3GPP TSG RAN WG1 #107-e			R1-2112385

e-Meeting, November 11th – 19th, 2021

Source:	WILUS Inc.

Title:	Remaining issues on channel access for NR from 52.6GHz to 71GHz

Agenda item:	8.2.6

Document for:	Discussion/Decision

- Introduction

There were many discussions from RAN1#104-e to RAN1#106bis-e meeting regarding channel access mechanism beyond 52.6GHz in many aspects such as LBT bandwidth in single carrier and CA, No-LBT mode and LBT to No-LBT switching, sensing structure for LBT mode, ED threshold adaptation based on BW, COT sharing aspects, whether to introduce CWS, CAPC, and Cat-2 LBT and multi-channel access. And the followings were agreed [1]-[5].

Agreement at RAN1#104-e: For Cat 2 LBT, down-select from the following alternatives - Alt 1: Do not introduce Cat 2 LBT for 60GHz unlicensed band operation - Alt 2: Introduce Cat 2 LBT for 60GHz unlicensed band operation Agreement at RAN1#104-e: If Cat 2 LBT is introduced, the following use cases can be further studied: - Resume transmission after a gap Y: Cat 2 LBT may be used to resume transmission by the initiating device within the COT after a gap Y (FFS the value of Y) - COT sharing: Cat 2 LBT may be used before transmission by a responding node sharing a COT - Multi-Beam LBT: Cat 2 LBT may be used before switching to a new transmission beam (not used in earlier part of the COT) in a COT with TDM beams, or resume a previously used transmission beam after a gap Z (FFS the value of Z) - Rx-Assistance: Cat 2 LBT may be used for sensing at the receiver as a responding device for Rx-Assistance measurements and associated signalling Other use cases not precluded. FFS if Cat 2 LBT is mandated for each use case or not. Agreement at RAN1#104-e: Define Type A and Type B multi-channel channel access as: - Type A: Perform independent eCCA for each channel - Type B: Identify a primary channel and perform eCCA on the primary channel, while perform Cat 2 LBT for other channels in the last observation slot Down-selection between - Alt1: Support Type A multi-channel channel access only - Alt2: Support both Type A and Type B multi-channel channel access. Note: How eCCA is performed on each channel, and the BW of the channels over which eCCAs are performed are separately discussed Working assumption at RAN1#104b-e: For energy measurement in 5us observation slot, when performing single measurement, the location of the measurement within the 5us is left for implementation, i.e., anywhere within the 5us. Agreement at RAN1#105-e: For regions where LBT is not mandated, gNB should indicate to the UE this gNB-UE connection is operating in LBT mode or no-LBT mode - Support both cell specific (common for all UEs in a cell as part of system information or dedicated RRC signalling or both) and UE specific (can be different for different UEs in a cell as part of UE-specific RRC configuration) gNB indication Agreement at RAN1#106-e: For energy measurement in 8us deferral period, Alt 2 is supported while Alt 1 and Alt 3 can be considered as gNB/UE implementation (Alt. 1/2/3 are defined as per previous agreement) Conclusion at RAN1#106-e: There is no consensus in RAN1 to support the functionality of accessing a carrier if there is interference in part of the carrier in frequency. Agreement at RAN1#106-e: - For LBT for single carrier transmission, gNB/UE performs LBT over the channel bandwidth (or BWP bandwidth) (Alt SC.1. in earlier agreements) - For LBT for multi-carrier transmission in intra-band CA, gNB/UE performs multiple LBT, one for each channel bandwidth separately (Alt CA.1. in earlier agreements) - FFS: Additional support of performing single LBT over all CCs (Alt CA.2. in earlier agreements) Agreement at RAN1#106-e: On COT sharing from an initiating device transmission to responding device transmission, support both of the following two alternatives - Alt 1: No maximum gap defined between the initiating device transmission and responding device transmission. A responding device transmission can occur without LBT with any gap within the maximum COT duration - Alt 3: Define a maximum gap Y, such that a responding device transmission can occur without LBT only if the transmission starts within Y from the end of the initiating device transmission. If the responding device transmission starts after Y from the end of the initiating device transmission, a Cat 2 LBT is needed before the responding device transmission. - The Cat 2 LBT uses the same sensing structure as the 8 us initial deferral period as in eCCA - Further down-select between the following options: - Option 1: Y=8 us (motivated by need to operate in all regions) - Option 2: Y=a multiple number of OFDM symbols - Option 3: gNB determines Y (for example, according to local regulation) - Cat. 2 LBT is a UE capability - The usage of the two alternatives is a gNB choice and depends at least on local regulations. Note: Alt. 3 is motivated by the regulations in Japan, but use of Cat. 3 LBT is also an option for operation in Japan and Cat. 2 LBT is not restricted for use only in Japan. Note: Maximum gap allowed without Cat 2 LBT between two initiating device transmissions is to be separately discussed Note: Other use cases of Cat 2 LBT will be separately discussed Conclusion at RAN1#106bis-e: There is no consensus to support explicitly introducing in the spec using single LBT covering multiple CCs under CA. - Note: This does not rule out gNB/UE implementation to perform single LBT to cover multiple CCs. However, the EDT needs to be selected such that if interference on one of the CCs exceeds the CC EDT, the LBT is declared as failed Conclusion at RAN1#106bis-e: There is no consensus to support per beam LBT mode or no-LBT mode UE specific gNB indication. Conclusion at RAN1#106bis-e: For regions where LBT is not mandated, there is no consensus to introduce L1 signalling for gNB to indicate to the UE if the operation is in LBT mode or no-LBT mode. Note this is different from the DCI field indicate the LBT type for UL transmission. Conclusion at RAN1#106bis-e: There is no consensus to introduce CWS Adjustment for unlicensed operation in FR2-2 Conclusion at RAN1#106bis-e: There is no consensus to introduce CAPC for unlicensed operation in FR2-2

In this contribution, we discuss channel access mechanism such as whether to introduce Cat-2 LBT and multi-channel access for unlicensed operation in NR beyond 52.6GHz and provide our views.

Remaining issues on channel access mechanism for NR beyond 52.6GHz

Whether to introduce Cat-2 LBT

Regarding introduction of Cat-2 LBT for 60GHz unlicensed band operation, from the RAN1#104-e to RAN1#106b-e meeting, it was discussed whether or not to define Cat-2 LBT in the 3GPP specification. It was agreed to further study the following use cases at the RAN1#104-e. It might be quite useful to apply Cat-2 LBT for the device at least for the use cases such as resuming transmission after a certain gap, multi-beam LBT in multiple beam operation, COT sharing between an initiating node and a responding node for the fair coexistence with other technologies or other RAT using unlicensed band. However, the detailed condition to apply Cat-2 LBT for each use case in 60GHz unlicensed band can be further discussed.

- Proposal 1: We support Alt-2 to introduce Cat 2 LBT for 60GHz unlicensed band operation.

Multi-channel access

According to the TS 37.213 v16.6.0, in Rel-16 NR-U, two types of multiple channel/carrier access were specified from LTE-LAA in Rel-13 to NR-U in Rel-16. However, from the regulation of ETSI BRAN, it does not currently be required to define Cat-2 LBT or channel bonding rule for multiple carriers. Therefore, we propose that at least Type A multi-channel access which performs independent clear channel assessment (CCA) for each channel should be supported. For the Type B multi-channel access, it should be further discussed after the decision on support of Cat-2 LBT including the definition of Cat-2 LBT.

- Proposal 2: At least Type A multi-channel access which performs independent clear channel assessment (CCA) for each channel should be supported. For support of the Type B multi-channel access, it should be further discussed after the decision on support of Cat-2 LBT including the definition of Cat-2 LBT.

Conclusion

In this contribution, we have discussed channel access mechanism such as whether to introduce Cat-2 LBT and multi-channel access for unlicensed operation in NR beyond 52.6GHz. As a conclusion, we summarize our view as follows:

- Proposal 1: We support Alt-2 to introduce Cat 2 LBT for 60GHz unlicensed band operation.

- Proposal 2: At least Type A multi-channel access which performs independent clear channel assessment (CCA) for each channel should be supported. For support of the Type B multi-channel access, it should be further discussed after the decision on support of Cat-2 LBT including the definition of Cat-2 LBT.

References

[1] Final Report of 3GPP TSG RAN WG1 #104-e meeting v1.0.0

[2] Final Report of 3GPP TSG RAN WG1 #104b-e meeting v1.0.0

[3] Final Report of 3GPP TSG RAN WG1 #105-e meeting v1.0.0

[4] Final Report of 3GPP TSG RAN WG1 #106-e meeting v1.0.0

[5] Draft Report of 3GPP TSG RAN WG1 #106b-e meeting v0.1.0