3GPP TSG RAN WG1 Meeting #107-e	                                                                          R1-2112461

Nov 11th –  19th, 2021

Agenda item:    7.2.2

Source:              Moderator (Qualcomm Incorporated)

Title:                  Preparation phase email discussion for NR-U

Document for:  Discussion and Decision

# Introduction

The paper summarizes the preparation phase email discussion for contribution submitted to 7.2.2 on NR-U CR under the following email thread

- [107-e-Prep-AI7.2.2] Preparation phase for Rel-16 NR-U maintenance: Jing (Qualcomm)

# Issues identified

This section lists the issues discussed in submitted papers. I tried to put them under technical proposal or editorial proposal categories, but I might be wrong though.

Issues may need technical discussion:

| Issue ID | Brief summary | Summary document | Supporting document |
| --- | --- | --- | --- |
| T1 | Channel access updates for MIIT | [2] | [1]. CR for 37.213 [3]. CR for 38.212 |
| T2 | CG-UCI multiplexing condition | [4] | [4]. CR for 38.212 |
| T3 | Freq hopping for single/multi-PUSCH | [5], [8] | [9]. CR for 38.214 |
| T4 | Channel access for consecutive UL transmission | [6] | [7]. CR for 37.213 |
| T5 | Correction on unit of CP extension | [12] | [12]. CR for 38.211 |
| T6 | UL transmission in wideband operation | [13] | [13]. CR for 37.211 |
| T7 | Discussion on LS from RAN4 on measing CSI-RS during SCell activation | [14] |  |
| T8 | Changes of channel access procedure in TS 37.213 | [16] | [16]. CR for 37.213 |
| T9 | On additional PDSCH DM-RS dropping with double symbol | [17] | [17]. CR for 38.211 |

Issues more editorial in nature:

| Issue ID | Brief summary | Summary document | Supporting document |
| --- | --- | --- | --- |
| E1 | Correct IE name for availableRB-SetsToReleaseList | [10] | [10]. CR for 38.213 |
| E2 | Type 3 HARQ codebook construction, change “HARQ process number h” to “HARQ process h” | [11] | [11]. CR for 38.213 |
| E3 | Correction on usage of subCarrierSpacingCommon for unlicensed | [15] | [15]. CR for 38.213 |
| E4 | Correction on UL channel access procedure Type 2A/2B/2C | [18] | [18]. CR for 37.213 |

# Discussion on which CR to treat

Please provide your view below. “Y” to discuss.

| Company | T1 | T2 | T3 | T4 | T5 | T6 | T7 | T8 | T9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MTK | Y | Y | Y | Y | Y | Y | Y | Y | Y |
| Ericsson |  | Y (OK with 1st change) | Y |  |  | Y | N | Y |  |
| LG Electronics |  | Y (but the second change doesn’t seem to be needed) | Y |  | Y | Y |  | Y | Y |
| Transsion | Y | Y | Y |  |  | Y |  | Y | N |
| Huawei, HiSilicon | Y, comment below | Y, response to LG below | Y |  | Comment below | Y |  | Y | resolved in RAN1#104, |
| CAICT |  | Y | Y |  | Y | Y |  | Y | Y |
| ZTE, Sanechips | Y(specific CR can be further discussed) | Y(fine with 1st change, and not understand the motivation of 2nd change) | Y |  | Y(can be further clarified) | Y |  | Y | Y |
| Intel |  | Y | Y |  |  | Y | Y | Y | Y |
| vivo |  | Y | Y | Y (Comment below) |  | Y |  | Y |  |
| Sharp |  |  |  |  | Y | Y |  | Y |  |
| Nokia, NSB | Y | Y | Y |  |  | Y |  | Y |  |
| Lenovo, Motorola Mob. | Y | Y | Y | Y | Y |  | Y (see Comment below) | Y |  |
| Samsung |  | Y | Y |  |  | Y |  | Y |  |
| Apple |  | Y | Y |  |  | Y | N (Comment below) | Y |  |
| WILUS |  | Y | Y |  |  | Y |  | Y | Y |
| OPPO |  | Y |  |  | Y | Y |  |  |  |

| Company | E1 | E2 | E3 | E4 |
| --- | --- | --- | --- | --- |
| MTK | Y | Y | Y | Y |
| Ericsson | Y | N | Y | N |
| LG Electronics | Y | Y | Y | Y |
| Transsion | Y | Y | Y | Y |
| Huawei, HiSilicon | Y | Y | Y | Comment below |
| CAICT | Y | Y | Y | Y |
| ZTE, Sanechips | Y | Y(but suggest to align name between iffered specs) | Y | Y |
| Intel | Y | Y | Y | Y |
| vivo | Y | Y | Y | Y (Comment below) |
| Sharp | Y |  | Y |  |
| Nokia, NSB | Y | Y | Y | Y |
| Lenovo, Motorola Mob. | Y |  |  | Y (see Comment) |
| Samsung | Y |  | Y | Y |
| Apple | Y |  | Y | Y |
| WILUS | Y | Y | Y | Y |
| OPPO | Y | Y | Y | Y |

Other comments:

| Company Name | Comments |
| --- | --- |
| MTK | For T9, we are trying to discuss the issue about additional PDSCH DM-RS dropping with double symbol. With the Rel-16 NR-U introduced Type B PDSCH duration ld ∈8,9,10,11,12,13: In some condition, only the second symbol of an additional double-symbol DM-RS is out of bound according to current Rel-16 RAN1 spec, and it breaks the orthogonality of orthogonal cover code (OCC) applied on the DM-RS. As a result, we prefer to adopt Alt 1 below (drop both symbols when the second symbol of DMRS is out of bound) to avoid this kind of scenario. |
| Ericsson | T7: This issue has been discussed for 3 meetings now (one of them in Rel-15 maintenance) without consensus. We see little point in continuing the discussion which will end up with the same result. |
| Transsion | T9: This issue has been discussed and identified at the RAN1#104-e meeting, and the intention of the final CR R1-2102009 is the same as that of MTK. In current specification, “for all values of the PDSCH duration ld other than 2, 5, and 7 symbols, the UE is not expected to receive DM-RS beyond the (ld - 1):th symbol” means that for double-symbol DMRS, the UE is not expected to receive the two DM-RS symbols. The language “DM-RS” highlighted in yellow in current spec is for simplicity and to be consistent with the description in “if the PDSCH duration ld is 12 or 13 symbols, the UE is not expected to receive DM-RS mapped to symbol 12 or later in the slot”. Hence, we believe that this issue should not be discussed again. |
| MTK2 | Thanks Transsion for pointing out that T9 is already discussed before and had a corresponding CR R1-2102009. The information is very helpful to us. We agree with Transsion that T9 is not needed to be discussed again in RAN1 #107e. |
| Huawei, HiSilicon | T1: The new regulation for 2.4GHz, 5.1GHz and 5.8GHz in China was formally issued on Sept 8. It will take effect from Jan 1 2022. Current FBE mechanism did not conform to the regulation and should be resolved. T2: As for the 2nd change, the current spec allocate all RE of PUSCH to CG-UCI, CSI-part- 1 and CSI-part-2. There is no space for CG-PUSCH. T5: We think this correction is inaccurate as Text is used for defining the time interval tstart,lμ-Text≤t<tstart,lμ on which the extension time-continuous signal sext(p,μ)t is generated from the discrete samples using Tc. Therefore, Δi should be also calculated in a time-continuous manner which also captures the gap duration accurately. Then, the calculation of the signal sl(p,μ)t takes care of the Tc time unit. E4: This clarifying editorial can be handled, if needed, without deletion |
| vivo | T4: We are trying to discuss the scenario that consecutive UL transmissions are scheduled by multiple UL grant(s) and one example is illustrated below: If following current spec below in 37.213, if Type 2B for PUSCH 1 failed, PUSCH2 and PUSCH3 will be transmitted by performing Type 2A LBT, which is not correct since it is already out of COT. If a UE is scheduled to transmit a set of UL transmissions using one or more UL grant(s) or DL assignment(s), and if the UE cannot access the channel for a transmission in the set prior to the last transmission according to one of Type 1, Type 2, or Type 2A UL channel access procedures, the UE shall attempt to transmit the next transmission according to the channel access type indicated in the corresponding UL grant or DL assignment. Otherwise, if the UE cannot access the channel for a transmission in the set prior to the last transmission according to Type 2B UL channel access procedure, the UE shall attempt to transmit the next transmission according to Type 2A UL channel access procedure. If looking at the agreement below, it only involves the UL transmission(s) by a single UL grant. For multiple UL grants, it should follow the channel access type in DCI. Agreement: For LBT type and CP extension, after failing to transmit first PUSCH(s) of a set scheduled by a single UL grant, - If a UE fails to access the channel with UL Type 2B channel access, Type 2A UL channel access shall be used for the following consecutively scheduled transmissions. - If a UE fails to access the channel prior to the first of the consecutive UL transmissions, it shall use “0” CP extension for the subsequent UL transmissions irrespective of the CP extension indicated in the scheduling grant. E4: This clarification editorial can be handled by adding ‘,’ before ‘where’. |
| Sharp | We think that T8 is more like editorial corrections. |
| Lenovo, Motorola Mobility | T7: CSI-RS measurement during SCell activation is also addressed by R1-2111846 submitted to AI 7.1, it might be better adressed there (if at all). E4: We consider this as an essential correction to remove ambiguity, and not just an editorial change. |
| Apple | T7: The discussion point in R1-2111846 is totally different with the issue T7, which clarifies the timeline of CSI reporting during SCell activation procedure based on the periodic CSI-RS. It has nothing related to PDCCH monitoring for the ‘to-be-activated’ SCell (i.e., T7). This issue was discussed several times in different sessions and unlikely to reach consensus |
| OPPO | T4: from 37.213 section 4.2.1.0.3, we understand that the condition for indication type 2 LBT should ensure that if there are consecutive UL transmissions without gap, all the transmissions should be contained within gNB COT. In this case, we think the example given in T4 the gNB is not allowed to indicate type 2B for the first PUSCH. T5: we suggest to discuss this CR and we disagree with Huawei’s comment. The spec should clearly define the UE operation for generating the OFDM symbol. If the Text is a continuous time, then we don’t know how to implement it as UE only has time domain samples (in unit of Tc). If you look at legacy NR OFDM symbol generation, the CP length is clearly defined in samples. So the Text should be, this guarantees the exact length of Text generated by UE, otherwise, the generated Text length would be different from the required length specified in specification. E2: in the pseudo-code for Type 3 CB generation, there is inconsistency between HARQ process and HARQ process number, we suggest to have an aligned and consistent description. E4: we support to discuss, we think without correction the UE behavior may be different from what has been agreed during the WI. |

# Conclusion

After email discussion, the following email discussion threads are assigned

[107-e-NR-NRU-01] Email discussion/approval on minimum sensing duration (Issue T1 in R1-2112461), until November 17 – Jiayin (Huawei)

[107-e-NR-NRU-02] Email discussion/approval on CG-UCI multiplexing in TS38.212 (Issue T2 in R1-2112461), until November 17 – Jiayin (Huawei)

[107-e-NR-NRU-03] Email discussion/approval on frequency hopping for single/multi PUSCH transmission (Issue T3 in R1-2112461), until November 17 – Gen Li (vivo)

[107-e-NR-NRU-04] Email discussion/approval on UL Transmissions in Wideband Operation (Issue T6 in R1-2112461), until November 17 – Steve (Ericsson)

[107-e-NR-NRU-05] Email discussion on editorial changes to be recommended to the specification editors on issues E1, E3, E4 and T8 in R1-2112461, until November 17 – Jing (Qualcomm)

# Reference

- R1-2110822, Changes of channel access procedure  in TS 37.213 according to MIIT regulation, Huawei, HiSilicon

- R1-2110823, Discussion on the impact of MIIT consultation to channel access procedure, Huawei, HiSilicon

- R1-2110824, Changes of channel access types tables in TS 38.212, Huawei, HiSilicon

- R1-2110825, Corrections on CG-UCI multiplexing in TS38.212, Huawei, HiSilicon

- R1-2110826, Discussion on the frequency hopping for single/multi PUSCH transmission, Huawei, HiSilicon

- R1-2110974, Discussion on channel access procedures for consecutive UL transmissions, vivo

- R1-2110975, Correction on channel access procedures for consecutive UL transmission(s), vivo

- R1-2110977, Discussions on frequency hopping for PUSCH,PUCCH and SRS, vivo

- R1-2110978, Draft CR on 38.214 on frequency hopping for multi-PUSCH scheduling by a single DCI, vivo

- R1-2111082, Correction on slot configuration in TS 38.213, ZTE, Sanechips

- R1-2111339, Correction on Type-3 HARQ-ACK codebook, OPPO

- R1-2111340, Draft CR for correction on unit of CP extension, OPPO

- R1-2111461, UL Transmissions in Wideband Operation, Ericsson, Nokia, NSB, LG Electronics, Qualcomm, Huawei, HiSilicon

- R1-2111462, Discussion on LS from RAN4 on measing CSI-RS during SCell activation, Ericsson

- R1-2111714, Correction on usage of subCarrierSpacingCommon for unlicensed, Samsung

- R1-2111927, Changes of channel access procedure in TS 37.213, Huawei, HiSilicon

- R1-2112294, On additional PDSCH DM-RS dropping with double symbol, MediaTek Inc.

- R1-2112350, Correction on UL channel access procedure Type 2A/2B/2C , Lenovo, Motorola Mobility, ZTE, Sanechips, Xiaomi, Intel, OPPO