3GPP TSG-RAN WG1 Meeting #107-e	R1-2112490

# Electronic Meeting, November 11th – 19th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | <CR#> | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of NR Sidelink enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2021-11-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Introduction of NR Sidelink enhancements |  |  |  |  |  |  |  |  |
| Summary of change: |  | In section 8.1.4, specified the operation of periodic-based partial sensing... In section 8.1.4A, specified the UE procedure for determining the set of preferred and/or non-preferred resources in inter-UE coordination In section 8.1.4B, specified the UE procedure for determining the presence of an expected/potential resource conflict in inter-UE coordination In section 8.1.4C, specified the UE procedure for combining the received resource set with its own sensing result in inter-UE coordination |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete support of NR Sidelink enhancements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4, 8.1.4A (new), 8.1.4B (new), 8.1.4C (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<omitted text>

### 8.1.4	UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

In resource allocation mode 2, the higher layer can request the UE to determine a subset of resources from which the higher layer will select resources for PSSCH/PSCCH transmission. To trigger this procedure, in slot n, the higher layer provides the following parameters for this PSSCH/PSCCH transmission:

-	the resource pool from which the resources are to be reported;

-	L1 priority, prioTX;

-	the remaining packet delay budget;

-	the number of sub-channels to be used for the PSSCH/PSCCH transmission in a slot, LsubCH;

-	optionally, the resource reservation interval, Prsvp_TX, in units of msec.

-	if the higher layer requests the UE to determine a subset of resources from which the higher layer will select resources for PSSCH/PSCCH transmission as part of re-evaluation or pre-emption procedure, the higher layer provides a set of resources (r0,r1,r2,…) which may be subject to re-evaluation and a set of resources (r0',r1',r2',…) which may be subject to pre-emption.

-	it is up to UE implementation to determine the subset of resources as requested by higher layers before or after the slot ri'' - T3, where ri'' is the slot with the smallest slot index among (r0,r1,r2,…) and (r0',r1',r2',…) , and T3 is equal to Tproc,1SL, where Tproc,1SL  is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP.

- 	Optionally, the indication of resource selection mechanism(s), as allowedResourceSelectionConfig, which may comprise of full sensing only, partial sensing only, random resource selection only, or any combination(s) thereof.

The following higher layer parameters affect this procedure:

-	sl-SelectionWindowList: internal parameter T2min is set to the corresponding value from higher layer parameter sl-SelectionWindowList for the given value of prioTX.

-	sl-Thres-RSRP-List: this higher layer parameter provides an RSRP threshold for each combination pi, pj, where pi is the value of the priority field in a received SCI format 1-A and pj is the priority of the transmission of the UE selecting resources; for a given invocation of this procedure, pj = prioTX.

-	sl-RS-ForSensing selects if the UE uses the PSSCH-RSRP or PSCCH-RSRP measurement, as defined in clause 8.4.2.1.

-	sl-ResourceReservePeriodList

-	sl-SensingWindow: internal parameter T0 is defined as the number of slots corresponding to sl-SensingWindow msec

-	sl-TxPercentageList: internal parameter X for a given prioTX is defined as sl-TxPercentageList (prioTX) converted from percentage to ratio

-	sl-PreemptionEnable: if sl-PreemptionEnable is provided, and if it is not equal to 'enabled', internal parameter priopre is set to the higher layer provided parameter sl-PreemptionEnable.

-	Optionally, minimum number of Y slots as Y_min (minNumCandidateSlots), which indicates the minimum number of Y slots that are included in the resources corresponding to periodic-based partial sensing.

-	Optionally, sensing occasion as periodicSensingOccasionReservePeriodList, which indicates the subset of periodicity values from sl-ResourceReservePeriodList used to determine periodic sensing occasions in periodic-based partial sensing. If not configured, all periodicity values from sl-ResourceReservePeriodList are used to determine periodic sensing occasions in periodic-based partial sensing.

-	Optionally, additional sensing occasions as additionalPeriodicSensingOccasion, which indicates that UE additionally monitors periodic sensing occasions that correspond to a set of values. The possible values of the set at least includes the most recent sensing occasion before the first slot of the candidate slots for a given reservation periodicity and the last periodic sensing occasion prior to the most recent one for the given reservation periodicity. If not configured, the UE monitors the most recent sensing occasion before the first slot of the candidate slots for the given periodicity used to determine periodic sensing occasions in periodic-based partial sensing.

-	Optionally, indication of the size of contiguous partial sensing window as contiguousSensingWindowPeriodic.

The resource reservation interval, Prsvp_TX, if provided, is converted from units of msec to units of logical slots, resulting in Prsvp_TX' according to clause 8.1.7.

When the resource pool is (pre-)configured with allowedResourceSelectionConfig including full sensing, the UE may perform full sensing.

When periodic reservation for another TB (sl-MultiReserveResource) is enabled for the resource pool, the resource pool is (pre-)configured with allowedResourceSelectionConfig including partial sensing, and partial sensing is (pre-) configured in the UE by higher layer, the UE may perform periodic-based partial sensing.

When a UE is triggered by higher layer to report resources for resource (re-)selection in a mode 2 Tx pool, the resource pool is (pre-)configured with allowedResourceSelectionConfig for partial sensing, and partial sensing is configured by higher layer, the UE may perform contiguous partial sensing.

Notation:

t'0SL, t'1SL, t'2SL,… denotes the set of slots which belongs to the sidelink resource pool and is defined in Clause 8.

The following steps are used:

1)	A candidate single-slot resource for transmission Rx,y is defined as a set of LsubCH contiguous sub-channels with sub-channel x+j in slot t'ySL where j=0,...,LsubCH-1. The UE shall assume that any set of LsubCH contiguous sub-channels included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource, where

-	selection of T1 is up to UE implementation under 0 ≤ T1≤ Tproc,1SL  , where Tproc,1SL  is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP;

-	if  T2min is shorter than the remaining packet delay budget (in slots) then T2 is up to UE implementation subject to  T2min ≤ T2 ≤ remaining packet delay budget (in slots); otherwise T2 is set to the remaining packet delay budget (in slots).

The total number of candidate single-slot resources is denoted by Mtotal.

2)	The sensing window is defined by the range of slots [n –T0,n–Tproc,0SL) where T0 is defined above and Tproc,0SL is defined in slots in Table 8.1.4-1 where μSL is the SCS configuration of the SL BWP. The UE shall monitor slots which belongs to a sidelink resource pool within the sensing window except for those in which its own transmissions occur. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots.

When the UE performs periodic-based partial sensing, the UE shall monitor slots at ty0-k×PreserveSL, where ty0SL is the first slot of the selected candidate slots. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots.

The value of Preserve corresponds to periodicSensingOccasionReservePeriodList if configured, otherwise, the values correspond to all periodicity from sl-ResourceReservePeriodList.

The UE monitors k sensing occasions determined by additionalPeriodicSensingOccasion, as previously described, and not earlier than n –T0.

When the UE performs contiguous partial sensing, the sensing window is defined by the range of slots [n+TA, n+TB], where TA and TB can be zero, positive or negative.

When the UE performs periodic-based partial sensing and contiguous partial sensing with periodic reservation for another TB (sl-MultiReserveResource) enabled, if Prsvp_TX≠0 , the sensing window is defined by the range of slots [n+TA, n+TB]. n+TA is M logical slots earlier than slot ty0SL, and n+TB is Tproc,0SL+Tproc,1SL slots earlier than ty0SL, where ty0SL is the first slot of the selected Y candidate slots of PBPS, and Tproc,0SL, Tproc,1SL are in units of physical time/slots. The value of M is (pre-)configured with the contiguousSensingWindowPeriodic. If contiguousSensingWindowPeriodic is not (pre-)configured, M equals to 31.

3)	The internal parameter Th(pi,pj)  is set to the corresponding value of RSRP threshold indicated by the i-th field in sl-Thres-RSRP-List, where i=pi+pj-1*8.

4)	The set SA is initialized to the set of all the candidate single-slot resources.

5)	The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions:

-	the UE has not monitored slot t'mSL in Step 2.

-	for any periodicity value allowed by the higher layer parameter sl-ResourceReservePeriodList and a hypothetical SCI format 1-A received in slot t'mSL with 'Resource reservation period' field set to that periodicity value and indicating all subchannels of the resource pool in this slot, condition c in step 6 would be met.

5a)	If the number of candidate single-slot resources Rx,y remaining in the set SA is smaller than X⋅Mtotal, the set SA is initialized to the set of all the candidate single-slot resources as in step 4.

6)	The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions:

a)	the UE receives an SCI format 1-A in slot t'mSL, and 'Resource reservation period' field, if present, and 'Priority' field in the received SCI format 1-A indicate the values Prsvp_RX and prioRX, respectively according to Clause 16.4 in [6, TS 38.213];

b)	the RSRP measurement performed, according to clause 8.4.2.1 for the received SCI format 1-A, is higher than ThprioRX,prioTX;

c)	the SCI format received in slot t'mSL or the same SCI format which, if and only if the 'Resource reservation period' field is present in the received SCI format 1-A, is assumed to be received in slot(s) t'm+q×Prsvp_RX'SL determines according to clause 8.1.5 the set of resource blocks and slots which overlaps with Rx,y+j×Prsvp_TX' for q=1, 2, …, Q and j=0, 1, …, Cresel-1. Here, Prsvp_RX' is Prsvp_RX converted to units of logical slots according to clause 8.1.7, Q=TscalPrsvp_RX  if Prsvp_RX< Tscal and  n'-m≤Prsvp_RX', where t'n'SL = n if slot n belongs to the set t'0SL,t'1SL,...,t'T'max-1SL, otherwise slot t'n'SL is the first slot after slot n belonging to the set t'0SL,t'1SL,...,t'T'max-1SL; otherwise Q=1. Tscal is set to selection window size T2 converted to units of msec.

7)	If the number of candidate single-slot resources remaining in the set SA is smaller than X⋅Mtotal, then Th(pi,pj) is increased by 3 dB for each priority value Th(pi,pj) and the procedure continues with step 4.

The UE shall report set SA to higher layers.

If a resource ri from the set (r0,r1,r2,…) is not a member of SA, then the UE shall report re-evaluation of the resource ri to higher layers

If a resource ri' from the set (r0',r1',r2',…) meets the conditions below then the UE shall report pre-emption of the resource ri' to higher layers

-	ri' is not a member of SA, and

-	ri' meets the conditions for exclusion in step 6, with ThprioRX,prioTX set to the final threshold after executing steps 1)-7), i.e. including all necessary increments for reaching X⋅Mtotal, and

-	the associated priority prioRX, satisfies one of the following conditions:

-	sl-PreemptionEnable is provided and is equal to 'enabled' and prioTX>prioRX

-	sl-PreemptionEnable is provided and is not equal to 'enabled', and prioRX<priopre and prioTX>prioRX

Table 8.1.4-1: Tproc,0SL depending on sub-carrier spacing

| μSL | Tproc,0SL [slots] |
| --- | --- |
| 0 | 1 |
| 1 | 1 |
| 2 | 2 |
| 3 | 4 |

Table 8.1.4-2: Tproc,1SL depending on sub-carrier spacing

| μSL | Tproc,1SL [slots] |
| --- | --- |
| 0 | 3 |
| 1 | 5 |
| 2 | 9 |
| 3 | 17 |

### 8.1.4A	UE procedure for determining a set of preferred and/or non-preferred resources for another UE’s transmission

A UE configured with the higher layer parameter interUECoordinationScheme1 enabling transmission of a preferred resource set considers [any candidate single-slot resource(s), as defined in clause 8.1.4] satisfying all the following conditions as preferred resources:

-	resource(s) excluding those overlapping with reserved resource(s) indicated by a received SCI format 1-A whose RSRP measurement is larger than an RSRP threshold.

-	if the UE is an intended receiver of transmission(s) from the UE(s) for which the preferred resource set is being determined, resource(s) excluding slot(s) in which the UE does not expect to perform SL reception due to half-duplex operation, unless this condition is disabled by the higher layer parameter condition1A2Scheme1Disabled.

When the inter-UE coordination information transmission is triggered by reception of an explicit request, the preferred resource(s) are determined in the same way as described in clause 8.1.4 with at least the following parameters indicated in the received explicit request:

-	Priority value to be used for PSCCH/PSSCH transmission replaces prioTX

-	Number of sub-channels to be used for PSCCH/PSSCH transmission in a slot replaces LsubCH

-	Resource reservation interval replaces Prsvp_TX

A UE configured with the higher layer parameter interUECoordinationScheme1 enabling transmission of a non-preferred resource set considers any resource(s) satisfying at least one of the following conditions as non-preferred resource(s):

-	resource(s) indicated by a received [SCI format 1-A], satisfying at least one of the following criteria:

-	the RSRP measurement performed, according to clause 8.4.2.1 for the received [SCI format 1-A], is higher than ThprioRX where prioRX is the value of the priority field in the received [SCI format 1-A]. The internal parameter Th(pi) is set to the corresponding value of RSRP threshold indicated by the i-th field in thresRSRPCondition1B1Option1Scheme1, where i=pi.

-	the UE would receive a TB ssociated with the received [SCI format 1-A] and the RSRP measurement performed, according to clause 8.4.2.1 for the received [SCI format 1-A], is lower than Th'prioRX where prioRX is the value of the priority field in the received [SCI format 1-A]. The parameter Th'(pi) is set to the corresponding value of RSRP threshold indicated by the i-th field in thresRSRPCondition1B1Option2Scheme1, where i=pi.

-	[slot(s)] in which the UE does not expect to perform SL reception from other UE due to half duplex operation, if the UE is an intended receiver of transmission(s) from the UE(s) for which the non-preferred resource set is being determined.

### 8.1.4B	UE procedure for determining a potential resource conflict

A UE configured with the higher layer parameter interUECoordinationScheme2 enabling transmission of a resource conflict indication considers that a potential resource conflict occurs on a reserved resource indicated by a received SCI if at least one of the following conditions is satisfied:

-	the reserved resource indicated by the received SCI overlaps with a resource indicated by another received SCI.

-	the reserved resource indicated by the received SCI occurs in a slot in which the UE does not expect to perform SL reception due to half-duplex operation and the UE is an intended receiver of the transmission in the reserved resource indicated by the received SCI.

### 8.1.4C	UE procedure for using a received resource set

A UE configured with the higher layer parameter interUECoordinationScheme1 enabling reception of preferred or non-preferred inter-UE co-ordination information uses a received resource set as follows when performing resource (re-)selection:

-	if the received resource set is a preferred resource set, the UE uses in its resource (re-)selection resource(s) belonging to the preferred resource set when the UE has no own sensing result, otherwise the UE uses in its resource (re-)selection resource(s) belonging to the preferred resource set in combination with its own sensing result.

-	if the received resource set is a non-preferred resource set, the UE excludes resource(s) overlapping with the non-preferred resource set.

### 8.1.5	UE procedure for determining slots and resource blocks for PSSCH transmission associated with an SCI format 1-A

The set of slots and resource blocks for PSSCH transmission is determined by the resource used for the PSCCH transmission containing the associated SCI format 1-A, and fields 'Frequency resource assignment', 'Time resource assignment' of the associated SCI format 1-A as described below.

'Time resource assignment' carries logical slot offset indication of N = 1 or 2 actual resources when sl-MaxNumPerReserve is 2, and N = 1 or 2 or 3 actual resources when sl_MaxNumPerReserve is 3, in a form of time RIV (TRIV) field which is determined as follows:

if N=1

TRIV=0

elseif N=2

TRIV=t1

else

if t2-t1-1≤15

TRIV=30t2-t1-1+t1+31

else

TRIV=3031-t2+t1+62-t1

end if

end if

where the first resource is in the slot where SCI format 1-A was received, and ti denotes i-th resource time offset in logical slots of a resource pool with respect to the first resource where for N = 2, 1≤t1≤31; and for N = 3, 1≤t1≤30, t1<t2≤31.

The starting sub-channel nsubCH,0start of the first resource is determined according to clause 8.1.2.2. The number of contiguously allocated sub-channels for each of the N resources LsubCH≥1 and the starting sub-channel indexes of resources indicated by the received SCI format 1-A, except the resource in the slot where SCI format 1-A was received, are determined from "Frequency resource assignment" which is equal to a frequency RIV (FRIV) where.

If sl-MaxNumPerReserve is 2 then

FRIV=nsubCH,1start+i=1LsubCH-1N subchannel SL+1-i

If sl-MaxNumPerReserve is 3 then

FRIV=nsubCH,1start+nsubCH,2start⋅N subchannel SL+1-LsubCH+i=1LsubCH-1N subchannel SL+1-i2

where

-	nsubCH,1start denotes the starting sub-channel index for the second resource

-	nsubCH,2start denotes the starting sub-channel index for the third resource

-	N subchannel SL is the number of sub-channels in a resource pool provided according to the higher layer parameter sl-NumSubchannel

If TRIV indicates N < sl-MaxNumPerReserve, the starting sub-channel indexes corresponding to sl-MaxNumPerReserve minus N last resources are not used.

The number of slots in one set of the time and frequency resources for transmission opportunities of PSSCH is given by Cresel where Cresel= 10*SL_RESOURCE_RESELECTION_COUNTER [10, TS 38.321] if configured else Cresel is set to 1.

If a set of sub-channels in slot t'mSL is determined as the time and frequency resource for PSSCH transmission corresponding to the selected sidelink grant (described in [10, TS 38.321]), the same set of sub-channels in slots t'm+j×Prsvp_TX'SL are also determined for PSSCH transmissions corresponding to the same sidelink grant where j=1, 2,…, Cresel-1, Prsvp_TX, if provided, is converted from units of msec to units of logical slots, resulting in Prsvp_TX' according to clause 8.1.7, and t'0SL, t'1SL, t'2SL,… is determined by Clause 8. Here, Prsvp_TX is the resource reservation interval indicated by higher layers.

<omitted text>