3GPP TSG RAN WG1 #107-e	R1-2112592

e-Meeting, November 11th – 19th, 2021

Agenda Item:	8.12.2

Source:	Moderator (Huawei)

Title:	FL summary#4 on improving reliability for MBS for RRC_CONNECTED UEs

Document for:	Discussion and Decision

# Introduction

This summary summarizes the contributions submitted in AI 8.12.2 to discuss how to improve the reliability for MBS for RRC_CONNECTED UEs.

This summary includes HARQ-ACK feedback specific issues, NACK-only specific issues, HARQ-ACK feedback common issues, HARQ-ACK feedback for SPS multicast, PDSCH repetition, and CSI feedback. In each of high level issue, a sub-level list of issues are organized.

For each of listed issue, proposal(s) is/are suggested from moderator’s perspective according to the submitted individual company’s proposal(s). Companies are welcome to make comments in the table “collect views”. The proposals may be updated in subsequent rounds according to the comments collected in previous rounds so as to strive to converge to consensus. Note that moderator may only tend to collect concerns when time is right for some specific rounds, for which companies are expected to only provide concerns in the table “collect concerns” if any instead of inputting views again and again to alleviate efforts.

People can use “navigation pane” to quickly overview the organization of the summary and proposal(s) for each issue for discussion and provide views/comments into the table of “collect view”/“collect concerns” under each proposal.

Note: for all proposals FL suggests, companies are encouraged to input views, situation can be known better, so as to progress fruitfully.

# ACK/NACK-based feedback specific

## (CLOSED)CB types/PUCCH structures

Submitted Proposals

“Different codebook types”

(Huawei) Proposal 1:

  - When UE is configured with different codebook types for unicast and multicast, support generating separate sub-codebooks and concatenating them by appending one sub-codebook to the other sub-codebook for both overlapping and non-overlapping cases.

(ZTE) Proposal 10:

  - For UE supports both unicast and multicast, different codebook types for unicast and multicast with the same priority can be configured, and it is up to gNB to avoid multiplexing between codebooks of different codebook types.

(vivo) Proposal 5:

  - The HARQ-ACK codebook types for unicast and multicast with the same priority can be different.

    - The HARQ-ACK codebook for unicast and multicast are separately constructed and then concatenated into one if they are multiplexed on one PUCCH.

(Nokia) Proposal 11:

  - The UE can be configured with different codebook types for unicast and multicast respectively with the same priority. The codebook is generated using the following alternative for the overlapping / non-overlapping PUCCHs cases in the same slot:

    - Alt2: support generating separate sub-codebooks and concatenating them by appending one sub-codebook to the other sub-codebook for both overlapping and non-overlapping cases.

(CATT) Proposal 12:

  - To configure one codebook for unicast and one codebook for multicast and the two codebooks are of different priorities, the current mechanism (i.e. different PUCCH-configs are configured for different priorities) can be applied without further enhancement.

(CATT) Proposal 13:

  - When UE is configured with different CB types for unicast and multicast respectively with the same priority, we support generating separate sub-codebooks and concatenating them by appending one sub-codebook to the other sub-codebook for both overlapping and non-overlapping cases.

(Xiaomi) Proposal 1:

  - A MBS UE can be configured with different CB types for unicast and multicast respectively.

(Xiaomi) Proposal 2:

  - A MBS UE can generate separate sub-codebooks for unicast and multicast, and concatenate them by appending one sub-codebook to the other sub-codebook for both overlapping and non-overlapping cases (alt2).

(DOCOMO) Proposal 3:

  - When a unicast HARQ-ACK codebook and a multicast HARQ-ACK codebook with the same priority are separately generated in the same PUCCH slot, they are concatenated into one codebook regardless of whether corresponding PUCCH resources are overlapped at least in a symbol or not.

(Qualcomm) Proposal 1:

  - In case that unicast and multicast with same priority are configured with different codebook types,

    - For overlapping case, UE is not expected to be configured with different codebook types for unicast and multicast with the same priority.

    - For non-overlapping case, support generating the separate codebooks with the same priority that are to be transmitted on two non-overlapping PUCCHs, subject to UE capability.

(Ericsson) Proposal 20:

  - When the UE is configured with separate pdsch-HARQ-ACK-CodebookList and the codebook types for multicast and unicast are different, and of the same priority, the UE sends feedback for both unicast and multicast by concatenating the unicast and multicast HARQ codebooks into one single codebook. The UE uses the last PRI received in the unicast assignments corresponding to the UL feedback slot.

“Different PUCCH structures”

- (Huawei) Proposal 2:

  - UE is not expected to be configured with different PUCCH structures. Alternatively, UE is not expected to be scheduled to feedback the HARQ-ACK with the same priority in the same PUCCH slot when configured with different PUCCH structures.

- (ZTE) Proposal 11:

  - For UE supports both unicast and multicast, different PUCCH structures for unicast and multicast with the same priority can be configured, and it is up to gNB to avoid overlapping between different PUCCH structures.

(CATT) Proposal 14:

  - UE is not expected to be configured with different PUCCH structure for unicast and multicast with the same priority and it should be captured into the specification.

(Qualcomm) Proposal 2:

  - In case that unicast and multicast with same priority are configured with different PUCCH structures,

    - UE is not expected to be configured with different PUCCH structures for unicast and multicast with the same priority in a slot.

“codebook for the same priority”

separate codebooks

(Spreadtrum) proposal 1:

  - For type-2 HARQ-ACK codebook, when unicast and multicast are with same priorities, separate codebooks should be generated.

(OPPO) Proposal 1:

  - For the case of same priority for multicast and uncast, separate codebook can be configured for multicast and unicast.

Single codebook

(Nokia) proposal 7:

  - Multiplexing of HARQ-ACK feedback of same priority is supported in case PUCCH transmissions are in the same slot, not only when the corresponding PUCCH resources overlap in time.

(CMCC) proposal 4:

  - Single codebook for unicast and multicast is constructed for the case of same priority, for Type-1 and Type-2 HARQ codebook.

- “PUCCH-config codebook”

- Separate config-> separate codebook

(OPPO) Proposal 2:

  - If PUCCH-config/PUCCH-ConfigurationList for unicast is also applied for MBS, HARQ-ACK bits of unicast and MBS with same priority are multiplexed and transmitted in the PUCCH of the same priority level.

(OPPO) Proposal 4:

  - If separate PUCCH-config/PUCCH-ConfigurationList is configured for MBS, HARQ-ACK codebook for MBS and unicast are constructed separately and transmitted in PUCCH resource dedicated for MBS or unicast.

- “Configuration for codebook type ”

(LGE) Proposal 5:

  - Clarify whether HP HARQ-ACK can be configured for multicast in case that HP HARQ-ACK is not configured for unicast.

### Round-1 (closed)

FL’s comment:

Since only Type-1 and Type-2 codebook are supported for NR MBS and unicast can support other codebook types than Type-1 or Type-2, restricting UE to be configured with the same codebook type for both unicast and multicast seems not desirable.

Assuming UE can be configured with different codebook types, regarding whether/how UE generates a single codebook for the same priority case, companies were invited to express the views on the following alternatives and some companies proceed with this discussion in this meeting. Based on the review of contributions, here is the situation:

- Alt1: support generating the separate CBs with the same priority that are to be transmitted on two non-overlapping case. UE is not expected to be configured with different CB types for the overlapping cases.

  - Supported by: Qualcomm

- Alt2: support generating separate sub-codebooks and concatenating them by appending one sub-codebook to the other sub-codebook for both overlapping and non-overlapping cases.

  - Supported by: Huawei, vivo, Nokia, CATT, Xiaomi, DOCOMO, Ericsson

- Alt3: UE is not expected to be configured with different CB types for unicast and multicast with the same priority and it is to be captured into the spec.

  - Supported by: None (from contributions)

- Alt4: Up to network to configure to avoid. No spec impact.

  - Supported by: None (from contributions)

- Alt5: The UE is not scheduled with unicast and multicast PUCCHs in the same slot with same priority, if unicast and multicast are configured to with CB types.

  - Supported by: ZTE

Based on the majority view, instead of leaving more options open given it is the last meeting, the following proposal is suggested:

#### Proposal 2.1.1-1

When UE is configured with different codebook types for unicast and multicast and when UE is scheduled to multiplex HARQ-ACK for unicast and HARQ-ACK for multicast with the same priority in the same PUCCH slot,

- UE generates two separate sub-codebooks for unicast and multicast respectively and then concatenates them by appending sub-codebook for multicast to the sub-codebook for unicast.

  - Note: The PUCCH resource for transmitting the codebook is based on the last unicast DCI.

Collect views:

| Company | Comments |
| --- | --- |
| - Lenovo, Motorola Mobility | - Agree. |
| - LG Electronics | - We are fine with this proposal. |
| - CATT | - Agree. |
| - CMCC | - Agree |
| - Nokia, NSB | - Agree |
| - vivo | - Agree |
| - Xiaomi | - Support |
| - NEC | - Agree |
| - Samsung | - Support |
| - NTT DOCOMO | - Support |
| - Ericsson | - Support with the following clarification: - The proposal should only apply to ACK/NACK-based HARQ-ACK feedback, since NACK-only codebook cannot be appended with unicast codebook. |
| - Intel | - Support |

FL’s comment:

Another question was asked in the last meeting to collect companies’ views regarding different PUCCH structures for unicast and multicast. Since sub-slot PUCCH is probably not supported for multicast and slot-based PUCCH is supported for unicast, can UE be configured with slot-based PUCCH for multicast and sub-slot based PUCCH for unicast and scheduled to multiplex the HARQ-ACK for unicast and for multicast with the same priority? If yes, how is the codebook generated? The following is observed from contributions:

- Alt1: UE is not expected to be configured with different PUCCH structure for unicast and multicast with the same priority and it is to be captured into the spec.

  - Supported by: Huawei, CATT, Qualcomm

- Alt2: up to network to configure to avoid such configuration. No spec impact.

  - Supported by: None (from contribution)

- Alt3: UE is not scheduled to feedback the HARQ-ACK with the same priority in the same PUCCH slot when configured with different PUCCH structures.

  - Supported by: Huawei, ZTE

#### Proposal 2.1.1-2

UE is not expected to be configured with different PUCCH structures for unicast and multicast for which the HARQ-ACK are with the same priority and are to be multiplexed.

Collect views:

| Company | Comments |
| --- | --- |
| - Lenovo, Motorola Mobility | - Agree. |
| - LG Electronics | - We are fine with this proposal. |
| - CATT | - Agree with the proposal. |
| - CMCC | - Agree |
| - Nokia, NSB | - Agree |
| - NEC | - Support. If non-overlapping HARQ-ACK codebooks with the same priority for unicast and multicast do not exist in a same slot, it means that UE always multiplexes the two codebooks to one codebook, thus we suggest delete ‘and are to be multiplexed’ in proposal 2.1.1-2. #### Proposal 2.1.1-2 UE is not expected to be configured with different PUCCH structures for unicast and multicast for which the HARQ-ACK are with the same priority and are to be multiplexed. |
| - Samsung | - Support |
| - NTT DOCOMO | - Support |
| - Qualcomm | - Is it just for the case that PUCCH for unicast and multicast are in the same slot? - If not in the same slot, do we still need to have this restriction. |
| - Apple | - Ok with this proposal |
| - ZTE | - We support Alt.3. - We think it would be too restrictive if network can’t configure different PUCCH slot lengths for unicast and multicast. Note that currently, only slot-length is supported for multicast feedback. If UE is configured with sub-slot for unicast for both high and low priority, then it basically means this UE can’t be configured with multicast in this case. - We think the more reasonable way is Alt.3, i.e., UE can be configured with different PUCCH slot length for unicast and multicast for the same priority, however network will guarantee UE is not scheduled to feedback the HARQ-ACK with the same priority in the same PUCCH slot when configured with different PUCCH structures. |
| - OPPO | - Support |
| - MediaTek | - Support the proposal |
| - FL’s response | - @NEC, Qualcomm and ZTE - “and are to be multiplexed” are intended and useful to be kept which can also address Qualcomm and ZTE’s question/comment, i.e., actually only when the HARQ-ACK is the same priority and are to be multiplexed into the same slot, there is such restriction. If they are not multiplexed in the same slot, then there is no such restriction. |
| - ZTE | - Thanks FL for the clarification. - With FL’s clarification above, it seems the proposal is exactly what Alt.3 means. However, the wording may not be that clear. Thus, we propose to directly use the Alt.3 wording. |

## (UPDATED)Type-1 HARQ codebook

Submitted Proposals

“FDM-ed for more than one G-RNTI”

(ZTE) Proposal 8:

  - Regarding Type-1 codebook construction in FDM-ed manner for more than one G-RNTI scheduling, support one of the following Alternatives,

    - Alt1: Type-1 codebook for multicast is generated jointly for all G-RNTIs.

      - If more than one G-RNTI scheduling in FDM-ed is supported, multiple HARQ-ACK bits should be generated for the SLIV group for multicast.

    - Alt2: Type-1 codebook for multicast is generated per G-RNTI and then all these sub-codebooks are concatenated into one codebook.

(Nokia) Proposal 12:

  - When FDM-ed transmissions of unicast and multicast / broadcast services are allowed, the UE constructs separate Type-1 HARQ-ACK sub-codebooks using Rel-15 / 16 mechanisms for each multicast / broadcast service.

(Nokia) Proposal 13:

  - The UE concatenates the constructed Type-1 sub-codebooks and sends them in the same PUCCH resource in case their HARQ-ACK feedback is scheduled for the same time instance.

(Nokia) Proposal 14:

  - The order of concatenation of the MBS sub-codebooks to construct a HARQ-ACK codebook, when the HARQ-ACK feedback of different services are scheduled for the same time instance, follows the increasing order of the G-RNTI values that are used to map PDSCH HARQ-ACK to MBS sub-codebook. MBS sub-codebooks are preceded by unicast sub-codebook, as agreed before.

(Nokia) Proposal 15:

  - RAN1 decides whether the UE produces HARQ-ACK bits for all the multicast services that the UE had indicated interest or only for services that the UE has received PDSCH(s) when constructing multiplexed Type-1 codebook for FDM-ed unicast / multicast case.

(Nokia) Proposal 16:

  - To overcome the problem of HARQ-ACK codebook size ambiguity, the constructed HARQ-ACK sub-codebooks are prepended by the UE with the information (with a known size) about which service the following sub-codebook belongs to using the following method:

    - The received services can be indexed, and that index can be prepended to the HARQ-ACK sub-codebook. The services can be indexed based on the order of RNTIs. For instance, unicast can be of index 0, the service with minimum configured G-RNTI value is index 1, service with second minimum G-RNTI is index 2 ….

(CATT) Proposal 15:

  - when the number of services configured including unicast and multicast is greater than number of FDM-ed PDSCHs reported by UE, the enhancement on Type-1 HARQ-ACK codebook generation shall be studied.

(LGE) Proposal 22:

  - For Type-1 HARQ-ACK codebook construction for FDM-ed unicast and multicast, HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells for unicast, precede, HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells where a CFR is configured for an interested service for multicast

“TDM-ed cases”

(CMCC) Proposal 5:

  - For TDM-ed unicast and multicast Type-1 HARQ-ACK codebook construction to be multiplexed in the same PUCCH resource, which PDSCH reception candidate occasions is determined based on Alt 1, UE determines a set of  occasions for candidate PDSCH receptions or SPS PDSCH releases in descending order of the slot timing values in K1 set, and for each K1 in the K1 set, corresponding row index set R of the TDRA table is determined as Alt 1.

Others

(Intel) Proposal 5:

  - If UE is configured one CB for multicast and one CB for unicast of different priorities, the pdsch-HARQ-ACK-Codebook configuration can include a priority indicator which applies to all multicast codebooks.

### Round-1 (closed)

FL’s Comments

The earlier agreement on Type-1 codebook construction for FDM-ed unicast and multicast with the same priority as follows at least applies to the case of one G-RNTI for multicast as shown in the following figure and procedure in the text box:

Agreement:

For Type-1 HARQ-ACK codebook construction for FDM-ed unicast and multicast with the same priority from the same TRP, support

- Opt 4: HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells for unicast, precede, HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells for multicast. (This is similar to the joint Type-1 codebook for mTRP).

- FFS: If UE reports the capability of supporting the FDM-ed unicast and multicast in the same slot, UE can be indicated semi-statically to generate Type-1 HARQ-ACK codebook as FDM-ed manner (i.e., Opt 4).

  - Otherwise, UE does not expect unicast and multicast are to be scheduled in FDM-ed.

Step1- Generate a sub-codebook#1 for unicast within all the slots, Step2- Generate another sub-codebook#2 for multicast within all the slots, Step3- Append the sub-codebook#2 to sub-codebook#1. In the following figure, the order of joint codebook should be (A, B, C, D, E, F, G, G)

Now when UE is configured with multiple G-RNTIs for multicast, the question is whether the above agreement applies or anything new needs to be agreed. Note that the current 38.213 draft CR is drafted per G-RNTI and the multiple G-RNTI case needs more discussion/agreement.

As discussed in the last meeting and also proposed in this meeting by some companies (ZTE, Nokia, LGE),  the above agreement can still hold with clarification that the multicast within the agreement includes the case of multiple G-RNTI configured to UE for which the sub-codebook for multicast constitutes the sub-codebooks for each G-RNTI by appending one to another in ascending order of G-RNTI value. The maximum numbers of G-RNTI configured to UE is up to UE capability which will be discussed in UE features.

Step1- Generate a sub-codebook#1 for unicast within all the slots, Step2- Generate another sub-codebook#2 for multicast within all the slots, - Generate sub-codebook#2-1 for G-RNTI values 1 - Generate sub-codebook#2-2 for G-RNTI values 2 - Append sub-codebook#2-2 to sub-codebook#2-1 Step3- Append the sub-codebook#2 to sub-codebook#1. In the following figure, the order of joint codebook should be (A, B, C, D, E, F, G, H)

#### Proposal 2.2.1-1

For the Type-1 codebook construction for FDM-ed unicast and multicast via Opt 4 (from the previous agreement), when UE is configured with multiple G-RNTIs and UE is configured with fdmed-Reception-Multicast, the sub-codebook for multicast consists of the sub-codebooks for each G-RNTI by appending one to another in ascending order of G-RNTI value.

- Note: The maximum numbers of G-RNTI configured to UE is up to UE capability which will be discussed in UE features.

Collect views:

| Company | Comments |
| --- | --- |
| - Lenovo, Motorola Mobility | - OK. |
| - LG Electronics | - We are generally fine with this proposal. Considering ‘for all serving cells for multicast‘ in Opt 4, the proposal could be revised to the following because RAN1 agree that G-RNTI is configured per serving cell and therefore this proposal would work for a serving cell where multiple G-RNTIs are configured, not for a serving cell where only one G-RNTI or no G-RNTI is configured, when UE is configured with multiple G-RNTIs for multiple serving cells: For the Type-1 codebook construction for FDM-ed unicast and multicast via Opt 4 (from the previous agreement), when UE is configured with multiple G-RNTIs for a serving cell and UE is configured with fdmed-Reception-Multicast, the sub-codebook for multicast consists of the sub-codebooks for each G-RNTI by appending one to another in ascending order of G-RNTI value for the serving cell. - Note: The maximum numbers of G-RNTI configured to UE is up to UE capability which will be discussed in UE features. - FL’s response: not necessarily limited to a serving cell because the same G-RNTI can be configured to multiple serving cells though it is configured per serving cell. |
| - CATT | - Ok with Proposal 2.2.1-1. |
| - CMCC | - Ok |
| - Nokia, NSB | - Ok with the intention of the proposal. - However, in case F belongs to G-RNTI 2 and H belongs to G-RNTI 1, would the codebook look like: (A, B, C, D, E, G, F, H)? - FL’s response: yes. I assume so. - What if there are some SLIV values where the UE did not receive any PDSCH, which codebook do they belong to? - FL’s response: This is Type-1 codebook, as long as there is PDSCH occasion for one SLIV, although UE did not receive any PDSCH, UE will report with NACK. - Instead, assuming that G-RNTI 1 and 2 use the same TDRA table and k1 set (different than unicast), the constructed codebook can be: - (A, B, C, D, E, F, G, H, E, F, G, H) - FL’s response: If two G-RNTIs has the same TDRA and K1, since only one parameter fdmed-Reception-Multicast representing FDM-ed unicast and multicast, though with unicast, G-RNTI1, and G-RNTI2 FDM-ed is not the case being considered here. From UE perspective, if on one PDSCH occasion from one SLIV nothing is received assuming PDSCH sent by NW is missing, but UE does not know which GRNTI the PDSCH is belonging to when fill NACK in the sub-codebook for a G-RNTI, so I will assume in this case, it is always (A, B, C, D, E, F, G, H, E, F, G, H) - This is also easily adopted to a case where G-RNTIs use separate TDRA tables with different k1 sets. - FL’s response: Follow the same logic, whether different G-RNTI has the same or different TDRA for Type-1 does not make difference. - The other option would be to create a joint codebook for separate G-RNTIs (if they use the same TDRA tables and k1 sets), since they cannot be FDM-ed in a slot. This would again lead to (A, B, C, D, E, F, G, H) - FL’s response: as explained, when considering multiple G-RNTIs, the codebook will be always (unicast sub-codebook, GRNTI1 sub-codebook, G-RNTI2 sub-codebook, … ) assuming this fdmed-Reception-Multicast parameter only represents FDM-ed unicast and multicast. However, this proposal still holds. |
| - vivo | - ok |
| - Xiaomi | - Support. It is a straightforward mechanism. |
| - NEC | - Support this proposal in principle. Just one question should be clarified. In the illustration shown by moderator, if the positions of PDSCH occasion F and G are changed, should the joint codebook be - - Comprehension 1: (A B C D E G F H), or - - Comprehension 2: (A B C D E F G H)? - -> FL’s response: please check the responses to Nokia above. It is eventually up to the TDRA for a specific G-RNTI. Maybe I can further clarify that the sub-codebook for each G-RNTI is generated per its K1 and TDRA - We think Comprehension 1 is in line with the agreement in previous discussion(‘… over all the slots’), but the proposal 2.2.1-1 may not clearly indicate which comprehensions is right. Therefore we suggest modify the proposal as, #### Proposal 2.2.2-1 For the Type-1 codebook construction for FDM-ed unicast and multicast via Opt 4 (from the previous agreement), when UE is configured with multiple G-RNTIs and UE is configured with fdmed-Reception-Multicast, the sub-codebook for multicast consists of the sub-codebooks for each G-RNTI over all the slots by appending one to another in ascending order of G-RNTI value. - Note: The maximum numbers of G-RNTI configured to UE is up to UE capability which will be discussed in UE features. |
| - Samsung | - Support |
| - Ericsson | - Support |
| - Qualcomm | - Ok in principle |
| - NTT DOCOMO | - Support in principle |
| - Intel | - OK |
| - Apple | - Ok with this proposal |
| - ZTE | - There are two different alternatives to address this issue. - Alt1: Type-1 codebook for multicast is generated jointly for all G-RNTIs. - Alt2: Type-1 codebook for multicast is generated per G-RNTI and then all these sub-codebooks are concatenated into one codebook. - Alt.2 is simpler and is forward compatible if FDMed multicast PDSCHs are supported in the future. However, Alt.2 has higher feedback overhead since it has to generate feedbacks for the same SLIV for different G-RNTI. Alt.1 can reduce the HARQ-ACK overhead as mentioned by companies above. Thus, we slightly prefer Alt.1. - FL’s response: Please check the discussion with Nokia whether we are on the same page. - Since we have agreed FDM-ed unicast and multicast, the sub-codebooks are concatenated. In light of this, Alt2 is more nature. In addition, I am not sure whether Alt1 can reduce the overhead. If considering multiple G-RNTI but only allowing one G-RNTI is FDM-ed with unicast, UE does not know which G-RNTI the PDSCH belongs to when it is missing since it is dynamically changed which G-RNTI is FDM-ed with unicast in a given slot. So eventually Alt1/2 may lead to the same CB size. |
| - OPPO | - OK |
| - MediaTek | - Ok with the proposal |
| - FL’s response: | - Based on the above discussion: the proposal still holds and would be clearer with the following clarification in red: #### Proposal 2.2.1-1 For the Type-1 codebook construction for FDM-ed unicast and multicast via Opt 4 (from the previous agreement), when UE is configured with multiple G-RNTIs and UE is configured with fdmed-Reception-Multicast, the sub-codebook for multicast consists of the sub-codebooks for each G-RNTI by appending one to another in ascending order of G-RNTI value. - The sub-codebook for each G-RNTI is generated per the k1 and TDRA configurations for the same G-RNTI as the legacy procedure. - Note: The maximum numbers of G-RNTI configured to UE is up to UE capability which will be discussed in UE features. |

FL’s Comments

Likewise, for TDM-ed unicast and multicast, for Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, further clarification is also needed whether the previous agreement as follows still holds or anything new needs to be agreed. Note that the current 38.213 draft CR is drafted per G-RNTI and the multiple G-RNTI case needs more discussion/agreement.

Agreement:

For TDM-ed unicast and multicast, for Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, determining PDSCH reception candidate occasions can be configured between the following alternatives from the previous agreement:

- Alt 1:

  - for slot timing values K1 in the intersection of K1 set for unicast (termed set A) and K1 set for multicast (termed set B), based on union of the PDSCH TDRA sets,

  - for slot timing values K1 in set A but not in set B, based on PDSCH TDRA set for unicast, and

  - for slot timing values K1 in set B but not in set A, based on PDSCH TDRA set for multicast.

- Alt 2: for slot timing values K1 in the union of K1 set for unicast and K1 set for multicast, based on the union of the PDSCH TDRA sets.

- Support of Alt. 1 is a UE capability

As discussing the draft CR, when UE is configured with multiple G-RNTI, whether the k1 set or the TDRA table is G-RNTI specific or applicable to all G-RNTI affects the procedure how Type-1 codebook is generated, especially for TDM-ed unicast and multicast. If the k1 set or TDRA table is configured per G-RNTI and is different from each other, the Type-1 codebook for TDM-ed unicast and multicast with multiple G-RNTIs is quite complex especially for Alt1. As noted by editor, the current draft CR can extend to multiple G-RNTI naturally if the k1 set and TDRA when configured are applicable to all configured G-RNTIs. No proposals from any companies but they are essential for WI complete, so FL suggests taking the simpler solution by the following proposals.

#### Proposal 2.2.1-2

For UE configured with multiple G-RNTIs to generate Type-1 codebook, when UE is not provided fdmed-Reception-Multicast but provided Type1-Intersection, the same TDRA table applies to all G-RNTIs if configured.

Collect views:

| Company | Comments |
| --- | --- |
| FL’s explanation | This proposal aims to simplify the Type-1 codebook generation for TDM-ed case if UE is configured with multiple G-RNTIs by applying the same TDRA table to all configured G-RNTIs. |
| Lenovo, Motorola Mobility | One question for clarification: Is same TDRA table configured for all G-RNTIs of the UE? - FL’s response: it is what the proposal says, so yes. |
| CMCC | We are fine with the intention of this proposal. But in AI 8.12.1, we had the agreements to support one CFR per active BWP in Rel-17 and only one PDSCH-config (one TDRA table) for MBS is configured in CFR, from this point of view, the only one TDRA table is applied for multiple G-RNTIs. We want to see other companies’ views whether the following two agreements have covered this proposal 2.2.1-2. - FL’s response: this makes sense to me. Agreement: From RAN1 perspective, the CFR (common frequency resource) for multicast of RRC-CONNECTED UEs, which is confined within the frequency resource of a dedicated unicast BWP and using the same numerology (SCS and CP), includes the following configurations: - Starting PRB and the number of PRBs - One PDSCH-config for MBS (i.e., separate from the PDSCH-Config of the dedicated unicast BWP) - One PDCCH-config for MBS (i.e., separate from the PDCCH-Config of the dedicated unicast BWP) - SPS-config(s) for MBS (i.e., separate from the SPS-Config of the dedicated unicast BWP) - FFS: Other configurations and details including whether signaling of starting PRB and the length of PRBs is needed when CFR is equal to the unicast BWP - FFS: Whether a unified CFR design is also used for broadcast reception for RRC_IDLE/INACTIVE and RRC_CONNECTED - FFS: Whether Coreset(s) for CFR in addition to existing Coresets in UE dedicated BWP is needed - Note: The terminology of CFR is only aiming for RAN1 discussion, and the detailed signaling design is up to RAN2 - Note: This agreement does not negate any previous agreements made on CFR Agreement: The number of CFRs for multicast is no more than one per dedicated unicast BWP in Rel-17. |
| Nokia, NSB | Support the proposal, considering the limited time in the WI. |
| NEC | We share the same view with CMCC. We should confirm whether there is only one TDRA table configured for multiple G-RNTIs or not first. If yes, it seems the agreement reached in the last e-meeting can be applied to the multiple G-RNTI case. |
| Samsung | Support. That issue was also mentioned in past discussions and was an (additional) argument against Alt.1. |
| NTT DOCOMO | Support |
| - Ericsson | - Support |
| - Qualcomm | It is better to add ‘in a serving cell’ to address the CMCC’s concern. The G-RNTI(s) configured in a serving cell will use the TDRA configured in the CFR-Config-Multicast included in the active BWP of the serving cell. #### Proposal 2.2.1-2 - For UE configured with multiple G-RNTIs to generate Type-1 codebook, when UE is not provided fdmed-Reception-Multicast but provided Type1-Intersection, the same TDRA table applies to all G-RNTIs if configured in a serving cell. - Not sure whether it helps, since TDM-ed Type-1 is generated per serving cell first. For a given serving cell, only one CFR per active BWP for multiple G-RNTI if configured, so TDRA included in PDSCH-config will naturally apply to all G-RNTIs as CMCC explained. |
| - Apple | Ok with this proposal |
| - ZTE | We are ok with the intention of this proposal. However, we don’t understand why we need to add the condition “when UE is not provided fdmed-Reception-Multicast but provided Type1-Intersection”. For all other cases, the same TDRA table should also apply to all G-RNTIs for multicast. Thus, we have the following update. For UE configured with multiple G-RNTIs to generate Type-1 codebook, when UE is not provided fdmed-Reception-Multicast but provided Type1-Intersection, the same TDRA table applies to all G-RNTIs for multicast if configured. - This was intended to solve how to get the intersection of k1 sets for multiple G-RNTIs case. As CMCC explained, anyhow TDRA is applied to all G-RNTIs if configured, and then yes, no need to have ““when UE is not provided fdmed-Reception-Multicast but provided Type1-Intersection”.”. Then this proposal is not really needed. |
| - OPPO | Agree with the intention of the proposal, however, we tend to agree with CMCC, it seems more proper to make it as a conclusion as it has already ben implied by previous agreements. |
| - MediaTek | We have the similar view with CMCC/NEC. Before we discuss the proposal, we should clarify whether the same TDRA table is used for multiple G-RNTIs. |
| - FL’s response | Based on the above discussion, we either update the proposal as follows for conclusion or drop this proposal assuming the previous agreement (and assuming TDRA is still included in PDSCH-config for multicast) implies the same thing. #### Proposal 2.2.1-2-r1 (for conclusion) For a given serving cell, the same TDRA table applies to all G-RNTIs if configured for UE supports multicast. |

#### Proposal 2.2.1-3

PUCCH-Config for ACK/NACK based feedback for multicast if separately configured from unicast, the PUCCH-Config is applied to all G-RNTIs with the same priority.

- Note: The dl-DataToUL-ACK is included in PUCCH-Config

Collect views:

| Company | Comments |
| --- | --- |
| FL’s explanation | This proposal aims to simplify the Type-1 codebook generation for TDM-ed case if UE is configured with multiple G-RNTIs by applying the same k1 (dl-DataToUL-ACK) set to all configured G-RNTIs. Since k1 is included in PUCCH-config, a more generic proposal is suggested that PUCCH-Config is applied to all configured G-RNTIs. |
| Lenovo, Motorola Mobility | One question for clarification: Is same K1 set configured for all G-RNTIs of the UE? - FL’s response: the intention was to make the same k1 for all configured G-RNTIs for a given serving cell. |
| CMCC | Support. |
| Nokia, NSB | Support the proposal, considering the limited time in the WI. |
| NEC | Support. There is no need to configure PUCCH-Config for each G-RNTI. |
| Samsung | Support. The specification/complexity benefit is clear and the restriction does not have a practical impact. |
| NTT DOCOMO | Support |
| - Ericsson | - Support |
| - Qualcomm | Questions for clarification: The PUCCH-Config is configured in a BWP-UplinkDedicated. The G-RNTI(s) is(are) configured for all the DL BWP(s) in a serving cell. Shall we clarify the PUCCH-Config is applied to all G-RNTIs with same priority in the same serving cell? - What about PUCCH-ConfigCommon? Is PUCCH-ConfigCommon applied to unicast C-RNTI as well as multicast G-RNTIs. - FL’s response: I was assuming PUCC-config is always for the same priority in light of agreement made for PUCC-configurationList including two PUCC-configs addressing low and high priority respectively . Ok to clarify for the same given serving cell. - This proposal mainly for dedicated PUCCH-config for multicast and if is configured, it is applied to all G-RNTIs. - Earlier we discussed if PUCCH-config for multicast is not configured, PUCCH-config for unicast applies. Your question here seemingly asks what if PUCCH-config for multicast and unicast are neither configured. This could be separately discussed. I welcome the answer to this question since I have no idea how to make the proposal right now because no company discussed it in their paper if I did not miss anything. |
| - Apple | Ok with this proposal |
| - ZTE | We support the proposal. Regarding the note, is its intention to say that the same K1 candidate values are applicable to all G-RNTIs? |
| - OPPO | Support |
| - MediaTek | Support the proposal. |

### Round-2 (closed)

#### Proposal 2.2.2-1

For the Type-1 codebook construction for FDM-ed unicast and multicast via Opt 4 (from the previous agreement), when UE is configured with multiple G-RNTIs and UE is configured with fdmed-Reception-Multicast, the sub-codebook for multicast consists of the sub-codebooks for each G-RNTI by appending one to another in ascending order of G-RNTI value.

- The sub-codebook for each G-RNTI is generated per the k1 and TDRA configurations for the same G-RNTI as the legacy procedure.

- Note: The maximum numbers of G-RNTI configured to UE is up to UE capability which will be discussed in UE features.

Collect views:

| Company | Comments |
| --- | --- |
| - FL’s explanation | - Please check the discussion and FL’s responses for Round-1 to get why the proposal is written as is. |
| - Samsung | - Support |
| - Qualcomm | - ok |
| - ZTE | - Regarding FL’s previous comments “In addition, I am not sure whether Alt1 can reduce the overhead. If considering multiple G-RNTI but only allowing one G-RNTI is FDM-ed with unicast, UE does not know which G-RNTI the PDSCH belongs to when it is missing since it is dynamically changed which G-RNTI is FDM-ed with unicast in a given slot. So eventually Alt1/2 may lead to the same CB size.” If Alt.1 is adopted, we don’t think we need to differentiate whether such a bit is for which G-RNTI, as long as the PDSCH is missing, UE just indicates NACK in this placeholder in the codebook. It is certainly beneficial for overhead reduction. - But anyway, we understand the current proposal is a more natural way to go, we can live with the current proposal. |
| - CMCC | - Ok |
| - vivo | - ok |
| - OPPO | - OK |
| - Lenovo, Motorola Mobility | - OK |
| - Xiaomi | - OK |
| - CATT | - OK |
| - NTT DOCOMO | - OK |
| - Nokia, NSB. | - OK |
| - Apple | - OK |
| - NEC | - OK |
| - Ericsson | - OK |

#### Proposal 2.2.2-2 (closed)

For a given serving cell, the same TDRA table applies to all G-RNTIs if configured for UE supports multicast.

Collect views:

| Company | Comments |
| --- | --- |
| - FL’s explanation | - Please check the discussion and FL’s responses for Round-1 to get why the proposal is written as is. |
| - Samsung | - Support (wording can be a bit improved – e.g. multicast can also go to the preamble for the sentence). Although the restriction for same TDRA is not necessary for Type-2 CB, OK to have it in general for simplicity as it does not create any problems. |
| - Qualcomm | - Ok to take it as a conclusion |
| - ZTE | - OK |
| - CMCC | - Ok |
| - vivo | - ok |
| - OPPO | - OK |
| - Lenovo, Motorola Mobility | - OK |
| - Xiaomi | - OK |
| - CATT | - OK |
| - NTT DOCOMO | - OK |
| - Nokia, NSB. | - OK |
| - Apple | - OK |
| - NEC | - OK |
| - Ericsson | - OK |

#### Proposal 2.2.2-3 (closed)

PUCCH-Config for ACK/NACK based feedback for multicast if separately configured from unicast, the PUCCH-Config is applied to all G-RNTIs with the same priority.

- Note: The dl-DataToUL-ACK is included in PUCCH-Config

Collect views:

| Company | Comments |
| --- | --- |
| - FL’s explanation | - Please check the discussion and FL’s responses in Round-1 to get why the proposal is not updated but the comments were noted and addressed in Round-1. |
| - Samsung | - Agree |
| - Qualcomm | - We prefer to add ‘in a serving cell’ in the main bullet for clarification. - Regarding PUCCH-ConfigCommon, it includes the parameters, such as p0-norminal, pucch-GroupHopping, different from that of PUCCH-Config per our understanding. We can have separate discussion for this whether to support separate configure PUCCH-ConfigCommon for unicast and multicast. |
| - ZTE | - Agree |
| - CMCC | - Ok |
| - vivo | - ok |
| - OPPO | - OK |
| - Lenovo, Motorola Mobility | - We are generally OK with it. Maybe add “for a given serving cell” in the bullet is better as Proposal 2.2.2-2. |
| - Xiaomi | - OK |
| - Spreadtrum | - Ok |
| - CATT | - OK |
| - NTT DOCOMO | - OK |
| - Nokia, NSB. | - OK |
| - Apple | - OK |
| - NEC | - OK |
| - Ericsson | - OK |

### Round-3 (closed)

#### Proposal 2.2.3

For the Type-1 codebook construction for FDM-ed unicast and multicast via Opt 4 (from the previous agreement), when UE is configured with multiple G-RNTIs and UE is configured with fdmed-Reception-Multicast, the sub-codebook for multicast consists of the sub-codebooks for each G-RNTI by appending one to another in ascending order of G-RNTI value.

- The sub-codebook for each G-RNTI is generated per the k1 and TDRA configurations for the same G-RNTI as the legacy procedure.

- Note: The maximum numbers of G-RNTI configured to UE is up to UE capability which will be discussed in UE features.

Collect views:

| Company | Comments |
| --- | --- |
| - FL’s explanation | - Based on the comments online: fdmed-Reception-Multicast is for unicast and multicast FDM-ed (at least G-RNTI). Whether more than one G-RNTI FDMed (or even more than one G-RNTi and unicast FDMed) is supported can be discussed in UE feature. I suggest we don’t imply anything for this issue because this proposal is quite generic and should be applicable to all above cases. Regarding whether the first sub-bullet is needed and correctly worded. Since the proposal is saying generate sub-codebook for each G-RNTI, I assume it is the same procedure for unicast. I am not sure whether all companies are on the same page on how exactly the CB is generated based on the discussion in first two rounds. - Your suggestions on how to improve this proposal are always welcome! |
| - ZTE | - We agree with the FL, the number of FDMed PDSCHs in each slot is up to UE feature discussion. |
| - Samsung | - Support |
| - Spreadtrum | - Ok |
| - CMCC | - Ok |
| - CATT | - Generally OK. |
| - NTT DOCOMO | - Support |
| - vivo | - ok |
| - Lenovo, Motorola Mobility | - OK |
| - Nokia, NSB | - We support the proposal. - In our view, based on the reply in the previous rounds from the FL, in case FDM-ed unicast and multicast is configured, the UE constructs sub-codebooks for each G-RNTI and concatenates those in ascending order of G-RNTI. The constructed codebook is then appended to the unicast (based on the previous agreements) for FDM-ed Type-1 codebook. |
| - Qualcomm | - Some questions for clarification: - RAN1 has agreed a UE can be configured with multiple G-RNTIs per serving cell (up to a UE capability). When we say the number of G-RNTIs in the FDMed Type-1 codebook is up to UE capability, is it same or different UE capability as the number of G-RNTIs configured per serving cell? If it is same, does it mean all the G-RNTIs will be concatenated to generate Type-1 CB even if some G-RNTIs may not be scheduled in the CFR of an active BWP? |
| - Ericsson | - OK |
| - FL’s response | - To question from Qualcomm: - Of course, the number of G-RNTIs configured to UE is supposed to be less than the number UE supports for capability reporting. This is Type-1 CB and the size is not affected by whether some G-RNTI is scheduled or not. As long as UE is configured with multiple G-RNTI, the sub-codebook for each configured G-RNTI should be generated and all the sub-codebooks are concatenated together. - I think the proposal is clear and not sure whether this clarification addressed Qualcomm’s question or anything more Qualcomm would like to add? |
| - Lenovo 2 | - Based on the FL response, we are a bit confused that “As long as UE is configured with multiple G-RNTI, the sub-codebook for each configured G-RNTI should be generated and all the sub-codebooks are concatenated together.” - E.g., assuming UE1 is configured with 4 G-RNTIs, in one slot, UE1 receives one GC-PDCCH associated with one G-RNTI, should the UE generate 4 sub-codebooks with each sub-codebook for one G-RNTI? - In addition, regarding FDMed unicast and multicast, for UE1, is it assuming the unicast and the 4 multicast services are purely FDMed or FDM between unicast and the 4 multicast where the 4 multicast are TDMed? - Could you elaborate it a bit more? |
| - FL’s response | - To Lenovo2: - Not sure why we need to talk about one specific slot. Since following this proposal, all sub-codebooks for all configured G-RNTI are concatenated. For each sub-codebook for a G-RNTI, the Type-1 CB is across all slots determined by k1. - It has been clarified already. The parameter of fdmed-Reception-Multicast is for unicast and multicast in general and regardless of whether multicast can be FDM-ed and how many Multicast can be FDM-ed (which are supposed to be discussed in UE feature or AI 8.12.1), this proposal works for all. |

### Round-4

#### (EMAIL)Proposal 2.2.4

For the Type-1 codebook construction for FDM-ed unicast and multicast via Opt 4 (from the previous agreement), when UE is configured with multiple G-RNTIs and UE is configured with fdmed-Reception-Multicast, the sub-codebook for multicast consists of the sub-codebooks for each G-RNTI by appending one to another in ascending order of G-RNTI value.

- The sub-codebook for each G-RNTI is generated per the k1 and TDRA configurations for the same G-RNTI as the legacy procedure.

- FFS: whether/how to reduce the Type-1 codebook size when multiple G-RNTIs are configured.

- Note: The maximum number of G-RNTI(s) configured to UE for the FDMed unicast and multicast Type-1 codebook is up to UE capability which will be discussed in UE features.

Collect views:

| Company | Comments |
| --- | --- |
| - FL’s explanation | - Based on the comments and response from Round-3, the proposal is updated with adding red font. - Given this is converging, I would suggest to have it email approved later. |
| - LG Electronics | - We are OK with the updated proposal. |

## (CLOSED)Type-2 HARQ codebook

Submitted Proposals

“Codebook for more than one per G-RNTI”

(ZTE) Proposal 9:

  - Regarding Type-2 HARQ-ACK codebook construction for MBS, the maximum number of Type-2 HARQ-ACK sub-codebooks for multicast is determined by UE capability for MBS.

(Intel) Proposal 9:

  - When UE is configured with more than one G-RNTI as well as more than one priority index, for Type 2 codebooks, DAI is separately counter per G-RNTI per priority index and the codebook concatenation is also per priority index. Same principle is also followed for NACK only feedback.

- “max number of G-RNTI”

(OPPO) Proposal 3:

  - At most one Type 2 MBS HARQ-ACK sub-codebook is concatenated with type 2 unicast HARQ-ACK sub-codebook.

- Others

(CATT) Proposal 18:

  - To achieve the common understanding on codebook size between gNB and UE, RAN1 should specify the following two aspects for generating Type-2 codebooks when UE joint an ongoing multicast transmission:

    - T0 indication: The slot that UE begins to monitor the PDCCH occasion and acquire scheduling information.

    - Missing DCI: The UE assume that DAI is not circled (e.g.  j=0) and (n-1) DCIs are missed when the DCI value is n (n is not equal 1) on T0 slot.

(CATT) Proposal 19:

  - For Type-2 HARQ-ACK feedback, nHARQ-ACK for PUCCH transmission can be modified as following:

nHARQ-ACK=nHARQ-ACK(unicast) +OACK(multicast),  in where nHARQ-ACK(unicast)  is computed as below formula which is same as R15/R16:

nHARQ-ACK=nHARQ-ACK,TB=VDAI,mlastDL-c=0NcellsDL-1UDAI,cmodTDNTB,maxDL+c=0NcellsDL-1m=0M-1Nm,creceived+NSPS,c ;

    - OACK(multicast) is the total number of bit for all multicast traffic.

### Round-1 (closed)

FL’s Comments

The maximum numbers of G-RNTI UE supports can be discussed in UE feature and is expected to be not many for the first release of NR MBS.

The discussion for Type-2 codebook in this section can focus on PUCCH power control for Type-2 codebook since editor noted agreement is needed to extend the current specification to support multicast, though there is only one proposal from CATT discussing this issue. I will take CATT’s proposal with some update for initial discussion.

FL’s Proposal:

#### Proposal 2.3.1

For obtaining a transmission power for a PUCCH, for Type-2 codebook, nHARQ-ACK is determined as follows:

- nHARQ-ACK=nHARQ-ACK(unicast) +OACK(multicast),  where

  - nHARQ-ACK(unicast)  is computed as in R15/R16.

  - OACK(multicast) is the total number of bits for all configured G-RNTIs.

Collect views:

| Company | Comments |
| --- | --- |
| - Lenovo, Motorola Mobility | - Generally fine with us. - One question for clarification: for NACK-only based feedback, does O_ACK_multicast include only NACK bits or both ACK and NACK bits? - FL’s responses, NACK-only has no CB, for Alt1 NACK-only converted into ACK/NACK, it can be included here. I can make a note to clarify it if there is no problem to have this case included. |
| - CATT | - Agree with the proposal. - @Lenovo Per our understanding, for NACK-only based feedback, the O_ACK esource only NACK bits. We did not see why the can be both ACK and NACK bits. |
| - CMCC | - Ok |
| - Nokia, NSB | - Ok. |
| - Vivo | - ok |
| - Xiaomi | - OK |
| - NEC | - We agree with the proposal in principle. But as mentioned by Lenovo and CATT, it should consider that OACK(multicast) indicates only NACK or ACK/NACK. For the overlapping case between NACK-only PUCCH and ACK/NACK PUCCH or CSI PUCCH, we have agreed that UE multiplexes the NACK-only based feedback with the HARQ-ACK feedback/CSI on PUCCH or on to PUSCH by transforming NACK-only into the ACK/NACK HARQ bit, thus at least a note should be added to clarify NACK-only case. - FL’s responses: yes, will do so. Agreement: When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for HARQ-ACK feedback/CSI for unicast for the same priority or PUSCH transmission for the same priority, support UE multiplexing the NACK-only based feedback with the HARQ-ACK feedback/CSI on PUCCH or on to PUSCH by transforming NACK-only into the ACK/NACK HARQ bit. - This applies to at least the case of the feedback addressing one TB. NACK-only based feedback for more than one TBs is to be handled separately. - Note: When the TB is correctly decoded, the ACK will be transmitted and multiplexed with others. - FFS the case of PUCCH for SR. |
| - Samsung | - Support. - @Lenovo/CATT: We understand the case of NACK-only (and no unicast) to not be part of the proposal – there is no Type-2 CB for “NACK-only”. |
| - Ericsson | - Support |
| - Qualcomm | - We think the proposal should be limited to ACK/NACK-based feedback for multicast. - If NACK-only is to be multiplexed with unicast, it will be firstly transformed to ACK/NACK-based feedback based on the previous RAN1 agreement. |
| - NTT DOCOMO | - Support |
| - Lenovo 2 | - @CATT @Samsung: when multiple NACK-only feedback is to be multiplexed with unicast PUCCH, the multiple NACK-only feedback may be transformed to ACK/NACK bits. In that sense, maybe O_ACK needs to include both ACK and NACK bits. |
| - ZTE | - We also think this proposal should be limited to ACK/NACK based feedback. To clarify this, we propose to change OACK(multicast) to nHARQ-ACK(multicast). |
| - OPPO | - Support the proposal. |
| - Spreadtrum | - Support the proposal |
| - FL’s responses, | - Based on the above discussion, the unclear point is if Alt1 for more than one NACK-only is agreed whether it is included in this proposal. Let see try the following clarification whether it is acceptable.: #### Proposal 2.3.1-r1 For obtaining a transmission power for a PUCCH, for Type-2 codebook, nHARQ-ACK is determined as follows: - nHARQ-ACK=nHARQ-ACK(unicast) +nACK(multicast), where - nHARQ-ACK(unicast) is computed as in R15/R16. - nACK(multicast) is the total number of bits for all configured G-RNTIs. - Note: If Alt1(from the previous agreement) for more than one NACK-only are available for transmission in the same PUCCH slot is agreed, the ACK/NACK converted from NACK-only are part of nACK(multicast). |

### Round-2 (closed)

#### Proposal 2.3.2

For obtaining a transmission power for a PUCCH, for Type-2 codebook, nHARQ-ACK is determined as follows:

- nHARQ-ACK=nHARQ-ACK(unicast) +OACK(multicast),  where

  - nHARQ-ACK(unicast)  is computed as in R15/R16.

  - OACK(multicast) is the total number of bits for all configured G-RNTIs.

    - Note: If Alt1(from the previous agreement) for more than one NACK-only are available for transmission in the same PUCCH slot is agreed, the ACK/NACK converted from NACK-only are part of nACK(multicast).

Collect views:

| Company | Comments |
| --- | --- |
| - FL’s explanation | - Please check the discussion and FL’s responses in Round-1 to get why the proposal is updated as is. |
| - Samsung | - Do not support as is. Clarification is needed. - For unicast, it is nHARQ-ACK(unicast). For multicast, it is OACK(multicast). In the note, it is nACK(multicast). Is that a typo? Also, why is it OACK(multicast) and not nHARQ-ACK(multicast)? If it is indeed OACK(multicast), that is not according to Rel-15/16 - also, the UE may also end up transmitting PUCCH with much more power than necessary (we missed that aspect in the previous response). That cannot be agreeable. - It also seems better to remove the note and just capture that the proposal is for when HARQ-ACK bits have ACK/NACK values. |
| - ZTE | - We are ok with the following one. #### Proposal 2.3.1-r1 For obtaining a transmission power for a PUCCH, for Type-2 codebook, nHARQ-ACK is determined as follows: - nHARQ-ACK=nHARQ-ACK(unicast) +nACK(multicast), where - nHARQ-ACK(unicast) is computed as in R15/R16. - nACK(multicast) is the total number of bits for all configured G-RNTIs. - Note: If Alt1(from the previous agreement) for more than one NACK-only are available for transmission in the same PUCCH slot is agreed, the ACK/NACK converted from NACK-only are part of nACK(multicast). |
| - vivo | - fine with ZTE’s update. |
| - OPPO | - Support the version proposed by ZTE. |
| - Lenovo, Motorola Mobility | - We suggest limiting the proposal for ACK/NACK based feedback. - For NACK-only based feedback, we can wait for the decision of NACK-only multiplexing. |
| - Spreadtrum | - Fine with the version from ZTE |
| - CATT | - The reason that why it is OACK(multicast) in the sub-bullet is that we assuming that the nACK(multicast)=OACK(multicast). As discussed in our contribution, since small number of MIMO layer is needed due to multicast transmission targets a group UEs, so it is more reasonable that NTB,maxDL=1 for multicast. This will lead nHARQ-ACKof multicast is equal to OACK, where OACK is the HARQ-ACK bit number that computed by C-DAI/T-DAI. If this can be a common understanding, we believe that OACK(multicast) in the sub-bullet is more straightforward instead of nACK(multicast). Thus, the proposal can be as below with updated note: For obtaining a transmission power for a PUCCH, for Type-2 codebook, nHARQ-ACK is determined as follows: - nHARQ-ACK=nHARQ-ACK(unicast) +OACK(multicast), where - nHARQ-ACK(unicast) is computed as in R15/R16. - OACK(multicast) is the total number of bits for all configured G-RNTIs. - Note: If Alt1(from the previous agreement) for more than one NACK-only are available for transmission in the same PUCCH slot is agreed, the ACK/NACK converted from NACK-only are part of nACK(multicast)OACK(multicast). |
| - NTT DOCOMO | - We are fine with ZTE’s update. |
| - Nokia, NSB. | - More clarification needed, as indicated by Samsung. |
| - Ericsson | - Fine with ZTE version |
| - TD Tech, Chengdu TD Tech | - ok |

## Others

### Non-overlapping/sub-slot PUCCH

Submitted Proposals

“sub-slot based PUCCH”

(vivo) Proposal 7:

  - Support sub-slot based PUCCH transmission for multicast HARQ-ACK.

(CMCC) proposal 1:

  - Support sub-slot based PUCCH transmission for multicast HARQ-ACK.

“Non-overlapping slot-based PUCCH”

(vivo) Proposal 4:

  - For the cases of HARQ-ACK feedback (at least for ACK/NACK based feedback) is available for multicast and unicast for a given UE receiving multicast,

    - For the case of non-overlapping PUCCHs resources for HARQ-ACK in the same slot, separate transmission of HARQ-ACK PUCCH for unicast and HARQ-ACK PUCCH for multicast is supported.

(CMCC) proposal 2:

  - If separate PUCCH(s) is configured for HARQ-ACK for multicast, the PUCCH structure for PUCCH-Config of multicast and unicast HARQ-ACK with the same priority index should be the same. The PUCCH structure for PUCCH-Config for multicast and unicast HARQ-ACK with different priority index can be different.

(DOCOMO) Proposal 4:

  - When HARQ-ACK feedback for unicast and multicast is indicated as transmitted at the same PUCCH slot, the UE multiplexes HARQ-ACK with the same priority regardless whether their PUCCH resources are overlapped or not.

### UCI multiplexing/prioritizing

Submitted Proposals

“last unicast DCI”

(CATT) Proposal 11:

  - It is not necessary to make them different from current specifications when last DCI is missed by the UE.

(CMCC) Proposal 6:

  - For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, determining the PUCCH resources for transmission should be based on the PRI indicated in the last DCI for unicast.

UL-DAI

(vivo) Proposal 6:

  - For PUCCH carrying multicast HARQ-ACK overlaps with PUSCH, reuse NR Rel-16 multiplexing/ prioritizing rule between the HARQ-ACK for unicast and PUSCH.

    - FFS UL-DAI indication when both unicast HARQ-ACK and multicast HARQ-ACK, or HARQ-ACK for multiple G-RNTIs are multiplexed on one PUSCH

(CATT) Proposal 16:

  - Additional UL DAI fields should be added in format DCI0_1/0_2 for supporting the multiplexing between sub-codebooks for unicast and multicast on PUSCH.

(CATT) Proposal 17:

  - When the number of configured G-RNTI is larger than the number of UL DAI, the association between the UL DAI and G-RNTI can be configured by RRC.

“whether multiplexing different priorities”

(Intel) Proposal 11:

  - Multiplexing across priorities is not allowed in a single PUCCH occasion. For overlapping PUCCH transmissions in time, dropping rule is used.

### PUCCH resource

Submitted Proposals

“Resource sets/resources”

(OPPO) Proposal 9:

  - For ACK/NACK based HARQ-ACK feedback for multicast, both shared and separate PUCCH resources among Ues within the group are supported.

(OPPO) Proposal 10:

  - For ACK/NACK based feedback and NACK only based feedback for multicast, RSRP based PUCCH resource configuration is supported.

(OPPO) Proposal 11:

  - Whether shared or separate PUCCH resources are used can be up to gNB configuration or scheduling.

(CMCC) Proposal 3:

  - For the separate PUCCH-ConfigurationList configured for multicast for two simultaneously constructed HARQ-ACK codebooks with different priorities,

    - different PUCCH Resource IDs are configured in the two PUCCH-Config within the PUCCH-ConfigurationList.

- “k1 indication”

(LGE) Proposal 1:

  - For PTM scheme 1, group common DCI indicates a single PUCCH resource indicator and a single PDSCH-to-HARQ_feedback timing indicator at least for ACK/NACK based HARQ-ACK.

(LGE) Proposal 2:

  - For UE specific PUCCH resource allocation for ACK/NACK based HARQ-ACK to group common DCI, different Ues in the group can be configured with different values of at least PUCCH-Resource and dl-DataToUL-ACK in UE dedicated PUCCH-config for multicast or for unicast (unless PUCCH-config for multicast is configured).

    - Different Ues can be allocated with different PUCCH resources by the same PUCCH resource indicator and the same PDSCH-to-HARQ_feedback timing indicator of the group common DCI.

(LGE) Proposal 3:

  - For PTP retransmission, the PUCCH resource indicator and the PDSCH-to-HARQ_feedback timing indicator in UE specific DCI are interpreted based on PUCCH-config for unicast, regardless of whether PUCCH-config for multicast is configured or not.

# NACK-only based feedback specific

## (UPDATED)NACK-only overlapping with SR

Submitted Proposals

“Overlap with other PUCCH/PUSCH” for the same priority

(Huawei) Proposal 5:

  - For the case that PUCCH for NACK-only overlaps with PUCCH for SR,

    - if the SR is negative SR, NACK-only HARQ-ACK is transmitted on the PUCCH configured for NACK-only, and

    - if the SR is positive SR, NACK-only HARQ-ACK is transformed into ACK/NACK HARQ-ACK and transmitted on the PUCCH configured for SR.

- (ZTE) Proposal 3:

  - For multiplexing between NACK-only PUCCH and unicast PUCCH/PUSCH which indicated in the same slot,

    - Unicast ACK/NACK/SR PUCCH F0/F1 multiplexes with NACK-only PUCCH.

    - Unicast ACK/NACK/CSI PUCCH F2/F3/F4 multiplexes with NACK-only PUCCH;

    - Unicast PUSCH multiplexes with NACK-only PUCCH.

- (vivo) Proposal 8:

  - When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR for the same priority, the following solutions can be considered.

    - Option 1: If SR is positive, NACK-only is dropped. If SR is negative, transmit NACK-only on its resource

    - Option 2: If SR is positive, transmit NACK-only on SR PUCCH resource. If SR is negative, transmit NACK-only PUCCH.

      - Option 2-1: only NACK-only is transmitted on SR PUCCH resource. If it is ACK, transmit SR only.

    - Option 3: When SR is positive, for the case that both SR PUCCH and NACK-only are PUCCH format 1, transmit NACK on SR using the same rule as NR R15/R16. Otherwise, NACK-only/SR is dropped. When SR is negative, transmit NACK-only PUCCH.

- (Nokia) proposal 8:

  - When the PUCCH resource for the NACK-only feedback overlaps in time with SR of the same priority, support UE multiplexing the NACK-only based feedback with the SR on the PUCCH resource of the SR by transforming NACK-only into the ACK / NACK bit.

    - If the PUCCH resource for SR cannot accommodate multiplexed UCI (e.g., in case of multiple NACK-only bits are multiplexed with SR), SR is dropped.

    - Note: When the TB is correctly decoded, an ACK will be transmitted.

- (LGE) Proposal 10:

  - Discuss UE procedure for multiplexing NACK-only based HARQ-ACK and SR for PUCCH Format 0 and/or 1.

- (DOCOMO) Proposal 6:

  - When a PUCCH format 0 with NACK-only based feedback overlaps with a PUCCH with positive SR, the NACK-only based feedback is transformed into ACK/NACK based HARQ-ACK bits, multiplexed with SR, and transmitted in another PUCCH resource for ACK/NACK based feedback.

- (Lenovo) Proposal 11:

  - For SR multiplexing with single NACK-only based feedback, positive SR multiplexing with NACK-only feedback is transmitted in SR resource.

- (Lenovo) Proposal 12:

  - For SR multiplexing with single NACK-only based feedback, different cyclic shifts are used to differentiate positive SR only and positive SR multiplexing with NACK-only feedback.

- “Overlap with other PUCCH/PUSCH” for different priorities

(Nokia) Proposal 10:

  - The prioritization between NACK-only based HARQ-ACK feedback for multicast and other UCI / PUSCH, the existing framework of unicast should be re-used also for multicast, i.e., dropping the less prior transmission.

### Round-1 (closed)

FL’s Comments

Regarding transmission for NACK-only collides with PUCCH/PUSCH, the following was agreed in the last meeting with FFS for the case of SR:

Agreement:

When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for HARQ-ACK feedback/CSI for unicast for the same priority or PUSCH transmission for the same priority, support UE multiplexing the NACK-only based feedback with the HARQ-ACK feedback/CSI on PUCCH or on to PUSCH by transforming NACK-only into the ACK/NACK HARQ bit.

- This applies to at least the case of the feedback addressing one TB. NACK-only based feedback for more than one TBs is to be handled separately.

- Note: When the TB is correctly decoded, the ACK will be transmitted and multiplexed with others.

- FFS the case of PUCCH for SR.

Multiplexing ACK/NACK PF0/1 with SR PF0/PF1 from legacy rule is summarized in the following table:

|  | SR using PF0 | SR using PF1 |
| --- | --- | --- |
| HARQ-ACK using PF0 | Positive SR: HARQ-ACK on PUCCH resource for HARQ-ACK using additional cyclic shift(s) Negative SR: HARQ-ACK on PUCCH resource for HARQ-ACK using original cyclic shift(s) Note: same “original cyclic shift(s)” are used for HARQ-ACK feedback when - HARQ-ACK collides with SR - HARQ-ACK doesn’t collide with SR |  |
| HARQ-ACK using PF1 | Drop SR | Positive SR: HARQ-ACK on PUCCH resource for SR Negative SR: HARQ-ACK on PUCCH resource for HARQ-ACK |

Following legacy rule of using PUCCH resource for NACK-only for positive SR when NACK-only uses PF0 will cause positive SR lost because the NACK-only resources may be shared by more than one UE.

From contributions, 8 companies proposed solution to address such issue and views are summarized as follows:

When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmission carrying SR for the same priority:

- Opt1: transmission on PUCCH resource for SR for positive SR; transmission on PUCCH resource for NACK-only for negative SR.

  - Supported by: Huawei, vivo, Lenovo

- Opt2: transmission on PUCCH resource for SR always despite positive or negative SR.

  - Supported by: Nokia

- Opt3: transmission on PUCCH resource for ACK/NACK based for positive SR

  - Supported by: DOCOMO

- Opt4: drop NACK-only for positive SR; transmission on PUCCH resource for NACK-only for negative SR

  - Supported by: vivo

- Opt5: when SR is positive, for the case that both SR PUCCH and NACK-only are PUCCH format 1, transmit NACK on SR using the same rule as NR R15/R16. Otherwise, NACK-only/SR is dropped. When SR is negative, transmit NACK-only PUCCH.

  - Supported by: vivo

Based on this situation, FL suggests Opt1 for initial discussion.

FL’s Proposal:

#### Proposal  3.1.1

When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmission carrying SR for the same priority, adopt the following option:

- Opt1: for positive SR, HARQ-ACK is transmitted on PUCCH resource for SR; for negative SR, HARQ-ACK is transmitted on PUCCH resource for NACK-only.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | We are generally OK with this proposal. |
| - CATT | - OK with this proposal. |
| - CMCC | - Ok |
| - Nokia, NSB | Ok with the intention of the proposal. In addition, as we discussed in our contribution, the case where the SR PUCCH cannot accommodate multiple NACK-only feedback (that are multiplexed) should be discussed. We propose the rewording as: When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmission carrying SR for the same priority, adopt the following option: - Opt1: for positive SR, HARQ-ACK is transmitted multiplexed on PUCCH resource for SR; for negative SR, HARQ-ACK is transmitted on PUCCH resource for NACK-only and SR is dropped. - If the PUCCH resource for SR cannot accommodate multiplexed UCI (e.g., in case of multiple NACK-only bits are multiplexed with SR), SR is dropped. - FL’s response: SR is for unicast. NACK-only is for multicast. Comparing these two, if have to drop one, I tend to agree dropping NACK-only as vivo proposed. |
| - Vivo | Generally ok. Agree with Nokia, the case with more than one NACK-only should be considered. In this case, if alt 4 is adopted, than how to transmit more than 2 NACK on SR PUCCH considering that the current spec can multiplex at most 2-bit HARQ-ACK on SR PUCCH. - FL’s response: agree to clarify how many NACK-only is considered in the proposal. |
| - Xiaomi | Support. We think option 1 is a completed solution for multiplexing SR and NACK-only feedback. Other options cannot guarantee both of them. |
| - Samsung | Support. One clarification is that “NACK-only” is converted to “ACK/NACK” when the SR resource is used (only possible way). Another clarification is that the proposal is applicable only for 1-2 “NACK-only” bits. For >2 “NACK-only” bits, it can depend on the eventual design and can be left FFS. - FL’s response: will update the proposal with these clarified. |
| - Ericsson | - Support |
| - Qualcomm | RAN1 has already agreed to transform NACK-only into ACK/NACK in case of collision. For the case of one NACK-only, we just follow the legacy rules for multiplexing ACK/NACK with SR and no need to have this proposal. - FL’s response: The problem is legacy rule may not work in some cases because the resources carrying ack/nack is limited, e.g., SR PF0 only has one sequence. When NACK-only is converted to ACK/NACK, it needs two sequences. - For the case of more than one NACK-only (discussed in 3.2), RAN1 to further study whether to avoid collision with SR by gNB scheduling, or define multiplexing/dropping rules. |
| - NTT DOCOMO | Not support. Since only 2 bits can be transmitted on PUCCH resource for SR, we prefer to use PUCCH resources for ACK/NACK based. - It depends on SR is positive or negative. For positive SR, using NACK-only resources is not helpful for network to get the SR because NACK-only resources can be shared. |
| - Apple | Ok with this proposal |
| - Lenovo 2 | Agree with Samsung that here “NACK-only” needs to be transformed to “ACK/NACK” in case of positive SR. |
| - ZTE | We are fine with the direction of this proposal. However, there are some details need to be addressed. - is this proposal applicable to both F0 and F1 for both SR and NACK-only feedback? -> intended to stick to legacy configuration of formats for SR and agreement of PF0/PF1 for NACK-only. 2) what’s the cyclic shift for PUCCH transmission for each case? -> This can be further discussed. |
| - OPPO | Agree with clarification that it is applied only for one NACK-only bit. |
| - FL’s response | Based on the discussion, I observed most companies tend to agree firstly that NACK-only is transformed into ack/nack when collides SR, let’s try the following update: #### Proposal 3.1.1-r1 - When PUCCH transmission for one TB of NACK-only based feedback for multicast collides with PUCCH transmission carrying SR for the same priority,: - for positive SR, NACK-only is transformed into ACK/NACK and ACK/NACK is transmitted on PUCCH resource configured for SR; - This applies to PF1 SR and PF0/PF1 NACK-only - FFS PF0 SR and PF0/PF1 NACK-only - for negative SR, HARQ-ACK (i.e., NACK-only) is transmitted on PUCCH resource configured for NACK-only. |

### Round-2 (closed)

#### Proposal  3.1.2

- When PUCCH transmission for one TB of NACK-only based feedback for multicast collides with PUCCH transmission carrying SR for the same priority,:

  - for positive SR, NACK-only is transformed into ACK/NACK and ACK/NACK is transmitted on the PUCCH resource configured for SR;

    - This applies to PF1 SR and PF0/PF1 NACK-only

    - FFS PF0 SR and PF0/PF1 NACK-only

  - for negative SR, HARQ-ACK (i.e., NACK-only) is transmitted on the PUCCH resource configured for NACK-only.

- FFS PUCCH transmission for one TB of NACK-only based feedback for multicast collides with PUCCH transmission carrying SR for the same priority.

Collect views:

| Company | Comments |
| --- | --- |
| - FL’s explanation | - Please check the discussion and FL’s responses in Round-1 to get why the proposal is updated as is. |
| - Samsung | - Why the limitation to 1 TB when 2 TBs can also be supported in the same manner? - The FFS seems to have typos – is it “for more than one TB”? As is, it is same as the first bullet. |
| - Qualcomm | - Still not sure why the problem is specific for NACK-only. In case of ACK/NACK-based feedback, we also need two sequences. If NACK-only is transformed to ACK/NACK, reuse the same rules for the collision with SR. |
| - LG Electronics | - If “NACK-only” is transformed to “ACK/NACK” in case of positive SR, it seems better to follow the legacy rule for PF1 SR and PF0 HARQ-ACK and the other cases in FFS, except PF1 for both SR and HARQ-ACK. |
| - ZTE | - We are ok with this proposal. Actually since we restrict the SR to PUCCH F1 in this proposal, it is already aligned with the existing spec. |
| - CMCC | - Same view as LGE, if “NACK-only” is transformed into “ACK/NACK”, the same multiplexing rule of unicast HARQ-ACK with SR in Rel-15/16 should be reused. |
| - vivo | - Why do we need to transform “NACK-only” to “ACK/NACK”? If it is transformed, “ACK/NACK” can transmit on one PUCCH configured for “ACK/NACK” feedback, and reuse legacy multiplexing rule. This solution seems better than the proposed one. |
| - OPPO | - Support the proposal as it is aligned with existing rule in spec, we also think there is a typo in the FFS as pointed out by Samsung. |
| - Lenovo, Motorola Mobility | - We are generally OK with this proposal and support both PF0/PF1 SR case. - The last FFS is unclear. |
| - Xiaomi | - Same view as LGE/CMCC/vivo. |
| - CATT | - Same view as LGE/CMCC, if NACK-only is transformed into ACK/NACK and ACK/NACK, the sequence with different cyclic shifts to represent different state combinations of ACK/NACK and negative/positive SR can be used. We also think there is a typo in the FFS as pointed out by Samsung. |
| - NTT DOCOMO | - We are generally fine with the proposal. |
| - Nokia, NSB. | - The main difference between ACK/NACK and NACK-only is that in which PUCCH resource multiplexed UCI is transmitted, since NACK-only resource cannot be used as it is group-common. For that reason, the cases where HARQ-ACK is format 0 PUCCH are problematic. Thus, if we were to keep the legacy rules as much as possible, the proposal should apply only to case HARQ-ACK with format 0 PUCCH. |
| - Lenovo 2 | - Since companies have concern on transforming NACK-only to ACK/NACK, a compromised way forward is we can agree NACK-only feedback is transmitted on SR resource in case of positive SR with a different CS from positive SR only. - One suggestion from our side for reference: - When PUCCH transmission for one TB of NACK-only based feedback for multicast collides with PUCCH transmission carrying SR for the same priority,: - for positive SR, NACK-only is transmitted on the PUCCH resource configured for SR with a different cyclic shift from that for positive SR only; - This applies to PF1 SR and PF0/PF1 NACK-only - FFS PF0 SR and PF0/PF1 NACK-only - for negative SR, NACK-only feedback is transmitted on the PUCCH resource configured for NACK-only. |
| - NEC | - If it attend to multiplex NACK-only and SR by selecting different resources, we think there is no necessary to transform NACK-only to ACK/NACK since the HARQ-ACK feedback type before multiplexing will not have any impact on the multiplexing result. If it attend to reuse the multiplexing rules between ACK/NACK and SR, we agree with Nokia that some cases like SR with PF0/1 and HARQ-ACK with PF0 should be redesigned since the PUCCH resource for NACK-only(also called ACK/NACK after transforming) is group-common. |
| - Ericsson | - Same view as Samsung |

### Round-3 (closed)

#### Proposal  3.1.3

- When PUCCH transmission for [one TB of] NACK-only based feedback for multicast collides with PUCCH transmission carrying SR for the same priority, :

  - for positive SR, NACK-only is transmitted on the PUCCH resource configured for SR with a different cyclic shift from that for positive SR only;

    - This applies to PF1 SR and PF0/PF1 NACK-only

    - FFS PF0 SR and PF0/PF1 NACK-only

  - for negative SR, NACK-only feedback is transmitted on the PUCCH resource configured for NACK-only.

- FFS PUCCH transmission for more than one TB of NACK-only based feedback for multicast collides with PUCCH transmission carrying SR for the same priority.

Collect views:

| Company | Comments |
| --- | --- |
| - FL’s explanation | - Based on the comments in previous rounds, it seems companies are not one the same page what should be assumed and what needs to discuss. - Let’s see whether the updated proposal is going to the direction agreeable to companies. I appreciate any suggestions for improving the proposal to close this issue. |
| - ZTE | - We are ok with this proposal. It would be good if we can address the issue for PF1 SR as well. |
| - Samsung | - Do not support - “NACK-only” is converted to “ACK/NACK” when multiplexed with other UCI or data. There is no reason to not follow Rel-16 procedure for ACK/NACK and SR multiplexing. |
| - CMCC | - Agree with Samsung |
| - CATT | - According to the current proposal, we summarized it as below (if not right, please correct me!): NACK-only with PF0; NACK-only with PF1. SR with PF0; positive SRFFS negative SR Mechanism2; positive SRFFS negative SR Mechanism2. SR with PF1; positive SR Mechanism1 negative SR Mechanism2; positive SR Mechanism1 negative SR Mechanism2. Mechanism1: for positive SR, NACK-only is transmitted on the PUCCH resource configured for SR with a different cyclic shift from that for positive SR only Mechanism2: for negative SR, NACK-only feedback is transmitted on the PUCCH resource configured for NACK-only.. - Comparing with the legacy rules (see Nokia summarised in last discussion round), it seems that the corresponding mechanisms change a lot according to Proposal 3.1.3. We are not sure this can be acceptable for majority. - Moreover, in the first sub-bullet, what the sequences with different cyclic shifts represent should be clarified. |
| - NTT DOCOMO | - We are generally fine with the proposal. |
| - vivo | - The proposal makes me confused. - 1) For PF1 SR, ACK/NACK is transmitted on SR using BPSK or QPSK. And gNB detects SR by energy detection. Different CSs are used for MU. We think the added “with a different cyclic shift from that for positive SR only” is not needed. Only PF0 using different CS for SR and SR+NACK in current spec. - 2) if we were to keep the legacy rules as much as possible, the proposal should apply only to case HARQ-ACK with format 1 PUCCH and SR with PUCCH format 1. since only in this case, HARQ-ACK is multiplexed on SR PUCCH resource. For PF1 SR and PF 0ACK/NACK, SR is multiplexed on ACK/NACK PUCCH according to the current multiplexing rule. |
| - Lenovo, Motorola Mobility | - We support this proposal. - In Rel-16, in case of SR multiplexing with HARQ-ACK feedback, positive SR is transmitted on the PUCCH resource configured for HARQ-ACK feedback. However, this can’t work for NACK-only based feedback multiplexing with SR. Anyway, Rel-16 behaviour needs to be minor changed. - To address concern from other companies, maybe we can clearly indicate the first sub-bullet is applied for PF0 SR then FFS PF1 SR. |
| - Nokia, NSB | - As we indicated before, the main difference between ACK/NACK and NACK-only is that in which PUCCH resource multiplexed UCI is transmitted, since NACK-only resource cannot be used as it is group-common. For that reason, the cases where HARQ-ACK is format 0 PUCCH are problematic, as can be seen from the following table. Thus, if we were to keep the legacy rules as much as possible, the proposal should apply only to case HARQ-ACK with format 0 PUCCH. - We propose the following, since the rest can follow the legacy behavior: - When PUCCH transmission for [one TB of] NACK-only based feedback for multicast collides with PUCCH transmission carrying SR for the same priority,: - for positive SR, NACK-only is transformed into ACK/NACK and ACK/NACK is transmitted on the PUCCH resource configured for SR; - This applies to PF0/PF1 SR and PF0/PF1 NACK-only - Other configurations follow legacy behavior - FFS PF0 SR and PF0/PF1 NACK-only - for negative SR, HARQ-ACK (i.e., NACK-only) is transmitted on the PUCCH resource configured for NACK-only. - FFS PUCCH transmission for more than [one TB of] NACK-only based feedback for multicast collides with PUCCH transmission carrying SR for the same priority. |
| - Qualcomm | - Similar view as Samsung. |
| - LG Electronics | - Similar view as Samsung. |

### Round-4 (closed)

FL’s Comments

Based on the comments in Round-3, I have been struggling how to move forward. Basically, there are two Alternatives to do multiplexing SR and NACK-only for the same priority:

Alt1: NACK-only is transformed into ACK/NACK first and multiplex ACK/NACK and SR as the legacy procedure in Rel-15/16.

- FL’s view: I suppose for this Alt1, the PUCCH transmission will be always on PUCCH resources configured for SR since PUCCH resource configured for NACK-only may be shared by more than one UE and it could be even the resource defined for different TB combination of NACK-only. Are we assuming PUCCH resources for unicast might be used instead when following legacy multiplexing rule which results in PUCCH transmission is on PUCCH resource for HARQ-ACK?

Alt2: Depending on the PUCCH configuration (including PUCCH formats) for SR and that for NACK-only, either the PUCCH transmission is on the PUCCH resource configured for SR or on the PUCCH resource configured for NACK-only for which NACK-only is not transformed into ACK/NACK.

- FL’s view: the original proposal is worded following this Alt2. Which resource to be used for PUCCH transmission depending on whether the SR is positive or negative. If it is negative SR, no problem to use the PUCCH resources for NACK-only which is applicable to Alt4 (defining combination of NACK-only corresponding to a specific sequence or PUCCH transmission) as well. Otherwise, PUCCH resources configured for SR will be used so that network can differentiate which UE this SR is coming from.

#### Proposal  3.1.4

When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR for the same priority, support [Alt1] from the following alternatives:

- Alt1: up to two TBs of NACK-only feedback is transformed into ACK/NACK HARQ bits and is multiplexed with SR as legacy procedure as Rel-15/16.

- Alt2:

  - for positive SR, NACK-only is transmitted on the PUCCH resource configured for SR;

    - If the PUCCH resource for SR cannot accommodate multiplexed UCI (e.g., in case of multiple NACK-only bits are multiplexed with SR), SR is dropped.

  - for negative SR, NACK-only feedback is transmitted on the PUCCH resource configured for NACK-only.

Collect views:

| Company | Comments |
| --- | --- |
| FL’s explanation | I temporarily put Alt1 in brackets ([Alt1]) in the main bullet. Please companies check whether each of alternatives are correctly described and which alternative is preferred and the reasoning for your preference. |
| Samsung | Understand the comments by Nokia/CATT. Prefer Alt.2 to avoid dropping NACKs – there is always a next SR opportunity (and SRs are dropped since Rel-15 based on MAC decision and NACK won’t be happening continuously unless something is broken). Alt.1 can be acceptable but also need to say which NACKs/HARQ-ACKs are multiplexed and which are dropped (e.g. the HARQ-ACK is for the first 2 TBs). |
| ZTE | We see some issues with both Alt.1 and Alt.2. For Alt.1, if the SR+ACK/NACK is transmitted in the NACK-only PUCCH resource, how can the network successfully receive and decode the PUCCH considering that multiple UEs can share the same PUCCH resource for NACK-only feedback? For Alt.2, we don’t understand the following “If the PUCCH resource for SR cannot accommodate multiplexed UCI (e.g., in case of multiple NACK-only bits are multiplexed with SR), SR is dropped.”, if 3bits of NACK-only need to be transmitted in the SR PUCCH resource, even if SR is dropped, the SR PUCCH resource can NOT accommodate 3bits. We don’t understand why we need to drop SR. |
| vivo | For alt 1, we think if we want to legacy procedure, the PUCCH for ACK/NACK should be UE-specific. So we think UE should determine a PUCCH for the ACK/NACK (after transforming) from the PUCCH resource set configured for ACK/NACK or unicast. Otherwise, the PUCCH resource for NACK only is group-common, the legacy procedure cann’t be reused since SR cann’t be multiplexed on group-common PUCCH resource. |
| Nokia, NSB | We agree with vivo on Alt. 1 and support Alt. 1, as minimum change will be needed compared to legacy approach. As in Alt. 2, if the SR resource cannot accommodate multiplexed feedback, SR can be dropped. When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR for the same priority, support [Alt1] from the following alternatives: - Alt1: up to two TBs of NACK-only feedback is transformed into ACK/NACK HARQ bits and is multiplexed with SR as legacy procedure as Rel-15/16. - If the PUCCH resource for SR cannot accommodate multiplexed UCI (e.g., in case of multiple NACK-only bits are multiplexed with SR), SR is dropped. - Alt2: - for positive SR, NACK-only is transmitted on the PUCCH resource configured for SR; - If the PUCCH resource for SR cannot accommodate multiplexed UCI (e.g., in case of multiple NACK-only bits are multiplexed with SR), SR is dropped. - for negative SR, NACK-only feedback is transmitted on the PUCCH resource configured for NACK-only. |
| CMCC | Agree with vivo, the unicast PUCCH can be used as ACK/NACK feedback for multicast in Alt 1. As the comment of Nokia, since the upper bound of NACK-only feedback is 2bits as the condition in Alt 1, we don’t need the new adding sentence, but more than 2 bits scenario is also needed to be discussed. |
| Lenovo, Motorola Mobility | We prefer Alt 2. For Alt 1, we are not sure why up to 2 TBs are mentioned. Which PUCCH resource is used for multiplexing SR and NACK-only converted A/N feedback? In legacy operation, it is multiplexed on HARQ-ACK feedback resource. However, here, we are talking about NACK-only feedback multiplexing with SR. For Alt 2, the sub-bullet on dropping SR is not clear. We suggest FFS multiple NACK-only feedback multiplexing with SR. |
| Ericsson | Not sure about how Alt1 work. For legacy procedure as R-15/16, when SR and HARQ need to multiplexed together when SR and HARQ using PF0, the PUCCH resource for multiplexing is from HARQ. However, PUCCH resource of NACK-only is a shared resource and not suitable for multiplexing with UE specific SR. We prefer Alt2, but suggest to make it more clear by adding the boldface text: - Alt2: - for positive SR, NACK-only is transmitted on the PUCCH resource configured for SR; - If the PUCCH resource for SR cannot accommodate multiplexed UCI (e.g., in case of multiple NACK-only bits are multiplexed with SR), SR is dropped and NACK-only is transmitted on the PUCCH resources configured for NACK-only.. Since PUCCH resources configured for SR can only be of Format 0 or 1, we think Alt2 could be phrased more explicitely: - for positive SR; - If PUCCH resource configured for SR is of Format 0 and the UE has to provide feedback for up to 2 TBs then the feedback is multiplexed with the SR and transmitted on this PUCCH resource. - If PUCCH resource configured for SR is of Format 1 and the UE has to provide feedback for up to 2 TBs then the feedback is transmitted on this PUCCH resource. The gNB by detecting energy on this PUCCH resource detects the SR from the UE. - otherwise SR is dropped and NACK-only is transmitted on the PUCCH resources configured for NACK-only.. for negative SR, NACK-only feedback is transmitted on the PUCCH resource configured for NACK-only |
| Qualcomm | We are not sure whether positive SR should be always dropped if colliding with multiple multicast NACK-only bits in Alt2. For example, some urgent UL scheduling may need to be prioritized. Alt1 can work for the case of up to two TBs of NACK-only feedback. We can add FFS more than two TBs for further discussion. #### Proposal 3.1.4 When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR for the same priority, support: - in case of up to two TBs of NACK-only feedback, the NACK-only feedback is transformed into ACK/NACK HARQ bits and is multiplexed with SR as legacy procedure as Rel-15/16. - FFS: more than two TBs of NACK-only feedback |
| NTT DOCOMO | We agree with vivo/CMCC. If the existing rule to multiplex PF0 HARQ-ACK and SR is directly applied to a PUCCH resource for NACK-only based feedback, gNB will not be able to identify which UE sent the SR since PUCCH resources for NACK-only based feedback will be shared among UEs. It is preferable to use the PUCCH resources configured for ACK/NACK based in Alt 1. |

### Round-5

#### Proposal  3.1.5

When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR for the same priority, support [Alt1] from the following alternatives:

- Alt1: up to two TBs of NACK-only feedback is transformed into ACK/NACK HARQ bits and is multiplexed with SR as legacy procedure as Rel-15/16.

  - PUCCH resources configured for unicast will be used in concerned cases subject to legacy procedure as Rel-15/16.

  - For the case of more than two TBs of NACK-only, [FFS: SR] is dropped.

- Alt2:

  - for positive SR, NACK-only is transmitted on the PUCCH resource configured for SR;

    - If the PUCCH resource for SR cannot accommodate multiplexed UCI (e.g., in case of multiple NACK-only bits are multiplexed with SR), [FFS: SR] is dropped.

  - for negative SR, NACK-only feedback is transmitted on the PUCCH resource configured for NACK-only.

Collect views:

| Company | Comments |
| --- | --- |
| FL’s explanation | I doubt we can agree on this proposal in this meeting. However, I am not confident I get companies’ comments clearly and correctly. so let’s at least strive for making the proposals clear enough and we can further discuss the down-selection later or in maintenance. Please suggest the wording modification instead of arguing for which option/alt to go. |

## (UPDATED)More than two NACK-only

Submitted Proposals

Alt 1: NACK-only->ACK/NACK

(Huawei) Proposal 6:

  - When more than one NACK-only based feedback are available for transmission in the same PUCCH slot, the NACK-only feedback is transformed into the ACK/NACK based feedback.

(ZTE) Proposal 2:

  - When multiple NACK-only based feedbacks are indicated to be transmitted in the same slot, support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits(Alt1).

(ZTE) Proposal 4:

  - When one NACK-only PUCCH is transformed into ACK/NACK bit due to multiplexing with unicast PUCCH format 0 carrying 1 bit HARQ-ACK, mapping of values for two HARQ-ACK information bits to sequences reuses what defined for unicast in Rel-16.When 3.1.2one NACK-only PUCCH is transformed into ACK/NACK bit due to multiplexing with unicast PUCCH format 1 carrying 1 bit HARQ-ACK, transmission of two HARQ-ACK information bits reuses what defined for unicast in Rel-16.

(ZTE) Proposal 5:

  - When one NACK-only PUCCH is transformed into ACK/NACK bit due to multiplexing with unicast PUCCH format 0/1 carrying 2 bits HARQ-ACK, HARQ-ACK information bits are transmitted on one new PUCCH format 2/3/4 which is determined by the PRI in the last unicast DCI.

(vivo) Proposal 9:

  - When more than one NACK-only based feedback is available for transmission in the same PUCCH slot, Alt 1 is supported.

    - Alt1: Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits.

(Spreadtrum) Proposal 2:

  - When more than one NACK-only based feedback are available for transmission in the same PUCCH slot, support Alt 1.

(Nokia) Proposal 1:

  - When more than one NACK-only based feedback with the same priority are available for transmission in the same slot, Alt. 1 is used to increase PUCCH capacity of NACK-only based HARQ-ACK feedback:

    - Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits.

(Nokia) Proposal 3:

  - In case of Alt.1, if supported, concatenation of the converted NACK-only bits is based on the ascending order of PUCCH resource IDs that are originally scheduled for NACK-only feedback.

- (Nokia) proposal 9:

  - When PUCCH transmission for the NACK-only based feedback for one MBS collides with PUCCH transmissions for ACK / NACK feedback for another MBS with the same priority, support UE multiplexing the NACK-only based feedback with the ACK / NACK feedback on PUCCH by transforming NACK-only into the ACK / NACK HARQ bit.

    - This applies to at least the case of the feedback addressing one TB. NACK-only based feedback for more than one TBs is to be handled separately.

    - Note: When the TB is correctly decoded, the ACK will be transmitted and multiplexed with others.

    - Converted NACK-only bits are concatenated to the ACK / NACK feedback.

(CATT) Proposal 10:

  - Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits (Alt1) when more than one NACK-only based feedback are available for transmission in the same PUCCH slot.

    - The type of codebook should be indicated in RRC configuration.

    - The last DCI should refer the multicast DCI

(Apple) Proposal 3:

  - Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits.

(LGE) Proposal 7:

  - When different PUCCHs for different NACK only HARQ-ACKs are overlapped, all NACK only HARQ-ACKs are changed to ACK/NACK based HARQ-ACK to be multiplexed.

(Ericsson) Proposal 5:

  - Do not support Alt1.

(Google) Proposal 1:

  - If a UE is configured with NACK-only feedback without a separate PUCCH-Config/PUCCH-ConfigurationList configuration, the UE transforms the NACK-only into ACK/NACK, if the scheduled PUCCH resource is outside of the CFR.

(Google) Proposal 2:

  - Alt1 can be supported if at least one of the PUCCH resource is UE specific PUCCH resource.

(Google) Proposal 3:

  - For UE consuming multiple MBS simultaneously, the UE does not expect to transmit multiple NACK-only based HARQ feedback scheduled by different G-RNTI in the same PUCCH slot.

- Alt 4: combination of NACK-only

(OPPO) Proposal 7:

  - When more than one NACK-only based feedback are available for transmission in the same PUCCH slot, define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission (i.e., Alt 4).

(Xiaomi) Proposal 3:

  - When more than one NACK-only based feedback are available for transmission in the same PUCCH slot, define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission (alt.4).

(NEC) Proposal 1:

  - Support Alt-4 to transmit more than one NACK-only based HARQ-ACK feedback existing in the same PUCCH slot.

    - Alt4: Define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission.

(Samsung) Proposal 2:

  - A UE is provided up to 16 orthogonal PUCCH resources to select from according to combinations of up to 4 ACK/NACK values. For more than 4 ACK/NACK values, the UE reports HARQ-ACK as in Rel-16 if the UE is provided PUCCH resources; otherwise, the UE does not report HARQ-ACK.

(DOCOMO) Proposal 5:

  - When more than one NACK-only based feedback are available for transmission in the same PUCCH slot, support Alt 4.

    - If multiplexed payload size is over the maximum, bundling is applied.

(Lenovo) Proposal 10:

  - For multiple NACK-only feedback multiplexing in same slot, from per UE perspective, the PUCCH resource which corresponds to the first incorrectly decoded PDSCH is selected for transmitting the NACK-only feedback in the PUCCH slot

(Qualcomm) Proposal 5:

  - For RRC_CONNECTED Ues, when more than one NACK-only based feedback are available for transmission in the same PUCCH slot, support

    - Alt4: Define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission.

(Ericsson) Proposal 3:

  - To convey N bits the UE transmits one selected PUCCH resource in a slot, from a bundle of M=2^N-1 configured PUCCH resources. The actually used PUCCH resource in a slot therefore carries the required N HARQ feedback information bits of data received with a single G-RNTI pointing to this UL slot.

(Ericsson) Proposal 4:

  - Change the wording of Alt4 to make it more clear: N HARQ feedback bits are mapped to one out of 2^N-1 PUCCH resources.

(Ericsson) Proposal 6:

  - When one or more NACK-only based feedback bits need be transmitted in same PUCCH slot Alt 4 is supported.

- “Last DCI”

(ZTE) Proposal 6:

  - If HARQ-ACK PUCCH for unicast and NACK-only PUCCH for multicast are determined in the same UL slot, then they are multiplexed for the same priority (regardless of whether they overlap in the time domain).

    - The multiplexed PUCCH resource is determined based on the PRI in the last DCI corresponding to the unicast HARQ-ACK PUCCH.

(Nokia) Proposal 2:

  - In case of Alt.1, if supported, to transmit the multiplexed HARQ-ACK feedback, the UE is configured with specific dedicated PUCCH resource(s), and the UE uses the dedicated PUCCH resource(s) to transmit multiplexed NACK-only bits, rather than selecting the PUCCH resource based on the PRI field of the last DCI.

(CATT) Proposal 10:

  - Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits (Alt1) when more than one NACK-only based feedback are available for transmission in the same PUCCH slot.

    - The type of codebook should be indicated in RRC configuration.

    - The last DCI should refer the multicast DCI

(LGE) Proposal 8:

  - For multiplexing the NACK only based HARQ-ACK feedback for multicast and ACK/NACK-based HARQ-ACK feedback for unicast, determining the PUCCH resources for transmission is based on the PRI indicated in the “last DCI”, where the “last DCI” refers to the last DCI for unicast.

(LGE) Proposal 9:

  - For multiplexing the NACK only based HARQ-ACK feedback for multicast and ACK/NACK-based HARQ-ACK feedback for multicast, determining the PUCCH resources for transmission is based on the PRI indicated in the “last DCI”, where the “last DCI” refers to the last DCI indicating PRI for ACK/NACK-based HARQ-ACK feedback for multicast.

- “UL DAI for NACK-only”

(ZTE) Proposal 7:

  - The DAI filed in UL scheduling DCI is used to indicate the number of PDSCHs received by the UE corresponding to NACK-only feedback.

### Round-1 (closed)

FL’s Comments

For more than one NACK-only based feedback are available for transmission in the same PUCCH slot, two alternatives are still open based on the agreement reached in the last meeting:

Agreement:

When more than one NACK-only based feedback are available for transmission in the same PUCCH slot, further decide based on the following subset of alternatives (from previous agreement) with potential further down-selection:

- Alt1: Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits.

- Alt2: Support sub-slot based PUCCH for this case.

- Alt3: Support UE transmitting more than one slot-based PUCCHs in the same PUCCH slot.

- Alt4: Define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission.

- Alt5: NACK-only bundling

Based on the submitted proposals, the situation is summarized as follows:

Alt1 is supported by (9): Huawei, ZTE, vivo, Spreadtrum, Nokia, CATT, Apple, LGE, Google

- Further details are provided regarding PF 0/1 with 1/2 bits HARQ-ACK (ZTE)

- PUCCH resource is dedicated for NACK-only (Nokia)

- Type of codebook should be indicated in RRC configuration (CATT)

- when a separate PUCCH-Config/PUCCH-ConfigurationList is not configured and not applicable to multiple G-RNTIs (Google)

Alt4 is supported by (8): OPPO, Xiaomi, NEC, Samsung, DOCOMO, Lenovo, Qualcomm, Ericsson,

- up to 16 orthogonal PUCCH resources to select from according to combinations of up to 4 ACK/NACK values. For more than 4 ACK/NACK values, the UE reports HARQ-ACK as in Rel-16 if the UE is provided PUCCH resources; otherwise, the UE does not report HARQ-ACK. (Samsung)

- If multiplexed payload size is over the maximum, bundling is applied. (DOCOMO)

- the PUCCH resource which corresponds to the first incorrectly decoded PDSCH is selected for transmitting the NACK-only feedback in the PUCCH slot (Lenovo)

- To convey N bits the UE transmits one selected PUCCH resource in a slot, from a bundle of M=2^N-1 configured PUCCH resources. Change the wording of Alt4 to make it more clear: N HARQ feedback bits are mapped to one out of 2^N-1 PUCCH resources.  (Ericsson)

The concern for Alt1 is that it compromised adopting NACK-only based feedback for reducing PUCCH resources overhead. The concern for Alt2 is the specification effort and the demand of a lot of PUCCH resources to correspond to the different combinations of NACK-only for multiple TBs. The two alternatives are supported by 9 vs 8 and such situation has not been changed much for a couple of meetings except more details are being brought up by companies in the direction of the supported alternative.

Given it is the last meeting, FL would like to suggest a compromised solution for way forward so that more details can be finalized in time as well for the compromised solution. For the compromised solution, the proposal from Samsung can be a starting point, especially considering for Alt4 defining the combinations of NACK-only the maximum number of TBs for the feedback for combination of NACK-only has to be determined anyhow.

FL’s Proposal:

#### Proposal  3.2.1

- When more than one NACK-only feedback are available for transmission in the same PUCCH slot for a given G-RNTI,

- define up to 16 orthogonal PUCCH resources to select from according to combinations of up to 4 TBs with NACK-only feedback, e.g., (A,N,N,N)->PUCCH resource 0, (A,A,N,N)->PUCCH resource 1, ….;

  - The PUCCH resource for transmission is determined by the last multicast DCI.

- for more than 4 TBs with NACK-only feedback, NACK-only feedback is transformed into ACK/NACK based, i.e., the UE reports HARQ-ACK as in Rel-16 if the UE is provided PUCCH resources for unicast. Otherwise, the UE does not report HARQ-ACK.

- UE is not expected to transmit more than one NACK-only feedback corresponding to different G-RNTIs in the same PUCCH slot.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | We can support both alternatives for compromise: For compromise of Alt 1 and Alt 4, we think - Alt 1 can be adopted when the multiple NACK-only feedback is to be multiplexed on PUSCH with same priority or PUCCH for unicast with same priority; - Alt 4 can be adopted when the multiple NACK-only feedback is to be transmitted in one PUCCH without multiplexing with other PUSCH/PUCCHs. After Alt 4 is agreed, then we start discussion on details of Alt 4. |
| - LG Electronics | - We prefer Alt 1. We already agree to support UE multiplexing the HARQ-ACK bits with ACK/NACK by transforming NACK-only into ACK/NACK HARQ bits. The same approach can be applied to this case. |
| - CATT | - Prefer Alt1. |
| - CMCC | - We support Alt 4 in our contribution. |
| - Nokia, NSB | The proposal needs further discussion and explanation. We cannot agree with the current version. In our view, there is no reason to limit the proposal to one G-RNTI. There can be multiple NACK-only feedback scheduled in the same slot for, e.g., 2 G-RNTIs. The proposed methods can as well work the same if we do not limit them to one G-RNTI. On the other hand, we do not understand how the last multicast DCI determines the PUCCH resource (for <= 4 TBs), since the combination to PUCCH resource mapping is already in place. This is not a correct understanding. Moreover, what is technical reason to determine the threshold value “4”? Also, in case Alt 1 is used, since the last PRI will be a multicast DCI, we believe that this PRI cannot optimally select a unicast PUCCH resource. Thus, specific PUCCH resource(s) for the purpose of transmitting NACK-only multiplexed feedback should be configured to the Ues. |
| - Vivo | Not support. We echo LG’s comment. In addition, if alt 4 is adopted, then the PUCCH resource with more than one NACK overlaps with PUCCH for unicast HARQ-ACK/CSI/PUSCH, how to do? |
| - Xiaomi | We prefer option 4 in our contribution. If it is not acceptable, we can live with option 1 as it is much simpler than the current proposal. |
| - NEC | - We prefer Alt 4. For the proposal 3.2.1, maybe it should be explained why set 4 as the switching threshold. |
| - Samsung | Support. Alt.4 is the only “NACK-only” option – Alt.1 is basically saying there is no support of “NACK-only” when it actually matters more for PUCCH resource overhead. Alt.1 can be accepted for spec completeness due to the limitation of Alt.4 as the number of bits increases – however, Alt.4 is sufficient for practically all expected deployments. - The case of multiple G-RNTIs can be FFS although we are fine with the proposal on that aspect as well – it may be possible to support under certain restrictions (e.g. all Ues have same G-RNTIs) but it is unclear if that is meaningful (e.g. single G-RNTI could then be used). |
| Ericsson | P3.2.1: If the reason for using NACK-only is UL congestion, with lack of PUCCH resources for ACK/NACK, then we think Alt1 cannot work, since there are then not enough PUCCH resources for all UEs to send ACK/NACK (had there been, why use NACK-only in the first place?). For this scenario, only Alt4 is therefore applicable. However, there may be other scenarios for NACK-only, e.g. to allow for UE power saving. If that is the reason, there may very well be available PUCCH resources for ACK/NACK, so Alt1 could then work. Alt4/NACK-only would then be used, for power saving reasons, most of the time, but occasionally there could be a need to transmit more bits than what Alt4 can support and in this case Alt1 can be used as a temporary fallback. We do however not see the combination of UE power saving and occasional high number of HARQ feedback bits as an important enough scenario, but if other companies think it is, we could compromise on the FL’s proposal, provided the max no of Alt4 bits is not fixed to 4 NACK-only bits, but can be RRC configurable to a higher number of bits, the value of which can be further discussed. We note however that the current maximum number of PUCCH resources that can be configured for the UE is 128 (7 bits) and the maximum number of PUCCH resources per PUCCH resource set (maxNrofPUCCH-ResourcesPerSet) is in Rel-16 32 (5 bits). Regarding the last bullet point/sentence, we propose a reformulation to make this clearer: “the UE is not expected to transmit NACK-only feedback for more than one G-RNTI in an UL slot”. Finally, it should be noted that the required number of PUCCH resources to transmit N bits is 2^N-1, so with four bits 15 PUCCH resources are required (since AAAA does not require any PUCCH resource). A final comment is that this proposal (P3.2.1) needs to be harmonized with P3.3.1, which currently includes the value 16 for the maximum number of PUCCH resources in a PUCCH resource set. |
| Qualcomm | We prefer to avoid the case that “there are more than 4 TBs with NACK-only feedback and UE is not provided PUCCH resources for unicast” by gNB scheduling, instead of dropping HARQ-ACK by UE. |
| NTT DOCOMO | We can support this proposal as a compromise. |
| Apple | Alt1 is preferred. The Alt 4 is too restrictive, such as only up to 4TB for a given G-RNTI. If users subscribe multiple MBS services, Alt.4 could not provide HARQ-ACK feedback. |
| Lenovo 2 | If companies can’t agree with the proposed compromised way forward, we support Alt 4 as Alt 1 is against the motivation of introducing NACK-only feedback and more PUCCH resources should be reserved. For Alt 4, we think the details can be FFS |
| ZTE | Alt.1 is our first preference. For compromise, we can also support Alt.4. However, we don’t the current Proposal 3.2.1 is a good way forward considering the following reasons 1) it can only address the case when multiple NACK-only bits for the same G-RNTI in the PUCCH slot, but not for the multiple G-RNTI cases 2) the PUCCH configuration/overhead issue would be serious, e.g., 16*N PUCCH resources are needed per slot for N G-RNTIs. |
| OPPO | We suggest to agree Alt. 4 only, as Option 1 is ACK/NACK based HARQ-ACK feedback, which can be achieved by simply disabling NACK-only based HARQ-ACK feedback. |
| Spreadtrum | Our first preference is Alt1, for it is one general solution. We can also accept Alt4, if it is the majority. But we don’t want to support both. It is complex for UE’s implementation. |
| FL’s responses | Based on all the comments above, we either agree on the compromised direction as the proposal suggests (can be updated as suggested) or neither one is to be agreed otherwise. So I will continue making the efforts on revising the proposal, and please offer the suggestion if you have. #### Proposal 3.2.1-r1 - When more than one NACK-only feedback are available for transmission in the same PUCCH slot for a given G-RNTI, - define up to [A] orthogonal PUCCH resources to select from according to combinations of up to [B] TBs with NACK-only feedback, - for more than [B] TBs with NACK-only feedback, NACK-only feedback is transformed into ACK/NACK based, i.e., the UE reports HARQ-ACK as in Rel-16 if the UE is provided PUCCH resources for unicast. Otherwise, the UE does not report HARQ-ACK. - FFS: values of A and B. - FFS: UE is not expected to transmit more than one NACK-only feedback corresponding to different G-RNTIs in the same PUCCH slot. |

### Round-2 (closed)

#### Proposal  3.2.2

- When more than one NACK-only feedback are available for transmission in the same PUCCH slot for a given G-RNTI,

- define up to [A] orthogonal PUCCH resources to select from according to combinations of up to [B] TBs with NACK-only feedback,

- for more than [B] TBs with NACK-only feedback, NACK-only feedback is transformed into ACK/NACK based, i.e., the UE reports HARQ-ACK as in Rel-16 if the UE is provided PUCCH resources for unicast. Otherwise, the UE does not report HARQ-ACK.

- FFS: values of A and B.

- FFS: UE is not expected to transmit more than one NACK-only feedback corresponding to different G-RNTIs in the same PUCCH slot.

Collect views:

| Company | Comments |
| --- | --- |
| - FL’s explanation | - Please check the discussion and FL’s responses in Round-1 to get why the proposal is updated as is. |
| - Samsung | - Support the proposal in order to progress this issue. - In our opinion, it is also OK to not report HARQ-ACK for more than [B] TBs (i.e. not transform to ACK/NACK in that case) – at least if CA is not supported, having A=15 orthogonal resources can work for any deployment and is easily realizable (~1 RB). But for completeness of the specifications, something needs to be said for what happens when there are more than [B] TBs. - Regarding the FFS, we do not think it is possible to have the considered restriction, at least for TDD, even when there are only two G-RNTIs – with larger number of G-RNTIs, it may not be possible even for FDD without reductions on supportable data rates. |
| - Qualcomm | - Question for clarification: - Is the ACK/NACK-based PUCCH resources for unicast configured in the same PUCCH slot or different? If it can be different, how to decide the PUCCH for reporting? |
| - LG Electronics | - We do not support this proposal for which different G-RNTIs in the same PUCCH slot cannot be supported. - We think that for different G-RNTIs in the same PUCCH slot, NACK-only feedback can be also transformed into ACK/NACK based feedback. |
| - ZTE | - We don’t support this proposal. Similar view as LG, we think NACK-only bits for different G-RNTIs should be allowed to be in the same PUCCH slot. - As clarified in the previous, we can accept either Alt.1 or Alt.4. |
| - CMCC | - We can compromise this proposal for the sake of progress. |
| - vivo | - Not support. We prefer to select one rather than both. For alt 4, if the PUCCH for multiple NACK-only overlaps with other PUCCH for unicast HARQ-ACK/CSI/PUSCH, we still need to adopt alt 1 to transform NACK only to ACK/NACK, otherwise, dropping would happen. We already agree to support UE multiplexing the HARQ-ACK bits with ACK/NACK by transforming NACK-only into ACK/NACK HARQ bits for other cases. Unified solution is preferred. |
| - OPPO | - There is no need to restrict to one G-RNTI, the resource could be applied to multiple G-RNTIs if all UEs sharing the resource are configured with same G-RNTIs, anyhow it is up to gNB to configure. - Similar view as Samsung, if B can be defined up to 4, the 2nd sub-bullet is not necessary, if more than 4 TBs is possible, gNB can simply configure ACK/NACK based feedback. We suggest the following changes: - When more than one NACK-only feedback are available for transmission in the same PUCCH slot for a given G-RNTI, - define up to [A] orthogonal PUCCH resources to select from according to combinations of up to [B] TBs with NACK-only feedback, - for more than [B] TBs with NACK-only feedback, NACK-only feedback is transformed into ACK/NACK based, i.e., the UE reports HARQ-ACK as in Rel-16 if the UE is provided PUCCH resources for unicast. Otherwise, the UE does not expect to report HARQ-ACK for more than [B] TBs. - FFS: values of A and B. - FFS: UE is not expected to transmit more than one NACK-only feedback corresponding to different G-RNTIs in the same PUCCH slot. |
| - Lenovo, Motorola Mobility | - We are supportive of defining A and B and ask whether A is equal to B? - For more than B case, we don’t support ACK/NACK transformation due to resource overhead. In this case, we think Alt 4 should be supported to compress the bits to A. |
| - Xiaomi | - We prefer make a down selection between original alt 1 and original alt 4. The current mechanisms is too complicated. As mentioned before, we can accept either alt 1 or alt 4. |
| - Spreadtrum | - Either one of Alt 1 and Alt 4 is OK for us. But don’t support both. |
| - CATT | - Seems that supporting both alternatives make this issue more complicated. We prefer to select one rather than both. |
| - NTT DOCOMO | - We are generally fine with the proposal. |
| - Nokia, NSB. | For progress, we can support the proposal with minor modification. In case Alt 1 is used, since the last PRI will be a multicast DCI, we believe that this PRI cannot optimally select a unicast PUCCH resource. Thus, specific PUCCH resource(s) for the purpose of transmitting NACK-only multiplexed feedback should be configured to the UEs. - When more than one NACK-only feedback are available for transmission in the same PUCCH slot for a given G-RNTI, - define up to [A] orthogonal PUCCH resources to select from according to combinations of up to [B] TBs with NACK-only feedback, - for more than [B] TBs with NACK-only feedback, NACK-only feedback is transformed into ACK/NACK based, i.e., the UE reports HARQ-ACK as in Rel-16 if the UE is provided UE-specific PUCCH resources for unicast. Otherwise, the UE does not report HARQ-ACK. - FFS: Which PUCCH resources UE uses. - FFS: values of A and B. - FFS: UE is not expected to transmit more than one NACK-only feedback corresponding to different G-RNTIs in the same PUCCH slot. |
| - Apple | We prefer Alt1. As compromise, we are ok with this proposal in general, the last bullet, i.e., FFS, can be removed, as this limitation is to Alt4. It’s not necessary for the compromised proposal. |
| - Ericsson | - We are fine with the main part of the proposal, which covers the case with one G-RNTI (“for a given G-RNTI”). - However, there seems to be disagreements related to the case of different G-RNTIs per UE. The technical solution for a single G-RNTI per UE is clear, but how the solution should work for multiple G-RNTIs is unclear. We suggest to agree the single G-RNTI case and leave the multiple G-RNTI case FFS for now #### Proposal 3.2.2 - When more than one NACK-only feedback are available for transmission in the same PUCCH slot for a given G-RNTI, - define up to [A] orthogonal PUCCH resources to select from according to combinations of up to [B] TBs with NACK-only feedback, - for more than [B] TBs with NACK-only feedback, NACK-only feedback is transformed into ACK/NACK based, i.e., the UE reports HARQ-ACK as in Rel-16 if the UE is provided PUCCH resources for unicast. Otherwise, the UE does not report HARQ-ACK. - FFS: values of A and B. - FFS: NACK-only feedback for multiple G-RNTIs per UE in a given PUCCH slot - About the solution for different G-RNTIs: Our assumption from earlier discussions was that a UE shall not be required to transmit multiple NACK-only signals in the same UL slot. If this is correct, then either - a) the NACK-only codebook from multiple G-RNTIs need to be mapped to a common PUCCH resource bundle, or - b) the UE needs to send NACK-only feedback for different G-RNTIs in different UL slots. - We have not seen a solution for case a) has been presented yet. For the special case that all the NACK-only for multiple G-RNTIs is configured identical set of UEs the solution could be easy. |

### Round-3 (closed)

#### Questions for more discussion

Proposal  3.2.3

- When more than one NACK-only feedback are available for transmission in the same PUCCH slot for a given G-RNTI,

- define up to [A] orthogonal PUCCH resources to select from according to combinations of up to [B] TBs with NACK-only feedback,

- for more than [B] TBs with NACK-only feedback, NACK-only feedback is transformed into ACK/NACK based, i.e., the UE reports HARQ-ACK as in Rel-16 if the UE is provided UE-specific PUCCH resources. Otherwise, the UE does not report HARQ-ACK.

  - FFS: Which PUCCH resources UE uses.

- FFS: values of A and B.

- FFS: UE is not expected to transmit more than one NACK-only feedback corresponding to different G-RNTIs in the same PUCCH slot.

FL’s comments:

Still different views for this proposal from the last round but I assume this proposal is the only direction I see agreeable to the group. Therefore, I would try to make efforts in this direction.

Following the above updated proposal (or the original proposal from Samsung), there are 16 (or 15) PUCCH resources in one PUCCH resource set and up to 4 TBs for NACK-only based feedback. Are we assuming different TB combinations from different UEs can share the same PUCCH resource as the following table (i.e., UE1 for TB#1, UE2 for TB2-TB3, UE3 for TB#2-TB#3-TB#4, and UE4 for TB#4-TB#5-TB#6-TB#7 can share the same PUCCH resource ID#1)? Alternatively, are we assuming always the same TBs combination for different UEs (i.e., TB#4-TB#5-TB#6TB#7 for all UEs can share the PUCCH resource ID#1)?

If it is the former case, from network perspective, which TBs need to be retransmitted is not differentiable.

If it is the latter case, from network perspective, always the same number of TBs and the same TB combinations are scheduled to different UEs that are to be transmitted in the same PUCCH slot.

| PUCCH resource set | TB#1 | TB#2-TB#3 | TB#2-TB#3-TB#4 | TB#4-TB#5-TB#6-TB#7 |
| --- | --- | --- | --- | --- |
| PUCCH resource ID#1 | N | N-N | N-N-N | N-N-N-N |
| PUCCH resource ID#2 | (A) | A-N | A-N-N | A-N-N-N |
| PUCCH resource ID#3 |  | N-A | N-A-N | N-A-N-N |
| PUCCH resource ID#4 |  | (A-A) | A-A-N | A-A-N-N |
| PUCCH resource ID#5 |  |  | N-N-A | N-N-A-N |
| PUCCH resource ID#6 |  |  | A-N-A | A-N-A-N |
| PUCCH resource ID#7 |  |  | N-A-A | N-A-A-N |
| PUCCH resource ID#8 |  |  | (A-A-A) | A-A-A-N |
| PUCCH resource ID#9 |  |  |  | N-N-N-A |
| PUCCH resource ID#10 |  |  |  | A-N-N-A |
| PUCCH resource ID#11 |  |  |  | N-A-N-A |
| PUCCH resource ID#12 |  |  |  | A-A-N-A |
| PUCCH resource ID#13 |  |  |  | N-N-A-A |
| PUCCH resource ID#14 |  |  |  | A-N-A-A |
| PUCCH resource ID#15 |  |  |  | N-A-A-A |
| PUCCH resource ID#16 |  |  |  | (A-A-A-A) |

Collect views:

| Company | Comments |
| --- | --- |
| - FL’s explanation | - Please share your view on this question. The answer will be helpful to understand how more than one NACK-only feedback is going to work. It also affect the proposals regarding the PUCCH resources configurable for NACK-only. |
| - ZTE | - Thanks for the detailed explanation. Based on our understanding, different UEs may receive different multicast services, it would be difficult to pre-configure different PUCCH resources for NACK-only for different UEs. If Alt.4 is selected, similar issue will occur. Since we are at the last meeting of Rel-17, it seems Alt.1 may be the better and clean solution. - If companies have any concern on the PUCCH resource configuration for Alt.1, we can update it as following. In this case, UE can directly reuse the PUCCH resources configured for unicast. Proposal: When more than one NACK-only based feedback are available for transmission in the same PUCCH slot, support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits (Alt.1) if UE is configured with PUCCH resources for unicast. - FFS: Alt.4 if UE is NOT configured with PUCCH resources for unicast. |
| - Samsung | - Support in principle. - It should be obvious that if there is a resource problem for 1 ACK/NACK bit (in order to have NACK-only), there is a much bigger resource problem for >2 ACK/NACK bits. Alt.1 is not acceptable - it is preferable over Alt.1 to drop HARQ-ACK if more than 1 bit. There is also no issue with having mixtures of G-RNTIs as PUCCH resources are UE-specific and G-RNTIs are limited (no issue having orthogonal resources for G-RNTI mixtures – few RBs are enough). - For more than 4 HARQ-ACK bits, either converting to ACK/NACK or, preferably, dropping is OK. - For specification/operation consistency with the case of 1 bit, and with the agreement that the UE does not transmit when all HARQ-ACK bits are ACK, the last entry (#16) in the above table should be removed (and most of the time a UE will have “all ACK”). |
| - Spreadtrum | - Thanks FL for the detailed elaboration. In our understanding, the latter case is not practical, considering different UEs may support different number of G-RNTI/G-CS-RNTI. We prefer Alt1, which is one general solution. Regarding the PUCCH resource issue, we don’t think it is one issue for Alt1. We have agreed that PUCCH resources for unicast can be used when PUCCH resources for ACK/NACK based not configured, and there is no PUCCH capacity issue for unicast. But we are fine with ZTE’s revision. |
| - CMCC | - Support |
| - CATT | - Still prefer Alt1, and fine with ZTE’s version. |
| - NTT DOCOMO | - Support |
| - vivo | - We have similar view with ZTE. Alt 4 is not support to support multiple G-RNTI. Because different Ues may have different G-RNTIs, for example, UE 1 is configure with G-RNTI1 and G-RNTI2, UE 2 is configured with G-RNTI1 and G-RNTI3, UE 3 is configured with G-RNTI 2. gNB may need to reserve PUCCH resources for all kinds of combination of G-RNTIs. What's more, this is only for one TB, considering multiple TB case, it is impractical for gNB to configure the PUCCH resources. In addition, if alt 4 is adopted and the resultant PUCCH for multiple NACK is overlapped with other PUCCH/PUSCH, NACK-only still needs to be transformed to ACK/NACK. - We still prefer alt 1, and fine with ZTE’s version. |
| - TD Tech, Chengdu TD Tech | - ok |
| - Lenovo, Motorola Mobility | - We can accept the first sub-bullet and don’t support the second sub-bullet. - It is weird with second sub-bullet where PUCCH resources are sufficient for A/N based feedback when the number of TBs is larger than B while PUCCH resource is problem for A/N based feedback when the number of TBs is not larger than B. We can’t agree with it. - From our side, when the number of TBs is larger than B, HARQ-ACK bundling should be performed to guarantee the number of NACK-only feedback is not larger than B. E.g., setting A=4, B=2, when there are 4 bits need to be transmitted, denoted as a0,a1,a2,a3, then two bits, a0 AND a1, a2 AND a3, are transmitted using the 4 resources. |
| - Nokia, NSB | - In our view, to support multiple G-RNTIs, the latter explanation from the FL is valid. That would indeed require all the UEs receive the same G-RNTIs. If that is not the case, then the gNB should ensure that UEs receiving different G-RNTIs are scheduled for feedback in different slots. - Considering that there are significant opposition from both sides, if we agree on the above proposal, we will see that companies will keep defending their sides in the next meeting (since the above proposal does not preclude both options). Therefore, perhaps either a down-selection is made, or FFSs are targeted by the proposal itself. |
| - Qualcomm | - Ok |
| - LG Electronics | - We still prefer Alt 1 concerning lack of support for multiple G-RNTIs in alt 4. |
| - Ericsson | - We support the FL’s proposal. - It is true that, with Alt4, the UE cannot send NACK-only feedback for multiple G-RNTIs in the same UL slot. However, the main use case for NACK-only feedback is PUCCH UL congestion and in such circumstances Alt1 is not possible to use at all, so would not be a solution to this problem. In such situations the gNB can instead - by implementation – let the UE send NACK-only feedback in different UL slots. This is the best that can be done in such a case and we will have to accept that. For a use case where NACK-only is used without PUCCH congestion there may be PUCCH ACK/NACK capacity available and in that case the UE may fall back to Alt1. - @ZTE: You mention “Based on our understanding, different UEs may receive different multicast services, it would be difficult to pre-configure different PUCCH resources for NACK-only for different UEs.” With Alt4 the PUCCH configurations would be per G-RNTI, so that all UEs that receive a certain G-RNTI would need to have the same PUCCH configuration to be used with this G-RNTI. Then different G-RNTIs can use different non-overlapping PUCCH configurations – one set per G-RNTI. This is not difficult. |

### Round-4 (closed)

FL’s Comments

Following the discussion in Round-3:

Assuming only one resource set is defined for NACK-only and up to 16 PUCCH resources for the set to support up to 4 TBs, it could be up to network scheduling via k1 to indicate how many and which TBs for the NACK-only combination feedback in the same PUCCH slot. From this sense, it can work as well.

| PUCCH resource set | TB#1 | TB#2-TB#3 | TB#2-TB#3-TB#4 | TB#4-TB#5-TB#6-TB#7 |
| --- | --- | --- | --- | --- |
| PUCCH resource ID#1 | N | N-N | N-N-N | N-N-N-N |
| PUCCH resource ID#2 | (A) | A-N | A-N-N | A-N-N-N |
| PUCCH resource ID#3 |  | N-A | N-A-N | N-A-N-N |
| PUCCH resource ID#4 |  | (A-A) | A-A-N | A-A-N-N |
| PUCCH resource ID#5 |  |  | N-N-A | N-N-A-N |
| PUCCH resource ID#6 |  |  | A-N-A | A-N-A-N |
| PUCCH resource ID#7 |  |  | N-A-A | N-A-A-N |
| PUCCH resource ID#8 |  |  | (A-A-A) | A-A-A-N |
| PUCCH resource ID#9 |  |  |  | N-N-N-A |
| PUCCH resource ID#10 |  |  |  | A-N-N-A |
| PUCCH resource ID#11 |  |  |  | N-A-N-A |
| PUCCH resource ID#12 |  |  |  | A-A-N-A |
| PUCCH resource ID#13 |  |  |  | N-N-A-A |
| PUCCH resource ID#14 |  |  |  | A-N-A-A |
| PUCCH resource ID#15 |  |  |  | N-A-A-A |
| PUCCH resource ID#16 |  |  |  | (A-A-A-A) |

#### Proposal  3.2.4

- When more than one NACK-only feedback are available for transmission in the same PUCCH slot for a given G-RNTI,

- define up to [A] orthogonal PUCCH resources to select from according to combinations of up to [B] TBs with NACK-only feedback,

- for more than [B] TBs with NACK-only feedback, NACK-only feedback is transformed into ACK/NACK based, i.e., the UE reports HARQ-ACK as in Rel-16 if the UE is provided UE-specific PUCCH resources. Otherwise, the UE does not report HARQ-ACK.

  - FFS: Which PUCCH resources UE uses.

- FFS: values of A and B.

- FFS: UE is not expected to transmit more than one NACK-only feedback corresponding to different G-RNTIs in the same PUCCH slot.

Collect views:

| Company | Comments |
| --- | --- |
| - FL’s explanation | - the proposal is not updated. - Please check FL’s comments at the beginning of this Round-4. Hopefully we can agree on this proposal in this meeting. Otherwise, we may end up not supporting more than one TB of NACK-only in the same PUCCH slot. |
| - Samsung | - Still generally OK with the proposal. - Having multiple G-RTNIs is not a problem – e.g. for G-RNTI#0, G-RNTI#1, G-RNTI#2, the gNB can configure separate resources for applicable combinations of G-RNTIs having NACK-only – implementation issue - required resources are still small (e.g. <=8 RBs) and whether to have them is controlled by the gNB. PUCCH resource determination and overhead are actually much more challenging, if at all possible in practice, for ACK/NACK-based feedback for multiple G-RNTIs without C-RNTI (not discussed in this meeting, will need to resolve in maintenance). - The proposal is inconsistent with the case of 1 bit. UEs will transmit even when everything is ACK (last entries) - that will be typical and result to unnecessary transmissions that provide no information, interference spikes, and non-uniform specifications/UE behavior that depend on the number of bits. |
| - ZTE | - Thanks Ericsson’s explanation in last round. Now we understand the intention of this proposal. - Now there are two main motivations here - M1) multiplex NACK-only bits for multiple TBs of the same G-RNTI; - M2) multiplex NACK-only bits for multiple TBs from different G-RNTIs - Configuring one PUCCH resource set for each G-RNTI can address the motivation M1 above. However, it can’t address motivation M2. Configuring a PUCCH resource set for all the G-RNTIs can address motivation M2, but it may not well address the motivation of M1. We think this can be part of the further discussion of Alt.4. - For Alt.1, the main remaining issue is how to indicate the PUCCH resource for the transferred ACK/NACK feedback - Since both Alt.1 and Alt.4 have some left issues, we would suggest to resolve the remaining issues of them first and then try to pick one of them. We are not ready to agree on the current combined proposal at this stage with remaining issues for both Alt.1 and Alt.4. |
| - vivo | - We still prefer to support alt 1 only. In addition, we still get no answer if alt 4 is used, and the PUCCH for multiple NACK-only overlaps with PUCCH for unicast HARQ-ACK/CSI/PUSCH, how to do. |
| - CATT | - Fine with the proposal for progress. |
| - Nokia, NSB | - We agree with ZTE that remaining issues for both sides should be resolved. Otherwise, the proposal will lead to similar discussions in the next meeting. - Regarding ZTE’s question on which resource to use for Alt.1, one solution is the one we proposed in our contribution, so that the UEs are configured with specific resources for that purpose (in unicast or multicast PUCCH-config). - In our view, it does not make much sense to configure separate PUCCH resource sets for each G-RNTI, rather the same resource set should be used. Therefore, Alt. 4 could work for multiple G-RNTIs if their feedback is scheduled in separate slots. |
| - CMCC | - Support this proposal. We acknowledgement companies’ comments that the feedback of multiple TBs of multiple G-RNTIs can not be supported in Alt 4, but as other companies’ response, gNB can scheduling different PUCCHs in different slots. As the concern of vivo, we think even for multiple NACK-only feedback case, the previous agreement to transform NACK-only into ACK/NACK HARQ bit and multiplexing with unicast HARQ-ACK/CSI/PUSCH can still work. |
| - Lenovo, Motorola Mobility | - We don’t support this proposal which brings more issues. - It is weird with second sub-bullet where PUCCH resources are sufficient for A/N based feedback when the number of TBs is larger than B while PUCCH resource is problem for A/N based feedback when the number of TBs is not larger than B. We can’t agree with it. - From our side, when the number of TBs is larger than B, HARQ-ACK bundling should be performed to guarantee the number of NACK-only feedback is not larger than B. E.g., setting A=4, B=2, when there are 4 bits need to be transmitted, denoted as a0,a1,a2,a3, then two bits, a0 AND a1, a2 AND a3, are transmitted using the 4 resources. |
| - Ericsson | - Support - For companies that wish to have the bigger picture how Alt 4 could work with multiple G-RNTIs, please check the explanation below. - Referring to ZTE’s classification of M1 and M2: - M1 = multiplex NACK-only bits for multiple TBs of the same G-RNTI - M2 = multiplex NACK-only bits for multiple TBs from different G-RNTIs - With the current limitation of one transmitted PUCCH resource per UL slot, we do not think it is viable to support NACK feedback of multiple G-RNTIs per UE in the same UL slot, since this would require a jointly constructed set of PUCCH resources that would need to cover all NACK-only bits of several/many G-RNTIs by selecting one single PUCCH resource. This is not practically feasible considering the combinatorial explosion when potentially many G-RNTIs are involved (taken over all UEs). - Instead, we suggest that each UE send NACK-only feedback for only one G-RNTI in a given UL slot. At the same time, other UEs could send NACK-only feedback for the same or different G-RNTIs in the same UL slot. All G-RNTIs, for which NACK-only feedback is sent in the same UL slot would need to have their own separate set of PUCCH resources, which is common among all UEs receiving this G-RNTI. - With e.g. four NACK bits per G-RNTI, each G-RNTI would need 2^4-1=15 PUCCH resources in a slot. In the slot, many other orthogonal sets of PUCCH resources could also be configured for the same and different UEs. - Each UE is only configured with one or more sets of PUCCH resources, corresponding to the number of G-RNTIs, for which it needs to send NACK-only feedback. - When a UE needs to send NACK-only feedback for multiple G-RNTIs (M2 case), it transmits this in multiple UL slots, with one slot per G-RNTI. The particular PUCCH resource to be transmitted, corresponds, in each UL slot, to the NACK-only bit combination that applies for the G-RNTI associated with that UL slot. - For each received G-RNTI, the UE would use the DCI indication for which UL slot to use for the feedback. - However, when the UE needs to send feedback for multiple G-RNTIs, it can happen that the indication for NACK-only feedback for two different G-RNTIs points to the same UL slot, which causes a conflict. - To resolve this conflict, there is a need for a specification rule, e.g. that the UE sends feedback for the G-RNTI with the lowest RNTI in the indicated UL slot and the other G-RNTIs are sent in subsequent UL slots in order of their RNTI number. - Unless each G-RNTI is associated with a unique UL slot, with no other G-RNTI feedback in this slot, the issue of conflict is unavoidable, we think. Consider e.g. a simple case with four G-RNTIs: G-RNTI1, G-RNTI2, G-RNTI3 and G-RNTI4. - The DCIs of G-RNTI1 and G-RNTI2 both point to UL slot K, whereas the DCIs of G-RNTI3 and G-RNTI4 both point to UL slot K+1. - A UE that needs to receive G-RNTI1 and G-RNTI3 can do that just following the indication, without conflict, but a UE that needs to receive G-RNTI1 and G-RNTI2 will have a conflict if it just follows the DCI indication, since this will point to slot K for both G-RNTIs. Following the suggested rule above, if G-RNTI is counted as “lower value” than G-RNTI2, this means that the UE would transmit NACK only feedback for G-RNTI1 in UL slot K and for G-RNTI2 in UL slot K+1, which would resolve the conflict. - The specification impact is that - each UE can be configured with one or more sets of PUCCH resources (“PUCCH resource bundles” in our contribution), where each set corresponds to one G-RNTI. - a rule is specified to resolve conflict of UL slot, when the DCI of multiple G-RNTIs point to the same UL slot - With this, we believe Alt 4 will work for both ZTE’s cases M1 and M2 cases, with small specification impact. - We also think the HARQ ACK budling that Lenovo mentions is interesting. That could resolve the bit limitation issue of Alt4. When e.g. four bits are supported with Alt4, and there is NACK-only feedback for six TBs to transmit, then the UE could e.g. transmit the first three bits as usual and the last bit would be a NACK-only bundling bit that would cover remaining (here three) TBs bits. This bit would be '1' (meaning NACK) if any of the remaining (three) TBs related to the bundled bits are erroneous. The “Otherwise, the UE does not report HARQ-ACK.” in the proposal could therefore be replaced by: - “Otherwise, the UE uses the last NACK-only bit as a NACK-only HARQ-ACK bundling bit with all remaining bits bundled.” |
| - NTT DOCOMO | - We are generally fine with the proposal. |
| - Apple | - This proposal limit a given G-RNTI, do we need another proposal to support multiple G-RNTI in case of user subscribed multiple multicast services? Currently we agreed PUCCH-config is applied to all G-RNTIs with ACK/NACK based feedback, do we intend to define separate PUCCH-configs for each G-RNTI for NACK-based feedback to support multiple G-RNTI, how to select the resource in that case? - The issue is only coming from the sub-bullet, if it is allowed that NACK based feedback falls back to ACK/NACK based feedback, there is not any issue. |

### Round-5

FL’s Comments

#### Proposal  3.2.5: Continue discussing the following proposal

- When more than one NACK-only feedback are available for transmission in the same PUCCH slot for a given G-RNTI,

- define up to [A] orthogonal PUCCH resources to select from according to combinations of up to [B] TBs with NACK-only feedback,

- for more than [B] TBs with NACK-only feedback, NACK-only feedback is transformed into ACK/NACK based, i.e., the UE reports HARQ-ACK as in Rel-16 if the UE is provided UE-specific PUCCH resources. Otherwise, the UE does not report HARQ-ACK.

  - FFS: Which PUCCH resources UE uses.

- FFS: values of A and B.

- FFS: UE is not expected to transmit more than one NACK-only feedback corresponding to different G-RNTIs in the same PUCCH slot.

Collect views:

| Company | Comments |
| --- | --- |
| - FL’s explanation | - Given the situation from the last round discussion, people are not convinced enough by this proposal, so I would suggest we continue the discussion. |

## PUCCH resource

Submitted Proposals

“Resource sets/resources”

(ZTE) Proposal 1:

  - Regarding the separate PUCCH-Config/PUCCH -ConfigurationList that is optionally configured for NACK-only based HARQ-ACK feedback for multicast.

    - Up to 1 PUCCH resource set in each PUCCH-Config.

    - Up to 32 PUCCH resources can be configured in each PUCCH resource set.

    - The PUCCH resources apply to all G-RNTIs configured to UE for the NACK-only based feedback.

    - PUCCH index determination for the transmission of NACK-only based feedback reuse what defined for unicast.

(CATT) proposal 6:

  - NACK-only based feedback framework is designed based on Rel-15 NR ACK/NACK-based feedback mechanism by considering k1, PRI and DCI CCE index to indicate the PUCCH resource.

(CATT) proposal 7:

  - Up to 32 PUCCH resources can be configured for the PUCCH resource set for NACK-only feedback in MBS.

(CATT) proposal 8:

  - The PUCCH resource set can be used by all MBS services which applying NACK-only based HARQ-ACK feedback.

(CATT) Proposal 9:

  - When NACK-only based feedback for receiving multicast is not configured the separate PUCCH-Config/PUCCH-ConfigurationList from the configuration for unicast, the PUCCH-Config/ PUCCH-ConfigurationList for unicast is applied (Alt2).

(OPPO) Proposal 6:

  - The PUCCH resource for NACK-only based HARQ-ACK feedback is configured per G-RNTI.

(Intel) Proposal 9:

  - If separate PUCCH-ConfigurationList is not configured for NACK-only feedback for multicast, the UE can re-use the PUCCH configuration for ACK/NACK based feedback for multicast.

(Intel) Proposal 10:

  - If high priority NACK-only feedback is configured, the UE expects to be configured with a separate PUCCH-ConfigurationList for NACK-only feedback.

(Intel) Proposal 12:

  - For PUCCH resource selection the following rules can be considered for overlap involving different cases:

    - If MBS and unicast have separate PUCCH-Configuration but with the same priority index and the PUCCH overlap in time, the PUCCH carrying the multiplexed HARQ-ACK feedback is from the PUCCH-Configuration of unicast for the corresponding priority.

    - If multicast ACK/NACK based feedback and NACK-only feedback have separate PUCCH-Configuration but with the same priority index and overlap in time for the PUCCH resources carrying the feedback, then the PUCCH resource carrying the multiplexed feedback is from the PUCCH-Configuration of the ACK/NACK based feedback.

    - If unicast ACK/NACK based feedback and multicast NACK-only feedback have separate PUCCH-Configuration but with the same priority index and overlap in time for the PUCCH resources carrying the feedback, then the PUCCH resource carrying the multiplexed feedback is from the PUCCH-Configuration of the ACK/NACK based feedback for unicast

(Intel) Proposal 13:

  - HARQ priority rules are also applicable to feedback for MBS SPS wherein SPS configuration of priority 0/1 maps to HARQ codebook priority 0/1 respectively.

(Apple) Proposal 1:

  - PUCCH resources for NACK-only are independently configured in UL CFR.

(Apple) Proposal 2:

  - PUCCH resource pool is defined for NACK-based feedback, UE select the PUCCH resource according to the subscribed MSB sessions.

(LGE) Proposal 6:

  - Only the first PUCCH resource set with PUCCH resource set ID = 0 in PUCCH-config for multicast can be configured for NACK only based HARQ-ACK.

(Lenovo) Proposal 7:

  - For PTM transmission scheme 1, from per UE perspective, PUCCH resource configuration for NACK-only based feedback can’t be shared with PUCCH resource configuration for HARQ-ACK feedback for unicast.

(Lenovo) Proposal 8:

  - For PTM transmission scheme 1, from per UE perspective, PUCCH resource configuration for NACK-only based feedback can’t be shared with PUCCH resource configuration for ACK/NACK based feedback for multicast.

(Lenovo) Proposal 9:

  - For PTM transmission scheme 1, from per UE perspective, PUCCH resource configuration for ACK/NACK based feedback can be shared with PUCCH resource configuration for HARQ-ACK feedback for unicast.

(Ericsson) Proposal 7:

  - PUCCH configuration for NACK-only contains a single PUCCH resource set

(Ericsson) Proposal 8:

  - The gNB has to ensure that it does not transmit more than r Nmax=log2(Z+1) transport blocks with dl-DataToUL-ACK pointing to the same uplink slot.

(Ericsson) Proposal 9:

  - The PRI in the DCI is interpreted by Ues in NACK-only mode to select one of the PUCCH resource bundles configurations for NACK-only.

(Ericsson) Proposal 10:

  - For PUCCH config for NACK-only, increase maxNrofPUCCH-ResourcesPerSet to 128.

(Ericsson) Proposal 11:

  - For a given UE, feedback for different G-RNTIs are TDM’ed in different UL slots.

(Ericsson) Proposal 12:

  - A UE configured with NACK only is always configured with a separate PUCCH configuration for NACK only if there is more than one NACK-only feedback bit.

- Power control for NACK-only

(Nokia) proposal 6:

  - A dedicated PUCCH-PowerControl parameter set, distinct from the PUCCH-PowerControl parameter set for UE-specific PUCCH resources, is used for configuration of power control of group-common PUCCH resources for NACK-only HARQ-ACK feedback, in case NACK-only PUCCH-config is not configured and unicast PUCCH-config is used to transmit NACK-only feedback.

(OPPO) Proposal 8:

  - For a UE receiving group-common PDSCH transmitted with PTM scheme 1 a TPC-PUCCH-RNTI different from that for unicast should be configured.

### Round-1 (closed)

FL’s Comments

Following the proposal for more than one NACK-only feedback are available for transmission in the same PUCCH slot as suggested for compromise, the PUCCH resource for NACK-only needs to be discussed and finalized hopefully because it affects RRC parameters.

Regarding the feedback mode between ACK/NACK based and NACK-only based, it was agreed in the last meeting that the RRC signaling configures it is either ACK/NACK based or NACK-only based for a single G-RNTI. If NACK-only is configured and separate PUCCH-Config/PUCCH-ConfigurationList for NACK-only is not configured, the PUCCH-Config/PUCCH-ConfigurationList for unicast applies:

Agreement:

For UE supporting both ACK/NACK based and NACK-only based feedback for multicast, for the same G-RNTI, support the following

- UE can be configured with either ACK/NACK based or NACK-only feedback for a single G-RNTI.

  - Note: Case1-1: if configured with ACK/NACK based feedback, UE can be optionally configured a separate PUCCH-Config/PUCCH-ConfigurationList for multicast. Otherwise, PUCCH-Config/PUCCH-ConfigurationList for unicast applies (This has been agreed.)

  - Case 1-2: if configured with NACK-only based feedback, when separate PUCCH-Config/PUCCH-ConfigurationList for NACK-only is not configured, PUCCH-Config/PUCCH-ConfigurationList for unicast applies.

Now the discussion is, when the separate PUCCH-Config/PUCCH-ConfigurationList for NACK-only is configured, how many PUCCH resource sets and how many PUCCH resources per PUCCH resource set, etc.

FL’s Proposal:

#### Proposal  3.3.1

For the separate PUCCH-Config/ PUCCH-ConfigurationList configured to UE for NACK-only based feedback,

  - up to 1 PUCCH resource set in each PUCCH-Config.

  - up to 16 PUCCH resources in each PUCCH resource set

  - Note: the separate PUCCH-Config/PUCCH-ConfigurationList applies to all configured G-RNTIs configured with NACK-only based feedback.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | - We suggest removing “up to” in the first sub-bullet as zero PUCCH resource set is not valid. - We suggest adding “[16]” to the second sub-bullet because the max number of resources is dependent on whether Alt 4 is agreed and detailed design of Alt 4. - For the note, it is not quite clear. Does it imply a single PUCCH-Config/PUCCH-ConfigurationList applies to all configured G-RNTIs configured with NACK-only based feedback? - FL’s responses: ok for the first two. Yes for (3) |
| - LG Electronics | - We are generally OK with this proposal. ‘up to’ can be removed in the first bullet point. |
| - CATT | - OK |
| - CMCC | - Agree with Lenovo, the number of [16] is also related to the decision of Alt1 or Alt 4. |
| - Nokia, NSB | We support the intention. However, in our view, 32 PUCCH resources, instead of 16 PUCCH resources should be configured (as in case of resource set 0 currently). Even if we agree on 16 PUCCH resources for multiple NACK-only in a slot, limitation to one NACK-only to 16 is not needed. For the separate PUCCH-Config/ PUCCH-ConfigurationList configured to UE for NACK-only based feedback, - up to 1 PUCCH resource set in each PUCCH-Config. - up to 1632 PUCCH resources in each PUCCH resource set - Note: the separate PUCCH-Config/PUCCH-ConfigurationList applies to all configured G-RNTIs configured with NACK-only based feedback. |
| - Vivo | We are generally OK with this proposal except the second bullet. In the current spec, up to 32 PUCCH resources can be configured in the first PUCCH resource set, same number can be configured here. |
| Xiaomi | OK |
| - NEC | - Agree with Lenovo. |
| - Samsung | - Support the proposal. |
| Ericsson | The number of PUCCH resources in one PUCCH resource set needs to be harmonized with P3.2.1 (about Alt1/Alt4). If the number in P3.2.1 increases from current 16 this Proposal must follow. |
| Qualcomm | ok |
| NTT DOCOMO | Support |
| Apple | Agree with Lenovo |
| ZTE | Similar view as Nokia, it should be up to 32 PUCCH resource in each set according to the existing PUCCH resource configuration for ACK/NACK-based feedback. |
| OPPO | Fine in principle, “16” should be put into brackets. |
| MediaTel | Agree with Lenovo |
| Spreadtrum | Generally fine |
| FL’s response | Based the above comments, updated as follows: Proposal 3.3.1-r1 For the separate PUCCH-Config/ PUCCH-ConfigurationList configured to UE for NACK-only based feedback, - 1 PUCCH resource set in each PUCCH-Config. - up to [16] PUCCH resources in each PUCCH resource set - Note: the separate PUCCH-Config/PUCCH-ConfigurationList applies to all configured G-RNTIs configured with NACK-only based feedback. |
| Samsung | OK |

### Round-2

#### Proposal  3.3.2

For the separate PUCCH-Config/ PUCCH-ConfigurationList configured to UE for NACK-only based feedback,

  - 1 PUCCH resource set in each PUCCH-Config.

  - up to [16] PUCCH resources in each PUCCH resource set

  - Note: the separate PUCCH-Config/PUCCH-ConfigurationList applies to all configured G-RNTIs configured with NACK-only based feedback.

Collect views:

| Company | Comments |
| --- | --- |
| - FL’s explanation | - Please check the discussion and FL’s responses in Round-1 to get why the proposal is updated as is. |
| - Samsung | - Support. - We don’t identify any connection/relevance between the number of resources for NACK-only and the maximum of 32 resources for 1-2 HARQ-ACK bits from Rel-15. |
| - Qualcomm | - Ok for progress |
| - LG Electronics | - We are generally OK. - For clarification, we wonder if this separate PUCCH-Config/ PUCCH-ConfigurationList is configured only for NACK-only based feedback or can be also configured for both NACK-only and ACK/NACK based feedback. |
| - ZTE | - We think 32 PUCCH resources in each PUCCH resource set should be supported, which is aligned with Rel-15. |
| - CMCC | - OK |
| - vivo | - Ok for progress |
| - OPPO | - OK |
| - Lenovo, Motorola Mobility | - OK to support 16. |
| - Xiaomi | - OK |
| - Spreadtrum | - Ok |
| - CATT | - OK |
| - NTT DOCOMO | - OK |
| - Nokia, NSB. | - OK |
| - Apple | - OK |
| - NEC | - OK |
| - Ericsson | - Not sure why [16] is mentioned as nrof resource to be in PUCCH resource set. Otherwise it is fine |
| - TD Tech, Chengdu TD Tech | - ok |

## Others

### PUCCH format and HARQ bits

Submitted Proposals

“format and HARQ bits”

(Huawei) Proposal 3:

  - Support only one HARQ-ACK bit for NACK-only feedback for PUCCH format 0 or format 1.

(Huawei) Proposal 4:

  - The sequence cyclic shift for NACK-only is .

(TD Teck) Proposal 1:

  - Support PUCCH format 2/3/4 for NACK-only based HARQ-ACK feedback.

(TD Teck) Proposal 2:

  - If PUCCH format 2/3/4 is used to feed back the NACK information for an MBS session, N is configured as a part of the configuration information of the MBS session. UE maps the NACK information of 1 bit long onto a sequence of N>2 bits long. Then send the NACK information with PUCCH format 2/3/4.

(OPPO) Proposal 5:

  - The PUCCH format used for NACK-only based HARQ-ACK feedback is semi-statically configured by RRC.

### NACK-only codebook

Submitted Proposals

(Ericsson) Proposal 1:

  - For NACK-only transmission of HARQ feedback for group scheduling, both semistatic and dynamic codebooks are supported

(Ericsson) Proposal 2:

  - The NACK-only codebook for a G-RNTI is constructed independently from codebooks constructed for other G-RNTIs that the UE may also receive and from the codebook that includes feedback for PDSCH scheduled by C-RNTI.

# HARQ-ACK feedback common

## DCI signaling to enable/disable HARQ-ACK

Submitted Proposals

“DCI signalling”

- (Nokia) Proposal 22:

  - For group-common DCI indicating whether HARQ-ACK feedback is enabled / disabled, reuse one existing field in the group-common DCI.

- (CATT) Proposal 1:

  - For DCI indicating the HARQ ACK enabling/disabling, Alt1 (Reuse one existing field e.g. Identifier for DCI formats, K1 and PRI fields in the group-common DCI) can be supported.

- (ETRI) Proposal 2:

  - For group-common DCI indicating whether ACK/NACK based HARQ-ACK feedback is enabled/disabled, Alt 2: introduce a new field in the group-common DCI is supported.

- (Intel) Proposal 4:

  - Reuse one existing field in the group-common DCI for HARQ/ACK feedback enabled/disabled indication

- (Xiaomi) Proposal 5:

  - For group-common DCI indicating whether ACK/NACK based HARQ-ACK feedback is enabled/disabled, introduce a new field in the group-common DCI (alt2).

- (CMCC) Proposal 8:

  - Regarding the first and second DCI format for GC-PDCCH,

    - ‘Identifier for DCI formats’ field is reused as ‘HARQ-ACK feedback enable/disable indicator’

- (NEC) Proposal 3:

  - Reuse the existing field in the group-common DCI to indicate whether ACK/NACK based HARQ-ACK feedback is enabled/disabled.

- (Samsung) Proposal 1:

  - DCI-based HARQ-ACK enabling/disabling is by a 1-bit field in first/second DCI formats for multicast.

- (Apple) Proposal 4:

  - Introduce a new field in the group-common DCI to enable/disable HARQ-ACK feedback.

- (DOCOMO) Proposal 1:

  - For DCI indication for enabling/disabling HARQ-ACK feedback, reuse the same mechanism as sidelink scheduling, i.e. DCI format 3_0.

    - If PRI is zero and PDSCH-to-HARQ feedback timing indicator is zero, HARQ-ACK feedback is disabled. Otherwise, HARQ-ACK feedback is enabled.

- (Lenovo) Proposal 2:

  - Support a new field in the group-common DCI for indicating HARQ-ACK feedback enabling/disabling.

- (Qualcomm) Proposal 6:

  - For GC-DCI indicating whether ACK/NACK based HARQ-ACK feedback is enabled/disabled,

    - Introduce a new field in the group-common DCI for a G-RNTI (Alt2).

    - Not supported for the group-common DCI for a G-CS-RNTI.

    - Only applied to Type-2 HARQ-ACK codebook.

- (MediaTek) Proposal 5:

  - Define a new field (e.g., “HARQ feedback enable/disable”) within MBS DCI format to indicate whether HARQ feedback is used for multicast services.

- (Ericsson) Proposal 14:

  - Only one bit is needed in the group-common DCI to indicate HARQ feedback enabling/disabling. This bit is a new field in the group-common DCI.

- (Ericsson) Proposal 16:

  - When group-common DCI is used to indicate enabling/disabling between ACK/NACK based HARQ feedback and NACK-only HARQ feedback, UE is configured with both ACK/NACK based and NACK-only feedback for a single G-RNTI.

### Round-1 (continue)

FL’s Comments

Regarding the DCI signalling for indicating whether ACK/NACK based HARQ-ACK feedback is enabled/disabled, the following was agreed in the last meeting:

Agreement:

For group-common DCI indicating whether ACK/NACK based HARQ-ACK feedback is enabled/disabled, down-select from the following alternatives:

- Alt1: Reuse one existing field in the group-common DCI.

- Alt2: Introduce a new field in the group-common DCI.

From the contributions:

Alt1 is supported by (6): Nokia, CATT, Intel, CATT, NEC, DOCOMO

- Identifier for DCI formats, K1 and PRI fields, (CATT)

- ‘Identifier for DCI formats’ field for both first and second DCI format, (CMCC)

- If PRI is zero and PDSCH-to-HARQ feedback timing indicator is zero, HARQ-ACK feedback is disabled. Otherwise, HARQ-ACK feedback is enabled. (DOCOMO)

Alt2 is supported by (8): ETRI, Xiaomi, Samsung, Apple, Lenovo, Qualcomm, MediaTek, Ericsson

- 1-bit field in first/second DCI formats (Samsung)

The discussion includes two issues: which DCI format(s) and which field (existing or a newly added field) in the DCI format is used to indicate enabled/disabled HARQ-ACK.

In the current 38.212 draft CR, there are only 3 reserved bits in DCI format 1_0 scrambled by G-RNTI/G-CS-RNTI for scheduling multicast and “enabling/disabling HARQ-ACK feedback indication” is included in DCI format 1_1 scrambled by G-RNTI/G-CS-RNTI for scheduling multicast. Since whether DCI indicates the HARQ-ACK enabled/disabled is configured by UE RRC signaling and the number of reserved bits is limited in DCI format 1_0 scrambled by G-RNTI/G-CS-RNTI. Hence, FL suggests the “enabling/disabling HARQ-ACK feedback indication” is only included in DCI format 1_1 scrambled by G-RNTI/G-CS-RNTI for discussion.

For the second issue whether reuse an existing field (supported by 6 companies) and add a new field (supported by 8 companies), there is no substantial majority but a new field seems simpler.

FL’s Proposal:

#### (GTW)Proposal 4.1.1-1

For group-common DCI indicating whether ACK/NACK based HARQ-ACK feedback is enabled/disabled, the “enabling/disabling HARQ-ACK feedback indication” is only included in DCI format 1_1 scrambled by G-RNTI

- For DCI format 1_1 scrambled by G-CS-RNTI, it is discussed separately.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | We are generally OK with it. But we think such indicator is also included in DCI format 1-0. |
| CATT | According to the discussion of A1 8.12.1, the Identifier for DCI formats and the TPC command for scheduled PUCCH fields are not needed for first DCI format. So we believe that the first DCI format can be also used for indicating enabling/disabling with sufficient bits. |
| CMCC | Also support to include this filed in the first DCI format. |
| Nokia, NSB | Does this proposal intend to use the reserved bits in DCI 1_1 for enabling/disabling HARQ-ACK feedback indication? If that is the case, considering that there are 3 reserved bits in DCI 1_0, why do we limit the proposal to 1_1 only? Does the FL believe 3 reserved fields will serve another purpose? - For DCI format 1_1, there is not reserved bits because the field and and size of field are all configurable. |
| Vivo | ok |
| Xiaomi | Similar views as Lenovo. There are at least 3 reserved bits in the DCI format 1_0 as addressed in the draft CR. HARQ-ACK enabling/disabling only needs 1 bit. There is still enough room for DCI format 1_0 to carry enabling/disabling indication. |
| NEC | Fine with the proposal. |
| Samsung | Support (especially given past agreement to not have configurable fields/priority indicator in DCI format 1_0) |
| Ericsson | Support |
| Qualcomm | Ok |
| NTT DOCOMO | We agree with Lenovo. The indicator can be included in DCI format 1_0. |
| Apple | Ok with this proposal |
| ZTE | From our perspective, we think “enabling/disabling HARQ-ACK feedback indication” can also be supported in DCI format 1_0 since the indication applies to each PDSCH, without this field in DCI format 1_0, all PDSCHs scheduled by DCI format 1_0 has to follow the semi-static RRC configuration, which lose the flexibility of dynamic indication. |
| OPPO | OK |
| MeidaTek | Support to include the “enabling/disabling HARQ-ACK feedback indication” field in the first DCI format. |
| FL’s response | Earlier when we were discussing whether format 1_0/G-RNTI can include “priority index” and it was precluded because companies keep arguing it is more like fallback DCI format and at least the field size should be kept the same as legacy. In light of this, I would suggest we should be consistent and agree on this is only included in DCI format 1_1, especially it is very controversial. Half ok and half also ok to include in DCI 1_0. |
| Lenovo 2 | We think both DCI format 1-0 and DCI format 1-1 should support HARQ-ACK enabling/disabling function. If companies have concern on introducing one bit in the DCI, we suggest using the non-numerical K1 value for indicating enabling/disabling HARQ-ACK feedback. In that sense, there is no extra bit overhead and this enabling/disabling HARQ-ACK feedback has been specified in Rel-16 in NR-U WI. |
| Samsung2 | OK to limit to DCI format 1_1 – it is not a critical feature. If it is to be supported also for DCI format 1_0, it cannot be a configurable field and it cannot be a non-numerical K1 as UEs not supporting the feature will not interpret K1 as being non-numerical. |
| Nokia, NSB2 | Ok. We should not reserve any K1 value for that reason, which reduces scheduling flexibility. |
| ETRI | OK with FL’s suggestion. |
| Lenovo3 | Using NNK1 value for enabling/disabling is already specified in Rel-16. Why not follow legacy behavior here? Using one value of 8 values does not impact scheduling flexibility. |
| Qualcomm | Ok to limit it only in DCI format 1_1. |

#### (GTW)Proposal 4.1.1-2

For the DCI format including the field of “enabling/disabling HARQ-ACK feedback indication” for multicast scheduling, the field is a new field with 1 bit.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | OK. |
| - LG Electronics | - We prefer to use the existing field such as K1 or PRI. |
| - CATT | Not OK. - We believe that using an existing field to indicate the HARQ-ACK feedback enabling/disabling is quite straightforward. Since the Identifier for DCI formats field is not needed for both first and second DCI formats, so there will be a coincident design for both DCI formats for multicast. Moreover, different Ues in a MBS group may have different RRC states of HARQ-ACK enabling/disabling. Thus, whether the new field is needed in DCI for Ues for all RRC state configurations should be further discussed. Thus, per our understanding, introducing a new field will bring extra work and high complexity on protocol design. |
| - CMCC | Agree with CATT. |
| - Nokia, NSB | As we have asked above, we call it a new field but we would like to use one of the reserved fields, right? - As responded for the last proposal, it is included in DCI format 1_1/G-RNTI, no reason to reuse “reserved bits” because there is no reserved bits at all because all the fields are configurable as in current agreed draft 212 CR. |
| - Xiaomi | Support. |
| - NEC | We share the same view with CATT. |
| - Samsung | Support. Clean solutions and avoiding mixing functionalities of fields is always preferable especially when the cost is 1 bit (or actually less than 1 bit since, with implicit indication, the information allowed from another field is reduced and there may be some scheduler restrictions). |
| Ericsson | Support |
| Qualcomm | ok |
| NTT DOCOMO | Not support. The current specification already has mechanisms to disable HARQ-ACK feedback using the existing DCI fields (e.g., non-numerical k1 value). We don’t see the need to introduce a new DCI field. |
| Intel | Not sure what is meant by “new field”. We should reuse some of the unused existing fields of which there are quite a few for e.g., Identifier for DCI formats. - If it is in DCI format 1_1, now all fields are configurable, there is no existing field not needed at all but kept used for this purpose. |
| Apple | Support |
| ZTE | OK, we are also fine to use the existing field such as K1 or PRI. |
| OPPO | OK |
| MediaTek | Support. |
| Spreadtrum | We can wait for AI8.12.1 where the content of DCI format is being discussed. - Not sure if it is good because the discussing here is related with HARQ especially considering reusing existing bit (PRI,K1) is one candidate alternative. |
| FL’s explanation | I agree with some companies’ comments that using k1 by e.g., non-numerical k1 value to disable HARQ-ACK as supported already in spec. However, when we were discussing GC-DCI indicating this HARQ-ACK enabled/disabled, the issue of C/T-DAI misalignment was brought up. I am not confident whether/how the codebook or C/T-DAI issue will be affected if enabling using k1 for disabling purpose because it will end up some UE HARQ-ACK disabled but other enabled. It is very complicated surely. Hence, I still tend to suggest simplifying the design by “adding a new field” instead of using existing bit. |
| Lenovo 2 | We think both DCI format 1-0 and DCI format 1-1 should support HARQ-ACK enabling/disabling function. If companies have concern on introducing one bit in the DCI, we agree with CATT/DOCOMO using the non-numerical K1 value for indicating enabling/disabling HARQ-ACK feedback. In that sense, there is no extra bit overhead and this enabling/disabling HARQ-ACK feedback has been specified in Rel-16 in NR-U WI. |
| Samsung2 | OK to limit disabling to DCI format 1_1. |
| Nokia, NSB2 | Ok with current proposal. |
| ETRI | OK with FL’s suggestion. |
| MediaTek | Support the FL’s suggestion. |
| Qualcomm | support |

## (CLOSED)enable/disable HARQ-ACK for NACK-only

“Applicability to NACK-only”

- (Huawei) Proposal 7:

  - Support enabling/disabling NACK-only based HARQ-ACK feedback by replacing ACK/NACK with NACK-only as in the following previous agreement:

For enabling/disabling ACK/NACK-based HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast via dynamic group-common PDSCH: - RRC signaling configures the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback. - If RRC signaling configures the function of group-common DCI based indication, group-common DCI indicates (explicitly or implicitly) whether ACK/NACK based HARQ-ACK feedback is enabled/disabled - Otherwise, enabling/disabling ACK/NACK based HARQ-ACK feedback is configured by RRC signaling. - FFS details on RRC signaling and group-common DCI indicating.

- (FUTUREWEI) Proposal 1:

  - Extend the agreement of the Working Assumption on the enabling/disabling ACK/NACK-based HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast via dynamic group-common PDSCH to include the following::

    - This WA is extended to apply to NACK-only based feedback.

    - This WA is extended to the enabling/disabling ACK/NACK based HARQ-ACK feedback and NACK-only retransmission.

    - The default UE behavior, prior to receiving of RRC signaling, is that either ACK/NACK or NACK-only feedback is enabled.

- (vivo) Proposal 1:

  - Not support DCI indication function of enabling/disabling NACK-only based HARQ-ACK feedback.

- (Nokia) Proposal 17:

  - Confirmed WA is enhanced to also include NACK-only feedback, for which the same HARQ-ACK feedback enabling / disabling mechanisms to the ones agreed for ACK / NACK feedback apply.

- (Nokia) Proposal 18:

  - The group-common DCI indicating the enabling / disabling NACK-only based HARQ-ACK feedback is configured per G-RNTI by UE RRC signalling.

(Nokia) Proposal 19:

  - If the group-common DCI indicating the enabling / disabling NACK-only based HARQ-ACK feedback is not configured, enabling / disabling NACK-only based HARQ-ACK feedback is configured per G-RNTI by UE RRC signalling.

- (CATT) Proposal 2:

  - The enabling/disabling ACK/NACK-based HARQ-ACK feedback mechanism can be used for NACK-only based HARQ-ACK feedback.

- (ETRI) Proposal 1:

  - Support the functionality of group-common DCI based indication for enabling/disabling HARQ-ACK feedback for NACK-only based HARQ-ACK feedback.

- (Intel) Proposal 3:

  - For NACK-only feedback RRC configures ON/OFF with no DCI based indication.

- (DOCOMO) Proposal 2:

  - RRC configuration and DCI indication for enabling/disabling HARQ-ACK feedback for multicast also applies to NACK-only based feedback for multicast.

- (Lenovo) Proposal 4:

  - Using new field for indicating HARQ-ACK feedback option as well as enabling/disabling can be straightforwardly extended to NACK-only based feedback and multiple G-RNTI cases.

- (MediaTek) Proposal 2:

  - The ACK/NACK-based enabling/disabling HARQ-ACK feedback mechanism can be extended to apply to NACK-only based feedback enabling/disabling.

- (Ericsson) Proposal 13:

  - This HARQ enable/disable feature is extended to apply to NACK-only HARQ feedback including both DCI based and RRC based solution.  This means UE can be configured to switch between ACK/NACK HARQ feedback (enable) and NACK-only feedback (disable) in addition to ACK/NACK HARQ feedback and no HARQ feedback.

### Round-1

FL’s Comments

The enabling/disabling HARQ-ACK mechanism has been focusing on ACK/NACK-based feedback, whether the relevant agreements made earlier can be extended to support NACK-only based feedback is the issue to be discussed in this section.

Based on the submitted proposals: for enabling/disabling HARQ-ACK for NACK-only based feedback

- Support: Huawei, Futurewei, Nokia, CATT, ETRI, DOCOMO, Lenovo, MediaTek, Ericsson

  - The group-common DCI indicating the enabling / disabling NACK-only based HARQ-ACK feedback is configured per G-RNTI by UE RRC signalling. (Nokia)

  - If the group-common DCI indicating the enabling / disabling NACK-only based HARQ-ACK feedback is not configured, enabling / disabling NACK-only based HARQ-ACK feedback is configured per G-RNTI by UE RRC signalling. (Nokia)

- Not support: vivo, Intel

From the contributions, the concern from vivo is that DCI indicating enabling/disabling ACK/NACK is to combat the PUCCH resources shortage which should not be the case for NACK-only because all Ues can share the same resources for NACK-only. The view from Intel is that RRC based configuration of HARQ ON/OFF would suffice and complicated DCI based indication is not necessary.

Considering more than one NACK-only may need a list of PUCCH resources though they could be shared and whether PUCCH esource is in shortage or not may not only count that used for NACK-only nor even MBS. In addition, DCI is anyway to be designed (in fact it has been implemented in the draft CR) for enabling/disabling HARQ-ACK for ACK/NACK-based feedback. Hopefully, the majority view can be acceptable to all.

FL’s Proposal:

#### (EMAIL)Proposal 4.2.1

Support enabling/disabling HARQ-ACK for NACK-only based feedback.

    - The relevant agreements made for ACK/NACK based feedback can be extended for the support of NACK-only, including:

  - RRC signalling configures the presence of the field “enabling/disabling HARQ-ACK feedback indication” in the group-common DCI and the configuration is per G-RNTI.

  - RRC signalling configures directly whether the HARQ-ACK feedback is enabled or disabled and the configuration is per G-RNTI.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | OK. |
| CATT | OK with the proposal. |
| CMCC | Ok |
| Nokia, NSB | Ok. |
| Vivo | We think the proposal is over optimization. There is no need to enable/disable NACK only via DCI. |
| Xiaomi | We are fine with the proposal. Furthermore, we would like to make a clarification: there is no intention to introduce separate enabling/disabling indication in a DCI for ACK/NACK-based feedback and NACK-only feedback respectively, right? - Not separate, because the DCI formats are the same for ACK/NACK and NACK-only, so the bit is the same. |
| NEC | Support |
| Samsung | Support One example reason for disabling “NACK-only” is the case of using “Alt.1” (discussed before, if agreed) due to the resulting number of bits – then, there is no “NACK-only”, it is converted to “ACK/NACK”. |
| Futurewei | Support |
| NTT DOCOMO | Support |
| Ericsson | We find the current proposal unclear. When considering switching between two HARQ-ACK feedback modes there are three logical transitions but only two of these are of interest: - Switching between ACK/NACK and no feedback - Switching between ACK/NACK and NACK-only The third option, to switch between NACK-only and no feedback, has no practical interest, so does not need to be supported. - Other companies see there is a need to do so. The switching between ACK/NACK and no feedback is already agreed, so this Proposal should target switching between ACK/NACK and NACK-only. - Switching between ACK/NACK and NACK-only by RRC configuration has been agreed when competing against dynamic switching by DCI. We agree that the configuration should be per G-RNTI. We agree that RRC should configure the DCI functionality, including which HARQ feedback modes that are affected by the switch. Since ACK/NACK is always involved in the mechanism, RRC only needs to configure the UE – with one bit – whether the “disable” means “no feedback” or “NACK-only” feedback. The word pair “enable/disable” refers to ACK/NACK feedback, which is enabled or disabled. Which is the other state, is RRC configured. |
| Qualcomm | ok |
| Intel | We think only RRC based configuration is enough. No need to have DCI based configuration of NACK-only ON/OFF |
| Apple | ok |
| ZTE | We are ok with the proposal. The same bit can be used for ACK/NACK based feedback or NACK-only based feedback. |
| OPPP | OK |
| MediaTek | Support |
| Spreadtrum | Support |
| FL’s response | Based on the comments, the reasonable way forward seems to accept this proposal especially considering the DCI signaling design need to be in place which can be easily extended to NACK-only. Oppositly, additional effort is needed to preclude the “enabling/disalbing” indication to be used for NACK-only. |
| Samsung | OK |
| Nokia, NSB2 | Support. We need the agreement to complete functionality of NACK-only feedback. |
| ETRI | Support. |

## enable/disable HARQ-ACK for multicast SPS

“Applicability to SPS”

(vivo) Proposal 2:

  - For MBS for RRC_CONNECTED UEs, HARQ-ACK feedback options including disabled feedback, ACK/NACK and NACK-only for SPS group-common PDSCH is configured per SPS PDSCH index.

- (Spreadtrum) Proposal 3:

  - For SPS group-common PDSCH without PDCCH scheduling, support activation PDCCH to enable/disable HARQ-ACK feedback based on UE’s capability.

(Spreadtrum) Proposal 4:

  - For multicast SPS activation/deactivation PDCCH, support ACK/NACK based feedback, and the ACK/NACK based feedback can be enabled/disabled.

- (CATT) Proposal 4:

  - For SPS group-common PDSCH, the ACK/NACK based feedback of SPS activation should be always enabled and the ACK/NACK based feedback of SPS deactivation can be enabled/disabled.

- (CATT) Proposal 5:

  - If HARQ-ACK enabling/disabling is determined by DCI, the HARQ-ACK feedback of the PDSCH transmission without DCI (Case3) can be based on the DCI indication of the activation command.

(DOCOMO) Proposal 8:

  - Introduce a higher layer parameter to control feedback for SPS GC-PDSCH activation/deactivation

    - If configured, UE performs ACK/NACK-based feedback for activation/deactivation of SPS GC-PDSCH regardless of configuration/indication of feedback type and configuration/indication of enabled/disabled feedback

    - Otherwise, UE follows the configuration/indication

(Lenovo) Proposal 14:

  - For group-common SPS configuration, ACK/NACK feedback for confirming reception of activation/deactivation DCI can’t be disabled.

- (Qualcomm) Proposal 6:

  - For GC-DCI indicating whether ACK/NACK based HARQ-ACK feedback is enabled/disabled,

    - Introduce a new field in the group-common DCI for a G-RNTI (Alt2).

    - Not supported for the group-common DCI for a G-CS-RNTI.

    - Only applied to Type-2 HARQ-ACK codebook.

- (Qualcomm) Proposal 7:

  - For SPS GC-PDSCH without PDCCH scheduling, the enabling/disabling ACK/NACK based HARQ-ACK feedback can be configured per G-CS-RNTI by UE RRC signalling.

(Ericsson) Proposal 21:

  - For a UE configured to transmit multicast via SPS, configuration of the HARQ feedback option is done separately for each G-CS-RNTI of the SPS-configuration.

    - ACK/NACK feedback is always transmitted regardless of the configured HARQ-ACK feedback option for a given SPS config when the UE responds to SPS activation or release, i.e. ACK-NACK feedback is always used to follow a PDCCH scrambled with G-CS-RNTI.

    - If the UE is configured to monitor the DCI indicator for HARQ feedback options, for the SPS PDSCH which is associated with PDCCH activation and PDCCH deactivation, ACK/NACK feedback is always transmitted regardless of the indicator in this group-common PDCCH, i.e. ACK-NACK feedback is always used to follow a PDCCH scrambled with G-CS-RNTI. For SPS PDSCH without PDCCH, how to send HARQ feedback is according to the indicator in the group-common activation PDCCH.

### Round-1 (closed)

FL’s Comments

The FFS whether/how the HARQ-ACK for SPS is enabled/disabled is discussed in this section. The discussion for SPS activation/deactivation can be separate from that for multicast SPS PDSCH without PDCCH scheduling because the motivation for the support/not support could be potentially different.

For multicast SPS PDSCH without PDCCH scheduling,

- Alt1: the HARQ-ACK can be disabled by RRC configuration per SPS-Config-Multicast (vivo, Qualcomm)

- Alt2: the DCI for SPS activation can be configured to indicate the HARQ-ACK enabled/disabled. (spreadtrum, CATT, Ericsson)

For multicast SPS activation/deactivation,

- Alt1: the HARQ-ACK can be enabled/disabled. (spreadtrum)

- Alt2: activation should be always enabled. Deactivation can be enabled/disabled. (CATT)

- Alt3: specific parameter to control whether performs ACK/NACK for activation/deactivation (DOCOMO)

- Alt4: not support disabling HARQ-ACK. (Lenovo, Qualcomm, Ericsson)

Since the discussion of enabling/disabling functionality for dynamical scheduling is getting clearer and clearer and DCI format 1_1 including the field of “enabling/disabling HARQ-ACK feedback indication” can be applicable to CRC  scrambled by both G-RNTI and G-CS-RNTI which has been reflected in current 38.212 draft CR already, extending the enabling/disabling functionality for multicast SPS does not seem to require substantially additional efforts.

For multicast SPS activation/deactivation, there was concern to disable the HARQ-ACK because it may cause unnecessary retransmissions since network is not able to differentiate whether UE decodes the command correctly similar to the argument for only support ACK/NACK based for multicast SPS activation/deactivation. However, if network desires to know whether UE decodes the command correctly, the “enabling/disabling HARQ-ACK feedback indication” can be set “on”, so it is up to gNB anyhow.

Given there is slightly majority to not support disabling HARQ-ACK for SPS activation/deactivation and support DCI for SPS activation configured to indicate the HARQ-ACK enabled/disabled for multicast SPS PDSCH without PDCCH scheduling. We can try this direction for initial discussion.

FL’s Proposal:

#### Proposal 4.3.1-1

For multicast SPS PDSCH without PDCCH scheduling, support the following:

- RRC signalling configures the presence of the field “enabling/disabling HARQ-ACK feedback indication” in the group-common DCI for multicast SPS activation.

  - The configuration is per SPS configuration index.

- RRC signalling configures directly whether the HARQ-ACK feedback is enabled or disabled.

  - The configuration is per SPS configuration index.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | OK |
| CATT | We are confuse with the first-bullet, seems that it only related with the SPS activation. How about the SPS deactivation? - This proposal is talking about PDSCH without PDCCH, so the field in DCI is the DCI for activation. |
| CMCC | Ok to take the similar way as dynamic scheduling. |
| Nokia, NSB | Ok. |
| Vivo | Ok with the second bullet. For the first bullet, we think the DCI for SPS activation is the same as that for dynamic scheduling. - They can be separately discussed. Let’s take one step by one step. |
| Xiaomi | We are not OK with the first subbullet, i.e. the configuration is per SPS configuration index. It will make the DCI size different across different SPS configuration. A unified DCI payload size for different SPS configuration should be guaranteed. - Seems valid. I will put them FFS. |
| NEC | Support |
| Samsung | OK with the proposal. We understand the proposal to be that the same mechanism as for DCI-based PDSCH applies for SPS PDSCH with the only additional component being that RRC enabling/disabling is per SPS configuration index. |
| NTT DOCOMO | Support |
| Ericsson | Support |
| Qualcomm | We don’t see the motivation to have dynamic enabling/disabling in GC-DCI for SPS multicast. For the RRC-configured enabling/disabling of SPS multicast feedback, we prefer to configure it per G-CS-RNTI, similar as that of DG multicast configured per G-RNTI. |
| ZTE | We support this proposal since anyway the bit will be included in the DCI activating SPS. It doesn’t increase any additional overhead in the DCI. |
| OPPO | Support |
| MediaTek | Support |
| FL’s response | Based on the comments, updated as follows: Proposal 4.3.1-1-r1 For multicast SPS PDSCH without PDCCH scheduling, support the following: - RRC signalling configures the presence of the field “enabling/disabling HARQ-ACK feedback indication” in the group-common DCI for multicast SPS activation. - FFS: The configuration is per SPS configuration index. - RRC signalling configures directly whether the HARQ-ACK feedback is enabled or disabled. - FFS: The configuration is per SPS configuration index. |

#### Proposal 4.3.1-2

For multicast SPS activation/activation, disabling the HARQ-ACK feedback is not supported.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | OK |
| - LG Electronics | - We prefer to support disabling the HARQ-ACK feedback for PUCCH overhead reduction. Reliability of activation/deactivation DCI can be supported by blindly re-transmitting the DCI. |
| - CATT | Not OK with it. - Regarding SPS deactivation, since the scheduling information is not that important and will not affect the subsequent data reception. Thus, the ACK/NACK based feedback of SPS deactivation can be enabled or disabled by different RRC states for saving resource. |
| - CMCC | Ok |
| - Nokia, NSB | Ok but there is a typo: For multicast SPS activation/deactivation, disabling the HARQ-ACK feedback is not supported |
| - vivo | ok |
| - NEC | We not support this proposal and share the same views with CATT and LG. |
| - Samsung | Do not support. Agree with LG. |
| - NTT DOCOMO | Not support. We agree with LG. |
| Ericsson | Support with the understanding that HARQ-ACK includes both ACK/NACK and NACK-only. Disabling HARQ-ACK feedback therefore means that no feedback is sent. NACK-only can still be considered separately. |
| Qualcomm | Support (with typo corrected by Nokia). The acknowledgement of SPS activation/deactivation is essential for gNB to manage the SPS operation. |
| Apple | OK with this proposal |
| ZTE | We are ok with the intention of this proposal. If both enabling and disabling the HARQ-ACK feedback are supported for SPS activation, basically it means the same option will be used for both SPS PDSCH with PDCCH and SPS PDSCH without PDCCH since they share the same field in the PDCCH. This kind of restriction is unreasonable from our perspective. Coming back to this proposal, we understand the intention is say that HARQ-ACK feedback is always needed regardless of the RRC configuration or the dynamic indication in the PDCCH. Thus, we would propose to make it clear as following. For multicast SPS activation/activation, HARQ-ACK feedback is always supported regardless of the RRC signalling configures the presence of the field “enabling/disabling HARQ-ACK feedback indication” and the corresponding DCI field indication. |
| OPPO | Support |
| MediaTek | Ok with the proposal |
| Speadtrum | Not support. Agree with LG. |
| FL’s response | I will take ZTE’s suggestion with some update for next round discussion. I know some companies prefer the other option. But we have to compromise somehow. Let’s see if companies have strong conern about the updated proposal in the round-2. |

### Round-2

#### Proposal 4.3.2-1 (converging)

For multicast SPS PDSCH without PDCCH scheduling, support the following:

- RRC signalling configures the presence of the field “enabling/disabling HARQ-ACK feedback indication” in the group-common DCI for multicast SPS activation.

  - FFS: The configuration is per SPS configuration index.

- RRC signalling configures directly whether the HARQ-ACK feedback is enabled or disabled.

  - FFS: The configuration is per SPS configuration index.

Collect views:

| Company | Comments |
| --- | --- |
| - FL’s explanation | - Please check the discussion and FL’s responses in Round-1 to get why the proposal is updated as is. |
| - Samsung | - Isn’t the second bullet sufficient? What is the relevance of the first bullet for SPS PDSCH without PDCCH? The DCI format would anyway be same as that used for DG-PDSCH. |
| - Qualcomm | - We prefer only support second subbullet and the configuration is per G-CS-RNTI. - The dynamic enabling/disabling function in GC-DCI is configured per G-RNTI, and no need to be configured for G-CS-RNTI(s). |
| - ZTE | - We are ok with this proposal. |
| - CMCC | - Ok |
| - vivo | - Support the second bullet. For the first bullet, same view as Samsung, the DCI format for SPS activation is the same as that for DG PDSCH. |
| - OPPO | - Share the view that second bullet only is sufficient. |
| - Lenovo, Motorola Mobility | - OK |
| - Xiaomi | - OK |
| - Spreadtrum | - Ok |
| - CATT | - OK with this proposal. - Per our understanding, the first sub-bullet is related with DCI indicating. And it is natural that the HARQ-ACK feedback can be based on the DCI indication of the active command when DCI indication is configured. We are confused how the second bullet sufficient is? |
| - NTT DOCOMO | - OK |
| - Nokia, NSB. | - OK |
| - NEC | - OK |
| - Ericsson | - OK |
| - FL’s response: | - @Samsung, Qualcomm, vivo, and CATT - There are some comments (Samsung, Qualcomm, vivo) to delete the first sub-bullet for which one company (CATT) prefer to keep. - The question is whether the indication of “enabling/disabling HARQ-ACK” in DCI format for dynamic scheduling is assumed to be there as well for SPS scheduling. The issue is open although all companies may have the same understanding that the same format is used for both dynamic (scrambled by G-RNTI) and SPS (scrambled by G-CS-RNTI). |
| - Samsung | - Do not understand the open issue mentioned by the FL as the DCI format size is not different based on G-RNTI or C-GC-RNTI – the field either exists or it doesn’t. - Also, there is no HARQ-ACK for SPS activation DCI – the HARQ-ACK is associated with the SPS PDSCH, not with the activation DCI. In the PUCCH slot indicated by k1, the UE transmits 1 HARQ-ACK bit, not 2, and the HARQ-ACK bit is for the TB (with PUCCH DTX indicating missed DCI). That is why the first sub-bullet is unnecessary and problematic. - That is also one of the reasons we do not support the following proposal (4.3.2-2-r1) – there is no such thing as HARQ-ACK for SPS activation (there is for deactivation). - “For multicast SPS activation/deactivation, HARQ-ACK feedback is always enabled regardless whether PDSCH without PDCCH is configured/indicated with HARQ-ACK disabled.” |
| - CATT | - We believe that the dynamic enabling/disabling function in GC-DCI should be configured for G-CS-RNTI(s) same as G-RNTIs. Thus, the DCI indicating is needed. And it is naturally that the RRC signalling should configure the presence of the field “enabling/disabling HARQ-ACK feedback indication” in the group-common DCI for multicast SPS activation. - Further clarification is needed for the question that FL mentioned. |
| - Qualcomm | - We don’t think dynamic enabling/disabling is needed for SPS multicast. If support the first subbullet, a new UE capability of dynamic enabling/disabling configured for G-CS-RNTI(s) is needed, separate from that of dynamic enabling/disabling configured for G-RNTI(s). - We prefer FFS the first subbullet. #### Proposal 4.3.2-1 (converging) For multicast SPS PDSCH without PDCCH scheduling, support the following: - FFS: RRC signalling configures the presence of the field “enabling/disabling HARQ-ACK feedback indication” in the group-common DCI for multicast SPS activation. - FFS: The configuration is per SPS configuration index. - RRC signalling configures directly whether the HARQ-ACK feedback is enabled or disabled. - FFS: The configuration is per SPS configuration index. |

#### (GTW)Proposal 4.3.2-2

For multicast SPS activation/deactivation, HARQ-ACK feedback is always supported regardless whether PDSCH without PDCCH is configured/indicated with HARQ-ACK disabled.

Collect views:

| Company | Comments |
| --- | --- |
| - FL’s explanation | - Please check the discussion and FL’s responses in Round-1 to get why the proposal is updated as is. |
| - Samsung | - Not support. - No actual need exists to justify introduction of new UE behavior. If a UE provides HARQ-ACK for SPS PDSCH, that is enough. If a UE does not provide HARQ-ACK for SPS PDSCH, that is a bigger challenge for a NW as it is much more difficult to ensure (almost) error-free reception for all TBs than for the (much smaller) one DCI – also, the activation DCI can have blind retransmissions – trivial to do and without material penalty as SPS activation is not a frequent event. |
| - Qualcomm | - ok |
| - LG Electronics | - We agree with Samsung |
| - ZTE | - We are ok with this proposal. |
| - CMCC | - Ok |
| - Lenovo, Motorola Mobility | - OK |
| - Spreadtrum | - Agree with Samsung |
| - CATT | - It is more clear for us with the following modification: - For multicast SPS activation/deactivation, HARQ-ACK feedback is always supported enabled regardless whether PDSCH without PDCCH is configured/indicated with HARQ-ACK disabled. |
| - Nokia, NSB. | - OK |
| - Apple | - OK |
| - Ericsson | - OK |
| - NTT DOCOMO | - We agree with Samsung |
| - TD Tech, Chengdu TD Tech | - ok |
| - Lenovo, Motorola Mobility | - Agree with CATT’s update. |
| - MediaTek | - Ok with the current proposal. CATT’s version is also fine for us. |

## Codebooks for disabled HARQ-ACK

Submitted Proposals

“impact to codebooks”

- (Intel) Proposal 6:

  - For Type 1 Codebook if HARQ is disabled by DCI for a GC-PDSCH, Type 1 codebook generation does not change, and padding NACK can be used such that CB size is same as in the case if HARQ-ACK feedback was enabled.

- (Intel) Proposal 7:

  - For Type 2 Codebook if HARQ is disabled by DCI for a GC-PDSCH, the C-DAI can for the GC-PDSCH can be ignored but the T-DAI is still indicated.

- (Qualcomm) Proposal 6:

  - For GC-DCI indicating whether ACK/NACK based HARQ-ACK feedback is enabled/disabled,

    - Introduce a new field in the group-common DCI for a G-RNTI (Alt2).

    - Not supported for the group-common DCI for a G-CS-RNTI.

    - Only applied to Type-2 HARQ-ACK codebook.

- (Ericsson) Proposal 17:

  - The UE is only expected to follow the enable / disable indicator for HARQ feedback in group-common DCI when the two following conditions are fulfilled:

    - If it is configured to use the enable/disable indicator present in group common PDCCH

    - If the HARQ feedback codebook type is of type 2.

“same enabled/disabled status for the same PUCCH slot”

- (CMCC) Proposal 7:

  - The HARQ-ACK of PDSCH that feedbacks in the same PUCCH should be enabled/disabled simultaneously.

- (MediaTek) Proposal 3:

  - The same HARQ-ACK feedback behaviour for all the UEs in the same MBS group are required.

- (Ericsson) Proposal 18:

  - The enable/disable indicator in the PDCCHs scrambled by same G-RNTI which point to the same PUCCH for HARQ feedback must be the same.

### Round-1 (continue)

FL’s Comments

On FFS pertaining to the agreement of enabling/disabling HARQ-ACK is the relation to the HARQ-ACK codebook types and HARQ-ACK codebook construction.

It has been agreed that the codebook type configured for multicast is applied to all G-RNTIs. For a given G-RNTI, the codebook type configured is the same for all Ues within the group, regardless whether the HARQ-ACK enabled/disabled is configured by RRC signaling or is indicated in the group-common DCI. As long as the HARQ-ACK feedback is disabled anyhow, type-1 codebook type should not be configured for multicast, which seems too restrictive.

Since there is only a couple of proposals for solving this issue, I would take the proposal from Intel as a starting point for initial discussion. This issue has to be clarified/solved as editor also pointed out in 38.213 draft CR.

When discussing the codebook generation for “disabled HARQ” and especially it is Type-2 codebook, at least CMCC and Ericsson proposed the HARQ-ACK feedback of all PDSCHs that to be transmitted in the same PUCCH should be enabled/disabled simultaneously, so that codebook misalignment between UE and gNB can be avoided assuming UE will feedback nothing if the HARQ-ACK is disabled.

FL’s Proposal:

#### Proposal 4.4.1-1

For group-common DCI indicating HARQ-ACK enabled/disabled, the HARQ-ACK feedback for all PDSCHs to be transmitted in the same PUCCH slot are expected to be all enabled or all disabled.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | OK |
| CATT | Seems that this procedure is also too restrictive. |
| CMCC | Ok to support to avoid the codebook misalignment. |
| Nokia, NSB | Ok |
| vivo | Same view with CATT. |
| Xiaomi | Same view with CATT and vivo. We don’t see the necessity with such restriction. HARQ-ACK feedback is handled per UE and gNB knows which one is enabled or disabled. We don’t see any misalignment between gNB and UE. |
| Samsung | Support. The proposal simplifies the design without having a problem in practice. The proposal may not need to be captured in the specifications (e.g. it can be a consequence of no other specification support and, if any DCIs have different indications, the UE can disregard such DCIs as providing “inconsistent” information based on Rel-15 specs) – in that sense, it can also be a “conclusion”. |
| Ericsson | Support |
| Qualcomm | The proposal is confusing. There can be the case that a PUCCH slot is to multiplex unicast and multicast, the feedback for unicast cannot be disabled. |
| NTT DOCOMO | We wonder if such restriction needs to be imposed to prevent codebook misalignment. |
| Intel | We think it’s better to handle codebook misalignment directly rather than enforce such restrictions. |
| Apple | Only the HARQ-ACK for the group common PDSCHs can be disabled. |
| ZTE | First of all, it has been agreed in last meeting that the DAI mismatch issue can be up to network implementation. The FL proposal is a potential way to address the DAI mismatch issue, however we are not sure whether we need to explicitly specify it. Also, looking at the above proposal and proposal 4.4.1-2 together, our understanding is that Proposal 4.4.1-1 is only for type-2 codebook. If this is the case, we should make this clear in the proposal. |
| OPPO | We did not see the need to make such restriction either. |
| MediaTek | Support the proposal. |
| FL’s response | Very divergent views especially for 4.4.1.-2. They are correlated. I suggest continue the discussion taking into others comments into account. |
| Samsung | Support the proposal as no further discussion would then be needed on how to construct the codebooks (i.e. how to treat DCI format with ‘enable’ and DCI formats with ‘disable’ for a same codebook). The proposal is applicable only to multicast. No need to capture anything for error cases in specifications as the UE behavior when receiving both ‘enable’ and ‘disable’ would be undefined. |
| Nokia, NSB2 | Similar view with Samsung. |

#### Proposal 4.4.1-2

- If the ACK/NACK based feedback is disabled by RRC signalling,

  - For Type-1 codebook, padding NACK such that CB size is the same as in the case if HARQ-ACK feedback was enabled.

  - FFS Type-2 codebook generation

- If the ACK/NACK feedback is disabled by group-common DCI,

  - For Type-1 codebook,

    - UE generates ACK if PDSCH is decoded correctly; otherwise, generates NACK.

  - FFS Type-2 codebook generation.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | For type-1 CB, if A/N is disabled by RRC or DCI, there is no HARQ-ACK feedback. Why is Type-1 CB discussed here? |
| CMCC | Similar view as Lenovo, especially considering we have the proposal 4.4.1-1 which when all PDSCHs feedback in the same PUCCH are all disabled, UE doesn’t need to transmit the PUCCH, regardless of type 1 or type 2 codebook. |
| Nokia, NSB | If the UE has sent an interest indication, it produces a Type-1 codebook (if configured) in all the PUCCH transmissions, even when no PDSCH was received. In our view, the proposal targets that problem. There would be no Type-2 codebook for that G-RNTI in case feedback is disabled. So, Type-2 codebook generation related proposals can be removed. - If the ACK/NACK based feedback is disabled by RRC signalling, - For Type-1 codebook, padding NACK such that CB size is the same as in the case if HARQ-ACK feedback was enabled. - FFS Type-2 codebook generation - If the ACK/NACK feedback is disabled by group-common DCI, - For Type-1 codebook, - UE generates ACK if PDSCH is decoded correctly; otherwise, generates NACK. - FFS Type-2 codebook generation. |
| Vivo | Similar view as Lenovo. For type 1 codebook, UE anyway has to construct HARQ-ACK codebook based on the candidate PDSCH reception occasions, it is not meaningful to enable to the enabling/disabling function of GC-PDCCH if type 1 codebook is configured. |
| Samsung | If HARQ-ACK is disabled by RRC, there is nothing for the UE to transmit and there is no ambiguity – same opinion as Lenovo. For HARQ-ACK disabling by DCI, there is the previous proposal to have same indication in all DCIs – the result will be same as for RRC configuration with the only difference that RRC-enabled can become DCI-disabled. |
| Ericsson | Regarding the first bullet, if ACK NACK feedback is disabled by RRC signalling: - We agree that if type 1 codebook is configured, the codebook size should not change. However, why should we use NACK padding when obviously the UE has the information on whether to send ACK or NACK, and also has the resource provision to send it via type 1 codebook configuration. - We assume this is in the case the type 1 codebook is common the unicast and multicast. If it is a multicast only CB, there is nothing to transmit. We are ok with the second bullet. For type 2 codebook generation, it should be straightforward that no bit is allocated to the codebook when disabled harq is either signalled in DCI or RRC configured. |
| Qualcomm | We don’t support the proposal. For RRC-configured disabling, no feedback and no ambiguity of Type-1 CB. For GC-DCI disabling, Type-1 codebook should not be applied. Why still padding NACK but disable feedback by GC-DCI? |
| Intel | We want to note that our proposal was for the case of DCI bases disabling of ACK/NACK. The case for RRC based disabling is not required. |
| Apple | For RRC disabled HARQ-ACK feedback, there is not HARQ-ACK bits generated for group common PDSCH at all. For group common DCI disabled feedback, for type 1 codebook, why still generate ACK and NACK. |
| ZTE | We are fine with the direction of this proposal. However, lots of detailed comments and updates. First of all, we think this proposal is only for type-1 codebook; If ACK/NACK feedback is disabled by group-common DCI, UE can just feedback NACK for type-1 codebook. Proposal: For Type-1 codebook, - If the ACK/NACK based feedback is disabled by RRC signalling, no codebook feedback is needed for this G-RNTI. - If the ACK/NACK feedback is disabled by group-common DCI, UE generates NACK for the corresponding PDSCH. |
| OPPO | Share the view that this proposal is not necessary. |
| MediaTek | Similar view with Lenovo. If the ACK/NACK is disabled, it means no HARQ-ACK feedback and the proposal is not necessary. |
| FL’s response | Very divergent views. Two proposals are correlated. I suggest continue the discussion taking into others comments into account. |
| CATT | The intention of this proposal is not clear for us. For Type-2 codebook, we did not see any issues on codebook construction when HARQ-ACK feedback is disabled. For Type-1 codebook, the issue on codebook misalignment definitely exists. We believe that both Proposal 4.4.1-1 and Proposal 4.4.1-2 are potential solutions to solve the misalignment issue when Type-1 codebook is configured. Per our understanding, the method in Proposal 4.4.1-1 is too restrictive. Regarding Proposal 4.4.1-2, we are OK with the first bullet. However, regarding second bullet, why UE generates ACK/NACK when DCI disabled indicating? |
| Samsung | As previously commented, the proposal is unnecessary, especially if the previous one is agreed. |
| Nokia, NSB2 | As we commented above, if the UE has sent an interest indication for a service, it produces a Type-1 codebook (if configured) in all the scheduled PUCCHs (e.g., for a unicast transmission), even when no PDSCH was received for that service. In our view, the proposal targets that problem. We can also agree with ZTE’s proposal in that sense. |

## (CLOSED)enable/disable HARQ-ACK for retransmissions

Submitted Proposals

“Applicability to retransmission”

- (Nokia) Proposal 21:

  - If the UE follows the DCI indication for enabling / disabling HARQ-ACK feedback for a G-RNTI, the UE follows the DCI indication also for transmitting / not transmitting HARQ-ACK feedback upon retransmissions.

- (CATT) Proposal 3:

  - The indication of the enabling/disabling for retransmission depends on the RRC/DCI signaling same as initial transmission.

(Lenovo) Proposal 5:

  - Same HARQ-ACK feedback option should be kept for initial transmission and retransmission for same TB.

- (Ericsson) Proposal 19:

  - The enable/disable indicator in the group-common PDCCHs can be present for either initial transmission and retransmission as long as they are PTM transmission.

### Round-1

FL’s Comments

One FFS pertaining to the agreement of enabling/disabling HARQ-ACK is the relation to the enabling/disabling ACK/NACK based HARQ-ACK feedback for retransmission.

Assuming it is unicast, if the HARQ-ACK is disabled, there should be no retransmission because no HARQ-ACK feedback at the network side. However, it is PTM transmission targeting a group of Ues. The case exists that the HARQ-ACK is disabled for some Ues but enabled for some other Ues in the same group. Hence, network may transmit the previous TB upon receiving NACK from some UE with HARQ-ACK enabled and the transmission can be PTM or PTP. As such, it is valid to clarify/discuss whether the HARQ-ACK is enabled or disabled for retransmission.

FL’s Proposal:

#### Proposal 4.5.1

- For PTM retransmission,

  - if UE is configured to enable/disable HARQ-ACK per group-common DCI indication for initial transmission, whether HARQ-ACK is enabled/disabled for PTM retransmission also follows the indication in the group-common DCI scheduling the PTM retransmission.

  - if UE is configured directly whether the HARQ-ACK is enabled/disabled, it applies to both PTM initial transmission and retransmission.

- For PTP retransmission, the HARQ-ACK is always enabled.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | We are OK with this proposal. Suggestion from our side to make the proposal concise: - Same HARQ-ACK feedback option applies to both PTM initial transmission and retransmission for a TB. - For PTP retransmission, the HARQ-ACK is always enabled. |
| CATT | OK |
| CMCC | OK |
| Nokia, NSB | Ok. |
| Vivo | ok |
| Xiaomi | We are fine with the proposal. |
| NEC | OK |
| Samsung | OK |
| NTT DOCOMO | Support |
| Ericsson | Support, with the understanding that “configured directly” in the second bullet point means “RRC configured”. Otherwise, the proposal needs to be clarified. |
| Qualcomm | Ok, also fine with Lenovo’s wording. |
| ZTE | We are not clear about the motivation to have the limitation for the PTM retransmission. At least from our perspective, we didn’t see any problem if different feedback option is indicated for the initial transmission and retransmission. Can the proponents of this proposal clarify the motivation? - FL’s response: if it RRC configured, it does not make sense the state from RRC is changed before retransmission, otherwise, the retransmission should probably not be valid. If it is DCI indication, the proposal says still follow DCI indication, is not saying the state indicated should be the same. |
| OPPO | OK |
| MediaTek | Lenovo’s wording is fine for us. |
| FL’s response | I would prefer to keep the original version especially when see others comments. |
| ZTE | Thanks for the FL’s clarification. With that, we are fine with the proposal. |
| TD Tech, Chengdu TD Tech | ok |

## “UE reaction” by enabled/disabled HARQ-ACK

Submitted Proposals

“UE reaction”

- (Intel) Proposal 2:

  - If RRC enables dynamic indication of HARQ on/off, UE always expects to be signaled with HARQ on/off in DCI i.e., there is no case where UE can ignore DCI based indication

(Lenovo) Proposal 6:

  - HARQ-ACK feedback option indicated in the group-common DCI should be followed by UE.

- (Ericsson) Proposal 15:

  - UE can be configured to monitor or ignore the enabling/disabling bit in group-common DCI.

    - UE configured to ignore the DCI indication for HARQ feedback type rely on the feedback type configured in RRC

    - UE configured to monitor the DCI indication for HARQ feedback type use the indicator in DCI to select one of two possible feedback types out of the possible three.

- (Ericsson) Proposal 17:

  - The UE is only expected to follow the enable / disable indicator for HARQ feedback in group-common DCI when the two following conditions are fulfilled:

    - If it is configured to use the enable/disable indicator present in group common PDCCH

    - If the HARQ feedback codebook type is of type 2.

### Round-1 (continue)

FL’s Comments

On FFS pertaining to the agreement of enabling/disabling HARQ-ACK is whether/how to allow UE not to react to the DCI signaling, but instead follow UE-specific RRC configuration for HARQ feedback.

In my recollection, the issue originates from the assumption that the field will be always included in the group-common DCI as long as one UE within the group is configured to enable/disable HARQ-ACK per DCI indication and then how UE reacts to the field indication in the DCI when the HARQ-ACK enabled/disabled is configured by RRC directly for such given UE.

Following the discussion of DCI 1_1 size alignment and of existing/new field in the DCI for the enabling/disabling HARQ-ACK indication, according to the current 38.212 draft CR, the “enabling/disabling HARQ-ACK feedback indication” is added as the last field in DCI format 1_1 scrambled by G-RNTI. In light of this, there is no need to discuss whether/how UE should react to the DCI indication in such case. Instead, for UEs configured with HARQ-ACK enabled/disabled by RRC directly will not interpret the last field; for UE configured with DCI indicating enabling/disabling the HARQ-ACK feedback will assume such indication is indicated by the last field in the DCI format.

FL’s Proposal:

#### Proposal 4.6.1

The field of indicating whether HARQ-ACK feedback is enabled/disabled is always the last field in the DCI format ahead of any reserved/padding bits.

  - Note1: For UE configured with HARQ-ACK enabled/disabled by RRC directly will not interpret the last field.

  - Note2: For UE configured with DCI indicating the HARQ-ACK feedback enabled/disabled will assume such indication is indicated by the last field in the DCI format.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | Agree. |
| CATT | Since this proposal is related with proposal Proposal 4.1.1-2, we prefer defer it. |
| CMCC | Agree with CATT. |
| Nokia, NSB | Agree. |
| Xiaomi | We don’t think there is any issue for UE reaction. The bit field of enabling/disabling is configured by the RRC. If gNB configure the enabling/disabling indication in the DCI, why it further configure the enabling/disabling via RRC signaling at the same time? |
| NEC | Agree to defer it. |
| Samsung | OK. |
| Ericsson | Support |
| Qualcomm | There are multiple configurable fields in DCI format 1_1, we don’t see it is necessary to specify the location as last field here. |
| NTT DOCOMO | We agree with CATT. |
| Apple | OK |
| ZTE | We don’t think this proposal is needed. Even if the field is not located in the end of this DCI format, there is still no issue. If UE supports dynamic indication, then this field can be correctly interpreted; if UE doesn’t support dynamic indication, then this field is reserved for this UE. |
| OPPO | Support |
| MediaTek | Ok |
| FL’s response | Let me explain the intention again: Since some UE could be configured with DCI indication and other UEs are just configured with RRC for enabling/disabling HARQ. For UEs configured with RRC for enabling/disabling HARQ, they don’t know whether the DCI includes such filed. In order not to affect these UEs, such filed should be put in the end of the format. |
| ZTE | It seems the issue can be addressed if this field is always in the DCI regardless of its configuration. |
| CATT | Why there is no effect on UEs which is configured with RRC for enabling/disabling HARQ with putting the field in the end of the format? |
| TD Tech, Chengdu TD Tech | ok |

## Others for enabled/disabled HARQ-ACK

“Default mode”

- (MediaTek) Proposal 4:

  - Disabling the feedback is as the default mode if the RRC signalling doesn’t configure the HARQ feedback function.

## others

### Feedback mode selection

Submitted Proposals

“Dynamic switching”

- (NEC) Proposal 2:

  - Reuse the existing field in the group-common DCI to indicate the HARQ-ACK feedback manner between ACK/NACK and NACK-only.

- (Lenovo) Proposal 1:

  - Support dynamic HARQ-ACK feedback switch among NACK-only based feedback and UE-specific ACK/NACK-based feedback.

(Lenovo) Proposal 3:

  - Support HARQ-ACK feedback indication as Table 1.

Table 1: Indication of HARQ-ACK feedback options

| Two-bit field (MSBLSB) | HARQ-ACK feedback option | PUCCH resource configuration |
| --- | --- | --- |
| 00 | NACK-only based feedback | group shared PUCCH resource |
| 01 | NACK-only based feedback | UE-specific PUCCH resource |
| 10 | ACK/NACK based feedback | UE-specific PUCCH resource |
| 11 | Disabled HARQ-ACK feedback | - |

- (MediaTek) Proposal 1:

  - The HARQ feedback mode can be indicated dynamically by DCI field, e.g., “HARQ feedback option” field.

### Retransmission

Submitted Proposals

“CBG-based retransmission”

(Intel) Proposal 14:

  - For ACK/NACK based HARQ operation, support UE specific CBG based retransmission. Other advanced retransmission schemes are not precluded.

- “retransmission vs. Codebook”

(LGE) Proposal 4:

  - HARQ-ACK to PTP retransmission is treated as HARQ-ACK to normal unicast transmission for construction of HARQ-ACK codebook.

- “retransmission vs. HARQ-ACK feedback mode”

(MediaTek) Proposal 7:

  - PTM scheme 1 or PTP retransmission is supported for ACK/NACK based HARQ feedback mechanism.

(MediaTek) Proposal 8:

  - PTM scheme 1 retransmission is supported for NACK-only based HARQ feedback mechanism.

# HARQ-ACK feedback for multicast SPS

## (CLOSED)NACK-only for SPS activation/deactivation

Submitted Proposals

(Huawei) Proposal 8:

  - For multicast SPS activation/deactivation, NACK-only based HARQ-ACK feedback is not supported..

(FUTUREWEI) Proposal 2:

  - For multicast SPS activation/deactivation, only support ACK/NACK based feedback.

(ZTE) Proposal 11:

  - Regarding HARQ-ACK feedback for SPS-based MBS transmission, if NACK-only feedback is configured:

    - The NACK-only feedback mode applies to ‘PDSCH without scheduling PDCCH’ only

    - The feedback mode for ‘PDSCH with scheduling PDCCH’ can be,

- Option 1: fixed to ACK/NACK feedback

- Option 2: follow the configuration of feedback mode for DG-PDSCH

    - FFS: the feedback mode for deactivation PDCCH

(Spreadtrum) Proposal 4:

  - For multicast SPS activation/deactivation PDCCH, support ACK/NACK based feedback, and the ACK/NACK based feedback can be enabled/disabled.

(Xiaomi) Proposal 4:

  - For SPS activation/deactivation of multicast SPS PDSCH, UE does not support NACK-only based HARQ-ACK feedback.

(CMCC) Proposal 9:

  - Support ACK/NACK based HARQ-ACK feedback for SPS activation/deactivation.

(Apple) Proposal 5:

  - For multicast SPS activation/deactivation, only support ACK/NACK based feedback.

(LGE) Proposal 11:

  - For group common SPS, UE specific confirmation to group common SPS (de-)activation can be supported by PUCCH A/N.

    - UE specific PUCCH resource is allocated by DCI indicating SPS (de-)activation.

(DOCOMO) Proposal 8:

  - Introduce a higher layer parameter to control feedback for SPS GC-PDSCH activation/deactivation

    - If configured, UE performs ACK/NACK-based feedback for activation/deactivation of SPS GC-PDSCH regardless of configuration/indication of feedback type and configuration/indication of enabled/disabled feedback

    - Otherwise, UE follows the configuration/indication

(Lenovo) Proposal 13:

  - For group-common SPS configuration, ACK/NACK feedback for confirming reception of activation/deactivation DCI is supported for each UE in the group.

(Lenovo) Proposal 15:

  - For group-common SPS configuration, the UE-specific PUCCH resource for confirming reception of activation/deactivation DCI is also used for the UE to transmit ACK/NACK for the SPS PDSCH.

(Qualcomm) Proposal 4:

  - For multicast SPS activation/deactivation, only support ACK/NACK-based feedback.

(MediaTek) Proposal 6:

  - MBS SPS activation/deactivation’s feedback mechanism only support ACK/NACK based HARQ feedback mode.

(Google) Proposal 4:

  - Support UE specific PDCCH for SPS activation/deactivation, at least for UE configured with NACK-only based feedback

(Google) Proposal 5:

  - For multicast SPS activation/deactivation, UE configured with NACK-only based feedback can transform NACK into ACK/NACK if the scheduled PUCCH resource is a UE specific resource.

### Round-1

FL’s Comments

This section discusses the issue whether both ACK/NACK based and NACK-only based feedback are supported or only ACK/NACK based feedback is supported for multicast SPS activation/deactivation.

The submitted proposals show clearly the vast majority view is to only support ACK/NACK based. In fact, this issue was discussed in the last meeting but was not concluded due to limited time to converge, though only supporting ACK/NACK based seems agreeable. Whether the ACK/NACK based feedback for SPS activation/deactivation can be disabled can be separately discussed.

FL’s Proposal:

#### Proposal 5.1.1

For multicast SPS activation/deactivation, only ACK/NACK based feedback is supported.

Collect views:

| Company | Comments |
| --- | --- |
| CATT | OK. |
| CMCC | OK |
| Nokia, NSB | Ok. |
| vivo | ok |
| Xiaomi | Support. |
| NEC | OK |
| Samsung | OK – no need to further discuss at this point |
| Futurewei | Support |
| NTT DOCOMO | Support |
| Ericsson | Support. What is important is to support ACK/NACK of activation/deactivation even if the data uses NACK-only or no feedback. We can however also agree to supporting NACK-only for activation/deactivation, provided the UE can be assumed to disregard duplicate successful activations or deactivations. |
| Qualcomm | support |
| Apple | Support |
| ZTE | OK |
| OPPO | OK |
| MediaTek | Support |
| Spreadtrum | Support |
| TD Tech, Chengdu TD Tech | ok |

## HARQ-ACK option for SPS multicast

Submitted Proposals

(vivo) Proposal 2:

  - For MBS for RRC_CONNECTED UEs, HARQ-ACK feedback options including disabled feedback, ACK/NACK and NACK-only for SPS group-common PDSCH is configured per SPS PDSCH index.

(Nokia) Proposal 24:

  - For multicast SPS PDSCH without PDCCH scheduling, HARQ-ACK feedback option is configured by UE RRC signalling per SPS configuration index.

(CATT) Proposal 20:

  - For multicast SPS PDSCH without PDCCH scheduling, the configuration HARQ-ACK feedback option is per G-CS-RNTI.

(CMCC) Proposal 10:

  - For multicast SPS PDSCH without PDCCH scheduling, HARQ-ACK feedback option is configured per G-CS-RNTI.

(DOCOMO) Proposal 7:

  - For HARQ-ACK feedback for SPS GC-PDSCH without PDCCH, HARQ-ACK feedback enabling/disabling is configured per SPS configuration index.

(Qualcomm) Proposal 3:

  - For multicast SPS PDSCH without PDCCH scheduling, HARQ-ACK feedback option is configured per G-CS-RNTI.

(Ericsson) Proposal 21:

  - For a UE configured to transmit multicast via SPS, configuration of the HARQ feedback option is done separately for each G-CS-RNTI of the SPS-configuration.

    - ACK/NACK feedback is always transmitted regardless of the configured HARQ-ACK feedback option for a given SPS config when the UE responds to SPS activation or release, i.e. ACK-NACK feedback is always used to follow a PDCCH scrambled with G-CS-RNTI.

    - If the UE is configured to monitor the DCI indicator for HARQ feedback options, for the SPS PDSCH which is associated with PDCCH activation and PDCCH deactivation, ACK/NACK feedback is always transmitted regardless of the indicator in this group-common PDCCH, i.e. ACK-NACK feedback is always used to follow a PDCCH scrambled with G-CS-RNTI. For SPS PDSCH without PDCCH, how to send HARQ feedback is according to the indicator in the group-common activation PDCCH.

- (Ericsson) Proposal 22:

  - HARQ feedback option for multicast SPS PDSCH should be configured G-CS-RNTI.

### Round-1 (closed)

FL’s Comments

ACK/NACK-based and NACK-only based feedback are both supported for multicast SPS PDSCH without PDCCH transmission and it was agreed in the last meeting either of them is configured by RRC signalling. The remaining issue is to address the FFS pertaining to the agreement:

Agreement:

For multicast SPS PDSCH without PDCCH scheduling, HARQ-ACK feedback option is configured by UE RRC signalling.

- FFS: Whether the configuration is per SPS configuration index or per G-CS-RNTI.

- Note: Whether there is a UE capability for support of NACK-only based HARQ-ACK or not will be discussed as part of UE features discussion.

Based on the contributions,

- Per SPS configuration index is supported by: vivo, Nokia, DOCOMO

- Per G-CS-RNTI is supported by: CATT, CMCC, Qualcomm, Ericsson

Based on the agreements achieved in AI 8.12.1, it is possible that multiple SPS configuration indices may be associated with one common G-CS-RNTI but only one G-CS-RNTI can be associated with a single SPS configuration index. Presumably the signalling structure to configure a list of SPS configuration for multicast is similar to what is used in 38.331 for unicast SPS as follows:

SPS-ConfigToAddModList-r16 ::= SEQUENCE (SIZE (1..maxNrofSPS-Config-r16)) OF SPS-Config … SPS-Config ::= SEQUENCE { periodicity ENUMERATED {ms10, ms20, ms32, ms40, ms64, ms80, ms128, ms160, ms320, ms640, spare6, spare5, spare4, spare3, spare2, spare1}, nrofHARQ-Processes INTEGER (1..8), n1PUCCH-AN PUCCH-ResourceId OPTIONAL, -- Need M mcs-Table ENUMERATED {qam64LowSE} OPTIONAL, -- Need S ..., [[ sps-ConfigIndex-r16 SPS-ConfigIndex-r16 OPTIONAL, -- Cond SPS-List harq-ProcID-Offset-r16 INTEGER (0..15) OPTIONAL, -- Need R periodicityExt-r16 INTEGER (1..5120) OPTIONAL, -- Need R harq-CodebookID-r16 INTEGER (1..2) OPTIONAL, -- Need R pdsch-AggregationFactor-r16 ENUMERATED {n1, n2, n4, n8 } OPTIONAL -- Need S ]] } …. SPS-ConfigIndex-r16 ::= INTEGER (0.. maxNrofSPS-Config-1-r16)

Therefore, including the HARQ-ACK feedback option into each SPS-Config-Multicast seems apparently straightforward and has the most flexibility. Setting to an identical HARQ-ACK option to the SPS-Config-Multicast configurations associated with the same G-CS-RNTI is equivalent to the configuration per G-CS-RNTI.

FL’s Proposal:

#### Proposal 5.2.1

HARQ-ACK feedback option is configured per multicast SPS configuration index.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | Agree |
| CATT | Not OK. Prefer ‘per G-CS-RNTI’ |
| CMCC | Prefer per G-CS-RNTI similar as G-RNTI, since which HARQ feedback option to be configured is related to the UE number of one multicast service (G-CS-RNTI) but not related to the SPS configuration. |
| Nokia, NSB | Ok. |
| vivo | ok |
| Xiaomi | Typically, different G-CS-RNTI corresponds to different service respectively. For HARQ-ACK feedback, it is straightforward to use the same type for a service. We don’t see the necessity of additional flexibility introduced by per SPS configuration index based selection. |
| Samsung | Agree with CATT/CMCC |
| NTT DOCOMO | Support |
| Ericsson | Support. |
| Qualcomm | Agree with CATT/CMCC/Samsung |
| Intel | Agree with CATT/CMCC/Samsung/Qualcomm |
| ZTE | Either “per multicast SPS configuration index” or “per G-CS-RNTI” works well. We don’t see any issue for either way. Different ACK option is more related to different QoS, from this perspective, maybe “per G-CS-RNTI” is better. |
| OPPO | Support. |
| Spreadtrum | Since the mapping of SPS configure index to G-CS-RNTI/G-RNTI is one-to-one, we have not seen the essential difference. Either could work. If there is no consensus, it can be up to editor. |
| FL’s resposne | Based on the comments, more and more are tending to have it configured by per G-CS-RNTI. Let’s try to updated to be per G-CS-RNTI |

### Round-2 (closed)

#### (EMAIL)Proposal 5.2.2

HARQ-ACK feedback option is configured per G-CS-RNTI.

Collect views:

| Company | Comments |
| --- | --- |
| - FL’s explanation | - Please check the discussion and FL’s responses in Round-1 to get why the proposal is updated as is. |
| - Samsung | - Support |
| - Qualcomm | - support |
| - ZTE | - Support |
| - CMCC | - Support |
| - OPPO | - OK |
| - Lenovo, Motorola Mobility | - OK |
| - Xiaomi | - Support |
| - Spreadtrum | - OK |
| - CATT | - Suppprt |
| - NTT DOCOMO | - OK |
| - Ericsson | - We understand the intention, but it is unclear why we need to introduce the restriction. A UE will be configured with one or more SPS configurations for multicast, which will include HARQ feedback options. The G-CS-RNTI is only attateched to a configuration at scheduling time, when the DCI is scrambled with the G-CS-RNTI. - FL’s response: as commented/discussed in the last round, since it is configured per G-RNTI for multicast and it is probably configured per G-CS-RNTI as well especially when considering it might be one service addressed by one G-CS-RNTI and it seems the majority view. |
| - TD Tech, Chengdu TD Tech | - Ok |

## Others for multicast SPS HARQ-ACK

Codebook related

(Huawei) Proposal 9:

  - When UE is configured with NACK-only for multicast SPS without PDCCH scheduling and when the HARQ-ACK bits for more than one multicast SPS configuration, for multicast SPS and unicast SPS, or for multicast SPS and dynamic scheduling of unicast/multicast, needs to be transmitted in the same PUCCH slot, the NACK-only based feedback for multicast SPS is transformed into the ACK/NACK based feedback.

(vivo) Proposal 3:

  - For the HARQ-ACK codebook for SPS PDSCHs regardless of unicast or multicast, UE adopts the same rule for codebook generation as defined in NR Rel-16.

(Nokia) Proposal 28:

  - When NACK-only based HARQ-ACK feedback is used for multicast SPS PDSCH without PDCCH scheduling, the UE determines a priority index from the HARQ-ACK codebook index in the configuration for SPS multicast, using the same method with the one for ACK / NACK based feedback.

- Further pursue NACK-only

(Nokia) Proposal 25:

  - For NACK-only HARQ operation a mechanism should be used, in which Ues are made aware via RRC signaling that SPS might be used for an MBS and request retransmission of an SPS activation PDCCH only if they have not received it in a certain amount of time.

(Nokia) Proposal 26:

  - While gNBs can send SPS deactivation commands, that are in NACK-only mode not acknowledged by Ues, Ues can assume that SPS has been deactivated if they have not been able to decode a PDSCH for a certain period of time.

(Nokia) Proposal 27:

  - In NACK-only HARQ operation, a method is supported for Ues to check with the gNB whether an SPS (re-)activation has been sent by the gNB but missed by the UE. Options include:

    - (a) Option 1: Using a group-common uplink resource

    - (b) Option 2: Using UE-specific signaling (MAC-CE or RRC message)

    - (c) Other methods are not precluded.

- Activation/deactivation related

(LGE) Proposal 12:

  - For a UE not confirming SPS activation, gNB can schedule PTP initial transmission of missed TB(s).

(LGE) Proposal 13:

  - After group common SPS activation, all Ues autonomously release the group common SPS right after a pre-determined slot

    - The pre-determined time is determined by RRC and/or DCI.

- “HARQ for retransmission”

(LGE) Proposal 14:

  - Either NACK only based HARQ-ACK or UE specific ACK/NACK based HARQ-ACK is used for SPS PDSCH retransmission.

(LGE) Proposal 15:

  - For multiplexing HARQ-ACKs for dynamically scheduled multicast and unicast SPS, UE determines a PUCCH resource based on one of the following options:

    - Option 1: The PRI of the last DCI with dynamic scheduling.

    - Option 2: Unicast SPS

    - Option 3: No multiplexing by dropping e.g. multicast HARQ-ACK

- “others”

(LGE) Proposal 16:

  - For multiplexing HARQ-ACKs for multicast SPS and dynamically scheduled unicast, UE determines a PUCCH resource based on the last unicast DCI.

(LGE) Proposal 17:

  - For multiplexing HARQ-ACKs for multicast SPS and dynamically scheduled multicast, UE determines a PUCCH resource based on the last multicast DCI.

(LGE) Proposal 18:

  - For multiplexing ACK/NACK based HARQ-ACKs for group common SPS PDSCHs, UE determines a PUCCH resource based on sps-PUCCH-AN-List of PUCCH-config for multicast as specified in 9.2.1 of 38.213.

    - If PUCCH-config for multicast is not configured, PUCCH-config for unicast is used.

(LGE) Proposal 19:

  - For group common SPS PDSCH(s) and unicast SPS PDSCH(s), UE determines a PUCCH resource based on sps-PUCCH-AN-List of PUCCH-config for unicast as specified in 9.2.1 of 38.213.

    - If NACK only based HARQ-ACK is configured for group common SPS, NACK only based HARQ-ACK is transformed to ACK/NACK based HARQ-ACK.

(LGE) Proposal 20:

  - For multiplexing NACK only based HARQ-ACK and ACK/NACK based HARQ-ACK for group common SPS PDSCHs, UE determines a PUCCH resource based on sps-PUCCH-AN-List of PUCCH-config for multicast for ACK/NACK based HARQ-ACK as specified in 9.2.1 of 38.213.

    - If PUCCH-config for multicast is not configured, PUCCH-config for unicast is used.

(LGE) Proposal 21:

  - For multiplexing SR and HARQ-ACKs for group common SPS PDSCHs (and possibly unicast SPS PDSCHs), UE determines a PUCCH resource based on:

    - Option 1: UE determines a PUCCH resource based on sps-PUCCH-AN-List of PUCCH-config for unicast as specified in 9.2.1 of 38.213

    - Option 2: HARQ-ACKs for group common SPS PDSCHs or SR is dropped.

# others

## PDSCH repetition

Submitted Proposals

- (Nokia) Proposal 23:

  - The UE can receive blind repetitions of the initial transmission that can be scheduled by both first and second DCI formats for multicast using pdsch-AggregationFactor.

## CSI feedback and trigger

Submitted Proposals

(Ericsson) Proposal 23:

  - The existing Rel. 15/16 framework of CSI feedback is reused for multicast/PTM.

## Other proposals

Submitted Proposals

(Nokia) Proposal 4:

  - A mechanism is adopted to disable HARQ-ACK feedback (and optionally CSI reporting) of the outlier UEs. This is down-selected from the following:

    - The UE detects itself that it is an outlier UE (e.g. if the reliability criteria cannot be met for a specific amount of time) and disables its own HARQ-ACK feedback mechanism.

    - The UE detects itself that it is an outlier UE and sends a request to the gNB to disable its feedback (e.g. using a specific CQI value such as CQI 0).

(Nokia) Proposal 5:

  - An outlier UE’s NACK-only HARQ-ACK feedback on group-common PUCCH resources can be re-activated by using one of the following methods:

    - The gNB can assign dedicated ACK / NACK resources to the UE and / or the UE can keep reporting CSI feedback, so that the gNB can decide when to re-activate the NACK-only based HARQ-ACK feedback of the UE.

    - The UE can be configured with a QoS criterion, such as an average BLER, and by satisfying such a criterion, the UE can either directly re-activate its feedback mechanism, or request gNB to allow the UE to utilize the group-common HARQ-ACK resource for NACK-only HARQ-ACK feedback.

(Intel) Proposal 1:

  - For the case when unicast and MBS PDSCH partially overlap in time on different PRBs, out-of-order HARQ feedback may be supported wherein the HARQ feedback corresponding to the PDSCH ending earlier in time may be transmitted after the HARQ feedback for the PDSCH ending later in time

(TCL) Proposal 1:

  - Support multiplexing of PTP and PTM RAN upper layer initial transmission data units within a transport block for UE capable of supporting one transport block per time slot, and support multiplexing of PTP and PTM initial transmission within a time slot for UE supporting multiple transport blocks per slot.

(TCL) Proposal 2:

  - Discuss the support of UE awareness of which part of the slot or transport block is configured for PTP and PTM within a multiplexed PTP and PTM initial transmission.

(TCL) Proposal 3:

  - Discuss the support of configuration of a partial ACK/NACK feedback code bit by a UE to indicate to the network the erroneous or lost PTP/PTM part of the initial transmission.

# Proposals for GTW on Nov. 19th

#### Proposal 4.1.1-1

For group-common DCI indicating whether ACK/NACK based HARQ-ACK feedback is enabled/disabled, the “enabling/disabling HARQ-ACK feedback indication” is only included in DCI format 1_1 scrambled by G-RNTI

- For DCI format 1_1 scrambled by G-CS-RNTI, it is discussed separately.

| Company | Comments |
| --- | --- |
| - FL’s explanation | - From Round-1: - Ok with proposal: Lenovo, CMCC, vivo, NEC, Samsung, Ericsson, Qualcomm, Apple, OPPO, Huawei - Also included into format 1_0/G-RNTI: Lenovo, CATT, CMCC, xiaomi, DOCOMO, ZTE, MediaTek, |

#### Proposal 4.1.1-2

For the DCI format including the field of “enabling/disabling HARQ-ACK feedback indication” for multicast scheduling, the field is a new field with 1 bit.

| Company | Comments |
| --- | --- |
| - FL’s explanation | - From Round-1: - Ok with proposal: Lenovo, xiaomi, Samsung, Ericsson, Qualcomm, [Intel], [Nokia], Apple, ZTE, OPPO, MedieTek - Using K1/PRI in the DCI format: Lenovo, LGE, CATT, CMCC, CATT, DOCOMO, ZTE |

#### Proposal 4.3.2-2-r2

For multicast SPS activation/deactivation, HARQ-ACK feedback is always enabled regardless whether PDSCH without PDCCH is configured/indicated with HARQ-ACK disabled.

- Note: For SPS activation, HARQ-ACK feedback is for the first PDSCH with PDCCH (activation) scheduling.

| Company | Comments |
| --- | --- |
| - FL’s explanation | - From Round-2: - Support proposal: Qualcomm, ZTE, CMCC, Lenovo, CATT, Nokia, Apple, Ericsson, TD Tech - Not support: Samsung, LGE, Spreadtrum, DOCOMO |

#### Proposal 4.3.2-1-r1

For multicast SPS PDSCH without PDCCH scheduling, support the following:

- RRC signalling configures the presence of the field “enabling/disabling HARQ-ACK feedback indication” in the group-common DCI for multicast SPS activation.

  - The configuration is per G-CS-RNTI.

  - Separate UE capability is needed from that for dynamic scheduling for multicast.

- RRC signalling configures directly whether the HARQ-ACK feedback is enabled or disabled.

  - The configuration is per G-CS-RNTI.

| Company | Comments |
| --- | --- |
| - FL’s explanation | - We have agreed HARQ-ACK feedback option is configured per G-CS-RNTI. - The HARQ-ACK enabling/disabling is naturally configured per G-CS-RNTI. - Samsung, Qualcomm and vivo from the last round have the concern to agree on the first sub-bullet. Samsung and vivo’s concern is mainly there is no HARQ-ACK feedback for SPS activation alone which should always go with the first SPS PDSCH and the DCI format is same as for dynamic (G-RNTI) and SPS (G-CS-RNTI). - Qualcomm views dynamic enabling/disabling is not needed for SPS multicast |

# References

R1-2110778	Mechanisms to improve reliability for RRC_CONNECTED UEs	Huawei, HiSilicon, CBN

R1-2110890	Remaining Issues on MBS Reliability	FUTUREWEI

R1-2110896	PUCCH formats for NACK-ONLY based HARQ-ACK feedback	TD Tech, Chengdu TD Tech

R1-2110911	Discussion on Mechanisms to Improve Reliability for RRC_CONNECTED UEs	ZTE

R1-2111040	Remaining issues on mechanisms to improve reliability for RRC_CONNECTED Ues	vivo

R1-2111114	Mechanisms to improve MBS reliability for RRC_CONNECTED UEs	Spreadtrum Communications

R1-2111136	Reliability Improvements for RRC_CONNECTED UEs	Nokia, Nokia Shanghai Bell

R1-2111231	Discussion on reliability improvement mechanism for RRC_CONNECTED UEs in MBS	CATT

R1-2111304	UL feedback for RRC-CONNECTED UEs in MBS	OPPO

R1-2111472	Discussion on enabling/disabling HARQ-ACK feedback for multicast of RRC_CONNECTED UEs	ETRI

R1-2111517	Mechanisms to Improve Reliability of NR-MBS for RRC_CONNECTED UEs	Intel Corporation

R1-2111550	Discussion on mechanisms to improve reliability for RRC_CONNECTED UEs	Xiaomi

R1-2111628	Discussion on reliability improvement	CMCC

R1-2111696	Discussion on Reliability Improvements for RRC_CONNECTED UEs	NEC

R1-2111762	On mechanisms to improve reliability for RRC_CONNECTED UEs	Samsung

R1-2111898	Discussion on MBS reliability improvement for RRC_CONNECTED UEs	Apple

R1-2111985	Discussion on Reliability Enhancement for Simultaneous MBS and Unicast Reception	TCL Communication Ltd.

R1-2112064	Mechanisms to improve reliability of Broadcast/Multicast service	LG Electronics

R1-2112129	Discussion on HARQ-ACK feedback for multicast for RRC_CONNECTED UEs	NTT DOCOMO, INC.

R1-2112162	On reliability improvement for RRC-CONNECTED UEs	Lenovo, Motorola Mobility

R1-2112240	Views on UE feedback for Multicast RRC_CONNECTED UEs	Qualcomm Incorporated

R1-2112313	Discussion on mechanisms to improve reliability for RRC_CONNECTED UEs	MediaTek Inc.

R1-2112347	Mechanisms to improve reliability for RRC_CONNECTED Ues	Ericsson

R1-2112383	Discussion on MBS reliability for RRC_CONNECTED UEs	Google Inc.

# Appendix Agreements summary for AI 8.12.2

## 102e

Agreements:

For RRC_CONNECTED UEs, HARQ-ACK feedback is supported for multicast and no additional evaluation is needed to justify this.

  - FFS: The detailed HARQ-ACK feedback solutions, e.g., ACK/NACK based, NACK-only based.

  - FFS: HARQ-ACK feedback can be optionally disabled and/or enabled.

Agreements:

- For RRC_CONNECTED UEs, at least support slot-level repetition for group-common PDSCH.

  - FFS: whether enhancement is needed

Agreements:

- For RRC_CONNECTED UEs, existing CSI feedback can be used for multicast transmission.

  - FFS: whether enhancement is needed

## 103e

Agreements:

For RRC_CONNECTED UEs receiving multicast, at least for PTM scheme 1, support at least one of the following:

- ACK/NACK based HARQ-ACK feedback for multicast,

  - From per UE perspective, UE feedback ACK or NACK.

  - From UEs within the group perspective,

    - FFS: PUCCH resource configuration for ACK/NACK feedback e.g., shared or separate PUCCH resources.

  - FFS details including conditions for it to be used

- NACK-only based HARQ-ACK feedback for multicast,

  - From per UE perspective, UE only feedback NACK.

  - From UEs within the group perspective, further down-select between:

    - FFS: PUCCH resource configuration for NACK only feedback.

  - FFS details including conditions for it to be used

- To decide in RAN1#104-e whether or not to support only one or both of the above schemes

  - If both are supported, FFS configuration/selection of ACK/NACK-based and NACK-only based HARQ-ACK feedback

Agreements:

For RRC_CONNECTED UEs receiving multicast, for ACK/NACK based HARQ-ACK feedback if supported for group-common PDCCH scheduling, PUCCH resource configuration for HARQ-ACK feedback from per UE perspective is, down-select one of the following options:

- Option 1: shared with PUCCH resource configuration for HARQ-ACK feedback for unicast

- Option 2: separate from PUCCH resource configuration for HARQ-ACK feedback for unicast

- Option 3: Option 1 or option 2 based on configuration

Agreements:

For RRC_CONNECTED UEs receiving multicast, for NACK-only based HARQ-ACK feedback if supported for group-common PDCCH scheduling, PUCCH resource configuration for HARQ-ACK feedback from per UE perspective is separate from PUCCH resource configuration for HARQ-ACK feedback for unicast.

- FFS PUCCH format

Agreements:

Enabling/disabling HARQ-ACK feedback for MBS is supported, further down-select between:

- Option 1: DCI

- Option 2: RRC configures enabling/disabling

- Option 3: RRC configures the enabling/ disabling function and DCI indicates enabling /disabling

- FFS: Option 4: MAC-CE indicates enabling/disabling

- FFS: Option 5: RRC configures the enabling/ disabling function and MAC-CE indicates enabling /disabling

Agreements:

For slot-level repetition for group-common PDSCH of RRC_CONNECTED UEs, for indicating the repetition number, further down-select among:

- Opt 1: by DCI

- Opt 2: by RRC

- Opt 3: by RRC+DCI

- FFS: Opt 4: by MAC-CE

- FFS: Opt 5: by RRC+MAC-CE

- FFS details for each option.

- FFS further enhancements for configuration of slot-level repetition

Agreements:

From the perspective of RRC_CONNECTED UEs receiving multicast, at least for PTM scheme 1 initial transmission, retransmission supports, for the purpose of down-selection, options are:

- Option 1: group-common PDCCH scheduled group-common PDSCH

- Option 2: UE-specific PDCCH scheduled PDSCH

  - Alt 1: PDSCH is UE-specific PDSCH

  - Alt 2: PDSCH is group-common PDSCH

- Option 3: both option 1 and option 2

- FFS other options

- FFS CBG based retransmission

Agreements:

FFS whether CSI feedback enhancement is needed for MBS, including but not limited:

- New CQI measurement

- New CSI report formats

- Targeted BLER

- CSI-RS configuration

- A-CSI-RS transmission triggering

- SRS configuration

Agreements:

For ACK/NACK based HARQ-ACK feedback if supported, both Type-1 and Type-2 HARQ-ACK codebook are supported for RRC_CONNECTED UEs receiving multicast,

- FFS details of HARQ-ACK codebook design.

- FFS whether enhanced Type-2 and/or Type-3 HARQ-ACK codebook is supported or not.

## 104e

Agreement:

For ACK/NACK based feedback if supported for RRC_CONNECTED UEs receiving multicast, UE can be optionally configured a separate PUCCH-Config for multicast. Otherwise, PUCCH-Config for unicast applies.

Agreement:

The priority for HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast can be,

- Lower, higher than or equal to the HARQ-ACK feedback for unicast

  - FFS: How to reflect the priority in specification, e.g., whether it is configured or indicated to the UE

  - FFS: The total number of priorities across multicast and unicast

- FFS the priority between HARQ-ACK feedback for multicast and other UCI for unicast (SR, CSI) or PUSCH for unicast.

Agreement:

For ACK/NACK based feedback if supported for multicast, for Type-2 HARQ-ACK feedback construction for PTM scheme 1,

- DAI for unicast and DAI for multicast are separately counted.

- Concatenation of Type-2 HARQ-ACK codebook for unicast and multicast is supported.

  - FFS details on   the codebooks.

- FFS whether to support concatenating more than one Type-2 HARQ-ACK codebook for multicast.

Agreement:

For RRC_CONNECTED UEs receiving multicast, support the following:

- ACK/NACK based HARQ-ACK feedback for multicast,

  - It is up to network to configure orthogonal PUCCH resources among UEs within the same group.

- FFS: NACK-only based HARQ-ACK feedback for multicast,

  - It is up to network to configure the PUCCH resources and the PUCCH resources can be shared among UEs within the same group.

- FFS details.

Agreement:

For the cases of HARQ-ACK feedback (at least for ACK/NACK based feedback) is available for multicast and unicast for a given UE receiving multicast, for determining the PUCCH resource,

- Support multiplexing for the same priority and prioritizing for different priorities at least when the corresponding PUCCH resources overlap in time in a slot.

  - FFS whether it is subject to UE capability.

- FFS the case of non-overlapping PUCCHs resources for HARQ-ACK in the same slot.

- FFS whether sub-slot based PUCCH transmission for HARQ-ACK is supported.

- FFS the case of HARQ-ACK feedback for multicast and other UCI for unicast.

Agreement:

For ACK/NACK based feedback if supported for multicast, construction of Type-1 HARQ-ACK codebook based on the union of the PDSCH TDRA sets of the unicast service and the multicast service (if they are separately configured), at least of the same priority, is supported

- FFS details of Type-1 HARQ-ACK codebook construction for FDM-ed unicast and multicast.

- FFS details of Type-1 HARQ-ACK codebook construction for FDM-ed multicast and multicast if supported.

- FFS: whether/how to optimize the Type-1 codebook construction to reduce the HARQ-ACK feedback payload size.

Agreement:

For enabling/disabling HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast,

- Option 3: RRC signalling configures the enabling/ disabling function of DCI indicating the enabling /disabling HARQ-ACK feedback.

  - If RRC signalling configures the function, DCI indicates (explicitly or implicitly) whether HARQ-ACK feedback is enabled/disabled

    - FFS details on RRC signalling and DCI indicating.

  - If RRC signalling does not configure the function, DCI does not indicate enabling/disabling the HARQ-ACK feedback.

    - FFS whether enabling or disabling the feedback is the default mode.

- Option 2: RRC indicates enabling/disabling.

- FFS: whether down-selection between option 3 and option 2 is needed or support the both options.

- FFS: enabling/disabling by MAC-CE.

Agreement:

For slot-level repetition for group-common PDSCH for RRC_CONNECTED UEs receiving multicast,

- (Config A) UE can be optionally configured with pdsch-AggregationFactor.

- (Config B) UE can be optionally configured with TDRA table with repetitionNumber as part of the TDRA table.

- If UE is configured with Config B, UE does not expect to be configured with Config A for the same group-common PDSCH.

## 104be

Agreement:

Support NACK-only based HARQ-ACK feedback for RRC_CONNECTED UEs receiving multicast.

Agreement:

Two priority indexes are introduced for multicast, with

- Index 0 meaning low priority and index 1 meaning high priority.

- Priority index can be included in DCI formats scheduling the group-common PDSCH.

  - FFS details for DCI formats.

- FFS: the priority comparison between multicast and unicast with the same priority index.

Agreement:

For a separate PUCCH-ConfigurationList for multicast that is optionally configured, at least for ACK/NACK based HARQ-ACK feedback,

- The separate PUCCH-ConfigurationList for multicast configuration can be a list which includes up to 2 PUCCH-Config configurations corresponding low priority codebook and high priority codebook, respectively.

- FFS other configurations

Agreement:

For Type-2 HARQ-ACK codebook concatenation to be multiplexed in the same PUCCH resource,

- The first Type-2 HARQ-ACK sub-codebook for unicast precedes the second Type-2 HARQ-ACK sub-codebook for multicast.

- FFS: The number of Type-2 HARQ-ACK sub-codebooks for multicast.

- Note: The case of SPS PDSCH will be discussed separately.

Agreement:

For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, determining the PUCCH resources for transmission is based on the PRI indicated in the “last DCI”, where the “last DCI” refers to, down-select the following alternatives:

- Alt.1: the last DCI for unicast;

- Alt.2: the last DCI across unicast and multicast;

## 105e

Agreement:

- The signaling for URLLC feature can be reused to configure separate codebooks for unicast and multicast, respectively, at least for the case of different priorities, at least for Type-2 HARQ codebook

- FFS: The case for the same priority.

- FFS: The case of Type-1 HARQ codebook

- FFS: Whether this applies to separate PUCCH transmissions only

Agreement:

The priority of multicast is the same as the priority of unicast for the same priority index of HARQ-ACK at least for ACK/NACK based feedback.

Agreement:

NR supports at least the following cases for UE supporting multicast:

- UE supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast with different priorities in a slot subject to UE capability.

- UE supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast and unicast with different priorities, respectively, in a slot subject to UE capability.

Agreement:

For Type-1 HARQ-ACK codebook construction for FDM-ed unicast and multicast with the same priority from the same TRP, support

- Opt 4: HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells for unicast, precede, HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells for multicast. (This is similar to the joint Type-1 codebook for mTRP).

- FFS: If UE reports the capability of supporting the FDM-ed unicast and multicast in the same slot, UE can be indicated semi-statically to generate Type-1 HARQ-ACK codebook as FDM-ed manner (i.e., Opt 4).

  - Otherwise, UE does not expect unicast and multicast are to be scheduled in FDM-ed.

Agreement:

For TDM-ed unicast and multicast, for Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, determining PDSCH reception candidate occasions is based on down-selecting one of the two alternatives as follows:

- Alt 1:

  - for slot timing values K1 in the intersection of K1 set for unicast (termed set A) and K1 set for multicast (termed set B), based on union of the PDSCH TDRA sets,

  - for slot timing values K1 in set A but not in set B, based on PDSCH TDRA set for unicast, and

  - for slot timing values K1 in set B but not in set A, based on PDSCH TDRA set for multicast.

- Alt 2: for slot timing values K1 in the union of K1 set for unicast and K1 set for multicast, based on the union of the PDSCH TDRA sets.

- Companies are encouraged to continue discussion of pros and cons for each alternative for further down-selection in the next meeting.

Agreement:

For ACK/NACK based HARQ-ACK feedback for multicast, the multiplexing/prioritizing rule between the HARQ-ACK for multicast and SR/CSI can reuse Rel-16 multiplexing/ prioritizing rule between the HARQ-ACK for unicast and SR/CSI.

Working assumption:

For enabling/disabling ACK/NACK-based HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast via dynamic group-common PDSCH:

- RRC signaling configures the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback.

  - If RRC signaling configures the function of group-common DCI based indication, group-common DCI indicates (explicitly or implicitly) whether ACK/NACK based HARQ-ACK feedback is enabled/disabled

  - Otherwise, enabling/disabling ACK/NACK based HARQ-ACK feedback is configured by RRC signaling.

  - FFS details on RRC signaling and group-common DCI indicating.

- FFS whether/how this option is extended to apply to NACK-only based feedback and multiple G-RNTI cases.

- FFS the relation to the HARQ-ACK codebook types and HARQ-ACK codebook construction.

- FFS the relation to the enabling/disabling ACK/NACK based HARQ-ACK feedback for retransmission.

- FFS whether/how to allow UE not to react to the DCI signaling, but instead follow UE-specific RRC configuration for HARQ feedback.

- FFS whether/how to apply it to SPS group-common PDSCH.

- Agreement:

- Support PUCCH format 0 and format 1 for NACK-only based HARQ-ACK feedback for multicast.

Agreement:

Support NACK-only based HARQ-ACK feedback at least for multicast SPS PDSCH without PDCCH scheduling.

- FFS for SPS activation/deactivation.

Conclusion:

PUCCH resource for NACK-only can be shared by UEs transmitting the NACK-only based HARQ-ACK feedback.

Agreement:

For support of ACK/NACK based HARQ-ACK feedback for SPS multicast,

- the HARQ-ACK codebook index corresponding the HARQ-ACK codebook for SPS PDSCH is included in the configuration for SPS multicast.

  - UE determines a priority index from the HARQ-ACK codebook index

- UE can be optionally configured a separate SPS-PUCCH-AN-List for all SPS multicast configurations. Otherwise, a common SPS-PUCCH-AN-List applies to all SPS unicast and SPS multicast configurations.

## 106e

Agreement:

For UE supporting both unicast and multicast, the pdsch-HARQ-ACK-Codebook/pdsch-HARQ-ACK-CodebookList can be separately configured for multicast from that for unicast.

Agreement:

When UE is configured Type-1 codebooks for unicast and multicast with different priorities, respectively, the UE separately generates each of the Type-1 codebooks.

- FFS: How UE is configured one codebook for unicast and one codebook for multicast and the two codebooks are of different priorities.

Agreement:

For a UE configured with Type-1 HARQ-ACK codebook,

- If UE is not configured to receive FDM-ed unicast and multicast, Type-1 HARQ codebook is generated as the agreement for TDM-ed unicast and multicast.

- If UE is configured to receive FDM-ed unicast and multicast, Type-1 HARQ codebook is generated as the agreement for FDM-ed unicast and multicast.

Agreement:

For UEs supporting ACK/NACK-based HARQ-ACK feedback for multicast and unicast, the following values are unchanged compared to unicast in Rel-16:

  - The maximum number of PUCCH resources sets in each PUCCH-Config,

  - The maximum number of PUCCH resources in a PUCCH resource set in each PUCCH-Config,

  - The maximum number of UCI information bits for the first PUCCH resource set.

  - The total number of PUCCH resources from all PUCCH-Config/PUCCH-ConfigurationList.

  - Note:

    - This applies to both cases of whether or not UE is configured optionally with a separate PUCCH-Config or PUCCH-ConfigurationList for multicast.

    - The case of NACK-only based is discussed separately.

Agreement:

- When UE is configured with the pdsch-HARQ-ACK-Codebook/pdsch-HARQ-ACK-CodebookList for ACK/NACK based feedback for multicast, it is applied to all G-RNTIs configured to UE.

Agreement:

For the separate PUCCH-ConfigurationList that is optionally configured to UE for NACK-only based HARQ-ACK feedback for multicast,

  - The separate PUCCH-ConfigurationList for multicast configuration can be a list which includes up to 2 PUCCH-Config configurations corresponding low priority feedback and high priority feedback, respectively.

  - FFS: how to handle the case when separate PUCCH-ConfigurationList is not configured to UE for NACK-only based HARQ-ACK feedback for multicast.

Agreement:

The priority index is,

- for the second DCI format for GC-PDCCH, optionally configured to be included in the DCI format. If not configured, the priority index is not included in the DCI format and is low priory by default.

- for the first DCI format for GC-PDCCH, down-select from:

    - Alt1: Optionally configured to be included in the DCI format. If not configured, the priority index is not included in the DCI format and is low priory by default.

    - Alt2: Always low priority, i.e., the priority index is not included in the DCI format.

Agreement:

The priority of multicast for NACK-only based feedback is the same as the priority of unicast for the same priority index of HARQ-ACK.

Agreement:

When more than one NACK-only based feedback are available for transmission in the same PUCCH slot, down-select from the following alternatives:

  - Alt1: Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits.

  - Alt2: Support sub-slot based PUCCH for this case.

  - Alt3: Support UE transmitting more than one slot-based PUCCHs in the same PUCCH slot.

  - Alt4: Define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission.

  - Alt5: NACK-only bundling

Agreement:

When UE supports and is configured with more than one G-RNTI,

  - for Type-2 codebook construction, DAI is separately counted per G-RNTI.

  - Type-2 codebook is constructed by concatenating Type-2 sub-codebook of each RNTI following the ascending order of the G-RNTI value.

Agreement:

Update the WA made in RAN1#105-e meeting regarding enabling/disabling HARQ-ACK feedback as follows:

Working assumption:

For enabling/disabling ACK/NACK-based HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast via dynamic group-common PDSCH:

- RRC signaling configures the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback.

  - If RRC signaling configures the function of group-common DCI based indication, group-common DCI indicates (explicitly or implicitly) whether ACK/NACK based HARQ-ACK feedback is enabled/disabled

  - Otherwise, enabling/disabling ACK/NACK based HARQ-ACK feedback is configured by RRC signaling.

  - FFS details on RRC signaling and group-common DCI indicating.

- FFS whether/how this option is extended to apply to NACK-only based feedback and multiple G-RNTI cases.

- FFS the relation to the HARQ-ACK codebook types and HARQ-ACK codebook construction.

- FFS the relation to the enabling/disabling ACK/NACK based HARQ-ACK feedback for retransmission.

- FFS whether/how to allow UE not to react to the DCI signaling, but instead follow UE-specific RRC configuration for HARQ feedback.

- FFS whether/how to apply it to SPS group-common PDSCH.

- UE capability for enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback is introduced and FFS details.

- Note: It is up to network implementation to avoid any potential HARQ ACK mismatch between different UEs in the same multicast group

Agreement

For UE supports both ACK/NACK-based and NACK-only based HARQ-ACK feedback for multicast SPS PDSCH without PDCCH scheduling, select one or more of the following alternatives:

  - Alt1: HARQ-ACK feedback option is configured per SPS configuration index.

  - Alt2: HARQ-ACK feedback option is indicated in the SPS activation DCI.

  - Note: enabling/disabling HARQ-ACK feedback for multicast SPS can be discussed separately.

## 106be

Agreement:

The group-common DCI indicating the enabling/disabling ACK/NACK based HARQ-ACK feedback is configured per G-RNTI by UE RRC signalling.

Agreement:

If the group-common DCI indicating the enabling/disabling ACK/NACK based HARQ-ACK feedback is not configured, enabling/disabling ACK/NACK based HARQ-ACK feedback is configured per G-RNTI by UE RRC signalling.

Agreement:

When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for HARQ-ACK feedback/CSI for unicast for the same priority or PUSCH transmission for the same priority, support UE multiplexing the NACK-only based feedback with the HARQ-ACK feedback/CSI on PUCCH or on to PUSCH by transforming NACK-only into the ACK/NACK HARQ bit.

- This applies to at least the case of the feedback addressing one TB. NACK-only based feedback for more than one TBs is to be handled separately.

- Note: When the TB is correctly decoded, the ACK will be transmitted and multiplexed with others.

- FFS the case of PUCCH for SR.

Agreement:

When more than one NACK-only based feedback are available for transmission in the same PUCCH slot, further decide based on the following subset of alternatives (from previous agreement) with potential further down-selection:

- Alt1: Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits.

- Alt2: Support sub-slot based PUCCH for this case.

- Alt3: Support UE transmitting more than one slot-based PUCCHs in the same PUCCH slot.

- Alt4: Define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission.

- Alt5: NACK-only bundling

Agreement:

Confirm the WA made in RAN1#106-e meeting regarding enabling/disabling HARQ-ACK feedback.

Agreement:

For group-common DCI indicating whether ACK/NACK based HARQ-ACK feedback is enabled/disabled, down-select from the following alternatives:

- Alt1: Reuse one existing field in the group-common DCI.

- Alt2: Introduce a new field in the group-common DCI.

Agreement:

For multicast SPS PDSCH without PDCCH scheduling, HARQ-ACK feedback option is configured by UE RRC signalling.

- FFS: Whether the configuration is per SPS configuration index or per G-CS-RNTI.

- Note: Whether there is a UE capability for support of NACK-only based HARQ-ACK or not will be discussed as part of UE features discussion.

Agreement:

- If configured, the pdsch-AggregationFactor for multicast dynamic scheduling is configured per G-RNTI.

- If configured, the pdsch-AggregationFactor for multicast SPS is configured per SPS-Config-Multicast.

Agreement:

For slot-level repetition for SPS GC-PDSCH for multicast RRC_CONNECTED UEs.

- Config A or Config B can be configured to UE:

    - (Config A) UE can be optionally configured with pdsch-AggregationFactor per SPS-Config-Multicast.

    - (Config B) UE can be optionally configured with TDRA table with repetitionNumber as part of the TDRA table in PDSCH-Config-Multicast.

    - If UE is configured with Config B, UE does not expect to be configured with Config A for the same SPS group-common PDSCH.

- For Config A, if pdsch-AggregationFactor in SPS-Config-Multicast is not configured, default value is

    - Alt1: equal to 1.

Agreement:

For UE supporting both ACK/NACK based and NACK-only based feedback for multicast, for the same G-RNTI, support the following

- UE can be configured with either ACK/NACK based or NACK-only feedback for a single G-RNTI.

  - Note: Case1-1: if configured with ACK/NACK based feedback, UE can be optionally configured a separate PUCCH-Config/PUCCH-ConfigurationList for multicast. Otherwise, PUCCH-Config/PUCCH-ConfigurationList for unicast applies (This has been agreed.)

  - Case 1-2: if configured with NACK-only based feedback, when separate PUCCH-Config/PUCCH-ConfigurationList for NACK-only is not configured, PUCCH-Config/PUCCH-ConfigurationList for unicast applies.

Agreement:

For the priority index for the first DCI format for GC-PDCCH, support the following Alt2 from the previous agreement:

  - Alt2: Always low priority, i.e., the priority index is not included in the DCI format.

Agreement:

For TDM-ed unicast and multicast, for Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, determining PDSCH reception candidate occasions can be configured between the following alternatives from the previous agreement:

- Alt 1:

  - for slot timing values K1 in the intersection of K1 set for unicast (termed set A) and K1 set for multicast (termed set B), based on union of the PDSCH TDRA sets,

  - for slot timing values K1 in set A but not in set B, based on PDSCH TDRA set for unicast, and

  - for slot timing values K1 in set B but not in set A, based on PDSCH TDRA set for multicast.

- Alt 2: for slot timing values K1 in the union of K1 set for unicast and K1 set for multicast, based on the union of the PDSCH TDRA sets.

- Support of Alt. 1 is a UE capability

Agreement:

For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, determining the PUCCH resources for transmission is based on the PRI indicated in the “last DCI”, where the “last DCI” refers to the following Alt1 from the previous agreement:

- Alt.1: The last DCI for unicast

- FFS: Any details when last DCI is missed by the UE if it is necessary to make them different from current specifications for this case.