3GPP TSG-RAN WG1 Meeting #107	R1-2112601

e-Meeting, 11th – 19th November 2021

Agenda Item:	8.6.1.2

Title:	FL summary #3 on other aspects for RedCap

Source:	Moderator (Qualcomm Inc.)

Document for:	Discussion, Decision

# Introduction

This feature lead (FL) summary (FLS) concerns the Rel-17 work item (WI) for support of reduced capability (RedCap) NR devices [1]. Earlier RAN1 agreements for this WI are summarized in [2].

This document summarizes contributions [3] – [25] submitted to agenda item 8.6.1.2 and captures this email discussion on other aspects operation for RedCap:

/This one is to use NWM – please use RAN1-107-e-NWM-NR-R17-RedCap-02 as the document name [107-e-NR-R17-RedCap-02] Email discussion regarding other aspects of UE complexity reduction – Chao (Qualcomm) - 1st check point: November 15 - Final check point: November 19

The following proposals are candidates for email endorsement.

FL3 High Priority Proposal 5.3-1: - The “back-to-back” non-overlapping UL/DL without sufficient gap between cell-specific configured DL and dedicated configured UL may happen, i.e., allowed for HD-FDD UEs - E.g., SSB vs. CG PUSCH, PUCCH or SRS - Configured UL transmission is cancelled (as in the overlapping case) - The “back-to-back” non-overlapping UL/DL without sufficient gap between dedicated configured DL and cell-specific configured UL may happen, i.e., allowed for HD-FDD UEs - E.g., PDCCH in USS, SPS PDSCH, CSI-RS or DL PRS vs. valid RO - Leave it to UE implementation to cancel either DL reception or UL transmission to ensure sufficient switching time FL3 High Priority Proposed Conclusion 5.4-1: - No additional UE behavior for DL/UL collision handling is specified in Rel-17 if SFI monitoring is supported for HD-FDD RedCap UEs. Updated FL3 High Priority Proposal 5.1-1: - For Case 5 of SSB overlapping with Msg3 (re)transmission and or PUCCH for Msg4, reuse the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over Msg3 and PUCCH for Msg4

Attached to this cover-page document is the report of the NWM discussion (v0.0.3) with the detailed discussion included.