3GPP TSG RAN WG1 #107-e			R1-2112620

e-Meeting, November 11th – 19th, 2021

Agenda Item:	8.2.5

Source: 	Moderator (LG Electronics)

Title: 	Summary #2 of PDSCH/PUSCH enhancements (Scheduling/HARQ)

Document for:	Discussion and decision

Introduction

This is the summary document for 8.2.5 on PDSCH/PUSCH enhancements (especially for scheduling and HARQ) for NR above 52.6 GHz, based on the contributions listed in reference section.

The following email thread is assigned for discussion of this topic:

[107-e-NR-52-71GHz-06] Email discussion/approval on scheduling particularly w.r.t. multi-PDSCH/PUSCH with a single DCI, HARQ, with checkpoints for agreements on November 15 and 19 – Seonwook (LGE)

Multi-PDSCH/PUSCH scheduling

CBG-based (re)transmission

| Company | Views |
| --- | --- |
| [3] vivo | Proposal 6: For CBG based scheduling, the same behaviour for multi-PUSCH scheduling with 120 kHz SCS is applied to 480/960 kHz SCS as well, i.e., CBG based scheduling is supported only when a DCI schedules a single PUSCH. Proposal 7: For multi-PDSCH scheduling, do not support CBG based scheduling for 120/480/960 kHz SCS. |
| [10] Panasonic | Proposal 2: For SCSs of 480 kHz and 960 kHz, for a DCI that can schedule multiple PUSCHs and is configured with the TDRA table containing at least one row with multiple SLIVs, - If CBG-based (re)transmission is configured, CBGTI field is not present when more than one PUSCHs are scheduled, but is present when a single PUSCH is scheduled, as in Rel. 16. Proposal 3: For SCSs of 120 kHz, 480 kHz, and 960 kHz, for a DCI that can schedule multiple PDSCHs and is configured with the TDRA table containing at least one row with multiple SLIVs, - If CBG-based (re)transmission is configured, CBGTI/CBGFI fields are not present when more than one PDSCHs are scheduled, but are present when a single PUSCH is scheduled, as in Rel. 16. |
| [11] Ericsson | Proposal 6: For 480/960 kHz SCS, for a DCI that can schedule single and/or multiple PUSCHs, configuration of CBG-based (re)-transmission is not supported, and thus the CBGTI fields is not present. Proposal 7: For a DCI that can schedule single and/or multiple PDSCHs, configuration of CBG-based (re)-transmission is not supported, and thus the CBGTI and CBGFI fields are not present |
| [12] Intel | Proposal 1 - For multi-PDSCH/PUSCH scheduling, CBG based transmission is supported for 120/480/960kHz subcarrier spacing when a single PDSCH/PUSCH is scheduled. |
| [16] Samsung | Proposal 6: For multi-PUSCH scheduling DCI: - CBG: - Not support CBG-based transmission for single and multi-PUSCH scheduling for 480/960 KHz. - Not support CBG-based transmission for multi-PUSCH scheduling for 120KHz, but applicable for single-PUSCH scheduling for 120KHz. - Frequency hopping: Support intra-PUSCH hopping - FDRA: Support increased RBG size using the same mechanism introduced in Rel-16 URLLC Proposal 7: For multi-PDSCH scheduling, the bit field common for DL and UL grant use the same design as multi-PUSCH scheduling, and at least following DL-specific bit field should be specified, - CBG-based transmission is not applicable to single and multi-PDSCH scheduling - HARQ-ACK relevant bit field is applicable to all PDSCHs and single PUCCH |
| [17] InterDigital | Proposal 5: For 480/960 kHz SCS, apply the same behavior of 120 kHz SCS for CBGTI field configuration in the DCI that can schedule multiple PUSCHs, i.e., if CBG-based (re)transmission is configured, CBGTI field is not present when more than one PUSCHs are scheduled, but is present when a single PUSCH is scheduled, as in Rel-16. Proposal 6: The same behavior of multi PUSCH could be applied for CBGTI/CBGFI fields when a DCI schedule multiple PDSCHs, i.e., CBGTI/CBGFI fields are not present if multiple PDSCHs are scheduled, but present if only one PDSCH is scheduled. |
| [18] Apple | Proposal 4: For Rel-17 multi-PUSCH transmission - The maximum number of PUSCHs that can be scheduled can be further restricted based on UE capabilities. - For 480 kHz and 960 kHz SCS, no support for CBGTI field configuration in the DCI that can schedule multiple PUSCHs - The FDRA size should be optimized to reduce the FDRA overhead. - Support inter-slot frequency hopping and NOT intra-slot frequency hopping for 480 kHz and 960 kHz Proposal 5: For Rel-17 multi-PDSCH transmission - The maximum number of PDSCHs that can be scheduled can be further restricted based on UE capabilities. - For a DCI that can schedule multiple PDSCHs and is configured with the TDRA table containing at least one row with multiple SLIVs, do not support/configure CBGTI/CBGFI fields - The FDRA size should be optimized to reduce the FDRA overhead. - Support inter-slot frequency hopping and NOT intra-slot frequency hopping for 480 kHz and 960 kHz |
| [19] LG Electronics | Proposal #5: For multi-PDSCH scheduling DCI, CBGTI/CBGFI cannot be configured, for 120/480/960 kHz SCSs. Proposal #6: For multi-PUSCH scheduling DCI, if CBG-based (re)transmission is configured, CBGTI is not present when more than one PUSCH is scheduled, but is present when a single PUSCH is scheduled, for 120/480/960 kHz SCSs. |
| [20] NTT DOCOMO | Proposal 1: - For multi-PUSCH scheduled by single DCI, - CBG based scheduling is not supported when multiple PUSCHs are scheduled by one DCI. - Support frequency hopping for multi-PUSCH scheduling, by reusing existing PUSCH frequency hopping scheme, i.e. intra-slot PUSCH frequency hopping. - For multi-PDSCH scheduled by single DCI, - CBG based scheduling is not supported when multiple PDSCHs are scheduled by one DCI. - For two-TB scheduling, support limitation on the maximum number of PDSCHs when 2-TB is scheduled. When RRC parameter enables two TB scheduling, - If the number of scheduled PDSCHs is no larger than value X (X>1), two TBs can be scheduled for each PDSCH. - If the number of scheduled PDSCHs is larger than value X (X>1), only single TB can be scheduled for each PDSCH. |
| [22] MediaTek | Proposal 3: CBG (re)transmission feature for 480kHz and 960kHz is not supported in FR2-2. |

- Issue 2.1-1) CBGTI field in multi-PUSCH scheduling DCI:

Agreement: (RAN1#105-e)

- At least for 120 kHz SCS, for a DCI that can schedule multiple PUSCHs and is configured with the TDRA table containing at least one row with multiple SLIVs,

  - If CBG-based (re)transmission is configured, CBGTI field is not present when more than one PUSCHs are scheduled, but is present when a single PUSCH is scheduled, as in Rel-16.

- FFS:

  - For 480/960 kHz SCS, whether to apply the same behavior with 120 kHz SCS or not to support CBGTI field configuration in the DCI that can schedule multiple PUSCHs

  - For a DCI that can schedule multiple PDSCHs and is configured with the TDRA table containing at least one row with multiple SLIVs, whether/how to configure CBGTI/CBGFI fields

Company views on CBGTI field in multi-PUSCH scheduling DCI:

- Same behaviour for all SCSs as in Rel-16

  - Supported by vivo, Panasonic, Intel, InterDigital, LG Electronics, NTT DOCOMO

- Do not support CBGTI field configuration for multi-PUSCH scheduling DCI for 480/960 kHz

  - Supported by Ericsson, Samsung, Apple, MediaTek

[Moderator’s note] According to the slight majority view, the following proposal can be made. It is noted that the support of CBGTI field for multi-PUSCH scheduling DCI doesn’t lead to additional specification impact. This issue is indicated as “HIGH” since it is essential for WI completion.

- [HIGH] Proposal #2.1-1 (CBG for UL):

- In addition to 120 kHz SCS, for 480/960 kHz SCS, for a DCI that can schedule multiple PUSCHs and is configured with the TDRA table containing at least one row with multiple SLIVs,

  - If CBG-based UL (re)transmission is configured, CBGTI field is not present when more than one PUSCH are scheduled, but is present when a single PUSCH is scheduled, as in Rel-16.

  - Note: Specification impact is not expected.

Companies are encouraged to provide views on Proposal #2.1-1.

| Company | Views |
| --- | --- |
| Samsung | Not support. For specification perspective, we tend to agree with moderator that no further specification impacts, but it is important to justify that this feature is important in FR2-2 even if it causes additional DCI overhead. Given the short symbol duration, we cannot see any benefits for CBG-based transmission for 480/960kHz SCS. |
| DOCOMO | We are fine with the proposal. |
| Nokia/NSB | Fine with the proposal for the progress |
| Lenovo, Motorola Mobility | Support the proposal |
| Qualcomm | We are okay with proposal |
| InterDigital | We support this proposal. |
| Futurewei | The Note does not seem necessary. |
| Qualcomm 2 | We prefer to have a unified design for UL and DL as well as across the SCSs For 480/960kHz, there is no need to support CBG retransmission for both UL and DL as the slot is too short, so the CBG fields will not be present in the DCI that schedule multi-PDSCH/PUSCH |
| Xiaomi | Agree with Samsung and Qualcomm |
| Panasonic | We are fine with the proposal #2.1-1. |
| OPPO | Support the proposal. |
| Fujitsu | We are fine with the proposal. |
| NEC | We are okay with proposal |
| vivo | We are fine with the proposal. |
| Huawei, HiSilicon | We do not see the necessity to configure CBG based transmission and multi-PUSCH scheduling at same time for 480kHz and 960kHz because of the short slot duration. The unified schemes for DL and UL are expected and it would be better to make agreement together with multi PDSCH scheduling. |
| Apple | Not support. Although the proposal seeks to have the same behavior for all SCSs as in Rel-16, we can would prefer to have the same behavior for PDSCH and PUSCH for 480 kHz and 960 kHz for Rel-17 especially given the short symbol/slot duration for 480 kHz and 960 kHz. |
| ZTE, Sanechips | For 480 and 960khz SCS, the slot duration is relatively short, so that the channel change will not be too large. Therefore, we understand that it is not necessary to support CBG transmission for 480 and 960khz SCS. |
| Intel | We are fine with the proposal. |
| Ericsson | We do not support Proposal 2.1-1. CBGs are questionable even for 120 kHz. We share the same view as some other companies that there are no benefits for CBG based (re)transmission for 480/960 kHz due to short symbol duration. All CBGs either pass or fail, hence there is no gain in re-transmission efficiency by selective re-transmission of specific CBGs. |
| Moderator | - Supported by NTT DOCOMO, Nokia, Lenovo, InterDigital, Futurewei, Panasonic, OPPO, Fujitsu, NEC, vivo, Intel - Objected by Samsung, Qualcomm, Xiaomi, Huawei, Apple, ZTE, Ericsson Let’s see companies’ views on the opposite direction. |

- [HIGH] Proposal #2.1-1a (CBG for UL):

- For a DCI that can schedule multiple PUSCHs for 480/960 kHz SCS, CBGTI field is not present in the DCI.

- [HIGH] Proposal #2.1-1a (CBG for UL) clean version:

- For a DCI that can schedule multiple PUSCHs for 480/960 kHz SCS, CBGTI field is not present in the DCI when one or more PUSCHs is scheduled.

Companies are encouraged to provide views on Proposal #2.1-1a.

| Company | Views |
| --- | --- |
| Ericsson | Support Proposal #2.1-1a We probably need to add a note saying that this applies for SCS 120 kHz and greater, since for multi-PUSCH for FR1 introduced in Rel-16, CBGs are supported. CBGs can make sense for FR1; however, for 120 kHz and greater for FR2, CBGs have no benefit. |
| Samsung | Support |
| OPPO | Support |
| CATT | Support |
| MediaTek | We support the proposal. Just a clarification question: if this proposal was agreed, can a UE be configured with CBG for UL? |
| Fujitsu | We are fine with the proposal. |
| Moderator | To Ericsson, We already agreed in RAN1#105-e that CBGTI can be present if a single PUSCH is scheduled for 120 kHz. For clarity, I added applicable SCS values. To MediaTek, Precisely, this proposal is limited to the case of multi-PUSCH scheduling. I think we need further discussion on whether CBG feature itself is supported or not for FR2-2 for UL. |
| WILUS | Support this Proposal @2.1-1a |
| Intel | For the sake of progress, we can compromise to support “CBGTI field is not present in the DCI” for 480/960kHz SCS. We prefer a unified solution for both DL and UL. |
| Ericcson | We still support the proposal, but with Moderator's clarification, wouldn't it be more clear to say the following to clarify that even when only single PUSCH is scheduled, the CBGTI field is not present? - For a DCI that can schedule multiple PUSCHs for 480/960 kHz SCS, CBGTI field is not present in the DCI when one or more PUSCHs is scheduled. |
| Panasonic | We support the proposal #2.1-1a. |
| Lenovo, Motorola Mobility | We support the proposal with Ericson’s updates |
| DOCOMO | We are fine with the proposal, also fine with Ericsson’s update. |
| Transsion | We support this Proposal and fine with Ericsson’s update. |
| vivo | We still prefer Proposal #2.1-1(not a). According to the above agreement, for 120 kHz SCS for multi-PUSCH scheduling, the mechanism agreed in NR-U Rel-16 is reused, it is undesirable to introduce different behaviours for different SCSes, which would increase the complexity of UE decoding. |
| Huawei,HiSilicon | To be more precisely, as for “a DCI that can schedule multiple PUSCHs”, does it mean TDRA table is configured at least one entry with multiple SLIV? If the answer is yes, we can support proposal 2.1-1a . |
| ZTE, Sanechips | We support the updated proposal from Ericsson |
| Futurewei | We supported the updated proposal. |
| Apple | We support the proposal and the update from Ericsson |
| Xiaomi | support the updated proposal from Ericsson |

- Issue 2.1-2) CBGTI/CBGFI field in multi-PDSCH scheduling DCI:

Working assumption: (RAN1#106bis-e)

UE does not expect to be configured with both of CBG operation and multi-PDSCH scheduling in the same PUCCH cell group with a Type 2 codebook.

- If time bundling operation is supported, this working assumption can be revisited

Company views on CBGTI/CBGFI field in multi-PDSCH scheduling DCI:

- Same behaviour as in Rel-16

  - Supported by Panasonic, Intel, InterDigital, NTT DOCOMO

- Do not support CBGTI/CBGFI field configuration for multi-PDSCH scheduling DCI for 120/480/960 kHz

  - Supported by vivo, Ericsson, Samsung, Apple, LG Electronics, MediaTek

[Moderator’s note] According to the slight majority view and working assumption as captured above, the following proposal can be made. This issue is indicated as “HIGH” since it is essential for WI completion.

- [HIGH] Proposal #2.1-2 (CBG for DL):

- If a UE configured with a DCI that can schedule multiple PDSCHs and is configured with the TDRA table containing at least one row with multiple SLIVs, for a serving cell,

  - The UE does not expect to be configured with CBG-based DL (re)transmission for the serving cell, and CBGTI and CBGFI fields are not present in the DCI.

Companies are encouraged to provide views on Proposal #2.1-2.

| Company | Views |
| --- | --- |
| Samsung | We are basically fine with the proposal. Since CBG-based PDSCH reception can be configured for a serving cell in FR1/FR2_1, the proposal can be revised as follows: - If a UE configured with a DCI that can schedule multiple PDSCHs and is configured with the TDRA table containing at least one row with multiple SLIVs, for a serving cell in FR2_2, - The UE does not expect to be configured with CBG-based DL (re)transmission for the serving cell, and CBGTI and CBGFI fields are not present in the DCI. |
| DOCOMO | We are fine with the proposal. |
| Nokia/NSB | Fine with the proposal. |
| Lenovo, Motorola Mobility | Support |
| Qualcomm | We are okay with the proposal |
| InterDigital | We support this proposal. |
| Futurewei | We are ok with the Proposal #2.1-2. |
| Qualcomm 2 | We prefer to have unified design for UL and DL as well as across the SCSs We propose to support CBG retransmission for single PDSCH of SCS 120kHz similar to the single PUSCH For 480/960kHz, there is no need to support CBG retransmission for both UL and DL as the slot is too short, so the CBG fields will not be present in the DCI that schedule multi-PDSCH/PUSCH |
| Xiaomi | Agree, and also think it is necessary to have the same agreement applied for Type 1 HARQ-ACK codebook |
| Panasonic | We are fine with the proposal #2.1-2. |
| OPPO | Support the proposal. |
| Fujitsu | We are generally fine with the proposal. But we prefer not to restrict the proposal to FR2-2. Multi-PDSCH scheduling seems also beneficial for other frequency range. If it is also supported for frequency range other than FR2-2 (may depend on UE feature discussion?), we think the restriction of configuration should also be applied. |
| NEC | We are fine with the proposal #2.1-2. |
| vivo | We generally support this proposal. However, we would like to clarify if this proposal applies to both type 1 and type 2 HARQ-ACK codebook or not. At least, from our side, the wording includes the both types, so, we think it is better to discuss this proposal until the conclusion from section 3.3 and 3.4 is made. |
| Huawei, HiSilicon | We are fine with proposal 2.1-2. It would be better to have a unified DL and UL behavior at least for 480kHz and 960kHz SCS. |
| Apple | We support this proposal. |
| ZTE, Sanechips | We are fine with this proposal. |
| Intel | We do not support this proposal. Same mechanism as for PUSCH scheduling should be applied for multi-PDSCH scheduling. We prefer a unified solution for both multi-PDSCH/PUSCH scheduling. |
| Ericsson | We support the direction of this proposal; however, there is a dependence on resolving the remaining issues of the below working assumption as discussed in Section 3.3. and 3.4 of this FL Summary. Hence, we think those issues should be resolved first before making an agreement on Proposal #2.1-2. If it is agreed that the UE does not expect to be configured with both of CBG operation and multi-PDSCH scheduling in the same PUCCH cell group regardless of time bundling and codebook type (our preference), then we can come back and finish this agreement more simply, e.g., something like the following: - If a UE configured with For a DCI that can schedule multiple PDSCHs and is configured with the TDRA table containing at least one row with multiple SLIVs, for a serving cell, - The UE does not expect to be configured with CBG-based DL (re)transmission for the serving cell, and CBGTI and CBGFI fields are not present in the DCI. Working assumption: (RAN1#106bis-e) UE does not expect to be configured with both of CBG operation and multi-PDSCH scheduling in the same PUCCH cell group with a Type 2 codebook. - If time bundling operation is supported, this working assumption can be revisited |
| Moderator | - Supported by Samsung, NTT DOCOMO, Nokia, Lenovo, Qualcomm, InterDigital, Futurewei, Xiaomi, Panasonic, OPPO, Fujitsu, NEC, vivo, Huawei, Apple, ZTE, Ericsson - Objected by Intel Based on several companies’ comments, it seems better to discuss CBG field and codebook issues at once. With this direction, the following proposal can be made. |

- [HIGH] Proposal #2.1-2a (CBG for DL):

- For a DCI that can schedule multiple PDSCHs, CBGTI and CBGFI fields are not present in the DCI.

- UE does not expect to be configured with both of CBG operation and multi-PDSCH scheduling in the same [PUCCH cell group or serving cell] with a Type 1 codebook.

- Confirm the working assumption from RAN1#106bis-e with the following modification.

Working assumption: (RAN1#106bis-e)

- UE does not expect to be configured with both of CBG operation and multi-PDSCH scheduling in the same PUCCH cell group with a Type 2 codebook.

  - If time bundling operation is supported, this working assumption can be revisited

- [HIGH] Proposal #2.1-2a (CBG for DL) clean version:

- For a DCI that can schedule multiple PDSCHs, CBGTI and CBGFI fields are not present in the DCI.

- UE does not expect to be configured with both of CBG operation and multi-PDSCH scheduling in the serving cell with a Type 1 codebook.

- Confirm the working assumption from RAN1#106bis-e with the following modification.

Working assumption: (RAN1#106bis-e)

- UE does not expect to be configured with both of CBG operation and multi-PDSCH scheduling in the same PUCCH cell group with a Type 2 codebook.

  - If time bundling operation is supported, this working assumption can be revisited

Companies are encouraged to provide views on Proposal #2.1-2a.

| Company | Views |
| --- | --- |
| Moderator | - The first bullet is simplified based on Ericsson’s comment. - For the second sub-bullet, if UE does not expect to be configured with both of CBG operation and multi-PDSCH scheduling in the same serving cell with a Type 1 codebook, then it is allowed that CBG for cell#1 and multi-PDSCH for cell#2, which does not affect type-1 HARQ-ACK codebook at all for which codebook construction is performed per cell basis. Please share your views on [PUCCH cell group or serving cell]. - For the third bullet, by deleting the sub-bullet of previous working assumption, regardless of whether time domain bundling (if supported) is configured or not, simultaneous CBG and multi-PDSCH configurations are not allowed. |
| Ericsson | We support Proposal 2.1-2a We think this is a simple and practical WF given the remaining time in the WI. Regarding the 2nd sub-bullet, what the moderator says makes sense and it seems like it would be sufficient to say "serving cell." This is based on our understanding that the Type-1 codebook is generated independently per serving cell, and then the bits for each serving cell are concatenated. In other words for Type-1 codebook construction, there is no dependence between serving cells. |
| Samsung | We are find with the proposal. Fort the 2nd sub-bullet, we support “serving cell”, because it can allow configured CBG operation for a serving cell and the multi-PDSCH scheduling in another serving cell and the type-1 CB is generated per serving cell. We would like to ask the motivation to have such restriction. |
| OPPO | We can accept the proposal for progress. For the second bullet, we also think “serving cell” is fine, as Type-1 codebook is constructed per cell. |
| CATT | Support the proposal. |
| MediaTek | We support the proposal with “serving cell” in the second bullet. |
| Fujitsu | We are fine with the proposal. For the 2nd bullet, we prefer “serving cell”. |
| WILUS | We support this Proposal #2.1.-2a with an addition of “serving cell” in the 2nd sub-bullet as mentioned by E/// and Samsung. |
| Intel | We prefer to have unified design for PDSCH/PUSCH for the same SCS. Since it is already agreed that CBGTI is applicable when single PUSCH is scheduled for SCS 120kHz, we prefer to allow CBGTI for single PDSCH scheduling for SCS 120kHz too. Regarding SCS 480/960kHz, for sake of progress, we can compromise to no CBGTI/CBGFI in the DCI. For the second bullet, as commented by Moderator, there is no impact on Type1 codebook generation is CBG or multi-PDSCH scheduling are configured in different cells in a PUCCH cell group. Therefore, we prefer to apply it per cell, i.e. [PUCCH cell group or serving cell]. We support to FL proposal in the third bullet. |
| Panasonic | We support the proposal #2.1-2a. Regarding [PUCCH cell group or serving cell], we support “serving cell” as it provides flexible scheduling among serving cells. |
| Lenovo, Motorola Mobilty | We support the proposal and are fine with adding “serving cell” in the 2nd bullet |
| DOCOMO | We are fine with the proposal. For the second bullet, we are fine with either “PUCCH cell group” or “serving cell”. |
| Transsion | We are fine with this proposal. Regarding the 2nd sub-bullet, we share the same views as Ericsson and Samsung that “serving cell” is sufficient. |
| Moderator | To Intel, Thanks a lot for being flexible. However, assuming the working assumption in the third bullet is agreed, how CBG can be configured with multi-PDSCH scheduling DCI even for 120 kHz SCS? Do I miss something? |
| vivo | Regarding the second bullet, in our opinion, it would be better to say “PUCCH cell group” which is consistent with type-2 codebook. While if majorities support “serving cell”, we can comprise. Regarding the first and third bullet, we are ok with it. |
| Huawei, HiSilicon | We support the proposal #2.1-2a if “a DCI that can schedule multiple PDSCHs” is interpret as “the TDRA table containing at least one row with multiple SLIVs”. We support to use “serving cell” for the 2nd bullet. |
| ZTE, Sanechips | For the 2nd bullet, we prefer “serving cell”. |
| Futurewei | Ok with the proposal and support “serving cell” for the second bullet. |
| Apple | We are fine with the proposal and “serving cell” is okay. |
| Xiaomi | Support the Proposal |

Frequency hopping

| Company | Views |
| --- | --- |
| [3] vivo | Proposal 10: For frequency hopping for multi-PUSCH scheduling, only intra-slot frequency hopping is applicable, and is applied to each scheduled PUSCH when configured and enabled, while inter-slot frequency hopping is inapplicable. |
| [11] Ericsson | Proposal 8: Support intra-slot frequency hopping for multi-PUSCH in Rel-17. |
| [12] Intel | Proposal 4 - For multi-PDSCH scheduling, - Up to 8 PDSCHs can be configured and scheduled for 2 TBs. - For multi-PUSCH scheduling, - Support intra-slot frequency hopping for scheduled PUSCHs. - Do not support enhancement on FDRA. |
| [13] Xiaomi | Proposal 5: Support to study intra-TTI frequency hopping and its enabling mechanism for multi-TTI scheduling. |
| [16] Samsung | Proposal 6: For multi-PUSCH scheduling DCI: - CBG: - Not support CBG-based transmission for single and multi-PUSCH scheduling for 480/960 KHz. - Not support CBG-based transmission for multi-PUSCH scheduling for 120KHz, but applicable for single-PUSCH scheduling for 120KHz. - Frequency hopping: Support intra-PUSCH hopping - FDRA: Support increased RBG size using the same mechanism introduced in Rel-16 URLLC |
| [17] InterDigital | Proposal 10: When multiple PUSCHs are scheduled using the same DCI, support only intra-slot frequency hopping |
| [18] Apple | Proposal 4: For Rel-17 multi-PUSCH transmission - The maximum number of PUSCHs that can be scheduled can be further restricted based on UE capabilities. - For 480 kHz and 960 kHz SCS, no support for CBGTI field configuration in the DCI that can schedule multiple PUSCHs - The FDRA size should be optimized to reduce the FDRA overhead. - Support inter-slot frequency hopping and NOT intra-slot frequency hopping for 480 kHz and 960 kHz |
| [20] NTT DOCOMO | Proposal 1: - For multi-PUSCH scheduled by single DCI, - CBG based scheduling is not supported when multiple PUSCHs are scheduled by one DCI. - Support frequency hopping for multi-PUSCH scheduling, by reusing existing PUSCH frequency hopping scheme, i.e. intra-slot PUSCH frequency hopping. - For multi-PDSCH scheduled by single DCI, - CBG based scheduling is not supported when multiple PDSCHs are scheduled by one DCI. - For two-TB scheduling, support limitation on the maximum number of PDSCHs when 2-TB is scheduled. When RRC parameter enables two TB scheduling, - If the number of scheduled PDSCHs is no larger than value X (X>1), two TBs can be scheduled for each PDSCH. - If the number of scheduled PDSCHs is larger than value X (X>1), only single TB can be scheduled for each PDSCH. |

- Summary (on frequency hopping):

Company views on frequency hopping enhancement:

- Intra-slot frequency hopping

  - Supported by: vivo, Ericsson, Intel, Samsung, InterDigital, NTT DOCOMO

- Inter-slot frequency hopping

  - Supported by: Apple

[Moderator’s note] Even though the clarification on frequency hopping for multi-PUSCH scheduling in Rel-16 has not yet been resolved, it seems to be a common sense that intra-slot frequency hopping is supported for multi-PUSCH scheduling in Rel-16. For Rel-17 multi-PUSCH scheduling case, a majority of companies suggested to support intra-slot PUSCH hopping, so the following proposal can be made. This issue is indicated as “HIGH” since it is essential for WI completion.

- [HIGH] Proposal #2.2 (FH):

- For multi-PUSCH scheduling DCI in Rel-17, support intra-slot frequency hopping which is applicable to each of multiple PUSCH transmissions scheduled by the DCI, and do not support inter-slot frequency hopping.

Companies are encouraged to provide views on Proposal #2.2.

| Company | Views |
| --- | --- |
| Samsung | We support the proposal |
| DOCOMO | We are fine with the proposal. |
| Nokia/NSB | Fine with the proposal. |
| Lenovo, Motorola Mobility | We think that inter-slot frequency hopping can also be supported for multi-PUSCH scheduling along with intra-slot frequency hopping. |
| InterDigital | We support this proposal. |
| Futurewei | We support Proposal #2.2. |
| Xiaomi | Support for the completion of this WI. but we do think inter-slot frequency hopping is beneficial. |
| Panasonic | Support the proposal #2.2 |
| OPPO | We are fine with the proposal. |
| Fujitsu | We are fine with the proposal. |
| vivo | We support proposal #2.2. |
| Huawei, HiSilicon | We are generally fine with the concept. To be more accurate, the definition of intra slot frequency hopping in Rel-16 is applied for PUSCH repetition type A where there is only one PUSCH in a slot. Considering the agreement in last meeting that only one PUSCH is scheduled in one slot for 480kHz/960kHz SCS, the calculation of OFDM symbols for each hop in a slot can be reused. However, for 120kHz SCS, there might be multiple PUSCH in a slot scheduled by single DCI. The NsymbPUSCH,s used to calculate number of OFDM symbols for each hop defined in TS38.214 should be clarified. Whether it is the number symbols for a PUSCH or for all PUSCHs in a slot scheduled by a single slot. Our understanding is the former one. More accurately, we should call it intra PUSCH frequency hopping. |
| Apple | We can agree to this proposal for the sake of progress |
| ZTE, Sanechips | We are fine with this proposal. |
| Intel | We are fine with the proposal. |
| Ericsson | Support Proposal #2.2 |
| Moderator | - Supported by: Samsung, NTT DOCOMO, Nokia, InterDigital, Futurewei, Panasonic, OPPO, Fujitsu, vivo, Huawei, Apple, ZTE, Intel, Ericsson - Also support inter-slot hopping: Lenovo, Xiaomi To Lenovo and Xiaomi, Given that clear majority support of this proposal, could you accept this proposal? Regarding inter-slot hopping, I doubt that it can be beneficial even when each PUSCH has individual TB, which is different from multi-slot PUSCH in Rel-15. To Huawei, I understand you point. However, intra-slot frequency hopping is the term that is used in 214 specification and “applicable to each of multiple PUSCH transmissions” in the proposal implies intra-PUSCH frequency hopping. With that, could you accept this proposal as it is? |
| WILUS | We support this proposal. |
| Lenovo, Motorola Mobility | For sake for progress, we can accept the proposal |
| Transsion | We are fine with the proposal. |
| Huawei, HiSilicon | Thanks for moderator’s the clarification. We support the proposal 2.2 |
| Xiaomi | Yes we can support this proposal. |

2-TB transmission

| Company | Views |
| --- | --- |
| [1] Huawei | Proposal 6: No limitation on the maximum number of PDSCHs when 2 TB is enabled or when 2 TB is scheduled. |
| [2] Futurewei | Proposal 5. No further restriction on the number of PDSCHs is needed when 2-TB is enabled or scheduled. |
| [3] vivo | Proposal 11: When two-codeword transmission is enabled, the maximum number of configured SLIVs in a row of TDRA table for multi-PDSCH scheduling can be further restricted. Proposal 12: Regarding TB disabling for multi-PDSCH scheduling, when two codeword transmission is configured, for a DCI format scheduling more than one PDSCH, a given TB can be disabled for each scheduled PDSCH individually, by setting IMCS = 26 and the 1-bit RV for a scheduled PDSCH to a predefined value, e.g. ‘1’, to indicated the given TB for the scheduled PDSCH is disabled. |
| [4] ZTE | Proposal 1: The maximum number of PDSCHs can be restricted as 2 or 4 when 2 TB is actually scheduled. |
| [6] Nokia | Proposal 3: UE can be scheduled for up-to 8 PDSCHs also when 2 TB is enabled or 2 TB is scheduled. |
| [7] CATT | Proposal 8: The gNB can implement limitation on maximum number of PDSCHs when 2 TB is enabled or when 2 TB is scheduled, and there is no further specification needed. |
| [11] Ericsson | Proposal 9: Do not introduce a constraint on the maximum number of PDSCHs that can be scheduled by a DCI when 2 TB is enabled or when 2 TB is scheduled. |
| [12] Intel | Proposal 4 - For multi-PDSCH scheduling, - Up to 8 PDSCHs can be configured and scheduled for 2 TBs. - For multi-PUSCH scheduling, - Support intra-slot frequency hopping for scheduled PUSCHs. - Do not support enhancement on FDRA. |
| [16] Samsung | Proposal 5: If two codeword transmission is supported and enabled for FR2-2, the maximum configured number of PDSCHs in a row of TDRA table in a DCI format scheduling multi-PDSCH is limited to 2. |
| [19] LG Electronics | Proposal #8: For NR FR2-2, if 2-TB is enabled, 2-TB transmission can be allowed only when a DCI schedules up to N (e.g., N=1) PDSCH(s). Proposal #9: Discuss how to disable one of 2 TBs if 2-TB is enabled and 2 TB transmission is allowed for the case where more than one PDSCH is scheduled by multi-PDSCH scheduling DCI. |
| [20] NTT DOCOMO | Proposal 1: - For multi-PUSCH scheduled by single DCI, - CBG based scheduling is not supported when multiple PUSCHs are scheduled by one DCI. - Support frequency hopping for multi-PUSCH scheduling, by reusing existing PUSCH frequency hopping scheme, i.e. intra-slot PUSCH frequency hopping. - For multi-PDSCH scheduled by single DCI, - CBG based scheduling is not supported when multiple PDSCHs are scheduled by one DCI. - For two-TB scheduling, support limitation on the maximum number of PDSCHs when 2-TB is scheduled. When RRC parameter enables two TB scheduling, - If the number of scheduled PDSCHs is no larger than value X (X>1), two TBs can be scheduled for each PDSCH. - If the number of scheduled PDSCHs is larger than value X (X>1), only single TB can be scheduled for each PDSCH. |
| [21] Qualcomm | Proposal 16: To indicate that the second TB is disabled for a certain DCI that schedules multiple PDSCHs, use a combination of MCS and rvid such that rvid bit of PDCSH i-1 is the complement of the one of PDSCH i for i=1 : number of maximum PDSCHs -1. Proposal 17: Do not introduce additional limitations on the maximum number of PDSCHs when 2 TB is enabled or when 2 TB is scheduled. |

- Issue 2.3-1) Whether or not to restrict the maximum number of PDSCHs when 2 TB is enabled or scheduled:

Agreement: (RAN1#106bis-e)

For a DCI that can schedule multiple PDSCHs, and if RRC parameter configures that two codeword transmission is enabled,

- MCS for the 2nd TB: This appears only once in the DCI and applies commonly to the 2nd TB of each PDSCH

- NDI for the 2nd TB: This is signaled per PDSCH and applies to the 2nd TB of each PDSCH

- RV for the 2nd TB: This is signaled per PDSCH, with 2 bits if only a single PDSCH is scheduled or 1 bit for each PDSCH otherwise and applies to the 2nd TB of each PDSCH

- FFS: the maximum number of PDSCHs when 2 TB is enabled or when 2 TB is scheduled

Company views on whether or not to restrict the maximum number of PDSCHs when 2 TB is enabled or scheduled:

- No restriction

  - Supported by Huawei, Futurewei, Nokia, CATT, Ericsson, Intel, Qualcomm

- Up to X SLIVs can be configured for multi-PDSCH scheduling DCI

  - Supported by vivo, Samsung (X=2)

- 2-TB can be scheduled only if up to N PDSCHs are scheduled, but 1-TB is scheduled otherwise

  - Supported by ZTE (N=2 or 4), LG Electronics, NTT DOCOMO

[Moderator’s note] Considering the majority view, the following proposal can be made. This issue is indicated as “HIGH” since it is essential for WI completion.

- [HIGH] Proposal #2.3-1 (2-TB TX):

- The maximum number of PDSCHs/PUSCHs that can be scheduled with a single DCI in Rel-17 is also 8 when 2 TB is enabled or when 2 TB is scheduled, for SCS of 120, 480 and 960 kHz.

  - Note: This is to handle FFS (the maximum number of PDSCHs when 2 TB is enabled or when 2 TB is scheduled) in previous agreement in RAN1#106bis-e.

Companies are encouraged to provide views on Proposal #2.3-1.

| Company | Views |
| --- | --- |
| Samsung | Not support. We still see no use cases with 2-TB transmission and up to 8 PDSCH scheduling because of the unacceptable DCI overhead increase (i.e., 21 bits in a DCI) |
| DOCOMO | Though we prefer to limit maximum number of PDSCHs for 2-TB scheduling, we can compromise for progress. |
| Nokia/NSB | Fine with the proposal. |
| Lenovo, Motorola Mobility | Support the proposal |
| Qualcomm | We support the proposal |
| InterDigital | We support this proposal. |
| Futurewei | Ok with the Proposal #2.3-1. |
| Xiaomi | We think the DCI size issue raised by Samsung needs to be discussed. We would like to hear companies views about it. |
| Fujitsu | We are fine with the proposal. |
| vivo | We prefer to restrict the maximum number of PDSCHs/PUSCHs when 2-TB transmission is enabled to avoid excessive DCI overhead. |
| Huawei, HiSilicon | We support the proposal |
| Apple | We are fine with the proposal. |
| ZTE, Sanechips | We are fine with the proposal. |
| Intel | We are fine with the proposal. |
| Ericsson | Support Proposal #2.3-1 |
| CATT | We are fine with the proposal. |
| Moderator | - Support: NTT DOCOMO, Nokia, Lenovo, Qualcomm, InterDigital, Futurewei, Fujitsu, Huawei, Apple, ZTE, Intel, Ericsson,CATT - Do not support: Samsung, Xiaomi, vivi To Samsung, Xiaomi, and vivo Given that situation, would it be possible to compromise to the proposal? I think the DCI size can be controlled by gNB’s configuration for TDRA table if DCI overhead can lead to any problem (e.g., robustness of DCI reception). |
| Samsung | It is not a good practice in RAN1. When we made the working assumption on 2TB transmission, we compromised the proposal with the explicit sentence “Strive to minimize the increase in the number of bits in the DCI needed to support this feature”. Now, RAN1 does not take into account the spirit of the sentence. Even though gNB may configure the DCI size properly, why do we introduce un-usable configurations in RAN1 specifications? So far, we don’t see valid use cases with up to 8 PDSCHs and 2 TB transmission. For a potential compromise, we suggest to consider the following proposal and hear the company’s views on that. - The maximum number of PDSCHs/PUSCHs that can be scheduled with a single DCI in Rel-17 is also 84 when 2 TB is enabled or when 2 TB is scheduled, for SCS of 120, 480 and 960 kHz. - Note: This is to handle FFS (the maximum number of PDSCHs when 2 TB is enabled or when 2 TB is scheduled) in previous agreement in RAN1#106bis-e. The intention of this compromise is to limit up to 4 SLIVs in a row of TDRA table when 2 TB is enabled. This is aligned with the spirit of the working assumption and can minimize the specification impacts and the additional overhead (5 bits additionally required). |
| Transsion | We are fine with the proposal. |
| vivo | We support Samsung’s updated proposal. |
| Xiaomi | We agree with Moderator that gNB can control the DCI size. but we also thine the DCI size for 2TB should not be bigger than 1 TB by a larger margin. We can go with majority on this issue, but also think Samsung’s version is better. |

- Issue 2.3-2) TB-disabling mechanism:

[Moderator’s note] Several companies (vivo, LG Electronics, and Qualcomm) brought up the issue on how to disable a TB when bit-width of RV field corresponding to a PDSCH is one and the RV field cannot indicate RV index 1. In Rel-15 NR, one TB between two TBs can be disabled when MCS=26 & RV index=1. To resolve this issue, the following three alternatives are identified:

- Alt 1 in [3]: For the M bits for indicating RV information for a given TB in a DCI format scheduling more than one PDSCH, a predefined value vector can be used to indicate the given TB is disabled for all PDSCHs scheduled by the DCI format, e.g. assuming M = 8, the 8-bit RV corresponding to a TB in the DCI format can be set to ‘11111111’ to indicate the TB is disabled for all PDSCHs scheduled by the DCI format.

- Alt 2 in [3]: For each PDSCH scheduled by a DCI format scheduling more than one PDSCH, a given TB can be disabled individually, i.e. a 1-bit RV for the given TB and for a PDSCH scheduled by the DCI format can be set to a predefined value, e.g. ‘1’, to indicate the given TB of the PDSCH is disabled.

- Alt 3 in [21]: To indicate that the second TB is disabled for a certain DCI that schedules multiple PDSCHs, use a combination of MCS and rvid such that rvid bit of PDCSH i-1 is the complement of the one of PDSCH i for i=1 : number of maximum PDSCHs -1

Companies are encouraged to provide views on the above three alternatives (or any other alternatives) to disable a TB.

| Company | Views |
| --- | --- |
| Qualcomm | Alt 1 and Alt 2 will prevent gNB from using different rv for the retransmission of PDSCHs with MCS 26, while with Alt 3 the rv vector to be used to disable the second TB is not likely to be useful for retransmission. |
| vivo | Alt. 1 disables a TB on a per-DCI basis, while Alt. 2 disables a TB on a per-PDSCH basis. Since NDI and RV are indicated for each scheduled PDSCH individually, it may also be beneficial to perform TB disabling for each scheduled PDSCH individually, due to that each scheduled PDSCH correspond to different traffic. From the perspective of flexibility, Alt. 2 is slightly preferred. |
| Intel | We think simple extension of existing mechanism is desirable, i.e. MCS =26 and RV index = a specific value for all PDSCHs for 2nd TB. We understand the additional flexibility provided by Alt 3, compared with using MCS=26 and ‘1’ value for all PDSCHs, but, the practical gain would be limited. Considering this is the last meeting, it would be desirable to have a simple solution. We suggest Alt. 1, but would like to add that a combination MCS=26 and ‘1’ for RV bit fields for all PDSCHs associated with the TB is used to indicate the second TB is disabled |

[Closed] FDRA enhancement

| Company | Views |
| --- | --- |
| [3] vivo | Proposal 5: Legacy frequency domain scheduling in NR Rel-15/16 is reused for multi-PUSCH/PDSCH scheduling. |
| [10] Panasonic | Proposal 4: No need to have the optimization of FDRA size. |
| [11] Ericsson | Proposal 4: Introduce new RBG configuration for PDSCH/PUSCH frequency resource allocation Type 0 to reduce FDRA granularity and DCI size. Proposal 5: Support configurable Resource Allocation Granularity (P) up to 32 for DCI Format 0_1 and 1_1 with PUSCH/PDSCH frequency resource allocation Type 1 to reduce FDRA granularity and DCI size. |
| [12] Intel | Proposal 4 - For multi-PDSCH scheduling, - Up to 8 PDSCHs can be configured and scheduled for 2 TBs. - For multi-PUSCH scheduling, - Support intra-slot frequency hopping for scheduled PUSCHs. - Do not support enhancement on FDRA. |
| [13] Xiaomi | Observation 1: The current DCI 0-2/1-2 can be reused to allow frequency domain resource by multi-PRB granularity. |
| [16] Samsung | Proposal 6: For multi-PUSCH scheduling DCI: - CBG: - Not support CBG-based transmission for single and multi-PUSCH scheduling for 480/960 KHz. - Not support CBG-based transmission for multi-PUSCH scheduling for 120KHz, but applicable for single-PUSCH scheduling for 120KHz. - Frequency hopping: Support intra-PUSCH hopping - FDRA: Support increased RBG size using the same mechanism introduced in Rel-16 URLLC |
| [17] InterDigital | Observation 3: It is observed that required payloads of DCI for frequency domain resource allocation do not increase as maximum number of RBs does not increase. Observation 4: Larger RB size reduces frequency domain resource allocation flexibility, and this may be a crucial disadvantage as higher SCSs occupies larger bandwidths than lower SCSs with the same RBG size. Proposal 11: The benefits from frequency domain resource allocation enhancements should be carefully evaluated. |
| [18] Apple | Proposal 4: For Rel-17 multi-PUSCH transmission - The maximum number of PUSCHs that can be scheduled can be further restricted based on UE capabilities. - For 480 kHz and 960 kHz SCS, no support for CBGTI field configuration in the DCI that can schedule multiple PUSCHs - The FDRA size should be optimized to reduce the FDRA overhead. - Support inter-slot frequency hopping and NOT intra-slot frequency hopping for 480 kHz and 960 kHz Proposal 5: For Rel-17 multi-PDSCH transmission - The maximum number of PDSCHs that can be scheduled can be further restricted based on UE capabilities. - For a DCI that can schedule multiple PDSCHs and is configured with the TDRA table containing at least one row with multiple SLIVs, do not support/configure CBGTI/CBGFI fields - The FDRA size should be optimized to reduce the FDRA overhead. - Support inter-slot frequency hopping and NOT intra-slot frequency hopping for 480 kHz and 960 kHz |

- Summary on FDRA enhancement:

Company views on FDRA enhancement:

- Same as in Rel-16 (i.e., no enhancement): vivo, Panasonic, Intel

- FDRA field enhancement to reduce DCI overhead

  - Supported by Ericsson, Samsung, Apple

[Moderator’s note] Given the low interest from companies, we can conclude that FDRA enhancement is deprioritized in this release. This issue is indicated as “HIGH” since it is essential for WI completion.

[HIGH] Proposed conclusion #2.4 (FDRA):

- For multi-PDSCH or multi-PUSCH scheduling DCI, FDRA enhancement is deprioritized in Rel-17.

Companies are encouraged to provide views on Proposed conclusion #2.4.

| Company | Views |
| --- | --- |
| DOCOMO | We are fine with the proposal. |
| Nokia/NSB | Fine with the conclusion. |
| Lenovo, Motorola Mobility | Support the conclusion |
| Qualcomm | We are fine with proposal |
| InterDigital | We support deprioritizing this topic. |
| Futurewei | Agree with the Proposed conclusion #2.4. |
| Xiaomi | agree |
| Panasonic | We are fine with the conclusion. |
| OPPO | Agree. |
| Fujitsu | We support the conclusion. |
| vivo | Support this conclusion #2.4. |
| Huawei, HiSilicon | We are fine with the proposal |
| Apple | We can accept the conclusion for the sake of progress in the WI. |
| ZTE, Sanechips | We are fine with the proposal |
| Intel | We are fine to deprioritize this issue. |
| Ericsson | For the sake of progress we can compromise and support Conclusion #2.4. There are enough other issues to solve. |
| CATT | We are fine with the proposal |
| Moderator | Seems stable. |

During email discussion, the following agreement was made:

- Agreement:

- For multi-PDSCH or multi-PUSCH scheduling DCI, FDRA enhancement is deprioritized in Rel-17.

TDRA enhancement

| Company | Views |
| --- | --- |
| [11] Ericsson | Proposal 3: If the UE is configured with a TDRA table in which one or more rows contains multiple SLIVs, the UE is not expected to be configured with legacy single TRP PDSCH/PUSCH repetition. Legacy single-TRP repetition refers to either Rel-15 repetition through configuration of pdsch-AggregationFactor / pusch-AggregationFactor, or Rel-16 repetition through configuration of repetitionNumber / numberOfRepetitions within the TDRA table. |
| [21] Qualcomm | Proposal 11: Support the ability to schedule a single TB to be repeated over multiple allocations and multiple TBs, with no repetitions, using the same DCI format. |

- Summary on TDRA enhancement:

Company views on TDRA-related issues for multi-PDSCH/PUSCH scheduling:

- A DCI format that is configured with a TDRA table containing at least one row with multiple SLIVs, can schedule PDSCH/PUSCH repetition schemes (which are supported from Rel-15 or Rel-16) by using different rows in the TDRA table

  - Supported by Qualcomm

  - Objected by Ericsson

[Moderator’s note] Although the number of inputs is quite small, this issue needs to be figured out. Ericsson’s approach seems straight-forward and aligned with Rel-16 multi-PUSCH DCI design. With this regard, the following proposal can be made. This issue is indicated as “HIGH” since it is related to RRC parameter discussion.

- [HIGH] Proposal #2.5 (TDRA):

- If a UE is configured with a TDRA table in which one or more rows contain multiple SLIVs, the UE does not expect to be configured with pdsch-AggregationFactor or repetitionNumber for PDSCH, and with pusch-AggregationFactor or numberOfRepetitions for PUSCH.

  - Note: Under agenda item 8.2.4, in RAN1#106-bis, it was already agreed that within the TDRA table for multi-PDSCH scheduling, the UE does not expect to be configured with the higher layer parameter repetitionNumber.

Companies are encouraged to provide views on Proposal#2.5.

| Company | Views |
| --- | --- |
| Samsung | As a consequence of this proposal, PDSCH repetition (or PUSCH repetition) cannot be scheduled by DCI format 1_2 (or 0_2). But other option is that DCI format 1_2 (or 0_2) can be used to schedule a single PDSCH (or PUSCH) with repetition by pdsch-AggregationFactor (or pusch-AggregationFactor) if configured. If the intention of the proposal is to preclude PDSCH repetition by DCI format 1_2 if multi-PDSCH scheduling is configured, then we are ok with the proposal. |
| DOCOMO | We are fine with the proposal. |
| Nokia/NSB | Fine with the proposal. |
| Lenovo, Motorola Mobility | Support the proposal |
| InterDigital | We are fine with this proposal. |
| Futurewei | Ok with the proposal, but it is better to clarify for the case when a single PxSCH is scheduled despite that UE is configured a TDRA table in which one or more rows contain multiple SLIVs. |
| Xiaomi | Agree with Samsung. UE should still be able to schedule single PUSCH with repetition when UE is configured with a TDRA table in which one or more rows contain multiple SLIVs. |
| Panasonic | We are fine with the proposal. |
| OPPO | Support the proposal. |
| Fujitsu | We share the same view with Samsung. In addition, according to the main bullet, it seems that once a TDRA table for either DL or UL is configured like that, both DL and UL are not expected to be configured with the RRC parameters. We guess that is not the intention. Assuming the understanding is correct, we suggest clarifying e.g. that the TDRA table and the RRC parameters are for same DCI. |
| NEC | Share the same view with Futurewei |
| vivo | We are not totally OK with proposal #2.5, when a row configured with only one SLIV is scheduled, pdsch-AggregationFactor/ pusch-AggregationFactor can be applied to the single scheduled PDSCH/PUSCH to achieve higher reliability. |
| Huawei, HiSilicon | Fine with the proposal |
| Apple | We are okay with the proposal for multi-PxSCH transmission scheduled with DCI Format x_1, but as mentioned, (1) the behavior for DCI Formats x_0 and x_2 and (2) the behavior for single PxSCH should be clarified to make sure that this is the behavior we want. |
| ZTE, Sanechips | We support this proposal. |
| Intel | We are fine with the proposal. |
| Ericsson | Support Proposal #2.5 |
| Moderator | First of all, let me clarify the relevant procedure in Rel-16. - If pusch-TimeDomainAllocationListForMultiPUSCH is configured, - TDRA table in pusch-TimeDomainAllocationListForMultiPUSCH cannot be simultaneously configured with pusch-AggregationFactor or with numberOfRepetitions. Thus, DCI format 0_1 does not support any repetition mechanism - For DCI format 0_0, default TDRA table is applied so any repetition mechanism is not applied. - For DCI format 0_2, pusch-TimeDomainAllocationListDCI-0-2 can be separately configured and numberOfRepetitions can be configured, which means PUSCH repetition type B can be scheduled by DCI format 0_2. My original intention was to follow Rel-16 principle. If the above is the correct understanding (please instruct me if I’m wrong. ), Proposal #2.5 can be modified as follows. |

- [HIGH] Proposal #2.5a (TDRA):

- If a UE is configured with a TDRA table in which one or more rows contain multiple SLIVs for PDSCH, the UE does not expect to be configured with pdsch-AggregationFactor in PDSCH-config or repetitionNumber for the TDRA table.

  - Note: These does not preclude pdsch-AggregationFactor or repetitionNumber is configured for TDRA table corresponding to DCI format 1_2

  - Note: Under agenda item 8.2.4, in RAN1#106-bis, it was already agreed that within the TDRA table for multi-PDSCH scheduling, the UE does not expect to be configured with the higher layer parameter repetitionNumber.

- If a UE is configured with a TDRA table in which one or more rows contain multiple SLIVs for PUSCH, the UE does not expect to be configured with pusch-AggregationFactor in PUSCH-config or numberOfRepetitions for the TDRA table.

  - Note: These does not preclude pusch-AggregationFactor or numberOfRepetitions is configured for TDRA table corresponding to DCI format 0_2

- [HIGH] Proposal #2.5a (TDRA) clean version:

- If a UE is configured with a TDRA table in which one or more rows contain multiple SLIVs for PDSCH, the UE does not expect to be configured with pdsch-AggregationFactor in PDSCH-config or repetitionNumber for the TDRA table.

  - Note: These does not preclude repetitionNumber is configured for TDRA table corresponding to DCI format 1_2

  - Note: Under agenda item 8.2.4, in RAN1#106-bis, it was already agreed that within the TDRA table for multi-PDSCH scheduling, the UE does not expect to be configured with the higher layer parameter repetitionNumber.

- If a UE is configured with a TDRA table in which one or more rows contain multiple SLIVs for PUSCH, the UE does not expect to be configured with pusch-AggregationFactor in PUSCH-config or numberOfRepetitions for the TDRA table.

  - Note: These does not preclude numberOfRepetitions is configured for TDRA table corresponding to DCI format 0_2

Companies are encouraged to provide views on Proposal#2.5a.

| Company | Views |
| --- | --- |
| Ericsson | Support Proposal #2.5a |
| Samsung | The wording is not clear to us In Rel-15, pdsch-AggregationFactor can be configured in PDSCH-Config and in Rel-16 pdsch-AggregationFactor can be also configured in SPS-Config. If the intention is to avoid repetition for a PDSCH scheduled by DCI 1_1, we would like to suggest the wording “pdsch-AggregationFactor configured in PDSCH-Config does not apply”. For UL, in Rel-15, pusch-AggregationFactor is configured in PUSCH-Config instead of in the TDRA table. |
| OPPO | Fine with the proposal. |
| Fujitsu | We are fine with the proposal. |
| Moderator | Based on Samsung’s comment, further modification is applied, as highlighted in yellow, considering Rel-15 slot aggregation is configured in PDSCH-config or PUSCH-config, not in the TDRA table. |
| Intel | We are fine with the proposal in principle. For the note, does that mean for DCI format 1_2 or 0_2, when pdsch-AggregationFactor or repetitionNumber is configured, this is only applied for single PDSCH/PUSCH case? It may be good to clarify. |
| Ericsson | Support Proposal #2.5a Minor correction: PDSCH-config in 2nd main bullet should be PUSCH-Config. |
| DOCOMO | We don’t support the proposal. Based on above comments, we prefer Samsung’s proposal that “pdsch-AggregationFactor configured in PDSCH-Config does not apply”. With wording in current proposal, “DCI 1_2 scheduled PDSCH repetition based on pdsch-AggregationFactor” is also precluded, if multiple SLIVs configured for the DCI 1_1 TDRA table. It is too restricted. We think it’s better to allow pdsch-AggregationFactor configured in PDSCH-Config, but it only applies to DCI 1_2 and does not apply to DCI 1_1. BTW. There is typo in the second bullet: it should be PUSCH-Config. |
| Transsion | We are fine with this Proposal. |
| Moderator | Typo in the second bullet is fixed. Thank you all for spotting this error. To Intel, With the note, UE CANNOT be configured with multi-PDSCH scheduling in DCI format 1_1 and pdsch-AggregationFactor, at the same time. Rather, UE CAN be configured with multi-PDSCH scheduling in DCI format 1_1 and repetitionNumber in DCI format 1_2, at the same time. Also, DCI format 0_2 or 1_2 can only schedule a single PUSCH or PDSCH. To NTT DOCOMO, This is the same principle in Rel-16, as I captured 331 specification below. |
| vivo | We still prefer that pdsch-AggregationFactor/ pusch-AggregationFactor apply when “only one SLIV in a row”. With relaxing this restriction, the gNB scheduling is more flexible and the higher reliability can be achieved with single PDSCH scheduling. While for the sake of process, if majority companies support Proposal #2.5a, we can accept the it following the Rel-16 principle as a compromise. |
| Huawei, HiSilicon | Support the proposal #2.5a. |
| ZTE, Sanechips | We are fine with proposal #2.5a |
| DOCOMO | Thanks for moderator’s clarification. We still prefer more flexible scheduling for DCI 1_2. In our understanding, such limitation was introduced in Rel-16 NR-U, while DCI 1_2 was introduced in Rel-16 URLLC in parallel. Not sure whether the Rel-16 NR-U colleagues are intended to impose the limitation also on DCI 1_2. It would be more flexible if pdsch-AggregationFactor can be configured but only applied to DCI 1_2. However, if majority companies hope to have the limitation that pdsch-AggregationFactor not configured if multi-PDSCH TDRA table is configured for DCI 1_1. |
| Futurewei | We supported the Proposal #2.5a. |
| Apple | We are fine with the updated proposal. |
| Xiaomi | Fine with the proposal if it is majority’s view. |

Out-of-order handling

| Company | Views |
| --- | --- |
| [1] Huawei | Proposal 7: Legacy specification could be used with minimum change if regarding the first valid PDSCH/PUSCH of multiple PDSCH/PUSCH scheduling by single DCI as the “first PDSCH/PUSCH” in current specification for OOO scheduling restriction. |
| [2] Futurewei | Proposal 7. Regarding data, consider allowing for SCS 480/960kHz the out-of-order scheduling for the case of one multi-PxSCH scheduling DCI and one single-PxSCH scheduling DCI, where multi-PxSCH scheduling DCI schedules more than one PxSCH, i.e., the single PxSCH scheduled by its DCI starting later than the multi-PxSCH scheduling DCI can be scheduled before the last PxSCH of the multi-PxSCH. Proposal 8. Regarding HARQ, only consider allowing for SCS 480kHz/960kHz the out-of-order scheduling for a PDSCH scheduled by multi-PDSCH scheduling DCI and other unicast PDSCH scheduled by single-PDSCH scheduling DCI if the range of k1 is to be notably extended. Proposal 9. Consider allowing OOO scheduling for SCS 480/960kHz for the case where two multi-PxSCH scheduling DCIs end in the same symbol but two multi-PxSCH scheduling DCIs have overlapping spans. |
| [3] vivo | Proposal 9: For multi-PDSCH scheduling, UE does not expect any of the scheduled/SPS PDSCHs and the resource for the HARQ-ACK transmission to lead to out-of-order scheduling. |
| [4] ZTE | Proposal 2: It is not recommended to introduce additional specification impact to handle the two cases listed in FFS except existing OOO rule. |
| [7] CATT | Proposal 6: For scheduling multi-PDSCH (or multi-PUSCH), the following two OOO cases are not supported. - for the case of one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, where multi-PDSCH (or multi-PUSCH) scheduling DCI schedules more than one PDSCH (or PUSCH) - for the case where two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but two multi-PDSCH (or multi-PUSCH) scheduling DCIs have overlapping spans, where the span is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV |
| [9] OPPO | Proposal 1: Support scheduling of the following case: - One multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, where multi-PDSCH (or multi-PUSCH) scheduling DCI schedules more than one PDSCH (or PUSCH), and the single-PDSCH (or single-PUSCH) is transmitted at least later than the first PDSCH (or PUSCH) of the multi-PDSCH (or multi-PUSCH). Proposal 2: Do not specify the following case: - Two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but two multi-PDSCH (or multi-PUSCH) scheduling DCIs have overlapping spans, where the span is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV. |
| [10] Panasonic | Proposal 5: For the case of one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, support the single-PDSCH (or single-PUSCH) scheduling DCI having higher priority than the multi-PDSCH (or multi-PUSCH) scheduling DCI - FFS on details of scheduling and out-of-order handling. Proposal 6: For the case where two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol, but two multi-PDSCH (or multi-PUSCH) scheduling DCIs have overlapping spans, UE does not expect any of the scheduled PDSCHs (or PUSCHs) to lead to out-of-order scheduling. Proposal 7: For multi-PDSCH scheduling, UE does not expect any of the scheduled/SPS PDSCHs and the resource for the HARQ-ACK transmission lead to out-of-order scheduling. |
| [11] Ericsson | Proposal 11: Out-of-Order scheduling is allowed for the case of one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, where multi-PDSCH (or multi-PUSCH) scheduling DCI schedules more than one PDSCH (or PUSCH). Proposal 12: Out-of-Order scheduling is NOT allowed for the case where two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but two multi-PDSCH (or multi-PUSCH) scheduling DCIs have overlapping spans, where the span is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV. |
| [12] Intel | Figure 2 Figure 3 Proposal 5 - For two PDCCHs and the associated PDSCH(s) and PUSCH(s), if at least one PDCCH is scheduling multiple PDSCHs/PUSCHs, referring to Figure 2, - Case A/B/C/D are invalid; - The existing specification should be updated to reflect that Case A/C are invalid - For two PDSCHs and associated PUCCH, if at least one PDSCH is of multiple PDSCHs/PUSCHs, referring to Figure 3, - Case F/G are valid, and Case H/I is invalid. - The existing specification is sufficient for cases F/G/H/I. |
| [15] NEC | Proposal 1: Allow the scheduling case where two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but scheduled PDSCHs (or PUSCHs) are interlaced |
| [16] Samsung | Proposal 11: For single PDSCH (or PUSCH) scheduling DCIs and multi-PDSCH (or multi-PUSCH) scheduling DCIs, UE does not expect any of the scheduled PDSCHs (or PUSCHs) and the scheduling DCIs to lead to out-of-order scheduling. Observation 1: SPS PDSCH reception has large scheduling restriction on multi-PDSCH scheduling. Proposal 12: UE is not expected to receive a SPS PDSCH if the SPS PDSCH is configured to be received between a PDCCH with a DCI scheduling multiple PDSCHs and the last PDSCH scheduled by the DCI. |
| [18] Apple | Proposal 7: To simplify UE implementation, we propose that for the DCI-to-data out of order issue, the UE does not expect any out-of-order scheduling for the following cases: - for the case of one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, where multi-PDSCH (or multi-PUSCH) scheduling DCI schedules more than one PDSCH (or PUSCH). - for the case where two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but two multi-PDSCH (or multi-PUSCH) scheduling DCIs have overlapping spans, where the span is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV Proposal 8: For the PDSCH-to-HARQ-ACK out-of-order issue, - for multi-PDSCH scheduling, the UE does not expect any of the scheduled/SPS PDSCHs and the resource for the HARQ-ACK transmission to lead to out-of-order scheduling in the case of a PDSCH scheduled by multi-PDSCH scheduling DCI and other unicast PDSCH scheduled by single-PDSCH scheduling DCI |
| [19] LG Electronics | Proposal #10: For one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, UE does not expect any of the scheduled PDSCHs (or PUSCHs) and the scheduling DCI to lead to out-of-order scheduling. Proposal #11: For the case where two DCIs end in the same symbol but two DCIs have overlapping spans, where the span is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV, UE drops the PDSCHs scheduled by one of the two DCIs in the overlapping duration. |
| [20] NTT DOCOMO | Proposal 2: The following two cases are OoO scheduling, and should not be allowed: - the case of one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, where multi-PDSCH (or multi-PUSCH) scheduling DCI schedules more than one PDSCH (or PUSCH). - the case where two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but two multi-PDSCH (or multi-PUSCH) scheduling DCIs have overlapping spans, where the span is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV. |
| [21] Qualcomm | Proposal 19: The UE does not expect to be scheduled with two DCIs that schedule DL (UL) data allocations with overlapping spans, where the span of the allocations scheduled by one DCI is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV by the same DCI. |
| [22] MediaTek | Proposal 6: For the case of one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, UE doesn’t expect any of the scheduled PDSCHs(or PUSCHs) and the scheduling DCI lead to out-of-order scheduling. Proposal 7: For multi-PDSCH scheduling, UE doesn’t expect any of the scheduled PDSCHs and the resource for the HARQ-ACK transmission lead to out-of-order scheduling. |

- Issue 2.6-1) DCI-to-data out-of-order issue:

TS 38.214 For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start receiving a first PDSCH starting in symbol j by a PDCCH ending in symbol i, the UE is not expected to be scheduled to receive a PDSCH starting earlier than the end of the first PDSCH with a PDCCH that ends later than symbol i. … For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start a first PUSCH transmission starting in symbol j by a PDCCH ending in symbol i, the UE is not expected to be scheduled to transmit a PUSCH starting earlier than the end of the first PUSCH by a PDCCH that ends later than symbol i.

Agreement: (RAN1#106bis-e)

For two multi-PDSCH (or two multi-PUSCH) scheduling DCIs, UE does not expect any of the scheduled PDSCHs (or PUSCHs) and the scheduling DCI to lead to out-of-order scheduling.

- FFS: whether to allow OOO scheduling for the following two cases:

  - for the case of one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, where multi-PDSCH (or multi-PUSCH) scheduling DCI schedules more than one PDSCH (or PUSCH)

  - for the case where two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but two multi-PDSCH (or multi-PUSCH) scheduling DCIs have overlapping spans, where the span is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV

- Note: The above FFS aspect applies only to multi-PDSCH and multi-PUSCH scheduling with single DCI

Company views on DCI-to-data out-of-order issue:

- For the first cast of above highlighted FFS,

  - Can be regarded as OOO scheduling: vivo, ZTE, CATT, Samsung, Apple, LG Electronics, NTT DOCOMO, Qualcomm, MediaTek

  - Can be allowed: Huawei, Futurewei, OPPO (if single PXSCH DCI is transmitted later than the first PXSCH scheduled by multi-PXSCH DCI), Panasonic?, Ericsson

- For the second cast of above highlighted FFS,

  - Can be regarded as OOO scheduling: CATT, Panasonic, Ericsson, Intel, Apple, LG Electronics, NTT DOCOMO, Qualcomm

  - Can be allowed: Huawei, Futurewei, ZTE, OPPO, NEC

[Moderator’s note] Considering the majority view, the following proposal can be made.

- Proposal #2.6-1 (DCI-to-data OOO):

- UE does not expect any of the scheduled PDSCHs (or PUSCHs) and the scheduling DCI to lead to out-of-order scheduling, also for the case of one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, where multi-PDSCH (or multi-PUSCH) scheduling DCI schedules more than one PDSCH (or PUSCH)

- The case where two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but two multi-PDSCH (or multi-PUSCH) scheduling DCIs have overlapping spans, where the span is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV, is considered as out-of-order scheduling and is not expected by UE.

Companies are encouraged to provide views on Proposal #2.6-1.

| Company | Views |
| --- | --- |
| DOCOMO | We are fine with the proposal. |
| Nokia/NSB | Fine with the proposal. |
| Lenovo, Motorola Mobility | Support the proposal |
| Qualcomm | We support the proposal |
| InterDigital | We are fine with this proposal. |
| Futurewei | We can accept the proposal, although we think that delay can be a concern given the extended k0, k2 if these cases are considered OOO. |
| Panasonic | For the case of one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, we see the merit so that we support the single-PDSCH (or single-PUSCH) scheduling DCI having higher priority than the multi-PDSCH (or multi-PUSCH) scheduling DCI. On the other hand, we do not support the case where two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol, but two multi-PDSCH (or multi-PUSCH) scheduling DCIs have overlapping spans. Therefore, we suggest removing the 1st bullet for further discussion and we can conclude the 2nd bullet first as follows Updated Proposal #2.6.1 - UE does not expect any of the scheduled PDSCHs (or PUSCHs) and the scheduling DCI to lead to out-of-order scheduling, also for the case of one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, where multi-PDSCH (or multi-PUSCH) scheduling DCI schedules more than one PDSCH (or PUSCH) - The case where two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but two multi-PDSCH (or multi-PUSCH) scheduling DCIs have overlapping spans, where the span is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV, is considered as out-of-order scheduling and is not expected by UE. |
| OPPO | For the first case, we think it can be allowed if single PXSCH DCI is transmitted later than the first PXSCH scheduled by multi-PXSCH DCI, e.g., For the second case, it can be considered as OOO scheduling. |
| NEC | We can accept the proposal, although we think the second case is not considered as invalid by legacy OOO rule |
| vivo | Support the proposal. |
| Huawei, HiSilicon | We can accept the proposal although we think allowing OOO in the listed two case provide more flexibility. |
| Apple | We are fine with the proposal |
| Intel | We support the FL proposal. Further, for the case of one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, the two DCIs can end in the same symbol. Such case can be considered as OOO too, which is similar to the second bullet in FL proposal. |
| Samsung | We would like to ask a clarification question for the 2nd bullet, are we going to separate discuss the case where a DCI schedules a single PDSCH/PUSCH and another DCI schedules multiple PDSCHs/PUSCHs? Also, we would like to ask the difference from PDSCH/PUSCH repetition? We don’t have such restrictions for repetition. |
| CATT | We are fine with the proposal |
| Fujitsu | We are fine with the proposal. |
| WILUS | We support this proposal. |
| Transsion | We support this proposal. |
| ZTE, Sanechips | We are fine with the proposal. |

- Issue 2.6-2) PDSCH-to-HARQ-ACK out-of-order issue:

TS 38.214 In a given scheduled cell, the UE is not expected to receive a first PDSCH in slot i, with the corresponding HARQ-ACK assigned to be transmitted in slot j, and a second PDSCH starting later than the first PDSCH with its corresponding HARQ-ACK assigned to be transmitted in a slot before slot j.

Company views on PDSCH -to-HARQ-ACK out-of-order issue:

- For multi-PDSCH scheduling, UE does not expect any of the scheduled/SPS PDSCHs and the resource for the HARQ-ACK transmission to lead to out-of-order scheduling.

  - Supported by vivo, Panasonic, Apple, MediaTek

  - Futurewei and Ericsson suggested to allow PDSCH-to-HARQ-ACK out-of-order scheduling for the case of one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI

- UE is not expected to receive a SPS PDSCH if the SPS PDSCH is configured to be received between a PDCCH with a DCI scheduling multiple PDSCHs and the last PDSCH scheduled by the DCI.

  - Supported by Samsung

[Moderator’s note] Given a small number of inputs, it is encouraged for companies to provide views on the above proposals, if any.

| Company | Views |
| --- | --- |
| Qualcomm | We prefer a unified design where the OOO rules applies for all PDSCHs whether the are configured via dynamic grant or SPS |
| Futurewei | Prefer a uniform handling for the OOO for data and OOO for HARQ for the case of one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI. |
| vivo | We share the same view with Qualcomm. |
| DOCOMO | We share same view as Qualcomm. |
| Intel | We are supportive to the first bullet. The OOO description does not need to differentiate whether the PDSCH is scheduled by DCI or not. The second bullet is too restrictive and is not preferred. |
| Samsung | Dynamic grant has higher priority than configured grant, the SPS PDSCH should not restrict the scheduling flexibility of dynamic scheduling. If SPS PDSCH is configured and activated, it will restrict dynamic scheduling which is not preferred. As we explained in our contribution, the following cases are not allowed which brings additional restrictions for dynamic scheduling. The 2nd bullet is to provide flexibility of dynamic scheduling, could Intel clarify a bit why it is restrictive? |
| Fujitsu | We share the same view with Qualcomm. And according to the discussion in the last meeting, if we understand correctly, the cases for SPS raised by Samsung are already covered by the 1 first bullet and not allowed. |
| WILUS | We share the same view with Qualcomm. |
| Apple | We do not support the occurrence of the scenario raised by Samsung, but our understanding is that the first bullet covers this scenario for out-of-order scheduling. |

Maximum gap between PDSCHs/PUSCHs

| Company | Views |
| --- | --- |
| [2] Futurewei | Proposal 6. It can be beneficial under unlicensed operation to restrict the maximal allowable gap values between adjacent PxSCHs according to the practical needs to avoid excessively large gaps that negatively impact the latency/throughput of the system or triggers additional requirement for LBT. |
| [6] Nokia | Proposal 2: The maximum gap between scheduled PDSCHs/PUSCH does not require additional impact on specification |
| [10] Panasonic | Proposal 1: For TDRA in a DCI that can schedule multiple PDSCHs (or PUSCHs), - The maximum value of the gap between two consecutively scheduled PDSCHs or between two consecutively scheduled PUSCHs is 2 slots, - The maximum number of gaps is 2. |
| [11] Ericsson | Proposal 2: Do not introduce constraints on maximum value of the gap between two consecutively scheduled PDSCHs/PUSCHs or maximum value of the gap between the first and the last scheduled PDSCH/PUSCH other than that inherently provided by the range of K0/K2 value. |
| [14] Lenovo | Proposal 2: For NR operation between 52.6 GHz and 71 GHz, when multiple PDSCHs/PUSCHs can be scheduled by a single DCI, then only the maximum allowed gap between first and last PDSCH/PUSCH is defined, or alternatively, maximum duration to contain all the scheduled PDSCHs/PUSCHs can be defined (in case of non-contiguous allocation) - Maximum allowed gap between two adjacent PDSCHs/PUSCHs need not be defined as network implementation can handle it under the constraint that all the scheduled PDSCHs/PUSCHs are contained within the maximum allowed gap between first and last PDSCH/PUSCH Proposal 3: For NR operation between 52.6 GHz and 71 GHz, when multiple PDSCHs/PUSCHs can be scheduled by a single DCI, one value to define the maximum allowed gap between first and last PDSCH/PUSCH for each of the SCS value can be defined |
| [17] InterDigital | Proposal 9: As all scheduled PDSCHs/PUSCHs should be transmitted within the channel coherent time, the maximum value of the gap between the first scheduled PDSCH and the last scheduled PDSCH or between the first scheduled PUSCH and the last scheduled PUSCH should be carefully selected. |
| [18] Apple | Proposal 6: On the maximum gap for PxSCH transmission: - The maximum gap between the first and last PxSCH transmissions should be selected (a) based on the maximum values of k0 and k2 i.e. 128 slots and (b) account for the use of a single MCS in the DCI. - The maximum gap between two consecutive transmissions, can be set to the maximum value between the first and the last transmission in a 2 PDSCH/PUSCH transmission i.e. 128 slots. |
| [19] LG Electronics | Observation #1: Adjustment of the gap between PDSCHs (or PUSCHs) for multi-PDSCH (or multi-PUSCH) scheduling DCI can be left up to network implementation. |
| [21] Qualcomm | Proposal 9: Define the maximum slot gap between any two SLIVs, it can be either SCS dependent or fixed values for all SCSs. Proposal 10: Define a maximum allowed span per single DCI as X slots, where X >= 8. |
| [22] MediaTek | Proposal 5: For multi-PDSCH scheduling, if M PDSCHs are scheduled by a DCI, the M PDSCHs should be contained within at most M consecutive slots |

- Summary on the maximum gap between scheduled PDSCHs/PUSCHs:

Agreement: (RAN1#105-e)

For TDRA in a DCI that can schedule multiple PDSCHs (or PUSCHs),

- A row of the TDRA table can indicate PDSCHs (or PUSCHs) that are in consecutive or non-consecutive slots.

  - FFS: The maximum value of the gap between two consecutively scheduled PDSCHs or between two consecutively scheduled PUSCHs

  - FFS: The maximum value of the gap between the first scheduled PDSCH and the last scheduled PDSCH or between the first scheduled PUSCH and the last scheduled PUSCH

  - FFS: Details to introduce the gap between PDSCHs or between PUSCHs

Company views on the maximum gap between scheduled PDSCHs/PUSCHs:

- Between two consecutively scheduled PDSCHs or between two consecutively scheduled PUSCHs

  - No additional impact on specification: Nokia, Ericsson, Lenovo, Apple?, LG Electronics

  - To be specified: Futurewei, Panasonic (2 slots), Qualcomm

- Between the first scheduled PDSCH and the last scheduled PDSCH or between the first scheduled PUSCH and the last scheduled PUSCH

  - No additional impact on specification: Nokia, Ericsson, LG Electronics

  - To be specified: Lenovo, Apple, Qualcomm (>= 8 slots), MediaTek (M slot span for M PXSCHs)

[Moderator’s note] In general, company views are divided into two categories where one is to suggest specifying a certain value to restrict the maximum gap between PDSCHs or PUSCHs and the other is not to further specify the maximum gap between PDSCHs or PUSCHs. In addition, even for proponents suggesting to specify the maximum gap between shared channels, the exact values for the gap are not aligned. Therefore, since it seems hard to pick an agreeable value for each gap, it is proposed not to specify the gap between PXSCHs.

- Proposed conclusion #2.7 (Max gap):

- For multi-PDSCH or multi-PUSCH scheduling DCI, the following maximum value of a gap is not specified in Rel-17 and up to gNB scheduler.

  - The maximum value of the gap between two consecutively scheduled PDSCHs or between two consecutively scheduled PUSCHs

  - The maximum value of the gap between the first scheduled PDSCH and the last scheduled PDSCH or between the first scheduled PUSCH and the last scheduled PUSCH

Companies are encouraged to provide views on Proposed conclusion #2.7.

| Company | Views |
| --- | --- |
| DOCOMO | We are fine with the proposal. |
| Nokia/NSB | Fine with the proposal. |
| Lenovo, Motorola Mobility | We don’t support this proposal and see the need to at least specify the maximum gap between the first scheduled PDSCH and the last scheduled PDSCH |
| Qualcomm | With the new range of k0/k2 at least we need to specify the maximum span of multi-PDSCH or PUSCH grant. At the beginning of the discussion of this issue the assumption was that k0/k2 are up to 32 so at most we may have up to 33 slots scheduled by one DCI, but now this can be up to 129 slots. |
| InterDigital | Given that all PDSCHs are supposed to use the same MCS, at least the maximum gap between the first and the last PDSCH/PUSCH should be specified. |
| Futurewei | We can accept the proposal, although we see it is better to specify the maximal gap between consecutively scheduled PxSCHs for LBT interruption concern if the gap is too large. |
| Panasonic | We share the similar view with Qualcomm. |
| OPPO | Support the proposal. |
| vivo | We are OK with the proposal and left to gNB implementation. |
| Huawei, HiSilicon | We agree with the proposal. gNB can determine the gaps and total span of multiple PXSCH scheduling. |
| Apple | As mentioned in our proposal, the maximum gap should be based on at least the maximum values of k0 and k2 decided in RAN1 #106-bis-e. |
| Intel | We are fine with the conclusion. |
| Samsung | We are fine with the proposed conclusion |
| CATT | We are fine with the conclusion. |
| MediaTek | We prefer to have reasonable restriction on the gap. Based on our understanding, the motivation of having gap is for UL/DL switching and we really don’t see the need to have large gap for it. Also, since there is an out of order restriction, having large gap is not desirable for DL scheduling. Lastly, large gap also elevates the HARQ ID starvation issue, which should be avoided as much as possible. |
| WILUS | We are fine with the proposed conclusion |
| Transsion | We are fine with the conclusion. |
| ZTE, Sanechips | We are fine with the conclusion |

Handling of collision with semi-static DL/UL/flexible symbols

| Company | Views |
| --- | --- |
| [12] Intel | Proposal 2 - If a PUSCH is collided with symbol(s) indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set, the HARQ process number increment is not skipped for the PUSCH. |
| [13] Samsung | Proposal 13: - The UE does not expect to be scheduled with multiple PUSCHs by a single DCI, where every PUSCH is collided with downlink symbol(s) indicated by SSB - If a PUSCH among multiple PUSCHs that are scheduled by a single DCI is collided with SSB symbols indicated by ssb-PositionsInBurst, the UE does not transmit the PUSCH |
| [19] LG Electronics | Proposal #1: If one of multiple PUSCHs scheduled by the DCI collides with a flexible symbol (indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated), and if that PUSCH is collided with symbol(s) indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set, the HARQ process number increment is skipped for the PUSCH. Proposal #2: If a PDSCH among multiple PDSCHs that are scheduled by a single DCI is collided with uplink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, NDI/RV fields corresponding to the PDSCH are absent in the DCI. Proposal #3: If a PUSCH among multiple PUSCHs that are scheduled by a single DCI is collided with downlink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, NDI/RV fields corresponding to the PUSCH are absent in the DCI. Proposal #4: Discuss in which PUSCH aperiodic CSI report is included if M-th or (M-1)-th scheduled PUSCH is cancelled due to the collision with semi-static DL symbols. |
| [20] NTT DOCOMO | Proposal 3: If multiple PUSCHs are scheduled by single DCI, and there is at least one PUSCH collides with semi-static DL symbol, and/or symbol configured for SSB or CORESET#0 reception, - OoO scheduling limitation is based on valid PUSCHs. - If CBG based transmission is configured, CBGTI field is present in DCI for the case when multiple PUSCHs are scheduled but only one PUSCH is valid. - A-CSI reporting triggered by multi-PUSCH scheduling DCI is based on valid PUSCHs. When the A-CSI triggering DCI schedules N valid PUSCHs, the PUSCH that carries the aperiodic CSI feedback is N-th valid PUSCH for N <= 2, or (N-1)-th valid PUSCH for N > 2. - When timeline is satisfied, the CG PUSCH overlapping with the cancelled DG PUSCH can be transmitted. - When timeline is satisfied, the CG PUSCH with same HARQ process ID as the cancelled DG PUSCH can be transmitted. Proposal 4: If multiple PDSCHs are scheduled by single DCI, and there is at least one PDSCH collides with semi-static UL symbol, - OoO scheduling limitation is based on valid PDSCHs. - If CBG based transmission is configured, CBGTI/CBGFI fields are present in DCI for the case when multiple PDSCHs are scheduled but only one PDSCH is valid. - DCI scheduling multiple PDSCHs but with only one valid PDSCH is included in the first sub-codebook. - When timeline is satisfied, the SPS PDSCH overlapping with the cancelled DG PDSCH can be received. |
| [21] Qualcomm | Proposal 13: In the case of multi-PDSCH scheduling via a single DCI with 'tdmSchemeA', consider one of the following options to handle the overlap with semi-static UL symbols - Option 1: If one of the repetitions of the PDSCH collides with semi-static UL symbols, the corresponding PDSCH is considered as not valid - Option 2: If the first repetition of the PDSCH collides with semi-static UL symbols, the corresponding PDSCH is considered as not valid - On the other hand, if only the second repetition of the PDSCH collides with semi-static UL symbol, the PDSCH is still considered valid Proposal 18: For a single DCI that schedules multi-PDSCH/PUSCH, the NDI/ RV should be signaled per SLIV, i.e., a single bit will be assumed for each SLIV in NDI or RV vector even if the corresponding PDSCH/PUSCH is not valid. |

- Issue 2.8-1) How to handle collision between PUSCH and CORESET#0:

Agreement:

For multiple PDSCHs (or PUSCHs) scheduled by a single DCI,

- Rel-15/16 behavior that is described in TS 38.213 Clauses 11 and 11.1 for a PDSCH (or PUSCH) indicated by DCI also applies for multiple PDSCHs (or PUSCHs) schedule by a single DCI.

- If one of multiple PDSCHs (or PUSCHs) scheduled by the DCI collides with a flexible symbol (indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated),

  - If that PUSCH is collided with SSB symbols indicated by ssb-PositionsInBurst [or symbol(s) indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set], the HARQ process number increment is skipped for the PUSCH.

  - Otherwise, the HARQ process number increment is not skipped for that PDSCH (or PUSCH).

Company views on highlighted part above:

- If that PUSCH is collided with SSB symbols indicated by ssb-PositionsInBurst or symbol(s) indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set, the HARQ process number increment is skipped for the PUSCH.

  - Supported by Samsung, LG Electronics

  - Objected by Intel

[Moderator’s note] Given a small number of inputs, it is encouraged for companies to provide views on the above proposals, if any.

| Company | Views |
| --- | --- |
| Qualcomm | The HARQ increment can be skipped for this case. |
| Futurewei | Agree with the view of Samsung and LGE to skip the HARQ proc number for the case. |
| OPPO | In our view, if PUSCH repetition Type B is supported, then: (1) if PUSCH is collided with symbol(s) indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set, the HARQ process number increment can be skipped for the PUSCH. (2) if one or more repetitions of the K nominal repetitions are determined as invalid according to the invalid symbol pattern indicator in the DCI and the configured invalidSymbolPattern, the HARQ process number increment can be skipped for that PUSCH. |
| vivo | We prefer to perform HARQ process number incrementing for a scheduled PUSCH colliding with any CORESET#0 symbol, since there is no explicit description in chapter 11.1 in TS38.213 for UE not to transmit a PUSCH if the PUSCH collides with any CORESET#0 symbol. |
| Apple | We support the proposal that “If that PUSCH is collided with symbol(s) indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set, the HARQ process number increment is skipped for the PUSCH”. |
| DOCOMO | Same handling as PUSCH collision with semi-static DL symbol and SSB. |
| Intel | We do not support to consider CORESET0 with Type0-PDCCH CSS set for HPN determination. Based on Rel-15/16 spec as captured below, NB scheduler needs to ensure that there is no collision between scheduled PUSCH and flexible symbols indicated for CORESET with Type0-PDCCH CSS set. In this case, UE can still transmit the PUSCH on the flexible symbols which are indicated for CORESET with Type0-PDCCH CSS set. We do not need to change existing behavior. For a set of symbols of a slot indicated to a UE by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set, the UE does not expect the set of symbols to be indicated as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated. |
| Samsung | Symbols indicated by CORESET0 are considered as semi-static DL in Rel-16 Type-B PUSCH repetition and Rel-17 URLLC SPS HARQ-ACK deferral, the principle should apply here. |
| WILUS | We share the view with Samsung and LGE to support the proposal that “If that PUSCH is collided with symbol(s) indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set, the HARQ process number increment is skipped for the PUSCH”. |

- Issue 2.8-2) Clarification on whether “scheduled PXSCH” in previous agreements implies valid PXSCH or not:

Agreement: (RAN1#104-bis)

For a DCI that can schedule multiple PDSCHs,

- NDI for the 1st TB: This is signaled per PDSCH and applies to the first TB of each PDSCH

- RV for the 1st TB: This is signaled per PDSCH, with 2 bits if only a single PDSCH is scheduled or 1 bit for each PDSCH otherwise and applies to the first TB of each PDSCH

Conclusion: (RAN1#105-e)

For a DCI that can schedule multiple PUSCHs,

- CSI-request: When the DCI schedules M PUSCHs, the PUSCH that carries the aperiodic CSI feedback is M-th scheduled PUSCH for M <= 2, or (M-1)-th scheduled PUSCH for M > 2.

Agreement: (RAN1#105-e)

- At least for 120 kHz SCS, for a DCI that can schedule multiple PUSCHs and is configured with the TDRA table containing at least one row with multiple SLIVs,

  - If CBG-based (re)transmission is configured, CBGTI field is not present when more than one PUSCHs are scheduled, but is present when a single PUSCH is scheduled, as in Rel-16.

Agreement: (RAN1#106bis-e)

For two multi-PDSCH (or two multi-PUSCH) scheduling DCIs, UE does not expect any of the scheduled PDSCHs (or PUSCHs) and the scheduling DCI to lead to out-of-order scheduling.

Agreement: (RAN1#106bis-e)

For a DCI that can schedule multiple PDSCHs, and if RRC parameter configures that two codeword transmission is enabled,

- NDI for the 2nd TB: This is signaled per PDSCH and applies to the 2nd TB of each PDSCH

- RV for the 2nd TB: This is signaled per PDSCH, with 2 bits if only a single PDSCH is scheduled or 1 bit for each PDSCH otherwise and applies to the 2nd TB of each PDSCH

[Moderator’s note] The following clarifications seem to be needed.

- For NDI/RV, are NDI/RV fields for invalid PXSCHs present in multi-PXSCH scheduling DCI?

- For RV field, is the bit-width between 1 bit and 2 bits determined based on the number of configured SLIVs or valid SLIVs?

- For CSI-request, is the number M determined based on the number of configured SLIVs or valid SLIVs?

- For CBGTI field, is the presence of CBGTI field determined based on the number of configured SLIVs or valid SLIVs?

- For out-of-order scheduling, is the rule for OOO scheduling determined based on configured SLIVs or valid SLIVs?

| Company | Views |
| --- | --- |
| Qualcomm | To simplify the UE processing of the DCI, only consider the number SLIVs whether some of them are invalid or not for bullets 1)-4). For the OOO rules, they are defined based on the PDSCHs so it is reasonable to consider only the valid SLIVs |
| vivo | For bullets 1)-4), it is preferred to consider the number of configured SLIVs in the scheduled row for simplicity. Regarding bullet 3), it is gNB’s responsibility to guarantee that the PUSCH assigned for carrying the triggered A-CSI is valid. For bullet 5), we share the same view as QC that only valid PDSCH(s)/PUSCH(s) should be considered for OOO determination. |
| Huawei, HiSilicion | For 1-4), we support use configured SLIVs because it should be semi-static. For 5), it can be according to valid SLIV |
| DOCOMO | We think determination of 1)-5) based on valid PXSCHs will not cause any problem, but can provide more flexibility. Especially for A-CSI reporting, the determined PUSCH based on scheduled M may be skipped due to overlapping with DL symbol. It is not desired. |
| Intel | We prefer a simple design to follow configured SLIVs by default. - There is no real gain to remove NDI/RV of invalid PxSCH since the DCI size is constant and determined by the configured maximum number of SLIVs across all rows - We prefer to avoid unnecessary optimization. gNB can schedule a row with single configured SlIV if 2-bit RV is desired - gNB should indicate an applicable row in TDRA table, if there is a A-CSI report - We prefer to avoid unnecessary optimization. gNB can schedule a row with single configured SlIV if CBG based transmission is desired - If OOO is defined by valid SLIV, gNB can even schedule PDSCH transmission by two or multiple DCIs with same K1 value, if only the valid PDSCHs scheduled by the DCIs are not overlapped. Such a design makes time bundling problematic in Type-1 HARQ-ACK codebook generation. |
| Samsung | NDI/RV can be based on scheduled SLIVs CSI-request should be based on the valid SLIV, otherwise the CSI may not be triggered. CBG should not be supported together with M-PDSCH/PUSCH transmission. OOO can be checked based on the valid SLIVs |
| ZTE, Sanechips | For bullet 1)~4), we suggest to consider the number of configured SLIVs regardless of some of them is valid or invalid. For bullet 5), only valid PDSCH(s)/PUSCH(s) should be considered |

SPS/CG-related issues

| Company | Views |
| --- | --- |
| [1] Huawei | Proposal 8: For activation of SPS (or CG) by using multi-PDSCH (or multi-PUCH) scheduling DCI, only single SLIV-based activation is allowed. |
| [3] vivo | Proposal 8: For activation/de-activation of SPS/CG by using multi-PDSCH/PUSCH scheduling DCI, the first (valid) SLIV in the row indicated by an activation/de-activation DCI is used for determining SPS/CG occasions. |
| [5] Fujitsu | Proposal 2: For SPS activation/retransmission via DCI format 1_1 when multi-PDSCH scheduling is supported, the following 3 options can be considered, and Option 2 is slightly preferred for a well trade-off between flexibility and standardization effort. - Option 1: Allow only single SLIV-based (de)activation - Option 2: Based on the last configured SLIV - Option 3: Based on the first (valid) SLIV |
| [7] CATT | Proposal 7: for (de)activation of SPS (or CG) by using multi-PDSCH (or multi-PUSCH) scheduling DCI, Both the PDSCH time domain and k1 value are obtained based on first valid SLIV. |
| [8] CATT | Proposal 1: When one SPS configuration is activated by a DCI which schedules multiple PDSCHs: - The first valid PDSCH scheduled is used for the SPS PDSCH - K1 is counted from the first PDSCH slot Proposal 2: When one SPS configuration is released by a DCI which schedules multiple PDSCHs: - The first valid PDSCH scheduled is used for the SPS PDSCH - K1 is counted from the first PDSCH slot Proposal 3: More than one SPS configurations can be defined in a list by RRC. And more than one SPS configurations in one list can be activated or released by a DCI that schedules multiple PDSCHs. Proposal 4: For some special HARQ process ID（e.g. ID assigned to SPS PDSCH by RRC）, UE shall skip occupied HARQ process ID of SPS when the dynamic scheduling overlaps with these process ID. |
| [12] Intel | Proposal 3 - A HARQ process number configured for SPS PDSCH/CG PUSCH can be allocated to a PDSCH/PUSCH of multi-PDSCH/PUSCH scheduling, as long as the timeline is met. |
| [16] Samsung | Proposal 8: If a CG PUSCH is configured to be transmitted between the first scheduled PUSCH and the last scheduled PUSCH by a single DCI scheduling multiple PUSCHs, HARQ process number increment is skipped for the HARQ ID used for the CG PUSCH when determining the HARQ ID of the multiple scheduled PUSCHs. Proposal 9: For a DCI capable of scheduling multi-PDSCH/PUSCHs, gNB can only indicate a row with single SLIV for SPS PDSCH/CG PUSCH activation and retransmission. Proposal 10: If a PUCCH overlaps a PUSCH scheduled by a DCI format, UE checks DL collision for the overlapping PUSCH before UCI multiplexing. |
| [19] LG Electronics | Proposal #7: If a DCI that indicates a row index of the TDRA table associated with multiple SLIVs can be used for SPS PDSCH (or CG PUSCH) (de)activation, determine TDRA or PUCCH resource corresponding to SPS (or CG) based on the last SLIV value in the indicated TDRA row index. |

- Issue 2.9-1) How to handle HARQ process number when it collides with that assigned for SPS or CG:

Company views on how to handle HARQ process number when it collides with that assigned for SPS or CG:

- Option 1: Skip HARQ process number(s) pre-configured for SPS or CG when any of scheduled PDSCHs (or PUSCHs) is overlapped with an SPS PDSCH (or CG PUSCH), and when HARQ process numbers for PDSCHs (or PUSCHs) scheduled by a single DCI collide with HARQ process number(s) configured for the SPS PDSCH (or CG PUSCH).

  - Supported by CATT, Samsung

- Option 2: HARQ process number configured for SPS PDSCH (or CG PUSCH) can be allocated to a PDSCH (or PUSCH) of multi-PDSCH (or multi-PUSCH) scheduling, as long as the timeline is met.

  - Supported by Intel

Timeline for PDSCH:

TS 38.214 5.1 UE procedure for receiving the physical downlink shared channel ……………………………………..<omitted>……………………………………… The UE is not expected to decode a PDSCH in a serving cell scheduled by a PDCCH with C-RNTI, CS-RNTI or MCS-C-RNTI and one or multiple PDSCH(s) required to be received according to this Clause in the same serving cell without a corresponding PDCCH transmission if the PDSCHs partially or fully overlap in time except if the PDCCH scheduling the PDSCH ends at least 14 symbols before the earliest starting symbol of the PDSCH(s) without the corresponding PDCCH transmission, where the symbol duration is based on the smallest numerology between the scheduling PDCCH and the PDSCH, in which case the UE shall decode the PDSCH scheduled by the PDCCH.

Timeline for PUSCH:

TS 38.214 6.1 UE procedure for transmitting the physical uplink shared channel ……………………………………..<omitted>……………………………………… A UE is not expected to be scheduled by a PDCCH ending in symbol i to transmit a PUSCH on a given serving cell overlapping in time with a transmission occasion, where the UE is allowed to transmit a PUSCH with configured grant according to [10, TS38.321], starting in a symbol j on the same serving cell if the end of symbol i is not at least N2 symbols before the beginning of symbol j. The value N2 in symbols is determined according to the UE processing capability defined in Clause 6.4, and N2 and the symbol duration are based on the minimum of the subcarrier spacing corresponding to the PUSCH with configured grant and the subcarrier spacing of the PDCCH scheduling the PUSCH.

[Moderator’s note] Although the number of inputs is small, given the wide support of Option 2 in the last meeting, the following proposal can be made.

- Proposal #2.9-1 (SPS/CG HPN):

- HARQ process number configured for SPS PDSCH (or CG PUSCH) can be allocated to a PDSCH (or PUSCH) of multi-PDSCH (or multi-PUSCH) scheduling, as long as the timeline condition defined in Rel-15/16 is met.

Companies are encouraged to provide views on Proposal #2.9-1.

| Company | Views |
| --- | --- |
| DOCOMO | Support the proposal. We think such behavior is resulted by directly following Rel-15/16 rule. Moreover, we think we can further discuss UE transmitting (receiving) CG PUSCH (or SPS PDSCH) or not if the PUSCH (or PDSCH) of multi-PUSCH (or multi-PDSCH scheduling) is skipped/cancelled due to DL/UL collision. |
| Qualcomm | We are okay with the proposal |
| Futurewei | Ok with the proposal. The issue raised by Docomo can be further discussed as well. |
| OPPO | We are fine with the proposal. |
| vivo | Agree with the proposal. |
| Huawei, HiSilicon | We can accept this proposal. |
| Intel | We are fine with the proposal. |
| Samsung | Not support. As we clarified in our contribution, the HARQ buffer can be cleared due to same HARQ ID collision and as a consequence, it may cause UL data retransmission in higher layer which is not preferred. |
| WILUS | We support this proposal. |
| ZTE, Sanechips | We support this proposal. |

- Issue 2.9-2) Activation of SPS (or CG) by using multi-PDSCH (or multi-PUCH) scheduling DCI:

Company views on the issue for activation of SPS (or CG) by using multi-PDSCH (or multi-PUCH) scheduling DCI:

- Option 1: Allow only single SLIV-based activation

  - Supported by Huawei, Samsung

- Option 2: Based on the last (valid) SLIV

  - Supported by Fujitsu, LG Electronics

- Option 3: Based on the first (valid) SLIV

  - Supported by vivo, CATT

- Note: As Huawei pointed out, UE does not need to check TDRA table to transmit HARQ-ACK feedback corresponding to SPS release DCI. Therefore, this issue is relevant only to activation of SPS/CG.

[Moderator’s note] Given a small number of inputs, it is encouraged for companies to provide views on the above options, if any, including whether Option 3 affects HARQ timing determination rule.

| Company | Views |
| --- | --- |
| Qualcomm | We prefer option 3 |
| OPPO | Support Option 1. |
| vivo | We slightly prefer option 3. Each row in the TDRA table can be used for activation/de-activation, which can bring more flexibility, and using the first (valid) SLIV can lead to shorter latency between an activation DCI and the first PDSCH/PUSCH transmission corresponding to the activation DCI. |
| Huawei, HiSilicon | We support Option 1 which has the least spec impact. For Option 2, there is an agreement that for DCI scheduling multiple PDSCHs, the k1 indicated by the DCI (or provided by RRC signaling) indicates the slot offset between the last PDSCH scheduled by the DCI and slot carrying the corresponding HARQ-ACK information, if the last SLIV is invalid due to collision with UL symbol by TDD configuration, there is an spec impact on how to determine the PUCCH resource by k1 value. For Option3, the definition of k1 in DCI needs revised. |
| Apple | Support Option 1 |
| DOCOMO | Prefer option 1. We don’t see the benefit of indicating multiple SLIVs but only based on the first valid SLIV. |
| Intel | We prefer Option 1. It is not clear to us why gNB would activate SPS/CS using multi-SLIV row in the TDRA table. |
| Samsung | We prefer option 1 but option 3 is acceptable. The note is unclear to us. To validate DCI format and find the bit position and size of NDI field and RV field, the UE check TDRA field first. Also, it should be clarified whether or not to allow SPS (or CG) retransmission by mulit-PDSCH (or PUSCH) scheduling. |
| CATT | Support Option 3 |
| MediaTek | We have a clarification question. The occasions for SPS PDSCH release reception need to be considered in HARQ codebook construction and we are curious why this aspect is removed from discussion? |
| Fujitsu | To Moderator/Huawei: Actually, in our mind, option 2 is based on the last configured SLIV, regardless of validation, so there is no such spec impact. And we suggest the following modification: - Option 2: Based on the last (valid) configured SLIV As we explained in our contribution, our preference is Option 2 considering its well trade-off between flexibility and spec. impact. But if we are the minority, we are okey to accept either Option 1 or Option 3. |
| ZTE, Sanechips | For the sake of simplicity, we support option1 |
| Futurewei | Prefer Option 1 and can accept Option 3. |

[Closed] TDMed PDSCHs/PUSCHs in a slot

| Company | Views |
| --- | --- |
| [2] Futurewei | Proposal 4. For multi-TRP cases, a UE should not expect to be scheduled with more than one PDSCHs if they are from the same TRP, while it is suggested to allow more than one PDSCHs be scheduled by two different DCIs from two TRPs. |

- Summary on whether or not to allow TDMed PDSCHs/PUSCHs in a slot:

Agreement: (RAN1#106-e)

- For single TRP operation, for 480/960 kHz SCS,

  - FFS: A UE does not expect to be scheduled with more than one PDSCH in a slot, by a single DCI or multiple DCIs.

  - FFS: A UE does not expect to be scheduled with more than one PUSCH in a slot, by a single DCI or multiple DCIs.

- For single TRP operation, for 120 kHz SCS (same as current specification for FR2-1 for PUSCH),

  - Subject to UE capability, a UE can be scheduled with more than one PDSCH in a slot, by a single DCI or multiple DCIs.

  - Subject to UE capability, a UE can be scheduled with more than one PUSCH in a slot, by a single DCI or multiple DCIs.

- FFS for multi-TRP operation

[Moderator’s note] Although a single company expressed their view for multi-TPR case, the suggestion seems reasonable and the following proposal can be made.

Proposal #2.10 (TDMed PDSCHs/PUSCHs in a slot for mTRP):

- For multi-TRP operation, for 480/960 kHz SCS,

  - A UE does not expect to be scheduled with more than one unicast PDSCH in a slot, by a single DCI or multiple DCIs, from the same TRP.

  - A UE does not expect to be scheduled with more than one PUSCH in a slot, by a single DCI or multiple DCIs, from the same TRP.

  - Note: This does not preclude a UE being scheduled with two PDSCHs (or two PUSCHs) in the same slot from two different TRPs for multi-DCI based multi-TRP mechanism.

Companies are encouraged to provide views on Proposal #2.10.

| Company | Views |
| --- | --- |
| DOCOMO | We are fine with the proposal. |
| Nokia/NSB | Fine with the proposal. |
| Lenovo, Motorola Mobility | Support the proposal |
| Qualcomm | We are okay with the proposal |
| InterDigital | We are fine with this proposal. |
| Futurewei | Support the Proposal #2.10. |
| Panasonic | We are fine with the proposal #2.10. |
| OPPO | We are fine with the proposal. |
| vivo | OK with the proposal. |
| Huawei, HiSilicon | We are fine with the proposal. |
| Apple | We are fine with the proposal. |
| Intel | We are fine with the proposal. |
| Moderator | Seems stable |

During email discussion, the following agreement was made:

- Agreement:

- For multi-TRP operation, for 480/960 kHz SCS,

  - A UE does not expect to be scheduled with more than one unicast PDSCH in a slot, by a single DCI or multiple DCIs, from the same TRP.

  - A UE does not expect to be scheduled with more than one PUSCH in a slot, by a single DCI or multiple DCIs, from the same TRP.

  - Note: This does not preclude a UE being scheduled with two PDSCHs (or two PUSCHs) in the same slot from two different TRPs for multi-DCI based multi-TRP mechanism.

Others

| Company | Views |
| --- | --- |
| [13] Xiaomi | Proposal 3: Support to indicate more than one channel access types in a single DCI. |
| [16] Samsung | Proposal 7: For multi-PDSCH scheduling, the bit field common for DL and UL grant use the same design as multi-PUSCH scheduling, and at least following DL-specific bit field should be specified, - CBG-based transmission is not applicable to single and multi-PDSCH scheduling - HARQ-ACK relevant bit field is applicable to all PDSCHs and single PUCCH Proposal 14: Clarify that for Scell dormancy indication, a UE repurposes Npdsch,max-bit NDI and Npdsch,max-bit RV fields if TDRA indicates multi-PDSCH scheduling or 1-bit NDI and 2-bit RV fields if TDRA indicates single-PDSCH scheduling. - If Npdsch,max-bit NDI and Npdsch,max-bit RV fields are repurposed, the sequence order for a bitmap is 5-bit MCS, Npdsch,max-bit NDI, Npdsch,max-bit RV, HPN, antenna port(s), and DMRS sequence initialization fields |
| [17] InterDigital | Observation 2: Ability to schedule a single slot with SCSs 480 kHz and 960 kHz can be useful to support delay sensitive applications. Proposal 7: Minimum number of slots that can be schedule by a single DCI for SCSs 480 kHz and 960 kHz is 1. |

- Summary on other aspects for multi-PDSCH/PUSCH scheduling:

The following issues are brought up by several companies:

- Xiaomi: Support of more than one channel access type indication fields in a single DCI

- Samsung: HARQ-ACK relevant bit field is applicable to all PDSCHs and single PUCCH

- Samsung: Clarification on SCell dormancy indication of multi-PDSCH scheduling DCI

- InterDigital: Minimum number of slots that can be schedule by a single DCI for SCSs 480 kHz and 960 kHz is one.

[Moderator’s note] Given a small number of inputs for those issues, it is proposed to deprioritize them in this meeting but please feel free to express views on above issues, if any.

| Company | Views |
| --- | --- |
| InterDigital | We are fine with deprioritizing first 3 topics. However, in our opinion, supporting single PDSCH/PUSCH scheduling for new SCSs could be very useful. |
| Futurewei | Fine with the recommendation that these issues are deprioritized. |
| DOCOMO | Support to deprioritize. |
| Moderator | To InterDigital, My understanding is that single PDSCH/PUSCH scheduling should be supported also for 480/960 kHz SCS, at least by using DCI format 1_0/0_0. |
| Samsung | UCI multiplexing rule proposed in our tdoc is missing here. Please add this topic. Samsung: Dropping/multiplexing order for UCI multiplexing Proposal 10: If a PUCCH overlaps a PUSCH scheduled by a DCI format, UE checks DL collision for the overlapping PUSCH before UCI multiplexing. |

HARQ

Time domain bundling

| Company | Views for type-1 HARQ-ACK codebook |
| --- | --- |
| [1] Huawei | Proposal 9: For FR2-2, Time domain bundling of Type-1 HARQ-ACK codebook can be supported in granularity of DCI scheduling. Reuse the legacy specification that the size and mapping of the HARQ-ACK codebook are determined by the number and position of the last non-overlapped valid SLIV after pruning of the invalid SLIVs with UL symbols by semi-static TDD configuration. |
| [3] vivo | Proposal 15: Regarding time domain bundling for Type-1 codebook when multi-PDSCH scheduling is configured, consider the following alternatives: - Alt. 1: A set of occasions is determined based on the last (valid) SLIV in each row of the TDRA table, and time domain bundling is performed across all valid PDSCH(s) scheduled by a DCI by indicating a row in the TDRA table. - Alt. 2: A set of occasions is determined based on the last (valid) SLIV in each sub-row, which is divided from each row of the TDRA table, and time domain bundling is performed across all valid PDSCH(s) for each of one or more sub-rows scheduled by a DCI by indicating a row, from which the one or more sub-rows are divided, in the TDRA table. - Alt. 3: A set of occasions is determined based on all (valid) SLIVs in each row of the TDRA table, in the same way as the case when time domain bundling is not configured, and time domain bundling is performed for each subset of occasions divided from the set of occasions. |
| [4] ZTE | Proposal 3: If time domain bundling for Type1 HARQ-ACK codebook is performed across all PDSCHs scheduled by a DCI and pruning procedure is based on the last SLIV, method to ensure HARQ-ACK information report for valid SLIVs should be introduced. |
| [5] Fujitsu | Proposal 1: For Type-1 HARQ-ACK codebook, support time domain bundling. - For each K1, the corresponding candidate PDSCH reception occasion can be determined based on all the SLIVs of each row in the TDRA table. If at least one of SLIVs in a row in the TDRA table is not colliding with UL symbols configured by RRC signaling, it corresponds to one candidate PDSCH reception occasion. - For each determined candidate PDSCH reception occasion, HARQ-ACK information for all PDSCHs in slots that include SLIV(s) not colliding with UL symbols can be bundled as 1 bit. |
| [6] Nokia | Proposal 5: For Type-1 and Type-2 codebook, configurable time domain bundling of HARQ-ACK feedback with M bundling groups for PDSCHs scheduled by the same DCI is supported. Proposal 6: For Type-1 codebook with configurable time domain bundling of HARQ-ACK feedback: - Modified TDRA table is used in the codebook determination - TDRA rows are modified by keeping the last SLIV(s) of the row corresponding to the number of bundled HARQ-ACK bit(s) and removing other SLIVs from that row. |
| [9] OPPO | Proposal 3: Support time domain bundling operation for both Type-1 and Type-2 HARQ-ACK codebooks. - Time domain bundling is performed across subset of PDSCHs scheduled by a DCI. - gNB can configure the subset of scheduled PDSCHs. |
| [11] Ericsson | Proposal 14: Support Option 1 for time domain bundling for Type-1 HARQ-ACK codebook. I.e., time domain bundling is performed across all PDSCHs scheduled by a DCI. The time domain bundling is RRC configurable. |
| [12] Intel | Proposal 6 - Time domain bundling is supported in HARQ-ACK transmission. - The PDSCHs associated with the HARQ-ACKs that are time bundled should be scheduled by the same DCI. - The maximum number of PDSCHs for which HARQ-ACKs are bundled can be configured by high layer. - UE does not expect to be configured with both of CBG operation and time domain bundling for multi-PDSCH scheduling in the same PUCCH cell group. - Confirm the work assumption that UE does not expect to be configured with both of CBG operation and multi-PDSCH scheduling in the same PUCCH cell group with a Type 2 codebook. Proposal 7 - For Type-1 HARQ-ACK codebook with time domain bundling - For each row in TDRA table, N SLIVs are selected and associated with N bundled HARQ-ACK of the row. - It is beneficial that the selected SLIVs of each row in TDRA table for each K1 value can be mapped to the same slot(s). - A modified TDRA table can be obtained with each row only containing the N selected SLIVs. - Rel-16 Type-1 HARQ-ACK codebook generation can be applied based on the modified TDRA table. |
| [13] Xiaomi | Proposal 7: Support the HARQ-ACK bundling that all the scheduled multiple PDSCHs’ HARQ-ACK information are bundled as one HARQ-ACK information, and the PUCCH resource for HARQ-ACK information is based on the location of the last PDSCH. |
| [14] Lenovo | Proposal 5: For NR operation between 52.6 GHz and 71 GHz, for HARQ-ACK information corresponding to PDSCHs scheduled by the DCI, time domain bundling should be supported for both type-1 and type-2 HARQ-ACK codebook, where the time domain bundling is performed across a subset of PDSCHs scheduled by a DCI. |
| [16] Samsung | Proposal 19: Postpone the discussion on whether/how to support time domain bundling for type-1 HARQ-ACK codebook until RAN1 makes the decision for the HARQ-ACK bundling mechanism. |
| [17] InterDigital | Proposal 3: Support bundling of HARQ-ACK information bits for multiple PDSCHs. the number of HARQ-ACK information bits for a candidate PDSCH reception occasion is determined based on the number of bundled PDSCHs. |
| [18] Apple | Proposal 9: Time domain bundling is performed across subset of PDSCHs scheduled by a DCI. The bundling operation is performed after the codebook is constructed. |
| [19] LG Electronics | Proposal #13: If time domain bundling is to be supported for type-1 HARQ-ACK codebook construction, - Only allow bundling operation for all PDSCHs corresponding to each DCI. - Each PDSCH reception occasion is determined based on the last SLIV among multiple SLIVs associated with a row index. |
| [20] NTT DOCOMO | Proposal 6: Support time domain HARQ-ACK bundling for multi-PDSCH scheduling. - Time domain bundling is enabled/disabled by RRC parameter, where the enabling/disabling should depend on other configuration, e.g. whether CBG is supported/enabled for multiple PDSCH scheduling, and/or whether two-TB scheduling is supported/enabled for multiple PDSCH scheduling, and/or whether spatial bundling is enabled if two-TB scheduling is supported/enabled for multiple PDSCH scheduling. - Time domain bundling is performed across all PDSCHs scheduled by a DCI. - For type 1 HARQ-ACK feedback, PDSCH candidate occasion determination and pruning procedure is based on the last SLIV. - For type 2 HARQ-ACK feedback, multi-PDSCH DCI scheduled PDSCHs and single-PDSCH DCI scheduled PDSCHs are included in the same sub-codebook. |
| Company | Views for type-2 HARQ-ACK codebook |
| [1] Huawei | Proposal 10: For FR2-2, Time domain bundling of Type-2 HARQ-ACK codebook can be supported in granularity of DCI scheduling where only one ACK/NACK is feedback for all the scheduled valid/invalid PDSCHs. |
| [3] vivo | Proposal 16: Regarding time domain bundling for Type-2 codebook when multi-PDSCH scheduling is configured, consider the following alternatives: - Alt. 1: Time domain bundling is performed across all valid PDSCH(s) scheduled by a DCI by indicating a row in the TDRA table. - Alt. 2: Time domain bundling is performed across a subset of valid PDSCHs, which is divided from the set of valid PDSCH(s) scheduled by a DCI by indicating a row in the TDRA table. |
| [6] Nokia | Proposal 5: For Type-1 and Type-2 codebook, configurable time domain bundling of HARQ-ACK feedback with M bundling groups for PDSCHs scheduled by the same DCI is supported. |
| [7] CATT | Proposal 11: If Time bundling of HARQ-ACK feedback is used, the following two issues are suggested: - The number of HARQ-ACK bit N per multi-PDSCH scheduling after bundling process can be configured, the N can be {1,2,4,8} - Only the ACK/NACK bits of valid PDSCH can join the timing bundling process |
| [9] OPPO | Proposal 3: Support time domain bundling operation for both Type-1 and Type-2 HARQ-ACK codebooks. - Time domain bundling is performed across subset of PDSCHs scheduled by a DCI. - gNB can configure the subset of scheduled PDSCHs. |
| [11] Ericsson | Proposal 15: Support Option 2-1 for time domain bundling for Type-2 HARQ-ACK codebook. I.e., with configurable number of bundling groups. Proposal 16: When time domain bundling is applied to Type-2 HARQ-ACK codebook, ACK should be reported for invalid PDSCHs (due to collision with UL symbols) in the codebook prior to bundling. |
| [12] Intel | Proposal 6 - Time domain bundling is supported in HARQ-ACK transmission. - The PDSCHs associated with the HARQ-ACKs that are time bundled should be scheduled by the same DCI. - The maximum number of PDSCHs for which HARQ-ACKs are bundled can be configured by high layer. - UE does not expect to be configured with both of CBG operation and time domain bundling for multi-PDSCH scheduling in the same PUCCH cell group. - Confirm the work assumption that UE does not expect to be configured with both of CBG operation and multi-PDSCH scheduling in the same PUCCH cell group with a Type 2 codebook. Proposal 8 - For Type-2 HARQ-ACK codebook with time domain bundling, - If the maximum number of bundled HARQ-ACK per DCI is one, single HARQ-ACK codebook can be used. Otherwise, two sub-codebooks are generated. - When two sub-codebooks are used, if the actual number of bundled bits is 1 for a DCI, the first sub-codebook is used to carry the bundled HARQ-ACK for the DCI. Otherwise, the second sub-codebook is used. |
| [14] Lenovo | Proposal 5: For NR operation between 52.6 GHz and 71 GHz, for HARQ-ACK information corresponding to PDSCHs scheduled by the DCI, time domain bundling should be supported for both type-1 and type-2 HARQ-ACK codebook, where the time domain bundling is performed across a subset of PDSCHs scheduled by a DCI. |
| [15] NEC | Proposal 3: For Alt 1 of type-2 HARQ-ACK codebook determination: - If time domain bundling is supported, similar grouping way as CBG can be reused, and spatial bundling and time bundling should not be simultaneously configured or applied. - If there is a confliction between any of scheduled PDSCHs of a single DCI and uplink symbol(s) indicated by TDD configuration, how to fill the NACK bits for the collision slot(s) needs to be determined. - If there is a confliction between any of scheduled PDSCHs of a single DCI and uplink symbol(s) indicated by TDD configuration, and only 1 actual scheduled PDSCH left in this DCI scheduling, this PDSCH will belong to sub-codebook 1. |
| [16] Samsung | Proposal 16: If HARQ-ACK bundling is supported, bundling is performed within valid PDSCHs scheduled by a single DCI. Down-select one of the following alternatives: - Alt a: gNB configures a number of HARQ-ACK bundling groups (Nb) per DCI - Alt b: gNB configures a number of valid PDSCHs per HARQ-ACK bundling groups (Npb) - Alt c: gNB configures time duration of one HARQ-ACK bundling group (Tb). - Prioritize HARQ-ACK bundling for Type-2 HARQ-ACK codebook. Proposal 17: Support to multiplex bundled HARQ-ACK bit for multi-PDSCHs by a DCI with the first sub-codebook - This is applicable to the case where CBG operation is configured or not |
| [18] Apple | Proposal 10: Configure the number of PDSCHs per bundling group. - Introduce signaling mechanism to enable generating a HARQ-ACK bit per ‘M’ scheduled PDSCHs in a multi-PDSCH scheduling by performing HARQ-ACK bundling to compress the HARQ-ACK bits overhead. |
| [19] LG Electronics | Proposal #15: If time domain bundling is to be supported for (enhanced) type-2 HARQ-ACK codebook construction, - Only allow bundling operation for all PDSCHs corresponding to each DCI. - HARQ-ACK bits corresponding to single PDSCH reception and multi-PDSCH reception are merged into the same sub-codebook. |
| [20] NTT DOCOMO | Proposal 6: Support time domain HARQ-ACK bundling for multi-PDSCH scheduling. - Time domain bundling is enabled/disabled by RRC parameter, where the enabling/disabling should depend on other configuration, e.g. whether CBG is supported/enabled for multiple PDSCH scheduling, and/or whether two-TB scheduling is supported/enabled for multiple PDSCH scheduling, and/or whether spatial bundling is enabled if two-TB scheduling is supported/enabled for multiple PDSCH scheduling. - Time domain bundling is performed across all PDSCHs scheduled by a DCI. - For type 1 HARQ-ACK feedback, PDSCH candidate occasion determination and pruning procedure is based on the last SLIV. - For type 2 HARQ-ACK feedback, multi-PDSCH DCI scheduled PDSCHs and single-PDSCH DCI scheduled PDSCHs are included in the same sub-codebook. |
| [21] Qualcomm | Proposal 6: For type-2 codebook, in the case of time domain bundling of A/N bits corresponding to PDSCHs scheduled by the same DCI into one bit, a single codebook should be defined at least if CBG operation is not configured. Proposal 7: Allowing different numbers of A/N bits per multi-PDSCH grant, such that for each A/N occasion all the corresponding multi-PDSCH grants will have the same A/N bits, however, from one A/N occasion to another we can allow different number A/N bits per grant - If time domain bundling is enabled, then the bundling pattern can be changed from one A/N occasion to another. - Time-domain bundling patterns to be defined via RRC configuration and the active pattern can be changed by MAC-CE or PDCCH. |
| [22] MediaTek | Proposal 1: For Type-2 codebook construction based on the principle of DAI per DCI, support the following PDSCH grouping and HARQ-ACK bit reporting to manage the codebook size. - When a UE is configured with multi-PDSCH scheduling in a cell c, the scheduled PDSCHs from one DCI are grouped into M PDSCH groups based on Rel-15/16 CBG grouping principle - M=min⁡(N,C), where N is the maximum number of PDSCH groups per DCI configured by network and C is the number of scheduled PDSCHs in the DCI. - Let M1=modC,M, K1=C/M, K2=C/M - Each PDSCH group in the first M1 PDSCH groups contains K1 scheduled PDSCHs and each PDSCH group in the remaining PDSCH groups contains K2 scheduled PDSCHs. - UE reports one HARQ-ACK bit for each PDSCH group - If all PDSCHs within a PDSCH group are decoded correctly, UE reports “ACK” - Else, UE reports “NACK” - If M<N, UE will append N-M “NACK” bits after the M HARQ-ACK bits from the M TB groups to construct the codebook |
| Company | Views for type-3 HARQ-ACK codebook |
| [12] Intel | Proposal 9 - Time domain bundling can be applied to Type-3 HARQ-ACK codebook. - HARQ-ACK bits of adjacent HARQ process IDs that are scheduled by the same DCI can be bundled. |

- Summary on Time domain bundling:

Company views on time domain bundling for type-1 HARQ-ACK codebook:

- Option 1: Time domain bundling is performed across all PDSCHs scheduled by a DCI and pruning procedure is based on the last valid SLIVs (after removal of invalid SLIVs)

  - Supported by Huawei, vivo (Alt 1), Fujitsu, Ericsson, Xiaomi, LG Electronics, NTT DOCOMO

  - ZTE’s concern could be addressed by pruning the last valid SLIVs after removing invalid SLIVs?

- Option 2: Time domain bundling is performed across subset of PDSCHs scheduled by a DCI, FFS for pruning procedure

  - Supported by vivo, Nokia, OPPO (w/o details), Intel, Lenovo (w/o details), InterDigital (w/o details), Apple

  - vivo:

    - Alt 2: A set of occasions is determined based on all (valid) SLIVs in each row of the TDRA table, in the same way as the case when time domain bundling is not configured, and time domain bundling is performed for each subset of occasions divided from the set of occasions.

    - Alt 3: A set of occasions is determined based on all (valid) SLIVs in each row of the TDRA table, in the same way as the case when time domain bundling is not configured, and time domain bundling is performed for each subset of occasions divided from the set of occasions.

  - Nokia: Modified TDRA table is used in the codebook determination and TDRA rows are modified by keeping the last SLIV(s) of the row corresponding to the number of bundled HARQ-ACK bit(s) and removing other SLIVs from that row.

  - Intel: For each row in TDRA table, N SLIVs are selected and associated with N bundled HARQ-ACK of the row. A modified TDRA table can be obtained with each row only containing the N selected SLIVs.

  - Apple: The bundling operation is performed after the codebook is constructed.

Company views on time domain bundling for type-2 HARQ-ACK codebook:

- Option 1: Time domain bundling is performed across all PDSCHs scheduled by a DCI and corresponding HARQ-ACK bit belongs to the first sub-codebook.

  - Supported by Huawei, vivo (Alt 1), LG Electronics, NTT DOCOMO

- Option 2: Time domain bundling is performed across subset of PDSCHs scheduled by a DCI, FFS for how to determine the subset of scheduled PDSCHs

  - Supported by vivo (Alt 2), Nokia, CATT, OPPO, Ericsson, Lenovo, NEC

  - Nokia, Ericsson, Samsung: Configure the number of bundling groups

  - Samsung, Apple: Configure the number of PDSCHs per bundling group

  - NEC, MediaTek: Similar grouping way as CBG

  - Qualcomm: Time domain bundling pattern can be configured by higher layer parameter.

  - Samsung: Configure the time duration of bundling group

Company views on time domain bundling for type-3 HARQ-ACK codebook:

- Intel: HARQ-ACK bits of adjacent HARQ process IDs that are scheduled by the same DCI can be bundled.

[Moderator’s note] The following points can be observed from Moderator’s point of view:

- More than 10 companies are suggesting to support time-domain bundling operation for type-1 and type-2 HARQ-ACK codebook design.

- Some companies suggest different options for each type of HARQ-ACK codebook, however, it would be reasonable to provide the same granularity of time domain bundling to type-1 and type-2 HARQ-ACK codebook.

- For type-1 HARQ-ACK codebook, the majority of companies prefers Option 1, several companies prefer Option 2 but without any detailed designs.

- For type-2 HARQ-ACK codebook, slight majority companies prefer Option 2 but the details on how to define/configure a subset of scheduled PDSCHs are diverged among companies.

Considering this is the last RAN1 meeting for Rel-17 finalization, it must be realistic to go with Option 1 otherwise, time domain bundling operation will be dropped in Rel-17 since it is hard to converge with other options. This issue is indicated as “HIGH” since it is related to RRC parameter discussion.

- [HIGH] Proposal #3.1 (Time domain bundling):

- For FR2-2, introduce new RRC parameters to enable time domain bundling operation for type-1 HARQ-ACK codebook and type-2 HARQ-ACK codebook generation, respectively.

  - If the RRC parameter enables time domain bundling operation,

    - Logical AND operation is performed across all valid PDSCHs scheduled by a DCI.

    - For type-1 HARQ-ACK codebook, reuse Rel-16 codebook generation procedure by determining PDSCH reception occasion based on the last valid SLIV after removing invalid SLIVs.

    - For type-2 HARQ-ACK codebook, HARQ-ACK bits corresponding to single PDSCH reception and multi-PDSCH reception are merged into the same sub-codebook.

Companies are encouraged to provide views on Proposal #3.1.

| Company | Views |
| --- | --- |
| Samsung | We support the proposal. |
| DOCOMO | We don’t support the sub-bullet for “type 1 HARQ-ACK codebook” saying “based on last valid SLIV”. Does it mean the PDSCH-to-HARQ timing is based on the last valid PDSCH? We think PDSCH-to-HARQ timing should be based on the last scheduled PDSCH instead of last valid PDSCH. Considering the case of PDSCH repetition, the K1 is relative to the last repetition instead of the last valid repetition. In our understanding, for type 1 HARQ-ACK codebook with time domain bundling, Rel-16 procedure can be reused, but the pruning should take each SLIV in the row into account. For example, if the last SLIV overlaps with UL symbol, the position/candidate occasion will not be removed if any SLIV in the SLIV doesn’t overlap with UL symbol. |
| Nokia/NSB | For the progress, we are fine with the proposal. |
| Lenovo, Motorola Mobility | We don’t support the proposal. Majority of companies support option 2, at least for type-2 HARQ-ACK codebook. In our view, the enhancement is quite straightforward to determine the bundling pattern. For every maximum number of PDSCHs, a sub-set for time-bundling can be configured by RRC. |
| Qualcomm | We support the proposal |
| Futurewei | Ok with the proposal. |
| Xiaomi | Agree in general. But for the second bullet for Type 1 HARQ-ACK codebook, some modification may be needed as illustrated by DOCOMO. |
| Panasonic | For time bundling for type-2 HARQ-ACK codebook, we would like to clarify this statement “HARQ-ACK bits corresponding to single PDSCH reception and multi-PDSCH reception are merged into the same sub-codebook”. Does it mean that HARQ-ACK bits of single-PDSCH scheduling (by a DCI) and multi-PDSCH scheduling (by another DCI) are merged? For Type-2 HARQ-ACK codebook, we only support that time domain bundling is performed across all PDSCHs scheduled by a DCI and corresponding HARQ-ACK bits belongs to the first sub-codebook. |
| OPPO | We don’t support the proposal. We support option 2 for time domain bundling. The subset of PDSCHs for performing time domain bundling can be configured by gNB. |
| Fujitsu | We support the proposal. |
| NEC | We don’t support the sub-bullet for “type 2 HARQ-ACK codebook”, we support option 2: Time domain bundling is performed across subset of PDSCHs scheduled by a DCI for type 2 HARQ-ACK codebook, and the group size can be indicated by RRC. |
| vivo | Support the proposal in general. Regarding the sub-bullet for Type-1 codebook, we share the similar view as DOCOMO. Firstly, the last configured SLIV for each row should be considered for the PDSCH-to-HARQ timing, otherwise, additional operation such as determining the effective K1 for the last valid SLIV should be considered before reusing Rel-16 procedure. Hence, “The last valid SLIV” should be changed to “The last configured SLIV”. Secondly, for a row in the TDRA table if at least a SLIV in the set of SLIVs configured for the row does not collide with any semi-static UL symbol, the row should not be removed. Therefore, “removing invalid SLIVs” should add the condition of “all the SLIVs in the row are invalid.” |
| Huawei, HiSilicon | We support the proposal. Our understanding of the bullet for type 2 HARQ codebook is the HARQ bits for single PDSCH and multi PDSCH are carried in the same sub codebook. |
| Apple | For Type 1 HARQ-ACK codebook, we can agree in general but with a modification based on DOCOMO’s comment. For Type 2 codebook, there are quite a few companies that support option that were not captured 2 i.e. “Supported by vivo (Alt 2), Nokia, CATT, OPPO, Ericsson, Lenovo, NEC, Apple, Samsung, MediaTek, Qualcomm”. Before giving up on Type 2, we may want to see if we can come to an agreement on how to perform Option 2. |
| ZTE, Sanechips | We are a little bit confused about how to determine last valid SLIV. And whether the existing pruning procedure will be affected in this way. If the above issue can be further clarified, we are fine with the current proposal. |
| Intel | If HARQ-ACK for all PDSCHs scheduled by a DCI are bundled into one bit, it means the HARQ-ACK codebook size is either X or 8X. We prefer to allow better granularity of the potential payload size. Especially for Type2 codebook, support multiple bundled bits per DCI has little additional complexity comparing with the case w/o time bundling. If bundling to single bit per DCI is considered in Type1 codebook, it is not clear how to derive/use last valid SLIV. In general, for a K1 value, the last valid SLIV of the rows in TDRA table can be mapped to slot n-K1-x, where x can be different for different rows. It requires new K1 extension mechanism. If we reuse the Rel-15/16 handling of PDSCH repetition, i.e., the occasion for candidate PDSCH is generated by the last configured SLIV of any row (i.e. the last SLIV maps to slot n-K1), the whole procedure is much simpler. A last SLIV of a row is considered as valid if only one SLIV of the row is valid for PDSCH transmission by checking TDD UL-DL configuration. |
| Ericsson | For Type-1 codebook, we support Proposal 3.1 once the issue regarding scheduled/valid SLIV is resolved. The good thing about this proposal for Type-1 is that the Rel-16 codebook generation procedure can be reused "as is" when all ACK-NACKs bundled into 1 bit. For Type-2 codebook, however, we do not support Proposal 3.1. As commented by other companies, we support Option 2 with a configurable number of HARQ bundling groups for better configuration control trading-off re-transmission efficiency and feedback overhead. As commented by Intel, this comes with little extra complexity. Furthermore, it is a superset of the "full bundling" case, i.e., all ACK-NACK bundled into 1 bit. Please remove "For FR2-2" It will be discussed separately in UE features whether there is any differentiation between FR2-1 and FR2-2. |
| Moderator | - Supported by Samsung, Nokia, Qualcomm, Futurewei, Fujitsu, Huawei - Need clarification on type-1 HARQ-ACK codebook generation: NTT DOCOMO, Xiaomi, vivo, ZTE, Intel, Ericsson - Further enhancement for type-2 HARQ-ACK codebook: Lenovo, NEC, Apple, Intel, Ericsson - Objected by Fujitsu First of all, regarding scheduled or valid SLIV for type-1 HARQ codebook, what I assumed was like the following figure from Fujitsu. Current procedure of draft Rel-17 213 spec: - Set K1 and set R are defined by configured SLIVs - Removing invalid SLIVs - Pruning procedure based on proper pairs of {k1, r} What I intended was to just change the 3rd step, as follows: - Set K1 and set R are defined by configured SLIV - Removing invalid SLIVs - Pruning procedure based on the last SLIV for each r among proper pairs of {k1, r} Please let me know if you have a different opinion, and suggest better wording to make the above intention clearer. Next, for the enhancement of type-2 HARQ-ACK codebook, my first question to the proponents is that why do we need to provide different granularities of time domain bundling depending on HARQ-ACK codebook type? Furthermore, even though 5 companies explicitly shows their preference on improved time domain bundling particularly for type-2 HARQ-ACK codebook, I’m not sure whether what they have in mind for the improvement is aligned and can be acceptable to all other companies. With these regards, I made some modifications as follows. |

- [HIGH] Proposal #3.1a (Time domain bundling):

- Introduce new RRC parameters to enable time domain bundling operation for type-1 HARQ-ACK codebook and type-2 HARQ-ACK codebook generation, respectively.

  - If the RRC parameter enables time domain bundling operation,

    - Logical AND operation is performed across all valid PDSCHs scheduled by a DCI.

    - For type-1 HARQ-ACK codebook, reuse Rel-16 codebook generation procedure as much as possible and pruning procedure is performed based on the last valid SLIV.

      - Note: K1 timing is determined based on the previous agreement.

    - For type-2 HARQ-ACK codebook, HARQ-ACK bits corresponding to single PDSCH scheduling case and multi-PDSCH scheduling case are carried in the same sub-codebook.

Companies are encouraged to provide views on Proposal #3.1a.

| Company | Views |
| --- | --- |
| Ericsson | We still prefer a configurable number of HARQ bundling groups for Type-2 codebook (Option 2). In answer to the Moderator's question about what bundling approach to use for Option 2, the only approach that does not suffer an ambiguity issue is the one in which the number of HARQ bundling groups is RRC configured. With this approach, regardless of the number of scheduled PDSCHs, the feedback overhead is the same, and it is robust to missed DCI(s). The other approaches, e.g., the one in which the number of PDSCHs per bundling group is RRC configured, suffer a robustness issue if the UE misses one or more DCIs since the UE does not know how many PDSCHs were scheduled. Moreover, Option 2 with configurable number of HARQ bundling groups is very simple and robust with very little specification effort. Hence, we think it should be supported for Type 2 codebook. There is nothing to say that the same approach is needed for Type 1 codebook since the construction approach for Type 1 is totally different, and would take time to agree on how to support time domain bundling, and will most likely result in more specification effort. |
| Samsung | The intention of time domain bundling is to decrease the size of HARQ-ACK bits, for Type-1 HARQ-ACK codebook if the last valid PDSCH is used to determine the HARQ-ACK bits, UE needs to consider all the possible candidates of last SLIV. K1 extension is needed. Extension of K1 will increase the size of HARQ-ACK codebook which is not preferred. On the contrary, if the last scheduled SLIV is used as reference, K1 extension can be avoided. To avoid increase the size of HARQ-ACK codebook, we prefer to use the last scheduled SLIV as reference, also, for simplicity, we suggest to add a restriction that “UE does not expect the last schedule SLIV overlaps with a semi-static UL symbol” For Type-2, we are also fine with multiple bundles per DCI. |
| OPPO | We do not support this proposal. We share the same view with Ericsson. A configurable number of HARQ bundling groups for Type-2 codebook (Option 2) would provide a more reliability performance over HARQ bundling across all PDSCHs (Option 1), and the spec effort for Option 2 is acceptable. |
| Fujitsu | We support the proposal. For Type-2 codebook, we do not have a strong view. |
| Intel | We are still not clear how does it work based on last valid SLIV for EACH row. In the following figure/example, the last valid SLIV is 1-0 and 2-1 in different slots for the two rows respectively. Is it the intention to extend single K1 (K1=2 in Figure 1) to multiple K1 values? For example, SLIV 1-0 and 2-1 are the last valid SLIVs of the two rows. After extension, the two K1 values are: K1=3 for SLIV 1-0 and K1=2 for SLIV 2-1? Or, no extension of K1, we only have 1 DL slot as PDSCH candidate DL slot, and we check overlap on last valid SLIV 1-0 and 2-1 as if they are in the same slot K1=2? If they're overlapped, what is UE behavior? UE generates 2 bits for row 1 and row 2 respectively? Or UE does not expect gNB to schedule both row 1 and row 2 for the same PUCCH, so UE can report 1 bit for this DL slot ? Anyway, as also commented by other companies, we prefer to follow existing Rel-15 and Rel-16 mTRP handling PDSCH repetition. That is, for a PUCCH slot n and K1, if at least one SLIV is valid for PDSCH transmission for a row, the last configured SLIV (slot n-K1) of the row is considered as valid SLIV, then occasions are allocated based on all valid SLIVs in slot n-K1. For Type2 codebook, we still prefer to allow multiple bundled bits per DCI. Time bundling can be enabled for Type2 codebook by a very simple extension. It is sufficient to replace the number of HARQ-ACK bits per DCI with the number of bundled bits per DCI, then the Type2 codebook with time bundling can be generated. Our preference is to support multiple bundled bits for both Type1 and Type2 codebook. However, to save some standardization effort, we think it is acceptable to limit time bundling to Type2 codebook only. |
| Panasonic | In proposal#3.1a, we still have a bit confusion about time bundling operation for type-2 HARQ-ACK codebook. In our understanding, there are two interpretations as follows - Interpretation 1: Only one of single PDSCH scheduling case and multi-PDSCH scheduling case is particularly configured at a certain time, but both single PDSCH scheduling case and multi-PDSCH scheduling case can use the same sub-codebook. - Interpretation 2: Single PDSCH scheduling case is separately configured for serving cell A and multi-PDSCH scheduling case is separately configured for serving cell B at the same time. Time bundling bits corresponding for single PDSCH scheduling case and multi-PDSCH scheduling case are carried in the same sub-codebook. For interpretation 1, it is straightforward. However, for interpretation 2, it is necessary to additionally differentiate which bits correspond to single PDSCH scheduling case and multi-PDSCH scheduling case, respectively. Other interpretation is not precluded, but we do not have it in mind as for now. Therefore, we would like to ask proponents and FL further clarify the sub-sub-bullet for Type-2 HARQ-ACK codebook. |
| Lenovo, Motorola Mobility | For type-2 HARQ-ACK codebook, we tend to agree with Ericsson and think that the number of time-bundling groups configured by RRC seems to be the most robust way. With this option, we don’t see an issue with accepting above for type-2 HARQ-ACK codebook |
| DOCOMO | For type 2 sub-codebook, we are fine with the proposal. For type 1 sub-codebook, we don’t support the pruning based on the last valid SLIV. We share same view as Samsung that K1 extension is not needed if type 1 codebook is generated based on last scheduled SLIV. In our understanding, the situation is quite similar to slot based PDSCH repetition case. As in current specification, for slot-based PDSCH repetition, type 1 HARQ-ACK CB is generated based on the last scheduled repetition, while UL collision pruning is based on each repetition. Similarly, for multi-PDSCH scheduling, UL collision will be checked for all SLIVs in the row. If there is any one SLIV not collided with UL, the position of the last scheduled SLIV will be kept regardless of the last scheduled SLIV overlaps with UL or not. |
| Transsion | We are fine with the proposal. |
| Moderator | Regarding type-1 HARQ-ACK codebook, some clarifications seem needed for better understanding each other. To Samsung, Intel, and NTT DOCOMO, Is it correct understanding that you assume the different methods for set K1 and set R composition, depending on whether time domain bundling is configured or not? What I thought was to have unified method for set K1 and set R composition for Rel-17 multi-PDSCH scheduling DCI. Especially in Intel’s example, K1 set is extended, SLIV 1-0 in slot n-3 for row 0 and SLIV 2-1 in slot n-2 for row 1 are pruned, and the outcome is two PDSCH reception occasions. Based on your description, we may have the following two approaches: - Approach 1 - Set K1 is extended and set R is determined, same way with Rel-17 multi-PDSCH scheduling DCI regardless of whether time domain bundling is configured or not - Invalid SLIVs are removed based on endorsed Rel-17 213 spec, that is, if HARQ-ACK information for PDSCH time resource derived by row r in slot nD cannot be provided in slot nU - Pruning procedure is performed only with the last SLIV for each row index of TDRA table - Approach 2 - Set K1 and set R are determined based on the last configured SLIV, different from Rel-17 multi-PDSCH scheduling DCI w/o time domain bundling configuration - Invalid SLIVs are removed only when all SLIVs in a row index are invalid - Pruning procedure is performed with the remaining rows To all, Please check above two approaches and indicate your preference (and check if I captured two approaches correctly) To Panasonic, I’m not sure if I understood Panasonic’s argument well. It seems that both interpretations are valid. Regardless of whether multi-PDSCH scheduling DCI schedules the same or different serving cell with the one scheduled by single-PDSCH scheduling DCI, DAI values in each DCI are counted in turn (carrier-first, then slot-second). The number of HARQ-ACK bit corresponding to multi-PDSCH scheduling DCI or single-PDSCH scheduling DCI is always one, so there is no ambiguity issue on whether the DCI schedules single PDSCH or multiple PDSCHs. To proponents of supporting HARQ-ACK bundling of sub-set of scheduled PDSCHs for type-2 HARQ-ACK codebook, Please provide more details to see if we can converge somewhere in this week. - What RRC parameter configures (e.g., the number of PDSCH groups)? - The method to compose a PDSCH group depending on the number of scheduled or valid PDSCHs |
| vivo | We generally support approach 2 with removing the “Set K1 and” because K1 is configured by RRC parameters directly. |
| Huawei, HiSilicon | We support the proposal 3.1 For type 1 codebook, the reason to prune the SLIVs according to the last valid SLIV is trying to maintain the flexibility of scheduling when time bundling is configured. If based on the last configured SLIV and the last SLIV is collided with UL slot, the rest SLIVs of this entry cannot be scheduled. We had similar understanding as FL that pruning SLIVs in a slot according to last valid SLIV do not have impact on the previous steps to determine the set of DL slots of PDCCH reception occasion already agreed when time bundling is not configured, i.e. Approaching 1. For type 2 codebook, support multiple bundling bits per DCI is over optimization considering already small overhead in the feedback of type 2 codebook compared with other 2 type of feedback. As for the 2 interpretations mentioned by Panasonic, I think both cases may happen. For example, a UE is configured with TDRA table A including both single SLIV and multi SLIV in serving cell A and TDRA table B including only single SLIV for serving cell B. The HARQ feedback for the two serving cells can be feedback on the same sub-book if HARQ-ACKs for PDSCHs scheduled by single DCI are bundled with 1 bit. In such case, there is no difference from the existing type 2 HARQ-ACK codebook generation. |
| ZTE, Sanechips | We support proposal #3.1a |
| Fujitsu | For the approaches, we have some questions for clarification. Q1: It seems there is a common rule for the approaches. That is, for a K1 value, if at least one SLIV corresponding to the K1 value is valid, it has a candidate PDSCH reception occasion. Is that the correct understanding? Q2: It seems that the difference between approach 1 and approach 2 is kind of the ‘anchor’ DL slot for candidate PDSCH reception occasion. That is, for approach 1, the ‘anchor’ is the DL slot where the last valid SLIV locates, while for approach 2, it is the DL slot where the last configured SLIV locates. Then, they may result in different bit mapping order. Is that correct understanding? If the answers for Q1 and Q2 are yes, considering the draft Rel-17 spec as clarified by Moderator, approach 1 is slightly preferred for less spec impact. And if it is still controversial for down-selection, to make some progress, maybe we can first try to agree the common rule. |
| DOCOMO | Our understanding is approach 2 for type 1 HARQ-ACK CB with time domain bundling. For approach 1, it seems the only method is to perform time domain bundling after K1 set extension and PDSCH candidate occasion determination, which means bundling for different PDSCH candidate occasions. However, it would be complicated to bundle PDSCHs scheduled by one DCI, since one PDSCH candidate occasion may belong to SLIVs in different TDRA rows. As commented above, we think the time domain bundling for multi-PDSCH scheduling can refer to HARQ-ACK for PDSCH repetitions in Rel-15/16. That is, the HARQ-ACK timing is relative to the last scheduled repetition instead of valid repetition. And the UL collision pruning will take each repetition slot into account. We support approach 1 and we don’t support approach 2. |
| Futurewei | We support the proposal #3.1a and prefer approach 1. |
| Apple | For Type 1 codebook, we prefer Approach 1 to have a unified approach for both the bundled and non-bundled case. For Type 2, we are fine with configuring the number of bundling groups. Groups are based on the number of valid PDSCHs similar to approach 1 for the Type 1 codebook for a unified design. |
| Xiaomi | We support Approach 2. with Approach 2, basically multi-PDSCH scheduling can be treated as a single PDSCH on HARQ-ACK feedback behavior. It is simpler than Approach 1 |

Impact of invalid PDSCH on HARQ-ACK feedback

| Company | Views |
| --- | --- |
| [1] Huawei | Proposal 11: For Type-2 HARQ-ACK codebook, NACK should be reported for the invalid SLIV/PDSCH and mapped in a position according the configured SLIVs of the row of TDRA table. |
| [2] Futurewei | Proposal 10. Deprioritize the issue of HARQ-ACK bit ordering for Type-2 HARQ-ACK codebook generation from Rel-17. |
| [3] vivo | Proposal 19: NACK corresponding to an invalid PDSCH is mapped to an HARQ-ACK bit among L HARQ-ACK bits corresponding to the DCI scheduling the invalid PDSCH in a Type-2 codebook, wherein the position of the HARQ-ACK bit among L HARQ-ACK bits is determined by the position of the SLIV corresponding to the invalid PDSCH among all configured SLIVs in the row indicated by the DCI, and every L HARQ-ACK bits correspond to one DCI in the Type-2 codebook. |
| [4] ZTE | Proposal 4: HARQ-ACK information bits for Type 2 codebook can be generated according to the scheduled PDSCH sequence. Wherein, - HARQ-ACK information bit corresponding to a valid PDSCH is determined according to the decoding result. - HARQ-ACK information bit corresponding to a invalid PDSCH is padding as NACK. |
| [6] Nokia | Proposal 8: In case of Type-2 codebook and UE reporting NACK for the PDSCH skipped due to collision with semi-static UL symbol(s), the NACK is inserted to codebook after the HARQ-ACK bits for scheduled & valid PDSCHs. |
| [11] Ericsson | Proposal 17: Introduce a bit sequence manipulation scheme that is not dependent on the actual number of PDSCHs scheduled by a DCI but still able to relocate padding bits to the beginning of a HARQ-ACK codebook, e.g., based on block interleaver + bit reversal, so that the HARQ-ACK codebook is better suited for Polar coding. |
| [15] NEC | Proposal 3: For Alt 1 of type-2 HARQ-ACK codebook determination: - If time domain bundling is supported, similar grouping way as CBG can be reused, and spatial bundling and time bundling should not be simultaneously configured or applied. - If there is a confliction between any of scheduled PDSCHs of a single DCI and uplink symbol(s) indicated by TDD configuration, how to fill the NACK bits for the collision slot(s) needs to be determined. - If there is a confliction between any of scheduled PDSCHs of a single DCI and uplink symbol(s) indicated by TDD configuration, and only 1 actual scheduled PDSCH left in this DCI scheduling, this PDSCH will belong to sub-codebook 1. |
| [16] Samsung | Proposal 20: The HARQ-ACK bits are ordered according to the time order of valid PDSCHs scheduled by a DCI. |
| [19] LG Electronics | Proposal #12: For type-2 HARQ-ACK codebook generation, HARQ-ACK information bit ordering is based on configured SLIV position in the indicated TDRA row index, regardless of validity of scheduled PDSCH, followed by NACK padding for the last NHARQ-ACKmax -NTB,cDL⋅NPDSCH,c HARQ-ACK information bits. |
| [21] Qualcomm | Proposal 5: The NACK bits of the skipped PDSCHs should be appended at the end of the A/N bits corresponding to its scheduling DCI. |

- Summary on HARQ-ACK bit ordering for type-2 HARQ-ACK codebook:

Agreement: (RAN1#106bis-e)

For a PDSCH that is scheduled by multi-PDSCH scheduling DCI and is skipped due to collision with semi-static UL symbol(s),

- For Type-1 HARQ-ACK codebook generation, the PDSCH is not considered and the HARQ-ACK bit corresponding to the PDSCH is not reported by UE.

  - Note: Rel-16 procedure can be reused to handle this case.

- For Type-2 HARQ-ACK codebook generation, UE reports NACK for the PDSCH.

  - FFS on HARQ-ACK bit ordering

- Note: Codebook generation in case time domain bundling is enabled can be separately discussed if time domain bundling is supported.

Company views on HARQ-ACK bit ordering for type-2 HARQ-ACK codebook:

- Alt 1: Bit ordering according to the configured SLIVs of the row of TDRA table, regardless of the validity of scheduled PDSCH

  - Supported by Huawei, vivo, LG Electronics

- Alt 2: Bit ordering for valid PDSCHs first, then NACK padding for the remaining bits

  - Supported by ZTE, Nokia, Samsung, Qualcomm

- Alt 3: Bit ordering based on bit sequence manipulation scheme (see above figure or [11])

  - Supported by Ericsson

[Moderator’s note] Unless technical benefit is clearly shown for Alt 2 or Alt 3, Alt 1 seems straight-forward method since UE can construct HARQ-ACK codebook based on configured SLIV and doesn’t need to care the validity of scheduled PDSCHs. However, more company views are needed to draw a proposal, so companies are encouraged to provide more views on the above three alternatives on HARQ-ACK bit ordering for type-2 HARQ-ACK codebook.

| Company | Views |
| --- | --- |
| Nokia/NSB | Both Alt 1 and Alt 2 are equivalent. We can select either option. |
| Qualcomm | We prefer Alt 2 as it simplifies the procedure, i.e., start with A/N bits of the valid PDSCHs then append NACKs to fix the feedback size per DCI |
| InterDigital | We are fine with Alt 1. |
| Futurewei | We are fine with Alt-1/Alt-2. |
| NEC | We are fine with Alt-1/Alt-2. |
| vivo | We prefer Alt 1 for simplicity. |
| Huawei, HiSilicon | We prefer Alt 1 which requires least spec impact. |
| Apple | Fine with Alt-1/Alt-2. |
| DOCOMO | We are fine with either Alt 1 or Alt 2. |
| Intel | We support Alt 1 since it is the most straightforward solution. Essentially, there will not no performance difference since all alternatives have the same payload size. |
| Ericsson | As summarized above, we prefer Alt-3 which ensures that the UCI payload bits occupy the later bit positions (more reliable bits) to ensure robust polar decoding performance. We cannot accept Alt-2 since that will lead to a degradation in polar decoding performance since the UCI payload bits will be mapped to less reliable bit positions as discussed in our contribution. If any re-ordering is performed, it should result in the UCI payload occupying later bit positions, not earlier positions. |
| Moderator | - Alt 1: Bit ordering according to the configured SLIVs of the row of TDRA table, regardless of the validity of scheduled PDSCH - Supported by Huawei, vivo, LG Electronics, Nokia, InterDigital, Futurewei, NEC, Apple, NT DOCOMO, Intel - Alt 2: Bit ordering for valid PDSCHs first, then NACK padding for the remaining bits - Supported by ZTE, Nokia, Samsung, Qualcomm, Futurewei, NEC, Apple, NTT DOCOMO - Objected by Ericsson - Alt 3: Bit ordering based on bit sequence manipulation scheme (see above figure or [11]) Supported by Ericsson Based on majority view, the following proposal can be made based on Alt 1. This issue is indicated as “HIGH” since it is essential for WI completion. |

- [HIGH] Proposal #3.2 (HARQ-ACK bit ordering):

- For type-2 HARQ-ACK codebook generation, HARQ-ACK bit ordering is based on configured SLIV position in the indicated TDRA row index, regardless of the validity of each scheduled PDSCH.

Companies are encouraged to provide views on Proposal #3.2.

| Company | Views |
| --- | --- |
| Samsung | We can live with the proposal although our first preference is Alt 2. Alt 3 is not acceptable to us, it is optimization and complicates the spec. |
| Fujitsu | We are fine with the proposal. |
| WILUS | We support this Proposal #3.2. |
| Intel | We support the FL proposal |
| Lenovo, Motorola Mobility | Support the FL proposal |
| Transsion | We are fine with Alt 1. |
| Huawei, HiSilicon | Support proposal 3.2 |
| ZTE, Sanechips | Our opinion is not correctly captured in the last round. Our preference is Alt 1, that is, current proposal #3.2 |
| Futurewei | We are ok with Proposal #3.2. |

Remaining issues of Type-1 (semi-static) HARQ-ACK codebook

| Company | Views |
| --- | --- |
| [1] Huawei | Proposal 13: For Type-1 and Type-2 HARQ-ACK codebook construction, option 3 is preferred. |
| [7] CATT | Proposal 9: The scheme for pruning candidate PDSCH occasions is based on number of DCIs that can be scheduled for a given PUCCH carrying HARQ-ACK. |
| [9] OPPO | Proposal 4: Clarify whether one PDSCH/PUSCH of the multi-PDSCH/PUSCH scheduled by a single DCI can be cancelled by a dynamic indication. |
| [11] Ericsson | Proposal 13: Confirm the working assumption from RAN1#106bis-e that UE does not expect to be configured with both of CBG operation and multi-PDSCH scheduling in the same PUCCH cell group with a Type-2 codebook. Extend the configuration striction also to Type-1 codebook and clarify that the configuration restriction applies regardless of whether time domain bundling is applied or not. |
| [19] LG Electronics | Proposal #14: Support to configure CBG-based (re)transmission for a serving cell and configure pdsch-TimeDomainResourceAllocationListForMultiPDSCH for the other serving cell within the same PUCCH cell group, if type-1 HARQ-ACK codebook is configured but type-2 HARQ-ACK codebook is not configured with those serving cells. |

- Summary on Type-1 HARQ-ACK codebook generation:

Working assumption: (RAN1#106bis-e)

UE does not expect to be configured with both of CBG operation and multi-PDSCH scheduling in the same PUCCH cell group with a Type 2 codebook.

- If time bundling operation is supported, this working assumption can be revisited

[Moderator’s note] Three companies expressed their views on whether both of CBG operation and multi-PDSCH scheduling in the same PUCCH cell group can be configured with a Type 1 codebook.

- Alt 1 (in [1] and [11]): Do not support

- Alt 2 (in [19]): Support to configure CBG-based (re)transmission for a serving cell and configure pdsch-TimeDomainResourceAllocationListForMultiPDSCH for the other serving cell within the same PUCCH cell group, if type-1 HARQ-ACK codebook is configured but type-2 HARQ-ACK codebook is not configured with those serving cells.

It should be noted that CBG configuration in cell#1 and multi-PDSCH configuration in cell#2 does not affect type-1 HARQ-ACK codebook at all for which codebook construction is performed per cell basis.

Please feel free to express views on two alternatives and Moderator’s note, if any.

| Company | Views |
| --- | --- |
| vivo | We prefer unified behavior for Type-1/2 HARQ-ACK codebook, i.e. UE does not expect to be configured with both of CBG operation and multi-PDSCH scheduling in the same PUCCH cell group with a Type 1/2 codebook. |
| DOCOMO | We support Alt 1. |
| Intel | We prefer Alt 2. We agree with moderator’s note that supporting the combination does not have any impact on the Type1 codebook generation, since HARQ-ACK bits for each serving cell are generated independently. |
| Ericsson | We prefer unified behavior for Type-1 and Type-2 codebook to reduce implementation and spec complexity. Furthermore, we think the main bullet of the working assumption should be confirmed regardless of codebook type, and regardless of time bundling. |
| Samsung | We prefer Alt 2. We share similar view as Intel. |
| WILUS | We prefer Alt 2 and share the same view as Intel. |
| Futurewei | Prefer Alt 2. |

Remaining issues of Type-2 (dynamic) HARQ-ACK codebook

| Company | Views |
| --- | --- |
| [1] Huawei | Proposal 12: For Type-2 HARQ-ACK codebook, legacy DAI counting procedure can be reused for multiple PDSCH/PUSCH scheduling without enhancement. |
| [3] vivo | Proposal 17: For Type-2 codebook, UE does not expect to be configured with both of CBG operation and multi-PDSCH scheduling in the same PUCCH cell group, irrespective of whether time domain bundling operation is supported or not. Proposal 18: Confirm the following working assumption with one update. Working assumption: UE does not expect to be configured with both of CBG operation and multi-PDSCH scheduling in the same PUCCH cell group with a Type 2 codebook. - If time bundling operation is supported, this working assumption can be revisited |
| [4] ZTE | Proposal 5: Confirm the working assumption as follows: - UE does not expect to be configured with both of CBG operation and multi-PDSCH scheduling in the same PUCCH cell group with a Type 2 codebook if time bundling operation is not configured. |
| [6] Nokia | Proposal 7: If HARQ-ACK time bundling operation is supported and bundles HARQ-ACK feedback for all PDSCHs scheduled by single DCI, UE can be configured with both of CBG operation and multi-PDSCH scheduling in the same PUCCH cell group with a Type 2 codebook. Otherwise UE does not expect to be configured with both of CBG operation and multi-PDSCH scheduling in the same PUCCH cell group with a Type 2 codebook. |
| [7] CATT | Proposal 10: It is confirmed that UE does not expect to be configured with both of CBG operation and multi-PDSCH scheduling in the same PUCCH cell group with a Type 2 codebook. |
| [9] OPPO | Proposal 5: For Type-2 HARQ-ACK codebook construction, HARQ-ACK bits corresponding to CBG-based PDSCH reception and multi-PDSCH reception are merged into the same sub-codebook. |
| [11] Ericsson | Proposal 13: Confirm the working assumption from RAN1#106bis-e that UE does not expect to be configured with both of CBG operation and multi-PDSCH scheduling in the same PUCCH cell group with a Type-2 codebook. Extend the configuration striction also to Type-1 codebook and clarify that the configuration restriction applies regardless of whether time domain bundling is applied or not. |
| [12] Intel | Proposal 6 - Time domain bundling is supported in HARQ-ACK transmission. - The PDSCHs associated with the HARQ-ACKs that are time bundled should be scheduled by the same DCI. - The maximum number of PDSCHs for which HARQ-ACKs are bundled can be configured by high layer. - UE does not expect to be configured with both of CBG operation and time domain bundling for multi-PDSCH scheduling in the same PUCCH cell group. - Confirm the work assumption that UE does not expect to be configured with both of CBG operation and multi-PDSCH scheduling in the same PUCCH cell group with a Type 2 codebook. |
| [13] Xiaomi | Proposal 8: CBG operation should not be configured along with HARQ-ACK bundling. |
| [15] NEC | Proposal 3: For Alt 1 of type-2 HARQ-ACK codebook determination: - If time domain bundling is supported, similar grouping way as CBG can be reused, and spatial bundling and time bundling should not be simultaneously configured or applied. - If there is a confliction between any of scheduled PDSCHs of a single DCI and uplink symbol(s) indicated by TDD configuration, how to fill the NACK bits for the collision slot(s) needs to be determined. - If there is a confliction between any of scheduled PDSCHs of a single DCI and uplink symbol(s) indicated by TDD configuration, and only 1 actual scheduled PDSCH left in this DCI scheduling, this PDSCH will belong to sub-codebook 1. |
| [16] Samsung | Observation 2: Including HARQ-ACK bits for 2 PDSCHs scheduled by a DCI in the first HARQ-ACK sub-codebook complicates the specification with marginal gain. Proposal 18: when a UE supports UE capability type2-HARQ-ACK-Codebook (FG 18-9), and there are >1 DCIs belonging to the same MOs and scheduling PDSCHs to the same serving cell. And these DCIs are configured to be able to schedule multiple PDSCHs. The counting procedure for the PDSCHs scheduled by these DCIs are: - PDSCHs are separated into different sets and each set of PDSCHs are scheduled by the same DCI. PDSCHs are counted separately for different sets. - The counting order between different sets of PDSCHs are based on the reception time of the first PDSCH in each set. |
| [17] InterDigital | Proposal 4: Type-2 HARQ-ACK codebook construction procedure when CBG is configured for a cell within the same PUCCH cell group should be carefully evaluated. |
| [20] NTT DOCOMO | Proposal 5: Confirm the working assumption that UE does not expect to be configured with both of CBG operation and multi-PDSCH scheduling in the same PUCCH cell group with a Type 2 codebook. |
| [21] Qualcomm | Proposal 6: For type-2 codebook, in the case of time domain bundling of A/N bits corresponding to PDSCHs scheduled by the same DCI into one bit, a single codebook should be defined at least if CBG operation is not configured. Proposal 8: If all PDSCHs scheduled by a DCI that schedules multi-PDSCHs (TDRA row has multiple SLIVs) except one PDSCH will not be transmitted due to overlap with semi-static UL symbols, then A/N bit of the valid PDSCH will be carried in the codebook of fallback and single-PDSCH grants. |

- Summary on Type-2 HARQ-ACK codebook generation:

Working assumption: (RAN1#106bis-e)

UE does not expect to be configured with both of CBG operation and multi-PDSCH scheduling in the same PUCCH cell group with a Type 2 codebook.

- If time bundling operation is supported, this working assumption can be revisited

Company views on the above working assumption:

- Confirm the working assumption, with that CBG + multi-PDSCH cannot be simultaneously configured regardless of time bundling configuration

  - Supported by vivo, CATT?, Intel, NTT DOCOMO?

- Confirm the working assumption, with that CBG + multi-PDSCH can be simultaneously configured when time bundling is not configured

  - Supported by ZTE, Nokia, Qualcomm?

- UE does not expect to be configured with both of CBG operation and time domain bundling for multi-PDSCH scheduling in the same PUCCH cell group

  - Supported by Intel, Xiaomi

[Moderator’s note] Whether/how to confirm the above working assumption can be revisited once the decision on time domain bundling is made. But, it is encouraged for companies to provide views on the above proposal, if any.

| Company | Views |
| --- | --- |
| Qualcomm | We are okay with unified design, i.e., confirming the working assumption for the case of time domain bundling |
| Futurewei | Agree that the working assumption can be revisited after the decision on time domain bundling is made. If no agreement can be made over there, the current version that has time domain bundling not configured as a condition can be confirmed. |
| vivo | If no agreement on time domain bundling is made, the sub-bullet should be deleted, then confirm the working assumption. |
| DOCOMO | We support confirming the working assumption regardless of time domain bundling. |
| Intel | It would be fine to simply delete the sub-bullet. By this way, the main bullet applies no matter time bundling is configured or not. |
| Ericsson | We prefer unified behavior to reduce implementation and spec complexity. Hence, we think the main bullet of the working assumption should be confirmed regardless of codebook type, and regardless of time bundling. |

Multi-PUCCH corresponding to single multi-PDSCH DCI

| Company | Views |
| --- | --- |
| [3] vivo | Proposal 13: For multi-PDSCH scheduling, support reporting HARQ-ACK information corresponding to different PDSCHs scheduled by a DCI on different PUCCH(s). Proposal 14: For reporting HARQ-ACK feedback on different PUCCHs, further study how to divide the PDSCHs scheduled by a single DL DCI, as well as indicate or determine more than one PUCCH carrying HARQ-ACK feedback. |
| [4] ZTE | Proposal 6: HARQ-ACK information corresponding to different PDSCHs scheduled by the DCI can be carried by different PUCCH(s) considering HARQ-ACK feedback delay. |
| [6] Nokia | Proposal 4: Single transmission of HARQ feedback per multi-PDSCH DCI is only supported. |
| [9] OPPO | Proposal 6: Separate the scheduled PDSCHs into two groups, consider two PUCCH resources allocated for the two PDSCH groups, an earlier PUCCH is used to report HARQ-ACK information of the earlier PDSCH group. |
| [10] Panasonic | Proposal 8: Not to support HARQ-ACK information corresponding to different PDSCHs scheduled by the DCI can be carried by different PUCCH(s) in Rel. 17. Observation 1: Different PUCCHs for multi-PDSCH scheduling from a span can be achieved by multiple DCIs using the functionality of FG3-5b specified in TR 38.822. |
| [11] Ericsson | Proposal 18: Do not support HARQ-ACK information corresponding to different PDSCHs scheduled by the DCI to be carried by different PUCCH occasions. |
| [13] Xiaomi | Proposal 4: For latency sensitive service, separate HARQ-ACK PUCCH resources for multiple PDSCHs scheduled by single DCI can be considered. |
| [14] Lenovo | Proposal 4: For NR operation between 52.6 GHz and 71 GHz, for HARQ-ACK information corresponding to PDSCHs scheduled by the DCI, different PUCCH(s) can be used where the PUCCH carrying the HARQ-ACK can be transmitted in the middle of non-contiguous PDSCHs transmissions to allow earlier/faster transmission of HARQ-ACK associated with earlier PDSCHs |
| [15] NEC | Proposal 2: HARQ-ACK information corresponding to the PDSCHs scheduled by a single DCI can be carried in an uplink slot or at most 2 uplink slots. |
| [16] Samsung | Proposal 15: HARQ-ACK information corresponding to different PDSCHs scheduled by the DCI carried by different PUCCH(s) is not supported in Rel-17. |
| [17] InterDigital | Observation 1: Supporting only one PUCCH transmission for HARQ-ACK of all the PDSCHs scheduled by a single DCI introduces excessive HARQ-ACK round trip delay and negative impact on the expected performance gains. Proposal 1: Support multiple PUCCHs carrying HARQ information of multiple PDSCHs scheduled by a single DCI. To this end, multiple sub-codebooks, one for each PUCCH, with HARQ-ACK information of a sub-set of scheduled PDSCHSs can be constructed. Proposal 2: To support multiple PUCCHs carrying HARQ-ACK information of a group of PDSCHs scheduled by a single DCI, extend TDRA table such that each row indicates multiple slot offsets (K0 values) corresponding to multiple HARQ-ACK sub codebooks. |
| [21] Qualcomm | Proposal 2: All HARQ-ACK information corresponding to different PDSCHs scheduled by the same DCI to be carried by the same PUCCH. |
| [22] MediaTek | Proposal 2: The HARQ-ACK information corresponding to the PDSCHs scheduled by a DCI should only be carried by single PUCCH to simplify Type-2 codebook design. |

- Summary on whether or not HARQ-ACK information corresponding to different PDSCHs scheduled by the DCI can be carried by different PUCCH(s):

Company views on whether or not HARQ-ACK information corresponding to different PDSCHs scheduled by the DCI can be carried by different PUCCH(s):

- Supported by vivo, ZTE, OPPO, Xiaomi, Lenovo, NEC, IDC

- Objected by Nokia, Panasonic, Ericsson, Samsung, Qualcomm, MediaTek

[Moderator’s note] At least 7 companies suggest to support that HARQ-ACK information corresponding to different PDSCHs scheduled by a DCI is carried by different PUCCHs while 6 companies are against it.

It is observed that the main motivation of this feature is to lower HARQ-ACK feedback delay, which is not well matched with FR2-2 operation. Furthermore, to implement this feature, at least the following aspects should be discussed.

- How to determine resources for two PUCCHs (e.g., K1 timing, PRI)

- How to signal individual DAI values corresponding to two PUCCHs

- Under which condition(s) two PUCCHs are indicated by the DCI (e.g., in case more than N PDSCHs are scheduled)

Considering that the specification impact seems quite large compared to expected advantages, the following proposal can be made. This issue is indicated as “HIGH” since it is essential for WI completion.

- [HIGH] Proposed conclusion #3.5 (two PUCCHs):

- HARQ-ACK information corresponding to different PDSCHs scheduled by a single DCI is not carried over multiple PUCCHs in Rel-17.

Companies are encouraged to provide views on Proposed conclusion #3.5.

| Company | Views |
| --- | --- |
| Samsung | We support the proposal |
| DOCOMO | We are fine with the proposal. |
| Nokia/NSB | We are fine with the proposal. |
| Lenovo, Motorola Mobility | Support the proposal |
| Qualcomm | We support the proposal |
| InterDigital | We do not support this proposal. Supporting multiple PUCCHs for HARQ-ACK information corresponding to different PDSCHs scheduled by a single DCI can support UEs that are only capable of supporting 16 HARQ processes. Also, when the gap between the first and the last PDSCH is high, this can reduce HARQ-ACK roundtrip delay. Further, supporting multiple PUCCH for HARQ-ACK can improve the coverage by reducing the UCI payload. |
| Futurewei | We are ok with the Proposed conclusion #3.5. |
| Xiaomi | We can agree for the progress’s sake. |
| Panasonic | We are fine with the proposal. |
| OPPO | We can accept the proposal for progress. |
| Fujitsu | We support the proposal. |
| NEC | We can accept the proposal for progress. |
| vivo | We share the same view as InterDigital. |
| Huawei, HiSilicon | We support the proposal |
| Apple | We support this proposal |
| ZTE, Sanechips | We support this proposal |
| Intel | We support the FL proposal. The feedback delay is not a real issue, since gNB can always do a proper scheduling. For SCS 480kHz/960kHz, the slot length is only 1/4 or 1/8 of that of SCS 120kHz. Even with multi-PDSCH transmission, the HARQ RTT will not increase a slot. For SCS 120kHz, if fast HARQ-ACK feedback is desired, gNB may schedule small number of PDSCHs by a DCI. |
| Ericsson | Support Proposal #3.5 |
| Moderator | - Supported by Samsung, NTT DOCOMO, Nokia, Lenovo, Qualcomm, Futurewei, Xiaomi, Panasonic, OPPO, Fujitsu, NEC, Huawei, Apple, ZTE, Intel, Ericsson - Objected by InterDigital, vivo To InterDigital and vivo, Given the majority view, could InterDigital and vivo accept this proposal? |
| WILUS | We support this proposal |
| Transsion | We support this proposal. |

HARQ process

| Company | Views |
| --- | --- |
| [11] Ericsson | Observation 1: The HARQ process ID fields in various DCI formats need to be extended to support 32 HARQ processes. The bit field extension can be handled by the on-going work in the Rel-17 NTN WI . Proposal 1: Monitor the progress on feedback-disabled HARQ process and its impact on Type-1 and Type-2 HARQ-ACK codebook construction in the Rel-17 NTN WI to capture any potential conflicts with HARQ-ACK codebook enhancement for multi-PDSCH scheduling in the Rel-17 60GHz WI. |
| [21] Qualcomm | Proposal 3: In case of BWP switching between SCS 120kHz, and 480/960kHz and when different numbers of HARQ processes are configured, consider one of the following options: - Option 1: No retransmission can be allowed over different SCSs. - Option 2: No soft combining is assumed between retransmissions over different SCSs. Proposal 4: To define different numbers of HARQ processes for 480/960kHz SCS and 120kHz SCS, consider one of the following options: - Option 1: Reuse the same parameter in PDSCH-ServingCellConfig and add more values, e.g., 24 and 32. - If UE is configured with more than 16 HARQs and the operating SCS is 120kHz or less, it will assume that number of HARQ processes is 16. - Option 2: Introduce new parameter(s) for SCSs 480kHz/960kHz. |

- Summary (on the number of HARQ processes):

Agreement: (RAN1#106bis-e)

For NR FR2-2 at least for 480/960 kHz SCS, support 32 as the maximum number of HARQ processes for DL and UL, subject to UE capability.

- Note: Up to 32 maximal supported HARQ process number is already agreed in Rel-17 NTN WI.

- Working assumption: The same solution to support up to 32 HARQ process number in Rel-17 NTN WI is reused for NR FR2-2.

Qualcomm brought up two issues when a UE capable of 32 HARQ processes is configured with multiple BWPs having 120/480/960 kHz SCSs in a serving cell:

- Issue 1: Data soft combining after BWP switching

- Issue 2: Whether or not to introduce new higher layer parameter to configure the number of HARQ processes for 480/960 kHz SCS

[Moderator’s note] Given that a single company brought up the above issues, it is encouraged for companies to provide views on them, if any.

| Company | Views |
| --- | --- |
| DOCOMO | We are not sure whether the two issues need to be discussed in RAN1, which seems more like higher layer issues. |
| Intel | We are open to consider up to 32 HARQ processes for SCS 120kHz |
| Moderator | Above two issues brought up by Qualcomm seem valid when 32 HARQ processes are supported for 480/960 kHz SCS but not supported for 120 kHz SCS. To avoid those issues, one way (as Intel pointed out) could be to support 32 as the maximum number of HARQ processes also for 120 kHz SCS in FR 2-2, which seems to be a simple extension and UE capable of 32 HARQ processes for 480/960 kHz SCS would be capable of 32 HARQ processes for 120 kHz SCS as well. With that, the following proposal can be made. This issue is indicated as “HIGH” since it is related to UE feature discussion. |

- [HIGH] Proposal #3.6 (HARQ process):

- For NR FR2-2 for 120 kHz SCS (in addition to 480/960 kHz), support 32 as the maximum number of HARQ processes for DL and UL, subject to UE capability.

Companies are encouraged to provide views on Proposal #3.6.

| Company | Views |
| --- | --- |
| Ericsson | Question: Can Qualcomm confirm that the issue is only within a single carrier configured with multiple BWPs with different numerologies? Is this a likely deployment scenario? |
| Intel | We are supportive to the FL proposal |
| Huawei, HiSilicon | We support proposal 3.6. |
| ZTE, Sanechips | We support proposal #3.6 |
| Futurewei | Support Proposal #3.6. |

Others

| Company | Views |
| --- | --- |
| [13] Xiaomi | Proposal 1: For multi-slot PDSCH scheduling, the HARQ ID for the PDSCH(s) exceeding the COT is/are still reserved. Proposal 2: For multi-slot PDSCH scheduling, the HARQ-ACK PUCCH resource for the scheduled multi-slot PDSCH is determined by the last PDSCH among the multiple PDSCHs scheduled by a single DCI, even if the last PDSCH exceeds the COT. |
| [18] Apple | Observation 1: HARQ-ACK information corresponding to different PDSCHs scheduled by a single DCI carried by different PUCCHs affects the UE complexity, signaling overhead and transmission latency. Proposal 11: RAN1 should decide whether a multi-PxSCH transmission can occur across multiple COTs and the specify the UE HARQ-ACK feedback behavior in the case that one or more of the PDSCH transmissions occurs outside a valid COT. Proposal 12: RAN1 should support a single HARQ-ACK feedback for multi-PDSCH transmissions within a single COT only. Proposal 13: In the case of BWP switching during multi-PxSCH transmission - Option 1: The UE does not expect an UL or DL BWP change on the serving cell after the DCI scheduling the multi-PDSCH transmission and until the PUCCH is transmitted - Option 2: The UE will only send HARQ-ACK bits for the effective K1 values after the BWP switch. |
| [22] MediaTek | Proposal 4: The UCI information bits including HARQ-ACK information bits should reuse the existing PUCCH payload size limit 1706. |

- Summary on other aspects for multi-PDSCH/PUSCH scheduling:

The following issues are brought up by several companies:

- Xiaomi and Apple: Relationship between HARQ-ACK transmission and COT

- Apple: Clarification on BWP switching during multi-PDSCH reception (or multi-PUSCH transmission)

- MediaTek: Reuse the existing PUCCH payload size limit 1706.

[Moderator’s note] Given a small number of inputs for those issues, it is proposed to deprioritize them in this meeting but please feel free to express views on above issues, if any.

| Company | Views |
| --- | --- |
| Futurewei | Ok with deprioritizing these issues. |
| DOCOMO | Fine to deprioritize. |

Reference

- R1-2110831	PDSCH/PUSCH enhancements for 52-71GHz spectrum	Huawei, HiSilicon

- R1-2110876	Enhancements to support PDSCH/PUSCH for Beyond 52.6GHz	FUTUREWEI

- R1-2111002	Remaining issues on PDSCH/PUSCH enhancements for NR operation from 52.6GHz to 71GHz	vivo

- R1-2111078	Discussion on the data channel enhancements for 52.6 to 71GHz	ZTE, Sanechips

- R1-2111147	Considerations on multi-PDSCH/PUSCH with a single DCI and HARQ for NR from 52.6GHz to 71 GHz	Fujitsu

- R1-2111199	PDSCH/PUSCH enhancements	Nokia, Nokia Shanghai Bell

- R1-2111245	PDSCH/PUSCH enhancements for up to 71GHz operation	CATT

- R1-2111247	Some issues on SPS for one DCI scheduling multiple PDSCHs case	CATT

- R1-2111311	Discussion on PDSCH/PUSCH enhancements	OPPO

- R1-2111424	Discussion on PDSCH/PUSCH enhancements for NR 52.6-71 GHz	Panasonic Corporation

- R1-2111468	PDSCH-PUSCH Enhancements	Ericsson

- R1-2111487	Discussion on PDSCH/PUSCH enhancements for extending NR up to 71 GHz	Intel Corporation

- R1-2111565	PDSCH and PUSCH enhancements for NR 52.6-71GHz	Xiaomi

- R1-2111644	PDSCH/PUSCH scheduling enhancements for NR from 52.6 GHz to 71GHz	Lenovo, Motorola Mobility

- R1-2111692	Discussion on PDSCH enhancements supporting NR from 52.6GHz to 71 GHz	NEC

- R1-2111728	PDSCH/PUSCH enhancements for NR from 52.6 GHz to 71 GHz	Samsung

- R1-2111837	Enhancement for PDSCH/PUSCH to support 52.6 GHz-71 GHz band in NR	InterDigital, Inc.

- R1-2111865	Discussion on PDSCH and PUSCH Enhancements	Apple

- R1-2112049	PDSCH/PUSCH enhancements to support NR above 52.6 GHz	LG Electronics

- R1-2112100	PDSCH/PUSCH enhancements for NR from 52.6 to 71 GHz	NTT DOCOMO, INC.

- R1-2112207	PDSCH/PUSCH enhancements for NR in 52.6 to 71GHz band	Qualcomm Incorporated

- R1-2112303	Multi-PDSCH scheduling design for 52.6-71 GHz NR operation	MediaTek Inc.

- Appendix: Previous agreements

- RAN1#104-e

Agreement:

- For a UE and for a serving cell, scheduling multiple PDSCHs by single DL DCI and scheduling multiple PUSCHs by single UL DCI are supported.

  - Each PDSCH or PUSCH has individual/separate TB(s) and each PDSCH/PUSCH is confined within a slot.

  - FFS: The maximum number of PDSCHs or PUSCHs that can be scheduled with a single DCI

  - FFS: Whether multiple PDSCH scheduling applies to 120 kHz in addition to 480 and 960 kHz

  - At least for 120 kHz SCS, single-slot scheduling with slot-based monitoring will still be supported as specified in Rel-15/Rel-16

- The followings will not be considered in this WI.

  - Single DCI to schedule both PDSCH(s) and PUSCH(s)

  - Single DCI to schedule one or multiple TBs where any single TB can be mapped over multiple slots, where mapping is not by repetition

  - Single DCI to schedule N TBs (N>1) where a TB can be repeated over multiple slots (or mini-slots)

- Note: This does not imply that existing slot aggregation and/or repetition for PDSCH and PUSCH by single DCI is precluded for the serving cell.

Agreement:

- For a DCI scheduling multiple PDSCHs, HARQ-ACK information corresponding to PDSCHs scheduled by the DCI is multiplexed with a single PUCCH in a slot that is determined based on K1,

  - where K1 (indicated by the PDSCH-to-HARQ_feedback timing indicator field in the DCI or provided by dl-DataToUL-ACK if the PDSCH-to-HARQ_feedback timing indicator field is not present in the DCI) indicates the slot offset between the slot of the last PDSCH scheduled by the DCI and the slot carrying the HARQ-ACK information corresponding to the scheduled PDSCHs.

    - It is noted that granularity of K1 can be separately discussed.

- FFS: If needed, further discuss whether or not HARQ-ACK information corresponding to different PDSCHs scheduled by the DCI can be carried by different PUCCH(s)

Agreement:

For generating type-2 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, the following alternatives can be considered to DAI counting and will be down-selected in RAN1#104bis-e.

- Alt 1: C-DAI/T-DAI is counted per DCI.

- Alt 2: C-DAI/T-DAI is counted per PDSCH.

- Alt 3: C-DAI/T-DAI is counted per M scheduled PDSCH(s), where M is configurable (e.g., 1, 2, 4, …).

- FFS: Codebook generation details

- FFS: How to signal DAI values (e.g., increase of DAI bits for Alt 2 and Alt 3)

- FFS: Whether to apply time domain bundling of HARQ-ACK feedback

Agreement:

The multi-PUSCH scheduling defined in Rel-16 NR-U is the baseline for multi-PUSCH scheduling in Rel-17.

- FFS: Applicability to multi-PDSCH scheduling.

Agreement:

- For the multi-PUSCH scheduling in Rel-17, study the enhancement of the following in addition to Rel-16 multi-PUSCH scheduling.

  - CBGTI: Whether or not CBG (re)transmission is supported when more than one PUSCHs are scheduled (Already supported when only one PUSCH is scheduled).

  - CSI-request: Whether to apply same or different rule compared to Rel-16 (e.g., the PUSCH that carries the AP-CSI feedback is the first PUSCH that satisfies the multiplexing timeline).

  - TDRA: Down-select among

    - Alt 1: TDRA table is extended such that each row indicates up to [X, FFS for X] multiple PUSCHs (continuous in time-domain). Each PUSCH has a separate SLIV and mapping type. The number of scheduled PUSCHs is signalled by the number of indicated valid SLIVs in the row of the TDRA table signalled in DCI.

    - Alt 2: TDRA table is extended such that each row indicates up to [X, FFS for X] multiple PUSCHs (that can be non-continuous in time-domain). Each PUSCH has a separate SLIV and mapping type. The number of scheduled PUSCHs is signalled by the number of indicated valid SLIVs in the row of the TDRA table signalled in DCI.

    - Alt 3: TDRA table is extended such that each row indicates up to 8 multiple PUSCH groups (that can be non-continuous between PUSCH groups). Each PUSCH group has a separate SLIV, mapping type and number of slots/PUSCHs N. Within each PUSCH group, N PUSCHs occupy the same OFDM symbols indicated by the SLIV and mapping type. The number of scheduled PUSCHs is the sum of number of PUSCHs in all PUSCH groups in the row of the TDRA table signalled in DCI.

  - FDRA: Whether/how to enhance FDRA e.g., by increasing RBG size or changing allocation granularity

  - Frequency hopping: Whether/how to support frequency hopping for scheduled PUSCHs, e.g., inter-PUSCH/intra-PUSCH hopping

  - URLLC related fields such as priority indicator and open-loop power control parameter set indication: Whether/how to apply URLLC related fields for scheduled PUSCHs

  - Applicability to multi-PDSCH scheduling in Rel-17.

  - Note: Other enhancements are not precluded.

- RAN1#104bis-e

Agreement:

- The maximum number of PDSCHs that can be scheduled with a single DCI in Rel-17 is 8 for SCS of 480 and 960 kHz.

  - FFS: Further restrictions for 480 kHz to 4

  - FFS: A UE capability to select between 4 and 8 for 480 kHz SCS

  - Note: Multi-PDSCH scheduling for the case of 120 kHz SCS is still FFS as per prior agreement. This case can be addressed after this FFS has been decided.

- The maximum number of PUSCHs that can be scheduled with a single DCI in Rel-17 is 8.

  - FFS: Further restrictions for 120 kHz and 480 kHz SCS

  - FFS: A UE capability to select between different values for 120 kHz and 480 kHz SCS

Agreement:

For a DCI that can schedule multiple PDSCHs,

- MCS for the 1st TB: This appears only once in the DCI and applies commonly to the first TB of each PDSCH

- NDI for the 1st TB: This is signaled per PDSCH and applies to the first TB of each PDSCH

- RV for the 1st TB: This is signaled per PDSCH, with 2 bits if only a single PDSCH is scheduled or 1 bit for each PDSCH otherwise and applies to the first TB of each PDSCH

- HARQ process number: This applies to the first scheduled PDSCH and is incremented by 1 for subsequent PDSCHs (with modulo operation, if needed)

- FFS:

  - MCS/NDI/RV for the 2nd TB for each PDSCH, including whether scheduling of the 2nd TB for each PDSCH can be supported or not

  - Details of resource allocation related fields such as VRB-to-PRB mapping, PRB bundling size indicator, rate matching indicator, and ZP CSI-RS trigger

  - Whether/how to signal CBGFI/CBGTI if CBGFI/CBGTI is supported for multi-PDSCH scheduling

  - Details of fields that are common with multi-PUSCH scheduling, e.g., TDRA, FDRA, priority indicator, including potential enhancements

Agreement:

- For a DCI that can schedule multiple PUSCHs,

  - TDRA: Alt 2 (TDRA table is extended such that each row indicates up to 8 multiple PUSCHs (that can be non-continuous in time-domain). Each PUSCH has a separate SLIV and mapping type. The number of scheduled PUSCHs is implicitly indicated by the number of indicated valid SLIVs in the row of the TDRA table signalled in DCI.), as per agreement made in RAN1#104-e

    - FFS: signaling details

  - Note: Alt 2 does not preclude continuous resource allocation in time-domain.

- For a DCI that can schedule multiple PDSCHs,

  - TDRA: TDRA table is extended such that each row indicates up to 8 multiple PDSCHs (that can be non-continuous in time-domain). Each PDSCH has a separate SLIV and mapping type. The number of scheduled PDSCHs is implicitly indicated by the number of indicated valid SLIVs in the row of the TDRA table signalled in DCI.

    - FFS: signaling details

  - Note: This does not preclude continuous resource allocation in time-domain.

  - Note: Multi-PDSCH scheduling for the case of 120 kHz SCS is still FFS as per prior agreement. This case can be addressed after this FFS has been decided.

Agreement:

For enhancements of generating type-1 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, the following options can be considered,

- Option 1: The set of candidate PDSCH reception occasions is determined according to each SLIV of each row in the TDRA table and based on extension of K1 set

- Option 1a: The set of candidate PDSCH reception occasions is determined according to each SLIV of each row in the TDRA table

- Option 2: The set of candidate PDSCH reception occasions is determined according to the last SLIV of each row in the TDRA table

- FFS: Codebook generation details, including how to handle the collision with TDD DL/UL configuration and whether/how to extend K1 set based on K1 and slot offset between last PDSCH and other PDSCHs in a row in the TDRA table

Conclusion:

The following is observed for alternative 1 from prior agreement.

- For Alt 1 (C-DAI/T-DAI is counted per DCI) of generating type-2 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs,

  - C-DAI/T-DAI in DL DCI: Same DAI overhead with Rel-16 single-PDSCH DCI

  - T-DAI in UL DCI:

    - In case of single codebook handling feedback for both single and multi-PDSCH scheduling, same DAI overhead with Rel-16 UL DCI

    - In case of separate sub-codebooks, need additional DAI field (with same bit-width of DAI with Rel-16 UL DCI), in UL DCI for all serving cells including a serving cell not configured with multi-PDSCH DCI

      - Note that DAI field increment for this case is similar for the case in Rel-15 where CBG is configured

  - HARQ-ACK codebook generation:

    - A separate sub-codebook can be generated when multi-PDSCH DCI is configured for a serving cell, similar to the way as 2nd sub-codebook is defined to handle CBG-based scheduling

      - FFS: whether single codebook or separate sub-codebooks is(are) generated when multi-PDSCH DCI is configured for a serving cell

      - FFS: how many sub-codebooks are generated when multi-PDSCH DCI is configured for a serving cell and CBG is configured for the serving cell and/or the other serving cell(s)

    - HARQ-ACK payload size is increased compared to single PDSCH scheduling only, since the number of HARQ-ACK bits corresponding to each DAI of the (sub-)codebook for multi-PDSCH DCI in case of separate sub-codebooks (or for all DL DCIs in case of single codebook) depends on the maximum configured number of PDSCHs for multi-PDSCH DCI across serving cells belonging to the same PUCCH cell group.

    - The number of HARQ-ACK bits for multi-PDSCH DCI in case of separate sub-codebooks, or for all DL DCIs in case of single codebook, does not depend on the number of actually scheduled PDSCHs, rather, it is fixed as the maximum configured number of PDSCHs.

    - FFS: time domain bundling of HARQ-ACK feedback, as per agreement in RAN1#104-e

  - Note that multi-PDSCH DCI refers to a DL DCI where at least one entry of the TDRA table allows scheduling more than one PDSCH

Conclusion:

The following is observed for alternative 2 from prior agreement.

- For Alt 2a (C-DAI/T-DAI is counted per PDSCH with a single codebook) of generating type-2 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs,

  - C-DAI/T-DAI in DL DCI: Bit-width can be increased (FFS: by how much), in DL DCI not only for multi-PDSCH DCI but also for single-PDSCH DCI for all serving cells including a serving cell not configured with multi-PDSCH DCI

  - T-DAI in UL DCI: Bit-width can be increased (FFS: by how much), in UL DCI for all serving cells including a serving cell not configured with multi-PDSCH DCI

  - C-DAI/T-DAI in DL DCI and T-DAI in UL DCI shall be designed such that at most 3 consecutive DCI missing can be resolved, same as in Rel-15/16 NR.

    - FFS: details on increment of DAI field size

    - FFS: whether/how to handle the case where different DCI formats (e.g., DCI format 1_0 and DCI format 1_1) have different field sizes for C-DAI/T-DAI

  - HARQ-ACK codebook generation:

    - The number of HARQ-ACK bits depends on the number of scheduled PDSCHs.

    - FFS: ordering of the PDSCHs for DAI counting

    - FFS: time domain bundling of HARQ-ACK feedback, as per agreement in RAN1#104-e

  - Note that multi-PDSCH DCI refers to a DL DCI where at least one entry of the TDRA table allows scheduling more than one PDSCH

Conclusion:

The following is observed for alternative 3 from prior agreement.

- For Alt 3 (C-DAI/T-DAI is counted per M scheduled PDSCH(s), where M is configurable) of generating type-2 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs,

  - If M equals to the maximum configured number of PDSCHs, Alt 3 is the same with Alt 1, if the same number of codebooks is assumed.

  - Else if M equals to 1, Alt 3 is the same with Alt 2.

  - Otherwise (i.e., 1<M<the maximum configured number of PDSCHs), Alt 3 is similar to Alt 2, except that

    - The number of HARQ-ACK bits corresponding to each DAI increases by M times.

    - NACK bits may be padded if the number of scheduled PDSCHs is not an integer multiple of M.

    - FFS: details on DAI field size

    - FFS: whether single codebook or separate sub-codebooks is(are) generated when multi-PDSCH DCI is configured for a serving cell

  - In addition, new RRC parameter to configure M needs to be introduced.

  - Note that multi-PDSCH DCI refers to a DL DCI where at least one entry of the TDRA table allows scheduling more than one PDSCH

- RAN1#105-e

Agreement:

- Do not use fallback DCI (i.e., DCI formats 0_0 and 1_0) for multi-PDSCH/PUSCH scheduling.

- Use DCI format 0_1 to schedule multiple PUSCHs with a single DCI.

- Use DCI format 1_1 to schedule multiple PDSCHs with a single DCI.

Conclusion:

For a DCI that can schedule multiple PUSCHs,

- CSI-request: When the DCI schedules M PUSCHs, the PUSCH that carries the aperiodic CSI feedback is M-th scheduled PUSCH for M <= 2, or (M-1)-th scheduled PUSCH for M > 2.

Agreement:

- If a PDSCH among multiple PDSCHs that are scheduled by a single DCI is collided with uplink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, the UE does not receive the PDSCH.

  - FFS on how to handle HARQ-related issue for the PDSCH (e.g., HARQ process numbering)

- The UE does not expect to be scheduled with multiple PDSCHs by a single DCI, where every PDSCH is collided with uplink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated.

- If a PUSCH among multiple PUSCHs that are scheduled by a single DCI is collided with downlink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, the UE does not transmit the PUSCH.

  - FFS on how to handle HARQ-related issue for the PUSCH (e.g., HARQ process numbering)

- The UE does not expect to be scheduled with multiple PUSCHs by a single DCI, where every PUSCH is collided with downlink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated.

Agreement:

For TDRA in a DCI that can schedule multiple PDSCHs (or PUSCHs),

- A row of the TDRA table can indicate PDSCHs (or PUSCHs) that are in consecutive or non-consecutive slots.

  - FFS: The maximum value of the gap between two consecutively scheduled PDSCHs or between two consecutively scheduled PUSCHs

  - FFS: The maximum value of the gap between the first scheduled PDSCH and the last scheduled PDSCH or between the first scheduled PUSCH and the last scheduled PUSCH

  - FFS: Details to introduce the gap between PDSCHs or between PUSCHs

Agreement:

For enhancements of generating type-1 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, the set of candidate PDSCH reception occasions corresponding to a UL slot with HARQ-ACK transmission is determined based on a set of DL slots and a set of SLIVs corresponding to each DL slot belonging to the set of DL slots.

- The set of DL slots includes all the unique DL slots that can be scheduled by any row index r of TDRA table in DCI indicating the UL slot as HARQ-ACK feedback timing.

- The set of SLIVs corresponding to a DL slot (belonging to the set of DL slots) at least include all the SLIVs that can be scheduled within the DL slot by any row index r of TDRA table in DCI indicating the UL slot as HARQ-ACK feedback timing.

  - FFS: details of further pruning of the set of SLIVs

  - FFS: impact if receiving more than one PDSCH in a slot is allowed, e.g., handling of overlapped SLIVs from different rows in the same and different DL slot

  - FFS impact of time domain bundling, if supported

Agreement:

- At least for 120 kHz SCS, for a DCI that can schedule multiple PUSCHs and is configured with the TDRA table containing at least one row with multiple SLIVs,

  - If CBG-based (re)transmission is configured, CBGTI field is not present when more than one PUSCHs are scheduled, but is present when a single PUSCH is scheduled, as in Rel-16.

- FFS:

  - For 480/960 kHz SCS, whether to apply the same behavior with 120 kHz SCS or not to support CBGTI field configuration in the DCI that can schedule multiple PUSCHs

  - For a DCI that can schedule multiple PDSCHs and is configured with the TDRA table containing at least one row with multiple SLIVs, whether/how to configure CBGTI/CBGFI fields

Agreement:

If Alt 1 (C-DAI/T-DAI is counted per DCI) is adopted for generating type-2 HARQ-ACK codebook corresponding to a DCI that can schedule multiple PDSCHs,

- At least two sub-codebooks are generated for a PUCCH cell group where

  - The first sub-codebook is for the following cases:

    - Any DCI that is not configured with CBG-based scheduling and is configured with TDRA table containing rows each with a single SLIV

    - Any DCI that is not configured with CBG-based scheduling and is configured with TDRA table containing at least one row with multiple SLIVs and schedules only a single PDSCH

  - The second sub-codebook is for the following case:

    - Any DCI that is configured with TDRA table containing at least one row with multiple SLIVs and schedules multiple PDSCHs

      - FFS: Methods (if needed) to align the size of HARQ-ACK feedback corresponding to different DCIs

      - FFS: Whether HARQ-ACK bits for 2 PDSCHs scheduled by this DCI can be included in the first sub-codebook in some cases

  - FFS: SPS PDSCH release, SCell dormancy indication without scheduled PDSCH

- FFS: 2 or 3 sub-codebooks if CBG is configured for a serving cell in the PUCCH cell group

- FFS: impact of time domain bundling, if supported, e.g., the number of sub-codebooks including single codebook if all A/N bits are bundled into a single bit per DCI

Agreement:

If Alt 2 (C-DAI/T-DAI is counted per PDSCH) is adopted for generating type-2 HARQ-ACK codebook corresponding to a DCI that can schedule multiple PDSCHs,

- PDSCH(s) scheduled by a single DCI is counted firstly, serving cell(s) in the same PUCCH cell group and same PDCCH monitoring occasion is counted secondly, and PDCCH monitoring occasion(s) is counted thirdly.

- The bit width of counter DAI field in fallback DCI (i.e., DCI formats 0_0 and 1_0) remains the same as in Rel-15 NR.

- Note: The DAI bit width and number of sub-codebooks shall ensure that at most 3 consecutive missed DCIs can be resolved, same as in Rel-15/16 NR

  - This shall not impose additional gNB’s scheduling restriction.

- In case where CBG retransmission is not configured for any serving cell in a same PUCCH cell group, the number of bits for each of counter DAI and total DAI in non-fallback DCI is extended (if needed) at least based on

  - The number of SLIVs associated with the row indexes in TDRA table

    - FFS: details

- FFS: the case with configuration of CBG retransmission

- FFS: the number of sub-codebooks

- FFS: for the UE indicating by type2-HARQ-ACK-Codebook support for more than one PDSCH reception on a serving cell that are scheduled from a same PDCCH monitoring occasion

- RAN1#106-e

Working assumption:

Scheduling multiple PDSCHs by single DL DCI applies to 120 kHz in addition to 480 and 960 kHz at least in FR2-2.

- FFS: Further limitations on maximum number of PDSCHs

Agreement:

Adopt Alt 1 (C-DAI/T-DAI is counted per DCI) for generating type-2 HARQ-ACK codebook corresponding to a DCI that can schedule multiple PDSCHs.

Agreement:

- The maximum number of PDSCHs/PUSCHs that can be scheduled with a single DCI in Rel-17 is 8 for SCS of 120, 480 and 960 kHz.

- FFS: Whether UE capability is introduced for restricting the maximum number of PDSCHs or PUSCHs that can be scheduled with a single DCI

Agreement:

If a scheduled PDSCH/PUSCH is dropped due to collision with UL/DL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, HARQ process number increment is skipped for the PDSCH/PUSCH and applied only for valid PDSCH(s)/PUSCH(s).

- FFS: HARQ process number determination for the case where a scheduled PDSCH/PUSCH collides with a flexible symbol (indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated) if the UE is configured to monitor DCI format 2_0.

Agreement:

- For a DCI that can schedule multiple PUSCHs,

  - Priority indicator and open loop power control parameter set indication fields are applied to all of scheduled PUSCHs.

- For a DCI that can schedule multiple PDSCHs,

  - Priority indicator field is applied to all of scheduled PDSCHs.

Agreement:

For TDRA in a DCI that can schedule multiple PDSCHs (or PUSCHs),

- A row of the TDRA table can indicate PDSCHs (or PUSCHs) that are in consecutive or non-consecutive slots, by configuring {SLIV, mapping type, scheduling offset K0 (or K2)} for each PDSCH (or PUSCH) in the row of TDRA table.

- Note: Whether and how to reduce RRC overhead is left to RAN2.

Agreement:

For a DCI that can schedule multiple PDSCHs,

- Each of VRB-to-PRB mapping, PRB bundling size indicator, ZP-CSI-RS trigger, and rate matching indicator fields appears only once in the DCI.

- VRB-to-PRB mapping and PRB bundling size indicator fields are applied to all the PDSCHs scheduled by the DCI.

- For ZP-CSI-RS trigger field, the triggered aperiodic ZP CSI-RS is applied to all the slot(s) in which the PDSCH(s) scheduled by the DCI are contained.

- When receiving a PDSCH scheduled by the DCI, the REs corresponding to configured resources in rateMatchPatternGroup1 or rateMatchPatternGroup2 (according to indication of rate matching indicator field) are not available for the scheduled PDSCH.

Working assumption:

For NR FR2-2, two codeword transmission is supported, subject to UE capability.

- RRC parameter configures whether two codeword transmission is enabled or disabled.

  - FFS: Details on signaling of MCS/NDI/RV for the second TB in a DCI that can schedule multiple PDSCHs when two codeword transmission is enabled

  - FFS: Whether unified or separate parameter to enable/disable 2-TB for single and for multiple PDSCH scheduling

  - Strive to minimize the increase in the number of bits in the DCI needed to support this feature

Agreement:

- For single TRP operation, for 480/960 kHz SCS,

  - FFS: A UE does not expect to be scheduled with more than one PDSCH in a slot, by a single DCI or multiple DCIs.

  - FFS: A UE does not expect to be scheduled with more than one PUSCH in a slot, by a single DCI or multiple DCIs.

- For single TRP operation, for 120 kHz SCS (same as current specification for FR2-1 for PUSCH),

  - Subject to UE capability, a UE can be scheduled with more than one PDSCH in a slot, by a single DCI or multiple DCIs.

  - Subject to UE capability, a UE can be scheduled with more than one PUSCH in a slot, by a single DCI or multiple DCIs.

- FFS for multi-TRP operation

- Note: The optimization of HARQ codebook size for Type 1 or Type 2 codebook design is considered as a low priority in Rel-17 (this does not preclude HARQ ACK bundling in time domain).

- The agreement made in RAN1#105-e is revised as follows.

Agreement: (RAN1#105-e) For enhancements of generating type-1 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, the set of candidate PDSCH reception occasions corresponding to a UL slot with HARQ-ACK transmission is determined based on a set of DL slots and a set of SLIVs corresponding to each DL slot belonging to the set of DL slots. - The set of DL slots contains all the unique DL slots determined by considering all combinations of the configured K1 values and the configured rows of the TDRA table. - The set of SLIVs corresponding to a DL slot (belonging to the set of DL slots) contains all the SLIVs for that slot determined by considering all combinations of the configured K1 values and the configured rows of the TDRA table. - The Rel-16 procedure is reused for determining the candidate PDSCH reception occasions for the set of SLIVs corresponding to each DL slot belonging to the set of DL slots - Note: The Rel-16 procedure already handles pruning of multiple SLIVs corresponding to a DL slot, for both UEs that are and are not capable of receiving multiple PDSCHs per slot - FFS impact of time domain bundling, if supported

Agreement:

Consider the following options to construct type-2 HARQ-ACK codebook when CBG operation is configured, and down-select to one of the following options in RAN1#106bis-e.

- Option 1: HARQ-ACK bits corresponding to CBG-based PDSCH reception and multi-PDSCH reception are merged into the same sub-codebook.

- Option 2: HARQ-ACK bits corresponding to CBG-based PDSCH reception and HARQ-ACK bits corresponding to multi-PDSCH reception are contained in separate sub-codebooks.

- Option 3: UE does not expect to be configured with both of CBG operation and multi-PDSCH scheduling in the same PUCCH cell group.

- Note: Multi-PDSCH reception refers to the case where multiple PDSCHs are scheduled by a DCI that is configured with TDRA table containing at least one row with multiple SLIVs.

Agreement:

For NR FR2-2 at least for 480/960 kHz SCS, support 32 as the maximum number of HARQ processes for DL and UL, subject to UE capability.

- Note: Up to 32 maximal supported HARQ process number is already agreed in Rel-17 NTN WI.

- Working assumption: The same solution to support up to 32 HARQ process number in Rel-17 NTN WI is reused for NR FR2-2.

- RAN1#106bis-e

Agreement:

Confirm the working assumption from RAN1#106-e with the following modification.

Working assumption: (RAN1#106-e)

Scheduling multiple PDSCHs by single DL DCI applies to 120 kHz in addition to 480 and 960 kHz at least in FR2-2.

- FFS: Further limitations on maximum number of PDSCHs

- Note: Further limitations (in addition to what was agreed earlier) on the maximum number of PDSCHs or PUSCHs can be separately discussed for all SCSs.

Working assumption:

UE does not expect to be configured with both of CBG operation and multi-PDSCH scheduling in the same PUCCH cell group with a Type 2 codebook.

- If time bundling operation is supported, this working assumption can be revisited

Agreement:

For a PDSCH that is scheduled by multi-PDSCH scheduling DCI and is skipped due to collision with semi-static UL symbol(s),

- For Type-1 HARQ-ACK codebook generation, the PDSCH is not considered and the HARQ-ACK bit corresponding to the PDSCH is not reported by UE.

  - Note: Rel-16 procedure can be reused to handle this case.

- For Type-2 HARQ-ACK codebook generation, UE reports NACK for the PDSCH.

  - FFS on HARQ-ACK bit ordering

- Note: Codebook generation in case time domain bundling is enabled can be separately discussed if time domain bundling is supported.

Agreement:

For generating type-2 HARQ-ACK codebook corresponding to a DCI that can schedule multiple PDSCHs,

- HARQ-ACK bit corresponding to SPS PDSCH release or SCell dormancy indication without scheduled PDSCH, belongs to the first sub-codebook (which is defined in the previous agreement made in RAN1#105-e)

Agreement:

For two multi-PDSCH (or two multi-PUSCH) scheduling DCIs, UE does not expect any of the scheduled PDSCHs (or PUSCHs) and the scheduling DCI to lead to out-of-order scheduling.

- FFS: whether to allow OOO scheduling for the following two cases:

  - for the case of one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, where multi-PDSCH (or multi-PUSCH) scheduling DCI schedules more than one PDSCH (or PUSCH)

  - for the case where two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but two multi-PDSCH (or multi-PUSCH) scheduling DCIs have overlapping spans, where the span is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV

- Note: The above FFS aspect applies only to multi-PDSCH and multi-PUSCH scheduling with single DCI

Agreement:

For multiple PDSCHs (or PUSCHs) scheduled by a single DCI,

- Rel-15/16 behavior that is described in TS 38.213 Clauses 11 and 11.1 for a PDSCH (or PUSCH) indicated by DCI also applies for multiple PDSCHs (or PUSCHs) schedule by a single DCI.

- If one of multiple PDSCHs (or PUSCHs) scheduled by the DCI collides with a flexible symbol (indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated),

  - If that PUSCH is collided with SSB symbols indicated by ssb-PositionsInBurst [or symbol(s) indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set], the HARQ process number increment is skipped for the PUSCH.

  - Otherwise, the HARQ process number increment is not skipped for that PDSCH (or PUSCH).

Conclusion:

For a DCI that can scheduled multiple PDSCHs (or PUSCHs), HARQ process number indicated in the DCI is applied to the first valid PDSCH (or PUSCH).

- Note: This is the consequence of previous agreements.

Agreement:

For single TRP operation, for 480/960 kHz SCS,

- A UE does not expect to be scheduled with more than one unicast PDSCH in a slot, by a single DCI or multiple DCIs.

- A UE does not expect to be scheduled with more than one PUSCH in a slot, by a single DCI or multiple DCIs.

Agreement:

For a DCI that can schedule multiple PDSCHs, and if RRC parameter configures that two codeword transmission is enabled,

- MCS for the 2nd TB: This appears only once in the DCI and applies commonly to the 2nd TB of each PDSCH

- NDI for the 2nd TB: This is signaled per PDSCH and applies to the 2nd TB of each PDSCH

- RV for the 2nd TB: This is signaled per PDSCH, with 2 bits if only a single PDSCH is scheduled or 1 bit for each PDSCH otherwise and applies to the 2nd TB of each PDSCH

- FFS: the maximum number of PDSCHs when 2 TB is enabled or when 2 TB is scheduled