3GPP TSG RAN WG1 Meeting #107e	R1-2112690

November 11th – November 19th, 2021

Agenda item:    8.2.6

Source:              Moderator (Qualcomm Incorporated)

Title:                  FL summary of channel access mechanism for 52.6GHz-71GHz band, ver03

Document for:  Discussion and Decision

# Introduction

This paper summarizes the channel access related proposals submitted to agenda item 8.2.6 in RAN1-107e.

# Summary of contributions

The section summarises key proposals and observations from submitted contributions.  Discussion points arising from each group of topics are captured separately in subsections.

Agreement:

The baseline ED threshold can be computed as

EDT=-80 dBm+10*log10PmaxPout+10*log10(Operating Channel BW in MHz)

Where Pout is RF output power (EIRP) and Pmax is the RF output power limit, Pout≤Pmax.

- FFS: Further adjustment on ED threshold based on the sensing beam and the transmission beam (further adjustment should not violate EDT requirements as per regulations)

- FFS: If Pout is max output EIRP of the device or instantaneous output EIRP

- FFS definition of Operating Channel BW

- FFS: Whether ED threshold for NR-U and NR-U coexistence scenarios (eg, at regulation level) can be appropriately relaxed compared with the threshold of coexistence between NR-U and Wi-Fi.

- FFS: EDT when the COT has time varying transmission beams and varying EIRP

ED Threshold computation FFS Items

| Company | Key Proposals/Observations/Positions |
| --- | --- |
| Huawei HiSilicon | Proposal 3: For operation in NR-U-60, the term ‘Operating Channel Bandwidth’ in the agreed baseline EDT formula is defined as the ‘LBT Bandwidth’ or the ‘bandwidth on which a channel access procedure is performed in shared spectrum’ Proposal 4: For operation in NR-U-60, the agreed baseline EDT formula should be adjusted such that, for a given RF output power (EIRP), the EDT proportionally increases with the effective transmit beamforming gain of the potential following transmission(s) by the device. Proposal 5: For operation in NR-U-60, when LBT is used, adopt the following formula to capture the potential adjustment to the baseline EDT formula based on the transmit beamforming gain: • GTX is the effective transmit antenna gain at the potential transmitter [dBi] • GTX,max is the maximum supported transmit antenna gain [dBi] • a is a scaling factor such that 0≤ a≤ 1 Proposal 6: For operation in NR-U-60, when LBT is used, the sensing beamforming gain of the LBT beam is deducted from the detected energy level before comparing it to the EDT. Proposal 7: The value of the adjustment to ED threshold based on the sensing beam and the transmission beam is zero if the transmit antenna gain reaches which is the maximum supported transmit antenna gain. Note: The effective transmit antenna gain includes the overall gain of the antenna element and the antenna array (beamforming gain).. |
| FUTUREWEI | Proposal 2: Utilize a separate EDT for each sensing beam. Proposal 3: Support additional adjustment to Energy Detection computation/threshold to include transmit beamforming and/or sensing beam. The value of the adjustment to ED threshold based on the sensing beam and the transmission beam should be zero if pseudo-omni (near 0dBi) gain sensing beam is used. |
| vivo | Proposal 5: The LBT bandwidth should be used as the operating channel bandwidth for EDT evaluation. Proposal 6: The EDT calculation equation is applicable for omni-directional sensing beam. Proposal 7: If UE is using a directional beam for sensing, either EDT is adjusted higher/looser by the antenna gain or the measurement energy is adjusted lower by the antenna gain before measured energy is compared with EDT. |
| ZTE Sanechips | Proposal 5: For NR-U and NR-U coexistence scenarios, ED threshold can be considered to be appropriately relaxed compared with the threshold of coexistence between NR-U and Wi-Fi. |
| Nokia Nokia Shanghai Bell | Observation 2: In LBT, energy is measured after antenna and antenna gain is included in the energy measurement. Proposal 4: Further adjustment of EDT based on the transmit or sensing beam gain is not specified. |
| CATT | Proposal 1: Adjustment value should be considered for the baseline ED threshold. Proposal 2: For adjustment value on baseline EDT, at least beamforming gain difference between the transmission beam and sensing beam should be considered. |
| Ericsson | Observation 3 ED threshold defined in EN 302 567 v2.2.0 is a function of the transmission’s EIRP Pout, which includes the transmission beamforming gain. It does not include the sensing beamforming gain. Observation 4 Including the sensing beam’s beamforming gain in EDT would require complex and accurate calibration of the compensation between the device’s antenna and the reference horn antenna used in ETSI EN 302 567 v2.2.1 regulatory test Proposal 3 Further adjustment on ED threshold based on the transmission and sensing beamforming gains could be up to implementation while not violating EDT requirements as per regulations. |
| Intel | Proposal 3: In the EDT determination, define “Operating channel BW” as the LBT BW using what RAN1 has defined for both single and multi-carrier operation. Proposal 4: When operating in unlicensed 60 GHz band, the ED threshold calculation shall account for the sensing beam used to perform the LBT procedure through an additional component which is added to the already agreed ED threshold formula. Proposal 5: In case the network is able to assess the absence of any other incumbent technology, the ED threshold value that a device may use during the LBT procedure is up to the gNB and may be configured via higher layer signaling. |
| NEC | Proposal 1: The ED threshold for directional LBT based channel access procedure should consider additional adjustment reflecting sensing/transmitting beamforming gain and relationship between transmission beam(s) and sensing beam. |
| Samsung | Proposal 1: ED threshold should depend on:  Whether other technology sharing the channel is absent or not on a long-term basis;  Beam parameters including beamforming gain and/or beam direction for transmission and/or receiving. |
| InterDigital Inc. | Proposal 8: Adapt EDT to account for beamforming gain of the sensing beam. Proposal 9: EDT of directional LBT is increased compared to EDT of omni-directional LBT. Proposal 10: The Operating Channel BW used in the EDT formula is equivalent to the LBT BW. |
| LG Electronics | Proposal #14: The intended transmissions for WA in RAN1#106bis-e meeting is needed to be clarified and it is necessary to consider the case when the UE is scheduled for additional transmission with Pout, which was not considered in initial EDT calculation after COT initiation. Proposal #16: The additional ED threshold adjustment should be introduced if UE is using a directional beam for transmission/sensing with positive antenna gain (i.e., lowered ED threshold compare to omni beam) Proposal #17: The energy should be derived from the measurement after antenna and the energy sensing indicator (ESI) can be defined to decide the IDLE/BUSY of channel during CCA procedure considering the energy measurement can be performed by each of antennas, individually. Proposal #18: Consider he additional ED threshold adjustment (e.g., 3 dB penalty) for a UE indicating beamCorrespondenceWithoutUL-BeamSweeping={0} and before the beam management procedure. |
| Qualcomm Incorporated | Proposal 2: Support additional adjustment to Energy Detection computation/threshold whenever the sensing beam has a lower beamforming gain than the transmission beam. |
| TCL Communications | Observation 1: The threshold is adaptable for LBT in 60GHz unlicensed band. A signaling with the similar function of ul-toDL-COT-SharingED-Threshold-r16 is necessary. |
| Xiaomi | Proposal 7: Support further adjustment on ED threshold based on the sensing and transmission beam. |

Working assumption:

- For Pout in EDT determination, define Pout as the maximum EIRP of the node determining EDT during a COT.

| Company | Key Proposals/Observations/Positions |
| --- | --- |
| Huawei HiSilicon | Proposal 1: For operation in NR-U-60, confirm the working assumption on Pout definition in RAN1 #104bis-e in its original form with Pout defined as the maximum EIRP of the intended transmissions by the node determining EDT during a COT. Proposal 2: For defining Pout, define the ‘transmission burst’ stated in the HS EN 302 567, if need be, as a set of transmissions from the node determining EDT without any gaps, or with gaps no greater than X μs. - Value of X is specified as one of 3us or 8us |
| FUTUREWEI | Proposal 1: For Pout in EDT determination, define Pout as the maximum EIRP of the intended transmissions by the node determining EDT during a COT.  EIRP of an intended transmission in a COT can be determined as the product of transmit power and beamforming gain estimated for that transmission. Observation 1. Using common Pout (common EDT) for multiple sensing beams can limit spatial reuse. |
| vivo | Proposal 3: The transmission burst is a set of transmissions from gNB/UE from one or more transmission beams which are “covered” by a sensing beam without any gaps greater than [16us]. Proposal 4: For Pout in EDT determination, define Pout as the maximum of mean EIRP of transmission burst for the node determining EDT during a COT. |
| ZTE Sanechips | Proposal 3: In order to align with the specification requirement of EN 302 567, the previous working assumption on the definition of Pout in EDT determination can be updated as follows:  For Pout in EDT determination, define Pout as the the maximum of mean EIRP of each transmission burst within the COT. |
| Nokia Nokia Shanghai Bell | Proposal 3: For Pout in EDT determination, define Pout as the maximum EIRP of the intended transmissions by the node determining EDT during a COT. |
| Ericsson | Observation 1 According to the regulations it is sufficient to use only the initiating device’s Pout to determine EDT. Observation 2 The argument to use both EIRPs from the initiating and responding devices to determine Pout for a node initiating a COT is insufficient as the responding device may also use a different bandwidth than the initiating device. Proposal 1 Confirm that Pout corresponds to the maximum or the maximum of the mean output power EIRPs of the intended transmissions or transmission bursts in a COT. The exact method to estimate Pout is left for implementation. Proposal 2 Confirm that Pout is estimated only based on the node initiating the COT even for COT sharing cases. |
| Intel | Proposal 2: For Pout in EDT determination, define Pout as the maximum EIRP of the node determining EDT during a COT. |
| NEC | Proposal 2: Confirm the working assumption on Pout definition as following: For Pout in EDT determination, define Pout as the maximum EIRP of the node determining EDT during a COT. |
| Apple | Proposal 1: Support Pout adjustment based on directional sensing, where no adjustment is needed with omni/quasi-omni sensing, similar as 802.11ad. Proposal 2: All transmission bursts within a COT should within the limitation of sensing EIRP and directivity. |
| LG Electronics | Proposal #15: When the COT is initiated at slot = and the additional UL transmission(s) with an EIRP larger than the Pout used for the initial EDT calculation are scheduled after slot. symbols, the maximum transmission power of UL transmission transmitted within the remaining COT may be limited to the max EIRP, or alternatively, UE can simply drop the corresponding UL transmission. |
| NTT DOCOMO INC. | Proposal 1: Revisit the definition of “A DL transmission burst” and “A UL transmission burst” by considering the following two aspects:  A duration of sensing performed by Cat-2 LBT  A duration of transient period |
| Qualcomm Incorporated | Proposal 1: Confirm the working assumption on Pout definition in RAN1 #104bis-e with the following updates:  For Pout in EDT determination, define Pout to be at least the maximum of mean EIRP of each transmission burst during the COT at the node initiating the COT. |
| OPPO | Proposal 4: When the measured energy with directional beam is larger than the measured energy with omni beam, the EDT value should be adjusted higher. Observation 1: The working assumption for Pout might limit the usage of the UE COT sharing. |
| Charter Communications | Proposal 1: Confirm the working assumption for the EDT definition: Pout is defined as the maximum EIRP of the node determining EDT during a COT. |
| GDCNI | Observation 1: By identifying maximal transmission-over-sensing gain(Xmax) and maximal sensing-over-transmission gain(Ymax) as mentioned above for similar shape and unaligned direction configuration of sensing beam and transmission beam, there could be a performance loss of (Xmax + Ymax)dB at the most in terms of transmit power in order to avoid violating regulations when there is no direction information about the neighboring user. Observation 2: By identifying maximal transmission-over-sensing gain(Xmax) or maximal sensing-over-transmission gain(Ymax) as mentioned above for different shape and aligned direction configuration of sensing beam and transmission beam, there could be a performance loss of Xmax or Ymax dB at the most in terms of transmit power in order to avoid violating regulations when there is no direction information about the neighboring user. Observation 3: For general case, by identifying maximal transmission-over-sensing gain(Xmax)dB and maximal sensing-over-transmission gain(Ymax)dB, power control with Xmax can be used to avoid violating regulations, and then a maximum performance loss is expected to be (Xmax + Ymax)dB. Proposal1: For directional sensing, maximal transmission-over-sensing gain and maximal sensing-over-transmission gain can defined as parameters for power control and performance evaluation. |

### First round discussions

From discussion from RAN1-106bis-e there is strong support to introduce additional EDT adjustment (19 companies support vs 2 companies not support). The following discussions are trying to further clarify the details.

Discussion 2.1.1-1 (closed)

It seems that we don’t have common understanding on which measured energy is used to compare with EDT, even before we consider if additional EDT adjustment is needed. There are two views below. Please provide your understanding

- View 1. The energy at gNB/UE is measured after antenna and antenna gain is included in the energy measurement. The energy measurement is directly compared with EDT

  - Support: Qualcomm, Huawei, OPPO, ZTE, DOCOMO, Futurewei, Ericsson, Lenovo, Nokia, Intel, Xiaomi, vivo, Transsion, InterDigital

- View 2. The energy used at gNB/UE is measured before antenna and does not include antenna gain. To come up with this measurement, the gNB/UE measures the energy after antenna with antenna gain included and need to deduct the antenna gain from the energy measured. After deduction, the energy is compared with EDT.

  - Support: Apple, HW

Please provide your view if not captured above:

| Company | View |
| --- | --- |
| Nokia, NSB | Our understanding of the ETSI regulation is according to view 1 |
| Futurewei | Our position was incorrectly captured. We fixed it. |
| Ericsson | Our understanding is View 1 in devices. However, for the adaptivity test in the ETSI regulations, the power of the interference at the receiver is calibrated using a reference antenna and the energy is measured after the “reference antenna” and only the “reference antenna gain”, which is non-zero and unknown, is included in the measurement. The following can be observed from EN 302 567 v2.2.1, testing clauses 5.3.7. and 5.3.8. - The tests in EN 302 567 use omni/quasi omni sensing for testing the adaptivity clause. - The interference power at the receiver is calibrated using a single standard reference antenna according to clause 5.3.7.2, which is different from the internal antennas of the device. This reference antenna is not as sophisticated as the antenna panel in the device, but a single omni/quasi-omni directional antenna, i.e., a horn antenna as stated in the Annex C.3 of EN 302 567 v2.2.1. - The reference RSSI value at the receiver compared with the EDT to determine whether channel is idle/busy is the RSSI after the standard reference antenna. Therefore, the reference RSSI value already includes the physical antenna gains of the reference antenna, which is non-zero but not defined in EN 302 567 v2.2.1[4]. Therefore, if it is agreed to include sensing gain in the determination of EDT (or internal adjustment based on sensing beamforming gain), it requires an accuracy calibration of the compensation BF gain considering: i) the offset of the narrow sensing beam (compared to the test interference direction or the transmission beam) and ii) the offset between the physical directional gains from the test device’s antenna panel and the reference antenna. Without an accuracy calibration of the compensation BF gain, the resulting EDT could be higher than the one in HS or the internal adjusted RSSI value at the receiver is lower than the reference RSSI value after the reference antenna, and hence violates the HS’s requirements. |
| Lenovo, Motorola Mobility | In our understanding, the energy measurement that is directly compared with EDT is energy at gNB/UE includes the antenna gain – basically same understanding as view 1 |
| Intel | Our understanding is that View 1 correctly captures the procedure defined within the ETSI BRAN adaptivity test. |
| Apple | EN 302 567 does not specify how the energy is measured, only the threshold is specified. Therefore, both views can be considered to meet the regulation requirement. However, considering 802.11ad, which is the primary technology in use in the band, omni-directional sensing is used and no adjustment for transmission beamforming gain is applied. If view 1 is used, then it will result in 3GPP technology has competitive disadvantaged comparing to 802.11ad, when omni-sensing is used. View 2 is aligned with what 802.11ad is doing therefore we think view 2 should be adopted. Otherwise, NR FR2-2 will be disadvantaged to 802.11ad/ay on CCS aspects. Moderator: There is a problem with View 2. The device will only measure with whatever beam is used (unless we always use omni beam for sensing). So for View 2 to work, I guess Apple’s proposal is to measure with narrow beam, collect a measurement, and deduct the antenna gain from the measurement to come up with an energy measurement “pretending” an omni antenna is used for sensing. However, the node does not know if the jammer is coming from the direction the sensing beam is pointing to. Then if the jammer is actually coming from a side lobe, the energy measurement post adjustment will be lower than what an omni beam will sense. Reply to modulator: Frist, we recommend reword view 2 to stating “The energy at gNB/UE is measured after antenna with 0dBi antenna gain” to clarify vivo’s comment. The reason we discuss view 1 versus view 2 is triggered by whether we adjust omni sensing by adding the beamforming gain. In our view, we should not, since this will cause NR device competitive dis-advantage to Wigig device. When UE measures with directional sensing with bit-1 beam correspondence, the jammer in the sidelobe should not stop the UE transmission since the sensing beam is supposed to cover the transmission beam. UE is not transmitting in the jammer’s direction. Moderator: Actually the discussion is more fundamental than if an adjustment to EDT is needed. The node will measure with a certain beam and the energy measurement will be after the antenna gain. The node cannot physically measure before the antenna, while the measurement before the antenna is some kind of reference device or genie device. The question now is, with the energy measured with antenna gain, does the node need to compensate for the antenna gain and “approximately” find out what the reference measurement will see. My example is about a jammer very close the node and measured energy is very strong and larger than EDT, though it is not from the main beam. If we deduct antenna gain from measurement, assuming the jammer is from the main lobe, the adjusted measurement will be below EDT and the transmission will happen, which will hurt the jammer. I feel this is not fair. |
| Xiaomi | Our understanding is view1 In R16 NR-U when directional LBT is not introduced, the antenna UE use to sensing the channel may not be real “omni-directional”, and antenna gain exists. And we don’t deduct the antenna gain when comparing to EDT. We think it is also OK to keep the same way in R17. |
| LG Electronics | Some questions for clarification: - For View 1, what if multiple antennas are being used? For example, if two energy values are achieved from two antennas, which one of them can be used for the comparison with ED threshold? Moderator: I guess you are talking about multiple antenna port case. For this, as in Rel.16 NR-U, we don’t address the problem when there are multiple antenna ports each with an energy measurement. How to handle this is left for implementation. - To Apple: Just to understand your comments. Could you elaborate on the reason why NR FR2-2 devices can be disadvantaged compared to WiGig devices if we go with View 1? |
| OPPO | As correctly captured by the FL, view 1 is our understanding. Pout in the baseline ED threshold includes transmission beamforming gain, so it’s not reasonable to use the energy measured before antenna when comparing with EDT. |
| Apple | Response to LG: Wigig used omni sensing and the sensed energy is compared to EDT directly. Following view 1, the if NR FR2-2 device used omni sensing, the measured energy needs to add the beamforming gain, then compared to EDT. For example, with omni sensing, measured energy is -55dBm, and EDT is -50dBm based on equation, and beamforming gain is 10dB. In this case, Wigig device will pass CCA by comparing -55dBm to -50dBm. However for NR device with omni sensing, beamforming gain needs to be adjusted which gives -55dBm +10dBm = -45dBm energy level. Then NR FR2-2 device will fail the CCA and can not transmit. This creates competitive disadvantage for NR FR2-2. Moderator: I believe 11ad is not forcing to use omni for sensing. It is just an implementation choice. It is not a real omni as well. If I remember right, it is a beam that the antenna gain is not lower than the transmission beam by more than a certain level. The 11ad design is more like “allow a wider beam to be used for sensing, but there is no additional EDT adjustment applied” |
| vivo | In our opinion, the energy at gNB/UE is measured after antenna and antenna gain is included in the energy measurement, i.e. view 1. However, we think the antenna gain for the EDT in the reregulate is 0 dBi. But it looks like that companies have different understanding of that. Therefore, we suggest to clarify the feasible scenarios for the EDT in the regulation. That is to say, the EDT in the previous agreement assuming 0 dBi receiving antenna gain (which is explicitly indicated in previous version of EN 302567), or the beamforming gain of the receiving antenna is the same as that of the transmission antenna. EDT=-80 dBm+10*log10PmaxPout+10*log10(Operating Channel BW in MHz) We would like to hear companies’ views on this issue. Moderator: Please see the issue I brought up in reply to Apple |
| NEC | We prefer View 1 which captures the effect of antenna gain on measured energy to make a calibration for being compared with EDT. |
| Transsion | Our understanding is View 1, and our position is captured. |
| ZTE, Sanechips | View 1 is our understanding, that is, the energy at gNB/UE is measured after antenna and antenna gain is included in the energy measurement. |
| DOCOMO | View 1 as captured above. |
| CATT | View 1 is our understanding. |
| Ericsson 2 | To Moderator: Please look at the issue we had raised regarding adjustment of EDT threshold and the ETSI regulations test. 3GPP devices could potentially fail the regulatory test if adjustment is allowed and not calibrated with the reference antenna in the ETSI test which is not a simple task. Moderator: I am not sure how you interpret the test procedure in 302 567. From what I see, it does not say anything about using a separate device to measure how much energy is received, and does not say anything about how to compare with a threshold. It only says, if a jammer is injected, the UUT should stop transmission. It does not say how far away the jammer should be (so it should be close enough). There is no calibration process as well. It is not a strict test to pass in the beginning. I am confused. Reply to Moderator: Please look at the following highlighted text form EN 302 567 v2.2.1. Although it is not stated in the Test clause for adaptivity, it uses the same test setup and is mentioned in a different clause. The power at the receiver is calibrated to about -47 dBm/2.16 GHz. However, this is done using a standard reference antenna (highlighted below in red from 5.3.7.2), i.e., a horn antenna (highlighted below in green from Annex C.3) connected to a Power Meter. Therefore, the UUT must have internal antennas with gains like or compensated with the reference antenna to be able to pass the test. If the adjustment (both in term of physical gain of the antenna array and the processing BF gain of the UUT) is not accurate, it will not ensure the devices will pass the regulatory test. EN 302 567 v2.2.1: 5.3.7.2 Test Method … Figure 2: Test Set-up for verifying the adjacent channel rejection of an equipment The Receive Power of the CW Blocking Signal should be calibrated, e.g. by replacing the UUT with a standard reference antenna connected to a Power Meter. … 5.3.8.2 Test method … Step 1: The UUT may connect to a companion device during the test. The set-up for radiated testing will include signal generator able to simulate an interferer, spectrum analyser or oscilloscope with two channels, with each channel connected to separate directed antenna, one directed to UUT and the other to the companion device, UUT and the companion device to connect with, but the signal generator does not yet generate any signals at this point in time. The spectrum analyser or oscilloscope is used to monitor the transmissions of the UUT in response to the interference signal. When performing testing on a UUT with a directional antenna system (such as array capable of beam-forming), the wanted communication link (between UUT and companion device) and the interference test signals shall be aligned to the direction corresponding to the UUT's maximum EIRP. ….. Step 4: Verification of reaction to the interference signal The analyser shall be used to monitor the transmissions of the UUT and the companion device on the selected operating channel after the interference signal was injected. This may require the analyser sweep to be triggered by the start of the interfering signal. Using the procedure defined in clause 5.3.8.3, it shall be verified that: ….. Annex C (normative): Test sites and arrangements for radiated measurements ….. C.3 Substitution antenna The substitution antenna shall be used to replace the equipment under test in substitution measurements. For measurements below 1 GHz the substitution antenna shall be a half wavelength dipole resonant at the frequency under consideration, or a shortened dipole, calibrated to the half wavelength dipole. For measurements between 1 GHz and 4 GHz either a half wavelength dipole or a horn radiator may be used. For measurements above 4 GHz a horn radiator shall be used. The centre of this antenna shall coincide with the reference point of the test sample it has replaced. This reference point shall be the volume centre of the sample when its antenna is mounted inside the cabinet, or the point where an outside antenna is connected to the cabinet. |
| Samsung | We agree to View 1. |
| InterDigital | We’ve added our support for view 1. |
| Huawei, HiSilicon | We think the formulation of the discussion is inaccurate. In View 2, we do not understand why the antenna gain needs to be deducted from the energy measured if it already does NOT include the antenna gain. We think that View 1 and View 2 should be modified as follows - Modified View 1. The energy at gNB/UE is measured after antenna and antenna gain is included in the energy measurement. The gNB/UE needs to deduct the antenna gain from the energy measured. After deduction, the energy is compared with EDT. - Modified View 2. The energy at gNB/UE is measured before antenna and does not include antenna gain. The energy measurement is directly compared with EDT. If FL agrees with our modifications, we can support modified View 1 |

Discussion 2.1.1-2 (closed)

On additional adjustment to EDT if introduced, at least at UE side, the following alternatives on how to adjust the EDT can be considered

- Scenario 1. For UE indicates a capability for beam correspondence with beamCorrespondenceWithoutUL-BeamSweeping ={1} and when the same TX beam is used for sensing, no additional EDT adjustment is introduced

  - Support: Ericsson, Intel, TCL, ZTE,NEC, OPPO, Huawei, LGE, Futurewei,InterDigital, DOCOMO, Nokia, Lenovo, Transsion, Samsung

- Scenario 2: For other cases (other than scenario 1) where sensing beam “covers” the transmission beam and has lower beamforming gain, the EDT is adjusted lower/tighter by the difference between the antenna gains of the sensing beam and transmission beam

  - Note: This is to make sure the same jammer at the transmission beam direction can be detected with the lower gain sensing beam

  - Support: Intel, ZTE, TCL, NEC, OPPO, Lenovo, Transsion, Samsung, IDC

  - Oppose: Ericsson, Nokia, DCM

  - Leave to Implementation: Docomo

- Scenario 3: If UE uses omni beam for sensing, no additional EDT adjustment is introduced. If UE is using a directional beam for sensing (with positive antenna gain, so the UE will see higher energy level compared with omni sensing beam), either EDT is adjusted higher/looser by the antenna gain or the measurement energy is adjusted lower by the antenna gain before measured energy is compared with EDT

  - Support: Samsung, InterDigital, Intel, Futurewei, Lenovo, Apple, vivo, Transsion, ZTE, NEC

  - Oppose: LGE, Nokia, Ericsson, Intel, DCM

- Other scenarios?

- Note: This does not rule out extra backoff (conservative) EDT being applied as UE implementation

Please provide your view if not captured above:

| Company | View |
| --- | --- |
| Nokia, NSB | Scenario 1: support Scenario 2: oppose (lowering based on implementation is always possible) Scenario 3: oppose |
| Futurewei | We slightly prefer Scenario 3 provided there is consensus on its validity. This is because there is uncertainty on the sensing gain assumed in ETSI BRAN and at the same time we do not wish to make channel access overly conservative. If such consensus cannot be achieved, we are ok to support Scenarios 1&2. |
| Ericsson | Scenario 1: support Scenario 2: oppose (lowering based on implementation is always possible, increasing the EDT above the threshold violates ETSI regulations), we do not support adjustment to EDT based on sensing beam, the reason is as our comment in Discussion 2.1.1-1 Scenario 3: oppose, we do not support adjustment to EDT based on sensing beam, the reason is as our comment in Discussion 2.1.1-1 |
| Lenovo, Motorola Mobility | We support all three scenarios for adjustment to EDT. |
| Intel | We support scenario 1 and 2 (notice that we have corrected the list of supportive companies for scenario 3), and we believe that lowering the EDT should not be left up to implementation, but it should be part of our specification since this is an important component of the channel access procedure which has large implications on the system performance, and leaving this up to implementation may cause coexistence problem for the whole ecosystem. As for scenario 3, we share same view as Ericsson, and we believe that scenario 3 may lead to an EDT adjustment that violates the minimum requirements from the ETSI BRAN. |
| Apple | Support scenario 3. Similar view as 2.1.1-1, no adjustment for omni sensing is what 802.11ad is using. If we adjust omni sensing tighter compared to 802.11ad, it is competitive disadvantage to 3GPP technology. To Intel/Ericsson on violates the minimum requirements from BRAN, we believe if 802.11ad can pass the test, there should be no issue. |
| LG Electronics | For scenario 1, we prefer to lower ED threshold for the UE with beamCorrespondenceWithoutUL-BeamSweeping ={0} before the beam management procedure. For scenario 2, we think it can be considered that UE uses lower ED threshold for beamformed transmission that the one used for omni-directional transmission. |
| OPPO | We support additional adjustment to EDT, but for Scenario 2, the EDT is adjusted lower or higher should be further discussed. When sensing beamforming gain is lower, the wider sensing beam also listens to interference in more directions. Moderator: Yes scenario 2 is a conservative approach. However, without additional information on which direction the interference comes from, not sure there is more we can do. |
| vivo | We support scenario 3. However, same as before, we think the first thing is to clarify the assumption in the regulation or the previous agreement. The EDT assumes of 0 dBi receiving antenna gain or the beamforming gain of the receiving antenna is the same as that of the transmission antenna. |
| NEC | We support all scenarios for EDT adjustment. |
| Transsion | We support all scenarios. |
| ZTE, Sanechips | We support the above mentioned three scenarios for EDT adjustment. |
| DOCOMO | Scenario 1: Support as captured above Scenario 2: We agree with Ericsson. Conservative threshold is NOT disallowed by reregulate. Scenario 3: We agree with Ericsson. |
| Samsung | We support all scenarios. |
| InterDigital | We support all three scenarios. We have a preference for Scenario 3, since it removes the inherent LBT penalty of directional LBT due to beamforming gain, when compared to omni-directional LBT. |
| Huawei, HiSilicon | Scenario 1: Oppose We think that even if the sensing beam is the same as the transmit beam, the EDT should not be the same for a wide transmit beam and a narrow transmit beam as these two Tx beams have different interference footprints. Scenario 2: Oppose The adjustment would basically mean EDT EDT+Gsensing - GTX . In fact, when comparing the sensed energy (Psensing +Gsensing) to the adjusted EDT, adding the term Gsensing to the baseline EDT is mathematically equivalent to deducting the sensing antenna gain from the sensed energy as discussed in previous discussion point which is in line with our view. However, deducting the term GTX results in penalizing the more directional transmissions for a given sensing beam which is counter intuitive: More directional transmissions have smaller spatial interference footprints and should be incentivised and not penalized. Therefore, following our Proposal 7, and for both scenarios 1 and 2, we think that EDT should be adjusted to EDT→EDT+ GTX -GTX,max to incentivize more directional transmitters while keeping the adjusted EDT below the value set by regulations. GTX is the effective transmit antenna gain at the potential transmitter that includes the overall gain of the antenna element and the antenna array (beamforming gain) and GTX,max is the maximum supported effective transmit antenna gain. Scenario 3: We can support the following modification If UE uses omni beam for sensing, no additional EDT adjustment is introduced. If UE is using a directional beam for sensing (with positive antenna gain, so the UE will see higher energy level compared with omni sensing beam), either EDT is adjusted higher/looser by the antenna gain or the measurement energy is adjusted lower by the antenna gain before measured energy is compared with EDT As explained earlier, we think the EDT should not be the same for a wide transmit beam and a narrow transmit beam as these two Tx beams have different interference footprints. So, we do not agree that no additional EDT adjustment is introduced if the UE uses omni beam for sensing. |

On WA for Pout definition:

Summary of positions so far:

- Confirm Working Assumption after Modification as follows :

- “For Pout in EDT determination, define Pout to be at least the maximum of mean EIRP of each transmission burst during the COT at the node initiating the COT”

  - FUTUREWEI (with clarifications), Qualcomm, Nokia, Lenovo, vivo, Ericsson, Apple, Oppo

- Confirm Working Assumption as it is

  - Huawei, Ericsson, LGE, Charter, Apple, Intel, Xiaomi, ZTE, Mediatek, Transsion, NEC, Futurewei, TCL, Samsung, CATT, Transsion

From the discussion, there is majority to support confirming the WA as is (15 companies vs 8 companies), consider the difference is not large between the two version, for the sake of progress, Moderator would recommend to confirm the WA as is, with some minor clarifications in red below

Proposal 2.1.1-3

Confirm the WA with some clarifications

Working assumption:

- For Pout in EDT determination, define Pout as the maximum EIRP of the intended transmissions by the node determining EDT during a COT.

  - The node is not expected to transmit in the COT with higher Pout than the Pout used to determine the EDT used to acquire the COT

- FFS: How the node determines maximum EIRP of intended transmissions in a COT

Please provide your view:

| Company | View |
| --- | --- |
| Nokia, NSB | For the sake of progress, we can support confirming the WA without changes. |
| Futurewei | We support proposal 2.1.1-3. A starting point for determining EIRP of an intended transmission in a COT is product of transmit power and beamforming gain estimated for that transmission. |
| Ericsson | We support the proposal. We are also ok with the WA without changes and leave the Pout determination to implementation. |
| Lenovo, Motorola Mobility | Support to confirm the WA with the updates |
| Intel | We support to confirm the WA without changes. |
| Apple | We support the main bullet, with the following FFS point. - FFS: Whether transmissions from responding device is limited to Pout used to acquire the COT. We list the issue in submission, when Pout is used to acquire the COT, do all transmission bursts within the COT need to be within the EIRP and beam directions used in the sensing including the initiating device and responding device, or only the initiating device need to follow Pout. ## If only the initiating device EIRP is limited within the COT, and the responding device can transmit maximum power, this can create serious interference issue. This is especially true for UE initiated COT sharing, where UE can acquire the COT with 20dBm EIRP directional sensing, while gNB share the COT will transmit at 40dBm to potentially any UE within the cell. In section 5.3.8 adaptivity test if EN 302 567, Fig 3 from 5.3.8.3 is copied below shows channel 1/2 chronogram. Although regulation only specify the requirement from the device under test point of view, the intention is clearly shown that the companion device should also follow fair sharing principle. |
| Xiaomi | We can agree with the working assumption. |
| LG Electronics | We support confirming WA as is. One additional point to be considered is when UE receives additional UL scheduling after Pout/EDT calculation and COT initiation. Let’s assume UE starts CCA to initiate COT from slot #n. From our understanding, UE can calculate Pout and EDT based on scheduling information until {slot #n – N2 symbols} considering UE’s processing timeline. If UE can access to channel after slot #n and another UL transmission can be scheduled in the middle of 5 msec COT, then the Pout of corresponding UL transmission could be higher than the calculated one before COT initiation. We would like to clarify what is the UE behaviour in this case. With that, we suggest to add the following FFS point. FFS: UE behaviour when the COT is initiated at slot #n, the additional UL transmission(s) are scheduled after {slot#n-N2 symbols}, and EIRP of the UL transmission could be larger than the Pout used for the initial EDT calculation. Moderator: My understanding is, if a node chooses to use a certain Pout for CCA, it should not increase the transmission power beyond that level in the COT. |
| OPPO | We support confirming the WA without changes. For proposal 2.1.1-3, whether the intended transmissions include the transmissions of responding devices still needs to be discussed. Moderator: If and how to capture the responding device Pout is a different discussion. This proposal is clear by itself that the responding device Pout is NOT considered. |
| vivo | In general, we would like to align with the regulation, which states that “The RF output power is the mean equivalent isotropically radiated power (EIRP) for the equipment during a transmission burst”. Therefore, we prefer to use the maximum of mean EIRP |
| NEC | We support to confirm the working assumption without modification, and also be ok with Proposal 2.1.1-3. |
| Transsion | We support to confirm the working assumption without changes. |
| ZTE, Sanechips | We tend to support the following wording: - “For Pout in EDT determination, define Pout to be the maximum of mean EIRP of each transmission burst during the COT at the node initiating the COT” For the proposal 2.1.1-3, we are not sure what “the intended transmission” means. In EN 302 567, Pout is defined based on transmission burst within a COT, not the transmission in transmission burst. Further, there is still a FFS to be resolved/discussed. Considering the limited time, we prefer to support proposal that align more with the definition of Pout in existing ETSI regulation. |
| LG Electronics2 | Regarding the Moderator’s response, if it is the common understanding, we think it should be captured in specification to clarify UE behavior since based on power control procedure in CA, UE’s transmit power can be changed within a COT. Therefore, we would suggest the following: Proposal: If a COT is initiated by UE at slot #n, and if an additional UL transmission(s) within the COT are scheduled after {slot#n-N2 symbols}, - EIRP of the UL transmission shall be no larger than the Pout used for EDT calculation for initiating the COT Moderator: I removed the FFS and added the statement the node will not transmit beyond Pout used to determine EDT |
| DOCOMO | Support to confirm as it is. |
| CATT | Confirm the WA. |
| InterDigital | We support to confirm the WA. |
| Huawei, HiSilicon | We support the WA as is. We can also support the proposal without FFS. We think the FFS is not needed specially at this late stage of WI. |
| Apple | To modulator: Can we have an explicit discussion topic to align companies’ understanding on the responding device behaviour? There are no related discussions in this meeting and previous meetings. The explicit question is: If UE acquires the COT using Pout=20dBm EIRP and shared it with gNB. Can gNB transmits at 40dBm EIRP without sensing. Moderator: Yes. Added a new discussion |
| Futurewei-2 | We believe device in the note means the initiating node. In that case “The device” in the note should be changed to “The node determining EDT”. Moderator: changed to “the node” |

For the discussion 2.1.1-2, I realize the language is not clear and is causing confusions. In my mind, scenario 2 and scenario 3 are opposite and cannot be both supported. Then I realize the language might be the issue. Let me try again

Discussion 2.1.1-4 (closed)

Please provide your view on the following scenarios

- Scenario 1. For UE indicates a capability for beam correspondence with beamCorrespondenceWithoutUL-BeamSweeping ={1} and when the same TX beam is used for sensing

  - Alt 1. No additional EDT adjustment is introduced

    - Support: Ericsson, Intel, TCL, ZTE,NEC, OPPO, Huawei, LGE, Futurewei,InterDigital, DOCOMO, Nokia, Lenovo, Transsion, Samsung, Apple

    - Not support: HW

  - Alt 2. The EDT is adjusted higher/looser by the difference between the antenna gains of the sensing beam and omni beam, or equivalently, the measured energy is adjusted lower by the difference between the antenna gains of the sensing beam and omni beam

    - Note: This is related to discussion 2.1.1-1 and assumes the jammer is in the peak EIRP direction, and the EDT is wrt to the an omni-sensing beam.

    - Support:

    - Not support: HW

- Scenario 2: For other cases (other than scenario 1) where sensing beam “covers” the transmission beam and has lower beamforming gain (this includes the case the UE is using (pseudo)-omni beam for sensing)

  - Alt 1. The EDT is adjusted lower/tighter by the difference between the antenna gains of the sensing beam and transmission beam

    - Note: This is to make sure the same jammer at the transmission beam direction can be detected with the lower gain sensing beam

    - Support: Qualcomm

    - Not support: HW

  - Alt 2. The EDT is not required to be adjusted lower/tighter

    - Note: The idea here is the EDT does not change to compensate sensing antenna gain difference.

    - Support: Apple, Ericsson

    - Not support: HW

- Note: This does not rule out extra backoff (conservative) EDT being applied as UE implementation

Please provide your view:

| Company | View |
| --- | --- |
| Apple | EN 302 567 has no clear definition/requirement how to measure the energy, unlike NR-U where 0dBi is clearly defined in regulation specification. Therefore, all alternatives do not violate the regulation. In 802.11ad, omni sensing is used and no adjustment is used. This is Scenario 2 Alt 2. NR devices do not need to be worse than this. From this point, since directional transmission with directional sensing cause less interference, therefore we propose Alt 2 in Scenario 1 to be consistent. However, we also see the diverse view based on each company’s understanding of the regulation. Therefore, we can compromise to no adjustment specified in 3GPP at all regardless to sensing beam, and all up to implementation. |
| Huawei, HiSilicon | Scenario 1, Alt 1: Oppose We think that even if the sensing beam is the same as the transmit beam, the EDT should not be the same for a wide transmit beam and a narrow transmit beam as these two Tx beams have different interference footprints. Scenario 1, Alt 2: modification required. This Alt proposes to make the following change to EDT: EDT→EDT+ GTX -GTX,omni While we should incentivise more directional Tx antennas by increasing EDT proportional to the antenna gain GTX , the total adjusted value of GTX -GTX,omni is always positive; pushing adjusted EDT to a value higher than the regulatory value, to avoid this problem, EDT should be adjusted by the followingEDT→EDT+(GTX -GTX,max) Moderator: Can you elaborate what GTX,max means? Reply to our Moderator: GTX,max is the maximum supported effective transmit antenna gain (maximum value for GTX). As explained in our t-doc this can be set to 30 dBi. Scenario 2, Alt 1: Oppose The adjustment mean EDT EDT+Gsensing - GTX . In fact, when comparing the sensed energy (Psensing +Gsensing) to the adjusted EDT, adding the term Gsensing to the baseline EDT is mathematically equivalent to deducting the sensing antenna gain from the sensed energy as discussed in previous discussion point which is in line with our view. However, deducting the term GTX results in penalizing the more directional transmissions for a given sensing beam which is counter intuitive: More directional transmissions have smaller spatial interference footprints and should be incentivised and not penalized. Moderator: In my understanding, this is adjustment is to penalize of using a wider beam for sensing than transmission beam, in case the lower antenna gain with the wider beam may “miss” the jammer. This is a conservative approach by definition. Reply to our Moderator: But using this solution, if two devices have the same Gsensing , the device with a more directional transmission will be penalized, correct? This can easily happen since the sensing beam is not very clearly specified in NRU 60Ghz for all cases so far and, for instance, two base stations may use omni-directional sensing beams but use different Tx beams. Scenario 2, Alt 2: Oppose In our view, EDT adjustment is to incentivise directional Tx antennas irrespective to the corresponding sensing beam. |
| OPPO | For Scenario 1, our position is correctly captured. For Scenario 1 Alt 2, the omni-sensing beam can listen to the interference in all the directions while the directional sensing beam can only listen to the interference in a particular direction. In our understanding, the node does not know which direction the interference comes from. If the jammer comes from a side lobe, the adjustment in Scenario 1 Alt 2 is not reasonable. For Scenario 2, do we assume that the jammer is at the transmission beam direction? Otherwise, the wider beam with lower beamforming gain also listens to interference in more directions. In other words, our concern is that the EDT may be adjusted in the opposite direction in Scenario 2 Alt 1. Moderator: Yes for scenario 2 alt 1, the worst case is assumed, without further knowledge on jammer direction. |
| ZTE, Sanechips | Scenario 1, we support Alt 1 and our position has been correctly captured. Scenario 2, for the note of Alt 1, we understand that it is assumed that the jammer is just within transmission beam direction and sensing beam direction. If Alt 1 is based on this assumption, we support Alt 1. But for the case that jammer is within Sensing beam direction (transmission beam)but not in transmission beam(sensing beam direction), we would like to clarify how to adjust EDT for this case. Moderator: The scenario 2 alt 1 is a conservative approach. |
| Ericsson | Please see our response to 2.1.1-1. Any adjustment introduced in 3GPP must ensure devices can pass the regulation test. It is not clear to us we can devise a mechanism for adjustment to ensure the conformity of 3GPP devices to EN 302 567 considering the complicated test involved for adaptivity. We suggest leaving the adjustment to implementation and not specify it in 3GPP. |
| Intel | For both scenario 1 and 2, our preference is for Alt.1. |

Discussion 2.1.1-5 (closed)

For Pout in EDT determination, should the initiating device also consider the Pout of the responding device?

- Yes: Apple, ZTE, Oppo.

- No: HW, Ericsson, Qualcomm

Please provide your view:

| Company | View |
| --- | --- |
| Huawei, HiSilicon | No. |
| Apple | Thanks modulator for adding discussion. The initiating device can determine the Pout. The main question at least in our mind is whether the responding device sharing the COT without LBT, should transmit within Pout limitation used to acquire the COT by the initiating device. This is in general not an issue for DL, where gNB has much higher power. The main issue is UL COT sharing, if configured, as gNB power is much higher than UE. For UE initiated COT sharing, we think gNB should not transmit higher than Pout. In other word, if UE acquired to COT using Pout = 20dBm, gNB can NOT transmit using a much higher EIRP. Of course, gNB can always choose not to enable UE acquired COT sharing. |
| OPPO | In our view, the responding device also cannot transmit beyond the Pout in EDT determination. Therefore, the initiating device should consider the Pout of the responding device, especially for gNB sharing the COT initiated by a UE. In this case, the Pout in EDT can be configured to the UE. |
| ZTE, Sanechips | In our view, if “the node” in the sentence “The node is not expected to transmit in the COT with higher Pout than the Pout used to determine the EDT used to acquire the COT” from proposal 2.1.1-3 is regarded as the responding node, then we think that the responding node has been considered when the initiating node determine Pout. Moderator: In the Pout discussion, the node is always the initiator. There is only one node mentioned in that proposal. The responding device is not mentioned. I don’t think there is any ambiguity there. The responding node is discussed here. |
| Ericsson | No. The initiating device need not consider Pout of responding device. Regulations do not mandate it, nor is it implemented by 802.11ad/ay. This is a complex mechanism for no real gain in our opinion. |
| Intel | No – we do not think that the initiating device should also account for the responding device’s Pout. The ETSI BRAN does not mandate anything in this regards, and the motivations/rationale used in Rel.16 do not hold here any longer. Therefore, while we do not see any technical reason for it, this may only complicate the design. |

### Second round discussions

From the discussion in 2.1.1-1 and 2.1.1-2 and 2.1.1-4, given the diversed view, the Moderator believe we can only agree on the minimum

Proposal 2.1.2-1

- For LBT purpose, the energy at gNB/UE is measured after antenna and antenna gain is included in the energy measurement.

- The energy measurement is directly compared with EDT with no further adjustment to EDT standardized in Rel.17

  - Note: This does not rule out extra backoff (conservative) EDT being applied as gNB or UE implementation

Proposal 2.1.2-1a

- For LBT purpose, the energy at gNB/UE is measured after antenna and antenna gain is included in the energy measurement.

- The energy measurement is directly compared with EDT with no further adjustment to EDT standardized in Rel.17

  - Note: This does not rule out extra backoff (conservative) EDT being applied as gNB or UE implementation

Please provide your view:

| Company | View |
| --- | --- |
| Lenovo, Motorola Mobility | Support |
| Intel | Given that this topic has been discussed for many meetings and positions have not changed sensibly, we are OK with this conclusion since it seems to be the only way forward. |
| Samsung | We are ok with the proposal. |
| Futurewei | We prefer if requirement of “energy measurement is directly compared” is changed to “energy measurement is compared”. In the note an additional sentence can be added: “This does not rule out processing energy measurement before comparing with EDT as long as regulation requirements are satisfied.” These changes are to allow implementation of Alt-2 option presented before in a compliant manner. We would like to see if there are persistent concerns on validity or compliance with regulations. Moderator: Added 2.1.2-1a version to remove “directly”. For the additional note, “as long as reregulate requirements are satisfied” may not seem to be clear. Since you are bringing this up, I assume there will be cases that equivalently EDT will be increased (looser)? If that is the case, I suspect there will be concerns. |
| LG Electronics | We are fine with either of Proposal 2.1.2-1 or Proposal 2.1.2-1a. |
| Apple | Support 2.1.2-1a. |
| OPPO | We are OK with the proposal for the sake of progress and limited time. |
| DOCOMO | We are fine with either Proposal 2.1.2-1 or Proposal 2.1.2-1a. |
| Ericsson | We can support the proposal 2.1.2-1 or proposal 2.1.2-1a. |
| ZTE, Sanechips | We are fine with either Proposal 2.1.2-1 or Proposal 2.1.2-1a. |
| Huawei, HiSilicon | Given the diverse views, we can go with Proposal 2.1.2-1a. |
| Futurewei-2 | Thanks for addressing our comment. We prefer 2.1.2-1a. |
| TCL | We are fine with both Proposal 2.1.2-1 and Proposal 2.1.2-1a. |
| CATT | We prefer Proposal 2.1.2-1a. |
| Nokia, NSB | We are fine with both proposals |

From the discussion in 2.1.1-6, the view is also not converging. The moderator recommend to agree to the minimum as well.

Proposal 2.1.2-2 (closed)

For Pout in EDT determination at the initiating device, the Pout of the responding device is not considered

Proposal 2.1.2-2a

For gNB initiated COT, for Pout in EDT determination at the initiating device (gNB), the Pout of the responding device (UE) is not considered

Proposal 2.1.2-2b

For UE initiated COT

- Alt 1. Pout in EDT determination at the initiating device (UE), the Pout of the responding device (gNB) is not considered

- Alt 2. Pout in EDT determination at the initiating device (UE), the Pout of the responding device (gNB) is not considered if a ul-toDL-COT-SharingED-Threshold is not configured. Otherwise, the EDT for the initiating device (UE) will use the configured ul-toDL-COT-SharingED-Threshold

Please provide your view:

| Company | View |
| --- | --- |
| vivo | OK |
| Lenovo, Motorola Mobility | Fine to support |
| Intel | We are OK with the proposal. |
| Samsung | We are ok with the proposal. |
| Futurewei | OK to support |
| LG Electronics | We are OK with the proposal only for gNB-initiated COT. For UE-initiated COT, we can apply the similar to Rel-16 NR-U, i.e., by configuring ul-toDL-COT-SharingED-Threshold-r16. |
| Apple | OK to support |
| OPPO | We agree with LG. For the proposal, it seems that the UE initiated COT sharing is not allowed given that the gNB transmits with higher EIRP. Moderator: Actually the proposal 2.1.2-2 covers both UE and gNB as initiating device case. However, given the comments, I can split that into two separate discussions |
| DOCOMO | We support the proposal. |
| Ericsson | We support the proposal. Regarding firstly we need an agreement similar to Rel 16 where the EDT threshold is RRC configured. Secondly, there are multiple issues regarding using ul-toDL-COT-SharingED-Threshold-r16 for Rel-17. - how to signal this? It was signalled per 20 MHz in Rel 16. - BWs are different and hence the range needs to be changed as well |
| ZTE, Sanechips | We are fine with the proposal. |
| Moderator | Separate the proposal into proposal 2.1.2-2a and proposal 2.1.2-2b. I see all companies above support proposal 2.1.2-2a. For proposal 2.1.2-2b, I see most companies support Alt 1 while LGE and maybe Oppo support Alt 2. |
| Huawei, HiSilicon | Support 2.1.2-2a and 2.1.2-2b Alt 1. |
| Intel | We support proposal 2.1.2-2a and Alt1 for proposal 2.1.2-2b. |
| OPPO2 | Thanks Moderator for the response. We support proposal 2.1.2-2a and 2.1.2-2b Alt 2 following R16 NRU principle. In our understanding, the gNB does not know the exact Pout value of the UE in EDT determination for UE initiated COT, so the Pout of the gNB cannot be determined when sharing the UE-initiated COT. Moderator: Understand your view. One more thing to note is, in NR-U, we are using the maximum power the node can transmit to determine EDT. But now we are using the maximum power the node is using in this COT to determine EDT. In some sense, the transmit power used is more dynamic now and harder to know by the other side. Given the majority of the other camp, it will be good if you can reconsider. |
| LG Electronics | Support Proposal 2.1.2-2a and prefer Alt 2 in Proposal 2.1.2-2b Moderator: Same reply to Oppo |
| Transsion | We support proposal 2.1.2-2a and proposal 2.1.2-2b. |
| CATT | OK |
| Nokia, NSB | OK with the proposals |
| Ericsson 2 | Support 2.1.2-2a and 2.1.2-2b Alt 1. However, we agree that we still need to configure the EDT threshold like Rel 16. Please see our previous comments. |

## LBT Bandwidth FFS Items

Agreement: - For LBT for single carrier transmission, gNB/UE performs LBT over the channel bandwidth (or BWP bandwidth) (Alt SC.1. in earlier agreements) - For LBT for multi-carrier transmission in intra-band CA, gNB/UE performs multiple LBT, one for each channel bandwidth separately (Alt CA.1. in earlier agreements) - FFS: Additional support of performing single LBT over all CCs (Alt CA.2. in earlier agreements) more than one alternative for at least multi-carrier transmission in intra-band CA is not precluded. Conclusion: There is no consensus to support explicitly introducing in the spec using single LBT covering multiple CCs under CA. - Note: This does not rule out gNB/UE implementation to perform single LBT to cover multiple CCs. However, the EDT needs to be selected such that if interference on one of the CCs exceeds the CC EDT, the LBT is declared as failed

| Company | Key Proposals/Observations/Positions |
| --- | --- |
| Huawei HiSilicon | Proposal 3: For operation in NR-U-60, the term ‘Operating Channel Bandwidth’ in the agreed baseline EDT formula is defined as the ‘LBT Bandwidth’ or the ‘bandwidth on which a channel access procedure is performed in shared spectrum’. |
| vivo | Proposal 5: The LBT bandwidth should be used as the operating channel bandwidth for EDT evaluation. |
| ZTE Sanechips | Observation 1: It is worth emphasizing that the OCB should be satisfied for each transmitter such as gNB or UE. Proposal 1: In order to avoid ambiguity about the understanding of nominal bandwidth and resolve the problem of unclear the conclusion for the OCB requirement, it is necessary to give a clear guidance on how to deal with the issue on the nominal bandwidth, e.g., introduce the definition of nominal bandwidth. Proposal 2: The nominal bandwidth can be defined as follows:  Nominal bandwidths for the purpose of OCB requirements at the UE are the channel BWs for transmission supported by the UE from the set of channel BWs (carrier BWs) to be defined in 38.101.  Nominal bandwidths for the purpose of OCB requirements at the gNB are the channel BWs for transmission supported by the gNB from the set of channel BWs (carrier BWs) to be defined in 38.104. |
| Convida Wireless | Proposal 15: To down-select the options of LBT BW with single carrier and multi-carrier operation for supporting NR form 52.6 GHz to 71 GHz, co-existence of single carrier and multi-carrier operation within a same channel BW should be considered. |
| OPPO | Proposal 1: For BWP bandwidth in Alt SC.1, sensing in DL BWP or UL BWP can be left for implementation. |

For this topic, the moderator does not see anything essential to be discussed for this meeting. If you see something worth discussion, please bring it up

Discussion 2.2-1 (open discussion)

| Company | Proposed discussion points |
| --- | --- |
| LG Electronics | We think at least one point should be clarified for the following highlighted part. Agreement: - For LBT for single carrier transmission, gNB/UE performs LBT over the channel bandwidth (or BWP bandwidth) (Alt SC.1. in earlier agreements) - For LBT for multi-carrier transmission in intra-band CA, gNB/UE performs multiple LBT, one for each channel bandwidth separately (Alt CA.1. in earlier agreements) - FFS: Additional support of performing single LBT over all CCs (Alt CA.2. in earlier agreements) more than one alternative for at least multi-carrier transmission in intra-band CA is not precluded. In our opinion, gNB may perform LBT over the whole channel bandwidth. However, UE needs to perform LBT only over BWP bandwidth, rather than channel bandwidth. Even if a UE is configured with BWP where its BW is smaller than channel bandwidth, does the UE have to perform LBT over the channel bandwidth? Moderator: My understanding is in RAN1 we use terminology BWP bandwidth, but RAN4 will use terminology channel bandwidth |
| vivo | We think moderator may capture our view in the wrong topic. We tried to solve the FFS in the following agreement: Agreement: The baseline ED threshold can be computed as EDT=-80 dBm+10*log10PmaxPout+10*log10(Operating Channel BW in MHz) Where Pout is RF output power (EIRP) and Pmax is the RF output power limit, Pout≤Pmax. - FFS: Further adjustment on ED threshold based on the sensing beam and the transmission beam (further adjustment should not violate EDT requirements as per regulations) - FFS: If Pout is max output EIRP of the device or instantaneous output EIRP - FFS definition of Operating Channel BW - FFS: Whether ED threshold for NR-U and NR-U coexistence scenarios (eg, at regulation level) can be appropriately relaxed compared with the threshold of coexistence between NR-U and Wi-Fi. - FFS: EDT when the COT has time varying transmission beams and varying EIRP The operating channel BW is not defined yet, which causes some problem for the editor of TS 37.213 when defining the operating channel BW. A few companies had already commented on the issue during the spec review last week. Moderator: Given we agree on the LBT bandwidth, I assume the operation channel BW here will be BWP bandwidth (in RAN1 term) or channel bandwidth (in RAN4 term) |
| ZTE, Sanechips | On the LBT bandwidth, we need to first clarify the previous agreement, that is, the conclusion on LBT bandwidth reached in RAN1 #106 e-meeting is only applied for per carrier, not per BWP since the functionality of accessing a carrier if there is interference in part of the carrier in frequency is not supported. Agreement and conclusion are copied below: Agreement: - For LBT for single carrier transmission, gNB/UE performs LBT over the channel bandwidth (or BWP bandwidth) (Alt SC.1. in earlier agreements) - For LBT for multi-carrier transmission in intra-band CA, gNB/UE performs multiple LBT, one for each channel bandwidth separately (Alt CA.1. in earlier agreements) - FFS: Additional support of performing single LBT over all CCs (Alt CA.2. in earlier agreements) Conclusion: There is no consensus in RAN1 to support the functionality of accessing a carrier if there is interference in part of the carrier in frequency. |
| LG Electronics2 | To Moderator: Thanks for the response. With that, would gNB perform LBT over BWP bandwidth, rather than carrier bandwidth? Moderator:I think that is gNB/UE implementation. From gNB point of view, if all UEs are in a narrow BWP, I don’t see a why to force the gNB to open up the RF to cover the entire carrier, which is a waste of power. Then in this case, gNB should be able to do LBT over the BWP. On the other hand, if some UEs are wide BWP and some UEs are narrow BWP, I don’t see gNB should switch RF to sense separately. Similarly on the UE side, if a UE is switched to a narrow BWP, there is no way to force the UE to keep the RF on for the entire carrier. The UE should be allowed to use a narrow band RF, which is the whole purpose to introduce BWP in the beginning. In that case, the UE should not be forced to sense channel with carrier bandwidth as well. |
| Samsung | We believe at least some conclusion on the understanding of “BWP bandwidth” and “channel bandwidth” is needed. If moderator believes it’s just a wording difference between RAN1 and RAN4, at least some explicit conclusion is needed to clarify, and guide the specification drafting for TS 37.213. |
| DOCOMO | We agree a conclusion may be needed. It may also help UE feature session a bit. |

### First round discussions

Proposal 2.2.1-1

Clarify the earlier agreement as follows

Agreement:

For LBT for single carrier transmission, gNB/UE performs LBT over the channel bandwidth (RAN4 terminology) (or BWP bandwidth in RAN1 terminology)

Please provide your view:

| Company | Proposed discussion points |
| --- | --- |
| Apple | Support this proposal |
| Huawei, HiSilicon | OK with the clarification |
| LG Electronics | Support Proposal 2.2.1-1 |
| OPPO | We support the proposal. |
| ZTE, Sanechips | Support this proposal. We would like to further confirm whether it implies LBT mode in RRC parameter excel sheet will be configured per BWP if this proposal is agreed. |
| Ericsson | We are ok with the proposal. As mentioned by the FL, whether to use the full carrier bandwidth or BWP BW is gNB/UE implentation. |
| vivo | Thanks for the clarification. For UE, it is obvious to perform LBT over the UL BWP. What confused us is the LBT bandwidth for gNB. If gNB wants to schedule 4 UEs with non-overlapping DL BWPs, how does gNB perform LBT? Perform one LBT over the carrier bandwidth or perform 4 LBTs over the 4 non-overlapping DL BWPs for the 4 UEs? Moderator: From UE point of view, the gNB only need to pass LBT for the UE’s BWP. However, from gNB point of view, a valid implementation is to perform wideband LBT covers all 4 UEs BWP, and for each UE BWP, the LBT passes. The gNB can also perform 4 LBTs each for one BWP to be accurate. These choices are up to gNB implementation |
| Intel | We are Ok with the proposal |
| Samsung | We are ok with the proposal. One further clarification, for agreement of CA case, the term “carrier bandwidth” is RAN1 or RAN4 terminology? Or whether it’s also allowed to perform LBT over the BWP bandwidth in each carrier (e.g. aligned with single carrier case)? Agreement: - For LBT for single carrier transmission, gNB/UE performs LBT over the channel bandwidth (or BWP bandwidth) (Alt SC.1. in earlier agreements) - For LBT for multi-carrier transmission in intra-band CA, gNB/UE performs multiple LBT, one for each channel bandwidth separately (Alt CA.1. in earlier agreements) - FFS: Additional support of performing single LBT over all CCs (Alt CA.2. in earlier agreements) Moderator: Sorry for using the terms interchangeably. Here I think it is RAN4 term. |
| Futurewei | We have similar question as Vivo. How is multiple BWP within carrier bandwidth handled by the gNB? If it is independent LBT per BWP then would it not violate the conclusion: “functionality of accessing a carrier if there is interference in part of the carrier in frequency is not supported”. Moderator: Please see reply to vivo. Simply speaking, what we define is independent LBT per BWP, but the implementation can choose to do wider LBT for convenience. |
| Convida Wireless | We are ok with the proposal. |
| Futurewei-2 | Thanks to the moderator for the clarification. We are OK to support. |
| TCL | We are fine with the proposal. |
| vivo | Thanks moderator’s reply to our previous question. We still think more clarification is needed. Can moderator confirm whether the proposal is equivalent to the following two sentences where only RAN1 terminology is used? For LBT for single carrier transmission, gNB performs LBT over BWP bandwidth. For LBT for single carrier transmission, UE performs LBT over BWP bandwidth. If that’s the correct understanding, we suggest split sentence into gNB and UE side. Furthermore, we don’t think it’s necessary to put RAN4 terminology into RAN1 agreement. Now on the reply, “from gNB point of view, a valid implementation is to perform wideband LBT covers all 4 UEs BWP, and for each UE BWP, the LBT passes. The gNB can also perform 4 LBTs each for one BWP to be accurate. These choices are up to gNB implementation”. Also looking at companies’ comment, we’re not sure that’s the common understanding among companies. For example, Ericsson said “whether to use the full carrier bandwidth or BWP BW is gNB/UE implementation”. To us, they interpreted as both channel bandwidth and BWP bandwidth (both in RAN1 terminology) are supported where gNB/UE choose one. |
| Transsion | We are fine with this clarification. |
| Nokia, NSB | We support the proposal. |

## Sensing Structures FFS Items

Agreement:

For energy measurement in 8us deferral period, at least a single measurement within 8us is performed, and the measurement duration is selected from one of the following alternatives:

- Alt 1: At least 3+X us (FFS X, such as X=1).

- Alt 2: At least X us, where X is the same as the minimum measurement duration in a 5 us observation slot and is within the 5 us observation slot.

- Alt 3: At least a contiguous duration of X+Y us where the Y us part of the measurement is done at the end of the first 3 us and X is the same as the minimum measurement duration in a 5 us observation slot and is at the beginning of the 5 us duration.

Agreement:

For energy measurement in 8us deferral period, Alt 2 is supported while Alt 1 and Alt 3 can be considered as gNB/UE implementation (Alt. 1/2/3 are defined as per previous agreement)

Agreement:

Confirm the WA with the following updates:

For energy measurement in 5us observation slot, when performing single measurement, the location of the measurement within the 5us is left for implementation, i.e., anywhere within the 5us.

| Company | Key Proposals/Observations/Positions |
| --- | --- |
| Huawei HiSilicon | Proposal 8: For operation in NR-U-60, when LBT is used, the measurement duration X us within the 5us observation is implementation specific with a maximum of 3us. Proposal 9: For operation in NR-U-60, when LBT is used, clarify that for Alt 2 agreed in RAN1#106-e on the energy measurement in the 8us deferral period, the 5us observation slot is at the end of the 8us deferral period. |
| FUTUREWEI | Proposal 4: For energy measurement in 5us observation slot duration of the measurement within the 5us is left for implementation. |
| ZTE Sanechips | Observation 8: For deferral period and 5us observation slot, the minimum duration of energy measurement can be configured as any integer value less than 3us. |
| Nokia Nokia Shanghai Bell | Proposal 5: Minimum duration for energy measurement during the 5 µs observation slot is not defined. LS is sent to RAN4 to consider defining a requirement on measurement or channel access accuracy. |
| Ericsson | Observation 17 8us deferral period in IEEE 802.11ad and IEEE 802.11ay ignali of the 5us observation slot at the end of the 8us period Proposal 23 For energy measurement in 8 µs deferral period, 5us observation slot is located at the end of the 8us deferral period like IEEE 802.11ad/ay. Proposal 24 The minimum measurement duration X within a 5 µs observation slot can be left for implementation with the maximum value as 3µs |
| Intel | Proposal 1: Within a 5us or 8us observation window, a device must perform a measurement of the medium for at least 2us. |
| Qualcomm Incorporated | Proposal 3: Minimum requirement for sensing for both 5us and 8us slots should be 1us irrespective of bandwidth. |
| MediaTek Inc. | Proposal 1: For sensing structure within a 8 us deferral period, support Alt 2. |
| OPPO | Proposal 2: The location of the 5us observation slot within the 8us deferral period can be left for implementation. Proposal 3: A minimum measurement duration of 2us can be considered. |
| CAICT | Proposal 1: X=2us could be considered as the minimum measurement duration for energy measurement in 5us observation slot. |

### First round discussions

Discussion:  2.3.1-1:  (closed)

On the minimum measurement duration X within a 5 µs observation slot the summary of positions is as follows.

- Implementation: Ericsson, Apple, LGE, Transsion, WILUS, Samsung, DCM, Charter, FW, Xiaomi, Transsion, HW (not exceeding 3us)

- Other :1 us (Qualcomm, CATT, Intel), 2us (OPPO, Intel, CAICT), 3us (Spreadtrum, Lenovo (also ok to config)), MTK, max 3us (ZTE, HW)

- RAN4: Nokia, CATT

Please provide your views on minimum duration of measurement for observation interval.

| Company | View |
| --- | --- |
| Nokia, NSB | We are fine to leave this for implementation. RAN4 may further decide if some restriction is defined for test purposes. |
| Futurewei | We prefer to leave it to implementation. We are OK with a check by RAN4 on whether implementation meets required channel access accuracy. |
| Ericsson | Our view is accurately captured. |
| Lenovo, Motorola Mobility | As captured, our preference is to support 3us as the minimum measurement duration within a 5us observation slot. However, we would also be okay to consider a configurable values. |
| Intel | We strongly prefer to set a reasonable lower bound for the measurement window in order to ensure a device would perform a proper sensing of the media, and for the exact value we are open to discuss, while we believe that a good compromise may be to set a minimum to either 1us or 2us. |
| Apple | Leave it to implementation. RAN4 can further discuss and define requirement is needed. |
| Xiaomi | Agree with Apple. |
| OPPO | We think it is necessary to specify the minimum measurement duration to ensure the sensing results are reliable. Furthermore, the minimum measurement duration can be scaled down with respect to NRU R16, where the duration is 4us for a sensing slot of 9us. Thus, for 5us sensing slot, a measurement duration of 2us can be considered. |
| Transsion | We prefer to leave it to implementation . |
| ZTE, Sanechips | We tend to support that the minimum duration of energy measurement can be configured as any integer value less than 3us. |
| CATT | For the minimum measurement, 1 us is preferred. We are also fine to leave the value of the minimum measurement to RAN4 discussion. |
| Huawei, HiSilicon | We would like to clarify our view. The minimum measurement duration X is still selected by implementation but not exceeding 3us. |

### Second round discussions

From the above discussion, left for implementation has small majority. For the sake of progress, the moderator recommend to agree to the implementation options.

Proposal:  2.3.2-1:

The minimum measurement duration X within a 5 µs observation slot is left for gNB or UE implementation

- Note: This agreement does not prevent RAN4 from setting minimum requirement for measurement duration X.

Please provide your views

| Company | View |
| --- | --- |
| Intel | For the sake of progress we are OK with the proposal, even though we still believe this would be a bad design choice as commented before. |
| Samsung | We support the proposal. |
| Futurewei | OK to support. We are open to RAN4 check/discussion. |
| LG Electronics | Support the proposal. |
| Apple | Support the proposal |
| OPPO | We still believe specifying the minimum measurement duration is necessary to ensure the sensing results are reliable. We are fine to RAN4 further discussion. |
| DOCOMO | Fine with the proposal. |
| Ericsson | We are ok with the proposal |
| ZTE, Sanechips | we share same view with Futurewei and OPPO and it can be further discussed in RAN4. |
| Moderator | Add a note above to address the concern that RAN4 can continue discussion |
| Huawei, HiSilicon | Support but “On” at the beginning can be removed. Moderator: Right |
| TCL | We support the proposal. |
| Transsion | We are fine with the proposal. |
| Mediatek | We prefer to specify the minimum duration for energy measurement. |
| CATT | We are ok with the proposal |
| Nokia, NSB | We support the proposal |

## COT Sharing

Agreement: On COT sharing from an initiating device transmission to responding device transmission, support both of the following two alternatives - Alt 1: No maximum gap defined between the initiating device transmission and responding device transmission. A responding device transmission can occur without LBT with any gap within the maximum COT duration - Alt 3: Define a maximum gap Y, such that a responding device transmission can occur without LBT only if the transmission starts within Y from the end of the initiating device transmission. If the responding device transmission starts after Y from the end of the initiating device transmission, a Cat 2 LBT is needed before the responding device transmission. - The Cat 2 LBT uses the same sensing structure as the 8 us initial deferral period as in eCCA - Further downselect between the following options: - Option 1: Y=8 us (motivated by need to operate in all regions) - Option 2: Y=a multiple number of OFDM symbols - Option 3: gNB determines Y (for example, according to local regulation) - Cat. 2 LBT is a UE capability - The usage of the two alternatives is a gNB choice and depends at least on local regulations. Note: Alt. 3 is motivated by the regulations in Japan but use of Cat. 3 LBT is also an option for operation in Japan and Cat. 2 LBT is not restricted for use only in Japan. Note: Maximum gap allowed without Cat 2 LBT between two initiating device transmissions is to be separately discussed Note: Other use cases of Cat 2 LBT will be separately discussed

| Company | Key Proposals/Observations/Positions |
| --- | --- |
| Huawei HiSilicon | Proposal 19: For COT sharing without LBT in NR-U-60, support Option 2 for defining the maximum gap Y within which a transmission from a responding device occurs without LBT(Y=a multiple number of OFDM symbols). - The value(s) of Y should be specified Proposal 21: For operation in a cell with shared spectrum access in FR2-2, the ChannelAccess field size in the non-fallback DCI formats 0_1/1_1/0_2/1_2 is configurable to 0, 1 or 2 bits based on the number of entries configured in the corresponding table of channel access types; and 0 bit otherwise. Proposal 22: For operation in a cell with shared spectrum access in FR2-2, the ChannelAccess field size in fallback DCI formats 0_0/1_0 is 2 bits; and 0 bit otherwise. |
| FUTUREWEI | Proposal 6: On the gap Y for Cat 2 LBT when COT Sharing is applied, gNB determines Y and the UE does not need to know the value for Y. The UE will follow DCI to determine if Cat 2 LBT is to be performed. Proposal 7: Define a maximum gap Y, such that a later transmission from an initiating node can share the COT without LBT only if the later transmission starts within Y from the end of the earlier transmission from the initiating node or a responding node. If the later transmission starts after Y from the end of the earlier transmission, a one-shot LBT is needed to share the COT:  FFS: Specific value of Y. |
| ZTE Sanechips | Proposal 17: For the maximum gap Y, similar rule as specified in LTE-LAA can be reused, such as Option1 that “Y=8 us (motivated by need to operate in all regions)” that is at least equal to the duration of Cat2 LBT. |
| Nokia Nokia Shanghai Bell | Observation 13: In case of Alt. 3 for COT sharing, there is need for a wide range of time gap Y values to facilitate efficient scheduling while fulfilling local regulations having a wide range in requirements. Proposal 16: Support Option 3 i.e., gNB determining maximum gap Y in Alt. 3. There is no need to signal the value Y to the Ues. Proposal 17: For dynamically scheduled UL transmissions, adopt Rel-16 DCI indication with appropriate modifications on the indicated channel access types. There is no need for an indication of CAPC or CP extension. Proposal 18: For configured UL transmissions like scheduling request and CG-PUSCH, consider and agree on the necessary ignaling indicating appropriate channel access type for the UE. |
| Ericsson | Observation 14 There is no reliable way for Ues (responding device) to know the end of the transmissions by the gNB (initiating device), to estimate the gap Y in the COT sharing case Proposal 20 For the maximum gap Y between initiating device and responding device in a COT sharing case for regions where sensing is needed, we support Option 3 with no need for gNB to specify or signal Y to the UE. |
| Intel | Proposal 18: Reservation signal in the form of cyclic prefix is applied by a UE soon after succeeding the LBT procedure so that to sufficiently postpone the actual UL transmission to account for beam-pairing time. |
| Lenovo Motorola Mobility | Proposal 15: For NR operation in unlicensed bands between 52.6 GHz and 71 GHz with LBT based channel access mechanism, COT sharing between the initiating device and responding device should be supported with at least Cat 2 LBT: - If the responding device is capable of beam correspondence and it is ignalin to use only any of the Rx beam(s) as Tx beam(s) for its transmission that have been used to receive at least one of the transmissions from the initiating device within the same COT - If the responding device determines at least one suitable beam on which it is allowed to transmit within the same COT, where the suitable beam can be determined as follows: o UE can be configured with a mapping table for determining suitable transmit beams for UL transmissions based on the receive beam(s) which the UE used to receive the prior DL transmissions in the same COT |
| NEC | Proposal 4: On COT sharing from an initiating device transmission to responding device transmission, the value of a maximum gap Y (if supported) should be defined as a multiple number of OFDM symbols depending on supported SCS. Proposal 5: On COT sharing between two initiating device ignalingn, a maximum gap Y should be defined, such that a later transmission can share the COT without LBT only if the later transmission starts within Y from the end of the earlier transmission. If the later transmission starts after Y from the end of the earlier transmission, a Cat 2 LBT is needed to share the COT. |
| Samsung | Proposal 2: For the gap duration Y in COT sharing, support Y as the duration of Cat 2 LBT, e.g. 8 us. |
| Apple | Proposal 3: For UE initiated COT sharing, the EIRP and UL TCI state/spatial relationship or omni-sensing used in EDT calculation is signaled in CG-UCI. Proposal 4: For UE initiated COT sharing,  When the UE performs directional sensing, any unicast transmission from the gNB that includes control and user plane data is only transmitted to the UE that initiated the COT  When the UE performs omni sensing, the gNB can transmit control/broadcast signals/channels for any Ues as long as the transmission contains transmission for the UE initiated the COT, and any unicast transmission from the gNB that includes user plane data is only transmitted to the UE that initiated the channel occupancy. Proposal 5: For Alt 1 COT sharing, 1 bit in DCI to indicate whether the transmission is within the COT or outside of the COT. Proposal 6: Use cell specific RRC configuration to indicate Y value. When Y is not configured, Alt-1 COT sharing is used. Proposal 7: Y can be configured as multiple of OFDM symbols depending on SCS. No CP extension is needed. |
| Convida Wireless | Proposal 5: For COT sharing consider Alt 3. Define a maximum gap Y, such that a later transmission can share the COT without LBT only if the later transmission starts within Y from the end of the earlier transmission. If the later transmission starts after Y from the end of the earlier transmission, an one-shot LBT is needed to share the COT. |
| NTT DOCOMO INC. | Proposal 2: Support Y to be 8 us (i.e., Option 1) or determined by gNB (i.e., Option 3)  A UE does NOT need to be aware of the exact duration of Y |
| Qualcomm Incorporated | Proposal 4: For Alt-3 for COT sharing, gNB determines the value of Y and is transparent to UE. Proposal 5: For Alt-3 for COT sharing, support the proposed conclusion 2.4.2-1 from [5], i.e., On the gap Y for Cat 2 LBT when COT Sharing is applied, no matter which option is chosen out of options 1/2/3, the UE does not need to know the value for Y, as the UE will follow DCI to determine if Cat 2 LBT is performed. Proposal 6: Cyclic Prefix extension is not required and need not be ignalin. Proposal 7: For FR2-2 operation, the CP extension column and CAPC column in 38.212 Table 7.3.1.1.1-4, Table 7.3.1.1.2-35 and Table 7.3.1.2.2-6 are not applicable |
| OPPO | Proposal 6: For maximum gap Y, Option 1, i.e., Y=8us should be supported. Proposal 7: For Alt 3, it should be clarified that the beam for initiating device transmission matches the beam for responding device transmission. |
| CAICT | Proposal 2: 8us could be considered for the maximum gap Y. Proposal 3: Cat2 LBT could be used for resume transmission after a gap Y, Multi-beam LBT and Multi-channel (Type B) LBT. |

### First round discussions

Proposed conclusion 2.4.1-1 (closed)

On the gap Y for Cat 2 LBT when COT Sharing is applied, no matter which option is chosen out of options 1/2/3, the UE does not need to know the value for Y, as the UE will follow DCI to determine if Cat 2 LBT is performed

Conclusion

On the gap Y for Cat 2 LBT when COT Sharing is applied, no matter which option is chosen out of options 1/2/3, the UE does not need to know the value for Y, as the UE will follow DCI to determine if Cat 2 LBT is performed

- Note: If Y is specified in 3GPP spec or not is discussed separately

Please provide your view if not captured above:

| Company | View |
| --- | --- |
| Nokia, NSB | We support the conclusion |
| Futurewei | Support the conclusion |
| Ericsson | We support the conclusion. |
| Lenovo, Motorola Mobility | Fine with the conclusion |
| Intel | We support the conclusion. |
| Apple | Our proposal is to use Y configuration to indicate Alt 1 versus Alt 3 is used. If Y is NOT configured, then it is alt-1 by default. The proposed conclusion is under the assumption if Alt 3 is configured, then Y is transparent to UE. If modulator can have a separate discussion topic on how to configure Alt1 and Alt 3 in higher level, we are fine with the conclusion. Moderator: Yes this is for Alt 3 only. I am not sure we need configuration for it. gNB can enforce it with the DCI channel access field. |
| Xiaomi | Agree with the conclusion |
| LG Electronics | Although we do not disagree with this conclusion, we are not sure whether this is needed since anyway the value Y will be captured in 37.213 specification, isn’t it? Moderator: The intention is not to capture it. There is no regulation guideline on what value to use. |
| OPPO | We are OK with the conclusion, but the value of Y still needs to be specified. |
| NEC | We support the conclusion. |
| Transsion | We support the conclusion. One question for clarification, if the value of Y is transparent to UE, then does the UE need to switch from Cat 2 LBT to No LBT for its configured UL transmission within the COT when the gap is smaller than Y? Moderator: We never do that. The UE will not be able to find out when is the end of DL transmission, |
| Sony | We support the conclusion. |
| ZTE, Sanechips | We support the conclusion. |
| DOCOMO | Support |
| CATT | Support. |
| Samsung | Support the conclusion. |
| InterDigital | We support the conclusion. |
| Huawei, HiSilicon | We support the conclusion in the sense that the UE does not need to know Y. However, we believe that, similar to Rel-16 NR-U, the conditions (applicable gap duration) under which gNB indicates performing CAT2 LBT to the UE should be specified. |

Moderator comment: The symbol duration in FR2-2 is smaller or comparable to the observation slot used for LBT contention. As a consequence, in any of the options 1/2/3 for Y, the timelines aligned to NR numerology can be applied without risk of UE losing the medium due to timelines for contention. The complexity of CP extension can be removed.

Proposed conclusion 2.4.1-2: (closed)

Rel.16 NR-U style Cyclic Prefix extension is not supported for FR2-2 at least for DCI scheduled UL transmission

FFS: If CP extension is supported for CG-PUSCH in FR2-2

Support: Nokia, Ericsson, Lenovo, Intel, Qualcomm, Apple, Xiaomi, Oppo, Transsion

Not support: LGE

Conclusion

Rel.16 NR-U style Cyclic Prefix extension is not supported for FR2-2 at least for DCI scheduled UL transmission

- FFS: If CP extension is supported for CG-PUSCH in FR2-2

Please provide your view:

| Company | View |
| --- | --- |
| Nokia, NSB | We support the conclusion |
| Ericsson | We support the conclusion. |
| Lenovo, Motorola Mobility | Support the conclusion |
| Intel | We think that we should first conclude on the option used, before making such a conclusion. If Option 2 is agree, then we are OK with such a conclusion, but if option 1 or 3 is supported then we think that cyclic prefix is needed. |
| Apple | Support the conclusion |
| Xiaomi | Agree with the conclusion |
| LG Electronics | We do not support this conclusion. CP extension could be beneficial in terms of channel occupancy aspects and NR-U principle can be easily extended as follows: Proposal: For the CP extension prior to PUSCH/PUCCH/SRS transmission, the CP extension is located in the symbol(s) immediately preceding the PUSCH/PUCCH/SRS allocation indicated by SLIV. The supported durations for CP extension at the UE are: - 0 (i.e., no CP extension) - C1*symbol length – 8us - C2*symbol length – 8us – TA - C1=1 for 120 kHz SCS, C1=4 for 480 kHz SCS, C1=8 for 960 kHz SCS - FFS: Whether C2 is fixed or implicitly derived based on TA for each subcarrier spacing |
| OPPO | We support the conclusion. |
| NEC | We support the proposal in principle. In our understanding, unlike NR-U with a limitation of maximum gap (25us) within a COT, in FR2-2 if the responding device can start transmission at any symbol within COT after Y from the end of the initiating device transmission and only a Cat LBT is needed immediately before the responding transmission, then no matter which option and symbol duration is used, CPE can be removed. Further, we think this conclusion is mainly about CPE for COT sharing, so what about CPE for CG-PUSCH transmission in FR2-2? In that case, a COT is initiated by UE. Moderator: That is a good point. We haven’t discussed this yet, and it needs a separate discussion. I clarified the scope in the proposal above. |
| Transsion | We support this conclusion. |
| Sony | We support the conclusion |
| ZTE, Sanechips | We support the conclusion. |
| DOCOMO | We support the conclusion. Our understanding is that CPE is generally to compensate the gap smaller than a symbol duration. Given the shorter symbol duration, we do not see it essential. |
| CATT | We support this conclusion. |
| Samsung | We are ok with the conclusion. |
| InterDigital | We support the conclusion. |
| Huawei, HiSilicon | We support the conclusion. |
| Futurewei | We support this conclusion. |
| Ericsson 2 | We supported the previous conclusion and propose to remove the first FFS and the text in red. We do not see a need for CP extension. CP extension was added in NRU to ensure the gaps of 16us or 25us could be met. However, there is no need to do that here as EN 302 567 do not specify any gap requirement. |

Moderator comment: The following discussion pertains to use of existing bits that signal CAPC, CP Extension and LBT mode in non-fallback and fallback DCI.   Refer to Table 7.3.1.1.1-4, Table 7.3.1.1.2-35 and Table 7.3.1.2.2-6 in 3GPP TS 38.212 Rel 16.

Proposal 2.4.1-3: (closed)

For fallback DCI formats 0_0 and 1_0, for FR2-2 operation, the ChannelAccess-Cpext field in DCI indicates the channel access type only. A new table similar to Table 7.3.1.1.1-4 is introduced with entries “Type 1 channel access in 4.4.1 of 37.213”, “Type 2 channel access in 4.4.2 of 37.213” and “Type 3 channel access in 4.4.3 of 37.213”, and “reserved”.

- For a UE not capable of supporting Cat 2 LBT, an indication for Cat 2 LBT for UL transmission will be treated as an indicate for Cat 3 LBT

Please provide your view:

| Company | View |
| --- | --- |
| Nokia, NSB | We support the proposal |
| Ericsson | We support the proposal in principle that the ChannelAccess-Cpext field in DCI indicates the channel access type only. However, we think that the Fallback DCI formats should not include the Type 2 channel access (CAT2 LBT) as it is only supported as an optional UE feature. Moreover, main use cases for fallback DCIs are for transmissions before RRC configuration, where UE features are not reported yet. Therefore, the gNB cannot indicate CAT2 LBT if the gNB does not know whether the UE supports the feature. Our proposal is: A similar table with title “Table 7.3.1.1.1-4B for FR2-2” which can include a single bit to indicate the entries “Type 1 Channel access” (CAT3 LBT) and “Type 3 Channel Access”(No LBT). Type 2 channel access could be incorporated in the Non-fallback DCI formats with a new table “7.3.1.1.2-35A for FR2-2” which includes the all the channel access types. Moderator: Understand Cat 2 LBT is optional. However, if a UE is capable of Cat 2 LBT and gNB wants to use it, not having this in the fallback DCI will imply only non-fallback DCI can use Cat 2 LBT, which may not be preferred. For the proposal, the intention is to leave it in fallback DCI, but gNB only use it when it knows the UE has the capability Response to Moderator: There seems very little motivation to include it in fallback DCI when cAT2 LBT is only optional and helps only as an optimization feature. Conventionally, fallback DCI is not used for optimized features to reduce the DCI overhead. Therefore, it may be better to just signal 1 bit and keep it as whether sensing is needed or not and use Non-fallback DCI to indicate which type of LBT is used. For COT sharing case, as you mentioned, it is a gNB choice whether CAT2 LBT is needed or not (depending on the regulations). Moderator: But the implication is, in Japan market, fallback DCI cannot be used to schedule UL transmission in COT anymore. I would recommend to add the bit for simplicity. |
| Lenovo, Motorola Mobility | Support the proposal |
| Intel | We support the proposal in principle, but we should first agree on the value of Y, before concluding on the specifics on how the ChannelAccess-Cpext field in DCI should be reinterpreted. |
| Apple | Support Ericsson’s revised proposal. Only 1 bit is needed. Moderator: Same answer to Ericsson. If we only budget 1 bit, we will not be able to use fallback DCI to schedule capable UE to transmit with Cat 2 LBT. |
| Xiaomi | Support the proposal |
| LG Electronics | We support the proposal in principle and we can further discuss the case where UE incapable of cat 2 LBT is indicated with cat 2 LBT in fallback DCI. Moderator: This is a good point. I guess we can introduce a rule says if a UE is not capable of cat 2 LBT but was indicated to perform Cat 2 LBT before transmission, it will use Cat 3 LBT instead. |
| OPPO | We support the proposal and agree with Moderator that Cat 2 LBT should be included in the fallback DCI. |
| NEC | We support the proposal in principle based on the same understanding about Proposed conclusion 2.4.1-2 |
| Transsion | We support the proposal. |
| Sony | We support the proposal |
| ZTE, Sanechips | We support the proposal and also agree to introduce a rule to deal with LG’s question. Moderator: Added some clarification above |
| DOCOMO | support |
| CATT | Support the proposal. |
| Samsung | We are ok with the proposal. |
| InterDigital | We support the proposal |
| Huawei, HiSilicon | We support the proposal We disagree with Ericsson’s argument for not including CAT2 LBT in the fallback DCI ignal. Fallback DCI can be used after UE capability signalling and RRC establishment. |
| Futurewei | Support |
| Huawei, Hisilicon 2 | We provide our views in the Email discussion also here. We can support the proposal with the following update: Proposal 2.4.1-3: For fallback DCI formats 0_0 and 1_0, for FR2-2 operation, the ChannelAccess-Cpext field in DCI indicates the channel access type only. A new table similar to Table 7.3.1.1.1-4 is introduced with entries “Type 1 channel access in 4.4.1 of 37.213”, “Type 2 channel access in 4.4.2 of 37.213” and “Type 3 channel access in 4.4.3 of 37.213”, and “reserved”. - FFS: For a UE not capable of supporting Cat 2 LBT, an indication for Cat 2 LBT for UL transmission will be treated as an indicate for Cat 3 LBT |
| Ericsson 2 | Response to Moderator 2: There is no CAT2 LBT requirement even in Japan. It was only added as an optional feature to support conformance to “sensing” requirement. We do not see the motivation to specify Cat2 LBT in fallback DCI and then add a sentence to say if the UE does not support CAT2 LBT, it means CAT3 LBT. We think this is overspecification for an optimization, as CAT2 is only an optional feature and not required by any regulatory domain. It was added to support operation in Japanese regulations. Even in Japan, a device may only implement CAT3 LBT and need not implement CAT2 LBT. Therefore, we cannot support this agreement. We prefer a simple 1-bit solution that reduces fallback DCI overhead and state changes associated with processing DCI. |

Proposal 2.4.1-4: (closed)

For Non-Fallback DCI formats, for FR2-2 operation, for the configuration of the ChannelAccess-Cpext field in DCI to indicate the channel access type only, new tables are introduced indicating channel access types for FR2-2, with entries “Type 1 channel access in 4.4.1 of 37.213”, “Type 2 channel access in 4.4.2 of 37.213” and “Type 3 channel access in 4.4.3 of 37.213”.

Agreement

For Non-Fallback DCI formats, for FR2-2 operation, for the configuration of the ChannelAccess-CPext field in DCI to indicate the channel access type only, new tables are introduced indicating channel access types for FR2-2, with entries “Type 1 channel access in 4.4.1 of 37.213”, “Type 2 channel access in 4.4.2 of 37.213” and “Type 3 channel access in 4.4.3 of 37.213”.

Please provide your view:

| Company | View |
| --- | --- |
| Nokia, NSB | We support the proposal. One could add “indicates the channel access type only.” As in proposal 2.4.1-3. |
| Ericsson | We support the proposal and agree with Nokia’s suggestion. |
| Lenovo, Motorola Mobility | Support the proposal |
| Intel | Same comment as above. |
| Apple | Support in general. Different table should be defined for Alt 1 and Alt 3. ChannelAccess field in DCI can be 1 bit or 2 bits depends on Alt 1 and Alt 3 higher layer configuration. For Alt 1, only 1 bit is needed to indicate CAT3, no LBT. For Alt 2: 2 bits are needed: CAT3, CAT2 and no LBT. Moderator: Not sure we need different tables. For the case Cat 2 LBT is not needed, gNB simply should avoid configure the UE will Cat 2 LBT for the UE specific table. Note this table is just all entries that the gNB can configure. gNB will form a smaller table by configuration for each UE. |
| Xiaomi | Support the proposal. |
| LG Electronics | Removal CAPC from ChannelAccess-Cpext field in DCI is OK but we need further discussion on whether CP extension can be used or not. |
| OPPO | We support the proposal. |
| NEC | Same comment as above. |
| Transsion | We support this proposal. |
| Sony | We support the proposal |
| ZTE, Sanechips | Support the proposal |
| DOCOMO | Support |
| CATT | Support the proposal. |
| Samsung | We are ok with the proposal. |
| InterDigital | We support the proposal |
| Huawei, HiSilicon | We support the proposal. |
| Futurewei | Support |

For the maximum gap Y, we have the following options from earlier agreement.

- Option 1: Y=8 us (motivated by need to operate in all regions)

- Option 2: Y=a multiple number of OFDM symbols

- Option 3: gNB determines Y (for example, according to local regulation)

If we can agree on the conclusion in 2.4.1-1 (UE does not need to know Y and will follow DCI), and different region may have different requirements, the moderator propose to adopt option 3 to leave the flexibility to gNB, so the proper number can be selected to comply with local regulation.

Proposed conclusion 2.4.1-5:

On COT sharing from an initiating device transmission to responding device transmission, when a maximum gap Y is defined, such that a responding device transmission can occur without LBT only if the transmission starts within Y from the end of the initiating device transmission, and a responding device transmission can occur with Cat 2 LBT if the transmission starts later than Y from the end of the initiating device transmission.

- gNB determines Y as gNB implementation (for example, according to local regulation) and Y will not be captured in 3GPP spec

  - Note: This is Option 3 in earlier agreement

Please provide your view:

| Company | View |
| --- | --- |
| Xiaomi | Support in general. |
| LG Electronics | We support Option 1. |
| OPPO | We do not support the proposal. Since Cat 2 LBT uses the same sensing structure as the 8us initial deferral period as in eCCA, the gap Y = 8us, i.e. Option 1, can be considered following NRU Rel-16 principle. Moderator: I believe this Cat 2 LBT restriction only implies Y should be no shorter than 8us. Since we don’t have any regulation says the node will loss the COT if the gap is larger than a certain value, I would suggest we don’t pick a number. Moreover, we have a clarification question. In the proposal, does the initiating device transmission also include the transmission when the initiating device uses a transmission beam mismatching the beam for responding device transmission? Moderator: Not sure I understand the question. Do you mean the initiating device transmits with a beam that the responding device cannot receive? For example the initiating device is serving another UE in a different direction before scheduling another UE to transmit? I think this should be supported. This is simply TDM serving different Ues in the same COT. |
| Transsion | One question for clarification, will the value of Y be indicated to UE? If the value of Y is transparent to UE, then does the UE need to switch from Cat 2 LBT to No LBT for its configured UL transmission within the COT when the gap is smaller than Y? Moderator: No the UE does not know. We don’t have mechanism to upgrade Cat 2 LBT to no LBT |
| ZTE, Sanechips | In our view, gNB may not notify the specific gap length to UE, but it needs to define a clear gap length to determined whether Cat2 LBT needs to be performed. So we prefer Option1. Moderator: Given we have Alt 1 (gNB does not need Cat 2 LBT at all, no matter how long the gap is), introducing a max gap in Alt 3 may not be essential anymore |
| DOCOMO | Support. No clear motivation to define Y. |
| CATT | Support. |
| Intel | We are not OK with the above conclusion, and to leave Y up to implementation. Either Option 1 or Option 2 are preferred, with slight preference for Option 2. In our view, Y should be chosen to be the smallest gap either allowed by the regulatory requirements or to permit CAT-2 from being performed. Choosing Y larger may only lead to a poor spectrum utilization. Moderator: The problem is, we don’t have any regulation provides a number. And if we do, and we pick the smallest number, it will be unfair for regions with a more relaxed regulation. Given the observation we hardly see any LBT failure event with the loose EDT, I highly doubt it matters with different Y choices. |
| Ericsson | We support the sub-bullet point but we are not clear why we need this elaborate agreement. We already have an agreement from RAN1 #106-e for COT sharing case that allows CAT2 LBT as an optional feature motivated by Japan regulations. Perhaps the agreement could focus only on this specific value of Y and not on the other aspects that have already been agreed. Moderator: Might be good to change it to a conclusion. |
| Huawei, HiSilicon | We do not support the proposal. We support Option 2. We believe that, similar to Rel-16 NR-U, the conditions (applicable gap duration) under which gNB indicates performing CAT2 LBT to the UE should be specified. |
| LG Electronics | We do not support the proposal. It cannot be understood how UE can determine whether to perform Cat2 or Cat 3 LBT, without specifying Y value. Moderator: I believe it is clear from all previous discussions that the UE does not know when the DL transmission ends, so there is nothing for UE to compare with Y, even if it is known. The UE will simply follow DCI indication on LBT type |
| Ericsson 2 | We are ok with the proposed conclusion. |
| TCL | We support the conclusion. It is roughly in the same spirit of NR-U. |

### Second round discussions

Proposal 2.4.2-1:

For fallback DCI formats 0_0 and 1_0, for FR2-2 operation, the ChannelAccess-Cpext field in DCI indicates the channel access type only.

- Option 1: A new table similar to Table 7.3.1.1.1-4 is introduced with entries “Type 1 channel access in 4.4.1 of 37.213”, “Type 2 channel access in 4.4.2 of 37.213” and “Type 3 channel access in 4.4.3 of 37.213”, and “reserved”.

  - Note: This option requires 2 bis in fallback DCI

  - For a UE not capable of supporting Type 2 LBT, an indication for Type 2 LBT for UL transmission will be treated as an indicate for Type 1 LBT

  - Support: Lenovo, Qualcomm, Futurewei, Intel, LGE, Oppo, DCM, vivo, ZTE, HW

- Option 2: A new table similar to Table 7.3.1.1.1-4 is introduced with entries “Type 1 channel access in 4.4.1 of 37.213” and “Type 3 channel access in 4.4.2 of 37.213”.

  - Note: This option requires 1 bit in fallback DCI. This option also implies in Japan, fallback DCI cannot be used to schedule UL transmission with Type 2 LBT

  - Support: Samsung, Apple, Ericsson, TCL

Please provide your view:

| Company | View |
| --- | --- |
| Lenovo, Motorola Mobility | Support Option 1 |
| Intel | We prefer Option 1, but once again we have concerns with the terminology used: Cat-3 should be replaced with type 3, since Cat-3 has not been defined and we do not see any technical benefit in introducing an additional LBT procedure compared to those already specified. - Option 1: A new table similar to Table 7.3.1.1.1-4 is introduced with entries “Type 1 channel access in 4.4.1 of 37.213”, “Type 2 channel access in 4.4.2 of 37.213” and “Type 3 channel access in 4.4.3 of 37.213”, and “reserved”. - Note: This option requires 2 bis in fallback DCI - For a UE not capable of supporting Cat 2 LBT, an indication for Cat 2 LBT for UL transmission will be treated as an indicate for Cat 3 Type 3 LBT |
| Samsung | Prefer Option 2 to save one bit from the DCI size. |
| Futurewei | Fine with Option 1 |
| LG Electronics | Support Option 1. |
| Apple | Prefer option 2 to save one bit for fall back DCI. CAT2 is optional and not the common use case. We do not agree on the note for option 2. For Japan, regional limitations can be added in 37.213 like current specification for “4.1.1.1 Regional limitation on channel occupancy time”, where Japan specific COT is defined. Similar description can be used for UL transmission, since current Japan regulation no LBT is not allowed. CAT-2 and CAT-3 are all possible up to implementation. Therefore, there is no need for DCI to indicate type1,2,3 LBT dynamically in Japan. |
| OPPO | We support Option 1. We understand Cat 2 LBT is an optional UE feature, but the use case that gNB indicates Cat 2 LBT in fallback DCI should be supported, e.g. for COT sharing and Rx-assisted LBT. |
| DOCOMO | On top of the revision by Intel, perhaps the following modification captures the intention of Intel more? - Option 1: A new table similar to Table 7.3.1.1.1-4 is introduced with entries “Type 1 channel access in 4.4.1 of 37.213”, “Type 2 channel access in 4.4.2 of 37.213” and “Type 3 channel access in 4.4.3 of 37.213”, and “reserved”. - Note: This option requires 2 bis in fallback DCI - For a UE not capable of supporting Cat Type 2 LBT, an indication for Cat Type 2 LBT for UL transmission will be treated as an indicate for Cat 3 Type 3 LBT We support Option 1. Moderator: Updated. BTW in the current 37.213 CR, Type 1/2/3 are Cat 3/2/1 LBT respectively |
| vivo | We support option 1 with the understanding that “ For a UE not capable of supporting Type 2 LBT” means UE indicating not support Type 2 LBT in capability signaling Moderator: Correct |
| Ericsson | We support Option 2. We also object the note in Option 2. We agree with Apple that for Japan CAT2 and CAT3 LBT both are applicable as there is no specific definition. In addition, there is a typo in Option 2. Option 2: A new table similar to Table 7.3.1.1.1-4 is introduced with entries “Type 1 channel access in 4.4.1 of 37.213” and “Type 32 channel access in 4.4.2 of 37.213”. - Note: This option requires 1 bit in fallback DCI. This option also implies in Japan, fallback DCI cannot be used to schedule UL transmission with Type 2 LBT Moderator: Right. Fixed |
| ZTE, Sanechips | We support Option1 |
| Huawei, Hisilicon | We can support Option 1 “as is” so if UE does not support Type2, it would perform Type1 (CAT3) instead. We cannot support Option 2 although we believe that in Option 2, “Type 2 channel access in 4.4.2 of 37.213” meant to be “Type 3 channel access in 4.4.3 of 37.213” (In either case, we don’t support it). |
| TCL | We support option 2. It is more elegant. |
| Transsion | We support option 1. |
| CATT | We prefer Option 1 |
| Nokia, NSB | We support Option 2. As Cat2 LBT is not supported by all UEs, it is more logical to use only Cat3 in the fallback DCI. |

In 8.2.2, there is a discussion on introduce beam-specific DCI 2_0 delivery

Several contributions show the following proposal; however one company prefers to not discuss this enhancement as part of Release 17.

Please comment on the following proposal.

Proposal: In DCI format 2_0, the following parameters can be indicated in a beam-specific manner

- Remaining CO duration

- Available RB set

- Search space group switching

| Company | Comment |
| --- | --- |
| Xiaomi | For the first bullet, It really depends on how to define COT. Whether it is per node or per beam, which we haven’t decided about it . For the second/third bullet, If directional LBT is adopted, it is possible to support per beam available RB set/SSGS. |
| Qualcomm | We support the proposal. |
| Transsion | We support the proposal. |
| Nokia, NSB | We support this proposal |
| ZTE, Sanechips | We suggest to de-prioritize this proposal. |
| LG Electronics | We support the proposal |
| NTT DOCOMO | The motivation is not clear for us, but we are open to discuss. |
| Intel | The proposal is fine in principle. |
| Lenovo, Motorola Mobility | Support the proposal |
| Apple | WE support the proposal. |
| Futurewei | It is related to channel access, should be treated in that AI |
| Samsung | We support the proposal. |
| Sony | We support |
| Ericsson | We do not support this proposal; it should be de-prioritized. It is not even clear what it means and what is the benefit of signaling these parameters in a beam-specific manner. |
| Sharp | We support the proposal. |
| vivo | We support the proposal |

Most companies support the proposal. Some companies point out that there is a lack of motivation, others consider this topic is preferably handled in the channel access AI. FL suggests to clarify where this topic should be handled, and continue discussion accordingly.

Since the topic is related to COT sharing, the FLs decide to continue the discussion in 8.2.6 intead.

The moderator believes the issue is, for beam forming transmission, the COT is for a certain beam. However, when DCI 2_0 is transmitted, all UEs inside or outside the beam can decode it, as long as it is close enough. Then DCI 2_0 can be received by unintended UEs. The proposal is to introduce beam specific DCI 2_0 to control which UEs can use the COT

Proposal 2.4.2-2

Introduce beam specific COT-SI (COT duration and (if introduced) available RB sets) delivery in DCI 2_0

- FFS: How to introduce beam specific COT-SI

- FFS: If this applies to SFI as well

- FFS: If this applies to SSGS as well

Please provide your view:

| Company | View |
| --- | --- |
| LG Electronics | We are generally fine with this proposal, but the second FFS (for SFI) doesn’t seem to be necessary since SFI should not be beam-specific. Moderator: That is why it is FFS |
| Apple | Not sure the exact proposal here. The COT can be used for transmission with multiple beams if the sensing beam cover transmission beams. Moderator: Right. But the problem is for UEs not in the wider sensing beams but happen to decode the COT-SI |
| NEC | We support the proposal and be open to consider all FFSs. |
| Lenovo, Motorola Mobility | Support the proposal and open to discuss all the FFS |
| DOCOMO | We are generally fine, while not sure it can be supported within Rel-17. Moderator: Yes it is quite late. The problem is I believe if we don’t have something like this, we may have some “accidental COT sharing”. Not sure if we can live with that. |
| vivo | We support the proposal. The beam related information can be carried in the DCI 2_0. The beam-specific COT-SI does not apply to SFI, but the application to SSGS is FFS. |
| Ericsson | We do not support this proposal. Beam-specific COT is not allowed according to the regulations. A COT may consist of multiple bursts or multiple beams (as long as sensing beam covers transmission beams) upto a maximum duration of 5 ms. Moreover, we have a conclusion that we don’t support per beam LBT. There is no motivation to support per-beam COT. Conclusion: There is no consensus to support per beam LBT mode or no-LBT mode UE specific gNB indication. Moderator: The problem is not about per beam LBT. It is about if a gNB acquires the channel with a beam, can we allow UE outside the beam to share the COT? Response to Moderator: Thank you for the clarification. Now we understand the problem. Although gNB can share the COT with any UE, our understanding is that this is not allowed according to regulations. If a gNB acquires a COT with a specific beam, anything outside the beam should not be allowed. This because, the gNB senses only in that direction to acquire the COT and ignores interferences in other directions. It is like acquiring a 20 MHz channel and not allowing transmissions from a UE in the neighbouring 20 MHz chunk even though the gNB can handle it. Hence, we object to this proposal. |
| ZTE, Sanechips | In principle, we are fine with this proposal, but maybe we need more time to discuss some potential issues such as the above listed FFSs and so on, so we tend to de-prioritize this issue. |
| Samusng | We are ok with the proposal. |
| Huawei, HiSilicon | We have concerns about the proposal and cannot agree with it at this time. First, unlike the explanation prior to the proposal and what the proposal itself seems to imply, we never agreed that COT is acquired per beam. In fact, the whole discussion in 2.7 and the corresponding agreements are related to multi-beam COT. Second, we are not sure what COT-SI means. Is it “COT duration and (if introduced) available RB sets”? If yes, no need to use a new terminology. Then, the question is why we want to indicate these values as well as SFI and SSGS beam specifically? Moderator: Even in the discussion in 2.7, we are talking about the sensing beam used to acquire the COT covers the transmission beam(s). My understanding is the COT is only acquired in the direction covered by the sensing beam. It may not make sense to claim the COT can be applied to direction not sensed. Note the intention of the proposal is to associate COT duration (at least) with sensing beam used to acquire the COT, not to associated it with transmission beam. |
| Intel | We support the proposal in principle, and open to discuss the FFSs. However, as other companies pointed out, we may need to conclude first the discussion related to Multi-beam COT and the specifics of the LBT procedure before we can continue discussion on this topic. |
| TCL | We support the proposal. We can have more discussions about the detail. Generally, the beam-centric design will make the COT beam related. The SFI may not related to beams. If it is, the control will become complex. The SSGS may be beam specific, which reduces the monitoring overhead. |
| Transsion | We support this proposal and open to discuss the first and the third FFS. |
| Mediatek | We propose to defer this issue since the motivation for this issue is not clear to us. Besides we have several clarification questions: 1. Under what circumstances that accidental COT sharing will occur and what’s the probability of this event, 2. What is the proposal to avoid/decrease the occurrence of this event, 3. The proposal seems to introduce a new DCI field to avoid accidental COT sharing, however, channel access field is not in DCI 2-0, does that mean we need to transmit DCI one of the format 0-0/0-1/1-0/1-1 and DCI 2-0 avoid the accidental COT sharing? |
| CATT | We are ok with the proposal. |
| Nokia, NSB | Although we are ok in principle, we can agree to deprioritize this feature given the little time we have left in this WI. |

Proposed conclusion 2.4.2-3

UL to DL COT sharing is supported for FR2-2 unlicensed operation, including from dynamically scheduled UL and CG-PUSCH.

Please provide your view:

| Company | View |
| --- | --- |
| LG Electronics | Support |
| Apple | Support |
| NEC | Support |
| OPPO | Support |
| Lenovo, Motorola Mobility | Support |
| DOCOMO | Support |
| vivo | We support the proposed conclusion. |
| Ericsson | We support the proposed conclusion. Perhaps, we need to discuss the issues raised by us in Proposal 2.1.2-2. |
| ZTE, Sanechips | Support |
| Samsung | Support |
| Huawei, HiSilicon | We support the proposal |
| Intel | Support |
| TCL | Support |
| Transsion | Support. |
| CATT | Support. |
| Nokia, NSB | We support this proposal |

Discussion 2.4.2-4 (close)

For CG-PUSCH in FR2-2 unlicensed operation, about CP extension

- Alt 1: Do not introduce CP extension for CG-PUSCH for FR2-2

  - Support: Apple, DCM, ZTE

- Alt 2: Introduce CP extension for CG-PUSCH for FR2-2 with maximum duration of 1 symbol at 15KHz

  - Since we don’t have interlaced waveform, the CP extension candidate numbers are the same as Rel.16 NR-U version with full bandwidth allocation, i.e. {1 sym @15KHz, 61us, 52us, 43us, 34us, 25us, and 16us} as in Table 5.3.1-2 of 38.211

  - Support (but may need to revisit the max CP extension length): LGE, NEC, Lenovo, Ericsson, vivo

Discussion 2.4.2-4a

For CG-PUSCH in FR2-2 unlicensed operation, about CP extension

- Alt 1: Do not introduce CP extension for CG-PUSCH for FR2-2

  - Support: Apple, DCM, ZTE

- Alt 2: Introduce CP extension for CG-PUSCH for FR2-2 with maximum duration of 1 symbol at 15KHz

  - Since we don’t have interlaced waveform, the CP extension candidate numbers can leverage the Rel.16 NR-U version with full bandwidth allocation

  - FFS: The set of CP extension lengths, including the maximum CP extension length

  - Support: LGE, NEC, Lenovo, Ericsson, vivo, Intel, TCL

Please provide your view:

| Company | View |
| --- | --- |
| LG Electronics | We prefer Alt 2 but need further discussion on whether the maximum duration is based on 15 kHz 1 symbol or other SCS (e.g., 120 kHz) 1 symbol. |
| Apple | Alt 1 |
| NEC | We prefer Alt 2 in principle, and share the similar view as LG. Since the 25/16us gap limit and 9us sensing slot duration are not applicable to FR2-2, we think the CPE candidate numbers should be reconsidered, as well as maximum duration of 1 symbol at 15kHz. |
| Lenovo, Motorola Mobility | We prefer Alt 2. |
| DOCOMO | Support Alt 1. |
| vivo | CP extension is still feasible for CG-PUSCH in FR 2-2 to avoid collision when full bandwidth allocation is adopted. However, the length of CPE should be further discussed, since the sensing slot duration in FR 2-2 is not the same as that in FR1. Therefore, we think it is not feasible to directly use the maximum duration of 1 OS at 15kHz for FR 2-2. Besides, the gap between the candidate CPEs should be determined based on the sensing slot duration in FR2-2, i.e., 5us. |
| Ericsson | We support Alt2. We think this CP extension is not to maintain gaps like the DL-UL COT sharing case we discussed before, but this is to ensure there are offsets between UEs trying to transmit CG-PUSCH so that it avoids collisions. |
| ZTE, Sanechips | We prefer Alt 1 |
| Moderator | Seems that all companies supporting Alt 2 suggest to revisit the maximum length of CP extension. I update the proposal to leave maximum length and step sizes as FFS |
| Huawei, HiSilicon | We support Alt 1. At this late stage of WI we do not think it is a good idea to start discussion of how to prioritize CG-PUSCH transmission inside or outside of COT using CP extension. Also, if we go with Alt2, we cannot use the offset values of Rel-16 which are based on the 9 us sensing slot. |
| Intel | We support Alt-2, and we share view with Ericsson regarding the rationale of why the CP extension is needed. |
| TCL | We support Alt-2, too. |
| Transsion | We support Alt 2 and share the same views of vivo and Ericsson. |
| Mediatek | We support Alt 1. |
| Nokia, NSB | Given that the symbol raster at the considered SCS is already much finer than at 5GHz, there is little need for CP extensions. Hence we are ok with Alt2 |

For CG-PUSCH in FR2-2 unlicensed operation, for CG-UCI COT sharing field, currently both duration and offset are from {1.. 39} in unit of slots. This may need to be extended

Proposal 2.4.2-5

For CG-PUSCH to DL COT sharing, extend the duration and offset range to {1, …, 319}.

Please provide your view:

| Company | View |
| --- | --- |
| LG Electronics | What is the rationale behind the maximum value as 639? If we consider 5 msec MCOT for 960 kHz, then the maximum value could be 319… Moderator: The rationale is I calculated wrong 😊 Fixed |
| Apple | Same question as LG. |
| NEC | We support the proposal in principle, and be open to discuss the parameter in detail. |
| DOCOMO | Same question as LG. |
| vivo | The maximum COT in FR 2-2 is 5 ms, which is different from that in FR1. Therefore, we should take this into account to determine the duration and offset range, a maximum value of 319 is enough. |
| Ericsson | We support the proposal in principle and can further support maximum COT durations allowed in FR2-2. |
| ZTE, Sanechips | Same question as LG |
| Samsung | Agree with the view from LG. |
| Huawei, HiSilicon | Agree in principle but, as LG pointed out, the offset and duration should be {1…319} |
| Intel | We support the current proposal with maximum value of 319. |
| TCL | We support the proposal. |
| LG Electronics2 | Support |
| Transsion | We support the proposal. |
| CATT | We are ok with the current proposal. |
| Nokia, NSB | We are ok with the updated proposal |

## Cat 2 LBT

Agreement:

For Cat 2 LBT, down-select from the following alternatives

- Alt 1: Do not introduce Cat 2 LBT for 60GHz unlicensed band operation

- Alt 2: Introduce Cat 2 LBT for 60GHz unlicensed band operation

Agreement:

If Cat 2 LBT is introduced, the following use cases can be further studied:

- Resume transmission after a gap Y:  Cat 2 LBT may be used to resume transmission by the initiating device within the COT after a gap Y (FFS the value of Y)

- COT sharing: Cat 2 LBT may be used before transmission by a responding node sharing a COT

- Multi-Beam LBT:  Cat 2 LBT may be used before switching to a new transmission beam (not used in earlier part of the COT) in a COT with TDM beams, or resume a previously used transmission beam after a gap Z (FFS the value of Z)

- Rx-Assistance:  Cat 2 LBT may be used for sensing at the receiver as a responding device for Rx-Assistance measurements and associated signalling

Other use cases not precluded.

FFS if Cat 2 LBT is mandated for each use case or not.

| Company | Key Proposals/Observations/Positions |
| --- | --- |
| Huawei HiSilicon | Proposal 20: The following use cases of CAT2 LBT related to COT initiation should be prioritized in the discussion due to the low complexity and overhead of CAT2 LBT compared to eCCA: - Starting transmission on a secondary channel in Type B multi-channel access, if supported - Energy measurement and reporting of Rx-assistance information by the receiver in Rx-assisted LBT, if supported |
| ZTE Sanechips | Proposal 9: Similar restriction as defined in Type 2C channel access procedure in TS 37.213 can also introduced in above 52.6GHz NR-U frequency band but the length of a transmission can be relaxed. Proposal 16: Cat 2 LBT can be considered in the following use cases:  1) Resuming transmission/beam switching situation;  2) Type B multi-channel access procedure;  3) Rx-assisted LBT when COT is initiated by transmitter. |
| Nokia Nokia Shanghai Bell | Proposal 1: A short contention window size of [4] is used in the LBT procedure Observation 1: For initiating device resuming transmission after a long transmission gap, Cat-2 LBT performance cannot be compared against Cat-3 LBT as long as Cat-3 LBT design remains open. Proposal 2: Do not support Cat-2 LBT in beam switching or in multi-channel LBT. Observation 4: Short contention window of [4] observation slots ignaling flexible LBT timing for SSB transmissions. Observation 10: Cat-2 LBT at every gNB beam switch would cause significant increase in overhead and is not even possible between the SSBs in the agreed SSB time locations. Observation 11: Use of LBT provides mostly loss in median throughput compared to no-LBT mode and reduces throughput for cell edge Ues Observation 12: Simulation results do not show any gain from introduction of additional Cat-2 LBT at gNB beam switch during COT. |
| CATT | Proposal 12: Performing Cat 2 LBT before beam switching within the COT could be supported, and it can be decided by gNB. |
| Sony | Proposal 2: Introduce Cat 2 LBT for 60 GHz unlicensed band operation |
| Ericsson | Observation 13 It is worthy to note that, use of CAT3 LBT is also an option for operation in Japan and CAT2 LBT is a UE capability feature Observation 15 Cat2 LBT is not specified in HS EN 302 567 Observation 16 Simulation studies show that there is no gain using Ca2 LBT compared to no LBT for the proposed used cases. Proposal 21 Reuse Rel-16 framework for indication of CAT2 LBT in the DCI. Proposal 22 Do not support Cat2 LBT for any of the use cases in 52.6 GHz to 71 GHz. It is not precluded to do CAT2 LBT in addition to CAT3 LBT requirements. |
| Intel | Proposal 6: The value of Zmin and Zmax are configured by the network, and the following conditions shall be always met:  Zmax > Zmin;  Zmin ≥ 0;  Zmax≥ 3. Proposal 7: When a UE is capable to perform Cat-2 LBT, whether to operate with or without Cat-2 LBT would be dynamically indicated by the gNB via scheduling DCIs. Proposal 8: Y is defined as:  1 OFDM symbol for 120 KHz SCS,  4 OFDM symbols for 480 KHz SCS,  8 OFDM symbols for 960 KHz SCS. Proposal 9: If an initiating device is capable to perform Cat-2 LBT, and if the initiating device performs an additional burst within the initiated COT which may be separated with any prior burst of at least a minimum gap Y, then under Alt-3 a Cat 2 LBT is needed before the initiating device transmission. Proposal 10: In addition to support CAT-2 LBT for COT-sharing procedure, the gNB may configure the UE to use CAT-2 LBT for RX-assisted LBT. |
| NEC | Proposal 6: Cat 2 LBT for 60GHz unlicensed band operation should be introduced for resuming transmission by the initiating device within the COT after a gap Y. Proposal 7: Cat 2 LBT for 60GHz unlicensed band operation should be introduced for channel sensing of receiver assistance measurements. Proposal 8: Cat 2 LBT for 60GHz unlicensed band operation should be introduced for Type B multi-channel access. |
| Panasonic | Proposal 3: Agree on Proposal 2.7.1-3 in Feature Lead Summary [1], and further select Alt 3 by recognizing that it is a valid use case for Cat-2 LBT i.e. Within a COT with TDM of beams with beam switching, at least support Alt 1 • Alt 3 is additionally supported |
| InterDigital Inc. | Proposal 12: A UE determines whether to use Cat 2 LBT based on the gap duration Y between the upcoming transmission and a preceding transmission on the same beam. |
| LG Electronics | Proposal #1: Regarding the gap Y for Cat-2 LBT, Option 1 (Y=8 us) should be supported and the CP extension may need to be discussed when the transmission cannot be started at the OFDM symbol boundary. Proposal #2: If ChannelAccess-Cpext field is kept as 2 bits in DCI format 0_0 and 1_0 for FR2-2 unlicensed band same as in NR-U, it is necessary to define UE behaviour for LBT type indication before identifying the Cat-2 LBT capability of the UE, such as initial access. |
| NTT DOCOMO INC. | Proposal 3: Use of Cat-2 LBT should be considered for the transmission of a certain signal/channel, for which LBT is not needed in a region (e.g., BRAN with short control ignaling), while LBT is always needed in another region (e.g., Japan). |
| Qualcomm Incorporated | Proposal 8: Introduce Cat 2 LBT for the use case of Multi-Beam LBT. |
| WILUS Inc. |  Proposal 1: We support Alt-2 to introduce Cat 2 LBT for 60GHz unlicensed band operation. |
| OPPO | Proposal 9: Cat-2 LBT should be introduced for resuming transmission within the COT after a gap and Rx-assisted LBT. Proposal 10: For resuming transmission after a gap, the beam-specific gap duration can be considered. Proposal 12: Introduce Cat 2 LBT for the independent per-beam LBT sensing procedure. |

### First round discussions

Discussion 2.5.1-1 (low priority for now)

Support potential CAT2  LBT use cases:

- Resume transmission after a gap Y:  Cat 2 LBT may be used to resume transmission by the initiating device within the COT after a gap Y (FFS the value of Y)

  - ZTE, Intel, Lenovo, Motorola Mobility, NEC, Transsion, Futurewei, Apple, OPPO, WILUS, TCL, Sony, Samsung, InterDigital, Transsion, Convida

- Multi-Beam LBT:  Cat 2 LBT may be used before switching to a new transmission beam (not used in earlier part of the COT) in a COT with TDM beams, or resume a previously used transmission beam after a gap Z (FFS the value of Z)

  - FUTUREWEI, Spreadtrum, , CATT, Lenovo, Motorola Mobility, ZTE, vivo, LG, NEC, WILUS, TCL, Sony, Samsung (could be applicable to certain area up to regulation), InterDigital, Convida

  - No: Intel, Nokia

- Rx-Assistance:  Cat 2 LBT may be used for sensing at the receiver as a responding device for Rx-Assistance measurements and associated signalling

  - Huawei, ZTE, Intel, Lenovo, Motorola Mobility, vivo, LG, NEC, Futurewei, OPPO, WILUS, TCL, Samsung, Convida

- Multi-channel Type B access if supported

  - Huawei, ZTE. NEC, vivo, WILUS, Samsung

  - No: Intel Nokia

- For a certain transmission, which can be treated as Short Control Signaling in BRAN, in a region where Short Control Signaling is NOT defined but LBT is mandatory

  - Docomo

- In general

  - CAICT

- No

  - Ericsson, Nokia (no for beam switch, multichannel,), Charter

Please provide your view if not captured above:

| Company | View |
| --- | --- |
| Nokia, NSB | We see no need for Cat 2 in any of the use cases above, except potentially” For a certain transmission, which can be treated as Short Control Signaling in BRAN, in a region where Short Control Signaling is NOT defined but LBT is mandatory”. |
| Ericsson | We do not see a need to support CAT2 LBT for the use cases above. However, cat2 LBT is already supported, which can be optionally used for those use cases based on implementation or gNB scheduling (e.g., indicating cat2 LBT in DCI). Therefore, we do not see the need for ignali discussions or new agreement to support cat2 LBT for those use cases. For regions where short control signalling transmissions are not defined, we have a proposal to include a bit in the SIB1 to indicate if LBT is needed for these transmissions or not. This way, whether CAT3 or CAT3 LBT is used for short control signalling transmissions is left to implementation. |
| Intel | We support Cat-2 for two specific use cases: - Resume transmission after Y gap. As already mentioned, this is motivated by Japanese regulatory requirements, which is mandated by ARIB, and whose text is very generic and does not define any concept of initiating or responding device, but rather distinguishes a device from a transmitter to a receiver. If the transmission power of the transmitter exceeds 10 mW, provide a carrier sense that will operate at beginning of the transmission.. - In this matter, our understanding is that carrier sensing would be needed at the beginning of every transmission, unless the transmissions are back-to-back. - Receiver assisted LBT: Cat-2 LBT is preferred to support scheme 2 for the RX assistance given that by using Cat4 at the receiver the LBT overhead may limit and overcome the benefits from using a receiver assisted mechanism. |
| OPPO | Our position is correctly captured. However, if the potential Cat 2 LBT use cases of resuming transmission after a gap and multi-beam LBT are supported simultaneously, the gap duration is counted per beam or per initiating device should be discussed. |
| Vivo | We support Cat 2 LBT for Receiver assisted LBT, beam switching and type B multi-channel access. |
| DOCOMO | We understand that Ericsson’s last part could cover “For a certain transmission, which can be treated as Short Control Signaling in BRAN, in a region where Short Control Signaling is NOT defined but LBT is mandatory”. We are actually ok with that direction. As mentioned by Intel, anyway LBT is needed for each non-consecutive transmission in Japan. In this sense, “Resume transmission after a gap Y” and “Multi-Beam LBT” will require LBT, which could be Cat-3 or Cat-2. If there will be a functionality to make it possible to use Cat-2 LBT for such case, that could be beneficial. |

## Rx Assistance

Agreement:

For receiver to provide assistance in channel access, channel sensing and reporting need to be performed. The following schemes can be further considered. Target down-selection by RAN1 #106bis-e

- Scheme 1: L1-RSSI based receiver assistance

  - Resource used for RSSI measurement

    - Alt 1: RSSI measurement is based on the time/frequency resources configured for ZP-CSI-RS

      - FFS: any enhancement needed for ZP-CSI-RS for this purpose (e.g., ZP-CSI-RS over all REs in BWP over one or more symbols).

    - Alt 2: Energy measurement on operating BW over indicated or specified number of symbols or time interval

  - L1-RSSI is reported in an AP-CSI report

  - L1-RSSI trigger in UL grant

    - FFS if L1-RSSI trigger can also be carried in DL grant

  - Timeline for L1-RSSI reporting is at least equal to AP-CSI reporting and RAN1 strives to tighten the timeline

    - Note: If L1-RSSI reporting timeline cannot be tighter than AP-CSI reporting timeline, this scheme is not needed

  - FFS: How to indicate the measurement beam for L1-RSSI

  - FFS: What is included in the L1-RSSI report, such as the value of RSSI measurement, comparison outcome with Energy Detection threshold, etc

- Scheme 2: CCA or eCCA based receiver assistance with existing phy channel/signals

  - Scheme 2-1: gNB schedules/triggers UL PUCCH/SRS transmission with the DL assignment DCI and indicates CCA or eCCA in the DCI. UE performs CCA or eCCA for the scheduled/triggered UL transmission and if LBT passes, transmits the Receiver-assistance information (implicitly or explicitly) in the PUCCH (or SRS in the case of 1-bit Rx-assistance) to indicate the LBT outcome. gNB detects the scheduled UL transmission to tell if UE passes the CCA or eCCA. After detecting the Receiver-assistance information, the downlink data transmission happens.

    - FFS if the downlink data transmission can be granted with the same DL DCI that schedules/triggers the first UL PUCCH/SRS transmission, in which case, the CCA or eCCA is performed for at least the first UL PUCCH/SRS transmission

  - Scheme 2-2: gNB schedules/triggers UL transmission PUSCH with the UL assignment DCI and indicates CCA or eCCA in the DCI. UE performs CCA or eCCA for the scheduled/triggered UL transmission and if LBT passes, transmits the Receiver-assistance information (implicitly or explicitly) in the PUSCH to indicate the LBT outcome. gNB detects the scheduled UL transmission to tell if UE passes the CCA or eCCA. After detecting the Receiver-assistance information, the downlink data transmission happens.

- Scheme 3: CCA or eCCA based receiver assistance with new RTS/CTS type transmission

  - New RTS/CTS-like signaling introduced.

  - gNB sends RTS-like signaling to UE. UE performs CCA or eCCA and if LBT passes, transmits CTS-like signaling to explicitly indicate the LBT outcome. gNB detects the CTS-like signaling to identify if the UE passed CCA or eCCA. After detecting the CTS-like signal, the data transmission happens

- Scheme 4: Legacy L3-RSSI with potential enhancements

  - FFS potential enhancements, e.g., supporting gNB indicating the beam used for UE RSSI measurement, supporting gNB indicating new reference SCS and measurement bandwidths

- Note: The schemes listed above are not mutually exclusive and should be discussed separately.

Conclusion:

There is no consensus to support CCA or eCCA based receiver assistance with new RTS/CTS type transmission

Agreement:

Support extending Rel.16 L3-RSSI to unlicensed operation in FR2-2

- Introduce RRC configuration for reference SCS, measurement duration, and measurement bandwidth

  - Extend the reference SCS/CP field (ref-SCS-CP-r16) and measurement duration field (measDurationSymbols-r16) in RMTC-Config

    - FFS value range and valid combinations for ref-SCS-CP-r16 and measDurationSymbols-r16

  - Introduce parameter in RMTC-Config to indicate the measurement bandwidth

    - FFS: Value range for measurement bandwidth

- For the QCL Type-D of L3-RSSI measurement, down-select one or both of the following alternatives

  - Alt 1: gNB configures the beam when configures the L3-RSSI measurement

  - Alt 2: Use the QCL type-D of the latest received PDSCH and the latest monitored CORESET

| Company | Key Proposals/Observations/Positions |
| --- | --- |
| Huawei HiSilicon | Observation 2: Schemes decoupling the Rx-assistance feedback from the scheduled DL reception have the following issues: - Reporting a high level of interference would result in latency and waste of the scheduling opportunity if the hidden node interfering burst has already ended by the time of scheduled DL reception. - In the converse, reporting a low level of interference would result in collision and poor performance especially for cell edge Ues if a hidden node interfering burst starts by the time of scheduled DL reception - The overall dynamic overhead is increased in the cell since two DCIs are required compared to Scheme 2-1 (with same DL assignment scheduling/triggering the UL transmission for CTS/interference level feedback) - Performance evaluations of the decoupled Schemes have never been provided/discussed in the SI phase during which Rx-assisted LBT was identified as a beneficial channel access mechanism to combat the hidden node issue in Rel-17 Proposal 23: For a receiver UE to provide assistance ignalingn in channel access in the DL scenario, support Scheme 2-1 with the downlink data transmission being scheduled by the same DL DCI that schedules/triggers the first UL PUCCH/SRS transmission. Proposal 25: For a receiver UE to provide assistance ignalingn in channel access in the DL scenario, support introducing a new field in DCI format 1_1 scrambled with C-RNTI, CS-RNTI or MCS-C-RNTI, to schedule/trigger PUCCH/A-SRS resource before the start of the scheduled PDSCH(s) - PUCCH: A 3-bit field ‘ChannelAccess-PUCCH resource indicator’ is introduced and the existing mechanism for indicating PUCCH resource can be reused o UCI Payload size is configurable between 1 bit (CTS only) or 7 bits (energy measurement report such as L1-RSSI) - A-SRS: 2-bit ‘Channel access indicator’ indicates the SRS trigger mode for reusing existing ‘SRS Request’ field to trigger a single aperiodic SRS resource set for receiver-assisted channel access, or trigger aperiodic SRS resource set(s) for legacy MIMO/positioning purposes, or both. O The UE can be configured with one or more aperiodic SRS resource set(s) in SRS-Config (Currently supported). For the configured aperiodic SRS resource sets, an optional RRC parameter (e.g., ‘SRS-ChannelAccess’) is configured to indicate that the SRS resource set is for receiver assistance report for channel access only. Proposal 26: For a receiver UE to provide assistance ignalingn in channel access in the DL scenario, support configuring/indicating a time offset of a small value range to the UE for transmitting the scheduled/triggered PUCCH/A-SRS resource with respect to the beginning of the scheduled PDSCH(s) - PUCCH: Add a new field of a configurable bitwidth (0, 1 or 2 bits) in the DCI format 1_1 to indicate the slot level offset from the indicated PUCCH resource to the start of the scheduled PDSCH(s), e.g., ‘ChannelAccess-PUCCH-to-PDSCH timing indicator’. - A-SRS: Higher layer parameters startPosition and slotOffset and can be reused such that slotOffset for an aperiodic SRS resource (set) triggered for providing receiver assistance in channel access is reinterpreted as the number of slots from the actual transmission of the triggered aperiodic SRS resource (set) to the start of the scheduled PDSCH(s). Proposal 27: For a receiver UE to provide assistance ignalingn in channel access in the DL scenario, support configuring a higher layer parameter providing the LBT type for the UE to access the channel and transmit the scheduled/triggered PUCCH/A-SRS - This can be provided using common or dedicated signaling. Proposal 28: For a receiver UE to provide assistance ignalingn in channel access in the DL scenario, the following procedures are applied: 1) A UE that has received a DCI format 1_1 scheduling/triggering PUCCH/A-SRS resource before the start of the scheduled PDSCH(s) transmits the triggered A-SRS or the scheduled PUCCH, including the detected energy level if configured, only if it has accessed the channel according to the UE-side LBT performed prior to the indicated time resource for transmitting the scheduled/triggered PUCCH/A-SRS. 2) A gNB that has transmitted a DCI format 1_1 to a UE scheduling/triggering PUCCH/A-SRS resource before the start of the scheduled PDSCH(s) may transmit the scheduled PDSCH(s) and any subsequent DL control/data only if it has received the scheduled/triggered PUCCH/A-SRS from that UE, the transmission of the scheduled PDSCH(s) is dropped otherwise. |
| Vivo | Proposal 14: Adopt the modified scheme 2. Scheme 2: CCA or eCCA based receiver assistance with existing phy channel/signals Scheme 2-1: gNB schedules/triggers UL PUCCH/SRS only transmission with the DL assignment DCI and indicates CCA or eCCA in the DCI. UE performs CCA or eCCA for the scheduled/triggered UL transmission and if LBT passes, transmits the Receiver-assistance information (implicitly or explicitly) in the PUCCH (or SRS in the case of 1-bit Rx-assistance) to indicate the LBT outcome. gNB detects the scheduled UL transmission to tell if UE passes the CCA or eCCA. After detecting the Receiver-assistance information, gNB determines if the downlink data transmission happens depending on the detection of the Receiver-assistance information. Scheme 2-2: gNB schedules/triggers UL transmission PUSCH with the UL assignment DCI and indicates CCA or eCCA in the DCI. UE performs CCA or eCCA for the scheduled/triggered UL transmission and if LBT passes, transmits the Receiver-assistance information (implicitly or explicitly) in the PUSCH to indicate the LBT outcome. gNB detects the scheduled UL transmission to tell if UE passes the CCA or eCCA. After detecting the Receiver-assistance information, gNB determines if the downlink data transmission happens depending on the detection of the Receiver-assistance information. |
| ZTE Sanechips | Proposal 18: Scheme 2 can be considered for CCA/eCCA based receiver assistance and propose to use the same DL DCI signalling to trigger/schedule UL transmission and DL data transmission considering complexity. |
| Fujitsu | Proposal 2: For Scheme 2 for receiver to provide assistance in channel access, support Scheme 2-1 for lower latency and signaling overhead. |
| Nokia Nokia Shanghai Bell | Observation 14: The network can operate scheme 2 in a fully standards transparent manner. There is no need to define further mechanisms to support scheme 2. |
| Ericsson | Proposal 14 Do not continue to discuss Scheme 2-1 and Scheme 3 for receiver assisted channel access. Proposal 19 Conclude that Scheme 2-2 with implicit feedback is already supported by the current specs. |
| InterDigital Inc. | Proposal 2: Directional receiver assistance is supported. |
| Convida Wireless | Proposal 10: Receiver assisted LBT and channel access should be supported in 52.6 GHz to 71 GHz. Proposal 12: For receiver to provide assistance, the following can be further discussed: legacy RSSI measurement and reporting with possible enhancements, AP-CSI report with possible enhancements and LBT at receiver using eCCA or Cat2 LBT. Proposal 13: For receiver to provide assistance, CCA or eCCA based receiver assistance with existing physical layer channel/signals can be considered. |
| LG Electronics | Proposal #4: For the receiver to provide assistance, the feedback mechanisms already supported by the current specification such as implicit method in Scheme 2 (appearance of the scheduled PUCCH/SRS/PUSCH) can be considered and it is not preferred to introduce the additional or new mechanism (such as added explicit payload bit in PUSCH/PUCCH or introduction of new RTS/CTS-like signalling in Scheme 3). |
| NTT DOCOMO INC. | Proposal 6: For Rx assistance: |
| Qualcomm Incorporated | Proposal 18: Approve the proposed conclusions 2.6.2-1, 2.6.2-2 and 2.6.2-3 from RAN1-106-bis-e discussions, [5] regarding Schemes 2-1 and 2-2 for Rx Assistance. |
| OPPO | Proposal 16: RTS-like signal can be carried in a PDCCH and CTS-like signal can be carried in a PUCCH. |
| Charter Communications | Proposal 2a: Extend the L3-RSSI reference SCS/CP field (ref-SCS-CP-r16) in RMTC-Config to include 120, 480 and 960 kHz SCSs. Measurement bandwidth field includes at least the set of maximum BWP sizes for different SCSs 120/480/960kHz. Proposal 2b: For the QCL Type-D of L3-RSSI measurement, support Alt 1: gNB configures the beam when configures the L3-RSSI measurement. |

Views on L3-RSSI (Scheme 4)

| Company | Key Proposals/Observations/Positions |
| --- | --- |
| Huawei HiSilicon | Proposal 24: For L3-RSSI enhancements, consider the following: - Introduce the 120 kHz SCS to reference SCS/CP field and the corresponding value of 140 symbols in the measurement duration field. - For the QCL Type-D of L3-RSSI measurement beam, support Alt 1 (gNB configures the beam when configuring the L3-RSSI measurement) as a more flexible alternative |
| FUTUREWEI | Proposal 9: For the QCL Type-D of L3-RSSI measurement, gNB configures the beam when it configures the L3-RSSI measurement |
| ZTE Sanechips | Proposal 19: To extend RSSI and Channel occupancy measurement in above 52.6GHz spectrum, the reference SCS/CP field (ref-SCS-CP-r1x) in RMTC-Config needs to be extend to support 120kHz, 480kHz and 960kHz. Proposal 20: Reuse current measurement duration values for extending reference SCS in FR2-2. Proposal 21: For 480kHz and 960kHz SCS, more measurement symbols may need to be considered to more accurately reflect the current channel occupancy situation. Proposal 22: For QCL assumption of L3-RSSI measurement, the UE can assume the configured RSSI measurement resources are QCL-ed with Type-D to one of the latest received PDSCH and the latest monitored CORESET. |
| Nokia Nokia Shanghai Bell | Proposal 23: Add 120, 480 and 960 kHz as SCS options to ref-SCS-CP-r16. Proposal 24: Use the Rel-16 values for measurement duration (measDurationSymbols-r16). Proposal 25: Channel bandwidths defined by RAN4 are used as measurement bandwidths. Proposal 26: For the QCL Type-D of L3-RSSI measurement, gNB configures the beam when it configures the L3-RSSI measurement (Alt 2) |
| CATT | Proposal 3: The value of new SCS, i.e. 120 kHz, 480 kHz and 960 kHz should be added to the candidate values of the reference SCS/CP field (ref-SCS-CP-r16) in RMTC-Config. Proposal 4: Considering the transmitter transient period for the BS, for the duration of L3-RSSI measurement that are configured by measurement duration field (measDurationSymbols-r16) in RMTC-Config, the following two options can be further studied: - Option 1: Use the gNB implementation to avoid configuring the L3-RSSI measurement on the symbols of transmitter transient time for BS. - Option 2: Use UE implementation to exclude the symbols of transmitter transient time for BS from the duration of L3-RSSI measurement. |
| Sony | Proposal 5: For the QCL Type-D of L3-RSSI measurement, at least Alt 1 (gNB configures the beam when configures the L3-RSSI measurement) should be supported. Proposal 6: In addition to extension of L3-RSSI, L1-RSSI based receiver assistance should be introduced in Rel-17 |
| Ericsson | Proposal 15 Enhance the RSSI and CO measurement configuration (RMTC-Config) in Rel-16 to support NR unlicensed operation in FR2-2 in Rel-17. The enhancement includes: · Extending the current ref-SCS-CP to include SCS 120, 480 and 960 kHz · Introducing an indication of channel bandwidth for RSSI measurement. The enumeration of channel bandwidths should ignali the maximum and the minimum channel bandwidth and the intermediate channel bandwidths defined by RAN4. The ignaling details of the RRC configuration for RSSI and CO measurement should be decided by RAN2 Proposal 16 For RSSI and CO measurement in FR2-2, UE can assume the configured RSSI measurement resources are QCL-ed with Type-D to either the latest received PDSCH or the latest monitored CORESET. |
| Intel | Proposal 19: ref-SCS-CP-r16 is extended to include all the supported SCS for FR2-2 (i.e., 120, 480 and 960 KHz). Proposal 20: measDurationSymbols-r16 is extended to include at least sym2, sym4 and symb8. Proposal 21: For the QCL Type-D of L3-RSSI measurement use the QCL type-D of the latest received PDSCH and the latest monitored CORESET. |
| Lenovo Motorola Mobility | Proposal 18: For NR operation in unlicensed bands between 52.6 GHz and 71 GHz, for receiver to provide assistance, channel sensing and reporting need to be performed and following enhancements to legacy RSSI measurements should be supported: - for long term sensing to measure interference statistics from WiFi systems or other NR operators, a new category of ZP CSI-RS should be supported where the UE is not expected to receive any channel/signal (including NZP CSI-RS for interference measurement) and only measure potential interference from WiFi nodes or other NR operators and report back corresponding measurements. |
| Samsung | Proposal 8: For RX-assistant LBT, support:  Scheme 2 with DCI for triggering and UCI for reporting the assistant information;  Scheme 4 with supporting new SCS and measurement bandwidth for 60 GHz unlicensed band. Proposal 9: Support RSSI measurement outside the active BWP and in non-serving cell. |
| InterDigital Inc. | Proposal 3: Support Alt. 1: the gNB configured the beam when it configured the L3-RSSI measurement. |
| LG Electronics | Proposal #5: Extending Rel-16 L3-RSSI measurement can be supported by introducing RRC configuration for reference SCS and the reference SCS can be defined only for 120 kHz and the measurement durations can be maintained 1/14/28/42/70 symbols. Proposal #6: For the QCL Type-D of L3-RSSI measurement, adopt Alt 1 (gNB can configure the beam when configures the L3-RSSI measurement). |
| NTT DOCOMO INC. |  For Scheme 4:  Support all the SCSs (i.e., 120, 480 and 960 kHz)  Bandwidth configuration should follow the LBT bandwidth, i.e., CBW or BWP |
| Qualcomm Incorporated | Proposal 17: For QCL Type-D of L3-RSSI measurement, support both Alt-1 and Alt-3 behavior, i.e.,  The gNB may configure the beam when configuring L3-RSSI measurement  Otherwise the UE uses the QCL type-D of the latest received PDSCH and the latest monitored CORESET |
| MediaTek Inc. | Proposal 3: down-scale rmtc-Periodicity-r16 according to the supported numerology in RMTC-Config-r16. Proposal 4: For QCL type-D of L3-RSSI measurement, support Alt 2. |

Views on L1-RSSI (Scheme 1)

| Company | Key Proposals/Observations/Positions |
| --- | --- |
| FUTUREWEI | Proposal 10: Support L1-RSSI for FR2-2 unlicensed operation: L1-RSSI based receiver assistance is introduced with the following design components Resource used for RSSI measurement Alt 1: RSSI measurement is based on the time/frequency resources configured for ZP-CSI-RS FFS: any enhancement needed for ZP-CSI-RS for this purpose (e.g., ZP-CSI-RS over all Res in BWP over one or more symbols). Alt 2: Energy measurement on operating BW over indicated or specified number of symbols or time interval L1-RSSI is reported in an AP-CSI report L1-RSSI trigger in UL grant with existing AP-CSI triggering mechanism Timeline for L1-RSSI reporting is equal to AP-CSI reporting of L1-RSRP Reuse the same mechanism for L1-RSRP beam determination for L1-RSSI On the content of L1-RSSI report, support both of the following alternatives Alt 1. L1-RSSI provides the (quantized) value of RSSI measurement Alt 2. L1-RSSI provides the comparison outcome with a preconfigured Energy Detection threshold |
| Fujitsu | Proposal 1: For Scheme 1 for receiver to provide assistance in channel access, to reduce latency and signaling overhead, support triggering AP-CSI report directly by the DCI with DL grant. |
| Nokia Nokia Shanghai Bell | Proposal 22: For L1-RSSI use energy measurement on operating BW over indicated or specified number of symbols or time interval as resource used for RSSI measurement (Alt 2) |
| CATT | Proposal 5: For receiver assistance based on L1-RSSI measurement, Alt 2 (energy measurement on operating BW over specified number of symbols) is preferred. Proposal 6: For receiver assistance based on L1-RSSI measurement, L1-RSSI can be triggered by DL grant. Proposal 7: For receiver assistance based on L1-RSSI measurement, the L1-RSSI report can be 1 bit which is the outcome of the value of RSSI measurement and Energy Detection threshold. Proposal 8: For receiver assistance based on L1-RSSI measurement, it is recommended to add the QCL source information for the L1-RSSI measurement. |
| Ericsson | Proposal 17 If RAN1 decides to support Scheme 1, the following enhancements on the current CSI reporting can be considered: Proposal 18 Discuss Scheme 2-2 with explicit feedback in the context of Scheme 1. |
| Intel | Proposal 22: For receiver-assisted LBT procedure both scheme 1 and 2 could be supported, where both scheme 1 and 2 could be used up to UE’s capability. |
| InterDigital Inc. | Proposal 4: Support Scheme 1 L1-RSSI reporting, where the UE is configured with periodic resources on which to measure L1-RSSI and can be aperiodically triggered to report L1-RSSI. Proposal 5: L1-RSSI includes one or more values associated to one or more BWs or beams. Proposal 6: L1-RSSI includes a comparison outcome with a preconfigured energy detection threshold (Alt 2). |
| Apple | Proposal 9: L1-RSSI measurement resource is configured using Alt 2 for time domain measurement. Proposal 10: The UE measures L1-RSSI using the Rx beam associated with the active TCI state of the triggering PDCCH or the Rx beam based on the default PDSCH beam. L1-RSSI can use L3-RSSI report range with the same mapping table defined in 38.133. |
| LG Electronics | Proposal #3: Rx assistance Scheme 1 is not needed because L1-RSSI reporting timeline cannot be tighter than AP-CSI reporting timeline, according to the agreement made in RAN1#106 that for 480 kHz and/or 960 kHz SCS, only value(s) for CSI computation delay requirement 2 are to be defined. |
| NTT DOCOMO INC. |  For Scheme 1, whether to tighten the timeline for L1-RSSI reporting should be concluded at first.  If no tightening is introduced compared to AP-CSI reporting, Scheme 1 itself would not be beneficial |
| Qualcomm Incorporated | Proposal 9: Among Rx-Assistance schemes, prioritize and adopt L1-RSSI enhancements to AP-CSI framework. Proposal 10: L1-RSSI enhancements to AP-CSI framework should be considered independently of Rel 17 IIOT/URLLC AP-CSI enhancements. Proposal 11: Consider the use of RSSI compared to a configurable threshold as part of the L1-RSSI report. Proposal 12: Consider use of UL grant DCI for trigger of Beam Specific L1-RSSI measurement and reporting for enhanced AP-CSI in PUSCH. Proposal 13: L1-RSSI trigger should also be carried in DL grant. Consider use of PUCCH for sending Beam Specific L1-RSSI measurement and ignaling for enhanced AP-CSI. Proposal 14: Use Rel. 16 AP-CSI timelines as baseline for enhanced AP-CSI reporting with L1-RSSI and study further possible tightening of the timelines. Use worst case UE capability for BeamReportTiming for 120KHz SCS, namely 56 OFDM symbols, as a guideline for setting the minimum requirement for L1-RSSI reporting timeline. Proposal 15: Beam Specific L1-RSSI measurement and reporting should be supported. Proposal 16: Consider the design of timeline, triggering and beam indication mechanisms of L1-RSSI to be analogous to CSI-RS based L1-RSRP ignaling in AP-CSI. |
| TCL Communications | Proposal 3: For receiver assisted LBT, scheme 1 is in the same time scale with an ordinary LBT. It should be supported. |

### First round discussion

The following proposals have been stable in email discussions from RAN1-106bis-e . There original version is captured in Section 3 for reference.

Proposed conclusion 2.6.1-1: (closed)

For scheme 2-1 and 2-2 in earlier agreement, there is no consensus to introduce explicitly in the spec that

- In Scheme 2-1, the gNB should not perform DL transmission if PUCCH/SRS is not detected

- In Scheme 2-2, the gNB should not perform DL transmission if PUSCH is not detected

It is to the interest of gNB that the DL transmission is not performed given the CCA/eCCA fails at UE side, thus the good practice for gNB is not to perform the DL transmission. But this is left to gNB implementation

Please provide your view in case your have objections

| Company | View |
| --- | --- |
| Nokia, NSB | We support the conclusion |
| Ericsson | We support the conclusion |
| Intel | We are not OK with the conclusion, and we still believe that this essential part should be captured in the specification, so that a well-defined receiver assistance mechanism would be defined. Moderator: Understand you would like to introduce the proposed behavior. However, given this is the last meeting, and given the number of objecting companies, and also given the fact that the gNB can already enforce the mechanism by implementation. I don’t think we can agree on supporting the mechanism. Then with or without this conclusion, this feature will not be introduced. A conclusion is just for a peace of mind so we can stop the discussion and focus on other issues with some chance. |
| Xiaomi | Support the conclusion |
| LG Electronics | We support the conclusion |
| OPPO | We do not support the proposed conclusion. The handshake mechanism was already implemented in commercial practice and proved to have a good efficiency to address hidden node issue. Therefore, it is justified to introduce in the spec the DL transmission restriction that the gNB should not perform DL transmission if PUCCH/SRS/PUSCH is not detected. Moderator: Same reply to Intel |
| vivo | We can live with this conclusion. However, we think there should be an Rx-assisted mechanism in specification where gNB can refer to when performing DL transmissions. Leave this to implementation has no guarantee that all gNBs will follow which in turn will degrade the performance of the network. In summary, we think DL transmission can be decoupled from the receiving of the assistant information, but there must be a scheme when the DL transmission is coupled with the assistant information, e.g, PUCCH/SRS only triggered by DCI. Moderator: Same reply to Intel |
| Mediatek | We still think the hidden node problem cannot be properly addressed if the consequent DL transmission is left for implementation and therefore do not support the proposed conclusion. |
| Transsion | We support this conclusion. |
| Sony | We support the conclusion |
| ZTE, Sanechips | Considering the limited time, we agree with the conclusion. |
| CATT | We support the conclusion |
| InterDigital | We support the conclusion |
| Huawei, HiSilicon | We do not support the conclusion. If the PUCCH/SRS is triggered by a single DCI in Scheme 2-1, the LBT at UE side reflects the actual hidden-node interference during the potential DL reception, and for which case, the benefit of this conditional transmission of the DL has been shown by SLS evaluations in the SI phase in which Rx-assistance was identified as a beneficial channel access mechanism to combat the hidden node issue in Rel-17; neither Scheme 1 nor Scheme 4 led the group to such conclusions Also, in terms of combating the hidden node problem, the conditional occupancy of the channel based on the RTS/CTS-like handshake was proven to be practically efficient by 802.11 technologies. Conclusion suggests that “It is to the interest of gNB that the DL transmission is not performed given the CCA/eCCA fails at UE side, thus the good practice for gNB is not to perform the DL transmission. But this is left to gNB implementation”. However, there is no such RTS/CTS operating mode in which the subsequent transmission may or may not occur if CTS is not received based on the AP/non-AP STA implementation. |

Proposed conclusion 2.6.1-2: (closed)

For scheme 2-2 in earlier agreement, if we don’t enforce the behavior that the gNB should not transmit if the PUSCH is not detected, the scheme has no spec impact and can be left for implementation

Please provide your view in case your have objections

| Company | View |
| --- | --- |
| Nokia, NSB | We support the conclusion |
| Futurewei | We support the conclusion |
| Ericsson | We support the conclusion |
| Intel | If the conclusion 2.6.1-1 is supported, that this conclusion would be a direct consequence of it. |
| Xiaomi | Support the conclusion |
| LG Electronics | We support the conclusion |
| OPPO | We do not support the proposed conclusion. When the if condition is removed, it is no longer scheme 2-2. Thus, what we can conclude is this NEW scheme can be left for implementation instead of scheme 2-2 can be left for implementation. Moderator: Let’s not debate if this is a new scheme or not. |
| Vivo | Again, we can live with this conclusion if that’s the only possible outcome of the whole Rx-A discussion. However, we have strong concern on the direction RAN1 is going. The whole group spent a lot of efforts trying to specifying Rx-A LBT. Then in the end, we specify nothing and leave it for implementation. As we commented toward proposal 2.6.1-1, specifying some behaviour even as optional feature and hence not mandating gNB behaviour is still helpful for some gNBs which want to follow rather than leave it totally open. |
| Mediatek | Same comment as Proposed conclusion 2.6.1-1 |
| Transsion | We support this conclusion. |
| Sony | We support the conclusion |
| ZTE, Sanechips | Considering the limited time, we agree with the conclusion. |
| CATT | We support the conclusion. |
| InterDigital | We support the conclusion |
| Huawei, HiSilicon | We can support the conclusion |

Proposed conclusion 2.6.1-3: (closed)

For Scheme 2-1 in earlier agreement, there is no consensus to support the same DCI triggers the PUCCH/SRS transmission also schedules the DL transmission after the PUCCH/SRS transmission

Please provide your view in case your have objections

| Company | View |
| --- | --- |
| Nokia, NSB | We support the conclusion |
| Ericsson | We support the conclusion |
| Intel | We support the conclusion |
| Xiaomi | Support the conclusion |
| LG Electronics | Just to make sure, according to current specification, is it impossible to trigger SRS before DL transmission? Moderator: I am not so sure but I think it is supported. However, the important part of the scheme is, the DCI will trigger SRS and the gNB will detect the SRS is transmitted or not, and then condition on that detection to transmit the PDSCH portion of the grant. This is the difficult part to agree on. |
| Vivo | We support the conclusion |
| Transsion | We support the conclusion |
| Sony | We support the conclusion |
| ZTE, Sanechips | Considering the limited time, we agree with the conclusion. |
| LG Electronics2 | With the Moderator’s response, we are fine with the conclusion. |
| CATT | We support the conclusion |
| InterDigital | We support the conclusion |
| Huawei, HiSilicon | We do not think this conclusion is needed. This is no different from many other issues with split views/ no consensus, eg, the support of Scheme 1 or even its components. |

Following set of proposals go into further detail on the FFS items for extending Rel 16 L3-RSSI to unlicensed operation in FR2-2.

Proposal: 2.6.1-4: (closed)

For the QCL Type-D of L3-RSSI measurement for unlicensed operation in FR2-2., down-select one or both of the following alternatives

- Alt 1: gNB configures the beam when configures the L3-RSSI measurement

  - Support: Nokia, FW, Lenovo, Xiaomi, LGE, Transsion, DCM, Samsung, IDCC, HW

- Alt 2: Use the QCL type-D of the latest received PDSCH and the latest monitored CORESET

  - Support: Ericsson, Intel, CATT

- Support both Alt 1 and Alt 2. If explicit beam/TCI state is configured, use Alt 1. Otherwise use Alt 2.

  - Support: Apple, Qualcomm, vivo, Sony, ZTE

Proposal: 2.6.1-4a

For the QCL Type-D of L3-RSSI measurement for unlicensed operation in FR2-2., down-select one or both of the following alternatives

- Alt 1: gNB configures the beam when configures the L3-RSSI measurement by introducing TCI-State IE in RMTC-Config

  - Support: Nokia, FW, Lenovo, Xiaomi, LGE, Transsion, DCM, Samsung, IDCC, HW, TCL

- Alt 2: Use the QCL type-D of the latest received PDSCH and the latest monitored CORESET

  - Support: Ericsson, Intel, CATT

- Support both Alt 1 and Alt 2. If explicit beam/TCI state is configured, use Alt 1. Otherwise use Alt 2.

  - Support: Apple, Qualcomm, vivo, Sony, ZTE

Please provide your view:

| Company | View |
| --- | --- |
| Nokia, NSB | In our view Alt 1 is sufficient. The main use case for L3 RSSI is channel selection for operation, and in such case there may be no scheduling on the measured carrier. |
| Futurewei | We prefer Alt 1 |
| Ericsson | We support Alt2. There is already precedence for Alt2 as it is similar to CLI measurement in FR2, where the UE can assume the configured CLI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH or the latest monitored CORESET. Furthermore, for Alt1 it is unclear how to introduce beam configuration in the existing RMTC configuration framework. It is easier and simpler to use the existing method defined for CLI. |
| Lenovo, Motorola Mobility | Our preference is Alt where gNB explicitly configures the QCL assumption for the measurement. Additionally, the measurement can also be done on the QCL assumption of the latest monitored CORESET. |
| Intel | We support Alt2, and we share same view as Ericsson. |
| Apple | Support both Alt 1 and Alt 2. gNB can configure the beam used for L3-RSSI, particularly gNB can specify omni/quasi-omni sensing for L3-RSSI is L3-RSSI is used for channel/BWP selection etc. When gNB does not configure, or configure Alt 2 without explicit indication of TCI state sensing, Alt 2 can be used. |
| Xiaomi | We support Alt1. Alt 1 is more flexible. Moderator: I think even though Alt 1 has more flexible configuration control, the benefit of Alt 2 is the L1-RSSI measuring beam can “follow” the serving beam, instead of always need an explicit signaling (RRC or MAC-CE) to change beams. |
| LG Electronics | We support Alt 1. The different between RSSI measurement in NR-U and CLI measurement is whether inter-frequency measurement is allowed or not. From our understanding, CLI measurement is performed only within active DL BWP while RSSI measurement in NR-U can be performed via inter-frequency. If this is the case, Alt 2 doesn’t seem to work for inter-frequency measurement case of RSSI measurement. |
| Vivo | We support both Alt 1 and Alt 2. |
| Transsion | We support Alt 1 which is sufficient. |
| Sony | We support both Alt 1 and Alt 2. |
| ZTE, Sanechips | In principle, we prefer alt 2 to reuse the design of CLI-RSSI, but for the sake of progress, we can live with supporting both Alt 1 and Alt 2. |
| DOCOMO | Prefer Alt 1 by adding additional RRC parameter. |
| CATT | Alt 2 is our preferred. |
| Ericsson 2 | It is not clear to us how TCI state can be configured for this purpose in Alt 1. We request the proponents of Alt1 to come up with concrete proposals regarding how this can be specified and embodied. In our opinion, TCI update procedure (by DCI, MAC-CE or RRC re-configuration), is very costly and only adds unnecessary complexity. Moderator: I think the design is simply adding an TCI state IE in RMTC-Config. Other companies, please comment if you have other suggestions |
| Samsung | We support Alt 1, and a TCI/QCL source can be provided in the configuration for RSSI measurement. If such information is not provided, it should be up to UE’s implementation on how to choose the beam to perform the RSSI measurement. Moderator: Is the “up to UE implementation” part Alt 2, or it is really up to UE implementation (so gNB does not know which beam UE is measuring)? |
| InterDigital | We prefer Alt 1. Tying the RSSI report to the most recent received PDSCH limits the scheduling flexibility especially if the time between the last PDSCH transmission and a planned subsequent PDSCH transmission is large. |
| Huawei, HiSilicon | We support Alt 1 and we have same understanding as Nokia. |
| Ericsson 3 | One needs to think about what can be used as the source RS for L3-RSSI measurement in the RMTC. It probably needs to refer to the QCL RS used for PDCCH monitoring or PDSCH reception. So, it is not clear to use the motivation to include the TCI state RMTC-config. For beam tracking, gNB can indicate UE update of TCI state for PDCCH by MAC-CE or for PDSCH in DCI, how should the TCI state in RMTC-config be updated? Response to Moderator: We do not think that the design is simply by adding a TCI State IE in RMTC-Config. How does the TCI state in RMTC-config be updated then? At this stage of the WI, we cannot support methods like Alt 1 that is not clear. Simplest method is to use the serving beam, Alt 2 as you mentioned. Moderator: The basic proposal is to directly configure TCI state. If we need a dynamic mechanism to update it is not discussed yet. I don’t see any proposal for that and it is not in the scope of this discussion. To make this clear, I added the TCI state setting in Alt 1 for 2.6.1-4a Response: Thank you for the modification. We still think it does not solve the issue. We configure the TCI State in an RRC message and reference them here. So, what happens when the TCI state needs to be changed? The update mechanism must be brought into the scope of discussions if we are going to introduce it. At this stage of the WI, we cannot agree to any mechanism that has FFSs and are not complete. In this case, without knowing how the update mechanism would be done. |
| TCL | We support Alt.1 That is more flexbile. |

Proposal 2.6.1-5:

Regarding reference SCS/CP field (ref-SCS-CP-r16) down-select from the two alternatives

- Alt 1: Extend the reference SCS/CP field (ref-SCS-CP-r16) to include 120KHz, 480 KHz and 960KHz subcarrier spacing.

  - Support: Nokia, Ericsson, Lenovo, Intel, Apple, Xiaomi, vivo, Transsion, Sony, ZTE, DCM, CATT, Samsung, IDCC, TCL

- Alt 2:  Extend the reference SCS/CP field (ref-SCS-CP-r16) to include 120KHz only

  - Support: LGE, HW

Please provide your view

| Company | View |
| --- | --- |
| Nokia, NSB | We have a (slight) preference for Alt 1 |
| Ericsson | We support Alt 1. |
| Lenovo, Motorola Mobility | Support Alt 1 |
| Intel | We support Alt 1. |
| Apple | Alt 1 |
| Xiaomi | Support Alt1 |
| LG Electronics | We support Alt 2. Since we introduced scaled UE processing timelines for 480/960 kHz SCS, 120 kHz SCS for reference SCS and 1/14/28/42/70 symbols for measurement duration are enough. If reference SCS is set to 120 kHz and duration is set to 1 for 480 kHz SCS BWP, RSSI measurement can be performed for 4 symbols based on 480 kHz SCS. |
| Vivo | We support Alt1 |
| Transsion | We support Alt 1. |
| Sony | We support Alt 1 |
| ZTE, Sanechips | Support Alt 1. |
| DOCOMO | Support Alt 1. To LGE, why the timeline scaling approach should be considered here? |
| CATT | We support Alt 1 |
| Samsung | We prefer Alt 1 to be consistent with Rel-16 NR-U. |
| InterDigital | We support Alt 1 |
| Huawei, HiSilicon | We think Alt 2 is sufficient as only one reference SCS needs to be defined. |
| Huawei, Hisilicon 2 | We would like to further explain our views about ref-SCS-CP-r16. If the view is that a UE in FR2-2 can only be configured with 120KHz, 480 KHz and 960KHz ref-SCS-CP-r16 (and not 15 KHz, 30 KHz, or 60 KHz) as in FR1, then we cannot accept any new ref-SCS-CP-r16 without proportionally increasing the maximum measDurationSymbols-r16 from FR1 value of 70. That is: if 120KHz is supported for ref-SCS-CP-r16, support measDurationSymbols-r16 = 140. If 480KHz is supported for ref-SCS-CP-r16, support measDurationSymbols-r16 = 560. If 960KHz is supported for ref-SCS-CP-r16, support measDurationSymbols-r16 = 1120. The reason is increasing the ref-SCS-CP-r16 without increasing corresponding maximum value of measDurationSymbols-r16 results in the reduction of RSSI measurement occasion duration and its accuracy. Moderator: This discussion is only for the SCS, not for the duration (next discussions). Here the proposal is to extend the field. In other words, I believe 15/30/60 as reference SCS can still be configured in FR2-2. I get your point on measurement accuracy. That can be further discussed when we discuss duration |
| TCL | We support Alt.1. |

Proposal 2.6.1-6: (closed)

On measDurationSymbols-r16 with ref-SCS-CP-r16=120KHz

- Alt 1: reuse measDurationSymbols-r16 as is, i.e. {1,14,28,42,70}

  - Support: Nokia, Ericsson, Lenovo, Xiaomi, LGE, Transsion, Intel (also file), Sony, ZTE, DCM, CATT, Samsung,

- Alt 2: extend measDurationSymbols-r16 to {1,2,4,8,14,28,42,70}

  - Support: Nokia (open), Ericsson (open), Intel (prefer)

- Alt 3: extend measDurationSymbols-r16 to {1,14,28,42,70,140}

  - Support: HW

FFS ref-SCS-CP-r16=480KHz or 960KHz case if supported

Please provide your view:

| Company | View |
| --- | --- |
| Nokia, NSB | The existing values are the baseline, but we are open to consider extension of the range; however, extension to values larger than 70 is preferred . |
| Ericsson | We support Alt 1 as the baseline and are open to discuss further extensions if needed. |
| Lenovo, Motorola Mobility | Support Alt 1 to reuse |
| Intel | We have slight preference for Alt.2, since for higher SCS a single OFDM symbol may not be sufficient for the shortest measurement duration since the accuracy of the RSSI measurement would be highly degraded, and the network may need to only rely on configurations with much longer duration. |
| Xiaomi | Support Alt1 |
| LG Electronics | Support Alt 1. |
| Transsion | We support Alt 1. |
| Sony | We support Alt 1 |
| ZTE, Sanechips | Support Alt 1. |
| DOCOMO | Support Alt 1. |
| CATT | We support Alt 1 as the baseline. |
| Samsung | We support Alt 1. Didn’t see a strong need on optimization. |
| Huawei, HiSilicon | In our view, the baseline values should be reused. However, if at least 120kHz is supported as a ref SCS, the additional value of 140 symbols would need to added to the baseline values to ensure that the same maximum measurement duration is still achievable. Therefore, we propose to support Alt3 below - Alt 3: extend measDurationSymbols-r16 to {1,14,28,42,70, 140} We don’t see the need to add 2,4, 8 symbols. |

Given many companies are open for discussion of additional values and HW’s comments on maintaining the maximum measurement accuracy seems to be reasonable, the Moderator is proposing the following.

Proposal 2.6.1-6a:

- On measDurationSymbols-r16 with ref-SCS-CP-r16=120KHz, extend measDurationSymbols-r16 to {1,14,28,42,70,140}

- On measDurationSymbols-r16 with ref-SCS-CP-r16=480KHz (if supported), extend measDurationSymbols-r16 to {1,14,28,42,70,140, 560}

- On measDurationSymbols-r16 with ref-SCS-CP-r16=960KHz (if supported), extend measDurationSymbols-r16 to {1,14,28,42,70,140, 560,1120}

Moderator note: Given likely the IE has 3 reserved values, adding up to 3 more entries only might be easier.

Please provide your view:

| Company | View |
| --- | --- |
| TCL | We are OK with the proposal. |
| LG Electronics | We support only the first bullet, which is sufficient. |
| Nokia, NSB | We support the proposal |
| ZTE, Sanechips | We are find with the proposal |
| Ericsson | We support the proposal in principle. |

Proposal 2.6.1-7:

Introduce new parameter in RMTC-Config to indicate measurement bandwidth.

- FFS: channel bandwidths should include the maximum and the minimum channel bandwidth and the intermediate channel bandwidths defined by RAN4.

- Support: Nokia, Ericsson, Lenovo, Intel, Apple, Transsion, DCM, CATT, Sony, ZTE (starting PRB and # of PRBs)

Alternative proposal: Always use sensing bandwidth:

- Support: Xiaomi, LGE

Please provide your view :

| Company | View |
| --- | --- |
| Nokia, NSB | We support the proposal |
| Ericsson | We support the proposal. |
| Lenovo, Motorola Mobility | Fine to support |
| Intel | We support the proposal |
| Apple | OK to support. |
| Xiaomi | We are not sure about the intention of introducing a new parameter. From our understanding, the measurement bandwidth should be the same as the sensing bandwidth, that is the whole channel bandwidth. |
| LG Electronics | We would like to know the motivation of introducing new parameter. If not configured, measurement bandwidth should be confined within carrier bandwidth (but the exact location would be up to UE implementation) by default. Can this high layer parameter configure more than one measurement bandwidths for a carrier? |
| Transsion | We support the proposal. |
| Sony | We support the proposal |
| ZTE, Sanechips | Regarding measurement bandwidth, we think that similar method specified in RSSI-ResourceConfigCLI can be considered, such as measurement bandwidth is determined by the starting PRB index of the measurement bandwidth and size of the measurement bandwidth. |
| DOCOMO | Support |
| CATT | We support the proposal. |
| Samsung | We believe this proposal could be revisited after clarifying the LBT bandwidth. |
| InterDigital | We support the proposal |
| Huawei, HiSilicon | We support the proposal |
| Futurewei | Support |
| TCL | We support the proposal. |

The following Proposal and the current summary of support are related to Scheme 1 Rx-Assistance, i.e. Introducing L1-RSSI. Consider this is the last meeting and we don’t have time to discuss further tighten the timeline, the proposal is updated to align the timeline with L1-RSRP (faster than CSI timeline)

Discussion: 2.6.1-8: (closed)

L1-RSSI based receiver assistance is introduced with the following design components

- Resource used for RSSI measurement

  - Alt 1: RSSI measurement is based on the time/frequency resources configured for ZP-CSI-RS

    - FFS: any enhancement needed for ZP-CSI-RS for this purpose (e.g., ZP-CSI-RS over all Res in BWP over one or more symbols).

    - Qualcomm, Ericsson, Futurewei (1st choice), Fujitsu, DCM,

  - Alt 2: Energy measurement on operating BW over indicated or specified number of symbols or time interval

    - Intel, Lenovo, Apple, InterDigital, Futurewei (2nd choice), Nokia, CATT, Sony,Charter

- L1-RSSI is reported in an AP-CSI report

- L1-RSSI trigger in UL grant

  - FFS if L1-RSSI trigger can also be carried in DL grant

- Timeline for L1-RSSI reporting is at least equal to AP-CSI reporting of L1-RSRP

- Reuse the same mechanism for L1-RSRP beam determination for L1-RSSI

- On the content of L1-RSSI report, down-select one or more of the following alternatives

  - Alt 1. L1-RSSI provides the (quantized) value of RSSI measurement

    - Qualcomm, Ericsson, Apple, Futurewei, DCM, Nokia. Sony, Charter

  - Alt 2. L1-RSSI provides the comparison outcome with a preconfigured Energy Detection threshold

    - Qualcomm, Intel, Lenovo, Ericsson, InterDigital, Futurewei, Fujitsu, DCM, CATT

- Support: Intel, Lenovo, Qualcomm, Ericsson, Apple, InterDigital, Futurewei, Fujitsu, TCL, DCM, Nokia, CATT, Sony, Charter

- Not support: ZTE, vivo, LGE, Samsung, Huawei/HiSilicon, OPPO, Transsion, MTK,

Moderator: Given the discussion above, it is quite clear to me we don’t have consensus to support L1-RSSI, and there is  not time to discuss further. I put together the proposed conclusion in 2.6.1-9 to capture that.

Please provide your view if not captured.

| Company | View |
| --- | --- |
| Nokia, NSB | We support the proposal and our views have been correctly captured. |
| Futurewei | Our views are correctly captured and we support this proposal. |
| Ericsson | We support the proposal and our views are correctly captured. |
| Intel | Our view is correctly captured expect for the timeline. Our view is that this scheme could only be supported if the L1-RSSI reporting timeline is tightened compared to the legacy procedure as per the note in prior agreement, and if this cannot be reduced then we do not see any benefit in introducing such a scheme. In this case, we think that only scheme 2 should be supported, given less specification impact and given that this has been evaluated and benefits are clear compared to this scheme. Moderator: The way I see it, align with L1-RSRP timeline is already satisfy the previous note. If we don’t have this L1-RSSI feature with the L1-RSRP timeline, we can only use normal AP-CSI report to detect the interference level, which is slower. |
| Apple | Support the proposal |
| LG Electronics | To Moderator, could you refer to where L1-RSRP timeline (faster than CSI timeline) is specified? Moderator: This is the Z3 and Z3’ in the spec. We already adopted the timeline for L1-RSRP with same absolute value as 120KHz |
| OPPO | We do not support the proposal and add our position, since there are still many FFS points in Scheme 1 and the gain of Scheme 1 over Scheme 2 is not clear. Also, for reusing the same mechanism for L1-RSRP beam determination for L1-RSSI, our concern is whether the L1-RSSI measurement result can be representative of the experienced interference for data reception. Moderator: At this phase, I don’t think we should assume we have scheme 2. What we reuse from the L1-RSRP is only the timeline. We will measure Res not occupied by serving cell, so I believe it can capture interference level. |
| Vivo | Our comment in v016 to 2.6.1-8 was removed from v017 and v018. Here we copied back again. We have the same understanding as Intel on the L1-RSSI timeline. We also have the same question as LG to Moderator. For several meetings, the whole argument from proponent to support this as Rx-A is that L1-RSSI timeline can be tightened. Now that it’s not possible, what is the motivation/reason to support this when scheme 2 inherently has benefit in terms of timely feedback of assistance information. Furthermore, we think this scheme will have a lot of spec impact, we need a set of relevant parameters or schemes to support L1-RSSI. Therefore, we do not support this scheme. Moderator: I don’t understand the comment that the timeline is not tightened. Please see the reply to Intel above. Without this, we can only AP-CSI mechanism to measure interference under CSI framework. Now the proposal is the use AP-CSI reporting framework with L1-RSRP timeline. |
| Mediatek | We do not support the proposal and we share similar view with VIVO. The benefit of the scheme 1 is not clear for us. If it’s for long-term measurement, we can apply legacy L3-RSSI measurement; if short-term measurement is needed, we fail to understand how the applicability of scheme 1 is more advantageous over the scheme 2 after taking timeline factor into consideration. In Rel-13, L3-RSSI was introduced after it’s identified as a useful scheme, and the relevant agreement is listed as follows: Agreements RSSI: - For the purpose of detecting hidden node in channel selection UE reporting of RSSI measurements to the eNB is considered useful. The details of the RSSI measurement reporting should be discussed in stage-3. 5 The eNB indicates which carriers(s) the UE should report RSSI for.. We do not want to introduce a mechanism when its benefit and usefulness are not clear. Besides, we already have a Note in the agreement of the 106-e meeting, which is listed as follows - Note: If L1-RSSI reporting timeline cannot be tighter than AP-CSI reporting timeline, this scheme is not needed Question to the moderator: In our understanding, the Z3 and Z3’ in the spec is the CSI processing timeline where L1-RSRP is already part of AP-CSI report, why is aligning with L1-RSRP timeline is already satisfying the previous note? |
| Transsion | We do not support the proposal, and we share the same view as vivo. |
| Sony | We support the proposal and our views are correctly captured. |
| ZTE, Sanechips | Considering the limited time and there is no a basic consensus in previous several meeting, we tend to deal with scheme1(L1-RSSI) in the same way as scheme 2, that is , there is no consensus to support L1-RSSI based receiver assistance in FR2-2. |
| DOCOMO | Our view captured correctly. |
| CATT | We support the proposal and our views are correctly captured. |
| InterDigital | We support the proposal and our views are correctly captured |
| Huawei, HiSilicon | We do not support the proposal. While the specification impact of Scheme 1 (L1-RSSI reporting in an AP-CSI) is comparable to Scheme 2-1, Scheme 1 decouples the measurement from the scheduled DL reception and has the following issues: - Reporting a high level of interference would result in latency and waste of the scheduling opportunity if the hidden node interfering burst has already ended by the time of scheduled DL reception. - In the converse, reporting a low level of interference would result in collision and poor performance especially for cell edge Ues if a hidden node interfering burst starts by the time of scheduled DL reception - The overall dynamic overhead is increased in the cell since two DCIs are required compared to Scheme 2-1 (with same DL assignment scheduling/triggering the UL transmission for CTS/interference level feedback) Performance evaluations of the decoupled Schemes have never been provided/discussed in the SI phase during which Rx-assistance was identified as a beneficial channel access mechanism to combat the hidden node issue in Rel-17 |

Proposed conclusion: 2.6.1-9

There is no consensus to support introducing L1-RSSI mechanism for FR2-2 in Rel.17.

Moderator note: To summarize, for the RX assistance LBT topic in Rel.17 discussion, seems that we can only support some implementation based schemes under scheme 2, other than L3-RSSI.

Please provide your view if you think otherwise:

| Company | View |
| --- | --- |
| Huawei, Hisilicon | We can support the conclusion |
| LG Electronics | Support Proposed conclusion: 2.6.1-9 |
| Apple | Support the proposed conclusion. |
| OPPO | We support the proposed conclusion 2.6.1-9. However, we do not think the hidden-node issue can be addressed by implementation. In our understanding, the implementation based scheme under Scheme 2 is that the gNB schedules UL transmission and indicates the UE to perform LBT, but the DL transmission is independent with the scheduled UL transmission. Do I understand correctly? In this case, if the UL transmission is PUSCH, it requires a PUSCH transmission prior to PDSCH in Rx-assisted LBT. However, what if the UE does not have UL traffic before PDSCH should be discussed. Also, the validity duration of the Rx Assistance information should be specified. On the other hand, if the UL transmission is PUCCH/SRS as in Scheme 2-1, the gNB may not infer the LBT outcome upon detecting the scheduled UL transmission since the PUCCH/SRS can be transmitted with Contention Exempt Short Control Signaling rule as in Proposal 2.11.1-5. |
| Mediatek | We can support the proposed conclusion. |
| ZTE, Sanechips | We support this conclusion. |
| Ericsson | For the sake of progress, we can compromise and support the proposed conclusion. |
| Vivo | We support the proposed conclusion. We also think the hidden node problem is still not solved. A Rx-assisted mechanism should be specified. |
| Intel | We support this proposal. |
| Samsung | We support the conclusion. |
| Convida Wireless | We are ok with the proposal. |
| TCL | We support the conclusion. |
| Transsion | We support the conclusion. |
| Nokia, NSB | We support FL’s assessment. |

## Multi-Beam COT

Agreement: For a COT with MU-MIMO (SDM) transmission, further consider the follow alternatives (down-select or support both) - Alt 1: Single LBT sensing at the start of the COT with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold - Alt 2: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT Agreement: Within a COT with TDM of beams with beam switching, down-select one or more of the following LBT operations - Alt 1: Single LBT sensing with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold - FFS: Details on the definition of “cover” - Alt 2: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT - Alt 3: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT with additional requirement on Cat 2 LBT before beam switch Agreement: - SSB transmission with LBT is supported, at least when the conditions for contention exempt short control signalling based SSB transmission is not met - Note the channel access for SSB with LBT may not be different from a normal COT with multiple beams - FFS: If any difference from a multi-beam COT LBT needs to be introduced Agreement: For a COT with MU-MIMO (SDM) transmission, when independent per-beam LBT sensing at the start of COT is performed for beams used in the COT (Alt 2 in earlier agreement) is considered, the following alternatives are further considered - Alt A: The per-beam LBT for different beams is performed in TDM fashion - Alt A-1: The node completes one eCCA on one beam, and directly move on to the eCCA on the other beam, with no transmission in the middle - Alt A-2: The node completes one eCCA on one beam, start transmission with the beam to occupy the COT, then move on to the eCCA on the other beam - Alt A-3: The node performs eCCA of the different beams simultaneous, round robin between different beams - Alt B: The per-beam LBT for different beams is performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams Agreement: Within a COT with TDM of beams with beam switching, when independent per-beam LBT sensing at the start of COT is performed for beams used in the COT (Alt 2 or Alt 3 in earlier agreement) is considered, the following alternatives are further considered - Alt A: The per-beam LBT for different beams is performed one after another in time domain - Alt A-1: The node completes one eCCA on one beam, and directly move on to the eCCA on the other beam, with no transmission in the middle - Alt A-2: The node completes one eCCA on one beam, start transmission with the beam to occupy the COT, then move on to the eCCA on the other beam - Alt A-3: The node performs eCCA of the different beams simultaneous, round robin between different beams - Alt B: The per-beam LBT for different beams is performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams

| Company | Key Proposals/Observations/Positions |
| --- | --- |
| Huawei HiSilicon | Proposal 14: For initiating a COT with SDM or TDM of different beams using a single LBT beam that “covers” all the subsequent DL transmission beams, gNB selects a spatial sensing filter that minimizes the resulting [3]dB sensing beamwidth which at least contains all beam peak directions of the subsequent DL transmission beams within the COT (Alt-1E). Proposal 15: For initiating a COT with SDM or TDM of different beams, support multiple independent per-beam LBTs, i.e. Alt 2. Proposal 16: When gNB performs multiple independent per-beam LBTs, the spatial domain sensing filter for an LBT beam is the same as the spatial domain filter used for the corresponding transmission beam. Proposal 17: For initiating a COT with SDM or TDM of different beams, support one LBT beam covering all transmission beams (Alt 1) as a fallback mechanism when the one-to-one correspondence between the LBT beams and transmission beams cannot be established. Proposal 18: For initiating a COT with SDM or TDM of different beams, when independent per-beam LBT sensing at the start of COT is performed for beams used in the COT, support performing the per-beam LBTs simultaneously in parallel (Agree to FL Proposals 2.7.1-2 and 2.7.1-4 from RAN1#105-e). - If the node is incapable of sensing simultaneously in different beams, a single LBT beam covering the multiplexed transmission beams should be used. |
| FUTUREWEI | Proposal 8: When independent per-beam LBT sensing at the start of COT is performed for beams used in the COT, an additional requirement on Cat 2 LBT before switching to new beam during the COT should be specified if the time duration from that beam’s LBT sensing exceeds a threshold. |
| Vivo | Proposal 1: The Cat 2 LBT can be used before switching to a new beam in a COT with TDM beams, before response with assistant information at the receiver, and in the Type B multi-channel access scheme. Proposal 8: A node can initiate two (or more) (partially) overlapping COTs in two (or more) different beams. Proposal 9: For a COT with MU-MIMO (SDM) transmission, both Alt 1 and Alt 2 are supported. Proposal 10: For a COT with TDM transmission, both Alt 1 and Alt 3 are supported. Proposal 11: For a COT with MU-MIMO (SDM) transmission, both Alt A-1 and Alt B are supported. Proposal 12: Alt A-1 and Alt-B are supported for the transmission within a COT with TDM of beams with beam switching. |
| ZTE Sanechips | Proposal 13: Considering transmission opportunity and utilization of resource, Alt2 that “multiple per-beam LBT that cover multiple transmission beams used in COT” should be considered for the transmission with multiple beams in spatial domain multiplexing if directional LBT is supported. Proposal 14: Considering LBT overhead and transmission delay, Alt B that“The per-beam LBT for different beams is performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams” should be considered for the transmission with multiple beams in spatial domain multiplexing if Alt 2 is supported. Proposal 15: Considering transmission opportunity and unnecessary interference to other device that is going to transmit transmission, Alt-3 that “Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT with additional requirement on Cat 2 LBT before beam switch” can be considered for the transmission with multiple beams in time domain multiplexing, if directional LBT is supported.  Considering LBT overhead and transmission delay, Alt B that“The per-beam LBT for different beams is performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams” should be considered if Alt-2 or Alt-3 is supported |
| Spreadtrum Communications | Proposal 4: For a COT with MU-MIMO (SDM) transmission, single LBT sensing at the start of the COT with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold should be supported. Proposal 5: For a COT with MU-MIMO (SDM) transmission, independent per-beam LBT sensing at the start of COT is performed for beams used in the COT should be supported, and the per-beam LBT for different beams is performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams. Proposal 6: Within a COT with TDM of beams with beam switching, single LBT sensing at the start of the COT with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold should be supported. Proposal 7: Within a COT with TDM of beams with beam switching, when independent per-beam LBT sensing at the start of COT is performed for beams used in the COT: If the transmitter has the capability to simultaneously sense in different beams, the per-beam LBT for different beams is performed simultaneously in parallel - If the transmitter does not have the capability to simultaneously hanne in different beams, Alt A-1 should be supported. |
| Nokia Nokia Shanghai Bell | Proposal 12: COT initiating LBT with multiple independent per-beam LBT sensing should be hanneleed while completing the design for baseline channel access procedures. Proposal 13: For a COT with MU-MIMO (SDM) or within a COT with TDM of beams with beam switching, support both Alt 1 and Alt 2. Proposal 14: For a COT with MU-MIMO (SDM) transmission, support Alt B. Observation 7: It is important to maintain flexibility of gNB implementation for multi-beam COT. Observation 8: Alt A-1 should be modified as: The node completes one eCCA on one beam, and directly moves on to the eCCA on the other beam, with no transmission in the middle. After completing eCCA on all beams, a further round robin CCA check is carried out in all beams (except the last beam). Observation 9: Alt A-3 should be modified as: The node performs eCCA of the different beams simultaneous, round robin between different beams.  single contention window is shared by beams or each beam has a separate contention window.  the last CCAs shall indicate vacant channel on all beams that are part of the COT Proposal 15: For a COT with TDM transmission, support the modified Alt A-1 and Alt A-3:  Alt A-1: The node completes one eCCA on one beam, and directly moves on to the eCCA on the other beam, with no transmission in the middle. After hannele eCCA on all beams, a further round robin CCA check is carried out in all beams (except the last beam).  Alt A-3: The node performs eCCA of the different beams simultaneous, round robin between different beams. - single contention window is shared by beams or each beam has a separate contention window. O the last CCAs shall indicate vacant channel on all beams that are part of the COT |
| CATT | Proposal 9：Consider supporting both of single LBT sensing with wide beam and independent per-beam LBT sensing for all beams to be used within the COT at the start of the COT. Proposal 10: If supporting Alt A-1 or Alt A-2, the ‘blocking issue’ (failure of previous beam LBT causes subsequent beams unable to perform LBT) should be addressed. Proposal 11: Alt A-3 of which node performs eCCA round robin between different beams should be supported to increase the multi-beam LBT efficiency. |
| Sony | Proposal 3: For a COT with MU-MIMO (SDM) transmission, both Alt 1 (Single LBT sensing at the start of the COT with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold) and Alt 2 (Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT) should be supported. Proposal 4: Within a COT with TDM of beams with beam switching, both Alt 1 (single LBT sensing with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold) and Alt 3 (independent per-beam LBT sensing at the start of COT is performed for beams used in the COT with additional requirement on Cat 2 LBT before beam switch) should be supported. Observation: If per-beam LBT sensing is introduced, per beam COT indication may be needed. |
| Ericsson | Observation 12 All alternatives agreed to be considered for a COT with TDM and SDM of beams, depends solely on how directional LBT for a single beam would be specified. Proposal 12 If any enhancements to better enable multiple beam transmissions within a COT when LBT mode is used can be agreed now, it is to support Alt 1 in principle for TDM and SDM case where a single LBT at the beginning of the COT is performed with the definition of “cover” meaning omni-directional or quasi-omni-directional. Proposal 13 RAN1 needs to decide on whether and how to specify directional LBT for single sensing beam case before further discussing multiple sensing beams. |
| Intel | Proposal 12: Do not support, Cat-2 LBT for multi-beam switching and multi-beam TDM COT. Proposal 13: For a COT with MU-MIMO, both Alt-1 and Alt-2 are supported. As for Alt-2 both Alt-A-2 and Alt-B could be considered. Proposal 14: For a COT with beam switching, both single LBT sensing with wide beam and independent per-beam LBT sensing at the start of the COT are supported. Proposal: 15: An initiating device is able to initiate multiple overlapping COT over different beams. |
| Lenovo Motorola Mobility | Proposal 7: For NR operation in unlicensed bands between 52.6 GHz and 71 GHz with LBT based channel access mechanism, for a COT with MU-MIMO (SDM) transmission, all of the following should be supported: - Single LBT sensing at the start of the COT with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold - Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT Proposal 8: For NR operation in unlicensed bands between 52.6 GHz and 71 GHz with LBT based channel access mechanism, within a COT with TDM of beams with beam switching, all of the following should be supported: - Single LBT sensing with wide beam covering all beams to be used in the COT with appropriate ED threshold, where covering implies that the coverage region of wide beam contains the coverage region of all the beams - Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT - Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT with additional requirement on Cat 2 LBT before beam switch Proposal 9: For NR operation in unlicensed bands between 52.6 GHz and 71 GHz with LBT based channel access mechanism, for a COT with MU-MIMO (SDM) transmission, the per-beam LBT for different beams is performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams Proposal 10: For NR operation in unlicensed bands between 52.6 GHz and 71 GHz with LBT based channel access mechanism, for a COT with TDM transmission, the per-beam LBT for different beams can be supported with both alternatives below:  Alt A: The per-beam LBT for different beams is performed one after another in time domain - Alt A-1: The node completes one eCCA on one beam, and directly move on to the eCCA on the other beam, with no transmission in the middle - Alt A-2: The node completes one eCCA on one beam, start transmission with the beam to occupy the COT, then move on to the eCCA on the other beam - Alt A-3: The node performs eCCA of the different beams simultaneous, round robin between different beams  Alt B: The per-beam LBT for different beams is performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams Proposal 11: For NR operation in unlicensed bands between 52.6 GHz and 71 GHz with LBT based channel access mechanism, when multiple DL/UL transmissions are scheduled on multiple beams in TDM in same COT, then LBT can be performed at the beginning of the transmissions and also in the middle of same COT, if needed, which is depending upon following gaps: - Maximum allowed gap between the first symbol of the following scheduled transmission on a given beam and the last symbol of the transmitted (same) beam - Or if there is no previous transmission on the same beam within a COT, then the maximum allowed gap between the between the first symbol of the following scheduled transmission on a given beam and the time instance when Cat 4 LBT was successful on a beam covering the transmit beam Proposal 16: For NR unlicensed bands between 52.6 GHz and 71 GHz with directional LBT based channel access mechanism, multiple COT sharing indicators and their corresponding association to different beams can be signaled in a group common DCI and the association of COT sharing indicator to transmission is semi-statically signaled |
| NEC | Proposal 9: For a COT with SDM transmission, when independent per-beam LBT sensing at the start of COT is performed and the node does not has the capability to simultaneously sense in different beams, at least the following LBT operations should be supported:  The node performs eCCA of the different beams simultaneous, round robin between different beams. Proposal 10: Within a COT with TDM of beams with beam switching, when independent per-beam LBT sensing at the start of COT is performed for beams used in the COT and the node does not has the capability to simultaneously sense in different beams , the following LBT operations should be supported:  The node completes one eCCA on one beam, start transmission with the beam to occupy the COT, then move on to the eCCA on the other beam.  The node performs eCCA of the different beams simultaneous, round robin between different beams. |
| Panasonic | Proposal 1: Agree on Proposal 2.7.1-1 in Feature Lead Summary [1] i.e. For a COT with MU-MIMO (SDM) transmission, support both Alt 1 and Alt 2 below: Alt 1: Single LBT sensing at the start of the COT with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold Alt 2: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT Proposal 2: Agree on Proposal 2.7.1-2 in Feature Lead Summary [1] i.e. For a COT with MU-MIMO (SDM) transmission if Alt 2 is supported (independent per beam LBT), and if the node has the capability to perform simultaneous sensing in different beams, simultaneous per-beam LBT for different beams is supported. Proposal 4: Agree on Proposal 2.7.1-4 in Feature Lead Summary [1] i.e. Within a COT with TDM of beams with beam switching, if Alt 2 or Alt 3 is supported (independent per beam LBT), and if the node has the capability to perform simultaneous sensing in different beams, simultaneous per-beam LBT for different beams is supported. Proposal 5: Support A-1 and A-3 in the discussion point 2.7.1-5 in Feature Lead Summary [2] of RAN1#105e. It means to support following. For a gNB/UE to initiate a COT with SDM or TDM multiple beams with separate LBT per beam and the gNB/UE does not have the capability to simultaneously sense in different beams, the following alternatives are supported: •Alt A-1: The node completes one eCCA on one beam, and directly move on to the eCCA on the other beam, with no transmission in the middle •Alt A-3: The node performs eCCA of the different beams simultaneous, round robin between different beams |
| Samsung | Proposal 6: Support directional channel sensing in multi-beam operation:  For multi-beam SDM scenario, both Alt 1 and Alt 2 can be supported.  For multi-beam TDM scenario, Alt 1 can be supported as baseline, and selection between Alt 2 and Alt 3 depends on whether sensing is required for switching beams within a COT. Proposal 7: For per-beam LBT for different beams,  Support both Alt A and Alt B, and up to implementation to choose between Alt A and Alt B.  Within Alt A, support Alt A-1 as the baseline. |
| InterDigital Inc. | Proposal 11: A node that has initiated a first COT and wishing to transmit on a new transmission beam not applicable to the first COT, performs LBT on a sensing beam covering at least the new transmission beam and if possible, hannel a new COT and terminates the first COT. Proposal 13: For a COT with MU-MIMO (SDM) transmission, support at least independent per-beam LBT sensing (Alt 2) and support simultaneous round robin eCCA between different beams (Alt A-3). Proposal 14: For a COT with TDM of beams with beam switching, support at least independent per-beam LBT sensing at the start of COT with additional requirement on CAT 2 LBT before beam switch (Alt 3) and support Alt A-2 or A-3. Proposal 15: Support of Alt B for SDM or TDM of beams can be considered for some Ues. |
| Convida Wireless | Proposal 6: For a COT with MU-MIMO (SDM) transmission, support both single LBT sensing at the start of the COT with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold and independent per-beam LBT sensing at the start of COT performed for beams used in the COT. Proposal 7: Within a COT with TDM of beams with beam switching, support both single LBT sensing with wide beam ‘cover’ all beams and independent per-beam LBT sensing at the start of COT performed for beams used in the COT. Further discuss independent per-beam LBT sensing at the start of COT for beams used in the COT with additional requirement on Cat 2 LBT before beam switch. Proposal 8: For a COT with MU-MIMO (SDM) transmission, consider both per-beam LBT for different beams performed in TDM fashion and per-beam LBT for different beams performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams. Proposal 9: Within a COT with TDM of beams with beam switching, consider both per-beam LBT for different beams performed in TDM fashion and per-beam LBT for different beams performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams. |
| LG Electronics | Proposal #7: For a COT with MU-MIMO (SDM) and TDM of beams transmission, adopt Alt A-1 (the node completes one eCCA on one beam, and directly move on to the eCCA on the other beam, with no transmission in the middle) when independent per-beam LBT sensing at the start of COT. |
| Qualcomm Incorporated | Proposal 19: For SDM transmission, support both (Alt1) single LBT sensing with wide beam covers all beams used in the COT and (Alt 2) independent per beam sensing. Proposal 20: For a COT with MU-MIMO (SDM) transmission if independent per beam LBT is supported, and if the node has the capability to perform simultaneous sensing in different beams, simultaneous per-beam LBT for different beams is supported. Proposal 21: Within a COT with TDM of beams with beam switching, if independent per beam LBT is supported, and if the node has the capability to perform simultaneous sensing in different beams, simultaneous per-beam LBT for different beams is supported. Proposal 22: Within a COT with TDM of beams with beam switching, when independent per-beam LBT sensing at the start of COT is performed for beams used in the COT (Alt 2 or Alt 3 in earlier agreement is considered), select, Alt A-2, namely, the node completes one eCCA on one beam, start transmission with the beam to occupy the COT, then move on to the eCCA on the other beam. Proposal 23: Within a COT with TDM of beams with beam switching, downselect to the following LBT operations Alt A: Support both Alt-1 and Alt-2, where Alt-1 and Alt -2 are part of earlier agreement as follows:  Alt 1: Single LBT sensing with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold - FFS: Details on the definition of “cover”  Alt 2: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT |
| OPPO | Proposal 11: for COT containing multiple beams, including MU-MIMO (SDM) and TDM of beams, Alt A-2 is not supported. Alt A-1 and Alt A-3 can be left for implementation. |
| Xiaomi | Proposal 3: COT should be per sensing beam based. If a sensing beam can “cover” several transmission beams, the transmission beams will share the same COT. Proposal 4: Multi-beam transmission for semi-static configured channels, such as CG-PUSCH should be studied to fully take advantage of spatial diversity. |
| CAICT | Proposal 4: Both single LBT sensing with wide beam and independent per-beam LBT sensing should be supported for COT with MU-MIMO transmission. Alt. B for per-beam LBT should be supported. |
|  | Proposal 5: For LBT within a COT with TDM of beams with beam switching, Alt 1 and 3 should be supported. |
| ITRI | Proposal 2: For a COT with MU-MIMO (SDM) transmission, the per-beam LBT for different beams is performed simultaneously in parallel. Proposal 3: For a COT with TDM transmission, the per-beam LBT for different beams is performed one after another in time domain. |

### First round discussion

### Second round discussion

Proposal 2.7.2-1

For a COT with MU-MIMO (SDM) transmission, support both Alt 1 and Alt 2 below:

- Alt 1: Single LBT sensing at the start of the COT with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold

- Alt 2: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT, if the node has the capability to perform simultaneous sensing in different beams

Note: On UE side, no UE capability will be introduced for this purpose.

Summary of Positions:

- Support both Alt 1 and Alt 2: Samsung, CATT, FUTUREWEI, CAICT, Qualcomm, Intel, Huawei/HiSilicon (Alt1 as a fallback mechanism), ITRI, Spreadtrum, TCL, NEC

- Decide single beam sensing first, deprioritize independent per beam sensing: Ericsson, Nokia

Please provide your view if not captured above:

| Company | View |
| --- | --- |
| Lenovo, Motorola Mobility | Support the proposal to support both Alt 1 and Alt 2 |
| Intel | Our view has been correctly captured and we would be OK to support both alternatives |
| LG Electronics | In general, we are fine with supporting both of two alternatives. Moreover, we think Alt 2 can be supported without if statement, i.e., irrespective of the capability, UE can perform independent per-beam LBT. |
| Apple | Support Alt 1 and Alt 2. |
| OPPO | We support both Alt1 and Alt2. When the node has the capability to simultaneously sense in differed beams, it can still apply Alt 1 for simplicity. Moreover, will we continue discussing performing per-beam LBT for different beams in TDM fashion? |
| DOCOMO | Support Alt 1 and Alt 2. |
| vivo | We support the proposal, both Alt 1 and Alt2 can be applied. |
| Ericsson | We support Alt.1. Alt.2 can be left to implementation. It is not preferable to introduce a new UE capability (on performing parallel LBTs), new signalling, and new UE behaviour to cancel the scheduled UL beams, which failed LBT. Moreover, regulations do not specify independent per beam LBT for single COT. If a device decides to perform LBT in part beams and then apply LBT for a COT it can be an implementation method. Moderator: The Alt 2 is not intended as to introduce a UE capability. This is part of implementation choice. In other words, the gNB nodes not need to know. Response to Moderator: We support Alt 1. We can agree that Alt2 can be an implementation choice. |
| ZTE, Sanechips | Support Alt 1 and Alt 2. |
| Huawei, HiSilicon | We support the following modification: Proposal 2.7.2-1 (modified) For a COT with MU-MIMO (SDM) transmission, support both Alt 1 and Alt 2 below: - Alt 1: Single LBT sensing at the start of the COT with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold, if the node does not have the capability to perform simultaneous sensing in different beams - Alt 2: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT, if the node has the capability to perform simultaneous sensing in different beams Moderator: I don’t think we need to condition Alt 1 and Alt 2 on UE capability. Even if the node has the capability of performing simultaneous sensing in different beams, the node may still choose to use a wider beam for a particular COT Added a note above to clarify |
| Futurewei | Support Alt-1 and Alt-2 |
| TCL | Support both Alt 1 and Alt 2 |
| NEC | Our view is correctly captured, we support both Alt 1 and Alt 2. |
| Transsion | Support both Alt 1 and Alt 2. |
| CATT | Our view has been correctly captured, and we support the proposal. |
| Nokia, NSB | Both option s can be supported, but we see Alt 1 as the baseline |

Proposal 2.7.2-2

Within a COT with TDM of beams with beam switching, at least support Alt 1

- Alt 1 (from previous agreement): Single LBT sensing with wide beam ‘cover’ all beams to be used in the COT

Please provide your view:

| Company | View |
| --- | --- |
| Lenovo, Motorola Mobility | We would prefer to discuss all the alternatives with TDM case as well (proposal 2.7.2-2 and proposal 2.7.2-3 In principle, we support all three alternatives from previous agreement. Furthermore, there is also an FFS under Alt 1 to define the cover. Do we need to include this case as well for sending LS to RAN4 or discuss in RAN1 on how to define the cover for the case when a single sensing beam can cover multiple transmission beams? |
| Intel | We are Ok with the proposal. |
| LG Electronics | For TDM case and particularly for UE, we think gNB should indicate whether the UE needs to perform sensing beam same with TX beam or wider than TX beam. |
| Apple | OK with the proposal |
| OPPO | We are fine with the proposal. |
| DOCOMO | Ok with the proposal. |
| vivo | We are OK with the proposal. |
| Ericsson | Ok with the proposal. This is the baseline according to regulations. |
| ZTE, Sanechips | At least for UE, we think whether UE performs sensing using wide beam or same beam with transmission should be based on UE capability or determined based on RAN4 feedback on sensing beam for transmission beam |
| Huawei, HiSilicon | Similar to Lenovo, we prefer not to split the alternatives for TDM case in two proposals and discuss three alternatives together. In principle, we can support Alt 1 only as a fallback mechanism for Alt2 in Proposal 2.7.2-3: If the node has the capability to perform simultaneous sensing in different beams, independent per-beam LBT sensing at the start of COT is performed for beams used in the COT (Alt 2 in Proposal 2.7.2-3). If the node does not have the capability to perform simultaneous sensing in different beams, single LBT sensing with wide beam ‘cover’ all beams to be used in the COT. |
| Futurewei | OK to support |
| TCL | We support the proposal. |
| NEC | We are fine with the proposal. |
| Transsion | We support the proposal. |
| CATT | We are fine with the proposal. |
| Nokia, NSB | OK with the proposal |

Proposal 2.7.2-3 (closed)

Within a COT with TDM of beams with beam switching, Alt 2 or Alt 3 is supported if the node has the capability to perform simultaneous sensing in different beams.

- Alt 2 from previous agreement: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT

- Alt 3 from previous agreement: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT with additional requirement on Cat 2 LBT before beam switch

Proposal 2.7.2-3a

Within a COT with TDM of beams with beam switching, Alt 2 is supported if the node has the capability to perform simultaneous sensing in different beams. Alt 3 is allowed as node implementation choice if the node also supports Cat 2 LBT.

- Alt 2 from previous agreement: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT

- Alt 3 from previous agreement: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT with additional requirement on Cat 2 LBT before beam switch

Please provide your view:

| Company | View |
| --- | --- |
| Lenovo, motorola Mobility | First, we prefer to discuss this proposal together with proposal 2.7.2-2. Second, this proposal states that Alt 2 or Alt is supported, does it mean that the intention is to only support one of Alt 2 or Alt 3? We prefer to support both Alt 2 and Alt 3 (and also Alt 1) |
|  | Third, we don’t think that such restriction should be specified that only when UE has to perform simultaneous sensing in different beams, then only these options can be supported |
| Intel | Our preference is for Alt-2, and we believe that Alt-3 may not be complaint with the ETSI BRAN since this does not follow or meet the minimum requirements defined for the adaptivity mechanisms. |
| Apple | Alt 2 |
| OPPO | We can support both Alt 2 and Alt 3, and the usage of the two alternatives can be left to implementation. |
| DOCOMO | Support Alt 2. |
| vivo | We prefer Alt 3 due to the reason that before switching, the new beam has not been occupied during the past time, an addition CCA check is needed to avoid collision. |
| Ericsson | This can be left for implementation. Same comment as 2.7.2-1. |
| ZTE, Sanechips | We support Alt 3. if we have no consensus on Alt 2 or Alt 3, maybe we can support Alt 2 or Alt 3 based on the node’s capability to support Cat2 LBT |
| Huawei, HiSilicon | Similar to Lenovo, we prefer not to split the alternatives for TDM case in two proposals and discuss three alternatives together. In principle, we can support Alt 1 only as a fallback mechanism for Alt2 in Proposal 2.7.2-3: If the node has the capability to perform simultaneous sensing in different beams, independent per-beam LBT sensing at the start of COT is performed for beams used in the COT (Alt 2 in Proposal 2.7.2-3). If the node does not have the capability to perform simultaneous sensing in different beams, single LBT sensing with wide beam ‘cover’ all beams to be used in the COT. We do not see any need for Alt 3. |
| Futurewei | We share similar view as ZTE. |
| TCL | We support both alternatives. |
| Moderator | Given the comments and the fact the Cat 2 LBT is optional, the proposal is modified to 2.7.2-3a |
| NEC | We support the Proposal 2.7.2-3. |
| Transsion | We support this proposal. |
| CATT | We share same view as Lenovo and OPPO, and support both Alt 2 and Alt 3. In our view, performing Cat 2 LBT before beam switching within the COT could be supported to avoid collision, and it can be decided by gNB. |
| Nokia, NSB | We are ok to support this |
| Ericsson 2 | We cannot support this proposal. Please correct our understanding if its not accurate. We don’t see any issue in allowing these options as implementation choices. If not, it is not clear to us how this will be specified. For example, how will the independent per-beam sensing be done in an observation slot of 5us? What happens if the sensing fails in one or many of the beams? We do not have much time left in this WI to discuss all these nuances and these are implementation choices that need not be specified. In addition, the word “capability” might cause some issues, so we modified that. Hence, we propose the following modifications. Modified Proposal 2.7.2-3a: Within a COT with TDM of beams with beam switching, Alt 2 and Alt 3 is are supported as implementation choices if the node has the capability can perform simultaneous sensing in different beams. Alt 3 is allowed as node implementation choice if the node and supports Cat 2 LBT capability. - Alt 2 from previous agreement: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT - Alt 3 from previous agreement: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT with additional requirement on Cat 2 LBT before beam switch |

## Multi-Channel channel access

Agreement:

Define Type A and Type B multi-channel channel access as:

- Type A: Perform independent eCCA for each channel

- Type B: Identify a primary channel and perform eCCA on the primary channel, while perform Cat 2 LBT for other channels in the last observation slot

Down-selection between

- Alt1: Support Type A multi-channel channel access only

- Alt2: Support both Type A and Type B multi-channel channel access.

Note: How eCCA is performed on each channel, and the BW of the channels over which eCCAs are performed are separately discussed

| Company | View |
| --- | --- |
| Huawei HiSilicon | Proposal 10: For multi-channel access in NR-U-60, support both Type A and Type B procedures. Proposal 11: For the supported Type A multi-channel access procedure in NR-U-60, it should be clarified whether other aspects of legacy Type A are applicable such as resuming decrementing the backoff counter on a channel I after ceasing the transmission on a channel j when idle sensing slots are detected as in legacy Type A1. |
| Vivo | Proposal 2: Both Type A and Type B multi-channel channel access can be supported. |
| Nokia Nokia Shanghai Bell | Proposal 6: Only Type A multi-channel access procedure (i.e. Alt.1 defined in RAN1#104-e meeting) shall be supported in NR-U on 60GHz band. |
| Ericsson | Observation 5 ETSI regulation for 60 GHz bands do not support Type B multi-channel access. Proposal 4 Support Alt1 in the agreement that allows only Type A multi-channel access from 37.213. Proposal 5 Do not support Type B multi-channel access for NR operation in 52.6 GHz to 71 GHz. |
| Intel | Proposal 11: Do not support, type B channel access mode for multi-carrier operation. |
| Qualcomm Incorporated | Proposal 24: Adopt Alt-1 for multi-channel access, i.e., support Type A multi-channel access only. |
| MediaTek Inc. | Proposal 2: Support only type A multi-channel channel access scheme. |
| WILUS Inc. |  Proposal 2: At least Type A multi-channel access which performs independent clear channel assessment (CCA) for each channel should be supported. For support of the Type B multi-channel access, it should be further discussed after the decision on support of Cat-2 LBT including the definition of Cat-2 LBT. |
| CAICT | Proposal 6: Support both Type A and Type B multi-channel channel access. |

### First Round Discussion

Define Type A and Type B multi-channel channel access as:

- Type A: Perform independent eCCA for each channel

- Type B: Identify a primary channel and perform eCCA on the primary channel, while perform Cat 2 LBT for other channels in the last observation slot

Discussion 2.8.1-1

Summary of Positions based on contribution proposals:

- Alt1: Support Type A multi-channel channel access only

  - Ericsson, Nokia, Qualcomm, Intel, DCM, CATT, Apple, Mediatek, Transsion, Charter, InterDigital

- Alt2: Support both Type A and Type B multi-channel channel access.

  - CAICT, WILUS (reconcile as a use-case of Cat 2 LBT), Huawei/HiSilicon, vivo, Lenovo, LG, ZTE,  Samsung, Convida, NEC, TCL,Xiaomi, FW

Please provide your view if not captured above:

| Company | View |
| --- | --- |
| Nokia, NSB | Our views have been correctly captured. |
| Ericsson | Our views are correctly captured. |
| Intel | Our view has been correctly captured. |
| Xiaomi | Support Alt2. |
| Vivo | Our views are correctly captured. However, it was counted twice which we corrected in above summary. |
| Transsion | Our view has been correctly captured. |
| DOCOMO | Our view is correctly captured. |
| CATT | Our view is correctly captured. |
| InterDigital | We added our view above. |
| Huawei, HiSilicon | Our views are correctly captured |
| NEC | Our view is correctly captured. |

### Second round discussion

Given approximately equal support for two alternatives is proposal 2.8.1-1, the moderator suggests to go with Alt 2 and leave it for implementation choice.

Proposal 2.8.2-1 (closed)

Support both Type A and Type B multi-channel channel access. The choice is based on implementation

Proposal 2.8.2-1a

Type A multi-channel channel access is supported. Type B multi-channel channel access is allowed as node implementation if the node has Cat 2 LBT capability.

Please provide your view:

| Company | View |
| --- | --- |
| Intel | We are not OK with supporting Type B, since there is no sufficient technical motivation to support an additional channel access mode in FR2-2. Therefore, we cannot agree with this proposal. |
| TCL | We support both Type A and B. That gives more freedom. |
| LG Electronics | Support |
| Moderator | Proposal updated to 2.8.2-1a to see if this is more acceptable |
| NEC | We support the Proposal 2.8.2-1a. |
| Transsion | We support this proposal. |
| Nokia, NSB | We cannot agree to the proposal. For Type B to work, there would have to be an agreed scheme for the bonded channels, and related bandwidths, which does not exist. |
| ZTE, Sanechips | Our 1st preference is proposal 2.8.2-1. For proposal 2.8.2-1a, we think if the node has cat2 LBT capability, Type B channel access can be used, not seen it as an implementation. |
| Ericsson | We cannot support Type B in this proposal. Many questions are unknown: What is the primary channel in Type B, what is the Bandwidth of this primary channel? How frequently would the devices change the primary channel? Type B works in Rel 16 because this is specified in the regulations. There is no technical motivation nor regulatory motivation to support Type B. |

## Directional LBT

Agreement: 3GPP specification consider defining at least the relative relationship between all applicable sensing beam(s) and the transmission beam(s) to define sensing beam for LBT, where at least sensing beam(s) “covers” the transmission beam(s), considering following alternatives. Target down-selection by RAN1 #106bis-e - Alt 1: Specify necessary requirement/test procedure to guarantee sensing beam “covers” the transmission beam - Some methods to define “cover” have been discussed in RAN1 (may further down select the list) and are considered as acceptable from RAN1 perspective - Alt-1A: the angle included in the [3] dB beamwidth of the transmission beam is ncluding in the [X, FFS] dB beamwidth of the sensing beam. - Alt-1B: the sensing beam gain measured along the direction of peak transmission direction is at least X [FFS] dB of the transmission beam gain - Alt-1C: The sensing beam gain is measured in one or more directions where the transmission beam EIRP is within A [FFS] dB of the peak EIRP. The sensing beam gain measured along the chosen directions is at least X [FFS] dB of the transmission beam gain in those directions. - Alt-1D: The sensing beam gain is measured in one or more directions where the transmission beam EIRP is within A [FFS] dB of the peak EIRP and the sensing beam gain measured along the chosen directions is at least X [FFS] dB of the peak sensing beam gain - Alt-1E: Sensing beam has the minimum [3] dB beamwidth which at least contains all beam peak directions of transmission beams. - Sending LS to RAN4 and inform them the above and request them to make the final choice - RAN4 choice may not be limited by the list above, but if different method is selected, RAN1 would like to have an opportunity to check as well - Alt 2. Extending the beam correspondence framework and QCL/TCI/SpatialRelationInfo framework to define “cover” and to indicate sensing beam(s) associated with a transmission beam(s) - On gNB side sensing beam selection for a DL transmission beam, - Option 1: The selection of eligible sensing beam for a transmission beam is left for gNB implementation - No testing or enforcement introduced in 3GPP spec for this option - Option 2: Beam correspondence at gNB side is assumed. Supporting one or more of the following behaviors - A1. For a gNB transmission beam corresponding to TCI state A for a certain UE, the gNB can use the same beam for sensing - A2. If TCI B is used as QCL source (Type D) for TCI A for a certain UE, then gNB transmission beam corresponding to TCI B can be used as the sensing beam for transmission with TCI A. - A3. If TCI C is NOT used as QCL source (Type D) for TCI A for any UE, then gNB cannot use the transmission beam corresponds to TCI C as the sensing beam for transmission with TCI A. - FFS: How and if to support sensing with a beam without corresponding RS sent? For example, how to use quasi-Omni beam for sensing if there is no SSB transmitted with quasi-omni beam - On UE side sensing beam selection for a UL transmission beam - Beam correspondence is assumed at UE - FFS: What if beam correspondence is not supported at UE. - Supporting one or more of the following behaviors - If the UE is indicated to transmit with a beam corresponding to a certain SRI, the UE can use the same beam for sensing - Assuming Rel.17 unified TCI framework, if the UE is indicated to transmit with a beam corresponding to a certain unified TCI, the UE can use the reception beam corresponding to the TCI for sensing - FFS: How and if to support a wider sensing beam (such as pseudo-omni beam, which is supported in WiFi) to be used for a narrower transmission beam under QCL/TCI framework - Option 0: Not supported - Option 1: UE implementation. - No testing or enforcement introduced in 3GPP spec for this option - Option 2: gNB indication. - FFS details. - FFS: How and if to support multiple sensing beams to be used for a transmission beam under QCL/TCI framework - Note: Supporting both alternatives or a combination of the two alternatives is not precluded Agreement: - When UE indicates a capability for beam correspondence with beamCorrespondenceWithoutUL-BeamSweeping ={1}, support the following behaviors - If the UE is indicated to transmit with a beam corresponding to a certain SRI, the UE can use the same beam for sensing - Assuming Rel.17 unified TCI framework, if the UE is indicated to transmit with a beam corresponding to a certain unified TCI, the UE can use the reception beam corresponding to the TCI for sensing - FFS: The case when UE does not indicate a capability for beam correspondence - Note: The UE should meet local regulatory requirements

| Company | Key Proposals/Observations/Positions |
| --- | --- |
| Huawei HiSilicon | Observation 1: specifying the spatial relationship between a wide LBT beam and multiple subsequent transmission beams is feasible if spatial properties similar to those defined in TS 38.104 for a transmission beam are defined for the LBT beam, including beam peak direction, beam center direction and beamwidth. Proposal 12: For the agreement made in RAN1#106bis-e on supported behaviors when UE indicates a capability for beam correspondence with beamCorrespondenceWithoutUL-BeamSweeping ={1}, it should be clarified that using the same transmit/receive beam for sensing refers to using the respective spatial transmit/receive filter for sensing. Proposal 13: Support FL Proposal 2.9.2-3 discussed in RAN1#106bis-e with the following modifications: Modified Proposal 2.9.2-3 from [6] For the following situations  Selecting sensing beam at the gNB [selecting sensing beam]  Selecting sensing beam at the UE when UE does not indicate a capability for beam correspondence with beamCorrespondenceWithoutUL-BeamSweeping ={1}  Selecting sensing beam at the UE when UE chooses to use a different beam for sensing than the beam used for transmission, Specify necessary requirement/test procedure to guarantee sensing beam(s) “covers” the transmission beam(s)  Some methods to define “cover” have been discussed in RAN1 (may further down select the list) and are considered as acceptable from RAN1 perspective - Alt-1A: the angle included in the [3] dB beamwidth of the transmission beam is included in the [X, FFS] dB beamwidth of the sensing beam. - Alt-1B: the sensing beam gain measured along the direction of peak transmission direction is at least X [FFS] dB of the transmission beam gain - Alt-1C: The sensing beam gain is measured in one or more directions where the transmission beam EIRP is within A [FFS] dB of the peak EIRP. The sensing beam gain measured along the chosen directions is at least X [FFS] dB of the transmission beam gain in those directions. - Alt-1D: The sensing beam gain is measured in one or more directions where the transmission beam EIRP is within A [FFS] dB of the peak EIRP and the sensing beam gain measured along the chosen directions is at least X [FFS] dB of the peak sensing beam gain - Alt-1E: Sensing beam has the minimum [3] dB beamwidth which at least contains all beam peak directions of transmission beams.  Sending LS to RAN4 and inform them the above and request them to make the final choice - RAN4 choice may not be limited by the list above, but if different method is selected, RAN1 would like to have an opportunity to check as well o RAN4 can further decide for gNB or UE separately if such test or requirement is not needed or not practical and leave it for UE to gNB or UE implementation |
| FUTUREWEI | Proposal 5: In the following scenarios Sensing at gNB  Sensing at UE that does not indicate a capability for beam correspondence with beamCorrespondenceWithoutUL-BeamSweeping ={1}  Sensing at UE that uses a different beam for sensing than the beam used for transmission, Specify necessary requirement/test procedure to guarantee sensing beam(s) “covers” the transmission beam(s)  Some methods to define “cover” have been discussed in RAN1 (may further down select the list) and are considered as acceptable from RAN1 perspective - Alt-1A: the angle included in the [3] dB beamwidth of the transmission beam is included in the [X, FFS] dB beamwidth of the sensing beam. - Alt-1B: the sensing beam gain measured along the direction of peak transmission direction is at least X [FFS] dB of the transmission beam gain - Alt-1C: The sensing beam gain is measured in one or more directions where the transmission beam EIRP is within A [FFS] dB of the peak EIRP. The sensing beam gain measured along the chosen directions is at least X [FFS] dB of the transmission beam gain in those directions. - Alt-1D: The sensing beam gain is measured in one or more directions where the transmission beam EIRP is within A [FFS] dB of the peak EIRP and the sensing beam gain measured along the chosen directions is at least X [FFS] dB of the peak sensing beam gain - Alt-1E: Sensing beam has the minimum [3] dB beamwidth which at least contains all beam peak directions of transmission beams.  Sending LS to RAN4 and inform them the above and request them to make the final choice - RAN4 choice may not be limited by the list above, but if different method is selected, RAN1 would like to have an opportunity to igna as well It is up to RAN4 to further decide for gNB or UE separately if such test or requirement is not needed or not practical and leave it for UE implementation. |
| Vivo | Proposal 13: The “cover” for sensing beam is defined as: the angle included in the [3] dB beam width of the transmission beam(s) is included in the [X] dB beam width of the sensing beam. |
| ZTE Sanechips | Proposal 4: Considering potential mismatch between sensing beam and transmission beam, the ED threshold provided by the ETSI BRAN 302 567 can be modified to consider mismatching between sensing beam and transmission beam. Proposal 11: If directional LBT is applied, the definition of the relationship between sensing beam and transmission beam for gNB side can be left to the implementation. Proposal 12: If directional LBT is applied, for the case where UE has no beam correspondence capability or sensing beam is different with transmission beam, the relationship between sensing beam and transmission beam can be defined by using one or more alternatives listed in Alt1.  Alt-1A: the angle included in the [3] dB beamwidth of the transmission beam is included in the [X, FFS] dB beamwidth of the sensing beam.  Alt-1B: the sensing beam gain measured along the direction of peak transmission direction is at least X [FFS] dB of the transmission beam gain  Alt-1C: The sensing beam gain is measured in one or more directions where the transmission beam EIRP is within A [FFS] dB of the peak EIRP. The sensing beam gain measured along the chosen directions is at least X [FFS] dB of the transmission beam gain in those directions.  Alt-1D: The sensing beam gain is measured in one or more directions where the transmission beam EIRP is within A [FFS] dB of the peak EIRP and the sensing beam gain measured along the chosen directions is at least X [FFS] dB of the peak sensing beam gain  Alt-1E: Sensing beam has the minimum [3] dB beamwidth which at least contains all beam peak directions of transmission beams. |
| Spreadtrum Communications | Proposal 1: RAN4 decides whether sensing beam selection on the gNB side is an implementation issue. Proposal 2: For NCB PUSCH transmission, the reception beam of CSI-RS associated with SRS can be used as the sensing beam. Proposal 3: For CB PUSCH transmission, an extra RS indication field is needed to indicate the sensing beam. |
| Nokia Nokia Shanghai Bell | Observation 6: Generic requirements for relative relationship between LBT sensing beam(s) and transmission beam(s) should be done in RAN4, not in RAN1. Proposal 11: Proposal 2.9.2-3 from RAN1#106bis-e FL summary is agreed for defining the relative relationship between applicable LBT sensing beam(s) and the transmission beam(s) for gNB, when UE does not indicate capability for beam correspondence without UL beam sweeping or when UE uses different beam for sensing than for transmission. |
| Ericsson | Observation 7 The effectiveness of LBT itself as medium access mechanism for co-existence in unlicensed spectrum in 60 GHz band is questionable. Therefore, any further enhancement on LBT baseline from the HS need to be justified both on the performance gain and the required complexity. Observation 8 Common understanding in ETSI and IEEE 802.11ad and IEEE 802.11ay specs are omni-directional LBT or quasi-omnidirectional LBT Observation 9 Simulation studies in general indicate no significant gain from using directional LBT. Observation 10 Directional LBT is currently not precluded in the existing regulations. EN 302 567¨s tests intrinsically ensure sensing beam is in the direction of the transmission beam for devices equipped with directional antenna systems. Observation 11 Notion of “beams” is non-existent for gNBs in RAN4. Proposal 9 Support Alt. A in Discussion 2.9.2-1 from RAN1#106be, i.e., to leave the sensing beam selection on the gNB side up to implementation. Proposal 10 If Alt. B in Discussion 2.9.2-1 from RAN1#106be is agreed for gNBs, RAN1 sends an LS to RAN4 and RAN4 can further decide if such test or requirement is needed for gNBs or not. Proposal 11 For Ues that do not indicate a capability for beam correspondence with beamCorrespondenceWithoutUL-BeamSweeping ={1}, at least support omni-directional LBT or quasi-omni-directional LBT. |
| Intel | Proposal 16: When directional sensing is performed, the COT should be considered to be acquired only in the transmission beams for which the LBT is performed and the LBT measurements have indicated that the channel is idle. Proposal 17: When directional sensing is performed, and multiple concurrent COTs are acquired, these should be independently treated unless LBT measurements have overlapping beams. In this case, the COT duration should be hannel from the time when the LBT succeeds for the first time in one of these overlapping beams. Proposal 23: Beam correspondence mandatory capability signaling is set to 1 for all supported unlicensed bands in FR2-2. Proposal 24: For situations where UE uses a different beam for sensing than the beam used for transmission, specify necessary requirement/test procedure to guarantee sensing beam(s) “covers” the transmission beam(s), which should be left up to RAN4 to decide. |
| Lenovo Motorola Mobility | Observation 1: For NR unlicensed bands between 52.6 GHz and 71 GHz, with directional LBT based channel access mechanism, LBT failure on a beam could require a beam update procedure and that results in increased latency Proposal 1: For NR unlicensed bands between 52.6 GHz and 71 GHz, with directional LBT based channel access mechanism, when beam correspondence is not supported by UE, UE reporting can be enhanced such that UE can report sensing beams corresponding to the UL transmission beams (or activated TCI states or SRI for UL transmission) - gNB can then indicate sensing beams to UE corresponding to the indicate UL transmission beam Proposal 2: For NR unlicensed bands between 52.6 GHz and 71 GHz, with directional LBT based channel access mechanism, Alt 2 with option 2 should be supported for both UE side and gNB side i.e.:  Alt 2. Extending the beam correspondence framework and QCL/TCI/SpatialRelationInfo framework to define “cover” and to indicate sensing beam(s) associated with a transmission beam(s) - On gNB side sensing beam selection for a DL transmission beam,  Option 2: Beam correspondence at gNB side is assumed. Supporting one or more of the following behaviors  A1. For a gNB transmission beam corresponding to TCI state A for a certain UE, the gNB can use the same beam for sensing  A2. If TCI B is used as QCL source (Type D) for TCI A for a certain UE, then gNB transmission beam corresponding to TCI B can be used as the sensing beam for transmission with TCI A.  A3. If TCI C is NOT used as QCL source (Type D) for TCI A for any UE, then gNB cannot use the transmission beam corresponds to TCI C as the sensing beam for transmission with TCI A.  FFS: How and if to support sensing with a beam without corresponding RS sent? For example, how to use quasi-Omni beam for sensing if there is no SSB transmitted with quasi-omni beam Proposal 3: For NR unlicensed bands between 52.6 GHz and 71 GHz, with directional LBT based channel access mechanism, the relationship between sensing beam(s) used by an initiating device to initiate a COT and the transmission beam(s) used by a responding device to share the COT should be defined. Proposal 4: For NR unlicensed bands between 52.6 GHz and 71 GHz, with directional LBT based channel access mechanism, configuration and/or indication of multiple sensing beams to UE should be specified for beam-based UL transmission Proposal 5: For NR unlicensed bands between 52.6 GHz and 71 GHz, with directional LBT based channel access mechanism, the relationship between the sensing and transmission beams can be configured based on the TCI framework to be: - One-to-one mapping between sensing beam and transmission beam - One sensing beam to many transmission beams mapping - Many sensing beams to one transmission mapping Proposal 14: For NR unlicensed bands between 52.6 GHz and 71 GHz with directional LBT based channel access mechanism, within a COT, PDCCH monitoring is not supported in the CORESETs corresponding to Tx beams which are not covered by the sensing beam(s) used to initiate the COT. |
| NEC | Proposal 3: For gNB and UE without a capability for beam correspondence with beamCorrespondenceWithoutUL-BeamSweeping ={1}, necessary requirement/test procedure based on Alt-1A to guarantee sensing beam(s) “covers” the transmission beam(s) should be supported, namely the angle included in the [3] dB beamwidth of the transmission beam is included in the [X] dB beamwidth of the sensing beam. |
| Samsung | Proposal 5:  Support extending the beam correspondence framework and/or QCL/TCI framework to define “cover” (Alt 2), option.2;  Support option 2 gNB indication in the sense of broad sensing beam can be implicitly indicated by reusing the set of DL RS signals which are used as QCL-D sources for the covered UL narrow transmission beams. |
| InterDigital Inc. | Proposal 1: Support sensing beam determination using Alt.1 (at least for no beam correspondence or multiple associated transmission beams) and Alt.2 (for single associated transmission beam with beam correspondence). |
| Convida Wireless | Proposal 1: Both omni-directional and directional LBT should be supported for frequency range of 52.6GHz to 71GHz. Proposal 3: If beam correspondence is assumed, sensing beam may be determined from transmission beam corresponding to a certain TCI state for frequency range of 52.6GHz to 71GHz. Proposal 4: If beam correspondence is not assumed, sensing beam may be determined from a certain dB beamwidth for frequency range of 52.6GHz to 71GHz. |
| LG Electronics | Proposal #8: The relationship between the LBT beam with a specific direction to acquire the COT and the transmission beam(s) allowed to transmit in that COT should be defined considering the relationship between the CCA range of the LBT beam and the interference range of the transmission beam(s). Proposal #9: It would be beneficial for coexistence that channel occupancy acquired by directional LBT is shared only for DL and UL signals/hannel having spatial QCL relationship. Proposal #10: The relative relationship between all applicable sensing beams and the transmission beam can be defined in RAN1 by using the beam correspondence and the QCL/TCI framework in RAN1 without RAN4 involvement. Proposal #11: On gNB side sensing beam selection for a DL transmission beam, support Option 1 (the selection of eligible sensing beam for a transmission beam is left for gNB implementation). Proposal #12: On UE side sensing beam, UE can use the same beam for sensing corresponding to indicated transmission beam through a certain SRI or a certain unified TCI, regardless of whether UE indicates a capability for beam correspondence with beamCorrespondenceWithoutUL-BeamSweeping ={1} or beamCorrespondenceWithoutUL-BeamSweeping = {0}. Proposal #13: If the sensing beam that does not correspond to the transmission beam is allowed, it should be a wider sensing beam (e.g., pseudo-omni beam) than the transmission beam and it is necessary to indicate whether the sensing beam is corresponding to a transmission beam or a wider sensing beam, by extending QCL/TCI framework such that a certain reference signal (RS) for the sensing beam or the wider sensing beam (i.e., pseudo-omni beam) configured to each RS in spatialrelationinfo (e.g., SSB, CSI-RS, and SRS) and/or unified TCI state. |
| NTT DOCOMO INC. | Proposal 4: Support Alt 2 (Extending the beam correspondence framework and QCL/TCI/SpatialRelationInfo framework) for the definition of sensing beam for gNB and for UE not indicating BC={1}, or for UE to initiate transmissions with multiple beams, if needed  To define a sensing beam covering more than one transmission beams, 1-to-N or N-to-N QCL/TCI relationship can be considered Proposal 5: Support independent per-beam LBT sensing for SDMed/TDMed transmission at least for UE not supporting to have a single wide sensing beam covering mutilple transmission beams  Support Alt A-2 as it is available already without any specification impact |
| Qualcomm Incorporated | Proposal 25: If Alt 1 is chosen, Support Alt -1-D namely the sensing beam gain is measured in one or more directions where the transmission beam EIRP is within A [FFS] dB of the peak EIRP and the sensing beam gain measured along the chosen directions is at least X [FFS] dB of the peak sensing beam gain. Proposal 26: Alt-1-D Alternative formulation: The sensing beam gain is measured in one or more directions where the transmission beam EIRP is within A [FFS] dB of the peak EIRP and the sensing beam gain measured along the chosen directions is at least X [FFS] dB of the transmission beam gain in those directions. Proposal 27: If Alt -2 is chosen, adopt gNB behaviors A1 and A2 for sensing at gNB. Proposal 28: Adopt Discussion Proposal 2.9.2-2 from RAN1-106-bis-e [5], with further modification that the LS to RAN4 should include the text that RAN4 can further decide for UE if such test is not needed or not practical and leave it for UE implementation. |
| TCL Communications | Proposal 1: For a unified TCI framework, the relationship describing coverage with beam correspondence is supported. |
| OPPO | Proposal 5: The UE can still use QCL/TCI framework to define ‘cover’ when it does not indicate a capability for beam correspondence. |
| ITRI | Proposal 1: In order to avoid resource wastage and hidden node problem, the LBT beam should be the same as the transmission beam. |

### First Round Discussion

Please provide your views below on the compromise option. The proposal is modified from 2.9.2-3 from RAN1-106bis-e based on Huawei’s suggestions.

Proposal 2.9.1-1: (closed)

For the following situations

- Selecting sensing beam at the gNB

- Selecting sensing beam at the UE when UE does not indicate a capability for beam correspondence with beamCorrespondenceWithoutUL-BeamSweeping ={1}

- Selecting sensing beam at the UE when UE uses chooses to use a different beam for sensing than the beam used for transmission,

Specify necessary requirement/test procedure to guarantee sensing beam(s) “covers” the transmission beam(s)

- Some methods to define “cover” have been discussed in RAN1 (may further down select the list) and are considered as acceptable from RAN1 perspective

  - Alt-1A: the angle included in the [3] dB beamwidth of the transmission beam is included in the [X, FFS] dB beamwidth of the sensing beam.

  - Alt-1B:  the sensing beam gain measured along the direction of peak transmission direction is at least X [FFS] dB of the transmission beam gain

  - Alt-1C:  The sensing beam gain is measured in one or more directions where the transmission beam EIRP is within A [FFS] dB of the peak EIRP.  The sensing beam gain measured along the chosen directions is at least X [FFS] dB of the transmission beam gain in those directions.

  - Alt-1D: The sensing beam gain is measured in one or more directions where the transmission beam EIRP is within A [FFS] dB of the peak EIRP and the sensing beam gain measured along the chosen directions is at least X [FFS] dB of the peak sensing beam gain

  - Alt-1E: Sensing beam has the minimum [3] dB beamwidth which at least contains all beam peak directions of transmission beams.

- Sending LS to RAN4 and inform them the above and request them to make the final choice

  - RAN4 choice may not be limited by the list above, but if different method is selected, RAN1 would like to have an opportunity to check as well

  - RAN4 can further decide for gNB or UE separately if such test or requirement is not needed or not practical and leave it to gNB or UE implementation

Alternative proposal from LGE, Lenovo and Ericsson (maybe others)

Proposal 2.9.1-1a: (closed)

- Selecting sensing beam at the gNB is up to gNB’s implementation

- Sensing beam at the UE is the may use a wider beam for sensing or a quasipseudo-omni directional beam, when the UE does not indicate a capability for beam correspondence with beamCorrespondenceWithoutUL-BeamSweeping ={1}

Agreement

For the following situations

- Selecting sensing beam at the gNB

- Selecting sensing beam at the UE when UE does not indicate a capability for beam correspondence with beamCorrespondenceWithoutUL-BeamSweeping ={1}

- Selecting sensing beam at the UE when UE uses a different beam for sensing than the beam used for transmission,

Specify necessary requirement/test procedure to guarantee sensing beam(s) “covers” the transmission beam(s)

- Some methods to define “cover” have been discussed in RAN1

  - Alt-1A: the angle included in the [3] dB beamwidth of the transmission beam is included in the [X, FFS] dB beamwidth of the sensing beam.

  - Alt-1B:  the sensing beam gain measured along the direction of peak transmission direction is at least X [FFS] dB of the transmission beam gain

  - Alt-1C:  The sensing beam gain is measured in one or more directions where the transmission beam EIRP is within A [FFS] dB of the peak EIRP.  The sensing beam gain measured along the chosen directions is at least X [FFS] dB of the transmission beam gain in those directions.

  - Alt-1D: The sensing beam gain is measured in one or more directions where the transmission beam EIRP is within A [FFS] dB of the peak EIRP and the sensing beam gain measured along the chosen directions is at least X [FFS] dB of the peak sensing beam gain

  - Alt-1E: Sensing beam has the minimum [3] dB beamwidth which at least contains all beam peak directions of transmission beams.

  - Alt-1F:

    - Selecting sensing beam at the gNB is up to gNB’s implementation

    - Sensing beam at the UE may use a wider beam for sensing than the beam used for transmission, when the UE does not indicate a capability for beam correspondence with beamCorrespondenceWithoutUL-BeamSweeping ={1}

- Sending LS to RAN4 and inform them the above and request them to make the final choice

  - RAN4 choice may not be limited by the list above

  - RAN4 can further decide for gNB or UE separately if such test or requirement is not needed or not practical and leave it to gNB or UE implementation

Please provide your view:

| Company | View |
| --- | --- |
| Nokia, NSB | We support the modified proposal |
| Futurewei | We support the proposal |
| Ericsson | We can support the proposal in principle for Ues. For gNBs, we think that this can be left for implementation as there are no “beams” defined in RAN4 for gNBs, it is unclear how or where this requirement can be specified. Moderator: We already have the last bullet to give the RAN4 a chance to not developing tests or requirements. Is that enough? Response to Moderator: As a compromise, we are ok with the last sub-bullet and propose to remove the later part of the first sub-bullet point. We think this would avoid the unnecessary ping-pong between RAN1 and RAN4 that could potentially delay the completion of this WI. - Sending LS to RAN4 and inform them the above and request them to make the final choice - RAN4 choice may not be limited by the list above, but if different method is selected, RAN1 would like to have an opportunity to check as well - RAN4 can further decide for gNB or UE separately if such test or requirement is not needed or not practical and leave it to gNB or UE implementation |
| Lenovo, Motorola Mobility | In above proposal, basically three cases are discussed commonly. We would prefer to separate the discussion for gNB and UE side. For the gNb side, we are fine with the proposal. However, for UE side, when beam correspondence is not support and/or UE is not able to use the same beam for sensing as for transmission, then we would prefer gNB indication to use some other beams for sensing. UE report a set of sensing beams based on the testing requirements, but it is up to gNB to indicate the sensing beam(s) to be used. Moderator: I believe we discussed this before. I don’t think gNB has enough information on the shape of UE side sensing beams, and it may not be easy to introduce mechanism to support UE reporting the shape of beams. |
| Intel | We support this proposal |
| Apple | Support the proposal |
| Support | Support the proposal |
| LG Electronics | One question to the following bullet: Selecting sensing beam at the UE when UE chooses to use a different beam for sensing than the beam used for transmission What is the motivation UE chooses to use a different beam compared to sensing beam? From our understanding, mainly this is for pseudo-omni LBT. If this is the case, do we need to additionally define “cover” relationship for pseudo-omni LBT? Or, do we assume a kind of LBT narrower than pseudo-omni LBT and wider than transmission beam? Moderator: This may or may not be a pseudo-omni beam. For example if the node wants to transmit in two narrow beams, it can sense with a slightly wider beam that covers both transmission beam. |
| OPPO | We support the proposal. |
| Vivo | We support the proposal. |
| NEC | We support the proposal |
| Mediatek | We support the proposal. However, we have a clarification question: what is the definition of the sensing beam gain and transmission beam gain, is it going to be defined by RAN 4? Moderator: Yes I think this should be handled in RAN4 |
| Transsion | We support the proposal. |
| Sony | We support the proposal |
| ZTE, Sanechips | In principle, we agree with the modified proposal. But for sensing beam of gNB side, we still prefer to leave it to gNB implementation. Although RAN4 can decide that sensing beam of gNB side can be handled by the implementation, there is still a certain risk that sensing beam is defined for gNB side, which will limit the flexibility of gNB to a certain extent. Moderator: Thanks for being flexible. Let’s continue the discussion in RAN4 |
| LG Electronics2 | Thanks Moderator for the response. Still we don’t think RAN4 requirement is needed for gNB and for UE. However, for the sake of progress, we can be compromised with the following changes. Proposal 2.9.1-1: For the following situations - Selecting sensing beam at the gNB - Selecting sensing beam at the UE when UE does not indicate a capability for beam correspondence with beamCorrespondenceWithoutUL-BeamSweeping ={1} - Selecting sensing beam at the UE when UE is configured/indicated to use a different beam for sensing than the beam used for transmission, - FFS how to configure/indicate to UE the different beam for sensing than the beam used for transmission Specify necessary requirement/test procedure to guarantee sensing beam(s) “covers” the transmission beam(s) - Some methods to define “cover” have been discussed in RAN1 (may further down select the list) - Alt-1A: the angle included in the [3] dB beamwidth of the transmission beam is included in the [X, FFS] dB beamwidth of the sensing beam. - Alt-1B: the sensing beam gain measured along the direction of peak transmission direction is at least X [FFS] dB of the transmission beam gain - Alt-1C: The sensing beam gain is measured in one or more directions where the transmission beam EIRP is within A [FFS] dB of the peak EIRP. The sensing beam gain measured along the chosen directions is at least X [FFS] dB of the transmission beam gain in those directions. - Alt-1D: The sensing beam gain is measured in one or more directions where the transmission beam EIRP is within A [FFS] dB of the peak EIRP and the sensing beam gain measured along the chosen directions is at least X [FFS] dB of the peak sensing beam gain - Alt-1E: Sensing beam has the minimum [3] dB beamwidth which at least contains all beam peak directions of transmission beams. - Sending LS to RAN4 and inform them the above and request them to make the final choice - RAN4 choice may not be limited by the list above, but if different method is selected, RAN1 would like to have an opportunity to check as well - RAN4 can further decide for gNB or UE separately if such test or requirement is not needed or not practical and leave it to gNB or UE implementation Moderator: I modified a little to use “UE uses” instead of “UE chooses” to make it vague the decision is coming from gNB or from UE itself. I don’t think we should force the decision on beam to come from gNB as well. Please see if it works for you. |
| DOCOMO | We are fine with the proposal |
| CATT | We support the proposal. |
| InterDigital | We support the proposal |
| Huawei, HiSilicon | We support Proposal 2.9.1-1 |
| LG Electronics3 | We still have a concern on the following bullet. - Selecting sensing beam at the UE when UE uses chooses to use a different beam for sensing than the beam used for transmission, Our view is that which sensing beam is used by UE should be controlled by gNB, regardless of whether the beam is omni-beam or middle range beam between omni-beam and TX beamformed beam. We cannot accept this vague proposal if RAN1 have a consensus on this point. However, if we go with the proposal, for the simplicity, we may allow only omni-beam LBT in addition to sensing beam = TX beamformed beam. In this sense, the proposal 2.9.1-1 can be further modified as follows, which means Proposal 2.9.1-1: For the following situations - Selecting sensing beam at the gNB - Selecting sensing beam at the UE when UE does not indicate a capability for beam correspondence with beamCorrespondenceWithoutUL-BeamSweeping ={1} - Selecting sensing beam with quasi-omni direction at the UE when UE uses chooses to use a different beam for sensing than the beam used for transmission, Specify necessary requirement/test procedure to guarantee sensing beam(s) “covers” the transmission beam(s) - Some methods to define “cover” have been discussed in RAN1 (may further down select the list) and are considered as acceptable from RAN1 perspective - Alt-1A: the angle included in the [3] dB beamwidth of the transmission beam is included in the [X, FFS] dB beamwidth of the sensing beam. - Alt-1B: the sensing beam gain measured along the direction of peak transmission direction is at least X [FFS] dB of the transmission beam gain - Alt-1C: The sensing beam gain is measured in one or more directions where the transmission beam EIRP is within A [FFS] dB of the peak EIRP. The sensing beam gain measured along the chosen directions is at least X [FFS] dB of the transmission beam gain in those directions. - Alt-1D: The sensing beam gain is measured in one or more directions where the transmission beam EIRP is within A [FFS] dB of the peak EIRP and the sensing beam gain measured along the chosen directions is at least X [FFS] dB of the peak sensing beam gain - Alt-1E: Sensing beam has the minimum [3] dB beamwidth which at least contains all beam peak directions of transmission beams. - Sending LS to RAN4 and inform them the above and request them to make the final choice - RAN4 choice may not be limited by the list above, but if different method is selected, RAN1 would like to have an opportunity to check as well - RAN4 can further decide for gNB or UE separately if such test or requirement is not needed or not practical and leave it to gNB or UE implementation Alternatively, RAN1 can simply agree with the following proposal which doesn’t require any additional RAN4 work. Proposal: - Selecting sensing beam at the gNB is up to gNB’s implementation - Sensing beam at a UE is the pseudo-omni directional beam, when the UE does not indicate a capability for beam correspondence with beamCorrespondenceWithoutUL-BeamSweeping ={1} |
| Lenovo, Motorola Mobility | We also tend to have similar view as LG that gNB should have control over which beam is used by sensing at the UE. As we have described earlier, UE may indicate a set of suitable beams for sensing (without indicating the shape and/or width of the beam). This would be relatively easy to indicate. However, if above seems to be controversial, then we are also okay with the suggested proposal from LG |
| Ericsson 2 | We are ok to support LG’s proposal. Moreover, if LG’s new proposal is agreed, we do not see the need to have agreement on the current Proposal 2.9.1-1 as LG’s new proposal already covers the cases for gNB and UE with beamCorrespondenceWithoutUL-BeamSweeping ={0}. For Ues, it can choose a quasi-omni beam or a slightly wider beam than the transmission beam (but still narrower than the quasi-omni beam), therefore we do not see a need to mention only quasi-omni beam. According to quasi-omni definition in IEEE specs, it is the antenna configuration that yields the highest beamwidth attainable. Proposal: - Selecting sensing beam at the gNB is up to gNB’s implementation - Sensing beam at the UE is the may use a wider beam for sensing or a quasipseudo-omni directional beam, when the UE does not indicate a capability for beam correspondence with beamCorrespondenceWithoutUL-BeamSweeping ={1} |

## No LBT

Agreement: For regions where LBT is not mandated, gNB should indicate to the UE this gNB-UE connection is operating in LBT mode or no-LBT mode - Support both cell specific (common for all Ues in a cell as part of system information or dedicated RRC signalling or both) and UE specific (can be different for different Ues in a cell as part of UE-specific RRC configuration) gNB indication Conclusion: There is no consensus to support per beam LBT mode or no-LBT mode UE specific gNB indication. Conclusion: For regions where LBT is not mandated, there is no consensus to introduce L1 signalling for gNB to indicate to the UE if the operation is in LBT mode or no-LBT mode. Note this is different from the DCI field indicate the LBT type for UL transmission.

| Company | Key Proposals/Observations/Positions |
| --- | --- |
| Huawei HiSilicon | Observation 3: When network allows enabling/disabling the LBT mode, coexistence issues would arise as the performance of the nodes ignaling with LBT mode would be adversely impacted by the nodes operating with No-LBT on the channel without a time limit . Proposal 29: For operation in the 60 GHz band, in regions where LBT is not mandated, COT should be limited when No–LBT is used. |
| ZTE Sanechips | Proposal 6: Support gNB and its UE(s) having different LBT mode. Proposal 7: Support per cell for the indication of LBT mode or No LBT mode. Proposal 8: No LBT can be considered to be used in the following use cases:  Specific areas such as ITU region 2 and 3.  Interference controlled environment.  The transmission beams of nodes of different operators in the same system (e.g., NR-U) have little interference with each other. Proposal 10: Conditions for No LBT fallback to LBT should be further studied, e.g., based on the interference level or correctly decoding rate. Proposal 23: If per cell is agreed for the indication of LBT mode and No LBT mode, it is suggested to capture this feature in RRC parameters list on cell specific and UE specific configuration. |
| Nokia Nokia Shanghai Bell. | Observation 15: Channel access mechanism without LBT should fulfil the requirements of EN 303 722 as well as the expected requirements of EN 303 753. Proposal 27: Leave any additional conditions/mechanisms/restriction/fallback modes on the no-LBT channel access mode for gNB implementation. |
| Ericsson | Proposal 25 Cell-specific system information indication of LBT ON/OFF is included in SIB1 - Define same DCI_1_0 sizes for both LBT on/off (licensed and unlicensed operation) Proposal 26 Enabling use of LBT for contention exempt transmissions is indicated in SIB1. The type of LBT (CAT3 or CAT2 LBT) to be used can be left for implementation. |
| NEC | Observation 1: Based on long term measurement, the channel assessment in statistic could be considered to determine or switch the operation mode. Proposal 11: For regions where LBT is not mandated, the mechanism and conditions for LBT mode and no-LBT mode switching should be specified to simplify the system implement. |
| Samsung | Proposal 3: For indication of the LBT/no-LBT mode:  gNB determines its mode by implementation;  UE assumes both the gNB and UE operates according to the indicated mode in the cell-specific indication;  UE assumes the UE operates according to the indicated mode in the UE-specific indication;  the UE-specific indication overrides the cell-specific indication when both of them are provided. |
| InterDigital Inc. | Proposal 7: The UE receives indication of the channel access mode (omni-directional, directional, receiver assistance, no LBT) from the gNB. |
| Convida Wireless | Proposal 2: Adaptation and indication for LBT mode, no-LBT mode and LBT sub-modes for system performance optimization should be considered. |
| LG Electronics | Proposal #19: For regions where LBT is not mandated, the mechanism for switching between the no-LBT mode and LBT mode should be supported and specified at least for UL, and the channel access mode switching between no-LBT mode and LBT mode can be determined e.g., based on the consecutive decoding success or failure or interference measurement. |
| TCL Communications | Proposal 2: The beam specific LBT mode configuration is supported for flexibility. |
| OPPO | Proposal 8: RAN1 should discuss how to indicate the supported LBT types in 60GHz band in DCI. |
| Xiaomi | Proposal 1: How to prevent long time continuous channel occupying for Tx using No-LBT should be further studied. Proposal 2: Neglect the field ChannelAccess-Cpext/ChannelAccess-Cpext-CAPC in the scheduling DCI if gNB informs UE to apply No-LBT. |

For this topic, the moderator does not see anything essential to be discussed for this meeting. If you see something worth discussion, please bring it up

Discussion 2.10-1 (open discussion)

| Company | Proposed discussion points |
| --- | --- |
| ZTE, Sanechips | On the restriction of the duration of a transmission for using No LBT, we think that similar restriction as defined in Type 2C channel access procedure in TS 37.213 can also introduced in above 52.6GHz NR-U frequency band but the length of a transmission can be relaxed. If there is no any limitations on the duration of a transmission for using No LBT, it may lead to unfair the opportunities of channel access/occupancy and also violate the basic principle of friendly and fair coexistence, e.g., the transmission of some nodes is continuously blocked, or the effect of persistent interference on devices that have occupied the channel in advance. Moderator: For NR-U, the restriction on no-LBT is because there is a Cat 2 LBT. Now cat 2 LBT is not always needed, thus it seems to me adding restriction to no LBT is not needed as well? Of course, if there is strong interest, we can discuss more. |
| Samsung | We would ask moderator to clarify our previous agreement, especially the wording of “gNB-UE link”. - If a network provides a cell-specific indication on the mode of “gNB-UE link”, does it apply to gNB or UE or both gNB and UE? - If a network provides a UE-specific indication on the mode of “gNB-UE link”, does it apply to gNB or UE or both gNB and UE? We believe this discussion is needed, since there is a bracket and a note in TS 37.213 draft to ask for resolve this issue. Moderator: My understanding is this applies to both gNB and UE (the link). Let’s see if there is any other thoughts. |

### First round discussion

Proposal 2.10.1-1: (closed)

In the following agreement:

Agreement:

For regions where LBT is not mandated, gNB should indicate to the UE this gNB-UE connection is operating in LBT mode or no-LBT mode

- Support both cell specific (common for all Ues in a cell as part of system information or dedicated RRC signalling or both) and UE specific (can be different for different Ues in a cell as part of UE-specific RRC configuration) gNB indication

When LBT mode or no-LBT mode is indicated, the mode applies to both gNB and UE for the operation between the gNB and UE.

Please provide your view:

| Company | View |
| --- | --- |
| Huawei, HiSilicon | Support if “apples” is changed to “applies” |
| ZTE, Sanechips | If the interference between the gNB and the UE side is equivalent or close, we support to apply the same LBT mode for them. However, for the scenarios where gNB and UE are in different interference environments, we think it would be better to use different LBT mode between gNB and UE. Moderator: The point is, instead of introducing a new “mixed” mode operation, which may involves more spec impact, use LBT mode if one side of the link requires LBT. |
| NEC | We would ask a question, does it mean both gNB and UE should operate in the same mode? In our understanding, since different Ues in a cell can operate in different mode, it is implicitly supported in above agreement that gNB and UE can have different mode, maybe we need a clarification for this. Moderator: The proposal is gNB and UE operate in the same mode “for that link” |
| Ericsson | When LBT mode is on in regions where LBT is required, the initiating device (gNB)always needs to perform LBT. The responding device may or may not perform LBT. EN 302 567 allows responding device to transmit without LBT within a max a duration of 5ms. Therefore, this indication is only for the Ues in our opinion. The Ues need not know which mode the gNB is operating on. The UE only needs to know if it needs to perform LBT before its transmissions. Moderator: This also makes sense. How about we change the language to from UE perspective? |

### Second round discussion

Proposal 2.10.2-1:

In the following agreement:

Agreement:

For regions where LBT is not mandated, gNB should indicate to the UE this gNB-UE connection is operating in LBT mode or no-LBT mode

- Support both cell specific (common for all Ues in a cell as part of system information or dedicated RRC signalling or both) and UE specific (can be different for different Ues in a cell as part of UE-specific RRC configuration) gNB indication

When LBT mode or no-LBT mode is indicated to a UE, the UE assumes the mode applies to both gNB and UE for the operation between the gNB and UE.

- Note: The gNB still may or may not perform LBT, but UE does not need to know

Proposal 2.10.2-1a:

In the following agreement:

Agreement:

For regions where LBT is not mandated, gNB should indicate to the UE this gNB-UE connection is operating in LBT mode or no-LBT mode

- Support both cell specific (common for all Ues in a cell as part of system information or dedicated RRC signalling or both) and UE specific (can be different for different Ues in a cell as part of UE-specific RRC configuration) gNB indication

When LBT mode or no-LBT mode is indicated to a UE, the mode applies to the UE for the operation between the gNB and UE.

Please provide your view:

| Company | View |
| --- | --- |
| vivo | We’re okay with the first part and UE assumption/behaviour for UE’s LBT mode in the 2nd part. But we’re not sure about UE assumption of gNB’s LBT mode in the 2nd part. The note says ” The gNB still may or may not perform LBT, but UE does not need to know”. If UE does not need to know whether gNB perform LBT or not, why does UE need to assume what LBT mode the gNB is on. We’d like to understand what implication (specification impact) of such UE assumption on gNB’s LBT mode. |
| Lenovo, Motorola Mobility | Fine to support |
| Intel | We have same concerns as Vivo: since the UE does not need to know whether LBT or no LBT is used at the gNB, why the UE has to make an assumption of what the gNB may do? Moderator: The assumption here is there is no impact to UE behavior if gNB performs LBT or not |
| Samsung | We are ok with the proposal for cell-specific indication, but we have a clarification question for the UE-specific indication. If the indicated mode is for both gNB and UE, does it mean a gNB can have multiple modes, and each of them could be corresponding one UE (or group of Ues)? For example, it’s allowed that one gNB use LBT-mode to transmit to a set of Ues, and use a no-LBT-mode to transmit to another set of Ues? Moderator: Yes. This is the implication of per UE indication |
| Futurewei | Same concern as Vivo and Intel |
| LG Electronics | We are fine with this proposal. |
| Apple | Need clarification why UE needs to assume gNB perform LBT, regardless whether gNB do it or not. |
| NEC | We are fine with the part for cell-specific indication. But regarding the UE-specific indication and the note in this proposal, if a UE needs not know the operation mode of gNB, we think it is also not necessary to specify the UE to assume the indicated mode applies to both gNB and UE. The UE only needs to know whether it needs to perform LBT before its transmission. In other words, gNB and UE may have different mode for a link. Further, although we don’t mean to complicate the progress, we have not discussed how to determine the operation mode of UE in regions where LBT is not mandated, is it up to gNB’s implementation? |
| DOCOMO | We are generally fine. |
| Ericsson | We also have a similar question. If there is any implication or need for UE to know whether gNB performs LBT or not? If no, then we don’t see the need for the text in red. We propose the following changes: When LBT mode or no-LBT mode is indicated to a UE, the UE assumes the mode applies to both gNB and the UE for the operation between the gNB and UE. - Note: The gNB still may or may not perform LBT, but UE does not need to know |
| ZTE, Sanechips | We are fine with this proposal. |
| Huawei, HiSilicon | Similar to some other companies, we are not sure why UE needs to make any assumption about what gNB does. Moreover, if UE needs to make an assumption, why it is OK if this assumption is incorrect (eg UE assumes that gNB performs LBT while gNB doesn’t). Given above questions, we do not agree with the proposal at this time. |
| Convida Wireless | We are generally ok with the proposal. |
| Futurewei | We are fine with Ericsson’s proposed changes. |
| TCL | We support the proposal. Ericsson’s change makes the scenarios more clear. |
| ASUSTeK | Fine with Ericsson’ version while prefer to delete the note since after Ericsson’s edit the indicated mode applies to the UE and is silent on gNB). Also we are not sure about implication of the note. |
| Moderator | Added Ericsson’s version as Proposal 2.10.2-1a |
| NEC | We are fine with Proposal 2.10.2-1a. |
| Transsion | We are fine with this proposal. |
| CATT | We are fine with this proposal. We just have one question for clarification. In our understanding, if LBT mode is indicated to a UE, the gNB may not perform LBT before the DL transmission, is this case allowed based on the Proposal 2.10.2-1a? |
| Nokia, NSB | We are a bit puzzled about some of the comments. To our understanding, the reason for indicating the LBT mode in the first place is to assist the UE by letting it know if some signals / channels are guaranteed to be transmitted, or may be absent because of an LBT failure. |
| ZTE, Sanechips | Compare with the proposal 2.10.2-1a, we prefer the version raised by Ericssion, that is, add a note to make the behavior of gNB side more clearer. |

## Short Control Signaling and Contention Exempt Transmission

Agreement: - Contention Exempt Short Control Signaling rules can be applicable to the transmission of SS/PBCH. - FFS: What are the other DL signals and channels that can be multiplexed with SS/PBCH transmission under Contention Exempt Short Control Signaling rule - FFS: Whether this can be applied to all supported SCS or specific SCS. - FFS: Extension to discovery burst if it is defined including signals other than SS/PBCH - Note: Restriction for short control signalling transmissions apply (10% over any 100ms interval) - FFS: Other DL signals/channels can be transmitted with Contention Exempt Short Control Signaling rule, such as PDCCH, broadcast PDSCH, PDSCH without user plain data, CSI-RS, PRS, etc Agreement: For contention exemption short control signalling based DL transmission of SS/PBCH, further consider if the following signals/channels can be multiplexed with SS/PBCH block transmission. - RMSI PDCCH and RMSI PDSCH - Other broadcast PDSCH - PDSCH without user-plane data - PDCCH - CSI-RS - PRS - Other signals/channels contained in Discovery Burst (i.e., exemption applies to Discovery Burst) Note: Total exempted signals/channels should meet the restriction of 10% over any 100ms interval. FFS: If contention exemption short control signalling based DL transmission is allowed when not multiplexed with SS/PBCH block transmission.

Agreement: - Contention Exempt Short Control Signaling rules apply to the transmission of msg1 for the 4 step RACH and MsgA for the 2-step RACH for all supported SCS. - Note restriction for short control signalling transmissions apply (10% over any 100ms intervals) - Alt 1: The 10% over any 100ms interval restriction is applicable to all available msg1/msgA resources configured (not limited to the resources actually used) in a cell - Alt 2: The 10% over any 100ms interval restriction is applicable to the msg1/msgA transmission from one UE perspective - FFS: Other UL signals/channels can be transmitted with Contention Exempt Short Control Signaling rule, such as msg3, SRS, PUCCH, PUSCH without user plain data, etc

| Company | Key Proposals/Observations/Positions |
| --- | --- |
| Huawei HiSilicon | Proposal 30: In regions where channel sensing is mandated and short control ignaling exemption is allowed by regulations, only channels/signals included in the DB as defined for Rel-16 NR-U should be supported for contention exemption short control signaling based DL transmission. Proposal 31: In regions where channel sensing is mandated and short control ignaling exemption is allowed by regulations, contention-exempt short control signaling rules apply to the transmission of msg1 for 4 step RACH and msgA for 2-step RACH such that the 10% over any 100ms interval restriction is applicable to all available msg1/msgA resources configured in a cell (Alt 1). Proposal 32: In regions where channel sensing is mandated and short control ignaling exemption is allowed by regulations, contention-exempt short control signaling based transmission is not supported for UL signals/channels other than msg1/msgA. Proposal 33: Modify the earlier agreements in RAN1 105-e and RAN1 104bis-e as follows: Agreement:  In regions where channel sensing is mandated and short control ignaling exemption is allowed by regulation, Contention Exempt Short Control Signaling rules can be applicable to the transmission of SS/PBCH. - FFS: What are the other DL signals and channels that can be multiplexed with SS/PBCH transmission under Contention Exempt Short Control Signaling rule - FFS: Whether this can be applied to all supported SCS or specific SCS. - FFS: Extension to discovery burst if it is defined including signals other than SS/PBCH - Note: Restriction for short control ignaling transmissions apply (10% over any 100ms interval)  FFS: Other DL signals/channels can be transmitted with Contention Exempt Short Control Signaling rule, such as PDCCH, broadcast PDSCH, PDSCH without user plain data, CSI-RS, PRS, etc Agreement:  In regions where channel sensing is mandated and short control ignaling exemption is allowed by regulation, Contention Exempt Short Control Signaling rules apply to the transmission of msg1 for the 4 step RACH and MsgA for the 2-step RACH for all supported SCS. - Note restriction for short control ignaling transmissions apply (10% over any 100ms intervals) - Alt 1: The 10% over any 100ms interval restriction is applicable to all available msg1/msgA resources configured (not limited to the resources actually used) in a cell - Alt 2: The 10% over any 100ms interval restriction is applicable to the msg1/msgA transmission from one UE perspective  FFS: Other UL signals/channels can be transmitted with Contention Exempt Short Control Signaling rule, such as msg3, SRS, PUCCH, PUSCH without user plain data, etc |
| FUTUREWEI | Proposal 11: The 10% over any 100ms interval restriction is applicable to all available msg1/msgA resources configured (not limited to the resources actually used) in a cell.  Enforceability of 10% limit on other allowed candidates for contention exempt short control signalling is required. |
| Vivo | Proposal 15: The contention exempt short control signaling can be extended to discovery burst with duration at most 1ms. Proposal 16: The contention exempt short control signaling based SS/PBCH can be multiplexed with RMSI PDCCH, RMSI PDSCH and CSI-RS. Proposal 17: The 10% over any 100ms interval restriction is applicable to the msg1/msgA transmission from one UE perspective. |
| ZTE Sanechips | Observation 2: Other channel/signal is allowed to be multiplexed with a channel/signal that has been regarded as Short Control Signalling only if their total transmission time does not exceed 10ms limitation within 100ms observation period. Observation 3: If channel(s)/signal(s) is not regarded as Short Control Signalling and not multiplexed with any Short Control Signalling, it is a natural way that such channel(s)/signal(s) cannot apply Contention Exempt Short Control Signaling rule. Observation 4:  For 120 kHz SCS SS/PBCH, transmitted 64 SS/PBCH with 20ms SS/PBCH period exceeds 10ms limitation within a 100ms observation period required for short control ignaling.  For larger SCS (e.g., 240/480/960kHz) SS/PBCH, transmitted 64 SS/PBCH with 20ms SS/PBCH period does not exceed 10ms limitation within a 100ms observation period required for short control signalling. Observation 5: As long as total time corresponding to all available UL resources that be used to transmit Short Control Signalling (e.g., Msg1/Msg A/potential Msg 3 ) meets 10ms limitation within a 100ms observation period, Contention Exempt Short Control Signaling rule can be applied. Observation 6: Once the transmission of DL/UL channels/signals considered as Short Control Signalling exceeds 10ms limitation, it is a nature way to switch from No LBT mode to LBT mode. Observation 7: For the case of the transmission of DL/UL channels/signals considered as Short Control Signalling is in a COT ignalin by gNB or UE and LBT is performed before Short Control Signalling transmission, it is suggested that such transmission should not be counted into 10ms limitation within the 100ms observation period. |
| Nokia Nokia Shanghai Bell | Observation 3: EN 302 567, v2.2.0 allows for Short Control Signalling transmissions for up to 10% of time within an observation period of 100 ms. Proposal 7: NR-U design for 60 GHz bands supports transmission of the following DL and UL control and management signals as short control ignaling without LBT:  Downlink: SS/PBCH blocks (already agreed), PDCCH, CSI-RS and other reference signals, e.g., for beam management, SIBs, Paging  Uplink: HARQ-ACK feedback on either PUCCH or PUSCH, Scheduling Request, CSI feedback, Sounding RS, e.g., for beam management, RACH related transmissions Proposal 8: For the UL transmissions, the 10% short control signalling allowance is shared by all the Ues in the cell. Observation 5: Depending on SSB sub-carrier spacings and SSB periodicity, only sub-set of total SSBs can be covered by short control ignaling exemption Proposal 9: It is possible to apply SCSe to one part of actually transmitted SSBs and LBT procedure for other/rest of the SSBs. Proposal 10: Use of short control signal contention exemption and use of LBT for SSBs is predetermined or semi-statically determined, distributing the channel access uncertainty over the SSBs. |
| CATT | Proposal 13: The 10% over any 100ms interval restriction should be applicable to all Contention Exempt Short Control Signals from cell perspective. Proposal 14: In order to meet 10ms limit over 100ms, it should be supported to apply the Contention Exempt Short Signaling rules to sub-set of PRACH slots for msg1/msg3. Proposal 15: For UL signal, the Contention Exempt Short Control Signaling rules can be applied to the PUCCH and PUSCH without user-plane data. Proposal 16: The Contention Exempt Short Control Signaling can be applied to any signaling without user-plane data multiplexed with SS/PBCH block transmission. Observation 1: When the periodicity of SS/PBCH block is 20 msec and the number of SSB beams is 64, the total duration of SSB transmission is more than 10ms over 100ms. Proposal 17: In order to meet 10ms limit over 100ms, the Contention Exempt Short Signaling rules may be applied to sub-set of SSB beams for 120 kHz SCS when the up to 64 SSBs transmission is supported. |
| Sony | Proposal 1: Contention exempt short control ignaling should be adopted for transmission of RMSI PDCCH, RMSI PDSCH, and/or CSI-RS contained in Discovery Burst. |
| Ericsson | Observation 6 In HS EN 302 567, SCS transmissions have a duty cycle requirement but no limitations on the number of SCS transmissions within the observation period. Proposal 6 Support extending the Short control signaling transmissions exemption to Discovery Burst as defined in Rel-16. Proposal 7 Support Alt2 in which the short control ignaling transmissions requirement of 10ms over 100ms duration is applicable to control and management transmissions from a single UE perspective Proposal 8 Consistent with EN 302 567, a node can access the channel without LBT for control signal/channel transmissions, the total duration of which shall not exceed 10 ms within an observation period of 100ms. The following signals/channels shall be classified as short control signaling transmissions: 1 msg3 for the 4 step RACH and MsgB for the 2-step RACH |
| Intel | Proposal 25: The gNB indicates through a cell-specific RRC parameter whether the short signal exemption should be applied or not. Proposal 26: It is left up to gNB to decide and apply SSE to any signals/channels which are additionally multiplexed with SS/PBCH, as long as when it does the 10% duty cycle over a 100ms observation period is met. Proposal 27: Type0-PDCCH for 960kHz should at least be ignaling as part of short control signal exemption. Proposal 28: It is up to the gNB to decide and apply SSE to the discovery burst, as long as when it does the 10% duty cycle over a 100ms observation period is met. Observation 1: The contention exempt control signaling rules is interpreted as if the 10% over any observation period of 100ms is applicable per device. Proposal 29: The 10% over any observation period of 100ms is applicable to the msg1/msgA transmission from one UE perspective. Observation 2: For 120 kHz, 480kHz, and 960 kHz PRACH transmission, UE does not exceed total transmission duration of 10 msec for PRACH within a 100 msec observation period. Proposal 30: Contention Exempt Short Control Signaling rules apply to the transmission of msg3 for the 4-step RACH for all supported SCS. Proposal 31: It is up to the UE to decide and apply SSE to SRS, PUSCH without user plain data, and PUCCH, as long as when it does the 10% duty cycle over a 100ms observation period is met. Proposal 32: The gNB indicates through a cell-specific RRC parameter which specific channels/signals could be qualifies as short control signaling. |
| Samsung | Proposal 4: For “short control signalling”:  support discovery burst to be used as short control signaling for DL;  allow components in discovery burst to be used as short control signaling even when not multiplexing with SS/PBCH block;  support limitation on the duty cycle to use “short control signalling”, wherein the duty cycle are defined from the perspective of a node (e.g. per cell for DL and per UE for UL). |
| Apple | Proposal 8: Enable UE specific RRC signaling to indicate which DL/UL channel/signals other than SSB and RACH can be transmitted with contention exempt short control signaling rule. |
| LG Electronics | Observation #1: The interpretation of regulation for 10% over any 100ms interval restitution from one UE perspective (Alt-2) is likely to cause coexistence problems with the incumbent system operating in the same band. Proposal #20: Whether a short control signing rule is applicable or not to the configured msg1/msgA resources can be explicitly indicated by the gNB or can be implicitly determined by the UE. |
| NTT DOCOMO INC. | Proposal 7: Contention Exempt Short Control Signaling rules can be applicable irrespective of SCS Proposal 8: Support Alt 2 on the interpretation of Contention Exempt Short Control Signaling rules, i.e., the 10% over any 100ms interval restriction is applicable to the msg1/msgA transmission from one UE perspective |
| Qualcomm Incorporated | Proposal 29: Support Alt 2. Contention Exempt Short Control Signaling rules apply to the transmission of msg1 for the 4 step RACH and msgA for the 2-step RACH for all supported SCS. The 10% over any 100ms interval restriction is applicable from the perspective of the UE in accordance with per device requirement set by regulation. Proposal 30: SRS should be included towards contention exempt transmissions. Proposal 31: PUCCH should be included towards contention exempt transmissions. Proposal 32: PUSCH without user plane data, such as CSI or Ack/Knack, and msg3 should be included towards contention exempt transmissions. Proposal 33: Under the restrictions of duty cycle for short control signaling, allow SS/PBCH, PDCCH, CSI-RS and PRS for contention exempt transmission. |
| OPPO | Proposal 13: PUCCH carrying HARQ-ACK information belong to short control signaling. Proposal 14: For msg3, SRS, and PUSCH without user plain data, what the criterion is to judge if a channel is qualified to be contention exemption short control ignaling should rather be discussed. Proposal 15: Restriction for short control ignaling transmissions is applicable to all available msg1/msgA resources configured in a cell. |
| Xiaomi | Proposal 5: Support Alt 1, that is 10% over any 100ms interval restriction is applied to all available msg1/msgA resources configured. Proposal 6: Support to apply contention exempt short control signalling transmission to other UL channels by gNB indication or configuration. |
| AsusTek | Observation 1: In addition to the current agreed SSB/Msg1/MsgA, only a limited set of channel or signal applies short control signaling to simplify the design and avoid too much impact caused by short control signaling exemption. Observation 2: Discovery burst occupies a similar duty cycle as SSB and apply short control signaling exemption to discovery curst together with SSB would simply the design. Proposal 1: In addition to the agreed channel/signal, only discovery burst applies short control signaling exemption. Observation 3: 10% limitation is too restricted for all possible PRACH resources and could induce undesired delay. Observation 4: Handling the case actual transmitted Msg1/MsgA opportunities from a UE exceeding 10% limit is not required. Proposal 2: 10% limitation over 100 ms applies to actual transmitted Msg1/MsgA opportunities from a UE and the case of actual transmitted Msg1/MsgA opportunities from a UE exceeding such limit is not handled from specification perspective. |

### First Round Discussion

Proposal 2.11.1-1 (closed)

In regions where channel sensing is mandated and short control signalling exemption is allowed by regulations, contention Exempt Short Control Signaling rules can be applicable to the transmission of discovery burst (as defined in 37.213 6.0)

- Note: Restriction for short control signalling transmissions apply (10% over any 100ms interval)

Moderator Note: the discovery burst as define in 4.0 of 37.213 includes at least a SS/PBCH block and may be multiplexed with PDCCH scheduling PDSCH for SIB1, and PDSCH carries SIB1, and/or NZP-CSI-RS

Please provide your view:

| Company | View |
| --- | --- |
| Nokia, NSB | We support the proposal. Additionally, we think at least PTRS should be included. Moderator: PTRS for PDSCH carries SIB1 is similar to DMRS for PDSCH carries SIB1 and is considered as part of the PDSCH |
| Ericsson | We support the proposal. |
| Lenovo, Motorola Mobility | Support |
| Intel | We support this proposal. |
| Apple | We do not support the proposal. Current discovery burst in 37.213 says “at least” an SSB. There is no limitation on user plane data if the burst contains at least one SSB. If we would like to apply short control signaling to discovery burst, we need revise the discovery burst definition to transmissions only including SSB, PDCCH scheduling PDSCH for SIB1, and SIB1 and/or NZP CSI-RS. Copy of discovery burst transmission from 37.213: “Transmission(s) initiated by a gNB that includes at least an SS/PBCH block consisting of a primary synchronization signal (PSS), secondary synchronization signal (SSS), physical broadcast channel (PBCH) with associated demodulation reference signal (DM-RS) and may also include CORESET for PDCCH scheduling PDSCH with SIB1, and PDSCH carrying SIB1 and/or non-zero power CSI reference signals (CSI-RS).” Moderator: Not sure what you mean. I think the spec text is quite clear and defines the discovery burst at least has SSB, but may also contain SIB1 PDCCH/PDSCH, and NZP-CSI-RS. When other non-unicast signal is transmitted with DRS, the spec just say they can be multiplexed together with DRS with Cat 2 LBT. In my understanding, DRS is only the SSB, SIB1 PDCCH/PDSCH and NZP-CSI-RS part of that transmission, no matter if anything else is multiplexed with it or not |
| Xiaomi | Support the proposal |
| LG Electronics | We support this proposal with removal of the first sub-bullet. |
| ASUSTeK | We support this proposal and agree with LG to remove the note in the first sub-bullet. Moderator: Changed to moderator note, not to be included in the agreement |
| vivo | We support the proposal. |
| NEC | We support the proposal |
| Transsion | We support the proposal. |
| Sony | We support the proposal |
| ZTE, Sanechips | We support the proposal |
| DOCOMO | We support the proposal |
| CATT | We support the proposal. |
| Samsung | We support the proposal. |
| Huawei, HiSilicon | We support the proposal with the following modification: Proposal 2.11.1-1 (modified): In regions where channel sensing is mandated and short control signalling exemption is allowed by regulations, contention Exempt Short Control Signaling rules can be applicable to the transmission of discovery burst (as defined in 37.213 6.0) - Note: Restriction for short control signalling transmissions apply (10% over any 100ms interval) The added line at the beginning is necessary as the whole concept of short control signalling exemption is adopted from ETSI BRAN 302 567. Also, it makes the proposal aligned with the current drafCR of 37.213 (R1-2112428): ### 4.4.5 Exempted transmissions from sensing In regions where channel sensing is mandated and short control signalling exemption is allowed by regulation, a gNB/UE may transmit the following transmission(s) on a channel without sensing the channel if the condition is satisfied that the corresponding transmission(s) on the channel in total do not occupy the channel more than 10ms over any 100ms interval: - Transmission(s) of SSB by a gNB - Transmission(s) of the first message in a random access procedure by a UE. Moderator: Good catch. Updated |
| Apple | We support transmission of SSB, PDCCH scheduling SIB1, SIB1 and NZP-CSI-RS as short control signaling. The current 37.213 specification needs updated definition for DL burst, UL burst, and DRS which is one type of DL burst, since 16us is not applied here. A DL transmission burst is defined as a set of transmissions from a gNB without any gaps. A UL transmission burst is defined as a set of transmissions from a UE without any gaps. A discovery burst refers to a DL transmission burst including a set of signal(s) and/or channel(s) confined within a window and associated with a duty cycle. |
| Futurewei | We support the proposal. |

Proposal 2.11.1-1a:

In regions where channel sensing is required and short control signalling exemption is allowed by regulations, contention Exempt Short Control Signaling rules can be applicable to the transmission of discovery burst (as defined in 37.213 6.0)

- Note: Restriction for short control signalling transmissions apply (10% over any 100ms interval)

Please provide your view:

| Company | View |
| --- | --- |
| Nokia, NSB | We support the proposal |
| ZTE, Sanechips | We support this proposal. |
| Ericsson | We support this proposal. |
| Vivo | We support the proposal |
| Intel | We support the proposal |
| Samsung | We support the proposal |
| ASUSTeK | We support the proposal |

In 37.213 for Rel.16 NR-U, it is also possible to use Cat 2 LBT to transmit discovery burst multiplexed with non-unicast transmission. The following proposal is to extend that to FR2-2 and use short control signaling rule.

Proposal 2.11.1-2:

Contention Exempt Short Control Signaling rules can be applicable to the transmission of discovery burst multiplexed with non-unicast information.

- Note: Restriction for short control signalling transmissions apply (10% over any 100ms interval)

Please provide your view:

| Company | View |
| --- | --- |
| Nokia, NSB | We support the proposal |
| Ericsson | We support the proposal. |
| Lenovo, Motorola Mobility | Support |
| Intel | We support this proposal. |
| Apple | Do not support. Short control signaling is limited to management and control based on definition in EN 302 567. Non-unicast information is too board. It is better to list out which non-unicast information. #### “4.2.6.2 Definition Short Control Signalling Transmissions are transmissions used by the equipment to send management and control frames without sensing the channel for the presence of other signals.” Moderator: This “non-unicast information” is the wording in the current 37.213 on what can be multiplexed with DRS under NR-U Cat 2 LBT rule. I believe if we can accept such wording for NR-U, we can do it here as well. |
| Xiaomi | Support the proposal |
| ASUSTeK | Not support. We think apply contention exemption rule to legacy discovery burst is sufficient as proposed in Proposal 2.11.1-1. It is unclear what signal is included by this proposal in addition and its benefits. Moderator: Same reply to Apple. Additionally, I believe the reason we allow non-unicast information to be multiplexed with DRS is, we are already occupying the channel with DRS. Either we don’t use all the resources, say leave a few RBs not occupied or leave a few symbols not filled, and take some additional resource to send the non-unicast information, or we use the extra resources in the DRS transmission, it is obviously more efficient to do the latter, and same the other resource for other RAT. The non-unicast information rule was introduced to satisfy the regulation (short CONTROL signaling). |
| Vivo | We share the same view as Apple. We think it is better to identify the feasible non-unicast information. |
| NEC | We support the proposal |
| Transsion | We support the proposal |
| Sony | We support the proposal |
| ZTE, Sanechips | We share the same view with Apple. It would be better to clarify what “non-unicast information” contains. |
| DOCOMO | Support. Same view as FL’s clarifications above. |
| CATT | We support the proposal |
| Samsung | We support the proposal. |
| Huawei, HiSilicon | We do not support the proposal. We understand that, in Rel-16, CAT2 LBT may be performed prior to DB multiplexed with non-unicast information as long as the DB duration <1ms with duty cycle less than 1/20. However, we don’t see how this can be a reason to use a complete LBT exemption prior to DB multiplexed with non-unicast information in Rel-17. Such multiplexing of non-unicast information in 60 GHz is not motivated by regulations. We are not supportive of the idea of “selectively” drawing parallels between Rel-16 NR-U and 60 GHz NR-U and re-interpret and carry over some of the features from Rel-16 NR-U while leave some other features out while neither of these set of features are supported by regulations for 60 GHz. |
| Apple | FR2-2 short control signaling does not require LBT. And there is no limitation of 1ms either. We do not see why NR-U support this then 60GHz should support. |
| Ericsson | We support this proposal. This is supported by regulations in our opinion. Furthermore, the definition of short control signalling transmissions is broad and includes both unicast information like ACK/NACKs and non-unicast information like beacon frames that are broadcast and frames for beamforming that may be multi-cast. Would this satisfy Huawei’s and Apple’s concerns? This is elaborated in 5.3.8.2, step 4 as follows Apart from transmission of the frames for short control ignaling (such as, for example, ACK/NACK signals, beacon frames, other time synchronization frames and frames for beamforming) no frame shall be initiated. |
| TCL | We support the proposal. The DRS is kinds of Short Control Signalings. |

The following proposal identifies DL transmissions that are not part of the discovery burst and not multiplexed with SS/PBCH and proposes that they are considered for contention exemption under short control signaling.

Proposal 2.11.1-3:

The contention exemption for short control signaling applies to following DL transmission bursts not multiplexed with SS/PBCH block transmission,It  but does not contain unicast information. The transmission burst may contain

- PDSCH without user plane data

- PDCCH

- CSI-RS

- PRS

Note: Restriction for short control signalling transmissions apply (10% over any 100ms interval)

Please provide your view:

| Company | View |
| --- | --- |
| Nokia, NSB | We support the proposal |
| Ericsson | We support the proposal. |
| Lenovo, Motorola Mobility | Support |
| Intel | We support this proposal. |
| Apple | Support this proposal |
| Xiaomi | Support the proposal |
| LG Electronics | Support |
| ASUSTeK | Not support. We think apply contention exemption rule to legacy discovery burst is sufficient. It is unclear how to manage the restriction of 10% over 100ms and its corresponding impact if contention exemption rule is applicable for so many different cases and signals. |
| Vivo | We support the proposal. |
| NEC | We support the proposal |
| Transsion | We support the proposal. |
| Sony | We support the proposal |
| ZTE, Sanechips | We support the proposal |
| DOCOMO | Support |
| CATT | We support the proposal |
| Huawei, HiSilicon | Not support. Exempting DB is sufficient. Further exempting above signals/channels is not supported by regulations. |

Contention exemption for UL transmissions in RACH procedure has two alternatives roughly equally supported as follows upto RAN1-106-bis-e discussion.

Proposal 2.11.1-4: (closed)

Contention Exempt Short Control Signaling rules apply to the transmission of msg1 for the 4 step RACH and MsgA for the 2-step RACH for all supported SCS. Restriction for short control signalling transmissions apply (10% over any 100ms intervals). Down-select from the following alternatives

- Alt 1: The 10% over any 100ms interval restriction is applicable to all available msg1/msgA resources configured (not limited to the resources actually used) in a cell

  - Support: Oppo, HW, LG, Nokia (though regulation allows Alt 2), ZTE, Futurewei, CATT, Spreadtrum, Xiaomi, Transsion, TCL

- Alt 2: The 10% over any 100ms interval restriction is applicable to the msg1/msgA transmission from one UE perspective

  - Support: vivo, Charter, Intel, Lenovo, DCM, InterDigital, Ericsson, Samsung, Convida, Apple, Nokia, Qualcomm, Mediatek, WILUS, ASUSTeK

Please provide your view if modified from the above:

| Company | View |
| --- | --- |
| Nokia, NSB | Our views are correctly captured. |
| Ericsson | We support the proposal, and our views are correctly captured. |
| Intel | Our view has been correctly captured. |
| Apple | Our view has been correctly captured. |
| ASUSTeK | Our view is added above. |
| Vivo | Our view has been correctly captured. |
| Transsion | Our view has been correctly captured. |
| CATT | Our view has been correctly captured. |
| InterDigital | Our view is correctly captured. |
| Huawei, HiSilicon | Our views are correctly captured. |

On contention exemption for short control signalling for non-RACH UL transmissions

Proposal 2.11.1-5: (closed)

For contention exemption short control signalling based UL transmission consider the following signals and channels.

- Any transmission on PUCCH

  - Support: OPPO  (HARQ A/N only), CATT , Nokia, Qualcomm, Intel, Lenovo, Motorola Mobility, Ericsson, Mediatek, Apple, WILUS, DCM,Xiaomi, ZTE, Sanechips

- SRS

  - Support: Qualcomm, Intel, Ericsson, Apple, Nokia, WILUS, TCL, DCM. CATT, Xiaomi

  - Oppose:  OPPO

- PUSCH not carrying user plane data

  - HARQ A/N on PUSCH

    - Support: CATT, Nokia,  Qualcomm, Intel, Lenovo, Motorola Mobility, Ericsson, Apple, WILUS, DCM, ,Xiaomi, ZTE, Sanechips

    - Oppose: OPPO

  - CSI reporting on PUSCH

    - Support: CATT, Nokia , Qualcomm, Intel, Lenovo, Motorola Mobility, Ericsson, Apple, WILUS, DCM, Xiaomi

    - Oppose: OPPO

  - Msg 3

    - Support: CATT, Ericsson, Nokia, Qualcomm, Intel. Lenovo, Motorola Mobility, Ericsson, Apple, WILUS, TCL, DCM,Xiaomi, ZTE, Sanechips, Samsung

    - Oppose: Oppo

- No other Contention Exempt UL transmission should be permitted: Huawei, InterDigital, ASUSTeK

- Note: Restriction for short control signalling transmissions apply (10% over any 100ms interval)

Moderator: Seems that we don’t have consensus. Would recommend to have a conclusion on not having additional UL transmission with short control signaling other than msg1 and msg3

Please provide your view if modified from the above:

| Company | View |
| --- | --- |
| Nokia, NSB | Our views are correctly captured. |
| Ericsson | We support the proposal and our views are accurately captured. |
| Intel | Our view has been correctly captured. |
| Apple | Our view has been correctly captured. |
| LG Electronics | This issue should be discussed after two alternatives in Proposal 2.11.1-4 is resolved |
| ASUSTeK | Our view is added above. |
| ZTE, Sanechips | Add our position in Any transmission on PUCCH, HARQ A/N on PUSCH, msg 3 |
| CATT | Our view has been correctly captured. |
| Samsung | At least msg3 should be supported. Added our position in the list. |
| InterDigital | Our view is correctly captured |
| Huawei, HiSilicon | Our views are correctly captured. |

Proposal 2.11.1-6:

For contention exemption short control signalling based UL transmission, further introduce RRC configuration to allow gNB to control which channels can be transmitted with contention exemption.

- Support: Intel, Xiaomi, ZTE, Qualcomm, Apple, Nokia, CATT, TCL, Samsung, Huawei/HiSilicon (Only msg1/MsgA may use exemption)

- Not support: Lenovo, vivo, Ericsson, InterDigital, Mediatek, Transsion, WILUS, TCL

- Deprioritize: DCM

Please provide your view if modified from the above:

| Company | View |
| --- | --- |
| Nokia, NSB | We support the proposal. Furthermore, similar signalling is also beneficial for the DL to inform Ues which signals/channels are subject to LBT and which ones are always transmitted. |
| Ericsson | We do not see a need to introduce RRC configuration for short control signalling transmissions. |
| Intel | Our view has been correctly captured, and we support the proposal. This may also serve as a way to signal the UE whether the contention exemption should be used or not, given that this is applicable only to CEPT/EU regions, and this may not need to be used in other regions even when LBT may be used as an adaptivity method. |
| Apple | We support the proposal. We share the same view as Nokia. We think it is more important to signal which DL signal is transmitted as short control signaling, particularly the RSs used for RLM/BFD. It simplifies UE so when UE does not detect the RS, whether this is due to bad link or due to LBT. |
| Xiaomi | Support the Proposal. If there are multiple channels can all apply UL contention exemption short control signalling transmission, and the sum of which exceed the 10% restriction, then gNB has to let UE know exactly which channels to apply |
| LG Electronics | So far, we agreed to support PRACH/msgA for short control signaling exemption. Thus, we may need to introduce a RRC parameter in SIB1 to inform UE whether LBT needs to be performed for PRACH/msgA transmission. |
| ASUSTeK | Currently how many channel/signal is eligible to contention exemption rule is not finalized yet (pending on above proposal). Though we are open to this proposal, it should be discussed after the applicable channel/signal is settled down. |
| Vivo | Our view has been correctly captured. We think it is up to UE to transmit the UL signals based on short control signalling or with LBT. When the UL signals based on short control signalling reaches the 10% limits over 100ms, UE will switch to the normal channel access mechanism. |
| Transsion | We think this proposal is related to discussion 2.11.1-5, if no UL channels/signals except msg1/MsgA are introduced as short control signaling, then this RRC signaling is not needed. |
| CATT | We support the proposal. |
| Huawei, Hisilicon | The use of exemption at the UE side should be under the control of gNB. However, since only msg1/MsgA are exempted, proposal should be modified as below: Proposal 2.11.1-6 (modified) For contention exemption short control signalling based UL transmission, further introduce RRC configuration to allow gNB to control which channels can be transmitted with contention exemption. - Only msg1/MsgA may use contention exemption short control signalling based UL transmission. |
| Huawei, HiSilicon | We do not support the proposal. If it is left to each individual UE to use CESCS for msg1 for the 4 step RACH and MsgA for the 2-step RACH, then the total time resources at which at least one UE within the cell transmits msg1/ MsgA can easily far exceed the 10% occupancy time for short control signaling exemption. In our view, this is a misuse of the exemption that is introduced in regulations for “short control signaling”. |

### Second Round Discussion

Moderator would recommend taking simple majority to go with Alt 2 of 2.11.1-4

Proposal 2.11.2-1:

Contention Exempt Short Control Signaling rules apply to the transmission of msg1 for the 4 step RACH and MsgA for the 2-step RACH for all supported SCS. Restriction for short control signalling transmissions apply (10% over any 100ms intervals).

- The 10% over any 100ms interval restriction is applicable to the msg1/msgA transmission from one UE perspective

Please provide your view:

| Company | View |
| --- | --- |
| Lenovo, Motorola Mobility | Support |
| Intel | We support the proposal |
| LG Electronics | We do not support this proposal. Still we believe the requirement of 10% over any 100ms interval should be applied to all available msg1/msgA resources configured (not limited to the resources actually used) in a cell, not from each UE perspective, for coexistence with other nodes. Moderator: Combined with proposed conclusion 2.11.2-2, the chance to have combined transmission exceeds 10% is very slim. If there is no practical difference, I would recommend to go with more regulation compliant version |
| Apple | Support |
| DOCOMO | We support the proposal. To interpret the rule as “per cell” needs further specification impact, which is unnecessary as per the regulation though. |
| vivo | We support the proposal. |
| Ericsson | We support the proposal. Regulations provide this exemption and IEEE 802.11ad/ay avails it too. We are not sure why 3GPP also shouldn’t make use of this exemption. |
| ZTE, Sanechips | We disagree this proposal and to avoid misuse of this rule and coexist with other nodes, we support restriction for SCS is for all available msg1/msgA resources configured in a cell |
| TCL | We support it. |
| ASUSTeK | Support. Also agree with the Moderator that if contention exemption applies to Msg1/MsgA only, it’s fairly unlikely the 10% restriction would be exceeded. |
| NEC | We support the proposal. |
| CATT | We only support the main bullet that the Contention Exempt Short Control Signaling rules apply to the transmission of msg1 for the 4 step RACH and MsgA for the 2-step RACH for all supported SCS. For the sub-bullet, we share same view as ZTE. Consider the worst case, if there are several UEs, i.e, 3 UEs, in the same cell and the duration used for transmission of msg1/msgA of each UE accounts for different 10% over 100ms interval, it means that the total duration transmission applied Contention Exempt Short Control Signaling rules in one cell will account for at least 30% over 100ms interval. This is unfair to other nodes who comply with the 10% over 100ms interval restriction in the same unlicensed band. Therefore, we still think the 10% over any 100ms interval restriction for Contention Exempt Short Control Signaling should from one cell perspective |
| Nokia, NSB | For the cell or UE-specific definition of 10% allowance, we feel cell specific definition may be more fair. That said, we are ok with the UE specific definition too. |

Proposed conclusion 2.11.2-2:

In Rel.17, there is no consensus to apply contention exemption short control signalling to UL transmissions other than msg1 and msgA.

Please provide your view:

| Company | View |
| --- | --- |
| vivo | OK |
| Lenovo, Motorola Mobility | Ok |
| Intel | OK |
| LG Electronics | OK |
| Apple | OK |
| DOCOMO | OK |
| Ericsson | For the sake of progress, we are ok to support this conclusion. |
| ZTE, Sanechips | OK |
| Samsung | OK |
| Huawei, HiSilicon | OK |
| Convida Wireless | We are generally ok with the proposed conclusion. |
| TCL | OK |
| ASUSTeK | OK |
| NEC | OK |
| Transsion | OK |
| CATT | OK |
| Nokia, NSB | Although not our preference, we can agree with the conclusion. |

## CG PUSCH, Long Term Sensing, Interference Mitigation, ATPC, Other aspects

| Company | Key Proposals/Observations/Positions |
| --- | --- |
| Nokia Nokia Shanghai Bell | Proposal 19: CG PUSCH configuration shall include indication of whether the CG PUSCH configuration is used inside or outside of a gNB initiated COT, or both. Proposal 20: Study the benefits of sharing the ED measurements results at gNB to the UEs. Proposal 21: Support for Cat-3 LBT is UE capability. |
| Lenovo Motorola Mobility | Observation 2: For NR operation in unlicensed bands between 52.6 GHz and 71 GHz, long-term channel sensing could be useful for both LBT and no-LBT based channel access mechanism: - For LBT based channel access mechanism, long-term sensing at the UE could be utilized for receiver assistance LBT at the gNB - For no LBT based channel access mechanisms, long-term sensing could provide interference statistics in terms of potential interference from WiFi as well as interference from other NR operators Proposal 6: For NR unlicensed bands between 52.6 GHz and 71 GHz, with directional LBT based channel access mechanism, for UL transmissions on CG resources, time-based autonomous switching of UL Tx beam should be supported, where the switching can be based on a timer within which the UE is expected to receiver HARQ-ACK feedback Proposal 12: If a UE is going to transmit a set of consecutive PUSCH transmissions including both dynamically scheduled PUSCH transmissions and CG-PUSCH transmissions, the UE can select the latest indicated UL Tx beam to transmit the consecutive UL transmissions Proposal 13: For NR operation in unlicensed bands between 52.6 GHz and 71 GHz, then following potential enhancements related to periodic transmissions of RS such as P-TRS should be specified to deal with LBT failure: - Termination of periodic RS transmission on beams where consecutive LBT failures are encountered - Dynamic switching of the QCL assumption (beams) for periodic RS transmission where consecutive LBT failures are encountered, where: o Multiple QCL assumptions (multiple beams) can be configured to the RS resource and beam switch can be triggered once the continuous number of LBT failures reach a certain threshold value Proposal 17: For NR operation in unlicensed bands between 52.6 GHz and 71 GHz, long term sensing should be supported for both LBT based and no-LBT based channel access mechanism to consider potential interference. Observation 3: Currently, there is no mechanism is support long-term sensing including interference measurements from WiFi or other NR operators at the UE and corresponding reporting. Proposal 19: For NR operation in unlicensed bands between 52.6 GHz and 71 GHz, potential enhancements related to periodic transmission of DRS such as SSB/PBCH/CORESET#0 are needed including: - performing directional LBT prior to the transmission of SSB according to the ssb-PositionsInBurst - directional LBT on multiple beams at the same time at the beginning of the DRS window - Cat 2 LBT (depending on the gap) before actual transmission |
| NEC | Proposal 12: Due to uncertain LBT duration for initiating a channel occupancy in mmWave band, the alignment between the transmission starting point with the symbol boundary should be considered. |
| Convida Wireless | Proposal 11: Enhancement of resource utilization and interference mitigation in 52.6 GHz to 71 GHz should be considered. Proposal 14: Enhancement of beam operation for unlicensed bands should be investigated to mitigate interference and optimize system performance due to hidden node for NR up to 71 GHz. |
| ITRI | Proposal 4: PDCCH monitoring enhancement for M-TRP operation should be supported for 60 GHz NR-U. Proposal 5: Configuring multiple SRIs for a CG transmission should be supported for 60 GHz NR-U. |

For this topic, the moderator does not see anything essential to be discussed for this meeting. If you see something worth discussion, please bring it up

Discussion 2.12-1 (open discussion)

| Company | Proposed discussion points |
| --- | --- |

# References

- R1-2110832, Channel access mechanism for 60 GHz unlicensed operation, Huawei, HiSilicon

- R1-2110877, Channel access for Beyond 52.6GHz, FUTUREWEI

- R1-2110894, Discussion on relation between sensing beam and transmission beam, GDCNI

- R1-2111003, Remaining issues on channel access mechanism for NR operation from 52.6GHz to 71 GHz, vivo

- R1-2111079, Discussion on the channel access for 52.6 to 71GHz, ZTE, Sanechips

- R1-2111093, Discussion on channel access mechanism for above 52.6GHz, Spreadtrum Communications

- R1-2111148, Considerations on channel access mechanism for NR  from 52.6GHz to 71 GHz, Fujitsu

- R1-2111200, Channel access mechanism, Nokia, Nokia Shanghai Bell

- R1-2111246, Channel access mechanism for up to 71GHz operation, CATT

- R1-2111312, Discussion on channel access mechanism, OPPO

- R1-2111389, Channel access mechanism for 60 GHz unlicensed spectrum, Sony

- R1-2111418, Channel access mechanisms for NR above 52 GHz, Charter Communications

- R1-2111469, Channel Access Mechanisms, Ericsson

- R1-2111488, Discussion on channel access mechanism for extending NR up to 71 GHz, Intel Corporation

- R1-2111566, Discussion on channel access mechanism for NR on 52.6-71 GHz, Xiaomi

- R1-2111645, Channel access mechanisms for NR from 52.6 GHz to 71GHz, Lenovo, Motorola Mobility

- R1-2111651, Some discussions on channel access mechanism enhancements for 52.6-71 GHz, CAICT

- R1-2111702, Discussion on channel access mechanism supporting NR from 52.6 to 71GHz, NEC

- R1-2111707, Channel access for multi-beam operation, Panasonic

- R1-2111729, Channel access mechanism for NR from 52.6 GHz to 71 GHz, Samsung

- R1-2111838, Discussion on channel access mechanisms, InterDigital, Inc.

- R1-2111866, Channel access mechanisms for unlicensed access above 52.6GHz, Apple

- R1-2112032, Channel Access Mechanism for Supporting NR from 52.6 GHz to 71 GHz, Convida Wireless

- R1-2112051, Channel access mechanism to support NR above 52.6 GHz, LG Electronics

- R1-2112101, Channel access mechanism for NR from 52.6 to 71 GHz, NTT DOCOMO, INC.

- R1-2112166, Remaining issue on channel access scheme for above 52.6GHz, ASUSTeK

- R1-2112172, Discussion on multi-beam operation, ITRI

- R1-2112208, Channel access mechanism for NR in 52.6 to 71GHz band, Qualcomm Incorporated

- R1-2112268, Channel access mechanism for NR in 60GHz unlicensed band operation, TCL Communication Ltd.

- R1-2112291, On the channel access mechanisms for 52.6-71 GHz NR operation, MediaTek Inc.

- R1-2112385, Remaining issues on channel access for NR from 52.6GHz to 71GHz, WILUS Inc.

- R1-2112460, Email discussion summary for channel access for beyond 52.6GHz band, Moderator (Qualcomm)