# 3GPP TSG RAN WG1 Meeting #107-e	R1-2112713

# e-Meeting, November 11– November 19, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1405 | rev | - | Current version: | V16.7.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on HARQ bundling for LTE-M MTB scheduling in FDD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator(ZTE), Sanechips, Ericsson, Lenovo, Motorola Mobility |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC5-Core |  |  |  |  |  | Date: |  |  | 2021-11-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For FDD, if the UE is configured with higher layer parameter harq-AckBundling in ce-PDSCH-MultiTB-Config and multiple TB are scheduled in the corresponding DCI, the number of TBs in a bundle and the number of bundles determined by b=0,1,⋯NTBM-1 is not aligned with that in Table 7.3-1 defined in TS36.213 clause 7.3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify the bundling rule in TS36.213 clause 10.2 for multiple TB scheduling in FDD - Replace the formula b=0,1,⋯NTBM-1 as b=0,1,…,M-1 - Add the explanation for M as ‘ is the number of TB bundles, and is determined according to clause 7.3’ - Remove the redundant description for NTB and bundling size M. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Uplink timing determined by the bundling rule in FDD for multiple TBs scheduling is not correct according to the agreement and the current spec in TS36.213 clause 7.3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

10.2	Uplink HARQ-ACK timing

# <Unchanged parts are omitted>

For FDD, if a BL/CE UE is configured with CEModeA, and if the UE is configured with higher layer parameter harq-AckBundling in ce-PDSCH-MultiTB-Config and multiple TB are scheduled in the corresponding DCI, the BL/CE UE shall upon detection of a PDSCH intended for the UE and for which an HARQ-ACK shall be provided, transmit the HARQ-ACK response using the same  derived according to Clause 10.1.2.1 in subframe(s) sb+ki with b=0,1,⋯NTBMM-1,  i =0,1, …, N-1, where

-	 is the number of TB bundles, and is determined according to clause 7.3;

-	if the UE is not configured with higher layer parameter interleaving in ce-PDSCH-MultiTB-Config and the UE is not in half-duplex FDD operation

-	s0=n0+4, sb=max {nb+4,  sb-1+Nb-1}, b≠0

-	otherwise

-	subframe s0=max n0+4,  nL+2, sb=max {nb+4,  sb-1+Nb-1}, b≠0

-	subframe nb is the last subframe in which the PDSCH containing TB bundle b is transmitted;

-	subframe nL is the last subframe in which the PDSCH is transmitted;

-	Nb denotes the number of consecutive subframes including non-BL/CE subframes where the PUCCH with HARQ ACK for TB bundle b with repetition number of N is transmitted;

and

# <Unchanged parts are omitted>