3GPP TSG-RAN WG1 Meeting #107-e              	R1-2112808

e-Meeting, November 11th – 19th, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0219 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on UE PDSCH processing time for DCI format 1_2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Ericsson), Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_L1enh_URLLC-Core |  |  |  |  |  | Date: |  |  | 2021-11-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In Rel-16, DCI format 1_2 has been introduced. However, the tables for PDSCH processing time have not been updated to account for the newly introduced DCI format 1_2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The tables for PDSCH processing time for PDSCH processing capability 1 and 2 are updated to include the RRC parameters for DMRS of DCI format 1_2. In RAN1#107-e, RAN1 achieved the following consensus in email discussion [107-e-NR-L1enh-URLLC-02]: - For updating the Table 5.3-1 and Table 5.3-2 in TS 38.214 to reflect DCI format 1_2 and associated DMRS configurations, the PDSCH processing time does not change dynamically according to the DCI formats 1-1 vs 1-2 scheduling the PDSCH. In the email discussion, companies also showed preference to indicate faster PDSCH processing time when DMRS parameters associated with DCI format 1_2 are configured with {= 'pos0'}, even if dmrs-DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB are not configured. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For UE configured with DCI format 1_2, the PDSCH processing time for PDSCH scheduled by DCI format 1_2 is not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 5.3	UE PDSCH processing procedure time

<Unchanged parts are omitted>

Table 5.3-1: PDSCH processing time for PDSCH processing capability 1

|  | PDSCH decoding time N1 [symbols] |  |
| --- | --- | --- |
|  | dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in dmrs-DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB if either higher layer parameter is configured, and in dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 if either higher layer parameter is configured | dmrs-AdditionalPosition ≠ 'pos0' in DMRS-DownlinkConfig in any of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB, dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2, dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2, or if none of the higher layer parameters is configured |
| 0 | 8 | N1,0 |
| 1 | 10 | 13 |
| 2 | 17 | 20 |
| 3 | 20 | 24 |

Table 5.3-2: PDSCH processing time for PDSCH processing capability 2

|  | PDSCH decoding time N1 [symbols] |
| --- | --- |
|  | dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in dmrs-DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB if either higher layer parameter is configured, and in dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 if either higher layer parameter is configured |
| 0 | 3 |
| 1 | 4.5 |
| 2 | 9 for frequency range 1 |

<Unchanged parts are omitted>