3GPP TSG-RAN WG1 Meeting #107-e	R1-2112865

# e-Meeting, November 11th – 19th, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0267 | rev | 1 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial corrections for TS 38.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core, NR_eMIMO-Core, NR_L1enh_URLLC-Core, NR_UE_pow_sav-Core, NR_2step_RACH-Core, NR_unlic-Core |  |  |  |  |  | Date: |  |  | 2021-11-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Correct an ‘are’ to ‘is’ in a sentence in clause 6. - Parameter name dl-DataToUL-ACK-ForDCIFormat1_2 in clause 9.1.2.1 is incorrect (should be dl-DataToUL-ACK-DCI-1-2). - For NR-DC and Rel-16 PDCCH monitoring, ‘pdcch-BlindDetection’ should be ‘pdcch-BlindDetection2’ in clause 10. - For operation in shared spectrum, the bit for subCarrierSpacingCommon in MIB indicates N_SSB^QCL in clause 4.1 but a corresponding change is not captured in clause 10.1. - Parameter name size-DCI-2-6 in clause 10.3 is incorrect (should be sizeDCI-2-6). - Misaligned parameter name in 38.213 (availableRB-SetsToRelease) and 38.331 (availableRB-SetsToReleaseList) in clauses 11.1 and 11.1.1. - Misaligned parameter name in 38.213 (tdd-UL-DL-ConfigDedicated) and 38.331 (tdd-UL-DL-ConfigurationDedicated) in clause 11.1. - Misplaced word ‘grant’ in a sentence in clause 11.1.1. - Misaligned parameter name in 38.213 (timeresourcepool) and 38.331 (sl-TimeResource) in clause 16 - Parameter name ‘sl-filterCoefficient’ in clause 16.2.1 is incorrect (should be ‘sl-FilterCoefficient’). - Clarify that the priority value of SL reception is used when comparing the priority of SL reception to sl-PriorityThreshold-UL-URLLC in clause 16.2.4.3.1. - Parameter names ‘startSLsymbols’ and ‘lengthSLsymbols’ in clause 16.3 are incorrect (should be ‘sl-StartSymbol’ and ‘sl-LengthSymbols’). - Incorrect reference to TS38.133 in clause 16.3.1 (should be TS 38.101-4). - Parameter name ‘sl-PSFCH-ToPUCCH’ in clause 16.5.1 is incorrect (should be ‘sl-PSFCH-ToPUCCH-CG-Type1’). - As Type1 SL HARQ-ACK codebook with HARQ-ACK bits corresponding to PSSCH transmission occasions in more than one resource pool configured with PSFCH is not supported, the MA of occasions contain PSSCH candidate occasions in a same resource pool. However, in clause 16.5.1.1, the MA of occasions are derived based on a set of pool bitmaps. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change an ‘are’ to ‘is’ in a sentence in clause 6. - Change dl-DataToUL-ACK-ForDCIFormat1_2 to dl-DataToUL-ACK-DCI-1-2) in clause 9.1.2.1. - Change ‘pdcch-BlindDetection’ to ‘pdcch-BlindDetection2’ in clause 10. - Clarify the use of subCarrierSpacingCommon in MIB for shared/non-shared spectrum operation in clause 10.1 - Change size-DCI-2-6 to sizeDCI-2-6 in clause 10.3. - Change ‘availableRB-SetsToRelease’ to ‘availableRB-SetsToReleaseList’ in clauses 11.1 and 11.1.1. - Change ‘tdd-UL-DL-ConfigDedicated’ to ‘tdd-UL-DL-ConfigurationDedicated’ in clause 11.1. - Rearrange location of ‘grant’ in the applicable sentence in clause 11.1.1. - Change ‘timeresourcepool’ to sl-TimeResource in clause 16. - Change ‘sl-filterCoefficient’ to ‘sl-FilterCoefficient’ in 16.2.1. - Add ‘or reception’ to clarify that priority value of SL reception is used when comparing the priority of SL reception to sl-PriorityThreshold-UL-URLLC in clause 16.2.4.3.1. - Change ‘startSLsymbols’ and ‘lengthSLsymbols’ to ‘sl-StartSymbol’ and ‘sl-LengthSymbols’ in clause 16.3. - Change reference in clause 16.3.1 from TS 38.133 to TS 38.101-4. - Change ‘sl-PSFCH-ToPUCCH to ‘sl-PSFCH-ToPUCCH-CG-Type1’ in clause 16.5.1. - Clarify that the MA of occasions of a Type1 HARQ-ACK codebook for sidelink are determined based on a single pool bitmap. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6, 9.1.2,1, 10, 10.1, 10.3, 11.1, 11.1.1, 16, 16.2.1, 16.2.4.3.1, 16.3, 16.3.1, 16.5.1, 16.5.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications"

[2]	3GPP TS 38.201: "NR; Physical Layer – General Description"

[3]	3GPP TS 38.202: "NR; Services provided by the physical layer"

[4]	3GPP TS 38.211: "NR; Physical channels and modulation"

[5]	3GPP TS 38.212: "NR; Multiplexing and channel coding"

[6]	3GPP TS 38.214: "NR; Physical layer procedures for data"

[7]	3GPP TS 38.215: "NR; Physical layer measurements"

[8-1]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone"

[8-2]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone"

[8-3]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios"

[8-4]	3GPP TS 38.101-4: "NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements"

[9]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception"

[10]	3GPP TS 38.133: "NR; Requirements for support of radio resource management"

[11]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification"

[12]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification"

[13]	3GPP TS 36.213: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures"

[14]	3GPP TS 36.321: "Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification"

[15]	3GPP TS 37.213: "Physical layer procedures for shared spectrum channel access"

[16]	3GPP TS 38.473: "F1 application protocol (F1AP)"

## *** Unchanged text is omitted ***

# 6	Link recovery procedures

## *** Unchanged text is omitted ***

For the PCell or the PSCell, upon request from higher layers, the UE provides to higher layers the periodic CSI-RS configuration indexes and/or SS/PBCH block indexes from the set  and the corresponding L1-RSRP measurements that are larger than or equal to the Qin,LR threshold.

For the SCell, upon request from higher layers, the UE indicates to higher layers whether there is at least one periodic CSI-RS configuration index or SS/PBCH block index from the set  with corresponding L1-RSRP measurements that is larger than or equal to the Qin,LR threshold, and provides the periodic CSI-RS configuration indexes and/or SS/PBCH block indexes from the set  and the corresponding L1-RSRP measurements that are larger than or equal to the Qin,LR threshold, if any.

For the PCell or the PSCell, a UE can be provided a CORESET through a link to a search space set provided by recoverySearchSpaceId, as described in clause 10.1, for monitoring PDCCH in the CORESET. If the UE is provided recoverySearchSpaceId, the UE does not expect to be provided another search space set for monitoring PDCCH in the CORESET associated with the search space set provided by recoverySearchSpaceId.

## *** Unchanged text is omitted ***

#### 9.1.2.1	Type-1 HARQ-ACK codebook in physical uplink control channel

For a serving cell c, an active DL BWP, and an active UL BWP, as described in clause 12, the UE determines a set of MA,c occasions for candidate PDSCH receptions for which the UE can transmit corresponding HARQ-ACK information in a PUCCH in slot nU. If serving cell c is deactivated, the UE uses as the active DL BWP for determining the set of MA,c occasions for candidate PDSCH receptions a DL BWP provided by firstActiveDownlinkBWP-Id. The determination is based:

a)	on a set of slot timing values K1 associated with the active UL BWP

a)	If the UE is configured to monitor PDCCH for DCI format 1_0 and is not configured to monitor PDCCH for either DCI format 1_1 or DCI format 1_2 on serving cell c, K1 is provided by the slot timing values {1, 2, 3, 4, 5, 6, 7, 8}

b)	If the UE is configured to monitor PDCCH for DCI format 1_1 and is not configured to monitor PDCCH for DCI format 1_2 for serving cell c, K1 is provided by dl-DataToUL-ACK

c)	If the UE is configured to monitor PDCCH for DCI format 1_2 and is not configured to monitor PDCCH for DCI format 1_1 for serving cell c, K1 is provided by dl-DataToUL-ACK-DCI-1-2

d)	If the UE is configured to monitor PDCCH for DCI format 1_1 and DCI format 1_2 for serving cell c, K1 is provided by the union of dl-DataToUL-ACK and dl-DataToUL-ACK-DCI-1-2

## *** Unchanged text is omitted ***

If a UE is provided dl-DataToUL-ACK or dl-DataToUL-ACK-DCI-1-2, the UE does not expect to be indicated by DCI format 1_0 a slot timing value for transmission of HARQ-ACK information that does not belong to the intersection of the set of slot timing values {1, 2, 3, 4, 5, 6, 7, 8} and the set of slot timing values provided by K1 for the active DL BWP of a corresponding serving cell.

## *** Unchanged text is omitted ***

## 10	UE procedure for receiving control information

## *** Unchanged text is omitted ***

When the UE is configured for carrier aggregation operation over more than 2 cells, or for a cell group when the UE is configured for NR-DC operation, the UE does not expect to monitor per span a number of PDCCH candidates or a number of non-overlapped CCEs that is larger than the maximum number as derived from the corresponding value of Ncellscap-r16.

When a UE is configured for NR-DC operation with a total of NNR-DCDL,cells downlink cells on both the MCG and the SCG and the UE is provided monitoringCapabilityConfig = r16monitoringcapability for all downlink cells where the UE monitors PDCCH, the UE expects to be provided pdcch-BlindDetection2 for the MCG and pdcch-BlindDetection2 for the SCG with values that satisfy

-	pdcch-BlindDetection2 for the MCG + pdcch-BlindDetection2 for the SCG <= pdcch-MonitoringCA, if the UE reports pdcch-MonitoringCA, or

-	pdcch-BlindDetection2 for the MCG + pdcch-BlindDetection2 for the SCG <= NNR-DCDL,cells, if the UE does not report pdcch-MonitoringCA

When a UE is configured for NR-DC operation and the UE is provided monitoringCapabilityConfig = r16monitoringcapability for all downlink cells where the UE monitors PDCCH, the UE may indicate, through pdcch-BlindDetectionMCG-UE-r16 and pdcch-BlindDetectionSCG-UE-r16, respective maximum values for pdcch-BlindDetection for the MCG and pdcch-BlindDetection for the SCG.

## *** Unchanged text is omitted ***

## 10.1	UE procedure for determining physical downlink control channel assignment

## *** Unchanged text is omitted ***

If a UE monitors PDCCH candidates for DCI formats with CRC scrambled by a C-RNTI and the UE is provided a non-zero value for searchSpaceID in PDCCH-ConfigCommon for a Type0/0A/2-PDCCH CSS set, the UE determines monitoring occasions for PDCCH candidates of the Type0/0A/2-PDCCH CSS set based on the search space set associated with the value of searchSpaceID.

The UE may assume that the DM-RS antenna port associated with PDCCH receptions in the CORESET configured by pdcch-ConfigSIB1 in MIB, the DM-RS antenna port associated with corresponding PDSCH receptions, and the corresponding SS/PBCH block are quasi co-located with respect to average gain, quasi co-location 'typeA' and 'typeD' properties, when applicable [6, TS 38.214], if the UE is not provided a TCI state indicating quasi co-location information of the DM-RS antenna port for PDCCH reception in the CORESET. The value for the DM-RS scrambling sequence initialization is the cell ID. For operation without shared spectrum channel access, a SCS is provided by subCarrierSpacingCommon in MIB. For operation with shared spectrum channel access, a SCS is same as the SCS of a corresponding SS/PBCH block.

For single cell operation or for operation with carrier aggregation in a same frequency band, a UE does not expect to monitor a PDCCH in a Type0/0A/2/3-PDCCH CSS set or in a USS set if a DM-RS for monitoring a PDCCH in a Type1-PDCCH CSS set is not configured with same qcl-Type set to 'typeD' properties [6, TS 38.214] with a DM-RS for monitoring the PDCCH in the Type0/0A/2/3-PDCCH CSS set or in the USS set, and if the PDCCH or an associated PDSCH overlaps in at least one symbol with a PDCCH the UE monitors in a Type1-PDCCH CSS set or with an associated PDSCH.

## *** Unchanged text is omitted ***

## 10.3	PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells

A UE configured with DRX mode operation [11, TS 38.321] can be provided the following for detection of a DCI format 2_6 in a PDCCH reception on the PCell or on the SpCell [12, TS 38.331]

-	a PS-RNTI for DCI format 2_6 by ps-RNTI

-	a number of search space sets, by dci-Format2-6, to monitor PDCCH for detection of DCI format 2_6 on the active DL BWP of the PCell or of the SpCell according to a common search space as described in clause 10.1

-	a payload size for DCI format 2_6 by sizeDCI-2-6

-	a location in DCI format 2_6 of a Wake-up indication bit by ps-PositionDCI-2-6

-	a '0' value for the Wake-up indication bit, when reported to higher layers, indicates to not start the drx-onDurationTimer for the next long DRX cycle [11, TS 38.321]

-	a '1' value for the Wake-up indication bit, when reported to higher layers, indicates to start the drx-onDurationTimer for the next long DRX cycle [11, TS 38.321]

## *** Unchanged text is omitted ***

## 11.1	Slot configuration

## *** Unchanged text is omitted ***

If a UE is provided channelAccessMode ='dynamic' and is provided availableRB-SetsToAddModList and availableRB-SetsToReleaseList, the UE expects to be provided co-DurationsPerCellToAddModList and co-DurationsPerCellToReleaseList and/or slotFormatCombToAddModList and slotFormatCombToReleaseList.

## *** Unchanged text is omitted ***

### 11.1.1	UE procedure for determining slot format

This clause applies for a serving cell that is included in a set of serving cells configured to a UE by slotFormatCombToAddModList and slotFormatCombToReleaseList, availableRB-SetsToAddModList and availableRB-SetsToReleaseList, switchTriggerToAddModList and switchTriggerToReleaseList, or co-DurationsPerCellToAddModList and co-DurationsPerCellToReleaseList.

## *** Unchanged text is omitted ***

For a set of symbols of a slot indicated to a UE as flexible by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated if provided, or when tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated are not provided to the UE, and if the UE detects a DCI format 2_0 providing a format for the slot using a slot format value other than 255

-	if one or more symbols from the set of symbols are symbols in a CORESET configured to the UE for PDCCH monitoring, the UE receives PDCCH in the CORESET only if an SFI-index field value in DCI format 2_0 indicates that the one or more symbols are downlink symbols

-	if an SFI-index field value in DCI format 2_0 indicates the set of symbols of the slot as flexible and the UE detects a DCI format indicating to the UE to receive PDSCH or CSI-RS in the set of symbols of the slot, the UE receives PDSCH or CSI-RS in the set of symbols of the slot

-	if an SFI-index field value in DCI format 2_0 indicates the set of symbols of the slot as flexible and the UE detects a DCI format, a RAR UL grant, fallbackRAR UL grant, or successRAR indicating to the UE to transmit PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot the UE transmits the PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot

-	if an SFI-index field value in DCI format 2_0 indicates the set of symbols of the slot as flexible, and the UE does not detect a DCI format indicating to the UE to receive PDSCH or CSI-RS, or the UE does not detect a DCI format, a RAR UL grant, fallbackRAR UL grant, or successRAR indicating to the UE to transmit PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot, the UE does not transmit or receive in the set of symbols of the slot

-	if the UE is configured by higher layers to receive PDSCH or CSI-RS in the set of symbols of the slot, the UE receives the PDSCH or the CSI-RS in the set of symbols of the slot only if an SFI-index field value in DCI format 2_0 indicates the set of symbols of the slot as downlink and, if applicable, the set of symbols is within remaining channel occupancy duration

## *** Unchanged text is omitted ***

And if another cell among the cells configured with directionalCollisionHandling-r16 operates in the same frequency band as the reference cell, the UE does not expect

-	a symbol to be indicated as downlink or uplink on the reference cell and as uplink or downlink on another cell, respectively, by tdd-UL-DL-ConfigurationCommon or by tdd-UL-DL-ConfigurationDedicated,

-	tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated to indicate a symbol as downlink on the reference cell and to detect a DCI format scheduling a transmission on the symbol on another cell, and

-	to be configured by higher layers to receive PDCCH, PDSCH, or CSI-RS on a flexible symbol on the reference cell and to detect a DCI format scheduling a transmission on the symbol on another cell,

if the reference cell and another cell among the cells configured with directionalCollisionHandling-r16 operate in different frequency bands,

the UE

-	assumes symbol as flexible, is not required to receive higher layer configured PDCCH, PDSCH, or CSI-RS and not expected to transmit higher layer configured SRS, PUCCH, PUSCH, or PRACH, when tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated indicates symbol as downlink or uplink on another cell and as uplink or downlink for the reference cell, respectively,

-	transmits a signal/channel scheduled by a DCI format on a symbol of another cell when the symbol is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated for the reference cell,

-	is not required to receive a higher layer configured PDCCH, PDSCH, or CSI-RS on flexible symbols on the reference cell in a set of symbols, if the UE detects a DCI format scheduling a transmission on one or more symbols in the set of symbols on another cell,

## *** Unchanged text is omitted ***

# 16	UE procedures for sidelink

A UE is provided by SL-BWP-Config a BWP for SL transmissions (SL BWP) with numerology and resource grid determined as described in [4, TS 38.211]. For a resource pool within the SL BWP, the UE is provided by sl-NumSubchannel a number of sub-channels where each sub-channel includes a number of contiguous RBs provided by sl-SubchannelSize. The first RB of the first sub-channel in the SL BWP is indicated by sl-StartRB-Subchannel. Available slots for a resource pool are provided by sl-TimeResource and occur with a periodicity of 10240 ms. For an available slot without S-SS/PSBCH blocks, SL transmissions can start from a first symbol indicated by sl-StartSymbol and be within a number of consecutive symbols indicated by sl-LengthSymbols. For an available slot with S-SS/PSBCH blocks, the first symbol and the number of consecutive symbols is predetermined.

## *** Unchanged text is omitted ***

### 16.2.1	PSSCH

A UE determines a power PPSSCH(i) for a PSSCH transmission on a resource pool in symbols where a corresponding PSCCH is not transmitted in PSCCH-PSSCH transmission occasion i on active SL BWP b of carrier f as:

PPSSCH(i)=minPCMAX,PMAX,CBR,minPPSSCH,Di,PPSSCH,SL(i) [dBm]

where

## *** Unchanged text is omitted ***

where

-	PO,SL is a value of sl-P0-PSSCH-PSCCH, if provided

-	αSL is a value of sl-Alpha-PSSCH-PSCCH, if provided; else, αSL=1

-	PLSL= referenceSignalPower – higher layer filtered RSRP, where

-	referenceSignalPower is obtained from a PSSCH transmit power per RE summed over the antenna ports of the UE, higher layer filtered across PSSCH transmission occasions using a filter configuration provided by sl-FilterCoefficient, and

-	higher layer filtered RSRP is a RSRP, as defined in [7, TS 38.215], that is reported to the UE from a UE receiving the PSCCH-PSSCH transmission and is obtained from a PSSCH DM-RS using a filter configuration provided by sl-FilterCoefficient

## *** Unchanged text is omitted ***

##### 16.2.4.3.1	Prioritizations for sidelink and uplink transmissions/receptions

*** Unchanged text is omitted ***

For prioritization between SL transmission or PSFCH/S-SS/PSBCH block reception and UL transmission other than a PRACH, or a PUSCH scheduled by an UL grant in a RAR and its retransmission, or a PUSCH corresponding to Type-2 random access procedure and its retransmission, or a PUCCH with sidelink HARQ-ACK information report

-	if the UL transmission is for a PUSCH or for a PUCCH with priority index 1,

-	if sl-PriorityThreshold-UL-URLLC is provided

-	the SL transmission or reception has higher priority than the UL transmission if the priority value of the SL transmission or reception is smaller than sl-PriorityThreshold-UL-URLLC; otherwise, the UL transmission has higher priority than the SL transmission or reception

-	else

-	the UL transmission has higher priority than the SL transmission or reception

-	else

-	the SL transmission or reception has higher priority than the UL transmission if the priority value of the SL transmission(s) or reception is smaller than sl-PriorityThreshold; otherwise, the UL transmission has higher priority than the SL transmission or reception

*** Unchanged text is omitted ***

### 16.3 UE procedure for reporting HARQ-ACK on sidelink

*** Unchanged text is omitted ***

A UE is provided by sl-PSFCH-RB-Set a set of MPRB, setPSFCH PRBs in a resource pool for PSFCH transmission in a PRB of the resource pool. For a number of Nsubch sub-channels for the resource pool, provided by sl-NumSubchannel, and a number of PSSCH slots associated with a PSFCH slot that is less than or equal to NPSSCHPSFCH, the UE allocates the i+j⋅NPSSCHPSFCH⋅Msubch, slotPSFCH, i+1+j⋅NPSSCHPSFCH⋅Msubch, slotPSFCH-1 PRBs from the MPRB, setPSFCH PRBs to slot i among the PSSCH slots associated with the PSFCH slot and sub-channel j, where Msubch, slotPSFCH=MPRB, setPSFCHNsubch⋅NPSSCHPSFCH, 0≤i<NPSSCHPSFCH, 0≤j<Nsubch, and the allocation starts in an ascending order of i and continues in an ascending order of j. The UE expects that MPRB, setPSFCH is a multiple of Nsubch∙NPSSCHPSFCH.

The second OFDM symbol l' of PSFCH transmission in a slot is defined as l'=sl-StartSymbol+sl-LengthSymbols-2 .

*** Unchanged text is omitted ***

### 16.3.1	UE procedure for receiving HARQ-ACK on sidelink

A UE that transmitted a PSSCH scheduled by a SCI format 2-A or a SCI format 2-B that indicates HARQ feedback enabled, attempts to receive associated PSFCHs according to PSFCH resources determined as described in clause 16.3. The UE determines an ACK or a NACK value for HARQ-ACK information provided in each PSFCH resource as described in [8-4, TS 38.101-4]. The UE does not determine both an ACK value and a NACK value at a same time for a PSFCH resource.

*** Unchanged text is omitted ***

### 16.5.1	Type-1 HARQ-ACK codebook determination

This clause applies if the UE is configured with pdsch-HARQ-ACK-Codebook = semi-static.

If a UE is configured a SL configured grant Type 1, and the UE is configured a SL configured grant Type 2 or to monitor PDCCH for detection of DCI format 3_0 with CRC scrambled by SL-RNTI or SL-CS-RNTI, and the UE is provided a set of slot timing values K1 associated with a SL BWP by sl-PSFCH-ToPUCCH and sl-PSFCH-ToPUCCH-CG-Type1, the sl-PSFCH-ToPUCCH-CG-Type1 is one of sl-PSFCH-ToPUCCH.

A UE reports HARQ-ACK information for PSSCH transmissions with corresponding PSFCH reception occasions in slot n only in a HARQ-ACK codebook that the UE includes in a PUCCH or PUSCH transmission in slot n+k, where k is a number of slots indicated by the PSFCH-to-HARQ_feedback timing indicator field in a DCI format 3_0 scheduling the PSSCH transmissions, or by a value of PSFCH-to-HARQ feedback timing indicator field in a DCI format 3_0 activating a SL configured grant Type-2 transmission or by a value of sl-PSFCH-ToPUCCH-CG-Type1 for a SL configured grant Type-1. If the UE reports HARQ-ACK information for the PSSCH transmissions with corresponding PSFCH reception occasions in a slot other than slot n+k, the UE sets a value for each corresponding HARQ-ACK information bit to NACK.

*** Unchanged text is omitted ***

#### 16.5.1.1	Type-1 HARQ-ACK codebook in physical uplink control channel

For a SL BWP on a carrier, and an active UL BWP on the primary cell, as described in clause 12, a UE determines a set MA of occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions for which the UE can multiplex corresponding HARQ-ACK information in a PUCCH transmission in slot nU. The determination is based on:

a)	a set of slot timing values K1 associated with the SL BWP where K1 is provided by sl-PSFCH-ToPUCCH for DCI format 3_0 or by sl-PSFCH-ToPUCCH-CG-Type1

b)	the ratio 2μSL-μUL between the sidelink SCS configuration μSL and the uplink SCS configuration μUL provided by subcarrierSpacing in BWP-Sidelink and BWP-Uplink for the SL BWP and the active UL BWP, respectively

c)	a configured sidelink resource pool bitmap

d)	a value of a period of PSFCH transmission occasion resources for a sidelink resource pool provided by a respective sl-PSFCH-Period

*** Unchanged text is omitted ***