3GPP TSG-WG1 Meeting # 107-e	R1-2112911

# e-Meeting, November 11th - 19th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1412 | rev | - | Current version: | 16.7.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of NB-IoT/eMTC support for Non-Terrestrial Networks in 36.213 s8-s09 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Motorola Mobility |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN-Core |  |  |  |  |  | Date: |  |  | 2021-11-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Introduction of NB-IoT/eMTC support for Non-Terrestrial Networks. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support for NB-IoT/eMTC support for Non-Terrestrial Networks features according to RAN1 agreements: - timing relationship enhancements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of NB-IoT/eMTC support for Non-Terrestrial Networks. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8, 8.0, 8.2, 9.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 36.211 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 8	Physical uplink shared channel related procedures

If the UE is configured with a SCG, the UE shall apply the procedures described in this clause for both MCG and SCG

-	When the procedures are applied for MCG, the terms 'secondary cell', 'secondary cells' , 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells, serving cell, serving cells belonging to the MCG respectively.

-	When the procedures are applied for SCG, the terms 'secondary cell', 'secondary cells', 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells (not including PSCell), serving cell, serving cells belonging to the SCG respectively. The term 'primary cell' in this clause refers to the PSCell of the SCG.

If a UE is configured with a LAA SCell for UL transmissions, the UE shall apply the procedures described in this clause assuming frame structure type 1 for the LAA SCell unless stated otherwise.

For a UE configured with EN-DC/NE-DC and serving cell frame structure type 1, if the UE is configured with tdm-PatternConfig/tdm-PatternConfigNE-DC for the serving cell, the UE is not expected to transmit any uplink physical channel or signal in the serving cell on subframes other than offset-UL subframes, where the offset-UL subframes are determined by applying an offset value given by harq-Offset-r15 to the subframes denoted as uplink in the UL/DL configuration tdm-PatternConfig/tdm-PatternConfigNE-DC.

For a UE configured with EN-DC with primary cell frame structure type 1, if the UE is configured with tdm-PatternConfig2 for a serving cell, and if the UE indicates a capability fdd-PCellUL-TX-AllUL-Subframe-r16 (as specified in [11]), the UE transmits any uplink physical channel or signal without associated DCI if configured, in the serving cell on any uplink subframes. Otherwise, if the UE is configured with tdm-PatternConfig2 for the serving cell and if the UE does not indicate a capability fdd-PCellUL-TX-AllUL-Subframe-r16, the UE is not expected to transmit any uplink physical channel or signal without associated DCI except for PRACH in the serving cell on subframes other than offset-UL subframes, where the offset-UL subframes are determined by applying an offset value given by harq-Offset-r16 to the subframes denoted as uplink in the UL/DL configuration tdm-PatternConfig2.

For a UE configured with EN-DC with primary cell frame structure type 2, if the UE is configured with tdm-PatternConfig2 for a serving cell, and if the UE indicates a capability tdd-PCellUL-TX-AllUL-Subframe-r16 (as specified in [11]), the UE transmits any uplink physical channel or signal without associated DCI if configured, in the serving cell on any uplink subframes. Otherwise, if the UE is configured with tdm-PatternConfig2 for the serving cell and if the UE does not indicate a capability tdd-PCellUL-TX-AllUL-Subframe-r16, the UE is not expected to transmit any uplink physical channel or signal without associated DCI except for PRACH in the serving cell on subframes other than offset-UL subframes, where the offset-UL subframes are determined by applying an offset value given by harq-Offset-r16 to the subframes denoted as uplink in the UL/DL configuration tdm-PatternConfig2.

For a UE configured with EN-DC/NE-DC, if serving cell frame structure type 1 and if the UE is configured with tdm-PatternConfig/tdm-PatternConfigNE-DC for the serving cell, or if the UE is configured with tdm-PatternConfig2 for a serving cell with EN-DC, the UE is not expected to be configured with more than one serving cell in the uplink.

For a non-BL/CE UE, and for FDD and transmission mode 1 and a cell that is not a LAA SCell, there shall be 16 uplink HARQ processes per serving cell configured with higher layer parameter ul-STTI-Length, otherwise 8 uplink HARQ processes per serving cell for non-subframe bundling operation, i.e. normal HARQ operation, and 3 uplink HARQ processes for subframe bundling operation when parameter e-HARQ-Pattern-r12 is set to TRUE and 4 uplink HARQ processes for subframe bundling operation otherwise. For a non-BL/CE UE, and for FDD and transmission mode 2 configured for subframe-PUSCH and a cell that is not a LAA SCell, there shall be 32 uplink HARQ processes per serving cell configured with higher layer parameters ul-STTI-Length and shortProcessingTime, otherwise 16 uplink HARQ processes per serving cell for non-subframe bundling operation and there are two HARQ processes associated with a given subframe for subframe-PUSCH as described in [8]. The subframe bundling operation is configured by the parameter ttiBundling provided by higher layers.

For FDD and a BL/CE UE configured with CEModeA, there shall be at most 8 uplink HARQ processes per serving cell.

For FDD and a BL/CE UE configured with CEModeB, there shall be at most 4 uplink HARQ processes per serving cell if the UE is configured with higher layer parameter ce-PUSCH-MultiTB-Config, 2 uplink HARQ processes per serving cell otherwise.

For a BL/CE UE and PUSCH transmission using preconfigured uplink resource, there shall be 1 uplink HARQ process per serving cell.

For a LAA SCell, and transmission mode 1, there shall be 16 uplink HARQ processes. For a LAA SCell, and transmission mode 2, there shall be 32 uplink HARQ processes.

There shall be 16 uplink HARQ processes per TDD serving cell configured with higher layer parameter ul-STTI-Length.

In case higher layers configure the use of subframe bundling for FDD and TDD, the subframe bundling operation is only applied to UL-SCH, such that four consecutive uplink subframes are used.

A BL/CE UE is not expected to be configured with simultaneous PUSCH and PUCCH transmission.

Throughout this clause, for a BL/CE UE, the value of Koffset is given by,

-	if the UE is configured with the higher layer parameter CellSpecificKoffset,

-	Koffset= Kcell_offset-KUE_offset where

Kcell_offset is the parameter CellSpecificKoffset provided by higher layers, and

KUE_offset is the parameter UESpecificKoffset provided by higher layers, otherwise KUE_offset=0

-	otherwise,

-	Koffset=0.

## 8.0	UE procedure for transmitting the physical uplink shared channel

The term "UL/DL configuration" in this Clause refers to the higher layer parameter subframeAssignment unless specified otherwise.

Throughout this clause, if the UE is configured with higher layer parameter shortTTI and the corresponding PDCCH/SPDCCH with DCI format 7-0A/7-0B is detected in a subslot, if the UE is configured for subslot uplink transmissions, is determined based on higher layer configuration from , otherwise. If subslot number n is in subframe N, subslot refers to subslot number in subframe .

For a given serving cell, if a UE is configured with higher layer parameter shortProcessingTime, the UE is not expected to receive

-	more than one uplink scheduling grants for an uplink subframe.

-	PDCCH in common search space with DCI format 0 in subframe n and PDCCH in User-specific search space with DCI format 0/4 in the same subframe n.

For a serving cell, and a UE configured with higher layer parameter ul-STTI-Length, the UE is not expected to transmit subframe-PUSCH

-	in a given subframe corresponding to PDCCH with uplink DCI format other than 7-0A/7-0B or without a corresponding PDCCH if the UE detects PDCCH/SPDCCH with uplink DCI format 7-0A/7-0B corresponding to a PUSCH transmission in the same subframe or if the UE transmits a slot/subslot-PUSCH without a corresponding PDCCH/SPDCCH. The UE shall transmit the HARQ-ACK response corresponding to the subframe-PUSCH using the slot/subslot-PUSCH (as defined in Clause 7.3). The UE shall apply spatial HARQ-ACK bundling on the HARQ-ACK response

-	in case subslot-PUSCH is used

-	in case slot-PUSCH is used if the bundling is configured for the cell.

-	in a given subframe corresponding to PDCCH/EPDCCH with uplink DCI format other than 7-0A/7-0B received in subframe n if the UE detects PDCCH/SPDCCH with uplink DCI format 7-0A/7-0B in any subframe from subframe n+1 to subframe n+WUL corresponding to a PUSCH transmission, and if  is indicated by skipSubframeProcessing capability [12],

-	in case of a collision between the subframe-PUSCH and slot/subslot-PUCCH. The UE shall transmit the HARQ-ACK response corresponding to the subframe-PUSCH using the slot/subslot-PUCCH (as defined in Clause 7.3). The UE shall apply spatial HARQ-ACK bundling on the HARQ-ACK response

-	in case subslot-PUCCH is used

-	in case slot-PUCCH is used if the bundling is configured for the cell.

-	in case of a collision between the subframe-PUSCH, subframe-PUCCH, and slot/subslot-PUSCH when simultaneous PUSCH and PUCCH transmission is configured for the UE. The UE is also not expected to transmit subframe-PUCCH. The UE shall transmit the HARQ-ACK response corresponding to the subframe-PUCCH using the slot/subslot-PUSCH.

For a serving cell, and a UE configured with higher layer parameter shortTTI, the UE is not expected to transmit PUSCH corresponding to PDCCH/SPDCCH with CRC scrambled by the C-RNTI/SPS C-RNTI and with uplink DCI format 7-0A/7-0B

-	in UpPTS of the special subframe in frame structure type 2 with special subframe configuration 0-9 or,

-	for a transport block corresponding to a HARQ process with NDI not toggled if the previous PUSCH transmission of the transport block was signalled via PDCCH in UE specific search space with CRC scrambled by the C-RNTI/SPS C-RNTI with DCI format other than DCI format 7-0A/7-0B when the number of codewords for the previous PUSCH transmission is two or the transport block size is larger than the maximum transport block size supported for slot/subslot-PUSCH transmission.

For a UE configured with more than one serving cell and not capable of simultaneous transmission of different uplink signal durations to different serving cells as indicated by UE capability simultaneousTx-differentTx-duration, in case of a collision between

-	a slot-PUSCH of first serving cell and a subframe-PUSCH/PUCCH/SRS/PRACH of second serving cell or

-	a subslot-PUSCH of first serving cell and a subframe/slot-PUSCH/PUCCH/SRS/PRACH of second serving cell

the uplink transmission(s) of the second serving cell are dropped.

For a serving cell, and a UE configured with higher layer parameter shortTTI, the UE shall discard PDCCH/SPDCCH with uplink DCI format 7-0A/7-0B for subslot n if PDCCH/SPDCCH with uplink DCI format 7-0A/7-0B for subslot n-1 indicates the DMRS transmission in the first symbol of subslot n

-	if the PDCCH/SPDCCH with uplink DCI format 7-0A/7-0B for subslot n does not indicate DMRS transmission in the first symbol of subslot n, or

-	if the PDCCH/SPDCCH with uplink DCI format 7-0A/7-0B for subslot n indicates the DMRS transmission in the first symbol of subslot n,

-	if the cyclic shift and/or IFDMA comb of subslot n-1 is not identical to that of subslot n, or

-	if the PUSCH RBs of subslot n-1 is not identical to those of subslot n, or

-	if precoding information and number of layers of subslot n-1 are not identical to those of subslot n, or

-	if TPC field for subslot n is not '1' and if the UE is configured with higher layer parameter accumulationEnabledsTTI, or

-	if TPC field for subslot n-1 is not identical to that of subslot n and if the UE is not configured with higher layer parameter accumulationEnabledsTTI.

For a serving cell, and a UE configured with higher layer parameter totalNumberPUSCH-SPS-STTI-UL-Repetitions or totalNumberPUSCH-SPS-UL-Repetitions,

-	the UE is not expected to transmit PUSCH with a subframe/slot/subslot duration associated with a DCI scrambled by SPS C-RNTI colliding with ongoing PUSCH repetitions of the same subframe/slot/subslot duration associated with another DCI scrambled by SPS C-RNTI.

-	for a FDD cell, the UE shall upon detection of a PDCCH/EPDCCH/SPDCCH with DCI format 0/7-0A/7-0B with CRC scrambled by SPS C-RNTI with NDI set to 0, intended for the UE, transmit the corresponding PUSCH in k consecutive UL subframes/slots/subslots.

-	for a TDD cell not configured with higher layer parameter symPUSCH-UpPts-r14, the UE shall upon detection of a PDCCH/EPDCCH/SPDCCH with DCI format 0/7-0A/7-0B with CRC scrambled by SPS C-RNTI with NDI set to 0, intended for the UE, transmit the corresponding PUSCH in k consecutive UL subframes/slots according to the UL/DL configuration indicated by higher layer parameter subframeAssignment for the serving cell.

-	for a TDD cell configured with higher layer parameter symPUSCH-UpPts-r14, the UE shall upon detection of a PDCCH/EPDCCH/SPDCCH with DCI format 0/7-0A/7-0B with CRC scrambled by SPS C-RNTI with NDI set to 0, intended for the UE, transmit the corresponding PUSCH in k consecutive UL subframes/slots or UpPTS according to the UL/DL configuration indicated by higher layer parameter subframeAssignment for the serving cell.

-	for a TDD cell configured with UL/DL configuration 0 indicated by higher layer parameter subframeAssignment, the UE is not expected to receive a DCI of format 0 with CRC scrambled by SPS C-RNTI scheduling more than one PUSCH with a subframe duration by UL index.

-	for a TDD cell configured with UL/DL configuration 6 indicated by higher layer parameter subframeAssignment and configured with higher layer parameters symPUSCH-UpPts-r14, the UE is not expected to receive a DCI of format 0 with CRC scrambled by SPS C-RNTI scheduling more than one PUSCH with a subframe duration by UL index.

-	for a TDD cell configured with UL/DL configurations 0/6 indicated by higher layer parameter subframeAssignment, the UE is not expected to receive a DCI of format 7-0A/7-0B with CRC scrambled by SPS C-RNTI scheduling more than one PUSCH with a slot duration by UL index.

For a serving cell that is not a LAA SCell, and for FDD and normal HARQ operation, the UE shall upon detection on a given serving cell of a

-	PDCCH/EPDCCH with DCI format 0/4 and/or a PHICH transmission in subframe n intended for the UE, perform a corresponding PUSCH transmission in subframe n+ kp according to the PDCCH/EPDCCH and PHICH information where  if the UE is configured with higher layer parameter shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI is in the UE-specific search space, otherwise.

-	PDCCH/SPDCCH with DCI format 7-0A/7-0B intended for the UE in

-	slot n, perform a corresponding PUSCH transmission in slot

-	subslot n, perform a corresponding PUSCH transmission

-	in subslot if the UE is configured with subslot-based uplink transmissions, or

-	in slot 0 of subframe N if the UE is configured with slot-based uplink transmissions, and subslot n (with n being subslot numbered from 0 to 5 within a subframe) is only one of

-	subframe N-3, and subslot number n=4 or 5, or

-	subframe N-2, and subslot number n=0

-	in slot 1 of subframe N if the UE is configured with slot-based uplink transmissions, and subslot n belongs to subframe N-2, and n is only one of subslot number {1, 2, 3}

if a transport block corresponding to the HARQ process of the PUSCH transmission is generated as described in [8].

For FDD-TDD and normal HARQ operation and a PUSCH for serving cell  with frame structure type 1, the UE shall upon detection of a PDCCH/EPDCCH with DCI format 0/4 and/or a PHICH transmission in subframe n intended for the UE, perform a corresponding PUSCH transmission for serving cell c in subframe n+ according to the PDCCH/EPDCCH and PHICH information if a transport block corresponding to the HARQ process of the PUSCH transmission is generated as described in [8], where  if the UE is configured with higher layer parameter shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI is in the UE-specific search space, otherwise.

For normal HARQ operation, if the UE detects a PHICH transmission and if the most recent PUSCH transmission for the same transport block was using spatial multiplexing according to Clause 8.0.2 and the UE does not detect a PDCCH/EPDCCH with DCI format 4 in subframe n intended for the UE, the UE shall perform the corresponding PUSCH retransmission in the associated subframe according to the PHICH information, and using the number of transmission layers and precoding matrix according to the most recent PDCCH/EPDCCH, if the number of negatively acknowledged transport blocks is equal to the number of transport blocks indicated in the most recent PDCCH/EPDCCH associated with the corresponding PUSCH.

For normal HARQ operation, if the UE detects a PHICH transmission and if the most recent PUSCH transmission for the same transport block was using spatial multiplexing according to Clause 8.0.2 and the UE does not detect a PDCCH/EPDCCH with DCI format 4 in subframe n intended for the UE, and if the number of negatively acknowledged transport blocks is not equal to the number of transport blocks indicated in the most recent PDCCH/EPDCCH associated with the corresponding PUSCH then the UE shall perform the corresponding PUSCH retransmission in the associated subframe according to the PHICH information, using the precoding matrix with codebook index 0 and the number of transmission layers equal to number of layers corresponding to the negatively acknowledged transport block from the most recent PDCCH/EPDCCH. In this case, the UL DMRS resources are calculated according to the cyclic shift field for DMRS [3] in the most recent PDCCH/EPDCCH with DCI format 4 associated with the corresponding PUSCH transmission and number of layers corresponding to the negatively acknowledged transport block.

If a UE is configured with the carrier indicator field for a given serving cell, the UE shall use the carrier indicator field value from the detected PDCCH/EPDCCH with uplink DCI format to determine the serving cell for the corresponding PUSCH transmission.

For FDD and normal HARQ operation, if a PDCCH/EPDCCH/SPDCCH with CSI request field set to trigger an aperiodic CSI report, as described in Clause 7.2.1, is detected by a UE on subframe/slot/subslot n, and simultaneous PUSCH and PUCCH transmission is not configured for the UE or is detected on slot/subslot n, then UCI is mapped on the corresponding PUSCH transmission on,

-	slot n+4 for slot-PUSCH transmissions when the higher layer parameter dl-STTI-Length is set to 'slot'

-	slot 0 of subframe N+2 for slot-PUSCH transmissions in case of subslot number n=4 or 5 in subframe N-1, or subslot number n=0 in subframe N when the higher layer parameter dl-STTI-Length is set to 'subslot'

-	slot 1 of subframe N+2 for slot-PUSCH transmissions in case of subslot number n=1 or 2 or 3 in subframe N when the higher layer parameter dl-STTI-Length is set to 'subslot'

-	subslot for subslot-PUSCH transmissions

-	subframe n+ kp where  if the UE is configured with higher layer parameter shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI is in the UE-specific search space, otherwise for subframe-PUSCH transmissions.

For FDD and a BL/CE UE configured with CEModeA, if an MPDCCH with CSI request field set to trigger an aperiodic CSI report, as described in Clause 7.2.1, is detected by a UE on subframe n, then on subframe n+4+Koffset UCI is mapped on the corresponding PUSCH transmission, including all subframe repetitions of the PUSCH transmission.

For FDD-TDD and normal HARQ operation, for a serving cell with frame structure type 1, if a PDCCH/EPDCCH/SPDCCH with CSI request field set to trigger an aperiodic CSI report, as described in Clause 7.2.1, is detected by a UE on subframe n, and simultaneous PUSCH and PUCCH transmission is not configured for the UE or is detected on slot/sublost n, UCI is mapped on the corresponding PUSCH transmission on

-	slot n+4 for slot-PUSCH transmissions when the higher layer parameter dl-STTI-Length is set to 'slot';

-	slot 0 of subframe N+2 for slot-PUSCH transmissions in case of subslot number n=4 or 5 in subframe N-1, or subslot n=0 corresponding to subframe N when the higher layer parameter dl-STTI-Length is set to 'subslot';

-	slot 1 of subframe N+2 for slot-PUSCH transmissions in case of subslot number n=1 or 2 or 3 in subframe N when the higher layer parameter dl-STTI-Length is set to 'subslot';

-	subslot for subslot-PUSCH transmissions;

-	subframe n+ kp where  if the UE is configured with higher layer parameter shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI is in the UE-specific search space, otherwise for subframe-PUSCH transmissions.

For TDD, if a UE is configured with the parameter EIMTA-MainConfigServCell-r12 for at least one serving cell, if the UE is configured with one serving cell or if the UE is configured with more than one serving cell and the TDD UL/DL configuration of all the configured serving cells is the same, then for a given serving cell, the serving cell UL/DL configuration is the UL-reference UL/DL configuration.

For TDD, if a UE is configured with more than one serving cell and if the UL/DL configurations of at least two serving cells are different, if the serving cell is a primary cell or if the UE is not configured to monitor PDCCH/EPDCCH in another serving cell for scheduling the serving cell, the serving cell UL/DL configuration is the UL-reference UL/DL configuration.

For TDD, if a UE is configured with more than one serving cell and if the UL/DL configurations of at least two serving cells are different and if the serving cell is a secondary cell and if the UE is configured to monitor PDCCH/EPDCCH in another serving cell for scheduling the serving cell, then for the serving cell, the UL reference UL/DL configuration is given in Table 8-0A corresponding to the pair formed by (other serving cell UL/DL configuration, serving cell UL/DL configuration).

For FDD-TDD and primary cell frame structure type 2, if a serving cell is a primary cell, the serving cell UL/DL configuration is the UL-reference UL/DL configuration for the serving cell.

For FDD-TDD if the UE is not configured to monitor PDCCH/EPDCCH in another serving cell for scheduling a secondary serving cell with frame structure type 2, the serving cell UL/DL configuration is the UL-reference UL/DL configuration for the serving cell.

For FDD-TDD, and for secondary serving cell c with frame structure type 2, if the UE is configured to monitor PDCCH/EPDCCH in another serving cell with frame structure type 1 for scheduling the serving cell, the serving cell UL/DL configuration is the UL-reference UL/DL configuration for the serving cell.

For FDD-TDD, if a UE is configured with more than one serving cell with frame structure type 2, and if the serving cell is a secondary cell with frame structure type 2 and if the UE is configured to monitor PDCCH/EPDCCH in another serving cell with frame structure type 2 for scheduling the serving cell, then for the serving cell, the UL reference UL/DL configuration is given in Table 8-0A corresponding to the pair formed by (other serving cell UL/DL configuration, serving cell UL/DL configuration).

Table 8-0A: UL-reference UL/DL Configuration for serving cell based on the pair formed by (other serving cell UL/DL configuration, serving cell UL/DL configuration)

| Set # | (other serving cell UL/DL configuration, serving cell UL/DL configuration) | UL-reference UL/DL configuration |
| --- | --- | --- |
| Set 1 | (1,1),(1,2),(1,4),(1,5) | 1 |
|  | (2,2),(2,5) | 2 |
|  | (3,3),(3,4),(3,5) | 3 |
|  | (4,4),(4,5) | 4 |
|  | (5,5) | 5 |
| Set 2 | (1,0),(2,0),(3,0),(4,0),(5,0) | 0 |
|  | (2,1),(4,1),(5,1) | 1 |
|  | (5,2) | 2 |
|  | (4,3),(5,3) | 3 |
|  | (5,4) | 4 |
|  | (1,6),(2,6),(3,6),(4,6),(5,6) | 6 |
| Set 3 | (3,1) | 1 |
|  | (3,2),(4,2) | 2 |
|  | (1,3),(2,3) | 3 |
|  | (2,4) | 4 |
| Set 4 | (0,0),(6,0) | 0 |
|  | (0,1),(0,2),(0,4),(0,5),(6,1),(6,2),(6,5) | 1 |
|  | (0,3),(6,3) | 3 |
|  | (6,4) | 4 |
|  | (0,6),(6,6) | 6 |

If a UE is configured with the parameter EIMTA-MainConfigServCell-r12 for a serving cell, for a radio frame of the serving cell, PUSCH transmissions can occur only in subframes that are indicated by eIMTA-UL/DL-configuration as uplink subframe(s) for the serving cell unless specified otherwise.

For TDD and normal HARQ operation, if a PDCCH/EPDCCH/SPDCCH with CSI request field set to trigger an aperiodic CSI report, as described in Clause 7.2.1, is detected by a UE on subframe n and simultaneous PUSCH and PUCCH transmission is not configured for the UE or is detected by a UE on slot n, then on subframe/slot n+k UCI is mapped on the corresponding PUSCH transmission where k is given by

- Table 8-2m for special subframe configuration 1,2,3,4,6,7,8 if the UE is configured with higher layer parameter ul-STTI-Length, and the corresponding uplink DCI format is 7-0A/7-0B;

- Table 8-2n for special subframe configuration 0,5,9 if the UE is configured with higher layer parameter ul-STTI-Length, and the corresponding uplink DCI format is 7-0A/7-0B;

- Table 8-2p if the UE is configured with higher layer parameters ul-STTI-Length and symPUSCH-UpPts-r14, and the corresponding uplink DCI format is 7-0A/7-0B;

- Table 8-2i if the UE is configured with higher layer parameter shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI is in the UE-specific search space;

- Table 8-2 otherwise.

For TDD and a BL/CE UE configured with CEModeA, if an MPDCCH with CSI request field set to trigger an aperiodic CSI report, as described in Clause 7.2.1, is detected by a UE on subframe n, then on subframe n+k+Koffset UCI is mapped on the corresponding PUSCH transmission, including all subframe repetitions of the PUSCH transmission, where k is given by Table 8-2.

For FDD-TDD normal HARQ operation, for a serving cell with frame structure type 2, if a PDCCH/EPDCCH with CSI request field set to trigger an aperiodic CSI report on the serving cell, as described in Clause 7.2.1, is detected by a UE on subframe n, then on subframe n+k UCI is mapped on the corresponding PUSCH transmission where k is given by Table 8-2 and the "TDD UL/DL configuration" refers to the UL-reference UL/DL configuration for the serving cell, when simultaneous PUSCH and PUCCH transmission is not configured for the UE.

When a UE is configured with higher layer parameter ttiBundling and configured with higher layer parameter e-HARQ-Pattern-r12 set to FALSE or not configured, for FDD and subframe bundling operation, the UE shall upon detection of a PDCCH/EPDCCH with DCI format 0 in subframe n intended for the UE, and/or a PHICH transmission in subframe n-5 intended for the UE, perform a corresponding first PUSCH transmission in the bundle in subframe n+4 according to the PDCCH/EPDCCH and PHICH information if a transport block corresponding to the HARQ process of the first PUSCH transmission is generated as described in [8].

When a UE is configured with higher layer parameter ttiBundling and configured with higher layer parameter e-HARQ-Pattern-r12 set to TRUE, for FDD and subframe bundling operation, the UE shall upon detection of a PDCCH/EPDCCH with DCI format 0 in subframe n intended for the UE, and/or a PHICH transmission in subframe n-1 intended for the UE, perform a corresponding first PUSCH transmission in the bundle in subframe n+4 according to the PDCCH/EPDCCH and PHICH information if a transport block corresponding to the HARQ process of the first PUSCH transmission is generated as described in [8].

For both FDD and TDD serving cells, the NDI as signalled on PDCCH/EPDCCH/MPDCCH/SPDCCH, the RV as determined in Clause 8.6.1, and the TBS as determined in Clause 8.6.2, shall be delivered to higher layers.

If the UE is not configured with higher layer parameter ul-STTI-Length, for a non-BL/CE UE, for TDD and transmission mode 1, the number of HARQ processes per serving cell shall be determined by the UL/DL configuration (Table 4.2-2 of [3]), as indicated in Table 8-1 if the UE is not configured with higher layer parameter symPUSCH-UpPts-r14 for the serving cell, otherwise the number of HARQ processes per serving cell shall be determined as

-	, whereis indicated in Table 8-1a, if the UE is configured with shortProcessingTime and the corresponding PDCCH is in the UE-specific search space,

-	indicated in Table 8-1a.

For a non-BL/CE UE, for TDD and transmission mode 2 if the UE is not configured with higher layer parameter ul-STTI-Length, the number of HARQ processes per serving cell for non-subframe bundling operation shall be twice the number determined by the UL/DL configuration (Table 4.2-2 of [3]) for TDD and transmission mode 1 there are two HARQ processes associated with a given subframe as described in [8]. For TDD and both transmission mode 1 and transmission mode 2, the "TDD UL/DL configuration" in Table 8-1 and Table 8-1a refers to the UL-reference UL/DL configuration for the serving cell if UL-reference UL/DL configuration is defined for the serving cell and refers to the serving cell UL/DL configuration otherwise.

For a non-BL/CE UE configured higher layer parameter ul-STTI-Length, if the UE is configured with shortProcessingTime and transmission mode 2 for subframe-PUSCH the number of HARQ processes per TDD serving cell for non-subframe bundling operation is 32, and 16 otherwise. There are two HARQ processes for transmission mode 2 of subframe-PUSCH associated with a given subframe as described in [8].

For a BL/CE UE configured with CEModeA and for TDD, the maximum number of HARQ processes per serving cell shall be determined by the UL/DL configuration (Table 4.2-2 of [3]) according to the normal HARQ operation in Table 8-1. For TDD a BL/CE UE configured with CEModeB is not expected to support more than 4 uplink HARQ processes per serving cell if the UE is configured with higher layer parameter ce-PUSCH-MultiTB-Config, 2 uplink HARQ processes per serving cell otherwise.

Table 8-1: Number of synchronous UL HARQ processes for TDD

| TDD UL/DL configuration | Number of HARQ processes for normal HARQ operation | Number of HARQ processes for subframe bundling operation |
| --- | --- | --- |
| 0 | 7 | 3 |
| 1 | 4 | 2 |
| 2 | 2 | N/A |
| 3 | 3 | N/A |
| 4 | 2 | N/A |
| 5 | 1 | N/A |
| 6 | 6 | 3 |

Table 8-1a: Number of synchronous UL HARQ processes for TDD and UE configured with symPUSCH-UpPts-r14

| TDD UL/DL configuration | Number of HARQ processes for normal HARQ operation | Number of HARQ processes for subframe bundling operation |
| --- | --- | --- |
| 0 | 9 | N/A |
| 1 | 6 | N/A |
| 2 | 4 | 2 |
| 3 | 4 | 2 |
| 4 | 3 | N/A |
| 5 | 2 | N/A |
| 6 | 8 | N/A |

For TDD, if the UE is not configured with EIMTA-MainConfigServCell-r12 for any serving cell, and if a UE is configured with one serving cell, or if the UE is configured with more than one serving cell and the TDD UL/DL configuration of all the configured serving cells is the same,

-	For TDD UL/DL configurations 1-6 and normal HARQ operation and UE not configured with higher layer parameter symPUSCH-UpPts-r14 for the serving cell, the UE shall upon detection of a PDCCH/EPDCCH/SPDCCH with uplink DCI format in subframe/slot n and/or a PHICH transmission in subframe n intended for the UE, perform a corresponding PUSCH transmission in subframe/slot n+k, with k given in

-	Table 8-2m for special subframe configuration 1, 2, 3, 4, 6, 7, 8 if the UE is configured with higher layer parameter ul-STTI-Length, and the corresponding uplink DCI format is 7-0A/7-0B

-	Table 8-2n for special subframe configuration 0, 5, 9 if the UE is configured with higher layer parameter ul-STTI-Length, and the corresponding uplink DCI format is 7-0A/7-0B

-	For TDD UL/DL configuration 6 and for n=0, 1, 2, 10, 11, 12

-	If only the MSB of the UL index in the uplink DCI is set in slot n, the UE shall perform a corresponding PUSCH transmission in slot n+ k

-	If only the LSB of the UL index in the uplink DCI is set in slot n, the UE shall perform a corresponding PUSCH transmission in slot n+ k+1

-	If both the MSB and LSB of the UL index in the uplink DCI are set in slot n, the UE shall perform a corresponding PUSCH transmission in both slot n+ k and n+ k+1, where the HARQ process number of the PUSCH in slot n+k is  and the HARQ process number of the PUSCH in n+k+1 is  with  from the HARQ process number field in the corresponding DCI format.

-	Table 8-2i if the UE is configured with higher layer parameter shortProcessingTime and the corresponding PDCCH with DCI format 0/4 and with CRC scrambled by C-RNTI is in the UE-specific search space,

-	Table 8-2 otherwise,

according to the PDCCH/EPDCCH/SPDCCH and PHICH information if a transport block corresponding to the HARQ process of the PUSCH transmission is generated as described in [8].

-	For TDD UL/DL configuration 0 and normal HARQ operation the UE shall upon detection of a PDCCH/EPDCCH with uplink DCI format 0/4 and/or a PHICH transmission in subframe n intended for the UE, perform a corresponding PUSCH transmission in subframe n+k if a transport block corresponding to the HARQ process of the PUSCH transmission is generated as described in [8] and if the MSB of the UL index in the PDCCH/EPDCCH with uplink DCI format 0/4 is set to 1 or PHICH is received in subframe n=0 or 5 in the resource corresponding to , as defined in Clause 9.1.2, or PHICH is received in subframe n=1 or 6 corresponding to PUSCH transmission in subframe n-5 for UE configured with higher layer parameter symPUSCH-UpPts-r14 for the serving cell. If, for TDD UL/DL configuration 0 and normal HARQ operation, the LSB of the UL index in the DCI format 0/4 is set to 1 in subframe n or a PHICH is received in subframe n=0 or 5 in the resource corresponding to , as defined in Clause 9.1.2, or PHICH is received in subframe n=1 or 6 corresponding to PUSCH transmission in subframe n-4, the UE shall perform a corresponding PUSCH transmission in subframe n+ kp if a transport block corresponding to the HARQ process of the PUSCH transmission is generated as described in [8]. If, for TDD UL/DL configuration 0, both the MSB and LSB of the UL index in the PDCCH/EPDCCH with uplink DCI format 0/4 are set in subframe n, the UE shall perform a corresponding PUSCH transmission in both subframes n+ k and n+ kp if a transport block corresponding to the HARQ process of the PUSCH transmission is generated as described in [8], with k given in

-	Table 8-2g if the UE is configured with higher layer parameter symPUSCH-UpPts-r14 and the UE is either not configured with higher layer parameter shortProcessingTime for the serving cell or is configured with higher layer parameter shortProcessingTime for the serving cell and the corresponding PDCCH is in the common search space,

-	Table 8-2i if the UE is not configured with higher layer parameter symPUSCH-UpPts-r14 and the UE is configured with higher layer parameter shortProcessingTime for the serving cell and the corresponding PDCCH with CRC scrambled by C-RNTI is in the UE-specific search space,

-	Table 8-2j if the UE is configured with higher layer parameters symPUSCH-UpPts-r14 and shortProcessingTime for the serving cell and the corresponding PDCCH with CRC scrambled by C-RNTI is in the UE-specific search space,

-	Table 8-2 otherwise.

if the UE is configured with higher layer parameters symPUSCH-UpPts-r14 and shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI is in the UE-specific search space, or if n=1 or 6 and the UE is not configured with higher layer parameter symPUSCH-UpPts-r14 but is configured with shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI is in the UE-specific search space, otherwise.

In case the UE is configured with higher layer parameter shortProcessingTime for the serving cell and both the MSB and LSB of the UL index in the PDCCH with uplink DCI format 0/4 with the UE's C-RNTI in the UE-specific search space are set to 1, the HARQ process number of the PUSCH in subframe n+k is  and the HARQ process number of the PUSCH in subframe n+kp is , where  is determined according to the HARQ process number field in the corresponding DCI format and MUL_HARQ is the number of UL HARQ processes per cell for transmission mode 1 and half the number of UL HARQ processes per cell for transmission mode 2.

-	For TDD UL/DL configuration 0 and normal HARQ operation the UE shall upon detection of a PDCCH/SPDCCH with uplink DCI format 7-0A/7-0B in slot n intended for the UE, perform a corresponding PUSCH transmission in slot n+k if a transport block corresponding to the HARQ process of the PUSCH transmission is generated as described in [8], with k given in

-	Table 8-2m for special subframe configuration 1, 2, 3, 4, 6, 7, 8, and in Table 8-2n for special subframe configuration 0, 5, 9

-	If only the MSB of the UL index in the PDCCH/SPDCCH with uplink DCI format 7-0A/7-0B is set in slot n, the UE shall perform a corresponding PUSCH transmission in slot n+ k

-	If only the LSB of the UL index in the PDCCH/SPDCCH with uplink DCI format 7-0A/7-0B is set in slot n, the UE shall perform a corresponding PUSCH transmission in slot n+ k+1

-	If both the MSB and LSB of the UL index in the PDCCH/SPDCCH with uplink DCI format 7-0A/7-0B are set in slot n, the UE shall perform a corresponding PUSCH transmission in both slot n+ k and n+ k+1, where the HARQ process number of the PUSCH in slot n+k is  and the HARQ process number of the PUSCH in n+k+1 is  with  from the HARQ process number field in the corresponding DCI format.

-	The UE is not expected to receive LSB of the UL index in PDCCH/SPDCCH with uplink DCI format set to 1 in slot n=0, 1, 10 and 11 for special subframe configuration 1, 2, 3, 4, 6, 7, 8

-	Table 8-2p if the UE is configured with higher layer parameter symPUSCH-UpPts-r14 for the serving cell

-	If UL index in the PDCCH/SPDCCH with uplink DCI format 7-0A/7-0B in slot n=2 or n=12 is set to

-	'10', the UE shall perform a corresponding PUSCH transmission in slot n+k

-	'01', the UE shall perform a corresponding PUSCH transmission in slot n+ k+1

-	'11', the UE shall perform a corresponding PUSCH transmission in slot n+ k+5

-	'00', the UE shall perform a corresponding PUSCH transmission in slot n+ k, n+k+1, and n+k+5, where the HARQ process number of the PUSCH in slot n+k is , the HARQ process number of the PUSCH in n+k+1 is , and the HARQ process number of the PUSCH in n+k+5 is  with  from the HARQ process number field in the corresponding DCI format.

-	If UL index in the PDCCH/SPDCCH with uplink DCI format 7-0A/7-0B in slot n=0, 1, 10 or 11 is set to

-	'10', the UE shall perform a corresponding PUSCH transmission in slot n+ k

-	'01', the UE shall perform a corresponding PUSCH transmission in slot n+ k+1

-	'11', the UE shall perform a corresponding PUSCH transmission in slot n+ k and n+k+1, where the HARQ process number of the PUSCH in slot n+k is  and the HARQ process number of the PUSCH in n+k+1 is with  from the HARQ process number field in the corresponding DCI format.

-	For TDD UL/DL configurations 1-5 and normal HARQ operation and UE configured with higher layer parameter symPUSCH-UpPts-r14 for the serving cell, the UE shall upon detection of a PDCCH/EPDCCH/SPDCCH with uplink DCI format in subframe/slot n intended for the UE, and/or a PHICH transmission intended for the UE in subframe n+l with l given in Table 8-2h, perform a corresponding PUSCH transmission in subframe/slot n+k, with k given in Table 8-2j if the UE is configured with higher layer parameter shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI has DCI Format 0/4 and is in the UE-specific search space, Table 8-2p if the corresponding PDCCH/SPDCCH has DCI format 7-0A/7-0B, in Table 8-2g otherwise, according to the PDCCH/EPDCCH and/or PHICH information if a transport block corresponding to the HARQ process of the PUSCH transmission is generated as described in [8].

-	For TDD UL/DL configuration 6 and normal HARQ operation and UE configured with higher layer parameter symPUSCH-UpPts-r14 for the serving cell, the UE shall upon detection of a PDCCH/EPDCCH with uplink DCI format 0/4 and/or a PHICH transmission in subframe n intended for the UE, perform a corresponding PUSCH transmission in subframe n+k if a transport block corresponding to the HARQ process of the PUSCH transmission is generated as described in [8] and if the MSB of the UL index in the PDCCH/EPDCCH with uplink DCI format 0/4 is set to 1 or PHICH is received in subframe n=1 or 6 or 9, or PHICH is received in subframe n=0 corresponding to PUSCH transmission in subframe n-6, or PHICH is received in subframe n=5 corresponding to PUSCH transmission in subframe n-7, with k given in Table 8-2j if the UE is configured with higher layer parameter shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI has DCI format 0/4 and is in the UE-specific search space, in Table 8-2g otherwise. If, for TDD UL/DL configuration 6 and normal HARQ operation, the LSB of the UL index in the DCI format 0/4 is set to 1 in subframe n, or PHICH is received in subframe n=0 or 5 corresponding to PUSCH transmission in subframe n-4, the UE shall perform a corresponding PUSCH transmission in subframe n+ kp if a transport block corresponding to the HARQ process of the PUSCH transmission is generated as described in [8]. If, for TDD UL/DL configuration 6, both the MSB and LSB of the UL index in the PDCCH/EPDCCH with uplink DCI format 0/4 are set in subframe n, the UE shall perform a corresponding PUSCH transmission in both subframes n+ k and n+ kp if a transport block corresponding to the HARQ process of the PUSCH transmission is generated as described in [8], with k given in Table 8-2j if the UE is configured with higher layer parameter shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI has DCI format 0/4 and is in the UE-specific search space, in Table 8-2g otherwise. In case the UE is configured with higher layer parameter shortProcessingTime for the serving cell and both the MSB and LSB of the UL index in the PDCCH with CRC scrambled by C-RNTI has DCI format 0/4 in the UE-specific search space are set to 1, the HARQ process number of the PUSCH in subframe n+k is  and the HARQ process number of the PUSCH in subframe n+kp is , where  is determined according to the HARQ process number field in the corresponding DCI format and MUL_HARQ is the number of UL HARQ processes per cell for transmission mode 1 and half the number of UL HARQ processes per cell for transmission mode 2. Note that is given as,

-	 if  or 9 and the UE is configured with higher layer parameter shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI is in the UE-specific search space,

-	 if n=1, 5, or 6 and the UE is configured with higher layer parameter shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI is in the UE-specific search space,

-	otherwise.

The UE is not expected to receive LSB of the UL index in PDCCH/EPDCCH with uplink DCI format 0/4 set to 1 in subframe n=9 unless the UE is configured with higher layer parameter shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI has DCI format 0/4 in the UE-specific search space.

-	For TDD UL/DL configuration 6 and normal HARQ operation and the UE is configured with higher layer parameter symPUSCH-UpPts-r14 for the serving cell, the UE shall upon detection of a PDCCH/SPDCCH with uplink DCI format 7-0A/7-0B in slot n intended for the UE, perform a corresponding PUSCH transmission in slot n+k if a transport block corresponding to the HARQ process of the PUSCH transmission is generated as described in [8], with k given in Table 8-2p

-	If UL index in the PDCCH/SPDCCH with uplink DCI format 7-0A/7-0B in slot n=2 is set to

-	'10', the UE shall perform a corresponding PUSCH transmission in slot n+k

-	'01', the UE shall perform a corresponding PUSCH transmission in slot n+ k+1

-	'11', the UE shall perform a corresponding PUSCH transmission in slot n+ k+5

-	'00', the UE shall perform a corresponding PUSCH transmission in slot n+ k, n+k+1, and n+k+5, where the HARQ process number of the PUSCH in slot n+k is , the HARQ process number of the PUSCH in n+k+1 is , and the HARQ process number of the PUSCH in n+k+5 is  with  from the HARQ process number field in the corresponding DCI format.

-	If UL index in the PDCCH/SPDCCH with uplink DCI format 7-0A/7-0B in slot n=0, 1, 10, 11, 12, 19 is set to

-	'10', the UE shall perform a corresponding PUSCH transmission in slot n+ k

-	'01', the UE shall perform a corresponding PUSCH transmission in slot n+ k+1

-	'11', the UE shall perform a corresponding PUSCH transmission in slot n+ k and n+k+1, where the HARQ process number of the PUSCH in slot n+k is  and the HARQ process number of the PUSCH in n+k+1 is  with  from the HARQ process number field in the corresponding DCI format.

For TDD, if a UE is configured with more than one serving cell and the TDD UL/DL configuration of at least two configured serving cells is not the same or if the UE is configured with EIMTA-MainConfigServCell-r12 for at least one serving cell, or FDD-TDD,

-	For a serving cell with an UL-reference UL/DL configurations belonging to {1,2,3,4,5,6} and normal HARQ operation and UE not configured with higher layer parameter symPUSCH-UpPts-r14 for the serving cell, the UE shall upon detection of a PDCCH/EPDCCH with uplink DCI format 0/4 and/or a PHICH transmission in subframe n intended for the UE, perform a corresponding PUSCH transmission in subframe n+k for the serving cell according to the PDCCH/EPDCCH and/or PHICH information if a transport block corresponding to the HARQ process of the PUSCH transmission is generated as described in [8], with k given in Table 8-2i if the UE is configured with higher layer parameter shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI has DCI format 0/4 in the UE-specific search space, in Table 8-2 otherwise, where the "TDD UL/DL Configuration" given in Table 8-2 refers to the UL-reference UL/DL configuration.

-	For a serving cell with UL-reference UL/DL configuration 0 and normal HARQ operation the UE shall upon detection of a PDCCH/EPDCCH with uplink DCI format 0/4 and/or a PHICH transmission in subframe n intended for the UE, perform a corresponding PUSCH transmission in subframe n+k for the serving cell if a transport block corresponding to the HARQ process of the PUSCH transmission is generated as described in [8] and if the MSB of the UL index in the PDCCH/EPDCCH with uplink DCI format 0/4 is set to 1 or PHICH is received in subframe n=0 or 5 in the resource corresponding to , as defined in Clause 9.1.2, or PHICH is received in subframe n=1 or 6 corresponding to PUSCH transmission in subframe n-5 for UE configured with higher layer parameter symPUSCH-UpPts-r14 for the serving cell. If, for a serving cell with UL-reference UL/DL configuration 0 and normal HARQ operation, the LSB of the UL index in the DCI format 0/4 is set to 1 in subframe n or a PHICH is received in subframe n=0 or 5 in the resource corresponding to , as defined in Clause 9.1.2, or PHICH is received in subframe n=1 or 6 corresponding to PUSCH transmission in subframe n-4, the UE shall perform a corresponding PUSCH transmission in subframe n+ kp for the serving cell if a transport block corresponding to the HARQ process of the PUSCH transmission is generated as described in [8]. If, for a serving cell with UL-reference UL/DL configuration 0, both the MSB and LSB of the UL index in the PDCCH/EPDCCH with uplink DCI format 0/4 are set in subframe n, the UE shall perform a corresponding PUSCH transmission in both subframes n+ k and n+ kp for the serving cell if a transport block corresponding to the HARQ process of the PUSCH transmission is generated as described in [8]. In case the UE is configured with higher layer parameter shortProcessingTime for the serving cell and both the MSB and LSB of the UL index in the PDCCH with uplink DCI format 0/4 with the UE's C-RNTI in the UE-specific search space are set to 1, the HARQ process number of the PUSCH in subframe n+k is  and the HARQ process number of the PUSCH in subframe n+kp is , where  is determined according to the HARQ process number field in the corresponding DCI format and MUL_HARQ is the number of UL HARQ processes per cell for transmission mode 1 and half the number of UL HARQ processes per cell for transmission mode 2. Note that k is given in

-	Table 8-2gif the UE is configured with higher layer parameter symPUSCH-UpPts-r14 and the UE is either not configured with higher layer parameter shortProcessingTime for the serving cell or is configured with higher layer parameter shortProcessingTime for the serving cell and the corresponding PDCCH with CRC scrambled by C-RNTI is in the common search space,

-	Table 8-2i if the UE is not configured with higher layer parameter symPUSCH-UpPts-r14 and the UE is configured with higher layer parameter shortProcessingTime for the serving cell and the corresponding PDCCH with CRC scrambled by C-RNTI is in the UE-specific search space,

-	Table 8-2j if the UE is configured with higher layer parameters symPUSCH-UpPts-r14 and shortProcessingTime for the serving cell and the corresponding PDCCH with CRC scrambled by C-RNTI is in the UE-specific search space,

-	Table 8-2 otherwise,

where the "TDD UL/DL Configuration" given in Table 8-2, Table 8-2g, Table 8-2i, Table 8-2j refers to the UL-reference UL/DL configuration. Note that if the UE is configured with higher layer parameters symPUSCH-UpPts-r14 and shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI is in the UE-specific search space, or if n =1 or 6 and the UE is not configured with higher layer parameter symPUSCH-UpPts-r14 but is configured with shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI is in the UE-specific search space, otherwise.

-	For a serving cell with an UL-reference UL/DL configurations belonging to {1,2,3,4,5} and normal HARQ operation and UE configured with higher layer parameter symPUSCH-UpPts-r14 for the serving cell, the UE shall upon detection of a PDCCH/EPDCCH with uplink DCI format 0/4 in subframe n intended for the UE, and/or a PHICH transmission intended for the UE in subframe n+l with l given in Table 8-2h, perform a corresponding PUSCH transmission in subframe n+k for the serving cell according to the PDCCH/EPDCCH and/or PHICH information if a transport block corresponding to the HARQ process of the PUSCH transmission is generated as described in [8], with k given in Table 8-2j if the UE is configured with higher layer parameter shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI has DCI format 0/4 in the UE-specific search space, in Table 8-2g otherwise, where the "TDD UL/DL Configuration" given in Table 8-2g, Table 8-2h and Table 8-2j refers to the UL-reference UL/DL configuration.

-	For a serving cell with UL-reference UL/DL configuration configuration 6 and normal HARQ operation and UE configured with higher layer parameter symPUSCH-UpPts-r14 for the serving cell, the UE shall upon detection of a PDCCH/EPDCCH with uplink DCI format 0/4 and/or a PHICH transmission in subframe n intended for the UE, perform a corresponding PUSCH transmission in subframe n+k if a transport block corresponding to the HARQ process of the PUSCH transmission is generated as described in [8] and if the MSB of the UL index in the PDCCH/EPDCCH with uplink DCI format 0/4 is set to 1 or PHICH is received in subframe n=1 or 6 or 9, or PHICH is received in subframe n=0 corresponding to PUSCH transmission in subframe n-6, or PHICH is received in subframe n=5 corresponding to PUSCH transmission in subframe n-7, with k given in Table 8-2j if the UE is configured with higher layer parameter shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI has DCI format 0/4 in the UE-specific search space, in Table 8-2g otherwise. If, for a serving cell with UL-reference UL/DL configuration 6 and normal HARQ operation, the LSB of the UL index in the DCI format 0/4 is set to 1 in subframe n, or PHICH is received in subframe n=0 or 5 corresponding to PUSCH transmission in subframe n-4, the UE shall perform a corresponding PUSCH transmission in subframe n+ kp if a transport block corresponding to the HARQ process of the PUSCH transmission is generated as described in [8]. If, for a serving cell with UL-reference UL/DL configuration 6, both the MSB and LSB of the UL index in the PDCCH/EPDCCH with uplink DCI format 0/4 are set in subframe n, the UE shall perform a corresponding PUSCH transmission in both subframes n+ k and n+ kp if a transport block corresponding to the HARQ process of the PUSCH transmission is generated as described in [8], with k given in Table 8-2j if the UE is configured with higher layer parameter shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI has DCI format 0/4 in the UE-specific search space, in Table 8-2g otherwise, where the "TDD UL/DL Configuration" given in Table 8-2g and Table 8-2j refers to the UL-reference UL/DL configuration. In case the UE is configured with higher layer parameter shortProcessingTime for the serving cell and both the MSB and LSB of the UL index in the PDCCH with uplink DCI format 0/4 with the UE's C-RNTI in the UE-specific search space are set to 1, the HARQ process number of the PUSCH in subframe n+k is  and the HARQ process number of the PUSCH in subframe n+kp is , where  is determined according to the HARQ process number field in the corresponding DCI format and MUL_HARQ is the number of UL HARQ processes per cell for transmission mode 1 and half the number of UL HARQ processes per cell for transmission mode 2. Note that kp is given as,

-	 if n = 0 or 9 and the UE is configured with higher layer parameter shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI is in the UE-specific search space,

-	 if n=1, 5, or 6 and the UE is configured with higher layer parameter shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI is in the UE-specific search space,

-	otherwise.

The UE is not expected to receive LSB of the UL index in PDCCH/EPDCCH with uplink DCI format set to 1 in subframe n=9 unless the UE is configured with higher layer parameter shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI is in the UE-specific search space.

For TDD UL/DL configurations 1, 2, 3 and 6 and subframe bundling operation, the UE shall upon detection of a PDCCH/EPDCCH with DCI format 0 in subframe n intended for the UE, and/or a PHICH transmission intended for the UE in subframe n-l with l given in Table 8-2a, perform a corresponding first PUSCH transmission in the bundle in subframe n+k according to the PDCCH/EPDCCH and/or PHICH information if a transport block corresponding to the HARQ process of the first PUSCH transmission is generated as described in [8], with k given in Table 8-2 if the UE is not configured with higher layer parameter symPUSCH-UpPts-r14 for the serving cell, otherwise k given in Table 8-2g.

For TDD UL/DL configuration 0 and subframe bundling operation, the UE shall upon detection of a PDCCH/EPDCCH with DCI format 0 in subframe n intended for the UE, and/or a PHICH transmission intended for the UE in subframe n-l with l given in Table 8-2a, perform a corresponding first PUSCH transmission in the bundle in subframe n+k according to the PDCCH/EPDCCH and PHICH information if a transport block corresponding to the HARQ process of the first PUSCH transmission is generated as described in [8] and if the MSB of the UL index in the DCI format 0 is set to 1 or if , as defined in Clause 9.1.2, with k given in Table 8-2. If, for TDD UL/DL configuration 0 and subframe bundling operation, the LSB of the UL index in the PDCCH/EPDCCH with DCI format 0 is set to 1 in subframe n or if , as defined in Clause 9.1.2, the UE shall perform a corresponding first PUSCH transmission in the bundle in subframe n+7, according to the PDCCH/EPDCCH and PHICH information if a transport block corresponding to the HARQ process of the first PUSCH transmission is generated as described in [8].

Table 8-2: k for TDD configurations 0-6

| TDD UL/DL Configuration | subframe number n |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 | 4 | 6 |  |  |  | 4 | 6 |  |  |  |
| 1 |  | 6 |  |  | 4 |  | 6 |  |  | 4 |
| 2 |  |  |  | 4 |  |  |  |  | 4 |  |
| 3 | 4 |  |  |  |  |  |  |  | 4 | 4 |
| 4 |  |  |  |  |  |  |  |  | 4 | 4 |
| 5 |  |  |  |  |  |  |  |  | 4 |  |
| 6 | 7 | 7 |  |  |  | 7 | 7 |  |  | 5 |

Table 8-2a: l for TDD configurations 0, 1, 2, 3 and 6

| TDD UL/DL Configuration | subframe number n |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 | 9 | 6 |  |  |  | 9 | 6 |  |  |  |
| 1 |  | 2 |  |  | 3 |  | 2 |  |  | 3 |
| 2 |  | 3 |  | 0 |  |  | 3 |  | 0 |  |
| 3 | 1 |  |  |  |  |  |  | 7 | 0 | 1 |
| 6 | 5 | 5 |  |  |  | 6 | 6 |  |  | 8 |

Table 8-2g: k for TDD configurations 0-6 and UE configured with symPUSCH-UpPts-r14

| TDD UL/DL Configuration | subframe number n |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 | 4 | 5 |  |  |  | 4 | 5 |  |  |  |
| 1 | 6 | 6 |  |  | 4 | 6 | 6 |  |  | 4 |
| 2 |  | 5 |  | 4 |  |  | 5 |  | 4 |  |
| 3 | 4 |  |  |  |  |  |  | 4 | 4 | 4 |
| 4 |  |  |  |  |  |  |  | 4 | 4 | 4 |
| 5 |  |  |  |  |  |  |  | 4 | 4 |  |
| 6 | 7 | 7 |  |  |  | 7 | 7 |  |  | 5 |

Table 8-2h: l for TDD configurations 1-5 and UE configured with symPUSCH-UpPts-r14

| TDD UL/DL Configuration | subframe number n |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 1 | 1 | 0 |  |  | 0 | 1 | 0 |  |  | 0 |
| 2 |  | 2 |  | 0 |  |  | 2 |  | 0 |  |
| 3 | 0 |  |  |  |  |  |  | 1 | 0 | 0 |
| 4 |  |  |  |  |  |  |  | 1 | 0 | 0 |
| 5 |  |  |  |  |  |  |  | 1 | 0 |  |

Table 8-2i: k for TDD configurations 0-6 and UE configured with shortProcessingTime

| TDD UL/DL Configuration | subframe number n |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 | 3 | 3 |  |  |  | 3 | 3 |  |  |  |
| 1 | 3 |  |  |  | 3 | 3 |  |  |  | 3 |
| 2 |  |  |  |  | 3 |  |  |  |  | 3 |
| 3 | 3 | 3 |  |  |  |  |  |  |  | 3 |
| 4 | 3 |  |  |  |  |  |  |  |  | 3 |
| 5 |  |  |  |  |  |  |  |  |  | 3 |
| 6 | 4 | 6 |  |  |  | 3 | 6 |  |  | 4 |

Table 8-2j: k for TDD configurations 0-6 UE configured with shortProcessingTime and with symPUSCH-UpPts-r14

| TDD UL/DL Configuration | subframe number n |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 | 3 | 3 |  |  |  | 3 | 3 |  |  |  |
| 1 | 3 | 5 |  |  | 3 | 3 | 5 |  |  | 3 |
| 2 |  |  |  | 3 | 3 |  |  |  | 3 | 3 |
| 3 | 3 | 3 |  |  |  |  |  |  | 3 | 3 |
| 4 | 3 |  |  |  |  |  |  |  | 3 | 3 |
| 5 |  |  |  |  |  |  |  |  | 3 | 3 |
| 6 | 3 | 5 |  |  |  | 3 | 5 |  |  | 3 |

Table 8-2m: k for TDD configurations 0-6, special subframe configuration 1,2,3,4,6,7,8 and UE configured with ul-STTI-Length

| TDD UL/DL Configuration | slot number n |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| 0 | 4 | 4 | 4 | 5 |  |  |  |  |  |  | 4 | 4 | 4 | 5 |  |  |  |  |  |  |
| 1 | 4 | 4 | 4 | 4 |  |  |  |  |  |  | 4 | 4 | 4 | 4 |  |  |  |  |  |  |
| 2 | 4 | 4 |  |  |  |  |  |  |  |  | 4 | 4 |  |  |  |  |  |  |  |  |
| 3 | 6 | 6 | 6 | 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6 | 6 |
| 4 | 4 | 4 | 4 | 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 | 4 | 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 | 6 | 6 | 6 | 6 |  |  |  |  |  |  | 4 | 4 | 4 | 4 |  |  |  |  | 6 | 6 |

Table 8-2n: k for TDD configurations 0-6, special subframe configuration 0,5,9 and UE configured with ul-STTI-Length

| TDD UL/DL Configuration | slot number n |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| 0 | 4 | 5 | 6 |  |  |  |  |  |  |  | 4 | 5 | 6 |  |  |  |  |  |  |  |
| 1 | 5 | 5 | 5 |  |  |  |  |  |  | 5 | 5 | 5 | 5 |  |  |  |  |  |  | 5 |
| 2 | 4 | 4 |  |  |  |  |  |  |  |  | 4 | 4 |  |  |  |  |  |  |  |  |
| 3 | 7 | 7 | 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 | 7 | 7 |
| 4 | 5 | 5 | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5 |
| 5 | 4 | 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 | 4 | 5 | 6 |  |  |  |  |  |  |  | 4 | 4 | 4 |  |  |  |  |  |  |  |

Table 8-2p: k for TDD configurations 0-6, UE configured with symPUSCH-UpPts-r14, and ul-STTI-Length

| TDD UL/DL Configuration | slot number n |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| 0 | 4 | 5 | 6 |  |  |  |  |  |  |  | 4 | 5 | 6 |  |  |  |  |  |  |  |
| 1 | 5 | 5 | 5 |  |  |  |  |  | 5 | 5 | 5 | 5 | 5 |  |  |  |  |  | 5 | 5 |
| 2 | 4 | 4 |  |  |  |  |  |  |  | 4 | 4 | 4 |  |  |  |  |  |  |  | 4 |
| 3 | 7 | 7 | 7 |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 | 7 | 7 | 7 |
| 4 | 5 | 5 | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5 | 5 |
| 5 | 4 | 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4 |
| 6 | 4 | 5 | 6 |  |  |  |  |  |  |  | 4 | 4 | 4 |  |  |  |  |  |  | 4 |

For BL/CE UEs, the set of BL/CE UL subframes is indicated as follows

-	If UL resource reservation is enabled for the UE as specified in [11],

-	for PUSCH transmission associated with C-RNTI or SPS C-RNTI using UE-specific MPDCCH search space including PUSCH transmission without a corresponding MPDCCH,

-	if the Resource reservation field in the DCI is set to 0, then the set of BL/CE UL subframes corresponds to all uplink subframes during the PUSCH transmission;

-	if the Resource reservation field in the DCI is set to 1, then the set of BL/CE UL subframes corresponds to all uplink subframes that are not fully reserved according to higher layer parameters (a subframe is considered fully reserved if and only if all SC-FDMA symbols of the PUSCH transmission are reserved in the subframe);

-	for PUCCH transmission associated with C-RNTI or SPS C-RNTI using UE-specific MPDCCH search space including PUSCH transmission without a corresponding MPDCCH,

-	the set of BL/CE UL subframes corresponds to all uplink subframes that are not fully reserved according to higher layer parameters (a subframe is considered fully reserved if and only if all SC-FDMA symbols of the PUCCH transmission are reserved in the subframe).

-	In all other cases, the set of BL/CE UL subframes is indicated by the higher layers according to fdd-DownlinkOrTddSubframeBitmapBR and fdd-UplinkSubframeBitmapBR [11].

For BL/CE UEs, PUSCH transmission can be scheduled by a MPDCCH with DCI format 6-0A/6-0B, or the transmission can correspond to using preconfigured uplink resource configured by higher layers. Transmission using preconfigured uplink resource is initiated by higher layers as specified in [14], while retransmission of transport blocks transmitted using preconfigured uplink resource are scheduled by a MPDCCH with DCI format 6-0A/6-0B.

For a PUSCH transmission using preconfigured uplink resource, the UE shall use the repetition number configured by higher layers.

A BL/CE UE shall upon detection on a given serving cell of an MPDCCH with DCI format 6-0A/6-0B scheduling PUSCH intended for the UE, perform a corresponding PUSCH transmission in subframe(s) ni = n+ki +Koffset if a transport block(s) corresponding to the HARQ process(es) of the PUSCH transmission is generated as described in [8] with i = 0, 1, …, NTBN-1 according to the MPDCCH, where

-	subframe n is the last subframe in which the MPDCCH is transmitted;

-	the value of is the number of scheduled TB determined by the corresponding DCI if present,  otherwise;

-	 and the value of  is determined by the repetition number field in the corresponding DCI, where

-	if the UE is configured with higher layer parameter ce-pdsch-puschEnhancement-config with value 'On' are given by {1,2,4,8,12,16,24,32}

-	otherwise, are given in Table 8-2b and Table 8-2c; and

-	if the UE is configured with higher layer parameter ce-PUSCH-SubPRB-Config-r15, and the PUSCH resource assignment in the corresponding DCI is using uplink resource allocation type 5,  where N ≤ 32 for CE Mode A and N ≤ 2048 for CE Mode B,  is defined in [3] and  is determined according to procedure in clause 8.1.6,  otherwise

-	in case N>1, subframe(s) n+ki with i=0,1,…, NTBN-1 are NTBN consecutive BL/CE UL subframe(s) starting with subframe n+x, and in case N=1, k0=x;

-	for ,

-	if the UE is configured with higher layer parameter interleaving in ce-PUSCH-MultiTB-Config, and PUSCH corresponding to a MPDCCH with DCI CRC scrambled by C-RNTI and

-	where C=MslotsUL2∙MRU if the UE is configured with higher layer parameter ce-PUSCH-SubPRB-Config-r15 and the PUSCH resource assignment is using uplink resource allocation type 5, otherwise  for UE configured with CEModeA, and  for UE configured with CEModeB,

-	BL/CE UL subframes  with  are associated with TBr+1 ,

-	otherwise,

-	BL/CE UL subframes  with  are associated with TBr+1 ,

-	the HARQ process ID for each of the scheduled  TBs are determined from the value of the 'HARQ index with offset' in the 'Scheduling TBs for Unicast' field for CEmodeA or the 'HARQ index' in the 'Scheduling TBs for Unicast' field for CEmodeB in the corresponding DCI which is a combinatorial index r defined as +roffset, where

-	the set , () contains the sorted HARQ process IDs and  is the extended binomial coefficient, resulting in unique label +roffset,

-	roffset is the offset value as defined in 5.3.3.1.10 of [4] for CE mode A, and roffset=0 for CE mode B,

-	 if UE is configured with CEModeA, and  if UE is configured with CEModeB.

-	for FDD, x = 4;

-	for TDD UL/DL configurations 1-6, or for TDD UL/DL configuration 0 and a BL/CE UE in CEModeB, the value of x is given as the value of k in Table 8-2 for the corresponding TDD UL/DL configuration; If the value x is not given in Table 8-2 for subframe n, denote subframe n' as the first downlink/special subframe which has a value in Table 8-2 after subframe n, and substitute n with n' in the above procedure for performing the PUSCH transmission.

-	for TDD UL/DL configuration 0 and a BL/CE UE in CEModeA, if the MSB of the UL index in the MPDCCH with DCI format 6-0A is set to 1, the value of x is given as the value of k in Table 8-2 for the corresponding TDD UL/DL configuration; if the LSB of the UL index in the MPDCCH with DCI format 6-0A is set to 1, x = 7. The UE is not expected to receive DCI format 6-0A with both the MSB and LSB of the UL index set to 1 when N>1 or ce-PUSCH-MultiTB-Config is configured. In case both the MSB and LSB of the UL index are set to 1, the HARQ process number of the PUSCH corresponding the MSB of the UL index is  and the HARQ process number of the PUSCH corresponding the LSB of the UL index is , where  is determined according to the HARQ process number field in DCI format 6-0A

-	The higher layer parameter ttiBundling is not applicable to BL/CE UEs.

-	For a BL/CE UE, in case a PUSCH transmission with a corresponding MPDCCH collides with a PUSCH transmission without a corresponding MPDCCH in a subframe n, the PUSCH transmission without a corresponding MPDCCH is dropped from subframe n.

-	For a BL/CE UE, in case of collision between at least one physical resource block to be used for PUSCH transmission and physical resource blocks corresponding to configured PRACH resources for BL/CE UEs or non-BL/CE UEs (defined in [3]) in a same subframe, the PUSCH transmission is dropped in that subframe.

-	For a BL/CE UE in half-duplex FDD operation, in case a PUSCH transmission including half-duplex guard subframe without a corresponding MPDCCH collides partially or fully with a PDSCH transmission with a corresponding MPDCCH, the PUSCH transmission without a corresponding MPDCCH is dropped.

-	For a BL/CE UE in half-duplex FDD operation and configured with ce-pdsch-puschEnhancement-config, in case a PUSCH transmission including half-duplex guard subframe collides partially or fully with a PDSCH transmission without a corresponding MPDCCH, the PUSCH transmission is dropped.

For BL/CE UEs, and for a PUSCH transmission starting in subframe n+ k0 without a corresponding MPDCCH, the UE shall adjust the PUSCH transmission in subframe(s) n+ki with i = 0, 1, …, N-1, where

-	0≤k0<k1<…,kN-1 and the value of  is determined by the repetition number field in the activation DCI, where are given in Table 8-2b and Table 8-2c; and

-	if the UE is configured with higher layer parameter ce-PUSCH-SubPRB-Config-r15, and the PUSCH resource assignment in the activation DCI is using uplink resource allocation type 5,  where  is defined in [3] and  is determined according to procedure in clause 8.1.6,  otherwise

-	in case N>1, subframe(s) n+ki with i=0,1,…,N-1 are N consecutive BL/CE UL subframe(s), and in case N=1, k0=0;

A BL/CE UE configured with mpdcch-UL-HARQ-ACK-FeedbackConfig shall upon detection on a given serving cell of an MPDCCH with DCI format 6-0A/6-0B intended for the UE in the UE-specific search space indicating HARQ-ACK(s) corresponding to transport block(s) associated to HARQ process(es) in the most recent PUSCH transmission with N>1, drop the remaining PUSCH transmission(s) (if any) corresponding to the transport block(s) no later than subframe n+k+Koffset, where

-	subframe n is the last subframe in which the MPDCCH is transmitted; and

- for FDD, k = 4;

- for TDD the value of k is given in Table 8-2 for the corresponding TDD UL/DL configuration; If the value of k is not given in Table 8-2 for subframe n, denote subframe n' as the first downlink/special subframe which has a value in Table 8-2 after subframe n, and substitute n with n' in the above procedure;

- value of is determined by the repetition number field in the corresponding DCI associated with the most recent PUSCH transmission;

- if the UE is configured with higher layer parameter ce-PUSCH-SubPRB-Config-r15, and the PUSCH resource assignment in the corresponding DCI associated with the most recent PUSCH transmission is using uplink resource allocation type 5,  where  is defined in [3] and  is determined according to procedure in clause 8.1.6,  otherwise.

For a BL/CE UE configured with mpdcch-UL-HARQ-ACK-FeedbackConfig, if the UE detects a first MPDCCH with DCI format 6-0A/6-0B intended for the UE scheduling PUSCH in subframes {s0,…,sN-1}, and if the UE detects a second MPDCCH with DCI format 6-0A/6-0B intended for the UE scheduling PUSCH in subframes {q0,…,qL-1} with M≤q0≤sN-1, where

-	for any HARQ ID that is indicated in both the first MPDCCH and the second MPDCCH, the NDI indicated in the second MPDCCH is toggled with respect to the NDI indicated in the first MPDCCH

-	for each HARQ ID i indicated in the first MPDCCH, the first subframe M in which the second MPDCCH is transmitted meets M≥s0,i+k, where  s0,i is the first subframe in which the HARQ ID i is transmitted

-	for FDD, k = 4,

-	for TDD the value of k is given in Table 8-2 for the corresponding TDD UL/DL configuration; If the value of k is not given in Table 8-2 for subframe n, denote subframe n' as the first downlink/special subframe which has a value in Table 8-2 after subframe n, and substitute n with n' in the above procedure

the UE shall

-	drop the remaining PUSCH transmission(s) of the transport block(s) scheduled by the first MPDCCH starting from subframe K, where M<K≤q0, and

-	deliver HARQ-ACK feedback corresponding to the transport block(s) scheduled by the first MPDCCH to higher layers, and

-	transmit the PUSCH scheduled by the second MPDCCH in subframes {q0,…,qL-1}

Table 8-2b: PUSCH repetition levels (DCI Format 6-0A)

| Higher layer parameter 'pusch-maxNumRepetitionCEmodeA' |  |
| --- | --- |
| Not configured | {1,2,4,8} |
| 16 | {1,4,8,16} |
| 32 | {1,4,16,32 } |

Table 8-2c: PUSCH repetition levels (DCI Format 6-0B)

| Higher layer parameter 'pusch-maxNumRepetitionCEmodeB' |  |
| --- | --- |
| Not configured | {4,8,16,32,64,128,256,512} |
| 192 | {1,4,8,16,32,64,128,192} |
| 256 | {4,8,16,32,64,128,192,256} |
| 384 | {4,16,32,64,128,192,256,384} |
| 512 | {4,16,64,128,192,256,384,512} |
| 768 | {8,32,128,192,256,384,512,768} |
| 1024 | {4,8,16,64,128,256,512,1024} |
| 1536 | {4,16,64,256,512,768,1024,1536} |
| 2048 | {4,16,64,128,256,512,1024,2048} |

A UE configured with parameter pusch-EnhancementsConfig shall upon detection on a given serving cell of an PDCCH/EPDCCH with DCI Format 0C intended for the UE, perform a corresponding PUSCH transmission in subframe(s) n+ki if a transport block corresponding to the HARQ process of the PUSCH transmission is generated as described in [8] with i = 0, 1, …, N-1 according to the PDCCH/EPDCCH, where

-	subframe n is the subframe in which the PDCCH/EPDCCH is transmitted; and

-	x≤k0<k1<…,kN-1 and the value of N is given by Table 8-2k based on the repetition number field in the corresponding DCI Format 0C; and

-	in case N>1, subframe(s) n+ki with i=0,1,…,N-1 are N consecutive UL subframe(s) starting with subframe n+x, and in case N=1, k0=x;

-	for FDD, x = 4;

-	for TDD UL/DL configurations 1-5 or for TDD UL/DL configuration 6 and a UE not configured with higher layer parameter symPUSCH-UpPts-r14, the value of x is given as the value of k in Table 8-2 for the corresponding TDD UL/DL configuration; If the value x is not given in Table 8-2 for subframe n, denote subframe n' as the first downlink/special subframe which has a value in Table 8-2 after subframe n, and substitute n with n' in the above procedure for performing the PUSCH transmission.

-	for TDD UL/DL configuration 0, if the MSB of the UL index in the PDCCH with DCI format 0C is set to 1, the value of x is given as the value of k in Table 8-2 for the corresponding TDD UL/DL configuration; if the LSB of the UL index in the PDCCH with DCI format 0C is set to 1, x = 7. The UE is not expected to receive DCI format 0C with both the MSB and LSB of the UL index set to 1 when N>1. In case both the MSB and LSB of the UL index are set to 1, the HARQ process number of the PUSCH corresponding the MSB of the UL index is  and the HARQ process number of the PUSCH corresponding the LSB of the UL index is , where  is determined according to the HARQ process number field in DCI format 0C

-	for TDD UL/DL configuration 6 and a UE configured with higher layer parameter symPUSCH-UpPTS-r14, if the MSB of the UL index in the PDCCH with DCI format 0C is set to 1, the value of x is given as the value of k in Table 8-2 for the corresponding TDD UL/DL configuration; if the LSB of the UL index in the PDCCH with DCI format 0C is set to 1, x = 6. The UE is not expected to receive DCI format 0C with both the MSB and LSB of the UL index set to 1 when N>1. In case both the MSB and LSB of the UL index are set to 1, the HARQ process number of the PUSCH corresponding the MSB of the UL index is  and the HARQ process number of the PUSCH corresponding the LSB of the UL index is , where  is determined according to the HARQ process number field in DCI format 0C

Table 8-2k: PUSCH repetition levels (DCI Format 0C)

| Repetition Number field in DCI Format 0C | Number of repetitions N |
| --- | --- |
| 000 | 1 |
| 001 | 2 |
| 010 | 4 |
| 011 | 8 |
| 100 | 12 |
| 101 | 16 |
| 110 | 24 |
| 111 | 32 |

For a serving cell that is a LAA SCell, a UE shall

-	upon detection of an PDCCH/ EPDCCH with DCI format 0A/0B/4A/4B and with 'PUSCH trigger A' field set to '0' in subframe n intended for the UE, or

-	upon detection of PDCCH/ EPDCCH with DCI format 0A/0B/4A/4B in subframe n-p with 'PUSCH trigger A' field set to '1' intended for the UE for the serving cell and that has not been triggered by a 'PUSCH trigger B' field set to '1' received prior to subframe n on the serving cell, with p>=1 and p<=v, and upon detection of PDCCH with DCI CRC scrambled by CC-RNTI and with 'PUSCH trigger B' field set to '1' in subframe n on the serving cell

perform a corresponding PUSCH transmission, conditioned on the channel access procedures described in clause 4.2.1 of [13], in subframe(s) n+l+k+i with i = 0, 1, …, N-1 according to the PDCCH/EPDCCH and HARQ process ID , where

-	N =1 for DCI format 0A/4A, and value of N is determined by the 'number of scheduled subframes' field in the corresponding DCI format 0B/4B.

-	The UE is configured the maximum value of N by higher layer parameter maxNumberOfSchedSubframes-Format0B for DCI format 0B and higher layer parameter maxNumberOfSchedSubframes-Format4B for DCI format 4B;

-	value of timing offset k is determined by the 'Timing offset' field in the corresponding DCI 0A/0B/4A/4B according to Table 8-2d if 'PUSCH trigger A' field set to '0' or Table 8-2e otherwise;

-	value of is determined by the HARQ process number field in the corresponding DCI format 0A/0B/4A/4B and ;

-	for 'PUSCH trigger A' field set to '0' in the corresponding DCI format 0A/0B/4A/4B,

-	l = 3 if the UE is configured with higher layer parameter shortProcessingTime, and 4 otherwise

-	otherwise

-	value of l is the UL offset as determined by the 'UL duration and offset' field in the corresponding DCI with CRC scrambled by CC-RNTI according to the procedure in Clause 13A, if 'PUSCH trigger B' field set to '1',

-	value of validation duration v is determined by the 'Timing offset' field in the corresponding PDCCH/ EPDCCH with DCI format 0A/0B/4A/4B according to Table 8-2f

-	the smallest value of l+k supported by the UE is included in the UE-EUTRA-Capability

-	the value of p+l+k is at least 3 if the UE is configured with higher layer parameter shortProcessingTime, and 4 otherwise.

Table 8-2d: Timing offset for DCI format 0A/0B/4A/4B with 'PUSCH trigger A' field set to '0'.

| Value of 'Timing offset' field |  |
| --- | --- |
| 0000 | 0 |
| 0001 | 1 |
| 0010 | 2 |
| 0011 | 3 |
| 0100 | 4 |
| 0101 | 5 |
| 0110 | 6 |
| 0111 | 7 |
| 1000 | 8 |
| 1001 | 9 |
| 1010 | 10 |
| 1011 | 11 |
| 1100 | 12 |
| 1101 | 13 |
| 1110 | 14 |
| 1111 | 15 |

Table 8-2e: Timing offset for DCI format 0A/0B/4A/4B with 'PUSCH trigger A' field set to '1'.

| Value of the first two bits of 'Timing offset' field |  |
| --- | --- |
| 00 | 0 |
| 01 | 1 |
| 10 | 2 |
| 11 | 3 |

Table 8-2f: Validation duration  for DCI format 0A/0B/4A/4B with 'PUSCH trigger A' field set to '1'.

| Value of the last two bits of 'Timing offset' field |  |
| --- | --- |
| 00 | 8 |
| 01 | 12 |
| 10 | 16 |
| 11 | 20 |

For a serving cell that is an LAA SCell, a UE that is configured with autonomous uplink transmissions on the serving cell may perform a corresponding PUSCH transmission in subframe n, if the following conditions are met:

-	subframe n is configured as usable for autonomous uplink transmission; and

-	the UE has not received a grant according to DCI Format 0A/0B/4A/4B for transmission in subframe n; and

-	autonomous uplink transmissions on the serving cell have been activated and not released according to the procedure described in clause 9.2A; and

-	subframe n is not in the DMTC window of the serving cell; and

-	channel access procedures described in clause 4.2.1 of [13] are followed to obtain channel access on the serving cell for subframe n.

-	Additionally, if subframe n is a subframe in which the UE is not required to receive any downlink physical channels and/or physical signals according to clause 13A, then in order to perform a corresponding PUSCH transmission the UE shall have detected a PDCCH with DCI CRC scrambled by CC-RNTI indicating that subframe n is shared with the UE.

For an LAA serving cell where a UE is performing an autonomous uplink transmission in one or more contiguous subframe(s) on all  resource blocks, for the first such subframe the UE randomly determines a timing offset  to be applied for transmission according to [3] from a set of values configured by higher layers according to the following rule:

-	If the first such subframe is a subframe in which the UE is not required to receive any downlink physical channels and/or physical signals, the set of values is determined by 30.72 * aul-startingFullBW-insideCOT;

-	otherwise, the set of values is determined by 30.72 * aul-startingFullBW-outsideCOT.

-	 is common for all carriers if more than one carrier is activated for autonomous uplink transmission.

For an LAA serving cell where a UE is performing an autonomous uplink transmission in one or more contiguous subframe(s) on fewer than  resource blocks, for the first such subframe the UE determines a timing offset  to be applied for transmission according to [3] according to the following rule:

-	If the first such subframe is a subframe in which the UE is not required to receive any downlink physical channels and/or physical signals,  is equal to 30.72 * aul-startingPartialBW-insideCOT;

-	otherwise,  is equal to 30.72 * aul-startingPartialBW-outsideCOT.

For a LAA SCell, a UE is not expected to receive more than one uplink scheduling grant for a subframe.

For a LAA SCell, the HARQ process ID shall be delivered to higher layers.

For a BL/CE UE, the HARQ process ID shall be delivered to higher layers.

If a UE is configured with higher layer parameter shortTTI or shortProcessingTime, the HARQ process ID shall be delivered to higher layers.

A UE is semi-statically configured via higher layer signalling to transmit PUSCH transmissions signalled via PDCCH/EPDCCH with DCI formats other than 7-0A/7-0B according to one of two uplink transmission modes, denoted mode 1 - 2. If a UE is configured with higher layer parameter ul-STTI-Length, the UE is semi-statically configured via higher layer signalling to transmit PUSCH transmissions signalled via PDCCH/SPDCCH with DCI formats 7-0A/7-0B according to one of two uplink transmission modes, denoted mode 1 - 2.

For a LAA SCell, the uplink transmission mode for autonomous uplink transmissions is configured independently from the uplink transmission mode for grant-based uplink transmissions. A UE is not expected to be configured with uplink transmission mode 2 for autonomous transmissions and uplink transmission mode 1 for grant-based uplink transmissions on the same LAA Scell.

If a UE is configured by higher layers to decode PDCCHs with the CRC scrambled by the C-RNTI, the UE shall decode the PDCCH according to the combination defined in Table 8-3 and transmit the corresponding PUSCH if a transport block corresponding to the HARQ process of the PUSCH transmission is generated as described in [8]. The scrambling initialization of this PUSCH corresponding to these PDCCHs and the PUSCH retransmission for the same transport block is by C-RNTI.

If a UE is configured by higher layers to decode EPDCCHs with the CRC scrambled by the C-RNTI, the UE shall decode the EPDCCH according to the combination defined in Table 8-3A and transmit the corresponding PUSCH if a transport block corresponding to the HARQ process of the PUSCH transmission is generated as described in [8]. The scrambling initialization of this PUSCH corresponding to these EPDCCHs and the PUSCH retransmission for the same transport block is by C-RNTI.

If a UE is configured with higher layer parameter shortTTI and the UE is configured by higher layers to decode SPDCCH with the CRC scrambled by the C-RNTI, the UE shall decode the SPDCCH according to the combination defined in Table 8-3C and transmit the corresponding PUSCH if a transport block corresponding to the HARQ process of the PUSCH transmission is generated as described in [8]. The scrambling initialization of this PUSCH corresponding to these SPDCCHs and the PUSCH retransmission for the same transport block is by C-RNTI.

If a UE is configured with a higher layer parameter pusch-EnhancementsConfig, the UE shall decode PDCCH/EPDCCH DCI format 0C in UE specific search space. In this case the UE is not required to decode/monitor DCI format 0 in the UE specific search space.

If a UE is configured with a higher layer parameter shortTTI, the UE shall decode PDCCH DCI format 7-0A/7-0B in UE specific search space.

If a UE is configured with a higher layer parameter pusch-EnhancementsConfig, the UE may assume that PDCCH/EPDCCH for a PUSCH retransmission of a transport block will occur in the UE specific search space if the PDCCH/EPDCCH for the corresponding initial PUSCH transmission for the same transport block was decoded in the UE specific search space.

If a UE is configured with a higher layer parameter pusch-EnhancementsConfig, the UE may assume that PDCCH/EPDCCH for a PUSCH retransmission of a transport block will occur in the common search space if the PDCCH/EPDCCH for the corresponding initial PUSCH transmission for the same transport block was decoded in the common search space.

If a UE is configured with higher layer parameter pusch-EnhancementsConfig, and the UE decodes a DCI format 0 in the common search space, the UE shall calculate the HARQ ID nHARQ for the corresponding PUSCH transmission in subframe ns2 and radio frame nf as:

- For a transmission in a normal uplink subframe, nHARQ=xHARQns2+(xMAX+1)×nfmod MHARQ, where

  - For FDD, xHARQn=n, and xmax=9

  - For TDD, xHARQn is given by Table 8-2q, and xMAX=maxxHARQn.

  - MHARQ is the number of HARQ processes, which is MHARQ=8 for FDD, and given by the "Normal HARQ operation" column in table 8-1, in the case of TDD.

- For a transmission in a special subframe, nHARQ=MHARQ if the transmission happens in the first special subframe of the radio frame, and nHARQ=MHARQ+1 otherwise.

Table 8-2q: xHARQ for TDD

| TDD UL/DL Configuration | subframe number n |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 |  |  | 0 | 1 | 2 |  |  | 3 | 4 | 5 |
| 1 |  |  | 0 | 1 |  |  |  | 2 | 3 |  |
| 2 |  |  | 0 |  |  |  |  | 1 |  |  |
| 3 |  |  | 0 | 1 | 2 |  |  |  |  |  |
| 4 |  |  | 0 | 1 |  |  |  |  |  |  |
| 5 |  |  | 0 |  |  |  |  |  |  |  |
| 6 |  |  | 0 | 1 | 2 |  |  | 3 | 4 |  |

The UE may for handover purposes, and before acquiring the SFN at the target cell, assume an absolute value of the relative time difference between radio frame  in the current cell and the target cell of less than  when determining nHARQ.

If a UE is configured by higher layers to decode MPDCCHs with the CRC scrambled by the C-RNTI, the UE shall decode the MPDCCH according to the combination defined in Table 8-3B and transmit the corresponding PUSCH if a transport block corresponding to the HARQ process of the PUSCH transmission is generated as described in [8]. The scrambling initialization of this PUSCH corresponding to these MPDCCHs and the PUSCH retransmission for the same transport block is by C-RNTI.

Transmission mode 1 is the default uplink transmission mode for a UE until the UE is assigned an uplink transmission mode by higher layer signalling.

When a UE configured in transmission mode 2 receives a DCI Format 0/0A/0B/0C uplink scheduling grant, it shall assume that the PUSCH transmission is associated with transport block 1 and that transport block 2 is disabled.

Table 8-3: PDCCH and PUSCH configured by C-RNTI

| Transmission mode | DCI format | Search Space | Transmission scheme of PUSCH corresponding to PDCCH |
| --- | --- | --- | --- |
| Mode 1 | DCI format 0 | Common and UE specific by C-RNTI | Single-antenna port, port 10 (see Clause 8.0.1) |
|  | DCI format 0A or 0B or 0C or 7-0A | UE specific by C-RNTI | Single-antenna port, port 10 (see Clause 8.0.1) |
| Mode 2 | DCI format 0 | Common and UE specific by C-RNTI | Single-antenna port, port 10 (see Clause 8.0.1) |
|  | DCI format 0A or 0B or 0C | UE specific by C-RNTI | Single-antenna port, port 10 (see Clause 8.0.1) |
|  | DCI format 4 or 4A or 4B or 7-0B | UE specific by C-RNTI | Closed-loop spatial multiplexing (see Clause 8.0.2) |

Table 8-3A: EPDCCH and PUSCH configured by C-RNTI

| Transmission mode | DCI format | Search Space | Transmission scheme of PUSCH corresponding to EPDCCH |
| --- | --- | --- | --- |
| Mode 1 | DCI format 0 or 0A or 0B or 0C | UE specific by C-RNTI | Single-antenna port, port 10 (see Clause 8.0.1) |
| Mode 2 | DCI format 0 or 0A or 0B or 0C | UE specific by C-RNTI | Single-antenna port, port 10 (see Clause 8.0.1) |
|  | DCI format 4 or 4A or 4B | UE specific by C-RNTI | Closed-loop spatial multiplexing (see Clause 8.0.2) |

Table 8-3B: MPDCCH and PUSCH configured by C-RNTI

| Transmission mode | DCI format | Search Space | Transmission scheme of PUSCH corresponding to MPDCCH |
| --- | --- | --- | --- |
| Mode 1 | DCI format 6-0A or 6-0B | Type0-common (only for 6-0A) and UE specific by C-RNTI | Single-antenna port, port 10 (see Clause 8.0.1) |

Table 8-3C: SPDCCH and PUSCH configured by C-RNTI

| Transmission mode | DCI format | Search Space | Transmission scheme of PUSCH corresponding to SPDCCH |
| --- | --- | --- | --- |
| Mode 1 | DCI format 7-0A | UE specific by C-RNTI | Single-antenna port, port 10 (see Clause 8.0.1) |
| Mode 2 | DCI format 7-0B | UE specific by C-RNTI | Closed-loop spatial multiplexing (see Clause 8.0.2) |

If a UE is configured by higher layers to decode PDCCHs with the CRC scrambled by the C-RNTI and is also configured to receive random access procedures initiated by "PDCCH orders", the UE shall decode the PDCCH according to the combination defined in Table 8-4.

If a UE is configured by higher layers to decode EPDCCHs with the CRC scrambled by the C-RNTI and is also configured to receive random access procedures initiated by "PDCCH orders", the UE shall decode the EPDCCH according to the combination defined in Table 8-4A.

If a UE is configured by higher layers to decode MPDCCHs with the CRC scrambled by the C-RNTI and is also configured to receive random access procedures initiated by "PDCCH orders", the UE shall decode the MPDCCH according to the combination defined in Table 8-4B.

Table 8-4: PDCCH configured as "PDCCH order" to initiate random access procedure

| DCI format | Search Space |
| --- | --- |
| DCI format 1A | Common and UE specific by C-RNTI |

Table 8-4A: EPDCCH configured as "PDCCH order" to initiate random access procedure

| DCI format | Search Space |
| --- | --- |
| DCI format 1A | UE specific by C-RNTI |

Table 8-4B: MPDCCH configured as "PDCCH order" to initiate random access procedure

| DCI format | Search Space |
| --- | --- |
| DCI format 6-1A or 6-1B | Type0-common (only for 6-1A) and UE specific by C-RNTI |

If a UE is configured by higher layers to decode PDCCHs with the CRC scrambled by the SPS C-RNTI or UL-SPS-V-RNTI, the UE shall decode the PDCCH according to the combination defined in Table 8-5 and transmit the corresponding PUSCH if a transport block corresponding to the HARQ process of the PUSCH transmission is generated as described in [8] except when the UE is configured with higher layer parameter shortProcessingTime and with DCI format 0 mapped onto the UE-specific search space. 
The scrambling initialization of this PUSCH corresponding to these PDCCHs and PUSCH retransmission for the same transport block is by SPS C-RNTI or UL-SPS-V-RNTI. The scrambling initialization of initial transmission of this PUSCH without a corresponding PDCCH and the PUSCH retransmission for the same transport block is by SPS C-RNTI or UL-SPS-V-RNTI.

If a UE is configured by higher layers to decode EPDCCHs with the CRC scrambled by the SPS C-RNTI or UL-SPS-V-RNTI, the UE shall decode the EPDCCH according to the combination defined in Table 8-5A and transmit the corresponding PUSCH if a transport block corresponding to the HARQ process of the PUSCH transmission is generated as described in [8]. 
The scrambling initialization of this PUSCH corresponding to these EPDCCHs and PUSCH retransmission for the same transport block is by SPS C-RNTI or UL-SPS-V-RNTI. The scrambling initialization of initial transmission of this PUSCH without a corresponding EPDCCH and the PUSCH retransmission for the same transport block is by SPS C-RNTI or UL-SPS-V-RNTI.

If a UE is configured by higher layers to decode MPDCCHs with the CRC scrambled by the SPS C-RNTI, the UE shall decode the MPDCCH according to the combination defined in Table 8-5B and transmit the corresponding PUSCH if a transport block corresponding to the HARQ process of the PUSCH transmission is generated as described in [8]. 
The scrambling initialization of this PUSCH corresponding to these MPDCCHs and PUSCH retransmission for the same transport block is by SPS C-RNTI. The scrambling initialization of initial transmission of this PUSCH without a corresponding MPDCCH and the PUSCH retransmission for the same transport block is by SPS C-RNTI.

If a UE is configured by higher layers to decode SPDCCHs with the CRC scrambled by the SPS C-RNTI, the UE shall decode the SPDCCH according to the combination defined in Table 8-5C and transmit the corresponding PUSCH if a transport block corresponding to the HARQ process of the PUSCH transmission is generated as described in [8]. 
The scrambling initialization of this PUSCH corresponding to these SPDCCHs and PUSCH retransmission for the same transport block is by SPS C-RNTI. The scrambling initialization of initial transmission of this PUSCH without a corresponding SPDCCH and the PUSCH retransmission for the same transport block is by SPS C-RNTI.

Table 8-5: PDCCH and PUSCH configured by SPS C-RNTI or UL-SPS-V-RNTI

| Transmission mode | DCI format | Search Space | Transmission scheme of PUSCH corresponding to PDCCH |
| --- | --- | --- | --- |
| Mode 1 | DCI format 0 | Common and UE specific by C-RNTI | Single-antenna port, port 10 (see Clause 8.0.1) |
|  | DCI format 7-0A | UE specific by C-RNTI |  |
| Mode 2 | DCI format 0 | Common and UE specific by C-RNTI | Single-antenna port, port 10 (see Clause 8.0.1) |
|  | DCI format 7-0B | UE specific by C-RNTI |  |

Table 8-5A: EPDCCH and PUSCH configured by SPS C-RNTI or UL-SPS-V-RNTI

| Transmission mode | DCI format | Search Space | Transmission scheme of PUSCH corresponding to PDCCH |
| --- | --- | --- | --- |
| Mode 1 | DCI format 0 | UE specific by C-RNTI | Single-antenna port, port 10 (see Clause 8.0.1) |
| Mode 2 | DCI format 0 | UE specific by C-RNTI | Single-antenna port, port 10 (see Clause 8.0.1) |

Table 8-5B: MPDCCH and PUSCH configured by SPS C-RNTI

| Transmission mode | DCI format | Search Space | Transmission scheme of PUSCH corresponding to PDCCH |
| --- | --- | --- | --- |
| Mode 1 | DCI format 6-0A | Type0-common (only for 6-0A) and UE specific by C-RNTI | Single-antenna port, port 10 (see Clause 8.0.1) |

Table 8-5C: SPDCCH and PUSCH configured by SPS C-RNTI

| Transmission mode | DCI format | Search Space | Transmission scheme of PUSCH corresponding to SPDCCH |
| --- | --- | --- | --- |
| Mode 1 | DCI format 7-0A | UE specific by C-RNTI | Single-antenna port, port 10 (see Clause 8.0.1) |
| Mode 2 | DCI format 7-0B | UE specific by C-RNTI | Single-antenna port, port 10 (see Clause 8.0.1) |

If a UE is configured by higher layers to decode PDCCHs with the CRC scrambled by the Temporary C-RNTI regardless of whether UE is configured or not configured to decode PDCCHs with the CRC scrambled by the C-RNTI, the UE shall decode the PDCCH according to the combination defined in Table 8-6 and transmit the corresponding PUSCH. The scrambling initialization of PUSCH corresponding to these PDCCH is by Temporary C-RNTI.

If a UE is configured by higher layers to decode MPDCCHs with the CRC scrambled by the Temporary C-RNTI regardless of whether UE is configured or not configured to decode MPDCCHs with the CRC scrambled by the C-RNTI during random access procedure, the UE shall decode the MPDCCH according to the combination defined in Table 8-6A and transmit the corresponding PUSCH. The scrambling initialization of PUSCH corresponding to these MPDCCH is by Temporary C-RNTI.

If a Temporary C-RNTI is set by higher layers, the scrambling of PUSCH corresponding to the Random Access Response Grant in Clause 6.2 and the PUSCH retransmission for the same transport block is by Temporary C-RNTI. Else, the scrambling of PUSCH corresponding to the Random Access Response Grant in Clause 6.2 and the PUSCH retransmission for the same transport block is by C-RNTI.

If a UE is also configured by higher layers to decode MPDCCH with CRC scrambled by the C-RNTI during random access procedure, the UE shall decode the MPDCCH according to the combination defined in Table 8-6A and transmit the corresponding PUSCH. The scrambling initialization of PUSCH corresponding to these MPDCCH is by C-RNTI.

Table 8-6: PDCCH configured by Temporary C-RNTI

| DCI format | Search Space |
| --- | --- |
| DCI format 0 | Common |

Table 8-6A: MPDCCH configured by Temporary C-RNTI and/or C-RNTI during random access procedure

| DCI format | Search Space |
| --- | --- |
| DCI format 6-0A, 6-0B | Type2-Common |

If a UE is configured by higher layers to decode PDCCHs with the CRC scrambled by the TPC-PUCCH-RNTI, the UE shall decode the PDCCH according to the combination defined in table 8-7. The notation 3/3A implies that the UE shall receive either DCI format 3 or DCI format 3A depending on the configuration.

If a UE is configured by higher layers to decode MPDCCHs with the CRC scrambled by the TPC-PUCCH-RNTI, the UE shall decode the MPDCCH according to the combination defined in table 8-7A. The notation 3/3A implies that the UE shall receive either DCI format 3 or DCI format 3A depending on the configuration.

Table 8-7: PDCCH configured by TPC-PUCCH-RNTI

| DCI format | Search Space |
| --- | --- |
| DCI format 3/3A | Common |

Table 8-7A: MPDCCH configured by TPC-PUCCH-RNTI

| DCI format | Search Space |
| --- | --- |
| DCI format 3/3A | Type0-Common (for CEModeA only) |

If a UE is configured by higher layers to decode PDCCHs with the CRC scrambled by the TPC-PUSCH-RNTI, the UE shall decode the PDCCH according to the combination defined in table 8.8. The notation 3/3A implies that the UE shall receive either DCI format 3 or DCI format 3A depending on the configuration.

If a UE is configured by higher layers to decode MPDCCHs with the CRC scrambled by the TPC-PUSCH-RNTI, the UE shall decode the MPDCCH according to the combination defined in table 8.8A. The notation 3/3A implies that the UE shall receive either DCI format 3 or DCI format 3A depending on the configuration.

Table 8-8: PDCCH configured by TPC-PUSCH-RNTI

| DCI format | Search Space |
| --- | --- |
| DCI format 3/3A | Common |

Table 8-8A: MPDCCH configured by TPC-PUSCH-RNTI

| DCI format | Search Space |
| --- | --- |
| DCI format 3/3A | Type0-Common (for CEModeA only) |

If the UE is configured by higher layers to decode PDCCHs with the CRC scrambled by higher layer parameter srs-TPC-RNTI-r14, the UE shall decode the PDCCH according to the combination defined in Table 8-8B.

Table 8-8B: PDCCH configured by higher layer parameter srs-TPC-RNTI-r14

| DCI format | Search Space |
| --- | --- |
| DCI format 3B | Common |

If a UE is configured by higher layers to decode PDCCHs/EPDCCHs with the CRC scrambled by the AUL C-RNTI, the UE shall decode the PDCCH/EPDCCH according to the combination defined in Table 8-9.

Table 8-9: PDCCH/EPDCCH configured by AUL C-RNTI

| Autonomous uplink Transmission mode | DCI format | Search Space | Transmission scheme of corresponding autonomous PUSCH |
| --- | --- | --- | --- |
| Mode 1 | DCI format 0A | UE specific by C-RNTI | Single-antenna port, port 10 (see Clause 8.0.1) |
| Mode 2 | DCI format 4A | UE specific by C-RNTI | Closed-loop spatial multiplexing (see Clause 8.0.2) |

A UE may transmit PUSCH on preconfigured uplink resources as configured by higher layers. The scrambling initialization of PUSCH transmission using preconfigured uplink resource is by PUR-RNTI.

If a UE is configured by higher layers to decode MPDCCHs with the CRC scrambled by the PUR-RNTI, the UE shall decode the MPDCCH according to the combination defined in Table 8-10 and in case the indication in the DCI corresponds to the retransmission of a transport block transmitted using preconfigured uplink resource, transmit a corresponding PUSCH. The scrambling initialization of this PUSCH corresponding to these MPDCCHs and the PUSCH retransmission for the same transport block is by PUR-RNTI.

Table 8-10: MPDCCH and PUSCH configured by PUR-RNTI

| Transmission mode | DCI format | Search Space | Transmission scheme of PUSCH corresponding to MPDCCH |
| --- | --- | --- | --- |
| Mode 1 | DCI format 6-0A or 6-0B | UE specific by PUR-RNTI | Single-antenna port, port 10 (see Clause 8.0.1) |

<Unchanged parts are omitted>

## 8.2	UE sounding procedure

If the UE is configured with a PUCCH-SCell, the UE shall apply the procedures described in this clause for both primary PUCCH group and secondary PUCCH group unless stated otherwise

- When the procedures are applied for the primary PUCCH group, the terms 'secondary cell', 'secondary cells', 'serving cell', and 'serving cells' in this clause refer to secondary cell, secondary cells, serving cell or serving cells belonging to the primary PUCCH group respectively unless stated otherwise.

- When the procedures are applied for secondary PUCCH group, the terms 'secondary cell', 'secondary cells', 'serving cell' and 'serving cells' in this clause refer to secondary cell, secondary cells (not including the PUCCH-SCell), serving cell, serving cells belonging to the secondary PUCCH group respectively unless stated otherwise. The term 'primary cell' in this clause refers to the PUCCH-SCell of the secondary PUCCH group.

A UE shall transmit Sounding Reference Symbol (SRS) on per serving cell SRS resources based on three trigger types:

-	trigger type 0: higher layer signalling

-	trigger type 1: DCI formats 0/0A/0B/4/4A/4B/1A/6-0A/6-1A for FDD, TDD, and frame structure type 3 and DCI formats 2B/2C/2D/3B for TDD, and frame structure type 3, and DCI format 7-0A/7-0B/7-1E/7-1F/7-1G for TDD if the UE is configured by higher layers for SRS triggering via DCI format 7-0A and has indicated the capability srs-DCI7-Triggering-FS2-r15/ srs-DCI7-Triggering-FS2-r16 and the UE is configured for SRS triggering with srs-DCI7-TriggeringConfig-r15/ srs-DCI7-Triggering-FS2-r16.

- trigger type 2: DCI formats 0/4/1A for FDD and TDD, and DCI formats 2B/2C/2D/3B for TDD, and DCI format 7-0A/7-0B/7-1E/7-1F/7-1G for TDD if the UE is configured by higher layers for SRS triggering via DCI format 7-0A and has indicated the capability srs-DCI7-Triggering-FS2-r16 and the UE is configured for SRS triggering with srs-DCI7-TriggeringConfig-r16.

A UE is not expected to be configured with SRS trigger type 0 and trigger type 2 on a LAA SCell.

In case both trigger type 0 and trigger type 1 SRS transmissions would occur in the same subframe in the same serving cell, the UE shall only transmit the trigger type 1 SRS transmission. This prioritization rule shall be applied before other prioritization rules defined in this clause.

In case both trigger type 1 and trigger type 2 SRS transmissions would occur in the same subframe, the UE shall transmit both the trigger type1 and type 2 SRS transmissions.

In case both trigger type 0 and trigger type 2 SRS transmissions would occur in the same subframe, the UE shall transmit both the trigger type 0 and type 2 SRS transmissions.

If higher layer parameter specialSubframePatterns-v1430 indicates ssp10, or if higher layer parameter specialSubframePatterns-v1450 indicates ssp10-CRS-LessDwPTS, the UE shall assume for the purpose of determining kSRS that the special subframe configuration is that signalled by specialSubframePatterns (without suffix).

A UE may be configured with SRS parameters for trigger type 0 and trigger type 1/2 on each serving cell. A BL/CE UE configured with CEModeB is not expected to be configured with SRS parameters for trigger type 0 and trigger type 1. A BL/CE UE is not expected to be configured with SRS parameters for trigger type 2.The following SRS parameters are serving cell specific and semi-statically configurable by higher layers for trigger type 0 and for trigger type 1/2.

-	Number of combs  as defined in Clause 5.5.3.2 of [3] for trigger type 0 and each configuration of trigger type 1/2, if configured

-	srs-UpPtsAdd: two or four additional SC-FDMA symbols in UpPTS as defined in [11] for trigger type 0 and trigger type 1, if configured

-	Transmission comb , as defined in Clause 5.5.3.2 of [3] for trigger type 0 and each configuration of trigger type 1/2

-	Starting physical resource block assignment , as defined in Clause 5.5.3.2 of [3] for trigger type 0 and each configuration of trigger type 1/2 for a serving cell that is not a LAA SCell. For a serving cell that is a LAA SCell, .

-	duration: single or indefinite (until disabled), as defined in [11] for trigger type 0

-	srs-ConfigIndex ISRS for SRS periodicity  and SRS subframe offset , as defined in Table 8.2-1 and Table 8.2-2 for trigger type 0 and SRS periodicity  and SRS subframe offset , as defined in Table 8.2-4 for trigger type 1 and Table 8.2-5 trigger for type 1/2 for a serving cell that is not a LAA SCell

-	SRS bandwidth , as defined in Clause 5.5.3.2 of [3] for trigger type 0 and each configuration of trigger type 1/2 for a serving cell that is not a LAA SCell. For a serving cell that is a LAA SCell, .

-	Frequency hopping bandwidth, , as defined in Clause 5.5.3.2 of [3] for trigger type 0 and each configuration of type 2

-	Cyclic shift , as defined in Clause 5.5.3.1 of [3] for trigger type 0 and each configuration of trigger type 1/2

-	Number of antenna ports  for trigger type 0 and each configuration of trigger type 1/2

-	SRS subframe for each configuration of trigger type 1 for a serving cell that is a LAA SCell and DCI format 4B

- Starting SC-FDMA symbol l0 and duration N and repetition number R as defined in Clause 5.5.3.2.2 of [3] for each configuration of SRS trigger type 2.

For a TDD serving cell,

-	If the serving cell not configured for PUSCH/PUCCH transmission, or if the UE supports ce-srsEnhancement-r14

-	For trigger type 0, the UE can be configured with more than one configuration of SoundingRS-UL-ConfigDedicatedUpPTsExt and/or SoundingRS-UL-ConfigDedicated, and the SRS parameters in each of the configurations shall be used. The UE is not expected to receive more than one SRS configuration indicating SRS transmission on multiple symbols in different subframes within a half frame.

-	For trigger type 1, the UE can be configured with more than one configuration of SoundingRS-UL- ConfigDedicatedAperiodicUpPTsExt and/or SoundingRS-UL- ConfigDedicatedAperiodic, and the SRS parameters in each of the configurations shall be used. The UE is not expected to receive more than one SRS configuration indicating SRS transmission on multiple symbols in different subframes within a half frame.

-	For trigger type 2, the parameters in SoundingRS-UL-ConfigDedicatedAdd shall be used.

-	Otherwise

-	For trigger type 0, if SoundingRS-UL-ConfigDedicatedUpPTsExt is configured, the SRS parameters in SoundingRS-UL-ConfigDedicatedUpPTsExt shall be used; otherwise, SoundingRS-UL-ConfigDedicated shall be used.

-	For trigger type 1, if SoundingRS-UL-ConfigDedicatedAperiodicUpPTsExt is configured, the SRS parameters in SoundingRS-UL-ConfigDedicatedAperiodicUpPTsExt shall be used; otherwise, SoundingRS-UL-ConfigDedicatedAperiodic shall be used.

-	For trigger type 2, the parameters in SoundingRS-UL-ConfigDedicatedAdd shall be used.

For trigger type 1 and DCI format 4/4A/4B/7-0B, and trigger type 2 and DCI format 4/7-0B, three sets of SRS parameters, srs-ConfigApDCI-Format4, are configured by higher layer signalling. The 2-bit SRS request field [4] in DCI format 4/4A/4B/7-0B indicates the SRS parameter set given in Table 8.1-1. For trigger type 1 and DCI format 0/0A/0B /6-0A/7-0A, and trigger type 2 and DCI format 0/6-0A/7-0A, a single set of SRS parameters, srs-ConfigApDCI-Format0, is configured by higher layer signalling. For trigger type 1/2 and DCI formats 1A/2B/2C/2D/6-1A/7-1E/7-1F/7-1G, a single common set of SRS parameters, srs-ConfigApDCI-Format1a2b2c, is configured by higher layer signalling. For a serving cell that is not a LAA SCell, the SRS request field is 1 bit [4] for DCI formats 0/1A/2B/2C/2D/6-0A/6-1A/7-0A/7-1E/7-1F/7-1G, with type 1/2 SRS triggered if the value of the SRS request field is set to '1'. For a serving cell that is a LAA SCell, the SRS timing offset field is 3 bits [4] for DCI formats 1A/2B/2C/2D, with a type 1 SRS triggered if the value of the SRS timing offset field is not set to '000'. The SRS request field is 1 bit [4] for DCI formats 0A, with type 1/2 SRS triggered if the value of the SRS request field is set to '1'. The 2-bit SRS request field [4] in DCI format 0B indicates the type 1 SRS triggering and PUSCH subframe (as determined in Clause 8.0) with SRS as given in Table 8.2-0A.

For a serving cell that is not a LAA SCell, a 1-bit SRS request field shall be included in DCI formats 0/1A for frame structure type 1 and 0/1A/2B/2C/2D for frame structure type 2 if the UE is configured with SRS parameters for DCI formats 0/1A/2B/2C/2D by higher-layer signalling. A 1-bit SRS request field shall be included in DCI format 7-0A for frame structure type 2 if the UE is configured with SRS parameters srs-ConfigApDCI-Format0 and srs-DCI7-TriggeringConfig-r15 by higher-layer signalling. A 1-bit SRS request field shall be included in DCI formats 7-1E/7-1F/7-1G for frame structure type 2 if the UE is configured with SRS parameters srs-ConfigApDCI-Format1a2b2c and srs-DCI7-TriggeringConfig-r15 by higher-layer signalling. A 1-bit SRS request field shall be included in DCI formats 6-0A/6-1A, the value of which is reserved if the UE is not configured with SRS parameters for DCI formats 6-0A/6-1A by higher layer signalling.

For a TDD serving cell not configured for PUSCH/PUCCH transmission, and trigger type 1/2, a SRS request field [4] shall be included in DCI format 3B if the value of the higher layer parameter fieldTypeFormat3B is set to 3 or 4. If the UE is configured with more than 5 TDD serving cells without PUSCH/PUCCH transmission, a single SRS request field is included in DCI format 3B for a set of the TDD serving cells without PUSCH/PUCCH transmission as given in Table 8.2-0C; otherwise one or more SRS request fields is included in DCI format 3B each corresponding to a TDD serving cell without PUSCH/PUCCH transmission as configured by higher layers. If the UE is configured with no more than 5 TDD serving cells without PUSCH/PUCCH transmission, and the UE is not configured with srs-ConfigApDCI-Format4, the SRS request field [4] in DCI format 3B is 1-bit, 2-bits otherwise. For the 1-bit SRS request field [4] in DCI format 3B, type 1/2 SRS is triggered if the value of the SRS request field is set to '1' with SRS parameters, srs-ConfigApDCI-Format1a2b2c, configured by higher layer signalling. For the 2-bit SRS request field [4] in DCI format 3B, and UE configured with no more than 5 TDD serving cells without PUSCH/PUCCH transmission, the SRS request field indicates the SRS parameter set given in Table 8.1-1 with the three sets of SRS parameters, srs-ConfigApDCI-Format4, configured by higher layer signalling. For the 2-bit SRS request field [4] in Table 8.2-0C and DCI format 3B, and UE configured with more than 5 TDD serving cells without PUSCH/PUCCH transmission, SRS parameters, srs-ConfigApDCI-Format1a2b2c, configured by higher layer signalling for the associated serving cell, is used if type 1/2 SRS is triggered. For the 2-bit SRS request field [4] in Table 8.2-0C and DCI format 3B, and UE configured with more than 5 TDD serving cells without PUSCH/PUCCH transmission, if the UE receives an SRS request field with value '00', the DCI does not indicate type 1/2 SRS trigger, but the UE shall apply the power control commands received in the DCI format 3B according to Clause 5.1.3.1.

Table 8.1-1: SRS request value for trigger type 1/2 in DCI format 4/7-0B, or trigger type 1 in DCI format 4A/4B

| Value of SRS request field | Description |
| --- | --- |
| '00' | No type 1/2 SRS trigger |
| '01' | The 1st SRS parameter set configured by higher layers |
| '10' | The 2nd SRS parameter set configured by higher layers |
| '11' | The 3rd SRS parameter set configured by higher layers |

Table 8.2-0A: SRS request value for trigger type 1 in DCI format 0B

| Value of SRS request field | Description |
| --- | --- |
| '00' | No type 1 SRS trigger |
| '01' | Type 1 SRS trigger and first scheduled PUSCH subframe |
| '10' | Type 1 SRS trigger and second scheduled PUSCH subframe |
| '11' | Type 1 SRS trigger and last scheduled PUSCH subframe |

Table 8.2-0C: SRS request value for trigger type 1/2 in DCI format 3B and for UE configured with more than 5 TDD serving cells without PUSCH/PUCCH transmission

| Value of SRS request field | Description |
| --- | --- |
| '00' | No type 1/2 SRS trigger for a 1st set of serving cells configured by higher layers |
| '01' | Type 1/2 SRS trigger for a 2nd set of serving cells configured by higher layers |
| '10' | Type 1/2 SRS trigger for a 3rd set of serving cells configured by higher layers |
| '11' | Type 1/2 SRS trigger for a 4th set of serving cells configured by higher layers |

For all DCI formats with type 1/2 SRS trigger, each SRS parameter set contains one trigger type 1 and/or one trigger type 2 SRS configuration. When a SRS parameter set is triggered with the 1-bit SRS request set to 1 (for DCI formats with 1-bit SRS request field) or with the corresponding SRS request field value signalled (for DCI formats with 2-bits SRS request field), the configured trigger type 1 and/or trigger type 2 SRS is transmitted according to their respective configurations.

For a serving cell that is not a LAA SCell, the serving cell specific SRS transmission bandwidths  are configured by higher layers. The allowable values are given in Clause 5.5.3.2 of [3].

For a serving cell that is not a LAA SCell, the serving cell specific SRS transmission sub-frames are configured by higher layers. The allowable values are given in Clause 5.5.3.3 of [3].

For a TDD serving cell, trigger type 0 and 1 SRS transmissions can occur in UpPTS and uplink subframes of the UL/DL configuration indicated by the higher layer parameter subframeAssignment for the serving cell. Trigger type 2 SRS transmissions can occur in uplink subframes of the UL/DL configuration indicated by the higher layer parameter subframeAssignment for the serving cell except the last symbol of a subframe.

For trigger type 0 and type 1 SRS transmission, when closed-loop or open-loop UE transmit antenna selection is enabled for a given serving cell for a UE that supports transmit antenna selection, or for a UE that can be configured with ue-TxAntennaSelection-SRS-1T4R-Config or ue-TxAntennaSelection-SRS-2T4R-NrOfPairs,

- when higher layer parameter 'ue-TxAntennaSelection-SRS-1T4R-Config' is configured for a given serving cell, the index anSRS, of the UE antenna that transmits the SRS at time nSRS is given by

anSRS=nSRS mod 4, for both partial and full sounding bandwidth, and when frequency hopping is disabled (i.e., bhop≥BSRS),

anSRS=nSRS+nSRSmax4,K+βnSRS4modmax4,K4mod 4,when K is evennSRS mod 4,otherwise

with β=1,if N1=2, N2=20,otherwise             , when frequency hopping is enabled (i.e., bhop<BSRS).

- when higher layer parameter 'ue-TxAntennaSelection-SRS-2T4R-NrOfPairs' is configured for a given serving cell for a UE configured with Λ UE antenna pairs, where Λ={2 or 3} is given by higher layer parameter 'ue-TxAntennaSelection-SRS-2T4R-NrOfPairs', the index anSRS for the UE antenna pair as 2anSRS,  2anSRS+1 when Λ =2, or 0,  anSRS+1 when Λ =3 that transmits the SRS at time nSRS is given by

anSRS=nSRS mod Λ for both partial and full sounding bandwidth, and when frequency hopping is disabled (i.e., bhop≥BSRS),

with β=1,if K mod Λ2=00,otherwise             , when frequency hopping is enabled (i.e., bhop<BSRS).

-	otherwise, the index , of the UE antenna that transmits the SRS at time nSRS is given by

, for both partial and full sounding bandwidth, and when frequency hopping is disabled (i.e., ),

,

when frequency hopping is enabled (i.e., ),

where values BSRS, bhop, Nb, and nSRS are given in Clause 5.5.3.2 of [3], and (where  regardless of the  value), except when a single SRS transmission is configured for the UE. If a UE is configured with more than one serving cell, and for a group of cells belonging to bands that are signalled to be switched together in txAntennaSwitchUL the UE is not expected to transmit SRS on different antenna ports simultaneously. If a UE is configured with more than one serving cell, and for a group of cells belonging to bands that are signalled to be switched together in txAntennaSwitchUL the UE is not expected to transmit SRS and PUSCH on different antenna ports simultaneously.

For trigger type 2 SRS transmission, when closed-loop or open-loop UE transmit antenna selection is enabled for a given serving cell for a UE that supports antenna selection, or for UE that can be configured with ue-TxAntennaSelection-SRS-1T4R-Config or ue-TxAntennaSelection-SRS-2T4R-NrOfPairs, the index anSRS of the UE antenna that transmits the SRS at time nSRS is given by anSRS=nSRS when frequency hopping is disabled, and anSRS=nSRSNFH when frequency hopping is enabled, where NFH is the number of frequency hops defined in Clause 5.5.3.2.2 of [3]. The UE is not expected to be configured with anSRS≥NAS and

-	when higher layer parameter 'ue-TXAntennaSelection-SRS-1T4R-Config' is configured for a given serving cell, the number of antenna switches NAS = 4.

-	when higher layer parameter 'ue-TXAntennaSelection-SRS-2T4R-NrOfPairs' is configured for a given serving cell for a UE configured with Λ UE antenna pairs, where Λ={2 or 3} is given by higher layer parameter 'ue-TXAntennaSelection-SRS-2T4R-NrOfPairs', the number of antenna switches NAS = Λ, and the index for the UE antenna pair that transmits the SRS at time nSRS is 2anSRS,  2anSRS+1 when Λ =2, or 0,  anSRS+1 when Λ =3.

-	otherwise, the number of antenna switches NAS = 2.

When higher layer parameter 'ue-TxAntennaSelection-SRS-1T4R-Config' is configured or 'ue-TxAntennaSelection-SRS-2T4R-NrOfPairs' is configured for a serving cell, a UE is not expected to be configured with more than two antenna ports for any uplink physical channel or signal on that serving cell.

A UE may be configured to transmit SRS on  antenna ports of a serving cell where may be configured by higher layer signalling. For PUSCH transmission mode 1 and for PUSCH transmission mode 2  with two antenna ports configured for PUSCH and  with 4 antenna ports configured for PUSCH. A UE configured for SRS transmission on multiple antenna ports of a serving cell shall transmit SRS for all the configured transmit antenna ports within one SC-FDMA symbol of the same subframe of the serving cell. 
The SRS transmission bandwidth and starting physical resource block assignment are the same for all the configured antenna ports of a given serving cell. The UE does not support a value of  set to '4', if the UE is configured for SRS transmission on 4 antenna ports of a serving cell.

If a UE is not configured with multiple TAGs and the UE is not configured with the parameter srs-UpPtsAdd for trigger type 1 and the UE is not configured with trigger type 2 SRS transmission, or if a UE is not configured with multiple TAGs and the UE is not configured with more than one serving cell of different CPs and the UE is not configured with trigger type 2 SRS transmission, or if a UE is configured for PUSCH transmission in UpPTS and a SRS transmission overlaps with a PUSCH transmission on the same symbol in UpPTS within a TDD serving cell, the UE shall not transmit trigger type 0/1 SRS in a symbol whenever SRS and PUSCH transmissions happen to overlap in the same symbol, except when the SRS is on a TDD serving cell not configured for PUSCH/PUCCH transmission.

For the case when a trigger type 0/1 SRS transmission in a first serving cell happens to overlap in the same symbol as a PUSCH transmission in a second serving cell, and the first and second serving cells are in the same TAG, same band, and use the same cyclic prefix, the UE may drop the trigger type 0/1 SRS transmission.

For the case when a trigger type 2 SRS transmission in a first serving cell happens to overlap in the same symbol as the PUSCH/PUCCH transmission in a second serving cell, and the first and second serving cells are in the same TAG, same band, and use the same cyclic prefix, the UE may drop the trigger type 2 SRS transmission in the overlapped symbol.

For TDD serving cell, and UE not configured with additional SC-FDMA symbols in UpPTS, when one SC-FDMA symbol exists in UpPTS of the given serving cell, it can be used for SRS transmission, when two SC-FDMA symbols exist in UpPTS of the given serving cell, both can be used for SRS transmission and for trigger type 0 SRS both can be assigned to the same UE. For TDD serving cell, and if the UE is configured with two or four additional SC-FDMA symbols in UpPTS of the given serving cell, all can be used for SRS transmission and for trigger type 0 SRS at most two SC-FDMA symbols out of the configured additional SC-FDMA symbols in UpPTS can be assigned to the same UE, except for UE not configured for PUSCH/PUCCH transmission or for UE supporting ce-srsEnhancement-r14, where all can be assigned to the same UE. A UE is not expected to be configured with trigger type 2 SRS in UpPTS.

If a UE is not configured with multiple TAGs and the UE is not configured with the parameter srs-UpPtsAdd for trigger type 1 and the UE is not configured with trigger type 2 SRS transmission, or if a UE is not configured with multiple TAGs and the UE is not configured with more than one serving cell of different CPs and the UE is not configured with trigger type 2 SRS transmission, or if a UE is configured with multiple TAGs and trigger type 0/1 SRS and PUCCH format 2/2a/2b happen to coincide in the same subframe in the same serving cell, except when the SRS is on a TDD serving cell not configured for PUSCH/PUCCH transmission,

-	The UE shall not transmit type 0 triggered SRS whenever type 0 triggered SRS and PUCCH format 2/2a/2b transmissions happen to coincide in the same subframe;

-	The UE shall not transmit type 1 triggered SRS whenever type 1 triggered SRS and PUCCH format 2a/2b or format 2 with HARQ-ACK transmissions happen to coincide in the same subframe;

-	The UE shall not transmit PUCCH format 2 without HARQ-ACK whenever type 1 triggered SRS and PUCCH format 2 without HARQ-ACK transmissions happen to coincide in the same subframe.

If a UE is not configured with multiple TAGs and the UE is not configured with the parameter srs-UpPtsAdd for trigger type 1 and the UE is not configured with trigger type 2 SRS transmission, or if a UE is not configured with multiple TAGs and the UE is not configured with more than one serving cell of different CPs and the UE is not configured with trigger type 2 SRS transmission, or if a UE is configured with multiple TAGs and trigger type 0/1 SRS and PUCCH happen to coincide in the same subframe/slot/subslot in the same serving cell, except when the SRS is on a TDD serving cell not configured for PUSCH/PUCCH transmission,

-	The UE shall not transmit trigger type 0/1 SRS whenever SRS transmission and PUCCH transmission carrying HARQ-ACK and/or positive SR happen to coincide in the same subframe/slot/subslot if the parameter ackNackSRS-SimultaneousTransmission is FALSE;

-	For FDD-TDD and primary cell frame structure 1, the UE shall not transmit trigger type 0/1 SRS in a symbol whenever SRS transmission and PUCCH transmission carrying HARQ-ACK and/or positive SR using shortened format as defined in Clauses 5.4.1, 5.4.2A, 5.4.2B, 5.4.2C, and 5.4A of [3] happen to overlap in the same symbol if the parameter ackNackSRS-SimultaneousTransmission is TRUE.

-	Unless otherwise prohibited, the UE shall transmit trigger type 0/1 SRS whenever SRS transmission and PUCCH transmission carrying HARQ-ACK and/or positive SR using shortened format as defined in Clauses 5.4.1, 5.4.2A, and 5.4A of [3] happen to coincide in the same subframe/slot/subslot if the parameter ackNackSRS-SimultaneousTransmission is TRUE.

If a UE is not configured with multiple TAGs and the UE is not configured with the parameter srs-UpPtsAdd for trigger type 1 and the UE is not configured with trigger type 2 SRS transmission, or if a UE is not configured with multiple TAGs and the UE is not configured with more than one serving cell of different CPs and the UE is not configured with trigger type 2 SRS transmission, the UE shall not transmit SRS whenever SRS transmission on any serving cells and PUCCH transmission carrying HARQ-ACK and/or positive SR using normal PUCCH format as defined in Clauses 5.4.1, 5.4.2A, and 5.4A of [3] happen to coincide in the same subframe/slot/subslot.

In UpPTS, whenever SRS transmission instance overlaps with the PRACH region for preamble format 4 or exceeds the range of uplink system bandwidth configured in the serving cell, the UE shall not transmit SRS.

For a TDD serving cell d not configured for PUSCH/PUCCH transmission, denote as s0(d) the corresponding serving cell whose UL transmissions may be interrupted as signalled by srs-SwitchFromServCellIndex. Define the set S(d)= {s0(d)… sN-1(d)} as the set of serving cells that meet the all the following conditions:

-	{s0(d)… sN-1(d)} are in the same band as s0(d).

-	{s0(d)… sN-1(d)} have the same CP as s0(d).

-	{s0(d)… sN-1(d)} are in the same TAG as s0(d).

The following prioritization rules shall be applied in case of collision between a transmission of SRS over serving cell d and transmission of a physical signal/channel over a serving cell in set S(d):

-	If PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/PTI/CRI/wideband PMI only (PUCCH reporting type 2a in Clause 7.2.2) and/or PRACH on a serving cell in set S(d) overlaps in the same symbol with the SRS transmission (including any interruption due to uplink or downlink RF retuning time [10]) on serving cell d, then the UE shall not transmit trigger type 0/1 SRS or drop the overlapped symbol(s) of type 2 SRS (including any interruption due to uplink or downlink RF retuning time [10]). Otherwise,

-	if PUSCH transmission carrying aperiodic CSI on a serving cell in set S(d) overlaps in the same symbol with the SRS transmission (including any interruption due to uplink or downlink RF retuning time [10]) in serving cell d, and if the SRS transmission is a type 0 SRS transmission, then the UE shall not transmit the type 0 SRS. Otherwise,

-	if PUSCH transmission on a serving cell in set S(d) overlaps in more than one symbol with the SRS transmission (including any interruption due to uplink or downlink RF retuning time [10]) in serving cell d, then the UE shall drop the PUSCH transmission. If PUCCH/trigger type 0/1 SRS transmission on a serving cell in set S(d) overlaps in the same symbol with the SRS transmission (including any interruption due to uplink or downlink RF retuning time [10]) on serving cell d, the UE shall drop the PUCCH/trigger type 0/1 SRS transmission. If a subset of symbol(s) for type 2 SRS transmission on a serving cell in set S(d) overlaps with the SRS transmission (including any interruption due to uplink or downlink RF retuning time [10]) on serving cell d, the UE shall drop the subset of the symbol(s) for type 2 SRS on a serving cell in set S(d).

In case an SRS transmission in subframe N on serving cell d is dropped due to a collision with a higher priority transmission (as defined above) in subframe N+1, and there is a lower priority transmission (as defined above) in subframe N that would have been dropped had the transmission in subframe N+1 not occurred, the UE is not required to transmit the lower priority transmission in subframe N.

The UE is not expected to be triggered to transmit type 2 SRS on serving cell d in subframe N that overlaps (including any interruption due to uplink or downlink RF retuning time [10]) with uplink subframe N-1 on a serving cell in set S(d).

The parameter ackNackSRS-SimultaneousTransmission provided by higher layers determines if a UE is configured to support the transmission of HARQ-ACK on PUCCH and SRS in one subframe/slot/subslot. If it is configured to support the transmission of HARQ-ACK on PUCCH and SRS in one subframe/slot/subslot, then in the cell specific SRS subframes of the primary cell in case of subframe-PUCCH or in the last slot/subslot of the cell specific SRS subframes of the primary cell in case of slot/subslot-PUCCH,

-	if the UE transmits PUCCH format 1/1a/1b/3, the UE shall transmit HARQ-ACK and SR using the shortened PUCCH format as defined in Clauses 5.4.1, 5.4.2A, and 5.4A.3 of [3], where the HARQ-ACK or the SR symbol corresponding to the SRS location in the last symbol of the subframe is punctured.

-	If the UE transmits PUCCH format 4/5 partly or fully overlapping with the cell specific SRS bandwidth in the cell specific SRS subframes of the primary cell, then UE shall transmit UCI using the shortened PUCCH format as defined in Clauses 5.4.2B, 5.4.2C, and 5.4A.4 of [3].

For PUCCH format 1/1a/1b/3, this shortened PUCCH format shall be used in a cell specific SRS subframe or the last slot/subslot of the cell specific SRS subframe of the primary cell even if the UE does not transmit SRS in that subframe. For PUCCH format 4/5, this shortened PUCCH format shall be used if the PUCCH transmission partly or fully overlaps with the cell-specific SRS bandwidth in the cell specific SRS subframes or the last slot/subslot of the cell specific SRS subframes of the primary cell even if the UE does not transmit SRS in that subframe, or if the UE transmits SRS in the last symbol of that subframe even if the PUCCH format 4/5 does not partly or fully overlap with the cell-specific SRS. The cell specific SRS subframes are defined in Clause 5.5.3.3 of [3]. Otherwise, the UE shall use the normal PUCCH format 1/1a/1b as defined in Clause 5.4.1, and 5.4A.2 of [3] or normal PUCCH format 3 as defined in Clause 5.4.2A, and 5.4A.3 or normal PUCCH format 4 as defined in Clause 5.4.2B, and 5.4A.4 or normal PUCCH format 5 as defined in Clause 5.4.2C of [3].

For a BL/CE UE not configured with the higher layer parameter srs-UpPtsAdd, for a SRS transmission in subframe n and if the UE transmits PUSCH/PUCCH in subframe n and/or n+1, the UE shall not transmit the SRS in subframe n if the SRS transmission bandwidth in subframe n is not completely within the narrowband of PUSCH/PUCCH in subframe n and/or n+1

A BL/CE UE not configured with the higher layer parameter srs-UpPtsAdd shall not transmit SRS in UpPTS if SRS frequency location is different from DwPTS reception narrowband in the same special subframe.

For a BL/CE UE, the SRS transmission that falls into the reserved symbol of a BL/CE UL subframe is dropped.

For a TDD serving cell, , not configured for PUSCH/PUCCH transmission, the UE is not expected to be configured with SRS resource(s) such that the SRS transmission (including any interruption due to uplink or downlink RF retuning time [10]) may overlap in time with PDCCH monitoring in subframes 0 or 5 on serving cell , if the UE is not capable of simultaneous transmission and reception on serving cell  and serving cell .

Trigger type 0 SRS configuration of a UE in a serving cell for SRS periodicity,, and SRS subframe offset,, is defined in Table 8.2-1 and Table 8.2-2, for FDD and TDD serving cell, respectively. The periodicity  of the SRS transmission is serving cell specific and is selected from the set {2, 5, 10, 20, 40, 80, 160, 320} ms or subframes. 
For the SRS periodicity  of 2 ms in TDD serving cell configured for PUSCH and/or PUCCH transmission, two SRS resources are configured in a half frame containing UL subframe(s) of the given serving cell. For the SRS periodicity  of 2 ms in TDD serving cell not configured for PUSCH/PUCCH transmission, two or more SRS resources are configured in a half frame containing UL subframe(s) of the given serving cell.

Type 0 triggered SRS transmission instances in a given serving cell for TDD serving cell with  and for FDD serving cell are the subframes satisfying , where for FDD  is the subframe index within the frame, for TDD serving cell, if the UE is configured with the parameter srs-UpPtsAdd for trigger type 0,  is defined in Table 8.2-6; otherwise  is defined in Table 8.2-3. The SRS transmission instances for TDD serving cell with  are the subframes satisfying .

For TDD serving cell, and a UE configured for type 0 triggered SRS transmission in serving cell c, and the UE configured with the parameter EIMTA-MainConfigServCell-r12 for serving cell c, if the UE does not detect an UL/DL configuration indication for radio frame m (as described in Clause 13.1), the UE shall not transmit trigger type 0 SRS in a subframe of radio frame m that is indicated by the parameter eimta-HARQ-ReferenceConfig-r12 as a downlink subframe unless the UE transmits PUSCH in the same subframe.

For a serving cell that is not a LAA SCell, trigger type 1 SRS configuration of a UE in a serving cell for SRS periodicity,, and SRS subframe offset,, is defined in Table 8.2-4 and Table 8.2-5, for FDD and TDD serving cell, respectively; and trigger type 2 SRS configuration of a UE in a serving cell for SRS periodicity, , and SRS subframe offset,, is defined in Table 8.2-5, for TDD serving cell. The periodicity  of the SRS transmission is serving cell specific and is selected from the set {2, 5, 10} ms or subframes. 
For the SRS periodicity  of 2 ms in TDD serving cell configured for PUSCH and/or PUCCH transmission, two SRS resources are configured in a half frame containing UL subframe(s) of the given serving cell. For the SRS periodicity  of 2 ms in TDD serving cell not configured for PUSCH/PUCCH transmission, two or more SRS resources are configured in a half frame containing UL subframe(s) of the given serving cell.

For TDD serving cell configured for PUSCH and/or PUCCH transmission, and a UE configured for type 1/2 triggered SRS transmission in serving cell c and configured with the parameter srs-UpPtsAdd, the UE is not expected to receive trigger type 1/2 SRS configurations with SRS periodicity  of 2 ms.

A UE configured for type 1/2 triggered SRS transmission in serving cell c and not configured with a carrier indicator field shall transmit SRS on serving cell c upon detection of a positive SRS request in PDCCH/EPDCCH/MPDCCH/SPDCCH scheduling PUSCH/PDSCH on serving cell c.

A UE configured for type 1/2 triggered SRS transmission in serving cell c and configured with a carrier indicator field shall transmit SRS on serving cell c upon detection of a positive SRS request in PDCCH/EPDCCH/SPDCCH scheduling PUSCH/PDSCH with the value of carrier indicator field corresponding to serving cell c.

For a serving cell that is not a LAA SCell, a non-BL/CE UE configured for type 1/2 triggered SRS transmission on serving cell c upon detection of a positive SRS request in subframe n, slot 2n or slot 2n+1 of serving cell c shall commence SRS transmission in the first subframe satisfying , and

-	 if the positive SRS request in PDCCH/SPDCCH with DCI format 7-0A/7-1A is detected in slot 2n or slot 2n+1, for TDD

-	 if the UE is configured with higher layer parameter shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI with DCI format other than DCI format 7-0A/7-0B/7-1E/7-1F/7-1G is in the UE-specific search space,

-	otherwise, and

for TDD serving cell c with  and for FDD serving cell c,

for TDD serving cell c with

where for FDD serving cell c  is the subframe index within the frame , for TDD serving cell c, if the UE is configured with the parameter srs-UpPtsAdd for trigger type 1,  is defined in Table 8.2-6; otherwise  is defined in Table 8.2-3. For a TDD serving cell not configured for PUSCH/PUCCH transmission and the positive SRS request detected in PDCCH/EPDCCH scheduling PDSCH and the UE configured with soundingRS-FlexibleTiming-r14 by higher layer signalling, if the trigger type 1 SRS transmission (including any interruption due to uplink or downlink RF retuning time [10]) in the first subframe  happens to overlap with a HARQ-ACK transmission for any serving cell, the UE shall commence trigger type 1 SRS transmission in subframe n + k + l, where l = max( 5, ). The soundingRS-FlexibleTiming-r14 if configured by higher layer signaling is not applied to trigger type 2 SRS.

For a type 1/2 SRS triggered for more than one TDD serving cell in DCI format 3B and UE configured with more than 5 TDD serving cells without PUSCH/PUCCH transmission, the order of the triggered SRS transmission on the serving cells follow the order of the serving cells in the indicated set of serving cells configured by higher layers. For a type 1/2 SRS triggered for more than one TDD serving cell in DCI format 3B and UE configured with no more than 5 TDD serving cells without PUSCH/PUCCH transmission, the order of the triggered SRS transmission on the serving cells follow the order of the serving cells with type 1/2 SRS triggered in the DCI. The SRS resource for the n-th (n>=2) SRS transmission is determined such that it is the first SRS resource on or after the SRS resource for the (n-1)-th SRS transmission provided it does not collide with any previous SRS transmission triggered in the DCI format 3B, or interruption due to UL or DL RF retuning time [10].

For a serving cell c that is a LAA SCell, a UE configured for type 1 triggered SRS transmission on serving cell c upon detection of a positive SRS request in subframe n of serving cell c shall commence SRS transmission, conditioned on the channel access procedures described in clause 4.2.1 of [13], in subframe , where

- corresponds to the scheduled PUSCH subframe determined in Clause 8.0 if SRS is triggered in DCI format 0A/4A,

- is determined from Table 8.2-0A and the corresponding scheduled PUSCH subframe determined in Clause 8.0 if SRS is triggered in DCI format 0B,

- where the value of l is determined from SRS subframe parameter for the indicated SRS parameter set in Table 8.1, is determined from the first scheduled PUSCH subframe determined in Clause 8.0 and N is determined by the procedure in Clause 8.0 if SRS is triggered in DCI format 4B,

- where the value of l is determined by the SRS timing offset field in the corresponding DCI if SRS is triggered in DCI format 1A/2B/2C/2D according to Table 8.2-0B.

Table 8.2-0B:  for SRS trigger type 1/2 in DCI format 1A/2B/2C/2D

| Value of SRS timing offset field |  |
| --- | --- |
| '000' | No type 1/2 SRS trigger |
| '001' | 1 |
| '010' | 2 |
| '011' | 3 |
| '100' | 4 |
| '101' | 5 |
| '110' | 6 |
| '111' | 7 |

A BL/CE UE configured for type 1 triggered SRS transmission on serving cell c upon detection of a positive SRS request of serving cell c shall commence SRS transmission in the first subframe satisfying n+k+Koffset,, k≥4, where subframe n is the last subframe in which the DCI format 6-0A/6-1A with the positive SRS request is transmitted, and

for TDD serving cell c with  and for FDD serving cell c,

for TDD serving cell c with  where for FDD serving cell c  is the subframe index within the frame , for TDD serving cell c , if the UE is configured with the parameter srs-UpPtsAdd for trigger type 1,  is defined in Table 8.2-6; otherwise  is defined in Table 8.2-3.

A UE configured for type 1/2 triggered SRS transmission is not expected to receive type 1/2 SRS triggering events associated with different values of trigger type 1/2 SRS transmission parameters, as configured by higher layer signalling, for the same subframe and the same serving cell.

For a serving cell that is a LAA SCell, a UE configured for type 1 triggered SRS transmission is not expected to receive type 1 SRS triggering event in DCI format 0B associated with a subframe that is not scheduled for PUSCH transmission for the same serving cell.

For a serving cell that is an LAA SCell, if the uplink transmission in a subframe is ending in the end of symbol #3 or in the end of symbol #6, the UE shall not transmit SRS in that subframe.

A UE configured for type 2, type 1, or type 0 triggered SRS transmission and more than one TDD serving cell without PUSCH/PUCCH transmission is not expected to receive type 2, type 1, or type 0 SRS triggering events that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in the UE-EUTRA-Capability [12].

For TDD serving cell c, and a UE configured with EIMTA-MainConfigServCell-r12 for a serving cell c, the UE shall not transmit SRS in a subframe of a radio frame that is indicated by the corresponding eIMTA-UL/DL-configuration as a downlink subframe.

A UE shall not transmit SRS whenever SRS and a PUSCH transmission corresponding to a Random Access Response Grant or a retransmission of the same transport block as part of the contention based random access procedure coincide in the same subframe.

A UE not configured with higher layer parameter ul-STTI-Length is not expected to be triggered with trigger type 2 SRS transmission in the same subframe as a PUSCH/PUCCH transmission in the same serving cell.

A UE configured with higher layer parameter ul-STTI-Length shall drop the trigger type 2 SRS transmission in the overlapped symbol when the trigger type 2 SRS transmission and slot PUSCH/PUCCH transmission happens to overlap in the same symbol and same serving cell.

A UE shall not transmit trigger type 2 SRS when trigger type 2 SRS transmission and PRACH happen to overlap in the same subframe and same serving cell.

Table 8.2-1: UE Specific SRS Periodicity  and Subframe Offset Configuration 
 for trigger type 0, FDD

| SRS Configuration Index ISRS | SRS Periodicity (ms) | SRS Subframe Offset |
| --- | --- | --- |
| 0 – 1 | 2 | ISRS |
| 2 – 6 | 5 | ISRS – 2 |
| 7 – 16 | 10 | ISRS – 7 |
| 17 – 36 | 20 | ISRS – 17 |
| 37 – 76 | 40 | ISRS – 37 |
| 77 – 156 | 80 | ISRS – 77 |
| 157 – 316 | 160 | ISRS – 157 |
| 317 – 636 | 320 | ISRS – 317 |
| 637 – 1023 | reserved | reserved |

Table 8.2-2: UE Specific SRS Periodicity  and Subframe Offset Configuration 
 for trigger type 0, TDD

| SRS Configuration Index ISRS | SRS Periodicity (ms) | SRS Subframe Offset |
| --- | --- | --- |
| 0 | 2 | 0, 1 |
| 1 | 2 | 0, 2 |
| 2 | 2 | 1, 2 |
| 3 | 2 | 0, 3 |
| 4 | 2 | 1, 3 |
| 5 | 2 | 0, 4 |
| 6 | 2 | 1, 4 |
| 7 | 2 | 2, 3 |
| 8 | 2 | 2, 4 |
| 9 | 2 | 3, 4 |
| 10 – 14 | 5 | ISRS – 10 |
| 15 – 24 | 10 | ISRS – 15 |
| 25 – 44 | 20 | ISRS – 25 |
| 45 – 84 | 40 | ISRS – 45 |
| 85 – 164 | 80 | ISRS – 85 |
| 165 – 324 | 160 | ISRS – 165 |
| 325 – 644 | 320 | ISRS – 325 |
| 645 – 1023 | reserved | reserved |

Table 8.2-3:  for TDD

|  | subframe index n |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 |  | 2 | 3 | 4 | 5 | 6 |  | 7 | 8 | 9 |
|  |  | 1st symbol of UpPTS | 2nd symbol of UpPTS |  |  |  |  | 1st symbol of UpPTS | 2nd symbol of UpPTS |  |  |  |
| in case UpPTS length of 2 symbols |  | 0 | 1 | 2 | 3 | 4 |  | 5 | 6 | 7 | 8 | 9 |
| in case UpPTS length of 1 symbol |  | 1 |  | 2 | 3 | 4 |  | 6 |  | 7 | 8 | 9 |

Table 8.2-4: UE Specific SRS Periodicity  and Subframe Offset Configuration 
 for trigger type 1, FDD

| SRS Configuration Index ISRS | SRS Periodicity (ms) | SRS Subframe Offset |
| --- | --- | --- |
| 0 – 1 | 2 | ISRS |
| 2 – 6 | 5 | ISRS – 2 |
| 7 – 16 | 10 | ISRS – 7 |
| 17 – 31 | reserved | reserved |

Table 8.2-5: UE Specific SRS Periodicity  and Subframe Offset Configuration 
 for trigger type 1/2, TDD

| SRS Configuration Index ISRS | SRS Periodicity (ms) | SRS Subframe Offset |
| --- | --- | --- |
| 0 | reserved | reserved |
| 1 | 2 | 0, 2 |
| 2 | 2 | 1, 2 |
| 3 | 2 | 0, 3 |
| 4 | 2 | 1, 3 |
| 5 | 2 | 0, 4 |
| 6 | 2 | 1, 4 |
| 7 | 2 | 2, 3 |
| 8 | 2 | 2, 4 |
| 9 | 2 | 3, 4 |
| 10 – 14 | 5 | ISRS – 10 |
| 15 – 24 | 10 | ISRS – 15 |
| 25 – 31 | reserved | reserved |

Table 8.2-6:  for TDD and UE configured with two or four additional SC-FDMA symbols in UpPTS

|  | subframe index n |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 |  |  |  | 2 | 3 | 4 | 5 | 6 |  |  |  | 7 | 8 | 9 |
|  |  | 1st symbol of UpPTS | 2nd symbol of UpPTS | 3rd symbol of UpPTS | 4th symbol of UpPTS |  |  |  |  | 1st symbol of UpPTS | 2nd symbol of UpPTS | 3rd symbol of UpPTS | 4th symbol of UpPTS |  |  |  |
| in case UpPTS length of 4 symbols |  | 0 | 1 | 2 | 3 |  |  |  |  | 5 | 6 | 7 | 8 |  |  |  |
| in case UpPTS length of 2 symbols |  | 2 | 3 |  |  |  |  |  |  | 7 | 8 |  |  |  |  |  |

<Unchanged parts are omitted>

### 9.1.5	MPDCCH assignment procedure

A BL/CE UE shall monitor a set of MPDCCH candidates on one or more Narrowbands (described in Clause 6.2.7 of [3]) as configured by higher layer signalling for control information, where monitoring implies attempting to decode each of the MPDCCHs in the set according to all the monitored DCI formats. The Narrowband in a subframe used for MPDCCH monitoring is determined as described in [3].

A UE that is not a BL/CE UE is not required to monitor MPDCCH.

A BL/CE UE can derive the configuration of one or two MPDCCH-PRB-sets for MPDCCH monitoring from higher layer signalling. The PRB-pairs corresponding to MPDCCH-PRB-set  are indicated by higher layers. Each MPDCCH-PRB-set consists of set of ECCEs numbered from 0 to where is the number of ECCEs in MPDCCH-PRB-set  of subframe .

The MPDCCH-PRB-set(s) can be configured by higher layers for either localized MPDCCH transmission or distributed MPDCCH transmission.

The set of MPDCCH candidates to monitor are defined in terms of MPDCCH search spaces.

The BL/CE UE shall monitor one or more of the following search spaces

-	a Type0-MPDCCH common search space if configured with CEmodeA, or if configured with CEmodeB and higher layer parameter ce-ETWS-CMAS-RxInConn,

-	a Type1-MPDCCH common search space,

-	a Type1A-MPDCCH common search space,

-	a Type2-MPDCCH common search space,

-	a Type2A-MPDCCH common search space, and

-	a MPDCCH UE-specific search space.

A BL/CE UE configured with CEModeB is not required to monitor Type0-MPDCCH common search space unless the UE is configured with higher layer parameter ce-ETWS-CMAS-RxInConn.

The BL/CE UE is not required to simultaneously monitor MPDCCH UE-specific search space and Type1-MPDCCH common search space.

The BL/CE UE is not required to simultaneously monitor MPDCCH UE-specific search space and Type2-MPDCCH common search space.

The BL/CE UE is not required to monitor Type1A-MPDCCH common search space or Type2A-MPDCCH common search space if the set of subframes comprising the search space include any subframes in which it monitors Type1-MPDCCH common search space or any subframes in which the UE receives PDSCH assigned by PDCCH with DCI CRC scrambled by P-RNTI.

The BL/CE UE is not required to monitor Type2A-MPDCCH common search space if the set of subframes comprising the search space include any subframes in which it monitors Type1A-MPDCCH common search space or any subframes in which the UE receives PDSCH assigned by MPDCCH with DCI CRC scrambled by SC-RNTI.

A BL/CE UE is not required to monitor Type1-MPDCCH common search space or in case of half-duplex FDD operation MWUS if the set of subframes comprising the search space or the set of subframes where MWUS may be received include any subframes in which the UE has initiated a PUSCH transmission using preconfigured uplink resource on a given serving cell.

A BL/CE UE is not required to monitor Type1-MPDCCH common search space or MWUS in subframes in which the UE monitors a UE-specific MPDCCH search space given by PUR-RNTI.

A BL/CE UE is not expected to monitor an MPDCCH candidate, if an ECCE corresponding to that MPDCCH candidate is mapped to a PRB pair that overlaps with a transmission of PDSCH scheduled previously in the same subframe.

For aggregation level  or  ECCEs, the number of ECCEs refers to the MPDCCH mapping to the REs of the 2+4 PRB set as defined in [3]. An MPDCCH search space at aggregation level  and repetition level is defined by a set of MPDCCH candidates where each candidate is repeated in a set of  consecutive BL/CE downlink subframes starting with subframe . For an MPDCCH-PRB-set , the ECCEs corresponding to MPDCCH candidate m of the search space  are given by

where

,

is the number of MPDCCH candidates to monitor at aggregation level in MPDCCH-PRB-set  in each subframe in the set of  consecutive subframes.

for MPDCCH UE-specific search space is determined as described in Clause 9.1.4, and  for Type0-MPDCCH common search space, Type1-MPDCCH common search space, Type1A-MPDCCH common search space, Type2-MPDCCH common search space and Type2A-MPDCCH common search space.

For , if subframe is a special subframe that does not support MPDCCH according to table 6.8B.1-1 in [3], the UE shall calculate  by assuming for normal cyclic prefix and for extended cyclic prefix.

A BL/CE UE is not expected to monitor MPDCCH in subframes that are not BL/CE DL subframes as defined in clause 7.1.

Until BL/CE UE receives higher layer configuration of MPDCCH UE-specific search space, the BL/CE UE monitors MPDCCH according to the same configuration of MPDCCH search space and Narrowband as that for MPDCCH scheduling Msg4.

The aggregation and repetition levels defining the MPDCCH search spaces and the number of monitored MPDCCH candidates are given as follows:

For MPDCCH UE-specific search space

-	if the BL/CE UE is configured with =2 or =4 PRB-pairs, and mPDCCH-NumRepetition=1, and

-	if the MPDCCH-PRB-set is configured for distributed transmission, the aggregation levels defining the search spaces and the number of monitored MPDCCH candidates are listed in Table 9.1.4-1a and Table 9.1.4-1b, where  is substituted with  for ≤24, and  is substituted with .

-	if the MPDCCH-PRB-set is configured for localized transmission, the aggregation levels defining the search spaces and the number of monitored MPDCCH candidates are listed in Table 9.1.4-2a and Table 9.1.4-2b, where  is substituted with  and  is substituted with .

-	otherwise

-	if the UE is configured with CEModeA, and =2 or =4, the aggregation and repetition levels defining the search spaces and the number of monitored MPDCCH candidates are listed in Table 9.1.5-1a

-	if the UE is configured with CEModeA, and =2+4 , the aggregation and repetition levels defining the search spaces and the number of monitored MPDCCH candidates are listed in Table 9.1.5-1b

-	if the UE is configured with CEModeB, and =2 or =4, the aggregation and repetition levels defining the search spaces and the number of monitored MPDCCH candidates are listed in Table 9.1.5-2a

-	if the UE is configured with CEModeB, and =2+4 , the aggregation and repetition levels defining the search spaces and the number of monitored MPDCCH candidates are listed in Table 9.1.5-2b

is the number of PRB-pairs configured for MPDCCH UE-specific search space. When =2+4, it is given by the higher layer parameter numberPRB-Pairs-r13, and when =2 or =4, it is given by the higher layer parameter numberPRB-Pairs-r11, except for MPDCCH candidates associated with PUR-RNTI in which case it is given by the higher layer parameter mpdcch-PRB-Pairs-r16 in PUR-Config.

, , ,  are determined from Table 9.1.5-3 by substituting the value of  with the value of higher layer parameter mPDCCH-NumRepetition, except for MPDCCH candidates associated with PUR-RNTI in which case it is given by the value of the higher layer parameter mpdcch-NumRepetition-r16 in PUR-Config.

The PRB-pairs within a Narrowband corresponding to an MPDCCH-PRB-set are indicated by higher layers and are determined using the description given in Clause 9.1.4.4.

If higher layer configuration numberPRB-Pairs-r13 or numberPRB-Pairs in PUR-MPDCCH-Config for MPDCCH-PRB-set is 6, =2+4, and the number of PRB-pairs in an MPDCCH-PRB-set = 2+4.

If Type2-MPDCCH common search space or Type2A-MPDCCH common search space,

-	PRB-pairs of the 2 PRB set in the 2+4 PRB set correspond to PRB-pairs with the largest two PRB indices in MPDCCH-PRB-set .

-	PRB-pairs of the4 PRB set in the 2+4 PRB set correspond to PRB-pairs with the smallest 4 PRB indices in MPDCCH-PRB-set .

-	PRB-pairs of the 2+4 PRB set in the 2+4 PRB set correspond to all PRB-pairs in MPDCCH-PRB-set

Table 9.1.5-1a: MPDCCH candidates monitored by a BL/CE UE 
(CEModeA, MPDCCH-PRB-set size – 2PRBs or 4PRBs)

|  | R |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | L'=2 | L'=4 | L'=8 | L'=16 | L'=24 |
| 2 | r1 | 2 | 1 | 1 | 0 | 0 |
| 4 |  | 1 | 1 | 1 | 1 | 0 |
| 2 | r2 | 2 | 1 | 1 | 0 | 0 |
| 4 |  | 1 | 1 | 1 | 1 | 0 |
| 2 | r3 | 2 | 1 | 1 | 0 | 0 |
| 4 |  | 1 | 1 | 1 | 1 | 0 |
| 2 | r4 | 2 | 1 | 1 | 0 | 0 |
| 4 |  | 1 | 1 | 1 | 1 | 0 |

Table 9.1.5-1b: MPDCCH candidates monitored by a BL/CE UE 
(CEModeA, MPDCCH-PRB-set size – 2+4PRBs)

| MPDCCH PRB set | R |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | L'=2 | L'=4 | L'=8 | L'=16 | L'=24 |
| 2 PRB set in 2+4 PRB set | r1 | 1 | 1 | 0 | 0 | 0 |
| 4 PRB set in 2+4 PRB set |  | 0 | 0 | 2 | 1 | 0 |
| Both PRB sets in 2+4 PRB set |  | 0 | 0 | 0 | 0 | 1 |
| 2 PRB set in 2+4 PRB set | r2 | 0 | 1 | 1 | 0 | 0 |
| 4 PRB set in 2+4 PRB set |  | 0 | 0 | 2 | 1 | 0 |
| Both PRB sets in 2+4 PRB set |  | 0 | 0 | 0 | 0 | 1 |
| 2 PRB set in 2+4 PRB set | r3 | 0 | 0 | 0 | 0 | 0 |
| 4 PRB set in 2+4 PRB set |  | 0 | 0 | 1 | 1 | 0 |
| Both PRB sets in 2+4 PRB set |  | 0 | 0 | 0 | 0 | 1 |
| 2 PRB set in 2+4 PRB set | r4 | 0 | 0 | 0 | 0 | 0 |
| 4 PRB set in 2+4 PRB set |  | 0 | 0 | 0 | 0 | 0 |
| Both PRB sets in 2+4 PRB set |  | 0 | 0 | 0 | 0 | 1 |

Table 9.1.5-2a: MPDCCH candidates monitored by a BL/CE UE 
(CEModeB, MPDCCH-PRB-set size – 2PRBs or 4PRBs)

|  | R |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | L'=2 | L'=4 | L'=8 | L'=16 | L'=24 |
| 2 | r1 | 0 | 0 | 1 | 0 | 0 |
| 4 |  | 0 | 0 | 1 | 1 | 0 |
| 2 | r2 | 0 | 0 | 1 | 0 | 0 |
| 4 |  | 0 | 0 | 1 | 1 | 0 |
| 2 | r3 | 0 | 0 | 1 | 0 | 0 |
| 4 |  | 0 | 0 | 1 | 1 | 0 |
| 2 | r4 | 0 | 0 | 1 | 0 | 0 |
| 4 |  | 0 | 0 | 1 | 1 | 0 |

Table 9.1.5-2b: MPDCCH candidates monitored by a BL/CE UE 
(CEModeB, MPDCCH-PRB-set size – 2+4PRBs)

| MPDCCH PRB set | R |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | L'=2 | L'=4 | L'=8 | L'=16 | L'=24 |
| 2 PRB set in 2+4 PRB set | r1 | 0 | 0 | 1 | 0 | 0 |
| 4 PRB set in 2+4 PRB set |  | 0 | 0 | 0 | 1 | 0 |
| Both PRB sets in 2+4 PRB set |  | 0 | 0 | 0 | 0 | 1 |
| 2 PRB set in 2+4 PRB set | r2 | 0 | 0 | 1 | 0 | 0 |
| 4 PRB set in 2+4 PRB set |  | 0 | 0 | 0 | 1 | 0 |
| Both PRB sets in 2+4 PRB set |  | 0 | 0 | 0 | 0 | 1 |
| 2 PRB set in 2+4 PRB set | r3 | 0 | 0 | 1 | 0 | 0 |
| 4 PRB set in 2+4 PRB set |  | 0 | 0 | 0 | 1 | 0 |
| Both PRB sets in 2+4 PRB set |  | 0 | 0 | 0 | 0 | 1 |
| 2 PRB set in 2+4 PRB set | r4 | 0 | 0 | 1 | 0 | 0 |
| 4 PRB set in 2+4 PRB set |  | 0 | 0 | 0 | 1 | 0 |
| Both PRB sets in 2+4 PRB set |  | 0 | 0 | 0 | 0 | 1 |

Table 9.1.5-3: Determination of repetition levels

| 1 | 1 | - | - | - |
| --- | --- | --- | --- | --- |
| 2 | 1 | 2 | - | - |
| 4 | 1 | 2 | 4 | - |
| >=8 |  |  |  |  |

Table 9.1.5-4: Repetition levels for Type1/1A-MPDCCH common search space

| 256 | 2 | 16 | 64 | 256 |
| --- | --- | --- | --- | --- |
| 128 | 2 | 16 | 64 | 128 |
| 64 | 2 | 8 | 32 | 64 |
| 32 | 1 | 4 | 16 | 32 |
| 16 | 1 | 4 | 8 | 16 |
| 8 | 1 | 2 | 4 | 8 |
| 4 | 1 | 2 | 4 | - |
| 2 | 1 | 2 | - | - |
| 1 | 1 | - | - | - |

For Type0-MPDCCH common search space, the narrowband location and the MPDCCH-PRB-set  are the same as for MPDCCH UE-specific search space, and

-	if =2,

-	=1 for =8 and repetition levels ,, ,  given in Table 9.1.5.-3. For all other cases, =0

-	if =4,

-	=1 for =16 and repetition levels ,, ,  given in Table 9.1.5.-3. For all other cases, =0

-	if =2+4,

-	=1 for =24 and repetition levels ,, ,  given in Table 9.1.5.-3. For all other cases, =0

where , , ,  are determined from Table 9.1.5-3 by substituting the value of  with the value of higher layer parameter mPDCCH-NumRepetition.

For Type1-MPDCCH common search space and Type1A-MPDCCH common search space, the number of PRB-pairs in MPDCCH-PRB-set is 2+4 PRB-pairs, and

-	=1 for =24 and repetition levels ,, ,  where the repetition levels are determined from Table 9.1.5-4 by substituting the value of

-	with higher layer parameter mPDCCH-NumRepetition-Paging for Type1-MPDCCH common search space, and

-	with higher layer parameter mpdcch-NumRepetitions-SC-MCCH for Type1A-MPDCCH common search space.

-	For all other cases, =0

For Type2-MPDCCH common search space, the number of PRB-pairs in MPDCCH-PRB-set is 2+4 PRB-pairs, and

-	If the most recent coverage enhancement level used for PRACH is coverage enhancement level 0 and 1, the aggregation and repetition levels defining the search spaces and the number of monitored MPDCCH candidates are determined from Table 9.1.5-1b, by assuming that the number of candidates for <8 as zero.

-	If the most recent coverage enhancement level used for PRACH is coverage enhancement level 2 and 3, the aggregation and repetition levels defining the search spaces and the number of monitored MPDCCH candidates are determined from Table 9.1.5-2b.

where , , , are determined from Table 9.1.5-3 by substituting the value of  with the value of higher layer parameter mPDCCH-NumRepetition-RA.

For Type2A-MPDCCH common search space, the number of PRB-pairs in MPDCCH-PRB-set is 2+4 PRB-pairs, and

-	for CEModeA, the aggregation and repetition levels defining the search spaces and the number of monitored MPDCCH candidates are determined from Table 9.1.5-1b, by assuming that the number of candidates for <8 as zero,

-	for CEModeB, the aggregation and repetition levels defining the search spaces and the number of monitored MPDCCH candidates are determined from Table 9.1.5-2b,

where , , , are determined from Table 9.1.5-3 by substituting the value of  with the value of higher layer parameter mpdcch-NumRepetitions-SC-MTCH.

In tables 9.1.5-1a, 9.1.5-1b, 9.1.5-2a, 9.1.5-2b, and for MPDCCH UE-specific search space when BL/CE UE is configured with =2+4 or mPDCCH-NumRepetition > 1 or mpdcch-NumRepetition > 1 in PUR-MPDCCH-Config, Type0, Type1, Type1A, Type2, Type2A MPDCCH common search space,  is applied for =4, and  is applied for =8 wherein  substituting the values of .

If a BL/CE UE is configured with higher layer parameter localizedMappingType in CRS-ChEstMPDCCH-ConfigDedicated set to 'CSI-based' or 'Reciprocity-based', and the MPDCCH-PRB-set  is configured for localized transmission, and for MPDCCH UE-specific search space or Type0-MPDCCH common search space, the UE may assume the relation between the DMRS and the CRS ports follows the predefined mapping type (as defined in [3]), for the following MPDCCH candidates

-	if , MPDCCH candidates with aggregation level  and repetition levels ,, ,

-	if , MPDCCH candidates with aggregation level  and repetition levels ,, ,

-	if , MPDCCH candidates with aggregation level  and repetition levels ,, ,

and for other MPDCCH candidates,

-	if higher layer parameter localizedMappingType in CRS-ChEstMPDCCH-ConfigDedicated is set to 'CSI-based' the UE may assume the relation between DMRS and CRS ports, as defined in [3], is based on a most recent reported PMI ending no later than subframe k0-4.

-	if higher layer parameter localizedMappingType in CRS-ChEstMPDCCH-ConfigDedicated is set to 'Reciprocity-based', the UE shall not assume any relation between the DMRS and CRS ports.

If a BL/CE UE is configured with higher layer parameter localizedMappingType in CRS-ChEstMPDCCH-ConfigDedicated set to 'CSI-based' or 'Reciprocity-based', and the MPDCCH-PRB-set  is configured for localized transmission, and  or , and rmax>2, for MPDCCH UE-specific search space or Type0-MPDCCH common search space, the antenna port for MPDCCH candidates with aggregation level L'=2 shall be changed as specified in [3].

For Type1-MPDCCH common search space, Type1A-MPDCCH common search space, Type2-MPDCCH common search space and Type2A-MPDCCH common search space, distributed MPDCCH transmission is used.

For MPDCCH UE-specific search space given by PUR-RNTI, distributed MPDCCH transmission is used.

For MPDCCH UE-specific search space, Type0-MPDCCH common search space, Type1A-MPDCCH common search space, Type2-MPDCCH common search space and Type2A-MPDCCH common search space locations of starting subframe  are given by where is the th consecutive BL/CE DL subframe from subframe , and , and , and , where

-	subframe  is a subframe satisfying the condition , where

-	For MPDCCH UE-specific search space and Type0-MPDCCH common search space,  is given by the higher layer parameter mPDCCH-startSF-UESS, except for MPDCCH candidates associated with PUR-RNTI in which case it is given by the higher layer parameter mpdcch-startSF-UESS-r16 in PUR-Config,

-	For Type1A-MPDCCH common search space,  is given by the higher layer parameter mpdcch-startSF-SC-MCCH

-	For Type2-MPDCCH common search space,  is given by the higher layer parameter mPDCCH-startSF-CSS-RA-r13

-	For Type2A-MPDCCH common search space,  is given by the higher layer parameter mpdcch-startSF-SC-MTCH

-	is given by the higher layer parameter mpdcch-Offset-SC-MTCH for Type2A-MPDCCH common search space, and by the higher layer parameter mpdcch-Offset-PUR-SS-r16 in PUR-Config for MPDCCH candidates associated with PUR-RNTI, and otherwise; and

-	is given by the higher layer parameter mPDCCH-NumRepetition for MPDCCH UE-specific search space and Type0-MPDCCH common search space, except for MPDCCH candidates associated with PUR-RNTI in which case it is given by the higher layer parameter mpdcch-NumRepetition-r16 in PUR-Config, and mPDCCH-NumRepetition-RA for Type2-MPDCCH common search space, and mpdcch-NumRepetitions-SC-MCCH for Type1A-MPDCCH common search space, and mpdcch-NumRepetitions-SC-MTCH for Type2A-MPDCCH common search space and

-	, , , are given in Table 9.1.5-3.

A BL/CE UE is not expected to be configured with values of and that result in non-integer values of .

For Type1-MPDCCH common search space, = and is determined from locations of paging opportunity subframes,

If SystemInformationBlockType1-BR or SI message is transmitted in one narrowband in subframe , a BL/CE UE shall assume MPDCCH in the same narrowband in the subframe  is dropped.

The BL/CE UE is not required to monitor an MPDCCH search space if any ECCEs corresponding to any of its MPDCCH candidates occur within a frame before and also occur within frame.

The BL/CE UE is not required to monitor an MPDCCH search space during the PUSCH transmission gap as defined in clause 5.3.4 of [3].

The BL/CE UE is not expected to be configured with overlapping MPDCCH search spaces of the same type.

A BL/CE UE configured to monitor MPDCCH candidates with CRC scrambled by C-RNTI or SPS C-RNTI with the same payload size and with the same aggregation level in the Type0-MPDCCH common search space and the MPDCCH UE-specific search space shall assume that for the MPDCCH candidates with CRC scrambled by C-RNTI or SPS C-RNTI, only the MPDCCH in the UE specific search space is transmitted.

For MPDCCH UE-specific search space or for Type0-MPDCCH common search space if the higher layer parameter mPDCCH-NumRepetition is set to 1 or the higher layer parameter mpdcch-NumRepetition in PUR-MPDCCH-Config is set to 1; or for Type2-MPDCCH common search space if the higher layer parameter mPDCCH-NumRepetition-RA is set to 1; or for Type2A-MPDCCH common search space if the higher layer parameter mpdcch-NumRepetitions-SC-MTCH is set to 1;

-	The BL/CE UE is not required to monitor MPDCCH

-	For TDD and normal downlink CP, in special subframes for the special subframe configurations 0 and 5 shown in Table 4.2-1 of [3], or for the special subframe configuration 10 configured by the higher layer signalling ssp10-CRS-LessDwPTS

-	For TDD and extended downlink CP, in special subframes for the special subframe configurations 0, 4 and 7 shown in Table 4.2-1 of [3];

otherwise

-	The BL/CE UE is not required to monitor MPDCCH

-	For TDD, in special subframes, if the BL/CE UE is configured with CEModeB

-	For TDD and normal downlink CP, in special subframes for the special subframe configurations 0, 1, 2, 5, 6, 7, 9, and 10 shown in Table 4.2-1 of [3], if the BL/CE UE is configured with CEModeA

-	For TDD and extended downlink CP, in special subframes for the special subframe configurations 0, 4 and 7 shown in Table 4.2-1 of [3], if the BL/CE UE is configured with CEModeA.

-	For TDD, in special subframes, for MPDCCH in Type1/1A-MPDCCH common search space.

If the UE has initiated a PUSCH transmission using preconfigured uplink resource ending in subframe n, the UE shall monitor the MPDCCH UE-specific search space in a search space window starting in subframe n+4+Kmac with duration given by higher layer parameter pur-MPDCCH-SS-window-duration where Kmac is provided by higher layer parameter K-mac, otherwise Kmac=0. Upon detection of a MPDCCH with DCI format 6-0A/6-0B with CRC scrambled by PUR-RNTI intended for the UE within the search space window and the corresponding DCI is for PUR ACK/fallback indication (as defined in [4]), the UE is not required to monitor the MPDCCH UE-specific search space for the remaining search space window duration.

The number of MPDCCH repetitions is indicated in the 'DCI subframe repetition number' field in the DCI according to the mapping in Table 9.1.5-5. For a BL/CE UE in half-duplex FDD operation, if the UE is configured with CEModeA, and configured with higher layer parameter ce-HARQ-AckBundling, and 'HARQ-ACK bundling flag' in the corresponding DCI is set to 1, the UE shall assume the number of MPDCCH repetitions as 1.

Table 9.1.5-5: Mapping for DCI subframe repetition number

| R | DCI subframe repetition number |
| --- | --- |
|  | 00 |
|  | 01 |
|  | 10 |
|  | 11 |

<Unchanged parts are omitted>