3GPP TSG RAN WG1 #107-e		                                   					       												                 R1-2111811

e-Meeting, November 11th – 19th, 2021

Agenda Item:	8.16.10

Source:	Moderator (AT&T)

Title:	Summary of UE features for IAB enhancements

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email discussion/approval [107-e-R17-UE-features-eIAB-01] during RAN1 #107-e. According to the Chairman’s Notes:

[107-e-R17-UE-features-eIAB-01] Email discussion UE features for IAB enhancements – Ralf (AT&T) - 1st check point: November 15 - Final check point: November 19

The following was discussed and/or agreed during RAN1 #107-e within the scope of [107-e-R17-UE-features-eIAB-01]. All proposals are based on the latest RAN1 UE features list for Rel-17 NR in Error: Reference source not found.

# Summary of Contributions Submitted to RAN1 #107-e

The following is the moderator’s summary of contributions submitted to RAN1 #107-e in this agenda item.

| 31. NR_IAB_enh | 31-1 | Guard symbols | 1) Support [Rel-17 DesiredGuardSymbols] reporting 2) Support [Rel-17 ProvidedGuardSymbols] reception |  | Yes | N/A | Guard symbols reporting and reception associated with Case 6 and 7 timings are not supported | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact Note: If an IAB node does not support a certain timing mode, the reported/provided values shall be ignored | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | In Rel-17, guard symbols enhancements are considering the adaption of the Case 6 and/or Case7 timing. Regarding the components of FG 31-1, we suggest to remove the brackets on Rel-17 ProvidedGuardSymbols and Rel-17 DesiredGuardSymbols. 31. NR_IAB_enh; 31-1; Guard symbols; 1) Support [Rel-17 DesiredGuardSymbols] reporting 2) Support [Rel-17 ProvidedGuardSymbols] reception; Yes; N/A; Guard symbols reporting and reception associated with Case 6 and 7 timings are not supported; per IAB node; No; No; support mixture of FDD/TDD and/or FR1/FR2; IAB-MT impact Note: If an IAB node does not support a certain timing mode, the reported/provided values shall be ignored; Optional with capability signalling. |
| Nokia/Nokia Shanghai Bell [3] | o Remove the brackets on components 1 and 2 (i.e. keep the text) o Pre-requisites: add "one of {31-4, 31-5} or both" |
| Intel Corporation [4] |  |
| Samsung [5] |  |
| Ericsson [6] |  |
| vivo [7] |  |
| ZTE/Sanechips [8] |  |

| 31. NR_IAB_enh | 31-2 | [Child IAB-DU beam restriction indication] | Support [DU Beam Indication] reception |  | Yes | N/A | Parent-node cannot indicate restricted beams at the IAB-DU. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | Feature group 31-2 is intended for IAB-MT to receive the indication on restricted beam(s) from parent node, which the restricted beam referred to beams at the IAB-DU. From IAB-MT perspective, the name of this FG can be update as “IAB-DU beam restriction indication”. 31. NR_IAB_enh; 31-2; Restricted DU beam indication [Child IAB-DU beam restriction indication]; Support [Simultaneous Operation DU Beam Indication] reception; Yes; N/A; Parent-node cannot indicate restricted beams at the IAB-DU.; per IAB node; No; No; support mixture of FDD/TDD and/or FR1/FR2; IAB-MT impact; Optional with capability signalling. |
| Nokia/Nokia Shanghai Bell [3] |  |
| Intel Corporation [4] | Index; Feature group; Components; Consequence if the feature is not supported by the UE. 31-2; [IAB-DU beam restriction indication]; Support [restricted DU Beam Indication] reception; Parent-node cannot indicate restricted beams at the IAB-DU.. |
| Samsung [5] | Restricted beam reception by IAB MT is for IAB DU not for child IAB DU and therefore, "Child” is removed. Proposal: "Child” is removed in Feature 31-2 |
| Ericsson [6] |  |
| vivo [7] |  |
| ZTE/Sanechips [8] |  |

| 31. NR_IAB_enh | 31-3 | [Child IAB-MT beam recommendation indication] | Support [MT Beam Indication] transmission 1) IAB-MT DL beam 2) IAB-MT UL beam |  | yes | N/A | IAB-node cannot indicate recommended/[non-preferred] MT DL/UL beam to parent node | Per IAB-node | no | no | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | FG 31-3 has the same issue. Feature group 31-3 is for beam recommendation indication from IAB MT. From IAB-MT perspective, the name of this FG can be updated as “IAB-MT beam recommendation indication”. 31. NR_IAB_enh; 31-3; Recommended/[Non-preferred] MT beam indication [Child IAB-MT beam recommendation indication]; Support [Simultaneous Operation MT Beam Indication] transmission 1) [IAB-MT DL beam 2) IAB-MT UL beam; Yes; N/A; IAB-node cannot indicate recommended/[non-preferred] MT DL/UL beam to parent node; Per IAB-node; no; no; support mixture of FDD/TDD and/or FR1/FR2; IAB-MT impact; Optional with capability signalling. |
| Nokia/Nokia Shanghai Bell [3] |  |
| Intel Corporation [4] | Index; Feature group; Components; Consequence if the feature is not supported by the UE. 31-3; [IAB-MT beam recommendation indication]; Support [recommended MT Beam Indication] transmission 1) IAB-MT DL beam 2) IAB-MT UL beam; IAB-node cannot indicate recommended/[non-preferred] MT DL/UL beam to parent node. |
| Samsung [5] | Similar to the discussion in Feature 31-2, recommended beam indication by IAB MT is for IAB MT not for child IAB MT and therefore, "Child” is removed which is aligned with decriptions in Component. Proposal: "Child” is removed in Feature 31-3 |
| Ericsson [6] |  |
| vivo [7] |  |
| ZTE/Sanechips [8] | According to the following agreements from previous meeting, only recommended beam is supported for the beam indication from the IMT-MT to its parent node. Agreement The recommended beam indication from the IAB-MT to the parent node are provided via MAC-CE: - For DL Rx beam(s): using one or more of the following: - DL TCI state ID - FFS: UE/IAB-MT does not assume that DL Tx power adjustment (if provided) is applied to the SSB index (if supported) indicated as QCLed reference signal in DL TCI state ID. - [RS ID] - [R17 DL TCI, or joint DL/UL TCI ID] - For UL Tx beam(s): using one or more of the following: - [Spatial relation] - [RS ID] - [R17 UL TCI, or joint DL/UL TCI ID] - [SRI]. So we propose to update FG31-3 to reflect the above agreement Proposal: Adopt the following modification to 31-3 to reflect the agreement of RAN1#106bis-e 31-3; [Child IAB-MT beam recommendation indication]; Support [MT Beam Indication] transmission 1) IAB-MT DL beam 2) IAB-MT UL beam; yes; N/A; IAB-node cannot indicate recommended/[non-preferred] MT DL/UL beam to parent node; Per IAB-node; no; no; support mixture of FDD/TDD and/or FR1/FR2; IAB-MT impact; Optional with capability signalling.. |

| 31. NR_IAB_enh | 31-4 | [Case 6 timing alignment] | Support [Case 6 transmission] |  | Yes | N/A | Case 6 timing at the IAB-node is not supported. Switching across different timing cases (i.e., Case 1 at IAB-node, Case 6 at IAB-node, and/or Case 7 at the Parent) is not supported. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] |  |
| Intel Corporation [4] | This FG 31-4 should be related to RAN1#106e agreement: - An IAB-node is explicitly indicated by the parent node when Case 6 timing is performed at the IAB-node at least for specific time resources. However, current FG description does not clearly reflect the signaling reception from parent-node to indicate Case#6 timing operation. The following modifications are suggested. Index; Feature group; Components; Consequence if the feature is not supported by the UE. 31-4; [Case 6 timing alignment indication]; Support [Case 6 timing alignment indication reception ]; Case 6 timing at the IAB-node is not supported. Switching across different timing cases (i.e., Case 1 at IAB-node, Case 6 at IAB-node, and/or Case 7 at the Parent) is not supported.. |
| Samsung [5] |  |
| Ericsson [6] |  |
| vivo [7] | Regarding the timing mode indication, case 6 timing mode indication and case 7 timing mode indication should be defined separately. Especially considering that, Case 6 timing mode indication is indicated from parent node to IAB node to enable the Case 6 timing mode, but Case 7 timing mode indication is indicated from IAB node which to enable the Case 7 timing mode for its child node. Since the transmission and reception nodes of Case 6 and Case 7 timing indication are different nodes, it is reasonable to split the timing mode indication into two sub-features. Based on the updated features, Case 6 timing alignment and Case 7 timing alignment are described by FG 31-4 and FG 31-5, respectively. It is believed that Case 6 timing mode indication should be captured in this FG 31-4, and Case 7 timing mode indication should be captured in this FG 31-5. Moreover, in RAN1#106e e-meeting, it was concluded that “Details on the design of the indication of when Case 6 timing and Case 7 timing is performed at the IAB-node are to be discussed under 8.10.1.”. That means, whether to use timing case indication or multiplexing case indication is still not decided. Proposal: The FG for timing case indication is respectively defined for case 6 timing indication and case 7 timing indication, e.g., defined in FG 31-4 and FG 31-5, respectively. Proposal: RAN1 further discuss whether to term the FG 31-4/FG 31-5 as ‘timing case alignment’ or ‘multiplexing case indication’. Regarding the feature that child MT to receive a timing offset for Case 7 timing alignment, a separated FG can be defined, if timing case indication is finally replaced by multiplexing case indication. Proposal: If FG 31-4/FG 31-5 as ‘multiplexing case indication’, additional FG is defined for child MT to receive a timing offset for Case 7 timing alignment. |
| ZTE/Sanechips [8] | Moreover, based on the following agreements in RAN1#106-e: Agreement For Case 7 timing at a parent node, the IAB-MT Tx timing of the node is obtained via the legacy TA loop plus an offset from the parent node. - FFS range, granularity, and signaling details of the offset. Agreement An IAB-node is explicitly indicated by the parent node when Case 6 timing is performed at the IAB-node at least for specific time resources. - FFS: whether the indication should be associated with another dimensions, e.g. multiplexing cases - FFS whether an IAB-node is explicitly indicated by the parent node when Case 7 timing is performed at the IAB-node. Agreement An IAB-node is explicitly indicated by the parent node when Case 7 timing is performed at the parent node. - FFS for signalling details. A parent node may explicitly indicated the following information to an IAB-MT: - When Case 6 timing is performed at the IAB-node. - When Case 7 timing is performed at the parent node. - IAB-MT Tx timing offset for Case 7 timing at the parent node. - FFS whether when Case 7 timing is performed at the IAB-node is also indicated. So we propose to update FG31-4 and 31-5 to reflect the above agreements. Proposal: Adopt the following modification to 31-4 and 31-5 to reflect the agreements of RAN1#106-e 31-4; [Case 6 timing alignment]; 1.) Support [Case 6 transmission] 2.) Support [Case 6 timing at IAB-node indication] reception; Yes; N/A; Case 6 timing at the IAB-node is not supported. Switching across different timing cases (i.e., Case 1 at IAB-node, Case 6 at IAB-node, and/or Case 7 at the Parent) is not supported. When to perform Case 6 timing at the IAB-node cannot be controlled by the parent node.; per IAB node; No; No; support mixture of FDD/TDD and/or FR1/FR2; IAB-MT impact; Optional with capability signalling.. |

| 31. NR_IAB_enh | 31-5 | Case 7 timing [alignment] | Support [Case7 transmission] |  | Yes | N/A | Parent-node cannot adopt both (and switch between) Case 1 and Case 7 timing. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 31. NR_IAB_enh; 31-5; Case 7 timing [offset alignment]; Support [Case7 transmission] - support Case 7 timing; - switching between Case 7 and other timing case.; Yes; N/A; Parent-node cannot adopt both (and switch between) Case 1 and Case 7 timing.; per IAB node; No; No; support mixture of FDD/TDD and/or FR1/FR2; IAB-MT impact; Optional with capability signalling. |
| Nokia/Nokia Shanghai Bell [3] |  |
| Intel Corporation [4] | This FG 31-5 should be related to RAN1#106e-RAN1#106bis-e agreements: - For Case 7 timing at a parent node, the IAB-MT Tx timing of the node is obtained via the legacy TA loop plus an offset from the parent node. - Case 7 UL timing offset is indicated by the parent-node via MAC-CE. However, current FG 31-5 description does not clearly reflect the Case#7 offset reception from parent-node. The following modifications are suggested. Index; Feature group; Components; Consequence if the feature is not supported by the UE. 31-5; Case 7 timing offset; Support [Case7 timing offset reception; Parent-node cannot adopt both (and switch between) Case 1 and Case 7 timing.. |
| Samsung [5] |  |
| Ericsson [6] |  |
| vivo [7] | Regarding the timing mode indication, case 6 timing mode indication and case 7 timing mode indication should be defined separately. Especially considering that, Case 6 timing mode indication is indicated from parent node to IAB node to enable the Case 6 timing mode, but Case 7 timing mode indication is indicated from IAB node which to enable the Case 7 timing mode for its child node. Since the transmission and reception nodes of Case 6 and Case 7 timing indication are different nodes, it is reasonable to split the timing mode indication into two sub-features. Based on the updated features, Case 6 timing alignment and Case 7 timing alignment are described by FG 31-4 and FG 31-5, respectively. It is believed that Case 6 timing mode indication should be captured in this FG 31-4, and Case 7 timing mode indication should be captured in this FG 31-5. Moreover, in RAN1#106e e-meeting, it was concluded that “Details on the design of the indication of when Case 6 timing and Case 7 timing is performed at the IAB-node are to be discussed under 8.10.1.”. That means, whether to use timing case indication or multiplexing case indication is still not decided. Proposal: The FG for timing case indication is respectively defined for case 6 timing indication and case 7 timing indication, e.g., defined in FG 31-4 and FG 31-5, respectively. Proposal: RAN1 further discuss whether to term the FG 31-4/FG 31-5 as ‘timing case alignment’ or ‘multiplexing case indication’. Regarding the feature that child MT to receive a timing offset for Case 7 timing alignment, a separated FG can be defined, if timing case indication is finally replaced by multiplexing case indication. Proposal: If FG 31-4/FG 31-5 as ‘multiplexing case indication’, additional FG is defined for child MT to receive a timing offset for Case 7 timing alignment. |
| ZTE/Sanechips [8] | Moreover, based on the following agreements in RAN1#106-e: Agreement For Case 7 timing at a parent node, the IAB-MT Tx timing of the node is obtained via the legacy TA loop plus an offset from the parent node. - FFS range, granularity, and signaling details of the offset. Agreement An IAB-node is explicitly indicated by the parent node when Case 6 timing is performed at the IAB-node at least for specific time resources. - FFS: whether the indication should be associated with another dimensions, e.g. multiplexing cases - FFS whether an IAB-node is explicitly indicated by the parent node when Case 7 timing is performed at the IAB-node. Agreement An IAB-node is explicitly indicated by the parent node when Case 7 timing is performed at the parent node. - FFS for signalling details. A parent node may explicitly indicated the following information to an IAB-MT: - When Case 6 timing is performed at the IAB-node. - When Case 7 timing is performed at the parent node. - IAB-MT Tx timing offset for Case 7 timing at the parent node. - FFS whether when Case 7 timing is performed at the IAB-node is also indicated. So we propose to update FG31-4 and 31-5 to reflect the above agreements. Proposal: Adopt the following modification to 31-4 and 31-5 to reflect the agreements of RAN1#106-e 31-5; Case 7 timing [alignment]; 1.) Support [Case7 transmission] 2.) Support [Case 7 timing at parent-node indication] and [offset indication] reception; Yes; N/A; Parent-node cannot adopt both (and switch between) Case 1 and Case 7 timing. When to perform Case 7 timing at the parent IAB-node cannot be indicated to its child node.; per IAB node; No; No; support mixture of FDD/TDD and/or FR1/FR2; IAB-MT impact; Optional with capability signalling.. |

| 31. NR_IAB_enh | 31-6 | DL TX power adjustment | 1.) Support [Desired DL TX Power Adjustment] reporting 2.) Support [DL TX Power Adjustment] reception |  | Yes | N/A | Parent-node’s DL TX power adjustment reporting and reception is not supported. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | For FG 31-6, there are two components: 1.) Support [Desired DL TX Power Adjustment] reporting; 2) support [DL TX Power Adjustment] reception. We suggest to remove the brackets of components as it precisely describes this FG. For the consequence, “parent-node’s DL TX power adjustment reporting” should be updated as “Desired parent-node’s DL TX power adjustment reporting and DL TX power adjustment reception is not supported”. 31. NR_IAB_enh; 31-6; DL TX power adjustment; 1.) Support [Desired DL TX Power Adjustment] reporting 2) support [DL TX Power Adjustment] reception; Yes; N/A; Parent-node’s Desired DL TX power adjustment reporting and reception is not supported; per IAB node; No; No; support mixture of FDD/TDD and/or FR1/FR2; IAB-MT impact; Optional with capability signalling. |
| Nokia/Nokia Shanghai Bell [3] |  |
| Intel Corporation [4] |  |
| Samsung [5] |  |
| Ericsson [6] |  |
| vivo [7] |  |
| ZTE/Sanechips [8] |  |

| 31. NR_IAB_enh | 31-7 | [Desired] UL TX power adjustment | Support [Desired IAB-MT PSD range] reporting |  | Yes | N/A | Desired MT’s UL PSD range reporting is not supported. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] |  |
| Intel Corporation [4] |  |
| Samsung [5] |  |
| Ericsson [6] |  |
| vivo [7] |  |
| ZTE/Sanechips [8] |  |

| 31. NR_IAB_enh | [31-8] | [Dynamic indication of Rel-17 or FDM soft resource availability] | Support monitoring DCI Format 2_5 scrambled by AI-RNTI for indication of FDM soft resource availability to an IAB node |  | Yes | N/A | The IAB-node is unable to receive explicit availability indication for Soft resources | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | On FG 31-9, the consequence should be ““The IAB node cannot receive explicit availability indication for Rel-17 FDM soft resources” since the IAB node may still receive explicit availability indication for Rel-16 soft resources depending on capability. 31. NR_IAB_enh; [31-8]; [Dynamic indication of Rel-17 or FDM soft resource availability]; Support monitoring DCI Format 2_5 scrambled by AI-RNTI for indication of FDM soft resource availability to an IAB node; Yes; N/A; Desired MT’s UL PSD range reporting is not supported The IAB-node is unable to receive explicit availability indication for Rel-17 FDM Soft resources; per IAB node; No; No; support mixture of FDD/TDD and/or FR1/FR2; IAB-MT impact; Optional with capability signalling. |
| Nokia/Nokia Shanghai Bell [3] |  |
| Intel Corporation [4] |  |
| Samsung [5] |  |
| Ericsson [6] |  |
| vivo [7] |  |
| ZTE/Sanechips [8] |  |

| 31. NR_IAB_enh | 31-9 | Simultaneous transmission and reception from multiple parent nodes | Support simultaneous transmission and reception from multiple parent nodes |  | Yes | N/A | Simultaneous transmission and reception is not supported in DC scenario | per BC | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] |  |
| Intel Corporation [4] |  |
| Samsung [5] |  |
| Ericsson [6] |  |
| vivo [7] |  |
| ZTE/Sanechips [8] |  |

Other issues

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Nokia/Nokia Shanghai Bell [3] |  |
| Intel Corporation [4] |  |
| Samsung [5] |  |
| Ericsson [6] | In the RAN1#106bis meeting, there was a discussion on multiplexing operation adaptation and the feature leader has the following proposal: Proposal 2.3.1a: In Rel-17, the semi-static indication of the multiplexing capability of an IAB node for the case of no-TDM between IAB MT and IAB DU is additionally provided with respect to each transmission-direction combination (per MT CC/DU cell pair): - Support for FDM (FFS: desired guard band) - Support for SDM (i.e. parent beam restrictions and/or child beam recommendations) - Supported timing modes - Support for DL Tx power adjustment The feature leader conclusion is given as below: Given the discussion, it seems the issue of supported multiplexing capabilities can be handled directly in 8.17.10 (IAB “UE” Features) Even though the companies are supportive to the intent of the proposal, but most companies (7 of 13) have indicated a need of further clarification and discussion of Proposal 2.3.1a. It should be noted that in Rel-16 IAB, the semi-static multiplexing capability reporting is not considered as a UE feature since this capability is reported by the IAB-DU to the donor IAB-node via F1 signaling. We partly agree to the feature leader that the Rel-16 semi-static indication of multiplexing capability should be enhanced. Specifically, an IAB-node should report its FDM capability to the donor-CU, accordingly the donor-CU may provide the IAB-DU with the semi-static frequency domain resource configuration. One example is to simply add one more value “supported-FDM required” to the two cases DU_RX/MT_RX and DU_TX/MT_TX in the current Multiplexing Info IE Error: Reference source not found. - Extend the semi-static IAB node multiplexing capability to include FDM-required for multiplexing modes MT RX/DU RX and MT TX/DU TX. Adapt the following change to TS 38.473. -------------------- Proposed modification of TS 38.473 ---------------------------- #### 9.3.1.108 Multiplexing Info This IE contains information about the multiplexing capabilities between the gNB-DU’s cell and the cells configured on the collocated IAB-MT. IE/Group Name; Presence; Range; IE type and reference; Semantics description. IAB-MT Cell List; 1. >IAB-MT Cell Item; 1 .. <maxnoofServingCells>. >>NR Cell Identity; M; BIT STRING (SIZE(36)); Cell identity of a serving cell configured for a collocated IAB-MT.. >>DU_RX/MT_RX; M; ENUMERATED (supported, not supported, supported-FDM required); An indication of whether the IAB-node supports simultaneous reception at its DU and MT side.. >>DU_TX/MT_TX; M; ENUMERATED (supported, not supported, supported-FDM required); An indication of whether the IAB-node supports simultaneous transmission at its DU and MT side.. >>DU_TX/MT_RX; M; ENUMERATED (supported, not supported); An indication of whether the IAB-node supports simultaneous transmission at its DU and reception at its MT side.. >>DU_RX/MT_TX; M; ENUMERATED (supported, not supported); An indication of whether the IAB-node supports simultaneous reception at its DU and transmission at its MT side.. Range bound; Explanation. maxnoofServingCells; Maximum no. of serving cells for IAB-MT. Value is 32, as defined by the maxNrofServingCells in TS 38.331 [8].. -------------------- End of proposed modification of TS 38.473 ---------------------------- After receiving the semi-static configuration from the donor-CU, the IAB-node should initialize the exchange of the operational parameters, i.e., multiplexing conditions, for simultaneous operation with its parent node and child node. We do not see a need to inform the donor-CU about IAB-node’s ability to exploit simultaneous operation regarding timing mode, power adjustment, beam indication and guard band. The two agreements below from RAN1#104e and RAN1#105e clearly state that the IAB-node the information on multiplexing conditions (e.g., timing mode, power control, beam indication etc) can be dynamically provided to the parent IAB-node. Agreement (RAN#1104e) Support indication/reporting of information between an IAB node and its parent node to assist in the determination of the applicability of a given multiplexing capability in case of simultaneous operation. The following solutions are considered (other solutions not precluded): - Temporal applicability of a given multiplexing capability - Time/frequency resource restrictions (e.g. access vs. backhaul links, DL vs. UL resources) - Indications of conditions/reporting information required to realize the given multiplexing capability, (e.g. timing mode, power control, guard symbols, etc.) FFS: channels/signals used for indicating/reporting information Agreement (RAN1#105e) The parent IAB-node is dynamically provided with conditions/parameters to facilitate adaptation between multiplexing operation modes: - FFS: Required number of guard symbols for switching of multiplexing mode (FFS: per timing mode or per multiplexing mode) for IAB-DU - FFS: Signalling procedure - FFS: Required guard band for FDM - FFS: other conditions, e.g. required timing mode, required power control parameters, and preferred TCI.. In case of simultaneous MT TX/DU TX, the IAB-node can report to the parent node about the multiplexing conditions on whether it requires 1) Case 6 timing, 2) Parent node indication of restricted IAB-DU beams, and 3) IAB-node indication to parent node to support its MT’s UL TX power control. Similarly, in case of simultaneous MT RX/DU RX, the IAB-node can report to the parent node about the multiplexing conditions on whether it requires or supports 1) Case 7 timing, 2) parent node in Case 7 timing, 3) IAB-node indication of recommended IAB-MT beams to parent node, and 4) IAB-node indication to support parent node DL power control. - The IAB-node may report to the parent node a set of multiplexing conditions on the required timing mode, power control, beam indication. - For simultaneous MT-TX/DU-TX, the multiplexing conditions include whether the IAB-node requires: - Case-6 timing - Parent node indication of restricted IAB-DU beams - IAB-node indication to its parent node on its MT’s minimum UL TX power level - For simultaneous MT RX/DU RX, the multiplexing conditions include whether the IAB-node: - Supports adjustment of child node UL TX timing in Case-7 timing - Supports adjustment of UL TX timing for parent node in Case-7 timing - Requires IAB-node indication of recommended IAB-MT beams - Requires IAB-node indication to support parent node maximum DL power level |
| vivo [7] |  |
| ZTE/Sanechips [8] |  |

# Discussion/Approval Items during RAN1 #107-e — First Checkpoint

After review of contributions submitted to RAN1 #107-e in this agenda item, the following topics were identified by the moderator for discussion/approval during RAN1 #107-e.

| Company | Comments/Questions/Suggestions |
| --- | --- |

# Issue 1: FG 31-1

After review of contributions submitted to RAN1 #107-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in red

| 31. NR_IAB_enh | 31-1 | Guard symbols | 1) Support [Rel-17 DesiredGuardSymbols] reporting 2) Support [Rel-17 ProvidedGuardSymbols] reception | one or more of {31-4, 31-5} | Yes | N/A | Guard symbols reporting and reception associated with Case 6 and 7 timings are not supported | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact Note: If an IAB node does not support a certain timing mode, the reported/provided values shall be ignored | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | Agree |
| Nokia, NSB | Support |
| Vivo | Agree |

# Issue 2: FG 31-2

After review of contributions submitted to RAN1 #107-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in red

| 31. NR_IAB_enh | 31-2 | [Child IAB-DU beam restriction indication] | Support [restricted DU Beam Indication] reception |  | Yes | N/A | Parent-node cannot indicate restricted beams at the IAB-DU. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | Agree, and to be even more correct, the Component text should be Support [restricted IAB-DU Beam Indication] reception |
| Nokia, NSB | Support |
| vivo | Support |

# Issue 3: FG 31-3

After review of contributions submitted to RAN1 #107-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in red

| 31. NR_IAB_enh | 31-3 | [Child IAB-MT beam recommendation indication] | Support [recommended MT Beam Indication] transmission 1) IAB-MT DL beam 2) IAB-MT UL beam |  | yes | N/A | IAB-node cannot indicate recommended/[non-preferred] MT DL/UL beam to parent node | Per IAB-node | no | no | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | Agree, and to be even more correct the Component text should be Support [recommended IAB-MT Beam Indication] transmission and Consequence text should be IAB-node cannot indicate recommended/[non-preferred] IAB-MT DL/UL beam to parent node |
| Nokia, NSB | Support |
| vivo | Support |

# Issue 4: FG 31-4

After review of contributions submitted to RAN1 #107-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in red

| 31. NR_IAB_enh | 31-4 | [Case 6 timing alignment indication] | 1.) Support [Case 6 transmission] 2.) Support Case 6 timing alignment indication reception |  | Yes | N/A | Case 6 timing at the IAB-node is not supported. Switching across different timing cases (i.e., Case 1 at IAB-node, Case 6 at IAB-node, and/or Case 7 at the Parent) is not supported. When to perform Case 6 timing at the IAB-node cannot be controlled by the parent node. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | We agree with all changes but the proposed Components. The motivation for the Component is the indication by the parent node of when to perform Case-6 timing. In this respect, we agree with adding the second Component and propose to remove the first Component Support [Case 6 transmission], since it is at least redundant but also misleading as it could be understood now as the ability of performing Case-6 transmissions. |
| Nokia, NSB | OK in general, but it is unclear what is the value of having both components together. |
| vivo | OK |

# Issue 5: FG 31-5

After review of contributions submitted to RAN1 #107-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in red

| 31. NR_IAB_enh | 31-5 | Case 7 timing [alignment] | 1.) Support [Case7 transmission] 2.) Support Case7 timing offset indication reception 3.) Support Case 7 timing at parent-node indication |  | Yes | N/A | Parent-node cannot adopt both (and switch between) Case 1 and Case 7 timing. When to perform Case 7 timing at the parent IAB-node cannot be indicated to its child node. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | We agree with all changes but the proposed Components. The motivation for the Components is the signaling of the Case-7 timing offset and indication by the parent node of when to perform Case-7 timing. In this respect, we agree with adding the second and third Component and propose to remove the first Component Support [Case7 transmission], since it is at least redundant but also misleading as it could be understood now as the ability of performing Case-7 UL transmissions. |
| Nokia, NSB | The new components are unclear. Otherwise OK with the changes. Another issue is the consequence if not supported, which is a bit too strict in our view. In principle case 1 and case 7 timing can both be supported using legacy TA, although quickly switching between the two may not be possible. The new offset simply allows the IAB MT to maintain two timing references which could be switched between. Without the offset the timing reference could be changed by providing a TA update, although there are some limitations with latency and range of offset. |
| vivo | OK |

# Issue 6: FG 31-6

After review of contributions submitted to RAN1 #107-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in red

| 31. NR_IAB_enh | 31-6 | DL TX power adjustment | 1.) Support [Desired DL TX Power Adjustment] reporting 2.) Support [DL TX Power Adjustment] reception |  | Yes | N/A | Parent-node’s DL TX power adjustment reporting and reception is not supported. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | Agree |
| Nokia, NSB | Support |
| vivo | Agree |

# Issue 7: FG 31-7

After review of contributions submitted to RAN1 #107-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in red

| 31. NR_IAB_enh | 31-7 | [Desired] UL TX power adjustment | Support [Desired IAB-MT PSD range] reporting |  | Yes | N/A | Desired MT’s UL PSD range reporting is not supported. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | Agree in principle. It is not customary to have “wanted”, “desired” etc. in the FG name. Furthermore, the feature is about reporting a preferred power range and not about adjusting power. We propose renaming the FG to “UL Tx power range for simultaneous transmission”. We also propose a change in the Consequences: “Reporting of Desired IAB-MT’s desired UL PSD range for simultaneous transmission reporting is not supported.” |
| Nokia, NSB | Ericsson’s revisions are OK to us. |
| vivo | Support the current version. The relation between UL PSD report and multiplexing case is still under discussion in 8.10.2 |

# Issue 8: FG 31-8

After review of contributions submitted to RAN1 #107-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Confirm FG 31-8 and adopt the following changes highlighted in red

| 31. NR_IAB_enh | [31-8] | [Dynamic indication of Rel-17 or FDM soft resource availability] | Support monitoring DCI Format 2_5 scrambled by AI-RNTI for indication of FDM soft resource availability to an IAB node |  | Yes | N/A | The IAB-node is unable to receive explicit availability indication for Rel-17 FDM soft resources | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | Agree in principle. The description of Consequences and Components should be aligned with the FG name where it is stated “Rel-17 or FDM soft resource” which is also our preference, since the design details are not yet agreed. |
| Nokia, NSB | Given design details are still being worked on, it might make more sense to revisit the definitions later. |
| vivo | We prefer to use agreed wording for the feature description, i.e., Rel-17 frequency soft resources, instead of Rel-17 FDM soft resources. |

# Issue 9: Set of Multiplexing Conditions

One company is proposing the following, see summary in Section 2 or R1-2112359 [6] for details.

Proposal: The IAB-node may report to the parent node a set of multiplexing conditions on the required timing mode, power control, beam indication.

- For simultaneous MT-TX/DU-TX, the multiplexing conditions include whether the IAB-node requires:

  - Case-6 timing

  - Parent node indication of restricted IAB-DU beams

  - IAB-node indication to its parent node on its MT’s minimum UL TX power level

- For simultaneous MT RX/DU RX, the multiplexing conditions include whether the IAB-node:

  - Supports adjustment of child node UL TX timing in Case-7 timing

  - Supports adjustment of UL TX timing for parent node in Case-7 timing

  - Requires IAB-node indication of recommended IAB-MT beams

  - Requires IAB-node indication to support parent node maximum DL power level

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | Agree |
| Nokia, NSB | It is unclear why this is being discussed in this thread. Do not support. |
| vivo | The beam reporting/indication or power reporting/indication have been captured in other FGs. Not sure why to repeat the feature. |

# Issue 10: Text Proposal for TS 38.473

One company is proposing the following, see summary in Section 2 or R1-2112359 [6] for details incl. the text proposal to TS 38.473

Proposal:

- Extend the semi-static IAB node multiplexing capability to include FDM-required for multiplexing modes MT RX/DU RX and MT TX/DU TX

- Adapt the text proposal to TS 38.473 in R1-2112359

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | Agree to the first bullet. The second bullet should be changed to - Send LS to RAN3, asking for a change in TS 38.473 according to the proposed modification. |
| Nokia, NSB | Do not agree, RAN1 does not have the mandate to decided on text proposals to TS38.473. Any such discussions should take place in the WI AIs, not in UE feature discussions. |
| vivo | No. Rel-16 multiplexing capability report is sufficient. |

# Discussion/Approval Items during RAN1 #107-e — Second Checkpoint

Based on the comments/questions/suggestions received by the first checkpoint, the following are the revised proposals and/or proposed agreements by the moderator. Companies submitted the following views on the moderator’s proposals.

[Please submit all comments/questions/suggestions here, late comments/questions/suggestions submitted in Section 3 will not be considered]

General comments

| Company | Comments/Questions/Suggestions |
| --- | --- |

# Issue 1: FG 31-1

Proposed Agreement: Adopt the following changes highlighted in chromatic formatting

| 31. NR_IAB_enh | 31-1 | Guard symbols | 1) Support [Rel-17 DesiredGuardSymbols] reporting 2) Support [Rel-17 ProvidedGuardSymbols] reception | one or more of {31-4, 31-5} | Yes | N/A | Guard symbols reporting and reception associated with Case 6 and 7 timings are not supported | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact Note: If an IAB node does not support a certain timing mode, the reported/provided values shall be ignored | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | Agree |
| Nokia, NSB | Support |
| ZTE, Sanechips | Agree |
| vivo | Agree |
| AT&T | Agree |
| Samsung | Support |
| Huawei, HiSilicon | Agree |

# Issue 2: FG 31-2

Proposed Agreement: Adopt the following changes highlighted in chromatic formatting

| 31. NR_IAB_enh | 31-2 | [Child IAB-DU beam restriction indication] | Support [restricted IAB-DU Beam Indication] reception |  | Yes | N/A | Parent-node cannot indicate restricted beams at the IAB-DU. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | Agree, and to be even more correct, the Component text should be Support [restricted IAB-DU Beam Indication] reception |
| Nokia, NSB | Support |
| ZTE, Sanechips | Agree |
| vivo | Agree |
| AT&T | Agree |
| Samsung | Support |
| Huawei, HiSilicon | Support |

# Issue 3: FG 31-3

Proposed Agreement: Adopt the following changes highlighted in chromatic formatting

| 31. NR_IAB_enh | 31-3 | [Child IAB-MT beam recommendation indication] | Support [recommended IAB-MT Beam Indication] transmission 1) IAB-MT DL beam 2) IAB-MT UL beam |  | yes | N/A | IAB-node cannot indicate recommended/[non-preferred] IAB-MT DL/UL beam to parent node | Per IAB-node | no | no | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | Agree, and to be even more correct the Component text should be Support [recommended IAB-MT Beam Indication] transmission and Consequence text should be IAB-node cannot indicate recommended/[non-preferred] IAB-MT DL/UL beam to parent node |
| Nokia, NSB | Support |
| ZTE, Sanechips | Agree |
| vivo | Agree |
| AT&T | Agree |
| Samsung | Support |
| Huawei, HiSilicon | Support |

# Issue 4: FG 31-4

Proposed Agreement: Adopt the following changes highlighted in chromatic formatting

| 31. NR_IAB_enh | 31-4 | [Case 6 timing alignment indication] | 1.) Support [Case 6 transmission] 2.) Support Case 6 timing alignment indication reception |  | Yes | N/A | Case 6 timing at the IAB-node is not supported. Switching across different timing cases (i.e., Case 1 at IAB-node, Case 6 at IAB-node, and/or Case 7 at the Parent) is not supported. When to perform Case 6 timing at the IAB-node cannot be controlled by the parent node. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | We agree with all changes but the proposed Components. The motivation for the Component is the indication by the parent node of when to perform Case-6 timing. In this respect, we agree with adding the second Component and propose to remove the first Component Support [Case 6 transmission], since it is at least redundant but also misleading as it could be understood now as the ability of performing Case-6 transmissions. |
| Nokia, NSB | OK in general, but it is unclear what is the value of having both components together. |
| ZTE, Sanechips | Support. |
| vivo | Agree |
| AT&T | If the first component is removed we should remove the following from the consequence description: “Case 6 timing at the IAB-node is not supported.” Since that is now only an implicit assumption. |
| Samsung | Similar view with AT&T |
| Ericsson | To be aligned with the Case 1 (Case 7) timing, we would prefer a FG name “Case 6 timing alignment” |
| Huawei, HiSilicon | We think the name of the FG should be changed back to “Case 6 timing alignment” since it represent the IAB-MT can transmit UL using case 6 timing. Then both components are fine as they are. |

# Issue 5: FG 31-5

Proposed Agreement: Adopt the following changes highlighted in chromatic formatting

| 31. NR_IAB_enh | 31-5 | Case 7 timing [alignment] | 1.) Support [Case7 transmission] 1.) Support Case7 timing offset indication reception 2.) Support Case 7 timing at parent-node indication |  | Yes | N/A | Parent-node cannot adopt both (and switch between) Case 1 and Case 7 timing. | Per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | We agree with all changes but the proposed Components. The motivation for the Components is the signaling of the Case-7 timing offset and indication by the parent node of when to perform Case-7 timing. In this respect, we agree with adding the second and third Component and propose to remove the first Component Support [Case7 transmission], since it is at least redundant but also misleading as it could be understood now as the ability of performing Case-7 UL transmissions. |
| Nokia, NSB | The new components are unclear. Otherwise OK with the changes. Another issue is the consequence if not supported, which is a bit too strict in our view. In principle case 1 and case 7 timing can both be supported using legacy TA, although quickly switching between the two may not be possible. The new offset simply allows the IAB MT to maintain two timing references which could be switched between. Without the offset the timing reference could be changed by providing a TA update, although there are some limitations with latency and range of offset. |
| ZTE, Sanechips | Agree in principle, for the new Components 1) and 2), we can live with it , but since they are always coupled, one combined component is slightly preferred. |
| Vivo | Agree |
| AT&T | If component 1 is removed, the name of the FG should be “Case 7 timing indication” since the FG only implicitly indicates support of Case 7 timing at the IAB node |
| Samsung | Similar view with AT&T |
| Ericsson | To be aligned with the Case 1 (Case 6) timing, we would prefer a FG name “Case 7 timing alignment” |
| Huawei, HiSilicon | We support the FG as its current form. |

# Issue 6: FG 31-6

Proposed Agreement: Adopt the following changes highlighted in chromatic formatting

| 31. NR_IAB_enh | 31-6 | DL TX power adjustment | 1.) Support [Desired DL TX Power Adjustment] reporting 2.) Support [DL TX Power Adjustment] reception |  | Yes | N/A | Parent-node’s DL TX power adjustment reporting and reception is not supported. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | Agree |
| Nokia, NSB | Support |
| ZTE, Sanechips | Agree |
| vivo | agree |
| AT&T | Agree |
| Samsung | Support |
| Huawei, HiSilicon | Support |

# Issue 7: FG 31-7

Proposed Agreement: Adopt the following changes highlighted in chromatic formatting

| 31. NR_IAB_enh | 31-7 | [Desired] UL TX power adjustment range for simultaneous transmission | Support [Desired IAB-MT PSD range] reporting |  | Yes | N/A | Desired Reporting of IAB-MT’s desired UL PSD range for simultaneous transmission reporting is not supported. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | Agree in principle. It is not customary to have “wanted”, “desired” etc. in the FG name. Furthermore, the feature is about reporting a preferred power range and not about adjusting power. We propose renaming the FG to “UL Tx power range for simultaneous transmission”. We also propose a change in the Consequences: “Reporting of Desired IAB-MT’s desired UL PSD range for simultaneous transmission reporting is not supported.” |
| Nokia, NSB | Ericsson’s revisions are OK to us. |
| ZTE, Sanechips | Agree. |
| vivo | The relation between UL PSD report and multiplexing case is still under discussion in 8.10.2. Not prefer to make conclusion in this AI. Reporting of IAB-MT’s desired UL PSD range for simultaneous transmission reporting is not supported |
| AT&T | Suggest to keep the FG name in brackets until the matching higher layer parameter indicating the PSD range is finalized. |
| Samsung | OK with keeping current one because the issue is still being discussed in 8.10.2 |
| Huawei, HiSilicon | Okay |

# Issue 8: FG 31-8

Proposed Agreement: Confirm FG 31-8 and adopt the following changes highlighted in chromatic formatting

| 31. NR_IAB_enh | [31-8] | [Dynamic indication of Rel-17 or FDM soft resource availability] | Support monitoring DCI Format 2_5 scrambled by AI-RNTI for indication of FDM soft resource availability to an IAB node |  | Yes | N/A | The IAB-node is unable to receive explicit availability indication for Rel-17 or FDM soft resources | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | Agree in principle. The description of Consequences and Components should be aligned with the FG name where it is stated “Rel-17 or FDM soft resource” which is also our preference, since the design details are not yet agreed. |
| Nokia, NSB | Given design details are still being worked on, it might make more sense to revisit the definitions later. |
| ZTE, Sanechips | Share similar view with Nokia, this could be revisited later. |
| vivo | We prefer to use agreed wording for the feature description, i.e., Rel-17 frequency soft resources, instead of ‘Rel-17 or FDM soft resources soft resources’. |
| AT&T | Suggest to keep the brackets for now. This can be revisited after RAN1 has finalized DCI Format 2_5 details. |
| Samsung | OK with AT&T’s suggestion. |
| Huawei, HiSilicon | Agree with ATT’s suggestion. |

# Issue 9: Set of Multiplexing Conditions

Proposal: The IAB-node may report to the parent node a set of multiplexing conditions on the required timing mode, power control, beam indication.

- For simultaneous MT-TX/DU-TX, the multiplexing conditions include whether the IAB-node requires:

  - Case-6 timing

  - Parent node indication of restricted IAB-DU beams

  - IAB-node indication to its parent node on its MT’s minimum UL TX power level

- For simultaneous MT RX/DU RX, the multiplexing conditions include whether the IAB-node:

  - Supports adjustment of child node UL TX timing in Case-7 timing

  - Supports adjustment of UL TX timing for parent node in Case-7 timing

  - Requires IAB-node indication of recommended IAB-MT beams

  - Requires IAB-node indication to support parent node maximum DL power level

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | Agree |
| Nokia, NSB | It is unclear why this is being discussed in this thread. Do not support. |
| ZTE, Sanechips | Share similar view with Nokia, the motivation is not clear to us. |
| vivo | The beam reporting/indication or power reporting/indication have been captured in other FGs. Not sure why to repeat the feature. |
| AT&T | This proposal would effectively lead to grouping of FGs based on multiplexing mode. We are supportive of the proposed grouping, however the details of how to provide this capability indication to the parent node should likely be decided in RAN3. |
| Samsung | Support the main bullet and the details for the sub-bullets may be up to further discussion. |
| Huawei, HiSilicon | We are not sure why this should be discussed in UE feature discussion. |

# Issue 10: Text Proposal for TS 38.473

Revised Proposal:

- Extend the semi-static IAB node multiplexing capability to include FDM-required for multiplexing modes MT RX/DU RX and MT TX/DU TX

- Send LS to RAN3, asking for a change in TS 38.473 according to the proposed modification in R1-2112359

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | Agree to the first bullet. The second bullet should be changed to - Send LS to RAN3, asking for a change in TS 38.473 according to the proposed modification. |
| Nokia, NSB | Do not agree, RAN1 does not have the mandate to decided on text proposals to TS38.473. Any such discussions should take place in the WI AIs, not in UE feature discussions. |
| ZTE, Sanechips | Do not support, it is not an issue related to UE feature, and share similar view with Nokia. |
| vivo | No. Rel-16 multiplexing capability report is sufficient. |
| ETRI | We have the following RAN1 agreements from the last online session: Agreement: Support indication of whether FDM is required or not for an enhanced multiplexing operation mode to donor-CU. Based on the agreements above, it seems that one of the following options should be down-selected: - Option 1: Define a new L1 signaling to indicate FDM-required. - Option 2: Define a new IAB-MT capability signaling to indicate FDM-required. - Option 3: Send LS to RAN2 to define a new signaling to indicate FDM-required. - Option 4: Send LS to RAN3 to define a new signaling to indicate FDM-required. Among the four options, we think option 4 is the way to go at either of WI AI or UE feature discussion (no matter which session covers it). |
| AT&T | As pointed out by ETRI, this has effectively already been agreed in 8.10.1. We believe this can be handled by the higher layer parameter email discussion and does not need further endorsement in this agenda item. |
| Samsung | OK with further discussing in WI AI. |
| Huawei, HiSilicon | This can be discussed in RRC parameter session. One alternative is leave this to implementation without introducing a dedicated parameter. |

# Conclusion

After further discussion on the RAN1 email reflector the following was agreed as part of this email discussion:

# References

- R1-2110587, Updated RAN1 UE features list for Rel-17 NR after RAN1 #106bis-e, Moderators (AT&T, NTT DOCOMO, INC.)

- R1-2110836, Rel-17 UE features for IAB, Huawei/HiSilicon

- R1-2111161, On UE features for IAB enhancements, Nokia/Nokia Shanghai Bell

- R1-2111533, UE features for IAB, Intel Corporation

- R1-2111777, UE features for NR IAB, Samsung

- R1-2112359, UE features for enhanced IAB, Ericsson

- R1-2111057, Discussion on UE features for IAB, vivo

- R1-2111594, Discussion on NR Rel-17 IAB MT Features, ZTE/Sanechips