3GPP TSG RAN WG1 #107-e		                                   					       												                 R1-2111809

e-Meeting, November 11th – 19th, 2021

Agenda Item:	8.16.4

Source:	Moderator (AT&T)

Title:	Summary of UE features for NR NTN

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email discussion/approval [107-e-R17-UE-features-NR-NTN-01] during RAN1 #107-e. According to the Chairman’s Notes:

[107-e-R17-UE-features-NR-NTN-01] Email discussion UE features for NR NTN – Ralf (AT&T) - 1st check point: November 15 - Final check point: November 19

The following was discussed and/or agreed during RAN1 #107-e within the scope of [107-e-R17-UE-features-NR-NTN-01]. All proposals are based on the latest RAN1 UE features list for Rel-17 NR in Error: Reference source not found.

# Summary of Contributions Submitted to RAN1 #107-e

The following is the moderator’s summary of contributions submitted to RAN1 #107-e in this agenda item.

| 26. NR_NTN_solutions | 26-1 | Uplink Time pre-compensation | - UE specific TA calculation in RRC_IDLE and RRC_INACTIVE states based on its GNSS-acquired position and the serving satellite ephemeris. - UE specific TA calculation in RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris. - UE applies common TA according to the parameters provided by the network (if any) - For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops - FFS: UE (re)acquire the serving satellite ephemeris and common TA upon the expiration of the corresponding validity timer |  | No | No | Release 17 UE cannot access [NTN/ satellite/HAPS/ATG] | [Per UE/per band] | No | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] | An NTN UE is required to at least support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris [Note: Serving satellite ephemeris Epoch time is implicitly known as a reference time defined by the starting time of a DL slot and/or frame] FFS: Whether this starting time is given by predefined rule or it is indicated by the Network | Optional with capability signalling For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported. [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei, HiSilicon [2] | - Comment 1: Component 1 and component 2 can be merged together. - Comment 2: On component 5, a NTN UE can either acquire the ephemeris and common TA before or after the validity timer expires. Component 5 is also implicitly covered by component 1 hence it can be removed. - Comment 3: As the typical altitude of HAPS and ATG are below 20 km, which is similar to a large cell from physical layer point of view. There is no need of UE specific TA pre-compensation. - Comment 4: The serving satellite ephemeris epoch time is not a UE feature, it is a part of the ephemeris. Thus, the description can be removed. - Comment 5: This FG is a per UE capability. 26. NR_NTN_solutions; 26-1; Uplink Time pre-compensation; - UE specific TA calculation in RRC_IDLE and RRC_INACTIVE states based on its GNSS-acquired position and the serving satellite ephemeris. - UE specific TA calculation in RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris. - UE applies common TA according to the parameters provided by the network (if any) - For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops - FFS: UE (re)acquire the serving satellite ephemeris and common TA upon the expiration of the corresponding validity timer; No; No; Release 17 UE cannot access [NTN/ satellite/HAPS /ATG]; [Per UE/per band]; No; No; [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2]; An NTN UE is required to at least support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris. [Note: Serving satellite ephemeris Epoch time is implicitly known as a reference time defined by the starting time of a DL slot and/or frame] FFS: Whether this starting time is given by predefined rule or it is indicated by the Network; Mandatory Optional with capability signalling For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported. [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported]. |
| Nokia, Nokia Shanghai Bell [3] | These are basic features for UEs supporting NR over NTN.” Hence, they should be combined into a single FG. |
| OPPO [4] |  |
| Ericsson [5] | We propose to remove the misleading text “support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2” in the column “Capability interpretation for mixture of FDD/TDD and/or FR1/FR2”. - The issue related to “The capability interpretation for mixture of FDD/TDD and/or FR1/FR2” was asked in RAN2 LS [5], to which RAN1 answered in [6]. - Specifically, the issue occurs when there are multiple serving cells. This is however not the case for the NTN UE features defined in the table below. Therefore, the misleading text “support mixture of FDD/TDD (for HAPS) and/or FR1/FR2” in the column “Capability interpretation for mixture of FDD/TDD and/or FR1/FR2” should be removed. |
| Intel Corporation [6] | 26. NR_NTN_solutions; 26-1; Uplink Time pre-compensation and Enhancement on the timing relationship; - UE specific TA calculation in RRC_IDLE and RRC_INACTIVE states based on its GNSS-acquired position and the serving satellite ephemeris. - UE specific TA calculation in RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris. - UE applies common TA according to the parameters provided by the network (if any) - For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops - FFS: UE (re)acquire the serving satellite ephemeris and common TA upon the expiration of the corresponding validity timer - estimate of UE-gNB RTT - delaying the start of ra-ResponseWindow [by UE-gNB RTT] - delaying the start of msgB-ResponseWindow [by UE-gNB RTT] - FFS: delaying the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS with cell-specific K_offset - FFS: delaying the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated - FFS: separate FGs for cell specific Koffset and Kmac/UE-gNB RTT estimation/delay of RAR/MsgB response window; No; No; Release 17 UE cannot access [NTN/ satellite/HAPS/ATG]; [Per UE/per band]; No; No; [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2]; An NTN UE is required to at least support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris [Note: Serving satellite ephemeris Epoch time is implicitly known as a reference time defined by the starting time of a DL slot and/or frame] FFS: Whether this starting time is given by predefined rule or it is indicated by the Network; Optional with capability signalling For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported. [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported]. |
| ZTE [7] | For component 3, UE applies common TA according to the parameters provided by the network (if any) (UE considers common TA as 0 if the parameter is not provided). Because a UE will apply the timing advance TTA according to the formula wherein the parameter of common TA should be either 0 or another value. For component 5, following UE feature can be added to capture the agreement in RAN1#106bis. i.e., A UE starts/restarts the validity timer at the epoch time of the serving satellite ephemeris and common TA. 26. NR_NTN_solutions; 26-1; Uplink Time pre-compensation; - UE specific TA calculation in RRC_IDLE and RRC_INACTIVE states based on its GNSS-acquired position and the serving satellite ephemeris. - UE specific TA calculation in RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris. - UE applies common TA according to the parameters provided by the network (if any) (UE considers common TA as 0 if the parameter is not provided). - For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops - FFS: UE (re)acquire the serving satellite ephemeris and common TA upon the expiration of the corresponding validity timer. A UE starts/restarts the validity timer at the epoch time of the serving satellite ephemeris and common TA.; No; Release 17 UE cannot access [NTN/ satellite/HAPS/ATG]; [Per UE/per band]; No; No; [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2]; An NTN UE is required to at least support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris [Note: Serving satellite ephemeris Epoch time is implicitly known as a reference time defined by the starting time of a DL slot and/or frame] FFS: Whether this starting time is given by predefined rule or it is indicated by the Network; Optional with capability signalling For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported. [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported]. |
| Samsung [8] | The current version has mandatory for FG26-1/26-2/26-3. Since Rel-16, all features are basically optional. Instead, the following changes can be made. Also, FG26-4 “UE reporting of information about the UE specific TA pre-compensation” should be one of the basic FG because the gNB needs to know its value in order to schedule with proper offset. Features; Index; Feature group; Components; Mandatory/Optional. 26. NR_NTN_solutions; 26-1; Uplink Time pre-compensation; - UE specific TA calculation in RRC_IDLE and RRC_INACTIVE states based on its GNSS-acquired position and the serving satellite ephemeris. - UE specific TA calculation in RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris. - UE applies common TA according to the parameters provided by the network (if any) - For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops - FFS: UE (re)acquire the serving satellite ephemeris and common TA upon the expiration of the corresponding validity timer; Mandatory Optional with capability signalling For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported. [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] This is the basic FG for NTN.. |
| Apple [9] | The feature 26-1 was agreed [2] as baseline for NR NTN as uplink time pre-compensation. It was agreed that an NR NTN UE, in RRC_IDLE, RRC_INACTIVE or RRC_CONNECTED state, pre-compensates time advance in any uplink transmissions including PRACH. Similarly, it was agreed that an NR NTN UE, in RRC_IDLE, RRC_INACTIVE or RRC_CONNECTED state, pre-compensates frequency offset to counter service link Doppler shift. The acquisition of service link Doppler shift is based on UE’s acquired GNSS position and satellite ephemeris. In other words, a UE always pre-compensates both time and frequency offset for any of its uplink transmissions, where the amount of time advance and frequency offset to be pre-compensated is based on UE’s acquired GNSS position and satellite ephemeris. Hence, it is preferred to combine feature 26-2 to feature 26-1. The combined feature name could be “uplink pre-compensation”. Proposal: Merge feature 26-2 to feature 26-1 and rename feature 26-1 as “uplink pre-compensation”. |
| Qualcomm Incorporated [10] |  |
| Beijing Xiaomi Mobile Software [11] |  |
| LG Electronics [12] |  |

| 26. NR_NTN_solutions | 26-2 | Uplink Frequency/Doppler pre-compensation | - [In RRC_IDLE and RRC_INACTIVE states calculate frequency pre-compensation to counter shift the Doppler experienced on the service link] - In RRC_CONNECTED states, calculate frequency pre-compensation to counter shift the Doppler experienced on the service link |  | No | No | Release 17 UE cannot access [NTN/ satellite/HAPS/ATG] | [Per UE/per band] | No | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] | An NR NTN UE shall be capable of at least using its acquired GNSS position and satellite ephemeris to calculate frequency pre-compensation to counter shift the Doppler experienced on the service link FFS: whether this FG gets merged with FG 26-1 | Optional with capability signalling For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported. [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei, HiSilicon [2] | - Comment 1: Component 1 and component 2 can be merged together. - Comment 2: Uplink Frequency/Doppler pre-compensation is mainly relavant for NGSO. It is preferred to keep FG26-2 as a separate FG to allow flexibility UE implementation. - Comment 3: This FG is a per UE capability. 26. NR_NTN_solutions; 26-2; Uplink Frequency/Doppler pre-compensation; - [In RRC_IDLE and RRC_INACTIVE states UE calculates frequency pre-compensation to counter shift the Doppler experienced on the service link] - in RRC_CONNECTED states, calculate frequency pre-compensation to counter shift the Doppler experienced on the service link.; No; No; Release 17 UE cannot access [NTN/ satellite/HAPS/ATG]; [Per UE/per band]; No; No; [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2]; An NR NTN UE shall be capable of at least using its acquired GNSS position and satellite ephemeris to calculate frequency pre-compensation to counter shift the Doppler experienced on the service link. FFS: whether this FG gets merged with FG 26-1; Mandatory Optional with capability signalling For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported. [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported]. |
| Nokia, Nokia Shanghai Bell [3] | These are basic features for UEs supporting NR over NTN.” Hence, they should be combined into a single FG. |
| OPPO [4] |  |
| Ericsson [5] | We propose to remove the misleading text “support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2” in the column “Capability interpretation for mixture of FDD/TDD and/or FR1/FR2”. - The issue related to “The capability interpretation for mixture of FDD/TDD and/or FR1/FR2” was asked in RAN2 LS [5], to which RAN1 answered in [6]. - Specifically, the issue occurs when there are multiple serving cells. This is however not the case for the NTN UE features defined in the table below. Therefore, the misleading text “support mixture of FDD/TDD (for HAPS) and/or FR1/FR2” in the column “Capability interpretation for mixture of FDD/TDD and/or FR1/FR2” should be removed. |
| Intel Corporation [6] |  |
| ZTE [7] |  |
| Samsung [8] | The current version has mandatory for FG26-1/26-2/26-3. Since Rel-16, all features are basically optional. Instead, the following changes can be made. Also, FG26-4 “UE reporting of information about the UE specific TA pre-compensation” should be one of the basic FG because the gNB needs to know its value in order to schedule with proper offset. Features; Index; Feature group; Components; Mandatory/Optional. 26. NR_NTN_solutions; 26-2; Uplink Frequency/Doppler pre-compensation; - [In RRC_IDLE and RRC_INACTIVE states calculate frequency pre-compensation to counter shift the Doppler experienced on the service link] - in RRC_CONNECTED states, calculate frequency pre-compensation to counter shift the Doppler experienced on the service link.; Mandatory Optional with capability signalling For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported. [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] This is the basic FG for NTN.. |
| Apple [9] | Remove FG |
| Qualcomm Incorporated [10] |  |
| Beijing Xiaomi Mobile Software [11] |  |
| LG Electronics [12] |  |

| 26. NR_NTN_solutions | 26-3 | Enhancement on the timing relationship | - estimate of UE-gNB RTT - delaying the start of ra-ResponseWindow [by UE-gNB RTT] - delaying the start of msgB-ResponseWindow [by UE-gNB RTT] - FFS: delaying the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS with cell-specific K_offset - FFS: delaying the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated - FFS: separate FGs for cell specific Koffset and Kmac/UE-gNB RTT estimation/delay of RAR/MsgB response window | [26-1][26-2] | No | No | Release 17 UE cannot access [NTN/ satellite/HAPS/ATG] | [Per UE/per band] | No | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] | FFS: whether this FG gets merged with FG 26-1 | Optional with capability signalling For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei, HiSilicon [2] | - Comment 1: Component 1, 2, 3 can be merged together. The enhancement on K_offset and K_mac can be separate. - Comment 2: FG 26-3 can either be a basic feature for NTN or a separate FG in order not to results long list of basic features. - Comment 3: This FG is a per UE capability. 26. NR_NTN_solutions; 26-3; Start of RAR window Enhancement on the timing relationship; - estimate of UE-gNB RTT - delaying the start of ra-ResponseWindow [by UE-gNB RTT] - delaying the start of msgB-ResponseWindow [by UE-gNB RTT] - FFS: delaying the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS with cell-specific K_offset - FFS: delaying the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated - FFS: separate FGs for cell specific Koffset and Kmac/UE-gNB RTT estimation/delay of RAR/MsgB response window - UE estimates UE-gNB RTT and delays the delays the start of RAR window by UE-gNB RTT - UE delays the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS with cell-specific K_offset - UE delays the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac; [26-1][26-2]; No; No; Due to large propagation delay in NTN the UE RAR is not expected within a few milliseconds like in TN. The UE does not need to monitor the PDCCH for RAN right after transmitting RA Preamble. Release 17 UE cannot access [NTN/ satellite/HAPS/ATG]; [Per UE/per band]; No; No; [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2]; FFS: whether this FG gets merged with FG 26-1; Mandatory Optional with capability signalling For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported. [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported]. |
| Nokia, Nokia Shanghai Bell [3] | - These are basic features for UEs supporting NR over NTN.” Hence, they should be combined into a single FG. - On ”Consequence if…” column, remove HAPS/ATG from the list, as not all HAPS/ATG deployments are impacted. |
| OPPO [4] |  |
| Ericsson [5] | We propose to remove the misleading text “support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2” in the column “Capability interpretation for mixture of FDD/TDD and/or FR1/FR2”. - The issue related to “The capability interpretation for mixture of FDD/TDD and/or FR1/FR2” was asked in RAN2 LS [5], to which RAN1 answered in [6]. - Specifically, the issue occurs when there are multiple serving cells. This is however not the case for the NTN UE features defined in the table below. Therefore, the misleading text “support mixture of FDD/TDD (for HAPS) and/or FR1/FR2” in the column “Capability interpretation for mixture of FDD/TDD and/or FR1/FR2” should be removed. |
| Intel Corporation [6] | Remove FG/row |
| ZTE [7] | The two time relationships were missing such as activation of TA command, first PUSCH transmission in CG Type 2. Thus, they are added in each block. 26. NR_NTN_solutions; 26-3; Enhancement on the timing relationship; - estimate of UE-gNB RTT - delaying the start of ra-ResponseWindow [by UE-gNB RTT] - delaying the start of msgB-ResponseWindow [by UE-gNB RTT] - FFS: delaying the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS, activation of TA command, first PUSCH transmission in CG Type 2 with cell-specific K_offset - FFS: delaying the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated - FFS: separate FGs for cell specific Koffset and Kmac/UE-gNB RTT estimation/delay of RAR/MsgB response window; [26-1][26-2]; No; Release 17 UE cannot access [NTN/ satellite/HAPS/ATG]; [Per UE/per band]; No; No; [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2]; FFS: whether this FG gets merged with FG 26-1; Optional with capability signalling For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported]. |
| Samsung [8] | The current version has mandatory for FG26-1/26-2/26-3. Since Rel-16, all features are basically optional. Instead, the following changes can be made. Also, FG26-4 “UE reporting of information about the UE specific TA pre-compensation” should be one of the basic FG because the gNB needs to know its value in order to schedule with proper offset. Features; Index; Feature group; Components; Mandatory/Optional. 26. NR_NTN_solutions; 26-3; Enhancement on the timing relationship; - estimate of UE-gNB RTT - delaying the start of ra-ResponseWindow [by UE-gNB RTT] - delaying the start of msgB-ResponseWindow [by UE-gNB RTT]; Mandatory Optional with capability signalling For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported. [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] This is the basic FG for NTN.. |
| Apple [9] | A potential feature 26-3 is named as “enhancement on the timing relationship”. The components of this feature include estimate of UE-gNB RTT, delaying the start of ra-ResponseWindow and msgB-ResponseWindow. It is open whether this feature is merged with feature 26-1. In our view, this feature should not be merged with feature 26-1. Feature 26-1 is related to uplink transmissions pre-compensation, while feature 26-3 is related to the downlink reception of the RA response or msgB response. The estimate of UE-gNB RTT is equal to the sum of UE’s full TA and . The acquisition of UE’s full TA is achieved by feature 26-1. The is indicated by network as system information. The estimate of UE-gNB RTT is used to determine the offset of ra-ResponseWindow” or “msgB-ResponseWindow”. The same is also used in determining the activation time for downlink configuration MAC CE. Hence, the component of “delaying the UE action and assumption on downlink configuration indicated by a MAC-CE common in PDSCH” should be also included in the feature. Proposal: Introduce feature 26-3 of “enhancement on the timing relationship with ”, which includes the components 1). reception of via system information 2). estimate of UE-gNB RTT and delaying the start of ra-ResponseWindow and/or msgB-ResponseWindow by estimated UE-gNB RTT 3). delaying the UE action and assumption on a downlink configuration carried by MAC CE command by if it is indicated. |
| Qualcomm Incorporated [10] |  |
| Beijing Xiaomi Mobile Software [11] |  |
| LG Electronics [12] | Component 4 and 5 can be separated into two new FGs. |

| 26. NR_NTN_solutions | 26-4 | UE reporting of information about the UE specific TA pre-compensation | - [The exact content of UE reporting of information about the UE specific TA pre-compensation] - FFS: frequency of the reports - FFS: granularity of the reported content | [26-1][26-2] | Yes | No |  | [Per UE/Per band] | No | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] | FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive | Optionalwith capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei, HiSilicon [2] | - Comment 1: Two kinds UE-specific TA reporting are under discussion, i.e. UE specific TA pre-compensation or UE location information. Depending on further input from SA3, component 1 may be updated later. - Comment 2: Component 2 and 3 can be removed since there is no need to list the detailed signalling design. - Comment 3: This is not a basic feature for NTN and there is need to merge it with FG26-1. Therefore, the notes in the last column can be removed. - Comment 4: This FG is a per UE capability. 26. NR_NTN_solutions; 26-4; UE reporting of information about the UE specific TA pre-compensation; - UE reports UE specific TA pre-compensation or UE location information - [The exact content of UE reporting of information about the UE specific TA pre-compensation] - FFS: frequency of the reports - FFS: granularity of the reported content; [26-1][26-2]; Yes; No; [Per UE/per band]; No; No; [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2]; FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive; [Optional] with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported]. |
| Nokia, Nokia Shanghai Bell [3] | - Component 1: It is unclear what is signalled here. We propose to change the text to “Support UE reporting of information about the UE specific TA pre-compensation” - Component 2: remove component 2 or change the text to make it clear all report frequencies are supported. - Component 3: remove component 3 or change the text to make it clear all report granularities are supported |
| OPPO [4] |  |
| Ericsson [5] | We propose to remove the misleading text “support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2” in the column “Capability interpretation for mixture of FDD/TDD and/or FR1/FR2”. - The issue related to “The capability interpretation for mixture of FDD/TDD and/or FR1/FR2” was asked in RAN2 LS [5], to which RAN1 answered in [6]. - Specifically, the issue occurs when there are multiple serving cells. This is however not the case for the NTN UE features defined in the table below. Therefore, the misleading text “support mixture of FDD/TDD (for HAPS) and/or FR1/FR2” in the column “Capability interpretation for mixture of FDD/TDD and/or FR1/FR2” should be removed. |
| Intel Corporation [6] | 26. NR_NTN_solutions; 26-4; UE reporting of information about the UE specific TA pre-compensation; - [The exact content of UE reporting of information about the UE specific TA pre-compensation] - FFS: frequency of the reports - FFS: granularity of the reported content; [26-1][26-2]; Yes; No; [Per UE/Per band]; No; No; [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2]; FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive; Optionalwith capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported]. |
| ZTE [7] | For FG 26-4, the component 3 can be updated with the latest agreement, i.e. The granularity of the reported TA is slot. FFS how to round TA value to slot level granularity. 26. NR_NTN_solutions; 26-4; UE reporting of information about the UE specific TA pre-compensation; - [The exact content of UE reporting of information about the UE specific TA pre-compensation] - FFS: frequency of the reports - FFS: granularity of the reported contentThe granularity of the reported TA is slot. FFS how to round TA value to slot level granularity; [26-1][26-2]; Yes; [Per UE/Per band]; No; No; [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2]; FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive; Optionalwith capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported]. |
| Samsung [8] | Features; Index; Feature group; Components; Mandatory/Optional. 26. NR_NTN_solutions; 26-4; UE reporting of information about the UE specific TA pre-compensation; - [The exact content of UE reporting of information about the UE specific TA pre-compensation] - FFS: frequency of the reports - FFS: granularity of the reported content; [Optional] with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] This is the basic FG for NTN.. |
| Apple [9] |  |
| Qualcomm Incorporated [10] |  |
| Beijing Xiaomi Mobile Software [11] | It is agreed in last meeting that the granularity of the reported TA is slot, so it is suggested to remove the FFS parts: Agreement: The granularity of the reported TA is slot. - FFS how to round TA value to slot level granularity Proposal: Remove the FFS on the granularity of TA reporting in FG26-4. 26. NR_NTN_solutions; 26-4; UE reporting of information about the UE specific TA pre-compensation; - [The exact content of UE reporting of information about the UE specific TA pre-compensation] - FFS: frequency of the reports; [26-1][26-2]; Yes; No; [Per UE/Per band]; No; No; [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2]; FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive; Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported]. |
| LG Electronics [12] | Component 2 and 3 can be removed or move to the note column. 26. NR_NTN_solutions; 26-4; UE reporting of information about the UE specific TA pre-compensation; - [The exact content of UE reporting of information about the UE specific TA pre-compensation] - FFS: frequency of the reports - FFS: granularity of the reported content; [26-1][26-2]; Yes; No; [Per UE/Per band]; No; No; [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2]; FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive; Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported]. |

| 26. NR_NTN_solutions | 26-5 | Increasing the number of HARQ processes | - The maximal supported HARQ process number is 32 for both UL and DL - FFS: Support on the maximal HARQ process number is up to UE capability - FFS: separate features for DL and UL |  | Yes | No | [Increasing the number of HARQ processes avoids HARQ stalling] | FFS | No | No | [support mixture of FDD/TDD (for HAPS and/or STG) and/or FR1/FR] | FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive | Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei, HiSilicon [2] | - Comment 1: FG 26-5 can be split into DL and UL for UE implementation flexibility. - Comment 2: This feature is also supported for 52.6GHz, a common FG can defined as per band capability. - Comment 3: This is not a basic feature for NTN and there is need to merge it with FG26-1. The notes in the last column can be removed. 26. NR_NTN_solutions; 26-5; Increasing the number of HARQ processes for DL; - The maximal supported HARQ process number is 32 for both UL and DL - FFS: Support on the maximal HARQ process number is up to UE capability - FFS: separate features for DL and UL; Yes; No; [Increasing the number of HARQ processes avoids HARQ stalling]; FFS Per band; No; No; [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2]; FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive; Optional with capability signalling [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported]. 26. NR_NTN_solutions; 26-5a; Increasing the number of HARQ processes for UL; - The maximal supported HARQ process number is 32 for both UL; Yes; No; [Increasing the number of HARQ processes avoids HARQ stalling]; FFS Per band; No; No; [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2]; FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive; Optional with capability signalling [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported]. |
| Nokia, Nokia Shanghai Bell [3] |  |
| OPPO [4] |  |
| Ericsson [5] | We propose to remove the misleading text “support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2” in the column “Capability interpretation for mixture of FDD/TDD and/or FR1/FR2”. - The issue related to “The capability interpretation for mixture of FDD/TDD and/or FR1/FR2” was asked in RAN2 LS [5], to which RAN1 answered in [6]. - Specifically, the issue occurs when there are multiple serving cells. This is however not the case for the NTN UE features defined in the table below. Therefore, the misleading text “support mixture of FDD/TDD (for HAPS) and/or FR1/FR2” in the column “Capability interpretation for mixture of FDD/TDD and/or FR1/FR2” should be removed. |
| Intel Corporation [6] | 26. NR_NTN_solutions; 26-5; Increasing the number of HARQ processes; - The maximal supported HARQ process number is 32 for both UL and DL - FFS: Support on the maximal HARQ process number is up to UE capability - FFS: separate features for DL and UL; Yes; No; [Increasing the number of HARQ processes avoids HARQ stalling]; FFS; No; No; [support mixture of FDD/TDD (for HAPS and/or STG) and/or FR1/FR]; FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive; Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported]. |
| ZTE [7] |  |
| Samsung [8] | It is proposed that the type of FG 26-5 should be per FSPC. Features; Index; Feature group; Components; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC). 26. NR_NTN_solutions; 26-5; Increasing the number of HARQ processes; - The maximal supported HARQ process number is 32 for both UL and DL - FFS: Support on the maximal HARQ process number is up to UE capability - FFS: separate features for DL and UL; Per FSPC. |
| Apple [9] | Feature 26-5 mentions that the maximal supported HARQ process number is 32 for both UL and DL. Although it was agreed in NTN, the maximal supported HARQ process number is up to 32, it is possible that the maximal supported HARQ process number of a UE can be between 16 and 32, based its capability. Furthermore, the maximal supported HARQ process number can be different for UL and for DL. Hence, it is preferred to make this feature more flexible. We could modify the component to “the maximal supported HARQ process number is [X] for UL and [Y] for DL.” Proposal: In feature 26-5, the component is modified to “the maximal supported HARQ process number is [X] for UL and is [Y] for DL”. |
| Qualcomm Incorporated [10] |  |
| Beijing Xiaomi Mobile Software [11] |  |
| LG Electronics [12] |  |

| 26. NR_NTN_solutions | 26-6 | Type-2 HARQ codebook Enhancement | - Enhancement on Type-2 HARQ codebook in NTN - FFS: HARQ disabling | [26-1, 26-2] | Yes | No |  | [Per UE/per band] | No | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] | FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive | Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei, HiSilicon [2] | - Comment 1: Add a component “UE supports HARQ disabling” since HARQ disabling is a pre-requisit for HARQ-ACK codebook enhancement. - Comment 2: FG is not basic features for NTN and there is need to merge them with FG 26-1. The notes in the last column can be removed. - Comment 3: FG is per UE capability. 26. NR_NTN_solutions; 26-6; Type-2 HARQ CODEBOOK Enhancement; - Enhancement on Type-2 HARQ codebook in NTN - FFS: UE supports HARQ disabling - Enhancement on Type-1 HARQ codebook in NTN; [26-1, 26-2]; Yes; No; [Per UE/per band]; No; No; [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2]; FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive; Mandatory Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported.] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported]. |
| Nokia, Nokia Shanghai Bell [3] | - These are basic features for UEs supporting NR over NTN.” Hence, they should be combined into a single FG. - It is not clear what enhancement the FG refers to. We propose to change the text to: ” Support type-1 and type-2 HARQ codebook enhancements for feedback-disabled HARQ processes”, and merge it with FG 26-1. |
| OPPO [4] | For 26-6, we think HARQ disabling should be an independent FG which is not always under the type-2 HARQ CB enhancement. A UE can claim supporting HARQ disabling while not supporting type-2 enhanced codebook. Thus, the network can still configure Type-1 CB to the UE. |
| Ericsson [5] | We propose to remove the misleading text “support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2” in the column “Capability interpretation for mixture of FDD/TDD and/or FR1/FR2”. - The issue related to “The capability interpretation for mixture of FDD/TDD and/or FR1/FR2” was asked in RAN2 LS [5], to which RAN1 answered in [6]. - Specifically, the issue occurs when there are multiple serving cells. This is however not the case for the NTN UE features defined in the table below. Therefore, the misleading text “support mixture of FDD/TDD (for HAPS) and/or FR1/FR2” in the column “Capability interpretation for mixture of FDD/TDD and/or FR1/FR2” should be removed. 26. NR_NTN_solutions; 26-6; Type-2 HARQ codebook Enhancement; - Enhancement on Type-2 HARQ codebook in NTN - FFS: HARQ disabling; Yes; No; [Per UE/per band]; No; No; FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive; Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported]. |
| Intel Corporation [6] |  |
| ZTE [7] |  |
| Samsung [8] |  |
| Apple [9] | Feature 26-6 is still open. We think this feature is related to feedback disabled HARQ processes. Specifically, it was agreed to reduce codebook size with HARQ-ACK codebook only including HARQ-ACK of PDSCH with feedback enabled HARQ processes for Type-2 HARQ codebook in NTN. Also, it was agreed that a UE is expected to receive a PDCCH carrying the DCI for a HARQ process with disabled HARQ feedback, after T_(proc,1) after the reception of the last PDSCH or slot-aggregated PDSCH for the given HARQ process. Overall, we do not think this feature is related to uplink transmission pre-compensation, and hence, it is a separate UE feature. Proposal: Introduce feature 26-6 “Type-2 HARQ codebook enhancement”, with components: - reducing codebook size with HARQ-ACK codebook only including HARQ-ACK of PDSCH with feedback enabled HARQ processes. - receiving PDCCH carrying the DCI for a HARQ process with disabled HARQ feedback, after T_(proc,1) after the reception of the last PDSCH or slot-aggregated PDSCH for the given HARQ process. |
| Qualcomm Incorporated [10] |  |
| Beijing Xiaomi Mobile Software [11] |  |
| LG Electronics [12] | We think HARQ disabling feature should be separated with HARQ-ACK codebook. So, we suggest to remove second component and add new FG for HARQ disabling 26. NR_NTN_solutions; 26-6; Type-2 HARQ codebook Enhancement; - Enhancement on Type-2 HARQ codebook in NTN - FFS: HARQ disabling; [26-1, 26-2]; Yes; No; [Per UE/per band]; No; No; [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2]; FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive; Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported]. 26. NR_NTN_solutions; 26-X; Disabling of HARQ feedback; - Disabling on HARQ feedback for downlink transmission should be at least configurable per HARQ process via UE specific RRC signaling; Yes; No; [Per UE/per band]; No; No; [support mixture of FDD/TDD (for HAPS) and/or FR1/FR2]; Optional with capability signalling. |

| 26. NR_NTN_solutions | [26-6a] | [Type-1 HARQ codebook enhancement] | - Enhancement on Type-1 HARQ codebook in NTN - FFS: HARQ disabling | [26-1, 26-2] | Yes | No |  | [Per UE/per band] | No | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] | FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive | Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei, HiSilicon [2] | - Comment 1: Add a component “UE supports HARQ disabling” since HARQ disabling is a pre-requisit for HARQ-ACK codebook enhancement. - Comment 2: FG is not basic features for NTN and there is need to merge them with FG 26-1. The notes in the last column can be removed. - Comment 3: FG is per UE capability. 26. NR_NTN_solutions; 26-6a; Type-1 HARQ codebook enhancement; - Enhancement on Type-1 HARQ codebook in NTN - FFS: UE supports HARQ disabling; [26-1, 26-2]; Yes; No; [Per UE/per band]; No; No; [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2]; FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive; Mandatory Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported.] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported]. |
| Nokia, Nokia Shanghai Bell [3] | These are basic features for UEs supporting NR over NTN.” Hence, they should be combined into a single FG. |
| OPPO [4] | For 26-6a: this FG should be removed for the moment as there is no agreement on the type 1 CB enhancement. |
| Ericsson [5] | We propose to remove the misleading text “support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2” in the column “Capability interpretation for mixture of FDD/TDD and/or FR1/FR2”. - The issue related to “The capability interpretation for mixture of FDD/TDD and/or FR1/FR2” was asked in RAN2 LS [5], to which RAN1 answered in [6]. - Specifically, the issue occurs when there are multiple serving cells. This is however not the case for the NTN UE features defined in the table below. Therefore, the misleading text “support mixture of FDD/TDD (for HAPS) and/or FR1/FR2” in the column “Capability interpretation for mixture of FDD/TDD and/or FR1/FR2” should be removed. 26. NR_NTN_solutions; [26-6a]; [Type-1 HARQ codebook enhancement]; - Enhancement on Type-1 HARQ codebook in NTN - FFS: HARQ disabling; Yes; No; [Per UE/per band]; No; No; FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive; Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported]. |
| Intel Corporation [6] |  |
| ZTE [7] |  |
| Samsung [8] |  |
| Apple [9] | Feature 26-6a is still open. We think this feature is related to feedback disabled HARQ processes. So far, we have two potential enhancements for Type-1 HARQ codebook. The first potential enhancement is HARQ codebook construction when DCIs carrying feedback disabled and feedback enabled HARQ processes are detected by UE. There are two options to be down-selected: Option 1 is that UE reports NACK only for feedback disabled HARQ process regardless of decoding results of corresponding PDSCH. Option 2 is that UE reports ACK/NACK for feedback disabled HARQ process depending on decoding results of corresponding PDSCH. Note that Option 2 is legacy approach, and it does not enhance Type-1 HARQ codebook. The second potential enhancement is HARQ codebook transmission when only DCIs carrying feedback disabled HARQ process is detected by UE. There are two options to be down-selected: Option 1 is that UE reports HARQ-ACK and Option 2 is that UE skips HARQ-ACK reporting when the feedback is carried by PUCCH. Note that Option 1 is legacy approach, and it does not enhance Type-1 HARQ codebook. It is possible that Type-1 HARQ codebook is not enhanced, depending on the latest RAN1 agreements. Hence, we propose to hold the discussion on the component of “enhancement on Type-1 HARQ codebook in NTN” in feature 26-6a, until RAN1 makes new agreements on Type-1 HARQ codebook. Proposal: Hold the discussion on the component of “enhancement on Type-1 HARQ codebook in NTN” in feature 26-6a until RAN1 makes new agreements on Type-1 HARQ codebook. |
| Qualcomm Incorporated [10] |  |
| Beijing Xiaomi Mobile Software [11] |  |
| LG Electronics [12] | We think HARQ disabling feature should be separated with HARQ-ACK codebook. So, we suggest to remove second component and add new FG for HARQ disabling 26. NR_NTN_solutions; [26-6a]; [Type-1 HARQ codebook enhancement]; - Enhancement on Type-1 HARQ codebook in NTN - FFS: HARQ disabling; [26-1, 26-2]; Yes; No; [Per UE/per band]; No; No; [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2]; FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive; Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported]. 26. NR_NTN_solutions; 26-X; Disabling of HARQ feedback; - Disabling on HARQ feedback for downlink transmission should be at least configurable per HARQ process via UE specific RRC signaling; Yes; No; [Per UE/per band]; No; No; [support mixture of FDD/TDD (for HAPS) and/or FR1/FR2]; Optional with capability signalling. |

| 26. NR_NTN_solutions | [26-7] | [NTN Performance enhancement] | - The maximum number of supported aggregation factor (i.e., pdsch-AggregationFactor) for DL PDSCH is [X] FFS: X = 16 or 32 |  | Yes | No |  | [Per UE/per band/Per BC] | [No] | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] |  | Optional with capability signalling [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei, HiSilicon [2] |  |
| Nokia, Nokia Shanghai Bell [3] | - These are basic features for UEs supporting NR over NTN.” Hence, they should be combined into a single FG. - To be merged with FG 26-1. |
| OPPO [4] |  |
| Ericsson [5] | We propose to remove the misleading text “support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2” in the column “Capability interpretation for mixture of FDD/TDD and/or FR1/FR2”. - The issue related to “The capability interpretation for mixture of FDD/TDD and/or FR1/FR2” was asked in RAN2 LS [5], to which RAN1 answered in [6]. - Specifically, the issue occurs when there are multiple serving cells. This is however not the case for the NTN UE features defined in the table below. Therefore, the misleading text “support mixture of FDD/TDD (for HAPS) and/or FR1/FR2” in the column “Capability interpretation for mixture of FDD/TDD and/or FR1/FR2” should be removed. 26. NR_NTN_solutions; [26-7]; [NTN Performance enhancement]; - The maximum number of supported aggregation factor (i.e., pdsch-AggregationFactor) for DL PDSCH is [X] FFS: X = 16 or 32; Yes; No; [Per UE/per band]; [No]; No; Optional with capability signalling [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported]. |
| Intel Corporation [6] |  |
| ZTE [7] |  |
| Samsung [8] | In order to clarify, the following changes are proposed. Also, it does not need value 8 since it is already supported. Features; Index; Feature group; Components. 26. NR_NTN_solutions; [26-7]; [NTN Performance enhancement] Increased PDSCH aggregation factor; 1. The maximum number of supported aggregation factor (i.e., pdsch-AggregationFactor) for DL PDSCH is [X] FFS: X = 16 or 32. |
| Apple [9] | Feature 26-7 is proposing a UE capability of receiving PDSCH with aggregation factor larger than or equal to 8. However, the feature name of “NTN performance enhancement” is too broad. Hence, we prefer to modify the feature name to reflect the exact capability, e.g., “increasing the aggregation factor of PDSCH transmission”. It is agreed that the maximum number of supported aggregated factor for PDSCH is [X], where the value of X is open between 8, 16, 32. If it is finally agreed that X is 8, then we do not need this feature at all. Hence, we propose to hold the discussion of feature 26-7, until RAN1 makes new agreements on the aggregation factor of PDSCH transmission. Proposal: Hold the discussion on feature 26-7 until RAN1 makes new agreements on the aggregation factor of PDSCH transmission. |
| Qualcomm Incorporated [10] |  |
| Beijing Xiaomi Mobile Software [11] |  |
| LG Electronics [12] |  |

| 26. NR_NTN_solutions | 26-8 | Support of polarization [signalling/ information/reception] in NR NTN | - Support polarization indication reception in SIB indicating DL and/or UL polarization information using respective polarization type parameters to indicate: RHCP or LHCP or linear - FFS: polarization information for DL is indicated in SIB by the network |  | No | No | [It is assumed by the network that UE supports at least linear polarization] | [Per UE/Per band] | No | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] | [Value range for component 1: {(RHCP, LHCP, Linear), (RHCP, LHCP), (RHCP), (LHCP), (Linear)}] FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive | Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei, HiSilicon [2] | - Comment 1: Component 1 can be simplified and Component 2 has already been covered by the component 1 hence can be removed. - Comment 2: This is not a basic feature for NTN and there is need to merge it with FG26-1. The notes in the last column can be removed. - Comment 3: This FG is a per UE capability. 26. NR_NTN_solutions; 26-8; Support of polarization [signalling/ information/reception] in NR NTN; - UE polarization capability (RHCP, LHCP, Linear) - Polarization information for UL may be indicated in SIB by the network - UE assumes a same polarization for UL and DL, when the UL polarization information is absent. - Support polarization indication reception in SIB indicating DL and/or UL polarization information using respective polarization type parameters to indicate: RHCP or LHCP or linear - FFS: polarization information for DL is indicated in SIB by the network; No; No; [It is assumed by the network that UE supports at least linear polarization]; [Per UE/Per band]; No; No; [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2]; [Value range for component 1: {(RHCP, LHCP, Linear), (RHCP, LHCP), (RHCP), (LHCP), (Linear)}] FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive; Mandatory Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported.] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported]. |
| Nokia, Nokia Shanghai Bell [3] | - These are basic features for UEs supporting NR over NTN.” Hence, they should be combined into a single FG. - FG name is misleading, as the FG is not about the signalling, but about the UE being able to apply the information it receives in the signalling. It should be merged with 26-1 in any case. |
| OPPO [4] |  |
| Ericsson [5] | We propose to remove the misleading text “support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2” in the column “Capability interpretation for mixture of FDD/TDD and/or FR1/FR2”. - The issue related to “The capability interpretation for mixture of FDD/TDD and/or FR1/FR2” was asked in RAN2 LS [5], to which RAN1 answered in [6]. - Specifically, the issue occurs when there are multiple serving cells. This is however not the case for the NTN UE features defined in the table below. Therefore, the misleading text “support mixture of FDD/TDD (for HAPS) and/or FR1/FR2” in the column “Capability interpretation for mixture of FDD/TDD and/or FR1/FR2” should be removed. |
| Intel Corporation [6] | 26. NR_NTN_solutions; 26-8; Support of polarization [signalling/ information/reception] in NR NTN; - Support polarization indication reception in SIB indicating DL and/or UL polarization information using respective polarization type parameters to indicate: RHCP or LHCP or linear - FFS: polarization information for DL is indicated in SIB by the network; No; No; [It is assumed by the network that UE supports at least linear polarization]; [Per UE/Per band]; No; No; [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2]; [Value range for component 1: {(RHCP, LHCP, Linear), (RHCP, LHCP), (RHCP), (LHCP), (Linear)}] FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive; Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported]. |
| ZTE [7] |  |
| Samsung [8] |  |
| Apple [9] |  |
| Qualcomm Incorporated [10] |  |
| Beijing Xiaomi Mobile Software [11] | It is agreed in last meeting that the UE-specific signalling rather than SIB can support the polarization signalling, it is suggested capture this in the FG 26-8: Agreement: Support polarization signalling for target serving cell in handover command message. Agreement: Support polarization signalling for non-serving cell in RRM measurement configuration. Proposal: Capture the signalling support for polarization indication in FG 26-8. 26. NR_NTN_solutions; 26-8; Support of polarization [signalling/ information/reception] in NR NTN; - Support polarization indication reception in SIB indicating DL and/or UL polarization information using respective polarization type parameters to indicate: RHCP or LHCP or linear - FFS: polarization information for DL is indicated in SIB by the network - Support polarization signalling for target serving cell in handover command message - Support polarization signalling for non-serving cell in RRM measurement configuration; No; No; [It is assumed by the network that UE supports at least linear polarization]; [Per UE/Per band]; No; No; [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2]; [Value range for component 1: {(RHCP, LHCP, Linear), (RHCP, LHCP), (RHCP), (LHCP), (Linear)}] FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive; Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported]. |
| LG Electronics [12] | Component 2 can be removed. 26. NR_NTN_solutions; 26-8; Support of polarization [signalling/ information/reception] in NR NTN; - Support polarization indication reception in SIB indicating DL and/or UL polarization information using respective polarization type parameters to indicate: RHCP or LHCP or linear - FFS: polarization information for DL is indicated in SIB by the network; No; No; [It is assumed by the network that UE supports at least linear polarization]; [Per UE/Per band]; No; No; [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2]; [Value range for component 1: {(RHCP, LHCP, Linear), (RHCP, LHCP), (RHCP), (LHCP), (Linear)}] FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive; Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported]. |

| 26. NR_NTN_solutions | 26-9 | UE-specific K_offset | - Reception of UE-specific K_offset via MAC-CE - delaying the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS with UE-specific Koffset | 26-3, 26-4 | Yes | No |  | FFS | No | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] | FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive | Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei, HiSilicon [2] |  |
| Nokia, Nokia Shanghai Bell [3] | These are basic features for UEs supporting NR over NTN.” Hence, they should be combined into a single FG. |
| OPPO [4] |  |
| Ericsson [5] | We propose to remove the misleading text “support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2” in the column “Capability interpretation for mixture of FDD/TDD and/or FR1/FR2”. - The issue related to “The capability interpretation for mixture of FDD/TDD and/or FR1/FR2” was asked in RAN2 LS [5], to which RAN1 answered in [6]. - Specifically, the issue occurs when there are multiple serving cells. This is however not the case for the NTN UE features defined in the table below. Therefore, the misleading text “support mixture of FDD/TDD (for HAPS) and/or FR1/FR2” in the column “Capability interpretation for mixture of FDD/TDD and/or FR1/FR2” should be removed. |
| Intel Corporation [6] |  |
| ZTE [7] | The two time relationships were missing such as activation of TA command, first PUSCH transmission in CG Type 2. Thus, they are added in each block. In addition, for component 1 in FG 26-9, it’s preferred to add a bracket on to [UE-specific K_offset], considering the solution of updating K_offset is pending, e.g., UE may receive adjustment of K_offset if MAC CE indicates the differential value. 26. NR_NTN_solutions; 26-9; UE-specific K_offset; - Reception of [UE-specific K_offset] via MAC-CE - delaying the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS, activation of TA command, first PUSCH transmission in CG Type 2 with UE-specific Koffset.; 26-3, 26-4; Yes; FFS; No; No; [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2]; FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive; Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported]. |
| Samsung [8] |  |
| Apple [9] |  |
| Qualcomm Incorporated [10] |  |
| Beijing Xiaomi Mobile Software [11] |  |
| LG Electronics [12] | For PDCCH ordered PRACH transmission, it is still FFS on which value (cell specific or UE specific) of K_offset is applied as agreed in RAN1#106-e meeting. So, it would be better to remove PDCCH ordered PRACH in the second component in FG 26-9. Agreement: For random access procedure initiated by a PDCCH order received in downlink slot , UE determines the next available PRACH occasion after uplink slot to transmit the ordered PRACH. - Note: The UE’s TA is based on the RAN1#104bis-e agreement on Timing Advance applied by an NR NTN UE given by , where is assumed for PDCCH ordered PRACH. - FFS: Which value of should be applied - FFS: Whether the timing relationship is impacted by UE behavior within or after the validity duration. 26. NR_NTN_solutions; 26-9; UE-specific K_offset; - Reception of UE-specific K_offset via MAC-CE - delaying the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, and transmission of aperiodic SRS with UE-specific Koffset; 26-3, 26-4; Yes; No; FFS; No; No; [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2]; FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive; Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported]. |

Other issues

| Company | Summary |
| --- | --- |
| Huawei, HiSilicon [2] |  |
| Nokia, Nokia Shanghai Bell [3] |  |
| OPPO [4] |  |
| Ericsson [5] |  |
| Intel Corporation [6] |  |
| ZTE [7] |  |
| Samsung [8] |  |
| Apple [9] | According to NR NTN timing relationship enhancement, UE will receive cell specific and apply it in various timing relationship enhancements, e.g., the transmission timing of DCI scheduled PUSCH (including CSI on PUSCH), the transmission timing of RAR grant scheduled PUSCH, the first transmission opportunity of PUSCH in configured grant type 2, the transmission timing of HARQ-ACK on PUCCH (including to MsgB), the CSI reference resource timing, the transmission timing of aperiodic SRS, MAC CE activation time for TA command, PDCCH ordered PRACH, etc. Hence, a new feature of enhancing on the timing relationship with cell specific is needed. This feature has the components of receiving of cell specific and applying this cell specific in various timing relationships including the scheduling of PUSCH, PUCCH, PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS, and MAC CE activation time for TA command. This feature is related to scheduling slot adjustment, which is independent of uplink transmission pre-compensation. Hence, this feature is preferred not to be merged with feature 26-1. Proposal: Introduce a new feature of “enhancing on the timing relationship with cell specific ”, which includes the components: - reception of cell specific via system information - delaying the scheduling of PUSCH, PUCCH, PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS, MAC CE activation time for TA command. |
| Qualcomm Incorporated [10] | Based on the latest summary, additional comments on UE features for NR-NTN are provided below. - Since NTN is designed with the assumption that legacy NR features are supported in NTN whenever needed, NR container should be used for NTN features. As such, NTN features should by default be per band or per band combination. - Since a UE might be able to support 32 HARQ processes for the DL but not for the UL, and vice versa, UE feature 26-5, “increasing the number of HARQ processes” should be separated into two, one for DL and the other for UL. - The “capability interpretation for mixture of FDD/TDD and/or FR1/FR2” needs to be discussed on a per feature basis. Support of TDD has yet to be addressed in general. Some of the features may not be needed for ATG and HAPS but be mandatory for LEO and GEO. We suggest leaving the discussion for ATG and HAPS to future. On the other hand, some of the NTN features could be useful for ATG/HAPS even in terrestrial bands. This is another reason to use NR container for NTN features. How to indicate the support of legacy UE features in NTN is an important question concerning NTN UE features and capability signalling. NR NTN is developed with the assumption that any legacy NR feature, if needed, is supported. However, not all legacy UE features are applicable. Two options are: - If a separate NTN container is used, legacy features applicable to NTN needs to be identified. This approach however does not promote the sharing of UE features between TN and NTN networks. - If NR container is used, all NTN features must be per band or per band combination by default. Some legacy per-UE features may require different capability in NTN or may not be applicable to NTN. In such case, we may create a separate column in UE feature table such as /TN/NTN differentiation; or declare some of the features not applicable to NTN bands. To answer the above questions, we have the following proposal: Proposal: - NTN uses the NR capability container. - Legacy NR features are applicable to NTN by default and exceptions are handled only when requested. - NTN UE features should be at least per band differentiated so that NTN and non-NTN capabilities can be independently set. |
| Beijing Xiaomi Mobile Software [11] | In RAN1 104e meeting, an agreement on the K1 extension was made as follows: Agreement: For unpaired spectrum, extend the value range of K1 from (0..15) to (0..31) FFS: Whether there is an impact on the size of the PDSCH-to-HARQ_feedback timing indicator field in DCI. The motivation to have extension of K1 is to support the contiguous DL slots in specific TDD deployment scenario. UE should be able to support this feature when operating with some specific TDD configuration. Thus this should be added as an optional UE feature. Proposal: Add the extension of K1 value as the UE optional feature specifically to TDD cases. |
| LG Electronics [12] |  |

# Discussion/Approval Items during RAN1 #107-e — First Checkpoint

After review of contributions submitted to RAN1 #107-e in this agenda item, the following topics were identified by the moderator for discussion/approval during RAN1 #107-e.

| Company | Comments/Questions/Suggestions |
| --- | --- |

# Issue 1: FG 26-1

After review of contributions submitted to RAN1 #107-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in red

| 26. NR_NTN_solutions | 26-1 | Uplink Time pre-compensation | - UE specific TA calculation in RRC_IDLE and RRC_INACTIVE states based on its GNSS-acquired position and the serving satellite ephemeris. - UE specific TA calculation in RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris. - UE applies common TA according to the parameters provided by the network (if any) (UE considers common TA as 0 if the parameter is not provided) - For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops - FFS: UE (re)acquire the serving satellite ephemeris and common TA upon the expiration of the corresponding validity timer |  | No | No | Release 17 UE cannot access [NTN/ satellite/HAPS/ATG] | [Per UE/per band] | No | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] | An NTN UE is required to at least support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris [Note: Serving satellite ephemeris Epoch time is implicitly known as a reference time defined by the starting time of a DL slot and/or frame] FFS: Whether this starting time is given by predefined rule or it is indicated by the Network | Optional with capability signalling For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported. [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | In the last column, make a corresponding update: For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported. |
| Apple | We are fine with the changes except the following 2 comments: 1. We are still discussing which common TA parameters are mandatory/optional in AI 8.4.2. Hence, in component 3, we may keep “UE considers common TA as 0 if the parameter is not provided.” in bracket. 2. We may need to add a component of “UE pre-compensates the calculated TA in its uplink transmissions”. |

# Issue 2: FG 26-2

After review of contributions submitted to RAN1 #107-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal:

- Alt. 1:

- Adopt the following changes highlighted in red

- Alt. 2:

- Adopt the following changes highlighted in red

- Remove the FG as independent row

- Merge the component with FG 26-1

  - “Support of frequency pre-compensation to counter shift the Doppler experienced on the service link”

| 26. NR_NTN_solutions | 26-2 | Uplink Frequency/Doppler pre-compensation | - [In RRC_IDLE and RRC_INACTIVE states calculate Support of frequency pre-compensation to counter shift the Doppler experienced on the service link] - In RRC_CONNECTED states, calculate frequency pre-compensation to counter shift the Doppler experienced on the service link |  | No | No | Release 17 UE cannot access [NTN/ satellite/HAPS/ATG] | [Per UE/per band] | No | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] | An NR NTN UE shall be capable of at least using its acquired GNSS position and satellite ephemeris to calculate frequency pre-compensation to counter shift the Doppler experienced on the service link FFS: whether this FG gets merged with FG 26-1 | Optional with capability signalling For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported. [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | - Alt 2 - In the “Consequence if the feature is not supported by the UE” column, make a similar update as 26-1: Release 17 UE cannot access [NTN/ satellite/HAPS/ATG] - In the last column, make a corresponding update: For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported. |
| Apple | We prefer Alt 2, since both FG 26-1 and FG 26-2 are related to pre-compensation of uplink transmissions. |

# Issue 3: FG 26-3

After review of contributions submitted to RAN1 #107-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal:

- Adopt the following changes highlighted in red

- Remove the FG as independent row

- Merge the components with FG 26-1

  - At least “Support of estimating UE-gNB RTT and delaying the start of RAR window [by UE-gNB RTT]”

  - Continue discussion whether FFS components should be separated into new FGs/rows

| 26. NR_NTN_solutions | 26-3 | Enhancement on the timing relationship | - estimate of UE-gNB RTT Support of estimating UE-gNB RTT and delaying the start of RAR window [by UE-gNB RTT] - delaying the start of ra-ResponseWindow [by UE-gNB RTT] - delaying the start of msgB-ResponseWindow [by UE-gNB RTT] - FFS: delaying the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS activation of TA command, first PUSCH transmission in CG Type 2 with cell-specific K_offset - FFS: delaying the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated - FFS: separate FGs for cell specific Koffset and Kmac/UE-gNB RTT estimation/delay of RAR/MsgB response window | [26-1][26-2] | No | No | Release 17 UE cannot access [NTN/ satellite/HAPS/ATG] | [Per UE/per band] | No | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] | FFS: whether this FG gets merged with FG 26-1 | Optional with capability signalling For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | - Support merging the following component with 26-1 - At least “Support of estimating UE-gNB RTT and delaying the start of RAR window [by UE-gNB RTT]” - Make 26-3 a row dedicated for timing relationship enhancements listed in the FFS components, and remove [26-1][26-2] from the “Prerequisite feature groups” column. - In the last column, make a corresponding update: For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported. |
| Apple | - We do not think it should be merged with 26-1. FG 26-1 is about the time advance pre-compensation of uplink transmissions. However, FG26-3 is about the determination of uplink slot index. We have two independent agenda items to handle the topics of 26-1 and 26-3, respectively. - We support to further split the FG into two: one related to “cell specific Koffset” (e.g., component 4) and one related to “K_mac” (e.g., components 1 and 5) |

# Issue 4: FG 26-4

After review of contributions submitted to RAN1 #107-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in red

| 26. NR_NTN_solutions | 26-4 | UE reporting of information about the UE specific TA pre-compensation | Support UE reporting of information about the UE specific TA pre-compensation - [The FFS exact content of UE reporting of information about the UE specific TA pre-compensation], e.g., frequency of the reports, granularity of the reported conten, etc. - FFS: frequency of the reports - FFS: granularity of the reported content | [26-1][26-2] | Yes | No |  | [Per UE/Per band] | No | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] | FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive | Optiona lwith capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | Remove [26-2] from the “Prerequisite feature groups” column, b/c UE does not need [26-2] for TA reporting |
| Apple | We support the proposal. |

# Issue 5: FG 26-5

After review of contributions submitted to RAN1 #107-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in red

| 26. NR_NTN_solutions | 26-5 | Increasing the number of HARQ processes | - The maximal supported HARQ process number is X32 for both UL and Y DL - FFS: Support on the maximal HARQ process number is up to UE capability - FFS: separate features for DL and UL |  | Yes | No | [Increasing the number of HARQ processes avoids HARQ stalling] Increased number of HARQ processes is not supported | FFS [Per band or per FSPC] | No | No | [support mixture of FDD/TDD (for HAPS and/or STG) and/or FR1/FR] | FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive Candidate component values for X: {16,32} Candidate component values for Y: {16,32} | Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | - UE shouldn’t report (X, Y) = (16, 16), which is the existing case already. So it’s better to write the value set (X, Y): (16, 32), (32, 16), and (32, 32) - In the type column, delete “FSPC”, and update it to be [Per UE/Per band] - It shouldn’t be FSPC. In Rel-15, supporting up to 16 HARQ processes is a basic feature, i.e., mandatory without capability signaling, and certainly it’s per UE. Here the support is optional, but the support should continue to be per UE. |
| Apple | - We support the proposal. |
| OPPO | - We support this proposal and we also support per FSPC. Given in NR-NTN R17, CA is not supported. Thus, it is enough that the UE only needs to report that the UE supports the feature for the access carrier. |

# Issue 6: FG 26-6

After review of contributions submitted to RAN1 #107-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal:

- Confirm FG 26-6 as independent row/FG

- Adopt the following changes highlighted in red

- Continue discussing the relationship with HARQ disabling

| 26. NR_NTN_solutions | 26-6 | Type-2 HARQ codebook Enhancement | - Enhancement on Type-2 HARQ codebook in NTN Support of type-2 HARQ codebook enhancements [for feedback-disabled HARQ processes] - FFS: HARQ disabling | [26-1, 26-2] | Yes | No |  | [Per UE/Per band] | No | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] | FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive | Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | Remove [26-1][26-2] from the “Prerequisite feature groups” column, as the support of this UE capability does not need them as prerequisite feature groups. |
| Apple | We support the proposal. |
| OPPO | We support by adding [for feedback-disabled HARQ process]. We suggest to make HARQ disabling as an independent FG rather than under type 2 HARQ codebook enhancement. |

# Issue 7: FG 26-8

After review of contributions submitted to RAN1 #107-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in red

| 26. NR_NTN_solutions | 26-8 | Support of polarization [signalling/ information/reception] in NR NTN | - Support polarization indication reception in SIB indicating DL and/or UL polarization information using respective polarization type parameters to indicate: RHCP or LHCP or linear - FFS: polarization information for DL is indicated in SIB by the network Support polarization signalling for target serving cell in handover command message - Support polarization signalling for non-serving cell in RRM measurement configuration |  | No | No | [It is assumed by the network that UE supports at least linear polarization] | [Per UE/Per band] | No | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] | [Value range for component 1: {(RHCP, LHCP, Linear), (RHCP, LHCP), (RHCP), (LHCP), (Linear)}] FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive | Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | We are fine with the proposal. |

# Issue 8: FG 26-9

After review of contributions submitted to RAN1 #107-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in red

| 26. NR_NTN_solutions | 26-9 | UE-specific K_offset | - Reception of UE-specific K_offset via MAC-CE - delaying the scheduling of PUSCH, PUCCH [and PDCCH ordered PRACH], CSI reference resource, transmission of aperiodic SRS, activation of TA command, first PUSCH transmission in CG Type 2 with UE-specific Koffset | 26-3, 26-4 | Yes | No |  | FFS | No | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] | FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive | Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | It is not always delaying the scheduling of x, y, z. Revise the component description as: delaying determining the scheduling timing of PUSCH, PUCCH [and PDCCH ordered PRACH], CSI reference resource, transmission of aperiodic SRS, activation of TA command, first PUSCH transmission in CG Type 2 with UE-specific Koffset |
| Apple | We support the proposal. |

# Discussion/Approval Items during RAN1 #107-e — Second Checkpoint

Based on the comments/questions/suggestions received by the first checkpoint, the following are the revised proposals and/or proposed agreements by the moderator. Companies submitted the following views on the moderator’s proposals.

[Please submit all comments/questions/suggestions here, late comments/questions/suggestions submitted in Section 3 will not be considered]

General comments

| Company | Comments/Questions/Suggestions |
| --- | --- |

# Issue 1: FG 26-1

Proposal: Adopt the following changes highlighted in chromatic formatting

| 26. NR_NTN_solutions | 26-1 | Uplink Time pre-compensation | - UE specific TA calculation in RRC_IDLE and RRC_INACTIVE states based on its GNSS-acquired position and the serving satellite ephemeris. - UE specific TA calculation in RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris. - UE applies common TA according to the parameters provided by the network (if any) [(UE considers common TA as 0 if the parameter is not provided)] - For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops - FFS: UE (re)acquire the serving satellite ephemeris and common TA upon the expiration of the corresponding validity timer - FFS: UE pre-compensates the calculated TA in its uplink transmissions |  | No | No | Release 17 UE cannot access [NTN/ satellite/HAPS/ATG] | [Per UE/per band] | No | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] | An NTN UE is required to at least support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris [Note: Serving satellite ephemeris Epoch time is implicitly known as a reference time defined by the starting time of a DL slot and/or frame] FFS: Whether this starting time is given by predefined rule or it is indicated by the Network | Optional with capability signalling For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported. [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | In the last column, make a corresponding update: For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported. |
| Apple | We are fine with the changes except the following 2 comments: 1. We are still discussing which common TA parameters are mandatory/optional in AI 8.4.2. Hence, in component 3, we may keep “UE considers common TA as 0 if the parameter is not provided.” in bracket. 2. We may need to add a component of “UE pre-compensates the calculated TA in its uplink transmissions”. |
| Huawei, HiSilicon | We are fine with this FG |
| Nokia, NSB | - While we can always add a note saying a group of features must be indicated together, aka basic features, we should use caution with such definitions when it is known already that they belong together. If this is done late we create unnecessary overhead by using several bits to indicate something where a single bit would suffice. For example, IOT-NTN is consolidating nearly all features into a single FG, for essentially same type of functionality as for NTN in general. Hence, we propose to consider merging all features currently proposed as basic features together, i.e. merge 26-1, 26-2, 26-3, 26-6. - We agree with Ericsson’s revision on notes column, which applies to all FGs where such a note is present (see comment above in any case about minimizing number of basic features). |
| LG | In AI 8.4.2, it is still discussed the value range of common TA parameters. Therefore, (UE considers common TA as 0 if the parameter is not provided) in 3 can be removed and discussed after the regarding issue is determined. |

# Issue 2: FG 26-2

Proposal:

- Alt. 1:

- Adopt the following changes highlighted in chromatic formatting

- Alt. 2:

- Adopt the following changes highlighted in chromatic formatting

- Remove the FG as independent row

- Merge the component with FG 26-1

  - “Support of frequency pre-compensation to counter shift the Doppler experienced on the service link”

| 26. NR_NTN_solutions | 26-2 | Uplink Frequency/Doppler pre-compensation | - [In RRC_IDLE and RRC_INACTIVE states calculate Support of frequency pre-compensation to counter shift the Doppler experienced on the service link] - In RRC_CONNECTED states, calculate frequency pre-compensation to counter shift the Doppler experienced on the service link |  | No | No | Release 17 UE cannot access [NTN/ satellite/HAPS/ATG] | [Per UE/per band] | No | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] | An NR NTN UE shall be capable of at least using its acquired GNSS position and satellite ephemeris to calculate frequency pre-compensation to counter shift the Doppler experienced on the service link FFS: whether this FG gets merged with FG 26-1 | Optional with capability signalling For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported. [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | - Alt 2 - In the “Consequence if the feature is not supported by the UE” column, make a similar update as 26-1: Release 17 UE cannot access [NTN/ satellite/HAPS/ATG] - In the last column, make a corresponding update: For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported. |
| Apple | We prefer Alt 2, since both FG 26-1 and FG 26-2 are related to pre-compensation of uplink transmissions. |
| Huawei, HiSilicon | Okay with Alt 2. |
| Nokia, NSB | Alt 2 |
| LG | Alt 2 |

# Issue 3: FG 26-3

Revised Proposal:

- Adopt the following changes highlighted in chromatic formatting

- Merge component 1 “Support of estimating UE-gNB RTT and delaying the start of RAR window [by UE-gNB RTT]” with FG 26-1

- Keep the row and the remaining components as placeholder (with yellow background) and continue discussion

| 26. NR_NTN_solutions | 26-3 | Enhancement on the timing relationship | - estimate of UE-gNB RTT Support of estimating UE-gNB RTT and delaying the start of RAR window [by UE-gNB RTT] - delaying the start of ra-ResponseWindow [by UE-gNB RTT] - delaying the start of msgB-ResponseWindow [by UE-gNB RTT] - FFS: delaying the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS activation of TA command, first PUSCH transmission in CG Type 2 with cell-specific K_offset - FFS: delaying the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated - FFS: separate FGs for cell specific Koffset and Kmac/UE-gNB RTT estimation/delay of RAR/MsgB response window | [26-1][26-2] | No | No | Release 17 UE cannot access [NTN/ satellite/HAPS/ATG] | [Per UE/per band] | No | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] | FFS: whether this FG gets merged with FG 26-1 | Optional with capability signalling For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | - Support merging the following component with 26-1 - At least “Support of estimating UE-gNB RTT and delaying the start of RAR window [by UE-gNB RTT]” - Make 26-3 a row dedicated for timing relationship enhancements listed in the FFS components, and remove [26-1][26-2] from the “Prerequisite feature groups” column. - In the last column, make a corresponding update: For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported. |
| Apple | - We do not think it should be merged with 26-1. FG 26-1 is about the time advance pre-compensation of uplink transmissions. However, FG26-3 is about the determination of uplink slot index. We have two independent agenda items to handle the topics of 26-1 and 26-3, respectively. - We support to further split the FG into two: one related to “cell specific Koffset” (e.g., component 4) and one related to “K_mac” (e.g., components 1 and 5) |
| Huawei, HiSilicon | Support to merge 26-3 with 26-1 since they are still basic features in order to operate NTN. |
| Nokia, NSB | - Support FL proposal in general - No need for separating the FGs |
| LG | Support FL proposal. |

# Issue 4: FG 26-4

Proposal: Adopt the following changes highlighted in chromatic formatting

| 26. NR_NTN_solutions | 26-4 | UE reporting of information about the UE specific TA pre-compensation | Support UE reporting of information about the UE specific TA pre-compensation - [The FFS exact content of UE reporting of information about the UE specific TA pre-compensation], e.g., frequency of the reports, granularity of the reported conten, etc. - FFS: frequency of the reports - FFS: granularity of the reported content | [26-1][26-2] | Yes | No |  | [Per UE/Per band] | No | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] | FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive | Optiona lwith capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | Remove [26-2] from the “Prerequisite feature groups” column, b/c UE does not need [26-2] for TA reporting |
| Apple | We support the proposal. |
| Huawei, HiSilicon | The sub-bullet are not needed since there is no need to have different UE capabilities for the reporting. [26-2] can be removed from the “Prerequisite feature groups” |
| Nokia, NSB | Support the edits. |
| LG | Remove [26-2] from the “Prerequisite feature groups” |

# Issue 5: FG 26-5

Proposal: Adopt the following changes highlighted in chromatic formatting

| 26. NR_NTN_solutions | 26-5 | Increasing the number of HARQ processes | - The maximal supported HARQ process number is X32 for both UL and Y DL - FFS: Support on the maximal HARQ process number is up to UE capability - FFS: separate features for DL and UL |  | Yes | No | [Increasing the number of HARQ processes avoids HARQ stalling] Increased number of HARQ processes is not supported | FFS [Per band or per FSPC or per UE] | No | No | [support mixture of FDD/TDD (for HAPS and/or STG) and/or FR1/FR] | FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive Candidate component values for (X,Y): {(16,32),(32,16),(32,32)} Candidate component values for Y: {16,32} | Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | - UE shouldn’t report (X, Y) = (16, 16), which is the existing case already. So it’s better to write the value set (X, Y): (16, 32), (32, 16), and (32, 32) - In the type column, delete “FSPC”, and update it to be [Per UE/Per band] - It shouldn’t be FSPC. In Rel-15, supporting up to 16 HARQ processes is a basic feature, i.e., mandatory without capability signaling, and certainly it’s per UE. Here the support is optional, but the support should continue to be per UE. |
| Apple | - We support the proposal. |
| OPPO | - We support this proposal and we also support per FSPC. Given in NR-NTN R17, CA is not supported. Thus, it is enough that the UE only needs to report that the UE supports the feature for the access carrier. |
| Huawei, HiSilicon | Support to remove per FSPC since a UE may support NTN and TN at the same time. Defining it as a Per band UE capability is more sensible. We suggest to remove [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] given that 32 HARQ processes is also supported for above 52.6GHz. There is no need to define duplicate FGs for the same feature. |
| Nokia, NSB | It should not be considered as FSPC, [Per UE/Per band] are the valid options for further discussion from our point of view |
| LG | Support the proposal. |

# Issue 6: FG 26-6

Proposal:

- Confirm FG 26-6 as independent row/FG

- Adopt the following changes highlighted in chromatic formatting

- Continue discussing the relationship with HARQ disabling

| 26. NR_NTN_solutions | 26-6 | Type-2 HARQ codebook Enhancement | - Enhancement on Type-2 HARQ codebook in NTN Support of type-2 HARQ codebook enhancements [for feedback-disabled HARQ processes] - FFS: HARQ disabling | [26-1, 26-2] | Yes | No |  | [Per UE/Per band] | No | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] | FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive | Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | Remove [26-1][26-2] from the “Prerequisite feature groups” column, as the support of this UE capability does not need them as prerequisite feature groups. |
| Apple | We support the proposal. |
| OPPO | We support by adding [for feedback-disabled HARQ process]. We suggest to make HARQ disabling as an independent FG rather than under type 2 HARQ codebook enhancement. |
| Huawei, HiSilicon | Component 2 can be updated to “UE supports HARQ disabling” [26-1, 26-2] can be removed from the “Prerequisite feature groups”. |
| Nokia, NSB | HARQ disabling is an important feature of NTN, and this FG could be merged to 24-1. In any case, HARQ disabling and codebook enhancement needs to be addressed by the same FG. |
| LG | Remove [26-1][26-2] from the “Prerequisite feature groups”. We also support to make HARQ disabling as an independent FG. |

# Issue 7: FG 26-8

Proposal: Adopt the following changes highlighted in chromatic formatting

| 26. NR_NTN_solutions | 26-8 | Support of polarization [signalling/ information/reception] in NR NTN | - Support polarization indication reception in SIB indicating DL and/or UL polarization information using respective polarization type parameters to indicate: RHCP or LHCP or linear - FFS: polarization information for DL is indicated in SIB by the network Support polarization signalling for target serving cell in handover command message - Support polarization signalling for non-serving cell in RRM measurement configuration |  | No | No | [It is assumed by the network that UE supports at least linear polarization] | [Per UE/Per band] | No | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] | [Value range for component 1: {(RHCP, LHCP, Linear), (RHCP, LHCP), (RHCP), (LHCP), (Linear)}] FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive | Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | We are fine with the proposal. |
| Huawei, HiSilicon | Support |
| Nokia, NSB | Reception of the polarization indication itself is not a UE feature, the real benefit comes from the UE applying the information. |
| LG | Support |

# Issue 8: FG 26-9

Proposal: Adopt the following changes highlighted in chromatic formatting

| 26. NR_NTN_solutions | 26-9 | UE-specific K_offset | - Reception of UE-specific K_offset via MAC-CE - Determining delaying the scheduling timing of PUSCH, PUCCH [and PDCCH ordered PRACH], CSI reference resource, transmission of aperiodic SRS, activation of TA command, first PUSCH transmission in CG Type 2 with UE-specific Koffset | 26-3, 26-4 | Yes | No |  | FFS | No | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] | FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive | Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | It is not always delaying the scheduling of x, y, z. Revise the component description as: delaying determining the scheduling timing of PUSCH, PUCCH [and PDCCH ordered PRACH], CSI reference resource, transmission of aperiodic SRS, activation of TA command, first PUSCH transmission in CG Type 2 with UE-specific Koffset |
| Apple | We support the proposal. |
| Huawei, HiSilicon | Support |
| Nokia, NSB | Support. |
| LG | Support |

# Conclusion

After further discussion on the RAN1 email reflector the following was agreed as part of this email discussion:

# References

- R1-2110587, Updated RAN1 UE features list for Rel-17 NR after RAN1 #106bis-e, Moderators (AT&T, NTT DOCOMO, INC.)

- R1-2110807, Rel-17 UE features for NR NTN, Huawei/HiSilicon

- R1-2111155, On UE features for NR NTN, Nokia/Nokia Shanghai Bell

- R1-2111318, Discussion on UE features for NTN-NR, OPPO

- R1-2111417, On UE features for NR NTN, Ericsson

- R1-2111528, On UE features for NR NTN, Intel Corporation

- R1-2111665, Discussion on UE feature for NR-NTN, ZTE

- R1-2111772, UE features for NR NTN, Samsung

- R1-2111909, Views on Rel-17 NR NTN UE features, Apple

- R1-2112249, UE features for NR NTN, Qualcomm Incorporated

- R1-2112272, Discussion on UE features for NTN, Beijing Xiaomi Mobile Software

- R1-2111972, Discussion on Rel-17 UE feature for NR NTN, LG Electronics