3GPP TSG RAN WG1 #107-e		R1-2112646

e-Meeting, November 11th – 19th, 2021

Title:	Reply LS on MCCH change notification

Response to:	R1-2104165 (R2-2104639) LS on broadcast session delivery and MCCH design

Release:	Rel-17

Work Item:	NR_MBS

Source:	RAN1

To:	RAN2

Cc:

Contact person:	David Vargas

david.vargas@bbc.co.uk

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# 1	Overall description

RAN1 would like to thank RAN2 for the LS R1-2104165 (R2-2104639) on broadcast session delivery and MCCH design, and the LS R1-2106410 (R2-2106544) on update for MCCH design. The following is the RAN1 response for the question listed in R1-2104165 (R2-2104639) on details of the RNTI and DCI design for carrying MCCH change notifications.

Question: Details of the RNTI and DCI design for carrying MCCH change notifications.

RAN1 answer:

Regarding MCCH change notification, RAN1 has made the following agreements:

Agreement: For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, study the following alternatives for MCCH change notification indication due to session start: - Alt 1: Define a dedicated RNTI to scramble the CRC of a DCI indicating a MCCH change notification; - Alt 2: Use of a field in a DCI format scheduling a MCCH without a dedicated RNTI for MCCH change notification; Other solutions are not precluded and it is also not precluded whether to support both Alt1 and Alt2. Conclusion: It is up to RAN2 to decide the specific contents of the MCCH change notification, e.g, whether notification only informs about session start, whether or not notification also informs about session modification/stop or whether or not the notification informs about any other information. Agreement: Study and reach an agreement by RAN1#106b-e on whether Alt1 and Alt2 for MCCH change notification indication can accommodate at least 2 bits for the notification of MCCH configuration changes due to a session start and the notification of MCCH configuration changes of an ongoing session (including session stop). Working assumption: Alt 2 (from previous agreement) is supported for broadcast reception with RRC_IDLE/RRC_INACTIVE UEs for the notification of MCCH configuration changes. - Send an LS to RAN2 with the mechanism agreed in RAN1 Agreement Confirm the working assumption made at RAN1#106bis-e: Working assumption: Alt 2 (from previous agreement) is supported for broadcast reception with RRC_IDLE/RRC_INACTIVE UEs for the notification of MCCH configuration changes. - Send an LS to RAN2 with the mechanism agreed in RAN1

# 2	Actions

To: RAN2

ACTION: 	RAN1 respectfully asks RAN2 to take the above information into account.

# 3	Dates of next TSG RAN WG1 meetings

TSG RAN WG1 Meeting #107bis-e                       	17th Jan – 25th Jan, 2022			E-meeting

TSG RAN WG1 Meeting #108-e                       	21st Feb – 3rd Mar, 2022			E-meeting