3GPP TSG RAN WG1 #107-e	R1-2112744

e-Meeting, November 11th–19th, 2021

Title:	LS on UL SRS-RSRPP definition

Reply to:

Release:	Rel-17

Work Item:	NR_pos_enh-Core

Source:	RAN WG1

To:	RAN WG4

Cc:

Contact Person:

#### Name:	Alexey Khoryaev

#### Email Address:	alexey.khoryaev@intel.com

Send any reply LS to:	 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN1 has discussed definition of UL SRS-RSRPP for NR positioning. The following definition was agreed by RAN1 as an outcome of the discussion at RAN1#107e:

- Definition UL SRS reference signal received path power (UL SRS-RSRPP) is defined as the power of the received UL SRS signal configured for the measurement at the i-th path delay of the channel response, where UL SRS-RSRPP for 1st path delay is the power corresponding to the first detected path. Working assumption: For frequency range 1, the reference point for the UL SRS-RSRPP shall be the antenna connector of the gNB. For frequency range 2, UL SRS-RSRPP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. Note: The following two options are supported by gNB to LMF: - Option 1 (RX diversity for the first path UL SRS-RSRPP) - The same RX branch(es) as applied for the first path UL SRS-RSRPP measurements are used for the additional paths UL SRS-RSRPP measurements if those are provided together - For frequency range 1 and 2, if receiver diversity is in use by the gNB for UL SRS-RSRPP measurements, then reported UL SRS-RSRPP value for the first path shall not be lower than the corresponding UL SRS-RSRPP for the first path of any of the individual receiver branches - Option 2 (RX diversity for UL SRS-RSRP) - The same RX branch(es) as applied for UL SRS-RSRP measurements are used for UL SRS-RSRPP measurements (i.e., the first and additional paths UL SRS-RSRPP if those are provided)

RAN1 asks RAN4 to consider the above agreement during the work on NR positioning enhancements and provide feedback on working assumption for reference point if any issue is identified.

2. Actions:

To RAN4 group

ACTION: RAN1 asks RAN4 to consider the above agreement during the work on NR positioning enhancements and provide feedback on working assumption for reference point if any issue is identified

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #107-bis-e 	January 17 – 25, 2022		        Electronic Meeting

TSG-RAN WG1 Meeting #108-e 		February 21 – March 03, 2022       Electronic Meeting