3GPP TSG RAN WG1 #107-e	R1-2112754

e-Meeting, 11th – 19th November, 2021

Title:	Reply LS on capability related RAN2 agreements for RedCap

Response to:	R1-2108714 / R2-2109218

Release:	Rel-17

Work Item:	Support of reduced capability NR devices (NR_redcap)

Source:	RAN1

To:	RAN2

Cc:	RAN4

Contact person:	Johan Bergman, johan (dot) bergman (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN1 would like to thank RAN2 for the LS on capability related RAN2 agreements for Rel-17 NR RedCap WI.

RAN1 has discussed the RAN2 agreements listed in the LS and the applicability of Rel-15/16 UE features or capabilities for Rel-17 RedCap UEs and would like to inform RAN2 of the following.

The following Rel-15/16 UE features or capabilities are not applicable for Rel-17 RedCap UEs:

- Capabilities related to carrier aggregation (CA)

- Capabilities related to dual connectivity (DC)

- Capabilities related to UE bandwidths wider than 20 MHz in FR1 or wider than 100 MHz in FR2

- Capabilities related to more than 2 UE Rx branches or more than 2 DL MIMO layers

- Capabilities related to more than 2 UE Tx branches or more than 2 UL MIMO layers

There are many L1 UE feature groups (FGs) in TR 38.822 (“NR; User Equipment (UE) feature list”) related to these capabilities. If needed, RAN1 can provide detailed information regarding the applicability of each FG.

RAN1 has made the following agreement related to the maximum number of PDSCH MIMO layers:

Agreement: - Inform RAN2 that “From RAN1 perspective, it would be enough to indicate the maximum number of PDSCH MIMO layers per band for RedCap UEs, but RAN1 notes that the type of FG2-3 (maxNumberMIMO-LayersPDSCH) is currently per FSPC and that it is up to RAN2 whether to signal per band or per FSPC”

Furthermore, RAN1 has made the following agreements regarding support of the RF/RRM FGs related to 256QAM for PDSCH and PUSCH.

Agreement: - RedCap UE supports FG1-4 (256QAM for PDSCH) as optional with capability signalling both for FR1 and FR2 - Add a note in FG 1-4 (256QAM for PDSCH) that “For RedCap UEs, the 256QAM MCS table for PDSCH and CQI table 2 are only supported if the UE supports 256QAM for PDSCH” Agreement: - Add a note in FG 1-5 (256QAM for PUSCH) that “For RedCap UEs, the 256QAM MCS table for PUSCH is only supported if the UE supports 256QAM for PUSCH”

Finally, RAN1 has made the following agreements relevant to L1 FG 6-1 “Basic BWP operation with restriction” and L1 FG 6-1a “BWP operation without restriction on BW of BWP(s)”, where the differences between the agreements for FR1 and FR2 are indicated in blue colour.

Agreement: - For both FR1 and FR2, for a cell that allows a RedCap UE to access, network can configure a separate initial DL BWP for RedCap UEs in SIB. At least the case when the separate initial DL BWP includes CD-SSB and the entire CORESET#0 is supported - It can be used in idle/inactive mode (including paging) and during and after initial access, when applicable - It is no wider than the maximum RedCap UE bandwidth. - This applies to both TDD and FDD (including FD FDD and HD FDD) cases. Agreement: - For FR1, - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - If it is configured for random access while not for paging in idle/inactive mode, RedCap UE does NOT expect it to contain SSB/CORESET#0/SIB. - Note: RAN1 assumes REDCAP UE performing Random access in the separate DL BWP does not need to monitor paging in a BWP containing CORESET#0 - Working assumption: If it is configured for paging, RedCap UE expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB from RAN1 perspective - For an RRC-configured active DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - A RedCap UE can indicate the following as optional capability: - Not need NCD-SSB: A RedCap UE can in addition optionally support relevant operation based on CSI-RS (working assumption) and/or FG 6-1a by reporting optional capabilities. - Note: if a separate initial/RRC configured DL BWP is configured to contain the entire CORESET#0, CD-SSB is expected by RedCap UE. - Note: The network may choose to configure SSB or MIB-configured CORESET#0 or SIB1 to be within the respective DL BWP. - Note: If a separate SIB-configured initial DL BWP for RedCap UEs contains the entire CORESET#0, the RedCap UE shall use the bandwidth and location of the CORESET#0 in DL during initial access. - Note: NCD-SSB periodicity is not required to be configured the same as that of CD-SSB - Note: Periodicity of NCD-SSB shall be not less than periodicity of CD-SSB Agreement: - For FR2, - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - If it is configured for random access while not for paging in idle/inactive mode, RedCap UE does NOT expect it to contain SSB/CORESET#0/SIB. - Note: RAN1 assumes REDCAP UE performing Random access in the separate DL BWP does not need to monitor paging in a BWP containing CORESET#0 - Working assumption: If it is configured for paging, RedCap UE expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB from RAN1 perspective - For an RRC-configured active DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - A RedCap UE can indicate the following as optional capability: - Not need NCD-SSB: A RedCap UE can in addition optionally support relevant operation based on CSI-RS (working assumption) and/or FG 6-1a by reporting optional capabilities. - Note: For SSB and CORESET#0 multiplexing pattern 1, if a separate initial/RRC configured DL BWP is configured to contain the entire CORESET#0, CD-SSB is expected by RedCap UE. - Note: The network may choose to configure SSB or MIB-configured CORESET#0 or SIB1 to be within the respective DL BWP. - Note: If a separate SIB-configured initial DL BWP for RedCap UEs contains the entire CORESET#0, the RedCap UE shall use the bandwidth and location of the CORESET#0 in DL during initial access. - Note: NCD-SSB periodicity is not required to be configured the same as that of CD-SSB - Note: Periodicity of NCD-SSB shall be not less than periodicity of CD-SSB

# 2	Actions

To RAN2:

ACTION: 	RAN1 respectfully asks RAN2 to take the above information into account in the RedCap work.

# 3	Dates of next TSG-RAN WG1 meetings

TSG-RAN WG1 Meeting #107bis-e	17th – 25th January 2022		Online

TSG-RAN WG1 Meeting #108-e	21st February – 3rd March 2022		Online