3GPP TSG RAN WG1 #107bis-e		R1-2200078

e-Meeting, January 17th – 25th, 2022

Source:	vivo

Title:	Remaining issues on PDSCH PUSCH enhancements for NR operation from 52.6GHz to 71GHz

Agenda Item:	8.2.5

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss the DMRS bundling across the multiple PUSCHs and potential TRS enhancements first, followed by discussion on remaining issues for multi-PDSCH/PUSCH scheduling and feedback related design.

- DMRS bundling across the multiple PUSCHs

In RAN1#107-e meeting, it was discussed whether to apply DMRS bundling introduced in Rel-17 Coverage enhancement WI for NR operation in FR2-2.

In FR2-2, the symbol duration and slot duration are smaller for SCS 480/960 kHz, and the typical scenario is stationary scenario, thus the channels for adjacent slots are more coherent than those for smaller SCS(s). However, the impact of phase noise and SCS on the performance gain has not been well studied, it is unclear with what conditions (such as modulation, SNR, SCS, etc.) the DMRS bundling can bring obvious performance gain. More importantly, DMRS bundling introduced in Rel-17 Coverage enhancement WI only supports same TB over multiple slots, and the agreement on different TBs over multiple slots has not been reached yet, while multi-PUSCH scheduling in FR2-2 only supports different TBs over multiple slots.

Proposal 1: Closely monitor the progress on DMRS bundling in Rel-17 Coverage enhancement WI, especially the support on different TBs over multiple slots. Only when this is supported, it is possible to apply DMRS bundling for NR operation in FR2-2.

- TRS enhancements

In RAN1#107 meeting, [2] proposed to introduce new TRS configurations with higher densities in frequency domain, i.e. 6 or 12 tones per RB, to increase the TTL pull-in range when the SCS of SSB is smaller than that of data transmission. Regarding this proposal, some concerns were raised in terms of specification impact, limited remaining time for the WI and potentially increased PHY complexity.

The issue comes from limited pull-in range, i.e., +/-12 ×ΔkTRS*SCSPDSCH  based on TRS of 960 kHz SCS and large SSB timing error limit based on SSB of 120 kHz SCS. The SSB timing error limit is calculated as 6256*SCSSSB , where the value 256 means the SSB timing granularity is enhanced by using x2 oversampling based on 127 PSS/SSS sub-carriers, and the value 6 means the SSB timing error limit is reduced down to n=6 chips. Based on this calculation method, the SSB timing error limits for 120/960 kHz SCS are listed in Table 1 below. For a PDSCH of 960 kHz SCS, the pull-in range is +/-130ns. In addition to the CD SSB of 120 kHz SCS, if network transmits an NCD SSB of 960 kHz SCS and indicates UE to measure the NCD SSB, the SSB timing error limit can be reduced from 195.31 ns to 24.41 ns, which is within the pull-in range of a TRS of 960 kHz SCS. This can be up to the implementation of gNB’s configuration and/or scheduling, and no specification impact will be introduced.

Table 1 SSB timing error limits for 120/960 kHz SCS

| SSB SCS (kHz) | Timing granularity (ns) | Timing error limit (ns) |
| --- | --- | --- |
| 120 | 32.55 | 195.31 |
| 960 | 4.07 | 24.41 |

Proposal 2: The timing error issue due to smaller SCS of SSB than that of data transmission can be resolved by gNB implementation, e.g., gNB transmits an NCD SSB of 960 kHz SCS and indicates UE to measure it, and no specification impact shall be introduced.

- Multi-PDSCH/PUSCH scheduling

  - DCI design for multi-PDSCH/PUSCH scheduling

4.1.1 Common issues for DL and UL

4.1.1.1 (De)Activation of SPS/CG

Since RAN1#106b-e meeting, the issue about activation/de-activation of SPS/CG by using multi-PDSCH/PUSCH scheduling DCI has been discussed without consensus. During the discussion, three options were identified, as listed below.

- Option 1: Allow only single SLIV-based (de)activation

- Option 2: Based on the last (valid) SLIV

- Option 3: Based on the first (valid) SLIV

Among these options, Option 3 is slightly preferred due to that each row in the TDRA table can be used for activation/de-activation, which can bring more flexibility, and using the first (valid) SLIV can lead to shorter latency between an activation DCI and the first PDSCH/PUSCH transmission corresponding to the activation DCI. Based on some comments during RAN1#106-e meeting for Option 3, some clarification may be needed for K1 interpretation for SPS transmission in conjunction with multi-PDSCH scheduling, then existing Type-1 codebook construction procedure can be reused without any additional modification.

Proposal 3: For activation/de-activation of SPS/CG by using multi-PDSCH/PUSCH scheduling DCI, the first (valid) SLIV in the row indicated by an activation/de-activation DCI is used for determining SPS/CG occasions.

4.1.1.2 Out-of-order issue

Since RAN1#106b-e meeting, out-of-order related issues have been discussed extensively with no agreement. Two issues can be identified, i.e., DCI-to-data OOO issue and PDSCH-to-HARQ-ACK OOO issue.

- DCI-to-data OOO

During RAN1#107-e meeting, DCI-to-data OOO issue was discussed with the last version of corresponding proposal listed as below.

Proposal #2.6-1a (DCI-to-data OOO):

- UE does not expect any of the scheduled PDSCHs (or PUSCHs) and the scheduling DCI to lead to out-of-order scheduling, also for the case of one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, where multi-PDSCH (or multi-PUSCH) scheduling DCI schedules more than one PDSCH (or PUSCH)

- The case where two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but two multi-PDSCH (or multi-PUSCH) scheduling DCIs have overlapping spans, where the span is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV, is considered as out-of-order scheduling and is not expected by UE.

  - This applies also when one of two DCIs is single-PDSCH (or single-PUSCH) scheduling DCI.

Regarding the first bullet in the above proposal, we support it as a conclusion since it is aligned with current Rel-15/16 out-of-order rule and there is no strong motivation to allow such case for FR2-2.

Regarding the second bullet, in our understanding, for Rel-15/16 PDSCH/PUSCH repetition operation with pdsch-AggregationFactor/pusch-AggregationFactor or repetitionNumber-r16/numberOfRepetitions-r16, overlapping spans is allowed, where the span can be regarded as from the beginning of the first repetition till the end of the last repetition belonging to a given PDSCH/PUSCH with repetitions, as long as any two repetitions for the involved two PDSCHs/PUSCHs don’t overlap each other.

Observation 1: The case where two DCIs, each of which schedules a multi-slot PDSCH (or multi-slot PUSCH), end in the same symbol but the two scheduled multi-slot PDSCHs (or multi-slot PUSCHs) have overlapping spans, where the span is defined from the beginning of the first repetition till the end of the last repetition for a PDSCH/PUSCH, is allowed in Rel-15/16.

Based on this observation, we don’t see strong motivation to regard the case where two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but have overlapping spans as out-of-order scheduling.

Proposal 4: For multi-PDSCH/PUSCH scheduling, DCI-to-PDSCH/PUSCH out-of-order scheduling is defined as NR Rel-15/16 without any exception.

- PDSCH-to-HARQ-ACK OOO

Regarding PDSCH-to-HARQ-ACK OOO issue, little discussion was done during RAN1#107-e meeting due to limited input.

From our perspective, any unicast PDSCH in a given cell, including dynamically scheduled PDSCH and SPS PDSCH, and corresponding HARQ-ACK reporting can reuse legacy out-of-order rule for simplicity.    Regarding the case involving PDSCHs scheduled by a multi-PDSCH scheduling DCI and other unicast PDSCH scheduled by single-PDSCH scheduling DCI, we don’t see strong motivation to allow out-of-order HARQ-ACK reporting for it.

Proposal 5: For multi-PDSCH scheduling, UE does not expect any of the scheduled PDSCH(s)/SPS PDSCH and the PUCCH resource for the HARQ-ACK transmission to lead to out-of-order HARQ-ACK reporting.

4.1.2 DL specific issues

4.1.2.1 Two codewords

Since RAN1#106b-e meeting, an issue about TB disabling has been brought by some companies, but no discussion has been focused on it due to limited time and input.

In TS38.214[4] there is the following description for TB disabling, i.e. disabling a TB by setting IMCS = 26 and rvid = 1 for the TB in a DCI format 1_1.

#### 5.1.3.2	Transport block size determination

In case the higher layer parameter maxNrofCodeWordsScheduledByDCI indicates that two codeword transmission is enabled, then one of the two transport blocks is disabled by DCI format 1_1 if IMCS = 26 and if rvid = 1 for the corresponding transport block. If both transport blocks are enabled, transport block 1 and 2 are mapped to codeword 0 and 1 respectively. If only one transport block is enabled, then the enabled transport block is always mapped to the first codeword.

…

For a DCI format for multi-PDSCH scheduling, when more than one PDSCH is scheduled, for a given TB, there are M bits in the DCI format for indicating redundancy version (RV) information, each one bit of which corresponds to a PDSCH scheduled by the DCI format, where M is the maximum number of PDSCHs that can be scheduled by a single DCI format.

For 1-bit RV for each scheduled PDSCH in a DCI format for multi-PDSCH scheduling, the following Table in TS38.212[3] can be reused for mapping an indicated value to corresponding rvid.

Table 7.3.1.1.2-34: Redundancy version

| Value of the Redundancy version field | Value of to be applied |
| --- | --- |
| 0 | 0 |
| 1 | 2 |

Based on above information, legacy mechanism for TB disabling cannot be reused for multi-PDSCH scheduling directly, since there are more than one RV field in the scheduling DCI, and each RV field cannot be mapped to rvid = 1.

Regarding this issue, the condition IMCS = 26 can be reused, and only the condition for rvid should be discussed. With respect to the condition for rvid, two alternatives can be identified as below.

- Alt. 1: For the M bits for indicating RV information for a given TB in a DCI format scheduling more than one PDSCH, a predefined value vector can be used to indicate the given TB is disabled for all PDSCHs scheduled by the DCI format, e.g. assuming M = 8, the 8-bit RV corresponding to a TB in the DCI format can be set to ‘11111111’ to indicate the TB is disabled for all PDSCHs scheduled by the DCI format.

- Alt. 2: For each PDSCH scheduled by a DCI format scheduling more than one PDSCH, a given TB can be disabled individually, i.e. a 1-bit RV for the given TB and for a PDSCH scheduled by the DCI format can be set to a predefined value, e.g. ‘1’, to indicate the given TB of the PDSCH is disabled.

Alt. 1 disables a TB on a per-DCI basis, while Alt. 2 disables a TB on a per-PDSCH basis. Since NDI and RV are indicated for each scheduled PDSCH individually, it may also be beneficial to perform TB disabling for each scheduled PDSCH individually, due to that each scheduled PDSCH correspond to different traffic. From the perspective of flexibility, Alt. 2 is slightly preferred.

Proposal 6: Regarding TB disabling for multi-PDSCH scheduling, when two codeword transmission is configured, for a DCI format scheduling more than one PDSCH, a given TB can be disabled for each scheduled PDSCH individually, by setting IMCS = 26 and the 1-bit RV for a scheduled PDSCH to a predefined value, e.g. ‘1’, to indicated the given TB for the scheduled PDSCH is disabled.

  - HARQ-ACK feedback for multi-PDSCH scheduling

4.2.1 Type-1 HARQ-ACK codebook

- Time domain bundling operation

During RAN1#107-e meeting, an agreement about time domain bundling for Type-1 codebook has been achieved, listed as below.

Agreement

For multi-PDSCH scheduling with a single DCI

- Introduce a new RRC parameter, e.g., enableTimeDomainHARQ-Bundling, to enable time domain bundling operation for type-1 HARQ-ACK codebook per serving cell.

  - If the RRC parameter enables time domain bundling operation,

    - To determine the set of candidate PDSCH reception occasions,

      - A row index is removed if at least one symbol of every PDSCH associated with the row index is configured as semi-static UL. (NOTE: This is similar to the case of slot aggregated PDSCH in Rel-16)

      - Pruning procedure in Rel-16 is performed based on the last configured SLIV of each row index.

    - Logical AND operation is applied across all valid PDSCHs associated with a determined candidate PDSCH reception occasion, at least for 1-TB case.

    - FFS: UE does not expect the last scheduled SLIV overlaps with a semi-static UL symbol when parameter enableTimeDomainHARQ-Bundling is configured

Regarding the FFS point, it may be too restrictive for gNB scheduling. In contrast, restricting the number of actually scheduled TDRA rows mapped to a same candidate PDSCH reception occasion to not exceed 1 may be more straightforward and more feasible, that is, UE does not expect more than one TDRA rows mapped to a same candidate PDSCH reception occasion is actually scheduled.

Alternatively, no explicit restriction is introduced. When more than one TDRA rows mapped to a same candidate PDSCH reception occasion is actually scheduled, logical AND operation is applied across all valid PDSCHs associated with the same candidate PDSCH reception, which are scheduled by the more than one TDRA rows. It is up to gNB implementation to check if time domain bundling should be limited only within valid PDSCH(s) scheduled by a single DCI indicating a TDRA row or not.

Based on the above discussion, restricting the number of actually scheduled TDRA rows mapped to a same candidate PDSCH reception occasion to not exceed 1 may be simpler, and time domain bundling across more than one TDRA rows can be avoided explicitly.

Proposal 7: For multi-PDSCH scheduling, when configured with Type-1 codebook and time domain bundling is enabled, UE does not expect more than one TDRA rows mapped to a same candidate PDSCH reception occasion is actually scheduled.

In addition, for the operation of excluding TDRA row(s) based on semi-static UL symbol(s) when time domain bundling is enabled, the description in [6] does not consider the case when more than one PDSCH time resource is configured within one DL slot based on a TDRA row for 120 kHz SCS, listed as below with the critical text in yellow background, therefore the specification may be incomplete.

-------------------------------------------------------Start of text in [6]-----------------------------------------------------------

If a UE is not provided ca-SlotOffset for any serving cell of PDSCH receptions and for the serving cell of corresponding PUCCH transmission with HARQ-ACK information

while k<CK1

if modnU-K1,k+1,max2μUL-μDL,1=0 or subslotLengthForPUCCH is provided for the HARQ-ACK codebook

Set nD=0 – index of a DL slot overlapping with an UL slot

Set Nk to a number of DL slots overlapping with UL slot nU-K1,k if subslotLengthForPUCCH is provided for the HARQ-ACK codebook; otherwise, Nk=max2μDL-μUL,1

while nD<Nk

……

if slot nU starts at a same time as or after a slot for an active DL BWP change on serving cell c or an active UL BWP change on the PCell and slot n0,k+nD is before the slot for the active DL BWP change on serving cell c or the active UL BWP change on the PCell, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and slot n0,k+nD overlaps with UL slot nU-K1,k-1, k>0, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k,

nD=nD+1;

else

while r<CR

if the UE is not provided enableTimeDomainHARQ-Bundling and is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r is configured as UL where K1,k is the k-th slot timing value in set K1, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and the end of the PDSCH time resource for row r is not within any UL slot nU-K1,l, 0≤l<CK1, or if HARQ-ACK information for PDSCH time resource derived by row r in slot nU-K1,k⋅2μDL-μUL+nD cannot be provided in slot nU

R=R\r;

elseif the UE is provided enableTimeDomainHARQ-Bundling and tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot nU-K1,k⋅2μDL-μUL+nD- ∆K0,r(d), at least one symbol of the PDSCH time resource derived by row r of set R' is configured as UL, where d = 0,1,…,C∆K0,r-1, ∆K0,r=maxK0K0,r-K0,r, and C∆K0,r is the cardinality of ∆K0,r.

R=R\r;

R'=R'\r;

else

r=r+1;

end if

end while

……

nD=nD+1;

end if

end while

end if

k=k+1;

end while

else

……

end if

-------------------------------------------------------End of text in [6]-----------------------------------------------------------

To resolve this issue, the following TP1 can be considered, with the modified text in red.

------------------------------------------------------------Start TP1----------------------------------------------------------------

If a UE is not provided ca-SlotOffset for any serving cell of PDSCH receptions and for the serving cell of corresponding PUCCH transmission with HARQ-ACK information

while k<CK1

if modnU-K1,k+1,max2μUL-μDL,1=0 or subslotLengthForPUCCH is provided for the HARQ-ACK codebook

Set nD=0 – index of a DL slot overlapping with an UL slot

Set Nk to a number of DL slots overlapping with UL slot nU-K1,k if subslotLengthForPUCCH is provided for the HARQ-ACK codebook; otherwise, Nk=max2μDL-μUL,1

while nD<Nk

……

if slot nU starts at a same time as or after a slot for an active DL BWP change on serving cell c or an active UL BWP change on the PCell and slot n0,k+nD is before the slot for the active DL BWP change on serving cell c or the active UL BWP change on the PCell, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and slot n0,k+nD overlaps with UL slot nU-K1,k-1, k>0, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k,

nD=nD+1;

else

while r<CR

if the UE is not provided enableTimeDomainHARQ-Bundling and is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r is configured as UL where K1,k is the k-th slot timing value in set K1, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and the end of the PDSCH time resource for row r is not within any UL slot nU-K1,l, 0≤l<CK1, or if HARQ-ACK information for PDSCH time resource derived by row r in slot nU-K1,k⋅2μDL-μUL+nD cannot be provided in slot nU

R=R\r;

elseif the UE is provided enableTimeDomainHARQ-Bundling and tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot nU-K1,k⋅2μDL-μUL+nD- ∆K0,r(d), at least one symbol of each of the one or more PDSCH time resources derived by row r of set R' is configured as UL, where d = 0,1,…,C∆K0,r-1, ∆K0,r=maxK0K0,r-K0,r, and C∆K0,r is the cardinality of ∆K0,r.

R=R\r;

R'=R'\r;

else

r=r+1;

end if

end while

……

nD=nD+1;

end if

end while

end if

k=k+1;

end while

else

……

end if

------------------------------------------------------------End TP1----------------------------------------------------------------

Proposal 8: For multi-PDSCH scheduling, when configured with Type-1 codebook and time domain bundling is enabled, consider the TP1 in TS38.213 for the operation of excluding TDRA row(s) based on semi-static UL symbol(s).

- Calculation of nHARQ-ACK

In [6] the calculation of nHARQ-ACK reuses Rel-16 formula with no modification, listed as below for reference.

9.1.2.1	Type-1 HARQ-ACK codebook in physical uplink control channel

……

If OACK+OSR+OCSI≤11, the UE determines a number of HARQ-ACK information bits nHARQ-ACK for obtaining a transmission power for a PUCCH, as described in clause 7.2.1, as nHARQ-ACK=c=0NcellsDL-1m=0Mc-1Nm,creceived+c=0NcellsDL-1m=0Mc-1Nm,creceived,CBG where

-	NcellsDL are all DL cells where the UE is configured to receive unicast or multicast PDSCHs

-	Mc is the cardinality for the union of all sets MA,c of occasions for unicast or multicast PDSCH receptions or SPS PDSCH releases for serving cell c

-	Nm,creceived is the number of transport blocks the UE receives in PDSCH reception occasion m for serving cell c if harq-ACK-SpatialBundlingPUCCH and PDSCH-CodeBlockGroupTransmission are not provided, or the number of transport blocks the UE receives in PDSCH reception occasion m for serving cell c if PDSCH-CodeBlockGroupTransmission is provided and the PDSCH reception is scheduled by a DCI format that does not support CBG-based PDSCH receptions, or the number of PDSCH receptions if harq-ACK-SpatialBundlingPUCCH is provided or SPS PDSCH release or TCI state update in PDSCH reception occasion m for serving cell c and the UE reports corresponding HARQ-ACK information in the PUCCH.

-	Nm,creceived,CBG is the number of CBGs the UE receives in a PDSCH reception occasion m for serving cell c if PDSCH-CodeBlockGroupTransmission is provided and the PDSCH reception is scheduled by a DCI format that supports CBG-based PDSCH receptions and the UE reports corresponding HARQ-ACK information in the PUCCH.

The Rel-16 nHARQ-ACK calculation formula for Type-1 codebook can work well when time domain bundling is not enabled. Nevertheless, when time domain bundling is enabled, it cannot indicate the number of valid HARQ-ACK information bits contained in the Type-1 codebook precisely. Therefore, the transmit power of PUCCH transmission conveying the Type-1 codebook, set based on the calculated nHARQ-ACK, may not match the required one exactly.

To resolve the above issue, the following TP2 can be considered, with the modified text in red.

------------------------------------------------------------Start TP2----------------------------------------------------------------

9.1.2.1	Type-1 HARQ-ACK codebook in physical uplink control channel

……

If OACK+OSR+OCSI≤11, the UE determines a number of HARQ-ACK information bits nHARQ-ACK for obtaining a transmission power for a PUCCH, as described in clause 7.2.1, as nHARQ-ACK=c=0NcellsDL-1m=0Mc-1Nm,creceived+c=0NcellsDL-1m=0Mc-1Nm,creceived,CBG where

-	NcellsDL are all DL cells where the UE is configured to receive unicast or multicast PDSCHs

-	Mc is the cardinality for the union of all sets MA,c of occasions for unicast or multicast PDSCH receptions or SPS PDSCH releases for serving cell c

-	if enableTimeDomainHARQ-Bundling is not provided, Nm,creceived is the number of transport blocks the UE receives in PDSCH reception occasion m for serving cell c if harq-ACK-SpatialBundlingPUCCH and PDSCH-CodeBlockGroupTransmission are not provided, or the number of transport blocks the UE receives in PDSCH reception occasion m for serving cell c if PDSCH-CodeBlockGroupTransmission is provided and the PDSCH reception is scheduled by a DCI format that does not support CBG-based PDSCH receptions, or the number of PDSCH receptions if harq-ACK-SpatialBundlingPUCCH is provided or SPS PDSCH release or TCI state update in PDSCH reception occasion m for serving cell c and the UE reports corresponding HARQ-ACK information in the PUCCH.

-	if enableTimeDomainHARQ-Bundling is provided, Nm,creceived is the number of enabled transport blocks in a DCI format scheduling a PDSCH associated with PDSCH reception occasion m for serving cell c if harq-ACK-SpatialBundlingPUCCH is not provided, or the number of DCI formats scheduling a PDSCH associated with PDSCH reception occasion m for serving cell c if harq-ACK-SpatialBundlingPUCCH is provided or SPS PDSCH release or TCI state update in PDSCH reception occasion m for serving cell c and the UE reports corresponding HARQ-ACK information in the PUCCH.

-	Nm,creceived,CBG is the number of CBGs the UE receives in a PDSCH reception occasion m for serving cell c if PDSCH-CodeBlockGroupTransmission is provided and the PDSCH reception is scheduled by a DCI format that supports CBG-based PDSCH receptions and the UE reports corresponding HARQ-ACK information in the PUCCH.

------------------------------------------------------------End TP2----------------------------------------------------------------

In the TP2, a transport block (i.e. corresponding to transport block 1 or transport block 2 field) in a DCI format is regarded as enabled if for the transport block, at least for a PDSCH, which is scheduled by the DCI format and the UE receives in a slot as described in clause 11.1, the corresponding transport block is enabled. The same concept of “enabled transport block” will be used hereafter.

Proposal 9: For multi-PDSCH scheduling and Type-1 codebook, consider the TP2 in TS38.213 for the calculation of nHARQ-ACK.

4.2.2 Type-2 HARQ-ACK codebook

- Time domain bundling operation

During RAN1#107-e meeting, an agreement about time domain bundling for Type-2 codebook has been achieved, listed as below.

Agreement

For multi-PDSCH scheduling with a single DCI

- Introduce a new RRC parameter, e.g., numberOfHARQ-BundlingGroups, to configure the number of HARQ bundling groups with value range {1, 2, 4} for type-2 HARQ-ACK codebook per serving cell.

  - If the RRC parameter is not configured for a serving cell, time domain bundling for type-2 HARQ-ACK codebook is not enabled for the serving cell.

  - The maximum number of PDSCHs allocated to each bundling group is ceil(NPDSCH,MAX/NHBG) where NHBG is the number of bundling groups configured by numberOfHARQ-BundlingGroups for a serving cell and NPDSCH,MAX is the maximum configured number of PDSCHs for the serving cell.

  - The PDSCHs corresponding to [configured or valid] SLIVs in a TDRA row index indicated by multi-PDSCH scheduling DCI are allocated to the bundling groups, e.g., if NHBG =4, NPDSCH,MAX =8, and 5 PDSCHs are scheduled, then 2/1/1/1 PDSCHs are assigned to each group, by reusing CBG grouping method.

    - For a group that is empty or is filled with only invalid PDSCH(s), HARQ-ACK bits for the bundling group is set to NACK (same principle as when no time bundling configured)

    - Logical AND operation is applied to across all valid PDSCHs within the same bundling group to generate 1 HARQ-ACK bit per group, at least for 1-TB case

  - If the number of HARQ bundling groups is configured as 1 for a serving cell, HARQ-ACK bits corresponding to any DCI for the serving cell belong to the first sub-codebook.

  - At least for 1-TB case, if the number of HARQ bundling groups is configured as larger than 1 for a serving cell, HARQ-ACK bits corresponding to multi-PDSCH scheduling case (which implies a multi-PDSCH DCI schedules more than one PDSCH) for the serving cell belong to the second sub-codebook,

    - Where the number of HARQ-ACK bits corresponding to a multi-PDSCH DCI is determined based on the maximum of Q value across all serving cells within the same PUCCH cell group, and Q=maximum configured number of PDSCHs for a cell without numberOfHARQ-BundlingGroups configured or Q=number of configured HARQ bundling groups for a cell with numberOfHARQ-BundlingGroups configured

According to the above agreement, whether division of bundling groups is based on configured SLIVs or valid SLIVs of the indicated TDRA row index has not been determined so far.

Based on comments made by several companies during RAN1#107-e meeting, when division of bundling groups is based on valid SLIVs, valid SLIVs can be divided into different bundling groups more even, therefore number of valid SLIVs divided into any bundling group may be smaller, compared to division based on configured SLIVs. Consequently, division of bundling groups based on valid SLIVs may offer better re-transmission efficiency.

Proposal 10: For multi-PDSCH scheduling, when configured with Type-2 codebook and time domain bundling is enabled, division of bundling groups is based on valid SLIVs of the indicated TDRA row index.

- Calculation of nHARQ-ACK

In [6] the calculation of nHARQ-ACK for the second HARQ-ACK sub-codebook for multi-PDSCH scheduling is missing. Furthermore, when time domain bundling is enabled for a serving cell, and the number of bundling groups is 1, then HARQ-ACK for PDSCHs scheduled by a DCI scheduling more than one PDSCH will be bundled and contained in the first HARQ-ACK sub-codebook. Then the calculation of nHARQ-ACK for the first HARQ-ACK sub-codebook should also be adjusted to accommodate this case.

To address above issues, the following TP3 can be considered, with the modified text in red.

------------------------------------------------------------Start TP3----------------------------------------------------------------

9.1.3.1	Type-2 HARQ-ACK codebook in physical uplink control channel

……

If a UE is not provided PDSCH-CodeBlockGroupTransmission for any serving cells,

or for PDSCH receptions scheduled by a DCI format scheduling more than one PDSCH reception for a serving cell provided numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max=1,

or for PDSCH receptions scheduled by a DCI format that does not support CBG-based PDSCH receptions and schedules only a single PDSCH reception,

or for SPS PDSCH reception,

or for a DCI format having associated HARQ-ACK information without scheduling PDSCH reception,

and if OACK+OSR+OCSI≤11, the UE determines a number of HARQ-ACK information bits nHARQ-ACK for obtaining a transmission power for a PUCCH, as described in clause 7.2.1, as

nHARQ-ACK=nHARQ-ACK,TB=VDAI,mlastDL-c=0NcellsDL-1UDAI,cmodTDNTB,maxDL+c=0NcellsDL-1m=0M-1Nm,creceived+NSPS,c+g=0G-1VDAI,mlast,gDL-c=0Ncells,gDL-1UDAI,c,gmodTD,gNTB,max,gDL+c=0Ncells,gDL-1m=0Mg-1Nm,c,greceived+NSPS,c,g

where

-	NcellsDL is a number of serving cells where the UE is configured to receive unicast PDSCHs

-	Ncells,gDL is a number of serving cells where the UE is configured to receive multicast PDSCHs for a G-RNTI g or a G-CS-RNTI g

-	G is a total number of G-RNTIs or G-CS-RNTIs configured to the UE

-	M is the number of PDCCH monitoring occasions for unicast DCI formats

-	Mg is the number of PDCCH monitoring occasions for multicast DCI formats with CRC scrambled by G-RNTI g or G-CS-RNTI g

-	TD=2NC-DAIDL where NC-DAIDL the number of bits for the counter DAI field in unicast DCI formats

-	TD,g=2NC-DAI,gDL where NC-DAI,gDL the number of bits for the counter DAI field in multicast DCI formats with CRC scrambled by G-RNTI g or G-CS-RNTI g

-	if NcellsDL=1, VDAI,mlastDL is the value of the counter DAI in the last DCI format scheduling PDSCH reception or having associated HARQ-ACK information without scheduling PDSCH reception, that the UE detects within the M PDCCH monitoring occasions.

-	if Ncells,gDL=1, VDAI,mlast,gDL is the value of the counter DAI in the last multicast DCI format with G-RNTI g, or G-CS-RNTI g, scheduling PDSCH reception or having associated HARQ-ACK information without scheduling a PDSCH reception, that the UE detects within the Mg PDCCH monitoring occasions

-	if NcellsDL>1 or if Ncells,gDL>1

-	if the UE does not detect any DCI format that includes a total DAI field in a last PDCCH monitoring occasion within the M or Mg PDCCH monitoring occasions where the UE detects at least one DCI format scheduling PDSCH reception, or having associated HARQ-ACK information without scheduling PDSCH reception, for any serving cell c, VDAI,mlastDL or VDAI,mlast,gDL, respectively, is the value of the counter DAI in a last DCI format the UE detects in the last PDCCH monitoring occasion

-	if the UE detects at least one DCI format that includes a total DAI field in a last PDCCH monitoring occasion within the M or Mg, for G-RNTI g or G-CS-RNTI g, PDCCH monitoring occasions where the UE detects at least one DCI format scheduling PDSCH reception, or having associated HARQ-ACK information without scheduling PDSCH reception, for any serving cell c, VDAI,mlastDL or VDAI,mlast,gDL, respectively, is the value of the total DAI in the at least one DCI format that includes a total DAI field

-	VDAI,mlastDL=0 or VDAI,mlast,gDL=0 if the UE does not detect any DCI format scheduling PDSCH reception, or having associated HARQ-ACK information without scheduling PDSCH reception,  for any serving cell c in any of the M or Mg PDCCH monitoring occasions, respectively.

-	UDAI,c or UDAI,c,g, for G-RNTI g or G-CS-RNTI g, is the total number of DCI formats scheduling PDSCH receptions, or having associated HARQ-ACK information without scheduling a PDSCH reception, that the UE detects within the M or Mg PDCCH monitoring occasions, respectively, for serving cell c. UDAI,c=0 or UDAI,c,g=0 if the UE does not detect any DCI format scheduling PDSCH reception, or having associated HARQ-ACK information without scheduling PDSCH reception, for serving cell c in any of the M or Mg, respectively,PDCCH monitoring occasions.

-	NTB,maxDL=2 if the value of maxNrofCodeWordsScheduledByDCI is 2 for any serving cell c and harq-ACK-SpatialBundlingPUCCH is not provided; otherwise, NTB,maxDL=1.

-	NTB,max,gDL=1.

-	if the UE is not provided numberOfHARQ-BundlingGroups for serving cell c, or provided numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max>1 for serving cell c, Nm,creceived or Nm,c,greceived, for G-RNTI g or G-CS-RNTI g, is the number of transport blocks the UE receives in a PDSCH scheduled by a DCI format that the UE detects in PDCCH monitoring occasion m for serving cell c if harq-ACK-SpatialBundlingPUCCH is not provided, or the number of PDSCH scheduled by a DCI format that the UE detects in PDCCH monitoring occasion m for serving cell c if harq-ACK-SpatialBundlingPUCCH is provided, or the number of DCI formats that the UE detects and have associated a HARQ-ACK information without scheduling PDSCH reception in PDCCH monitoring occasion m for serving cell c.

-	if the UE is provided numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max=1 for serving cell c, Nm,creceived or Nm,c,greceived, for G-RNTI g or G-CS-RNTI g, is the number of enabled transport blocks in a DCI format that the UE detects in PDCCH monitoring occasion m for serving cell c if harq-ACK-SpatialBundlingPUCCH is not provided, or the number of DCI formats that the UE detects in PDCCH monitoring occasion m for serving cell c if harq-ACK-SpatialBundlingPUCCH is provided, or the number of DCI format that the UE detects and have associated a HARQ-ACK information without scheduling PDSCH reception in PDCCH monitoring occasion m for serving cell c.

-	NSPS,c or NSPS,c,g, for G-RNTI g or G-CS-RNTI g, is the number of SPS PDSCH receptions by the UE on serving cell c for which the UE transmits corresponding HARQ-ACK information in the same PUCCH as for HARQ-ACK information corresponding to PDSCH receptions within the M or Mg PDCCH monitoring occasions, respectively.

……

If a UE is not provided numberOfHARQ-BundlingGroups, detects a first DCI format scheduling one PDSCH reception or having associated HARQ-ACK information without scheduling a PDSCH reception, if any, and a second DCI format scheduling more than one PDSCH reception on a serving cell from the NcellsDL serving cells, if any, and the UE would provide corresponding HARQ-ACK information in a same PUCCH, the UE determines the o0ACK, o1ACK,⋯,oOACK-1ACK according to the previous pseudo-code with the following modifications

-	the UE determines a first HARQ-ACK sub-codebook based on each detected DCI format scheduling one PDSCH reception or having associated HARQ-ACK information without scheduling a PDSCH reception, or SPS PDSCH receptions, if any, and

-	the UE determines a second HARQ-ACK sub-codebook based on each detected DCI format scheduling more than one PDSCH reception, and

-	instead of generating one HARQ-ACK information bit per transport block for serving cell c, the UE generates NHARQ-ACKmax HARQ-ACK information bits where NHARQ-ACKmax is the maximum value of NTB,cDL⋅NPDSCH,cmax across all NcellsDL serving cells, NPDSCH,cmax is a maximum number of PDSCH receptions that can be scheduled by a DCI format on serving cell c as described in [6, TS 38.214], and NTB,cDL is the value of maxNrofCodeWordsScheduledByDCI for serving cell c. The UE generates the NHARQ-ACKmax HARQ-ACK information bits in ascending order of the NPDSCH,cmax PDSCHs, including any PDSCH that the UE does not receive in a slot as described in clause 11.1. If, for serving cell c, the UE detects a DCI format that schedules NPDSCH,c PDSCH receptions and NTB,cDL⋅NPDSCH,c<NHARQ-ACKmax , the UE generates NACK for the last NHARQ-ACKmax -NTB,cDL⋅NPDSCH,c HARQ-ACK information bits

-	The pseudo-code operation when PDSCH-CodeBlockGroupTransmission is provided is not applicable.

-	The counter DAI value and the total DAI value apply separately for each HARQ-ACK sub-codebook.

-	The UE generates the HARQ-ACK codebook by appending the second HARQ-ACK sub-codebook to the first HARQ-ACK sub-codebook.

If OACK+OSR+OCSI≤11, the UE also determines nHARQ-ACK=nHARQ-ACK,TB+nHARQ-ACK,multi1 for obtaining a PUCCH transmission power, as described in clause 7.2.1, with

nHARQ-ACK,multi1=VDAI,mlastDL-c=0NcellsDL,multi-1UDAI,cmultimodTDNHARQ-ACKmax+c=0NcellsDL,multi-1m=0M-1Nm,creceived,multi

where

-	if NcellsDL=1, VDAI,mlastDL is the value of the counter DAI in the last DCI format scheduling more than one PDSCH reception for any serving cell c that the UE detects within the M PDCCH monitoring occasions

-	if NcellsDL>1, VDAI,mlastDL is the value of the total DAI in the last DCI format scheduling more than one PDSCH reception for any serving cell c that the UE detects within the M PDCCH monitoring occasions

-	VDAI,mlastDL=0, if the UE does not detect any DCI format scheduling more than one PDSCH reception for any serving cell c in any of the M PDCCH monitoring occasions

-	NcellsDL,multi is the number of serving cells each of which is provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH in NcellsDL serving cells

-	UDAI,cmulti is the total number of DCI formats scheduling more than one PDSCH reception that the UE detects within the M PDCCH monitoring occasions for serving cell c. UDAI,cmulti=0 if the UE does not detect any DCI format scheduling more than one PDSCH reception for serving cell c in any of the M PDCCH monitoring occasions

-	Nm,creceived,multi is the number of transport blocks the UE receives in all PDSCHs scheduled by a DCI format scheduling more than one PDSCH reception that the UE detects in PDCCH monitoring occasion m for serving cell c if harq-ACK-SpatialBundlingPUCCH is not provided, or the number of PDSCHs, excluding any PDSCH that the UE does not receive in a slot as described in clause 11.1, scheduled by a DCI format scheduling more than one PDSCH reception that the UE detects in PDCCH monitoring occasion m for serving cell c if harq-ACK-SpatialBundlingPUCCH is provided and the UE reports corresponding HARQ-ACK information in the PUCCH

If a UE is provided numberOfHARQ-BundlingGroups for a serving cell c, the UE generates HARQ-ACK information over transport block groups (TBGs) for PDSCH receptions where, for a maximum number of NPDSCHmax PDSCH receptions scheduled by a DCI format on the serving cell, a maximum number of TBGs NHARQ-ACK,cTBG,max is provided by numberOfHARQ-BundlingGroups. If the UE detects a DCI format scheduling NPDSCH,c PDSCH receptions on the serving cell c, the UE generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for first TBs and, if applicable, generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for second TBs in the NPDSCH PDSCH receptions as described in clause 9.1.1 by setting NHARQ-ACKCBG/TB,max=NHARQ-ACK,cTBG,max and C=NPDSCH,c.

If a UE

-	is provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH and, if provided, numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max>1 for NcellsDL,TBG serving cells; and

-	is not provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH or is provided numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max=1, for NcellsDL,TB serving cells where NcellsDL,TB+NcellsDL,TBG=NcellsDL

the UE determines the o0ACK, o1ACK,⋯,oOACK-1ACK according to the previous pseudo-code with the following modifications

-	NcellsDL is used for the determination of a first HARQ-ACK sub-codebook for

-	SPS PDSCH reception,

-	any DCI format having associated HARQ-ACK information without scheduling PDSCH reception, and

-	PDSCH reception scheduled by a DCI format scheduling one PDSCH

-	PDSCH reception with NHARQ-ACKTBG,max=1 for TBG-based HARQ-ACK information on the NcellsDL,TB serving cells,

-	NcellsDL is replaced by NcellsDL,TBG for the determination of a second HARQ-ACK sub-codebook corresponding to the NcellsDL,TBG serving cells for TBG-based HARQ-ACK information, or for TB-based HARQ-ACK information corresponding to multiple PDSCH receptions scheduled by a single DCI format, and

-	if, for an active DL BWP of a serving cell, the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs, and is provided ackNackFeedbackMode = joint, the serving cell is counted as two times where the first time corresponds to the first CORESETs and the second time corresponds to the second CORESETs, and

-	instead of generating one HARQ-ACK information bit per transport block for a serving cell from the NcellsDL,TBG serving cells, the UE generates NHARQ-ACK,maxTBG,max HARQ-ACK information bits, where NHARQ-ACK,maxTBG,max is the maximum value between NTB,cDL⋅NHARQ-ACK,cTBG,max across all NcellsDL,TBG serving cells if the UE is provided numberOfHARQ-BundlingGroups, and NTB,cDL⋅NPDSCH,cmax across all NcellsDL,TBG serving cells where the UE is not provided numberOfHARQ-BundlingGroups, and NTB,cDL is the value of maxNrofCodeWordsScheduledByDCI for serving cell c. If for a serving cell c where the UE is provided numberOfHARQ-BundlingGroups, it is NTB,cDL⋅NHARQ-ACK,cTBG,max<NHARQ-ACK,maxTBG,max, the UE generates NACK for the last NHARQ-ACK,maxTBG,max-NTB,cDL⋅NHARQ-ACK,cTBG,max HARQ-ACK information bits for serving cell c. If for a serving cell c where the UE is not provided numberOfHARQ-BundlingGroups, it is NTB,cDL⋅NPDSCH,cmax <NHARQ-ACK,maxTBG,max, the UE generates NACK for the last NHARQ-ACK,maxTBG,max-NTB,cDL⋅NPDSCH,cmax  HARQ-ACK information bits for serving cell c.

-	The pseudo-code operation when PDSCH-CodeBlockGroupTransmission is provided is not applicable.

-	The counter DAI value and the total DAI value apply separately for each HARQ-ACK sub-codebook.

-	The UE generates the HARQ-ACK codebook by appending the second HARQ-ACK sub-codebook to the first HARQ-ACK sub-codebook.

If OACK+OSR+OCSI≤11, the UE also determines nHARQ-ACK=nHARQ-ACK,TB+nHARQ-ACK,multi2 for obtaining a PUCCH transmission power, as described in clause 7.2.1, with

nHARQ-ACK,multi2=VDAI,mlastDL-c=0NcellsDL,TBG-1UDAI,cmultimodTDNHARQ-ACK,maxTBG,max+c=0NcellsDL,TBG-1m=0M-1Nm,creceived,multi

where

-	if NcellsDL=1, VDAI,mlastDL is the value of the counter DAI in the last DCI format scheduling more than one PDSCH reception for any serving cell c provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH and, if provided, numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max>1, that the UE detects within the M PDCCH monitoring occasions

-	if NcellsDL>1, VDAI,mlastDL is the value of the total DAI in the last DCI format scheduling more than one PDSCH reception for any serving cell c provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH and, if provided, numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max>1, that the UE detects within the M PDCCH monitoring occasions

-	VDAI,mlastDL=0, if the UE does not detect any DCI format scheduling more than one PDSCH reception for any serving cell c provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH and, if provided, numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max>1, in any of the M PDCCH monitoring occasions

-	UDAI,cmulti is the total number of DCI formats scheduling more than one PDSCH reception that the UE detects within the M PDCCH monitoring occasions for serving cell c provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH and, if provided, numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max>1. UDAI,cmulti=0 if the UE does not detect any DCI format scheduling more than one PDSCH reception for serving cell c provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH and, if provided, numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max>1, in any of the M PDCCH monitoring occasions

-	if harq-ACK-SpatialBundlingPUCCH is not provided, Nm,creceived,multi is the number of transport blocks the UE receives in all PDSCHs scheduled by a DCI format scheduling more than one PDSCH reception that the UE detects in PDCCH monitoring occasion m for serving cell c provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH and not provided numberOfHARQ-BundlingGroups, or the number of bundling groups, each of which corresponds to an enabled transport block in a DCI format scheduling more than one PDSCH reception that the UE detects in PDCCH monitoring occasion m for serving cell c provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH and provided numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max>1, and contains at least a PDSCH that the UE receives in a slot as described in clause 11.1, and the UE reports corresponding HARQ-ACK information in the PUCCH

-	if harq-ACK-SpatialBundlingPUCCH is provided, Nm,creceived,multi is the number of PDSCHs, excluding any PDSCH that the UE does not receive in a slot as described in clause 11.1, scheduled by a DCI format scheduling more than one PDSCH reception that the UE detects in PDCCH monitoring occasion m for serving cell c provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH and not provided numberOfHARQ-BundlingGroups, or the number of bundling groups, each of which contains at least a PDSCH that is scheduled by a DCI format scheduling more than one PDSCH reception that the UE detects in PDCCH monitoring occasion m for serving cell c provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH and provided numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max>1, and the UE receives in a slot as described in clause 11.1, and the UE reports corresponding HARQ-ACK information in the PUCCH

------------------------------------------------------------End TP3----------------------------------------------------------------

Proposal 11: For multi-PDSCH scheduling and Type-2 codebook, consider the TP3 in TS38.213 for the calculation of nHARQ-ACK.

- Spatial bundling operation

In [6] Type-2 HARQ-ACK codebook determination for multi-PDSCH scheduling does not consider the case when spatial bundling is configured, therefore the specification may be incomplete.

To resolve this issue, the following TP4 can be considered, with the modified text in red.

------------------------------------------------------------Start TP4---------------------------------------------------------------

9.1.3.1	Type-2 HARQ-ACK codebook in physical uplink control channel

……

If a UE is not provided numberOfHARQ-BundlingGroups, detects a first DCI format scheduling one PDSCH reception or having associated HARQ-ACK information without scheduling a PDSCH reception, if any, and a second DCI format scheduling more than one PDSCH reception on a serving cell from the NcellsDL serving cells, if any, and the UE would provide corresponding HARQ-ACK information in a same PUCCH, the UE determines the o0ACK, o1ACK,⋯,oOACK-1ACK according to the previous pseudo-code with the following modifications

-	the UE determines a first HARQ-ACK sub-codebook based on each detected DCI format scheduling one PDSCH reception or having associated HARQ-ACK information without scheduling a PDSCH reception, or SPS PDSCH receptions, if any, and

-	the UE determines a second HARQ-ACK sub-codebook based on each detected DCI format scheduling more than one PDSCH reception, and

-	instead of generating one HARQ-ACK information bit per transport block for serving cell c, the UE generates NHARQ-ACKmax HARQ-ACK information bits where NHARQ-ACKmax is the maximum value of NTB,cDL⋅NPDSCH,cmax across all NcellsDL serving cells, NPDSCH,cmax is a maximum number of PDSCH receptions that can be scheduled by a DCI format on serving cell c as described in [6, TS 38.214], and NTB,maxDL=2 if the value of maxNrofCodeWordsScheduledByDCI is 2 for serving cell c and harq-ACK-SpatialBundlingPUCCH is not provided, or NTB,maxDL=1 otherwise NTB,cDL is the value of maxNrofCodeWordsScheduledByDCI for serving cell c. The UE generates the NHARQ-ACKmax HARQ-ACK information bits in ascending order of the NPDSCH,cmax PDSCHs, including any PDSCH that the UE does not receive in a slot as described in clause 11.1. If, for serving cell c, the UE detects a DCI format that schedules NPDSCH,c PDSCH receptions and NTB,cDL⋅NPDSCH,c<NHARQ-ACKmax , the UE generates NACK for the last NHARQ-ACKmax -NTB,cDL⋅NPDSCH,c HARQ-ACK information bits

……

If a UE

-	is provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH and, if provided, numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max>1 for NcellsDL,TBG serving cells; and

-	is not provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH or is provided numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max=1, for NcellsDL,TB serving cells where NcellsDL,TB+NcellsDL,TBG=NcellsDL

the UE determines the o0ACK, o1ACK,⋯,oOACK-1ACK according to the previous pseudo-code with the following modifications

-	NcellsDL is used for the determination of a first HARQ-ACK sub-codebook for

-	SPS PDSCH reception,

-	any DCI format having associated HARQ-ACK information without scheduling PDSCH reception, and

-	PDSCH reception scheduled by a DCI format scheduling one PDSCH

-	PDSCH reception with NHARQ-ACKTBG,max=1 for TBG-based HARQ-ACK information on the NcellsDL,TB serving cells,

-	NcellsDL is replaced by NcellsDL,TBG for the determination of a second HARQ-ACK sub-codebook corresponding to the NcellsDL,TBG serving cells for TBG-based HARQ-ACK information, or for TB-based HARQ-ACK information corresponding to multiple PDSCH receptions scheduled by a single DCI format, and

-	if, for an active DL BWP of a serving cell, the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs, and is provided ackNackFeedbackMode = joint, the serving cell is counted as two times where the first time corresponds to the first CORESETs and the second time corresponds to the second CORESETs, and

-	instead of generating one HARQ-ACK information bit per transport block for a serving cell from the NcellsDL,TBG serving cells, the UE generates NHARQ-ACK,maxTBG,max HARQ-ACK information bits, where NHARQ-ACK,maxTBG,max is the maximum value between NTB,cDL⋅NHARQ-ACK,cTBG,max across all NcellsDL,TBG serving cells if the UE is provided numberOfHARQ-BundlingGroups, and NTB,cDL⋅NPDSCH,cmax across all NcellsDL,TBG serving cells where the UE is not provided numberOfHARQ-BundlingGroups, and NTB,maxDL=2 if the value of maxNrofCodeWordsScheduledByDCI is 2 for serving cell c and harq-ACK-SpatialBundlingPUCCH is not provided, or NTB,maxDL=1 otherwise NTB,cDL is the value of maxNrofCodeWordsScheduledByDCI for serving cell c. If for a serving cell c where the UE is provided numberOfHARQ-BundlingGroups, it is NTB,cDL⋅NHARQ-ACK,cTBG,max<NHARQ-ACK,maxTBG,max, the UE generates NACK for the last NHARQ-ACK,maxTBG,max-NTB,cDL⋅NHARQ-ACK,cTBG,max HARQ-ACK information bits for serving cell c. If for a serving cell c where the UE is not provided numberOfHARQ-BundlingGroups, it is NTB,cDL⋅NPDSCH,cmax <NHARQ-ACK,maxTBG,max, the UE generates NACK for the last NHARQ-ACK,maxTBG,max-NTB,cDL⋅NPDSCH,cmax  HARQ-ACK information bits for serving cell c.

-	The pseudo-code operation when PDSCH-CodeBlockGroupTransmission is provided is not applicable.

------------------------------------------------------------End TP4----------------------------------------------------------------

Proposal 12: For multi-PDSCH scheduling and Type-2 codebook, consider the TP4 in TS38.213 for the case when spatial bundling is configured.

- Conclusion

In this contribution, we first discuss the DMRS bundling across the multiple PUSCHs and TRS enhancements, then the multi-PDSCH/PUSCH scheduling and feedback related design. We have the following observations and proposals.

- DMRS bundling across the multiple PUSCHs

Proposal 1: Closely monitor the progress on DMRS bundling in Rel-17 Coverage enhancement WI, especially the support on different TBs over multiple slots. Only when this is supported, it is possible to apply DMRS bundling for NR operation in FR2-2.

- TRS enhancements

Proposal 2: The timing error issue due to smaller SCS of SSB than that of data transmission can be resolved by gNB implementation, e.g., gNB transmits an NCD SSB of 960 kHz SCS and indicates UE to measure it, and no specification impact shall be introduced.

- DCI design for multi-PDSCH/PUSCH scheduling

Proposal 3: For activation/de-activation of SPS/CG by using multi-PDSCH/PUSCH scheduling DCI, the first (valid) SLIV in the row indicated by an activation/de-activation DCI is used for determining SPS/CG occasions.

Observation 1: The case where two DCIs, each of which schedules a multi-slot PDSCH (or multi-slot PUSCH), end in the same symbol but the two scheduled multi-slot PDSCHs (or multi-slot PUSCHs) have overlapping spans, where the span is defined from the beginning of the first repetition till the end of the last repetition for a PDSCH/PUSCH, is allowed in Rel-15/16.

Proposal 4: For multi-PDSCH/PUSCH scheduling, DCI-to-PDSCH/PUSCH out-of-order scheduling is defined as NR Rel-15/16 without any exception.

Proposal 5: For multi-PDSCH scheduling, UE does not expect any of the scheduled PDSCH(s)/SPS PDSCH and the PUCCH resource for the HARQ-ACK transmission to lead to out-of-order HARQ-ACK reporting.

Proposal 6: Regarding TB disabling for multi-PDSCH scheduling, when two codeword transmission is configured, for a DCI format scheduling more than one PDSCH, a given TB can be disabled for each scheduled PDSCH individually, by setting IMCS = 26 and the 1-bit RV for a scheduled PDSCH to a predefined value, e.g. ‘1’, to indicated the given TB for the scheduled PDSCH is disabled.

- HARQ-ACK feedback for multi-PDSCH scheduling

Proposal 7: For multi-PDSCH scheduling, when configured with Type-1 codebook and time domain bundling is enabled, UE does not expect more than one TDRA rows mapped to a same candidate PDSCH reception occasion is actually scheduled.

Proposal 8: For multi-PDSCH scheduling, when configured with Type-1 codebook and time domain bundling is enabled, consider the TP1 in TS38.213 for the operation of excluding TDRA row(s) based on semi-static UL symbol(s).

Proposal 9: For multi-PDSCH scheduling and Type-1 codebook, consider the TP2 in TS38.213 for the calculation of nHARQ-ACK.

Proposal 10: For multi-PDSCH scheduling, when configured with Type-2 codebook and time domain bundling is enabled, division of bundling groups is based on valid SLIVs of the indicated TDRA row index.

Proposal 11: For multi-PDSCH scheduling and Type-2 codebook, consider the TP3 in TS38.213 for the calculation of nHARQ-ACK.

Proposal 12: For multi-PDSCH scheduling and Type-2 codebook, consider the TP4 in TS38.213 for the case when spatial bundling is configured.

References

- R1-2112523, Discussion summary #1 of [107-e-NR-52-71GHz-05], Moderator (vivo), RAN WG1 #107-e, e-Meeting, November 11th – 19th, 2021.

- R1-2112207, PDSCH/PUSCH enhancements for NR in 52.6 to 71GHz band, Qualcomm Incorporated, RAN WG1 #107-e, e-Meeting, November 11th – 19th, 2021.

- TS 38.212, Multiplexing and channel coding, v16.8.0, Dec. 2021.

- TS 38.214, Physical layer procedures for data, v16.8.0, Dec. 2021.

- R1-2112714, Summary #4 of PDSCH/PUSCH enhancements (Scheduling/HARQ), Moderator (LG Electronics), RAN WG1 #107-e, e-Meeting, November 11th – 19th, 2021.

- TS 38.213, Physical layer procedures for control, v17.0.0, Dec. 2021.