3GPP TSG RAN WG1 #107bis-e                                              R1-2200116
e-Meeting, January 17th – 25th, 2022

Source: 	ZTE Corporation

Title: 	Discussion on RRC parameters for coverage enhancement

Agenda item:	8.8.4

Document for:	Discussion/Decision

Introduction

In this contribution, our view on RRC parameters for NR coverage enhancement is provided. The discussion is based on the outcome of the discussion in RAN1#107-e [1][2].

Discussion

- Indication of the number of repetitions for Msg3

In RAN1#106bis-e, the following RRC parameter for the number of repetitions for Msg3 PUSCH repetition is agreed [2].

| WI code | Sub-feature group | Parameter name in the spec | New or existing? | Description | Value range | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_cov_enh-Core | Type A PUSCH repetitions for Msg3 | numberOfMsg3Repetitions | new | The number of repetitions for Msg3 PUSCH repetition, including Msg3 initial transmission and Msg3 re-transmission. | FFS | FFS | Cell-specific | 38.331 | Working Assumption Down-select only one from the following methods for indication of the number of repetitions of Msg3 initial transmission. - Alt 1: If TDRA information field is chosen, Option 2 is supported. - The candidate values for repetition factor could be chosen from {[1], 2, 3, 4, 7, 8, [12], [16]} - Alt 2: If MCS information field is chosen, repurpose the MCS information field as follows. - 2 MSB bits of the MCS information field are used for selecting one repetition factor from a SIB1 configured set with 4 candidate values. - The set of candidate values for repetition factor could be chosen from {[1], 2, 3, 4, 7, 8, [12], [16]} Note: Whether ‘1’ is included depends on the outcome of interpretation of the selected information field. |

- Based on agreements in RAN1#107-e, the 4 candidate repetition factors can be configured by SIB1 and the default values if not configured is {1, 2, 3, 4}. In addition, the same mechanism of Msg3 PUSCH repetition is also applied to CFRA PUSCH with repetitions. The main remaining issue is the candidate values for repetition when configured by SIB1. According to the discussion in [2], we think a set with (1,2, 3, 4, 7, 8, 12) is a good middle-ground among companies. Therefore, we propose the following revisions for the RRC parameter numberOfMsg3Repetitions

- Proposal 1: Adopt the following revisions for the RRC parameter numberOfMsg3Repetitions.

| WI code | Sub-feature group | Parameter name in the spec | New or existing? | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_cov_enh-Core | Type A PUSCH repetitions for Msg3 | numberOfMsg3Repetitions | new | The number of repetitions for PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI Msg3 PUSCH repetition, including Msg3 initial transmission and Msg3 re-transmission. | SEQUENCE (SIZE (4)) OF INTEGER (1,2, 3, 4, 7, 8, 12 ) | {1, 2, 3, 4} | RACH-ConfigCommon | Cell-specific | 38.331 | Working Assumption Down-select only one from the following methods for indication of the number of repetitions of Msg3 initial transmission. - Alt 1: If TDRA information field is chosen, Option 2 is supported. - The candidate values for repetition factor could be chosen from {[1], 2, 3, 4, 7, 8, [12], [16]} - Alt 2: If MCS information field is chosen, repurpose the MCS information field as follows. - 2 MSB bits of the MCS information field are used for selecting one repetition factor from a SIB1 configured set with 4 candidate values. - The set of candidate values for repetition factor could be chosen from {[1], 2, 3, 4, 7, 8, [12], [16]} Note: Whether ‘1’ is included depends on the outcome of interpretation of the selected information field. Agreements - For indication of the number of repetitions of Msg3 initial transmission, Alt 2 (i.e., using MCS information field) is adopted. - Four candidate MCS indexes can be configured by SIB1 for Msg3 initial transmission. MCS 0~3 are applied if the configuration is absent. If the four candidate repetition factors are not configured, the default values are {1, 2, 3, 4}. Agreement For repetition indication for Msg3 re-transmission, Option 1 (i.e., use the same mechanism as supported for Msg3 initial transmission) is adopted. FFS: [8] MCS index to be used for Msg3 re-transmission Working assumption : support repetition for a PUSCH scheduled by RAR UL grant, including both Msg3 PUSCH and CFRA PUSCH. - Use the same mechanism of Msg3 PUSCH repetition, when applicable, for CFRA PUSCH with repetitions. - No separate CFRA preamble/RO for repetition of CFRA PUSCH is introduced. - No additional optimization specific for CFRA PUSCH is considered for CFRA PUSCH with repetition. - No additional RAN1 specification impact Note: UE reports Msg3 repetition capability after initial access. Note: The working assumption can be confirmed only if no additional RAN1 specification impact nor optimization specific for CFRA PUSCH. |

- Indication of the MCS index for Msg3 repetition

In RAN1#107-e, it was agreed to configure the candidate MCS indexes for Msg3 initial transmission by RRC, and FFS whether/how to introduce it for re-transmission. Based on the discussion in our companion contribution [3], MCS index 0~7 can be always used for Msg3 re-transmission, i.e., there is no need to configure MCS index for Msg3 re-transmission by RRC. In addition, the same mechanism can also be applied to CFRA PUSCH with repetition. Therefore, we have the following proposal.

- Proposal 2: Introduce the following the RRC parameter for indication of the MCS index for Msg3/CFRA repetition .

| WI code | Sub-feature group | Parameter name in the spec | New or existing? | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_cov_enh-Core | Type A PUSCH repetitions for Msg3 | mcs-Msg3Repetition | new | Configure four candidate MCS indexes for PUSCH transmission scheduled by RAR UL grant. | SEQUENCE (SIZE (4)) OF INTEGER (0..27) | {0, 1, 2, 3} | RACH-ConfigCommon | Cell-specific | 38.331 | Agreements - For indication of the number of repetitions of Msg3 initial transmission, Alt 2 (i.e., using MCS information field) is adopted. - Four candidate MCS indexes can be configured by SIB1 for Msg3 initial transmission. MCS 0~3 are applied if the configuration is absent. If the four candidate repetition factors are not configured, the default values are {1, 2, 3, 4}. Agreement For repetition indication for Msg3 re-transmission, Option 1 (i.e., use the same mechanism as supported for Msg3 initial transmission) is adopted. FFS: [8] MCS index to be used for Msg3 re-transmission Working assumption : support repetition for a PUSCH scheduled by RAR UL grant, including both Msg3 PUSCH and CFRA PUSCH. - Use the same mechanism of Msg3 PUSCH repetition, when applicable, for CFRA PUSCH with repetitions. - No separate CFRA preamble/RO for repetition of CFRA PUSCH is introduced. - No additional optimization specific for CFRA PUSCH is considered for CFRA PUSCH with repetition. - No additional RAN1 specification impact Note: UE reports Msg3 repetition capability after initial access. Note: The working assumption can be confirmed only if no additional RAN1 specification impact nor optimization specific for CFRA PUSCH. |

Conclusion

According to the analysis given above, we have the following proposals:

- Proposal 1: Adopt the following revisions for the RRC parameter numberOfMsg3Repetitions.

| WI code | Sub-feature group | Parameter name in the spec | New or existing? | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_cov_enh-Core | Type A PUSCH repetitions for Msg3 | numberOfMsg3Repetitions | new | The number of repetitions for PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI Msg3 PUSCH repetition, including Msg3 initial transmission and Msg3 re-transmission. | SEQUENCE (SIZE (4)) OF INTEGER (1,2, 3, 4, 7, 8, 12 ) | {1, 2, 3, 4} | RACH-ConfigCommon | Cell-specific | 38.331 | Working Assumption Down-select only one from the following methods for indication of the number of repetitions of Msg3 initial transmission. - Alt 1: If TDRA information field is chosen, Option 2 is supported. - The candidate values for repetition factor could be chosen from {[1], 2, 3, 4, 7, 8, [12], [16]} - Alt 2: If MCS information field is chosen, repurpose the MCS information field as follows. - 2 MSB bits of the MCS information field are used for selecting one repetition factor from a SIB1 configured set with 4 candidate values. - The set of candidate values for repetition factor could be chosen from {[1], 2, 3, 4, 7, 8, [12], [16]} Note: Whether ‘1’ is included depends on the outcome of interpretation of the selected information field. Agreements - For indication of the number of repetitions of Msg3 initial transmission, Alt 2 (i.e., using MCS information field) is adopted. - Four candidate MCS indexes can be configured by SIB1 for Msg3 initial transmission. MCS 0~3 are applied if the configuration is absent. If the four candidate repetition factors are not configured, the default values are {1, 2, 3, 4}. Agreement For repetition indication for Msg3 re-transmission, Option 1 (i.e., use the same mechanism as supported for Msg3 initial transmission) is adopted. FFS: [8] MCS index to be used for Msg3 re-transmission Working assumption : support repetition for a PUSCH scheduled by RAR UL grant, including both Msg3 PUSCH and CFRA PUSCH. - Use the same mechanism of Msg3 PUSCH repetition, when applicable, for CFRA PUSCH with repetitions. - No separate CFRA preamble/RO for repetition of CFRA PUSCH is introduced. - No additional optimization specific for CFRA PUSCH is considered for CFRA PUSCH with repetition. - No additional RAN1 specification impact Note: UE reports Msg3 repetition capability after initial access. Note: The working assumption can be confirmed only if no additional RAN1 specification impact nor optimization specific for CFRA PUSCH. |

- Proposal 2: Introduce the following the RRC parameter for indication of the MCS index for Msg3/CFRA repetition .

| WI code | Sub-feature group | Parameter name in the spec | New or existing? | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_cov_enh-Core | Type A PUSCH repetitions for Msg3 | mcs-Msg3Repetition | new | Configure four candidate MCS indexes for PUSCH transmission scheduled by RAR UL grant. | SEQUENCE (SIZE (4)) OF INTEGER (0..27) | {0, 1, 2, 3} | RACH-ConfigCommon | Cell-specific | 38.331 | Agreements - For indication of the number of repetitions of Msg3 initial transmission, Alt 2 (i.e., using MCS information field) is adopted. - Four candidate MCS indexes can be configured by SIB1 for Msg3 initial transmission. MCS 0~3 are applied if the configuration is absent. If the four candidate repetition factors are not configured, the default values are {1, 2, 3, 4}. Agreement For repetition indication for Msg3 re-transmission, Option 1 (i.e., use the same mechanism as supported for Msg3 initial transmission) is adopted. FFS: [8] MCS index to be used for Msg3 re-transmission Working assumption : support repetition for a PUSCH scheduled by RAR UL grant, including both Msg3 PUSCH and CFRA PUSCH. - Use the same mechanism of Msg3 PUSCH repetition, when applicable, for CFRA PUSCH with repetitions. - No separate CFRA preamble/RO for repetition of CFRA PUSCH is introduced. - No additional optimization specific for CFRA PUSCH is considered for CFRA PUSCH with repetition. - No additional RAN1 specification impact Note: UE reports Msg3 repetition capability after initial access. Note: The working assumption can be confirmed only if no additional RAN1 specification impact nor optimization specific for CFRA PUSCH. |

Reference

3GPP RAN1#106bis-e,  R1-2112976, Consolidated higher layers parameter list for Rel-17 NR, Moderator (Ericsson).

3GPP RAN1#106bis-e, R1-2112827, [107-e-R17-RRC-CovEnh] Summary of email discussion on Rel-17 RRC parameters for Coverage Enhancement, Moderator (China Telecom, Sharp, Nokia, Qualcomm, ZTE).

3GPP RAN1#107-e, R1-2200115 Discussion on remaining issues for Type A PUSCH repetitions for Msg3, ZTE Corporation.