3GPP TSG RAN WG1 #107bis-e	 																						   R1-2200139

e-Meeting, January 17th – 25th, 2022

Agenda Item:	8.15.5

Source:	CATT

Title:	Remaining issues on Rel-17 UE features for NR Positioning enhancements

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss remaining issues related to Rel-17 NR UE features for NR positioning enhancements.

At RAN1 #107-e within the scope of [107-e-R17-UE-features-ePos-01], the following agreements were achieved related to Rel-17 NR UE features for NR positioning enhancements in [1].

Agreement: Adopt the following changes highlighted in chromatic formatting

| 27. NR_pos_enh | 27-1-1 | Maximum number Support of UE-RxTEGs [for UE-assisted DL TDOA and/or Multi-RTT positioning] | The maximum number of UE-RxTEG, which is supported and reported by UE [for UE assisted DL TDOA and/or Multi-RTT positioning] FFS: the values (>1) FFS: whether to have a value=1 to indicate UE Rx timing errors is well calibrated FFS: whether to have separate values/FGs for DL TDOA and/or Multi-RTT positioning [If UE supports this capability with the values > 1, the UE supports including one UE Rx TEG ID for the RSTD reference time and one UE Rx TEG ID for each DL RSTD measurement (including each additional DL RSTD measurement), in a DL TDOA measurement report] | 13-1 [,one or more of {13-3, 13-4}], 27-1-1b | No |  | Mitigation of UE Rx timing delays is not supported UE-RxTEG reporting is not supported and no assumption can be made on the mitigation of UE Rx timing delays for the measurements | FFS: Per UE or [per band or FS] | n/a | n/a | n/a | [The candidate values are {[1,]2[, 3],4,6,8[,12,16,24,32]}] Need for location server to know if the feature is supported FFS: Separate row for “Support of UE-RxTEG reporting for DL-TDOA”, and “Support of UE-RxTEG reporting for M-RTT” If UE supports this capability with the values > 1, and if the UE does not include RxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE Rx timing delays for this measurement [If value=1 is indicated by the UE, the UE Rx timing errors differences between two measurements are within a margin only if the UE reports an the same Rx-TEG-ID associated with both the measurements, otherwise, no assumption can be made about the timing error differences between these measurements.] Note: The “per band” reporting on this capability does not imply, that the RxTEG IDs in the measurement report are grouped per band; In the measurement report, the RxTEG ID can span from 0, up to 31 | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic formatting

| 27. NR_pos_enh | 27-1-2 | Maximum number Support of UE-TxTEGs [for UL TDOA and/or Multi-RTT positioning] | The maximum number of UE-TxTEG, which is supported and reported by UE [for UL TDOA and/or Multi-RTT positioning] FFS: the values (>1) FFS: whether to have a value=1 to indicate UE Tx timing errors is well calibrated FFS: whether to have different values/FGs for UL TDOA and/or Multi-RTT positioning FFS: Separate row for “Support of UE-TxTEG reporting for MRTT”, and a separate row for the “maximum number of TxTEGs for RTT” [If UE supports this capability with the values > 1, the UE supports to provide the association information of UL SRS resources for positioning with Tx TEGs to the LMF. - FFS: Whether the association information is sent directly from UE to LMF, or is first provided to gNB and then forwarded to LMF] | [13-4, ] 13-8], 27-1-2b | FFS Yes |  | Mitigation of UE Tx timing delays is not supported UE-TxTEGs for UL TDOA is not supported and no assumption can be made on the mitigation of UE Tx timing for the SRS” and “UE-TxTEGs for RTT is not supported and no assumption can be made on the mitigation of UE Tx timing for the SRS | FFS: Per UE or per band or per FS | n/a | n/a | n/a | [The candidate values are {[1,] 2, 4, 6, 8, 16, 32}] [Need for location server to know if the feature is supported] Note: It should support the serving gNB to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB for UL TDOA [if UL TDOA is supported by UE] [Note: It should support the LMF to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF for Multi-RTT if Multi-RTT is supported by UE] | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-1-2a | Support of UE-TxTEGs for Multi-RTT [and/or UL TDOA] positioning | The maximum number of UE-TxTEG, which is supported and reported by UE for Multi-RTT positioning | 13-4, 13-8 | No |  | UE-TxTEGs for Multi-RTT positioning is not supported | [per band per FS] | n/a | n/a | n/a | The candidate values are {[1, ] 2, 4, 6, 8} Need for location server to know if the feature is supported If UE supports this capability with the values > 1, and if if the UE does not include TxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE Tx timing delays for this SRS resource [If value=1 is indicated by the UE, the UE Tx timing errors differences between two SRS resources are within a margin only if the UE reports an Tx-TEG-ID associated with the SRS resources, otherwise, no assumption can be made about the timing error differences between these SRS resources.] [Note: It should support the serving gNB to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB for UL TDOA if UL TDOA is supported by UE] [Note: It should support the LMF to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF for Multi-RTT if Multi-RTT is supported by UE] | Optional with capability signaling |

Agreements: Add the following new row/FG to the 27-x-y  family for NR_pos_enh

| 27. NR_pos_enh | 27-xx3 | Support of PRS TEG association information for UE-based DL-TDOA | Support of reception of association between PRS and TRP Tx TEG for UE-based positioning |  | No |  | Positioning calculation assistance data containing association between PRS and TRP Tx TEG is not supported by UE | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. Agreement: Support the LMF to provide the association information of DL PRS resources with Tx TEGs to a UE for UE-based positioning if the TRP has multiple TEGs | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG number/ID will be fixed by moderator before LS is sent to RAN2

Agreements: Add the following new row/FG to the 27-x-y  family for NR_pos_enh

| 27. NR_pos_enh | 27-uu1 | Support of lower Rx beam sweeping factor | 1. Support of the lower Rx beam sweeping factor than 8 for FR2 2. Number of Rx beam sweeping factors: {1,2,3,4,5,6,7} |  | No |  | UE only supports 8 as the Rx beam sweeping factor defined by RAN4. | Per band (FR2 only) | n/a | n/a | n/a | Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG number/ID will be fixed by moderator before LS is sent to RAN2

Agreements: Add the following new row/FG to the 27-x-y  family for NR_pos_enh

| 27. NR_pos_enh | 27-uu2 | Support of UL MAC CE based MG activation request | 1. Support of using UL MAC CE to request measurement gap. |  | Yes |  | Using UL MAC CE to indicate PRS measurement to the gNB is not supported. | Per UE | No | No | No |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG number/ID will be fixed by moderator before LS is sent to RAN2

Agreements: Add the following new row/FG to the 27-x-y  family for NR_pos_enh

| 27. NR_pos_enh | 27-uu3 | Support of DL MAC CE based MG activation | 1. Support of preconfiguration of MGs in RRC 2. Support of using DL MAC CE to activate the MG |  | Yes |  | Using DL MAC CE to activate the preconfigured MG is not supported | Per UE | No | No | No |  | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG number/ID will be fixed by moderator before LS is sent to RAN2

Agreements: Add the following new row/FG to the 27-x-y  family for NR_pos_enh

| 27. NR_pos_enh | 27-vv1 | LOS/NLOS indicator for UE-based positioning assistance data | Support reception of the assistance data containing the LOS/NLOS indicator. 1. LOS/NLOS indicator type: {softValue, hardValue, both} 2. LOS/NLOS indicator granularity {resourceSpecific, trpSpecific, both} |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG number/ID will be fixed by moderator before LS is sent to RAN2

Agreements: Add the following new row/FG to the 27-x-y  family for NR_pos_enh

| 27. NR_pos_enh | 27-vv2 | Additional path reporting for UE-assisted DL-TDOA | [1. Support of TOA reporting for more than 2 additional paths.] 2. Support of path RSRP reporting for additional paths if path RSRP reporting is supported. |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG number/ID will be fixed by moderator before LS is sent to RAN2

Agreements: Add the following new row/FG to the 27-x-y  family for NR_pos_enh

| 27. NR_pos_enh | 27-vv4 | Additional path reporting for Multi-RTT | 1. Support of TOA reporting for more than 2 additional paths 2. Support of path RSRP reporting for additional paths if path RSRP reporting is supported. |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG number/ID will be fixed by moderator before LS is sent to RAN2

Agreements: Add the following new row/FG to the 27-x-y  family for NR_pos_enh

| 27. NR_pos_enh | 27-ww2 | Support of positioning SRS transmission in RRC_INACTIVE state [for initial BWP] | 1. Max number of SRS Resource Sets for positioning supported by UE per BWP. Values = {1, 2, 4, 8, 12, 16}. 2. Max number of periodic SRS Resources for positioning per BWP. Values = {1,2,4,8,16,32,64} 3. Max number of periodic SRS Resources for positioning per BWP per slot. Values = {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} 4. FFS: Applicability for initial BWP OLPC for SRS for positioning based on SSB from the last serving cell (the cell that releases UE from connection) is part of this FG. Note: no dedicated capability signaling is intended for this component |  | Yes |  |  | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG number/ID will be fixed by moderator before LS is sent to RAN2

Agreements: Add the following new row/FG with yellow background to the 27-x-y  family for NR_pos_enh

| 27. NR_pos_enh | 27-ww5 | OLPC for positioning SRS in RRC_INACTIVE state | Same as LPP OLPC-SRS-Pos-r16 RRC OLPC-SRS-Pos-r16 |  | Yes |  |  | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG number/ID will be fixed by moderator before LS is sent to RAN2

Agreements: Add the following new row/FG to the 27-x-y  family for NR_pos_enh

| 27. NR_pos_enh | 27-ww1 | Support of [PRS measurement in RRC_INACTIVE] | Support of PRS measurement in RRC_INACTIVE Note: UE supporting this feature may support at least one from DL RSTD, DL PRS-RSRP, or UE Rx – Tx time difference |  | Yes |  |  | Per UE | No | No | No |  | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-ww1a | Support of PRS measurement in RRC_INACTIVE state for DL-TDOA | Support of PRS measurement in RRC_INACTIVE state for DL-TDOA Note: Other PRS capabilities follows the same as the RRC_CONNECTED state for DL-TDOA. |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-ww1b | Support of PRS measurement in RRC_INACTIVE state for DL-AoD | Support of PRS measurement in RRC_INACTIVE state for DL-AoD Note: Other PRS capabilities follows the same as the RRC_CONNECTED state for DL-AoD. |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-ww1c | Support of PRS measurement in RRC_INACTIVE state for Multi-RTT | 1. Support of PRS measurement in RRC_INACTIVE state for Multi-RTT 2. Support of positioning SRS transmission in RRC_INACTIVE state. Note: Other PRS capabilities follows the same as the RRC_CONNECTED state for Multi-RTT. |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |

- FG number/ID will be fixed by moderator before LS is sent to RAN2

Agreements: Add the following new row/FG to the 27-x-y  family for NR_pos_enh

| 27. NR_pos_enh | 27-ww6 | Spatial relation for positioning SRS in RRC_INACTIVE state | Same as LPP SpatialRelationsSRS-Pos-r16 RRC SpatialRelationsSRS-Pos-r16 |  | Yes |  |  | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG number/ID will be fixed by moderator before LS is sent to RAN2

Agreements: Add the following new row/FG to the 27-x-y  family for NR_pos_enh

| 27. NR_pos_enh | 27-c4b | DL PRS processing capabilities in RRC inactive state | 1. DL PRS buffering capability: Type 1 or Type 2 a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. a) Type 1 – sub-slot/symbol level buffering b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms 3. Max number of DL PRS resources that UE can process in a slot under it a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG number/ID will be fixed by moderator before LS is sent to RAN2

Agreements: Add the following new row/FG to the 27-x-y  family for NR_pos_enh

| 27. NR_pos_enh | 27-d5 | Maximum number of measurement instances which can be included in a single measurement report | Maximum number of measurement instances which can be included in a single measurement report |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG number/ID will be fixed by moderator before LS is sent to RAN2

Agreements: Adopt the following changes highlighted in chromatic formatting

| 27. NR_pos_enh | 27-1-3 | Maximum number Support of UE-RxTxTEGs for Multi-RTT | The maximum number of UE-RxTxTEG, which is supported and reported by UE for Multi-RTT positioning FFS: the values (>1) FFS: whether to have a value=1 to indicate UE RxTx timing errors is well calibrated [If a UE support this capability with the values > 1, the UE supports reporting of UE RxTx TEG ID with UE Rx-Tx time difference measurements for Multi-RTT positioning] | [13-4 or 13-8] | No |  | Mitigation of UE RxTx timing delays is not supported | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate values are {[1,] 2, 4, 6, 8, 12, 16, 24, 32[, 64, 128, 256]}] Need for location server to know if the feature is supported If UE supports this capability with the values > 1, and if the UE does not include RxTxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE RxTx timing delays for this measurement [If value=1 is indicated by the UE, the UE RxTx timing errors differences between two measurements are within a margin only if the UE reports an RxTx-TEG-ID associated with the measurements, otherwise, no assumption can be made about the timing error differences between these measurements] Note: The “per band” reporting on this capability does not imply, that the RxTxTEG IDs in the measurement report are grouped per band; In the measurement report, the RxTxTEG ID can span from 0, up to [255] FFS: Separate row for “Support of UE-RxTxTEG reporting for Multi-RTT” | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreements: Adopt the following changes highlighted in chromatic formatting incl. the yellow highlighting

| 27. NR_pos_enh | 27-1-4 | The maximum Number Support of UE Rx TEGs for measuring the same DL PRS resource | The maximum number of different UE-RxTEGs that a UE can support to measure the same DL PRS of a TRP. FFS: The values (>1) | 27-1-1 | No |  | Mitigation of UE Rx timing delays by using different Rx TEGs are not supported Up to 1 RxTEG is used to measure the same DL PRS resource of a TRP | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate values are {1, 2, 3, 4, 6, 8}] Need for location server to know if the feature is supported [Note: UE measures the same instance of the DL PRS with multiple RX TEGs] FFS: Separate row for “Support measuring the same DL PRS of a TRP with different UE-RxTEGs” FFS: Separate row for “The maximum Number of UE Rx TEGs for measuring the same DL PRS resource simultaneously” | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-1-4a | Support of UE Rx TEGs for measuring the same DL PRS resource simultaneously | The maximum number of UE Rx TEGs for measuring the same DL PRS resource simultaneously |  | No |  |  | Per band | n/a | n/a | n/a | [The candidate values are {1,2, 4, 8}] | Optional with capability signaling |

Agreements: Adopt the following changes highlighted in chromatic formatting

| 27. NR_pos_enh | 27-2-1 | [UE-assisted] DL PRS RSRP of the first [or additional] path [for DL-AoD] | 1.) Support of [measuring and reporting the] PRS RSRP of the first path [or additional] [for DL-AoD positioning method] 2.) The maximum number of first path PRS RSRP per TRP [Note: Applicable for DL-TDOA and Multi-RTT] | [13-2 or 13-3], 13-4, 13-5, 13-8] | No |  | PRS RSRP of the first path is not supported | FFS: Per UE or per band | n/a | n/a | n/a | Component 2 candidate values: [2,4,8,16,24] Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreements: Adopt the following changes highlighted in chromatic formatting

| 27. NR_pos_enh | 27-2-2 | DL PRS RSRP reporting for more than 8 measurements for UE-assisted DL-AoD positioning | Support reporting K> 8 DL PRS RSRP measurements per TRP. FFS: the values of K Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for different timestamps. FFS: Additional capability may be added to limit the maximum number of DL PRS RSRP associated with the same Rx beam index | [13-5, 13-2] | No |  | UE report of more than 8 DL PRS-RSRP is not supported. | FFS: Per UE or per band | n/a | FFS Yes | n/a | [The candidate values are {[12,] 16, 24[, 32, 64]}] If K is not reported, default value is 8. Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreements: Adopt the following changes highlighted in chromatic formatting

| 27. NR_pos_enh | 27-3-1 | M-sample measurements [of DL PRS measurement on single DL PRS period/occasion] | [The capability to support reporting a measurement based on measuring M samples (instances) of a DL PRS resource set] M=1[,2-3]. FFS: other values. If the UE does not provide the capability, the UE [is assumed to] support M=4 only. | [13-1, 13-4, 13-8] | No |  |  | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate values are {1, 2, 3, 4}] If the UE does not provide the capability, the UE [is assumed to] support M=4 only. Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Discussion

Proposed changes to FG 27-1-1

The following draft FG 27-1-1 was agreed with some remaining issues in RAN1#107-e.

| 27. NR_pos_enh | 27-1-1 | Maximum number Support of UE-RxTEGs [for UE-assisted DL TDOA and/or Multi-RTT positioning] | The maximum number of UE-RxTEG, which is supported and reported by UE [for UE assisted DL TDOA and/or Multi-RTT positioning] FFS: the values (>1) FFS: whether to have a value=1 to indicate UE Rx timing errors is well calibrated FFS: whether to have separate values/FGs for DL TDOA and/or Multi-RTT positioning [If UE supports this capability with the values > 1, the UE supports including one UE Rx TEG ID for the RSTD reference time and one UE Rx TEG ID for each DL RSTD measurement (including each additional DL RSTD measurement), in a DL TDOA measurement report] | 13-1 [,one or more of {13-3, 13-4}], 27-1-1b | No |  | Mitigation of UE Rx timing delays is not supported UE-RxTEG reporting is not supported and no assumption can be made on the mitigation of UE Rx timing delays for the measurements | FFS: Per UE or [per band or FS] | n/a | n/a | n/a | [The candidate values are {[1,]2[, 3],4,6,8[,12,16,24,32]}] Need for location server to know if the feature is supported FFS: Separate row for “Support of UE-RxTEG reporting for DL-TDOA”, and “Support of UE-RxTEG reporting for M-RTT” If UE supports this capability with the values > 1, and if the UE does not include RxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE Rx timing delays for this measurement [If value=1 is indicated by the UE, the UE Rx timing errors differences between two measurements are within a margin only if the UE reports an the same Rx-TEG-ID associated with both the measurements, otherwise, no assumption can be made about the timing error differences between these measurements.] Note: The “per band” reporting on this capability does not imply, that the RxTEG IDs in the measurement report are grouped per band; In the measurement report, the RxTEG ID can span from 0, up to 31 | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For this FG, we think separate rows for “Support of UE-RxTEG reporting for DL-TDOA” and “Support of UE-RxTEG reporting for M-RTT are not needed, and only one row for “Support of UE-RxTEGs for UE-assisted DL TDOA and/or Multi-RTT positioning” should be enough. The FG should be per band and about the candidate values of this FG, we think the values should include {1,2, 3,4,6,8,12,16,24,32}. For value=1, we think it is reasonable that if value=1 is indicated by the UE, the UE Rx timing errors differences between two measurements are within a margin only if the UE reports the same Rx-TEG-ID associated with both measurements, otherwise, no assumption can be made about the timing error differences between these measurements.

Based on the above discussions, our proposal on FG27-1-1 as follows,

Proposal 1: Adopt the following modifications marked as red colour to FG 27-1-1 based on the agreement in RAN1#107-e:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-1-1 | Support of UE-RxTEGs for UE-assisted DL TDOA and/or Multi-RTT positioning | The maximum number of UE-RxTEG, which is supported and reported by UE for UE assisted DL TDOA and/or Multi-RTT positioning | 13-1 one or more of {13-3, 13-4} | No |  | UE-RxTEG reporting is not supported and no assumption can be made on the mitigation of UE Rx timing delays for the measurements | per band | n/a | n/a | n/a | The candidate values are {1,2, 3,4,6,8,12,16,24,32} Need for location server to know if the feature is supported If UE supports this capability with the values > 1, and if the UE does not include RxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE Rx timing delays for this measurement If value=1 is indicated by the UE, the UE Rx timing errors differences between two measurements are within a margin only if the UE reports the same Rx-TEG-ID associated with both measurements, otherwise, no assumption can be made about the timing error differences between these measurements. Note: The “per band” reporting on this capability does not imply, that the RxTEG IDs in the measurement report are grouped per band; In the measurement report, the RxTEG ID can span from 0, up to 31 | Optional with capability signaling |

Proposed changes to FG 27-1-2/2a

The following draft FG 27-1-2/2a was agreed with some remaining issues in RAN1#107-e.

Agreement: Adopt the following changes highlighted in chromatic formatting

| 27. NR_pos_enh | 27-1-2 | Maximum number Support of UE-TxTEGs [for UL TDOA and/or Multi-RTT positioning] | The maximum number of UE-TxTEG, which is supported and reported by UE [for UL TDOA and/or Multi-RTT positioning] FFS: the values (>1) FFS: whether to have a value=1 to indicate UE Tx timing errors is well calibrated FFS: whether to have different values/FGs for UL TDOA and/or Multi-RTT positioning FFS: Separate row for “Support of UE-TxTEG reporting for MRTT”, and a separate row for the “maximum number of TxTEGs for RTT” [If UE supports this capability with the values > 1, the UE supports to provide the association information of UL SRS resources for positioning with Tx TEGs to the LMF. - FFS: Whether the association information is sent directly from UE to LMF, or is first provided to gNB and then forwarded to LMF] | [13-4, ] 13-8], 27-1-2b | FFS Yes |  | Mitigation of UE Tx timing delays is not supported UE-TxTEGs for UL TDOA is not supported and no assumption can be made on the mitigation of UE Tx timing for the SRS” and “UE-TxTEGs for RTT is not supported and no assumption can be made on the mitigation of UE Tx timing for the SRS | FFS: Per UE or per band or per FS | n/a | n/a | n/a | [The candidate values are {[1,] 2, 4, 6, 8, 16, 32}] [Need for location server to know if the feature is supported] Note: It should support the serving gNB to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB for UL TDOA [if UL TDOA is supported by UE] [Note: It should support the LMF to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF for Multi-RTT if Multi-RTT is supported by UE] | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-1-2a | Support of UE-TxTEGs for Multi-RTT [and/or UL TDOA] positioning | The maximum number of UE-TxTEG, which is supported and reported by UE for Multi-RTT positioning | 13-4, 13-8 | No |  | UE-TxTEGs for Multi-RTT positioning is not supported | [per band per FS] | n/a | n/a | n/a | The candidate values are {[1, ] 2, 4, 6, 8} Need for location server to know if the feature is supported If UE supports this capability with the values > 1, and if if the UE does not include TxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE Tx timing delays for this SRS resource [If value=1 is indicated by the UE, the UE Tx timing errors differences between two SRS resources are within a margin only if the UE reports an Tx-TEG-ID associated with the SRS resources, otherwise, no assumption can be made about the timing error differences between these SRS resources.] [Note: It should support the serving gNB to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB for UL TDOA if UL TDOA is supported by UE] [Note: It should support the LMF to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF for Multi-RTT if Multi-RTT is supported by UE] | Optional with capability signaling |

For the FG 27-1-2, we think this FG should be per band and about the candidate values of this FG, we think the values should include {1,2,4,6,8}. For value=1, we think it is reasonable that if value=1 is indicated by the UE, the UE Tx timing errors differences between two SRS resources are within a margin only if the UE reports an Tx-TEG-ID associated with the SRS resources, otherwise, no assumption can be made about the timing error differences between these SRS resources. We prefer to keep the note of “Need for location server to know if the feature is supported”. About the two notes below the note of “Need for location server to know if the feature is supported”, we prefer to keep the first one, and remove the note related to Multi-RRT.

For the FG 27-1-2a, we think this FG should be per band and about the candidate values of this FG, we think the values should include {1,2,4,6,8}. For value=1, we think it is reasonable that if value=1 is indicated by the UE, the UE Tx timing errors differences between two SRS resources are within a margin only if the UE reports an Tx-TEG-ID associated with the SRS resources, otherwise, no assumption can be made about the timing error differences between these SRS resources. About the two notes below the note of “Need for location server to know if the feature is supported”, we prefer to keep the second one, and remove the note related to UL-TDOA.

Based on the above discussions, our proposal on FG27-1-2/2a as follows,

Proposal 2: Adopt the following modifications marked as red colour to FG 27-1-2/2a based on the agreement in RAN1#107-e:

| 27. NR_pos_enh | 27-1-2 | Support of UE-TxTEGs for UL TDOA | The maximum number of UE-TxTEG, which is supported and reported by UE for UL TDOA | 13-4, 13-8 | Yes |  | UE-TxTEGs for UL TDOA is not supported and no assumption can be made on the mitigation of UE Tx timing for the SRS” and “UE-TxTEGs for RTT is not supported and no assumption can be made on the mitigation of UE Tx timing for the SRS | per band | n/a | n/a | n/a | The candidate values are {1, 2, 4, 6, 8} Need for location server to know if the feature is supported. If value=1 is indicated by the UE, the UE Tx timing errors differences between two SRS resources are within a margin only if the UE reports an Tx-TEG-ID associated with the SRS resources, otherwise, no assumption can be made about the timing error differences between these SRS resources. Note: It should support the serving gNB to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB for UL TDOA. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-1-2a | Support of UE-TxTEGs for Multi-RTT positioning | The maximum number of UE-TxTEG, which is supported and reported by UE for Multi-RTT positioning | 13-4, 13-8 | No |  | UE-TxTEGs for Multi-RTT positioning is not supported | per band | n/a | n/a | n/a | The candidate values are {1, 2, 4, 6, 8} Need for location server to know if the feature is supported If UE supports this capability with the values > 1, and if the UE does not include TxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE Tx timing delays for this SRS resource If value=1 is indicated by the UE, the UE Tx timing errors differences between two SRS resources are within a margin only if the UE reports an Tx-TEG-ID associated with the SRS resources, otherwise, no assumption can be made about the timing error differences between these SRS resources. Note: It should support the LMF to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF for Multi-RTT if Multi-RTT is supported by UE. | Optional with capability signaling |

Proposed changes to FG 27-1-3

The following draft FG 27-1-3 was agreed with some remaining issues in RAN1#107-e.

Agreements: Adopt the following changes highlighted in chromatic formatting

| 27. NR_pos_enh | 27-1-3 | Maximum number Support of UE-RxTxTEGs for Multi-RTT | The maximum number of UE-RxTxTEG, which is supported and reported by UE for Multi-RTT positioning FFS: the values (>1) FFS: whether to have a value=1 to indicate UE RxTx timing errors is well calibrated [If a UE support this capability with the values > 1, the UE supports reporting of UE RxTx TEG ID with UE Rx-Tx time difference measurements for Multi-RTT positioning] | [13-4 or 13-8] | No |  | Mitigation of UE RxTx timing delays is not supported | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate values are {[1,] 2, 4, 6, 8, 12, 16, 24, 32[, 64, 128, 256]}] Need for location server to know if the feature is supported If UE supports this capability with the values > 1, and if the UE does not include RxTxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE RxTx timing delays for this measurement [If value=1 is indicated by the UE, the UE RxTx timing errors differences between two measurements are within a margin only if the UE reports an RxTx-TEG-ID associated with the measurements, otherwise, no assumption can be made about the timing error differences between these measurements] Note: The “per band” reporting on this capability does not imply, that the RxTxTEG IDs in the measurement report are grouped per band; In the measurement report, the RxTxTEG ID can span from 0, up to [255] FFS: Separate row for “Support of UE-RxTxTEG reporting for Multi-RTT” | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For the FG 27-1-3, we think the candidate values of this FG should include {1, 2, 4, 6, 8, 12, 16, 24, 32, 64, 128, 256}. For value=1, we think it is reasonable that if value=1 is indicated by the UE, the UE RxTx timing errors differences between two measurements are within a margin only if the UE reports an RxTx-TEG-ID associated with the measurements, otherwise, no assumption can be made about the timing error differences between these measurements. For the last note, we support the RxTxTEG ID can span from 0, up to 255, in the measurement report.

Based on the above discussions, our proposal on FG27-1-3 as follows,

Proposal 3: Adopt the following modifications marked as red colour to FG 27-1-3 based on the agreement in RAN1#107-e:

| 27. NR_pos_enh | 27-1-3 | Support of UE-RxTxTEGs for Multi-RTT | The maximum number of UE-RxTxTEG, which is supported and reported by UE for Multi-RTT positioning | 13-4 or 13-8 | No |  | Mitigation of UE RxTx timing delays is not supported | per band | n/a | n/a | n/a | The candidate values are {1, 2, 4, 6, 8, 12, 16, 24, 32, 64, 128, 256} Need for location server to know if the feature is supported If UE supports this capability with the values > 1, and if the UE does not include RxTxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE RxTx timing delays for this measurement If value=1 is indicated by the UE, the UE RxTx timing errors differences between two measurements are within a margin only if the UE reports an RxTx-TEG-ID associated with the measurements, otherwise, no assumption can be made about the timing error differences between these measurements Note: The “per band” reporting on this capability does not imply, that the RxTxTEG IDs in the measurement report are grouped per band; In the measurement report, the RxTxTEG ID can span from 0, up to 255 | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed changes to FG 27-1-4a

The following draft FG 27-1-4/4a was agreed with some remaining issues in RAN1#107-e.

Agreements: Adopt the following changes highlighted in chromatic formatting incl. the yellow highlighting

| 27. NR_pos_enh | 27-1-4 | The maximum Number Support of UE Rx TEGs for measuring the same DL PRS resource | The maximum number of different UE-RxTEGs that a UE can support to measure the same DL PRS of a TRP. FFS: The values (>1) | 27-1-1 | No |  | Mitigation of UE Rx timing delays by using different Rx TEGs are not supported Up to 1 RxTEG is used to measure the same DL PRS resource of a TRP | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate values are {1, 2, 3, 4, 6, 8}] Need for location server to know if the feature is supported [Note: UE measures the same instance of the DL PRS with multiple RX TEGs] FFS: Separate row for “Support measuring the same DL PRS of a TRP with different UE-RxTEGs” FFS: Separate row for “The maximum Number of UE Rx TEGs for measuring the same DL PRS resource simultaneously” | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-1-4a | Support of UE Rx TEGs for measuring the same DL PRS resource simultaneously | The maximum number of UE Rx TEGs for measuring the same DL PRS resource simultaneously |  | No |  |  | Per band | n/a | n/a | n/a | [The candidate values are {1,2, 4, 8}] | Optional with capability signaling |

For the FG 27-1-4a, we think the candidate values of this FG should include {1, 2, 4, 8}.

Based on the above discussions, our proposal on FG27-1-4a as follows,

Proposal 4: Adopt the following modifications marked as red colour to FG 27-1-4a based on the agreement in RAN1#107-e:

| 27. NR_pos_enh | 27-1-4a | Support of UE Rx TEGs for measuring the same DL PRS resource simultaneously | The maximum number of UE Rx TEGs for measuring the same DL PRS resource simultaneously |  | No |  |  | Per band | n/a | n/a | n/a | The candidate values are {1,2, 4, 8} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed changes to FG 27-2-1

The following draft FG 27-2-1 was agreed with some remaining issues in RAN1#107-e.

Agreements: Adopt the following changes highlighted in chromatic formatting

| 27. NR_pos_enh | 27-2-1 | [UE-assisted] DL PRS RSRP of the first [or additional] path [for DL-AoD] | 1.) Support of [measuring and reporting the] PRS RSRP of the first path [or additional] [for DL-AoD positioning method] 2.) The maximum number of first path PRS RSRP per TRP [Note: Applicable for DL-TDOA and Multi-RTT] | [13-2 or 13-3], 13-4, 13-5, 13-8] | No |  | PRS RSRP of the first path is not supported | FFS: Per UE or per band | n/a | n/a | n/a | Component 2 candidate values: [2,4,8,16,24] Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For the FG 27-2-1, we think the texts “UE-assisted” can be added before the name of this FG. For the name of Component 1, we think the texts “measuring and reporting the” can be added into the name of this Component. We think this FG should be per band. We think the candidate values of this FG should include {2,4,8,16,24}.

Based on the above discussions, our proposal on FG27-2-1 as follows,

Proposal 5: Adopt the following modifications marked as red colour to FG 27-2-1 based on the agreement in RAN1#107-e:

| 27. NR_pos_enh | 27-2-1 | UE-assisted DL PRS RSRP of the first path for DL-AoD | 1.) Support of measuring and reporting the PRS RSRP of the first path for DL-AoD positioning method 2.) The maximum number of first path PRS RSRP per TRP | 13-2 or 13-3, 13-4, 13-5, 13-8 | No |  | PRS RSRP of the first path is not supported | per band | n/a | n/a | n/a | Component 2 candidate values: {2,4,8,16,24} Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed changes to FG 27-2-2

The following draft FG 27-2-2 was agreed with some remaining issues in RAN1#107-e.

Agreements: Adopt the following changes highlighted in chromatic formatting

| 27. NR_pos_enh | 27-2-2 | DL PRS RSRP reporting for more than 8 measurements for UE-assisted DL-AoD positioning | Support reporting K> 8 DL PRS RSRP measurements per TRP. FFS: the values of K Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for different timestamps. FFS: Additional capability may be added to limit the maximum number of DL PRS RSRP associated with the same Rx beam index | [13-5, 13-2] | No |  | UE report of more than 8 DL PRS-RSRP is not supported. | FFS: Per UE or per band | n/a | FFS Yes | n/a | [The candidate values are {[12,] 16, 24[, 32, 64]}] If K is not reported, default value is 8. Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For the FG 27-2-2, we think this FG should be per band. The candidate values of this FG should include {12, 16, 24, 32, 64}.

Based on the above discussions, our proposal on FG27-2-2 as follows,

Proposal 6: Adopt the following modifications marked as red colour to FG 27-2-2 based on the agreement in RAN1#107-e:

| 27. NR_pos_enh | 27-2-2 | DL PRS RSRP reporting for more than 8 measurements for UE-assisted DL-AoD positioning | Support reporting K> 8 DL PRS RSRP measurements per TRP. Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for different timestamps. | 13-5, 13-2 | No |  | UE report of more than 8 DL PRS-RSRP is not supported. | per band | n/a | Yes | n/a | The candidate values are {12, 16, 24, 32, 64} Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed changes to FG 27-3-1

The following draft FG 27-3-1 was agreed with some remaining issues in RAN1#107-e.

Agreements: Adopt the following changes highlighted in chromatic formatting

| 27. NR_pos_enh | 27-3-1 | M-sample measurements [of DL PRS measurement on single DL PRS period/occasion] | [The capability to support reporting a measurement based on measuring M samples (instances) of a DL PRS resource set] M=1[,2-3]. FFS: other values. If the UE does not provide the capability, the UE [is assumed to] support M=4 only. | [13-1, 13-4, 13-8] | No |  |  | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate values are {1, 2, 3, 4}] If the UE does not provide the capability, the UE [is assumed to] support M=4 only. Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For the FG 27-3-1, we think the name of this FG should be changed to “Support of M-sample measurements of DL PRS measurement on single DL PRS occasion” to better reflect the meaning of the FG. For the component of this FG, we are fine with current wording of “The capability to support reporting a measurement based on measuring M samples (instances) of a DL PRS resource set”. We think this FG should be per band and its candidate value is {1}.

Based on the above discussions, our proposal on FG27-3-1 as follows,

Proposal 7: Adopt the following modifications marked as red colour to FG 27-3-1 based on the agreement in RAN1#107-e:

| 27. NR_pos_enh | 27-3-1 | Support of M-sample measurements of DL PRS measurement on single DL PRS occasion | The capability to support reporting a measurement based on measuring M samples (instances) of a DL PRS resource set | 13-1, 13-4, 13-8 | No |  |  | per band | n/a | n/a | n/a | The candidate value is {1}. If the UE does not provide the capability, the UE [is assumed to] support M=4 only. Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Conclusion

In this contribution, we provided our views on Rel-17 UE features for NR positioning enhancements with the following proposals.

Proposal 1: Adopt the following modifications marked as red colour to FG 27-1-1 based on the agreement in RAN1#107-e:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-1-1 | Support of UE-RxTEGs for UE-assisted DL TDOA and/or Multi-RTT positioning | The maximum number of UE-RxTEG, which is supported and reported by UE for UE assisted DL TDOA and/or Multi-RTT positioning | 13-1 one or more of {13-3, 13-4} | No |  | UE-RxTEG reporting is not supported and no assumption can be made on the mitigation of UE Rx timing delays for the measurements | per band | n/a | n/a | n/a | The candidate values are {1,2, 3,4,6,8,12,16,24,32} Need for location server to know if the feature is supported If UE supports this capability with the values > 1, and if the UE does not include RxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE Rx timing delays for this measurement If value=1 is indicated by the UE, the UE Rx timing errors differences between two measurements are within a margin only if the UE reports the same Rx-TEG-ID associated with both measurements, otherwise, no assumption can be made about the timing error differences between these measurements. Note: The “per band” reporting on this capability does not imply, that the RxTEG IDs in the measurement report are grouped per band; In the measurement report, the RxTEG ID can span from 0, up to 31 | Optional with capability signaling |

Proposal 2: Adopt the following modifications marked as red colour to FG 27-1-2/2a based on the agreement in RAN1#107-e:

| 27. NR_pos_enh | 27-1-2 | Support of UE-TxTEGs for UL TDOA | The maximum number of UE-TxTEG, which is supported and reported by UE for UL TDOA | 13-4, 13-8 | Yes |  | UE-TxTEGs for UL TDOA is not supported and no assumption can be made on the mitigation of UE Tx timing for the SRS” and “UE-TxTEGs for RTT is not supported and no assumption can be made on the mitigation of UE Tx timing for the SRS | per band | n/a | n/a | n/a | The candidate values are {1, 2, 4, 6, 8} Need for location server to know if the feature is supported. If value=1 is indicated by the UE, the UE Tx timing errors differences between two SRS resources are within a margin only if the UE reports an Tx-TEG-ID associated with the SRS resources, otherwise, no assumption can be made about the timing error differences between these SRS resources. Note: It should support the serving gNB to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB for UL TDOA. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-1-2a | Support of UE-TxTEGs for Multi-RTT positioning | The maximum number of UE-TxTEG, which is supported and reported by UE for Multi-RTT positioning | 13-4, 13-8 | No |  | UE-TxTEGs for Multi-RTT positioning is not supported | per band | n/a | n/a | n/a | The candidate values are {1, 2, 4, 6, 8} Need for location server to know if the feature is supported If UE supports this capability with the values > 1, and if the UE does not include TxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE Tx timing delays for this SRS resource If value=1 is indicated by the UE, the UE Tx timing errors differences between two SRS resources are within a margin only if the UE reports an Tx-TEG-ID associated with the SRS resources, otherwise, no assumption can be made about the timing error differences between these SRS resources. Note: It should support the LMF to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF for Multi-RTT if Multi-RTT is supported by UE. | Optional with capability signaling |

Proposal 3: Adopt the following modifications marked as red colour to FG 27-1-3 based on the agreement in RAN1#107-e:

| 27. NR_pos_enh | 27-1-3 | Support of UE-RxTxTEGs for Multi-RTT | The maximum number of UE-RxTxTEG, which is supported and reported by UE for Multi-RTT positioning | 13-4 or 13-8 | No |  | Mitigation of UE RxTx timing delays is not supported | per band | n/a | n/a | n/a | The candidate values are {1, 2, 4, 6, 8, 12, 16, 24, 32, 64, 128, 256} Need for location server to know if the feature is supported If UE supports this capability with the values > 1, and if the UE does not include RxTxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE RxTx timing delays for this measurement If value=1 is indicated by the UE, the UE RxTx timing errors differences between two measurements are within a margin only if the UE reports an RxTx-TEG-ID associated with the measurements, otherwise, no assumption can be made about the timing error differences between these measurements Note: The “per band” reporting on this capability does not imply, that the RxTxTEG IDs in the measurement report are grouped per band; In the measurement report, the RxTxTEG ID can span from 0, up to 255 | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 4: Adopt the following modifications marked as red colour to FG 27-1-4a based on the agreement in RAN1#107-e:

| 27. NR_pos_enh | 27-1-4a | Support of UE Rx TEGs for measuring the same DL PRS resource simultaneously | The maximum number of UE Rx TEGs for measuring the same DL PRS resource simultaneously |  | No |  |  | Per band | n/a | n/a | n/a | The candidate values are {1,2, 4, 8} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 5: Adopt the following modifications marked as red colour to FG 27-2-1 based on the agreement in RAN1#107-e:

| 27. NR_pos_enh | 27-2-1 | UE-assisted DL PRS RSRP of the first path for DL-AoD | 1.) Support of measuring and reporting the PRS RSRP of the first path for DL-AoD positioning method 2.) The maximum number of first path PRS RSRP per TRP | 13-2 or 13-3, 13-4, 13-5, 13-8 | No |  | PRS RSRP of the first path is not supported | per band | n/a | n/a | n/a | Component 2 candidate values: {2,4,8,16,24} Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 6: Adopt the following modifications marked as red colour to FG 27-2-2 based on the agreement in RAN1#107-e:

| 27. NR_pos_enh | 27-2-2 | DL PRS RSRP reporting for more than 8 measurements for UE-assisted DL-AoD positioning | Support reporting K> 8 DL PRS RSRP measurements per TRP. Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for different timestamps. | 13-5, 13-2 | No |  | UE report of more than 8 DL PRS-RSRP is not supported. | per band | n/a | Yes | n/a | The candidate values are {12, 16, 24, 32, 64} Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 7: Adopt the following modifications marked as red colour to FG 27-3-1 based on the agreement in RAN1#107-e:

| 27. NR_pos_enh | 27-3-1 | Support of M-sample measurements of DL PRS measurement on single DL PRS occasion | The capability to support reporting a measurement based on measuring M samples (instances) of a DL PRS resource set | 13-1, 13-4, 13-8 | No |  |  | per band | n/a | n/a | n/a | The candidate value is {1}. If the UE does not provide the capability, the UE [is assumed to] support M=4 only. Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- References

- R1-2111810, Summary of UE features for NR positioning enhancements, Moderators (AT&T), RAN1#107-e.