3GPP TSG RAN WG1 Meeting #107bis-e			                                  R1- 2200170

e-Meeting, January 17th – 25th, 2022

______________________________________________________________________Agenda item: 8.11.1.2

Source: Moderator (LG Electronics)

Title: Feature lead summary #5 for AI 8.11.1.2 Inter-UE coordination for Mode 2 enhancements

Document for: Discussion and information

- Draft proposals for Monday’s GTW (January 17th)

According to the chairman’s guideline that the topics will be treated by following the order of the open issues in the status report, draft proposals for Scheme 1 to be treated at Monday’s GTW session (January 17th) are limited to the topic of “Finalization of contents and containers of UE-A’s inter-UE coordination information and UE-B’s explicit request, including determination of destination UE(s) for UE-A’s inter-UE coordination information and UE-B’s explicit request”.

  - Scheme 1

FL’s observation:

For inter-UE coordination information triggered by an explicit request, majority companies support that starting and ending time locations of resource selection window to determine the set of preferred resources is provided by the explicit request. On the other hand, few companies proposed that at least starting time location of the resource selection window can be determined by UE-A and indicated by UE-A’s inter-UE coordination information.

Draft proposal 1-1

Alt 1:

- For determining preferred resource set in Scheme 1, when the inter-UE coordination information transmission is triggered by UE-B’s explicit request,

  - Starting/Ending time locations of resource selection window is provided by UE-B’s explicit request

    - Starting/Ending time locations of resource selection window is a form of combination of DFN index and slot index

Alt 2:

- For determining preferred resource set in Scheme 1, when the inter-UE coordination information transmission is triggered by UE-B’s explicit request,

  - Ending time locations of resource selection window is provided by UE-B’s explicit request

    - Ending time locations of resource selection window is a form of combination of DFN index and slot index

  - Starting time locations of resource selection window is determined by UE-A and indicated by UE-A’s inter-UE coordination information

    - Starting time locations of resource selection window is a form of combination of DFN index and slot index

FL’s observation:

For inter-UE coordination information triggered by an explicit request, a number of companies supports that the value of Cresel to determine the set of preferred resources is provided by the explicit request since the value of Cresel is necessary to determine the set of preferred resoruces by following mode 2 RA specified in TS 38.214 section 8.1.4.

Draft proposal 1-2

Alt 1:

- For determining preferred resource set in Scheme 1, when the inter-UE coordination information transmission is triggered by UE-B’s explicit request,

  - The value of Cresel is provided by UE-B’s explicit request

Alt 2:

- For determining preferred resource set in Scheme 1, when the inter-UE coordination information transmission is triggered by UE-B’s explicit request,

  - The value of Cresel is (pre)configured

FL’s observation:

For the contents of the request for the inter-UE coordination information, a variety of information is proposed by companies. For progress, the prospoal focus on contents supported by a number of companies.

Draft proposal 1-3

- For Scheme 1, at least following parameters are additionally provided by UE-B’s request for the inter-UE coordination information

  - Resource set type

FL’s observation:

For the contents of the inter-UE coordination information, a variety of information is proposed by companies. For progress, the prospoal focus on contents supported by a number of companies. Moreover, for a 2nd SCI design, majority companies supports that a SCI format 2-A is a baseline.

Draft proposal 1-4

- For Scheme 1, at least following parameters are provided by UE-A’s inter-UE coordination information

  - Indication of resource set

  - Resource set type

- When a SCI format 2-C is used,

  - SCI format 2-C includes at least all the SCI fields in a SCI format 2-A as specified in TS 38.212 section 8.4.1.1 on top of the inter-UE coordination information

  - The same set of SCI fields for a SCI format 2-C is supported for both inter-UE coordination information triggered by an explicit request and inter-UE coordination information triggered by a condition other than explicit request reception

FL’s observation:

For the remaining details on first resource location of each TRIV, some companies provides their views on the definition of the first resource location and its candidate.

Draft proposal 1-5

Alt 1:

- For Scheme 1, candidates of first resource location of each TRIV are (pre)configured and defined by

  - Slot offset to the slot where inter-UE coordination information is transmitted for first TRIV

  - Slot offset to the last actual indicated slot in immediate previous TRIV for other TRIV(s)

Alt 2:

- For Scheme 1, candidates of first resource location of each TRIV are (pre)configured and defined by

  - Slot offset to the earliest slot of a resource pool within a resource selection window for determing the set of resources

FL’s observation:

For the remaining details on TRIV/FRIV, some companies proposed that the assumption on Sl-MaxNumPerReserve.

Draft proposal 1-6

- For the indication of resource set in Scheme 1, the value of Sl-MaxNumPerReserve is 3.

FL’s observation:

For the container of the request for the inter-UE coordination information, clear majority is not observed. Meanwhile, some companies supporting MAC CE only commented that a 2nd SCI format needs to be reserved for future uses. For progress, FL suggests to employ similar approach of the container of inter-UE coordination information.

Draft proposal 1-7

- For Scheme 1,

  - MAC CE and 2nd SCI are used as the container of an explicit request transmission from UE-B to UE-A

    - The same information is indicated in the 2nd SCI and the MAC CE

    - Size of the 2nd SCI for the explicit request is the same as the size of a SCI format 2-C

      - A format indicator is included in both a SCI format 2-C and the 2nd SCI format for the explicit request

FL’s observation:

A number of companies prefer that the value of N is (pre)configured. Considering that additional information other than indication of resource set is conveyed in 2nd SCI, a note is added that the value of N is (pre)configured so that the size of the 2nd SCI excluding 24-bit CRC is no greater than 140.

Draft proposal 1-8

- Confrim the following working assumption with red color marked changes:

    - Alt 1 (Working Assumption): MAC CE or 2nd SCI are used as the container of inter-UE coordination information transmission from UE A to UE B.

      - For the indication of resource set, the following is supported:

        - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request.

          - First resource location of each TRIV is separately indicated by the inter-UE coordination information

        - If [N <= a (pre)configured threshold 3], MAC CE is used and it is up to UE implementation to additionally use 2nd SCI. When 2nd SCI and MAC CE are both used, the same resource set is indicated in the 2nd SCI and the MAC CE. If [N > 3] Otherwise, only MAC CE is used.

          - FFS: UE capability details

          - 2nd SCI is UE RX optional

          - Note: A UE expects that the (pre)configured threshold for N is selected so that the size of the 2nd SCI excluding 24-bit CRC is no greater than 140 bits

FL’s observation:

For the cast type(s) of inter-UE coordination information and its request, majority companies supports unicast for the inter-UE coordination information triggered by an explicit request. On the other hand, when the inter-UE coordination information triggered by a condition, majority companies supports both unicast and groupcast. For the request signalling, clear majority is not observed between unicast and groupcast.

Draft proposal 1-9

- For Scheme 1,

  - Unicast is supported for an explicit request transmission for inter-UE coordination information

    - Unicast is used for the inter-UE coordination information transmission triggered by the explicit request

  - Following cast type(s) are supported for inter-UE coordination information transmission tirggred by a condition other than explicit request reception

    - Unicast

    - Groupcast

FL’s observation:

For determining preferred resource set triggered by a condition other than a request reception, clear majority is not observed on how to assume the values of parameters specified in TS 38.214 section 8.1.4.

Draft proposal 1-10

Alt 1:

- For determining preferred resource set in Scheme 1, when inter-UE coordination information transmission is triggered by a condition other than explicit request reception,

  - Values of following parameters are (pre)configured for a resource pool

    - prio_TX

    - L_subCH

    - P_rsvp_TX

    - Resource selection window size

    - Cresel

  - UE-A determines a value of following parameter and indicates it in its inter-UE coordination information

    - n+T_1

Alt 2:

- For determining preferred resource set in Scheme 1, when inter-UE coordination information transmission is triggered by a condition other than explicit request reception,

  - UE-B’s prior SCI determines values of following parameters

    - prio_TX

    - L_subCH

    - P_rsvp_TX

  - UE-A determines values of following parameters and indicates them in its inter-UE coordination information

    - n+T_1, n+T_2

    - Cresel

- Draft proposals for Tuesday’s GTW (January 18th)

According to the chairman’s guideline that the topics will be treated by following the order of the open issues in the status report, draft proposals for Scheme 2 to be treated at Tuesday’s GTW session (January 18th) are limited to the topic of “Finalization of determination of PSFCH resource/index for conflict indication”.

  - Scheme 2

FL’s observation:

On the setting of m_CS, clear majority is not observed between m_CS setting based on a condition and m_CS = 0. On PSFCH resource-to-(m_0,PRB) mapping, clear majority is not observed. Meanwhile, some companies proposed to differently indicate resource conflict event (e.g. location of a resource conflict, condition type of a resource conflct). Some companies support it by using different values of m_CS while some companies support it by using different PSFCH resources. Other companies does not support it and want to keep Rel-16 PSFCH-to-(m_0, PRB) mapping and set the value of m_CS to 0.

Draft proposal 2-1

Alt 1:

- For Scheme 2,

  - m_0 and PRB for PSFCH used for a resource conflit indication are derived in the same way as specified in TS 38.213 Section 16.3

  - Value of m_CS is determined by a condition type of a resource confclit

    - 0: Condition 2-A-1

    - 6: Condition 2-A-2

Alt 2:

- For Scheme 2,

  - m_0 and PRB for PSFCH used for a resource conflit indication are derived in the same way as specified in TS 38.213 Section 16.3

  - Value of m_CS is always 0

Alt 3:

- For Scheme 2,

  - m_0 and PRB for PSFCH used for a resource conflit indication are derived in the same way as specified in TS 38.213 Section 16.3 except that

    - addition offset is applied to a PSFCH resource index to indicate a condition type of a resource conflict

  - Value of m_CS is always 0

FL’s observation:

Clear majority is observed for the time gap between a PSFCH and SCI(s) scheduling conflicting TBs when PSFCH occasion is derived by a slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B’s SCI. FL suggests to apply the same processing time for the case when PSFCH occasion is derived by a slot where UE-B’s SCI is transmitted.

Draft proposal 2-2

- When PSFCH occasion is derived by a slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B’s SCI, time gap between the PSFCH and SCI(s) scheduling conflicting TBs is larger than or equal to X value

  - X = sl-MinTimeGapPSFCH

- 1st email discussion (Due date: January 18th 4:59pm UTC)

I ask companies to provide inputs on questions in Section 3 until January 18th 4:59pm UTC. To prepare/make more stable draft proposals before the start of the next GTW session, it would be highly appreciated if companies make comments as soon as possible. Also to make progress more efficiently, I would like to encourage companies to directly provide “revised wording” or “new wording needed to be added”.

  - Scheme 1

    - Finalization of behaviour of UE-B receiving resource set(s) from UE-A(s)

FL’s observation:

On UE-B’s behavior for Scheme 1 with preferred resource set Option A, majority companies support having the same MAC layer procedure regardless of a container of the preferred resource set.

Q3-1: Do you agree following proposal?

- For Scheme 1 with preferred resource set Option A,

  - MAC layer selects resources using S_A and the received preferred resource set

    - MAC layer firstly selects resources for transmissions within the intersection of S_A and the preferred resource set until it becomes impossible to select a resource within the intersection under the constraint defined in Rel-16.

      - The received preferred resource set is reported by physical layer at UE-B if 2nd SCI is used as a container of the preferred resource set

    - After this, if the number of selected resources is smaller than the required number of transmissions for a TB, MAC layer selects resources for the remaining transmissions outside the intersection either inside S_A or inside preferred resource set first then  S_A based on (pre-)configured behavior or attributes of UE-A.

| Company | Answer (yes or no) | Comments |
| --- | --- | --- |

FL’s observation:

On UE-B’s behavior for Scheme 1 with preferred resource set Option B, consensus is observed to support that MAC layer at UE-B selects resoruces belonging to the received preferred resource set. Meanwhile, few companies considered the possibility that preferred resoruces are not sufficient.

Q3-2: Do you agree following proposal?

- For Scheme 1 with preferred resource set Option B,

  - MAC layer selects resources belonging to the received preferred resource set

      - The received preferred resource set is reported by physical layer at UE-B if 2nd SCI is used as a container of the preferred resource set

| Company | Answer (yes or no) | Comments |
| --- | --- | --- |

Q3-3: For Scheme 1 with preffered resource set Option B, is it necessary to specify how to handle the case when the amount of preferred resources does not exceed X*M_totoal? If yes, how to handle it?

| Company | Answer (yes or no) | Comments |
| --- | --- | --- |

FL’s observation:

On UE-B’s behavior for Scheme 1 with non-preferred resource set, few companies considered to introduce additional enhancement to handle a case where the amount of candidate single-slot resoruces is not sufficient.

Q3-4: For Scheme 1 with non-preffered resource set, is it necessary to specify how to handle the case when the amount of candidate single-slot resources obtained after the exclusion of resource(s) overlapping with non-preferred resources does not exceed X*M_totoal? If yes, how to handle it?

| Company | Answer (yes or no) | Comments |
| --- | --- | --- |

FL’s observation:

Few companies suggested to introduce latency bound of the inter-UE coordination information in Scheme 1. With the latency bound, UE-B will use the received inter-UE coordination information within the latency bound for its resource (re)selection.

Q3-4: For Scheme 1, is it necessary to specify latency bound for UE-B to consider the inter-UE coordination information trigerred by UE-B’s explicit request in its resource (re)selection? If yes, which option is supported to handle it?

- Option 1: Latency bound is provided by UE-B’s request

- Option 2: Latency bound is provided by a (pre)configuration

- Option 3: Latency bound is determined by UE-B so that T_2 is larger than or equal to T_2,min

- Option 4: Up to UE-B’s implementation

- Option 5: Other (please specify it)

| Company | Answer (yes or no) | Option(s) | Comments |
| --- | --- | --- | --- |

Q3-5: For Scheme 1, is it necessary to specify latency bound for UE-B to consider the inter-UE coordination information trigerred by a condition other than explicit request reception in its resource (re)selection? If yes, which option is supported to handle it?

- Option 1: Latency bound is provided by a (pre)configuration

- Option 2: Latency bound is determined by UE-B so that T_2 is larger than or equal to T_2,min

- Option 3: Up to UE-B’s implementation

- Option 4: Other (please specify it)

| Company | Answer (yes or no) | Option(s) | Comments |
| --- | --- | --- | --- |

    - Finalization of when and with which information UE-A generates and/or transmits an inter-UE coordination information, including triggering based on condition(s) other than an explicit request

FL’s observation:

Clear majority is not observed, and the situation is not so much different compared to the last meeting. FL suggests to provide two alternatives to finalize it.

Q3-6: Which alternative do you agree between following proposals?

Alt 1:

- For inter-UE coordination triggered by a condition rather than request reception in Scheme 1,

  - it is up to UE-A’s implementation whether or not to trigger the inter-UE coordination information generation.

  - Note: Rel-16 procedure of UL/SL prioritization, LTE SL/NR SL prioritization, and congestion control is applied to the transmission of the inter-UE coordination information triggered by a condition.

Alt2:

- For inter-UE coordination triggered by a condition rather than request reception in Scheme 1,

  - the inter-UE coordination information generation can be triggered if the following is met. For other cases, it is up to UE implementation whether or not to trigger the inter-UE coordination information generation.

    - UE-A has data that is transmitted together with the inter-UE coordination information to UE-B

  - Note: Rel-16 procedure of UL/SL prioritization, LTE SL/NR SL prioritization, and congestion control is applied to the transmission of the inter-UE coordination information triggered by a condition.

| Company | Alt(s) | Comments |
| --- | --- | --- |

Q3-7: Do you agree following proposals?

- For inter-UE coordination triggered by an explicit request in Scheme 1, whether or not to transmit the inter-UE coordination information upon the request reception is determined by at least following procedures

  - Rel-16 procedure of UL/SL prioritization, LTE SL/NR SL prioritization, and congestion control

| Company | Answer (yes or no) | Comments |
| --- | --- | --- |

FL’s observation:

A few companies suggested to specify sensing window for determining the set of resoruces provided in inter-UE coordination information.

Q3-8: Is it necessary to specify the sensing window for determining the set of resources? If yes, which option do you agree between following proposals?

- Option 1: Transmission time (slot n) of inter-UE coordination information is a reference point

  - Option 1-1: sensing window for the set of resources is given by [n – T_0, n – T_proc,0]

  - Option 1-2: sensing window for the set of resources is given by [n – T_0 – T_proc,1, n – T_proc,0 – T_proc,1]

- Option 2: Starting time location (slot n) of a resource selction window for determining the set of resources is a reference point

  - Option 2-1: sensing window for the set of resources is given by [n – T_0 – T_proc,1, n – T_proc,0 – T_proc,1]

- Option 3: Other (please specify it)

| Company | Answer (yes or no) | Option(s) | Comments |
| --- | --- | --- | --- |

    - Finalization of when UE-B generates and/or transmits an explicit request

FL’s observation:

Clear majority is not observed, and the situation is not so much different compared to the last meeting. FL suggests to provide two alternatives to finalize it.

Q3-9: Which alternative do you agree among following proposals?

Alt 1:

- For inter-UE coordination triggered by a condition rather than request reception in Scheme 1,

  - it is up to UE-A’s implementation whether or not to trigger the request generation

  - Note: Rel-16 procedure of UL/SL prioritization, LTE SL/NR SL prioritization, and congestion control is applied to the transmission of the request transmission.

Alt2:

- For inter-UE coordination triggered by a condition rather than request reception in Scheme 1,

  - the request generation can be triggered if the following is met. For other cases, it is up to UE implementation whether or not to trigger the request generation.

    - Priority value of UE-B’s transmission is smaller than a (pre)configured threshold

  - Note: Rel-16 procedure of UL/SL prioritization, LTE SL/NR SL prioritization, and congestion control is applied to the transmission of the request transmission.

Alt3:

- For inter-UE coordination triggered by a condition rather than request reception in Scheme 1,

  - the request generation can be triggered if the following is met. For other cases, it is up to UE implementation whether or not to trigger the request generation.

    - UE-B has data that is transmitted together with the request to UE-B

  - Note: Rel-16 procedure of UL/SL prioritization, LTE SL/NR SL prioritization, and congestion control is applied to the transmission of the request transmission.

| Company | Alt(s) | Comments |
| --- | --- | --- |

    - Finalization of resource selection and/or multiplexing with sidelink transmissions for UE-A’s inter-UE coordination information and UE-B’s explicit request

FL’s observation:

Consensus is observed for the UE-A’s behavior to select resoruces to be used for inter-UE coordination information transmission.

Q3-10: Do you agree following proposals?

- For inter-UE coordination information transmission in Scheme 1,

  - UE-A performs its resource (re)selection according to the same procedure in Rel-16 TS 38.214 Section 8.1.4 to transmit the inter-UE coordination information to UE-B.

| Company | Answer | Comments |
| --- | --- | --- |

FL’s observation:

Majority companies support that UE-B performs random selection to select resoruces to be used for the request transmission when UE-B does not perform sensing/resource exclusion.

Q3-11: Do you agree following proposals?

- For inter-UE coordination information transmission in Scheme 1,

  - UE-B performs its resource (re)selection according to the same procedure in Rel-16 TS 38.214 Section 8.1.4 to transmit the request for the inter-UE coordination information to UE-A if UE-B performs sensing/resource exclusion. Otherwise, at least UE-B can perform random selection

| Company | Answer | Comments |
| --- | --- | --- |

FL’s observation:

A few companies supported the possilbity of multiplexing inter-UE coordination information and/or its request with other data.

Q3-12: Do you agree following proposals?

- For inter-UE coordination information transmission in Scheme 1,

  - Inter-UE coordination information can be multiplexed with other data only if the same source/destination IDs are used

- For request transmission in Scheme 1,

  - Request can be multiplexed with other data only if the same source/destination IDs are used

| Company | Answer | Comments |
| --- | --- | --- |

FL’s observation:

A few companies suggeseted to introduce dedicated resources for inter-UE coordination information transmission and its request transmission.

Q3-13: Is it necessary to (pre)configure dedicated resources for transmission(s) of inter-UE coordination information and/or its request? If yes, please specify details on this.

| Company | Answer | Comments |
| --- | --- | --- |

    - Finalization of prioritization of inter-UE coordination information and explicit request

FL’s observation:

Clear majority is not observed, and the situation is not so much different compared to the last meeting. FL tires to update the latest version of proposals in the last meeting.

Q3-13: Do you agree following proposals?

- For inter-UE coordination information triggered by an explicit request in Scheme 1, the priority value of the inter-UE coordination information is (pre)configured priority value if it is provided by (pre)configuration. Otherwise, the priority value is the same as indicated by UE-B’s request.

  - For the case when inter-UE coordination information is transmitted together with other data (if supported), the priority value of the inter-UE coordination information transmission is determined by the smallest priority value between the inter-UE coordination information and data

| Company | Answer | Comments |
| --- | --- | --- |

Q3-14: Do you agree following proposals?

- For inter-UE coordination information triggered by an explicit request in Scheme 1, the priority value of request is (pre)configured priority value if it is provided by (pre)configuration. Otherwise, the priority value is the same as that of a TB to be transmitted by UE-B.

  - For the case when the request is transmitted together with other data (if supported), the priority value of the request transmission is determined by the smallest priority value between the request and data

| Company | Answer | Comments |
| --- | --- | --- |

FL’s observation:

Majority companies support that the priority value of inter-UE coordination information triggered by a condition other than explicit request reception is (pre)configured.

Q3-15: Do you agree following proposals?

- For inter-UE coordination information triggered by a condition other than explicit request reception in Scheme 1, the priority value of the inter-UE coordination information is (pre)configured priority value

  - For the case when inter-UE coordination information is transmitted together with other data (if supported), the priority value of the inter-UE coordination information transmission is determined by the smallest priority value between the inter-UE coordination information and data

| Company | Answer | Comments |
| --- | --- | --- |

    - Combination of preferred/non-preferred resources with explicit request/condition triggers

FL’s observation:

Clear majority is not observed, and the situation is not so much different compared to the last meeting. FL suggests to down-select one of two alternatives for progress.

Q3-16: Which alternative do you agree following proposals?

Alt 1:

- For Scheme 1, following combinations are supported

  - Inter-UE coordination information of preferred resource set triggered by an explicit request

  - Inter-UE coordination information of non-preferred resource set triggered by an explicit request

  - Inter-UE coordination information of preferred resource set triggered by a condition other than the explicit request reception

  - Inter-UE coordination information of non-preferred resource set triggered by a condition other than the explicit request reception

Alt2:

- For Scheme 1, following combinations are supported

  - Inter-UE coordination information of preferred resource set triggered by an explicit request

  - Inter-UE coordination information of non-preferred resource set triggered by an explicit request

  - Inter-UE coordination information of non-preferred resource set triggered by a condition other than the explicit request reception

| Company | Answer | Comments |
| --- | --- | --- |

  - Scheme 2

    - Finalization of determination of PSFCH resource/index for conflict indication

FL’s observation:

A few companies suggested to indicate the time location of a resource conflict when PSFCH occasion is derived by a slot where UE-B’s SCI is transmitted. Since it is related to UE-B’s behavior upon a reception of a resrouce conflict indication, it seems necessary to resolve it.

Q3-17: Is it necessary to indicate time location of a resource conflict when PSFCH occasion is derived by a slot where UE-B’s SCI is transmitted? If yes, how to indicate it.

| Company | Answer | Comments |
| --- | --- | --- |

    - Finalization of behaviour of UE-B receiving a conflict indication from UE-A

FL’s observation:

After deciding how to use PSFCH resource and m_CS value in terms of time location or condition type of a resource confclit, we can continue to discuss the UE behavior upon a reception of a resource confclit indication.

    - Finalization of prioritization of conflict indication

FL’s observation:

Clear majority is not observed on whether PSFCH TX/RX of SL HARQ-ACK feedback is always prioritized over that of a resource conflict indication or the existing prioritization rule is reused. In addition, slight majority is observed on that smallest priority of conflicting TBs is used as a priority value of PSFCH TX and that priority value indicated by UE-B’s SCI is used as a priority value of PSFCH RX. Considering both issues, one possible way to make progress is to allow the possibility of (pre)configuring priority values of PSFCH TX/RX including 9 is used to always prioritize PSFCH TX/RX for SL HARQ-ACK feedback over that of a resource confclit indication.

Q3-18: Do you agree following proposal?

- For Scheme 2,

  - Priority value of PSFCH TX for a resource conflict indication is the smallest priority value of the confliting TBs if (pre)configured value is not provided. Otherwise, the priorty value is a (pre)configured one including 9.

  - Priority value of PSFCH RX for a resource conflict indication is priority value indicated by UE-B’s SCI if (pre)configured value is not provided. Otherwise, the priorty value is a (pre)configured one including 9.

  - Note: For PSFCH TX/RX prioritization between SL HARQ-ACK feedback and a resource conflict indication, the prioritization rule as specified in TS 38.213 Section 16.2.4.2 is reused.

| Company | Answer | Comments |
| --- | --- | --- |

    - Finalization of how to determine UE-B among UEs scheduling conflicting TBs, including whether/how to handle, or differently handle, the case when at least one of UEs scheduling conflicting TBs doesn’t support Scheme 2

FL’s observation:

A few companies suggested to apply the UE-B selection procedure for the case when UE’s PSFCH occasions for a resource confclit indication is not passed.

Q3-18: Do you agree following proposal?

- Confirm the following working assumption with red-color changes:

  - Working Assumption:

    - For Condition 2-A-1 in Scheme 2, when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is enabled or when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is disabled and the destination UE of the conflicting TBs is UE-A, for each pair of UEs scheduling the conflicting TBs whose PSFCH occassoins for a resource conflict indication is not yet passed, a UE with the higher priority value is UE-B.

| Company | Answer | Comments |
| --- | --- | --- |

FL’s observation:

Majoirty companies support the possibility that UB’s SCI indicating whether or not the UE can be UE-B. Meanwhile, updating the working assumption considering this indication needs to resolve the working assumption itself first as in Q3-18. In other words, if the working assumption is updated as a result of Q3-18, then the updated one will be applied to the proposal in Q3-20.

Q3-19: Do you agree following proposal?

- For Scheme 2,

  - One of reserved bits in a SCI format 1-A is used to indicate whether UE scheduling a conflict TB can be UE-B or not.

| Company | Answer | Comments |
| --- | --- | --- |

Q3-20: If the answer of Q3-19 is yes, do you agree following proposal? Note that if the working assumption is updated as a result of Q3-18, then the updated one will be applied to the proposal in Q3-20.

- For Scheme 2,

  - Following working assumption is applied to the case when all the UEs scheduling confclit TBs indicate that the UE can be UE-B

    - Working Assumption:

      - For Condition 2-A-1 in Scheme 2, when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is enabled or when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is disabled and the destination UE of the conflicting TBs is UE-A, for each pair of UEs scheduling the conflicting TBs, a UE with the higher priority value is UE-B.

  - When at least one of UEs scheduling conflicting TBs does not support scheme 2, all other UEs with unknown capability or supporting scheme 2 are UE-Bs

| Company | Answer | Comments |
| --- | --- | --- |

  - Others

Q3-21: If any essential issues that should be addressed (epeciall those with RAN2 impact) are missing, please provide them.

| Company | Comments |
| --- | --- |

- Summary of contributions

  - Scheme 1

- Finalization of contents and containers of UE-A’s inter-UE coordination information and UE-B’s explicit request, including determination of destination UE(s) for UE-A’s inter-UE coordination information and UE-B’s explicit request

  - Remaining details on determining preferred resource set

    - If inter-UE coordination information is triggered by an explicit request

      - Starting/ending time location of resource selection window (n+T_1, n+T_2)

        - Provided by UE-B’s request

          - Supported by [Futurewei,2] [Huawei,3] [CATT,6] [LGE,7](for ending time location) [OPPO,16](for ending time location) [ETRI,17] [Intel,19] [Apple,20] [xiaomi,22](for ending time location) [Lenovo,23] [Sharp,24] (11)

        - Indicated by UE-A’s inter-UE coordination information

          - Supported by [LGE,7](for starting time location) [OPPO,16](for starting time location) [Sharp,24] (3)

        - Provided by UE-A’s MAC layer based on time location of inter-UE coordinaotin information transmission and remaining PDB informed by UE-B

          - Supported by [vivo,4] [Samsung,10] [Fraunhofer,29] (3)

        - Determined by UE-B’s request reception time

          - Supported by [xiaomi,22](for starting time location) (1)

        - Slot n

          - time location where UE-A transmits the inter-UE coordination information [OPPO,16]

          - Provided by UE-A’s MAC layer [vivo,4]

      - C_resel

        - Provided by UE-B’s request

          - Supported by [LGE,7] [OPPO,16] [Apple,20] [CMCC,28] [Panasonic,31] (5)

    - If inter-UE coordination information is triggered by a condition rather than request reception

      - Setting of prio_TX, L_subCH, P_rsvp_TX

        - Indicated by a (pre)configuration

          - Supported by [Futurewei,2] [LGE,7] [Intel,19] (3)

        - Indicated by UE-B’s prior SCI

          - Supported by [Futurewei,2] [InterDigital,9] [OPPO,16] (3)

        - Indicated by PC5-RRC

          - Supported by [Futurewei,2] (1)

        - Determined by UE-A’s implementation, and they are included in UE-A’s inter-UE coordination information

          - Supported by [vivo,4] (1)

      - Setting of resource selection window

        - Inidcated by a (pre)configuration

          - Supported by [Futurewei,2] [LGE,7](for window size) [Intel,19](for window size) [Lenovo,23] (4)

        - Indicated by PC5-RRC

          - Supported by [Futurewei,2] (1)

        - Determined by UE-A’s implementation

          - Supported by [vivo,4] [LGE,7](for starting time location) (2)

            - Indiacated by inter-UE coordination information in terms of DFN index and slot index [LGE,7]

      - Setting of other parameters

        - Cresel

          - (pre)configured

            - Suppoted by [LGE,7] (1)

  - Contents of the inter-UE coordination information in Scheme 1

    - Resource set type [Futurewei,2] [Huawei,3] [LGE,7] [CAICT,13] [Fraunhofer,29] [Panasonic,31] (6)

      - For each resource indication combination [CAICT,13]

    - Priority value associated with preferred or non-preferred resources

      - Supported by [Huawei,3] [vivo,4] [InterDigital,9] [ASUSTeK,26] (4)

    - Starting time location of a resource selection window

      - [LGE,7] [OPPO,16] [Intel,19] [Sharp,24] (4)

    - Location information

      - Supported by [InterDigital,9] (1)

    - Condition type indicator

      - Supported by [Intel,19] (1)

    - Strating sub-channel of the first resource

      - Supported by [Intel,19] (1)

    - Number of signaled resources

      - Supported by [Intel,19](for MAC CE) (1)

    - Ending time location of a resource selection window

      - Supported by [Intel,19] (1)

    - RSRP used in sensing procedure

      - Supported by [ASUSTeK,26] (1)

    - Remaining details on resource indication

      - First resource location of each TRIV

        - Candidates

          - (pre)configured: [Huawei,3] [LGE,7] [Apple,20] (3)

          - Among slots located multiples of 31 logical slots after starting time location of resource selection window [LGE,7] [Sharp,24] (2)

        - Signaling details

          - Time offset to the slot where inter-UE coordinaotin information is transmitted

            - Supported by [Huawei,3](for first TRIV) [Intel,19](for first TRIV in 2nd SCI) [Apple,20](with respect to last retransmission) (3)

          - Time offset to the last actual indicated slot in immediate previous TRIV

            - Supported by [Huawei,3](for other TRIV(s)) [ETRI,17] [Intel,19](for other TRIV(s) in 2nd SCI) [ASUSTeK,26](for other TRIV(s)) (4)

          - Time offset to first logical slot within a resource selection window given by DFN index and slot index

            - Supported by [LGE,7] [Intel,19](for MAC CE) (2)

      - Sl-MaxNumPerReserve

        - Fixed to 3

          - Suppoted by [Huawei,3] [LGE,7] [ETRI,17] [Intel,19] (4)

  - Contents of the request for the inter-UE coordination information in Scheme 1

    - Starting and/or ending time position of resource selection window

      - Supported by [Futurewei,2] [Huawei,3] [CATT,6] [LGE,7](at least ending time location) [OPPO,16](ending time location) [ETRI,17] [Intel,19] [Apple,20] [xiaomi,22](for ending time location) [Lenovo,23] [Sharp,24] [CMCC,28] [Fraunhofer,29] (13)

        - DFN index + slot index [LGE,7]

        - Time gap between last retransmission timing of the explicit request and the starting/ending time location of a resource selection window [Apple,20]

    - Resource set type

      - Supported by [Nokia,1] [Futurewei,2] [CATT,6] [InterDigital,9] [Samsung,10] [Apple,20] [ZTE,21] [Lenovo,23] (8)

    - Remaining PDB

      - Supported by [vivo,4] [Samsung,10] [ZTE,21] [CMCC,28] [Fraunhofer,29] (5)

    - C_resel

      - Supported by [LGE,7] [OPPO,16] [Apple,20] [CMCC,28] [Panasonic,31] (5)

    - Resoruces to be used for inter-UE coordination information signaling

      - Supported by [Nokia,1] [Fujitsu,5] [ITL,11] (3)

    - Number of resoruces to be reported

      - Supported by [Nokia,1] [Apple,20] (2)

    - Remaining PDB for inter-UE coordination information

      - Supported by [vivo,4] [Samsung,10] (2)

    - Number of (re)transmission(s) for a TB

      - Supported by [vivo,4] [Apple,20] (2)

    - Message size

      - Supported by [Nokia,1] (1)

    - ID(s) of the intended receiver(s)

      - Supported by [Nokia,1] (1)

    - ID(s) used by UE-B

      - Supported by [Nokia,1] (1)

    - Preferred or non-preferred resources determined at UE-B

      - Supported by [Nokia,1] (1)

    - X%

      - Supported by [Fujitsu,5] (1)

    - Zone ID and MCR

      - Supported by [Samsung,10] (1)

  - Container  of inter-UE coordination information and its request

    - Remaining details on container of the inter-UE coordinaotin information

      - Details condition to use 2nd SCI

        - When 2nd SCI is used, MAC CE containing coordination information is not included in a TB

          - Supported by [Nokia,1] [vivo,4] (2)

        - 2nd SCI can be used if N<=3 and total payload size of the 2nd SCI is no greater than 140 bits [LGE,7]

        - 2nd SCI can be used if N<=2 [DCM,12]

        - Preferred resource set only [Intel,19]

        - Toal payload size does not exceed (pre)configured value [Sharp,24]

        - N is (pre)configured [Huawei,3] [Intel,19] [Apple,20] [Sharp,24] (4)

    - Details on a SCI format 2-C

      - SCI fields for a SCI format 2-A

        - Supported by [Huawei,3] [vivo,4] [DCM,12] [Intel,19](for 1st new SCI format) [Ericsson,30] [Panasonic,31] (6)

      - SCI fields for a SCI format 2-B

        - Supported by [Intel,19](for 2nd new SCI format) (1)

      - Union of SCI fields for a SCI format 2-A and 2-B

        - Supported by [LGE,7] [Fraunhofer,29] (2)

      - Resource indication combinations

        - Supported by [Huawei,3] [vivo,4] [LGE,7] [Intel,19] [xiaomi,22] [Fraunhofer,29] [Ericsson,30] [Panasonic,31] (8)

          - Resource reservation field for the inter-UE coordination information triggered by UE-B’s request

            - Always present: [Huawei,3] [LGE,7] (2)

              - All zeros: [Huawei,3]

      - First resource location

        - Supported by [Huawei,3] [vivo,4] [LGE,7] [Intel,19] [xiaomi,22] [Panasonic,31] (6)

      - Resource type

        - Supported by [Huawei,3] [LGE,7] [Fraunhofer,29] [Panasonic,31] (4)

      - Priority value associated with preferred or non-preferred resources

        - Supported by [Huawei,3] [vivo,4] [xiaomi,22] [Fraunhofer,29] (4)

      - Strating sub-channel of the first resource

        - Supported by [Intel,19] (1)

  - Container of the explicit request in Scheme 1

    - 2nd-stage SCI and MAC CE

      - Supported by [Nokia,1] [Huawei,3] [Sony,8] [Lenovo,23] (4)

    - 2nd-stage SCI

      - Supported by [Futurewei,2] [vivo,4] [CATT,6] [Samsung,10] [xiaomi,22] [Sharp,24] [ASUSTeK,26] [CMCC,28] (8)

        - A flag to distinguish inter-UE coordination information signaling and its request [vivo,4] [Sharp,24] (2)

        - 2nd SCI only without SL-SCH [Samsung,10]

    - MAC CE

      - Supported by [LGE,7] [Apple,20] [ZTE,21] [Sharp,24] [Fraunhofer,29] (5)

    - PC5-RRC

      - Supported by [Qualcomm,15] [Fraunhofer,29] (2)

    - PSFCH

      - Supported by [MediaTek,27] (1)

  - Other details for scheme 1

    - Inter-UE coordination triggered by UE-B’s request

      - Cast type

        - Unicast [Futurewei,2] [Huawei,3] [Fujitsu,5](for preferred resource) [CATT,6] [LGE,7] [Spreadtrum,14] [Intel,19] [Fraunhofer,29] (8)

        - Groupcast [Nokia,1]

        - Braodcast [Intel,19]

      - Source ID

        - Source ID of UE-A’s transmission to UE-B [LGE,7]

      - Destinatoin ID

        - Source ID of UE-B’s transmission to UE-A [CATT,6] [LGE,7] [Intel,19] (3)

        - Broadcast destination ID [Intel,19]

    - Request signaling

      - Cast type

        - Unicast [Futurewei,2] [Huawei,3] [LGE,7] [Fraunhofer,29] [Panasonic,31] (5)

        - Groupcast [Nokia,1] [Futurewei,2] [Fraunhofer,29] (3)

      - Source ID

        - Source ID to be used for UE-B’s transmission to UE-A [LGE,7] [Mitsubishi,18] (2)

      - Destinatoin ID

        - Destination ID to be used for UE-B’s transmission to UE-A [LGE,7] [Mitsubishi,18] (2)

    - Inter-UE coordination triggered by a condition rather than request reception

      - Cast type

        - Unicast [Futurewei,2] [Huawei,3] [LGE,7] [Spreadtrum,14] [Fraunhofer,29] (5)

        - Groupcast [Nokia,1] [Futurewei,2] [LGE,7] [OPPO,16](with small number of group members) [Fraunhofer,29] [Panasonic,31] (6)

        - Broadcast [Intel,19] [Panasonic,31] (2)

      - Source ID

        - One of source IDs available for UE-B’s transmission [LGE,7]

      - Destinatoin ID

        - One of destination IDs available for UE-B’s transmission [LGE,7]

        - (Pre)configured broadcast destination ID [Intel,19]

- Finalization of behaviour of UE-B receiving resource set(s) from UE-A(s)

  - Scheme 1 with preferred resource set Option A

    - When MAC CE is used,

      - Alt 1: [Futurewei,2] [Huawei,3] [vivo,4] [CATT,6] [LGE,7] [Sony,8] [DCM,12] [Apple,20] [ZTE,21] [CMCC,28] (10)

        - S_A report from PHY layer of UE-B is the same as the outcome after Step 7) of Rel-16 TS 38.214 Section 8.1.4

        - MAC layer selects resources using S_A and the received preferred resource set

          - MAC layer firstly selects resources for transmissions within the intersection of S_A and the preferred resource set until it becomes impossible to select a resource within the intersection under the constraint defined in Rel-16.

          - After this, if the number of selected resources is smaller than the required number of transmissions for a TB, MAC layer selects resources for the remaining transmissions outside the intersection either inside S_A or inside preferred resource set first then  S_A based on (pre-)configured behavior or attributes of UE-A.

      - Alt 2: [Intel,19] [xiaomi,22] [Ericsson,30] (3)

        - Physical layer at UE-B reports the intersection set between the preferred resource set and S_A obtained after Step 7) of Rel-16 TS 38.214 Section 8.1.4 and the S_A to higher layer for its resource (re-)selection,

          - If the number of candidate single-slot resources belonging to the intersection set is smaller than a threshold,

            - Physical layer at UE-B includes replenishes the intersection set by adding preferred resources that belong to S_A

      - Alt 3: [Fujitsu,5] [Samsung,10] (2)

        - Physical layer at UE-B reports the intersection set between the preferred resource set and S_A obtained after Step 7) of Rel-16 TS 38.214 Section 8.1.4 and the S_A to higher layer for its resource (re-)selection,

          - If the number of candidate single-slot resources belonging to the intersection set is smaller than a threshold,

            - Physical layer at UE-B includes replenishes the intersection set by adding preferred resources that have been excluded in Step 5) of Rel-16 TS 38.214 Section 8.1.4.

              - If the size of the updated intersection set is smaller than a threshold, it is up to UE-B’s implementation to determine a set of candidate single-slot resources that is larger than or equal to a threshold and it reports the updated intersection set instead S_A to higher layer for its resource (re-)selection.

    - When 2nd SCI is used,

      - Alt 1: [Futurewei,2] [Huawei,3] [vivo,4] [LGE,7] [Sony,8] [DCM,12] [Apple,20] [CMCC,28] (8)

        - UE-B PHY reports both preferred resource set and S_A obtained after Step 7) of Rel-16 TS 38.214 Section 8.1.4

        - MAC layer selects resources using S_A and the received preferred resource set

          - MAC layer firstly selects resources for transmissions within the intersection of S_A and the preferred resource set until it becomes impossible to select a resource within the intersection under the constraint defined in Rel-16.

          - After this, if the number of selected resources is smaller than the required number of transmissions for a TB, MAC layer selects resources for the remaining transmissions outside the intersection either inside S_A or inside preferred resource set first then  S_A based on (pre-)configured behavior or attributes of UE-A.

      - Alt 2: [Intel,19] [xiaomi,22] [Ericsson,30] (3)

        - Physical layer at UE-B reports the intersection set between the preferred resource set and S_A obtained after Step 7) of Rel-16 TS 38.214 Section 8.1.4 and the S_A to higher layer for its resource (re-)selection,

          - If the number of candidate single-slot resources belonging to the intersection set is smaller than a threshold,

            - Physical layer at UE-B includes replenishes the intersection set by adding preferred resources that belong to S_A

      - Alt 3: [NEC,25] [Fraunhofer,29] (2)

        - PHY layer at UE-B reports both the intersection set between the preferred resource set and S_A obtained after Step 7) of Rel-16 TS 38.214 Section 8.1.4 and the S_A to higher layer for its resource (re-)selection

        - MAC layer selects resources using S_A and the intersection set

          - MAC layer firstly selects resources for transmissions within the intersection set until it becomes impossible to select a resource within the intersection under the constraint defined in Rel-16.

          - After this, if the number of selected resources is smaller than the required number of transmissions for a TB, MAC layer selects resources for the remaining transmissions outside the intersection but inside S_A

      - Alt 4: [Fujitsu,5] (1)

        - Physical layer at UE-B reports the intersection set between the preferred resource set and S_A obtained after Step 7) of Rel-16 TS 38.214 Section 8.1.4 and the S_A to higher layer for its resource (re-)selection,

          - If the number of candidate single-slot resources belonging to the intersection set is smaller than a threshold,

            - Physical layer at UE-B includes replenishes the intersection set by adding preferred resources that have been excluded in Step 5) of Rel-16 TS 38.214 Section 8.1.4.

              - If the size of the updated intersection set is smaller than a threshold, it is up to UE-B’s implementation to determine a set of candidate single-slot resources that is larger than or equal to a threshold and it reports the updated intersection set instead S_A to higher layer for its resource (re-)selection.

      - Alt 5: [CATT,6] (1)

        - Physical layer at UE-B set the received preferred resource set as the candidate resource set (S_A) in step 4)

  - Scheme 1 with preferred resource set Option B

    - When MAC CE is used,

      - Alt 1: [Futurewei,2] [Huawei,3] [vivo,4] [CATT,6] [LGE,7] [Sony,8] [Inel,19] [Apple,20] (8)

        - MAC layer selects resources belonging to the received preferred resource set

          - If preferred resoruces are not sufficient, UE-B performs random selection

            - Supported by [vivo,4] [Inel,19] [Apple,20] (3)

    - When 2nd SCI is used,

      - Alt 1: [Futurewei,2] [Huawei,3] [vivo,4] [CATT,6] [LGE,7] [Sony,8] [Inel,19] [Apple,20] (8)

        - PHY layer at UE-B reports the preferred resource set

        - MAC layer selects resources belonging to the received preferred resource set

          - If preferred resoruces are not sufficient, UE-B performs random selection

            - Supported by [vivo,4] [Inel,19] [Apple,20] (3)

  - Scheme 1 with non-preferred resource set

    - Update the definition of M_total

      - Supported by [Fujitsu,5] [CATT,6] (2)

    - Introduce additional threshold to check amount of candidate resources after the exclusion [Intel,19]

  - Latency bound of inter-UE coordination information to be used for UE-B’s resource (re)selection procedure

    - Provided by UE-B’s request [vivo,4] [Samsung,10] (2)

    - Provided by a (pre)configuration [CATT,6] [xiaomi,22] [Lenovo,23] (3)

    - Implicitly determined to ensure UE-B’s resource selection window is no less than a threshold [LGE,7] [Sharp,24] (2)

    - Based on feedback aging time indicated by inter-UE coordination information [Intel,19] (1)

- Finalization of when and with which information UE-A generates and/or transmits an inter-UE coordination information, including triggering based on condition(s) other than an explicit request

  - Triggering condition(s)

    - Up to UE’s implementation [Futurewei,2] [Huawei,3] [vivo,4](for preferred resource) [LGE,7] [Panasonic,31] (5)

    - Potential/expected resource conflict is detected on the resources reserved by UE-B [Futurewei,2] [OPPO,16] [Fraunhofer,29] (3)

    - UE has data to UE-B which is multiplexed with feedback payload [DCM,12] [Intel,19] (2)

    - UE-A completes its resource selection [vivo,4] [Qualcomm,15] (2)

    - Change in resource to be sent via inter-UE coordination [Nokia,1] (1)

    - Based on CBR, priority, consecutive decoding failures [Futurewei,2] (1)

    - Feedback was not transmitted for a certain amount of time [Intel,19] (1)

    - CBR is higher than a threshold [Apple,20] (1)

    - Distance between UE-A and UE-B is larger than a threshold [xiaomi,22] (1)

    - Number of failure of TB decoding at UE-A side is larger than a threshold [Lenovo,23] (1)

    - UE-A detects a resource re-selection is to be performed by UE-B [Ericsson,30] (1)

  - Sensing window for determining the set of resources

    - Sensing window prior to the transmission time (slot n) of UE-A’s iner-UE coordination information

      - Supported by [Huawei,3] [LGE,7] [Intel,19] (3)

        - [n-T_0, n-T_proc,0]: [Huawei,3]

        - [n-T_0-T_proc,1, n-T_proc,0-T_proc,1]: [LGE,7]

        - [n-X, n-T_proc,1]: [Intel,19]

    - Sensing window prior to the resource selection window for determing the set of resoruces

      - Supported by [Futurewei,2] (1)

- Finalization of when UE-B generates and/or transmits an explicit request

  - Up to UE’s implementation [Futurewei,2] [Huawei,3] [CATT,6] [LGE,7] [Samsung,10] [ZTE,21] [Sharp,24] [Panasonic,31] (8)

  - Priority vlaue of UE-B’s packet is smaller than a threshold [Futurewei,2] [OPPO,16] [xiaomi,22] [NEC,25] (4)

  - Resource (re)selection is triggered by UE-B [OPPO,16] [Intel,19] [xiaomi,22] (3)

  - Remainig PDB of UE-B’s packet is larger than a threshold [Futurewei,2] [OPPO,16] [xiaomi,22] (3)

  - Measured CBR is larger than a threshold [Futurewei,2] [vivo,4] (2)

  - TB(s) arrive at UE-B [vivo,4] [Apple,20] (2)

  - UE-B has data/TB for transmission that can be multiplexed with request to UE-A [DCM,12] [Intel,19] (2)

  - Resource re-selection is expected to be performed by UE-B  [Intel,19] [Fraunhofer,29] (2)

  - UE-B’s sensing results are not available [Futurewei,2] (1)

  - Retransmission time of a prior TB is beyond a threshold [vivo,4] (1)

  - Number of resoruces within the set S_A is larger than a threshold [OPPO,16] (1)

  - UE-B does not have valid inter-UE coordination information [Intel,19] (1)

  - Elapsed time from the previous inter-UE coordination feedback request exceeds pre-configured value [Intel,19] (1)

  - Number of sensing slots at UE-B is belo a threshold [Apple,20] (1)

  - NACK ratio is larger than a threshold [NEC,25] (1)

- Finalization of resource selection and/or multiplexing with sidelink transmissions for UE-A’s inter-UE coordination information and UE-B’s explicit request

  - Resource selection

    - UE-A performs its resource (re)selection according to the same procedure in Rel-16 TS 38.214 Section 8.1.4 to transmit the inter-UE coordination information to UE-B.

      - Supported by [Futurewei,2] [Huawei,3] [vivo,4] [LGE,7] [ITL,11] [Intel,19] [Apple,20] (7)

    - UE-B performs its resource (re)selection according to the same procedure in Rel-16 TS 38.214 Section 8.1.4 to transmit the request for the inter-UE coordination information to UE-A if UE-B performs sensing/resource exclusion.

      - Alt 1:

        - Otherwise, at least UE-B can perform random selection

          - Supported by [Futurewei,2] [vivo,4] [LGE,7] [Apple,20] (4)

      - Alt 2:

        - Otherwise, UE-B performs random selection, or uses resources indicated by UE-A to transmit for the request to UE-A and receive the inter-UE coordination information from UE-A.

          - Supported by [Huawei,3] (1)

  - Multiplexing with other data

    - Multiplexing inter-UE coordination information with other data

      - Support: [Futurewei,2](2nd SCI+MAC CE) [vivo,4] [LGE,7] [Qualcomm,15] (4)

        - Only if they have the same IDs [LGE,7]

      - Not support: [Futurewei,2](MAC CE only) (1)

      - Mandated: [DCM,12] [Intel,19] (2)

    - Multiplexing a request signaling with other data

      - Support: [Futurewei,2] [vivo,4] [LGE,7] [Panasonic,31] (4)

        - Only if they have the same IDs [LGE,7]

      - Not support:

      - Mandated: [DCM,12] [Intel,19] (2)

- Finalization of prioritization of inter-UE coordination information and explicit request

  - Priority value

    - Inter-UE coordination information triggered by UE-B’s request

      - (pre)configured [LGE,7] [Samsung,10] [Qualcomm,15] [Sharp,24] [Panasonic,31] (5)

      - Indicated by UE-B’s request [Futurewei,2] [Huawei,3] [Intel,19] [Apple,20] (4)

      - Priority value used for the resource selection of the inter-UE coordination information transmission [vivo,4] (1)

      - Smaller priority value between priority value of data (if present) and priority value conveyed on the request [DCM,12] (1)

      - Up to UE-A’s implementation [CAICT,13] (1)

    - Request signaling

      - (pre)configured [CATT,6] ]LGE,7] [Samsung,10] [Sharp,24] [Panasonic,31] (5)

      - Priority value to be used for UE-B’s transmission [Futurewei,2] [Huawei,3] [CATT,6] (3)

      - Priority value used for the resource selection of the inter-UE coordination information transmission [vivo,4] (1)

      - Smaller priority value between priority value of data (if present) and priority value conveyed on the request [DCM,12] (1)

    - Inter-UE coordination information triggered by a condition rather than request reception

      - (Pre)configured value [Futurewei,2] [LGE,7] [Qualcomm,15] [Intel,19] [Apple,20] [Sharp,24] [Panasonic,31] (7)

      - Lowest priority value [DCM,12](when no data is multiplexed) [Intel,19] (2)

      - Indicated by UE-B’s prior SCI [Futurewei,2] (1)

      - UE-A’s implementation [Huawei,3] (1)

      - Priority value used for the resource selection of the inter-UE coordination information transmission [vivo,4] (1)

      - Priority value of the data [DCM,12](when data is multiplexed) (1)

      - Up to UE-A’s implementation [CAICT,13] (1)

- Combination of preferred/non-preferred resources with explicit request/condition triggers

  - All combinations

    - Supported by [Nokia,1] [Futurewei,2] [Sony,8] (3)

  - A subset of combinations

    - Alt 1: [Fujitsu,5] [DCM,12] [Qualcomm,15] (3)

      - Inter-UE coordination information of preferred resource set triggered by an explicit request

      - Inter-UE coordination information of non-preferred resource set triggered by a condition other than the explicit request reception

    - Alt 2: [CATT,6] [Samsung,10] [Panasonic,31] (3)

      - Inter-UE coordination information of preferred resource set triggered by an explicit request

      - Inter-UE coordination information of non-preferred resource set triggered by an explicit request

    - Alt 3: [Apple,20] [Fraunhofer,29] (2)

      - Inter-UE coordination information of preferred resource set triggered by an explicit request

      - Inter-UE coordination information of non-preferred resource set triggered by an explicit request

      - Inter-UE coordination information of non-preferred resource set triggered by a condition other than the explicit request reception

  - Scheme 2

- Finalization of determination of PSFCH resource/index for conflict indication

  - Frequency and code domain resources derived by

    - m_CS

      - Option 1: [Futurewei,2] [vivo,4](for option 2 timing) [CATT,6] [LGE,7](for option 2 timing) [InterDigial,9] [Spreadtrum,14] [OPPO,16] (7)

        - 0 for Condition 2-A-1,

        - 6 for Condition 2-A-2

      - Option 2: [Samsung,10] [DCM,12] [Apple,20] [ZTE,21] [Ericsson,30] [Panasonic,31] (6)

        - 0

      - Option 3: [vivo,4](for option 1 timing) [LGE,7](for option 1 timing) [Intel,19] (3)

        - 0 for 2nd reserved resource,

        - 6 for 3rd reserved resource

      - Option 4: [Huawei,3] (1)

        - 0 for 2nd reserved resoruce

        - 2 for 3rd reserved resoruce

        - 4 for both 2nd and 3rd reserved resource

        - 6 for the case when no UE transmitted SCI with periodic reservation on the non-moniotred slot of UE-B

        - 8 for Condition 2-A-2

    - m_0/PRB determination based on PSFCH resource index

      - In the same way as specified in TS 38.213 Section 16.3

        - Supported by [CATT,6] [LGE,7] [DCM,12] [OPPO,16] [Ericsson,30] (5)

      - Update it to indicate the time location and/or type of expected/potential resrouce conflict

        - Supported by [Futurewei,2] [Samsung,10] [CAICT,13] (3)

      - Update it to indicate whether UE-A is the intended receiver of UE-B or not

        - Supported by [Fujitsu,5] (1)

      - (pre)configure the value of m_0

        - Supported by [Panasonic,31] (1)

  - When PSFCH occasion is derived by a slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B’s SCI, time gap between the PSFCH and SCI(s) scheduling conflicting TBs is larger than or equal to X value

    - X = T_proc,0: [LGE,7] [InterDigital,9] [Sharp,24] (3)

    - X = sl-MinTimeGapPSFCH: [Samsung,10] [DCM,12] [Qualcomm,15] [ETRI,17] (4)

    - X = T_3: [Intel,19] (1)

- Finalization of behaviour of UE-B receiving a conflict indication from UE-A

  - Resource reselection upon a resource conflict indication

    - Alt 1: [LGE,7](for Condition 2-A-1) [Samsung,10] [Intel,19](for Condition 2-A-1) [Apple,20] [xiaomi,22] (5)

      - Among reserved resources indicated by UE-B’s SCI, PHY layer at UE-B reports resource(s) indicated by conflict indicator and S_A as specified in Rel-16 TS 38.214 Section 8.1.4 to higher layer.

      - Higher layer at UE-B re-selects the reported resource(s) among the S_A excluding the reported resource(s).

    - Alt 2: [Futurewei,2] [CATT,6] [Fraunhofer,29] (3)

      - UE-B performs exclusion of the candidate single-slot resources overlapping with resources corresponding to the expected/potential resource conflict after Step 6) of TS 38.214 Section 8.1.4

    - Alt 3: [LGE,7](for condition 2-A-2 or without condition indication) [Intel,19](for Condition 2-A-2) (2)

      - Among resources in slot(s) including reserved resources indicated by UE-B’s SCI, PHY layer at UE-B reports resources in slot(s) including resource(s) indicated by conflict indicator and S_A as specified in Rel-16 TS 38.214 Section 8.1.4 to higher layer.

      - Higher layer at UE-B re-selects the reported resource(s) among the S_A excluding the reported resource(s).

    - Alt 4: [vivo,4] (1)

      - UE-B performs exclusion of the candidate single-slot resources overlapping with resources corresponding to the expected/potential resource conflict after Step 7) of Rel-16 TS 38.214 Section 8.1.4

    - Alt 5: [DCM,12] (1)

      - UE-B excludes the single-slot resources corresponding to the collision indication right before resource exclusion based on its own sensing results.

      - PHY layer reports S_A with ‘resource conflict’ to MAC layer.

- Finalization of prioritization of conflict indication

  - Priority value of PSFCH transmission at UE-A

    - Smallest priority value of the confliting TBs

      - Supported by [Fujitsu,5] [LGE,7]((pre)configurable) [Samsung,10] [DCM,12] [Panasonic,31] (5)

    - Indicated by UE-B’s SCI

      - Supported by [Futurewei,2] [Apple,20] [Lenovo,23] [NEC,25] (4)

    - (pre)configured

      - Supported by [LGE,7] [Qualcomm,15] (2)

  - Priority value of PSFCH reception at UE-B

    - Indicated by UE-B’s SCI

      - Supported by [Futurewei,2] [LGE,7]((pre)configurable) [Samsung,10] [DCM,12] [Apple,20] [Lenovo,23] [NEC,25] [Panasonic,31] (8)

    - (pre)configured

      - Supported by [LGE,7] [Qualcomm,15] (2)

    - Smallest priority value of the confliting TBs

      - Supported by [Fujitsu,5] (1)

  - Prioritization rule

    - PSFCH for SL HARQ-ACK feedback is always prioritized over PSFCH for Scheme 2

      - Supported by [Huawei,3] [vivo,4] [Qualcomm,15] [ETRI,17] [Intel,19] [Apple,20] [Ericsson,30] (7)

    - Reuse the prioritization rule as specified in TS 38.213 Section 16.2.4.2

      - Supported by [Futurewei,2] [CATT,6] [LGE,7] [DCM,12] [ETRI,17](2nd pref with configurability) [Lenovo,23] (6)

- Finalization of how to determine UE-B among UEs scheduling conflicting TBs, including whether/how to handle, or differently handle, the case when at least one of UEs scheduling conflicting TBs doesn’t support Scheme 2

  - Based on UE capability on supporting Scheme 2 indicated by UE-B’s SCI: [Nokia,1] [Futurewei,2] [CATT,6] [InterDigital,9] [DCM,12] [OPPO,16] [ETRI,17] [Intel,19](2nd SCI signaling) [Apple,20] [ZTE,21] [Sharp,24] (11)

    - At least one of UEs scheduling conflicting TBs does not support scheme 2, all other UEs with unknown capability or supporting scheme 2 are UE-Bs

      - Supported by [Futurewei,2] [CATT,6] [OPPO,16] [Apple,20] (4)

    - Seleting UE-B based on priority is applied to a case when all the UEs are unknown or supporintg scheme 2

      - Supported by [Futurewei,2] [Apple,20] [ZTE,21] (3)

    - Drop PSFCH TX when the selected UE-B does not support Scheme 2

      - Supported by [ETRI,17] (1)

  - Based on whether PSFCH occasion(s) for resource conflict indication is paased or not: [Huawei,3] [LGE,7] (2)

    - Seleting UE-B based on priority is applied to UEs whose PSFCH occassoins for a resource conflict indication is not yet passed

      - Supported by [Huawei,3] [LGE,7] (2)

  - Based on priority value of UE-B’s transmission [InterDigital,9] (1)

  - Cast type of UE-B’s transmission

    - Any cast type: [Fujitsu,5] [OPPO,16](UE-A is non-destination of UE-B) [Ericsson,30] (3)

    - Unicast and groupcast: [CATT,6] [OPPO,16](UE-A is a destination of UE-B) (2)

  - No additional criteria [Samsung,10] [Qualcomm,15] [Panasonic,31] (3)

  - Scheme 1 and 2

- Details on a (pre)configuration to enable or disable or control feature of the inter-UE coordination

  - Alt 1: [vivo,4] [Qualcomm,15] (2)

    - Scheme 1 with preferred-resource indication

    - Scheme 1 with non-preferred resource indication

    - Scheme 2

  - Alt 2: [LGE,7] [Sony,8] (2)

    - Scheme 1

      - Preferred resource vs non-preferred resource

      - Expclicit request-based manner vs condition-based manner

    - Scheme 2

  - Alt 3: [Futurewei,2]

    - Scheme 1 with explicit request-based manner

    - Scheme 1 with condition-based manner

    - Scheme 2 + Scheme 1 with condition-based manner

  - Alt 4: [CATT,7]

    - Scheme 1 with explicit request-based manner

    - Scheme 1 with condition-based manner

    - Scheme 2

  - Alt 5: [ZTE,21]

    - Scheme 1

    - Scheme 2

- Others

  - Further consideration on modifying condition or procedures of determing the set of resources [Nokia,1] [vivo,4] [Fujitsu,5] [ITL,11] [Qualcomm,15] [OPPO,16] [Intel,19] [ZTE,21] [Lenovo,23] [Fraunhofer,29] (10)

  - Further restrict or expand on the condition to be UE-A and/or UE-B [Huawei,3] [vivo,4] [InterDigital,9] [Mitsubishi,18] [Lenovo,23] [Fraunhofer,29] (6)

  - Further consideration on specifying conditions to skip inter-UE coordination information transmission [LGE,7] [DCM,12] [Mitsubishi,18] [Intel,19] (4)

  - Further consideration on restricting UE(s) transmitting a resource conflict indication [Nokia,1] [vivo,4] [Fujitsu,5] (3)

  - Further consideration on specifying additional details on Condition 1-A-2/1-B-2/2-A-2 [vivo,4] [DCM,12] [Qualcomm,15] (3)

  - Further consideration on skipping the received resource conflict indication [Fujitsu,5] [Fraunhofer,29] [Ericsson,30] (3)

  - Further consideration on handling the case where UE-B receives multiple inter-UE coordinaotin information from same or different UE-A [Samsung,10] [DCM,12] [Apple,20] [Fraunhofer,29] (3)

  - Further consideration on multiplexing multiple inter-UE coordination information (e.g. request-based and condition-based information or preferred and non-preferred resources) [Intel,19] [Lenovo,23] [Ericsson,30] (3)

  - Further consideration on determining applicable scenario to transmit a preferred resource set or a non-preferred resource set [Nokia,1] [OPPO,16] (2)

  - Further consideration on modifying UE-B’s resource (re)selection procedure upon a reception on the set of non-preferred resources [Nokia,1] [Qualcomm,15] (2)

  - Further consideration on dedicated resources for inter-UE coordination information transmission in Scheme 1 [Nokia,1] [Qualcomm,15] (2)

  - Further consideration on tie-breaking for the case when conflicting TBs have the same priority [Futurewei,2] [Fujitsu,5] (2)

  - Further consideration on ID sharing mechanism between UE-A and UE-B [Nokia,1] (1)

  - Further considerinatoin on modifying a (pre)configuration granularity for a RSRP threshold [Nokia,1] (1)

  - Further consideration modifying UE-B’s resource (re)selection procedure upon a reception of 1st SCI from UE-A [vivo,4] (1)

  - Further consideration on changing 2nd SCI format size by using 1st SCI format [Apple,20] (1)

  - Further consideration on inter-UE coordination with mode 1 operation [Lenovo,23] (1)

  - Further consideration on DRX active time for determing the set of resrouces [ASUSTeK,26] (1)

  - Further consideration on updating UE-A’s resource (re)selection procedure for its transmission based on UE-A’s inter-UE coordination information [ASUSTeK,26] (1)

- Reference

- R1-2200016	Inter-UE coordination for Mode 2 enhancements	Nokia, Nokia Shanghai Bell

- R1-2200022	Discussion on techniques for inter-UE coordination	FUTUREWEI

- R1-2200042	Inter-UE coordination in sidelink resource allocation	Huawei, HiSilicon

- R1-2200092	Remaining issues on mode-2 enhancements	vivo

- R1-2200126	Considerations on inter-UE coordination for mode 2 enhancements	Fujitsu

- R1-2200131	Remaining issues  on Inter-UE coordination for Mode 2 enhancements	CATT, GOHIGH

- R1-2200169	Discussion on inter-UE coordination for Mode 2 enhancements	LG Electronics

- R1-2200182	Discussion on inter-UE coordination for Mode 2 enhancements	Sony

- R1-2200190	Discussions on remaining issues for Mode 2 inter-UE coordination	InterDigital, Inc.

- R1-2200211	On Inter-UE Coordination for Mode2 Enhancements	Samsung

- R1-2200225	Inter-UE coordination for mode 2 enhancements	ITL

- R1-2200242	Resource allocation for reliability and latency enhancements	NTT DOCOMO, INC.

- R1-2200270	Considerations on mode2 enhancements	CAICT

- R1-2200282	Discussion on inter-UE coordination in sidelink resource allocation	Spreadtrum Communications

- R1-2200307	Reliability and Latency Enhancements for Mode 2	Qualcomm Incorporated

- R1-2200348	Inter-UE coordination in mode 2 of NR sidelink	OPPO

- R1-2200360	Discussion on inter-UE coordination for Mode 2 enhancements	ETRI

- R1-2200361	Inter-UE coordination for enhanced resource allocation	Mitsubishi Electric RCE

- R1-2200385	Inter-UE Coordination Solutions for Sidelink Communication	Intel Corporation

- R1-2200426	On Remaining Issues of Inter-UE Coordination	Apple

- R1-2200437	Remaining issues on the inter-UE coordiantion	ZTE, Sanechips

- R1-2200450	Discussion on inter-UE coordination	xiaomi

- R1-2200475	Inter-UE coordination for Mode 2 enhancements	Lenovo, Motorola Mobility

- R1-2200504	Discussion on inter-UE coordination for mode 2 enhancements	Sharp

- R1-2200511	Discussion on mode 2 enhancements	NEC

- R1-2200524	Remaining issues on V2X mode 2 enhancements	ASUSTeK

- R1-2200554	Discussion on Mode 2 enhancements	MediaTek Inc.

- R1-2200594	Remaining issues on inter-UE coordination for mode 2 enhancement	CMCC

- R1-2200630	Inter-UE Coordination for Mode 2 Enhancements	Fraunhofer HHI, Fraunhofer IIS

- R1-2200642	Details on mode 2 enhancements for inter-UE coordination	Ericsson

- R1-2200675	Inter-UE coordination for Mode 2 enhancements	Panasonic

- Appendix

  - Conclusions made in RAN1#103-e meeting

- Conclusion:

  - The schemes of inter-UE coordination in Mode 2 are categorized as being based on the following types of “A set of resources” sent by UE-A to UE-B:

    - UE-A sends to UE-B the set of resources preferred for UE-B’s transmission

        - e.g., based on its sensing result

    - UE-A sends to UE-B the set of resources not preferred for UE-B’s transmission

        - e.g., based on its sensing result and/or expected/potential resource conflict

    - UE-A sends to UE-B the set of resource where the resource conflict is detected

    - FFS: details of resource conflict, e.g., including type of resource conflict

    - FFS: details of sensing operation at UE-A side

    - FFS: which type(s) of resource set information is(are) beneficial/feasible to which cast type(s)

    - Note: these different types may be used in combination with each other

  - From RAN1 perspective, further study on the feasibility/benefit of inter-UE coordination is required

  - Send an LS to RAN plenary

    - Final LS in R1-2009841

- Conclusion:

  - For the schemes of inter-UE coordination identified as feasible/beneficial, at least the following aspects are further discussed.

    - How/when UE-A determines the contents of ”A set of resources”, including consideration of UL scheduling

    - When UE-A sends ”A set of resources” to UE-B, including which UE(s) sends it

    - How UE-A and UE-B are determined

    - How UE-A sends ”A set of resources” to UE-B, including container used for carrying it, implicitly or explicitly or both

    - How/when/whether UE-B receives “A set of resources” and takes it into account in the resource selection for its own transmission

    - How/whether to define the relationship between support/signaling of inter-UE coordination and cast type

  - Conclusions made in RAN1#104-e meeting

- Conclusion:

  - RAN1 concludes that the inter-UE coordination in Mode 2 is feasible, and is beneficial (e.g., reliability, etc.) compared to Rel-16 Mode 2 RA, and thus recommends specification of the feature.

    - The detailed observations can be found in the attachment of the LS

- Draft LS in R1-2102165, along with the attachment R1-2102166, is approved (with a typo fix)

  - Final LS in R1-2102168

  - Agreements made in RAN1#104bis-e meeting

- Agreement:

  - Support the following schemes of inter-UE coordination in Mode 2:

    - Inter-UE Coordination Scheme 1:

      - The coordination information sent from UE-A to UE-B is the set of resources preferred and/or non-preferred for UE-B’s transmission

        - FFS details including a possibility of down-selection between the preferred resource set and the non-preferred resource set, whether or not to include any additional information other than indicating time/frequency of the resources within the set in the coordination information

      - FFS condition(s) in which Scheme 1 is used

    - Inter-UE Coordination Scheme 2:

      - The coordination information sent from UE-A to UE-B is the presence of expected/potential and/or detected resource conflict on the resources indicated by UE-B’s SCI

        - FFS details including a possibility of down-selection between the expected/potential conflict and the detected resource conflict

      - FFS condition(s) in which Scheme 2 is used

- Agreement:

  - Study further to determine the conditions for UEs to be UE-A(s)/UE-B(s) for inter-UE coordination:

    - Details include applicable scenario(s)/inter-UE coordination scheme(s)

    - E.g., only UE(s) among the intended receiver(s) of UE-B can be a UE-A, any UE can be a UE-A, high-layer configured, etc.

      - Including the possibility of being subject to certain conditions and/or capability

- Agreement:

  - When UE-B receives the inter-UE coordination information from UE-A, consider at least one of the following options (with details FFS including possibly down-selecting/merging one or more of the options below, applicable scenario(s)/condition(s) for each option, UE behavior) for UE-B’s to take it into account in the resource (re)-selection for its own transmission

    - For scheme 1:

      - Option 1-1: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based on both UE-B’s sensing result (if available) and the received coordination information

      - Option 1-2: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based only on the received coordination information

      - Option 1-3: UE-B’s resource(s) to be re-selected based on the received coordination information

      - Option 1-4: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based on the received coordination information

    - For scheme 2:

      - Option 2-1: UE-B can determine resource(s) to be re-selected based on the received coordination information

      - Option 2-2: UE-B can determine a necessity of retransmission based on the received coordination information

  - Agreements made in RAN1#106-e meeting

- Agreement:

  - For scheme 1, the following inter-UE coordination information signalling from UE-A is supported. FFS details including condition(s)/scenario(s) under which each information is enabled to be sent by UE-A and used by UE-B.

    - Set of resources preferred for UE-B’s transmission

    - Set of resources non-preferred for UE-B’s transmission

- Agreement:

  - For scheme 2, the following inter-UE coordination information signalling from UE-A is supported. FFS details including condition(s)/scenario(s) under which each information is enabled to be sent by UE-A and used by UE-B

    - Presence of expected/potential resource conflict on the resources indicated by UE-B’s SCI

      - FFS: UE behaviour when the presence of expected/potential resource conflict is detected by the transmitter

    - FFS: Whether to additionally support the presence of detected resource conflict on the resources indicated by UE-B’s SCI

- Agreement:

  - In scheme 1, the following is supported for UE(s) to be UE-A(s)/UE-B(s) in the inter-UE coordination information transmission triggered by an explicit request in Mode 2:

    - A UE that sends an explicit request for inter-UE coordination information can be UE-B

    - A UE that received an explicit request from UE-B and sends inter-UE coordination information to the UE-B can be UE-A

    - (Working assumption) At least a destination UE of a TB transmitted by UE-B can be UE A

    - The above feature can be enabled or disabled or controlled by (pre-)configuration

      - FFS: Details on how to support this, including (pre-)configuration signaling granularity

    - FFS: Additional details and conditions on UE-A and UE-B

  - (Working Assumption) In scheme 1, the following is supported for UE(s) to be UE-A(s)/UE-B(s) in the inter-UE coordination information transmission triggered by a condition other than explicit request reception in Mode 2:

    - A UE that satisfies the condition mentioned in the main bullet and sends inter-UE coordination information is UE-A

    - A UE that received inter-UE coordination information from UE-A and uses it for resource (re-)selection is UE-B

    - The above feature can be enabled or disabled or controlled by (pre-)configuration

      - FFS: Details on how to support this, including (pre-)configuration signaling granularity

    - FFS: Additional details and conditions on UE-A and UE-B

- Agreement:

  - In scheme 2, at least the following is supported for UE(s) to be UE-A(s)/UE-B(s) in the inter-UE coordination transmission triggered by a detection of expected/potential resource conflict(s) in Mode 2:

    - A UE that transmitted PSCCH/PSSCH with SCI indicating reserved resource(s) to be used for its transmission, received inter-UE coordination information from UE-A indicating expected/potential resource conflict(s) for the reserved resource(s), and uses it to determine resource re-selection is UE-B

    - A UE that detects expected/potential resource conflict(s) on resource(s) indicated by UE-B’s SCI sends inter-UE coordination information to UE-B, subject to satisfy one of the following conditions, is UE-A

      - (Working assumption) At least a destination UE of one of the conflicting TBs, i.e., TBs to be transmitted in the expected/potential conflicting resource(s)

        - Whether a non-destination UE of a TB transmitted by UE-B can be UE-A is (pre-)configured

      - FFS: Additional details and condition(s) on UE-A and UE-B

    - The above feature can be enabled or disabled or controlled by (pre-)configuration

      - FFS: Details on how to support this, including (pre-)configuration signaling granularity

    - FFS: Definition of expected/potential resource conflict(s) and other details (if any)

- Agreement:

  - In scheme 2, the following UE-B’s behavior in its resource (re)selection is supported when it receives inter-UE coordination information from UE-A:

    - UE-B can determine resource(s) to be re-selected based on the received coordination information

      - UE-B can reselect resource(s) reserved for its transmission when expected/potential resource conflict on the resource(s) is indicated

        - FFS: Other details (if any)

- Agreement:

  - In scheme 1, at least following UE-B’s behavior in its resource (re-)selection is supported when it receives inter-UE coordination information from UE-A:

    - For preferred resource set, the following two options are supported:

      - Option A): UE-B’s resource(s) to be used for its transmission resource (re-)selection is based on both UE-B’s sensing result (if available) and the received coordination information

        - UE-B uses in its resource (re-)selection, resource(s) belonging to the preferred resource set in combination with its own sensing result

          - UE-B uses in its resource (re-)selection, resource(s) not belonging to the preferred resource set when condition(s) are met

            - FFS: Details of condition(s)

          - This option is supported when UE-B performs sensing/resource exclusion

          - FFS: Other details (if any)

      - Option B): UE-B’s resource(s) to be used for its transmission resource (re-)selection is based only on the received coordination information

        - UE-B uses in its resource (re-)selection, resource(s) belonging to the preferred resource set

          - This option is supported at least when UE-B does not support sensing/resource exclusion

            - FFS: Whether the support is conditional or UE capability

          - FFS: Other details (if any)

      - FFS: Other option(s), and other details (if any)

    - For non-preferred resource set,

      - UE-B’s resource(s) to be used for its transmission resource (re-)selection is based on both UE-B’s sensing result (if available) and the received coordination information

        - UE-B excludes in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set

          - FFS: Details including

            - Whether/how UE-B can use in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set, definition of the overlap, and other details (if any)

            - When UE-B excludes in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set

        - FFS: UE-B reselects in its resource (re-)selection, resource(s) to be used for its transmission when the resource(s) are fully/partially overlapping with the non-preferred resource set

      - FFS: Other option(s), and other details (if any)

- Agreement:

  - In scheme 2, at least the following is supported to determine inter-UE coordination information:

    - Among resource(s) indicated by UE-B’s SCI, UE-A considers that expected/potential resource conflict occurs on the resource(s) satisfying at least one of the following condition(s):

      - Condition 2-A-1:

        - Other UE’s reserved resource(s) identified by UE-A are fully/partially overlapping with resource(s) indicated by UE-B’s SCI in time-and-frequency

        - FFS: Other details (if any)

        - FFS: Whether/how to specify additional criteria and other details (if any) including signaling details of conflict indication

      - (Working Assumption) Condition 2-A-2:

        - Resource(s) (e.g., slot(s)) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B due to half duplex operation

          - FFS: Other details (if any)

      - FFS: Other condition(s)

    - FFS: Other details (if any)

- Agreement:

  - In scheme 1, at least the following is supported to determine inter-UE coordination information of preferred resource set:

    - UE-A considers any resource(s) satisfying all the following condition(s) as set of resource(s) preferred for UE-B’s transmission

      - Condition 1-A-1:

        - Resource(s) excluding those overlapping with reserved resource(s) of other UE identified by UE-A whose RSRP measurement is larger than a RSRP threshold

          - FFS: Other details (if any)

      - FFS: Condition 1-A-2:

        - Resource(s) excluding slot(s) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B

          - FFS: Other details (if any)

      - FFS: Condition 1-A-3:

        - Resource(s) satisfying UE-B’s traffic requirement (if available)

          - FFS: Other details (if any)

      - FFS: Other condition(s)

    - FFS: Other details (if any)

- Agreement:

  - In scheme 1, at least the following is supported to determine inter-UE coordination information of non-preferred resource set:

    - UE-A considers any resource(s) satisfying at least one of the following condition(s) as set of resource(s) non-preferred for UE-B’s transmission

      - Condition 1-B-1:

        - Reserved resource(s) of other UE identified by UE-A from other UEs’ SCI (including priority field) and RSRP measurement

          - FFS: Other details (if any)

      - FFS: Condition 1-B-2:

        - Resource(s) (e.g., slot(s)) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B

          - FFS: Other details (if any)

      - FFS: Other condition(s)

    - FFS: Other details (if any)

  - Agreements made in RAN1#106bis-e meeting

- Agreement:

  - For Scheme 2, PSFCH format 0 is used to convey the presence of expected/potential resource conflict on reserved resource(s) indicated by UE-B’s SCI

- Agreement:

  - For Condition 2-A-1 of Scheme 2, down-select one or more of following additional criteria to determine resource(s) where expected/potential resource conflict occurs

    - Option 1: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger than a RSRP threshold according to the priorities included in the SCI:

      - prio_TX and prio_RX are the priorities indicated in the SCI making the overlapping reservations

      - Strive to reuse Rel-16 specification wherever possible

    - Option 2: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is within a (pre)configured RSRP threshold compared to the RSRP measurement of UE-B’s reserved resource.

      - FFS: Whether the threshold depends on priority

    - Option 3: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) and the other UE is within a distance threshold of UE-B as determined by both UEs’ SCIs.

    - Option 4: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger a (pre)configured RSRP threshold compared to the RSRP measurement of UE-B’s reserved resource.

      - FFS: Whether the threshold depends on priority

    - FFS: In case of collisions of resources for two UEs having TBs with UE A as destination UE, if needed

- Working Assumption

  - For Condition 1-B-1 of Scheme 1, the following two options are supported

    - Option 1: Reserved resource(s) of other UE(s) identified by UE-A whose RSRP measurement is larger than a (pre)configured RSRP threshold which is determined by at least priority value indicated by SCI of the UE(s)

    - Option 2: Reserved resource(s) of other UE identified by UE-A whose RSRP measurement is smaller than a (pre)configured RSRP threshold which is determined by at least priority value indicated by SCI of the UE(s) when UE-A is a destination of a TB transmitted by the UE(s)

- Working Assumption

  - For Scheme 1 with non-preferred resource set, support following condition:

    - Condition 1-B-2:

      - Resource(s) (e.g., slot(s)) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B due to half duplex operation

- Agreement:

  - For Condition 1-A-1 of Scheme 1, the set of resources preferred for UE-B’s transmission is a form of candidate single-slot resource as specified in Rel-16 TS 38.214 Section 8.1.4

    - When the inter-UE coordination information transmission is triggered by UE-B’s explicit request, the candidate single-slot resource(s) are determined in the same way according to Rel-16 TS 38.214 Section 8.1.4 with at least following parameters provided by signaling from UE-B. FFS whether or not to apply RSRP threshold increase in Step 7) of Rel-16 TS 38.214 Section 8.1.4.

      - Priority value to be used for PSCCH/PSSCH transmission

        - It replaces prio_TX

      - Number of sub-channels to be used for PSSCH/PSCCH transmission in a slot

        - It replaces L_subCH

      - Resource reservation interval

        - It replaces P_rsvp_TX

      - FFS: Starting/ending time location of resource selection window

    - FFS : In addition to Rel-16 procedure, use inter-UE coordination information from other UEs

      - If there is no consensus in RAN1#106bis-e, no further discussions for Rel-17

- Conclusion:

  - No consensus that UE-A uses inter-UE coordination information from other UEs when it determines the preferred resource set for Condition 1-A-1 of Scheme 1.

- Working Assumption

  - For Scheme 1 with preferred resource set, support following condition:

    - Condition 1-A-2:

      - Resource(s) excluding slot(s) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B due to half duplex operation

      - This can be disabled by RRC (pre-)configuration

- Agreement:

  - For allocating PSFCH resources in Scheme 2, at least following can be (pre)configured separately from those for SL HARQ-ACK feedback.

    - Set of PRBs for PSFCH transmission/reception (sl-PSFCH-RB-Set)

- Agreement:

  - For Scheme 2,

    - Index of a PSFCH resource for inter-UE coordination information transmission is determined in the same way according to Rel-16 TS 38.213 Section 16.3 with at least following modification

      - P_ID is L1-Source ID indicated by UE-B’s SCI

      - M_ID is 0

    - FFS: How to set m_CS

    - FFS: How to set m_0

    - FFS: Whether M_ID can be (pre)configured

  - Agreements made in RAN1#107-e meeting

- Agreement:

  - A resource pool level (pre-)configuration uses either of the following options

    - Option 1: PSFCH occasion is derived by a slot where UE-B’s SCI is transmitted

      - Reuse PSSCH-to-PSFCH timing as specified in TS 38.213 Section 16.3 to determine the PSFCH occasion for resource conflict indication

      - Time gap between the PSFCH and a slot where expected/potential resource conflict occurs is larger than or equal to T_3

    - Option 2: PSFCH occasion is derived by a slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B’s SCI

      - UE-A transmits the PSFCH in a latest slot that includes PSFCH resources for inter-UE coordination information and is at least T_3 slots of the resource pool before the PSSCH resource indicated by UE-B’s SCI in which expected/potential resource conflict occurs

      - FFS: How to account for processing timeline

    - Note that it is possible not to configure either option1 or option 2.

- Agreement:

  - For Condition 1-A-2 of Scheme 1, the set of resources preferred for UE-B’s transmission is a form of candidate single-slot resource as specified in Rel-16 TS 38.214 Section 8.1.4

    - UE-A excludes candidate single-slot candidate(s) belonging to “slot(s) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B due to half duplex operation” after Step 6) of TS 38.214 Section 8.1.4

- Agreement:

  - When PSFCH TX/RX for Scheme 2 is overlapping with LTE SL TX/RX and/or UL in a UE, reuse prioritization rule as specified in TS 38.213 Section 16.2.4.1 and 16.2.4.3.1.

- Conclusion:

  - For Scheme 2, the values of the following parameters are the same as those for SL HARQ-ACK feedback in the same resource pool

    - Period of PSFCH resources (sl-PSFCH-Period)

    - Number of cyclic shift pairs used for a PSFCH transmission that can be multiplexed in a PRB (sl-NumMuxCS-Pair)

    - Number of PSFCH resources available for multiplexing information in a PSFCH transmission (sl-PSFCH-CandidateResourceType)

- Agreement:

  - For Scheme 1, a resource pool level (pre-)configuration can enable one of the following alternatives:

    - Alt 1 (Working Assumption): MAC CE or 2nd SCI are used as the container of inter-UE coordination information transmission from UE A to UE B.

      - For the indication of resource set, the following is supported:

        - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request.

          - First resource location of each TRIV is separately indicated by the inter-UE coordination information

        - If [N <= 3], MAC CE is used and it is up to UE implementation to additionally use 2nd SCI. When 2nd SCI and MAC CE are both used, the same resource set is indicated in the 2nd SCI and the MAC CE. If [N > 3], only MAC CE is used.

          - FFS: UE capability details

          - 2nd SCI is UE RX optional

    - Alt 2: MAC CE is used as the container of inter-UE coordination information transmission from UE A to UE B.

      - For the indication of resource set, the following is supported:

        - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request.

          - First resource location of each TRIV is separately indicated by the inter-UE coordination information

    - FFS: Whether/How to use resource reservation information as coordination information

- Working Assumption:

  - A resource pool level (pre-)configuration can enable one of the following options:

    - Option 1:

      - For Condition 2-A-1 of Scheme 2, support following additional criteria to determine resource(s) where expected/potential resource conflict occurs

        - For the case when UE-A is a destination UE of a TB transmitted by UE-B

          - The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger than a RSRP threshold according to the priorities included in the SCI:

            - prio_TX and prio_RX are the priorities indicated in the SCI making the overlapping reservations for UE-B and other UE respectively

        - For the case when UE-A is a destination UE of a TB transmitted by another UE

          - The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) when RSRP measurement of UE-B’s reserved resource is larger than a RSRP threshold according to the priorities included in the SCI:

            - prio_TX and prio_RX are the priorities indicated in the SCI making the overlapping reservations for other UE and UE-B respectively

    - Option 4:

      - For Condition 2-A-1 of Scheme 2, support following additional criteria to determine resource(s) where expected/potential resource conflict occurs

        - For the case when UE-A is a destination UE of a TB transmitted by UE-B

          - The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger than a (pre)configured RSRP threshold compared to the RSRP measurement of UE-B’s reserved resource.

        - For the case when UE-A is a destination UE of a TB transmitted by another UE

          - The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) when RSRP measurement of UE-B’s reserved resource is larger than a (pre)configured RSRP threshold compared to the RSRP measurement of the resource(s).

      - Support of Option 4 is subject to UE capability

    - FFS: Whether/how RSRP threshold depends on priority, MCS, overlap

- Agreement:

  - For Scheme 1 with non-preferred resource set,

    - Physical layer at UE-B excludes in its resource (re-)selection, candidate single-slot resource(s) obtained after Step 6) of Rel-16 TS 38.214 Section 8.1.4 overlapping with the non-preferred resource set

- Agreement:

  - For Condition 1-A-1 of Scheme 1, when UE-A determines the set of resources preferred for UE-B’s transmission, apply RSRP threshold increase in the same way according to Rel-16 TS 38.214 Section 8.1.4.

    - FFS: Whether/how to introduce the maximum limit of RSRP threshold increase

- Agreement:

  - For Scheme 1, at least following parameters are provided by UE-B’s request:

    - Priority value to be used for PSCCH/PSSCH transmission

    - Number of sub-channels to be used for PSSCH/PSCCH transmission in a slot

    - Resource reservation interval

- Agreement:

  - For Scheme 2, when PSFCH occasion is derived by a slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B’s SCI,

    - Time gap between the PSFCH and SCI(s) scheduling conflicting TBs is larger than or equal to X value.

      - FFS: Details of X

- Working Assumption:

  - For Condition 2-A-1 in Scheme 2, when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is enabled or when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is disabled and the destination UE of the conflicting TBs is UE-A, for each pair of UEs scheduling the conflicting TBs, a UE with the higher priority value is UE-B.

    - FFS whether/how to set additional condition for UE-A to send PSFCH.

    - Conclude on whether/how to handle, or differently handle, the case when at least one of UEs scheduling conflicting TBs doesn’t support Scheme 2 at the subsequent meetings

- Agreement:

  - For inter-UE coordination information triggered by an explicit request in Scheme 1,

    - UE-A uses a TX resource pool used for UE-B’s request transmission to determine the set of resources and to transmit the set of resources to UE-B

- Agreement:

  - For inter-UE coordination information triggered by a condition rather than request reception in Scheme 1,

    - UE-A transmitting in a resource pool provides inter-UE coordination information associated with the same resource pool

  - Agreements made in RAN#94-e meeting

- Agreement:

  - RAN1 is tasked to complete the remaining normative work for Rel-17 NR sidelink enhancement by Q1 of 2022

    - All RAN1 decisions that impact other WGs should be finalized in RAN1#107bis-e

  - Use the list of open issues provided RP-212880 (status report of WI: NR sidelink enhancement) as a starting point for technical discussions in RAN1.

    - This does not mean that all the issues included in the list are considered essential or the list is complete

    - RAN1 should not spend additional effort to further refine the list