3GPP TSG RAN WG1 #107bis-e	R1-2200193

e-Meeting, January 17th – 25th, 2022

Agenda item:	8.2.2

Source:	Samsung

Title:	Maintenance on PDCCH monitoring enhancements for NR from 52.6 GHz to 71 GHz

Document for:	Discussion and Decision

# Introduction

This contribution discusses the remaining issues for the PDCCH monitoring enhancements for NR 52.6 to 71 GHz, including the following aspects, and the TPs, if needed, associated with the proposals are listed in the appendix:

- Remaining issues for multi-slot PDCCH monitoring

- Remaining issues for search space set configuration

- Remaining issues for multi-cell

- Enhancement to GC-PDCCH

# Remaining issues for multi-slot PDCCH Monitoring

In RAN1 #107, a framework for multi-slot PDCCH monitoring was agreed to be supported for 480 kHz and 960 kHz. The following aspects are remaining issues to be discussed.

## Multi-slot PDCCH monitoring in IDLE mode

The discussion on multi-slot PDCCH monitoring was all based on UE capability, and one remaining issue is the UE behavior in IDLE mode, wherein the UE capability on the multi-slot PDCCH monitoring is not available. One way is to support single-slot based PDCCH monitoring in IDLE mode, but it may not be efficient due to the limited number of BD/CCE budget, and will need further specification impact. Another way is to adopt the mandatory UE capability for multi-slot PDCCH monitoring as the default UE behavior in IDLE mode, which is much simpler and saves the effort on defining the controversial number of BD/CCE budget for single-slot based PDCCH monitoring.

Proposal 1: For IDLE mode:

- Support the mandatory UE capability for multi-slot PDCCH monitoring;

- Don’t support slot based PDCCH monitoring;

- Adopt TP#1 for TS 38.213.

## UE capability in slot other than Ys

In RAN1#107-e, the following was agreed regarding the mandatory UE capability of multi-slot PDCCH monitoring within the Ys slots:

- A UE capable of multi-slot monitoring mandatorily supports the following PDCCH monitoring within Y slots

  - For Y>1: FG3-1 (monitoring Group (1) SSs in the first 3 OFDM symbols of each of the Y slots)

  - For 960 kHz SCS For Y=1: FG3-5b with set1 = (7, 3)

  - For 480 kHz SCS For Y=1: FG3-5b with set2 = (4, 3) and (7, 3) with a modification with maximum two monitoring spans in a slot

The remaining issue is to define the mandatory UE capability of multi-slot PDCCH monitoring outside the Ys slots. Since only Group (2) SS is monitored outside the Ys slots, it can simply follow the FG3-1.

Proposal 2: A UE capable of multi-slot monitoring mandatorily supports FG3-1 for each slot outside Ys slots.

## UE capability determination with multiple Xs,Ys combinations reported

Similar to Rel-16 span-based PDCCH monitoring, a UE can indicate multiple Xs,Ys combinations for multi-slot PDCCH monitoring in FR2-2, then the remaining issue is how to determine the combination that the UE will choose from the multiple indicated Xs,Ys combinations. A similar approach as Rel-16 span-based PDCCH monitoring can be reused, with the consideration of potential tie of values on MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ.

Proposal 3: If a UE indicates a capability to monitor PDCCH according to multiple Xs,Ys combinations, a UE monitors PDCCH on the cell based on the following steps:

- Step 1: Choose the Xs,Ys combination, from the Xs,Ys combinations, that is associated with the largest maximum number of MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ;

- Step 2: If there is a tie on the largest maximum number of MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ, choose the Xs,Ys combination with a larger value of Ys.

- Adopt TP#2 for TS 38.213.

## (Xs, Ys)= (2, 1) for 480 kHz

There is a FFS on whether to support Xs,Ys=(2, 1) for 480 kHz SCS, as one of the reported UE capability. The intention of supporting a smaller value of Xs, comparing to the mandatory UE capability with Xs=4 for 480 kHz SCS, is to allow flexible network configuration on the SS sets. Also, other than adding the corresponding values of MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ in the table, no other spec impact is expected, then it’s beneficial to support extra values of Xs for better flexibility.

Proposal 4: Support Xs,Ys=(2, 1) for 480 kHz SCS.

- Adopt TP#3 for TS 38.213.

# Remaining issues for search space set configurations

To support multi-slot PDCCH monitoring, a UE would expect that a time gap between any two consecutive PDCCH monitoring occasions in a search space set is not smaller than X_s, while any PDCCH monitoring duration is up to Ys slots. PDCCH monitoring occasions are determined according to configured search space sets, where PDCCH monitoring periodicity ks, and duration Ts, are configured in a number of slots for a search space set s, wherein those parameters should be enhanced to be compatible with the multi-slot PDCCH monitoring capability.

Proposal 5: For μ=5 and μ=6 with multi-slot PDCCH monitoring based on combination (Xs, Ys), the PDCCH monitoring periodicity is ks=k⋅Xs, k=1, 2, …, and the PDCCH monitoring duration is Ts≤Ys.

# Remaining issues for multi-cell

When a UE supports CA or NR-DC operation in NR from 52.6 GHz to 71 GHz, UE can be configured with multiple downlink cells for multi-slot based PDCCH monitoring. It’s necessary to determine a capability to monitor a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs that corresponds to Ncellscap downlink cells, which is in analogy to  Ncellscap-r15 for slot-based PDCCH monitoring capability introduced in NR Rel-16 and  Ncellscap-r16 for span based PDCCH monitoring capability introduced in NR Rel-17.

The legacy mechanism for determine  Ncellscap-r16 can be extended to determine a capability of  Ncellscap-r17for multi-slot based PDCCH monitoring considering scenarios, such as

- Case 1: UE is configured with CA but not NR-DC operation, and UE is configured with only R17monitoringcapability for all downlink cells

- Case 2: UE is configured for NR-DC operation, and UE is configured with only R17monitoringcapability for all downlink cells

When the UE is configured for carrier aggregation operation over more than 2 cells, or for a cell group when the UE is configured for NR-DC operation, the UE does not expect to monitor per group of Xs slots according to combination Xs,Ys a number of PDCCH candidates or a number of non-overlapped CCEs that is larger than the maximum number as derived from the corresponding value of Ncellscap-r17.

Proposal 6: Adopt TP#4 for TS 38.213 to determine a capability to monitor a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs per Xs slots that corresponds to Ncellscap downlink cells, i.e.  Ncellscap-r17.

It was agreed in RAN1#107-e meeting to support PDCCH dropping per Xs slots based on combination Xs,Ys. The PDCCH dropping rule should be applied to both single cell case and CA. For UE configured with CA or NR-DC operation, the PDCCH candidates and non-overlapping CCE limits should be determined based on Ncellscap-r17downlink cells for all scheduling cells. The same principle to determine MPDCCHtotal,(X,Y),μ and CPDCCHtotal,(X,Y),μ for R16monitoringcapability can be reused to determine MPDCCHtotal,(Xs,Ys),μ and CPDCCHtotal,(Xs,Ys),μ for R17monitoringcapability in CA mode.

Proposal 7: Adopt TP#5 for TS 38.213 to determine maximum number of PDCCH candidates, MPDCCHtotal,(Xs,Ys),μ, and a maximum number of non-overlapped CCEs, CPDCCHtotal,(Xs,Ys),μ, per Xs slots in CA mode.

# Enhancement to GC-PDCCH

For 60 GHz unlicensed band, transmissions are expected to be highly directional. To address the channel access efficiency, a transmitter can choose an intended beam direction to perform the channel access procedure, and the sensed result is exclusively applicable to that intended beam direction only. Hence, indicating COT, available RB set, and search space group switching should be associated with the beam direction, wherein such feature was introduced in Rel-16 NR-U by using DCI format 2_0 and in a cell-specific manner. Generalizing the feature to a beam-specific manner is beneficial to address different interference situations along beam directions, and is compatible with the intention to introduce directional LBT.

Proposal 8: Support indicating COT, available RB set, and search space group switching in a beam-specific manner for 60 GHz licensed band.

# Conclusion

The observations and proposals made in this contribution are summarized below:

Proposal 1: For IDLE mode:

- Support the mandatory UE capability for multi-slot PDCCH monitoring;

- Don’t support slot based PDCCH monitoring;

- Adopt TP#1 for TS 38.213.

Proposal 2: A UE capable of multi-slot monitoring mandatorily supports FG3-1 for each slot outside Ys slots.

Proposal 3: If a UE indicates a capability to monitor PDCCH according to multiple Xs,Ys combinations, a UE monitors PDCCH on the cell based on the following steps:

- Step 1: Choose the Xs,Ys combination, from the Xs,Ys combinations, that is associated with the largest maximum number of MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ;

- Step 2: If there is a tie on the largest maximum number of MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ, choose the Xs,Ys combination with a larger value of Ys.

- Adopt TP#2 for TS 38.213.

Proposal 4: Support Xs,Ys=(2, 1) for 480 kHz SCS.

- Adopt TP#3 for TS 38.213.

Proposal 5: For μ=5 and μ=6 with multi-slot PDCCH monitoring based on combination (Xs, Ys), the PDCCH monitoring periodicity is ks=k⋅Xs, k=1, 2, …, and the PDCCH monitoring duration is Ts≤Ys.

Proposal 6: Adopt TP#4 for TS 38.213 to determine a capability to monitor a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs per Xs slots that corresponds to Ncellscap downlink cells, i.e.  Ncellscap-r17.

Proposal 7: Adopt TP#5 for TS 38.213 to determine maximum number of PDCCH candidates, MPDCCHtotal,(Xs,Ys),μ, and a maximum number of non-overlapped CCEs, CPDCCHtotal,(Xs,Ys),μ, per Xs slots in CA mode.

Proposal 8: Support indicating COT, available RB set, and search space group switching in a beam-specific manner for 60 GHz licensed band.

# Appendix

## TP#1 for TS 38.213

============================== Start of TP #1 for TS 38.213 ==================================

10 	UE procedure for receiving control information

=============================== Unchanged Text Omitted ===================================

If a UE is provided monitoringCapabilityConfig for a serving cell, the UE obtains an indication to monitor PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs

-	per slot, as in Tables 10.1-2 and 10.1-3, if monitoringCapabilityConfig = r15monitoringcapability, or

-	per span, as in Tables 10.1-2A and 10.1-3A, if monitoringCapabilityConfig = r16monitoringcapability

-	per group of Xs slots according to combination Xs,Ys, as in Tables 10.1-2B and 10.1-3B, if μ=5 or 6

If the UE is not provided monitoringCapabilityConfig for μ=0, 1, 2, or 3, the UE monitors PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per slot.

If the UE is not provided monitoringCapabilityConfig for μ=5 or 6, the UE monitors PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per group of Xs slots according to combination Xs,Ys = (4, 1) or (8, 1), for μ=5 or 6, respectively.

=============================== Unchanged Text Omitted ===================================

============================== End of TP #1 for TS 38.213 ==================================

## TP#2 for TS 38.213

============================== Start of TP #2 for TS 38.213 ==================================

10 	UE procedure for receiving control information

=============================== Unchanged Text Omitted ===================================

For SCS configuration μ=5 or μ=6, a UE can indicate a capability to monitor PDCCH according to one or more combinations Xs,Ys, where Xs and Ys are numbers of consecutive slots, groups of Xs slots are consecutive and non-overlapping, and the Ys slots are within the Xs slots. The first group of Xs slots starts from the beginning of a subframe. The start of two consecutive groups of Ys slots is separated by Xs slots.

If a UE monitors PDCCH on a cell according to combination Xs,Ys, the UE can monitor PDCCH for Type1-PDCCH CSS set provided by dedicated higher layer signalling, Type3-PDCCH CSS sets, and USS sets in any slot of the Ys slots, and the UE can monitor PDCCH for Type0/0A/2-PDCCH CSS set and Type1-PDCCH CSS set provided in SIB1 in any slot of the Xs slots. The UE determines the number of monitored PDCCH candidates and the number of non-overlapped CCEs for combination Xs,Ys based on all search space sets within the Xs slots, as applicable according to the search space set configurations, and maximum corresponding values are provided in Table 10.1-2B and Table 10.1-3B, respectively.

If a UE indicates a capability to monitor PDCCH according to multiple Xs,Ys combinations and a configuration of search space sets to the UE for PDCCH monitoring on a cell satisfies Type1-PDCCH CSS set provided by dedicated higher layer signalling, Type3-PDCCH CSS sets, and USS sets in any slot of the Ys slots within the Xs slots, for one or more of the multiple combinations Xs,Ys, the UE monitors PDCCH on the cell according to the combination Xs,Ys, from the one or more combinations Xs,Ys, that is associated with the largest maximum number of MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ defined in Table 10.1-2B and Table 10.1-3B, respectively. If multiple combinations Xs,Ys are associated with the same maximum number of MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ, the UE monitors PDCCH on the cell according to the combination Xs,Ys with the largest value of Ys.

=============================== Unchanged Text Omitted ===================================

============================== End of TP #2 for TS 38.213 ==================================

## TP#3 for TS 38.213

============================== Start of TP #3 for TS 38.213 ==================================

10.1	UE procedure for determining physical downlink control channel assignment

=============================== Unchanged Text Omitted ===================================

Table 10.1-2B provides the maximum number of monitored PDCCH candidates, MPDCCHmax,Xs,Ys,μ, per slot group for combination (Xs,Ys) for a UE in a DL BWP with SCS configuration μ for operation with a single serving cell.

Table 10.1-2B: Maximum number MPDCCHmax,(Xs,Ys),μ of monitored PDCCH candidates per slot group for combination Xs,Ys for a DL BWP with SCS configuration μ∈5, 6 for a single serving cell

|  | Maximum number MPDCCHmax,Xs,Ys,μ of monitored PDCCH candidates per combination Xs,Ys and per serving cell |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| μ | (2, 1) | (4, 1) | (4, 2) | (8, 1) | (8, 4) |
| 5 | 10 | 20 | 20 | - | - |
| 6 | - | 10 | 10 | 20 | 20 |

=============================== Unchanged Text Omitted ===================================

Table 10.1-3B provides the maximum number of non-overlapped CCEs, CPDCCHmax,Xs,Ys,μ, for a DL BWP with SCS configuration μ that a UE is expected to monitor corresponding PDCCH candidates for combination Xs,Ys for operation with a single serving cell.

Table 10.1-3B: Maximum number CPDCCHmax,(Xs,Ys),μ of non-overlapped CCEs in a slot group for combination Xs,Ys for a DL BWP with SCS configuration μ∈5, 6 for a single serving cell

|  | Maximum number of non-overlapped CCEs per combination Xs,Ys and per serving cell CPDCCHmax,(Xs,Ys),μ |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| μ | (2, 1) | (4, 1) | (4, 2) | (8, 1) | (8, 4) |
| 5 | 16 | 32 | 32 | - | - |
| 6 | - | 16 | 16 | 32 | 32 |

=============================== Unchanged Text Omitted ===================================

============================== End of TP #3 for TS 38.213 ==================================

## TP#4 for TS 38.213

============================== Start of TP #4 for TS 38.213 ==================================

10	UE procedure for receiving control information

=============================== Unchanged Text Omitted ===================================

If the UE reports pdcch-MonitoringCA,

-	the value range of pdcch-BlindDetectionMCG-UE-r16 or of pdcch-BlindDetectionSCG-UE-r16 is [1, …, pdcch-MonitoringCA-1], and

-	pdcch-BlindDetectionMCG-UE-r16 + pdcch-BlindDetectionSCG-UE-r16 >= pdcch-MonitoringCA.

Otherwise, if NNR-DC,  max, r16DL,cells is a maximum total number of downlink cells for which the UE is provided monitoringCapabilityConfig = r16monitoringcapability and the UE is configured on both the MCG and the SCG for NR-DC as indicated in UE-NR-Capability

-	the value of pdcch-BlindDetectionMCG-UE-r16 or of pdcch-BlindDetectionSCG-UE-r16 is 1,

-	pdcch-BlindDetectionMCG-UE-r16 + pdcch-BlindDetectionSCG-UE-r16 >= NNR-DC,  max, r16DL,cells.

If a UE indicates in UE-NR-Capability a carrier aggregation capability larger than two downlink cells, the UE includes in UE-NR-Capability an indication for a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs that the UE can monitor per group of Xs slots according to combination Xs,Ys when the UE is configured for carrier aggregation operation over more than 2 downlink cells with monitoringCapabilityConfig = r17monitoringcapability. When a UE is not configured for NR-DC operation and the UE is provided monitoringCapabilityConfig = r17monitoringcapability for all downlink cell where the UE monitors PDCCH, the UE determines a capability to monitor a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs per span that corresponds to Ncellscap-r17 downlink cells, where

-	Ncellscap-r17 is the number of configured downlink cells if the UE does not provide pdcch-MonitoringCA

-	otherwise, Ncellscap-r17 is the value of pdcch-MonitoringCA

When a UE is configured for NR-DC operation and the UE is provided monitoringCapabilityConfig = r17monitoringcapability for all downlink cells where the UE monitors PDCCH, the UE determines a capability to monitor a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs per group of Xs slots according to combination Xs,Ys that corresponds to

-	Ncellscap-r17= Ncells, r17MCG downlink cells for the MCG where Ncells, r17MCG is provided by pdcch-BlindDetection3 for the MCG, and

-	Ncellscap-r17= Ncells, r17SCG downlink cells for the SCG where Ncells, r17SCG is provided by pdcch-BlindDetection3 for the SCG

When the UE is configured for carrier aggregation operation over more than 2 cells, or for a cell group when the UE is configured for NR-DC operation, the UE does not expect to monitor per group of Xs slots according to combination Xs,Ys a number of PDCCH candidates or a number of non-overlapped CCEs that is larger than the maximum number as derived from the corresponding value of Ncellscap-r17.

When a UE is configured for NR-DC operation with a total of NNR-DCDL,cells downlink cells on both the MCG and the SCG and the UE is provided monitoringCapabilityConfig = r17monitoringcapability for all downlink cells where the UE monitors PDCCH, the UE expects to be provided pdcch-BlindDetection for the MCG and pdcch-BlindDetection for the SCG with values that satisfy

-	pdcch-BlindDetection3 for the MCG + pdcch-BlindDetection3 for the SCG <= pdcch-MonitoringCA, if the UE reports pdcch-MonitoringCA, or

-	pdcch-BlindDetection3 for the MCG + pdcch-BlindDetection3 for the SCG <= NNR-DCDL,cells, if the UE does not report pdcch-MonitoringCA

When a UE is configured for NR-DC operation and the UE is provided monitoringCapabilityConfig = r17monitoringcapability for all downlink cells where the UE monitors PDCCH, the UE may indicate, through pdcch-BlindDetectionMCG-UE-r17 and pdcch-BlindDetectionSCG-UE-r17, respective maximum values for pdcch-BlindDetection for the MCG and pdcch-BlindDetection for the SCG.

If the UE reports pdcch-MonitoringCA,

-	the value range of pdcch-BlindDetectionMCG-UE-r17 or of pdcch-BlindDetectionSCG-UE-r17 is [1, …, pdcch-MonitoringCA-1], and

-	pdcch-BlindDetectionMCG-UE-r17 + pdcch-BlindDetectionSCG-UE-r17 >= pdcch-MonitoringCA.

Otherwise, if NNR-DC,  max, r17DL,cells is a maximum total number of downlink cells for which the UE is provided monitoringCapabilityConfig = r17monitoringcapability and the UE is configured on both the MCG and the SCG for NR-DC as indicated in UE-NR-Capability

-	the value of pdcch-BlindDetectionMCG-UE-r17 or of pdcch-BlindDetectionSCG-UE-r17 is 1,

-	pdcch-BlindDetectionMCG-UE-r17 + pdcch-BlindDetectionSCG-UE-r17 >= NNR-DC,  max, r17DL,cells.

============================== End of TP #4 for TS 38.213 ==================================

## TP#5 for TS 38.213

============================== Start of TP #5 for TS 38.213 ==================================

10.1	UE procedure for determining physical downlink control channel assignment

=============================== Unchanged Text Omitted ===================================

For each scheduled cell from the Ncells,r16DL,(X,Y),μ downlink cells using combination X,Y, the UE is not required to monitor on the active DL BWP with SCS configuration μ of the scheduling cell, more than minMPDCCHmax,(X,Y),μ,MPDCCHtotal,(X,Y),μ PDCCH candidates or more than minCPDCCHmax,(X,Y),μ,CPDCCHtotal,(X,Y),μ non-overlapped CCEs per span.

If a UE is configured only with Ncells,r17DL,μ downlink cells for which the UE is provided monitoringCapabilityConfig = r17monitoringcapability and with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cells using SCS configuration μ, and with Ncells,r17DL,(Xs,Ys),μ of the Ncells,r17DL,μ downlink cells using combination Xs,Ys for PDCCH monitoring, where μ=01Ncells,r17DL,μ>Ncellscap-r17, a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell, the UE is not required to monitor more than MPDCCHtotal,(X,Y),μ=Ncellscap-r17⋅MPDCCHmax,(Xs,Ys),μ⋅Ncells,r17DL,(Xs,Ys),μj=01Ncells,r17DL,j PDCCH candidates or more than CPDCCHtotal,(Xs,Ys),μ=Ncellscap-r17⋅CPDCCHmax,(Xs,Ys),μ⋅Ncells,r17DL,(Xs,Ys),μj=01Ncells,r17DL,j non-overlapped CCEs

-	per set of groups of Xs slots on the active DL BWP(s) of all scheduling cell(s) from the Ncells,r17DL,(Xs,Ys),μ downlink cells within every Xs slots

where Ncells,r16DL,j is a number of configured cells with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cells using SCS configuration j.

For each scheduled cell from the Ncells,r17DL,(X,Y),μ downlink cells using combination Xs,Ys, the UE is not required to monitor on the active DL BWP with SCS configuration μ of the scheduling cell, more than minMPDCCHmax,(Xs,Ys),μ,MPDCCHtotal,(Xs,Ys),μ PDCCH candidates or more than minCPDCCHmax,(Xs,Ys),μ,CPDCCHtotal,(Xs,Ys),μ non-overlapped CCEs per group of Xs slots.

A UE does not expect to be configured CSS sets that result to corresponding total, or per scheduled cell, numbers of monitored PDCCH candidates and non-overlapped CCEs per slot, per group of Xs slots for a corresponding combination Xs,Ys, or per span that exceed the corresponding maximum numbers per slot, or per group of Xs slots for a corresponding combination Xs,Ys, or per span, respectively.

============================== End of TP #5 for TS 38.213 ==================================