3GPP TSG RAN WG1 #107bis-e			    R1-2200388

e-Meeting, January 17th – 25th, 2022

Source:	Intel Corporation

Title:	Broadcast for RRC_IDLE/INACTIVE UEs

Agenda item:	8.12.3

Document for:	Discussion and Decision

# Introduction

In this paper, remaining issues and maintenance for support of NR multicast and broadcast services (MBS) for RRC_IDLE/INACTIVE UEs is discussed.

# Discussion

In this section we provide initial details on design considerations for NR MBS for RRC_IDLE or RRC_INACTIVE mode UEs. RAN2 has already agreed that low QoS Delivery mode 2 (broadcast) is supported for RRC_IDLE/INACTIVE UEs and multicast is currently deprioritized. Some remaining details for broadcast operation include finalizing CFR configuration, MCCH change notification etc. In the following sections we provide our views on some open topics to finalize the broadcast service design from a RAN1 perspective

## 2.1. Group Scheduling

### 2.1.1. CFR Configuration MCCH/MTCH

The following agreements were reached in recent RAN1 meetings for CFR configuration of RRC_IDLE/INACTIVE UEs:

Agreement Only one CFR can be configured for group-common PDCCH/PDSCH carrying MCCH for broadcast reception with UEs in RRC_IDLE/INACTIVE state. Agreement: For broadcast reception, RRC_IDLE/RRC_INACTIVE UEs can use the same bandwidth configurations for the CFR of GC-PDCCH/PDSCH carrying MCCH and the CFR of GC-PDCCH/PDSCH carrying MTCH. - FFS: use of different bandwidth configurations for the CFR of GC-PDCCH/PDSCH carrying MCCH and the CFR of GC-PDCCH/PDSCH carrying MTCH Agreement: For broadcast reception with RRC_IDLE/RRC_INACTIVE UEs: - The CFR frequency resources used for MCCH and MTCH are configured by SIBx; - PDCCH-config/PDSCH-config for broadcast reception with GC-PDCCH/PDSCH carrying MCCH is configured by SIBx - PDCCH-config/PDSCH-config for broadcast reception with GC-PDCCH/PDSCH carrying MTCH is configured by MCCH. If the PDCCH-config/PDSCH-config for MTCH is not configured, the PDCCH-config/PDSCH-config for GC-PDCCH/PDSCH carrying MCCH configured by SIBx is reused for GC-PDCCH/PDSCH carrying MTCH.

- However, in the current version of TS38.213 [4], the following is specified:

A UE can be configured by cfr-Config-MCCH-MTCH an MBS frequency resource for PDCCH and PDSCH receptions providing MCCH and MTCH [12, TS 38.331]; otherwise, the MBS frequency resource is same as for the CORESET with index 0 that is associated with the Type0-PDCCH CSS set for PDCCH and PDSCH receptions providing MCCH and MTCH. In clauses referring to a higher layer parameter value provided by PDCCH-ConfigCommon or PDSCH-ConfigCommon, when applicable a corresponding higher layer parameter value for MCCH/MTCH PDCCH receptions or PDSCH receptions, respectively, is provided as described in [12, TS 38.331]. A UE can be configured by cfr-Config-Broadcast, an MBS frequency resource within the initial DL BWP for PDCCH and PDSCH receptions [4, TS 38.211]. If cfr-Config- Broadcast does not include locationAndBandwidth-Broadcast, the MBS frequency resource is the initial DL BWP. A UE monitors PDCCH for scheduling PDSCH receptions for MCCH or MTCH as described in clause 10.1.

- Based on the highlighted parts, it appears that there are two possible CFR configurations for broadcast i.e., cfr-Config-MCCH-MTCH and cfr-Config-Broadcast which would be a cause for confusion for UEs which receive both configurations. From the current specification, it is not clear which CFR should be used for broadcast reception. Since CONNECTED mode UEs can also receive cfr-Config-MCCH-MTCH, two configurations are unnecessary.

- In addition, the last agreement which states that the PDDCH-Config and PDSCH-Config for MTCH can be provided by MCCH and if not provided is the same as that provided in cfr-Config-MCCH-MTCH. This needs to be captured in the specification. Therefore, we propose the following:

CR for TS 38.213 # 18 Multicast Broadcast Services ## *** Unchanged text is omitted *** A UE can be configured by cfr-Config-MCCH-MTCH an MBS frequency resource for PDCCH and PDSCH receptions providing MCCH and MTCH [12, TS 38.331]; If cfr-Config-MCCH-MTCH does not contain locationAndBandwidth-Broadcast, the MBS frequency resource is the initial DL BWP configured by SIB-1. If initial DL BWP is not configured by SIB-1, the MBS frequency resource is same as for the CORESET with index 0 that is associated with the Type0-PDCCH CSS set for PDCCH and PDSCH receptions providing MCCH and MTCH. MCCH can provide the PDCCH-Config-MTCH and PDSCH-Config-MTCH for MTCH reception; if not provided by MCCH, the MTCH reception uses the PDCCH-Config-MCCH and PDSCH-Config-MCCH provided by cfr-Config-MCCH-MTCH in SIBx. In clauses referring to a higher layer parameter value provided by PDCCH-ConfigCommon or PDSCH-ConfigCommon, when applicable a corresponding higher layer parameter value for MCCH/MTCH PDCCH receptions or PDSCH receptions, respectively, is provided as described in [12, TS 38.331]. A UE can be configured by cfr-Config-Broadcast, an MBS frequency resource within the initial DL BWP for PDCCH and PDSCH receptions [4, TS 38.211]. If cfr-Config- Broadcast does not include locationAndBandwidth-Broadcast, the MBS frequency resource is the initial DL BWP. A UE monitors PDCCH for scheduling PDSCH receptions for MCCH or MTCH as described in clause 10.1.

# Conclusions

In this paper we have provided a CR for TS 38.213 to address CFR configuration for broadcast.

# References

- RP-193248, New Work Item on NR Support of Multicast and Broadcast Services, Huawei, RAN#86, Sitges, Spain, December 2019.

- 3GPP TS 38.211 v17.0.0, NR Physical Channels and Modulation (Release 17)

- 3GPP TS 38.212 v17.0.0, NR Multiplexing and Channel Coding (Release 17)

- 3GPP TS 38.213 v17.0.0, NR Physical Layer Procedures for Control (Release 17)

- 3GPP TS 38.214 v17.0.0, NR Physical Layer Procedures for Data (Release 17)