3GPP TSG RAN WG1 #107bis-e							    R1-2200391

e-Meeting, January 17th – 25th, 2022

Source:	Intel Corporation

Title:	On UE features for enhanced IIoT and URLLC

Agenda item:	8.15.3

Document for:	Discussion and Decision

Introduction

This contribution provides further comments on the draft UE features available in R1-2112902 for feature group 25 related to further enhancements to URLLC/IIOT in Release 17.

Discussion on features

## HARQ enhancements

- 25-3a Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots using dynamic repetition indication

  - Regarding the pre-requisite of 30-5 (dynamic slot-based repetition), we still think it should be removed. In our view, there is no direct relation with CovEnh feature. Slot-based repetitions target coverage improvement, while sub-slot based repetitions target better latency-reliability tradeoff. In that sense, a URLLC UE may choose to implement sub-slot based repetitions without support slot-based repetitions.

- 25-3b Inter-subslot frequency hopping for PUCCH repetitions

  - Sub-slot PUCCH repetition operation and frequency hopping should be pre-requisites

- 25-9 Semi-static PUCCH cell switching

- 25-10 PUCCH cell switching based on dynamic indication

  - Regarding the FFS whether to separate the capability for different numerologies, we are supportive. As it can be seen in current discussions, the different numerologies require additional handling mechanisms for the cases of SCS reference larger or smaller than SCS of switched cell.

## CSI enhancements

No comments.

## Unlicensed band

No comments.

## Intra-UE multiplexing and prioritization

- 25-16 HARQ-ACK with different priorities multiplexing on a PUCCH/PUSCH

  - Regarding the FFS whether to separate capability for different UCI type, we have a slight preference of not doing that to allow a greater network scheduling flexibility to support intra-UE multiplexing use cases.

## Propagation delay compensation

- 25-19 RTT-based Propagation delay compensation based on CSI-RS for tracking and SRS

- 25-19a RTT-based Propagation delay compensation based on DL PRS and SRS

  - Regarding the FFS whether to merge FG 25-19 and FG 25-19a, we clearly don’t see the need since DL PRS support may require additional implementation efforts comparing to CSI-RS based

  - Depending on discussion on “SRS for positioning” support, additional row e.g., “RTT-based Propagation delay compensation based on DL PRS and SRS for positioning” may be introduced

## Discussion on Type (Per UE or Per Band or Per BC or Per FS or Per FSPC)

From technical point of view, complexity of the following capabilities does not scale with bands, and therefore ‘Per UE’ signalling type is appropriate:

- 25-1	SPS HARQ-ACK deferral in case of TDD collision

- 25-2	Repetitions for PUCCH format 0, and 2 over multiple slots with K = 2, 4, 8

- 25-3	Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with configured K = 2, 4, 8

- 25-3a	Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots using dynamic repetition indication

- 25-3b	Inter-subslot frequency hopping for PUCCH repetitions

- 25-4	One-shot HARQ ACK feedback triggered by DCI format 1_2

- 25-5	PHY priority handling for one-shot HARQ ACK feedback

- 25-6	Enhanced type 3 HARQ-ACK codebook feedback

- 25-7	Triggered HARQ-ACK codebook re-transmission

- 25-8	Semi-static HARQ-ACK codebook for sub-slot PUCCH

- 25-11	4-bits subband CQI

- 25-16	HARQ-ACK with different priorities multiplexing using on a PUCCH/PUSCH

- 25-18	Parallel PUCCH and PUSCH transmission across CCs in inter-band CA

- 25-20	Propagation delay compensation based on legacy TA procedure

Regarding the issue with testing mentioned by some companies last meeting, we think these need to be discussed not per each UE feature but in general for all work items and agenda items. If accepted, then some ‘per UE’ features can be promoted to per FSPC/BC/FS etc.

Conclusion

In this contribution we went over the updated UE features list generated by moderators and provided comments. We have the following proposal:

Proposal

- Consider comments in section 2 of this contribution (R1-2200391) in Release 17 URLLC/IIOT UE features discussion