3GPP TSG RAN WG1 #107bis-e			    R1-2200394

e-Meeting, January 17th – 25th, 2022

Source:	Intel Corporation

Title:	UE Features for NR MBS

Agenda item:	8.16.12

Document for:	Discussion and Decision

# Introduction

In this paper, we provide our views on the latest version of RAN1 UE features for Rel-17 NR MBS [2].

# Discussion

The initial version of NR MBS UE feature list provides a set of feature groups based on current design of NR MBS. We have provided our initial feedback in Table 1. The proposed changes are marked in red in the table. In summary we propose the following updates to the initial version of the UE Feature list:

- FG 33-2

  - Item 6: The support of ACK/NACK based HARQ feedback should be separated into a separate dependent feature group since the default mode is HARQ disabled and RRC is used to enable it. UEs may not support HARQ feedback.

  - Separate support of DCI 1_1 and rename 1_0/1_1 to 4_1/4_2 respectively

  - The PTP and PTM retransmission for MBS should be separated from 33-2 and added to dependent FG on ACK/NACK based HARQ feedback

- FG 33-2b:

  - Add NACK-only based feedback option based on latest agreements.

- FG 33-4:

  - Add component for support of enabling/disabling NACK-only based HARQ-ACK feedback configured by RRC signaling

- FG 33-7:

  - Add NACK-only feedback also to this FG

The changes proposed are shown in red text in the table below from [2].

| Features | Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-1 | Broadcast | - Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format 1_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support MCCH change notification indication via DCI. |  |
| 33. NR_MBS | 33-2 | Dynamic scheduling for multicast | - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 1_0 / 1_1 4_1 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - [Support ACK/NACK based HARQ-ACK feedback, and support enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling.] - Support PTM retransmission for multicast. - Support PTP retransmission for multicast. FFS whether to separate the capability for support of DCI format 1_1 with CRC scrambled with G-RNTI for multicast FFS whether to support NACK-only and/or ACK/NACK based HARQ-ACK feedback FFS whether to separate the capability for support of PTP retransmission for multicast FFS whether to separate the capability for support of PTM retransmission for multicast |  |
| 33. NR_MBS | 33-2b | DCI-based enabling/disabling ACK/NACK-HARQ/ACK based feedback for dynamic scheduling for multicast | Support of DCI-based enabling/disabling ACK/NACK and NACK-only based HARQ-ACK feedback configured per G-RNTI by RRC signaling | 33-2a |
| 33. NR_MBS | 33-2-x | Multiple G-RNTIs for group-common PDSCHs | Capability on number of G-RNTI for groupcast FFS details. | 33-2 |
| 33. NR_MBS | 33-3-1 | Slot-level repetition for group-common PDSCH | - Support slot-level repetition for group-common PDSCH for multicast. FFS whether to split FG 33-1-1 into two FGs for semi-static slot-level repetition and dynamic slot-level repetition | 33-2 |
| 33. NR_MBS | 33-3-2 | FDM-ed unicast PDSCH and group-common PDSCH | - Support FDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support FDM-ed Type-1 HARQ-ACK codebook for multicast. - Support FDM-ed Type-2 HARQ-ACK codebook for multicast. FFS whether/how to separate the capability for HARQ-ACK codebook | 33-1, 33-2 |
| 33. NR_MBS | 33-3-3 | Intra-slot TDM-ed unicast PDSCH and group-common PDSCH | - Support TDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - Support TDM among N (N>1) group-common PDSCHs in a slot per CC - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). - Support TDM-ed Type-1 HARQ-ACK codebook for multicast. - Support TDM-ed Type-2 HARQ-ACK codebook for multicast. FFS whether/how to separate the capability for HARQ-ACK codebook | 33-1, 33-2 |
| 33. NR_MBS | 33-4-1 | ACK-NACK based HARQ-ACK feedback for multicast | - Support ACK/NACK based HARQ-ACK feedback, and support enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling. - Support PTM retransmission for multicast. - Support PTP retransmission for multicast. | 33-2 |
| 33. NR_MBS | 33-4-2 | NACK-only based HARQ-ACK feedback for multicast | - Support NACK-only based HARQ-ACK feedback and support enabling/disabling NACK-only based HARQ-ACK feedback configured by RRC signaling. | 33-2 |
| 33. NR_MBS | 33-5-1 | SPS group-common PDSCH for multicast | - Support one SPS group-common PDSCH configuration for multicast - Support ACK/NACK based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. - Support NACK-only based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. - FFS: HARQ-ACK feedback for SPS group-common with PDCCH scheduling and SPS release PDCCH. - Support slot-level repetition for SPS group-common PDSCH FFS whether/how to separate the above capabilities from FG 33-5-1 | 33-2 |
| 33. NR_MBS | 33-5-2 | Multiple SPS group-common PDSCH configuration | - Support [N>1] SPS group-common PDSCH configuration for multicast - Support [M>=1] activated SPS group-common PDSCH configurations among the N SPS group-common PDSCH configurations per CFR for multicast | 33-2 |
| 33. NR_MBS | 33-6-1 | DL priority indication for multicast in DCI | 1. Support of priority indicator field configured in DCI formats 1_1 with CRC scrambled with G-RNTI for multicast. FFS whether/how to separate the above capability from FG 33-6-1 | 33-2 |
| 33. NR_MBS | 33-6-2 | Two HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities for multicast or for unicast and multicast at a UE. | 1. Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for multicast or for unicast and multicast at a UE. | 33-6-1 |
| 33. NR_MBS | 33-6-3 | More than one PUCCH for HARQ-ACK transmission for multicast or for unicast and multicast within a slot | 1. Supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast or for unicast and multicast with different priorities in a slot. | 33-6-1, 33-6-2 |
| 33. NR_MBS | 33-7 | Supporting group-common DCI indicating the enabling/disabling [ACK/NACK based] HARQ-ACK feedback | 1. Supports the function of group-common DCI indicating the enabling/disabling [ACK/NACK and NACK-only based] HARQ-ACK feedback | 33-2 |

# Conclusions

In this paper, we provided feedback on the RAN1 UE feature list for NR MBS.

# References

- RP-193248, New Work Item on NR Support of Multicast and Broadcast Services, Huawei, RAN#86, Sitges, Spain, December 2019.

- R1-2112902, Updated RAN1 UE features list for Rel-17 NR after RAN1 #107-e, RAN1#107-e, Nov 2021

- 3GPP TS 38.211 v17.0.0, NR Physical Channels and Modulation (Release 17)

- 3GPP TS 38.212 v17.0.0, NR Multiplexing and Channel Coding (Release 17)

- 3GPP TS 38.213 v17.0.0, NR Physical Layer Procedures for Control (Release 17)

- 3GPP TS 38.214 v17.0.0, NR Physical Layer Procedures for Data (Release 17)