3GPP TSG RAN WG1 #107bis	R1-2200623

e-Meeting, 17 – 25 January, 2022

Agenda item:		8.16.2

Source:	Nokia, Nokia Shanghai Bell

Title:	On UE features for supporting NR from 52.6 GHz to 71 GHz

Document for:		Discussion and Decision

# Introduction

In RAN1#107-e the baseline for UE features has been further refined [1,2]. In this contribution we present our views on the UE features for supporting NR from 52.6 GHz to 71 GHz.

# Discussion

Detailed comments on UE features are listed below. For reference, the baseline agreements from RAN1#107-e are available in the Annex.

- In general, per Band indication is sufficient for the FGs in this WI, given they apply to a limited set of bands, and further savings on complexity and/or overhead when choosing between per UE/per Band are not significant.

- 24-1b: Confirm the FG

- 24-1c: Can be combined with 24-1a

- 24-2: No need to split the capability into SA and DC

- 24-1b/c, 24-4b/c, 24-5c: feature can be support with or without shared spectrum channel access. No need to separate the FG though, per band indication is sufficient.

- 24-4a: Add 24-4 (480kHz DL SCS) as pre-requisite.

- 24-5a: Add 24-5 (960kHz DL SCS) as pre-requisite.

- Basic feature groups:

  - Given the characteristics of high frequency band and practical operation considerations, we do not see much value in defining a mapping between FGs and scenarios where they are basic. This was a valid exercise for NR-U, but we are not convinced the same applies here. Hence, our preference is as follows:

    - 24-1: Basic FG

    - 24-1a: Basic FG

    - 24-1b: Optional with capability signaling

    - 24-1c: Optional with capability signaling

    - 24-1d: Optional with capability signaling

    - 24-1e: Optional with capability signaling

    - 24-2: Basic FG

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# Conclusion

In this contribution we have presented our views on UE features for supporting NR from 52.6 GHz to 71 GHz, with the following proposal:

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# References

- Chair’s Notes RAN1#107-e, Nov. 2021

- R1-2112902, ”Updated RAN1 UE features list for Rel-17 NR after RAN1 #107-e”, Moderators (AT&T, NTT DOCOMO, INC.), Nov. 2021

# Annex

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-1 | Basic FR2-2 DL support | 1. Support reception of 120kHz subcarrier spacing for DL data and control channels, SSB, and reference signals in FR2-2 for non-initial access |  |  |  | FR2-2 is not supported | [per UE][per band] |  |  |  |  | Optional with capability signalling A UE that supports FR2-2 must indicate this FG is supported |
| 24. NR_ext_to_71GHz | 24-1a | Basic FR2-2 UL support | 1. PRACH with 120KHz SCS and length 139 2. Support transmission of 120kHz subcarrier spacing for UL data and control channels and reference signals in FR2-2 | [24-1] |  |  |  |  |  |  |  |  | Optional with capability signalling [A UE that supports FR2-2 must indicate this FG is supported] |
| 24. NR_ext_to_71GHz | 24-1b | Wideband PRACH [with/without shared spectrum channel access] | Enhanced PRACH design for operation by adopting a single long ZC sequence, with ZC sequence equal to 1151 for 120kHz and ZC sequence equal to 571 for 120kHz | [24-1a] |  |  |  |  |  |  |  | FFS: whether to split this FG for SA and DC | Optional [with/without]capability signalling [A UE that supports FR2-2 must indicate this FG is supported] |
| 24. NR_ext_to_71GHz | 24-1c | Multi-RB support PUCCH format 0/1/4 for 120 kHz [with/without shared spectrum channel access] | 1. Support multi-RB PUCCH format 4 for 120 kHz 2. Support multi-RB PUCCH format 0/1 for 120 kHz | [24-1a] |  |  |  |  |  |  |  |  | Optional with capability signalling [A UE that supports FR2-2 must indicate this FG is supported] |
| 24. NR_ext_to_71GHz | 24-1d | Multiple PDSCH scheduling by single DCI for 120kHz | 1. Multi-PDSCH scheduling by single DCI for the operation with 120 kHz SCS 2. HARQ enhancements | [24-1] |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-1e | Multiple PUSCH scheduling by single DCI for 120kHz | 1. Multi-PUSCH scheduling by single DCI for the operation with 120 kHz SCS | [24-1a] |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-2 | 120KHz SSB support for SA/DC in FR2-2 | 1. Support 120KHz SSB for SA/DC in FR2-2 | [24-1, 24-1a] | N/A | N/A | 120KHz SSB based stand-alone in FR2-2 is not supported | N/A | N/A | N/A | N/A | per band FFS: whether to split this FG for SA and DC | Optional [with/without] capability signalling [A UE that supports FR2-2 must indicate this FG is supported] |
| 24. NR_ext_to_71GHz | 24-3 | 480KHz SSB support for SA/DC in FR2-2 | 1. Support 480KHz SSB for SA/DC in FR2-2 | 24-1[, 24-2, 24-4] | FFS |  |  | [per UE][per band] |  |  |  | FFS: whether to split this FG for SA and DC | Optional [with/without] capability signalling |
| 24. NR_ext_to_71GHz | 24-4 | 480KHz SCS support for DL | 1. 480KH SCS for DL data and control channels, SSB, and reference signal reception in FR2-2 for non-initial access 2. Multiple-slot PDCCH monitoring for 480KHz with X=4 slots FFS: 3. Multi- PDSCH scheduling by single DCI for the operation with 480 kHz SCS and corresponding HARQ enhancements | 24-1 | Yes |  |  | [Per UE/band] |  |  |  |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-4a | 480KHz SCS support for UL | 1. PRACH with 480KHz and length 139 2. 480KHz SCS for UL data and control channels and reference signal transmission in FR2-2 3. Multi-PUSCH scheduling by single DCI for the operation with 480 kHz SCS |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-4b | Wideband PRACH for 480 kHz [with/without shared spectrum channel access] | PRACH with 480KHz and length 571 |  |  |  |  |  |  |  |  | FFS: whether to split this FG for SA and DC [Agreement: Do not support PRACH length L=571, 1151 for 960kHz PRACH and at least L =1151 for 480kHz PRACH] | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-4c | Multi-RB PUCCH format 0/1/4 for 480 kHz [with/without shared spectrum channel access] | Support multi-RB PUCCH format 0/1/4 for 480 kHz |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-4f | Enhanced PDCCH monitoring for 480KHz | Multiple-slot PDCCH monitoring for 480KHz with X=[2] slots |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-5 | 960KHz SCS support for DL | 1. 960KHz SCS for DL data and control channels, SSB, and reference signal reception in FR2-2 for non-initial access 2. Multiple-slot PDCCH monitoring for 960KHz with X=8 slots FFS: 3. MultiPDSCH scheduling by single DCI for the operation with 960 kHz SCS and corresponding HARQ enhancements | 24-1 | Yes |  |  | [Per UE/band] |  |  |  |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-5a | 960KHz SCS support for UL | 1. PRACH with 960KHz and length 139 2. 960KHz SCS for UL data and control channels and reference signal transmission in FR2-2 [3. Multi-PUSCH scheduling by single DCI for the operation with 960 kHz SCS] |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-5c | Multi-RB PUCCH format 0/1/4 for 960 kHz [with/without shared spectrum channel access] | Support multi-RB PUCCH format 0/1/4 for 960 kHz |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-5f | Enhanced PDCCH monitoring for 960KHz | Multiple-slot PDCCH monitoring for 960KHz with X=4 slots |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-6 | Support [Type 1] channel access procedure in uplink for FR2-2 unlicensed operation | - Support [Type 1] channel access procedure - [Support LBT performed per carrier/BWP bandwidth] | 24-1 |  |  |  | per band |  |  |  |  | Optional with capability signalling [A UE that supports FR2-2 must indicate this FG is supported when required by regulation] |
| 24. NR_ext_to_71GHz | 24-7 | Support [Type 2] channel access procedure in uplink for FR2-2 unlicensed operation | 1. Support [Type 2] channel access procedure | 24-1, 24-6 |  |  |  | per band |  |  |  |  | Optional with capability signalling [A UE that supports FR2-2 must indicate this FG is supported when required by regulation] |
| 24. NR_ext_to_71GHz | 24-8 | 32 DL HARQ processes for FR 2-2 | Support 32 HARQ processes in DL for 480/960 kHz |  |  |  |  | [Per UE/per FSPC/per band] |  |  |  | FFS: 120 kHz | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-9 | 32 UL HARQ processes for FR 2-2 | Support 32 HARQ processes in UL for 480/960 kHz |  |  |  |  | [Per UE/per FSPC/per band] |  |  |  | FFS: 120 kHz | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-10 | Additional beam switching time delay | Supported additional beam switching time delay d for 480 kHz SCS |  |  |  |  |  |  |  |  | Candidate value set: 56 or 112 symbols | Optional with capability signalling |