3GPP TSG RAN WG1 Meeting #107bis-e	R1-2200646

e-Meeting, January 17th – 25th, 2022

Agenda Item:	8.12.4

Source:	Huawei, HiSilicon

Title:	Views on overall configuration for broadcast scheduling

Document for:	Discussion and Decision

# Introduction

The following agreement was reached in RAN1#106e meeting:

Agreement:

From RAN1 perspective, the CFR for broadcast reception of RRC_IDLE/INACTIVE UEs, includes at least the following configurations:

- One set of parameters configured for PDSCH for broadcast reception with GC-PDSCH

- One set of parameters configured for PDCCH for broadcast reception with GC-PDCCH

- FFS: whether some parameters configured for PDSCH/PDCCH are optional/needed for the supported cases of CFR.

- FFS: If necessary, depending on the cases supported, starting PRB and the number of PRBs

  - The reference for starting PRB is Point A. (Following the same approach to determine reference for starting PRB as that defined in AI8.12.1.)

Based on such an agreement and after some intermediate discussion/clarification, RAN1 group tended to assume there is no need to explicitly discuss each parameter one-by-one especially for those parameters with the same usage for unicast though there was no formal conclusion stating this for broadcast.

However, many parameters included in PDCCH-Config/PDSCH-Config for unicast are not needed for MBS broadcast scheduling based on the current reached agreements, which is a different case from MBS multicast that can mostly reuse the IEs for unicast. Instead of using the same IEs for unicast, TS 38.331 running CR from RAN2 [1] temporarily attempts to introduce new IEs PDCCH-ConfigBroadcast/PDSCH-ConfigBroadcast for broadcast which can be included in SIBx for MCCH or included in MCCH for MTCH.

In light of this, the RRC parameters list for NR MBS needs to enumerate explicitly what parameters are needed for MBS broadcast from RAN2 perspective for compiling the new IEs. To achieve this, this paper aims to provide the parameter list for MBS broadcast based on the analysis whether the parameters are needed or not for MBS broadcast.

# Discussion on configurations for MBS broadcast scheduling

The following table starts from all the parameters currently included in PDSCH-Config for unicast in the first column and derives whether they are needed or not in the last column based on the agreement (if any) in the middle column:

Table 1: Analysis of parameters for PDSCH carrying MBS broadcast

| Parameters in PDSCH-config | RAN1 agreement | Whether needed for MBS Broadcast? |
| --- | --- | --- |
| dataScramblingIdentityPDSCH INTEGER (0..1023) | Agreement: For initializing scrambling sequence generator for GC-PDSCH for MCCH/MTCH for broadcast, - nID equals the higher layer parameter dataScramblingIdentityPDSCH if it is configured in a CFR used for GC-PDSCH for MCCH/MTCH and the RNTI equals the G-RNTI or MCCH-RNTI; nID=NIDcell otherwise. - nRNTI corresponds to the RNTI associated with the GC-PDSCH transmission. | Yes FFS whether it is per G-RNTI or per serving cell. |
| dmrs-DownlinkForPDSCH-MappingTypeA SetupRelease { DMRS-DownlinkConfig } | Agreement: For LBRM and TBS determination for GC-PDSCH for broadcast reception: - the maximum number of layers is 1 | No DMRS configuration for PDSCH mapping Type A. When legacy UE receiving PDSCH scheduled by DCI format 1_1, the DMRS configuration of type, number of DMRS symbols can support more layers or more antenna ports. MBS broadcast only supports one layer, number of DMRS symbols, DMRS location and port can be fixed as receiving PDSCH scheduled by DCI format 1_0. Only DCI format 4_0 is supported for MBS broadcast (based on DCI format 1_0). |
| dmrs-DownlinkForPDSCH-MappingTypeB SetupRelease { DMRS-DownlinkConfig } |  | No DMRS configuration for PDSCH mapping Type B. When legacy UE receiving PDSCH scheduled by DCI format 1_1, the DMRS configuration of type, number of DMRS symbols can support more layers or more antenna ports. MBS broadcast only supports one layer, number of DMRS symbols, DMRS location and port can be fixed as receiving PDSCH scheduled by DCI format 1_0. Only DCI format 4_0 is supported for MBS broadcast (based on DCI format 1_0). |
| tci-StatesToAddModList |  | No A list of Transmission Configuration Indicator (TCI) states indicating a transmission configuration which includes QCL-relationships between the DL RSs in one RS set and the PDSCH DMRS ports. No TCI field in DCI format 4_0 for MBS broadcast. MBS broadcast are received by all RRC states UEs and targets to lower band. |
| rb-ToPRB-Interleaver ENUMERATED {n2, n4} | Agreement For broadcast reception, the following options is supported for VRB-to-PRB mapping field in the DCI format 1_0 for GC-PDCCH scheduling a GC-PDSCH - Opt-1: DCI includes the VRB-to-PRB mapping field with 1 bit according to Table 7.3.1.2.2-5 in TS 38.212 - Note: DL resource allocation type 0 is not supported in DCI format 1_0 | No DCI includes the VRB-to-PRB mapping field with 1 bit according to Table 7.3.1.2.2-5 in TS 38.212. No RRC Configuration is needed. |
| resourceAllocation ENUMERATED { resourceAllocationType0, resourceAllocationType1, dynamicSwitch} | Agreement For broadcast reception, the following options is supported for VRB-to-PRB mapping field in the DCI format 1_0 for GC-PDCCH scheduling a GC-PDSCH - Opt-1: DCI includes the VRB-to-PRB mapping field with 1 bit according to Table 7.3.1.2.2-5 in TS 38.212 - Note: DL resource allocation type 0 is not supported in DCI format 1_0 | No Only Type1 is supported for DCI format 4_0 scheduling Broadcast. |
| pdsch-TimeDomainAllocationList SetupRelease { PDSCH-TimeDomainResourceAllocationList } pdsch-TimeDomainAllocationList-r16 SetupRelease { PDSCH-TimeDomainResourceAllocationList-r16 } | Agreement For RRC_IDLE/INACTIVE UEs, for slot-level repetition for MTCH, support: - (Config A) UE can be configured with pdsch-AggregationFactor per G-RNTI, applied to DCI format 1_0 with the G-RNTI. - (Config B) UE can be configured with TDRA table with repetitionNumber as part of the TDRA table in PDSCH-Config-Broadcast - If UE is configured with Config B, UE does not expect to be configured with Config A for the same GC-PDSCH. | Yes. PDSCH-TimeDomainResourceAllocationList-R16 can be used including all parameters in PDSCH-TimeDomainResourceAllocationList, and thus PDSCH-TimeDomainResourceAllocationList is not needed. |
| pdsch-AggregationFactor ENUMERATED { n2, n4, n8 } | Agreement For RRC_IDLE/INACTIVE UEs, for slot-level repetition for MTCH, support: - (Config A) UE can be configured with pdsch-AggregationFactor per G-RNTI, applied to DCI format 1_0 with the G-RNTI. - (Config B) UE can be configured with TDRA table with repetitionNumber as part of the TDRA table in PDSCH-Config-Broadcast - If UE is configured with Config B, UE does not expect to be configured with Config A for the same GC-PDSCH. | Yes It is Per G-RNTI. |
| rateMatchPatternToAddModList |  | Yes The motivation was clarified in RAN1#106bis-e meeting and it was not pursued in RAN1#107-e meeting by assuming no need to explicitly agree on this parameters because it is included in PDSCH-config for unicast. |
| rateMatchPatternGroup1 rateMatchPatternGroup2 |  | No. Supporting semi-static rate-matching resource set configuration for MBS broadcast (as mandatory for unicast) is sufficient. No “Rate matching indicator” in DCI format 4_0 to dynamically shift the rateMatchPattern. |
| rbg-Size ENUMERATED {config1, config2}, |  | No This is not needed because “The UE ignores this field if resourceAllocation is set to resourceAllocationType1 (see TS 38.214 [19], clause 5.1.2.2.1).” |
| mcs-Table ENUMERATED {qam256, qam64LowSE} | Agreement For determination of maximum modulation order for LBRM and TBS determination for GC-PDSCH, - if mcs-Table in PDSCH-Config for MBS is not configured in CFR, Table 5.1.3.1-1 in TS38.214 is used (similar as the default value in R16). | Yes |
| maxNrofCodeWordsScheduledByDCI ENUMERATED {n1, n2} | Agreement For LBRM and TBS determination for GC-PDSCH for broadcast reception: - the maximum number of layers is 1 - the maximum modulation order can be determined from mcs-Table in PDSCH-Config for broadcast. - If mcs-Table in PDSCH-Config is not configured in CFR for broadcast, Table 5.1.3.1-1 in TS38.214 is used. | No The maximum number of 1 layer can only support one codeword. |
| prb-BundlingType CHOICE { staticBundling SEQUENCE { bundleSize ENUMERATED { n4, wideband } OPTIONAL -- Need S }, dynamicBundling SEQUENCE { bundleSizeSet1 ENUMERATED { n4, wideband, n2-wideband, n4-wideband } OPTIONAL, -- Need S bundleSizeSet2 ENUMERATED { n4, wideband } OPTIONAL -- Need S } }, |  | No The field prb-BundlingType applies to DCI format 1_1 for dynamically changing the precoding granularity. MBS broadcast can take the same approach as PDSCH with DCI format 1_0, i.e., UE shall assume that it is equal to 2 PRBs. |
| zp-CSI-RS-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-Resources)) OF ZP-CSI-RS-Resource zp-CSI-RS-ResourceToReleaseList SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-Resources)) OF ZP-CSI-RS-ResourceId aperiodic-ZP-CSI-RS-ResourceSetsToAddModList SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSet aperiodic-ZP-CSI-RS-ResourceSetsToReleaseList SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSetId sp-ZP-CSI-RS-ResourceSetsToAddModList SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSet sp-ZP-CSI-RS-ResourceSetsToReleaseList SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSetId p-ZP-CSI-RS-ResourceSet SetupRelease { ZP-CSI-RS-ResourceSet } | Agreements: - For RRC_CONNECTED UEs, existing CSI feedback can be used for multicast transmission. - FFS: whether enhancement is needed | No No CSI enhancement for multicast nor for broadcast (implicitly). DCI format 4_0 does not include the field of “ZP CSI-RS trigger”. |
| maxMIMO-Layers-r16 SetupRelease { MaxMIMO-LayersDL-r16 } | Agreement For LBRM and TBS determination for GC-PDSCH for broadcast reception: - the maximum number of layers is 1 - the maximum modulation order can be determined from mcs-Table in PDSCH-Config for broadcast. - If mcs-Table in PDSCH-Config is not configured in CFR for broadcast, Table 5.1.3.1-1 in TS38.214 is used. | No Maximum layer is 1 so no need to configure it. |
| minimumSchedulingOffsetK0-r16 SetupRelease { MinSchedulingOffsetK0-Values-r16 } |  | No DCI format 4_0 does not include the field of “Minimum applicable scheduling offset indicator”. |
| antennaPortsFieldPresenceDCI-1-2-r16 aperiodicZP-CSI-RS-ResourceSetsToAddModListDCI-1-2-r16 SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSet aperiodicZP-CSI-RS-ResourceSetsToReleaseListDCI-1-2-r16 SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSetId dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2-r16 SetupRelease { DMRS-DownlinkConfig } dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2-r16 SetupRelease { DMRS-DownlinkConfig } dmrs-SequenceInitializationDCI-1-2-r16 harq-ProcessNumberSizeDCI-1-2-r16 mcs-TableDCI-1-2-r16 numberOfBitsForRV-DCI-1-2-r16 pdsch-TimeDomainAllocationListDCI-1-2-r16 SetupRelease { PDSCH-TimeDomainResourceAllocationList-r16 } prb-BundlingTypeDCI-1-2-r16 priorityIndicatorDCI-1-2-r16 rateMatchPatternGroup1DCI-1-2-r16 rateMatchPatternGroup2DCI-1-2-r16 resourceAllocationType1GranularityDCI-1-2-r16 vrb-ToPRB-InterleaverDCI-1-2-r16 referenceOfSLIVDCI-1-2-r16 resourceAllocationDCI-1-2-r16 ENUMERATED { resourceAllocationType0, resourceAllocationType1, dynamicSwitch} |  | No These parameters can be independently configured for “URLLC” from that for “eMBB” or for scheduling using DCI format 1_2 from that for using DCI format 1_1. Only DCI format 4_0 is used for MBS broadcast scheduling. |
| priorityIndicatorDCI-1-1-r16 | Agreement: For the priority index for the first DCI format for GC-PDCCH, support the following Alt2 from the previous agreement: - Alt2: Always low priority, i.e., the priority index is not included in the DCI format. | No DCI format 4_0 (based on the first DCI format) does not include the field of “Priority indicator”. |
| dataScramblingIdentityPDSCH2-r16 |  | No The dataScramblingIdentityPDSCH2 is configured if coresetPoolIndex is configured with 1 for at least one CORESET in the same BWP (for the case mTRP). Standardized mTRP transmission is not assumed for MBS broadcast. |
| repetitionSchemeConfig-r16 SetupRelease { RepetitionSchemeConfig-r16} repetitionSchemeConfig-v1630 SetupRelease { RepetitionSchemeConfig-v1630 |  | No repetitionSchemeConfig configure the UE with repetition schemes. The network does not configure repetitionSchemeConfig-r16 and repetitionSchemeConfig-v1630 simultaneously to setup in the same PDSCH-Config. This is introduced by mTRP. Standardized mTRP transmission is not assumed for MBS broadcast. |
| xOverhead | Agreement The following agreements for RRC_CONECTED UEs also apply for broadcast reception with UEs in RRC_IDLE/ RRC_INACTIVE states, with the following updates: - xOverhead can be provided in PDSCH-Config for MBS in CFR; if not provided, a default value of zero is used. | Yes This IE is in PDSCH-ServingCellConfig , can be merged to PDSCH-Config for MBS |

Based on the above analysis, all parameters needed for MBS broadcast scheduling are:

- dataScramblingIdentityPDSCH

- PDSCH-TimeDomainResourceAllocationList

- pdsch-AggregationFactor (per G-RNTI)

- rateMatchPatternToAddModList

- mcs-Table

- xOverhead

The RRC parameters for MBS broadcast needs to update to reflect and enumerate all above parameters so as to support or help RAN2 compile TS 38.331 signaling formulation as expected.

Proposal: Update the RRC parameters list for MBS broadcast to enumerate the following parameters needed:

    - dataScramblingIdentityPDSCH

    - PDSCH-TimeDomainResourceAllocationList

    - pdsch-AggregationFactor (per G-RNTI)

    - rateMatchPatternToAddModList

    - mcs-Table

    - xOverhead

# Conclusions

This paper presents an analysis of what parameters are needed for MBS broadcast scheduling and proposes coordinating with RAN2 for TS 38.331 running CR compiling, based on the current parameters included in PDSCH-Config for unicast and the agreements reached, which leads to the following proposal:

Proposal: Update the RRC parameters list for MBS broadcast to enumerate the following parameters needed:

    - dataScramblingIdentityPDSCH

    - PDSCH-TimeDomainResourceAllocationList

    - pdsch-AggregationFactor (per G-RNTI)

    - rateMatchPatternToAddModList

    - mcs-Table

    - xOverhead

# References

R2-2200814, 38.331 running CR for NR MBS, Huawei, HiSilicon, RAN2#116bis-e, Jan. 17-25, 2022.