3GPP TSG RAN WG1 Meeting #107bis-e	R1-2200650

e-Meeting, January 17th – 25th, 2022

Agenda Item:	8.3.4

Source:	Huawei, HiSilicon

Title:	Enhancements for support of time synchronization

Document for:	Discussion and Decision

Introduction

Based on previous agreements, this contribution provides our further views on the RTT-based propagation delay compensation.

RTT based PDC

For RTT-based PDC, one remaining issue is whether to support positioning SRS for Rx-Tx time difference measurement [1]. Based on previous agreements, the detection error is mainly related to the SRS bandwidth, i.e. 0.5/(RS BW). From the gNB perspective, the detection error for positioning SRS and Rel-15 SRS may be similar if their bandwidth is configured to be the same. Thus there seems no need to consider positioning SRS for PDC since the Rel-15 SRS already is enough to satisfy the budget, and no additional benefits are obtained from introducing the positioning SRS for PDC. Considering RAN1 is in the maintenance phase now, we prefer not to consider positioning SRS, and it will also simplify the RRC signaling.

Proposal 1: Positioning SRS is not supported for Rx-Tx time difference measurement for RTT-based PDC in Rel-17.

RRC parameters

PRS for PDC

The Rel-16 PRS configuration in TS37.355 is copied below.

NR-DL-PRS-ResourceSet-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceSetID-r16			NR-DL-PRS-ResourceSetID-r16,

dl-PRS-Periodicity-and-ResourceSetSlotOffset-r16

NR-DL-PRS-Periodicity-and-ResourceSetSlotOffset-r16,

dl-PRS-ResourceRepetitionFactor-r16	ENUMERATED {n2, n4, n6, n8, n16, n32, ...}

OPTIONAL,	-- Need OP

dl-PRS-ResourceTimeGap-r16			ENUMERATED {s1, s2, s4, s8, s16, s32, ...}

OPTIONAL, 	-- Cond Rep

dl-PRS-NumSymbols-r16				ENUMERATED {n2, n4, n6, n12, ...},

dl-PRS-MutingOption1-r16			DL-PRS-MutingOption1-r16			OPTIONAL,	-- Need OP

dl-PRS-MutingOption2-r16			DL-PRS-MutingOption2-r16			OPTIONAL,	-- Need OP

dl-PRS-ResourcePower-r16			INTEGER (-60..50),

dl-PRS-ResourceList-r16				SEQUENCE (SIZE (1..nrMaxResourcesPerSet-r16)) OF

NR-DL-PRS-Resource-r16,

...

}

The parameter dl-PRS-ResourceRepetitionFactor-r16, dl-PRS-ResourceTimeGap-r16, dl-PRS-ResourcePower-r16 was discussed in last meeting, but unfortunately it is not included in the latest RRC parameters list [2] yet due to no consensus on whether these are necessary or not. Theses parameters can be used for any serving cell where the PRS is transmitted in Rel-16 positioning, including the current cell and the neighbor cell. We think reusing these parameters for PDC is straightforward and necessary, so we have the following proposal.

Proposal 2: Add “dl-PRS-ResourceRepetitionFactor-r16, dl-PRS-ResourceTimeGap-r16, dl-PRS-ResourcePower-r16” in the RRC parameters list for RTT-based PDC.

SRS for PDC

The Rel-15 SRS configuration in TS38.331 is copied below.

SRS-ResourceSet ::=                     SEQUENCE {

srs-ResourceSetId                       SRS-ResourceSetId,

srs-ResourceIdList                      SEQUENCE (SIZE(1..maxNrofSRS-ResourcesPerSet)) OF SRS-ResourceId    OPTIONAL, -- Cond Setup

resourceType                            CHOICE {

aperiodic                               SEQUENCE {

aperiodicSRS-ResourceTrigger            INTEGER (1..maxNrofSRS-TriggerStates-1),

csi-RS                                  NZP-CSI-RS-ResourceId                                  OPTIONAL, -- Cond NonCodebook

slotOffset                              INTEGER (1..32)                                        OPTIONAL, -- Need S

...,

[[

aperiodicSRS-ResourceTriggerList            SEQUENCE (SIZE(1..maxNrofSRS-TriggerStates-2))

OF INTEGER (1..maxNrofSRS-TriggerStates-1)     OPTIONAL  -- Need M

]]

},

semi-persistent                         SEQUENCE {

associatedCSI-RS                        NZP-CSI-RS-ResourceId                                  OPTIONAL, -- Cond NonCodebook

...

},

periodic                                SEQUENCE {

associatedCSI-RS                        NZP-CSI-RS-ResourceId                                  OPTIONAL, -- Cond NonCodebook

...

}

},

usage                                   ENUMERATED {beamManagement, codebook, nonCodebook, antennaSwitching},

alpha                                   Alpha                                                          OPTIONAL, -- Need S

p0                                      INTEGER (-202..24)                                             OPTIONAL, -- Cond Setup

pathlossReferenceRS                     PathlossReferenceRS-Config                                     OPTIONAL, -- Need M

srs-PowerControlAdjustmentStates        ENUMERATED { sameAsFci2, separateClosedLoop}                   OPTIONAL, -- Need S

...,

[[

pathlossReferenceRSList-r16             SetupRelease { PathlossReferenceRSList-r16}                    OPTIONAL  -- Need M

]]

}

The SRS configuration for PDC was discussed in last meeting but it is also not in the latest RRC parameters list [2]. We think one alternative is to add PDC in the usage in the SRS-ResourceSet to enable Rel-15 SRS configuration for PDC. Considering that the max number of SRS-ResourceSet in a BWP can be 16, so this may not much impact to the MIMO SRS configurations. And the PRS can also be used for the path loss estimation for the SRS transmission since it is possible that the Rx-Tx measurement can be based on PRS and SRS. One example for the SRS configuration for PDC is below with yellow changes.

SRS-ResourceSet ::= SEQUENCE { srs-ResourceSetId SRS-ResourceSetId, srs-ResourceIdList SEQUENCE (SIZE(1..maxNrofSRS-ResourcesPerSet)) OF SRS-ResourceId OPTIONAL, -- Cond Setup resourceType CHOICE { aperiodic SEQUENCE { aperiodicSRS-ResourceTrigger INTEGER (1..maxNrofSRS-TriggerStates-1), csi-RS NZP-CSI-RS-ResourceId OPTIONAL, -- Cond NonCodebook slotOffset INTEGER (1..32) OPTIONAL, -- Need S ..., [[ aperiodicSRS-ResourceTriggerList SEQUENCE (SIZE(1..maxNrofSRS-TriggerStates-2)) OF INTEGER (1..maxNrofSRS-TriggerStates-1) OPTIONAL -- Need M ]] }, semi-persistent SEQUENCE { associatedCSI-RS NZP-CSI-RS-ResourceId OPTIONAL, -- Cond NonCodebook ... }, periodic SEQUENCE { associatedCSI-RS NZP-CSI-RS-ResourceId OPTIONAL, -- Cond NonCodebook ... } }, usage ENUMERATED {beamManagement, codebook, nonCodebook, antennaSwitching}, alpha Alpha OPTIONAL, -- Need S p0 INTEGER (-202..24) OPTIONAL, -- Cond Setup pathlossReferenceRS PathlossReferenceRS-Config OPTIONAL, -- Need M srs-PowerControlAdjustmentStates ENUMERATED { sameAsFci2, separateClosedLoop} OPTIONAL, -- Need S ..., [[ pathlossReferenceRSList-r16 SetupRelease { PathlossReferenceRSList-r16} OPTIONAL -- Need M ]] usage-r17 ENUMERATED {beamManagement, codebook, nonCodebook, antennaSwitching, PDC}, } PathlossReferenceRS-Config-r17 ::= CHOICE { ssb-Index SSB-Index, csi-RS-Index NZP-CSI-RS-ResourceId dl-PRS-PDC dl-PRS-ResourceId-r16 }

In last meeting, it was also discussed to introduce a separate IE for SRS configuration for PDC. One example is below.

SRS-PDCResourceSet-r17 SEQUENCE (SIZE(1)) OF srs-ResourceSet SRS-ResourceSet ::= SEQUENCE { srs-ResourceSetId SRS-ResourceSetId, srs-ResourceIdList SEQUENCE (SIZE(1..maxNrofSRS-ResourcesPerSet)) OF SRS-ResourceId OPTIONAL, -- Cond Setup resourceType CHOICE { aperiodic SEQUENCE { aperiodicSRS-ResourceTrigger INTEGER (1..maxNrofSRS-TriggerStates-1), csi-RS NZP-CSI-RS-ResourceId OPTIONAL, -- Cond NonCodebook slotOffset INTEGER (1..32) OPTIONAL, -- Need S ..., [[ aperiodicSRS-ResourceTriggerList SEQUENCE (SIZE(1..maxNrofSRS-TriggerStates-2)) OF INTEGER (1..maxNrofSRS-TriggerStates-1) OPTIONAL -- Need M ]] }, semi-persistent SEQUENCE { associatedCSI-RS NZP-CSI-RS-ResourceId OPTIONAL, -- Cond NonCodebook ... }, periodic SEQUENCE { associatedCSI-RS NZP-CSI-RS-ResourceId OPTIONAL, -- Cond NonCodebook ... } }, usage ENUMERATED {beamManagement, codebook, nonCodebook, antennaSwitching}, alpha Alpha OPTIONAL, -- Need S p0 INTEGER (-202..24) OPTIONAL, -- Cond Setup pathlossReferenceRS PathlossReferenceRS-Config OPTIONAL, -- Need M srs-PowerControlAdjustmentStates ENUMERATED { sameAsFci2, separateClosedLoop} OPTIONAL, -- Need S ..., [[ pathlossReferenceRSList-r16 SetupRelease { PathlossReferenceRSList-r16} OPTIONAL -- Need M ]] } PathlossReferenceRS-Config-r17 ::= CHOICE { ssb-Index SSB-Index, csi-RS-Index NZP-CSI-RS-ResourceId dl-PRS-PDC dl-PRS-ResourceId-r16 }

The two examples above are workable, and we slightly prefer to add PDC in the usage in the SRS-ResourceSet to simplify the spec.

Proposal 3: In the RRC parameters list for RTT-based PDC, prefer to add “PDC” in the usage IE in the SRS-ResourceSet.

Proposal 4: In the RRC parameters list for RTT-based PDC, add PRS as the path loss reference RS for SRS transmission.

Conclusion

In this paper, we give our considerations for the enhancements in Rel-17.

Proposal 1: Positioning SRS is not supported for Rx-Tx time difference measurement for RTT-based PDC in Rel-17.

Proposal 2: Add “dl-PRS-ResourceRepetitionFactor-r16, dl-PRS-ResourceTimeGap-r16, dl-PRS-ResourcePower-r16” in the RRC parameters list for RTT-based PDC.

Proposal 3: In the RRC parameters list for RTT-based PDC, prefer to add “PDC” in the usage IE in the SRS-ResourceSet.

Proposal 4: In the RRC parameters list for RTT-based PDC, add PRS as the path loss reference RS for SRS transmission.

References

- R1- 2112978 Summary of Email discussion on Rel-17 RRC parameters for LS to RAN2 Moderator (Ericsson)

- R1-2112976 Consolidated higher layers parameter list for Rel-17 NR