3GPP TSG RAN WG1 #107bis-e		      R1-2200671

e-meeting, January 17th – 25th, 2021

Source:	Lenovo, Motorola Mobility

Title: 	Remaining issues on PDCCH for NR from 52.6 GHz to 71GHz

Agenda Item:   8.2.2

Document for:	Discussion

# 1	Introduction

In RAN#93-e, a revised work item for supporting NR from 52.6 GHz to 71 GHz in NR Rel. 17 has been approved with following objectives for PDCCH monitoring in [1]:

- Physical layer aspects including [RAN1]:

  - In addition to 120kHz SCS, specify new SCS, 480kHz and 960kHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported.

Note: Except for timing line related aspects, a common design framework shall be adopted for 480kHz to 960kHz

  - Time line related aspects adapted to 480kHz and 960kHz, e.g., BWP and beam switching timeing, HARQ timing, UE processing, preparation and computation timelines for PDSCH, PUSCH/SRS and CSI, respectively.

  - Support enhancements for multi-PDSCH/PUSCH scheduling and HARQ support with a single DCI

Note: coverage enhancement for multi-PDSCH/PUSCH scheduling is not pursued

  - Support enhancement to PDCCH monitoring, including blind detection/CCE budget, and multi-slot span monitoring, potential limitation to UE PDCCH configuration and capability related to PDCCH monitoring.

In this contribution, we discuss some of the remaining issues for multi-slot PDCCH monitoring for NR operation between 52.6 GHz and 71GHz.

# 2	Multi-slot PDCCH monitoring

In RAN1#107 [2], following agreement related to multi-slot PDCCH monitoring has been made:

Agreement

- For Group (1) SS: Type 1 CSS with dedicated RRC configuration and type 3 CSS, UE specific SS

  - A SS is monitored within Y consecutive slots within a slot group of X slots

  - The Y consecutive slots can be located anywhere within the slot group of X slots

    - Note: There is no requirement to align the Y consecutive slots across UEs or with slot n0

  - The location of the Y consecutive slots within the slot group of X slots is maintained across different slot groups

  - BD attempts for all Group (1) SSs are restricted to fall within the same Y consecutive slots

- For Group (2) SS: Type 1 CSS without dedicated RRC configuration and type 0, 0A, and 2 CSS

  - SS monitoring locations can be anywhere within a slot group of X slots, with the following exception

    - BD attempts for Type0-CSS for SSB/CORESET 0 multiplexing pattern 1, and additionally for Type0A/2-CSS if searchSpaceId = 0, occur in slots with index n0 and n0+X0, where n0 is as in Rel-15, X0=4 for 480 kHz SCS and X0=8 for 960 kHz SCS.

- Supported combinations of (X,Y)

  - A UE capable of multi-slot monitoring mandatorily supports

    - For SCS 480 kHz: (X,Y) = (4,1)

    - For SCS 960 kHz: (X,Y) = (8,1)

  - A UE capable of multi-slot monitoring optionally supports

    - For SCS 480 kHz: (X,Y) = (4,2)

    - For SCS 960 kHz: (X,Y) = (8,4), (4,2), (4,1)

      - Working assumption: BD/CCE budget for (4,2), (4,1) is half that of X=8

- A UE capable of multi-slot monitoring mandatorily supports the following PDCCH monitoring within Y slots

  - For Y>1: FG3-1 (monitoring Group (1) SSs in the first 3 OFDM symbols of each of the Y slots)

  - For 960 kHz SCS For Y=1: FG3-5b with set1 = (7, 3)

    - [FL Note: The first number is the minimum gap in symbols between the start of two spans, the second number is the span duration in symbols (cf. TS 38.822)]

  - For 480 kHz SCS For Y=1: FG3-5b with set2 = (4, 3) and (7, 3) with a modification with maximum two monitoring spans in a slot

    - [FL Note: The first number is the minimum gap in symbols between the start of two spans, the second number is the span duration in symbols (cf. TS 38.822)]

  - The following supersedes FG3-5b and FG3-1 definition:

  - Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot group of X slots per scheduled CC for FDD

  - Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot group of X slots per scheduled CC for TDD

From the above agreement, there could be the issue of back-to-back monitoring across slot groups e.g. when the beam changes, e.g. with respect to the above proposal when n0 changes and also when the periodicity of Group(2) SS MO is every slot group.

Observation 1: For supporting NR between 52.6 GHz and 71 GHz with high subcarrier spacing values including 480kHz and 960kHz, when multi-slot PDCCH monitoring is applied with shifting of Group(2) SS due to n0 change, then potential back-to-back monitoring issue can arise across slot groups where the shift is applied, and periodicity of Group(s) SS MOs is every slot group

One potential solution could be to drop any Group(1) SS MOs and/or Group(2) SS MOs in the slot group in which shifting needs to be applied such that back-to-back monitoring issue can be avoided.

Proposal 1: For supporting NR between 52.6 GHz and 71 GHz with high subcarrier spacing values including 480kHz and 960kHz, then dropping of Group(1) SS MOs and/or Group(2) SS MOs in the slot where the shift is first applied should be supported to avoid back-to-back monitoring issue.

# 3	Search Space Configuration

With multi-slot PDCCH monitoring, the search space configuration enhancements are also required in terms of periodicity and indication of monitoring slots within the slot group. For the periodicity configuration, it makes sense to allow only multiple of X i.e. slot group duration to have alignment across slot groups

Proposal 2: For supporting NR between 52.6 GHz and 71 GHz with high subcarrier spacing values including 480kHz and 960kHz, support periodicity of PDCCH monitoring in search space configuration only in multiples of slot group duration.

Furthermore, currently in search space configuration, a bitmap is configured to indicate the symbols within a slot for PDCCH monitoring occasions within a slot. Similarly for multi-slot PDCCH monitoring, a bitmap to indicate the slots containing PDCCH monitoring occasions should be configured. An example illustration for a 4-slot duration with a 4-bit bitmap for monitoringSlotsWithinGroup indicating “1010” is shown in Figure 1, where the first and third slot within the group have monitoring occasions.

Figure 1: Example of monitoring slots indication for a multi-slot PDCCH monitoring

Proposal 3: For supporting NR between 52.6 GHz and 71 GHz with high subcarrier spacing values including 480kHz and 960kHz, support configuration of a new bitmap to indicate which slots within the slot group contain monitoring occasions. The size of the bitmap is equal to the number of slots per slot group (i.e. X). The indication of symbols within a slot for PDCCH monitoring applies only to slots indicated by the new bitmap.

# 4	PDCCH multi-beam aspects

Another important aspect for PDCCH monitoring is related to directional LBT. Directional LBT may cause some issues in comparison with omni-directional LBT. For example, different Tx beams used by gNB may correspond to different COTs, thus different CORESETs which are configured with different Tx beams by higher layer signaling may also correspond to different COTs. From power saving perspective, according to the indicated sensing beam(s) used by the gNB to initiate a COT, a UE does not need to monitor PDCCH occasions in a CORESET corresponding to a different Tx beam which is not ‘covered’ by the indicated sensing beam(s) until the current COT ends, which can reduce the power consumption caused by blind decoding.

Proposal 4: For NR unlicensed bands between 52.6 GHz and 71 GHz with directional LBT based channel access mechanism, within a COT, PDCCH monitoring is not supported in the CORESETs corresponding to other COTs (PDCCH monitoring restricted to monitoring corresponding to only one COT at a time)

# 5	Conclusion

In this contribution, we discussed enhancements for PDCCH monitoring for NR operation between 52.6 GHz to 71 GHz with high SCS values and provided following observations/proposals:

Observation 1: For supporting NR between 52.6 GHz and 71 GHz with high subcarrier spacing values including 480kHz and 960kHz, when multi-slot PDCCH monitoring is applied with shifting of Group(2) SS due to n0 change, then potential back-to-back monitoring issue can arise across slot groups where the shift is applied, and periodicity of Group(s) SS MOs is every slot group

Proposal 1: For supporting NR between 52.6 GHz and 71 GHz with high subcarrier spacing values including 480kHz and 960kHz, then dropping of Group(1) SS MOs and/or Group(2) SS MOs in the slot where the shift is first applied should be supported to avoid back-to-back monitoring issue.

Proposal 2: For supporting NR between 52.6 GHz and 71 GHz with high subcarrier spacing values including 480kHz and 960kHz, support periodicity of PDCCH monitoring in search space configuration only in multiples of slot group duration.

Proposal 3: For supporting NR between 52.6 GHz and 71 GHz with high subcarrier spacing values including 480kHz and 960kHz, support configuration of a new bitmap to indicate which slots within the slot group contain monitoring occasions. The size of the bitmap is equal to the number of slots per slot group (i.e. X). The indication of symbols within a slot for PDCCH monitoring applies only to slots indicated by the new bitmap.

Proposal 4: For NR unlicensed bands between 52.6 GHz and 71 GHz with directional LBT based channel access mechanism, within a COT, PDCCH monitoring is not supported in the CORESETs corresponding to other COTs (PDCCH monitoring restricted to monitoring corresponding to only one COT at a time)

# 6	References

[1]	3GPP RP-211584, “Revised WID:  Extending current NR operation to 71 GHz”, CMCC

[2] 	3GPP RAN1#107-e, “Chairman Notes