3GPP TSG RAN WG1#107bis-e		R1-2200711

e-Meeting, January 17th – 25th, 2022

Agenda Item:	8.2.2

Source:	Moderator (Lenovo)

Title:	Feature lead summary #3 for B52.6 GHz PDCCH monitoring enhancements

Document for:	Discussion, Decision

# Introduction

As stated by the chairman:

[107bis-e-R17-52-71GHz-02] Email discussion/approval on PDCCH monitoring enhancements – Alex (Lenovo)

- 1st check point: January 20

- Final check point: January 25

Depending on the progress, new questions or proposals may be added after the defined checkpoints.

# Discussion

FL NOTE: Excerpts from submitted documents are listed in Section 3.

## Topic A1: Blind Decoding Capability, Multi-slot monitoring

In RAN1#107-e, the following agreement has been achieved:

Agreement - For Group (1) SS: Type 1 CSS with dedicated RRC configuration and type 3 CSS, UE specific SS - A SS is monitored within Y consecutive slots within a slot group of X slots - The Y consecutive slots can be located anywhere within the slot group of X slots - Note: There is no requirement to align the Y consecutive slots across UEs or with slot n0 - The location of the Y consecutive slots within the slot group of X slots is maintained across different slot groups - BD attempts for all Group (1) SSs are restricted to fall within the same Y consecutive slots - For Group (2) SS: Type 1 CSS without dedicated RRC configuration and type 0, 0A, and 2 CSS - SS monitoring locations can be anywhere within a slot group of X slots, with the following exception - BD attempts for Type0-CSS for SSB/CORESET 0 multiplexing pattern 1, and additionally for Type0A/2-CSS if searchSpaceId = 0, occur in slots with index n0 and n0+X0, where n0 is as in Rel-15, X0=4 for 480 kHz SCS and X0=8 for 960 kHz SCS. - Supported combinations of (X,Y) - A UE capable of multi-slot monitoring mandatorily supports - For SCS 480 kHz: (X,Y) = (4,1) - For SCS 960 kHz: (X,Y) = (8,1) - A UE capable of multi-slot monitoring optionally supports - For SCS 480 kHz: (X,Y) = (4,2) - For SCS 960 kHz: (X,Y) = (8,4), (4,2), (4,1) - Working assumption: BD/CCE budget for (4,2), (4,1) is half that of X=8 - A UE capable of multi-slot monitoring mandatorily supports the following PDCCH monitoring within Y slots - For Y>1: FG3-1 (monitoring Group (1) SSs in the first 3 OFDM symbols of each of the Y slots) - For 960 kHz SCS For Y=1: FG3-5b with set1 = (7, 3) - [FL Note: The first number is the minimum gap in symbols between the start of two spans, the second number is the span duration in symbols (cf. TS 38.822)] - For 480 kHz SCS For Y=1: FG3-5b with set2 = (4, 3) and (7, 3) with a modification with maximum two monitoring spans in a slot - [FL Note: The first number is the minimum gap in symbols between the start of two spans, the second number is the span duration in symbols (cf. TS 38.822)] - The following supersedes FG3-5b and FG3-1 definition: - Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot group of X slots per scheduled CC for FDD - Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot group of X slots per scheduled CC for TDD

### Issue A1-1: Configuration of multi-slot monitoring – Potential RRC impact

#### First round discussion

Huawei (R1-2200045):

Value r17monitoringcapability should be added to monitoringCapabilityConfig.

Ericsson (R1-2200401):

Inform RAN2 that the value range for the existing parameter monitoringCapabilityConfig has been extended with new value r17monitoringcapability. A note can be added to the RRC parameter spreadsheet for RAN2 to update the field description as follows:

monitoringCapabilityConfig

Configures either Rel-15 PDCCH monitoring capability or Rel-16 PDCCH monitoring capability for PDCCH monitoring on a serving cell. Value r15monitoringcapablity enables the Rel-15 monitoring capability, and value r16monitoringcapablity enables the Rel-16 PDCCH monitoring capability. Value r17monitoringcapablity enables the Rel-17 PDCCH monitoring capability only applicable to 480 and 960 kHz SCS (see TS 38.213 [13], clause 10.1).

ZTE (R1-2200260):

  - Proposal 1: There is no need to introduce monitoringCapabilityConfig = r17monitoringcapability if per-slot monitoring is not supported for SCS 480/960 kHz.

| Company | Comment |
| --- | --- |
| Ericsson | Since the spec editor has written Section 10 of 38.213 using monitoringCapabilityConfig = r17monitoringcapability, we think it is fine to keep this configuration, and then define appropriate default behavior in 38.213 for when the parameter monitoringCapabilityConfig is absent, e.g., during initial access. For example, the default behavior for 480/960 can be to monitor according to (Xs,Ys) = (4,1)/(8,1), respectively. This issue overlaps with Issue A1-4. Please see our more detailed comments under Issue A1-4. |
| Qualcomm | We agree with Ericsson’s view. For both backward and forward compatible design, we think it would be good to keep a consistent structure of configuration parameters. Also, for 480/960 kHz SCSs for Rel-17, it should be restricted that monitoringCapabilityConfig should always be configured (or assumed, if not configured) as r17monitoringcapability. |
| Intel | We share ZTE’s view that r17monitoringcapability is not needed. The configuration of monitoringCapabilityConfig is per BWP per cell. Since we concluded in the GTW session that per-slot PDCCH monitoring is not support for SCS 480/960kHz, r17monitoringcapability is redundant. On the other hand, we share other companies’ view that default combination (Xs, Ys) = (4,1)/(8,1) should be reflected in specification. |
| Nokia, NSB | Agree with the Proposal 1. |
| vivo | We share ZTE and Intel’s view that r17monitoringcapability is not needed and redundant since it is already agreed that single slot PDCCH monitoring is not supported for 480K and 960K SCS. Besides, we already provide our comment in spec review stage on this issue and spec editor add the following note to this sentence: Editor’s choice to write the statement following similar ones from R15/16. All references to r17monitoringcapability are subject to removal as the capability may be implicit based on operation with 480/960 kHz. |
| Apple | We agree with ZTE’s proposal that there is no need for r17monitoringcapability. Note that as it is currently in the specification, it should be removed. |
| InterDigital | We also share the view that r17monitoringcapability is not needed and the default combination should be captured in the spsecification. |
| Xiaomi | Prefer Ericsson and Qualcomm’s view, but we are flexible on this issue |
| Samsung | Technically we agree that there seems no essential need to introduce such a RRC parameter value for multi-slot based PDCCH monitoring, but since it’s already in the specification, and we observed that it will remarkably reduce the complexity to describe multi-slot based PDCCH monitoring in the specification with such RRC parameter, then it would be good to keep it. |
| OPPO | Agree that r17monitoringcapability is not needed. |
| Futurewei | Because it is already in the specs it would be useful to keep it to describe the multi-slot PDCCH monitoring. |
| NTT DOCOMO | We are fine to introduce r17monitoringcapability for monitoringCapabilityConfig to follow Rel-15/16. In addition, indeed monitoringCapabilityConfig would be absent during initial access and /or IDLE mode and agree that multi-slot PDCCH monitoring capability should be applied for such cases at least for 480 kHz SCS. For 960 kHz SCS, it should be clarified whether monitoringCapabilityConfig is always configured or can be absent for 960 kHz SCS. |
| ZTE, Sanechips | The following agreements/conclusions are made in the previous meetings: Conclusion: For 120 kHz SCS, no multi-slot UE capability for PDCCH monitoring is needed. Agreement: - A UE supporting 480 kHz SCS supports multi-slot PDCCH monitoring for 480 kHz SCS. - A UE supporting 960 kHz SCS supports multi-slot PDCCH monitoring for 960 kHz SCS. - FFS: whether to apply multi-slot PDCCH monitoring at all times and for all search spaces. Conclusion (GTW session on 17-Jan-2022) Single-slot PDCCH monitoring for 480 kHz and 960 kHz is not supported. For SCS configuration of μ=0 and μ=1, UE can support both r15monitoringcapability and r16monitoringcapability, it is necessary to use an indication parameter to specify the exact capability. If the UE is not provided monitoringCapabilityConfig, the UE monitors PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per slot. However, it will not cause any ambiguity since multi-slot PDCCH monitoring is only supported for SCS 480/960kHz and there is no need to introduce monitoringCapabilityConfig = r17monitoringcapability. Moreover, we can further clarify this: If a UE is provided monitoringCapabilityConfig for a serving cell, the UE obtains an indication to monitor PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs - per slot, as in Tables 10.1-2 and 10.1-3, if monitoringCapabilityConfig = r15monitoringcapability, or - per span, as in Tables 10.1-2A and 10.1-3A, if monitoringCapabilityConfig = r16monitoringcapability If the UE is not provided monitoringCapabilityConfig, the UE monitors PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per slot for μ∈0, 1,2,3 . For μ=5 or μ=6, Regardless of monitoringCapabilityConfig, the UE monitors PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per group of X_s slots according to combination Xs,Ys, as in Tables 10.1-2B and 10.1-3B. |
| Lenovo, Motorola Mobility | We support Proposal 1 |
| Transsion | We share the same view as ZTE that the parameter r17monitoringcapability is redundant, since only multi-slot PDCCH monitoring is supported for 480/960kHz SCS. |
| Sharp | We agree to leave r17monitoringcapability because it is already specified. It is assumed that the default monitoring will be used if this parameter is not configured. |
| LG Electronics | We share the same view with ZTE, Intel, vivo, Apple and ZTE. We think r17monitoringcapability is redundant since there is no other option other than multi-slot monitoring for 480/960 kHz SCS. We also share the view with other companies that default (X,Y) combination, i.e., (4,1) for 480 kHz and (8,1) for 960 kHz, should be reflected in specification. |
| Panasonic | We share the same view as ZTE, Intel, vivo, Apple, ZTE and LGE that r17monitoringcapability is not needed. |
| Huawei/HiSilicon | Although there is no ambiguity that multi-slot PDCCH monitoring is the only supported choice for 480/960 kHz, we think it is better to keep r17monitoringcapability as an efficient way to refer to the Rel-17 multi-slot PDCCH monitoring capability for 480/960 kHz. Agree with Ericsson that the default behavior for 480/960 kHz when monitoringCapabilityConfig is not provided should be specified. |
| LG Electronics2 | Aside from agreeing with the above proposals, reading related TPs and comments from Samsung and Ericsson, it seems that the TP proposed by vivo also changes the existing behavior. In this case, it may be necessary to check what kind of spec impact there is. |
| CATT | We share the same view as ZTE r17monitoringcapability is not needed. |

Vivo (R1-2200075):

Proposal 3: For NR Rel-17 Ues, PDCCH monitoring capability is defined per BWP and configuration of 480K/960K SCS for a BWP implies multi-slot-based capability for that BWP.

Proposal 4: PDCCH monitoring capability for a serving cell is the capability for its active BWP or configured first active BWP when it is deactivated.

| Company | Comment |
| --- | --- |
| Intel | We agree with the two vivo proposals |
| vivo | We support the above proposals as proponent company. Looking at the text below in TS 38.213, it seems that PDCCH monitoring capability is defined per serving cell. This is fine for Rel-16 but not reasonable for Rel-17 with multi-slot monitoring capability. If a serving cell has two BWPs where one BWP is configured with 120KHz SCS and the other BWP with 480/960KHz, it is not possible to have the same capability type per serving cell. So PDCCH monitoring capability should be defined per BWP. Currently monitoringCapabilityConfig is already configured per BWP and no need to change this RRC signaling. For BWP with 480/960KHz SCS, as described above, there is no need to have r17monitoringcapability and 480/960KHz SCS imply multi-slot-based capability for that BWP. If a UE is provided monitoringCapabilityConfig for a serving cell, the UE obtains an indication to monitor PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs - per slot, as in Tables 10.1-2 and 10.1-3, if monitoringCapabilityConfig = r15monitoringcapability, or - per span, as in Tables 10.1-2A and 10.1-3A, if monitoringCapabilityConfig = r16monitoringcapability - per group of Xs slots according to combination Xs,Ys, as in Tables 10.1-2B and 10.1-3B, if monitoringCapabilityConfig = r17monitoringcapability If the UE is not provided monitoringCapabilityConfig, the UE monitors PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per slot. In multi-serving cell case, the serving cells with the same capability type will be grouped together for further check on whether to exceed corresponding cell capacity limit. This should be based on determination of monitoring capability type for each serving cell. As long as PDCCH monitoring capability type is defined per BWP, it is naturally that PDCCH monitoring capability for a serving cell is the capability for its active BWP or configured first active BWP when it is deactivated. |
| Apple | Agree with Vivo’s argument that the PDCCH monitoring capability be defined per BWP. |
| InterDigital | We are fine with the proposals. |
| Samsung | We understand the intention of the proposals, but don’t fully agree with the TP associated with the proposals. |
| Futurewei | We are fine with the proposals. |
| ZTE, Sanechips | We are generally fine with this proposal, it’s better to clarify “PDCCH monitoring capability” in the proposal refers to monitoringCapabilityConfig. |
| Transsion | We are fine with the proposals. |
| Sharp | Agree with the proposals. |
| LG Electronics | We are fine with the proposals. |
| Panasonic | We are fine with the proposals. |
| Ericsson | We have similar concerns as Samsung on the TP associated with this proposal. Also, does the TP change legacy behavior? As pointed out by vivo monitoringCapabilityConfig is configured per BWP, and so is SCS, to isn’t it already clear that the UE follows the multi-slot monitoring behavior corresponiding to the configured SCS for the active BWP, even if 38.213 says “for a serving cell.” The UE is still monitoring on a serving cell irrespective of what is the active BWP. |
| Huawei, HiSilicon | We think it is only required to agree on the following modified version of Proposal 3 and TP from vivo: Proposal 3 (modified): For NR Rel-17 Ues, PDCCH monitoring capability monitoringCapabilityConfig is defined per BWP and configuration of 480K/960K SCS for a BWP implies multi-slot-based capability for that BWP. TP: If a UE is provided monitoringCapabilityConfig for a serving cell BWP, the UE obtains an indication to monitor PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs - per slot, as in Tables 10.1-2 and 10.1-3, if monitoringCapabilityConfig = r15monitoringcapability, or - per span, as in Tables 10.1-2A and 10.1-3A, if monitoringCapabilityConfig = r16monitoringcapability - per group of Xs slots according to combination Xs,Ys, as in Tables 10.1-2B and 10.1-3B, if monitoringCapabilityConfig = r17monitoringcapability For BWP SCS configuration μ<=3, if the UE is not provided monitoringCapabilityConfig, the UE monitors PDCCH on the serving cell BWP for a maximum number of PDCCH candidates and non-overlapping CCEs per slot. Of course, the default PDCCH monitoring behavior for 480/960 kHz should be additionally captured (this is an independent subject though) |
| Qualcomm | We share the same view as Ericsson – the issue is the same for Rel-16, since monitoringCapabilityConfig is already per-BWP configuration in Rel-16. So, if there really is ambiguity and we need to fix it, the fix should also be applied to the legacy Rel-16 behavior. |
| Vivo | We agree with Huawei’s modified TP except that monitoringCapabilityConfig is not needed. If a UE is provided monitoringCapabilityConfig for a serving cell BWP, the UE obtains an indication to monitor PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs - per slot, as in Tables 10.1-2 and 10.1-3, if monitoringCapabilityConfig = r15monitoringcapability, or - per span, as in Tables 10.1-2A and 10.1-3A, if monitoringCapabilityConfig = r16monitoringcapability - per group of Xs slots according to combination Xs,Ys, as in Tables 10.1-2B and 10.1-3B, if monitoringCapabilityConfig = r17monitoringcapability For BWP SCS configuration μ<=3, if the UE is not provided monitoringCapabilityConfig, the UE monitors PDCCH on the serving cell BWP for a maximum number of PDCCH candidates and non-overlapping CCEs per slot. For BWP SCS configuration μ=5 or 6, the UE monitors PDCCH per group of Xs slots according to combination Xs,Ys, as in Tables 10.1-2B and 10.1-3B. @Samsung, Ericsson and Qualcomm: In the above TP, the only change for Rel-16 spec is from a serving cell to a BWP. As we indicated, monitoringCapabilityConfig is already configured per BWP in NR Rel-16 and it is still aligned with Rel-16 behavior. |
| CATT | We are fine with vivo’s TP |
| Ericsson2 | We don't think this TP is needed, despite the fact that monitoringCapabilityConfig is configured per BWP – it doesn't create any ambiguity. There is only one BWP monitored on a serving cell at a time anyway. We also note that the maximum number of PDCCH candidates and non-overlapping CCEs is defined per serving cell, so the 2nd instance of changing serving cell to BWP makes it look like that is defined per BWP, which is not correct. |

ZTE (R1-2200260):

  - Proposal 2: Further clarification in TS 38.213: if a UE does not provide any monitoring capability combinations Xs,Ys or if a UE indicates a capability to monitor PDCCH according to more combinations Xs,Ys, Xs,Ys = (4, 1) or (8, 1) is mandatorily supported for SCS configuration μ=5 or μ=6 respectively if per-slot monitoring is not supported for SCS 480/960kHz.

| Company | Comment |
| --- | --- |
| Qualcomm | We support the proposal. |
| Intel | We share other companies’ view that default combination (Xs, Ys) = (4,1)/(8,1) should be reflected in specification |
| Nokia, NSB | We are ok with the proposal. |
| vivo | The mandatory support of Xs,Ys = (4, 1) or (8, 1) for SCS configuration μ=5 or μ=6 could be reflected in UE feature. |
| Apple | We are fine with this proposal. |
| InterDigital | We also think that the default combinations should be captured in the specification. |
| Samsung | We support the proposal, and proposed the similar one summarized into Issue A1-4. |
| OPPO | Agree to clarify |
| Futurewei | We are fine with the proposal |
| CATT1 | Fine with the intention but not sure if this needs to be clarified in 38.213. |
| ZTE, Sanechips | We support this proposal as proponent. |
| Transsion | We are fine with the proposal. |
| Sharp | Agree with the proposal. |
| LG Electronics | We agree that the default combination (Xs, Ys) = (4,1)/(8,1) should be reflected in specification |
| Panasonic | The mandatory support of (Xs, Ys) = (4,1) for u=5 and (8,1) for u=6 can be captured in UE feature. |
| Ericsson | This issue overlaps A1-4, and there are TPs proposed in that section to resolve this issue. |
| Huawei, HiSilicon | We already concluded that per-slot monitoring is not supported for SCS 480/960 kHz. We also think the text in proposal 2 should not be restricted to the condition that “if a UE indicates a capability to monitor PDCCH according to more combinations Xs,Ys”. We think specification only need to agree on this: Proposal: Capture the following in TS 38.213 Xs,Ys = (4, 1) or (8, 1) is mandatorily supported for SCS configuration μ=5 or μ=6, respectively. |

#### Summary of first round discussion

There is an almost even split between companies preferring to add r17monitoringcapability should be added to monitoringCapabilityConfig and removing references to monitoringCapabilityConfig = r17monitoringcapability from 38.213. Companies who prefer to have/keep it see it mainly as convenience, while opponents seem to think that if there is no strict need identified, no such parameter should be used/introduced.

Companies also note that regardless of this issue, it is necessary to specify the multi-slot monitoring behaviour for cases such as UEs in RRC_IDLE or during initial access, i.e. to aply the mandatory (X,Y) for 480/960 kHz. It therefore seems necessary to write specifications anyway considering the absence of monitoringCapabilityConfig. FL notes further that an approach that is fully within RAN1's domain, i.e. without RRC impact, is generally preferable for maintenance.

Regarding changing the configuration reference of monitoringCapabilityConfig from serving cell to BWP, most companies agree that the configuration is per BWP since Rel-16. Some companies point out that any change in the spec should not change the behaviour of the previous releases. FL points out that 38.213 clause 10 refers in numerous cases to "active DL BWP of a serving cell" instead of just BWP, so FL suggests to adopt this kind of expression.

FL suggestion: Continue discussion based on FL's TP in the second round discussion.

#### Second round discussion

FL proposal:

- Conclude on the following

  - There is no need to introduce monitoringCapabilityConfig = r17monitoringcapability

- Agree to the following TP for 38.213 clause 10

# 10UE procedure for receiving control information *** Unchanged text is omitted *** If a UE is provided monitoringCapabilityConfig for an active DL BWP of a serving cell, the UE obtains an indication to monitor PDCCH on the active DL BWP of the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs - per slot, as in Tables 10.1-2 and 10.1-3, if monitoringCapabilityConfig = r15monitoringcapability, or - per span, as in Tables 10.1-2A and 10.1-3A, if monitoringCapabilityConfig = r16monitoringcapability XsXs,Ys If the UE is not provided monitoringCapabilityConfig for an active DL BWP of a serving cell - for μ ∈ {0,1,2,3} the UE monitors PDCCH on the active DL BWP of the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per slot. - for μ ∈ {5,6} the UE monitors PDCCH on the active DL BWP of the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per group of Xs slots according to combination Xs,Ys, as in Tables 10.1-2B and 10.1-3B. For an active DL BWP of the serving cell where a UE has not been configured with a combination Xs,Ys the UE applies Xs,Ys=(4,1) for μ=5 and Xs,Ys=(8,1) for μ=6. *** Unchanged text is omitted *** # 10.1 UE procedure for determining physical downlink control channel assignment *** Unchanged text is omitted *** The UE allocates PDCCH candidates for monitoring to USS sets for the primary cell having an active DL BWP with SCS configuration μ in a slot if the UE is not provided monitoringCapabilityConfig for the primary cell or if the UE is provided monitoringCapabilityConfig = r15monitoringcapability for the primary cell, or in the first span of each slot if the UE is provided monitoringCapabilityConfig = r16monitoringcapability for the primary cell, or in a group of Xs slots for a corresponding combination Xs,Ys if the UE monitors PDCCH with SCS configuration μ∈{5,6} on the primary cell, according to the following pseudocode. If for the USS sets for scheduling on the primary cell the UE is not provided coresetPoolIndex for first CORESETs, or is provided coresetPoolIndex with value 0 for first CORESETs, and is provided coresetPoolIndex with value 1 for second CORESETs, and if minγ∙MPDCCHmax,slot,μ,MPDCCHtotal,slot,μ>minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ or minγ∙CPDCCHmax,slot,μ,CPDCCHtotal,slot,μ>minCPDCCHmax,slot,μ,CPDCCHtotal,slot,μ, the following pseudocode applies only to USS sets associated with the first CORESETs. A UE does not expect to monitor PDCCH in a USS set without allocated PDCCH candidates for monitoring. In the following pseudocode, if the UE is provided monitoringCapabilityConfig = r16monitoringcapability for the primary cell, MPDCCHmax,slot,μ and CPDCCHmax,slot,μ are replaced by MPDCCHmax,(X,Y),μ and CPDCCHmax,(X,Y),μ respectively, and MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ are replaced by MPDCCHtotal,(X,Y),μ and CPDCCHtotal,(X,Y),μ respectively. In the following pseudocode, if the UE monitors PDCCH with SCS configuration μ∈{5,6} on the primary cell, MPDCCHmax,slot,μ and CPDCCHmax,slot,μ are replaced by MPDCCHmax,(Xs,Ys),μ and CPDCCHmax,(Xs,Ys),μ respectively, and MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ are replaced by MPDCCHtotal,(Xs,Ys),μ and CPDCCHtotal,(Xs,Ys),μ respectively. *** Unchanged text is omitted ***

| Company | Comment |
| --- | --- |
| Lenovo, Motorola Mobility | Support the FL’s proposal |
| Apple | Fine with FL’s proposal |
| Samsung | First, we don’t prefer to fully remove r17monitoringcapability, since the spec impact is much larger than we see from the TP (the intention of the TP is only to remove the holder of r17monitoringcapability added from last meeting’s CR, and there are numerous places need to use r17monitoringcapability in TS 38.213). We would like to remind the consequence of not supporting such a RRC parameter. Then, even following the spirit of not using r17monitoringcapability, we have further comments to the TP: - Changes on adding “BWP” in the first paragraph is not needed. After removing the last bullet, the text is purely legacy Rel-15/16, and no change should be made for legacy texts. Even from the technical point of view, the RRC parameter monitoringCapabilityConfig is specified in the following track: ServingCellConfig -> BWP-Downlink -> PDCCH-Config -> monitoringCapabilityConfig, then the wording “for a BWP” is not needed since it’s implied in the RRC parameter name. - For the second paragraph, same comment that “for a BWP” is not needed. Also, the formulating of this paragraph is not correct, if we don’t support r17monitoringcapability for 480/960. monitoringCapabilityConfig would not be applicable for 480/960 kHz, so we should use the wording “not provided”. The correct formulation in our opinion should be as follow (we used colored text to avoid Word crashing when many track marks exist in a long document): For μ ∈ {0,1,2,3}, if the UE is not provided monitoringCapabilityConfig, the UE monitors PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per slot. For μ ∈ {5,6}, the UE monitors PDCCH per group of Xs slots according to combination Xs,Ys, as in Tables 10.1-2B and 10.1-3B. If a UE is configured with a combination Xs,Ys, the UE assumes Xs,Ys=(4,1) forμ=5 and Xs,Ys=(8,1) for μ=6. |
| Ericsson | We also do not agree to remove monitoringCapabilityConfig = r17monitoringcapability. As pointed out by Samsung, it is already defined in 38.213, and it makes sense to keep it. Otherwise, we need to tell RAN2 that for μ ∈ {5,6}, monitoringCapabilityConfig must be configured as a NULL value. It would not work just to make the parameter absent, since then the UE would always monitor using the default capabilities (4,1) or (8,1). While this is correct for initial access (prior to RRC), it would not be correct if the UE is capable of other values, e.g., (8,4), (4,2). We also do not agree to changing the wording to BWP; the current wording using "serving cell" is fully functional, despite the fact that monitoringCapabilityConfig is configured per BWP. Hence, the only text in the TP that we agree to is to define the default monitoring behavior for μ ∈ {5,6} when the parameter monitoringCapabilityConfig is absent. We suggest the following alternative to Samsung's wording: Alternative TP: For μ ∈ {0,1,2,3}, if the UE is not provided monitoringCapabilityConfig, the UE monitors PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per slot. For μ ∈ {5,6}, if the UE is not provided monitoringCapabilityConfig, the UE monitors PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per group of Xs slots according to combination Xs,Ys=(4,1) or Xs,Ys=(8,1), respectively, as in Tables 10.1-2B and 10.1-3B. |
| Intel | We support the conclustion to remove ‘monitoringCapabilityConfig = r17monitoringcapability’ Regarding the following text in the TP, there is no agreement to configure the active combination (Xs, Ys) for a UE. Therefore, we suggest the following update, which should be applicable independent from valid combination (Xs, Ys) ‘For an active DL BWP of the serving cell if a UE does not have dedicated RRC connection where a UE has not been configured with a combination Xs,Ys the UE applies Xs,Ys=(4,1) for μ=5 and Xs,Ys=(8,1) for μ=6.’, so |
| ZTE, Sanechips | We agree with FL’s proposal and not introduce monitoringCapabilityConfig = r17monitoringcapability. We also share the same view with Samsung that changing “serving cell” to “active DL BWP of a serving cell” is not necessary, a minor correction (high lighted) based on Samsung is as follows: For μ ∈ {0,1,2,3}, if the UE is not provided monitoringCapabilityConfig, the UE monitors PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per slot. For μ ∈ {5,6}, the UE monitors PDCCH per group of Xs slots according to combination Xs,Ys, as in Tables 10.1-2B and 10.1-3B. If a UE is not configured with a combination Xs,Ys, the UE assumes Xs,Ys=(4,1) forμ=5 and Xs,Ys=(8,1) for μ=6.. |
| NTT DOCOMO | We are fine with the FL proposal in general, but for 960 kHz SCS, it should be clarified whether monitoringCapabilityConfig is always configured or can be absent. In our understanding, during initial access and IDLE mode, 960 kHz SCS is not supported and if monitoringCapabilityConfig is always configured for 960 kHz SCS, the default capability definition for the case when monitoringCapabilityConfig is not configured is not required for 960 kHz SCS. |
| LG Electronics | We support the FL proposed conclusion that there is no need to introduce monitoringCapabilityConfig = r17monitoringcapability. Regarding TP, we agree with Intel’s view and support the amended text by Intel. |
| Futurewei | We support keeping ‘monitoringCapabilityConfig = r17monitoringcapability’, and we support if a UE does not have dedicated RRC connection the UE applies Xs,Ys=(4,1) for μ=5 and Xs,Ys=(8,1) for μ=6. |
| vivo | Regarding monitoringCapabilityConfig, we support the conclusion to remove ‘monitoringCapabilityConfig = r17monitoringcapability’ since there is no need to have such RRC signaling. Agree with Intel that the configuraton of (Xs, Ys) is not correct. Actually, we don’t see critical need to include the mandatory value in spec. In UE feature, Xs,Ys=(4,1) for μ=5 and Xs,Ys=8,1 for μ=6 is already listed as a mandotarty component in 480/960KHz FG. When a UE doesn’t report any value of Xs,Ys, the only choice to a smart gNB will assume Xs,Ys=(4,1) for μ=5 and Xs,Ys=8,1 for μ=6 for the UE’s SS configuration. If majority wants, we are also fine to include this in spec but with the following wording: ‘For an active DL BWP of the serving cell where a UE has not been configured with a indicated any combination Xs,Ys the UE applies Xs,Ys=(4,1) for μ=5 and Xs,Ys=(8,1) for μ=6.’ Comment to Samsung: Regarding “the spec impact is much larger than we see from the TP (the intention of the TP is only to remove the holder of r17monitoringcapability added from last meeting’s CR, and there are numerous places need to use r17monitoringcapability in TS 38.213)”, could you please clarify more? Our understanding is that the places to use r17monitoringcapability is equavialent to use operation with SCS configuration μ∈{5,6}. Comment to Ericsson: We understand the concern that the UE would always monitor using the default capabilities (4,1) or (8,1). The above revised text proposal could solve your concern. Regarding the ‘BWP’ wording, we support the revision. @Samsung and Ericsson, if our understanding is correct, we are in the same page that PDCCH monitoring capability is defined per BWP and the difference lies in that whether to update the spec. Assuming a case that two BWP exist in one serving cell, i.e. one BWP with 120KHz SCS provided monitoringCapabilityConfig = r15monitoringcapability and the other BWP with 480KHz SCS where monitoringCapabilityConfig is not provided. In this case where BWP with 480KHz SCS is active, how to understand “If a UE is provided monitoringCapabilityConfig for a serving cell”? Actually monitoringCapabilityConfig is provided for the serving cell, should the UE perform the action after it? We don’t think the spec is clear in this case. In NR Rel-16, we could understand monitoringCapabilityConfig configuration should be the same for all BWPs in a serving cell. However, in NR Rel-17, this is not possible anymore that motivates the change of adding ‘BWP’ in TS 38.213. |
| Qualcomm | As we commented in the first round, we still think keeping r17monitoringcapability is better, because it will make the specification text more compact and clear. Thus, we share similar view with Samsung and Ericsson. Regarding the TP, we are fine with Ericsson’s update. |
| Panasonic | We are fine to the conclustion to remove ‘monitoringCapabilityConfig = r17monitoringcapability’. We also share the same view with Samsung that changing “serving cell” to “active DL BWP of a serving cell” is not necessary. |
| Samsung2 | Response to vivo: That’s true that we can try to use the wording μ∈{5,6} to replace every place needs ‘monitoringCapabilityConfig = r17monitoringcapability’, and we believe this makes the specification complicated and inconsistent. Anyway, this is just a preference on the flavor of writing the specification. |
| Huawei, Hisilicon | - we prefer not to remove r17monitoringcapability as it is already used in 38.213 in multiple places and can facilitate forward compatibility. In the future, there maybe some new PDCCH monitoring capabilities that will be supported for mu=5, 6 and if r17monitoringcapability removed, distincting them from the Rel-17 multi-slot PDCCH monitoring requires a more tedius specification change. - Regarding the the following part of the text in the TP: “For an active DL BWP of the serving cell where a UE has not been configured with a combination Xs,Ys the UE applies Xs,Ys=(4,1) for μ=5 and Xs,Ys=(8,1) for μ=6”. This implies that an RRC signaling is used to configure (Xs,Ys) for monitoring. As we discussed in 2.1.3.3, we disagree with such RRC configuration. Moreover, this is subject of an active discussion in 2.1.3.3 and, procedurally, it should not be concluded in a TP in another section. We think a final TP regarding this part can be agreed only after making a decision on whether or not a RRC signaling is required to configure UE with (Xs,Ys) combination for monitoring or a rule will be specified to pick the propoer (Xs,Ys) for monitoring (after finalizing the discussion in 2.1.3.3) - Given above, we think that, at this point, only an “interim TP” is agreeable that clarifiesthe default monitoring behavior of the UE when its capability is not signaled. We suggest the following TP: Proposal: Support the following TP for 38.213 clarifying the default monitoring behavior of the UE. - Further update on the TP may be required after concluding the discussion in section 2.1.3 “Issue A1-3: Determination of X in case of multiple supported X values for multi-slot monitoring” If a UE is provided monitoringCapabilityConfig for a serving cell, the UE obtains an indication to monitor PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs - per slot, as in Tables 10.1-2 and 10.1-3, if monitoringCapabilityConfig = r15monitoringcapability, or - per span, as in Tables 10.1-2A and 10.1-3A, if monitoringCapabilityConfig = r16monitoringcapability - per group of Xs slots according to combination Xs,Ys, as in Tables 10.1-2B and 10.1-3B, if monitoringCapabilityConfig = r17monitoringcapability. If UE has not indicated any combination Xs,Ys, the UE applies Xs,Ys=(4,1) for μ=5 and Xs,Ys=(8,1) for μ=6 For μ ∈ {0,1,2,3}, if the UE is not provided monitoringCapabilityConfig, the UE monitors PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per slot.. |

#### Summary of second round discussion

8 companies support the FL proposal to remove r17monitoringcapability from 38.213 and not introduce a corresponding entry in RRC, while 5 companies prefer to keep the parameter in 38.213 and to introduce a corresponding parameter in RRC.

The moderator notes that there is neither consensus to remove r17monitoringcapability from 38.213, nor is there consensus to introduce a corresponding parameter to monitoringCapabilityConfig. Still a resolution is highly desirable in this meeting if r17monitoringcapability should be added to monitoringCapabilityConfig as this has an obvious effect on RRC. In the moderator's opinion, removing r17monitoringcapability from 38.213 does not break the specifications but is seen as inconvenient by some companies, while keeping it might simplify inclusion of new behaviour in forthcoming releases, if necessary.

Moderator seeks guidance from the chair how to proceed in this case.

Regarding the change the wording to introduce "active DL BWP", several companies pointed out that this is not required. Moderator suggests to revisit this aspect in RAN1#108-e, and to take the following as a starting point for discussion whether/how to cover the default monitoring behaviour for 480 kHz and 960 kHz:

For μ ∈ {0,1,2,3}, if the UE is not provided monitoringCapabilityConfig, the UE monitors PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per slot.

For μ ∈ {5,6}, the UE monitors PDCCH per group of Xs slots according to combination Xs,Ys, as in Tables 10.1-2B and 10.1-3B. If a UE is not configured with a combination Xs,Ys, the UE assumes Xs,Ys=(4,1) forμ=5 and Xs,Ys=(8,1) for μ=6.

### Issue A1-2: Support of optional/FFS (X,Y) values

#### First round discussion

Samsung (R1-2200193):

Proposal 4: Support Xs,Ys=(2, 1) for 480 kHz SCS.

Intel (R1-2200193):

- X=2 can be optionally supported for SCS 480kHz, which corresponds to combination (X, Y) = (2, 1)

LG (R1-2200565):

Proposal #1: Support (X,Y)=(2,1) as an optional combination for 480 kHz SCS.

| Company | Comment |
| --- | --- |
| Ericsson | We don't see the need for supporting (2,1). |
| MediaTek | We don’t support the proposal. The use case of (X,Y)=(2,1) is not clear to us compared to (X,Y)=(4,1) and (4,2). Also, RAN1 has agreed to not supporting even (X,Y)=(1,1) and we don’t see any motivation to support (X,Y)=(2,1) either. From UE perspective, (X,Y)=(2,1) greatly increase power consumption and UE implementation complexity with no obvious benefit on latency which is governed by the agreed processing timelines. The last point we would like to bring up is the BD/CCE limit for (X,Y)=(2,1) might not be scheduling friendly. For example, based on our evaluation, the BD/CCE limit for (X,Y)=(2,1) should be around half of the BD/CCE limit of (X,Y)=(4,1), i.e., 10 BD/16 CCE per X=2 slots. If that’s the case, then the Type-0 PDCCH monitoring for consecutive slots or even for consecutive slot groups of X=2 slots will be an issue, which is the reason RAN1 changed the Type-0 PDCCH monitoring from per slot to per slot group in FR2-2 in order to avoid Type-0 PDCCH monitoring consuming too many BD/CCE within a slot group from the 20/32 budget. In order the words, RAN1 has identified that two Type-0 monitoring occasions based on BD/CCE=20/32 per 4 slots for 480kHz is not feasible. Therefore we don’t see it’s feasible to support one Type-0 monitoring based on BD/CCE=10/16 per 2 slots for 480kHz as well. |
| Qualcomm | We support the proposal. According to the discussion in the GTW session, it seems that the concern is with supporting (4,3) within Y=1 slot. As Apple suggested during the GTW session, we are okay to support only (7,3) for (X,Y)=(2,1). |
| Intel | We don’t see any additional complexity to support (2, 1) for SCS 480kHz. As commented in the online session, a reference for the decision on (2, 1) for SCS 480kHz should be (4, 1) for 960kHz. The same absolute duration of slot group, i.e. half slot length of SCS 120kHz and the same value Y=1. The existing agreement on (4, 1) for 960kHz can be directly reused to (2, 1) for SCS 480kHz |
| Nokia, NSB | We do not see a reason to re-open the capability discussion. Hence, we don’t support this. |
| vivo | We are open to support (X, Y)=(2, 1) as an optional value. |
| Apple | We are open to this if there is a modification in the FG defined in this case. (X,Y) = (2,1), if supported, should support only FG3-5b with set1 = (7, 3). The option of “ FG3-5b with set2 = (4, 3) and (7, 3) with a modification with maximum two monitoring spans in a slot” was a compromise agreed based on providing additional flexibility to the gNB. With X = 2, this additional flexibility is not needed. |
| InterDigital | We believe that it’s better not to open the discussion again. |
| Xiaomi | Open to support (X, Y)=(2, 1) as an optional value. |
| Samsung | If the only spec impact is to add a column into the tables for multi-slot PDCCH monitoring, we prefer to support (X_s,Y_s )=(2,1). |
| OPPO | Suggest not to support (X,Y)=(2,1) |
| Futurewei | We do not see the strong need to support (X, Y)=(2, 1) |
| NTT DOCOMO | We don’t see the need for supporting (X,Y)=(2,1) slots for 480 kHz SCS. |
| CATT1 | agree not to support (X,Y)=(2,1) |
| ZTE, Sanechips | We do not strongly against (X,Y)=(2,1) but prefer not to support this proposal for the sake of progress since it is still controversial to bring the capability discussion out again. |
| Lenovo, Motorola Mobility | We also don’t see the need to reconsider this, so we prefer to not support (X,Y) = (2,1). |
| Transsion | We do not support (X, Y)=(2, 1) |
| Sharp | We don’t see the need to support (X,Y)=(2,1). |
| LG Electronics | We share the same view with Intel. (X,Y)=(2,1) can be supported as optional for 480 kHz without any additional spec impact compared to supporting X=4 for 960 kHz. |
| Sony | Suggest not to open the discussion again. |
| Huawei, HISilicon | We do not support inclusion of (X,Y)=(2,1) for 480 kHz. Suggest to make the following conclusion: Conclusion: (X,Y)=(2,1) for 480 kHz is not supported in Rel-17. |

MediaTek (R1-2200540):

Proposal 1: For 960kHz multi-slot PDCCH monitoring, only (X,Y)=(8,1) and (8,4) are considered in Rel-17.

| Company | Comment |
| --- | --- |
| MediaTek | Support the proposal |
| Qualcomm | We don’t support the proposal. Since the additional (X,Y) values are already agreed as optional, we don’t see any strong need to revert the agreement. |
| Intel | It is not desired to revert an agreement. |
| Nokia, NSB | We do not see a reason to re-open the capability discussion. Hence, we don’t support this. |
| vivo | No need to revert the agreement |
| Apple | We are fine with the existing agreement as additional values are optional. |
| InterDigital | We believe that it’s better not to open the discussion again. |
| Samsung | It’s not preferable to revert an agreement in the maintenance phase. We are open to discuss the maximum BD/CCE number for (X,Y)=(8,1) and (8,4), if that’s the concern. |
| Futurewei | We prefer to keep the existing agreement. |
| NTT DOCOMO | As it has been already agreed, X=4 slots should be supported as optional capability for 960 kHz SCS to improve scheduling flexibility. |
| CATT1 | Prefer not to change previous agreement |
| ZTE, Sanechips | We do not support this proposal. |
| Lenovo, Motorola Mobility | We don't see a need to revert the earlier agreement to remove the optional values. |
| Transsion | We prefer to keep the previous agreement. |
| Sharp | We don’t support to revert the agreement. |
| LG Electronics | We share the view with other companies that it is not desired to revert what has already been agreed upon. |
| Sony | Suggest not to open this discussion again. |
| Panasonic | We do not support the proposal. It is better to keep the previous agreement. |
| MediaTek2 | We apologize that we didn’t provide a complete comment at the beginning. The reason we suggest reverting the previous agreement is we are not sure whether the BD/CCE limit for X=4 in 960kHz can satisfy Type-0 PDCCH monitoring BD/CCE number specified in the spec (shown in the following table) CCE Aggregation Level; Number of Candidates. 4; 4. 8; 2. 16; 1. As can be seen in the table, one Type-0 SS set in a slot will consume 7BD/(up to)28CCE and we simply don’t see a feasible way to support X=4 with a BD/CCE limit around 10BD/16CCE per 4 slots when Type-0 SS set is involved. We can accept the majority view but at least we suggest to agree on the associated BD/CCE limit corresponding to X=4 for 960kHz and discuss how to resolve the Type-0 PDCCH monitoring BD/CCE budget together. |
| Qualcomm | We still don’t support the proposal, but we agree with MeidiaTek’s comment in general. We share the same view that further discussion is needed on the BD/CCE budget for X=4 in 960 kHz, since it is still a WA. |
| CATT | We think the problem raised by Mediatek2 is caused by the working assumption: BD/CCE budget for (4,2), (4,1) is half that of X=8, and the assigned CCE value in table 10.1-3B. |

#### Summary of first round discussion

Overall, there is no consensus to support (X,Y)=(2,1) for 480 kHz SCS.

A large majority of companies opposes the introduction of (X,Y)=(2,1) as an optional combination for 480 kHz SCS. Some companies support (X,Y)=(2,1) as an optional combination for 480 kHz SCS claiming little extra specification effort and no new absolute time issues compared to what (4, 1) for 960 kHz implies. Apple points out that for (X,Y)=(2,1) the agreement "FG3-5b with set2 = (4, 3) and (7, 3) with a modification with maximum two monitoring spans in a slot" is not required.

A majority of companies does not intend to revert the earlier agreement and support only (X,Y)=(8,1) and (8,4) for 960 kHz. As MediaTek clarified, a major concern for the proposal is that they don’t see a feasible way to support X=4 with a BD/CCE limit around 10BD/16CCE per 4 slots when Type-0 SS set is involved.

FL proposal:

- Conclude that (X,Y)=(2,1) for 480 kHz SCS is not supported

- Continue discussion about BD/CCE issue for X=4 with 960 kHz SCS in the context of BD budget/dropping in RAN1#108e.

### Issue A1-3: Determination of X in case of multiple supported X values for multi-slot monitoring – Potential RRC impact

#### First round discussion

vivo (R1-2200075):

Proposal 1: Select one (Xs, Ys) value from multiple (Xs, Ys) combinations reported by a UE according to the following steps:

1) Determine a set of (Xs, Ys) values according to which the search space configurations meets the limitation, i.e. configured Group (1) SSs are located within Y consecutive slots within a slot group of X slots;

2) Determine one or more (Xs, Ys) with the largest Xs value (i.e. the largest BD/CCE budget) first;

3) Select (Xs, Ys) with the smallest Ys value from the above selected (Xs, Ys) combinations.

Intel (R1-2200193):

Proposal 5:

- For a UE capable of multiple combinations (X, Y), if the configured SS sets are aligned with more than one combination (X, Y), the active combination (X, Y) is determined that is associated with the largest X and smallest Y.

LG (R1-2200565):

Proposal #4: In the multi-slot monitoring, when the monitoring occasions corresponding to multiple (X,Y) combinations are configured, the UE should operate assuming the (X,Y) combination corresponding to the largest X (and Y that can be combined therewith) among the reported combinations that match the monitoring occasions.

| Company | Comment |
| --- | --- |
| Ericsson | We think it would be simplest to just configure the actual (Xs,Ys) value to the UE via RRC. When the parameter is absent, the UE uses a default value, i.e., (4,1) or (8,1) for 480 and 960 kHz, respectively. |
| Qualcomm | We agree with the general idea of introducing a rule for determining a (X,Y) value. First, we don’t think a condition for Y (i.e., either smallest or largest Y) is needed, since the BD/CCE budget only depends on X. Also, rather than the length of a slot group, X, we think the number of BD/CCE is a more important factor, although they end up with the same result. Since we already have a similar rule in Rel-16, we think it can simply be extended. That is, if there are more than one (X,Y) combination that comply with the configuration MOs, the UE monitors PDCCH according to the combination (X,Y) that is associated with the largest maximum number of BD and CCE. |
| Intel | We prefer to adopt similar handling from Rel-16 URLLC and avoid unnecessary RRC impact. The active combination (X, Y) is determined that is associated with the largest X and smallest Y. The largest X should be determined since it allows more BD/CCE. The smallest Y should be determined otherwise UE will always assume (4, 2) or (8, 4) for SCS 480 or 960kHz if largest Y is assumed. |
| Nokia, NSB | Agree with Ericsson’s view. Additional rules just complicate the design without true benefits. |
| vivo | Share the same view as Intel. If there are multiple combinations that comply with SS configuration, largest X means more BD/CCE. If multiple Y values exists for the largest X value, selection of smallest Y means more UE power saving. |
| Apple | Agree with Intel that we could use the same handling as Rel-16 URLLC. |
| InterDigital | We agree with Ericsson and Nokia. Simple RRC configuration should be enough. |
| Xiaomi | Share Ericsson’s view. just configure the actual (Xs,Ys) value to the UE via RRC. When the parameter is absent, the UE uses a default value, i.e., (4,1) or (8,1) for 480 and 960 kHz, respectively |
| Samsung | We agree that the issue of multiple reported (X,Y) combinations needs to be addressed. Regarding the proposals, we are wondering if choosing the largest value of X and smallest value of Y, does it imply (8,1)/(4,1) is always selected as the actual combination since they are the mandatory combinations to be reported for 960/480 kHz respectively. |
| OPPO | Agree with QC and Intel |
| Futurewei | We prefer simple configuration via RRC with default values (4,1) and (8,1) in the absence of RRC config. |
| NTT DOCOMO | We agree that similar handling in Rel-16 URLLC should be supported, i.e., X associated with the largest BD/CCE budget should be applied. We also share the same view with Qualcomm that it is not necessary to determine which Y to apply since BD/CCE budget is defined associated only with value of X and not with Y. |
| CATT1 | We don’t agree to use largest X and smallest Y. In fact there are only very limited combinations the spec can specify what UE should expect with different combinations. This reduce signalling overhead compare with Ericsson’s suggestion |
| ZTE, Sanechips | In principle, we think the UE monitors PDCCH according to the combination (X, Y) that is associated with the largest maximum number of BD/CCE budget among multiple combinations (X, Y). However, UE mandatorily support (4,1) or (8,1) for 480 and 960 kHz respectively, that is to say, if UE can optionally support (4,1) for 960kHz, UE can indicate a capability to monitor PDCCH according to two combinations X,Y = (8, 1), (4, 1) for 960kHz. In order to find the (X,Y) associated with the largest maximum number of BD/CCE budget among multiple combinations (X, Y), (4,1) or (8,1) for 480 and 960 kHz respectively are always chosen even when the parameter is absent. We may need to consider RRC signaling to configure the (X, Y) instead of always using (4,1) or (8,1) for 480 and 960 kHz respectively. |
| Lenovo, Motorola Mobility | We also prefer to following similar handling as Rel-16 URLLC where the active combination is determined according to largest X and smallest Y. We don’t see the need to introduce additional RRC parameter for actual (X,Y) |
| Transsion | We share the same view as Ericsson, just follow the actual (Xs, Ys) value in the RRC configuration, which is a simplest way. |
| Sharp | Simple RRC configuration should be enough. |
| LG Electronics | We agree with Intel and vivo. We prefer to introduce a rule to determine an (X,Y) when UE reports multiple (X,Y) combinations. This is a simple way to extend the rule of Rel-16 and does not require additional RRC impact. We also believe it makes sense that the selected (X,Y) be the mandatorily supporting (X,Y) for each SCS. |
| Sony | We agree with Ericsson to configure the actual (Xs,Ys) to the UE via RRC for further simplicity. |
| Panasonic | We prefer not to introduce additional RRC impact. We share the view that to follow the similar as Rel-16 URLLC, would be sufficient: if there are more than one (X,Y) combination that comply with the configuration MOs, the UE monitors PDCCH according to the combination (X,Y) that is associated with the largest maximum number of BD and CCE. |
| Huawei, HiSilicon | We don’t think it is required to RRC configure actual (Xs, Ys) to UE. We are also not sure why the active/actual value of Y should be determined as it has no impact on BD/CCE budgeting. Aligned with Rel-16 span-based monitoring, we think it is enough to agree on this: Proposal: If among the reported combinations of (Xs, Ys), more than one combinations comply with the SS configurations, UE monitors PDCCHs according to the complying combination (Xs, Ys) that is associated with the largest maximum number of BD/CCE budget (largest value of Xs) |
| CATT | We prefer to reuse rel-16 rules and support the proposal from HW. |

CATT (R1-2200143):

Proposal 2: For PDCCH monitoring capability of 480 kHz/960kHz, if a UE indicates a capability with multiple (X, Y) combinations, the (X,Y) combination determination method of Rel 16 can be reused.

Proposal 4：When the UE indicates a capability with multiple  combinations of (X, Y) for 480 kHz/960 kHz , RAN1 needs further study the following schemes to determine the value of  X for search space configuration, such as:

- Scheme #1: Adding a new IE for configuring the value of X for in the higher layer parameter SearchSpace.

- Scheme #2: Determining the smallest value of X from multiple combinations of (X, Y) reported by UE as the value of X.

- Scheme #3: Defining a default value for the value of X, such as X= 4 slots for both 480 kHz/960 kHz.

FL Note: It would be beneficial if the proponent could clarify how these two proposals work together, as they appear potentially conflicting

| Company | Comment |
| --- | --- |
| Intel | We prefer to adopt similar handling from Rel-16 URLLC and avoid unnecessary RRC impact. |
| Ericsson | Prefer RRC signaling of (Xs,Ys) with a default if parameter not present. |
| vivo; See comment above.. | vivo; See comment above.. |

Samsung (R1-2200193):

Proposal 3: If a UE indicates a capability to monitor PDCCH according to multiple Xs,Ys combinations, a UE monitors PDCCH on the cell based on the following steps:

- Step 1: Choose the Xs,Ys combination, from the Xs,Ys combinations, that is associated with the largest maximum number of MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ;

- Step 2: If there is a tie on the largest maximum number of MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ, choose the Xs,Ys combination with a larger value of Ys.

| Company | Comment |
| --- | --- |
| Intel | We share same view to use a combination (Xs, Ys) with larger MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ; |
| Nokia, NSB | We think the simplest is just to configure the actual values of (Xs,Ys) via RRC signaling. |
| vivo | Support the proposal in principle. Smaller value of Ys should be sufficient. |
| Samsung | We support the proposal as the proposing company. Step 1 is same as Rel-16 span-based PDCCH monitoring, and Step 2 is an extra step to address the potential tie issue for BD/CCE budget. |
| Futurewei | We prefer a RRC configuration-based solution. |
| CATT1 | No need to specify rules |
| Ericsson | Prefer RRC signaling of (Xs,Ys) with a default if parameter not present. |
| Huawei, HiSilicon | As discussed above, we don’t think it is required to RRC configure actual (Xs, Ys) to UE. We are also not sure why the active/actual value of Y should be determined as it has no impact on BD/CCE budgeting. Aligned with Rel-16 span-based monitoring, we think it is enough to agree on this: Proposal: If among the reported combinations of (Xs, Ys), more than one combinations comply with the SS configurations, UE monitors PDCCHs according to the complying combination (Xs, Ys) that is associated with the largest maximum number of BD/CCE budget (largest value of Xs) |
| LG Electronics2 | We share the view with vivo that smaller value of Ys should be sufficient. |

Apple (R1-2200410):

- For each SCS 480 kHz and 960 kHz, the minimum configurable multi-slot PDCCH monitoring periodicity is the smallest value X that a UE supports when reporting its PDCCH monitoring capabilities for the corresponding SCS and are UE specific.

| Company | Comment |
| --- | --- |
| Qualcomm | We are okay with the proposal, and it could be considered as a conclusion. |
| Nokia, NSB | Agree with QC |
| Apple | Support as the proposing company |
| Samsung | This can be considered as a combination of “effective (X,Y) combination in multiple reported (X,Y)” and restriction to the periodicity for PDCCH monitoring. We believe the issue will be clear after resolving those two, and no need for separate discussion. |
| CATT1 | This seems correct but it is not necessary to be included in the spec |
| Lenovo, Motorola Mobility | Fine with the proposal; we note that this will get us a little closer to converging on the PDCCH configuration aspect, so it may be beneficial to discuss in that context. |
| Ericsson | This should be discussed under Topic A2 since it relates to SS configuration |
| Huawei, HiSilicon | We agree with setting the minimum periodicity but it is more important that UE is configured with a periodicity that is multiple of Xs slots. We suggest the following proposal Proposal: - UE expects to be configured with a multi-slot PDCCH monitoring periodicity of n* X_s slots where n is an integer at least equal to 1 (FFS: value range of n) and - For 480 kHz, X_s = 4 - For 960 kHz, X_s= 4 if (X_s,Y_s) = (4,2) or (X_s,Y_s) = (4,1) is supported; otherwise X_s= 8. |
| LG Electronics2 | Agree with Lenovo and Ericsson that it relates to SS configuration. |
| Intel | The proposal is understandable, but it may not need to be specified. Assuming the valid combination (Xs, Ys) is determined, the periodicity of any configured SS set must be a value which can result in the valid combination (Xs, Ys) |

Sharp (R1-2200495):

Proposal 1: Introduce a new RRC parameter for configuring Xs for allocation of PDCCH candidates for UEs with multiple Xs capabilities.

| Company | Comment |
| --- | --- |
| Qualcomm | We don’t support the proposal. As discussed above, if we introduce a rule for determining the value of X, we don’t need an explicit configuration of the X value. |
| Nokia, NSB | We think the simplest is just to configure the actual values of (Xs,Ys) via RRC signaling. |
| vivo | We don’t support the proposal. |
| Samsung | In general, we think a selection rule similar to Rel-16 can be utilized. |
| OPPO | Not needed. |
| Futurewei | We are OK with the (X,Y) configuration via RRC signaling |
| CATT1 | Not needed. |
| Lenovo, Motorola Mobility | As mentioned above, we don’t see a need to introduce additional RRC parameter for this purpose. Even if such a parameter were adopted, another solution would be required before the RRC configuration is received. |
| Sharp | We support the proposal as the proposing company. |
| LG Electronics | We don’t support the proposal and share the same view with Qualcomm, vivo, Samsung, OPPO, CATT and Lenovo. |
| Sony | We think the index of actual (Xs,Ys) can be configured to the UE via RRC for simple signaling, where the index of (Xs,Ys) is included in UE monitoring capability reporting. |
| Panasonic | As mentioned in our reply above, we do not support to introduce new RRC signalling. |
| Ericsson | Agree that RRC signaling can be used, and that a default should be defined if the parameter is absent. |
| Huawei, HiSilicon | As discussed in some of the earlier proposals. We don’t see the need for an RRC configuration. |
| Intel | We don’t support an RRC signaling for value Xs since it is not necessary. Similar handling from URLLC span is applicable. |

#### Summary of first round discussion

A minority of companies expressed a preference to not introduce RRC signalling, though a number of companies support it due to the simplicity. Nevertheless, it was pointed out that a simple rule could be defined without RRC signalling, e.g. the UE monitors PDCCH according to the combination (X,Y) that is associated with the largest maximum number of BD and CCE for the SCS (e.g. proposal by Samsung). Due to the definitions, this may further reduce to monitoring according to (4,1) for 480 kHz and (8,1) for 960 kHz.

There seems to be common understanding that the minimum configurable multi-slot PDCCH monitoring periodicity is the smallest value X that a UE supports when reporting its PDCCH monitoring capabilities for the corresponding SCS.

FL suggestion:

Discuss in a second round whether there are any deficiencies or problems when monitoring according to (X,Y)=(4,1) for 480 kHz and (X,Y)= (8,1) for 960 kHz if multiple (X,Y) combinations are reported.

Discuss minimum configurable periodicity in the context of A2, taking into account that there seems to be common understanding that the minimum configurable multi-slot PDCCH monitoring periodicity is the smallest value X that a UE supports when reporting its PDCCH monitoring capabilities for the corresponding SCS.

#### Second round discussion

Please state if you see any deficiencies or problems with monitoring according to (X,Y)=(4,1) for 480 kHz and (X,Y)=(8,1) for 960 kHz if multiple (X,Y) combinations are reported. Especially point out a motivation or need for introducing an RRC signal to deviate from that behaviour.

| Company | Comment |
| --- | --- |
| Lenovo, Motorola Mobility | In our view, we don’t see any issue with monitoring for above combinations and also don’t see the need to introduce RRC signaling if the proposed rule can be applied. |
| Samsung | We can be ok with this proposal, but prefer to reuse current specification wording from span-based PDCCH monitoring as much as possible. |
| Ericsson | We think a much simpler, cleaner solution is to simply define an RRC parameter for which (Xs,Ys) combination is configured. This will avoid discussion time on what the rule should be. |
| Intel | Is it proposed to use (X,Y)=(4,1)/(8,1) for 480/960 kHz whenever UE report the support of multiple combinations (X, Y)? It seems the consequence is UE will use always used X,Y)=(4,1)/(8,1) for 480/960 kHz - If a UE does not report capability of multiple combinations, the UE must use the default X,Y)=(4,1)/(8,1) for 480/960 kHz - If a UE reports capability of multiple combinations, according to this proposal, UE still uses X,Y)=(4,1)/(8,1) for 480/960 kHz It hence conflicts with an early agreement of support Y>1. For example, if UE report capability of combinations (8,1)(8,4)(4,1)(4,2), gNB may configure Group (1) SS in 4 consecutive slots, which means the active combination should be (8,4) We prefer to reuse the principle from URLLC span, i.e. checking the SS set configurations and determining a valid combination with largest X and smallest Y. |
| ZTE, Sanechips | We have to point out that determination of X for multi-slot PDCCH monitoring is different from that in Rel-16. There is no mandatory capability among the combinations X,Y = (2, 2), (4, 3), and (7, 3) per SCS configuration of μ=0 and μ=1 in Rel-16 and a UE can indicate a capability to monitor PDCCH according to one or more of the combinations. If in Rel-17 the UE monitors PDCCH according to the combination Xs,Ys that is associated with the largest maximum number of BD and CCE for the SCS, Xs,Ys = (4,1) or (8,1) for 480 and 960 kHz respectively are always chosen. However, in our understanding, the value of X should be configurable for more flexible operation in above 52.6 GHz band, that depends on UE capability. We may need to consider RRC signaling to configure the Xs,Ys with smaller value of X for some scenarios, such as in low latency traffic and flexible . |
| NTT DOCOMO | In our understanding, the dropped USS can be different depending on which BD/CCE budget to apply. Thus, it is necessary to define the rule to avoid misunderstanding between UE and gNB on which BD/CCE budget to apply, especially for the case when UE supports multiple (X, Y) and multiple (X, Y) can meet the SS set configuration by gNB. For multi-slot PDCCH monitoring, BD/CCE budget is defined depending on the value of X, and Y is irrelevant. In that sense, it is sufficient to specify the rule to determine X when multiple (X, Y) meet SS set configuration. Furthermore, the applied value of Y is determined based on the SS set configuration, i.e., Y does not have to be 1. For example, if UE supports (X, Y)=(8, 1), (8, 2) and only (8, 2) can meet the SS set configuration, UE should assume applying (8,2) not (8,1). Therefore, we don’t see the need to specify the determination rule of Y while we are fine that UE monitors according to X=4 for 480 kHz SCS and X=8 for 960 kHz SCS if multiple (X, Y) can meet the SS set configuration by gNB. |
| LG Electronics | We support the direction to reuse or extend the rule of Rel-16 without additional RRC impact. However, as pointed out by Intel, it is not meant that UE always monitor PDCCH according to (4,1)/(8,1) for 480/960 kHz when multiple (X,Y) are reported. It should be such that UE monitors PDCCH according to the (X,Y) combination with largest X and smallest Y among the reported (X,Y) combinations that match the monitoring occasions. |
| Huawei, HiSilicon | we share DOCOMO’s concern. If UE “supports (X, Y)=(8, 1), (8, 2) and only (8, 2) can meet the SS set configuration, UE should assume applying (8,2) not (8,1)”. So, to answer our FL question “state if you see any deficiencies or problems with monitoring according to (X,Y)=(4,1) for 480 kHz and (X,Y)=(8,1) for 960 kHz if multiple (X,Y) combinations are reported”, we think, yes, there is a deficiency in such monitoring (as explained above by DoCoMo). We also don’t see any reason to introduce RRC configuration to choose between reported (X,Y) pairs. We think that only the following proposal needs to be agreed: Proposal: If among the reported combinations of (Xs, Ys), more than one combinations comply with the SS configurations, UE monitors PDCCHs according to the complying combination (Xs, Ys) that is associated with the largest maximum number of BD/CCE budget (largest value of Xs) |
| Xiaomi | Share similar views as Ericsson. |
| Nokia, NSB | We agree with Ericsson. The simplest is just to configure the actual values of (Xs,Ys) via RRC signaling. This should be done within capabilities supported by UE |
| Futurewei | We think that RRC configuration could be simpler solution, however if there is no consensus on the RRC we can support a rule based to avoid delaying RAN2 work. |
| vivo | We think a rule to determine (Xs, Ys) if multiple combinations are reported by UE with similar handling for Rel-16 span-based capability. An explicite RRC parameter is not needed. Agree with Intel to reuse span-based mechanism, i.e. If a UE indicates a capability to monitor PDCCH according to multiple Xs,Ys combinations where one or more Xs,Ys combinations comply with SS set configuration, the UE monitors PDCCH on the cell according to the combination (Xs,Ys), from the one or more combinations, that is with the largest Xs first and the smallest Ys second. |
| Qualcomm | We don’t see any need for introducing an RC signal, but the proposal “monitoring according to (X,Y)=(4,1) for 480 kHz and (X,Y)=(8,1) for 960 kHz if multiple (X,Y) combinations are reported” is unclear to me. Is it really mean that (4,1) and (8,1) are always used for 480 kHz and 960 kHz, respectively, as Intel pointed out above? If that is truly the intention, we think it’s deficient, because there is no point for UE to report multiple (X,Y) combinations. As pointed out by many other companies, we think the Rel-16 rule for selecting a (X,Y) combination can largely be reused. |
| Panasonic | As pointed out by e.g. Intel, DOCOMO, Huawei, QC, there is indeed a deficiency if “monitoring according to (X,Y)=(4,1) for 480kHz and (X,Y)=(8,1) for 960kHz if multiple (X,Y) combinations are reported”. To solve the issue, we prefer to reuse the Rel-16 rules for selecting a (X,Y), instead of introducing RRC configuration. |
| MediaTek | As pointed our by many companies that UE might always monitors (4,1)/(8,1) for 480kHz/960kHz respectively as the outcome of this proposal since those are the basic UE capability, which is a concern to us. We prefer resuing Rel-16 span definition rule as much as possible with the modification on how to decide between multiple (X,Y) with the same BD/CCE limits, which is not occurred in Rel-16. |

#### Summary of second round discussion

Several companies still prefer an RRC configuration for the behaviour rather than defining a rule for the UE behaviour, however a majority of companies prefers to not introduce an RRC signal but rather define a rule. It was pointed out that "monitoring according to (X,Y)=(4,1) for 480 kHz and (X,Y)=(8,1) for 960 kHz if multiple (X,Y) combinations are reported" is in conflict of supporting (X,Y) combinations with Y>1. Several companies have stated that a re-use of Rel-16 span-based mechanism is sufficient and desirable (valid combination with largest X and smallest Y).

It seems there is no consensus on a detailed agreement, but based on the current majority view regarding intorduction of an RRC parameter the FL suggests the following conclusion:

Proposed conclusion 2.1.3.4

For (Xs, Ys) in case a UE is capable of multiple (Xs, Ys) combinations

- No RRC parameter is introduced to configure the applicable (Xs, Ys) combination

- Continue discussion on determining (Xs, Ys) in RAN1#108-e

### Issue A1-4: Multi-slot monitoring for IDLE UEs or prior to dedicated configuration

#### First round discussion

Samsung (R1-2200193):

Proposal 1: For IDLE mode:

- Support the mandatory UE capability for multi-slot PDCCH monitoring;

- Don’t support slot based PDCCH monitoring;

- Adopt TP#1 for TS 38.213.

| Company | Comment |
| --- | --- |
| Qualcomm | Further clarification may be needed. Since the proposal is restricted to “IDLE mode”, it has nothing to do with unicast PDCCH, for which we have already concluded not to support per-slot PDCCH monitoring in the GTW session. Also, in RAN1 #107-e, we agreed that, for SS set #0, a UE monitors slot n0 and n0+X0. Thus, multi-slot PDCCH monitoring is already default for SS set #0. Then, is the proposal intended to be applied for Type0A/2 CSS with searhSpaceId ≠ 0? From TP#1 in R1-2200193, it is not very clear. |
| Intel | We understood this proposal is already implied by the conclusion of last GTW session, i.e. per-slot scheduling is not supported. However, it may be more clear if an explicit agreement can be made |
| Apple | Existing agreements seem to indicate this. |
| Xiaomi | Support the Proposal 1 |
| Samsung | We support the proposal as the proposing company. |
| Futurewei | We are fine in principle with the proposal, but we agree with QC that further text clarifications are needed |
| NTT DOCOMO | We think this proposal can be discussed in Issue A1-1. |
| CATT | This is already agreed in GTW except the TP. |
| Lenovo, Motorola Mobility | Support the proposal |
| Sharp | We agree with QC and Futurewei that the text need to be clarified for discussion. |
| LG Electronics | We support TP#1 in R1-2200193. As commented in A1-1, the default (X,Y) combination should be reflected in specification. |
| Panasonic | We understand this is already agreed in GTW except the TP. |
| Ericsson | We agree that a default behavior needs to be defined for 480 and 960 kHz, otherwise, due to the following spec text in 38.213 Section 10, the UE should monitor per-slot if monitoringCabilityConfig is absent (which it is during initial access), and this clearly contradicts agreed behavior: [38.213 Section 10] If the UE is not provided monitoringCapabilityConfig, the UE monitors PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per slot. To rectify this there are at least two Options: Option A: The TP#1 in R1-2200193 proposed by Samsung which defines default behavior for 480/960 kHz when monitoringCapabilityConfig is absent. For this option, we will need to inform RAN2 that monitoringCapabilityConfig must be absent if 480/960 kHz is configured (e.g., by adding a Conditional Presence flag to the parameter). Option B: TP#1a below. For this option we will need to inform RAN2 that the value range for monitoringCapabilityConfig must be extended to include the new value r17monitoringcapability, and that the this value must be configured if 480/960 kHz is configured. >>> Start TP#1a for 38.213 Section 10 >>> *** Unchanged text omitted *** If a UE is provided monitoringCapabilityConfig for a serving cell, the UE obtains an indication to monitor PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs - per slot, as in Tables 10.1-2 and 10.1-3, if monitoringCapabilityConfig = r15monitoringcapability, or - per span, as in Tables 10.1-2A and 10.1-3A, if monitoringCapabilityConfig = r16monitoringcapability - per group of Xs slots according to combination Xs,Ys, as in Tables 10.1-2B and 10.1-3B, if monitoringCapabilityConfig = r17monitoringcapability If the UE is not provided monitoringCapabilityConfig, the UE monitors PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per slot for SCS configuration μ≤3. For SCS configuration μ=5 or μ=6, if the UE is not provided monitoringCapabilityConfig, the UE monitors PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per group of Xs slots according to combination Xs,Ys=(4,1) or Xs,Ys=(8,1), respectively, as in Tables 10.1-2B and 10.1-3B. *** Unchanged text omitted *** >>> End TP #1a >>> We are okay with either approach, but the RRC parameter spreadsheet sent to RAN2 needs to be updated with either option. |
| Huawei, HiSilicon | Bullet one is already agreed for Idle/Inactive/Active cases. Bullet two is already a conclusion in the last GTW for all cases. We agree with this part of TP#1 in bullet three: ########################################### If the UE is not provided monitoringCapabilityConfig for μ=0, 1, 2, or 3, the UE monitors PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per slot. If the UE is not provided monitoringCapabilityConfig for μ=5 or 6, the UE monitors PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per group of Xs slots according to combination Xs,Ys = (4, 1) or (8, 1), for μ=5 or 6, respectively. ############################################# |
| vivo | Further clarification is needed. For idle mode, gNB has no information on UE reported capability on Xs,Ys. It will assume the worst case that UE will only support mandatory capability. Naturally gNB will configure SSs like this. Besides, how to capture idle mode in spec? In the proposed TP, it seems that “the UE is not provided monitoringCapabilityConfig” means idle mode? In our understanding, the case “the UE is not provided monitoringCapabilityConfig” could apply to both idle UEs and connected UEs. Furthermore, for connected UEs, we don’t think UE will monitor the PDCCH according to Xs,Ys = (4, 1) or (8, 1) or μ=5 or 6, respectively if the UE report multiple combinations. |

Ericsson (R1-2200401):

Default r17monitoringcapability of Xs,Ys=(4,1) for 480 kHz SCS before dedicated RRC configuration needs to be captured in the specs. RAN1 discusses which of the following options to adopt:
- Option A: introduce r17monitoringcapability indication in PDCCH-ConfigCommon in the TR 38.331 for 480 kHz SCS only.
- Option B: introduce default r17monitoringcapability indication for 480 kHz SCS in TR 38.213

| Company | Comment |
| --- | --- |
| Ericsson | We prefer Option B since 38.213 Section 10 already defines default behavior when the parameter monitoringCapabilityConfig is absent for the case of per-slot monitoring for SCS <= 120 kHz. Hence, for consistence, it would be better to define default behavior when the parameter is absent also when SCS = 480/960 kHz. |
| Samsung | We are ok with either of the options. |
| Huawei, HiSilicon | We agree that “Default r17monitoringcapability of Xs,Ys=(4,1) for 480 kHz SCS before dedicated RRC configuration needs to be captured in the specs”. We prefer Option B. Additionally, this should be clarified in spec that “Xs,Ys = (4, 1) or (8, 1) is mandatorily supported for SCS configuration μ=5 or μ=6, respectively.” |

#### Summary of first round discussion

Companies are generally fine with adding specification text to capture default behaviour applicable for RRC_IDLE UEs in 38.213 clause 10. FL has included a corresponding section in the TP for 2nd round discussion of issue A1-1. Companies are requested to continue the discussion there, if necessary.

### Issue A1-5: Support of single-slot / multi-slot PDCCH monitoring

#### First round discussion

Intel (R1-2200193):

- X=1, i.e., per-slot PDCCH monitoring capability is not supported for SCS 480/960kHz

Apple (R1-2200410):

- Single-slot PDCCH monitoring for 480 kHz and 960 kHz is not supported.

LG (R1-2200565):

Proposal #1: #2: Do not support X=1, i.e., single slot monitoring, for 480 kHz and 960 kHz SCS.

Conclusion (GTW session on 17-Jan-2022)

- Single-slot PDCCH monitoring for 480 kHz and 960 kHz is not supported.

### Issue A1-6: Other multi-slot monitoring behaviour

#### First round discussion

Samsung (R1-2200193):

Proposal 2: A UE capable of multi-slot monitoring mandatorily supports FG3-1 for each slot outside Ys slots.

| Company | Comment |
| --- | --- |
| Ericsson | We think this is an important issue in order to complete the multi-slot monitoring definition. It seems the agreement from RAN1#107-e wasn't quite complete. We agree to the direction of this proposal from Samsung, and this seems like a straight forward approach. However, we think it needs some refinement since Group (2) can be monitored inside the Ys slots too, and the existing agreement for monitoring within the Ys slots only refers to Group (1). Hence, we would propose the following instead: Revised proposal: A UE capable of multi-slot monitoring mandatorily supports monitoring Group (2) SSs according to FG 3-1 within each of the Xs slots of a slot-group |
| MediaTek | We have some question regarding the proposal. Supporting FG 3-1 in each slot outside the Y slots means only monitors the first 3 symbols? Or it can be any 3 symbols? |
| Qualcomm | We generally agree with the direction, but we think FG3-1 should be extended. As we discussed in RAN1 #107-e, there are several issues with having multiple CSS MOs within a slot group (e.g., exhaustion of BD/CCE budget). FG3-1 also restricts the number of CSS MOs within a slot: - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within a slot As such, the same principle can be extended to multi-slot PDCCH monitoring, such that PDCCH MOs for Group (2) SS sets can be anywhere within a slot group, but, for any of different types of CSSs, there is a single span of three consecutive OFDM symbols within a slot group. |
| Intel | The existing agreement mainly focus on Group (1) SS. So an agreement on Group (S) SS are necessary. Question to Samsung and Ericsson - Is it allowed that all X slots in a slot group are configured with MOs of Group (2) SS? If so, any restriction on UE PDCCH monitoring? - Within the Ys slot, what is the maximum number of spans, 2 or 3? Note: Group (1) SS can have up to 2 spans for Ys=1 and up to 1 span for Ys>1 - Any limitation on the maximum number of spans that a UE can monitor in a slot group? |
| Nokia, NSB | It is true that the previous agreement covers literally only “Group(1) SSs” and “each of the Y slots” - A UE capable of multi-slot monitoring mandatorily supports the following PDCCH monitoring within Y slots - For Y>1: FG3-1 (monitoring Group (1) SSs in the first 3 OFDM symbols of each of the Y slots) Obviously this is not enough for Group(2) SSs. We think that Ericsson’s revised proposal captures all the needed elements. |
| vivo | We agree that monitoring occasions for Group (2) SS should be determined. Just a clarification: FG 3-1 is applied means Group (2) SS could be in any symbol within a slot but within a single span of three symbols. |
| Apple | We agree with the goal of the proposal and agree with QC that some more detail should be provided. |
| InterDigital | We support Ericsson’s update. |
| Xiaomi | Share Ericsson’s view |
| Samsung | We are ok with Ericsson’s clarification. Actually by seeing only Group (2) SS is outside of Ys slots, the capability is by default applicable to Group (2) SS only. |
| OPPO | We agree that the previous agreement was not complete. But we would like also to have answers to the questions raised by Intel. It is true that in the last meeting we agreed that Group (2) can be in any slot of the slot group. But normally a UE does not expect that the PDCCH monitoring in Group (2) happens in every slot of a slot group. |
| Futurewei | We agree that the PDCCH monitoring for Group (2) SS should be determined and we are OK in principle with Ericsson’s update. Not quite clear how the FG 3-1 formulation is extended to slot group. Is the intention to apply FG 3-1 restriction each slot of the slot group, and have each slot be configured with MO? Some clarification note to this proposal may be necessary. |
| NTT DOCOMO | We agree that it should be discussed to complete multi-slot PDCCH monitoring capability definition. Ericsson’s Revised proposal seems fine for us. |
| CATT | We don’t oppose refining rule s for groups(2) but directly use FG3-1 is not clear in the context of multi-slot monitoring. |
| ZTE, Sanechips | We also think defining the mandatory UE capability of multi-slot PDCCH monitoring for Group(2) SSs is necessary. As Qualcomn points out, we need further discuss the limitation on the maximum number of spans of Group(2) SSs that a UE can monitor. |
| Lenovo, Motorola Mobility | We support Ericsson’s update |
| Transsion | We support Ericsson’s revised proposal. |
| Sharp | We agree with Ericsson’s Revised proposal. |
| LG Electronics | We sympathize that monitoring occasions for Group (2) SS should be determined. From this point of view, Ericsson’s revised proposal seems more appropriate than the original one. However, for a better understanding of the proposal, we may need some clarifications for questions from companies above. |
| Panasonic | We support Ericsson’s revised proposal. |
| Ericsson2 | To try and answer some questions above, the description of FG3-1 is copied below for convenience. The cyan highlighted parts refer to Group (2) SSs and the yellow highlights to Group (1) SSs. - vivo's comment is correct; for a given SS from Group (2), the monitoring occasions can be any OFDM symbol(s) in a slot within a single span of three consecutive OFDM symbols within a slot. - Regarding Qualcomm's question, the words "any of" are important in the FG3-1 description (see below). FG 3-1 does NOT say that all Group(2) SSs should be in the same span. This is quite natural, as Type0 can be in the middle of the slotm, and Type 2 could be in the first 3 symbols of a slot. In that sense, we don't agree with Qualcomm's statement that for different types of CSS, there should be a single span within the slot group. - That being said, it is important to note that for each of the Group (s) SS, the monitoring is according to the detected/preferred SSB (once per 20 ms), hence there is no danger of MOs of a single Group (2) SS to occur in multiple slots of a slot group. - If restrictions are introduced to require this (as hinted by Qualcomm and Intel), it would impose quite some changes on the CSS monitoring configuration to force alignment of Group (2) SSs. We don't think that would be wise at this point. 3. DL control channel and procedure; 3-1; Basic DL control channel; 1) One configured CORESET per BWP per cell in addition to CORESET0 - CORESET resource allocation of 6RB bit-map and duration of 1 – 3 OFDM symbols for FR1 - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSSs, CORESET resource allocation of 6RB bit-map and duration 1-3 OFDM symbols for FR2 - For type 1 CSS with dedicated RRC configuration and for type 3 CSS, UE specific SS, CORESET resource allocation of 6RB bit-map and duration 1-2 OFDM symbols for FR2 - REG-bundle sizes of 2/3 RBs or 6 RBs - Interleaved and non-interleaved CCE-to-REG mapping - Precoder-granularity of REG-bundle size - PDCCH DMRS scrambling determination - TCI state(s) for a CORESET configuration 2) CSS and UE-SS configurations for unicast PDCCH transmission per BWP per cell - PDCCH aggregation levels 1, 2, 4, 8, 16 - UP to 3 search space sets in a slot for a scheduled SCell per BWP This search space limit is before applying all dropping rules. - For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within a slot 3) Monitoring DCI formats 0_0, 1_0, 0_1, 1_1 4) Number of PDCCH blind decodes per slot with a given SCS follows Case 1-1 table 5) Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot per scheduled CC for FDD 6) Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot per scheduled CC for TDD. |
| Huawei, Hisilicon | We support Ericsson’s revised proposal. |
| Qualcomm | To answer Ericsson’s comment: It seems that our original comment was misunderstood. We didn’t mean to align MOs of different types of CSS in a single slot, and it is neither required by FG 3-1. If we repeat our original comment, “As such, the same principle can be extended to multi-slot PDCCH monitoring, such that PDCCH MOs for Group (2) SS sets can be anywhere within a slot group, but, for any of different types of CSSs, there is a single span of three consecutive OFDM symbols within a slot group.” Although it could be misleading, what we really meant was that, for each type of CSS, there is a single span of three OFDM symbols within a slot group. It would be clearer in our other proposals, which is discussed below: Proposal 9: For a CSS set with searchSpaceID ≠ 0, a UE does not expect to monitor more than one MO for the CSS set per slot group. Proposal 10: Per slot group, a UE does not expect to process information from more than one DCI format with CRC scrambled by a RNTI from any of SI-RNTI, RA-RNTI, MsgB-RNTI or P-RNTI. |
| Intel 2 | It would be helpful to firstly align the understanding on the following bullet of FG3-1. Based on input from Ericsson and Qualcomm, there is a single span for each Group (2) SS set in a slot, and the different Group (2) SS sets can be configured in different spans in the slot. Not sure if all companies share the same understanding. - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within a slot Then, regarding the configuration of group (2) SS, we share the views from some companies that it is not clear how to extend FG3-1 to a slot group. As we commented earlier, we prefer to clarify this issue from two aspects 1) limitation on SS set configuration; 2) limitation on UE PDCCH monitoring. In general, UE may only monitor a subset of configured MOs of Group (2) SS. limitation on SS set configuration by gNB - Q1: Any limitation on Group (2) SS configuration in same or different slots or multiple slots in a slot group? We prefer to allow full flexibility of the Group (2) SS. - Q2: Any limitation on the maximum number of spans of Group (2) SS or both Group (1) &(2) SS configured in a slot or in a slot group? Our current preference is to limit number of spans per slot to 2 to align with existing agreement on Group (1) SS. We are open for the number of spans configured in a slot group limitation on UE PDCCH monitoring - Q3: Any limitation on the number of spans of Group (2) SS or both Group (1) &(2) S that can be monitored by UE in a slot or in a slot group? In general, it is preferred to limit the maximum number of spans monitored by UE in a slot or slot group. UE must be able to decode >=2 spans since it is already agreed that group (1) SS can have up to 2 spans. On the other hand, for Y=4 for SCS 960kHz, UE can decode >=4 spans in a slot group. We would like to hear more views from other companies. |

Qualcomm (R1-2200290):

Observation 1: For multi-slot PDCCH monitoring, putting Type1 CSS with dedicated RRC configuration within Group (1) may be infeasible and should be reconsidered.

Proposal 8: A rule for determining a UE-specific MO of cell-specific Type1 CSS configuration should be introduced.

Proposal 9: For a CSS set with searchSpaceID ≠ 0, a UE does not expect to monitor more than one MO for the CSS set per slot group.

Proposal 10: Per slot group, a UE does not expect to process information from more than one DCI format with CRC scrambled by a RNTI from any of SI-RNTI, RA-RNTI, MsgB-RNTI or P-RNTI.

| Company | Comment |
| --- | --- |
| MediaTek | We support proposal 9 and 10 from R1-2200290. For Proposal 8, based on our understanding, the issue raised by R1-2200290 is how to align Type-1 CSS configuration among UEs, which should be should resolved by gNB configuration. Therefore, we don’t think Proposal 8 is necessary. |
| Intel | In general, we prefer to clearly define two aspects: 1) limitation on SS set configuration; 2) limitation on UE PDCCH monitoring Regarding Proposal 9, we think it is fine to restrict UE to monitor at most one MO for group (2) SS in a slot group, however, it is still possible for gNB to configure more than one MO for group (2) SS, which is to align with existing SS configuration behavior For Proposal 10, we have a similar proposal. We are open to discuss alternative capability too. |
| Nokia, NSB | Agree with MediTek |
| vivo | We support proposal 9 and 10. |
| Apple | We support proposals 9 and 10. |
| Samsung | We believe more justification and discussion may be needed to revert part of agreement in the previous meeting. We believe some of the proposals can be achieved by implementation based on current framework, and no change to specification is needed. |
| Futurewei | We are OK with proposal 9 and 10. We agree with Intel’s comment and suggest that a note is added to clarify that gNB may configure more than one MO per slot group. Proposal 8 is not necessary in our opinion. |
| CATT | Not sure the justification to have these limit. For example, proposal 9 may lead to the dropping of some CSS for the UE |
| LG Electronics | Same view with Samsung. But, we are open to discuss. |
| Huawei, HiSilicon | We prefer to discuss this issues further. If gNB implementation can handle the issue of cell-specific Type 1 CSS configuration by, eg, aligning the Y slots of UEs, we prefer to avoid reverting the agreement (as suggested in observation 1) or doing some tedious specification work (as suggested by Proposal 8). Likewise, the BD/CCE budget exhaustion that seems to be the motivation behind Proposal 9 and 10 should also be quantified and better studied to see if some restriction on monitoring occasions is required. |

Intel (R1-2200367):

Proposal 2:

- The maximum number of spans in any slot in a slot group remains to be 2 which reuses the current agreement for the Y=1 slot

- Up to 2 or 3 spans for group (2) SS except the SS set with searchSpaceId = 0 can be configured in a slot group

  - UE may decide to monitor only one MO or even not monitor any MO at all

- UE is capable to monitor up to 3 or 4 spans of SS sets in a slot group

- The limitation on number of spans in a slot group should be discussed in UE feature for WI NR_ext_to_71GHz

| Company | Comment |
| --- | --- |
| MediaTek | This proposal is related to the above proposal: Proposal 2: A UE capable of multi-slot monitoring mandatorily supports FG3-1 for each slot outside Ys slots. If we can reach a agreement on Proposal2 in Samsung’s Tdoc, then we don’t see the need to discuss this proposal |
| Intel | We prefer to clarify all issues in the proposal. In general, two aspects: 1) limitation on SS set configuration; 2) limitation on UE PDCCH monitoring should be clarified. It is necessary to limit UE monitoring from complexity point of view. However, it is not desired to restrict gNB configuration for flexibility. |
| Samsung | Agree with MediaTek’s comment that we can discuss together. |
| Huawei, HiSilicon | Agree with MediaTek and Samsung assessment. |

Panasonic (R1-2200654):

Proposal 2: In case that new MO of Group(1) SS is configured, the location of Ys within Xs can be adapted accordingly.

Proposal 3: Location of Ys should first include all CSS MOs (of Group(1) SS) that are monitored by UE, then USS MOs can be further included in Ys from USS with lower to higher indices. Certain USS MO would be dropped if it cannot be included in Ys.

| Company | Comment |
| --- | --- |
| Samsung | We are wondering the specification impact of these two proposals, since we thought they are already supported by current spec. |
| Panasonic | Current 38.213 has the following description in section 10: TS 38.213 v17.0.0, Section 10 … For SCS configuration μ=5 or μ=6, a UE can indicate a capability to monitor PDCCH according to one or more combinations Xs,Ys, where Xs and Ys are numbers of consecutive slots, groups of Xs slots are consecutive and non-overlapping, and the Ys slots are within the Xs slots. The first group of Xs slots starts from the beginning of a subframe. The start of two consecutive groups of Ys slots is separated by Xs slots. If a UE monitors PDCCH on a cell according to combination Xs,Ys, the UE can monitor PDCCH for Type1-PDCCH CSS set provided by dedicated higher layer signalling, Type3-PDCCH CSS sets, and USS sets in any slot of the Ys slots, and the UE can monitor PDCCH for Type0/0A/2-PDCCH CSS set and Type1-PDCCH CSS set provided in SIB1 in any slot of the Xs slots. The UE determines the number of monitored PDCCH candidates and the number of non-overlapped CCEs for combination Xs,Ys based on all search space sets within the Xs slots, as applicable according to the search space set configurations, and maximum corresponding values are provided in Table 10.1-2B and Table 10.1-3B, respectively. …. The yellow highlighted part implies the location of Ys within Xs is always the same across different slot groups. This cannot be true. The Proposal 2 is to allow the location of Ys to change across different slot groups if new MO is configured. This only happens temporally. After the location of Ys is adapted to the new configuration of MOs, it would maintain the same across future slot groups. The Proposal 3 is to define a criterion for the location determination of Ys. Because of the fact that it is not be feasible to restrict all MOs of Group (1) SS within Ys for all the time (e.g.considering the beam switching), we propose to prioritize the CSS MO. This is currently missing in 38.213. |

#### Summary of first round discussion

Regarding Ericsson's revision of Samsung's proposal, it seems there is support by many companies at least for the general direction of the proposal. Further discussion seems necessary to clarify remaining questions and comments e.g. by Intel and Qualcomm. This is preferably treated together with Qualcomm's and Intel's proposals in a second round.

Regarding Panasonic's proposal, there has been little input, so it seems premature to conclude. Discussion can continue in a second round. FL notes that there seems to be no immediate impact to RRC, therefore it would be acceptable if no conclusion is reached in this meeting.

#### Second round discussion

Please continue discussion on the following aspects from the first round. For convenience, some proposals and comments are copied here.

Samsung's proposal: A UE capable of multi-slot monitoring mandatorily supports FG3-1 for each slot outside Ys slots.

Ericsson's revised proposal: A UE capable of multi-slot monitoring mandatorily supports monitoring Group (2) SSs according to FG 3-1 within each of the Xs slots of a slot-group

Qualcomm's proposal 9: For a CSS set with searchSpaceID ≠ 0, a UE does not expect to monitor more than one MO for the CSS set per slot group.

Qualcomm's proposal 10: Per slot group, a UE does not expect to process information from more than one DCI format with CRC scrambled by a RNTI from any of SI-RNTI, RA-RNTI, MsgB-RNTI or P-RNTI.

Intel's proposal:

- The maximum number of spans in any slot in a slot group remains to be 2 which reuses the current agreement for the Y=1 slot

- Up to 2 or 3 spans for group (2) SS except the SS set with searchSpaceId = 0 can be configured in a slot group

  - UE may decide to monitor only one MO or even not monitor any MO at all

- UE is capable to monitor up to 3 or 4 spans of SS sets in a slot group

- The limitation on number of spans in a slot group should be discussed in UE feature for WI NR_ext_to_71GHz

| Company | Comment |
| --- | --- |
| Ericsson2 (from 1st round) | To try and answer some questions above, the description of FG3-1 is copied below for convenience. The cyan highlighted parts refer to Group (2) SSs and the yellow highlights to Group (1) SSs. - vivo's comment is correct; for a given SS from Group (2), the monitoring occasions can be any OFDM symbol(s) in a slot within a single span of three consecutive OFDM symbols within a slot. - Regarding Qualcomm's question, the words "any of" are important in the FG3-1 description (see below). FG 3-1 does NOT say that all Group(2) SSs should be in the same span. This is quite natural, as Type0 can be in the middle of the slotm, and Type 2 could be in the first 3 symbols of a slot. In that sense, we don't agree with Qualcomm's statement that for different types of CSS, there should be a single span within the slot group. - That being said, it is important to note that for each of the Group (s) SS, the monitoring is according to the detected/preferred SSB (once per 20 ms), hence there is no danger of MOs of a single Group (2) SS to occur in multiple slots of a slot group. - If restrictions are introduced to require this (as hinted by Qualcomm and Intel), it would impose quite some changes on the CSS monitoring configuration to force alignment of Group (2) SSs. We don't think that would be wise at this point. 3. DL control channel and procedure; 3-1; Basic DL control channel; 1) One configured CORESET per BWP per cell in addition to CORESET0 - CORESET resource allocation of 6RB bit-map and duration of 1 – 3 OFDM symbols for FR1 - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSSs, CORESET resource allocation of 6RB bit-map and duration 1-3 OFDM symbols for FR2 - For type 1 CSS with dedicated RRC configuration and for type 3 CSS, UE specific SS, CORESET resource allocation of 6RB bit-map and duration 1-2 OFDM symbols for FR2 - REG-bundle sizes of 2/3 RBs or 6 RBs - Interleaved and non-interleaved CCE-to-REG mapping - Precoder-granularity of REG-bundle size - PDCCH DMRS scrambling determination - TCI state(s) for a CORESET configuration 2) CSS and UE-SS configurations for unicast PDCCH transmission per BWP per cell - PDCCH aggregation levels 1, 2, 4, 8, 16 - UP to 3 search space sets in a slot for a scheduled SCell per BWP This search space limit is before applying all dropping rules. - For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within a slot 3) Monitoring DCI formats 0_0, 1_0, 0_1, 1_1 4) Number of PDCCH blind decodes per slot with a given SCS follows Case 1-1 table 5) Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot per scheduled CC for FDD 6) Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot per scheduled CC for TDD. |
| Qualcomm (from 1st round) | To answer Ericsson’s comment: It seems that our original comment was misunderstood. We didn’t mean to align MOs of different types of CSS in a single slot, and it is neither required by FG 3-1. If we repeat our original comment, “As such, the same principle can be extended to multi-slot PDCCH monitoring, such that PDCCH MOs for Group (2) SS sets can be anywhere within a slot group, but, for any of different types of CSSs, there is a single span of three consecutive OFDM symbols within a slot group.” Although it could be misleading, what we really meant was that, for each type of CSS, there is a single span of three OFDM symbols within a slot group. It would be clearer in our other proposals, which is discussed below: Proposal 9: For a CSS set with searchSpaceID ≠ 0, a UE does not expect to monitor more than one MO for the CSS set per slot group. Proposal 10: Per slot group, a UE does not expect to process information from more than one DCI format with CRC scrambled by a RNTI from any of SI-RNTI, RA-RNTI, MsgB-RNTI or P-RNTI. |
| Intel 2 (from 1st round) | It would be helpful to firstly align the understanding on the following bullet of FG3-1. Based on input from Ericsson and Qualcomm, there is a single span for each Group (2) SS set in a slot, and the different Group (2) SS sets can be configured in different spans in the slot. Not sure if all companies share the same understanding. - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within a slot Then, regarding the configuration of group (2) SS, we share the views from some companies that it is not clear how to extend FG3-1 to a slot group. As we commented earlier, we prefer to clarify this issue from two aspects 1) limitation on SS set configuration; 2) limitation on UE PDCCH monitoring. In general, UE may only monitor a subset of configured MOs of Group (2) SS. limitation on SS set configuration by gNB - Q1: Any limitation on Group (2) SS configuration in same or different slots or multiple slots in a slot group? We prefer to allow full flexibility of the Group (2) SS. - Q2: Any limitation on the maximum number of spans of Group (2) SS or both Group (1) &(2) SS configured in a slot or in a slot group? Our current preference is to limit number of spans per slot to 2 to align with existing agreement on Group (1) SS. We are open for the number of spans configured in a slot group limitation on UE PDCCH monitoring - Q3: Any limitation on the number of spans of Group (2) SS or both Group (1) &(2) S that can be monitored by UE in a slot or in a slot group? In general, it is preferred to limit the maximum number of spans monitored by UE in a slot or slot group. UE must be able to decode >=2 spans since it is already agreed that group (1) SS can have up to 2 spans. On the other hand, for Y=4 for SCS 960kHz, UE can decode >=4 spans in a slot group. We would like to hear more views from other companies. |
| Ericsson | Thanks Qualcomm for the clarification. We now understand that you did not imply that all Group (2) SSs should be aligned in the same span. However, regarding your Proposal 9 and 10, we wonder if these are really needed? Since for the Group (2) SSs, the PDCCH occasions that the UE actually monitors are based on the (preferred) detected SSB, we think it will not happen that there are more than one MO per search space within a slot group. Regarding processing more than one DCI format per RNTI per slot group – this would not occur for the same reason. Regarding Intel's concern on lack of clarity on how to extend FG3-1 to a slot group, we thought the alternative proposal mentioned above by the FL would clarify that Ericsson's revised proposal: A UE capable of multi-slot monitoring mandatorily supports monitoring Group (2) SSs according to FG 3-1 within each of the Xs slots of a slot-group If this is not clear enough, perhaps the following which reuses wording directly from the FG3-1 definition would clarify further (only change "a" to "each"). Further clarified proposal: A UE capable of multi-slot monitoring mandatorily supports monitoring Group (2) SSs according to FG 3-1 within each of the Xs slots of a slot-group. This means the following: For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of a each slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within a each slot. Since the monitoring of Group (2) SSs occurs so infrequently, we don't think explicit limitations on the number of spans sper slot group need to be specified for Group (2). |
| Apple | Thank you Ericsson and Qualcomm for the discussion. @ Ericsson: if we think it will not happen, then setting up a rule (similar to Qualcomm’s 1st proposal should not hurt). On the clarified proposal, we could have the wording “each slot of the slot group”. On explicit limitateion on the number of spans per slot group, our preference is that approximately once every twenty milliseconds, there is one slot that is loaded with USSs and CSSs subject to the BD/CCE limit and then the other slots can be optimized for power rather than the CSSs being spread out over multiple slots within the 20 msec duration. |
| ZTE, Sanechips | We are fine with Ericsson's revised proposal and support monitoring Group (2) SSs according to FG 3-1 within each of the Xs slots of a slot-group. The specific descriptions of FG3-1 are as follows: - For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within a slot. According to the agreement in RAN1#107e, a UE capable of multi-slot monitoring mandatorily supports monitoring Group (1) SSs in the first 3 OFDM symbols of each of the Y slots for Y>1. We propose to apply the similar rule for Group(2) SSs, a UE capable of multi-slot monitoring mandatorily supports monitoring Group (2) SSs in a single span of three consecutive OFDM symbols within each slot of slot group. We suggest describing multi-slot monitoring capability as follows: - For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of each of the Y slots for Y>1 - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within each slot of slot group. |
| NTT DOCOMO | We support Ericsson's further clarified proposal. Regarding the limitation on SS configuration/UE PDCCH monitoring, we don’t see the need to limit maximum number of spans of Group (1) and/or Group (2) SS in a slot/slot group either. |
| Nokia, NSB | We also support Ericsson’s revised proposal. |
| Futurewei | We are OK with UE should be capable of multi-slot monitoring mandatorily supports monitoring Group (2) SSs according to FG 3-1 within each of the Xs slots of a slot-group . |
| vivo | We support Ericsson’s clarified proposal |
| Qualcomm | For the monitoring span for Group (2) SS sets within a slot, we are fine with the proposal, e.g., Ericsson’s clarified one. Regarding the span within a slot group, we also think further limitation of the number of spans is needed. As Ericsson pointed out, in practice, it may be rare to happen that multiple spans of Group (2) SS sets are within a slot group, e.g., due to the association between CSS MO and SSB. However, technically, there is nothing preventing such configuration for connected mode UEs (e.g., by using commonSearchSpaceList and commonControlResourceSet in PDCCH-ConfigCommon IE) and, as commented by Apple above, it should not hurt to add clarification to the standard. |
| Panasonic | We share Qualcomm’s view for the need to add clarification to the spec. |
| LG Electronics | Regarding monitoring span in a slot, we are fine with either proposal from Ericsson. Regarding limiting the number of spans in a slot group, it is unclear to us what issues arise either when such limitations are addded or not added. We slightly prefer not to add an explicit limitation to minimize spec impact. |
| Huawei, HiSilicon | We prefer to allow full flexibility of the Group (2) SS, and hence we prefer a) for a given SS from Group (2), the monitoring occasions can be any OFDM symbol(s) in a slot within a single span of three consecutive OFDM symbols, and b) no further restriction on SS set configuration or UE PDCCH monitoring. As discussed in the first round, the necessity of further limitations on the number of spans per slot/slot group is not clear to us. We support the Further clarified proposal provided by Ericsson. |

#### Summary of second round discussion

After the second round, it seems Ericsson's revised proposal for Group (1) SSs is agreeable. Regarding a limitation on the number of spans per slot group of Group (2) SSs, opinions differ whether a limitation by specification is required or not. Therefore FL suggests to continue the discussion in RAN1#108-e

FL Proposal 2.1.6.4:

Revise/clarify earlier agreement as follows:

A UE capable of multi-slot monitoring mandatorily supports monitoring Group (2) SSs according to FG 3-1 within each of the Xs slots of a slot-group, such that:

For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of each slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within each slot.

Continue discussion on introducing a limitation for Group (2) SSs in RAN1#108-e.

## Topic A2: Search Space Configuration/Enhancement

In RAN1#107-e, the following working assumption has been reached:

Working assumption The following values are adopted as minimum value of Pswitch for 120/480/960 kHz - Support only search space set group switching processing capability 1 with the following values μ; Minimum Pswitch value for UE processing capability 1 [symbols]. 3; 40. 5; 160. 6; 320.

### Issue A2-1: Periodicity, offset, duration configuration for multi-slot monitoring

#### First round discussion

Discussion in RAN1#107-e has shown two major flavours for defining the periodicity for multi-slot PDCCH monitoring:

- Interpreting the existing RRC configurable values as being in unit of X slots

- Adding new periodicities in unit of slots to the RRC table that correspond to supported values of X

FL NOTE: This issue may be updated to concoct a 'package' for periodicity, offset, duration etc. Nevertheless any immediate concerns on any of the suggested values and approaches is helpful to converge on a working solution.

Huawei (R1-2200045):

For search space set configuration for multi-slot PDCCH monitoring,

- Add periodicities { sl32, sl64, sl128, sl5120, sl10240, sl20480} to the list of supported values in monitoringSlotPeriodicityAndOffset,

  - Supported set of offsets for each added periodicity X is 0,…,X-1;

- UE expects to be configured with a periodicity that is a multiple of X slots where

  - For 480 kHz, X = 4

  - For 960 kHz, X= 4 if (X,Y) = (4,2) or (X,Y) = (4,1) is supported; otherwise X= 8.

For search space set configuration for multi-slot PDCCH monitoring, introduce a new RRC parameter durationWithinSlotGroup, ranging from 1 to Y, to indicate the consecutive slots that contain monitoring occasions within each slot group of X slots

- Y is the value of multi-slot PDCCH monitoring capability in the reported (X, Y) pair by the UE. If UE is allowed to report more than one Y>1 for a given X, the maximum value of reported Y is used.

For search space set configuration for multi-slot PDCCH monitoring, the unit of “duration” should be changed to X slots where

  - For 480 kHz, X = 4

  - For 960 kHz, X= 4 if (X,Y) = (4,2) or (X,Y) = (4,1) is supported; otherwise X= 8.

| Company | Comment |
| --- | --- |
| Ericsson | We think that there are two fundamental approaches that can be considered for SS configuration when monitoring is per slot group. Regardless which approach is adopted, it is essential to be able to locate a particular slot or slots (Y = 1, 2, or 4) within a slot group. Approach #1 - Periodicity and offset are configured in terms of slots - Configuration restriction is needed that periodicity can only be a multiple of X - Duration is configured in terms of slot groups - Duration of Y within a slot group is configured - E.g., value of Y (1, 2, or 4) Approach #2 - Periodicity, offset, and duration are configured in terms of slot groups - Location of Y slots within a slot group is configured - E.g., a start slot and number of slots Y or a bitmap of which Y contiguous slots are used We think it should be agreed first which approach to take, then sort out the 2nd level details on how to achieve the configuration. For Approach #1, it seems necessary to configure some new periodicity values in order to maintain the same configuration flexibility of the periodicity in terms of slot groups as exists in Rel-15/16 for the periodicity in terms of slots. Most likely this will require defining new -r17 versions of existing parameters with an extended value range. For Approach #2, it seems this can be achieved by re-interpreting existing -r16 parameters in terms of slot groups instead of slots; alternatively, new -r17 parameters can be defined explicitly in terms of slot groups. |
| MediaTek | We prefer not to change the meaning of existing configuration parameters and have some scheduling restriction associated with X, i.e., the first part of the proposal For search space set configuration for multi-slot PDCCH monitoring, - Add periodicities { sl32, sl64, sl128, sl5120, sl10240, sl20480} to the list of supported values in monitoringSlotPeriodicityAndOffset, - Supported set of offsets for each added periodicity X is 0,…,X-1; - UE expects to be configured with a periodicity that is a multiple of X slots where - For 480 kHz, X = 4 - For 960 kHz, X= 4 if (X,Y) = (4,2) or (X,Y) = (4,1) is supported; otherwise X= 8. For the second part, the meaning of the proposed RRC parameter durationWithinSlotGroup is not clear to us. What is the difference between the new RRC parameters and the existing one duration? |
| Qualcomm | We agree with the first part of the proposal, i.e., keeping the same interpretation of monitoringSlotPeriodicityAndOffset for multi-slot PDCCH monitoring. However, we think all the existing parameters, i.e., monitoringSlotPeriodicityAndOffset, duration and monitoringSymbolsWithinSlot should not be changed, without re-interpretation. Regarding the new parameter, instead of durationWithinSlotGroup, we suggest adding a “durationSlotGroup” parameter. This parameter will provide the same level of configuration flexibility as durationWithinSlotGroup, without re-interpretation of duration. - durationSlotGroup: Number of consecutive slot groups of Xs slots (Xs = 2 or 4 for 480 kHz SCS and Xs = 4 or 8 for 960 kHz SCS) that a duration of SearchSpace is repeated. The maximum valid durationSlotGroup is Periodicity-1/Xs Example) - periodicity = 16 slots - offset = 1 slot - duration = 2 slot durationSlotGroup = 2 slot groups (Xs = 4) |
| Intel | We don’t think slot group needs to be emphasized too much in SS set configuration since combination (Xs, Ys) is clearly defined in 38.213, which enforces that SS set for group (1) SS must be in the Ys slots. by this way, we can avoid many tricky and unnecessary discussions. Therefore, our preference is - Periodicity/offset is in slot - Duration is in slot - The only parameter needs change is monitoringSymbolsWithinSlot. It should support the configuration in one slot (for Ys=1 for group (1) SS), multiple slots (for Ys>1 for group (1) SS), or all X slots (group (2) SS can be in any slot in a slot group) Above all, we don’t need complicate the specification on SS set configuration. gNB should do proper configuration otherwise it will violate combination (Xs, Ys) defined in 38.213 |
| Nokia, NSB | Agree with Ericsson that we could first decide the principle. We support slot-based configuration for monitoring periodicity and offset. We also agree with the first part of the proposal, i.e., keeping the same interpretation of monitoringSloteriodicityAndOffset for multi-slot PDCCH monitoring. And adding new values to support all periodicities available for 120 kHz SCS. We also think that the existing parameters, i.e., monitoringSlotPeriodicityAndOffset, duration and monitoringSymbolsWithinSlot should not be changed. We see that “durationSlotGroup” proposed by Qualcomm is an optimization (i.e. not mandatory for providing the basic functionality) We agree with Mediatek that certain restrictions are needed for configuration: - Consider monitoring periodicity (slots) resulting in non-integer monitoring periodicity (in terms of slot groups) as invalid - UE considers configured SS group (1) monitoring occasions outside of the Y slots as invalid |
| vivo | We prefer the least spec impact for SS configuration. All the parameters are the same meaning as before. The only needed change is to add periodicities { sl32, sl64, sl128, sl5120, sl10240, sl20480} to the list. As intel indicates, gNB should do proper configuration to comply with combination (Xs, Ys) defined in 38.213 |
| Apple | Although we prefer Approach #2 (as described by Ericsson), we are open to either option. Note that for Approach #1, - Periodicity/offset is in slots with additional periodicities and restrictions as suggested by MediaTek and Nokia, NSB {similar to legacy} - Duration of the # of slot groups within each period (can be in slots or slot-groups but slot-groups are more efficient) {New?} - Duration of # of slots within each slot group (in slots) {similar to legacy} - Monitoring symbols within slot (unchanged) {similar to legacy} |
| InterDigital | We also believe that the current interpretation of monitoringSlotPeriodicityAndOffset and the existing parameters of monitoringSlotPeriodicityAndOffset, duration and monitoringSymbolsWithinSlot should be kept. The only needed change is to add some more parameters for monitoringSlotPeriodicityAndOffset. |
| Samsung | We prefer to keep the unit for periodicity, duration and offset for SS configuration. We are open to add more values for periodicity with 480 and 960 kHz, and also support introducing restriction on the parameter values (e.g. relationship with X and Y). |
| OPPO | It seems Ericsson’s approach 1 is a simple solution. Thus, we support the principle of approach 1. |
| Futurewei | We support the first part of the proposal, which is slot-based and minimum impact to the existing spec. RRC parameter durationWithinSlotGroup definition needs further clarifications. We do not see necessary “durationSlotGroup” proposed by Qualcomm. |
| NTT DOCOMO | Before we discuss how to extend the SS set configuration, we would like to clarify the problem with the existing values at least as for periodicity and offset, i.e., which SS set configuration pattern cannot be provided with the existing values and whether it is essential pattern or not for 480/960 kHz SCS. |
| CATT1 | We do not want to introduce too many new definition and prefer to reuse the current definition. Therefore approach 2 in E//’s comment is preferred. We also agree with intel’s comment that the only change needed is regarding the monitoringSymbolsWithinSlot. One straightforward approach is extending the bitmap’s bit length to cover multi-slot. |
| ZTE, Sanechips | We prefer to configure periodicity and offset in terms of slots. We suggest adding other appropriate periodicity/duration values { sl32, sl64, sl128, sl5120, sl10240, sl20480} in addition to the existing configurable values. Moreover, additional restrictions are needed to make sure that the values of periodicity and duration for multi-slot PDCCH monitoring in FR2-2 are an integral multiple of slot groups. Also, we support Huawei’s view to set the offsets for each added periodicity X as 0,…,X-1. Moreover, a new parameter needs to be introduced to indicate which and how many slots have monitoring occasions winthin a slot group for SCS 480/960 kHz. If the “offset” in monitoringSlotPeriodicityAndOffset indicates the slot offset between PDCCH monitoring occasion and frame boundary, it seems like we only need to introduce a parameter indicating the value of Y in (X,Y). The following paragraph is extracted from TS 38.213: A UE determines a PDCCH monitoring occasion on an active DL BWP from the PDCCH monitoring periodicity, the PDCCH monitoring offset, and the PDCCH monitoring pattern within a slot. For search space set s, the UE determines that a PDCCH monitoring occasion(s) exists in a slot with number ns,fμ [4, TS 38.211] in a frame with number nf if (nf. Nslot frame, μ+ ns,f μ- os) modks=0nfNslotframe,μ+ns,fμ-op,smod kp,s=0. The UE monitors PDCCH candidates for search space set s for Ts consecutive slots, starting from slot ns,fμ, and does not monitor PDCCH candidates for search space set s for the next ks-Ts consecutive slots. |
| Lenovo, Motorola Mobility | We are fine to first agree on the approach rather than values first, if it helps to make better progress. Based on this, we support approach 2 described by Ericsson where, periodicity, offset and duration are configured in multiples of slot group Furthermore, configuring slots with monitoring occasions within a slot group should be supported as well. This should be simply provided by a bitmap corresponding to the slots within a slot group. |
| Transsion | Although we prefer Approach #2, we are open to ether approach. |
| Sharp | We prefer Approach #2, but we are open to discuss Approach #1. |
| LG Electronics | We support these parameters should be configured in unit of X (no matter either defining a new RRC parameter for Rel-17 or reinterpret the existing RRC parameters for multi-slot monitoring). In addition, two more things can be discussed together. - How to determine X for Group (1) SS which can be a criterion for setting the set of values for these parameters when the UE reports multiple X. For mixed X cases, the set of values for these parameters should be based on smallest X for a UE reporting multiple X values. - How to determine X for Group (2) SS which can be a criterion for setting the set of values for these parameters when the UE reports multiple X. In this case, legacy rule (i.e., parameters are configured in unit of slot) can be used, but alternatively, the smallest (reported) X can also be used like for the case of Group (1). |
| Panasonic | We agree with MediaTek and Nokia to introduce a limitation on the periodicity in the unit of X. We do not think it is a good idea to change or re-interpret the existing parameters such as monitoringSlotPeriodicityAndOffset, duration and monitoringSymbolsWithinSlot. On the second limitation mentioned by Nokia: “UE considers configured SS group (1) monitoring occasions outside of the Y slots as invalid”. We observe that it is not be feasible to restrict all MOs of Group (1) SS within Y for all the time. For example, considering the TDMed-beam transmission of CSS (of Group (1)), UE may need to monitor a different slot for CSS when UE’s serving beam is changed. However, for USS, gNB can simply change the serving beam without changing the MO location since USS MO is unicast to the UE. In this case, it can happen that CSS MO becomes far away from the USS MO of the new serving beam such that Y cannot include both CSS and USS MOs anymore even if it was the case before beam switching. |
| Xiaomi | For the first part “For search space set configuration for multi-slot PDCCH monitoring, - Add periodicities { sl32, sl64, sl128, sl5120, sl10240, sl20480} to the list of supported values in monitoringSlotPeriodicityAndOffset, - Supported set of offsets for each added periodicity X is 0,…,X-1; - UE expects to be configured with a periodicity that is a multiple of X slots where - For 480 kHz, X = 4 - For 960 kHz, X= 4 if (X,Y) = (4,2) or (X,Y) = (4,1) is supported; otherwise X= 8. ”, we are generally fine with it. For the second part, “For search space set configuration for multi-slot PDCCH monitoring, introduce a new RRC parameter durationWithinSlotGroup, ranging from 1 to Y, to indicate the consecutive slots that contain monitoring occasions within each slot group of X slots Y is the value of multi-slot PDCCH monitoring capability in the reported (X, Y) pair by the UE. If UE is allowed to report more than one Y>1 for a given X, the maximum value of reported Y is used.” 1, we think UE does not need to report X/Y as a pair. UE can report X or Y separately. And it is also the truth, when we are discussing multiple-slot PDCCH monitoring, X/ Y values are separately discussed. X is related to the BD/CCE budget, Y is related to how the searchspace are configured. we don’t need to put X and Y in a pair. 2, From our view, if UE can support multiple Y, then to facilitate gNB configuration, UE can just report the maximum Y to gNB, and UE does not need to report multiple Y values. For the third part, “For search space set configuration for multi-slot PDCCH monitoring, the unit of “duration” should be changed to X slots where - For 480 kHz, X = 4 - For 960 kHz, X= 4 if (X,Y) = (4,2) or (X,Y) = (4,1) is supported; otherwise X= 8. ”, we do not see the need.But we can go with majority on this issue. |
| Huawei, HiSilicon | It seems that most companies inclined to keep the periodicity and offset in terms of slot (without any re-interpretation) and only add some additional periodicities. We have a feeling that it might worth it to try to agree to the following (the first part of our proposal only) Proposal: For search space set configuration for multi-slot PDCCH monitoring, - Add periodicities { sl32, sl64, sl128, sl5120, sl10240, sl20480} to the list of supported values in monitoringSlotPeriodicityAndOffset, - Supported set of offsets for each added periodicity X is 0,…,X-1; - UE expects to be configured with a periodicity that is a multiple of X slots where - For 480 kHz, X = 4 - For 960 kHz, X= 4 if (X,Y) = (4,2) or (X,Y) = (4,1) is supported; otherwise X= 8. Once the issue of slot-based periodicity/offset is agreed, we think it is much easier to agree on how to configure SS duration and monitoring symbols. To Mediatek: Regarding to your question “For the second part, the meaning of the proposed RRC parameter durationWithinSlotGroup is not clear to us. What is the difference between the new RRC parameters and the existing one duration?” maybe it is easiest to explain this from the following figure from our t-doc R1-2200045. In the top sub-figure, durationWithinSlotGroup = 1 while duration=12 slots. In the bottom sub-figure, durationWithinSlotGroup = 2 while duration=12 slots. For 480 kHz with (X,Y) = (4,1): … os Ts=12 slots … ks … (a) For 480 kHz with (X,Y) = (4,2): … os Ts=12 slots … ks … (b) 1 slot 1 slot 1 slot 1 slot 1 slot 1 slot 1 slot 1 slot 1 slot 1 slot monitoring occasion |
| CATT2 | As stated previously we prefer to keep the periodicity and offset in terms of slot. For the other parameter, e.g monitoringSlotPeriodicityAndOffset, duration and monitoringSymbolsWithinSlot, we don’t think there is a need to re-define monitoringSlotPeriodicityAndOffset, duration, but to indicate the starting symbol for the search space within the multi-slot group, we need either to redefine monitoringSymbolsWithinSlot or introduce a new RRC parameter. |
| Ericsson2 | Contrary to Huawei's suggestion, we still think a package approach is needed, i.e., agree on how to configure (a) periodicity, offset, duration, and (b) how to indicate which Y slots of a slot group are configured. We do not agree to (a) without (b). This is why we outlined Approach #1 (configuration based on slots) and Approach #2 (configuration based on slot groups), even if different companies have proposed variations on one or the other of these approaches. We would also like to point out that Approach #1 requires considerably larger overhead than Approach #2. Approach #2 requires roughly the same overhead as Rel-15/16, whereas Approach #1 requires roughly 8 times the overhead due to the fact that more offsets are defined than actually needed. Fundamentally the monitoring pattern is based on groups of X slots slots, hence is more efficient to provide an offset in terms of slot groups, and then define an additional (low overhead) parameter that locates which Y slots within the group of X slots is configured for monitoring. Approach #1 - Periodicity and offset are configured in terms of slots - Configuration restriction is needed that periodicity can only be a multiple of X - Duration is configured in terms of slot groups (although some companies propose this to be configured in terms of slots) - Duration of Y within a slot group is configured - E.g., value of Y (1, 2, or 4) Approach #2 - Periodicity, offset, and duration are configured in terms of slot groups - Location of Y slots within a slot group is configured - E.g., a start slot and number of slots Y or a bitmap of which Y contiguous slots are used |

CATT (R1-2200143):

Proposal 3：For 480 kHz SCS and 960 kHz SCS, the search space configuration can be defined as follows.

- Duration: The unit of the duration is still slot.

- Adding a new bitmap monitoringSlotWithinMulti-slot indicating the slot that the search space exists within the multi-slot.

Lenovo (R1-2200671):

Proposal 3: For supporting NR between 52.6 GHz and 71 GHz with high subcarrier spacing values including 480kHz and 960kHz, support configuration of a new bitmap to indicate which slots within the slot group contain monitoring occasions. The size of the bitmap is equal to the number of slots per slot group (i.e. X). The indication of symbols within a slot for PDCCH monitoring applies only to slots indicated by the new bitmap.

| Company | Comment |
| --- | --- |
| Ericsson | Please see our comments in the table below Huawei's proposal (see above). We think that by keeping the duration in units of slot, there will be loss in flexibility for SS configuration. We think duration should be configured in terms of slot groups. |
| Qualcomm | Please see the comments under Huawei’s proposal. |
| Intel | Please refer to our reply to Huawei proposal above. |
| Nokia, NSB | Please see the comments under Huawei’s proposal. |
| vivo | Please refer to our reply to Huawei proposal above. |
| Apple | Please refer to our comments under Huawei’s proposal above. |
| Samsung | We believe the bitmap can be derived by the UE from the SS configuration and there is no need to explicitly support one. |
| Lenovo, Motorola Mobility | Please see our comments under Huawei’s proposal above. |
| Panasonic | Location of Y within X can be determined by search space configuration itself without additional bitmap indication. If Y cannot include both CSS (of Group(1)) and USS, CSS can be prioritized. |
| Xiaomi | Generally fine with the proposal. and from our view, the new bitmap“monitoringSlotWithinMulti-slot” is the same as the “durationWithinSlotGroup” in HW’s proposal. |
| Huawei, HiSilicon | Please see our comments in the discussion under Huawei (R1-2200045) |
| LG Electronics2 | We basically support that all parameters (i.e., periodicity, offset, duration) are in unit of X. However, as commented in GTW, all parameters should be discussed together. We are open to discuss on introducing a new bitmap. |

Nokia (R1-2200184):

Proposal 6: Keep the current Search space configuration principle (i.e. slot based configuration for monitoring periodicity and offset)

Proposal 7: Redesing monitoringSlotPeriodicityAndOffset in TS 38.331 for 480/960 kHz SCSs​

- Consider monitoring periodicity (slots) resulting in non-integer monitoring periodicity (slot groups) as invalid

- Add values to support all periodicitis available for 120 kHs SCS

- Introduce periodicites shown in Table 2 in TS 38.331.

Proposal 8: UE considers configured SS group (1) monitoring occasions outside the Y slots as invalid

Panasonic (R1-2200654):

Proposal 1: For Group(1) SS configuration, the periodicity can only be integer number of Xs.

Lenovo (R1-2200671):

Proposal 2: For supporting NR between 52.6 GHz and 71 GHz with high subcarrier spacing values including 480kHz and 960kHz, support periodicity of PDCCH monitoring in search space configuration only in multiples of slot group duration.

| Company | Comment |
| --- | --- |
| Ericsson | Please see our comments in the table below Huawei's proposal (see above). |
| Intel | Please refer to our reply to Huawei proposal above. |
| Nokia, NSB | We agree with the principle. Please see our detailed comments under Huawei’s proposal. |
| vivo | Please refer to our reply to Huawei proposal above. |
| Apple | Please refer to our comments under Huawei’s proposal above. |
| Samsung | We prefer to keep the unit for periodicity, duration and offset for SS configuration. We are open to add more values for periodicity with 480 and 960 kHz, and also support introducing restriction on the parameter values (e.g. relationship with X and Y). |
| Futurewei | Please refer to our comments to Huawei proposal above. |
| CATT1 | No need to enforce this in the spec, |
| Lenovo, Motorola Mobility | Support both proposals, further see our comments under Huawei’s proposal above. |
| Huawei, HiSilicon | Please see our comments in the discussion under Huawei (R1-2200045) |
| LG Electronics2 | We basically support that all parameters (i.e., periodicity, offset, duration) are in unit of X. However, as commented in GTW, all parameters should be discussed together. |

Samsung (R1-2200193):

Proposal 5: For μ=5 and μ=6 with multi-slot PDCCH monitoring based on combination (Xs, Ys), the PDCCH monitoring periodicity is ks=k⋅Xs, k=1, 2, …, and the PDCCH monitoring duration is Ts≤Ys.

| Company | Comment |
| --- | --- |
| Ericsson | Please see our comments in the table below Huawei's proposal (see above). |
| Intel | Please refer to our reply to Huawei proposal above. |
| Nokia, NSB | We agree with the principle. Please see our detailed comments under Huawei’s proposal. |
| vivo | Please refer to our reply to Huawei proposal above. |
| Apple | Please refer to our comments under Huawei’s proposal above. |
| Samsung | We support the proposal as the proposing company. |
| Futurewei | Please refer to our reply to Huawei proposal above. |
| Panasonic | Support the proposal. |
| Xiaomi | Support in general. |
| LG Electronics2 | We basically support that all parameters (i.e., periodicity, offset, duration) are in unit of X. However, as commented in GTW, all parameters should be discussed together. |

ZTE (R1-2200260):

Proposal 4: The search space configuration for 120kHz SCS in FR2-2 including monitoringSlotPeriodicityAndOffset , duration and monitoringSymbolsWithinSlot can be the same as that for 120kHz in FR2-1.

Proposal 5: For multi-slot PDCCH monitoring in FR2-2, we suggest introducing new parameters (e.g. monitoringSlotPeriodicityAndOffset-r17 and duration-r17 ) and adding other appropriate periodicity/duration values in addition to the existing configurable values. Moreover, additional restrictions are needed to make sure that the values of periodicity and duration for 480/960 kHz are an integral multiple of slot groups.

Proposal 6: A new slot level bitmap (e.g. monitoringSlotsWithinSlotGroup) needs to be introduced to indicate which slots have monitoring occasions winthin a slot group for SCS 480/960 kHz.

| Company | Comment |
| --- | --- |
| Ericsson | Please see our comments in the table below Huawei's proposal (see above). |
| Qualcomm | We support the first proposal (Proposal 4), but not support the third one (Proposal 6). For detailed comments, please see the comments under Huawei’s proposal. We also support adding a new parameter, monitoringSlotPeriodicityAndOffset-r17 – in the new parameter, new periodicity values (sl32, sl64, sl128, sl5120, sl10240, and sl20480) are added and invalid values from the existing ones (i.e., not multiples of Xs) are removed. |
| Intel | Please refer to our reply to Huawei proposal above. |
| Nokia, NSB | Agree with Qualcomm’s comment. Please see our detailed comments under Huawei’s proposal. |
| vivo | Please refer to our reply to Huawei proposal above. |
| Apple | Please refer to our comments under Huawei’s proposal above. |
| Samsung | We prefer to keep the unit for periodicity, duration and offset for SS configuration. We are open to add more values for periodicity with 480 and 960 kHz, and also support introducing restriction on the parameter values (e.g. relationship with X and Y). We believe the bitmap can be derived by the UE from the SS configuration and there is no need to explicitly support one. |
| Futurewei | Please refer to our reply to Huawei proposal above. |
| Lenovo, Motorola Mobility | Please see our comments under Huawei’s proposal above. |
| Panasonic | Please see our reply to Huawei’s proposal above. We think the Proposal 6 (bitmap indication) is not needed. |
| LG Electronics2 | We basically support that all parameters (i.e., periodicity, offset, duration) are in unit of X. However, as commented in GTW, all parameters should be discussed together. We are open to discuss on introducing a new bitmap. |

Qualcomm (R1-2200260):

Proposal 2: After a UE reports its supported (X,Y) values for multi-slot PDCCH monitoring for 480/960 kHz SCSs, the UE always expect that the search space set configurations comply with at least one of the reported (X,Y) values.

- The interpretation of the fields in searchSpaceSet IE, e.g., monitoringSlotPeriodicityAndOffset, monitoringSymbolWithinSlot, and duration, is not changed for the multi-slot PDCCH monitoring,

- A new field of periodicity and offset may be added, e.g., monitoringSlotPeriodicityAndOffset-r17.

  - New periodicities supported by 480/960 kHz: sl32, sl64, sl128, sl5120, sl10240, and sl20480.

Proposal 3: For GC DCI formats, the following periodicities are only applicable:

|  | 120 kHz (same as FR2) | 480 kHz | 960 kHz |
| --- | --- | --- | --- |
| DCI format 2_0 | sl1, sl2, sl4, sl5, sl8, sl10, sl16, sl20 | sl4, sl8, sl16, sl20, sl32, sl40, sl64, sl80 | sl8, sl16, sl32, sl40, sl64, sl80, sl128, sl160 |
| DCI format 2_1 | sl1, sl2, sl4 | sl4, sl8, sl16 | sl8, sl16, sl32 |
| DCI format 2_4 | sl1, sl2, sl4, sl5, sl8, sl10 | sl4, sl8, sl16, sl20, sl32, sl40 | sl8, sl16, sl32, sl40, sl64, sl80 |
| * Highlighted: New periodicity values to be introduced for 480/960 kHz SCSs |  |  |  |

| Company | Comment |
| --- | --- |
| Ericsson | Please see our comments in the table below Huawei's proposal (see above). |
| MediaTek | We support Proposal2 |
| Intel | Please refer to our reply to Huawei proposal above. |
| Nokia, NSB | We agree with the principle. Please see our detailed comments under Huawei’s proposal. |
| vivo | We support the proposal |
| Apple | Please refer to our comments under Huawei’s proposal above. |
| Samsung | We prefer to keep the unit for periodicity, duration and offset for SS configuration. We are open to add more values for periodicity with 480 and 960 kHz, and also support introducing restriction on the parameter values (e.g. relationship with X and Y), but we don’t think an explicit table is needed. |
| Futurewei | Please refer to our reply to Huawei proposal above. |
| Lenovo, Motorola Mobility | Fine with proposal, further see our comments under Huawei’s proposal above. |
| LG Electronics2 | We basically support that all parameters (i.e., periodicity, offset, duration) are in unit of X. However, as commented in GTW, all parameters should be discussed together. We also questionable that an explicit table is needed. |

Intel (R1-2200367):

Proposal 8:

- The existing parameters for SS set configuration are reinterpreted as below for both group (1) SS set and group (2) SS set.

  - monitoringSlotPeriodicityAndOffset: the periodicity and slot offset in a period that is configured with PDCCH MOs for a SS set

  - duration: multiple of X slots starting from the slot offset configured by monitoringSlotPeriodicityAndOffset. The configured MOs in a slot group repeat in all slot groups in duration.

  - monitoringSymbolsWithinSlot: A bitmap to indicate the start symbols of PDCCH MOs in one or multiple slots. The index of the first slot within a slot group can be modo,X. The bitmap length can be at least configured from {14, 28, 56, 112}

  - The configured MOs in a slot group by monitoringSymbolsWithinSlot repeat in all slot groups within the duration slots.

- Agree on TP 5 for SS set configuration

| Company | Comment |
| --- | --- |
| Ericsson | Please see our comments in the table below Huawei's proposal (see above). |
| Intel | Please refer to our reply to Huawei proposal above. |
| Nokia, NSB | Please see the comments under Huawei’s proposal. |
| vivo | Please refer to our reply to Huawei proposal above. |
| Apple | Please refer to our comments under Huawei’s proposal above. |
| Samsung | We prefer to keep the unit for periodicity, duration and offset for SS configuration. We are open to add more values for periodicity with 480 and 960 kHz, and also support introducing restriction on the parameter values (e.g. relationship with X and Y). We believe the bitmap can be derived by the UE from the SS configuration and there is no need to explicitly support one. |
| Futurewei | Please refer to our reply to Huawei proposal above. |
| Lenovo, Motorola Mobility | Please see our comments under Huawei’s proposal above. |
| LG Electronics2 | We basically support that all parameters (i.e., periodicity, offset, duration) are in unit of X. However, as commented in GTW, all parameters should be discussed together. TP can be discussed after high level agreements were drawn. |

Ericsson (R1-2200401):

A new RRC parameter monitoringSlotsWithinSlotGroup-r17 needs to be introduced to support RAN1 agreement that, the Ys consecutive slots can be located anywhere within the slot group of Xs slots.

RAN1 discusses how to remedy the existing search space configuration shortfalls for 480 and 960 kHz SCS serving cells based on the following two options:
- Option A: reinterpret existing RRC fields monitoringSlotPeriodicityAndOffset and duration as in unit of Xs slots;
- Option B: explicitly introduce new RRC field extensions based on slot groups.

| Company | Comment |
| --- | --- |
| Ericsson | Please see our comments in the table below Huawei's proposal (see above). |
| Intel | Please refer to our reply to Huawei proposal above. |
| Nokia, NSB | Please see the comments under Huawei’s proposal. |
| vivo | Please refer to our reply to Huawei proposal above. |
| Apple | Please refer to our comments under Huawei’s proposal above. |
| Samsung | We don’t think an explicit RRC parameter is needed for this purpose, since the UE can derive the start of PDCCH monitoring occasion(s) and PDCCH monitoring duration (up to Y slots) from the SS configuration. |
| Futurewei | Please refer to our reply to Huawei proposal above. |
| Lenovo, Motorola Mobility | Please see our comments under Huawei’s proposal above. |
| Panasonic | We share the view of Samsung that explicit RRC parameter is not needed for indicating the location of Y. |
| LG Electronics2 | We basically support that all parameters (i.e., periodicity, offset, duration) are in unit of X. However, as commented in GTW, all parameters should be discussed together. Regarding new RRC parameter, we have the same view with Samsung. |

Apple (R1-2200410):

Proposal 4: Search Space configuration parameters (periodicity, offset and duration) should be defined for multi-slot PDCCH monitoring. The unit of “periodicity”, “offset” and “duration” should be changed to X slots, with default value X=4 for 480 kHz and X=8 for 960 kHz. An additional parameter should be defined for offset within each slot group.

- The configurable values for multi-slot PDCCH monitoring operation should be same as the reported X value(s). The  UE is not expected to handle a scenario in which they are different, and a UE might report its monitoring capability for more than one (X,Y) combination.

Transsion (R1-2200558):

Proposal 3: For multi-slot PDCCH monitoring, the search space configuration should be in units of  slot group.

| Company | Comment |
| --- | --- |
| Ericsson | Please see our comments in the table below Huawei's proposal (see above). |
| Intel | Please refer to our reply to Huawei proposal above. |
| Nokia, NSB | Please see the comments under Huawei’s proposal. |
| vivo | Please refer to our reply to Huawei proposal above. |
| Apple | Please refer to our comments under Huawei’s proposal above. |
| Samsung | We prefer to keep the unit for periodicity, duration and offset for SS configuration. |
| Futurewei | Please refer to our reply to Huawei proposal above. |
| Lenovo, Motorola Mobility | Please see our comments under Huawei’s proposal above. |
| LG Electronics2 | We basically support that all parameters (i.e., periodicity, offset, duration) are in unit of X. However, as commented in GTW, all parameters should be discussed together. |

#### Summary of first round discussion

Most companies express a preference to keep the unit for periodicity and offset as slots (as in earlier Releases).

FL proposal:

For search space set configuration of multi-slot PDCCH monitoring:

- The unit of existing parameters monitoringSlotPeriodicityAndOffset, duration and monitoringSymbolsWithinSlot are slots (i.e. unchanged)

- Add periodicities {32, 64, 128, 5120, 10240, 20480} slots to the list of supported values in monitoringSlotPeriodicityAndOffset

  - For each supported periodicity value (Xp), the supported set of offsets is 0,…, Xp-1

    - NOTE: There may be no need to introduce the term "Xp" in the specifications

- A UE is not expected to be configured with a periodicity that is not an integer multiple of its supported value(s) Xs for a corresponding SCS

  - Further discuss how to determine monitoring symbols within slot, e.g. by

    - extending the existing bitmap to cover the number of slots in the multi-slot group

    - new parameter durationWithinSlotGroup

    - new parameter durationSlotGroup

    - new bitmap indicating the slots within a slot group, where monitoringSymbolsWithinSlot applies

#### Second round discussion

Please further comment/discuss the following proposal (using the state after 21-Jan-2022 GTW discussion).

For search space set configuration of multi-slot PDCCH monitoring:

- The unit of existing parameters monitoringSlotPeriodicityAndOffset, duration and monitoringSymbolsWithinSlot are slots (i.e. unchanged)

  - Add new values to duration, e.g. to go up to the maximum periodicity

    - FFS: which values

- Add periodicities {32, 64, 128, 5120, 10240, 20480} slots to the list of supported values in monitoringSlotPeriodicityAndOffset

  - For each supported periodicity value (Xp), the supported set of offsets is 0,…, Xp-1

    - NOTE: There may be no need to introduce the term "Xp" in the specifications

- A UE is not expected to be configured with a periodicity that is not an integer multiple of its supported value(s) Xs for a corresponding SCS for group 1 SSs

- Support a mechanism to determine monitoring slots within a slot group and monitoring symbols within those slot, e.g. by

  - extending the existing bitmap to cover the number of slots in the multi-slot group

  - new parameter durationWithinSlotGroup

  - new parameter durationSlotGroup

  - new bitmap indicating the slots within a slot group, where monitoringSymbolsWithinSlot applies

  - reuse the legacy bitmap

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | As discussed in GTW session, we think it is easier to try to disentangle “periodicity/offset”, “duration”, and “the monitoring symbols within slot/duration” and try to resolve the SS configuration problem hierarchically. Also, we think that one source of problem in progressing the discussion is the ambiguity in definition of “duration”. The current definition of “duration” in 38.331 is as follows: duration Number of consecutive slots that a SearchSpace lasts in every occasion, i.e., upon every period as given in the periodicityAndOffset.. Above definition should be modified to accommodate Rel-17 multi-slot PDCCH monitoring mechanism. Certainly, “Number of consecutive slots that a SearchSpace lasts in every occasion” is not applicable for group 1 SSs. Once the definition of “duration” is agreed, it is much easier to “determine monitoring slots within a slot group and monitoring symbols within those slot”. Based on the above discussion, we suggest the following proposal with the minimal changes to the proposal that captured in interim chairman report and brought up by our moderator above. Proposal: - For monitoringSlotPeriodicityAndOffset, - The unit is slot - Add periodicities {32, 64, 128, 5120, 10240, 20480} slots to the list of supported values in monitoringSlotPeriodicityAndOffset - For each supported periodicity value (Xp), the supported set of offsets is 0,…, Xp-1 - NOTE: There may be no need to introduce the term "Xp" in the specifications - A UE is not expected to be configured with a periodicity that is not an integer multiple of its supported value(s) Xs for a corresponding SCS for group 1 SSs - For duration - The definition is the number of consecutive groups of Xs slots that a SearchSpace lasts in every occasion i.e., upon every period as given in the monitoringSlotPeriodicityAndOffset. - Support a mechanism to determine monitoring slots within a slot group of Xs slots and monitoring symbols within those slot, e.g. by - extending the existing monitoringSymbolsWithinSlot bitmap to cover all the symbols within the slot group of Xs slots - Introducing a new parameter durationWithinSlotGroup along with using the legacy monitoringSymbolsWithinSlot. - Introducing a new parameter durationSlotGroup along with using the legacy monitoringSymbolsWithinSlot. - new bitmap indicating the slots within a slot group, where monitoringSymbolsWithinSlot applies |
| OPPO | We understand that the duration is the unit of slot. But for multi-slot PDCCH monitoring, we still need to decide whether the duration parameter is to be used or a new parameter duration_slotgroup will be used instead. We think that if a new parameter is to be used then there is no need to add more values for duration. Therefore we suggest some modifciation based on FL proposal For search space set configuration of multi-slot PDCCH monitoring: - The unit of existing parameters monitoringSlotPeriodicityAndOffset, duration and monitoringSymbolsWithinSlot are slots (i.e. unchanged) - Add new values to duration, e.g. to go up to the maximum periodicity - FFS: which values - Add periodicities {32, 64, 128, 5120, 10240, 20480} slots to the list of supported values in monitoringSlotPeriodicityAndOffset - For each supported periodicity value (Xp), the supported set of offsets is 0,…, Xp-1 - NOTE: There may be no need to introduce the term "Xp" in the specifications - A UE is not expected to be configured with a periodicity that is not an integer multiple of its supported value(s) Xs for a corresponding SCS for group 1 SSs - Support a mechanism to determine monitoring slots within a slot group and monitoring symbols within those slot, e.g. by - extending the existing bitmap to cover the number of slots in the multi-slot group - new parameter durationWithinSlotGroup - new parameter durationSlotGroup - new bitmap indicating the slots within a slot group, where monitoringSymbolsWithinSlot applies - reuse the legacy bitmap - Add new values to duration, e.g. to go up to the maximum periodicity |
| Lenovo, Motorola Mobility | In principle, we are fine with FL’s proposal. However, the updated proposal suggested by Huawei provides clarity in terms of units for each element and as a result ease the discussion on values Furthermore, as discussed during GTW, we should aim to agree on the exact mechanism to determine monitoring slots within a slot group and monitoring symbols within a slot. We think that the most straightforward extension is to simply add a new parameter (bitmap) to indicate monitoring slots within a slot group. Then for further determining symbols with those slots, the existing bitmap for symbols can be used. |
| Apple | From the discussion, the paraemters monitoringSlotPeriodicityAndOffset and monitoringSymbolsWithinSlot seem to have some level of agreement. However, the duration parameter can be “within the slot group” or “of the slot group” and has not converged (in the FFS). As such, duration should be removed from the first bullet point and either agreed to or moved into the FFS. Our preference would be to define a slotGroupDuration parameter and separately a duration within the slot group parameter. The slotGroupDuration can be in units of slots or slot groups although in units of slot groups would be more efficient. |
| CATT | In principle we are fine with FL’s proposal. Regarding duration, we don’t see the need to change the definition , or the unit of duration,. As long as configured values of duration are multiple of Xs slots, and the starting symbol is indicated by other parameters, there will be no problems. The definition that ‘Number of consecutive slots that a SearchSpace lasts in every occasion’ can still apply to multi-slot monitoring case because it does not mean that SS will exist in each of the consecutive slot. In Rel-16 because monitoringSymbolsWithinSlot is defined per symbol , therefore this is the case. But since the mechanism of indication of starting symbol within slot group will be defined now anyway, there is no need to redefine duration. |
| Samsung | From the GTW session on Thursday, we believe the following two aspects need to be completed for the SS configuration: - The unit of periodicity/offset/duration and the corresponding value range - Additional mechanism to determine monitoring slots within a slot group and monitoring symbols within those slot Although we preferred to keep the unit as Rel-16, after the discussion, we feel like directly apply the unit of periodicity/offset/duration as slot group seems simpler and can avoid lots of further discussion. Here is our understanding on the unit of periodicity/offset/duration and the corresponding value range: - Alt 1: The unit of existing parameters monitoringSlotPeriodicityAndOffset and duration are slots (i.e. unchanged) - Add extra values for monitoringSlotPeriodicityAndOffset and duration - Alt 2: The unit of existing parameters monitoringSlotPeriodicityAndOffset and duration are slot group of Xs slots - The value range for monitoringSlotPeriodicityAndOffset and duration maintain the same For the mechanism to determine monitoring slots within a slot group and monitoring symbols within those slot, we believe the following options are valid (based on our review of companies’ tdoc and comments, but of course there could be more to be added): - Option 1: Indicating monitoring slots within a slot group and monitoring symbols within those slot separately: - Option 1a: Support a new bitmap (e.g. with length Xs) to indicate monitoring slots within a slot group, and reuse the legacy bitmap monitoringSymbolsWithinSlot to indicate monitoring symbols within a slot - For this option, the monitored symbols within a slot are the same for all slots monitored in a slot group, and all slot groups in duration are monitored - Option 1b: Support a starting slot index and duration to indicate monitoring slots within a slot group, and reuse the legacy bitmap monitoringSymbolsWithinSlot to indicate monitoring symbols within a slot - For this option, the monitored symbols within a slot are the same for all slots monitored in a slot group, and monitored slots in a slot group are consecutive, and all slot groups in duration are monitored - Option 1c: Support a new bitmap (e.g. with length “duration” in term of slots) to indicate monitoring slots within duration, and reuse the legacy bitmap monitoringSymbolsWithinSlot to indicate monitoring symbols within a slot - For this option, the monitored symbols within a slot are the same for all slots monitored in a slot group, and the slot groups in duration to be monitored are indicated - Option 2: Indicating monitoring slots within a slot group and monitoring symbols within those slot jointly: - Extend the legacy bitmap monitoringSymbolsWithinSlot to indicate monitoring symbols within a slot group (e.g. with length 14*Xs) - For this option, the monitored symbols within a slot can be different across slots monitored in a slot group |
| Ericsson | We have the following comments, after which we summarize our view on which of two alternatives should be supported. Comment #1: Two general options have been discussed: - Alt1: Configuration parameters are in terms of slots - Value range of parameters is extended, e.g., to allow the same periodicity and duration (in absolute time) as for 120 kHz - The configured values are restricted to integer multiples of the slot group size X - Alt2 (our preference): Configuration parameters are in terms of slot groups As we have commented previously, we still prefer Alt-2, since it is simpler, cleaner, and more accurately captures the needed functionality of multi-slot monitoring without defining values in RRC that are ultimately never signaled due to configuration restrictions. Comment #2: Our firm position is that a consistent approach should be supported, i.e., either all parameters are defined in terms of slots groups (as in Alt-2, our preference) or if companies insist on Alt-1, then there should not be a mix of some parameters in terms of slots and some parameters in terms of slot groups. For example, some companies are proposing that monitoringSlotPeriodicityAndOffset is in terms of slots, and duration is in terms of slot groups. We don't agree to this mix. Comment #3: For either Alt-1 or Alt-2, it must allow at least the same search space periodicities and durations (in absolute time) as supported for legacy with 120 kHz SCS. Note that the existing value range for the legacy parameter monitoringSlotPeriodicityAndOffset is {1,2,4,5,8,10,16,20,40,80,160,320,640,1280,2560} and the existing value range for the legacy parameter duration is {2 .. 2559}. For this reason, we disagree with the suggestion (e.g., OPPO) to maintain the legacy value range for the duration parameter. For Alt-1, since the duration is defined in terms of slots, this reduces by a factor of 4 or 8 the duration (in absolute time) that can be configured for 480/960 kHz SCS. We object to this loss in configuration flexibility compared to 120 kHz SCS. Comment #4: Based on Comments #2 and #3, only the following value ranges are acceptable to us: - Alt-1 (not our preference): - monitoringPeriodicityAndOffset-r17: {1,2,4,5,8,10,16,20,32,40,64,80,128,160,320,640,1280,2560,5120, 10240,20480} - For periodicity value Xp, the value range for the offset O is {0 .. Xp-1} - Periodicity Xp is restricted to be an integer multiple of Xs - Note: The values in red are new compared to Rel-15/16 - duration-r17: all integer values in the range {2 .. 20479} - Duration is restricted to be an integer multiple of Xs - Note: The maximum value in red is new compared to Rel-15/16 - Alt-2 (our preference): - monitoringPeriodicityAndOffset-r17: {1,2,4,5,8,10,16,20,40,80,160,320,640,1280,2560} - For periodicity value Xp, the value range for the offset O is {0 .. Xp-1} - duration-r17: all integer values in the range {2 .. 2559} At least for Alt-1, new -r17 versions of the parameters will need to be introduced. For Alt-2, this is not strictly needed; however, since some companies have concerns on re-interpreting existing Rel-15/16 parameters in terms of slot groups, a viable alternative is to add new -r17 versions of the parameters defined in terms of slot groups. Comment #5: Regarding how to configure which slots and symbols within a slot group are monitored, our firm position is as follows: - The legacy parameter monitoringSymbolsWithinSlot (length 14 bitmap) is unchanged - This means that for whatever slot(s) are monitored within a slot group, the same OFDM symbols in each of those slots is monitored - A new parameter monitoringSlotsWithinSlotGroup is supported for configuring which slot(s) within a slot group are monitored - The parameter is a length-8 bitmap which contains a number of '1's indicating which consecutive slot(s) within a slot group are monitored - If 480 kHz SCS is configured, only the first 4 bits of the bitmap are relevant - If 960 kHz SCS is configured, all 8 bits of the bitmap are relevant - While we are not supportive, if a new parameter durationWithinSlotGroup is agreed (only applicable to Alt-1), the following conditions must hold: - The parameter indicates the number of consecutive slots within a slot group that is/are monitored, e.g., with value range {1,2,4} - The Offset O must not be restricted to be an integer multiple of X, since the parameter durationWithinSlotGroup only indicates the duration of the monitoring, not the starting location within a slot group. The offset O (in terms of slots) is needed to effectively indicate the first slot within a slot group that is monitored Some companies have proposed to extend the legacy parameter monitoringSymbolsWithinSlot to a length 14*X bitmap indicating which OFDM symbols in the entire slot group are monitored. We do not agree to such an approach. The flexibility is not needed, and the RRC overhead is excessive. Comment #6 @vivo: In the proposal above, the periodicity is restricted to an integer multiple of Xs slots only for Group (1) SSs. This condition was suggested by vivo during the GTW and captured by the vice chairman, but we would like further clarification. We note that for some search spaces in Group (2), i.e., for Type0A/2 CSS configured with SearchSpaceId ≠ 0, and for Type-1 CSS without RRC, the search space configuration uses the same SearchSpace IE as for configuring Group (1) search spaces, i.e., the configuration is according to parameters monitoringPeriodicityAndOffset, duration, monitoringSymbolsWithinSlot. What is the motivation for restricting the periodicity/duration as an integer multiple of Xs slots only for Group (1) SSs, and not for Group (2) SSs? This is an important point to clarify. Summary Our first preference is Alt-2 where parameters are defined in terms of slot groups: - monitoringPeriodicityAndOffset-r17: {1,2,4,5,8,10,16,20,40,80,160,320,640,1280,2560} - For periodicity value Xp, the value range for the offset O is {0 .. Xp-1} - Same value range as legacy parameter - duration-r17: all integer values in the range {2 .. 2559} - Same value range as legacy parameter - monitoringSymbolsWithinSlot (length-14 bitmap) - Legacy parameter is reused, and applies to each of the slot(s) within a slot group that are configured for monitoring - monitoringSlotsWithinSlotGroup-r17 (length-8 bitmap) - New parameter with a number of consecutive '1's indicating which consecutive slot(s) within a slot group is/are configured for monitoring We can compromise to Alt-1 (parameters defined in terms of slots) only according to the following: - monitoringPeriodicityAndOffset-r17: {1,2,4,5,8,10,16,20,32,40,64,80,128,160,320,640,1280,2560,5120, 10240,20480} - For periodicity value Xp, the value range for the offset O is {0 .. Xp-1} - Periodicity Xp is restricted to be an integer multiple of Xs - Note: The values in red are new compared to legacy - duration-r17: all integer values in the range {2 .. 20479} - Duration is restricted to be an integer multiple of Xs - Note: The maximum value in red is new compared to legacy - monitoringSymbolsWithinSlot (length-14 bitmap) - Legacy parameter is reused, and applies to each of the slot(s) within a slot group that are configured for monitoring - monitoringSlotsWithinSlotGroup-r17 (length-8 bitmap) - New parameter with a number of consecutive '1's indicating which consecutive slot(s) within a slot group is/are configured for monitoring While we are not supportive, if a new parameter durationWithinSlotGroup is agreed (only applicable to Alt-1), the following conditions must hold: - The parameter indicates the number of consecutive slots within a slot group that is/are monitored, e.g., with value range {1,2,4} - The Offset O must not be restricted to be an integer multiple of X, since the parameter durationWithinSlotGroup only indicates the duration of the monitoring, not the starting location within a slot group. The offset O (in terms of slots) is needed to effectively indicate the first slot within a slot group that is monitored |
| Intel | From the discussions, it is common supported that SS set configuration is done by the following 3 parameters - monitoringSymbolsWithinSlot, which includes periodicity and offset - duration - monitoring slots within a slot group and monitoring symbols within those slots For periodicity and duration, the main divergence is on the unit for the configured values. 3 alternatives can be considered - Option 1: defined in unit of Xs slots - Option 2: defined in unit of slot. Further, capture in the agreement that aUE is not expected to be configured with a periodicity that is not an integer multiple of its supported value(s) Xs for a corresponding SCS for group 1 SSs - Option 3: defined in unit of slot. No need to capture in the agreement that aUE is not expected to be configured with a periodicity that is not an integer multiple of its supported value(s) Xs for a corresponding SCS for group 1 SSs The first two options are under hot discussion. However, we want to emphasis Option 3 is also workable. Since combination (Xs, Ys) is clearly defined in 38.213, which enforces that SS set for group (1) SS must be in the Ys slots. Therefore, even we don’t explicitly describe Xs based restriction in SS set configuration, gNB must guarantee the configured values to be integer times of Xs. For offset which is determined by monitoringSymbolsWithinSlot, it should be in unit of slot as in existing NR. By this way, ‘offset’ can directly identify a slot or a starting slot in a slot group that are configured with MOs. For example, the slot index is mod(offset, Xs) Anyway, we don’t think unit of the value is the critical issue for now. The last bullet in FL proposal should be the major issue for discussion. Samsung provides a good list of options with clear definitions. - Option 1 enforce the same position of MOs in different slots, while Option 2 allows the different positions of MOs in different slots in a slot group. One motivation for Option 2 is the flexible configuration of Group (2) SS. For example, since the MOs of SS set 0 can be in different position in the slots which is determined by the transmitted SSBs, the Type0A/2 CSS sets with searchSpaceID non-zero needs to be configured in different MOs in different slots to align with SS set 0. - Option 1a is preferred than Option 1b. considering flexibility of Group (2) SS, it is not flexible to limit the MOs in consecutive slots in a slot group. For Option 1a, we prefer to define the bitmap with variable length. For example, for Group (1) SS with Y=1, it is applicable to just omit this bitmap. For Group (1) SS with Y>1, Y bits are sufficient. Finally, for Group (2) SS, gNB may use X bits. - One clarification on Option 1c, assuming duration of 20480 slots, is it to use 20480/Xs bits for Option 1c? |
| ZTE, Sanechips | From Huawei’s comment above, we notice that the current definition of duration is “Number of consecutive slots that a SearchSpace lasts in every occasion”. For slot-group based PDCCH monitoring, we need to know the monitoring slots within a slot group and monitoring symbols within those slots, there are two choices we can accept: - Keep the current definition of duration (the unit is slot and it is not restricted to be an integer multiple of Xs of a slot group), and define a slotGroupDuration parameter (it is restricted to be an integer multiple of Xs of a slot group) as Apple mentioned above. - Change the current definition of duration to “Number of consecutive slot groups that a SearchSpace lasts in every occasion”(it is restricted to be an integer multiple of Xs of a slot group) , and further determine a mechanism indicating monitoring symbols with slot. Moreover, we would like to clarify again that if we keep the unit of “offset” in monitoringSlotPeriodicityAndOffset as slot, the “offset” indicates the slot offset between PDCCH monitoring occasion and frame boundary. We do not need to introduce a parameter indicating the monitoring slots within a slot group according to the specification. The following paragraph is extracted from TS 38.213: - a PDCCH monitoring periodicity of ks slots and a PDCCH monitoring offset of os slots, by monitoringSlotPeriodicityAndOffset A UE determines a PDCCH monitoring occasion on an active DL BWP from the PDCCH monitoring periodicity, the PDCCH monitoring offset, and the PDCCH monitoring pattern within a slot. For search space set s, the UE determines that a PDCCH monitoring occasion(s) exists in a slot with number ns,fμ [4, TS 38.211] in a frame with number nf if (nf. Nslot frame, μ+ ns,f μ- os) modks=0nfNslotframe,μ+ns,fμ-op,smod kp,s=0. The UE monitors PDCCH candidates for search space set s for Ts consecutive slots, starting from slot ns,fμ, and does not monitor PDCCH candidates for search space set s for the next ks-Ts consecutive slots. |
| NTT DOCOMO | We agree with Ericsson’s comment #2 that the unit of monitoringSlotPeriodicityAndOffset and duration should be unified with either way of slot or slot group. If we take the unit of monitoringSlotPeriodicityAndOffset and duration to be slot, we support the first three bullets of the FL proposal. For the last bullet, especially for how to indicate the monitored slot in a slot group, we agree with Qualcomm that a new parameter should be introduced to indicate which slot group included in a periodicity is monitored without changing the definition of duration. More specifically, the existing parameter duration indicates the duration of the monitored slot in a slot group (i.e., duration would be up to Y), and the new parameter indicates duration of consecutive slot groups which have monitoring occasion in an indicated periodicity. We believe that this mechanism will provide the smallest specification impact. |
| LG Electronics | We agree with Ericsson’s comments #1, #2, #3 and #4. Especially, we think the way of mixing of some parameters in terms of slots and some parameters in terms of slot groups would make the design complicated. And, we prefer to keep monitoringSymbolsWithinSlot as is and to introduce new parameter indicating which slots within a slot group are configured for monitoring. In addition, while the unit of existing parameters is slot (i.e., unchanged), the 3rd bullet point, i.e., “A UE is not expected to be configured with a periodicity that is not an integer multiple of its supported value(s) Xs for a corresponding SCS for group 1 SSs”, should be reflected in TS 38.213. If not, we need to further discuss on how UE handles the value of periodicity with non integral multiples of X, if configured. |
| Xiaomi | Generally OK with the proposal, and also OK with HW‘s Proposal. But for the parameters about duration, we think it is necessary to determine in this meeting since it has RAN2 impact. we can go with majority on this issue. |
| Nokia, NSB | We support FL’s proposal. This has indeed much smaller specification impact (compared to slot group -based configuration). We also support Oppo’s wording |
| Futurewei | We are OK with HW Proposal to have separate duration definition. For the units we prefer slots for the periodicity/offset and for the duration we could accept the majority’s preference. |
| vivo | @Ericsson: There is the following reasons not to restrict Group (2) SSs: - For the agreed multi-slot PDCCH monitoring capability, there is no restriction of slot location for Group (2) SS; - For Group (2) SSs, we agree that the same IE is used if search space ID is not 0. However, the SS configuration is for all SSBs and different monitoring slots may map to different SSBs. For one UE, only monitoring slots corresponding to one SSB is needed and thus there is no need to restrict SS period for all SSBs as multiple of Xs slots. For example, SS configuration could be a period of one slot and mapping to 4 SSBs. In this case, monitoring slot index#0, #4, #8 and #12 mapping to SSB#0 still belongs to different slot groups. Based on the above observation that Group (1) and Group (2) SS will share the same IE but with different restrictions. We prefer to align the parameters for both Group (1) and Group (2) SS by reusing legacy parameter and definition as much as possible. So we support the first 3 bullets in the proposal. |
| Qualcomm | We are generally supportive with the proposal. However, in the last bullet, it would be required to clarify the intention of “reuse the legacy bitmap”. If it implies no enhancement from Rel-16 for the “additional mechanism to determine monitoring slots within a slot group”, we think it could be a valid option and needs to be clarified. To be specific, without any enhancement, the monitoring slot within a slot group can be determined by the legacy “offset” and “duration”. |
| Panasonic | We support the FL’s first three bullets of the proposal. For the last bullet, we are ok to go with QC’s suggestion on introducing a new parameter“durationSlotGroup”. However, as pointed out by Nokia in the first round discussion, this can be seen as an optimization. Without such enhancement, the monitoring slot within a slot group can be still be determined by legacy “offset” and “duration”. |
| Samsung2 | In general we agree with the observations from Ericsson other than the need of “durationWithinSlotGroup”, and our preference is also Alt 2 in Ericsson’s summary considering the spec impact. |
| Huawei, HiSilicon | According to discussion so far, companies seems have common understanding/requirement on the monitoring behavior while have different views on how to reuse/re-interpret the existing parameters, especially on the field of “duration” to achieve the requirement. Maybe we can suggest first to align the behavior at first: - Periodicity: Same flexibility/range as those for 120kHz SCS in terms of absolute time, should be integer multiple of X slots, - re-interpret existing Ts in monitoringSlotPeriodicityAndOffset in the unit of slot group - introduce new Ts in monitoringSlotPeriodicityAndOffset in the unit of slot - Start of MO in a period: should allow start from any slot in a slot group. - require additional configuration on MO within slot group (see 4a) if existing field of monitoringSlotPeriodicityAndOffset is re-interpreted in the unit of slot group. - by slot level offset corresponding to the existing/new Ts if monitoringSlotPeriodicityAndOffset is defined in the unit of slot. - Number of consecutive slot groups where Searchspace lasts - re-interpret existing field of “duration” in the unit of slot group. - introduce a new field, e.g. “durationSlotgroup” in the unit of slot group - Location of searchspace within slot group: 1/2/3/4 consecutive slot - new field indicating slot level bitmap indicate which slot MO locates - new field indicating duration from the offset indicated by monitoringSlotPeriodicityAndOffset - reuse existing “duration” field and restrict the value range to 1-4 slot - Requirement on Location of searchspace within a slot: - by existing monitoringSymbolsWithinSlot - For the periodicity, both 1a and 1b can work. 1a has less overhead but with larger standard impact. - For start of MO, 2a should work with 1a in periodicity while 2b should work with 1b in periodicity. - For the number of consecutive slot groups (3rd bullet), no matter 3a or 3b is selected, the definition should be clarified. It should be noted that the searchspace is not contiguous in a period (occasion) in terms of slot because the group (1) SS will only located with Y slot in the X slot group. the existing definition of duration field is not valid any more. We do not agree that if, for instance, 3b is used along with 1b it would be a mixed unit because they are defining different things. Even in Rel-15, there are two unit, slot and symbol. as long as the overhead is reduced without impact on flexibility, introducing another unit does not harm as no additional complexity it really introduced. - For the location within slot group, 4a should work together with 1a, while selecting between 4b or 4c depends on wheher 3a or 3b is adopted. - in summary, we can accept following three sets of combinations - set 1: 1a+3a/3b+4a+5a - set 2: 1b+2b+3a+4b+5a - set 3: 2a+2b+3b+4c+5a |
| MediaTek | We share the same view with Apple and ZTE. In particular, we still prefer to keep the same definition on the existing IEs and introduce one new parameter to address the scheduling flexibility of multi-slot PDCCH monitoring configuration. Based on our observation, two approaches mentioned by Apple and ZTE are on the table - keep current IEs and introduce one new IE to describe the number of consecutive slotggroups within a period - reinterpret the unit of current IEs as slot-group and introduce one new IE to describe the consecutive slots within a slotgroup - Both approaches can work but approach 2 requires more discussion since directly changing the unit from slot to slotgroup on existing IEs might not be desirable, e.g., how does UE determine the slot-group size in the PDCCH monitoring configuration is 4 or 8 if the monitoring configuration unit definition is related to slot-group size? With the principle of approach 1(stated above), we suggest the following change on the proposal due to the reason:1. Duration definition is the consecutive slots for PDCCH monitoring and no need to go up to the periodicity 2. Companies may have different understanding on the new IE proposals and we prefer to keep FFS for now - The unit of existing parameters monitoringSlotPeriodicityAndOffset, duration and monitoringSymbolsWithinSlot are slots (i.e. unchanged) - Add new values to duration, e.g. to go up to the maximum periodicity - FFS: which values - … - FFS:Support a mechanism to determine monitoring slots within a slot group and monitoring symbols within those slot or consecutive slot groups within a period, e.g. by - extending the existing bitmap to cover the number of slots in the multi-slot group - new parameter durationWithinSlotGroup - new parameter durationSlotGroup - new bitmap indicating the slots within a slot group, where monitoringSymbolsWithinSlot applies - reuse the legacy bitmap |

#### Summary of second round discussion

Many companies implicitly or explicitly stated a preference to use the same units for parameters monitoringSlotPeriodicityAndOffset, duration and monitoringSymbolsWithinSlot; and overall, more companies express support for using a slot rather than those supporting a slot group as a unit for these parameters.

At this point, there is no consensus on a single package proposal that includes a substantial level of detail for RRC parameters. However since the target is to make progress on issues related to RRC signalling as much as possible, FL suggests two sets of proposals.

FL proposal 2.2.1.4A (for substantial progress in FL's opinion):

For search space set configuration of multi-slot PDCCH monitoring:

- monitoringSlotPeriodicityAndOffset and duration are appended with "-r17", and

  - For monitoringPeriodicityAndOffset-r17

    - The values represent slots

    - Add periodicity values {32,64,128,5120,10240,20480} to the existing values in monitoringSlotPeriodicityAndOffset

      - Note: Total list of supported periodicity values: {1,2,4,5,8,10,16,20,32,40,64,80,128,160,320,640,1280,2560,5120,10240,20480}

    - For each periodicity value Xp

      - The value range for the offset O is {0 .. Xp-1} slots

      - Note: There may be no need to introduce the term "Xp" in the specifications

    - The configured periodicity at least for Group (1) SSs is restricted to be an integer multiple of Xs slots

  - For duration-r17

    - The values represent slots

    - The value range is {4, 8, …, 20476}

    - The configured duration is restricted to be an integer multiple of Xs slots

- monitoringSymbolsWithinSlot applies to each slot in a slot group configured for multi-slot PDCCH monitoring

  - Note: This parameter can be directly re-used from earlier releases.

- Introduce new parameter monitoringSlotsWithinSlotGroup-r17

  - Working assumption:

    - The size is Xs bits

    - Each bit in monitoringSlotsWithinSlotGroup-r17 represents a slot in a slot group

    - A slot in the slot group is configured for multi-slot PDCCH monitoring if the corresponding bit in the slot group is set to '1'

      - Note: Further configuration of the monitoring symbols in such a slot is done by monitoringSymbolsWithinSlot

FL proposal 2.2.1.4B (minimal progress in FL's opinion):

For search space set configuration of multi-slot PDCCH monitoring:

- monitoringSlotPeriodicityAndOffset is appended with "-r17", and

  - For monitoringPeriodicityAndOffset-r17

    - The values represent slots

    - Add periodicity values {32,64,128,5120,10240,20480} to the existing values in monitoringSlotPeriodicityAndOffset

      - Note: Total list of supported periodicity values: {1,2,4,5,8,10,16,20,32,40,64,80,128,160,320,640,1280,2560,5120,10240,20480}

    - For each periodicity value Xp

      - The value range for the offset O is {0 .. Xp-1} slots

      - Note: There may be no need to introduce the term "Xp" in the specifications

    - The configured periodicity at least for Group (1) SSs is restricted to be an integer multiple of Xs slots

- monitoringSymbolsWithinSlot applies to each slot in a slot group configured for multi-slot PDCCH monitoring

  - Note: This parameter can be directly re-used from earlier releases.

- Support a mechanism to determine monitoring slots within a slot group and monitoring symbols within those slots

  - Working assumption: A new RRC parameter is introduced for this purpose

    - FFS: Determine the functionality of the new parameter, e.g. one of the following:

      - Indication of slots within a slot group where monitoringSymbolsWithinSlot applies (e.g. monitoringSlotsWithinSlotGroup)

      - Indicate the number of consecutive slots that contain monitoring occasions within each slot group of X slots (e.g. durationWithinSlotGroup)

      - Indication of number of consecutive slot groups of Xs slots that a duration of SearchSpace is repeated (e.g. durationSlotGroup)

### Issue A2-2: CORESET duration

#### First round discussion

Previous discussion in earlier meetings has shown limited support for a duration of more than 3 OFDM symbols for PDCCH with 480/960 kHz SCS. From the contributions submitted to RAN1#107bis-e, it seems there is no fundamental change of opinion for this matter.

FL suggestion: Conclude that a duration of more than 3 OFDM symbols for PDCCH with 480/960 kHz SCS is not supported.

| Company | Comment |
| --- | --- |
| Ericsson | Support proposed conclusion |
| MediaTek | Support the conclusion |
| Qualcomm | We support the proposal. |
| Intel | We support the FL proposal. in fact, it is already implicitly agreed in last meeting agreement, i.e. relying on FG 3-1/3-5b |
| Nokia, NSB | Support the conclusion |
| vivo | Support the proposed conclusion |
| Apple | Support the conclusion. |
| InterDigital | We are fine with the conclusion. |
| Xiaomi | support |
| Samsung | We are ok with the proposal. Maybe further clarify: FL suggestion: Conclude that a duration of more than 3 OFDM symbols for PDCCH with 480/960 kHz SCS is not supported in Rel-17. |
| Futurewei | We are OK with the proposed conclusion. |
| NTT DOCOMO | We support the FL suggestion. |
| ZTE, Sanechips | We are fine with FL’s suggestion. |
| Lenovo, Motorola Mobility | Fine with the proposed conclusion |
| Transsion | We support the conclusion. |
| Sharp | We support the FL suggestion. |
| LG Electronics | Support the FL suggestion. |
| Sony | We think a large CORESET duration with more than 3 symbols for SCS 480kHz and 960kHz could alleviate UE processing capability for PDCCH decoding. However, we are also fine with the proposal if it is the majority view. |
| Panasonic | We support the conclusion |
| Huawei, HiSilicon | Support FL proposal |

#### Summary of first round discussion

While some companies identify a benefit of extending the CORESET beyond 3 OFDM symbols, all companies indicated support or acceptance of the suggested conclusion. Samsung suggest to clarify that the conclusion applies only to Release 17, however FL notes that the whole WI and this agenda items only has a scope and mandate for Rel-17, so explicitly mentioning Rel-17 is not required.

FL suggestion: Conclude that a duration of more than 3 OFDM symbols for PDCCH with 480/960 kHz SCS is not supported.

### Issue A2-3: SS set group switching minimum time

#### First round discussion

Most companies intend confirm the working assumption reached in RAN#107-e for the minimum switching times, i.e.:

Working assumption

The following values are adopted as minimum value of Pswitch for 120/480/960 kHz

- Support only search space set group switching processing capability 1 with the following values

| μ | Minimum Pswitch value for UE processing capability 1 [symbols] |
| --- | --- |
| 3 | 40 |
| 5 | 160 |
| 6 | 320 |

FL suggestion: Promote the working assumption to an agreement.

| Company | Comment |
| --- | --- |
| Ericsson | Agree to confirm the working assumption |
| MediaTek | We prefer to wait until the functionality of the search space set group switching is finalized. |
| Qualcomm | We support the proposal |
| Intel | Agree to confirm the working assumption |
| Nokia, NSB | We support the proposal |
| vivo | Support the proposal |
| Apple | Agree with the suggestion |
| InterDigital | We support the proposal. |
| Xiaomi | support |
| Samsung | We support the proposal. |
| Futurewei | We support the proposal. |
| NTT DOCOMO | Agree with the FL suggestion. |
| ZTE, Sanechips | We agree to confirm the working assumption. |
| Lenovo, Motorola Mobility | We support the proposal |
| Transsion | Agree to confirm the working assumption. |
| Sharp | We prefer to wait until the functionality of the search space set group switching is finalized. |
| LG Electronics | Support the FL suggestion. |
| Panasonic | We support the proposal. |
| Huawei, HiSilicon | Support |

#### Summary of first round discussion

While almost all companies support the proposal to convert the existing WA to an agreement, two companies (MediaTek, Sharp) suggest to keep it as WA until SSSG switching is finalized. FL notes that this doesn't seem to affect RRC, therefore it should be acceptable to keep the WA for the time being.

### Issue A2-4: SS set group switching behaviour

#### First round discussion

Sony (R1-2200174):

Proposal 1: SSSG switching schemes with finer granularity can be considered for NR from 52.6 GHz to 71GHz.

| Company | Comment |
| --- | --- |
| Samsung | Better to clarify “finer granularity” in the proposal. Does it mean “beam-specific”? |
| Sony | The “finer granularity” means to introduce a new dimension of SSS switch, which is not just on sparse/dense (as in Rel-16), but also considering the number of BD attempts.”. In other words, on the one hand, if only monitoring capability based SSSG switching is considered, with (X,Y) = (8,1) as mandatory and (8,4), (4,2), (4,1) as optional for SCS 960kHz, when more than two monitoring capacity are supported by UE, monitoring capability based SSSG switching will introdce finer granularity of SSSG than that in R-16. On the other hand, since 52.6 GHz to 71GHz have dual features with unlicensed band and multi monitoring capability, a two level SSSG switching with both monitoring capability combinations (X, Y) based SSSG switching and legacy R-16 SSSG switching, which leads to further finer granularity of SSSG can be considered,. |

Qualcomm (R1-2200290):

Proposal 4: For multi-slot PDCCH monitoring for 480/960 kHz SCSs, a boundary of SSSG switching is always aligned with a boundary of a slot group.

- The interpretation of configuration parameters and timer operation of SSSG switching is not changed from those of Rel-16 SSSG switching.

Proposal 5: The location of Y consecutive slots within a slot group is maintained for all Group (1) SS sets within a BWP, regardless of the associated SSSG indices.

- FFS: configuration granularity of the location of the Y slots – per BWP, per CC, per UE, etc.

Figure 1: An example of SSSG switching for multi-slot PDCCH monitoring.

| Company | Comment |
| --- | --- |
| Intel | Since P_switch (WA for now) is quite large value, UE should already well prepare for the SSSG switching, we prefer to allow UE to do immediate SSSG switching at time t0+P_switch, t0 is the time of the trigger. Such a proposal fit well with the agreement that Ys slot(s) can be anywhere in a slot group. For example, assuming t0+P_switch is in the middle of a slot group, there is 50% possibility that UE can benefit from early using the new SSSG (whenever Ys is in second half of a slot group). |
| Nokia, NSB | We support the proposal 4 We don’t see a need for the Proposal 5. |
| vivo | We support the proposal 4 |
| Apple | Support proposal 4 |
| Xiaomi | We don’t see the reason for Proposal 4. Even boundary of SSSG switching is not aligned with a boundary of a slot group, UE can still monitor the new SSSG after the boundary of SSSG switching. We don’t see the need for Proposal 5. Since we have already agreed that The location of the Y consecutive slots within the slot group of X slots is maintained across different slot groups, why have to mention “associated SSSG indices”? |
| Samsung | In general we are ok with the proposals. |
| OPPO | We have a question to proposal 4: if the SSSG switching is aligned with slot group, then the timer should also be aligned with slot group. Isn’t it ? |
| Futurewei | We are ok with the proposal 4. |
| NTT DOCOMO | Given that the boundaries of slot groups and SSSG switching are aligned and the location of Y within the X slot is maintained across slot groups even if SSSG is switched, back-to-back monitoring issue would be avoided. Thus, such restriction as proposal 4, i.e., alignment of the boundaries of slot groups and SSSG switching and the fixed location of Y slots within X slots, seems reasonable. In addition, it needs more investigation whether other cases, e.g., boundaries of slot groups and SSSG switching are not aligned and/or applied X can be switched associated with SSSG switching, can avoid back-to-back monitoring issue. It maybe depends on the determination rule of applied (Xs, Ys) from multiple (Xs, Ys) discussed in Issue A1-3. |
| ZTE, Sanechips | Question to Qualcom: For Proposal 5, by suggesting the location of Y consecutive slots within a slot group is the same for both SSSG#0 and SSSG#1 within a BWP, do you assume those two SSSGs having the same value of Y? However, in our opinion Proposal 5 is not quite accurate if SSSG switching between two different capabilities of multi-slot PDCCH monitoring for 480/960kHz (e.g. switch from (X,Y)=(4,1) to (X,Y)=(4,2)) . |
| Transsion | We support proposal 4. Regarding proposal 5, we believe it is already covered by the previous agreement: Agreement - For Group (1) SS: Type 1 CSS with dedicated RRC configuration and type 3 CSS, UE specific SS - A SS is monitored within Y consecutive slots within a slot group of X slots - The Y consecutive slots can be located anywhere within the slot group of X slots - Note: There is no requirement to align the Y consecutive slots across UEs or with slot n0 - The location of the Y consecutive slots within the slot group of X slots is maintained across different slot groups |
| Sharp | We share same view with DOCOMO. If back-to-back problems occur, a solution will be needed. |
| LG Electronics | We agree with Proposal 4. We think SSSG switching boundary should be aligned with a slot-group boundary. But it may be redundant to count the timer value in unit of slot-group. So, timer decrement procedure can be kept as Rel-16 SSSG switching. We agree with Proposal 5. In our understanding, Y slots within a slot group should be determined considering all Group (1) SSs in a BWP, so the location of Y slots within a slot group is maintained even after SSSG switching. |
| Ericsson | We don't see a need for either proposal. To support SSSG switching, it seems straightforward to support for 480/960 kHz SCS only by defining the minimum switch times. Further optimizations seem unnecessary; the feature will still work. |
| Huawei, HiSilicon | We can support proposal 4. We don’t see why proposal 5 is necessary. Y MO slots within a distance less than X slots on either side of SSSG switching boundary can be avoided by gNB using a proper switching trigger time or by UE using discarding the first MO in the second SSSG after switching. |
| Qualcomm | To clarify the motivation of Proposal 4, if the SSSG switching occurs in the middle of a slot group and, as a result, if SS sets from both SSSG#0 and SSSG#1 are included in the same group, BD/CCE counting, overbooking, and dropping should be re-calculated. To answer OPPO’s question: In our view, the timer operation can be the same as Rel-16, i.e., the timer decreases by one after each slot. To answer ZTE’s question: Yes, the proposal is based on the assumption that the same (X,Y) capability is applied for both of the SSSGs. However, we think it can be extended for the case that different (X,Y) capabilities are applied to different SSSGs. For example, if the UE supports (8,1) and (4,1), and SSSG#0 is associated with (8,1) and SSSG#1 with (4,1), the position of Y can be determined to comply with (4,1) (i.e., the one with smaller X value) across all SSSGs. That is, if we divide the slot group of X=8 into two half slot groups of 4 slots, the Y slot can be positioned in either the first or the second half. Then the position of the Y slot in the half slot is the same the position of Y slot in the slot group of X=4 associated with SSSG#1. |

Intel (R1-2200367):

Proposal 7:

- Dynamic SSSG switching is supported for all SCSs 120, 480 and 960kHz.

- The search space set configurations of the two SSSG can correspond to two different PDCCH monitoring capabilities combinations (X, Y)

- UE can monitor the PDCCHs in the X slots before the time of SSSG switching in the slots that are the intersection of the slot patterns of the two combinations (X, Y) before and after SSSG switching

- PDCCH monitoring following the second SSSG can start right after the time of SSSG switching

- Agree on TP 4 to do SSSG switching with different multi-slot PDCCH monitoring capability combinations (X, Y).

| Company | Comment |
| --- | --- |
| Qualcomm | We do not support the fourth bullet. Alignment of SSSG switching and slot group boundaries is preferred for simplicity. |
| Intel | Since P_switch (WA for now) is quite large value, UE should already well prepare for the SSSG switching, we prefer to allow UE to do immediate SSSG switching at time t0+P_switch, t0 is the time of the trigger. Such a proposal fit well with the agreement that Ys slot(s) can be anywhere in a slot group. For example, assuming t0+P_switch is in the middle of a slot group, there is 50% possibility that UE can benefit from early using the new SSSG (whenever Ys is in second half of a slot group). |
| Nokia, NSB | Agree with Qualcomm’s view |
| vivo | We don’t support the second, third and fourth bullet. We think one (Xs, Ys) for all SSSGs is preferred for simplicity. |
| Apple | Prefer that SSSG should occur on a slot group boundary. |
| xiaomi | We support “Dynamic SSSG switching is supported for all SCSs 120, 480 and 960kHz” And do not support others. |
| Samsung | In general we support adaptation of SSSG switching for multi-PDCCH monitoring, and the TP can be further discussed. However, since PDCCH monitoring capability is configured per cell to follow legacy principle, SSSG switching to adapt combination (X, Y) may require much spec efforts for new design on SSSG switching or PDCCH monitoring capability configuration. We prefer to reuse the existing of SSSG switching framework. |
| Futurewei | Support first bullet, we prefer the SSSG at the slot group boundary. |
| Transsion | We support the first bullet. And don’t support the others. |
| Sharp | We are OK with the proposal, but we should discuss solutions to the possible back-to-back problems. |
| LG Electronics | We could only support the first bullet among the listed proposals. Regarding the 2nd and 5th bullets, we believe that it is sufficient to support the switching between two SSSGs with different parameters (e.g., periodicities) while they share the same multi-slot monitoring capability combination (X,Y). Regarding the 4th bullet, we share the view with Qualcomm. |
| Sony | We support the first bullet. We support the second bullet but think it may not necessary to be limited to two SSSG. |
| Ericsson | While we support the first bullet, but we also don't think an agreement is needed for this. As long as the minimum switch times are defined for 120, 480, 960, then it is inherently supported. We so need for further optimizations or spec changes to support SSSG switching. |
| Huawei, HiSilicon | We support the first bullet. |

NEC (R1-2200507):

Proposal 1: For operation in unlicensed band with 960 kHz SCS, support SSSG switching along with changing different PDCCH monitoring capability.

Proposal 2: For operation in unlicensed band with 480 kHz and 960 kHz SCS, the switching boundary and the timer counter should be modified to slot group based.

Transsion (R1-2200558):

Proposal 5: SSSG switching mechanism should take the slot group as the unit.

LG (R1-2200565):

Proposal #6: For 480 kHz or 960 kHz multi-slot monitoring, SSSG switching should be performed at the slot-group boundary after at least P_switch symbols from the switching triggering.

| Company | Comment |
| --- | --- |
| Qualcomm | We generally support the proposals of aligning SSSG switching and slot group boundaries. However, for the timer operation, we prefer per-slot timer decrement to keep the same configuration and operation as Rel-16 SSSG switching. |
| Intel | Assuming UE reports the capability to support multiple combinations (Xs, Ys), we don’t see a reason to forbid the use of such multiple combinations for the two SSSGs. |
| Nokia, NSB | Agree with Qualcomm’s view |
| Apple | Support LG’s proposal #6 and Transsion’s proposa. |
| Xiaomi | Support “Proposal 1: For operation in unlicensed band with 960 kHz SCS, support SSSG switching along with changing different PDCCH monitoring capability.” But we think it has no spec impact, can be done by gNB configuration We don’t see the need of aligning SSSG switching and slot group boundaries. Even boundary of SSSG switching is not aligned with a boundary of a slot group, UE can still monitor the new SSSG after the boundary of SSSG switching. |
| Samsung | In general we support adaptation of SSSG switching for multi-PDCCH monitoring, and the TP can be further discussed. |
| OPPO | support |
| Futurewei | We support in principle the proposals and open to further discuss the TP |
| Transsion | We support in principle. |
| Sharp | We support the proposals. |
| LG Electronics | Regarding NEC’s Proposal 1, we believe that it is sufficient to support the switching between two SSSGs with different parameters (e.g., periodicities) while they share the same multi-slot monitoring capability combination (X,Y). Regarding SSSG switching mechanism, we share the view with Qualcomm that aligning SSSG switching and slot-group boundary is necessary, but per-slot timer decrement could be kept. |
| Sony | We support NEC’s proposal #1 on SSSG switching along with different PDCCH monitoring capability, and we are also supportive on a two level SSSG switching based on both monitoring capability and sparse/dense in R-16. |
| Ericsson | As commented above, we see no need to align with slot group boundaries. |
| Huawei, HiSilicon | We appreciate the intention behind the proposals. We think it is enough to agree on proposal 4 of R1-2200290. |

CATT (R1-2200143):

Proposal 5: The Legacy SSSG switching mechanism should be reused for the 120 kHz SCS, 480 kHz SCS and 960 kHz SCS in 60 GHz NR-U.

| Company | Comment |
| --- | --- |
| Xiaomi | Agree in general. |
| Samsung | In general we support adaptation of SSSG switching for multi-PDCCH monitoring, and the TP can be further discussed. |
| Ericsson | Agree in principle, but only spec change needed is to define the minimum switching time for 480/960 kHz. |
| Huawei, HiSilicon | we think such general proposals are interpretable and not needed in the maintenance phase. |

#### Summary of first round discussion

A majority of companies show support to align the SSG switching boundary with the boundary of a slot group.

A majority of companies do not support SSSG switching between two different PDCCH monitoring combinations (X, Y). It may therefore be beneficial to agree that SSSG switching is only supported between SSSGs that have the same (X,Y) PDCCH monitoring combination.

A majority of companies support to keep the unit for the SSSG switching timer as slots instead of slot groups.

FL suggests to continue discussion in the second round based on these proposals/conclusion.

#### Second round discussion

FL proposed conclusion : SSSG switching timer is in units of slots

FL proposed agreements :

- The SSSG switching boundary is aligned with the boundary of a slot group

- SSSG switching is only supported between SSSGs that have the same (X,Y) PDCCH monitoring combination.

| Company | Comment |
| --- | --- |
| Lenovo, Motorola Mobility | Support the FL conclusion and proposal |
| Apple | We are fine with the conclusion and proposal |
| Samsung | In general, we are ok with the proposal. - The meaning of “The SSSG switching boundary” is not very clear, maybe “the timing UE receives the SSSG switching trigger” may be better? But anyway we can understand the intention of the proposal, and the wording can be left for TP discussion. - We wonder the necessity of the spec impact of SSSG switching is only supported between SSSGs that have the same (X,Y) PDCCH monitoring combination, and again can be discussed when TP is ready. |
| Ericsson | We are fine with the proposed conclusion that SSSG switching timer is in units of slots However, we don't think that either of the proposed agreements (aligning switching boundary, and restrictions on (X,Y) within a group). It is not clear the benefit, or problem that it attempts to solve. |
| Intel | We support the FL proposed conclusion. Regarding the first bullet of FL proposed agreement, we still prefer to do PDCCH monitoring according to new SSSG from ‘trigger time + P_switch’. If max BD/CCE handling/overbooking is a concern, UE can simply skip PDCCH monitoring in the immediately Xs slots before ‘trigger time + P_switch’. Further, skipping PDCCH in the Xs slots has an advantage of avoiding back-to-back issue around the SSSG switching boundary. Note the current first bullet of FL proposed agreement may end up with back-to-back issue. Regarding the second bullet in FL proposed agreement, we cannot agree the limitation on same combination (X, Y) for the two SSSGs. For example, for a UE supporting combination (8,1) and (4,1), gNB may choose to use (4,1) for the first SSSG for frequent PDCCH monitoring. On the other hand, gNB may use configure SS set with long periodicity in the gNB COT. However, if restricting same (X,Y) for the two SSSGs, it results in (4,1) has to be assumed in the COT even when the PDCCH MOs only repeats in every 8 slots or longer. The key drawback here is UE only assumes half number of BD/CCE in the PDCCH monitoring which largely impact PDCCH transmission flexibility |
| ZTE, Sanechips | In our understanding, SSSG switching between two different capabilities of multi-slot PDCCH monitoring for 480/960kHz should be supported and this can be further discussed. We can compromise to FL’s proposal for the sake of progress. |
| NTT DOCOMO | We support the FL proposed conclusion and agreement. In addition, if we support the case when SSSG switching boundary and slot group boundary is not aligned, we think the following points need to be ensured to avoid back-to-back monitoring issue even if SSSG is switched since it is unclear whether it is common understanding or not according to the current agreement: - The applied value of (X, Y) is not changed. - The location of Y slot(s) in a X slot group. - Slot groups are consecutive and non-overlapping. |
| LG Electronics | We support the FL proposed conclusion. We support the first bullet of FL proposed agreement. In Rel-16 NR-U, BD/CCE budget was defined in units of slots, and SSSG switching boundary is aligned with slot boundary, not symbol boundary within a slot. This principle can be naturally extended to Rel-17. In multi-slot PDCCH monitoring, BD/CCE budget is defined in unit of slot-group. Thus, the SSSG switching boundary can be similarily extended to align with the boundary of a slot-group, rather than slot boundary. Since the BD/CCE budget is checked in units of slot-group, SSSG change in the middle of slot-group may cause to increase UE complexity compared to that for the SSSG switching at slot-group boundary. Regarding the second bullet of FL proposed agreement, we agree with the concept of the proposal. However, it should be the conclusion instead of the agreement. In our understanding, all configured Group (1) SSs are associated with only one (X,Y), at least in a certain BWP, even when UE reports multiple (X,Y) combinations. So, SSSG switching is automatically supported only between SSSGs that have the same (X,Y) combination. SSSG switching between different (X,Y) combinations or between different monitoring capaiblities can be supported via e.g., BWP switching without any specification impact. Regarding the first question from Samsung, we think “SSSG switching boundary” means the timing UE starts monitoring PDCCH according to SSSG#0 and stops monitoring PDCCH for SSSG#1 (or vice versa), rather than triggering timing. |
| Huawei, HiSilicon | We do not support the second sub-bullet of “FL proposed agreement”. We don’t see why the two different SSSGs have to support the same (X,Y) combination. This is an unnecessarily conservative approach to guarantee that the distance between two consecutive Ys SS slots on either side of the switching boundary is not less than Xs. For instance (4,1) can switch to (8,1) as long as the distance between the last monitoring slot in (4,1) to the first monitoring slot in (8,1) is not less than 4. This can be easily supported by gNB implementation. Alternatively, UE can just drop monitoring the first SS in the “switched to” SSSG if the distance between two consecutive Ys SS slots on either side of the switching boundary is not less than Xs. We are OK with the following: FL proposed conclusion : SSSG switching timer is in units of slots FL proposed agreements : - The SSSG switching boundary is aligned with the boundary of a slot group - SSSG switching is only supported between SSSGs that have the same (X,Y) PDCCH monitoring combination. |
| Xiaomi | We support the FL proposed conclusion. We do not support FL proposed agreement. as we commented earlier, We don’t see the need of aligning SSSG switching and slot group boundaries. Even boundary of SSSG switching is not aligned with a boundary of a slot group, UE can still monitor the new SSSG after the boundary of SSSG switching. |
| Nokia, NSB | We support the FL proposal, ecept the last bullet. We think that the last bullet is not necessary. |
| Futurewei | We support FL conclusion. We support FL first bullet for the SSSG switching boundary. We do not see the benefit of restricting SSSG switching between same (X,Y) combinations |
| vivo | We are fine with the proposed conclusion. For the first bullet of proposed agreement, we are OK with the proposal. For the second bullet of proposed agreement, we think the key issue is how to check whether SS configuration comply (Xs, Ys) combinations: Alt. 1: All SS configurations including both SSSGs should comply with at least one of (Xs, Ys) combinations supported by UE; Alt. 2: SS configurations for each SSSG should comply with at least one of (Xs, Ys) combinations supported by UE. Our preference is Alt. 1 for simplicity since other parameters such as BD/CCEs are also defined for all SS configurations including both SSSGs in Rel-16 SSSG switching framework. |
| Qualcomm | We support FL proposed conclusion and the first bullet of FL proposed agreements. However, we are not quite supportive with the second bullet of FL proposed agreements. We don’t see any reason to give up such a flexibility. |
| Panasonic | We are supportive of the FL proposed conclusion and agreements. With the agreements, the SSSG switching behavior is clarified without the need to define additional dropping rules. |
| Huawei, HiSilicon2 | We just noticed that in our previous reply, the second bullet to FL proposed agreement in not crossed out. This was a mistake. As explained above, we are OK with FL proposed conclusion and the first bullet of FL proposed agreements but NOT the second bullet. - SSSG switching is only supported between SSSGs that have the same (X,Y) PDCCH monitoring combination. |
| MediaTek | We support the proposal. If such proposal can be agreed, we can confirm the WA for issue A2-3. If SSSG switching between different (X,Y) is allowed, then more discussion on A2-3 and 2-5 is needed. |

#### Summary of second round discussion

After the second round, most companies expressed support for the conclusion that the SSSG switching timer is in units of slots.

Regarding alignment of SSSG switching boundary with slot groups, three companies do not see a need to align applying the new SSSG for PDCCH monitoring with the slot group boundary, while nine companies support the proposal to align the SSSG switching boundary with the slot group boundary.

Regarding supporting only SSSG switching between SSSGs with the same (X,Y) PDCCH monitoring combination, there is no clear majority at this stage. Six companies expressed a preference to limit SSSG switching SSSGs with the same (X,Y) PDCCH monitoring combination, while five companies do not agree to such a limitation. LG points out that

In our understanding, all configured Group (1) SSs are associated with only one (X,Y), at least in a certain BWP, even when UE reports multiple (X,Y) combinations. So, SSSG switching is automatically supported only between SSSGs that have the same (X,Y) combination. SSSG switching between different (X,Y) combinations or between different monitoring capaiblities can be supported via e.g., BWP switching without any specification impact.

FL proposal 2.2.4.4:

Conclude that the SSSG switching timer is in units of slots.

Agree that the SSSG switching boundary, i.e. the time of applying the switching, is aligned with the boundary of a slot group.

Discussion on supporting SSSG switching between different (X,Y) PDCCH monitoring combinations can continue in RAN1#108-e.

### Issue A2-5: Maximum value of searchSpaceSwitchTimer (RRC impact)

#### First round discussion

LG (R1-2200567):

Proposal #7: Define 40/160/320 slots as the maximum value of searchSpaceSwitchTimer for 120/480/960 kHz SCS, respectively.

| Company | Comment |
| --- | --- |
| Samsung | We are ok with the proposal. |
| NTT DOCOMO | We think it should be discussed in AI 8.2.5. |
| LG Electronics | We support the proposal as a proponent. The maximum value of searchSpaceSwitchTimer is 20/40/80 for 15/30/60 kHz, respectively. These are values derived from the maximum COT duration, 20 msec, for the NR-U in Rel-16. Since the maximum COT duration in FR2-2 was determined to be 5 msec, the maximum value of searchSpaceSwitchTimer can be defined as the number of slots corresponding to that time for each SCS, i.e., 40/160/320 slots for 120/480/960 kHz SCS, respectively. |
| Moderator | This issue is closed; please continue discussion in this under Issue A2-6 Q2 below. |

### Issue A2-6: SSSG switching timer and PDCCH candidate skipping values

#### First round discussion

FL NOTE: This issue has been inherited from AI 8.2.5. Please provide your input below if it hasn't been captured yet. In case of updating your view, please add a new entry at the end of the table.

The following was agreed in RAN1#107-e.

Agreement

From RAN1 perspective, for NR operation with 480 kHz and/or 960 kHz SCS, the value of minimum time gap for wake-up and Scell dormancy indication (DCI format 2_6) is scaled by 4 and 8 of the corresponding value of 120 kHz SCS for 480 kHz and 960 kHz SCS respectively.

- Note: X in 38.213 Section 10.3 and 38.133 Section 8.2.1.2.7.

- Send LS to RAN4 to inform about RAN1’s agreement of the reference values and ask RAN4 to make final decision

[17, Xiaomi] observed that R16 power saving WUS feature can be applied on NR 52.6-71GHz given the above agreement. [17, Xiaomi] proposed to allow R17 DCI-based power saving feature for FR2-2 with 480 and/or 960 kHz SCS as well where SSSG switching timer and PDCCH skipping values for 480kHz and 960kHz SCS should be defined. The same scaling factor 4 and 8 can be applied based on the values for 120kHz. For example, the value of the SSSG switching timer in slots for SSSG#1 and/or SSSG#2 can be configured as, {[4,8,12,16,…,640,1280,1600,2560,3200]} for 480kHz SCS,  {[8,16,24,32,…, 1280,1600,2560,3200,6400]} for 960kHz SCS. The candidate skipping values can be configured as {[4,8,12,16,…,640,1280,1600,2560,3200]} for 480kHz SCS,  {[8,16,24,32,…, 1280,1600,2560,3200,6400]} for 960 kHz SCS.

The following are the relevant agreements made in Rel-17 UE power saving enhancements WI.

Agreement

Confirm the working assumptions with the following updates (extract from RAN1#106-bis agreements)

- The value of the timer in slots for monitoring PDCCH in the active DL BWP of the serving cell before moving to the default search space group is

- {1,2,3,…,20,30, 40, 50, 60, 80, 100} for 15 kHz SCS,

- {1,2,3,…,40, 60, 80, 100, 100,160,200} for 30 kHz SCS,

- {1,2,3,…,80, 120, 160, 200, 240, 320,400} for 60kHz SCS,

- {1,2,3,…,160, 240, 320,400, 480, 640,800} for 120kHz SCS

Agreement

- For value X in Beh 1A, candidate skipping values are

  - Up to [100ms] length is supported,

    - The X is configured and indicated in the unit of slot.

      - Working assumption for candidate values for X

        - {1,2,3,…,20,30, 40, 50, 60, 80, 100} for 15 kHz SCS,

        - {1,2,3,…,40, 60, 80, 100, 120,160,200} for 30 kHz SCS,

        - {1,2,3,…,80, 120, 160, 200, 240, 320,400} for 60kHz SCS,

        - {1,2,3,…,160, 240, 320,400, 480, 640,800} for 120kHz SCS

  - FFS: Equal to or longer than the applicable minimum scheduling offset

  - FFS: additional symbol level / PDCCH monitoring period level skipping duration

Moderator’s comment:

It is moderator’s understanding that SSSG switching timer and PDCCH candidate skipping values agreed so far in Rel-17 UE power saving enhancements WI do not cover 480 and/or 960 kHz SCS yet. It is worth to discuss on high level whether feature introduced in Rel-17 UE power saving enhancements WI can be extended for NR operation with 480 and/or 960 kHz SCS. Once agreed, then we can discuss further what values for 480 and/or 960 kHz SCS to enable that feature. Note that the scaling principle proposed in [17] is straight forward and consistent with what applied to other timelines for 480 and/or 960 kHz SCS. Formulate the following questions for discussion.

Q1: Do you think SSSG switching and PDCCH skipping feature introduced in Rel-17 UE power saving enhancement WI can be extended for NR operation in FR2-2 with 480 and/or 960 kHz SCS? Please elaborate your reasoning.

Q2: If the answer to Q1 is yes, do you agree to scale the values corresponding for 120 kHz by 4 and 8 for 480 and 960 kHz SCS, respectively? Please elaborate your reasoning.

Q3: Do you think there are other expected additional specification changes to support this feature for NR operation in FR2-2 with 480 and/or 960 kHz SCS? If so, please elaborate.

| Company | Comment |
| --- | --- |
| Xiaomi | For Q1, Yes, SSSG switching and PDCCH skipping feature introduced in Rel-17 UE power saving enhancement WI can be extended for NR operation in FR2-2 with 480 and/or 960 kHz SCS. We don’t see any discrepancy and R16 DCI based power saving has already extended to FR2-2 with 480 and/or 960 kHz SCS, R17 DCI based power saving For Q2, Agree. Follow the same principle as we do to R16 minimum time gap for wake-up and Scell dormancy indication (DCI format 2_6). For Q3, No, currently we don’t see other additional specification changes. |
| Samsung | Q1. Yes. Our view is that a feature agreed in other WI in parallel is applicable to FR2_2 unless explicit agreements precluding the feature are made. So, we are ok to support SSSG switching and PDCCH skipping for FR2_2 in principle. But, if we identify any issues for FR2_2, we are also ok to not support these for FR2_2. Q2. Basically, we agree with the scaling the maximum value. But, the candidate values should be further discussed. For example, the following two options are considered. Option 1. {1,2,3,…,160, 240, 320,400, 480, 640,800}*4 for 480kHz {1,2,3,…,160, 240, 320,400, 480, 640,800}*8 for 480kHz Option 2. {1,2,3,…,640, 960, 1280, 1600, 1920, 2560, 3200} for 480kHz {1,2,3,…,1280, 1920, 2560, 3200, 3840, 5120, 6400} for 9600kHz Q3. No additional specification works are expected. To be clear, it should be confirmed in Rel-17 UE power saving WI. |
| Qualcomm | Q1: Yes, SSSG switching and PDCCH skipping feature introduced in Rel-17 UE power saving enhancement WI can be extended for NR operation in FR2-2 with 480 and/or 960 kHz SCS. We do not see strong motivation to prevent such extension. Q2: Yes, we agree. As mentioned by Xiaomi, we followed the same approach with R16 minimum time gap for wake-up and Scell dormancy indication (DCI format 2_6). Q3: No. |
| DOCOMO | Q1: Yes. Q2: Yes. To scale the values corresponding for 120 kHz by 4 and 8 for 480 and 960 kHz SCS is the most simplest way. Q3: No. |
| MediaTek | Q1:yes Q2: scaling by 4 and 8 for 480kHz and 960kHz should be considered in the maintenance stage. Q3:no |
| Futurewei | Q1: Yes. We agree with Xiaomi and see no strong motivation so far to prevent the feature be extended to FR2-2; Q2: Yes. Support to scale the values for 480/960kHz SCS; Q3: No. Currently see no additional specification change needed. |
| ZTE, Sanechips | Q1:Yes Q2: Yes, it is straightforward to scale the value of the SSSG switching timer and candidate skipping values. Q3: No |
| Nokia, NSB | SSSG switching is discussed under PDCCH AI (AI 8.2.2). We proposed to discuss it there (and not in here). The following working assumption was made in RAN1 #107-e |
| Apple | Q1: yes Q2: Yes, i.e. scaled by 4 and 8 of the corresponding value of 120 kHz SCS for 480 kHz and 960 kHz SCS respectively Q3: No |
| Huawei, HiSilicon | Q1: yes Q2: support to simply scale the value for 120kHz by 4 and 8 for 480kHz and 960kHz SCS Q3: No We are also fine to discuss it in 8.2.2 as P_switch had be determined in that AI. |
| vivo | Q1: Yes. Q2: Yes. Agree to scale by 4 and 8 for SCS 480kHz and 960kHz respectively. Q3: No. |
| Intel | Q1. We are ok to support SSSG switching and PDCCH skipping for FR2-2. Q2. We are ok to scale the candidate values based on subcarrier spacing using 120kHz as reference. However, the instead of agreement, we should consider WA so that companies can double check. Q3. We are not aware of other changes, but we are reviewing the impact from other WI, so it might not be wise to conclude that there are no other changes and just have companies provide inputs as they find them. |
| CATT | Q1: yes Q2: agree scale the value for 120kHz by 4 and 8 for 480kHz and 960kHz SCS Q3: No |
| Ericsson | If we make any agreement in this area, it should probably be a WA (as pointed out by Intel in response to Q2). We agree with Intel's response on Q3. Generally, we think that it is safer to make agreements on an incremental basis as companies find issues, rather than broad brush agreements such as "Support R17 power savings features". |
| Moderator (AI 8.2.5) | Summary of company views: For Q1: other than Nokia, all companies support to extend SSSG switching timer and PDCCH skipping feature introduced in Rel-17 UE power saving enhancement WI for NR operation in FR2-2 with 480 and/or 960 kHz SCS. Nokia proposed to discuss this issue in AI 8.2.2. For Q2: most companies support to simply scale the values of 120 kHz by 4 and 8 for 480 kHz and 960 kHz SCS. However, Samsung suggest to discuss further on the candidate values as there’re other options proposed. Intel and Ericsson proposed to take WA rather than agreement. For Q3: most companies do not see any additional specification impact. Though there’re comments suggesting to confirm in UE power saving WI. Before proceed further on this issue, regarding which agenda to discuss, moderator believe a guidance from Chairman is required. |
| Moderator (AI 8.2.5) | Per the guidance from Chairman (email from VC copied below), this discussion is moved to AI 8.2.2. Regarding discussion point 1-3 in section 2.1.2.3, let’s move that discussion to the PDCCH monitoring agenda 8.2.2 since the issue is about PDCCH monitoring behavior rather than processing timeline, and other PDCCH monitoring aspects for SSSG switching are being discussed in 8.2.2 already. |
| Ericsson 2 | Regarding Q2, we think it is sufficient to scale the values by 4 and 8, as in Samsung's Option 1: Option 1. {1,2,3,…,160, 240, 320,400, 480, 640,800}*4 for 480kHz {1,2,3,…,160, 240, 320,400, 480, 640,800}*8 for 480kHz We don't think increased granularity in Option 2 is needed. |
| NTT DOCOMO 2 | At Rel-17 power saving WI, the following working assumption was discussed at the last Xiaodong’s GTW session: Working Assumption At least for non LBT case, 480kHz and 960kHz SCS is supported for Rel-17 PDCCH monitoring adaptation - The candidate skipping values and SSSG switching initial timer values in slots, e.g., - The candidate skipping values can be configured as - {[4,8,12,16,...,640,1280,1600,2560,3200]} for 480kHz SCS, - {[8,16,24,32,..., 1280,1600,2560,3200,6400]} for 960kHz SCS. - The value of the SSSG switching timer in slots can be configured as, - {[4,8,12,16,...,640,1280,1600,2560,3200]} for 480kHz SCS, - {[8,16,24,32,..., 1280,1600,2560,3200,6400]} for 960kHz SCS. Note: use the bit length instead of the detailed value set if the value set can not reach consensus, and the bit length is assumed to be the same as that for 120KHz SCS. It seems that a similar discussion is on-going in parallel between power saving WI and here, while only non LBT case is considered in PowSav. It may cause confusion and we think coordination would be necessary between power saving WI and 52.6-71 GHz WI. |
| Nokia, NSB | Q1: Yes Q2: we agree with Docomo that alignment with Power Saving WI is preferred. We may consider the values agreed there as the baseline for the LBT case as well. Q3: No |
| LG Electronics2 | We would like to ask one thing to clarify. Do the above mentioned timer values assume the same regardless of LBT or no LBT? We think that at least for LBT cases, the Timer value should be determined considering max COT duration in FR2-2. |
| Apple2 | For Q1/Q3, we would want to clarify that for licensed operation, things may be fine but for unlicensed operation, there may be an interaction between R16 format DCI 2-0 based SSSG switching and Rel-17 power-savings based SSSG switching that may need to be resolved. This could result in up to 5 SSs with a different default SS for Rel-16 and Rel-17. Our position is that if enabled, in an unlicensed band, both schemes should not be configured simultaneously. |
| Qualcomm2 | What is discussed in AI 8.7.2 is Rel-17 UE power saving feature, while what we have discussed in this AI is an extension of Rel-16 (NR-U) feature. As some other companies have also commented on the differentiation between LBT and non-LBT, we think we should not mix up the two cases. As such, the timer duration configuration should also be separate, as LGE commented above. Note that, the SSSG timer configuration of Rel-16 SSSG switching is per-cell, while the SSSG timer for Rel-17 SSSG switching is per-BWP. |

#### Summary of first round discussion

For Q1: Most companies support to extend SSSG switching timer and PDCCH skipping feature introduced in Rel-17 UE power saving enhancement WI for NR operation in FR2-2 with 480 and/or 960 kHz SCS. However Apple points out that DCI 2_0 based SSSG switching and Rel-17 power-savings based SSSG switching may be in conflict, so that both schemes should not be configured simultaneously. NTT DOCOMO points out that the skipping values and switching time values are still under discussion. LG is aksing for further clarification how LBT/no-LBT may affect the timer values. It is therefore suggested to continue discussion to at least resolve LG's and Apple's questions/concerns.

For Q2: Most companies support to simply scale the values of 120 kHz by 4 and 8 for 480 kHz and 960 kHz SCS. However, Samsung suggest to discuss further on the candidate values as there’re other options proposed. Intel and Ericsson proposed to take WA rather than agreement. FL suggests to take the scaling of 120 kHz values by 4 and 8 for 480 kHz and 960 kHz SCS as a baseline for continued discussion.

For Q3: most companies do not see any additional specification impact. Though there’re comments suggesting to confirm in UE power saving WI.

#### Second round discussion

Please continue the discussion esp. regarding comments/questions by LG and Apple, copied here for convenience.

| Company | Comment |
| --- | --- |
| LG Electronics2 (from 1st round) | We would like to ask one thing to clarify. Do the above mentioned timer values assume the same regardless of LBT or no LBT? We think that at least for LBT cases, the Timer value should be determined considering max COT duration in FR2-2. |
| Apple2 (from 1st round) | For Q1/Q3, we would want to clarify that for licensed operation, things may be fine but for unlicensed operation, there may be an interaction between R16 format DCI 2-0 based SSSG switching and Rel-17 power-savings based SSSG switching that may need to be resolved. This could result in up to 5 SSs with a different default SS for Rel-16 and Rel-17. Our position is that if enabled, in an unlicensed band, both schemes should not be configured simultaneously. |
| Intel | We share Apple’s view that NR-U based SSSG switching and Rel-17 PS based SSSG switching should not be configured simultaneously for unlicensed operation. |
| ZTE, Sanechips | For Q1: We are open to extend SSSG switching timer and PDCCH skipping feature in FR 2-2 but we think it is better to wait until the discussion is settled in Rel-17 power saving enhancement WI. For Q2: We agree with FL’s suggestion and make scaling of 120 kHz values by 4 and 8 for 480 kHz and 960 kHz SCS as a working assumption if further clarification can be made that “SSSG switching timer” here is assiociated with Rel-17 power-savings based SSSG switching. There is also a “SSSG switching timer” discussed under Issue A2-4 and it is introduced for Rel-16 NRU. FL proposed conclusion : SSSG switching timer is in units of slots For Q3:We do not see any additional specification impact for now and we are open for other companies’ suggestions. |
| NTT DOCOMO | We are fine to discuss LBT case and non-LBT case separately (non-LBT case may be decided in PowSav WI), but it may result in converged into one set of values. However, we think “LBT case” and “non-LBT case” may need clarification, e.g., whether non-LBT case includes unlicensed band operation with LBT turned off in addition to licensed band operation. |
| LG Electronics | We are open to discuss on adopting SSSG switching and PDCCH skipping feature introduced in Rel-17 UE power saving WI. However, before that, we should first focus on how to extend an SSSG switching feature from NR-U for 120/480/960 kHz SCS. For the timer value of SSSG switching in unlicensed band for FR2-2, we would like to echo our previous comment in 2.2.5.1 to clarify what timer value corresponding to the maximum COT duration is, as below. - The maximum value of searchSpaceSwitchTimer for NR-U is 20/40/80 for 15/30/60 kHz, respectively. These values are derived from the maximum COT duration, 20 msec. Now, since the maximum COT duration in FR2-2 was determined to be 5 msec, the maximum value of searchSpaceSwitchTimer can be defined as the number of slots corresponding to that time for each SCS, i.e., 40/160/320 slots for 120/480/960 kHz SCS, respectively. |
| Xiaomi | Q1: Agree with Apple that R16 SSSG switching and R17 SSSG switching should not be simultaneously applied. Not agree with LG’s view. If COT has ended but SSSG switching timer is still running, UE can simply not monitor the DL channels, or stop the timer if UE know where is the end of the COT. But it has no influence on the configured values or candidate values for SSSG timer. Q2: Agree with FL’s suggestion to take the scaling of 120 kHz values by 4 and 8 for 480 kHz and 960 kHz SCS as a baseline for continued discussion. |
| Futurewei | Q1: Agree with Apple that R16 and R17 SSSG switching should not be simultaneously applied. This can be further discussed in UE Features discussions. Q2:Agree with FL suggestion to take the scaling of 120kHz values as baseline for discussion. Q3: We do not see specifications impact. |
| vivo | Q1: W |
| LG Electronics2 | @Xiaomi: Thank you for your comments. The reason we proposed the maximum timer value (for U-band) corresponding to the maximum COT was to avoid the following inefficient situation. According to 213 spec, if (1) a UE is not provided SearchSpaceSwitchTrigger for a serving cell, and, (2) the UE monitors PDCCH for a serving cell according to search space sets with group index 1, and, (3) the UE is NOT provided a search space set to monitor PDCCH for detecting a DCI format 2_0, then UE can starts monitoring PDCCH for SSSG#0 and stops monitoring PDCCH for SSSG#1 only when the timer expires. At this time, if the UE is provided SearchSpaceSwitchTrigger as 160 slots (for 120 kHz), UE should monitor PDCCH according to SSSG#1 for about 15 msec (which is corresponding to 120 slots) even after the remaing CO duration is already over. We don’t think this is an efficient behavior in unlicensed band. However, since it may not be a serious problem from a different standpoint, we do not have a strong view on it and do not oppose to adopt the set of values {1,2,3,…,160, 240, 320,400, 480, 640,800}*4/8 for 480/960 kHz SCS, respectively. |

#### Summary of second round discussion

From the second round discussion most companies think that NR-U based SSSG switching and Rel-17 PS based SSSG switching should not be configured simultaneously for unlicensed operation. Several companies point out that in this WI, priority should be given to extending the SSSG swicthing feature from NR-U rather than harmonizing with power saving WI. Therefore it may be prudent to wait for further progress from the power saving WI discussion before deciding on an integration of these features with this WI, though some companies have shown interest in such an integration.

FL proposal 2.2.6.4:

Working assumption: NR-U based SSSG switching and Rel-17 PS based SSSG switching should not be configured simultaneously for unlicensed operation

Note: Discussion can continue after more progress in power saving WI, and should take the scaling of 120 kHz skipping/switching values by 4 and 8 for 480 kHz and 960 kHz SCS as a starting point for continued discussion.

## Topic A3: BD Budget/Dropping

Deprioritized for the time being as there seems to be no impact to other WGs – may be updated during RAN1#107bis-e.

In RAN1#107-e, the following agreement has been achieved:

Agreement - SS set overbooking can be allowed with multi-slot PDCCH monitoring capability same as the current specification but applied per slot group, i.e., SS set overbooking is allowed for USS in PCell and PSCell, and UE expects no overbooking for CSS in PCell and PSCell and no overbooking in SCell. - The dropping rule for multi-slot PDCCH monitoring capability is the same as the current specification but evaluated per slot group, i.e., a UE drops UE specific search space set(s) in a slot group with higher index when SS sets are overbooked. - Additional dropping rules are not precluded

## Topic B: Multi-Beam Aspects

### Issue B-1: Beam-specific indication in DCI format 2_0 (potential RRC impact)

#### First round discussion

vivo (R1-2200075):

Proposal 7: Specifying beam-specific COT indication in DCI 2_0 for NR operation from 52.6-71GHz when directional LBT is used.

Sony (R1-2200174):

Proposal 2: Support per beam indication of DCI format 2_0 for above 52 GHz unlicensed operation.

Nokia (R1-2200184):

Proposal 9: Beam-specific indication of remaining COT duration and search space group switching in DCI format 2_0 can be supported.

- Indicatation can be e.g. a bitmap of beam group indications, where UE’s TCI states are associated to a beam group via RRC signalling.

Samsung (R1-2200193):

Proposal 8: Support indicating COT, available RB set, and search space group switching in a beam-specific manner for 60 GHz licensed band.

Intel (R1-2200367):

Proposal 10:

- The DCI format 2_0 can be transmitted in a beam specific manner.

Apple (R1-2200410):

Proposal 9: Consider enhancement of DCI 2-0 transmission to signal COT duration and SS adaptation at the beginning of the COT.

Please state if and which of the following aspects shall be supported as a beam-specific indication in DCI format 2_0. Please also state for which of these you see a need for a corresponding RRC signal (e.g. field position within the DCI).

- Remaining COT duration – moved to AI 8.2.6

- Available RB set – moved to AI 8.2.6 (though several companies see no need to support this)

- Search space group switching

| Company | Comment |
| --- | --- |
| Ericsson | The problem definition and motivation seem a bit vague, and not essential either. We think this should be deprioritized, especially now that we are in maintenance. |
| MediaTek | We share the same view with Ericsson. |
| Intel | We prefer to conclude beam specific DCI 2_0 can be supported. FFS available RB set |
| Nokia, NSB | We think that beam-specific indication of remaining COT duration and search space group switching in DCI format 2_0 can be supported. Indication can be e.g. a bitmap indicator of beam groups served in the CO, where reference signals in UE’s PUCCH-SpatialRelationInfo or TCI-State_r17 are associated to a beam group via RRC signalling. We also think that this should be part of Channel Access -AI |
| vivo | We support to specify beam specific DCI 2_0 especially for COT duration. When directional LBT is used, COT will be specific to one or more beams for further UE behavior change, e.g. CSI-RS validation. If COT is obtained by gNB in beam 1, the CSI-RS in beam 2 should not be validated. In this sense, specifying beam specific COT duration is important. |
| Apple | Support this proposal. |
| InterDigital | We also support the enhancement to support beam specific indication of DCI format 2_0 especially for COT duration. |
| Samsung | We support beam-specific indication at least for “remaining COT indication” and “available RB set”. Open to discuss “search spacr group switching”. |
| OPPO | We support beam specific indication for remaining COT indication, available RB set and SSSG switching. |
| Futurewei | We think that multi-beam indication should be discussed in channel access AI together with Issue B-2 |
| NTT DOCOMO | It should be clarified which AI of 8.2.2 or 8.2.6 to handle this topic. |
| ZTE, Sanechips | Beam-specific DCI 2_0 should be discussed in the channel access AI. |
| Lenovo, Motorola Mobility | We support the proposal. We note that there seems to be a parallel item in channel access discussion. While for COT duration and RB set channel access AI may be suitable, we think SSSG switching should be discussed in the PDCCH enhancement context under 8.2.2. |
| Transsion | We support beam specific indication for “remaining COT duration” and “Search space set group switching”. Regarding RB set, since the LBT bandwidth is determined according to channel bandwidth or BWP bandwidth, we don’t see the necessity to introduce RB set in FR2-2. |
| Sharp | This discussion should be handled in AI 8.2.6. |
| LG Electronics | We share the view with Intel that the beam-specific DCI 2_0 can be supported in FR2-2. Details on how to configure each field of DCI 2_0 can be further discussed. BTW, it seems that the similar discussion is in Timeline AI, so any coordination may be needed. |
| Sony | We support beam specific DCI 2_0, and all three sub bullets are relevant. For the COT, a node could terminate some COT earlier than maximum COT duration when data transmission using the COT is completed if it is independent for each beam. In addition, information on SS set group switching should be also considered as per-beam information in this case since SS set group is switched depending on the state of COT. As for available RB set, available RB set should be per-beam because occupied bandwidth acquired by per-beam LBT sensing would be different. |
| Moderator | Per chairman guidance, COT aspects of this issue are moved to AI 8.2.6 (and should be treated there in RAN1#108e as well). Beam-specific SSSG switching will still be discussed in this AI. |

#### Summary of first round discussion

Some companies suggest to support a beam-specific SSSG switching indication, however apart from this high-level proposal only few details have been rovided so far. FL suggests to continue discussion in the second round to elaborate more on how to support beam-specific SSSG switching.

#### Second round discussion

Please continue discussion if and particularly how beam-specific SSSG switching should be supported. Especially if there is any RRC impact, we need to clarify this aspect as soon as possible.

| Company | Comment |
| --- | --- |
| Samsung | In general, we support this feature. The method for support beam-specific SSSG switching can be common for beam-specific COT and/or RB set indication (not sure whether it’s a good idea to separate the discussion). |
| Intel | We share Samsung’s view beam-specific SSSG switching can be discussed together with other parameters COT length, available RB set and SFI. |
| ZTE, Sanechips | We think beam-specific SSSG switching should also be discussed in AI 8.2.6. |
| NTT DOCOMO | We agree with moderator that more detailed specification impact need to be suggested. In addition, we share the same view with Samsung that the mechanism for beam-specific indication of SSSG sitching and COT duration/available RB set should be discussed together if supported. |
| LG Electronics | We share the view with Samsung. We support this feature and prefer to be supported in the same way as beam-specific COT. |
| Huawei/HiSilicon | Similar to beam-specific Remaining COT duration, we don’t see the need to support beam-based search space group switching. As SSSG switching is at least partly motivated by providing UE with two different MO periodicities inside and outside of the COT, it is not very well-justified to support beam-specific SSSG/Remaining COT duration and not support beam-specifc for Remaining COT duration/SSSG. Our reasons for not supporting beam-specific Remaining COT duration (or, in general, any content of DCI 2_0) are provided in Email discussion of 8.2.6 and copy-pasted below: We still don’t see why DCI 2_0 should identify the acquired beam to avoid the UE in a sidelobe of gNB that decodes DCI 2_0 not to share the COT. First, even if DCI 2_0 identifies the acquired beam and only UE that fall in the identified DL beam share the COT, there is no guarantee that its UL TX would correspond to the acquired DL beam. Second, if we are concerned with a Tx sidelobe, sensing beam has also a sidelobe. So, should we also devise a mechanism to make sure that the received energy from sidelobe of the LBT beam does not result in channel in the mainlobe direction to be sensed occupied? We hope that companies agree with us that we should not open all these “sidelobe” related issues because, at best, they are only marginal enhancement with a potentially huge specification impact.. Finally, a TCI-state to monitor Type3-PDCCH carrying Format 2_0 DCI is already indicated in MAC-CE to the UEs. In other words, network has already some control on which UEs can actually decode Format 2_0 DCI using the indicated TCI-state. We don’t see a value to additionally provide further TCI-state in the DCI payload to provide beam-specific information. |
| Nokia, NSB | We also support this feature. Beam indication can be e.g. a bitmap indicator of beam groups served in the COT, where reference signals in UE’s PUCCH-SpatialRelationInfo or TCI-State_r17 are associated to a beam group via RRC signalling. |
| Futurewei | We think that these discussions related to beam specific SSSG switching should be discussed in AI 8.2.6 |

#### Summary of second round discussion

Four companies have expressed support for beam-specific SSSG switching indication in DCI format 2_0, while one company does not see a need for such a support. Five companies expressed a preference to discuss the topic in conjunction with other beam-specific enhancements under AI 8.2.6.

FL suggestion 2.4.1.4: Discussion on beam-specific SSSG switching can continue under AI 8.2.6 in the next meeting.

### Issue B-2: Monitoring in CORESETs corresponding to specific beams

#### First round discussion

Oppo (R1-2200325):

Proposal 1: the UE can share gNB COT only if the UL transmission resources are within the gNB COT and the UL transmission beam is covered by the gNB sensing beam for directional LBT.

Proposal 2: For higher layer configured CSI-RS reception, the UE performs the reception if the CSI-RS resources are within the gNB COT and the gNB’s sensing beam covers the CSI-RS beam.

Proposal 3: In FR2-2 unlicensed band, the pre-configured downlink reception is not only confirmed by the SFI indication but also by gNB’s sensing beam, e.g., UE should cancel the downlink reception within the gNB COT if the gNB sensing beam does not cover the downlink transmission beam.

Proposal 4: R17 should allow UE to skip PDCCH monitoring in the CORESET associated with a beam uncovered by the gNB sensing beam within the gNB COT.

FL Note: Proposals may overlap with Channel Access AI and might be better discussed in that context.

| Company | Comment |
| --- | --- |
| Ericsson | It seems this is channel access related, and thus doesn't belong in this AI? |
| Intel | We agree this issue should be clarification and we are fine to handle it in channel access AI |
| Nokia, NSB | Agree with Ericsson |
| InterDigital | We also prefer to discuss this issue in channel access AI. |
| Samsung | Agree with FL’s note. |
| Futurewei | Agree with FL’s note. |
| Lenovo, Motorola Mobility | Fine to discuss under channel access agenda |
| Transsion | Agree with FL’s note. |
| LG Electronics | Agree with FL’s note. |
| Moderator | Per chairman's guidance, this discussion is closed here and moved to AI 8.2.6 (channel access) |

Lenovo (R1-2200671):

Proposal 4: For NR unlicensed bands between 52.6 GHz and 71 GHz with directional LBT based channel access mechanism, within a COT, PDCCH monitoring is not supported in the CORESETs corresponding to other COTs (PDCCH monitoring restricted to monitoring corresponding to only one COT at a time)

| Company | Comment |
| --- | --- |
| Ericsson | It seems this is channel access related, and thus doesn't belong in this AI? |
| Intel | Agree with Ericsson |
| Nokia, NSB | Agree with Ericsson |
| InterDigital | We also prefer to discuss this issue in channel access AI. |
| Samsung | Seems belong to channel access discussion. |
| Futurewei | We prefer to be discussed in the channel access AI. |
| Lenovo, Motorola Mobility | Fine to discuss under channel access agenda |
| LG Electronics | Same view with Ericsson. |
| Moderator | Per chairman's guidance, this discussion is closed here and moved to AI 8.2.6 (channel access) |

## Topic C: Multi-Cell Operation, Cross-carrier scheduling

### Issue C-1: MSM capability for multiple serving cells

#### First round discussion

vivo (R1-2200075):

Proposal 6: For multi-cell operation,  Support the following method to handle multi-slot PDCCH monitoring capability for multi-serving cell case, i.e. transform the serving cell with multi-slot-based capability to equivalent serving cell with slot-based capability for further BD/CCE budget calculation.

| Company | Comment |
| --- | --- |
| Intel | We understand the proposal is to share BD/CCE for a cell with SCS 120kHz, a cell with SCS 480kHz with combination (4, Ys) and a cell with SCS 960kHz with combination (8, Ys). We agree that it may be possible since the duration of slot/slot group are same, however, we wonder if there can be special considerations due to the different SCS. So more views/discussions are necessary |
| vivo | To handle multi-serving cell problem, the following two alternatives could be considered: Alt. 1: Serving cells with the same PDCCH monitoring type including multi-slot-based capability are grouped together for further BD/CCE budget calculation Alt. 2: Transform the serving cell with multi-slot-based capability to equivalent serving cell with slot-based capability for further BD/CCE budget calculation As indicated in our contribution, there are the following 3 cell types in NR Rel-17: - Cell Type 1 (FR1/FR2): Serving cell with slot-based PDCCH monitoring capability; - Cell Type 2 (FR1 only): Serving cell with span-based PDCCH monitoring capability; - Cell Type 3 (FR2-2 only): Serving cell with multi-slot-based PDCCH monitoring capability. In NR Rel-16, the following cases may occur for one UE where PDCCH monitoring capability related parameters is reported by UE for each case respectively. - Case 1: All serving cells belongs to cell type 1; - Case 2: All serving cells belongs to cell type 2; - Case 3: At least one serving cell belongs to cell type 1 and at least one serving cell belongs to cell type 2. In NR Rel-17, there may be more cases as listed below: - Case 4: All serving cells belongs to cell type 3; - Case 5: At least one serving cell belongs to cell type 1 and at least one serving cell belongs to cell type 3; - Case 6: At least one serving cell belongs to cell type 2 and at least one serving cell belongs to cell type 3; - Case 7: At least one serving cell belongs to cell type 1, at least one serving cell belongs to cell type 2 and at least one serving cell belongs to cell type 3. If Alt. 1 is adopted, many capability related parameters should be reported by UE for each of the above cases, which also requires large spec impact. Alt. 2 is preferred by us due to smaller spec impact. As Intel indicates, it is possible since the duration and BD/CCE budget of slot/slot group is the same. |
| Samsung | We are OK with the high level idea. The details of the proposed method was missing. |
| LG Electronics | We support the concept of treating serving cell with multi-slot-based capability as if they were cells with slot-based capability when calculating multi-cell BD/CCE distribution. We think it can be an efficient approach to avoid complicatedly handling all cases listed by Vivo above. |
| Huawei, HiSilicon | The two alternatives listed by vivo seem to be a good starting point for the discussion. We would like to see more details of Alt 2. If it turns functional, it seems to be an efficient way to avoid tedious discussions. One question regarding Alt 2 is that how a 960 kHz cell with SS sets corresponding to capability (4,1) or (4, 2) is transformed to a virtual 120 kHz cell with a per-slot based capability. |
| vivo | Thanks for the comments. Agree with Huawei that it is better to discuss the above two alternatives before going into TP discussion below. Here is a summary of more details on the above two alternatives: Alt. 1: Serving cells with the same PDCCH monitoring type including multi-slot-based capability are grouped together for further BD/CCE budget calculation - Alt. 1-1: Within a group of serving cells with multi-slot-based capability, the serving cells with the same SCS and (Xs, Ys) value are grouped together to follow a total BD/CCE budget - Alt. 1-2: Within a group of serving cells with multi-slot-based capability, the serving cells with the same SCS and Xs value are grouped together to follow a total BD/CCE budget Alt. 2: Transform the serving cell with multi-slot-based capability to equivalent serving cell with slot-based capability for further BD/CCE budget calculation - Alt. 2-1: A serving cell with SCS μ and multi-slot-based capability (Xs, Ys) is considered as an equivalent virtual cell with SCS μ-log2(Xs) and slot-based capability, where a slot group for the serving cell is considered as a slot for the virtual cell - Alt. 2-2: A serving cell with SCS μ and multi-slot-based capability is considered as an equivalent virtual cell with SCS μ=3 and slot-based capability, where 4/8 slots for the serving cell with SCS μ=5/6 is considered as a slot for the virtual cell For Alt. 2-1, it may need to define a 240KHz SCS virtual cell with slot-based capability, where BD/CCE budget is half of that for 120KHz； For Alt. 2-2, fixed number of slots are mapping to a slot for the virtual cell with 120KHz and slot-based capability irrespective of (Xs, Ys). @Huawei: Hopefully the above can answer your question. |

Samsung (R1-2200193):

Proposal 6: Adopt TP#4 for TS 38.213 to determine a capability to monitor a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs per Xs slots that corresponds to Ncellscap downlink cells, i.e.  Ncellscap-r17.

If a UE indicates in UE-NR-Capability a carrier aggregation capability larger than two downlink cells, the UE includes in UE-NR-Capability an indication for a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs that the UE can monitor per group of Xs slots according to combination Xs,Ys when the UE is configured for carrier aggregation operation over more than 2 downlink cells with monitoringCapabilityConfig = r17monitoringcapability. When a UE is not configured for NR-DC operation and the UE is provided monitoringCapabilityConfig = r17monitoringcapability for all downlink cell where the UE monitors PDCCH, the UE determines a capability to monitor a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs per span that corresponds to Ncellscap-r17 downlink cells, where - Ncellscap-r17 is the number of configured downlink cells if the UE does not provide pdcch-MonitoringCA - otherwise, Ncellscap-r17 is the value of pdcch-MonitoringCA When a UE is configured for NR-DC operation and the UE is provided monitoringCapabilityConfig = r17monitoringcapability for all downlink cells where the UE monitors PDCCH, the UE determines a capability to monitor a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs per group of Xs slots according to combination Xs,Ys that corresponds to - Ncellscap-r17= Ncells, r17MCG downlink cells for the MCG where Ncells, r17MCG is provided by pdcch-BlindDetection3 for the MCG, and - Ncellscap-r17= Ncells, r17SCG downlink cells for the SCG where Ncells, r17SCG is provided by pdcch-BlindDetection3 for the SCG When the UE is configured for carrier aggregation operation over more than 2 cells, or for a cell group when the UE is configured for NR-DC operation, the UE does not expect to monitor per group of Xs slots according to combination Xs,Ys a number of PDCCH candidates or a number of non-overlapped CCEs that is larger than the maximum number as derived from the corresponding value of Ncellscap-r17. When a UE is configured for NR-DC operation with a total of NNR-DCDL,cells downlink cells on both the MCG and the SCG and the UE is provided monitoringCapabilityConfig = r17monitoringcapability for all downlink cells where the UE monitors PDCCH, the UE expects to be provided pdcch-BlindDetection for the MCG and pdcch-BlindDetection for the SCG with values that satisfy - pdcch-BlindDetection3 for the MCG + pdcch-BlindDetection3 for the SCG <= pdcch-MonitoringCA, if the UE reports pdcch-MonitoringCA, or - pdcch-BlindDetection3 for the MCG + pdcch-BlindDetection3 for the SCG <= NNR-DCDL,cells, if the UE does not report pdcch-MonitoringCA When a UE is configured for NR-DC operation and the UE is provided monitoringCapabilityConfig = r17monitoringcapability for all downlink cells where the UE monitors PDCCH, the UE may indicate, through pdcch-BlindDetectionMCG-UE-r17 and pdcch-BlindDetectionSCG-UE-r17, respective maximum values for pdcch-BlindDetection for the MCG and pdcch-BlindDetection for the SCG. If the UE reports pdcch-MonitoringCA, - the value range of pdcch-BlindDetectionMCG-UE-r17 or of pdcch-BlindDetectionSCG-UE-r17 is [1, …, pdcch-MonitoringCA-1], and - pdcch-BlindDetectionMCG-UE-r17 + pdcch-BlindDetectionSCG-UE-r17 >= pdcch-MonitoringCA. Otherwise, if NNR-DC, max, r17DL,cells is a maximum total number of downlink cells for which the UE is provided monitoringCapabilityConfig = r17monitoringcapability and the UE is configured on both the MCG and the SCG for NR-DC as indicated in UE-NR-Capability - the value of pdcch-BlindDetectionMCG-UE-r17 or of pdcch-BlindDetectionSCG-UE-r17 is 1, - pdcch-BlindDetectionMCG-UE-r17 + pdcch-BlindDetectionSCG-UE-r17 >= NNR-DC, max, r17DL,cells.

| Company | Comment |
| --- | --- |
| Intel | Our understanding on the proposal is to determine M_total/C_total for cells with same SCS u, same X and same Y. However, we think mandating same Y is not necessary. Please refer to a more detailed clarification under Intel’s proposal. |
| vivo | Please see our comment above. In our understanding, this is proposing TP for Alt. 1. The proposed TP is only related to Case 4: All serving cells belongs to cell type 3. However, related spec for case 5-7 is also needed if Alt. 1 is adopted. |
| Samsung | We support the proposal as the proposing company. TP can be further discussed. To vivo: yes, this TP only covers the simplest case as the starting point, and should be further discussed for other cases. |
| LG Electronics | We have the same understanding with Intel. We also think that for a mixed situation of per-slot monitoring, per-span monitoring, and per-multi-slot monitoring, BD/CCE budget calculation per each (X,Y) may be too restrictive. |
| Ericsson | Please see our comments in the Table below Intel's proposal. |

Samsung (R1-2200193):

Proposal 7: Adopt TP#5 for TS 38.213 to determine maximum number of PDCCH candidates, MPDCCHtotal,(Xs,Ys),μ, and a maximum number of non-overlapped CCEs, CPDCCHtotal,(Xs,Ys),μ, per Xs slots in CA mode.

If a UE is configured only with Ncells,r17DL,μ downlink cells for which the UE is provided monitoringCapabilityConfig = r17monitoringcapability and with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cells using SCS configuration μ, and with Ncells,r17DL,(Xs,Ys),μ of the Ncells,r17DL,μ downlink cells using combination Xs,Ys for PDCCH monitoring, where μ=01Ncells,r17DL,μ>Ncellscap-r17, a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell, the UE is not required to monitor more than MPDCCHtotal,(X,Y),μ=Ncellscap-r17⋅MPDCCHmax,(Xs,Ys),μ⋅Ncells,r17DL,(Xs,Ys),μj=01Ncells,r17DL,j PDCCH candidates or more than CPDCCHtotal,(Xs,Ys),μ=Ncellscap-r17⋅CPDCCHmax,(Xs,Ys),μ⋅Ncells,r17DL,(Xs,Ys),μj=01Ncells,r17DL,j non-overlapped CCEs - per set of groups of Xs slots on the active DL BWP(s) of all scheduling cell(s) from the Ncells,r17DL,(Xs,Ys),μ downlink cells within every Xs slots where Ncells,r16DL,j is a number of configured cells with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cells using SCS configuration j. For each scheduled cell from the Ncells,r17DL,(X,Y),μ downlink cells using combination Xs,Ys, the UE is not required to monitor on the active DL BWP with SCS configuration μ of the scheduling cell, more than minMPDCCHmax,(Xs,Ys),μ,MPDCCHtotal,(Xs,Ys),μ PDCCH candidates or more than minCPDCCHmax,(Xs,Ys),μ,CPDCCHtotal,(Xs,Ys),μ non-overlapped CCEs per group of Xs slots.

| Company | Comment |
| --- | --- |
| Ericsson | Please see our comment below regarding Intel's Proposal 6. |
| MediaTek | We support the proposal which handle cells with different (X,Y) respectively. |
| Intel | Our understanding on the proposal is to determine M_total/C_total for cells with same SCS u, same X and same Y. However, we think mandating same Y is not necessary. Please refer to a more detailed clarification under Intel’s proposal. |
| vivo | Please see our comment above. In our understanding, this is proposing TP for Alt. 1. The proposed TP is only related to Case 4: All serving cells belongs to cell type 3. However, related spec for case 5-7 is also needed if Alt. 1 is adopted. |
| Samsung | We support the proposal as the proposing company. TP can be further discussed. |
| LG Electronics | We have the same understanding with Intel and Vivo. We also think that for a mixed situation of per-slot monitoring, per-span monitoring, and per-multi-slot monitoring, BD/CCE budget calculation per each (X,Y) may be too restrictive. |
| Ericsson | Please see our comments in the Table below Intel's proposal. |

Intel (R1-2200367):

Proposal 6:

- MPDCCHtotal,(X,Y),μ, CPDCCHtotal,(X,Y),μ needs to be determined and shared by all cells that are configured with scheduling cells having same SCS configuration μ and same value X in combinations (X, Y).

- Agree on TP 3 to determine MPDCCHtotal,(X,Y),μ, CPDCCHtotal,(X,Y),μ

If a UE is configured with Ncells,0DL,μ+Ncells,1DL,μ downlink cells for which the UE is provided monitoringCapabilityConfig = r17monitoringcapability and with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cells using SCS configuration μ, and with Ncells,0DL,(Xs,Ys),μ+Ncells,1DL,(Xs,Ys),μ of the Ncells,0DL,μ+Ncells,1DL,μ downlink cells using combination Xs,Ys with same Xs for PDCCH monitoring, where μ=06Ncells,0DL,μ+γ∙Ncells,1DL,μ≤Ncellscap, the UE is not required to monitor, per a group of Xs slots on the active DL BWPs of the scheduling cells, - more than MPDCCHtotal,(Xs,Ys),μ=MPDCCHmax,(Xs,Ys),μ PDCCH candidates or more than CPDCCHtotal,(Xs,Ys),μ=CPDCCHmax,(Xs,Ys),μ non-overlapped CCEs for each scheduled cell when the scheduling cell is from the Ncells,0DL,(Xs,Ys),μ downlink cells, or - more than MPDCCHtotal,(Xs,Ys),μ=γ∙MPDCCHmax,(Xs,Ys),μ PDCCH candidates or more than CPDCCHtotal,(Xs,Ys),μ=γ∙CPDCCHmax,(Xs,Ys),μ non-overlapped CCEs for each scheduled cell when the scheduling cell is from the Ncells,1DL,(Xs,Ys),μ downlink cells - more than MPDCCHmax,(Xs,Ys),μ PDCCH candidates or more than CPDCCHmax,(Xs,Ys),μ non-overlapped CCEs for CORESETs with same coresetPoolIndex value for each scheduled cell when the scheduling cell is from the Ncells,1DL,(Xs,Ys),μ downlink cells If a UE - is configured with Ncells,0DL,μ+Ncells,1DL,μ downlink cells for which the UE is provided monitoringCapabilityConfig = r17monitoringcapability and with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cells using SCS configuration μ, and - with Ncells,0DL,(Xs,Ys),μ+Ncells,1DL,(Xs,Ys),μ of the Ncells,0DL,μ+Ncells,1DL,μ downlink cells using combination Xs,Ys with same Xs for PDCCH monitoring, where μ=06Ncells,0DL,μ+γ∙Ncells,1DL,μ>Ncellscap, and - a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell, the UE is not required to monitor, per a group of Xs slots on the active DL BWP with SCS configuration μ of the scheduling cells - for the Ncells,0DL,(Xs,Ys),μ+Ncells,1DL,(Xs,Ys),μ, downlink cells more than MPDCCHtotal,(Xs,Ys),μ=Ncellscap⋅MPDCCHmax,(Xs,Ys),μ⋅Ncells,0DL,(Xs,Ys),μ+γ∙Ncells,1DL,(Xs,Ys),μj=06Ncells,0DL,(Xs,Ys),j+γ∙Ncells,1DL,(Xs,Ys),j PDCCH candidates or more than CPDCCHtotal,(Xs,Ys),μ=Ncellscap⋅CPDCCHmax,(Xs,Ys),μ⋅Ncells,0DL,(Xs,Ys),μ+γ∙Ncells,1DL,(Xs,Ys),μj=06Ncells,0DL,(Xs,Ys),j+γ∙Ncells,1DL,(Xs,Ys),j non-overlapped CCEs. - for each scheduled cell from the Ncells,0DL,(Xs,Ys),μ downlink cells, more than minMPDCCHmax,(Xs,Ys),μ,MPDCCHtotal,(Xs,Ys),μ PDCCH candidates or more than minCPDCCHmax,(Xs,Ys),μ,CPDCCHtotal,(Xs,Ys),μ non-overlapped CCEs. - for each scheduled cell from the Ncells,1DL,(Xs,Ys),μ downlink cells, - more than minγ∙MPDCCHmax,(Xs,Ys),μ,MPDCCHtotal,(Xs,Ys),μ PDCCH candidates or more than minγ∙CPDCCHmax,(Xs,Ys),μ,CPDCCHtotal,(Xs,Ys),μ non-overlapped CCEs - more than minMPDCCHmax,(Xs,Ys),μ,MPDCCHtotal,(Xs,Ys),μ PDCCH candidates or more than minCPDCCHmax,(Xs,Ys),μ,CPDCCHtotal,(Xs,Ys),μ non-overlapped CCEs for CORESETs with same coresetPoolIndex value

| Company | Comment |
| --- | --- |
| Ericsson | We agree that it is essential do define MPDCCHtotal,(Xs,Ys),μ CPDCCHtotal,(Xs,Ys),μ since these quantities are used in 38.213 but not yet defined. Not sure about the actual TP from Intel, but maybe it's more productive to agree on a high level principle, and then leave the actual TP to the spec editor. We agree with the high level principle proposed by Intel, specifically: - MPDCCHtotal,(Xs,Ys),μ, CPDCCHtotal,(Xs,Ys),μ are determined such that the total is shared by all cells having same SCS configuration μ and same value Xs in combination (Xs,Ys). In this way, we think the legacy Rel-15 principle can be reused: - If the number of cells configured with combination of Xs,Ys is no more than Ncellscap, MPDCCHtotal,(Xs,Ys),μ=MPDCCHmax,Xs,Ys,μ and CPDCCHtotal,(Xs,Ys),μ=CPDCCHmax,Xs,Ys,μ. - If the number of cells configured with combination Xs,Ys is more than than Ncellscap, the number of blind decodes Ncellscap⋅MPDCCHmax,Xs,Ys,μ is distributed to MPDCCHtotal,(Xs,Ys),μ based on a weighting of number of cells with numerology μ among all cells configured with combination Xs,Ys. Similarly, the total number of available channel estimation CCEs Ncellscap⋅CPDCCHmax,Xs,Ys,μ is distributed to CPDCCHmax,Xs,Ys,μ based on a weighting of number of cells with numerology μ among all cells configured with combination Xs,Ys. With this approach, both quantities are still upper bounded by MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ, respectively. |
| MediaTek | We don’t support the proposal which handle cells with same X but different Y together. |
| Intel | Based on the existing agreements - Value of max BD/CCE is only impacted by value Xs not value Ys - Though there is a limitation of Ys slots for group (1) SS, however there is no limitation on group (2) SS at all in a slot group. - Further, the BD/CCE for Type0 CSS or other group (2) SS can be quite larger. for example, it could be 28 CCEs for type0 CSS which is almost the whole capability of 32 CCEs in a slot group Therefore, we believe it is not necessary to concern about position of Ys slots in a slot group. in fact, it is even simpler to only consider SCS u and value X in M_total/C_total determination. |
| vivo | Please see our comment above. In our understanding, this is proposing TP for Alt. 1. The proposed TP is only related to Case 4: All serving cells belongs to cell type 3. However, related spec for case 5-7 is also needed if Alt. 1 is adopted. |
| Samsung | We agree with the issue identified in this proposal (actually same as our proposals), and detailed TP can be discussed further. |
| NTT DOCOMO | We agree with the first bullet of proposal 6, and also we support Ericsson’s suggestion that we should agree on a high level principle and reuse that in Rel-15. |
| Sharp | We support the first bullet of proposal 6. |
| LG Electronics | We agree that TP should be considered after generic consensus takes precedence. In our understanding, Intel proposed BD/CCE budget needs to be shared by all cells having same SCS and X. However, as commented at vivo’s proposal above, we support the concept of treating serving cell with multi-slot-based capability as if they were cells with slot-based capability when calculating multi-cell BD/CCE distribution. That is to say, BD/CCE budget needs to be shared by all cells having same monitoring time unit. For example, BD/CCE budget of all cells with X=8 for 960 kHz can be shared with that for 120 kHz. |

Ericsson (R1-2200401):

MPDCCHtotal,(Xs,Ys),μ and CPDCCHtotal,(Xs,Ys),μ should be defined in TS 38.213 following the same PDCCH monitoring capability allocation for carrier aggregation cases for the case of per-slot monitoring.

| Company | Comment |
| --- | --- |
| Ericsson | Please see our comment above regarding Intel's Proposal 6. |
| Samsung | We support the proposal in general, and TP can be further discussed. |

Xiaomi (R1-2200459):

Proposal 1: When UE is configured with multiple serving cells, within each serving cell, UE apply multi-slot PDCCH monitoring capability for a single serving cell independently.

13 UE procedure for monitoring Type0-PDCCH CSS sets If during cell search a UE determines from MIB that a CORESET for Type0-PDCCH CSS set is present, as described in clause 4.1, the UE determines a number of consecutive resource blocks and a number of consecutive symbols for the CORESET of the Type0-PDCCH CSS set from controlResourceSetZero in pdcch-ConfigSIB1, as described in Tables 13-1 through 13-10, for operation without shared spectrum channel access in FR1 and FR2-1, or as described in Tables 13-1A and 13-4A for operation with shared spectrum channel access in FR1, or as described in Tables 13-10A, 13-10B and 13-10C for FR2-2, and determines PDCCH monitoring occasions from searchSpaceZero in pdcch-ConfigSIB1, included in MIB, as described in Tables 13-11 through 13-15. SFNc and nc are the SFN and slot index within a frame of the CORESET based on SCS of the CORESET and SFNSSB,i and nSSB,i are the SFN and slot index based on SCS of the CORESET, respectively, where the SS/PBCH block with index i overlaps in time with system frame SFNSSB,i and slot nSSB,i. The symbols of the CORESET associated with pdcch-ConfigSIB1 in MIB or with searchSpaceSIB1 in PDCCH-ConfigCommon have normal cyclic prefix. *<omitted text>*. For operation without shared spectrum channel access and for the SS/PBCH block and CORESET multiplexing pattern 1, for FR1 and FR2-1, a UE monitors PDCCH in the Type0-PDCCH CSS set over two consecutive slots starting from slot . For FR2, UE monitors PDCCH in the Type0-PDCCH CSS set over two slots, slot and , where if SCS of the CORESET for Type0-PDCCH CSS is 480kHz, and if SCS of the CORESET for Type0-PDCCH CSS is 960kHz. For SS/PBCH block with index , the UE determines an index of slot as that is in a frame with system frame number (SFN) satisfying if , or in a frame with SFN satisfying if . and are provided by Tables 13-11 and 13-12, and based on the SCS for PDCCH receptions in the CORESET [4, TS 38.211]. The index for the first symbol of the CORESET in slots and and is the first symbol index provided by Tables 13-11 and 13-12. For operation with shared spectrum channel access and for the SS/PBCH block and CORESET multiplexing pattern 1, a UE monitors PDCCH in the Type0-PDCCH CSS set over slots that include Type0-PDCCH monitoring occasions associated with SS/PBCH blocks that are quasi co-located with the SS/PBCH block that provides a CORESET for Type0-PDCCH CSS set with respect to average gain, quasi co-location 'typeA' and 'typeD' properties, when applicable [6, TS 38.214]. For FR1 and FR 2-1, for a candidate SS/PBCH block index i, where 0≤i≤Lmax-1, two consecutive slots starting from slot n0 include the associated Type0-PDCCH monitoring occasions. For FR2, for a candidate SS/PBCH block index i, where 0≤i≤Lmax-1, two slots, slot n0 and , where if SCS of the CORESET for Type0-PDCCH CSS is 480kHz, and if SCS of the CORESET for Type0-PDCCH CSS is 960kHz, include the associated Type0-PDCCH monitoring occasions. The UE determines an index of slot n0 as n0=O⋅2μ+i⋅M mod Nslotframe, μ that is in a frame with system frame number (SFN) SFNC satisfying SFNC mod 2=0 if O⋅2μ+i⋅M/Nslotframe, μ mod 2=0, or in a frame with SFN satisfying SFNC mod 2=1 if O⋅2μ+i⋅M/Nslotframe, μ mod 2=1. M and O are provided by Table 13-11, and μ∈{0, 1} based on the SCS for PDCCH receptions in the CORESET [4, TS 38.211]. The index for the first symbol of the CORESET in slots n0 and n0+1 is the first symbol index provided by Table 13-11. The UE does not expect to be configured with M=1/2, or with M=2, when NSSBQCL=1. For the SS/PBCH block and CORESET multiplexing patterns 2 and 3, a UE monitors PDCCH in the Type0-PDCCH CSS set over one slot with Type0-PDCCH CSS set periodicity equal to the periodicity of SS/PBCH block. For the SS/PBCH block and CORESET multiplexing patterns 2 and 3, if the active DL BWP is the initial DL BWP, the UE is expected to be able to perform radio link monitoring, as described in Clause 5, and measurements for radio resource management [10, TS 38.133] using a SS/PBCH block that provides a CORESET for Type0-PDCCH CSS set. For a SS/PBCH block with index , the UE determines the slot index and based on parameters provided by Tables 13-13 through 13-15.

| Company | Comment |
| --- | --- |
| Samsung | We fail to understand the TP is associated with the proposal. Seems clarification is needed. |
| LG Electronics2 | We also feel that the proponent may need clarifications about the relationship between the proposal and the TP. |

MediaTek (R1-2200540):

Proposal 2: The Rel-15/16 multi-cell BD/CCE budget calculation method specified for slot-based PDCCH monitoring should be considered as the baseline for the multi-cell BD/CCE budget calculation for slot-group based PDCCH monitoring in 480kHz and 960kHz.

Proposal 3: For multi-cell operation, UE can report a capability on whether the location of the Y slots within a slot group of X slots is maintained across CCs associated with (X,Y) configuration.

| Company | Comment |
| --- | --- |
| MediaTek | We support the proposal. |
| Nokia, NSB | We don’t support Proposal 3. We think that location of Y slots within X slots should be flexible between CCs. |
| Samsung | We support the proposal in general, and TP can be further discussed. |
| Huawei, HiSilicon | We don’t see the necessity for Proposal 2. Aligning SS sets in different CCs in Proposal 3 for the sake of power saving does not seem to be well-justified. Such SS alignment across CCs could have been applied for slot based and/or span based in Rel-15/Rel-16 as well if found necessary but such a capability/mechanism was not introduced. |
| LG Electronics2 | We support the Proposal 2. And, we are open to further discuss on Proposal 3. |

LG (R1-2200565):

Proposal #8: When a UE is configured with DL cells greater than the reported number of DL cells, consider followings for BD/CCE budget calculation,

  - For a serving cell with mandatory X (i.e., X=4/8 for 480/960 kHz), BD/CCE budget is calculated by transforming the serving cell to the cell with 120 kHz SCS.

  - For a serving cell with optional X (e.g., X=4 for 960 kHz), BD/CCE budget is calculated by grouping the serving cell with other cells having the same PDCCH monitoring type including multi-slot-based capability.

| Company | Comment |
| --- | --- |
| Samsung | Clarification is needed on the wording “transforming the serving cell to the cell with 120 kHz SCS”. |
| LG Electronics | The first bullet in above proposal has a similar concept to a Vivo’s proposal Alt-2. For X=4/8 for 480/960 kHz, the absolute time of the slot-group and the BD/CCE limit per X=4/8 are the same as those for 120 kHz. In this case, when the multi-slot monitoring is configured with multiple serving cells with X=4/8 slots for 480/960 kHz, the BD/CCE limit calculation for cells with these SCS can be calculated as if they are cells with 120 kHz SCS. However, for X=4 for 960 kHz, separate BD/CCE calculation may be required since the length of slot-group and BD/CCE limit of X=4 slots are not the same as for 120 kHz. The second bullet provides a way to do this. |
| Huawei, HiSilicon | Bullet 2 is not very clear. Does it mean two 960 kHz cells with X=4, 1) are merged to a single 960 kHz of X=8 and then 2) further transferred to a slot-based 120 kHz cell? What happens if there is only a single 960 kHz cell with X=4? |
| LG Electronics2 | @Huawei: Thanks for asking the question. But, that's not our intention. Serving cells with X=4 for 960kHz requires the BD/CCE budget to be calculated in units of X=4 slots, not related to 120kHz cells. Our basic idea is to share the BD/CCE budget by the same unit of time. For example, serving cells with X=4 for 480kHz, cells with X=8 for 960kHz, and cells with 120kHz all share the same BD/CCE budget (This is the content of the first bullet). The second bullet means to share the BD/CCE budget by grouping all serving cells with optional Xs. In other words, it would mean that for 960 kHz SCS, only serving cells with X=4 share the BD/CCE budget. If X=2 for 480kHz is adopted to support, it would means that all serving cells with X=4 for 960kHz and with X=2 for 480kHz share the BD/CCE budget as a group. |

#### Summary of first round discussion

There is definitely a need to define MPDCCHtotal,(Xs,Ys),μ and CPDCCHtotal,(Xs,Ys),μ and some companies have expressed a desire to re-use the principle of Rel-15. At the same time, several companies show interest in further discussing based on vivo's detailed explanation in the first round. FL therefore suggest to continue discussion in the second round.

#### Second round discussion

Please continue discussion based on the contributions and comments in the first round. Vivo's and Ericsson's comments have been copied to serve as a starter.

| Company | Comment |
| --- | --- |
| vivo (from 1st round) | Thanks for the comments. Agree with Huawei that it is better to discuss the above two alternatives before going into TP discussion below. Here is a summary of more details on the above two alternatives: Alt. 1: Serving cells with the same PDCCH monitoring type including multi-slot-based capability are grouped together for further BD/CCE budget calculation - Alt. 1-1: Within a group of serving cells with multi-slot-based capability, the serving cells with the same SCS and (Xs, Ys) value are grouped together to follow a total BD/CCE budget - Alt. 1-2: Within a group of serving cells with multi-slot-based capability, the serving cells with the same SCS and Xs value are grouped together to follow a total BD/CCE budget Alt. 2: Transform the serving cell with multi-slot-based capability to equivalent serving cell with slot-based capability for further BD/CCE budget calculation - Alt. 2-1: A serving cell with SCS μ and multi-slot-based capability (Xs, Ys) is considered as an equivalent virtual cell with SCS μ-log2(Xs) and slot-based capability, where a slot group for the serving cell is considered as a slot for the virtual cell - Alt. 2-2: A serving cell with SCS μ and multi-slot-based capability is considered as an equivalent virtual cell with SCS μ=3 and slot-based capability, where 4/8 slots for the serving cell with SCS μ=5/6 is considered as a slot for the virtual cell For Alt. 2-1, it may need to define a 240KHz SCS virtual cell with slot-based capability, where BD/CCE budget is half of that for 120KHz； For Alt. 2-2, fixed number of slots are mapping to a slot for the virtual cell with 120KHz and slot-based capability irrespective of (Xs, Ys). |
| Ericsson (from 1st round) | We agree that it is essential do define MPDCCHtotal,(Xs,Ys),μ CPDCCHtotal,(Xs,Ys),μ since these quantities are used in 38.213 but not yet defined. Not sure about the actual TP from Intel, but maybe it's more productive to agree on a high level principle, and then leave the actual TP to the spec editor. We agree with the high level principle proposed by Intel, specifically: - MPDCCHtotal,(Xs,Ys),μ, CPDCCHtotal,(Xs,Ys),μ are determined such that the total is shared by all cells having same SCS configuration μ and same value Xs in combination (Xs,Ys). In this way, we think the legacy Rel-15 principle can be reused: - If the number of cells configured with combination of Xs,Ys is no more than Ncellscap, MPDCCHtotal,(Xs,Ys),μ=MPDCCHmax,Xs,Ys,μ and CPDCCHtotal,(Xs,Ys),μ=CPDCCHmax,Xs,Ys,μ. - If the number of cells configured with combination Xs,Ys is more than than Ncellscap, the number of blind decodes Ncellscap⋅MPDCCHmax,Xs,Ys,μ is distributed to MPDCCHtotal,(Xs,Ys),μ based on a weighting of number of cells with numerology μ among all cells configured with combination Xs,Ys. Similarly, the total number of available channel estimation CCEs Ncellscap⋅CPDCCHmax,Xs,Ys,μ is distributed to CPDCCHmax,Xs,Ys,μ based on a weighting of number of cells with numerology μ among all cells configured with combination Xs,Ys. With this approach, both quantities are still upper bounded by MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ, respectively. |
| Intel | Thanks vivo providing this summary of Alternatives. Alt 1 is to only share BD/CCE for cells with same SCS, while Alt 2 allows BD/CCE sharing for cells with different SCS. Though Alt 2 can share BD/CCE for a large group of cells, we are afraid whether it introduce additional complexity since UE has to coordinate the PDCCH decoding of cells with different SCSs. Therefore, Alt 1 is our preference for simplicity. Then, we prefer Alt 1-2 for its simplicity without any real performance loss. Based on the existing agreements, a Group (2) SS can be configured anywhere in a slot group. That is, even when we enforce same value of Ys, or even same position of the Ys slots, the PDCCH MOs are still not aligned due to free configuration of Group (2) SS. Note: the BD/CCE for Group (2) SS can be quite larger, e.g., 28 CCEs, which dominates the allocation of BD/CCE in a slot group. Therefore, we believe checking same Xs is enough in the BD/CCE handing. |
| ZTE, Sanechips | We prefer Alt 1-2 in vivo’s comment. Since the value of Ys in (Xs, Ys) does not impact the maximum BD/CCE budget, the serving cells with the same SCS and Xs value are grouped together to follow a total BD/CCE budget. |
| LG Electronics | Our first preference is Alt 2-1 and second is Alt 1-2 from vivo’s comment above. But, we do not support Alt 1-1 and Alt 2-2. In the first round discussion, Vivo kindly listed up several cases (Case1 – Case7) of configurations of multiple serving cells with different monitoring capabilities. Among them, Case7 may be the most complicated case. With Alt 1-1, there may be too many group of serving cells sharing the same BD/CCE budget for the Case7, since Alt 1-1 separates both the SCS and (X,Y) combinations to determine the BD/CCE budget respectively. We think that for a mixed situation of per-slot monitoring, per-span monitoring, and per-multi-slot monitoring, BD/CCE budget calculation per each (X,Y) may be too restrictive. On the other hand, Alt 1-2 can reduce the number of groups of serving cells sharing the BD/CCE budget since Alt 1-2 separates only X regardless of Y. Moreover, Alt 2-1 can further simplify the situation since BD/CCE budget could be shared according to the time unit (i.e., time duration of monitoring occasion) instead of SCS and/or X. |
| Futurewei | Our preference is Alt1-2 in vivo’s comment. |
| vivo | Our preference is Alt 2. As indicated in our first round comments and also mentioned by LG, the following additional cases should be handled for Alt. 1: - Case 4: All serving cells belongs to cell type 3; - Case 5: At least one serving cell belongs to cell type 1 and at least one serving cell belongs to cell type 3; - Case 6: At least one serving cell belongs to cell type 2 and at least one serving cell belongs to cell type 3; - Case 7: At least one serving cell belongs to cell type 1, at least one serving cell belongs to cell type 2 and at least one serving cell belongs to cell type 3. If Alt. 1 is adopted, many capability related parameters should be reported by UE for each of the above cases, which requires large spec impact. Based on this, Alt. 2 is slightly preferred by us due to smaller spec impact. |
| Huawei, HiSilicon | @vivo Very much appreciated for your input. Regarding Alt 1-2, as mentioned by ZTE, since the value of Ys in (Xs, Ys) does not impact the maximum BD/CCE budget, there’s no need to calculate the BD/CCE budgets of serving cells with the same SCS and Xs yet different Ys separately. Hence, Alt 1-2 is least preferred. In addition, does Alt 1 means that the BD/CCE for the same SCS but different Xs would be counted separately or there would be “further calculation” as mentioned at the beginning? If the different cases are counted separately, it might be a burden on the UE. What’s the benefit of this comparing to Alt 2? |

#### Summary of second round discussion

vivo provided the following set of alternatives for calculating the BD/CCE budget, where FL has added indicated show or lack of support:

Alt. 1: Serving cells with the same PDCCH monitoring type including multi-slot-based capability are grouped together for further BD/CCE budget calculation

- Alt. 1-1: Within a group of serving cells with multi-slot-based capability, the serving cells with the same SCS and (Xs, Ys) value are grouped together to follow a total BD/CCE budget

  - Supported by:

  - Not supported by: LG

- Alt. 1-2: Within a group of serving cells with multi-slot-based capability, the serving cells with the same SCS and Xs value are grouped together to follow a total BD/CCE budget

  - Supported by: Intel, ZTE, Sanechips, LG (2nd preference), Futurewei

  - Not supported by: Huawei

Alt. 2: Transform the serving cell with multi-slot-based capability to equivalent serving cell with slot-based capability for further BD/CCE budget calculation

- Alt. 2-1: A serving cell with SCS μ and multi-slot-based capability (Xs, Ys) is considered as an equivalent virtual cell with SCS μ-log2(Xs) and slot-based capability, where a slot group for the serving cell is considered as a slot for the virtual cell

  - Supported by: vivo

  - Not supported by:

- Alt. 2-2: A serving cell with SCS μ and multi-slot-based capability is considered as an equivalent virtual cell with SCS μ=3 and slot-based capability, where 4/8 slots for the serving cell with SCS μ=5/6 is considered as a slot for the virtual cell

  - Supported by: vivo

  - Not supported by: LG

Overall, FL thinks that more discussion is necessary.

FL suggestion 2.5.1.4: Continue discussion on MSM capability for multiple serving cells in the next meeting.

### Issue C-2: Cross-carrier scheduling limitations by SCS difference

FL Note: Discussion in earlier meetings has not shown consensus to introduce a cross-carrier scheduling limitation as a function of |μPDCCH − μPDSCH|

#### First round discussion

CATT (R1-2200143):

Proposal 6：In order to better support cross-carrier scheduling of the new SCS, i.e. 480 kHz and 960 kHz, the difference of SCS of cross-carrier scheduling should not be limited.

NTT DOCOMO (R1-2200227):

Proposal 2: To support cross-carrier scheduling of a cell within FR2-2 band from/to a cell of any FR1/FR2-1 band with different numerologies, any SCS combinations for PDCCH and PDSCH should be supported.

ZTE (R1-2200260):

Proposal 8: Cross-carrier scheduling of a cell within 52.6-71 GHz band from/to a cell within FR1/FR2-1 is supported and a UE supporting 480 kHz and 960 kHz SCS supports cross-carrier scheduling.

Qualcomm (R1-2200260):

Proposal 7: Cross-carrier scheduling of a cell in FR2-2 from/to a cell in FR1 or FR2-1 is supported.

- Potential limitation for the values of |μPDCCH − μPDSCH| is up to UE’s capability.

Intel (R1-2200367):

Proposal 9:

- CCS is only support for |μPDCCH − μPDSCH | ≤ k with k=5

- The limitation on |μPDCCH − μPDSCH | ≤ k, k=5 should be captured in UE feature for WI NR_ext_to_71GHz

Apple (R1-2200410):

Proposal 6: for cross-carrier scheduling, the max number of CCs that can be scheduled from a single CC is reported as UE capability.

| Company | Comment |
| --- | --- |
| Ericsson | We don't think there should be an artificial limitation on SCS difference. RAN4 is defining FR1 + FR2-2 band combinations, so in principle, the SCS difference ¦μPDCCH − μPDSCH¦ could be as large as 6, i.e., for 15 kHz PDCCH and 960 kHz PDSCH. If the FR1 SCS is 30 kHz, the difference would be 5. |
| MediaTek | We support to reuse the same limitation from FR1+FR2-1, i.e., k=3. For larger k, the processing timeline will be considerably large, which is not desirable. At least k=3 should be the basic UE capability and we are open to discuss more advanced UE capability. |
| Qualcomm | We support that ¦μPDCCH − μPDSCH¦ is up to UE’s capability. Since it has impact on UE complexity, it would be fair to leave it up to complexity. |
| Intel | It is questionable how a scheduling cell of 15kHz can flexibly schedule a cell of 960kHz, especially if the number of DCIs per slot on the scheduling cell is not increased. Therefore, it would be fine to define a limitation on max ¦μPDCCH − μPDSCH ¦. We prefer to discuss the issue here. If the principle can be agreeable, an exact value of k can be discussed in UE feature session. |
| Nokia, NSB | We also think that the SCS difference should not be limited. If cross carrier scheduling is supported, it is natural to support it between all SCSs involved. Hence, we don’t see a need for additional UE capability here. |
| vivo | WE support it is up to UE capability |
| Apple | We support that the difference should be limited and is based on UE capability. We also support that the maximum # of CCs that can be scheduled from a single CC should be a UE capability. |
| InterDigital | We do not support introducing a limitation based on SCS difference, however, we are open to discuss if it is based on UE capability. |
| Xiaomi | the SCS difference ¦μPDCCH − μPDSCH¦ for cross carrier scheduling can be defined as a UE capability. |
| Samsung | We support cross-carrier scheduling, but didn’t see a strong need to introduce a restriction on the values of ¦μPDCCH − μPDSCH¦, and we would like to understand the fundamental concern on why such restriction is needed. |
| NTT DOCOMO | Regarding CCS, our preference is no limitation on SCSs for PDCCH/PDSCH to support cross-carrier scheduling of a cell within FR2-2 band from/to a cell of any FR1/FR2-1 band with different numerologies. |
| ZTE, Sanechips | We support cross-carrier scheduling of a cell within 52.6-71 GHz band from/to a cell within FR1/FR2-1 and it is unnecessary to introduce a UE capability to limit the value of ¦μPDCCH − μPDSCH ¦. |
| Transsion | We support ¦μPDCCH − μPDSCH ¦ is up to UE’s capability. |
| Sharp | We do not see the need to set limits on SCS difference. |
| LG Electronics | We agree with Nokia that there is no need to restrict the SCS difference. |
| Huawei, HiSilicon | We support Proposal 2 from R1-2200227. |

#### Summary of first round discussion

The situation compared to earlier discussion is fundamentally the same. While there is no consensus to introduce a generic limitation on cross-carrier scheduling based on the SCS difference, several companies support a corresponding UE capability signalling.

FL suggestion: Continue discussion on cross-carrier scheduling limitations based on SCS difference in the context/agenda of UE capabilities for 52.6-71 GHz.

#### Second round discussion

Samsung notes the following regarding support of cross-carrier scheduling:

If supporting CA and cross-carrier scheduling for 480/960 kHz is a consensus from the group, we would request at least a conclusion on this such that the editors can start the corresponding specification work. At least in TS 38.213, there is a note by the editor asking whether CA is supported for 480/960 kHz, and the specification is not complete yet regarding this point

Therefore, FL suggests to take the following conclusion:

Proposed conclusion:

- Cross-carrier scheduling to and from cells operating in FR2-2 is supported

- Potential indications of UE capability related to a limited support of cross-carrier scheduling e.g. as a function of |μPDCCH − μPDSCH| can be discussed as part of the UE capability discussion

Please comment whether you support the proposed conclusion.

| Company | Comment |
| --- | --- |
| Lenovo, Motorola Mobility | Support the conclusion |
| Apple | We are fine with the conclusion. |
| CATT | We are not sure we can confidently declare that cross-carrier scheduling is supported for FR2-2, unless there’s no major specification effort. This is the maintenance phase anyway and we should prevent any risk of running out of our time. |
| Samsung | We support the conclusion. |
| Ericsson | Our assumption is that unless there is a technical problem in specifications, a particular feature can be viewed as supported unless stated otherwise. Perhaps it is more productive to identify what technical problems exist in the current specification regarding cross carrier scheduling? @Samsung: I understand that Samsung raised the potential issue that neither carrier aggregation nor cross carrier scheduling are supported currently, and this sounds quite a serious problem. Could Samsung elaborate on what specification text is missing or problematic to lead to this conclusion? |
| Intel | We support the conclusion. |
| ZTE, Sanechips | We are fine with the proposed conclusion but still our preference is not introducing a UE capability to limit the value of ¦μPDCCH − μPDSCH ¦. |
| NTT DOCOMO | We are fine with the conclusion. |
| LG Electronics | We support the proposed conclusion. |
| Huawei, HiSilicon | We can support the conclusion |
| Xiaomi | Support the conclusion. |
| Nokia, NSB | We support this conclusion |
| Futurewei | Support the conclusion. |
| Samsung2 | Response to Ericsson: At least in TS 38.213, when specifying multi-carrier operation and cross-carrier scheduling, many descriptions use “monitoringCapabilityConfig=XXX” to define the UE behavior, but it’s not applicable for 480/960 kHz SCS (not matter we support that RRC parameter or not). In this sense, we believe at least some wording improvement (e.g. adding a Rel-17 value of such RRC parameter and update the text, or use u=5 or 6) is needed to make sure the corresponding text for multi-carrier operation and cross-carrier scheduling is applicable for 480/960 kHz SCS (if we remember correctly, editor left a note in the agreed CR from last meeting regarding this point). |

#### Summary of second round discussion

Most companies express support of the FL suggestion. CATT points out that the support of cross-carrier scheduling should not be concluded before being confident that there is no major specification effort. Ericsson points out that unless there is a technical problem in specifications, a particular feature (i.e. cross-carrier scheduling to and from FR2-2) can be viewed as supported unless stated otherwise. FL shares Ericsson's understanding.

FL suggestion 2.5.2.4:

Conclude that potential indications of UE capability related to a limited support of cross-carrier scheduling e.g. as a function of |μPDCCH − μPDSCH| can be discussed as part of the UE capability discussion.

# Contribution Details

The following sections show extracted discussion and proposals from the contributions submitted to this AI, by a pure subjective decision by the FL.

## Topic A1: Blind Decoding Capability, Multi-slot monitoring and corresponding (X,Y) values

List of issues, proposals, and suggestions for handling in the email discussion phase.

### R1-2200045 (Huawei, HiSilicon)

Furthermore, to enable the configuration of the Rel-17 PDCCH monitoring capability, value r17monitoringcapability (already used in 38.213 Endorsed CR) should be added to monitoringCapabilityConfig. - Value r17monitoringcapability should be added to monitoringCapabilityConfig.

### R1-2200075 (vivo)

Based on the above agreement, it is obviously that multiple (Xs, Ys) values may be reported by UE, i.e. - 480KHz PDCCH: (4, 1) mandatory, (4, 2) optional - 960KHz PDCCH: (8, 1) mandatory, (8, 4), (4, 2) and (4, 1) optional If a UE indicates a capability to monitor PDCCH according to multiple Xs,Ys combinations, how to determine the value of (Xs, Ys) for a scheduling cell should be specified. First, UE should determine a set of (Xs, Ys) values according to which the search space configuration meets the limitation, i.e. configured Group (1) SSs are located within Y consecutive slots within a slot group of X slots where the location of the Y consecutive slots within the slot group of X slots is maintained across different slot groups. Among the set of (Xs, Ys) value, determine one or more (Xs, Ys) with the largest Xs value (i.e. the largest BD/CCE budget) first and then select (Xs, Ys) with the smallest Ys value. Proposal 1: Select one (Xs, Ys) value from multiple (Xs, Ys) combinations reported by a UE according to the following steps: 1) Determine a set of (Xs, Ys) values according to which the search space configurations meets the limitation, i.e. configured Group (1) SSs are located within Y consecutive slots within a slot group of X slots; 2) Determine one or more (Xs, Ys) with the largest Xs value (i.e. the largest BD/CCE budget) first; 3) Select (Xs, Ys) with the smallest Ys value from the above selected (Xs, Ys) combinations. To implement the above proposal, the following proposal is made: Proposal 2: Adopt TP1 in Appendix. When multi-slot-based capability is introduced for NR Rel-17 UEs, how to configure or determine the capability type needs to be considered. As long as multi-slot-based capability is the mandatory one for BWP with 480K/960K SCS according to configuration of 480K/960K SCS for a BWP implies multi-slot-based capability for that BWP, which means PDCCH monitoring capability should be defined per BWP. Proposal 3: For NR Rel-17 UEs, PDCCH monitoring capability is defined per BWP and configuration of 480K/960K SCS for a BWP implies multi-slot-based capability for that BWP. After defining the PDCCH monitoring capability per BWP, the capability for one serving cell should also be determined for calculation of BD/CCE budget in multiple serving cell case. The straight forward way is to adopt the PDCCH monitoring capability for active BWP or configured first active BWP as the capability of the serving cell. Proposal 4: PDCCH monitoring capability for a serving cell is the capability for its active BWP or configured first active BWP when it is deactivated. To implement the above proposals, the following proposal is made: Proposal 5: Adopt TP2 in Appendix.

### R1-2200143 (CATT)

In RAN1#107-e meeting, it has been agreed that multiple combinations (X,Y) were supported, including (4,1),(4,2) for 480 kHz SCS and (8,1), (8,4), (4,2), (4,1) for 960 kHz SCS. One remaining issue is that how to determine the combination (X,Y) for PDCCH monitoring capability if a UE indicates a capability with multiple combinations (X,Y) of 480 kHz/960 kHz. A potential solution is to reuse the (X,Y) combination determination method of Rel 16. For example, if the UE indicates a capability with combination (8,1) and combination (4,1) for 960kHz SCS, and a configuration of search space sets results in a minimal separation of 9 slots between two Group(1) SSs in two consecutive slot groups, the UE will monitor PDCCH according to the BD/CCE limit of combination (8,1). If a configuration of search space sets results in a minimal separation of 7 slots between two Group(1) SSs in two consecutive slot groups, the UE will monitor PDCCH according to the BD/CCE limit of combination (4,1). Proposal 2: For PDCCH monitoring capability of 480 kHz/960kHz, if a UE indicates a capability with multiple (X, Y) combinations, the (X,Y) combination determination method of Rel 16 can be reused. If a UE may indicate a capability with multiple combinations of (X, Y), the search space configuration of 480kHz/960kHz is related to the value of X. Thus, how to indicate the value of X for search space configuration of 480kHz/960kHz require to be further studied. There are three candidate schemes to determine the value of X for search space configuration as follows, - Scheme #1: Adding a new IE for configuring the value of X for in the higher later parameter SearchSpace. - Scheme #2: Determining the smallest value of X from multiple combinations of (X, Y) reported by UE as the value of X. - Scheme #3: Defining a default value for the value of X, such as X= 4 slots for both 480 kHz/960 kHz. Proposal 4：When the UE indicates a capability with multiple combinations of (X, Y) for 480 kHz/960 kHz , RAN1 needs further study the following schemes to determine the value of X for search space configuration, such as: - Scheme #1: Adding a new IE for configuring the value of X for in the higher layer parameter SearchSpace. - Scheme #2: Determining the smallest value of X from multiple combinations of (X, Y) reported by UE as the value of X. - Scheme #3: Defining a default value for the value of X, such as X= 4 slots for both 480 kHz/960 kHz.

### R1-2200193 (Samsung)

The discussion on multi-slot PDCCH monitoring was all based on UE capability, and one remaining issue is the UE behavior in IDLE mode, wherein the UE capability on the multi-slot PDCCH monitoring is not available. One way is to support single-slot based PDCCH monitoring in IDLE mode, but it may not be efficient due to the limited number of BD/CCE budget, and will need further specification impact. Another way is to adopt the mandatory UE capability for multi-slot PDCCH monitoring as the default UE behavior in IDLE mode, which is much simpler and saves the effort on defining the controversial number of BD/CCE budget for single-slot based PDCCH monitoring. Proposal 1: For IDLE mode: - Support the mandatory UE capability for multi-slot PDCCH monitoring; - Don’t support slot based PDCCH monitoring; - Adopt TP#1 for TS 38.213. In RAN1#107-e, the following was agreed regarding the mandatory UE capability of multi-slot PDCCH monitoring within the Ys slots: - A UE capable of multi-slot monitoring mandatorily supports the following PDCCH monitoring within Y slots - For Y>1: FG3-1 (monitoring Group (1) SSs in the first 3 OFDM symbols of each of the Y slots) - For 960 kHz SCS For Y=1: FG3-5b with set1 = (7, 3) - For 480 kHz SCS For Y=1: FG3-5b with set2 = (4, 3) and (7, 3) with a modification with maximum two monitoring spans in a slot The remaining issue is to define the mandatory UE capability of multi-slot PDCCH monitoring outside the Ys slots. Since only Group (2) SS is monitored outside the Ys slots, it can simply follow the FG3-1. Proposal 2: A UE capable of multi-slot monitoring mandatorily supports FG3-1 for each slot outside Ys slots. Similar to Rel-16 span-based PDCCH monitoring, a UE can indicate multiple Xs,Ys combinations for multi-slot PDCCH monitoring in FR2-2, then the remaining issue is how to determine the combination that the UE will choose from the multiple indicated Xs,Ys combinations. A similar approach as Rel-16 span-based PDCCH monitoring can be reused, with the consideration of potential tie of values on MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ. Proposal 3: If a UE indicates a capability to monitor PDCCH according to multiple Xs,Ys combinations, a UE monitors PDCCH on the cell based on the following steps: - Step 1: Choose the Xs,Ys combination, from the Xs,Ys combinations, that is associated with the largest maximum number of MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ; - Step 2: If there is a tie on the largest maximum number of MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ, choose the Xs,Ys combination with a larger value of Ys. - Adopt TP#2 for TS 38.213. There is a FFS on whether to support Xs,Ys=(2, 1) for 480 kHz SCS, as one of the reported UE capability. The intention of supporting a smaller value of Xs, comparing to the mandatory UE capability with Xs=4 for 480 kHz SCS, is to allow flexible network configuration on the SS sets. Also, other than adding the corresponding values of MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ in the table, no other spec impact is expected, then it’s beneficial to support extra values of Xs for better flexibility. Proposal 4: Support Xs,Ys=(2, 1) for 480 kHz SCS. - Adopt TP#3 for TS 38.213.

### R1-2200260 (ZTE, Sanechips)

- According to the previous agreements/conclusions, a UE supporting 480/960 kHz SCS supports multi-slot PDCCH monitoring for 480/960 kHz SCS and for 120 kHz SCS, no multi-slot UE capability for PDCCH monitoring is needed. In other words, it will not cause any ambiguity if per-slot monitoring is not supported for SCS 480/960 kHz and there is no need to introduce monitoringCapabilityConfig = r17monitoringcapability. - Proposal 1: There is no need to introduce monitoringCapabilityConfig = r17monitoringcapability if per-slot monitoring is not supported for SCS 480/960 kHz. - Moreover, according to TS 38.213, if the UE is not provided monitoringCapabilityConfig, the UE monitors PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per slot. However, for SCS configuration μ=5 or μ=6 in Rel-17, only multi-slot PDCCH monitoring capability is supported and per-slot monitoring is still under discussion. There is a high probability that per-slot monitoring will not be supported in FR2-2 based on previous discussions, we prefer to clarify this in the specification that if UE does not provide any monitoring capability combinations Xs,Ys, Xs,Ys = (4, 1) or (8, 1) is mandatorily supported for SCS configuration μ=5 or μ=6 respectively if per-slot monitoring is not supported for SCS 480/960 kHz. Moreover, for SCS configuration μ=5 or μ=6, a UE can indicate a capability to monitor PDCCH according to more combinations Xs,Ys, different Xs,Ys may correspond to different BD/CCE budge, we suggest indicating the mandatory Xs,Ys combination. - Proposal 2: Further clarification in TS 38.213: if a UE does not provide any monitoring capability combinations Xs,Ys or if a UE indicates a capability to monitor PDCCH according to more combinations Xs,Ys, Xs,Ys = (4, 1) or (8, 1) is mandatorily supported for SCS configuration μ=5 or μ=6 respectively if per-slot monitoring is not supported for SCS 480/960kHz.

### R1-2200290 (Qualcomm)

Further related to the joint configuration of multi-slot PDCCH monitoring and SSSG switching, a situation shown in Figure 1 may be considered. That is, at the boundary of SSSG switching, the locations of the two Y consecutive slots in the slot groups before and after the boundary may be different. Thus, the separation between the two Y consecutive slots may be less than X slots. In general, the issue is related to the granularity of configuration: Although the agreement says that “The location of the Y consecutive slots within the slot group of X slots is maintained across different slot groups”, it needs further clarification in which hierarchy of configuration, the statement is applied. For example, the location of the Y consecutive slots within the slot group could be maintained within a SSSG, within a BWP, within a CC, or across CCs of a UE. To avoid such a situation in Figure 1, the location of the Y consecutive slots should be maintained at least across SSSGs. For BWP switching, due to the presence of BWP switching delay, within which the UE does not expect to receive and transmit any channels, the situation in Figure 1 does not occur even if the location of the Y slots changes across BWPs. If the location of the Y slots is maintained across CCs within the same band, the power saving gain would be maximized, while the scheduling flexibility would be somewhat limited. Proposal 5: The location of Y consecutive slots within a slot group is maintained for all Group (1) SS sets within a BWP, regardless of the associated SSSG indices. - FFS: configuration granularity of the location of the Y slots – per BWP, per CC, per UE, etc. Figure 1: An example of SSSG switching for multi-slot PDCCH monitoring. In Rel-15 per-slot PDCCH monitoring, the cell-specific CSS configuration is rather straightforward. For example, with the basic PDCH monitoring feature (FG 3-1), a SS set other than SS set #0 can be configured as the Type1 CSS for all UEs via dedicated signaling, and the SS set has a MO within the first 3 symbols of every slot. For multi-slot PDCCH monitoring discussed in the previous sections, the Type1 CSS with dedicated configuration has been assumed to be in Group (1). However, with multi-slot PDCCH monitoring, the monitoring occasions of Group (1) SS sets are limited to the Y (< X) consecutive slots and the locations of the Y slots may be different across UEs. Therefore, in such a case, a common dedicated configuration of the Type1 CSS for all UE may be impossible. As a result, for multi-slot PDCCH monitoring, the plausibility of putting the Type1 CSS with dedicated RRC configuration within Group (1) should be reconsidered. Observation 1: For multi-slot PDCCH monitoring, putting Type1 CSS with dedicated RRC configuration within Group (1) may be infeasible and should be reconsidered. To address the issue, a special rule for determining MOs of Type1 CSS with dedicated RRC configuration (i.e., other than SS set #0) needs to be introduced. As an example, Type1 CSS may be configured with one-slot periodicity, commonly for all UEs, so that every slot in the slot group may include a nominal MO for Type1 CSS. Then, for actual monitoring for the Type1 CSS, each UE may select only one slot out of the X slots in the slot group. A simple rule for selecting the slot can be as follows: - Connected mode UE with contention-free RACH: select a slot for Type1 CSS monitoring that overlaps with the Y consecutive slot(s) of the UE. - Idle/inactive mode UE or connected mode UE with contention-based RACH: a slot offset (0,..,X-1 slots) is associated with a RACH resource/preamble, and the slot for Type1 CSS monitoring is selected based on the RACH resource/preamble that the UE used for the RACH procedure. Proposal 8: A rule for determining a UE-specific MO of cell-specific Type1 CSS configuration should be introduced. In the agreement of RAN1 #107-e, the design for the MOs of Type0A/2 CSS with searchSpaceId non-zero is still incomplete – although it was agreed that the MO can be anywhere within a slot group, it does not necessarily mean the same slot-based design as Rel-15/16. In fact, if we allow the same design as Rel-15/16, the issues we considered for the new design of SS set #0 should be revisited: - If a UE is required to monitor more than one MOs of Type0A/2 CSS within a slot group, the BD/CCE budget may get exhausted. - Likewise, if there are more than one MO associated with different CSS types within the same slot group, the BD/CCE budget of the slot group may get easily exhausted. Thus, similar to the new design of SS set #0 (i.e., monitoring slot n0 and n0+X0), it would be desirable to limit the total number of MOs for Group (2) SS sets within a slot group. Proposal 9: For a CSS set with searchSpaceID ≠ 0, a UE does not expect to monitor more than one MO for the CSS set per slot group. In Rel-15, for CSS monitoring within a slot, the following rule is applied: TS 38.213, Section 10.1: If a UE is provided - one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, or a CSS set by PDCCH-Config, and - a SI-RNTI, a P-RNTI, a RA-RNTI, a MsgB-RNTI, a SFI-RNTI, an INT-RNTI, a TPC-PUSCH-RNTI, a TPC-PUCCH-RNTI, or a TPC-SRS-RNTI then, for a RNTI from any of these RNTIs, the UE does not expect to process information from more than one DCI format with CRC scrambled with the RNTI per slot.. Thus, a similar design should be extended for multi-slot PDCCH monitoring. Proposal 10: Per slot group, a UE does not expect to process information from more than one DCI format with CRC scrambled by a RNTI from any of SI-RNTI, RA-RNTI, MsgB-RNTI or P-RNTI.

### R1-2200367 (Intel)

The following TP is proposed to capture additional X value 2 for SCS 480kHz. ------------------------------ TP#1: TS 38.213 ----------------------------------- *** Unchanged text is omitted *** ## 10.1 UE procedure for determining physical downlink control channel assignment *** Unchanged text is omitted *** Table 10.1-2B provides the maximum number of monitored PDCCH candidates, MPDCCHmax,Xs,Ys,μ, per slot group for combination (Xs,Ys) for a UE in a DL BWP with SCS configuration μ for operation with a single serving cell. Table 10.1-2B: Maximum number MPDCCHmax,(Xs,Ys),μ of monitored PDCCH candidates per slot group for combination Xs,Ys for a DL BWP with SCS configuration μ∈5, 6 for a single serving cell Maximum number MPDCCHmax,Xs,Ys,μ of monitored PDCCH candidates per combination Xs,Ys and per serving cell. μ; (2, 1); (4, 1); (4, 2); (8, 1); (8, 4). 5; 10; 20; 20; -; -. 6; -; 10; 10; 20; 20. *** Unchanged text is omitted *** Table 10.1-3B provides the maximum number of non-overlapped CCEs, CPDCCHmax,Xs,Ys,μ, for a DL BWP with SCS configuration μ that a UE is expected to monitor corresponding PDCCH candidates for combination Xs,Ys for operation with a single serving cell. Table 10.1-3B: Maximum number CPDCCHmax,(Xs,Ys),μ of non-overlapped CCEs in a slot group for combination Xs,Ys for a DL BWP with SCS configuration μ∈5, 6 for a single serving cell Maximum number of non-overlapped CCEs per combination Xs,Ys and per serving cell CPDCCHmax,(Xs,Ys),μ. μ; (2, 1); (4, 1); (4, 2); (8, 1); (8, 4). 5; 16; 32; 32; -; -. 6; -; 16; 16; 32; 32. *** Unchanged text is omitted ***. Proposal 1: - X=2 can be optionally supported for SCS 480kHz, which corresponds to combination (X, Y) = (2, 1) - X=1, i.e., per-slot PDCCH monitoring capability is not supported for SCS 480/960kHz - Agree on TP 1 to capture additional X value 2 for SCS 480kHz Since UE is mandatorily support Y=1 for the multi-slot PDCCH monitoring capability, the design on handling group (2) SS can take existing agreement for Y=1 as baseline, i.e., FG3-5b extension. That is, UE must be capable to handle up to two spans in a slot in a slot group. To avoid excessive increase of UE complexity in the configuration of the two groups of SS sets, it is preferred that the maximum number and positions of spans in any slot in a slot group remains to be 2 which keeps the same complexity as current agreement for the Y=1 slot. Further, the total number of spans of SS sets in a slot group should be carefully designed. A larger number of spans provide a better flexibility for PDCCH transmission. However, it also increases UE complexity and power consumption. To balance various factors, it is preferred that UE should be capable to monitor up to 3 or 4 spans of all SS sets in a slot group. Proposal 2: - The maximum number of spans in any slot in a slot group remains to be 2 which reuses the current agreement for the Y=1 slot - Up to 2 or 3 spans for group (2) SS except the SS set with searchSpaceId = 0 can be configured in a slot group - UE may decide to monitor only one MO or even not monitor any MO at all - UE is capable to monitor up to 3 or 4 spans of SS sets in a slot group - The limitation on number of spans in a slot group should be discussed in UE feature for WI NR_ext_to_71GHz Based on the agreed framework, UE may report the capability to support multiple combinations (X, Y) for SCS 480/960kHz. The configured search space sets for the UE must satisfy at least one supported combination (X, Y). - For SCS 480 kHz: (X,Y) = (4,1), (4,2), (2,1)? - For SCS 960 kHz: (X,Y) = (8,1), (8,4), (4,2), (4,1) Further, if the configured search space sets of the UE satisfy two or more supported combinations (X, Y), the UE needs to identify the active combination (X, Y). Since the supported maximum numbers of BD/CCE is only determined by value X, UE can determine a combination (X, Y) with larger X which enables larger maximum numbers of BD/CCE for PDCCH monitoring for better flexibility. If multiple potential combinations (X, Y) have same value X, the active combination (X, Y) could be determined as the combination (X, Y) with smallest value Y. The following TP is proposed to determine the active combination (X, Y). ------------------------------ TP#2: TS 38.213 ----------------------------------- *** Unchanged text is omitted *** # 10 UE procedure for receiving control information *** Unchanged text is omitted *** For SCS configuration μ=5 or μ=6, a UE can indicate a capability to monitor PDCCH according to one or more combinations Xs,Ys, where Xs and Ys are numbers of consecutive slots, groups of Xs slots are consecutive and non-overlapping, and the Ys slots are within the Xs slots. The first group of Xs slots starts from the beginning of a subframe. The start of two consecutive groups of Ys slots is separated by Xs slots. If a UE indicates a capability to monitor PDCCH according to multiple Xs,Ys combinations and a configuration of search space sets to the UE for PDCCH monitoring on a cell is allowed by one or more of the multiple combinations Xs,YsY, the UE monitors PDCCH on the cell according to the combination Xs,Ys with largest Xs and smallest Ys. *** Unchanged text is omitted ***. The number of DL or UL unicast DCIs per slot group that can be processed by UE was also agreed in last meeting. Additionally, UE should be able to process one broadcast DCI for SI/RACH/paging in the slot group of X slots. If cross-carrier scheduling is configured, for example, the scheduling cell has SCS 15kHz, it needs clarification whether the agreement implies that UE would be able to detect up to 8 unicast DCIs in a slot on the scheduling cell that schedule a unicast transmissions on the scheduled cell with SCS 960kHz. Agreements from early RAN1 meetings: - The following supersedes FG3-5b and FG3-1 definition: - Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot group of X slots per scheduled CC for FDD - Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot group of X slots per scheduled CC for TDD. Proposal 4: - UE should be able to process one broadcast DCI for SI/RACH/paging in addition to the agreed number of processed unicast DCI in a slot group of X slots. - To clarify whether a UE would be able to detect up to 8 unicast DCIs in a slot on the scheduling cell with SCS 15kHz - The limitation on number of detected DCIs in a slot group should be discussed in UE feature for WI NR_ext_to_71GHz Proposal 5: - For a UE capable of multiple combinations (X, Y), if the configured SS sets are aligned with more than one combination (X, Y), the active combination (X, Y) is determined that is associated with the largest X and smallest Y. - Agree on TP 2 to determine the active combination (X, Y).

### R1-2200401 (Ericsson)

For 480 kHz SCS, the DL support FG 24-4 should therefore be updated to capture the agreed mandatory support of Xs,Ys=(4,1) capabilities and behaviors: - For Group (1) SS: Type 1 CSS with dedicated RRC configuration and type 3 CSS, UE specific SS - A SS is monitored within Y consecutive slots within a slot group of X slots - A UE capable of multi-slot monitoring mandatorily supports - For SCS 480 kHz: (X,Y) = (4,1) - For 480 kHz SCS For Y=1: FG3-5b with set2 = (4, 3) and (7, 3) with a modification with maximum two monitoring spans in a slot - The following supersedes FG3-5b and FG3-1 definition: - Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot group of X slots per scheduled CC for FDD - Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot group of X slots per scheduled CC for TDD Similarly, for 960 kHz SCS, the DL support FG 24-5 should be updated to capture the agreed mandatory support of Xs,Ys=(8,1) capabilities and behaviors - For Group (1) SS: Type 1 CSS with dedicated RRC configuration and type 3 CSS, UE specific SS - A SS is monitored within Y consecutive slots within a slot group of X slots - A UE capable of multi-slot monitoring mandatorily supports - For SCS 960 kHz: (X,Y) = (8,1) - For 960 kHz SCS For Y=1: FG3-5b with set1 = (7, 3) - The following supersedes FG3-5b and FG3-1 definition: - Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot group of X slots per scheduled CC for FDD - Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot group of X slots per scheduled CC for TDD We provide further details on the above proposed changes in the companion contribution on UE features [2]. It is noted that prior to dedicated configuration, e.g., during initial access, the UE is not yet provided with monitoringCapabilityConfig. This is based on the fact that the parameter monitoringCapabilityConfig is defined only in PDCCH-Config for UE-specific configurations, but is not defined in PDCCH-ConfigCommon for cell-specific PDCCH configuration [4]. As shown in the highlighted text above, this would imply that the UE shall perfom per-slot PDCCH monitoring for Type0/0A/2-PDCCH CSS set and Type1-PDCCH CSS set provided in SIB1. This clearly contradicts the RAN1 #107-e agreement in which per-slot group monitoring is always performed for 480/960 kHz. It also contradicts the highlighted text above. Two possible solutions for 480 kHz SCS can be considered: - Option A is to introduce r17monitoringcapability indication in PDCCH-ConfigCommon in the TR 38.331 for 480 kHz SCS only. - Option B is to introduce default r17monitoringcapability indication for 480 kHz SCS in TR 38.213. Our preference is Option B. This is similar to how searchSpaceZero definitions for 480 and 960 kHz SCS are captured in TR 38.213. For example, the highlighted text could be amended as follows: If the UE is not provided monitoringCapabilityConfig, the UE monitors PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per slot, unless the SCS configuration μ = 5 in which case the UE monitors PDCCH per group of Xs slots according to combination Xs,Ys=(4,1) in Tables 10.1-2B and 10.1-3B. We point out that no change is needed for 960 kHz SCS since standalone operation is not supported. Default r17monitoringcapability of Xs,Ys=(4,1) for 480 kHz SCS before dedicated RRC configuration needs to be captured in the specs. RAN1 discusses which of the following options to adopt: - Option A: introduce r17monitoringcapability indication in PDCCH-ConfigCommon in the TR 38.331 for 480 kHz SCS only. - Option B: introduce default r17monitoringcapability indication for 480 kHz SCS in TR 38.213 Another issue is that the parameter monitoringCapabilityConfig has been extended with a new value r17monitoringcapability by the 38.213 spec editor; however, this has not been captured in the RRC parameter spreadsheet that was sent to RAN2 after RAN1#107-e. Hence we make the following additional proposal. Inform RAN2 that the value range for the existing parameter monitoringCapabilityConfig has been extended with new value r17monitoringcapability. A note can be added to the RRC parameter spreadsheet for RAN2 to update the field description as follows: monitoringCapabilityConfig Configures either Rel-15 PDCCH monitoring capability or Rel-16 PDCCH monitoring capability for PDCCH monitoring on a serving cell. Value r15monitoringcapablity enables the Rel-15 monitoring capability, and value r16monitoringcapablity enables the Rel-16 PDCCH monitoring capability. Value r17monitoringcapablity enables the Rel-17 PDCCH monitoring capability only applicable to 480 and 960 kHz SCS (see TS 38.213 [13], clause 10.1).

### R1-2200410 (Apple)

Proposal 1: On the values of X for Alt-1 and Alt-2: - Single-slot PDCCH monitoring for 480 kHz and 960 kHz is not supported. - The configurable values for multi-slot PDCCH monitoring operation should be same as the reported X value(s). The UE is not expected to handle a scenario in which they are different, and a UE might report its monitoring capability for more than one (X,Y) combination. - For each SCS 480 kHz and 960 kHz, the minimum configurable multi-slot PDCCH monitoring periodicity is the smallest value X that a UE supports when reporting its PDCCH monitoring capabilities for the corresponding SCS and are UE specific.

### R1-2200495 (Sharp)

In RAN1#107-e meeting, the agreement for BD drop in multi-slot monitoring is achieved and it says “The dropping rule for multi-slot PDCCH monitoring capability is the same as the current specification but evaluated per slot group”. Then, the slot group is just described as “group of Xs slots according to combination (Xs, Ys)” or “group of Xs slots for a corresponding combination (Xs, Ys)” in TS38.213. When a UE reports multiple (Xs, Ys) combinations, there is no determination rule such that which value of Xs is used for BD dropping per group. Therefore, we should make it clear what is meant by the statements in “according” and “corresponding”. Proposal 1: Introduce a new RRC parameter for configuring Xs for allocation of PDCCH candidates for UEs with multiple Xs capabilities. Proposal 2: Adopt Text proposal #1.

### R1-2200540 (MediaTek)

One remaining discussion is the BD/CCE budge for (X,Y)=(4,2) and (4,1) under 960kHz. In our view, the significance and usage of configuration (X,Y)=(4,1) and (4,2) are not clear compared to (8,1) and (8,4) and we prefer to remove such configurations. Furthermore, if the configuration (X,Y)=(4,1) and (4,2) were considered, the associated BD/CCE limit should be much less than the ones for (X,Y)=(8,1) and (8,4). Consequently, the PDCCH scheduling will be impacted, especially when Type-0 PDCCH monitoring is involved where monitoring in consecutive slot-groups is needed. Proposal 1: For 960kHz multi-slot PDCCH monitoring, only (X,Y)=(8,1) and (8,4) are considered in Rel-17.

### R1-2200565 (LG)

Among the supported combinations, X=8 for 960 kHz and X=4 for 480 kHz are the mandatorily supported X values for multi-slot PDCCH monitoring operations. These values can maintain the PDCCH monitoring burden of the UE at a level similar to that of the 120 kHz SCS since the slot-group length and BD/CCE limit for them are the same as for 120 kHz SCS. In addition, for 960 kHz SCS, there is an optionally supported X, i.e., X=4, which can allow more diverse and flexible monitoring operation than what the mandatory X can do. However, for 480 kHz SCS, whether additional X other than X=4 is supported has not been decided yet. For a flexible multi-slot monitoring operation like 960 kHz, support for additional optional X may be required. It should be noted that X=2 for 480 kHz and X=4 for 960 kHz share the same absolute time and their BD/CCE budget could be the same. These mean that the UE supporting X=4 of 960 kHz can support X=2 for 480 kHz without additional burden. From our point of view, there seems to be no critical issue to support an optional X for 480 kHz SCS. With X=2 for 480 kHz, a possible combination (X,Y) can be (2,1). Proposal #1: Support (X,Y)=(2,1) as an optional combination for 480 kHz SCS. Single slot monitoring, i.e., X=1, is not in line with the purpose of introducing multi-slot monitoring and it requires a slot based operation at the UE side. For 480/960 kHz SCS, many aspects were already designed with multi-slot based operation instead of a single-slot, e.g., most timeline parameters, multi-slot monitoring, scheduling, and so on. Due to the short slot duration for these SCSs, the slot based operation may cause an additional implementation burden on the UE. Moreover, supporting slot based monitoring removes the complexity reduction benefits of multi-slot monitoring and the low BD/CCE budget that can be expected will result in low aggregation levels and high blocking probabilities when scheduling. In this regard, we do not support slot based monitoring for 480/960 kHz. Proposal #2: Do not support X=1, i.e., single slot monitoring, for 480 kHz and 960 kHz SCS. For multi-slot monitoring operation, the length X of the slot-group becomes the criterion for BD/CCE budget management. For example, for 960 kHz SCS, the UE has the BD budget of 20 per X=8 slots or 10 per X=4 slots. If the slot-based monitoring is not supported for 480/960 kHz SCS (we do not prefer single-slot monitoring for these SCS), multi-slot monitoring becomes a mandatory monitoring operation including IDLE mode. Since UE assumption on the (X,Y) can affect BD/CCE budget handling within X slots, the default (X,Y) combination should be specified in the specification for when any combinations (X,Y) are not indicated by gNB (or not reported by UE). At the last meeting, RAN1 agreed to have a mandatorily supporting (X,Y) combinations for 480 kHz and 960 kHz SCS, i.e., (4,1) for 480 kHz and (8,1) for 960 kHz. These can be the default (X,Y) combination for each SCS, respectively. We propose to adopt the following text proposal (highlighted in yellow) to Clause 10 in TS 38.213 specification. Proposal #3: Adopt the following text proposal in TS 38.213 Clause 10, to specify the default (X,Y) combination for multi-slot PDCCH monitoring. ============ Start of Text Proposal for TS 38.213 [1] ================== 10 UE procedure for receiving control information << Other parts are omitted >> If a UE monitors PDCCH on a cell according to combination Xs,Ys, the UE can monitor PDCCH for Type1-PDCCH CSS set provided by dedicated higher layer signalling, Type3-PDCCH CSS sets, and USS sets in any slot of the Ys slots, and the UE can monitor PDCCH for Type0/0A/2-PDCCH CSS set and Type1-PDCCH CSS set provided in SIB1 in any slot of the Xs slots. The UE determines the number of monitored PDCCH candidates and the number of non-overlapped CCEs for combination Xs,Ys based on all search space sets within the Xs slots, as applicable according to the search space set configurations, and maximum corresponding values are provided in Table 10.1-2B and Table 10.1-3B, respectively. Xs,Ys=(4,1) or (8,1) applies for SCS configuration μ=5 or μ=6, respectively, unless the UE indicates a capability to monitor PDCCH according to other combinations. << Other parts are omitted >> ============ End of Text Proposal for TS 38.213 ==================. When the UE reports multiple (X,Y) combinations for multi-slot monitoring, the UE should determine the (X,Y) combination based on the configured SS set configuration parameters (e.g., periodicity, offset, …). However, it may be ambiguous which (X,Y) the UE should assume to check BD/CCE budget if the monitoring occasions are aligned with multiple (X,Y) combinations. For example, when periodicity is configured as 8 slots to a UE reporting (4,1) and (8,1) as a preferred (X,Y) combination, the monitoring occasion may be aligned with both (4,1) and (8,1) combinations. In this case, it is not clear in which X slots the BD/CCE budget should be managed. To remove this ambiguity, an additional rule to select one (X,Y) combination may be needed. As a conservative approach, it may be reasonable to select the combination with the largest X among the reported combinations that match the monitoring occasions. Proposal #4: In the multi-slot monitoring, when the monitoring occasions corresponding to multiple (X,Y) combinations are configured, the UE should operate assuming the (X,Y) combination corresponding to the largest X (and Y that can be combined therewith) among the reported combinations that match the monitoring occasions.

### R1-2200654 (Panasonic)

Even if the above-mentioned constraint on search space periodicity is introduced, it can still happen that the configured MOs sometimes fall outside the Ys. For example, considering the TDMed-beam transmission of CSS (of Group (1)), UE may need to monitor a different slot for CSS when UE’s serving beam is changed. However, for USS, gNB can simply change the serving beam without changing the MO location since USS MO is unicast to the UE. In this case, it can happen that CSS MO becomes far away from the USS MO of the new serving beam such that Ys cannot include both CSS and USS MOs anymore. The following Fig.2 illustrates one example. As shown, in the first slot group (UE is served by yellow beam), the location of Ys is the first slot, covering USS MO and CSS MO of the yellow beam. When UE moves from the coverage of yellow beam to green beam in the second slot group, the corresponding CSS MO location for the UE has to been changed as well because the previous CSS MO would still be used for other UEs covered by the yellow beam. Consequently, the new CSS MO for the UE becomes outside the Ys. According to the current spec, the UE may not receive such CSS because UE is not required to monitor Group(1) CSS outside Ys. Fig.2 Infeasibility to restrict all MOs into Ys due to beam switching To fix the above-mentioned issue, we need two amendments to existing multi-slot monitoring framework. The first one is to allow the location of Ys to change across different slot groups if new MO is configured. This only happens temporally. After the location of Ys is adapted to the new configuration of MOs, it would maintain the same across future slot groups. In particular, we propose the following: Proposal 2: In case that new MO of Group(1) SS is configured, the location of Ys within Xs can be adapted accordingly. The second amendment is to define a criterion for the location determination of Ys. Because of the fact that it is not be feasible to restrict all MOs of Group (1) SS within Ys for all the time, we propose to prioritize the CSS MO. Specifically, we have the following proposal: Proposal 3: Location of Ys should first include all CSS MOs (of Group(1) SS) that are monitored by UE, then USS MOs can be further included in Ys from USS with lower to higher indices. Certain USS MO would be dropped if it cannot be included in Ys. The following Fig.3 illustrates how the above two proposals are used. Similar to Fig.2, the UE is first served by yellow beam and therefore the location of Ys is determined by the CSS MO of the yellow beam (which is the first slot in a slot group). UE is not required to monitor USS2 MO in 1st slot group because it is outside Ys. When UE moves from the coverage of yellow beam to green beam in the 2nd slot group, gNB indicates the change of UE’ serving beam. This can be done using MAC CE to activate a new TCI state of the related CORESETs, according to the existing procedure. After receiving the beam switching indication, UE shall monitor a different MO of CSS corresponding to the new beam due the nature of beam-sweeping transmission of CSS. The beam-sweeping pattern (i.e. which MO of CSS corresponds to which beam) can be pre-configured to the UE such that no RRC reconfiguration is needed every time when beam is switched. According to Proposal 3, the location of Ys would change to the second slot from the 2nd slot group onwards to follow the new CSS MO (of green beam). As a result, USS1 MO is dropped, and USS2 MO will be monitored by the UE. With this procedure, UE and gNB would have the same understanding on which MO(s) are received or dropped. Fig.3 Determination of location of Ys

## Topic A2: Search Space Configuration/Enhancement

### R1-2200045 (Huawei, HiSilicon)

Among the existing configurable periodicities {sl1, sl2, sl4, sl5, sl8, sl10, sl16, sl20, sl40, sl80, sl160, sl320, sl640, sl1280, sl2560}, there are entries not divisible by X= 4 or 8, which cannot be configured to UE operating with 480kHz and 960kHz SCS. On the other hand, the range of configurable SS periodicity in terms of absolute time shrinks with the increasing SCS. In order to maintain the same range and flexibility of SS periodicity configuration as 120 kHz SCS, the following two alternatives of enhancement to monitoringSlotPeriodicityAndOffset can be considered: - Add periodicities of {sl32, sl64, sl128, sl5120, sl10240, sl20480} to the list of monitoringSlotPeriodicityAndOffset. The offset field should be extended accordingly, e.g., for sl32, the range of offset should be from 0 to 31. - Change the unit of monitoringSlotPeriodicityAndOffset to X slots instead of one slot. Additional slot level offset within X slot group should be introduced to allow MO locates in any slot From our perspective, Alternative 1 is more straightforward and with less standard impact. - For search space set configuration for multi-slot PDCCH monitoring, - Add periodicities { sl32, sl64, sl128, sl5120, sl10240, sl20480} to the list of supported values in monitoringSlotPeriodicityAndOffset, - Supported set of offsets for each added periodicity X is 0,…,X-1; - UE expects to be configured with a periodicity that is a multiple of X slots where - For 480 kHz, X = 4 - For 960 kHz, X= 4 if (X,Y) = (4,2) or (X,Y) = (4,1) is supported; otherwise X= 8. Note that, if Y=1, the current 14-bit bitmap monitoringSymbolsWithinSlot is sufficient to indicate the first symbol(s) of the PDCCH monitoring occasions that are configured in the single slot within the slot group of X slots. However, when Y>1 is supported, PDCCH monitoring is carried out in the first 3 OFDM symbols of up to Y consecutive slots within the slot group of X slots. A single 14-bit bitmap would not then be enough to indicate the first symbol(s) of the PDCCH monitoring occasions within these up to Y consecutive slots. To resolve this issue, any of the following solutions may be considered: - Introduce a parameter that indicates the slot-level duration (e.g., durationWithinSlotGroup). For instance, when (X,Y)=(4,2), durationWithinSlotGroup can be either “one” to indicate that the PDCCH monitoring occasions are only in the first 3 symbols of a single slot within the slot group of X slots or “two” to indicate that the PDCCH monitoring occasions are in the first 3 symbols of two consecutive slots within the slot group of X slots. - Introduce a slot-level bitmap (e.g., monitoringSlotsWithinSlotGroup). For instance, when (X,Y)=(4,2), monitoringSlotsWithinSlotGroup can be either “1000” to indicate that the PDCCH monitoring occasions are only in the first 3 symbols of the first slot within the slot group of X slots or “1100” to indicate that the PDCCH monitoring occasions are in the first 3 symbols of the first and second slots within the slot group of X slots. - Increase the number of bits of monitoringSymbolsWithinSlot to cover Y slots. For instance, when (X,Y)=(4,2), monitoringSymbolsWithinSlot can be either “11100000000000 00000000000000” or “11100000000000 11100000000000.” In our view, solution a) that introduces a slot-level duration parameter (e.g., durationWithinSlotGroup) is the most efficient and is preferred. - For search space set configuration for multi-slot PDCCH monitoring, introduce a new RRC parameter durationWithinSlotGroup, ranging from 1 to Y, to indicate the consecutive slots that contain monitoring occasions within each slot group of X slots - Y is the value of multi-slot PDCCH monitoring capability in the reported (X, Y) pair by the UE. If UE is allowed to report more than one Y>1 for a given X, the maximum value of reported Y is used. Regarding duration, the unit of duration can be simply changed to X slots. - For search space set configuration for multi-slot PDCCH monitoring, the unit of “duration” should be changed to X slots where - For 480 kHz, X = 4 - For 960 kHz, X= 4 if (X,Y) = (4,2) or (X,Y) = (4,1) is supported; otherwise X= 8.

### R1-2200060 (InterDigital)

Based on the discussion, the following values with only one capability (i.e., UE processing capability 1) was agreed as a working assumption. μ; Minimum Pswitch value for UE processing capability 1 [symbols]. 3; 40. 5; 160. 6; 320. As the values do not have any serious issues, it is preferred to agree the working assumption as it is. Observation 1: No serious issue has been observed for the working assumption on the minimum Pswitch value. Proposal 1: Confirm the working assumption on the minimum Pswitch value.

### R1-2200143 (CATT)

Proposal 3：For 480 kHz SCS and 960 kHz SCS, the search space configuration can be defined as follows. - Duration: The unit of the duration is still slot. - Adding a new bitmap monitoringSlotWithinMulti-slot indicating the slot that the search space exists within the multi-slot. Figure 4: Search space group switching for 60GHz NR-U The search space group set switching was introduced in Rel-16 NR-U with 15 kHz SCS, 30 kHz SCS and 30 kHz SCS for dynamic switching between different search spaces. Before the gNB obtains the COT, the frequent monitoring enable the gNB to transmit DCI as soon as possible if gNB’s LBT is successful. However, frequent monitoring is not conducive to power saving of the UE during the COT. When the search space group set switching is configured, the gNB can indicate to UE switching between a search space with long periodicity and a search space with short periodicity to meet different scheduling requirements. Therefore, we suggest the legacy SSSG switching mechanism should be reused for the 120 kHz SCS, 480 kHz SCS and 960 kHz SCS in 60GHz NR-U, as shown in Figure 3. Proposal 5: The Legacy SSSG switching mechanism should be reused for the 120 kHz SCS, 480 kHz SCS and 960 kHz SCS in 60 GHz NR-U.

### R1-2200174 (Sony)

For multi-slot PDCCH monitoring, supporting multi combination of (X,Y) as optional and multi span when Y=1 were agreed in RAN1#107-e meeting. Multi combination of (X,Y) and spans will benefit the flexibility of search space configuration for NW and the adjustment of power consumption for UE. However, it also triggers another issue on SSSG switching among different monitoring capacities/search space sets. From previous meetings, many companies support SSSG switching for 480/960 kHz with a natural extension of the Rel-16 functionality due to lack of time, then switching time Pswitch for UE processing capability with the main concern on whether to support multiple capability on Pswitch [3] has been discussed and agreed. However, from our perspective, a natural extension of Rel-16 SSSG switching is not enough for 52.6GHz to 71GHz. Take 960 kHz for illustration, (X,Y) = (8,1) with mandatory and (X,Y) = (8,4), (4,2), (4,1) with optional were already agreed, for UEs supporting all four monitoring capacities, SSSG switching with only two SSSG group may loss switching efficiency on power saving and also waste certain capabilities of these UEs. Thus, flexible SSSG switching schemes with finer granularity should be considered. One possible SSSG switching scheme to address the above power saving issues is to configure the search space set with monitoring capacity indices. The procedure can be as follows: - First, when UE reports multi monitoring capabilities, the corresponding monitoring capability indices are also reported. - Then, gNB configures each SS set with the associated monitoring capability index. - After SSSG switching is trigged, such as the scheduling requirement from gNB or power saving requirement from UE by preferred monitoring capacity index reporting, a DCI with monitoring capability index indication is scheduled so that UE can switch to the specified SSSG for monitoring. Take a simple UE monitoring capability with {(8,1); (4,1)} reported as an example. With the agreement and working assumption from RAN1#107 meeting, the maximum number of blind detection (BD) for monitoring capability (8,1) and (4,1) are 20 and 10 respectively. Denoting Rel-16 SSSG index as x, and UE monitoring capability index as y, allocate y=1 and 2 to these two capabilities when UE monitoring capability index is reported. Alt1: No monitoring capability index is configured, assume RRC configure SS1={2 BD}, SS2={7 BD}, SS3={6 BD},SS4={5 BD}, and SSSG0={SS1,SS2 and SS3}, SSSG1={SS4}; Alt2: Monitoring capability index is configured, assume RRC configure SS1={1,2; 2 BD}, SS2={1,2; 7BD}, SS3={1; 6BD}, SS4={1; 5BD}. From Alt2, firstly, we can see a new dimension of group division based on number of BD (UE monitoring capability) indicated by UE monitoring capability index is introduced. y=2 can be indicated to monitor SSSG 2={SS1, SS2}, which is not existed in Alt1. While x in Rel-16 can only indicate sparse or dense monitoring. Besides, with joint (x,y)=(0,2) indication, we have SSSG0_2={SS1,SS2} smaller than SSSG0 in Alt1, which shows the finer granularity feature. This corresponds to a double dimension of SSSG switching. In summary, with UE monitoring capability index reporting, SSSG can be finer granularity-based. Proposal 1: SSSG switching schemes with finer granularity can be considered for NR from 52.6 GHz to 71GHz. CORESET duration has also been discussed in RAN1#107-e meeting. Unfortunately, not consensus has been achieved on whether to support a duration of more than 3 OFDM symbols for PDCCH with 480/960 kHz SCS in Rel-17 NR above 52.6 GHz [3]. In our view, large SCSs with 480kHz and 960kHz cause a relatively short time duration of a symbol. Therefore, if CORESET duration remains up to 3 symbols as in R16, the real-time duration for PDCCH monitoring is quite small, which also puts extra time limitation of UE blind decoding. Therefore, we suggest a large CORESET duration with more than 3 symbols for SCS 480kHz and 960kHz alleviate UE processing capability for PDCCH decoding. Thus, we suggest PDCCH monitoring with a maximum duration of more than 3 OFDM symbols per PDCCH monitoring occasion. Proposal 3: PDCCH monitoring with a maximum duration of more than 3 OFDM symbols per PDCCH monitoring occasion is more suitable.

### R1-2200184 (Nokia, Nokia Shanghai Bell)

Proposal 5: Support only search space set group switching processing capability 1 with the following values Minimum value for UE processing capability 1 [symbols]. 3; 40. 5; 160. 6; 320. When considering the applicability of current periodicyt/offset configuration for multi-slot PDCCH monitoring, it can be noted that it works correctly if - Montoring-periodicity (slots) is an integer multiple of X (slots); and - Montoring-offset-PDCCH-slot is configured in such that it overlaps with Y slots Based on that, we propose to keep the current SS configuration principle (i.e. slot-based configuration). It is noted that the other option (i.e. X-based configuration) might have impacts also in other RAN1 specs (esp. TS 38.213). Table 2 shows the proposed monitoring periodicity in terms of slots and slot groups. The left column shows the RRC signalled periodicity (+ associated slot offset) according to the current parameter (monitoringSlotPeriodicityAndOffset). The right columns shows the actual periodicity in terms of slot groups. Table 2 is defined in the following way: - Keep slot periodicites multiple of X slots (i.e. 4 slots for 480 kHz and 8 slots for 960 kHz) - Consider periodicities not multiple of 4 or 8 slots, for SCS of 480 and 960 kHz as invalid configuration options. - Add periodicities to match monitoringSlotPeriodicities (in terms of absolute time) available for 120 kHz SCS, corresponding to X times the periodicity in 120 kHz SCS case. Proposal 6: Keep the current Search space configuration principle (i.e. slot based configuration for monitoring periodicity and offset) Proposal 7: Redesing monitoringSlotPeriodicityAndOffset in TS 38.331 for 480/960 kHz SCSs​ - Consider monitoring periodicity (slots) resulting in non-integer monitoring periodicity (slot groups) as invalid - Add values to support all periodicitis available for 120 kHs SCS - Introduce periodicites shown in Table 2 in TS 38.331. Proposal 8: UE considers configured SS group (1) monitoring occasions outside the Y slots as invalid Table 2. Monitoring-periodicity (slots and slot groups).

### R1-2200193 (Samsung)

To support multi-slot PDCCH monitoring, a UE would expect that a time gap between any two consecutive PDCCH monitoring occasions in a search space set is not smaller than X_s, while any PDCCH monitoring duration is up to Ys slots. PDCCH monitoring occasions are determined according to configured search space sets, where PDCCH monitoring periodicity ks, and duration Ts, are configured in a number of slots for a search space set s, wherein those parameters should be enhanced to be compatible with the multi-slot PDCCH monitoring capability. Proposal 5: For μ=5 and μ=6 with multi-slot PDCCH monitoring based on combination (Xs, Ys), the PDCCH monitoring periodicity is ks=k⋅Xs, k=1, 2, …, and the PDCCH monitoring duration is Ts≤Ys.

### R1-2200227 (NTT DOCOMO)

Proposal 1: Confirm the following working assumption: Working assumption The following values are adopted as minimum value of Pswitch for 120/480/960 kHz - Support only search space set group switching processing capability 1 with the following values μ; Minimum Pswitch value for UE processing capability 1 [symbols]. 3; 40. 5; 160. 6; 320.

### R1-2200260 (ZTE, Sanechips)

Proposal 4: The search space configuration for 120kHz SCS in FR2-2 including monitoringSlotPeriodicityAndOffset , duration and monitoringSymbolsWithinSlot can be the same as that for 120kHz in FR2-1. Multi-slot PDCCH monitoring is based on slots within a slot group, when configuring the search space set by higher layer parameter monitoringSlotPeriodicityAndOffset and duration, the gNB needs to ensure that PDCCH monitoring periodicity ks, the duration TS is an integral multiple of X slots (X slots consists a slot group). For example, if a slot group includes four slots (X=4), the duration TS can be configured as 4, 8, 12, 16, ... of slots. We can introduce new parameters (e.g. monitoringSlotPeriodicityAndOffset-r17 and duration-r17 ) and add other appropriate periodicity/duration values in addition to the existing configurable values. Moreover, additional restrictions are needed to make sure that the values of periodicity and duration for multi-slot PDCCH monitoring in FR2-2 are an integral multiple of slot groups. Proposal 5: For multi-slot PDCCH monitoring in FR2-2, we suggest introducing new parameters (e.g. monitoringSlotPeriodicityAndOffset-r17 and duration-r17 ) and adding other appropriate periodicity/duration values in addition to the existing configurable values. Moreover, additional restrictions are needed to make sure that the values of periodicity and duration for 480/960 kHz are an integral multiple of slot groups. Besides, for Group (2) SS: Type 1 CSS without dedicated RRC configuration and type 0, 0A, and 2 CSS, SS monitoring locations can be anywhere within a slot group of X slots. It is hard to indicate monitoring occasions by simply using the parameter monitoringSymbolsWithinSlot due to the lack of “monitoring slots location within the slot group” information. A new slot level bitmap (e.g. monitoringSlotsWithinSlotGroup) needs to be introduced to indicate which slots have monitoring occasions winthin a slot group for SCS 480/960 kHz. Proposal 6: A new slot level bitmap (e.g. monitoringSlotsWithinSlotGroup) needs to be introduced to indicate which slots have monitoring occasions winthin a slot group for SCS 480/960 kHz. Proposal 7: Confirm the working assumption: Support only search space set group switching processing capability 1 with the following values μ; Minimum Pswitch value for UE processing capability 1 [symbols]. 3; 40. 5; 160. 6; 320.

### R1-2200290 (Qualcomm)

The goal of the discussion is to let the search space configuration comply with UE’s multi-slot PDCCH monitoring capability (X,Y). In the discussion, two alternative views were identified: - Alt 1: Re-interpretation of the existing configuration parameters, i.e., in the unit of multi-slots (X), - Alt 2: Restrict the configurable values of the parameters, e.g., an integer multiple of X. In our view, re-interpreting the value of the existing RRC parameters (i.e., Alt 1) just for a certain SCS in a certain Release seems against the fundamental principle of scalable numerology of NR. When a feature is extended in a new Release, adding new configuration parameters and values (e.g., monitoringSlotPeriodicityAndOffset-r17, duration-r17, etc.) or introducing some restriction to the configurable values of existing parameters (such that UE only expects that the SS set periodicity is an integer multiple of X) is more common practice in NR. Furthermore, to support Alt 1, it seems necessary that the gNB should indicate the unit ‘X’ to the UE, if the UE supports multiple X values (e.g., 4 and 8 for 960 kHz SCS). However, as in the case of Rel-16 span-based PDCCH monitoring, the explicit indication of X would be unnecessary. Instead, the X value, which complies with one of the (X,Y) combinations that the UE has reported as its capability, can be implied by the search space set configuration, i.e., from the periodicity, offset, and duration. Proposal 2: After a UE reports its supported (X,Y) values for multi-slot PDCCH monitoring for 480/960 kHz SCSs, the UE always expect that the search space set configurations comply with at least one of the reported (X,Y) values. - The interpretation of the fields in searchSpaceSet IE, e.g., monitoringSlotPeriodicityAndOffset, monitoringSymbolWithinSlot, and duration, is not changed for the multi-slot PDCCH monitoring, - A new field of periodicity and offset may be added, e.g., monitoringSlotPeriodicityAndOffset-r17. - New periodicities supported by 480/960 kHz: sl32, sl64, sl128, sl5120, sl10240, and sl20480. In Rel-15/16, the periodicity configuration for some group-common DCI formats is restricted. Thus, for multi-slot PDCCH monitoring in Rel-17, similar restriction should be applied. To maintain the same level of flexibility as 120 kHz SCS in FR2-1, the restricted set of periodicities for 480/960 kHz SCSs may be determined by simple scaling. Proposal 3: For GC DCI formats, the following periodicities are only applicable: 120 kHz (same as FR2); 480 kHz; 960 kHz. DCI format 2_0; sl1, sl2, sl4, sl5, sl8, sl10, sl16, sl20; sl4, sl8, sl16, sl20, sl32, sl40, sl64, sl80; sl8, sl16, sl32, sl40, sl64, sl80, sl128, sl160. DCI format 2_1; sl1, sl2, sl4; sl4, sl8, sl16; sl8, sl16, sl32. DCI format 2_4; sl1, sl2, sl4, sl5, sl8, sl10; sl4, sl8, sl16, sl20, sl32, sl40; sl8, sl16, sl32, sl40, sl64, sl80. * Highlighted: New periodicity values to be introduced for 480/960 kHz SCSs. As we discussed in Section 2.2, however, re-interpretation of the existing RRC parameters just for a certain SCS in a certain Release seems against the principle of scalable numerology of NR. As such, for 480/960 kHz SCSs with multi-slot PDCCH monitoring, it would be desirable to only align the SSSG switching boundary with the slot group boundary, while maintaining the same interpretation of RRC parameters across all SCSs. Proposal 4: For multi-slot PDCCH monitoring for 480/960 kHz SCSs, a boundary of SSSG switching is always aligned with a boundary of a slot group. - The interpretation of configuration parameters and timer operation of SSSG switching is not changed from those of Rel-16 SSSG switching. A text proposal for the proposal is provided in the Appendix. Further related to the joint configuration of multi-slot PDCCH monitoring and SSSG switching, a situation shown in Figure 1 may be considered. That is, at the boundary of SSSG switching, the locations of the two Y consecutive slots in the slot groups before and after the boundary may be different. Thus, the separation between the two Y consecutive slots may be less than X slots. In general, the issue is related to the granularity of configuration: Although the agreement says that “The location of the Y consecutive slots within the slot group of X slots is maintained across different slot groups”, it needs further clarification in which hierarchy of configuration, the statement is applied. For example, the location of the Y consecutive slots within the slot group could be maintained within a SSSG, within a BWP, within a CC, or across CCs of a UE. To avoid such a situation in Figure 1, the location of the Y consecutive slots should be maintained at least across SSSGs. For BWP switching, due to the presence of BWP switching delay, within which the UE does not expect to receive and transmit any channels, the situation in Figure 1 does not occur even if the location of the Y slots changes across BWPs. If the location of the Y slots is maintained across CCs within the same band, the power saving gain would be maximized, while the scheduling flexibility would be somewhat limited. Proposal 5: The location of Y consecutive slots within a slot group is maintained for all Group (1) SS sets within a BWP, regardless of the associated SSSG indices. - FFS: configuration granularity of the location of the Y slots – per BWP, per CC, per UE, etc. Figure 1: An example of SSSG switching for multi-slot PDCCH monitoring.

### R1-2200367 (Intel)

The configured search space configuration in the two SSSGs may have different requirements on the PDCCH monitoring capability. Figure 1 provides two examples for the SSSG switching with corresponding switching between combination (4, 1) and (4, 2). In Figure 1A, at the time for SSSG switching, UE needs to decode more PDCCHs if no additional restriction is introduced. Since the agreed Pswitch values are much larger than the slot group size X=4/8, UE can know in a quite early time that there happens burst PDCCH detections at the time of SSSG switching. As a result, a simple solution is that UE can cancel the PDCCH detection in X slots before the time of SSSG switching. On the other hand, Figure 1B shows another example that there exists a distance between the slots for PDCCH monitoring before or after SSSG switching. There is practically no problem to monitor all PDCCHs around the time of SSSG switching. In this case, it is desired if PDCCH in the first SSSG in the X slots before the time of SSSG switching can still be detected. Based on Figure 1A/1B, a unified solution could be that UE can monitor the PDCCHs in the X slots before the time of SSSG switching in the slots that are the intersection of the slot patterns of combinations (X, Y) used before and after SSSG switching. Figure 1: SSSG switching resulting in burst PDCCH detections The time of SSSG switching after applying Pswitch may not be the boundary of a slot group. Since the second SSSG can provide better scheduling flexibility and the agreed Pswitch are rather large, it would be helpful if the PDCCH monitoring following the second SSSG can start as soon as possible. Therefore, it is preferred that PDCCH monitoring following the second SSSG can start from the time of SSSG switching. It is harmful to wait until the boundary of a slot group for the SSSG switching. Proposal 7: - Dynamic SSSG switching is supported for all SCSs 120, 480 and 960kHz. - The search space set configurations of the two SSSG can correspond to two different PDCCH monitoring capabilities combinations (X, Y) - UE can monitor the PDCCHs in the X slots before the time of SSSG switching in the slots that are the intersection of the slot patterns of the two combinations (X, Y) before and after SSSG switching - PDCCH monitoring following the second SSSG can start right after the time of SSSG switching - Agree on TP 4 to do SSSG switching with different multi-slot PDCCH monitoring capability combinations (X, Y). Proposal 8: - The existing parameters for SS set configuration are reinterpreted as below for both group (1) SS set and group (2) SS set. - monitoringSlotPeriodicityAndOffset: the periodicity and slot offset in a period that is configured with PDCCH MOs for a SS set - duration: multiple of X slots starting from the slot offset configured by monitoringSlotPeriodicityAndOffset. The configured MOs in a slot group repeat in all slot groups in duration. - monitoringSymbolsWithinSlot: A bitmap to indicate the start symbols of PDCCH MOs in one or multiple slots. The index of the first slot within a slot group can be modo,X. The bitmap length can be at least configured from {14, 28, 56, 112} - The configured MOs in a slot group by monitoringSymbolsWithinSlot repeat in all slot groups within the duration slots. - Agree on TP 5 for SS set configuration

### R1-2200401 (Ericsson)

With the existing mechanism, a search space for 120 kHz SCS can, for example, be configured such that it is monitored in the second 10 slots out of every radio frame (80 slots) using monitoringSlotPeriodicityAndOffset = (sl80, 10) duration = 10 This is illustrated in the upper part of Figure 1. Figure 1 Examples of search space configurations for 120 and 480 kHz SCS. However, a similar placement of monitoring occasions once every Xs=4 slot for the 480 kHz SCS case as illustrated in the lower part of Figure 1 cannot be done using the existing search space configuration fields. Indeed, the periodicity part of the existing monitoringSlotPeriodicityAndOffset could be set to 4 to achieve monitoring once every Xs=4 slots. However, since the existing offset and duration fields are both with respect to the periodicity, the configured values would need to be less than 4, meaning that the desired pattern with 10 monitoring locations separated by 4 symbols each would not be achieved. Therefore, there is a need to remedy the existing search space configuration shortfalls for 480 and 960 kHz SCS serving cells. The following configuration parameters are needed: - periodicity in terms of Xs-slot groups - offset in terms of Xs-slot groups within the period - duration in terms of Xs-slot groups within the period - monitoringSlotsWithinSlotGroup, denoted by the set of os,Xs⊂{0,1,…,Xs-1} This parameter is needed to support the RAN1 agreement that, the Ys consecutive slots can be located anywhere within the slot group of Xs slots. - monitoringSymbolsWithinSlot (no change from current specs) There are two possible solutions to implement the above parameters. - Option A is to reinterpret existing RRC fields monitoringSlotsPeriodicityAndOffset and duration as in unit of Xs slots. With this option, only one new field monitoringSlotsWithinSlotGroup will need to be introduced. - Option B is to explicitly introduce new RRC field extensions based on slot groups . The bit string size of monitoringSlotsWithinSlotGroup-r17 can be set to 8 to support both 480 and 960 kHz SCS. For a 480 kHz SCS serving cell, the last four bits are discarded. Depending on if Y = 1, 2, or 4 is configured, the bit string would have 1, 2, or 4 consecutive positions set to '1', respectively. In our view, Option A is an easier way forward for Rel-17 NR operation up to 71 GHz. For example, the monitoring occasions for the 480 kHz SCS as illustrated in the lower part of Figure 1 can be configured with monitoringSlotPeriodicityAndOffset = (sl80, 10) duration = 10 monitoringSlotsWithinSlotGroup-r17 = ’01000000’ In addition to the above two options, some sources discussed extending monitoringSymbolsWithinSlot to support monitoringSlotsWithinSlotGroup. For example, such an approach will have something like monitoringSymbolsWithinSlotGroup-r17 BIT STRING (SIZE (112)) In our view, this approach is undesirable since it requires more than five times the number of bits to signal than either option A or option B. A new RRC parameter monitoringSlotsWithinSlotGroup-r17 needs to be introduced to support RAN1 agreement that, the Ys consecutive slots can be located anywhere within the slot group of Xs slots. RAN1 discusses how to remedy the existing search space configuration shortfalls for 480 and 960 kHz SCS serving cells based on the following two options: - Option A: reinterpret existing RRC fields monitoringSlotPeriodicityAndOffset and duration as in unit of Xs slots; - Option B: explicitly introduce new RRC field extensions based on slot groups. The two working assumptions Working assumption: BD/CCE budget for (4,2), (4,1) is half that of X=8 Working assumption The following values are adopted as minimum value of Pswitch for 120/480/960 kHz - Support only search space set group switching processing capability 1 with the following values μ; Minimum Pswitch value for UE processing capability 1 [symbols]. 3; 40. 5; 160. 6; 320. have been captured and approved in TR 38.213 [3]. In our view, both can be considered confirmed.

### R1-2200410 (Apple)

Proposal 2: There is no need to increase the CORESET duration i.e., the maximum CORESET duration ≤ 3. Proposal 4: Search Space configuration parameters (periodicity, offset and duration) should be defined for multi-slot PDCCH monitoring. The unit of “periodicity”, “offset” and “duration” should be changed to X slots, with default value X=4 for 480 kHz and X=8 for 960 kHz. An additional parameter should be defined for offset within each slot group. In Rel-16, the switching boundary and the timer decrement value are on the order of slots. In the case of MSM PDCCH monitoring, as the PDCCH may be on the order of multiple slots, both the switching boundary and the timer decrements (captures in searchSpaceSwitchTimer) can be modified to be on the order of multi-slots as needed. Alternatively, the values can be modified to account for the change in SCSs and the effect of MSM. The effect of MSM on the transition boundary and the time unit of multiple slots (4 slot) is illustrated in Figure 2. Figure 2: Example of SSSG switching with multi-slot monitoring limitations Proposal 7: The following WA made in RAN1 #107-e should be confirmed: Working assumption The following values are adopted as minimum value of for 120/480/960 kHz - Support only search space set group switching processing capability 1 with the following values Minimum value for UE processing capability 1 [symbols]. 3; 40. 5; 160. 6; 320. Proposal 8: Consider the effect of the change in SCS and of MSS PDCCH monitoring on the searchSapce SwitchTimer and boundary for SSSG switching.

### R1-2200507 (NEC)

For operation in unlicensed band beyond 52.6GHz, in our understanding, the legacy SSSG switching can be reused for 120 kHz SCS, since it has been concluded: for 120 kHz SCS, no multi-slot UE capability for PDCCH monitoring is needed. So the monitoring capability before and after the SSSG switch is the same, both are per-slot based. While for 960 kHz SCS, it was agreed in last meeting [1], there are 2 kinds of PDCCH monitoring capability, X=8 slots and X=4 slots (optionally support). To handle those use cases with low-latency, denser PDCCH monitoring occasion are configured in some period, then PDCCH monitoring capability may be changed along with SSSG switching, e.g. there are 2 configured SSSG, PDCCH is configured to be monitored per 8-slots by the first search space set group, and the second search space set configures PDCCH to be monitored per 4-slots. In this case, the monitoring time unit and capability is different before and after the switching. In our view, SSSG switching along with changing different PDCCH monitoring capability should be supported. Proposal 1: For operation in unlicensed band with 960 kHz SCS, support SSSG switching along with changing different PDCCH monitoring capability. As mentioned in [3], in R16, the switching boundary is the first slot that is at least Pswitch symbols after some switch indication and the timer decrement value is counted by slot. For 480 kHz and 960 kHz SCS, PDCCH monitoring capability is multi-slot based, e.g. 4 slots for 480 kHz SCS and 8 slots for 960 kHz SCS, the switching boundary and the timer counter should be modified to multi-slot based accordingly. Proposal 2: For operation in unlicensed band with 480 kHz and 960 kHz SCS, the switching boundary and the timer counter should be modified to slot group based.

### R1-2200558 (Transsion Holdings)

Regarding the parameter of “monitoringSlotPeriodicityAndOffset” in “SearchSpace”, in NR Rel-15 it means the slots for PDCCH monitoring configured as periodicity and the relevant offset. For each slot group, it is better to restrict the PDCCH monitoring occasions within Y slots. Otherwise, the multi-slot PDCCH monitoring capability will be violated. Therefore, the parameter of period in “SearchSpace” should be an integer multiple of X. Meanwhile, to ensure the PDCCH monitoring occasions are located within the Y slots, the offset values in “SearchSpace” should be associated with the position of Y slots within a slot group. Regarding the parameter of duration in “SearchSpace”, in NR Rel-15 it refers to the number of consecutive slots that a “SearchSpace” lasts in every period. However, for 480kHz SCS and 960kHz SCS, these consecutive slots within a search space period may be located outside the Y consecutive slots within a slot group, which may violet the definition of multi-slot PDCCH monitoring capability. Therefore, a new interpretation of duration should be defined. Considering the period in “SearchSpace” is defined as the integer multiple of X, it is straight forward to interpret the duration as the number of consecutive slot groups that a “SearchSpace” lasts in every period. Proposal 3: For multi-slot PDCCH monitoring, the search space configuration should be in units of slot group. Regarding the parameter Pswitch, in NR-U the value of Pswitch is based on SPS release timeline. For 120kHz SCS, the value of SPS release timeline is 25 symbols. Considering the implementation margin, 40 symbols is reasonable for 120kHz SCS. Then, the values for 480/960kHz SCS can be directly scaled by a factor of 4/8. Furthermore, when SSSG switching operates on a slot group basis, it is also necessary to determine to which slot the switching operation applies. When reusing the Rel 16 rule, if the first slot after the time of Pswitch symbols is in the middle of the slot group, some potential issues may arise. Considering the SS set configuration can be switched in a slot group, the UE may be required to monitor two search space set groups during a slot group, in that case the BD/CCE budget may exceed the the limitation of the UE. As a result, unnecessary search space dropping occurs. Therefore, SSSG switching mechanism should take the slot group as the unit. Proposal 4: Confirm the working assumption that the value of Pswitch should be 40, 160, 320 respectively for 120/480/960 kHz SCS. Proposal 5: SSSG switching mechanism should take the slot group as the unit.

### R1-2200565 (LG)

SS set configuration can also be set appropriately for the multi-slot monitoring. Through SS set configuration based on slot-group, PDCCH monitoring occasion could be adjusted properly and then additional power saving effects would be expected. At the last meeting, it was agreed that all search spaces for Group (1) SS (i.e., Type 1 CSS with dedicated RRC configuration and type 3 CSS, UE specific SS) are monitored within Y slots within X slot-group. In order for the monitoring occasions to be placed at a fixed region (i.e., Y slots) for each slot-group, the SS set configuration parameters such as periodicity, offset and duration should be configured in X units (e.g., to define a new RRC parameter defined only as a multiple of X, or to reinterpret the existing RRC parameters for multi-slot monitoring). One more thing to discuss may be to determine X, which is a criterion for setting the set of values for these parameters (i.e., periodicity, offset, duration) when the UE reports multiple X. For mixed X cases, the set of values for these parameters should be based on smallest X for a UE reporting multiple X values. Proposal #5: Consider to configure PDCCH monitoring occasions to be compliant with the slot-group (or integral multiples of X). Regarding SSSG switching, in Rel-15/16 NR, one SSSG could be switched to another SSSG at the slot boundary after at least P_switch symbols from the switching triggering. However, for the multi-slot monitoring, if SSSG switching occurs at the slot boundary inside the slot-group rather than the slot-group boundary, the complexity of the UE may increase. For example, when a switch occurs within the X slot-group, it may operate as SSSG#0 before the switching and may operate as SSSG#1 after that. At this time, since the BD/CCE budget for 480/960 kHz SCS is checked in units of slot-group, SSSG change in the middle of slot-group may cause to increase UE complexity compared to that for the SSSG switching at slot-group boundary. In this regard, SSSG switching should be performed at the slot-group boundary. Proposal #6: For 480 kHz or 960 kHz multi-slot monitoring, SSSG switching should be performed at the slot-group boundary after at least P_switch symbols from the switching triggering. In Rel-16 NR-U, timer-based SSSG switching was introduced. A UE sets the timer value to the number of slots provided by searchSpaceSwitchTimer when SSSG switching is triggered from the default SS group (SSSG#0) to the other one (SSSG#1). The UE decrements the timer value by one after each slot based on the smallest SCS among all configured DL BWPs in the serving cell. The maximum value of searchSpaceSwitchTimer is 20/40/80 for 15/30/60 kHz, respectively. These are values derived from the maximum COT duration, 20 msec, for the NR-U in Rel-16. In FR2-2, RAN1 agreed P_switch values to support SSSG switching for 120/480/960 kHz as a working assumption. Since the maximum COT duration in FR2-2 was determined to be 5 msec, the maximum value of searchSpaceSwitchTimer can be defined as the number of slots corresponding to that time for each SCS. Proposal #7: Define 40/160/320 slots as the maximum value of searchSpaceSwitchTimer for 120/480/960 kHz SCS, respectively.

### R1-2200654 (Panasonic)

At first glance, it seems feasible to configure all Group (1) SS MOs within Ys. However, one pre-requisite would be the periodicity of all search spaces is an integer number of Xs, upon which there is no agreement yet. Without such restriction on the periodicity, it can happen that some MOs would fall outside the Ys for some slot group. The following figure shows one example, where USS has periodicity of 4 slots (=Xs) but the CSS has periodicity of 6 slots. Fig.1 Infeasibility to restrict all MOs into Ys due to mismatched periodicity As shown in Fig.1, since the location of Ys must be maintained across different slot groups, if it is determined based on the 1st slot group to cover both USS and CSS MOs then it cannot cover the CSS MO in the 2nd slot group. On the other hand, if the location of Ys is determined based on the 2nd slot group to cover both MOs at slots#5 and #6, it cannot cover the CSS MO at slot#0 in the 1st slot group. The dilemma comes from the that fact that the search space periodicity does not match the size of slot group. Therefore, the constraints on periodicity configuration of Group (1) SS should be introduced because it is aligned with the intention of multiple slot monitoring behaviour. Proposal 1: For Group(1) SS configuration, the periodicity can only be integer number of Xs.

### R1-2200671 (Lenovo, Motorola Mobility)

With multi-slot PDCCH monitoring, the search space configuration enhancements are also required in terms of periodicity and indication of monitoring slots within the slot group. For the periodicity configuration, it makes sense to allow only multiple of X i.e. slot group duration to have alignment across slot groups Proposal 2: For supporting NR between 52.6 GHz and 71 GHz with high subcarrier spacing values including 480kHz and 960kHz, support periodicity of PDCCH monitoring in search space configuration only in multiples of slot group duration. Furthermore, currently in search space configuration, a bitmap is configured to indicate the symbols within a slot for PDCCH monitoring occasions within a slot. Similarly for multi-slot PDCCH monitoring, a bitmap to indicate the slots containing PDCCH monitoring occasions should be configured. An example illustration for a 4-slot duration with a 4-bit bitmap for monitoringSlotsWithinGroup indicating “1010” is shown in Figure 1, where the first and third slot within the group have monitoring occasions. Figure 1: Example of monitoring slots indication for a multi-slot PDCCH monitoring Proposal 3: For supporting NR between 52.6 GHz and 71 GHz with high subcarrier spacing values including 480kHz and 960kHz, support configuration of a new bitmap to indicate which slots within the slot group contain monitoring occasions. The size of the bitmap is equal to the number of slots per slot group (i.e. X). The indication of symbols within a slot for PDCCH monitoring applies only to slots indicated by the new bitmap.

## Topic A3: BD Budget/Dropping

### R1-2200143 (CATT)

In RAN1#107-e meeting, the definition for multi-slot PDCCH monitoring capability was agreed. The remaining issue on SS overbooking across different slot groups requires further study. Since Group (2) SS monitoring locations can be anywhere within a slot group, the total number of BD/ CCE for those Group(2) SSs within the current slot group and SSs within the next slot group may exceed the BD/ CCE budgets for a slot group, as shown in Figure 1. In our view, the SS overbooking across different slot groups can be avoided by gNB implementation. No additional dropping rule is needed to solve this problem. Figure 1: SS overbooking across different slot groups Proposal 1: It can be up to gNB implementation to avoid the overbooking issue across different slot groups.

### R1-2200184 (Nokia, Nokia Shanghai Bell)

We propose to convert the working assumption to an agreement. At the same time, we should agree the exact values for the BD/CCE budgets for different multi-slot PDCCH monitoring scenarios. We provide a proposal for the values in Table 1. Table 1. Proposed valus for BD / CCE budget per slot SCS; (X,Y); BD budget per X slots; CCE budget per X slots. 480 kHz; (4,1); 20; 32. 480 kHz; (4,2). 960 kHz; (8,1). 960 kHz; (8,4). 960 kHz; (4,1); 10; 16. 960 kHz; (4,2). Proposal 1: The maximum number of monitored PDCCH candidates per X slots for a single serving cell is 20 - For SCS 480 kHz: (X,Y) = (4,1) - For SCS 480 kHz: (X,Y) = (4,2) - For SCS 960 kHz: (X,Y) = (8,1) - For SCS 960 kHz: (X,Y) = (8,4) Proposal 2: The maximum number of non-overlapped CCEs per X slots for a single serving cell is 32 - For SCS 480 kHz: (X,Y) = (4,1) - For SCS 480 kHz: (X,Y) = (4,2) - For SCS 960 kHz: (X,Y) = (8,1) - For SCS 960 kHz: (X,Y) = (8,4) Proposal 3: The maximum number of monitored PDCCH candidates per X slots for a single serving cell is 10 - For SCS 960 kHz: (X,Y) = (4,1) - For SCS 960 kHz: (X,Y) = (4,2) Proposal 4: The maximum numberof non-overlapped CCEs per X slots for a single serving cell is 16 - For SCS 960 kHz: (X,Y) = (4,1) - For SCS 960 kHz: (X,Y) = (4,2)

### R1-2200227 (NTT DOCOMO)

Since RAN1 has agreed that overbooking is not allowed for CSS even for multi-slot PDCCH monitoring, by defining the maximum number of CCEs as 16, the available configurations for CSS monitoring are restricted. Since type0-PDCCH monitoring can consume all the BD/CCE budgets in a slot group with X=4, a UE cannot monitor other SSs including other CSSs than type0-PDCCH CSS in the same slot group, or configurations, e.g., number of CCEs or aggregation level for other SSs would be limited. In addition, at the last RAN1 meeting, it was pointed out by companies that if two consecutive slots in the same slot group are monitored for type0-PDCCH CSS of SSB/CORESET#0 multiplexing pattern 1, the required number of BD/CCE would be at most 14/56 considering the above requirement for type0-PDCCH CSS monitoring, and thus UE may not be able to monitor the CSS with the BD/CCE budget for X=4/8 slots for SCS 480/960 kHz (i.e., 20 BD and 32 CCE). As a result, it was agreed that the monitored slots for type0-PDCCH CSS is n0 and n0+X0 slots to distribute the monitoring occasions for the CSS into different slot groups. In that sense, it seems fair to ensure enough PDCCH candidates/CCEs configuration flexibility even for X=4 slots for 960 kHz SCS which is supported as an optional UE capability. Therefore, regarding maximum number of CCE for 960 kHz SCS, we suggest considering larger value for X=4 slots, e.g., 28 CCE as maximum, than the simply halved value from that for X=8 slots. Text Proposal #1 -------------------------- Start of Text Proposal for TS 38.213 -------------------------- <Unchanged parts omitted> ##### 10.1 UE procedure for determining physical downlink control channel assignment … Table 10.1-3B provides the maximum number of non-overlapped CCEs, CPDCCHmax,Xs,Ys,μ, for a DL BWP with SCS configuration μ that a UE is expected to monitor corresponding PDCCH candidates for combination Xs,Ys for operation with a single serving cell. Table 10.1-3B: Maximum number CPDCCHmax,(Xs,Ys),μ of non-overlapped CCEs in a slot group for combination Xs,Ys for a DL BWP with SCS configuration μ∈5, 6 for a single serving cell Maximum number of non-overlapped CCEs per combination Xs,Ys and per serving cell CPDCCHmax,(Xs,Ys),μ. μ; (4, 1); (4, 2); (8, 1); (8, 4). 5; 32; 32; -; -. 6; 16 [28]; 16[28]; 32; 32. <Unchanged parts omitted> -------------------------- End of Text Proposal for TS 38.213 --------------------------.

### R1-2200260 (ZTE, Sanechips)

- Proposal 3: Confirm the working assumption: BD/CCE budget of 960 kHz for (4,2), (4,1) is half that of X=8. Table 1: Maximum number MPDCCHmax,(Xs,Ys),μ of monitored PDCCH candidates per slot group for combination Xs,Ys for a DL BWP with SCS configuration μ∈5, 6 for a single serving cell Maximum number MPDCCHmax,Xs,Ys,μ of monitored PDCCH candidates per combination Xs,Ys and per serving cell. μ; (4, 1); (4, 2); (8, 1); (8, 4). 5; 20; 20; -; -. 6; 10; 10; 20; 20. Table 2: Maximum number CPDCCHmax,(Xs,Ys),μ of non-overlapped CCEs in a slot group for combination Xs,Ys for a DL BWP with SCS configuration μ∈5, 6 for a single serving cell Maximum number of non-overlapped CCEs per combination Xs,Ys and per serving cell CPDCCHmax,(Xs,Ys),μ. μ; (4, 1); (4, 2); (8, 1); (8, 4). 5; 32; 32; -; -. 6; 16; 16; 32; 32.

### R1-2200290 (Qualcomm)

In RAN1 #107-e, a relevant issue was discussed in the dynamic spectrum sharing agenda, and the following has been agreed: Agreement - Following approaches for PDCCH monitoring and BD limit handling is supported for Type A UE - Additional simplifications to PDCCH monitoring - Type A UE as per RAN1#105-e agreement and - no simultaneous monitoring between ‘USS sets (for P(S)Cell scheduling) on sSCell’ and ‘Type 0/0A/1/2/CSS sets on P(S)Cell for DCI formats with CRC scrambled by C-RNTI/MCS-C-RNTI/CS-RNTI’ - simultaneous monitoring of ‘USS sets (for P(S)Cell scheduling) on sSCell’ and ‘Type 0/0A/1/2/CSS sets on P(S)Cell for DCI formats with CRC not scrambled by C-RNTI/MCS-C-RNTI/CS-RNTI’. In Short, if there is a USS sets monitored on the sSCell, the UE is not required to monitor DCI formats 0_0 and 1_0 with CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI in the CSS sets in the overlapping slot on the sPCell. The motivation behind the above agreement is removing redundancy and improving power efficiency. Therefore, in the same vein, when the UE is required to monitor both Group (1) and Group (2) SS sets in the same slot group, if there is at least one valid PDCCH MO of Group (1) SS sets, considering overbooking and dropping, the UE may refrain from monitoring DCI formats 0_0 and 1_0 with CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI in Group (2) SS sets. Proposal 1: If PDCCH MOs of both Group (1) and Group (2) SS sets are configured in the same slot group, and there is at least one valid PDCCH MO of USS set(s) after overbooking and dropping, the UE does not monitor DCI formats 0_0 and 1_0 with CRC scrambled by C-RNTI, MCS-C-RNTI, and CS-RNTI in Group(2) SS set(s). In RAN1 #107-e, the following has been agreed: Agreement - SS set overbooking can be allowed with multi-slot PDCCH monitoring capability same as the current specification but applied per slot group, i.e., SS set overbooking is allowed for USS in PCell and PSCell, and UE expects no overbooking for CSS in PCell and PSCell and no overbooking in SCell. - The dropping rule for multi-slot PDCCH monitoring capability is the same as the current specification but evaluated per slot group, i.e., a UE drops UE specific search space set(s) in a slot group with higher index when SS sets are overbooked. - Additional dropping rules are not precluded. For the detailed dropping rules in the third bullet of the agreement, two alternatives were identified: If a SS set has multiple monitoring occasions within a slot group, - Alt 1: All MOs of the SS set shall be dropped as a whole​. - Alt 2: Each MO of the SS set shall be dropped individually​. An example showing the difference of the two alternatives is illustrated in Figure 2. Figure 2: Alternatives of dropping rules. Between the two alternatives, Alt 2 has a benefit over Alt 1 in some cases, like the one shown in Figure 2. That is, Alt 2 may accommodate more PDCCH candidates than Alt 1 under the same BD/CCE budget. However, as an expense, the overbooking procedure may get complicated. Furthermore, the impact of Alt 1 on the existing specification is marginal, since it is a simple extension of Rel-15 overbooking. On the other hand, Alt 2 may require some changes of the specification. Therefore, to keep the design simple and reduce the burden of standardization, Alt 1 would be preferred. Proposal 6: If a SS set to be dropped by overbooking has multiple MOs within a slot group, they are dropped as a whole.

### R1-2200325 (OPPO)

When it comes to SSSG switching, the motivation of introduction of this feature is to allow UE to reduce the PDCCH monitoring effort in the gNB COT. With the similar reasoning as described above, when the transmission beam is not covered by the gNB sensing beam, the gNB is not allowed to perform this transmission within the gNB COT. Therefore, if a CORESET beam is not covered by the gNB sensing beam, the UE should be allowed to skip the PDCCH monitoring in the CORESET. Proposal 4: R17 should allow UE to skip PDCCH monitoring in the CORESET associated with a beam uncovered by the gNB sensing beam within the gNB COT.

### R1-2200367 (Intel)

There was a proposal in early RAN1 discussion that the configuration of MOs in consecutive slots should be avoided. The main concern is that it may increase UE buffering of PDCCH symbols considering one slot with SCS 480/960kHz may not be enough for the PDCCH decoding. The issue of MOs in consecutive slots may happen not only in a slot group, but also across two adjacent slot groups. The agreed framework for multi-slot PDCCH monitoring capability generally allows any kinds of SS sets that can be configured in consecutive slots. It would be complicated to define a dropping rule. We prefer not to define additional solutions to handle SS sets configured in consecutive slots. Proposal 3: - A span of SS sets configured in a slot group, if it is not monitored by the UE, may not be counted in the number of spans in the slot group. - If the maximum number of spans in a slot group is exceeded, UE can drop one or more spans of the group (1) SS - Additional solution to handle SS sets configured in consecutive slots is not preferred - The limitation on number of spans in a slot group should be discussed in UE feature for WI NR_ext_to_71GHz

### R1-2200401 (Ericsson)

The two working assumptions Working assumption: BD/CCE budget for (4,2), (4,1) is half that of X=8 Working assumption The following values are adopted as minimum value of Pswitch for 120/480/960 kHz - Support only search space set group switching processing capability 1 with the following values μ; Minimum Pswitch value for UE processing capability 1 [symbols]. 3; 40. 5; 160. 6; 320. have been captured and approved in TR 38.213 [3]. In our view, both can be considered confirmed.

### R1-2200410 (Apple)

Proposal 3: Additional dropping rules for PDCCH multi-slot monitoring should be defined to limit back-to-back SS monitoring for Group 1 and Group 2 SSs. A window-based approach around the CSS maybe considered to determine whether or not a USS monitoring occasion is dropped or not for PDCCH monitoring Proposal 5: Confirm the following Working Assumption: the BD/CCE budget for (4,2), (4,1) is half that of X=8.

### R1-2200495 (Sharp)

In RAN1#107-e meeting, the working assumption for SSSG switching at 120/480/960kHz is achieved. The working assumption was directly described in 38.213, and the minimum switching time Pswitch was defined as one value for each SCS. Here, we have one concern about the SSSG switching at 480kHz/960kHz, which is that it may cause a back-to-back problem when switching. It is important to avoid this problem, which we have been considering for a long time in past meetings. In the current specification, SSSG-related monitoring is stopped and started at the same time in the first slot after the Pswitch. However, in multi-slot monitoring, SSSG switching may be performed after the UE has monitored in a later slot of the slot group, and monitoring may start in the slot immediately after. In this case, a back-to-back problem occurs. To avoid this situation, a potential solution is that Pswitch is set to different values for staggering timing of stopping current monitoring and starting new monitoring. Figure 1:Possible problem with SSSG switching with multi-slot monitoring. Proposal 3: Adopt Text proposal #2.

### R1-2200558 (Transsion Holdings)

Proposal 1: Confirm the working assumption that the maximum number of PDCCH candidates and the maximum number of non-overlapped CCEs should be 10 and 16 for combination (4, 2), (4, 1). In the current specification, slot-group based PDCCH monitoring capability has been specified and the overbooking rule is performed on slot group basis. With these methods, UE can handle the slot-group based PDCCH detection very well. However, when the PDCCH monitoring occasions change, the UE may need to perform PDCCH detection on consecutive slots belonging to different slot groups, which may result in the UE being unable to perform blind detection in time. For example, for SS/PBCH block and CORESET#0 multiplexing pattern 1, the PDCCH monitoring occasions are located within two consecutive slot groups which are associated with the SS/PBCH block. However, when the UE changes SS/PBCH block it tracks due to mobility, the relevant PDCCH monitoring occasions may be changed based on the slot index of n0. In a special case, for the Type0 CSS or Type0A/Type 2 CSS if “searchSpaceId=0”, the PDCCH monitoring occasions may be changed to the end of the slot group. If the PDCCH monitoring occasions of the Group (1) SS are located in the first slot of an adjacent slot group, then the requirement for the PDCCH detection capability of the UE is almost doubled, which may exceed the BD/CCE limit of the UE. Considering that it is difficult for gNB to avoid this back-to-back issue, it is better to drop the USS in these adjacent slot groups to ensure that the UE’s PDCCH detection budget does not exceed its BD/CCE limit. Proposal 2: Drop the USS with the higher index in these adjacent slot groups, when back-to-back issue happens.

### R1-2200671 (Lenovo, Motorola Mobility)

From the above agreement, there could be the issue of back-to-back monitoring across slot groups e.g. when the beam changes, e.g. with respect to the above proposal when n0 changes and also when the periodicity of Group(2) SS MO is every slot group. Observation 1: For supporting NR between 52.6 GHz and 71 GHz with high subcarrier spacing values including 480kHz and 960kHz, when multi-slot PDCCH monitoring is applied with shifting of Group(2) SS due to n0 change, then potential back-to-back monitoring issue can arise across slot groups where the shift is applied, and periodicity of Group(s) SS MOs is every slot group One potential solution could be to drop any Group(1) SS MOs and/or Group(2) SS MOs in the slot group in which shifting needs to be applied such that back-to-back monitoring issue can be avoided. Proposal 1: For supporting NR between 52.6 GHz and 71 GHz with high subcarrier spacing values including 480kHz and 960kHz, then dropping of Group(1) SS MOs and/or Group(2) SS MOs in the slot where the shift is first applied should be supported to avoid back-to-back monitoring issue.

## Topic B: Multi-Beam Aspects

### R1-2200075 (vivo)

In NR Rel-16, remaining COT information can be indicated to UE in DCI 2_0 for further operation, i.e. CSI-RS validation. However, it is assumed that omni-directional LBT is used for COT initiator. However, directional LBT is introduced to cope with beamforming and directional transmission on the 60GHz unlicensed band. In this case, specifying beam-specific COT information in DCI 2_0 is needed, i.e. the sensing beam direction at gNB side when receiving the COT information. Proposal 7: Specifying beam-specific COT indication in DCI 2_0 for NR operation from 52.6-71GHz when directional LBT is used.

### R1-2200174 (Sony)

As haven been mentioned by multiple companies in previous meetings [4]-[8]: In Rel-16 NR-U, several fields such as CO duration, SS set-group switching trigger, and available RB-sets, were introduced to DCI format 2_0 which is node-specific information because a node does not acquire more than one COT at a time on an unlicensed cell. In the frequency range above 52.6 GHz, independent per-beam LBT sensing has been introduced in RAN1#107-e meeting [9], which implies that a node can acquire multiple COTs in different beams at the same time on an unlicensed cell. If COTs in different beams are independent, that information should be beam-dependent. Therefore, there is a need to consider per beam indication of DCI format 2_0. Proposal 2: Support per beam indication of DCI format 2_0 for above 52 GHz unlicensed operation.

### R1-2200184 (Nokia, Nokia Shanghai Bell)

Support for beam-specific configurations of DCI format 2_0 is not possible in FR2 currently. Although a UE can be indicated a change of active-TCI, DCI format 2_0 PDCCH candidates and payload location remain the same and thus cannot be beam specific. Hence, new field indicating the beams that are relevant for the CO duration and SSGS should be introduced. Given the large number of possible beams and combinations of beams served during the COT, considerable compression of information is needed. One possible is to introduce a beam groups: UE’s TCI states are associated via UE-specific RRC to a certain beam group (which may be seen e.g. as a set of SSB beams) and a bitmap of beam group indicators is introduced to DCI format 2_0. Proposal 9: Beam-specific indication of remaining COT duration and search space group switching in DCI format 2_0 can be supported. - Indicatation can be e.g. a bitmap of beam group indications, where UE’s TCI states are associated to a beam group via RRC signalling.

### R1-2200193 (Samsung)

For 60 GHz unlicensed band, transmissions are expected to be highly directional. To address the channel access efficiency, a transmitter can choose an intended beam direction to perform the channel access procedure, and the sensed result is exclusively applicable to that intended beam direction only. Hence, indicating COT, available RB set, and search space group switching should be associated with the beam direction, wherein such feature was introduced in Rel-16 NR-U by using DCI format 2_0 and in a cell-specific manner. Generalizing the feature to a beam-specific manner is beneficial to address different interference situations along beam directions, and is compatible with the intention to introduce directional LBT. Proposal 8: Support indicating COT, available RB set, and search space group switching in a beam-specific manner for 60 GHz licensed band.

### R1-2200325 (OPPO)

Since gNB can perform directional LBT and RAN1 has agreed that the sensing beam should cover the transmission beams, it implies that the transmission beams within the initiated gNB COT should be covered by the sensing beam. Moreover, the remaining COT duration indication in DCI 2_0 was used in R16 for the UE to determine whether the UE can share the gNB COT and also whether the UE should perform reception on the pre-configured downlink transmissions, e.g. CSI-RS. Thus, it would be more reasonable that the COT sharing as well as the legacy P-CSI-RS reception behavior should be checked by the sensing beam or the transmission beams. For COT sharing mechanism, in the legacy system, the UE can share gNB’s COT as long as the uplink transmission resources in time and frequency domain are within the gNB COT. However, when directional LBT is applied at gNB side, a beam level condition should be added such that the gNB LBT sensing beam should cover the UE’s transmission beam, otherwise, the gNB COT should not be considered sharable by the UE. The ‘cover’ definition can reuse the same definition in the directional LBT procedure. Proposal 1: the UE can share gNB COT only if the UL transmission resources are within the gNB COT and the UL transmission beam is covered by the gNB sensing beam for directional LBT. For P-CSI-RS reception within the gNB COT, in NRU system due to the FR1 frequency range, the UE only checks whether the CSI-RS resources are within the gNB COT to decide if the reception is to be canceled or not. But for FR2-2, if the gNB’s sensing beam does not cover the CSI-RS beam, the CSI-RS is not expected to be transmitted. In this case, if the CSI-RS resources are within the gNB’s remaining COT duration, as long as the gNB sensing beam does not cover the CSI-RS beam, the UE should also cancel the CSI-RS reception. Proposal 2: For higher layer configured CSI-RS reception, the UE performs the reception if the CSI-RS resources are within the gNB COT and the gNB’s sensing beam covers the CSI-RS beam. In the unlicensed spectrum, when COT duration indication is not configured in DCI 2_0, the COT duration is determined by the SFI periodicity. Moreover, the higher layer configured downlink reception, e.g. CSI-RS and PDSCH, is to be confirmed by the SFI indication. In FR2-2 with unlicensed spectrum where LBT is mandatory by the regional regulation, similar to the proposal 2, the confirmation should take into account the gNB sensing beam, because if the sensing beam does not cover the transmission beam, the pre-configured reception should also be canceled, as the gNB is not allowed to transmit in the direction uncovered by the directional LBT. Proposal 3: In FR2-2 unlicensed band, the pre-configured downlink reception is not only confirmed by the SFI indication but also by gNB’s sensing beam, e.g., UE should cancel the downlink reception within the gNB COT if the gNB sensing beam does not cover the downlink transmission beam. When it comes to SSSG switching, the motivation of introduction of this feature is to allow UE to reduce the PDCCH monitoring effort in the gNB COT. With the similar reasoning as described above, when the transmission beam is not covered by the gNB sensing beam, the gNB is not allowed to perform this transmission within the gNB COT. Therefore, if a CORESET beam is not covered by the gNB sensing beam, the UE should be allowed to skip the PDCCH monitoring in the CORESET. Proposal 4: R17 should allow UE to skip PDCCH monitoring in the CORESET associated with a beam uncovered by the gNB sensing beam within the gNB COT.

### R1-2200367 (Intel)

Another proposal under discussion is on beam-specific DCI 2_0 transmission. Considering a FR2-2 cell will work with very narrow beam and the interference on the different narrow beam can be quite different, it is expected the channel occupation status can be different on different narrow beams if LBT is required. Therefore, the DCI format 2_0 can be transmitted in a beam specific manner, which may be applied to both licensed operation and unlicensed operation. Proposal 10: - The DCI format 2_0 can be transmitted in a beam specific manner.

### R1-2200410 (Apple)

In R16 NR-U, DCI format 2-0 is enhanced to carry channel access related information: RB allocation, COT duration and search space set switching indication. While RB allocation information may not be needed depending on LBT bandwidth discussion, the COT duration and SSSG switching should be supported. COT duration and SSSG switching information should be sent at the beginning of the COT as shown in Fig.1. However, current design of DCI format 2-0 transmission limit to one beam per slot. Therefore, it takes multiple slots to finish the beam sweeping transmission of DCI format 2-0. For example, with 120KHz SCS and 32 beams, it takes 4ms to finish beam sweeping. Considering maximum COT duration is 5ms defined by EN 302 567, more efficient transmission scheme of DCI format 2-0 is needed. Proposal 9: Consider enhancement of DCI 2-0 transmission to signal COT duration and SS adaptation at the beginning of the COT.

### R1-2200671 (Lenovo, Motorola Mobility)

Another important aspect for PDCCH monitoring is related to directional LBT. Directional LBT may cause some issues in comparison with omni-directional LBT. For example, different Tx beams used by gNB may correspond to different COTs, thus different CORESETs which are configured with different Tx beams by higher layer signaling may also correspond to different COTs. From power saving perspective, according to the indicated sensing beam(s) used by the gNB to initiate a COT, a UE does not need to monitor PDCCH occasions in a CORESET corresponding to a different Tx beam which is not ‘covered’ by the indicated sensing beam(s) until the current COT ends, which can reduce the power consumption caused by blind decoding. Proposal 4: For NR unlicensed bands between 52.6 GHz and 71 GHz with directional LBT based channel access mechanism, within a COT, PDCCH monitoring is not supported in the CORESETs corresponding to other COTs (PDCCH monitoring restricted to monitoring corresponding to only one COT at a time)

## Topic C: Multi-Cell Operation, Cross-carrier scheduling

### R1-2200075 (vivo)

Therefore, in multi-cell operation scenario, BD/CCE budget calculation becomes more complex by introducing such multi-slot-based BD/CCE budget definition, i.e. more additional cases as described above. The following alternatives could be considered: Alt. 1: Serving cells with the same PDCCH monitoring type including multi-slot-based capability are grouped together for further BD/CCE budget calculation As a straightforward alternative, the BD/CCE budget calculation adopts the same way for NR Rel-16, i.e. serving cells with the same PDCCH monitoring type are grouped together for further handling. Particularly, the follows steps apply: - Determination of NcellsCap: UE needs to report respective NcellsCap for different cases, i.e. Case 1-7 as described above. For the case with mixed capability, L NcellsCap values need to be reported where L is the number of capability types in that case (e.g. 3 in case 7); - Determination of total limit for each group of serving cells: - If the group adopts slot-based or span-based capability, legacy way is used; - If the group adopts multi-slot-based capability, further divide the cell group into different parts depending on SCS and/or value of X/Y. Then BD/CCE budget for the serving cells will follow one total limit. Note that there may have certain limits in the group or part of serving cells. Alt. 2: Transform the serving cell with multi-slot-based capability to equivalent serving cell with slot-based capability for further BD/CCE budget calculation As another alternative, the serving cell with SCS µ and multi-slot-based capability can be transformed to an equivalent virtual serving cell with SCS µ’ and slot-based capability, e.g. e.g. cell A with 480KHz SCS and BD/CCE budget per 4 slots is equivalent to a virtual cell A’ with 120KHz and BD/CCE budget per slot. After this, legacy operation as NR Rel-16 could be reused to calculate the BD/CCE budget. By comparing the above alternatives, Alt. 2 works well with less spec impact. Thus, Alt. 2 is preferred to handle multi-slot PDCCH monitoring capability for multi-serving cell case. Proposal 6: For multi-cell operation, Support the following method to handle multi-slot PDCCH monitoring capability for multi-serving cell case, i.e. transform the serving cell with multi-slot-based capability to equivalent serving cell with slot-based capability for further BD/CCE budget calculation.

### R1-2200143 (CATT)

In the RAN1#106-e, the maximum difference of SCS of cross-carrier scheduling, i.e, uPDCCH-uPDSCH, has been discussed. In the Rel-16, the maximum difference of SCS is equal to 3. However, 480 kHz SCS and 960 kHz SCS have been supported for 52.6-71GHz. We believe the maximum difference of SCS of cross-carrier scheduling should be expanded to support of new SCS, and there is no motivation to limit the difference of SCS of cross-carrier scheduling. Proposal 6：In order to better support cross-carrier scheduling of the new SCS, i.e. 480 kHz and 960 kHz, the difference of SCS of cross-carrier scheduling should not be limited.

### R1-2200193 (Samsung)

When the UE is configured for carrier aggregation operation over more than 2 cells, or for a cell group when the UE is configured for NR-DC operation, the UE does not expect to monitor per group of Xs slots according to combination Xs,Ys a number of PDCCH candidates or a number of non-overlapped CCEs that is larger than the maximum number as derived from the corresponding value of Ncellscap-r17. Proposal 6: Adopt TP#4 for TS 38.213 to determine a capability to monitor a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs per Xs slots that corresponds to Ncellscap downlink cells, i.e. Ncellscap-r17. It was agreed in RAN1#107-e meeting to support PDCCH dropping per Xs slots based on combination Xs,Ys. The PDCCH dropping rule should be applied to both single cell case and CA. For UE configured with CA or NR-DC operation, the PDCCH candidates and non-overlapping CCE limits should be determined based on Ncellscap-r17downlink cells for all scheduling cells. The same principle to determine MPDCCHtotal,(X,Y),μ and CPDCCHtotal,(X,Y),μ for R16monitoringcapability can be reused to determine MPDCCHtotal,(Xs,Ys),μ and CPDCCHtotal,(Xs,Ys),μ for R17monitoringcapability in CA mode. Proposal 7: Adopt TP#5 for TS 38.213 to determine maximum number of PDCCH candidates, MPDCCHtotal,(Xs,Ys),μ, and a maximum number of non-overlapped CCEs, CPDCCHtotal,(Xs,Ys),μ, per Xs slots in CA mode.

### R1-2200227 (NTT DOCOMO)

Regarding the discussion, some companies pointed out that the following aspects regarding band combination should be considered which is described in updated work item description at the RAN#93-e meeting [4]: - Specify gNB and UE RF core requirements for the band(s) in the above frequency range, including a limited set of example band combinations (see Note 1). - For the case of FR2-2 DC or CA with an anchor in FR1 the following three example band combinations shall be considered: - n79 + Nx - n77 + Nx - n41 + Nx - where Nx is the 57-71 GHz band for unlicensed operation and the [66-71] GHz for licensed operation. - RAN4 to further discuss the need for single or multiple bands relevant for FR2-2 licensed/unlicensed operation. Note 1: The WI can be completed provided requirements for at least one band combination involving a new NR-U band is specified as long as it is in line with country-specific regulatory directives.. According to the WID, since the band of n79/n77/n41 are included in FR1, cross-carrier scheduling of a cell within FR2-2 band from/to FR1 band with different numerologies should be considered. In that sense, same as the current specification, no limitation on SCSs is preferable to support cross-carrier scheduling with any SCSs of a cell within FR2-2 from/to FR1 as far as the system works. Proposal 2: To support cross-carrier scheduling of a cell within FR2-2 band from/to a cell of any FR1/FR2-1 band with different numerologies, any SCS combinations for PDCCH and PDSCH should be supported.

### R1-2200260 (ZTE, Sanechips)

As agreed in the RAN1 #107 e-meeting, the Npdsch value of 120 kHz SCS is scaled by 4 and 8 for 480 kHz and 960 kHz SCS respectively to maintain the same absolute time duration as that of 120 kHz SCS in FR2-1. Npdsch in 38.214 Section 5.5 is the UE PDSCH reception preparation time with cross carrier scheduling with different subcarrier spacings for PDCCH and PDSCH. TS 38.214 Table 5.5-1: Npdsch as a function of the subcarrier spacing of the scheduling PDCCH µPDCCH; Npdsch [symbols]. 0; 4. 1; 5. 2; 10. 3; 14. 5; 56. 6; 112. The cross-carrier scheduling here mainly refers to cross-carrier scheduling of a cell within 52.6-71GHz band from/to a cell within FR1/FR2-1. Although a cell within 52.6-71GHz band cross-carrier schedules other cells within FR1/FR2-1 is less likely, it should not be ruled out unless we find enough proof. In our understanding, a UE supporting 480 kHz and 960 kHz SCS supports cross-carrier scheduling. Proposal 8: Cross-carrier scheduling of a cell within 52.6-71 GHz band from/to a cell within FR1/FR2-1 is supported and a UE supporting 480 kHz and 960 kHz SCS supports cross-carrier scheduling.

### R1-2200290 (Qualcomm)

To support both SA and NSA operations efficiently for cells in FR2-2, extending the use of cross-carrier scheduling is necessary. However, when the SCS difference between the scheduling and scheduled cells are very large (e.g., scheduling from 15 kHz SCS to 960 kHz SCS, and vice versa), the gain of cross-carrier scheduling may be harmed, while the design and implementation (e.g., the timeline design and the memory requirement) would be challenging. Therefore, it would be desirable to put some restriction on the selection of SCSs. On the other side, in RAN #93-e, the WID was updated to specify some FR1 + FR2-2 band combinations: - Specify gNB and UE RF core requirements for the band(s) in the above frequency range, including a limited set of example band combinations (see Note 1). - For the case of FR2-2 DC or CA with an anchor in FR1 the following three example band combinations shall be considered: - n79 + Nx - n77 + Nx - n41 + Nx - where Nx is the 57-71 GHz band for unlicensed operation and the [66-71] GHz for licensed operation. - RAN4 to further discuss the need for single or multiple bands relevant for FR2-2 licensed/unlicensed operation.. Considering the band combinations specified above, some extreme situation, such as a FR1 cell with 15 kHz SCS scheduling a FR2-2 cell with 960 kHz SCS, would be unavoidable. Thus, to address the complexity concern, it would be fair to put the supported values of ¦μPDCCH − μPDSCH ¦ to UE’s capability. Then the question is whether we need to define a new UE capability for the supported values of ¦μPDCCH − μPDSCH ¦. Since the supported band combinations for CA is already per UE’s capability, it would be thought that a support for a specific combination of FR1 + FR2-2 bands implies a support for cross-carrier scheduling between them, and no separate capability for ¦μPDCCH − μPDSCH ¦ values are needed. However, although a band combination is supported, the support for cross-carrier scheduling may be dependent on the SCSs of the CCs, due to the complexity issue as discussed above. Therefore, a separate UE capability would be necessary for the supported values of ¦μPDCCH − μPDSCH ¦. Proposal 7: Cross-carrier scheduling of a cell in FR2-2 from/to a cell in FR1 or FR2-1 is supported. - Potential limitation for the values of ¦μPDCCH − μPDSCH¦ is up to UE’s capability.

### R1-2200367 (Intel)

When CA is considered, the total number of BD/CCEs that are shared by multiple cells/TRPs. MPDCCHtotal,(X,Y),μ, CPDCCHtotal,(X,Y),μ need to be determined considering both SCS configuration μ and the combination (X, Y) of the scheduling cells. Three options are discussed as below. - Option 1: MPDCCHtotal,(X,Y),μ is determined and shared by all cells that are configured with scheduling cells having same SCS configuration μ and same combination (X, Y). Based on the agreed framework, the slot group always starts from the subframe boundary. Consequently, the slot groups of difference cells must be aligned if same SCS configuration μ are configured. On the other hand, though the group (1) SS is limited to the Y slots, the group (2) SS can be configured in any slot in a slot group. Since the PDCCH monitoring for the two scheduled cells may not be fully aligned due to the arbitrary MOs for group (2) SS, it is not necessary to align the Y slots in the slot group for multiple cells for BD/CCE sharing. Based on the above discussions, option 1 can be considered without alignment of the Y slots. - Option 2: MPDCCHtotal,(X,Y),μ is determined and shared by all cells that are configured with scheduling cells having SCS configuration μ and same value X in combinations (X, Y). In the option 1, the BD/CCE for the scheduling cells with combination (X, Y) of same value X and different value Y are separately handled. In fact, since the maximum numbers of BD/CCE is only determined by SCS configuration μ and value X, but not the value Y, it is possible to share the BD/CCE for the scheduling cells with same value X of the combinations (X, Y). This option can provide gNB more freedom to share the PDCCH scheduling capability for more cells. - Option 3: MPDCCHtotal,(X,Y),μ is determined and shared by all cells that are configured with scheduling cells having same duration of slot or slot group. A slot of SCS 120kHz and a slot group of X=4/8 slots for SCS 480/960kHz are fully overlapped in time with same the maximum numbers of BD/CCE. It is then expected the UE implementation on the PDCCH detection could be common in some extent. Consequently, it can be considered to share the BD/CCE for the scheduling cells with SCS 120kHz and for SCS 480/960kHz if X=4/8 slots apply. Option 3 can provide even more flexibility on PDCCH transmission than Option 2. However, Option 3 also requires UE to jointly share maximum BD/CCE budget across cells with different numerology, which results in additional complexity. Based on the above analysis, we prefer Option 2 since it provides a good balance between flexibility and complexity. The following TP is proposed to determine MPDCCHtotal,(X,Y),μ, CPDCCHtotal,(X,Y),μ. Proposal 6: - MPDCCHtotal,(X,Y),μ, CPDCCHtotal,(X,Y),μ needs to be determined and shared by all cells that are configured with scheduling cells having same SCS configuration μ and same value X in combinations (X, Y). - Agree on TP 3 to determine MPDCCHtotal,(X,Y),μ, CPDCCHtotal,(X,Y),μ Proposal 9: - CCS is only support for ¦μPDCCH − μPDSCH ¦ ≤ k with k=5 - The limitation on ¦μPDCCH − μPDSCH ¦ ≤ k, k=5 should be captured in UE feature for WI NR_ext_to_71GHz

### R1-2200401 (Ericsson)

MPDCCHtotal,(Xs,Ys),μ and CPDCCHtotal,(Xs,Ys),μ are used but not defined in the approved 38.213 CR [3]: The UE allocates PDCCH candidates for monitoring to USS sets for the primary cell having an active DL BWP with SCS configuration μ in a slot if the UE is not provided monitoringCapabilityConfig for the primary cell or if the UE is provided monitoringCapabilityConfig = r15monitoringcapability for the primary cell, or in the first span of each slot if the UE is provided monitoringCapabilityConfig = r16monitoringcapability for the primary cell, or in a group of Xs slots for a corresponding combination Xs,Ys if the UE is provided monitoringCapabilityConfig = r17monitoringcapability for the primary cell, according to the following pseudocode. If for the USS sets for scheduling on the primary cell the UE is not provided coresetPoolIndex for first CORESETs, or is provided coresetPoolIndex with value 0 for first CORESETs, and is provided coresetPoolIndex with value 1 for second CORESETs, and if minγ∙MPDCCHmax,slot,μ,MPDCCHtotal,slot,μ>minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ or minγ∙CPDCCHmax,slot,μ,CPDCCHtotal,slot,μ>minCPDCCHmax,slot,μ,CPDCCHtotal,slot,μ, the following pseudocode applies only to USS sets associated with the first CORESETs. A UE does not expect to monitor PDCCH in a USS set without allocated PDCCH candidates for monitoring. In the following pseudocode, if the UE is provided monitoringCapabilityConfig = r16monitoringcapability for the primary cell, MPDCCHmax,slot,μ and CPDCCHmax,slot,μ are replaced by MPDCCHmax,(X,Y),μ and CPDCCHmax,(X,Y),μ respectively, and MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ are replaced by MPDCCHtotal,(X,Y),μ and CPDCCHtotal,(X,Y),μ respectively. In the following pseudocode, if the UE is provided monitoringCapabilityConfig = r17monitoringcapability for the primary cell, MPDCCHmax,slot,μ and CPDCCHmax,slot,μ are replaced by MPDCCHmax,(Xs,Ys),μ and CPDCCHmax,(Xs,Ys),μ respectively, and MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ are replaced by MPDCCHtotal,(Xs,Ys),μ and CPDCCHtotal,(Xs,Ys),μ respectively. The quantities can be defined following the same Rel-15 PDCCH monitoring capability allocation for carrier aggregation cases: - If the number of cells configured with combination of Xs,Ys is no more than than Ncellscap, MPDCCHtotal,(Xs,Ys),μ=MPDCCHmax,Xs,Ys,μ and CPDCCHtotal,(Xs,Ys),μ=CPDCCHmax,Xs,Ys,μ. - If the number of cells configured with combination of Xs,Ys is more than than Ncellscap, the number of blind decodes Ncellscap⋅MPDCCHmax,Xs,Ys,μ is distributed to MPDCCHtotal,(Xs,Ys),μ based on a weighting of number of cells with numerology μ among all cells configured with combination of Xs,Ys; and the total number of available channel estimation CCEs Ncellscap⋅CPDCCHmax,Xs,Ys,μ is distributed to CPDCCHmax,Xs,Ys,μ based on a weighting of number of cells with numerology μ among all cells configured with combination of Xs,Ys. Both quantities are still upper bounded by MPDCCHmax,Xs,Ys,μ and CPDCCHmax,Xs,Ys,μ, respectively. MPDCCHtotal,(Xs,Ys),μ and CPDCCHtotal,(Xs,Ys),μ should be defined in TS 38.213 following the same PDCCH monitoring capability allocation for carrier aggregation cases for the case of per-slot monitoring.

### R1-220410 (Apple)

In RAN1 107-e, the following conclusion was made: There seems to be no consensus on introducing a capability for a potential limitations on ¦μPDCCH − μPDSCH¦ for cross-carrier scheduling.. To support cross carrier scheduling, one outstanding issue has to do with the maximum number of carriers that can be simultaneously scheduled from a single carrier given the increase in data rates for the new SCS. This can be defined as a UE capability. Proposal 6: for cross-carrier scheduling, the max number of CCs that can be scheduled from a single CC is reported as UE capability.

### R1-2200459 (Xiaomi)

From previous agreement, UE has a mandatory PDCCH monitoring capability and can also report an optionally supported capability to gNB. So it would be possible that gNB may configure optional PDCCH monitoring capability on some serving cells but leave other cells with mandatory capapbility. And there can be different SCS on different serving cells, so the applied multi-slot PDCCH monitoring capability can also be different. From our point of view, since we have very few time left, it is more preferred not to do PDCCH monitoring capability sharing among multiple serving cells. When UE is configured with multiple serving cells, within each serving cell, UE apply multi-slot PDCCH monitoring capability for a single serving cell independently. Proposal 1: When UE is configured with multiple serving cells, within each serving cell, UE apply multi-slot PDCCH monitoring capability for a single serving cell independently. It is already agreed in R1#107 meeting that BD attempts for Type0-CSS for SSB/CORESET 0 multiplexing pattern 1, and additionally for Type0A/2-CSS if searchSpaceId = 0, occur in slots with index n0 and n0+X0, where n0 is as in Rel-15, X0=4 for 480 kHz SCS and X0=8 for 960 kHz SCS. Related TP can be as follows, TP#2 for TS 38.213 Clause 13 ============================= Unchanged part omitted ========================================= 13 UE procedure for monitoring Type0-PDCCH CSS sets If during cell search a UE determines from MIB that a CORESET for Type0-PDCCH CSS set is present, as described in clause 4.1, the UE determines a number of consecutive resource blocks and a number of consecutive symbols for the CORESET of the Type0-PDCCH CSS set from controlResourceSetZero in pdcch-ConfigSIB1, as described in Tables 13-1 through 13-10, for operation without shared spectrum channel access in FR1 and FR2-1, or as described in Tables 13-1A and 13-4A for operation with shared spectrum channel access in FR1, or as described in Tables 13-10A, 13-10B and 13-10C for FR2-2, and determines PDCCH monitoring occasions from searchSpaceZero in pdcch-ConfigSIB1, included in MIB, as described in Tables 13-11 through 13-15. SFNc and nc are the SFN and slot index within a frame of the CORESET based on SCS of the CORESET and SFNSSB,i and nSSB,i are the SFN and slot index based on SCS of the CORESET, respectively, where the SS/PBCH block with index i overlaps in time with system frame SFNSSB,i and slot nSSB,i. The symbols of the CORESET associated with pdcch-ConfigSIB1 in MIB or with searchSpaceSIB1 in PDCCH-ConfigCommon have normal cyclic prefix. *<omitted text>*. For operation without shared spectrum channel access and for the SS/PBCH block and CORESET multiplexing pattern 1, for FR1 and FR2-1, a UE monitors PDCCH in the Type0-PDCCH CSS set over two consecutive slots starting from slot . For FR2, UE monitors PDCCH in the Type0-PDCCH CSS set over two slots, slot and , where if SCS of the CORESET for Type0-PDCCH CSS is 480kHz, and if SCS of the CORESET for Type0-PDCCH CSS is 960kHz. For SS/PBCH block with index , the UE determines an index of slot as that is in a frame with system frame number (SFN) satisfying if , or in a frame with SFN satisfying if . and are provided by Tables 13-11 and 13-12, and based on the SCS for PDCCH receptions in the CORESET [4, TS 38.211]. The index for the first symbol of the CORESET in slots and and is the first symbol index provided by Tables 13-11 and 13-12. For operation with shared spectrum channel access and for the SS/PBCH block and CORESET multiplexing pattern 1, a UE monitors PDCCH in the Type0-PDCCH CSS set over slots that include Type0-PDCCH monitoring occasions associated with SS/PBCH blocks that are quasi co-located with the SS/PBCH block that provides a CORESET for Type0-PDCCH CSS set with respect to average gain, quasi co-location 'typeA' and 'typeD' properties, when applicable [6, TS 38.214]. For FR1 and FR 2-1, for a candidate SS/PBCH block index i, where 0≤i≤Lmax-1, two consecutive slots starting from slot n0 include the associated Type0-PDCCH monitoring occasions. For FR2, for a candidate SS/PBCH block index i, where 0≤i≤Lmax-1, two slots, slot n0 and , where if SCS of the CORESET for Type0-PDCCH CSS is 480kHz, and if SCS of the CORESET for Type0-PDCCH CSS is 960kHz, include the associated Type0-PDCCH monitoring occasions. The UE determines an index of slot n0 as n0=O⋅2μ+i⋅M mod Nslotframe, μ that is in a frame with system frame number (SFN) SFNC satisfying SFNC mod 2=0 if O⋅2μ+i⋅M/Nslotframe, μ mod 2=0, or in a frame with SFN satisfying SFNC mod 2=1 if O⋅2μ+i⋅M/Nslotframe, μ mod 2=1. M and O are provided by Table 13-11, and μ∈{0, 1} based on the SCS for PDCCH receptions in the CORESET [4, TS 38.211]. The index for the first symbol of the CORESET in slots n0 and n0+1 is the first symbol index provided by Table 13-11. The UE does not expect to be configured with M=1/2, or with M=2, when NSSBQCL=1. For the SS/PBCH block and CORESET multiplexing patterns 2 and 3, a UE monitors PDCCH in the Type0-PDCCH CSS set over one slot with Type0-PDCCH CSS set periodicity equal to the periodicity of SS/PBCH block. For the SS/PBCH block and CORESET multiplexing patterns 2 and 3, if the active DL BWP is the initial DL BWP, the UE is expected to be able to perform radio link monitoring, as described in Clause 5, and measurements for radio resource management [10, TS 38.133] using a SS/PBCH block that provides a CORESET for Type0-PDCCH CSS set. For a SS/PBCH block with index , the UE determines the slot index and based on parameters provided by Tables 13-13 through 13-15. ============================= Unchanged part omitted =========================================

### R1-2200540 (MediaTek)

Consequently, it is natural to link the slot group notion in 480kHz or 960kHz and the slot notion in 120kHz, and it is desirable to reuse the multi-cell BD/CCE budget calculation method specified for slot-based PDCCH monitoring when determining the multi-cell BD/CCE budget calculation method for slot-group based PDCCH monitoring. That is, when the number of scheduled cells is less than or equal to the number of cells UE can support, UE is not required to monitor more than the BD/CCE limit per slot-group specified for a single cell on a DL BWP of scheduling cell. When the number of scheduled cells is larger than the number of cells UE can support, the multi-cell BD/CCE budget per slot group across CCs UE will follow can be derived based on the multi-cell BD/CCE budget calculation method specified for slot-based PDCCH. Proposal 2: The Rel-15/16 multi-cell BD/CCE budget calculation method specified for slot-based PDCCH monitoring should be considered as the baseline for the multi-cell BD/CCE budget calculation for slot-group based PDCCH monitoring in 480kHz and 960kHz. The other open issue is the location of the Y slots within a slot group of X slots across CCs. Compared with the fixed location of the Y slots within a slot group across CCs, non-aligned locations of Y slots can provide potential scheduling flexibility with the cost of less power saving. For example, without any restriction, it is possible that the monitoring slots in a cell with (X,Y) multi-slot PDCCH monitoring configuration might be close to the monitoring slots in another cell with the same (X,Y) multi-slot PDCCH monitoring configuration, which is illustrated in Figure 1. In this example, both CCs follow (X=4,Y=1) multi-slot PDCCH monitoring configuration and UE has to monitoring PDCCH in different slots across CCs without any chance of micro-sleep due to the non-aligned PDCCH monitoring pattern. On the other hand, if the PDCCH monitoring pattern is aligned across CCs, UE will monitor the same slots across CCs and improve PDCCH monitoring power consumption accordingly. Proposal 3: For multi-cell operation, UE can report a capability on whether the location of the Y slots within a slot group of X slots is maintained across CCs associated with (X,Y) configuration. Figure 1 Example of non-aligned monitoring pattern across CCs

### R1-2200565 (LG)

According to this, the serving cells with the same PDCCH monitoring type can be grouped together for the BD/CCE limit calculation. That is, the serving cells for slot-based monitoring and span-based monitoring are treated as separate groups when calculating the BD/CCE limit. If three different monitoring operations are processed as separate groups in a CA situation including multi-slot monitoring, the number of cases becomes complicated, which may increase the complexity of the UE. Meanwhile, RAN1 agreed that X=4/8 for 480/960 kHz SCS (resp.) is supported as mandatory and other values of X as optional. For X=4/8 for 480/960 kHz, the absolute time of the slot-group and the BD/CCE limit per X=4/8 are the same as those for 120 kHz. In this case, when the multi-slot monitoring is configured with multiple serving cells with X=4/8 slots for 480/960 kHz, the BD/CCE limit calculation for cells with these SCS can be calculated as if they are cells with 120 kHz SCS. With this, BD/CCE limit calculation for CA can be simplified even in a situation where multi-slot monitoring is added. Additionally, the number of DL cells for multi-slot monitoring may be reported by pdcch-BlindDetectionCA, whose value indicates total the number of DL cells for both slot based monitoring and multi-slot based monitoring. Through this, the UE will be able to further simplify BD/CCE calculation for the multi-cell operation. Proposal #8: When a UE is configured with DL cells greater than the reported number of DL cells, consider followings for BD/CCE budget calculation, - For a serving cell with mandatory X (i.e., X=4/8 for 480/960 kHz), BD/CCE budget is calculated by transforming the serving cell to the cell with 120 kHz SCS. - For a serving cell with optional X (e.g., X=4 for 960 kHz), BD/CCE budget is calculated by grouping the serving cell with other cells having the same PDCCH monitoring type including multi-slot-based capability.

## Topic D: Other

# List of submitted TDocs

The following TDocs have been used to compile above summary:

R1-2200045	Remaining issues of PDCCH monitoring enhancement for 52-71GHz spectrum	Huawei, HiSilicon

R1-2200060	Remaining issues for PDCCH monitoring enhancements	InterDigital, Inc.

R1-2200075	Remaining issues on PDCCH monitoring enhancements for NR operation from 52.6GHz to 71GHz	vivo

R1-2200143	Remaining issues on PDCCH monitoring enhancements for up to 71GHz operation	CATT

R1-2200174	PDCCH enhancement for NR from 52.6GHz to 71GHz	Sony

R1-2200184	PDCCH monitoring enhancements	Nokia, Nokia Shanghai Bell

R1-2200193	Maintenance on PDCCH monitoring enhancements for NR from 52.6 GHz to 71 GHz	Samsung

R1-2200227	Remaining issues on PDCCH monitoring enhancements for NR in FR2-2	NTT DOCOMO, INC.

R1-2200260	Remaining issues on the PDCCH monitoring enhancements for 52.6 to 71GHz	ZTE, Sanechips

R1-2200290	PDCCH monitoring enhancements for NR in 52.6 to 71GHz band	Qualcomm Incorporated

R1-2200325	Discussion on remaining issue for PDCCH monitoring enhancement	OPPO

R1-2200367	Discussion on PDCCH monitoring enhancements for extending NR up to 71 GHz	Intel Corporation

R1-2200401	PDCCH Monitoring Enhancements	Ericsson

R1-2200410	On remaining issues for PDCCH Monitoring	Apple

R1-2200459	Remaining issues on PDCCH monitoring enhancement for NR 52.6-71GHz	xiaomi

R1-2200495	PDCCH monitoring enhancements	Sharp

R1-2200507	Remaining issues on PDCCH enhancement for NR operation from 52.6GHz to 71GHz	NEC

R1-2200540	Remaining discussion on PDCCH monitoring enhancement for 52.6-71 GHz NR operation	MediaTek Inc.

R1-2200558	Remaining issues of PDCCH monitoring enhancements for above 52.6GHz	Transsion Holdings

R1-2200565	PDCCH monitoring enhancements to support NR above 52.6 GHz	LG Electronics

R1-2200654	PDCCH monitoring for NR operation from 52.6 to 71 GHz	Panasonic

R1-2200671	Remaining issues on PDCCH for NR from 52.6 GHz to 71GHz	Lenovo, Motorola Mobility