3GPP TSG-RAN WG1 Meeting #107bis-e 	R1-2200830

e-Meeting, January 17–25, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR |  | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on NR sidelink enhancement in 38.212 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2021-1-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Capture new agreements made in RAN1#107bis-e for NR sidelink enhancement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Reflect the working assumption related on SCI format 1-A. - Reflect agreements related to SCI format 2-C. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification for NR sidelink enhancement is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1.1, 8.4, 8.4.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | 38.213, 38.214 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

8.3.1.1	SCI format 1-A

SCI format 1-A is used for the scheduling of PSSCH and 2nd-stage-SCI on PSSCH

The following information is transmitted by means of the SCI format 1-A:

-	Priority – 3 bits as specified in clause 5.4.3.3 of [12, TS 23.287] and clause 5.22.1.3.1 of [8, TS 38.321]. Value '000' of Priority field corresponds to priority value '1', value '001' of Priority field corresponds to priority value '2', and so on.

-	Frequency resource assignment – log2(N subChannel SLN subChannel SL + 12) bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 2; otherwise log2(N subChannel SLN subChannel SL + 12N subChannel SL + 16) bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 3, as defined in clause 8.1.5 of [6, TS 38.214].

-	Time resource assignment – 5 bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 2; otherwise 9 bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 3, as defined in clause 8.1.5 of [6, TS 38.214].

-	Resource reservation period –log2Nrsv_period bits as defined in clause 16.4 of [5, TS 38.213], where Nrsv_period is the number of entries in the higher layer parameter sl-ResourceReservePeriodList, if higher layer parameter sl-MultiReserveResource is configured; 0 bit otherwise.

-	DMRS pattern – log2Npattern bits as defined in clause 8.4.1.1.2 of [4, TS 38.211], where Npattern is the number of DMRS patterns configured by higher layer parameter sl-PSSCH-DMRS-TimePatternList.

-	2nd-stage SCI format – 2 bits as defined in Table 8.3.1.1-1.

-	Beta_offset indicator – 2 bits as provided by higher layer parameter sl-BetaOffsets2ndSCI and Table 8.3.1.1-2.

-	Number of DMRS port – 1 bit as defined in Table 8.3.1.1-3.

-	Modulation and coding scheme – 5 bits as defined in clause 8.1.3 of [6, TS 38.214].

-	Additional MCS table indicator – as defined in clause 8.1.3.1 of [6, TS 38.214]: 1 bit if one MCS table is configured by higher layer parameter sl-Additional-MCS-Table; 2 bits if two MCS tables are configured by higher layer parameter sl- Additional-MCS-Table; 0 bit otherwise.

-	PSFCH overhead indication – 1 bit as defined clause 8.1.3.2 of [6, TS 38.214] if higher layer parameter sl-PSFCH-Period = 2 or 4; 0 bit otherwise.

-	Reserved – a number of bits as determined by the following:

-	Nreserved bits as configured by higher layer parameter sl-NumReservedBits, with value set to zero, if higher layer parameter indicationUEBScheme2 is not configured, or if higher layer parameter indicationUEBScheme2 is configured and set to 'Disabled';

-	(Nreserved-1) bits otherwise, with value set to zero.

-	Second UE flag –  0 or 1 bit

-	1 bit if higher layer parameter indicationUEBScheme2 is configured and set to 'Enabled', where the bit value of 0 indicates that the UE cannot be a second UE and the bit value of 1 indicates that the UE can be a second UE as defined in Clause 16.3.0 of [5, TS 38.213];

-	0 bit otherwise.

< Unchanged parts are omitted >

8.4	Sidelink control information on PSSCH

SCI carried on PSSCH is a 2nd-stage SCI, which transports sidelink scheduling information, and/or inter-UE coordination related information.

< Unchanged parts are omitted >

8.4.1.3	SCI format 2-C

SCI format 2-C is used for the decoding of PSSCH, and providing inter-UE coordination information or requesting inter-UE coordination information.

The following information is transmitted by means of the SCI format 2-C:

-	HARQ process number – 4 bits

-	New data indicator – 1 bit

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2

-	Source ID – 8 bits as defined in clause 8.1 of [6, TS 38.214]

-	Destination ID – 16 bits as defined in clause 8.1 of [6, TS 38.214]

-	HARQ feedback enabled/disabled indicator – 1 bit as defined in clause 16.3 of [5, TS 38.213]

-	Providing/Requesting indicator – 1 bit, where value 0 indicates SCI format 2-C is used for providing inter-UE coordination information and value 1 indicates SCI format 2-C is used for requesting inter-UE coordination information

If the 'Providing/Requesting indicator' field is set to 0, all the remaining fields are set as follows:

-	Resource combination(s) – 3∙log2(N subChannel SLN subChannel SL + 12N subChannel SL + 16)+9+log2Nrsv_period  bits as defined in Clause 8.1.5A of [6, TS 38.214], where N subChannel SL and Nrsv_period are defined in Clause 8.3.1.1 for SCI format 1-A.

-	First resource location(s) – x bits as defined in Clause 8.1.5A of [6, TS 38.214].

-	Reference slot location – x bits as defined in Clause 8.1.5A of [6, TS 38.214].

-	Resource set type – 1 bit, where value 0 indicates preferred resource set and value 1 indicates non-preferred resource set, if higher layer parameter determineResourceSetTypeScheme1 is configured to 'UE-A’s implementation' or higher layer parameter interUECoordinationScheme1Condition is configured to 'enable'; otherwise, 0 bit.

If the 'Providing/Requesting indicator' field is set to 1, all the remaining fields are set as follows:

-	Priority – 3 bits as specified in clause 5.4.3.3 of [12, TS 23.287] and clause 5.22.1.3.1 of [8, TS 38.321]. Value '000' of Priority field corresponds to priority value '1', value '001' of Priority field corresponds to priority value '2', and so on.

-	Number of subchannels – log2N subChannel SL bits as defined in Clause 8.1.4A of [6, TS 38.214].

-	Resource reservation period – log2Nrsv_period bits as defined in Clause 8.1.4A of [6, TS 38.214].

-	Resource selection window location – x bits as defined in Clause 8.1.4A of [6, TS 38.214].

-	Resource set type – 1 bit, where value 0 indicates a request for inter-UE coordination information providing preferred resource set and value 1 indicates a request for inter-UE coordination information providing non-preferred resource set, if higher layer parameter determineResourceSetTypeScheme1 is configured to 'UE-B’s request'; otherwise, 0 bit.

-	Padding bits, if required.

Zeros shall be appended to SCI format 2-C of which 'Providing/Requesting indicator' field is set to 1 until the payload size equals that of SCI format 2-C of which 'Providing/Requesting indicator' field is set to 0.

< Unchanged parts are omitted >