3GPP TSG RAN WG1 Meeting #108-e	 R1-2200915

e-Meeting, February 21 – March 3, 2022

Agenda Item:	8.13.2

Source:	Huawei, HiSilicon

Title:	Discussion on efficient activation/de-activation mechanism for SCells

Document for:	Discussion and Decision

# Introduction

In RAN1#107-e meeting, RAN1 provided two example sets of RRC parameters for Alt 1 and Alt 2 respectively to facilitate the RAN2 decision on the two alternatives. In RAN2#116bis-e meeting, RAN2 discussed MAC CE and RRC signaling for TRS based SCell activation and made the following agreements [1].

Use alt1 with one octet used for TRS ID (including gap length if not configured by RRC).

In addition, RAN2 has provided the MAC CR [2] and RRC CR [3] for TRS based SCell activation. However, RAN2 would like to confirm whether RAN2’s understanding is correct and whether there is any limitation in TRS configuration for fast SCell activation in Rel-17 which needs to be captured in RAN2 spec. Thus, RAN2 asks RAN1 to consider the above agreements and corresponding MAC CR and RRC CR.

[To RAN1]: RAN2 respectfully asks RAN1 to consider the above RAN2 agreements, question and corresponding RRC CR and MAC CR.

# Discussion

Considering simple and flexible triggering of temporary RS per each of SCells individually, we support the agreement provided by RAN2 that Alt 1 with bitmap approach in MAC-CE is preferable for SCell activation. Regarding to the RRC CR and MAC CR, one question is about the max number of RS configurations for SCell activation based on Alt 1. RAN2 seems to have different opinion from what was agreed at RAN1#107-e meeting [4], where companies proposed that maximum number of TRS configurations supported per SCell is 15 assuming that a value zero indicated by a bit block in the MAC-CE means no RS resource transmitted.

- RAN1: max number of TRS configurations per SCell is 15.

- RAN2: max number of TRS configurations per SCell is 255.

As agreed in RAN1#107-e meeting, the max number of NZP CSI-RS resource set configurations for temporary RS per serving cell is the same as current maxNrofNZP-CSI-RS-ResourceSetsPerConfig. In RAN1’s understanding, a list of TRS configurations from which one configuration can be indicated by MAC-CE has a limited size of 16, for the consideration of limited overhead of MAC-CE. The list of 16 TRS configurations is associated with TRS configuration ID whose maximum value is equal to maxNrofCSI-ResourceConfigurations. Diagram is shown in Figure1. Benefit of this structure is that only 4-bit blocks are need for addressing the TRS ID per SCell, which has small bit overhead in MAC CE.

Figure 1 Diagram of Alt1 discussed in RAN1’s meeting

From the perspective of RAN2, the relationship between configurations indicated in MAC CE and temporary RS configuration ID in RRC is one-to-one. This means maximum number of temporary RS configuration for SCell activation and corresponding max temporary RS configuration ID have same value 255 and 8-bit blocks are need in MAC CE for addressing the TRS ID per SCell. Diagram of parameter structure is shown in Figure 2. This scheme can allow more configuration combinations, providing more flexibility.

Figure 2 Scheme diagram agreed by RAN2

From the above analysis, it can be seen that if maximum 15 TRS configurations agreed in RAN1 is supported per SCell, 4-bit blocks are need for addressing the TRS ID per SCell, which will lead to MAC CE design misalignment and lose some flexibility. Additionally, flexible gaps between two TRS bursts needs more TRS configurations that can be indicated by the MAC CE. Therefore, we agree with the proposal of RAN2 that TRS ID could be defined as 8 bits instead and corresponding max number of TRS configurations per SCell is extended to 255.

Proposal 1: Confirm the agreements and corresponding RRC CR and MAC CR from RAN2.

Regarding the question raised in RAN2 LS [1], as copied below, the trs-info is expected to be “TRUE”. There is no additional RAN2 spec impact identified for any other limitation.

In RAN2’s understanding, the trs-info in NZP-CSI-RS-ResourceSet will be set to TRUE if the CSI-RS for tracking is the temporary RS for fast SCell activation. Q1: RAN2 would like to confirm whether RAN2’s understanding is correct and whether there is any limitation in TRS configuration for fast SCell activation in Rel-17 which needs to be captured in RAN2 spec?

Proposal 2: To reply the RAN2 LS R1-2200890, the trs-info in NZP-CSI-RS-ResourceSet will be set to TRUE in case of fast SCell activation. There is no additional RAN2 spec impacts identified for any other limitation.

# Conclusion

According to the above discussions, we have the following proposal:

Proposal 1: Confirm the agreements and corresponding RRC CR and MAC CR from RAN2.

Proposal 2: To reply the RAN2 LS R1-2200890, the trs-info in NZP-CSI-RS-ResourceSet will be set to TRUE in case of fast SCell activation. There is no additional RAN2 spec impacts identified for any other limitation.

# References

R1-2200890/R2-2201715, “LS on RAN2 agreements for TRS-based Scell activation”.

R2-2201713, “38321 CR Introduction of TRS based SCell activation”.

R2-2201714, “38331 CR Introduction of TRS based SCell activation”.

R1-2112981, “Summary of LS on triggering signaling of temporary RS”.

# Appendix

Agreements in RAN1#107-e meeting.

Agreement The max number of NZP CSI-RS resource set configurations for temporary RS per serving cell is the same as current maxNrofNZP-CSI-RS-ResourceSetsPerConfig. Agreement For efficient SCell activation with assistance of temporary RS, a SSB P-TRS of the to-be-activated SCell is to be configured as a QCL source for the temporary RS in case of known SCell same as existing specification. - Note: a SSB of the to-be-activated SCell is a QCL source for the P-TRS per existing specification - Note: It is RAN1 understanding that Scell activation latency can be reduced compared to Rel-16 even when P-TRS is configured as QCL source for the temporary RS in case of known SCell Below Working Assumption does not need to be confirmed. Working Assumption For efficient SCell activation with assistance of temporary RS, a SSB of the to-be-activated SCell can be indicated as a QCL source for the temporary RS in case of known SCell - FFS: QCL type - FFS: the case of unknown SCell - FFS: other QCL source, e.g. the SSB/P-TRS of another active cell Agreement For both Alt 1 and Alt 2 of temporary RS triggering, - For Alt 1, the gap between temporary RS bursts is explicitly configured. - A set of possible gap lengths from which the triggering MAC-CE can indicate one from RAN1 perspective. Up to RAN2 to decide details. - For Alt 2, a gap length is configured by RRC for each temporary RS having two bursts. For different temporary RS, the value of the gap length can be different based on RRC configuration. - the number of bursts is up to 2. It can be either explicitly configured, or implicitly indicated by the gap configuration ((Up to RAN2 to decide one) Agreement For Alt 2 of temporary RS triggering, to avoid potential impact on the existing CSI-AperiodicTriggerStateList, a separate trigger-state list is used. - Note: it does not imply that Alt 2 has been selected by RAN2. Agreement For the RRC and MAC-CE designs of temporary RS triggering (both Alt1 and Alt2), from functionality perspective, the max number of to-be-activated SCells should be 15, irrespective of triggered number of temporary RS bursts per cell.  Note: UE capability for the max number of to-be-activated SCells with 2-burst temporary RS is not precluded.