3GPP TSG-RAN WG1 Meeting #108-e 	R1-2200919

e-Meeting, February 21 – March 3, 2022

Agenda Item:	8.16.6

Source:	Huawei, HiSilicon

Title:	Rel-17 UE features for RedCap

Document for:	Discussion and Decision

Introduction

In this contribution, we provide our considerations for Rel-17 NR RedCap UE feature regarding SSB.

Discussion

RAN1 introduces NCD-SSB for serving cell purpose. Although NCD-SSB is introduced for performing RRM/RLM/BFD measurement in an active BWP, RAN4 has reached conclusion that it is feasible for a RedCap UE to perform L1/L3 measurement based on CSI-RS and/or FG 6-1a by reporting optional capabilities, if the RRC-configured active DL BWP has no SSB in connected mode [1]. The previous RAN1 agreements state that there is an operation without need of NCD-SSB based on support of CSI-RS and/or FG 6-1a. Since the basic FG 6-1 also addresses CORESET#0, and it is not preferable to use an existing FG with different interpretation, for RedCap UEs. Also, IODT is an aspect that needs to be considered, therefore a signaling bit might be desirable. We suggest to introduce a new FG as mandatory for RedCap UE to report whether it can expect NCD-SSB in the active BWP, e.g. to perform RRM/RLM/beam measurement of the serving cell. Also, it is required for indication of support if a UE does not report either CSI-RS based measurements nor FG 6-1a, as per the RAN1 agreements.

Proposal 1: For RedCap, define a new FG to report whether a UE supports NCD-SSB based measurements in the active BWP or not, if neither FG 6-1a nor CSI-RS based measurement is supported.

Conclusions

In this contribution, we presented our views on UE features for Rel-17 RedCap UEs:

Proposal 1: For RedCap, define a new FG to report whether a UE supports NCD-SSB based measurements in the active BWP or not, if neither FG 6-1a nor CSI-RS based measurement is supported.

- References

- R4-2202773, “Reply LS on use of NCD-SSB or CSI-RS in DL BWPs for RedCap UE”, RAN4#101b-e.