3GPP TSG-RAN WG1 Meeting #108-e	 									R1-2200928

e-Meeting, February 21 – March 3, 2022

Agenda Item:	8.16.10

Source:	Huawei, HiSilicon

Title:	Remaining issues of Rel-17 IAB UE feature

Document for:	Discussion and Decision

Introduction

In R1-2200780, a list of UE features on IAB was provided. In this contribution, we share some of our considerations on Rel-17 IAB UE feature including some remaining issues.

Discussion

UL power control

The support of desired IAB-MT UL PSD range reporting is an important feature for IAB node to operate simultaneous Tx. By reporting the UL PSD range, the parent node can schedule the IAB-MT with UL Tx transmission power levels which is favorable for IAB-DU to transmit at the same time. For FG 31-7, the name of the feature and the description are still in brackets.

Proposal 1: Remove the brackets in FG 31-7 as follows

| 31. NR_IAB_enh | 31-7 | Desired UL TX power adjustment | Support Desired IAB-MT PSD range reporting |  | Yes | N/A | Desired MT’s UL PSD range reporting is not supported. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Dynamic indication of Rel-17 or FDM soft resource availability

For the dynamic FDM soft resource availability indication, the following was agreed in RAN1#107-e:

Agreement For DCI format 2_5 indicating availability for the soft resources of the respective RB sets corresponding to a given time resource of the child IAB-DU cell: - AvailabiltyCombination can be extended to include multiple resourceAvailabilty, where each resourceAvailabilty includes availability indication for one RB set group - One RB set group consists of one or multiple RB sets

FG 31-8 is in brackets since it was unclear then whether there will be any enhancement to the dynamic indication of IAB-DU soft resource. Given the above agreement, in Rel-17, “AvailabiltyCombination” can be extended to include multiple resourceAvailabilty, where each resourceAvailabilty includes availability indication for one RB set group. Hence we propose:

Proposal 2: For FG 31-8, adopt the following changes

- Remove the brackets for [31-8]

- Remove the bracket and “or” for [Dynamic indication of Rel-17 or FDM soft resource availability]

| 31. NR_IAB_enh | 31-8 | Dynamic indication of Rel-17 FDM soft resource availability | Support monitoring DCI Format 2_5 scrambled by AI-RNTI for indication of FDM soft resource availability to an IAB node |  | Yes | N/A | The IAB-node is unable to receive explicit availability indication for Soft resources | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Conclusions

In this paper, we observe and propose:

Proposal 1: Remove the brackets in FG 31-7 as follows

| 31. NR_IAB_enh | 31-7 | Desired UL TX power adjustment | Support Desired IAB-MT PSD range reporting |  | Yes | N/A | Desired MT’s UL PSD range reporting is not supported. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 2: For FG 31-8, adopt the following changes

- Remove the brackets for [31-8]

- Remove the bracket and “or” for [Dynamic indication of Rel-17 or FDM soft resource availability]

| 31. NR_IAB_enh | 31-8 | Dynamic indication of Rel-17 FDM soft resource availability | Support monitoring DCI Format 2_5 scrambled by AI-RNTI for indication of FDM soft resource availability to an IAB node |  | Yes | N/A | The IAB-node is unable to receive explicit availability indication for Soft resources | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |