3GPP TSG RAN WG1 Meeting #108e	R1-2200934

e-meeting, February 21 – March 3, 2022

Agenda Item:	8.1.3

Source:	Huawei, HiSilicon

Title:	Remaining issues on SRS enhancement in Rel-17

Document for:	Discussion and decision

Introduction

According to the agreements in last meeting [1], most aspect of SRS enhancement in Rel-17 was completed, while there are still some remaining issues. In this paper, we share our views on the remaining issues and provide corresponding text proposals.

On SRS antenna switching

Guard period

There is one remaining issue for the guard period discussion: How to handle the case where the interval between SRS resource sets is larger than Y. In the current spec, the UE behaviour for two SRS resource sets in the consecutive slots is missing. There are two understandings:

- Alt.1: When the interval between two SRS resource sets is larger than Y, there is no scheduling restriction on the symbols.

- Alt.2: When the interval between two SRS resource sets is larger than Y, no data could be scheduled on all of the symbols.

For such case, LS from RAN4  (R4-2202413) indicates that RAN4 thinks not sending PUSCH and PUCCH between SRS resources belonging to different sets where usage is set to antenna switching is inefficient. Actually,  the interval between two sets on consecutive slots could be more than 20 symbols in the case of two SRS resource sets in two consecutive slots. If these large number of OFDM symbols could not be used for any signals transmission due to the guard period Y=1 or 2 symbols, it will significantly impact the throughput. So, we prefer to define the UE behavior with Alt.1.

Proposal 1: If the interval between SRS resource sets is larger than Y, there is no scheduling restriction.

Text Proposal 1: We have the following text proposal for TS 38.214 V17.0.0:

< Start of the text proposal > #### 6.2.1.2 UE sounding procedure for DL CSI acquisition < Unchanged parts are omitted > The UE is configured with a guard period of Y symbols, in which the UE does not transmit any other signal, in the case the SRS resources of a set are transmitted in the same slot. The guard period is in-between the SRS resources of the set. For two SRS resource sets of an antenna switching located in two consecutive slots, if UE is capable of transmitting SRS in all symbols in one slot, a guard period of Y symbols exists between the last OFDM symbol occupied by the SRS resource set in the first slot and the first OFDM symbol occupied by the SRS resource set in the second slot. When the interval between the last OFDM symbol occupied by the SRS resource set in the first slot and the first OFDM symbol occupied by the SRS resource set in the second slot is larger than Y symbols, there is no scheduling restriction for the symbols in the interval, otherwise, UE does not transmit any other signal in the symbols. < End of the text proposal >

Text proposal on SRS configuration for T=R antenna switching in 38.214

Reason of changes:

For antenna switching T=R case, in Rel-15, up to two SRS resource sets are supported, including 2 periodic, or two aperiodic, or 1 periodic+1 aperiodic, etc. In Rel-17, when UE has the capability of “Maximum 2 SP and 1 periodic SRS sets for antenna switching”, the legacy configurations should be maintained, i.e., the configurations of 2 periodic, or 2 aperiodic, or 1 periodic+1 aperiodic should be supported. However, the mentioned legacy SRS configurations are missing for Rel-17 UE in the current wording of spec.

Consequence without changes:

For T=R, the legacy configurations of 2 periodic, or 2 aperiodic, or 1 periodic+1 aperiodic are missing for Rel-17 UE with the capability of “Maximum 2 SP and 1 periodic SRS sets for antenna switching”.

Proposed changes:

Text Proposal 2: We have the following text proposal for TS 38.214 V17.0.0

< Start of the text proposal > #### 6.2.1.2 UE sounding procedure for DL CSI acquisition < Unchanged parts are omitted > - For 1T=1R, or 2T=2R, or 4T=4R, up to two SRS resource sets each with one SRS resource, where the number of SRS ports for each resource is equal to 1, 2, or 4. If the UE is indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets],two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' and one SRS resource set configured with resourceType in SRS-ResourceSet set to 'periodic' also can be configured, where each SRS resource set has one SRS resource, the number of SRS ports for each resource is equal to 1, 2, or 4, and the two SRS resource sets configured with 'semi-persistent' are not activated at the same time, or < End of the text proposal >

On SRS capacity/coverage enhancement

Further restriction on applicable cases for partial sounding

In previous meetings, RB-level partial sounding was agreed and most of the aspects, including candidate values of PF, sequence design, RB location hopping, etc., were completed. The only one remaining issue is that whether further restriction on applicable cases is needed. In our understanding, the main design purpose of partial sounding is to reduce the SRS overhead and boost the power spectral density by only transmitting SRS in partial contiguous RBs in one symbol. So no matter whether frequency hopping is enabled or not, partial sounding works well and can increase SRS capacity and coverage. For the non-hopping case, some company claim that Rel-15 can achieve the same patterns as partial sounding by selecting suitable configuration parameters. However, Rel-15 can’t cover all the patterns supported by partial sounding. For instance, the bandwidth of partial sounding can be 38 RBs by configuring mSRS,BSRS=152 and PF = 4, which is not covered by Rel-15. So it’s unnecessary to further restrict the applicable cases for partial sounding.

Proposal 2: No need to restrict applicable cases for partial sounding, i.e., partial sounding is applicable for both frequency hopping and non-frequency hopping cases.

Text Proposal 3: We have the following text proposal for TS 38.211 V17.0.0:

< Start of the text proposal > #### 6.2.1.2 Mapping to physical resources < Unchanged parts are omitted > - kF∈0,1,…,PF-1 is given by the higher-layer parameter StartRBIndex if configured, otherwise kF=0; - khop is given by Table 6.4.1.4.3-3 with khop=nSRSb'=bhopBSRSNb' mod PF Nbhop=1 if the higher-layer parameter EnableStartRBHopping is configured, otherwise khop=0. If NBWPstart≤nshift the reference point for k0(pi)=0 is subcarrier 0 in common resource block 0, otherwise the reference point is the lowest subcarrier of the BWP. < End of the text proposal >

Conclusion

In this contribution, we discussed the remaining issues on SRS enhancements in Rel-17 and provide following proposals:

Proposal 1: If the interval between SRS resource sets is larger than Y, there is no scheduling restriction.

Proposal 2: No need to restrict applicable cases for partial sounding, i.e., partial sounding is applicable for both frequency hopping and non-frequency hopping cases.

Text Proposal 1: We have the following text proposal for TS 38.214 V17.0.0:

< Start of the text proposal > #### 6.2.1.2 UE sounding procedure for DL CSI acquisition < Unchanged parts are omitted > The UE is configured with a guard period of Y symbols, in which the UE does not transmit any other signal, in the case the SRS resources of a set are transmitted in the same slot. The guard period is in-between the SRS resources of the set. For two SRS resource sets of an antenna switching located in two consecutive slots, if UE is capable of transmitting SRS in all symbols in one slot, a guard period of Y symbols exists between the last OFDM symbol occupied by the SRS resource set in the first slot and the first OFDM symbol occupied by the SRS resource set in the second slot. When the interval between the last OFDM symbol occupied by the SRS resource set in the first slot and the first OFDM symbol occupied by the SRS resource set in the second slot is larger than Y symbols, there is no scheduling restriction for the symbols in the interval, otherwise, UE does not transmit any other signal in the symbols. < End of the text proposal >

Text Proposal 2: We have the following text proposal for TS 38.214 V17.0.0

< Start of the text proposal > #### 6.2.1.2 UE sounding procedure for DL CSI acquisition < Unchanged parts are omitted > - For 1T=1R, or 2T=2R, or 4T=4R, up to two SRS resource sets each with one SRS resource, where the number of SRS ports for each resource is equal to 1, 2, or 4. If the UE is indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets],two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' and one SRS resource set configured with resourceType in SRS-ResourceSet set to 'periodic' also can be configured, where each SRS resource set has one SRS resource, the number of SRS ports for each resource is equal to 1, 2, or 4, and the two SRS resource sets configured with 'semi-persistent' are not activated at the same time, or < End of the text proposal >

Text Proposal 3: We have the following text proposal for TS 38.211 V17.0.0:

< Start of the text proposal > #### 6.2.1.2 Mapping to physical resources < Unchanged parts are omitted > - kF∈0,1,…,PF-1 is given by the higher-layer parameter StartRBIndex if configured, otherwise kF=0; - khop is given by Table 6.4.1.4.3-3 with khop=nSRSb'=bhopBSRSNb' mod PF Nbhop=1 if the higher-layer parameter EnableStartRBHopping is configured, otherwise khop=0. If NBWPstart≤nshift the reference point for k0(pi)=0 is subcarrier 0 in common resource block 0, otherwise the reference point is the lowest subcarrier of the BWP. < End of the text proposal >

- References

3GPP, “RAN1#107e Meeting Chairman’s Notes”, E-meeting, November 11 – November 19, 2021.