3GPP TSG-RAN WG1 Meeting #108-e	       R1-2200941

e-Meeting, February 21 – March 3, 2022

Agenda Item:	8.15.1

Source:	Huawei, HiSilicon

Title:	Maintenance on time and frequency synchronization enhancement for IoT in NTN

Document for:	Discussion and Decision

# Introduction

This contribution discusses the remaining issues on enhancements to time and frequency synchronization for IoT in NTN, including UL segmentation for NPUSCH and NPRACH.

# Remaining issues on time and frequency synchronization enhancements

In RAN1#107-e, RAN1 discussed the enhancements on UL transmission segmentation of NPUSCH/NPRACH for NB-IoT and the following agreements are made.

Agreement UL transmission segment duration with one value X per NPUSCH for NB-IoT and PUSCH/PUCCH for eMTC may be indicated on SIB. … Agreement At least UL transmission segment duration with one value X for NPRACH for NB-IoT and PRACH for eMTC may be indicated on SIB. … Agreement For NB-IoT NTN, the network configures one of K values for the UL transmission segment duration of each PRACH preamble format in a k-bit field, where the size of the k-bit field and the number of K candidate values depend on the preamble format. - Format 0 and format 1: 3-bit field, K=6 candidate values 2.4.(TCP+TSEQ), 4.4.(TCP+TSEQ), 8.4.(TCP+TSEQ), 16.4.(TCP+TSEQ), 32.4.(TCP+TSEQ), 64.4.(TCP+TSEQ) - Format 2: 3-bit field, K=5 candidate values 1.6.(TCP+TSEQ), 2.6.(TCP+TSEQ), 4.6.(TCP+TSEQ), 8.6.(TCP+TSEQ), 16.6.(TCP+TSEQ) .

From the agreements, the exact segment duration value K when transmitting NPRACH preambles is agreed to be signaled in SIB with a 3-bit field indicating different values with respect to different preamble formats, where the value X should be captured in TS 36.133 and the corresponding description should be included in TS 36.213 at Clause 16.1.2. So, we have the following proposal:

Proposal 1: Capture TP#1 in clause 16.1.2 in TS 36.213.

=========================== Start of TP #1 for TS 36.213 =============================== 16.1.2 Timing synchronization ---------------------------------------------------- Unchanged Text Omitted ------------------------------------------------------- If a segment duration is configured by higher layer parameter TransmissionDurationNPRACH-NB-r17, the UE is expected to adjust the TTA per segment during the transmission of narrowband physical random access preamble. If a segment duration is configured by higher layer parameter TransmissionDurationNPUSCH-NB-r17, the UE is expected to adjust the TTA per segment during the transmission of narrowband physical uplink shared channel. ============================== End of TP #1 for TS 36.213 ===============================

In RAN1 #107-e [1], the following agreement were made for UL time and frequency synchronization for IoT in NTN:

Agreement: Confirm the working assumption made at RAN1#106-bis-e on serving satellite ephemeris bit allocations for LEO/MEO/GEO based non-terrestrial access network: - Support serving satellite ephemeris format bit allocations for LEO/MEO/GEO based non-terrestrial access network: - Position and velocity state vector ephemeris format is 17 bytes payload. - The field size for position (m) is 78 bits - Position range is driven by GEO : +/- 42 200 km - The quantization step is 1.3m for position - The field size for velocity (m/s) is 54 bits - Velocity range is driven by LEO@600 km: +/- 8000 m/s - The quantization step is 0.06 m/s for Velocity

According to our companion paper [2], the velocity range is calculated wrong in the above agreement, therefore we proposed the following observation and proposal.

Observation 1: The velocity range (+/- 8000 m/s) is not correct for the current agreement.

Proposal 2: The velocity range of +/- 8000 m/s should change to [-7864,7863] according to the bit allocation and granularity.

# Conclusion

In this contribution, we further discuss the issue of UL time and frequency synchronization enhancement for IoT in NTN, the following observations and proposals are presented:

Observation 1: The velocity range (+/- 8000 m/s) is not correct for the current agreement.

Proposal 1: Capture TP#1 in clause 16.1.2 in TS 36.213.

Proposal 2: The velocity range of +/- 8000 m/s should change to [-7864,7863] according to the bit allocation and granularity.

# References

[1] RAN1 #107bis-e, chairman notes.

[2] R1-2200938, Maintenance on UL time and frequency synchronization enhancement for NTN, Huawei, HiSilicon.