3GPP TSG-RAN WG1 Meeting #108-e	         R1- 2200958

e-Meeting, February 21 – March 3, 2022

Agenda Item:	8.16.2

Source:	Huawei, HiSilicon

Title:	Rel-17 UE features for extension to 71 GHz

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss our views on the UE features for extension to 71GHz according to Updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e[1].

Discussion on the feature group table

- FG24-1b and FG24-4b: Wideband PRACH

Note:

In RAN1#107bis-e, there were discussion and no consensus on whether the FG24-1b and FG24-4b should be restricted in unlicensed band. Although the scope of WID [2] might not be crystal clear depending on different interpretation from companies, the main motivation to introduce longer PRACH sequence in RAN1 is to make full use of UE TX power under the restriction of power spectrum density required by regional unlicensed band regulations. On the other hand, concentrating the transmit power in narrower bandwidth by power control mechanism is more efficient than introducing long PRACH sequence in licensed band. So we propose to remove the bracket on the sentence of “[Note: This FG is only supported in bands for shared spectrum operation]” in the note column for FG24-1b and FG24-4b.

There was discussion on whether FG24-1b should be supported if UE report the capability of FG24-2 (120KHz SSB support for initial access in FR2-2). To our understanding, it is not necessary because all basic uplink capabilities to fulfil the initial access procedures have been captured in FG24-1a, which is already the prerequisite of 24-2. Moreover, FG24-2 is applied for both licensed band unlicensed band while FG24-1b is only for unlicensed band from our perspective. The sentence of “[A UE that supports FG 24-2 must indicate this FG is supported]” should be deleted from the note column of FG24-1b.

Proposal 1: Remove the bracket on the sentence of “[Note: This FG is only supported in bands for shared spectrum operation]” in the note column for FG24-1b and FG24-4b

Proposal 2: Delete the sentence of “[A UE that supports FG 24-2 must indicate this FG is supported]” in the note column of FG24-1b.

- FG24-1c: Multi-RB support PUCCH format 0/1/4

Note:

In the note column, there is still one pending issue of “[A UE that supports [24-1a/24-2/FR2-2] must indicate this FG is supported]”.  According to the agreement in RAN1#107bis-e, FG24-1c is only applicable to “bands under PSD limitation in shared spectrum operation”. However, FG24-1a, FG24-2 or even the whole FR2-2 would include both licensed band and unlicensed band. It is not necessary for UE only support licensed band to report this capability. For the UE operating on the unlicensed band without PSD limitation, this FG is also redundant. Thus, we propose to delete the sentence of “[A UE that supports [24-1a/24-2/FR2-2] must indicate this FG is supported]”in the note column.

Proposal 3: Delete the sentence of “[A UE that supports [24-1a/24-2/FR2-2] must indicate this FG is supported]” in the note column of FG24-1c.

- FG24-1d and FG24-1e：multiple PDSCH/PUSCH scheduling by single DCI for 120 kHz SCS

Note:

In RAN1#107bis-e, several companies proposed to extend the support of multiple PDSCH/PUSCH scheduling by single DCI to other frequency ranges, e.g. FR2-1 and FR1. We also share the similar view because these FGs can reduce UE implementation complexity and power consumption on PDCCH monitoring while maintain the high throughput. Moreover, there were also precedents in NRU Rel-16. For example, multiple PUSCH scheduling by single DCI was originally introduced for unlicensed band in FR1 and then extended to licensed band in FR1 and FR2-1. The designs in FR2-2 in Rel-17 are following the same design as in Rel-16 except for allowing discontinuous resource allocation in time domain.

Proposal 4: Support to extend FG24-1d and FG24-1e to FR2-1 and FR1.

- FG24-4/24-4f and FG24-5/5f: 480/960 kHz SCS support for DL and multi slot PDCCH monitoring capability

Component:

Considering the reduced monitoring occasion within X slot group, support of multi PDSCH/PUSCH scheduling with single DCI is essential to maintain the peak throughput. We support to remove FFS before the 3rd component for both FG24-4 and FG24-5.

Agreement

Clarify earlier agreement as follows:

- A UE capable of multi-slot monitoring mandatorily supports monitoring Group (2) SSs according to FG 3-1 within each of the Xs slots of a slot-group, such that:

  - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of each slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within each slot of the slot group.

- Continue discussion on whether or not introducing other limitation for Group (2) SSs in RAN1#108-e.

Following agreement on Group (2) SS monitoring for UE with multi slot PDCCH monitoring capability has been reached in RAN1#107bis-e. Thus, the sentence of “(FFS: Monitoring capability within slots of type 1 CSS without dedicated RRC configuration and type0, 0A, and 2 CSS)” in FG24-4 and FG24-5 can be replaced with the yellow highlighted sentence in the agreement considering the guidance in the note column “FFS: component description without a reference to other R15 FGs”.

Further, if Group (2) SS monitoring capability is already described in FG24-4 and FG24-5, it does not need to be repeated in the corresponding advanced FG capabilities FG 24-4f and FG 24-5f. Therefore, we suggest to remove “(FFS: Monitoring capability within slots of type 1 CSS without dedicated RRC configuration and type0, 0A, and 2 CSS)” from FG 24-4f and FG 24-5f.

In RAN1#107e and RAN1#107bis-e, there is no consensus to introduce multi slot PDCCH monitoring capability with slot group of X=2 slots. Comparing with the already support capability of (Xs,Ys)=(4,2), the capability of (Xs,Ys)=(2,1) requires UE to at most monitor 4 occasions every 4 slots and every 2 monitoring occasions locate in the same slot. The UE complexity is increased significantly while the benefit is unclear.   So we propose to change the component description back to “Multiple-slot PDCCH monitoring for 480KHz with (Xs,Ys) = (4,2)”.

Proposal 5: In FG24-4 and FG24-5, remove FFS before 3rd component.

Proposal 6: In FG 24-4 and FG 24-5, replace the sentence of “FFS: Monitoring capability within slots of type 1 CSS without dedicated RRC configuration and type0, 0A, and 2 CSS” with the following agreement text as a separate component. “For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of each slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within each slot of the slot group.”

Proposal 7: In FG 24-4f and FG 24-5f, remove “FFS: Monitoring capability within slots of type 1 CSS without dedicated RRC configuration and type0, 0A, and 2 CSS”.

Proposal 8: In FG24-4f, component candidate value of (2,1) is not supported. The component 1 should be changed to support the optional capability with (Xs,Ys)=(4,2).

- FG24-6 and FG24-7: type 1/2 channel access procedure in uplink for FR2-2

Component:

In the AI8.2.6, the discussion on the LBT bandwidth is still ongoing. The controversial issue focuses on whether the LBT bandwidth should be equal to the active BWP or the channel bandwidth including the active BWP. In TS37.213, the terminology of “channel” is used and defined as “A channel refers to a carrier or a part of a carrier consisting of a contiguous set of resource blocks (RBs) on which a channel access procedure is performed in shared spectrum.”  It is extended from “carrier” in early LTE LAA because NR-U allows carrier bandwidth larger than 20MHz which is mandated by regulation for LBT bandwidth. Although a channel in 37.213 is described as a set of contiguous RB, it does not imply the LBT can be implemented with the granularity of RB because LBT is usually performed in time domain due to short response time and a sensing slot (9us in FR1) is not aligned with OFDM symbol.  Thus, it is further clarified in TS38.214 and TS38.101-1 that the RB set corresponds to 20MHz LBT bandwidth required by regulation.  Similarly in FR2-2, considering the sensing slot (5us) is not aligned with OFDM symbol, defining LBT bandwidth exactly equal to active BWP is hardly implementable even if there is no regulatory requirement on LBT bandwidth.  Moreover, it is not cost efficient to support various LBT bandwidths according to every configurable active BWP. So we propose to update “per carrier/BWP” as “per channel including active BWP” in the component of 24-6 and 24-7.

Proposal 9: Update “per carrier/BWP” as “per channel including active BWP” in the component of 24-6 and 24-7.

- FG24-8 and FG24-9: 32 HARQ processes

Component:

Agreement

- In NR FR2-2, a UE supporting 32 maximum number of HARQ processes for 480/960 kHz SCS for DL (or for UL) shall support 32 as the maximum number of HARQ processes for 120 kHz SCS for DL (or UL), subject to UE capability.

In RAN1#107bis-e, the following agreement is achieved on the support of 32 HARQ processes for 120kHz SCS.

Considering UE will or will not support 32 HARQ processes for all supported SCS in FR2-2, it is not necessary to differentiate the FG from numerologies. Therefore, we propose to at least remove the text “for 480/960 kHz” in the component description in FG24-8 and FG24-9.

The support of 32 HARQ processes was also introduced in NTN WI (FG26-5) for NTN cell in FR1 and FR2-1. It is under discussion under NTN UE feature whether such capability can be extended to other non-NTN cell. The answer should obviously be yes since it was also agreed to support 32 HARQ processes for FR2-2 as part of this WI. Moreover, if the support of multiple PDSCH/PUSCH scheduling by single DCI is extended to bands outside of FR2-2, as in NRU Rel-16, the support of 32 HARQ processes should be extended together to avoid HARQ processing starvation. So we think the FG26-5 discussed in NTN WI can be applied to all numerologies in both FR1 and FR2. The FG24-8 and FG24-9 are overlapping with FG26-5.

Proposal 10: Remove “for 480/960kHz” in the component of FG24-8 and FG24-9.

Observation 1: FG24-8 and FG24-9 are overlapping with FG26-5 (Increasing the number of HARQ processes) discussed in NTN WI. If FG26-5 were reported “per band” and defined independently of the numerologies and the feature (i.e. not limited to NTN or 60 GHz, etc.), FG24-8 and FG24-9 could be removed.

- FG24-10: Additional beam switching time delay

Following note was included in UE feature LS to RAN2 :

Note that in NR_ext_to_71GHz, FG 24-10 “Additional beam switching time delay”, RAN1 is still discussing whether to define default values or whether UEs need to signal this FG is supported if one or more other FG is supported.

In Rel-17, additional beam switching time delay was agreed to be specified as d=28 symbols for 120 kHz. Majority of companies were supportive of the idea of following the modus operandi in Rel-17 to support the same absolute time for the timeline related parameters in 120 kHz and 480 kHz and, as such, specify d=112 symbols for 480 kHz. However, some companies had a concern that d=112 would be excessively long and proposed to support d=56. As a compromise, RAN1 agreed to support a UE capability for this parameter with the candidate values of {56, 112}. Aligned with the practice throughout Rel-17, we think that the default value of d=112 should be supported for this FG so, in case that UE does not report this capability, the same timeline as in 120 kHz can be preserved.  Additionally, if UE does not report this FG and there is no default value for it, gNB may assume d=0 for 480 kHz. This would certainly be a problematic assumption since d is specified to be a non-zero value for all other numerologies and is agreed to be either 56 or 112 for 480 kHz.

We do not see any value in reporting this feature per band and we think that this should be reported per UE.

Finally, there is a sentence “[Additional beam switching time delay is not supported]” in the column “Consequence if the feature is not supported by the UE” (this sentence is actually included in the LS in the neighboring column by mistake). We think this sentence is misleading as the additional beam switching time delay as always supported (its value cannot be zero). The only question is whether and how to define a default value for it.

Proposal 11: For FG24-10, support the default value of d=112.

Proposal 12: For FG24-10, support “Per UE” report.

Proposal 13: For FG24-10, remove “[Additional beam switching time delay is not supported]”.

Conclusions

Based on the discussion above, we had following observation and proposals.

Proposal 1: Remove the bracket on the sentence of “[Note: This FG is only supported in bands for shared spectrum operation]” in the note column for FG24-1b and FG24-4b

Proposal 2: Delete the sentence of “[A UE that supports FG 24-2 must indicate this FG is supported]” in the note column of FG24-1b.

Proposal 3: Delete the sentence of “[A UE that supports [24-1a/24-2/FR2-2] must indicate this FG is supported]” in the note column of FG24-1c.

Proposal 4: Support to extend FG24-1d and FG24-1e to FR2-1 and FR1.

Proposal 5: In FG24-4 and FG24-5, remove FFS before 3rd component.

Proposal 6: In FG 24-4 and FG 24-5, replace the sentence of “FFS: Monitoring capability within slots of type 1 CSS without dedicated RRC configuration and type0, 0A, and 2 CSS” with the following agreement text as a separate component. “For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of each slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within each slot of the slot group.”

Proposal 7: In FG 24-4f and FG 24-5f, remove “FFS: Monitoring capability within slots of type 1 CSS without dedicated RRC configuration and type0, 0A, and 2 CSS”.

Proposal 8: In FG24-4f, component candidate value of (2,1) is not supported. The component 1 should be changed to support the optional capability with (Xs,Ys)=(4,2).

Proposal 9: Update “per carrier/BWP” as “per channel including active BWP” in the component of 24-6 and 24-7.

Proposal 10: Remove “for 480/960kHz” in the component of FG24-8 and FG24-9.

Observation 1: FG24-8 and FG24-9 are overlapping with FG26-5 (Increasing the number of HARQ processes) discussed in NTN WI. If FG26-5 were reported “per band” and defined independently of the numerologies and the feature (i.e. not limited to NTN or 60 GHz, etc.), FG24-8 and FG24-9 could be removed.

Proposal 11: For FG24-10, support the default value of d=112.

Proposal 12: For FG24-10, support “Per UE” report.

Proposal 13: For FG24-10, remove “[Additional beam switching time delay is not supported]”.

- References

- R1-2200780, “Updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e”, RAN1#107bis-e, Moderators (AT&T, NTT DOCOMO, INC.)

- RP-213540, “Revised WID:  Extending current NR operation to 71 GHz”, Qualcomm

- R1-2200781, “LS on updated Rel-17 RAN1 UE features list for NR”, RAN1#107bis-e, Moderators (AT&T, NTT DOCOMO, INC.)

- Appendix: proposed changes to the UE feature list corresponding to [1]

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-1b | Wideband PRACH for 120 kHz in FR2-2 | Enhanced PRACH design for operation by adopting a single long ZC sequence, with ZC sequence equal to 1151 for 120kHz and ZC sequence equal to 571 for 120kHz | 24-1a | Yes | N/A | Wideband PRACH for 120 kHz in FR2-2 is not supported | Per band | N/A | N/A | N/A | Note: This FG is only supported in bands for shared spectrum operation | Optional withcapability signalling |
|  | 24-1c | Multi-RB support PUCCH format 0/1/4 for 120 kHz in FR2-2 | 1. Support multi-RB PUCCH format 4 for 120 kHz 2. Support multi-RB PUCCH format 0/1 for 120 kHz | 24-1a | Yes | N/A | Multi-RB support PUCCH format 0/1/4 for 120 kHz in FR2-2 is not supported | Per band | N/A | N/A | N/A | This FG is only supported in bands under PSD limitation in shared spectrum operation | Optional with capability signalling |
|  | 24-1d | Multiple PDSCH scheduling by single DCI for 120kHz | 1. Multi-PDSCH scheduling by single DCI for the operation with 120 kHz SCS 2. HARQ enhancements | 24-1 | Yes | N/A | Multiple PDSCH scheduling by single DCI for 120kHz is not supported | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
|  | 24-1e | Multiple PUSCH scheduling by single DCI for 120kHz | 1. Multi-PUSCH scheduling by single DCI for the operation with 120 kHz SCS | 24-1a | Yes | N/A | Multiple PUSCH scheduling by single DCI for 120kHz is not supported | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
|  | 24-4 | 480KHz SCS support for DL | 1. 480KH SCS for DL data and control channels, SSB, and reference signal reception in FR2-2 for non-initial access 2. Multiple-slot PDCCH monitoring for 480KHz with (Xs,Ys) = (4,1) 3. Multi- PDSCH scheduling by single DCI for the operation with 480 kHz SCS and corresponding HARQ enhancements 4. Within the Ys = 1 slot, monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS with a maximum of two monitoring spans per slot with set2 = (4, 3) and (7, 3) symbols where set2 is defined in FG3-5b 5. For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of each slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within each slot of the slot group. 6. Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for FDD (This supersedes corresponding component of FG 3-5b) 7. Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for TDD (This supersedes corresponding component of FG 3-5b) | 24-1 | Yes | N/A | 480KHz SCS for DL is not supported | Perband | N/A | N/A | N/A | FFS: component description without a reference to other R15 FGs | Optional with capability signalling |
|  | 24-4b | Wideband PRACH for 480 kHz in FR2-2 | PRACH with 480KHz and length 571 | 24-4a | Yes | N/A | Wideband PRACH for 480 kHz in FR2-2 is not supported | Per band | N/A | N/A | N/A | Note: This FG is only supported in bands for shared spectrum operation | Optional with capability signalling |
|  | 24-4f | Enhanced PDCCH monitoring for 480KHz in FR2-2 | 1. Multiple-slot PDCCH monitoring for 480KHz with (Xs,Ys)=(4,2) 2.) Within each of the Ys = 2 slots, monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS in the first 3 OFDM symbols of each slot | 24-4 | Yes | N/A | Enhanced PDCCH monitoring for 480KHz in FR2-2 is not supported | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
|  | 24-5 | 960KHz SCS support for DL | 1. 960KHz SCS for DL data and control channels, SSB, and reference signal reception in FR2-2 for non-initial access 2. Multiple-slot PDCCH monitoring for 960KHz with (Xs,Ys)=(8,1) 3. MultiPDSCH scheduling by single DCI for the operation with 960 kHz SCS and corresponding HARQ enhancements 4. Within the Ys = 1 slot, monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS with set1 = (7, 3) symbols where set1 is defined in FG3-5b 5. For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of each slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within each slot of the slot group. 6. Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for FDD (This supersedes corresponding component of FG 3-5b) 7. Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for TDD (This supersedes corresponding component of FG 3-5b) | 24-1 | Yes | N/A | 960KHz SCS support for DL is not supported | Perband | N/A | N/A | N/A | FFS: component description without a reference to other R15 FGs | Optional with capability signalling |
|  | 24-5f | Enhanced PDCCH monitoring for 960KHz | 1. Multiple-slot PDCCH monitoring for 960KHz with (Xs,Ys) 2.) Within each of the Ys = 2 or 4 slots, monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS in the first 3 OFDM symbols of each slot | 24-5 | Yes | N/A | Enhanced PDCCH monitoring for 960KHz is not supported | Per band | N/A | N/A | N/A | Component 1 candidate values: one or more of {(4,1), (4,2), (8,4)} | Optional with capability signalling |
|  | 24-6 | Type 1 channel access procedure in uplink for FR2-2 with shared spectrum channel access | 1. Support Type 1 channel access procedure 2. Support LBT performed per channel including active BWP bandwidth | 24-1a | Yes | N/A | Type 1 channel access procedure in uplink for FR2-2 with shared spectrum channel access is not supported | per band | N/A | N/A | N/A | A UE that supports FR2-2 must indicate this FG is supported when required by regulation | Optional with capability signalling |
|  | 24-7 | Type 2 channel access procedure in uplink for FR2-2 with shared spectrum channel access | 1. Support Type 2 channel access procedure 2. Support LBT performed per channel including active BWP bandwidth | 24-1a, 24-6 | Yes | N/A | Type 2 channel access procedure in uplink for FR2-2 with shared spectrum channel access is not supported | per band | N/A | N/A | N/A | A UE that supports FR2-2 must indicate this FG is supported when required by regulation | Optional with capability signalling |
|  | 24-8 | 32 DL HARQ processes [for FR 2-2] | Support 32 HARQ processes in DL |  |  |  |  | per band | N/A | N/A | N/A | FFS: extend to other FRs | Optional with capability signalling |
|  | 24-9 | 32 UL HARQ processes [for FR 2-2] | Support 32 HARQ processes in UL |  |  |  |  | per band | N/A | N/A | N/A | FFS: extend to other FRs | Optional with capability signalling |
|  | 24-10 | Additional beam switching time delay | Supported additional beam switching time delay d for 480 kHz SCS | Yes | N/A | [ |  | Per UE | N/A | N/A | Yes | Candidate value set: 56 or 112 symbols The default value of 112 symbols is assumed if this capability is not reported | Optional with capability signalling |