3GPP TSG RAN WG1 #108-e	R1-2200980

e-Meeting, February 21st - March 3rd, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Resource allocation for power saving

Agenda item:		8.11.1.1

Document for:		Discussion and Decision

# Introduction

2. Resource allocation enhancement:

- Specify resource allocation to reduce power consumption of the UEs [RAN1, RAN2]

  - Baseline is to introduce the principle of Rel-14 LTE sidelink random resource selection and partial sensing to Rel-16 NR sidelink resource allocation mode 2.

  - Note: Taking Rel-14 as the baseline does not preclude introducing a new solution to reduce power consumption for the cases where the baseline cannot work properly.

  - This work should consider the impact of sidelink DRX, if any.

In the description of the work item [1] for NR sidelink enhancement, the following objectives were specified for sidelink resource allocation to reduce power consumption:

In previous few RAN1 meetings, significant progress has been made on both types of partial sensing and related issues. In this contribution, we continue the discussion and provide our views on remaining issues of SL random resource selection, partial sensing, and the DRX impact on resource allocation.

# Resource allocation for power saving in mode 2

The standardization of Rel-16 NR sidelink focused on vehicle-to-vehicle (V2V) communication in V2X use cases. The design is based on the assumption of “always-on” when a UE operates sidelink, since vehicular UEs are installed in vehicles with sufficient battery capacity. A UE performs blind decoding of PSCCHs in all slots except the slots in which it needs to transmit. This is for the following two purposes:

- The UE checks whether the associated PSSCH is of interest to it and hence decides whether it needs to go ahead and decode the PSSCH;

- The UE acquires information of resource allocations by other UEs to enable mode 2 (sensing-based) resource allocation.

In Rel-17, NR sidelink will support new use cases including V2P (vulnerable road users (VRUs)) in V2X use cases, public safety and commercial use cases, as identified in the WID [1]. UEs for these use cases, such as SL reception type-A/B/D UEs, are energy sensitive and therefore their power consumption needs to be minimized. Hence solutions for power saving in Rel-17 are required. In LTE V2X sidelink (Rel-14/15), random resource selection and partial sensing based resource selection were supported as power efficient RA schemes especially for the pedestrian UEs. These two RA schemes are assumed as the baseline in Rel-17 for power efficient RA schemes with potential further enhancements, as described in the WID [1]. Both resource allocation schemes, random selection and partial sensing are under discussion for specification as enhancement for SL resource allocation for power saving.

## Random resource selection

In RAN1#106-e [2], the following agreements were reached on random resource selection:

Agreement For random resource selection in a resource pool (pre-)configured with full/partial sensing and random resource selection, down-select to one of the followings in RAN1#106bis-e - Option 1: A priority threshold value or a range of priority levels is (pre-)configured for the resource pool, below or within which random resource selection is allowed - Note, lower value means higher priority - FFS whether resource pool partitioning can be additionally applied - Option 2: Increase the priority for the transmission based on random selection and indicate the new priority value in the priority field in the 1st-stage SCI - FFS: An extra field is added in SCI for indicating the original priority value associated with QoS requirement, - FFS: A 1-bit field in the SCI indicates that the UE is performing random resource selection, or - FFS: An extra field is added in SCI for indicating the mapping to the original priority value associated with QoS requirement. - Option 7: Exclude resources reserved by UE performing random selection without re-evaluation / pre-emption checking, regardless of their priorities. E.g. a 1-bit field in the SCI indicates that the UE is performing random resource selection and not performing re-evaluation and pre-emption checking - Option 12: No special consideration

With random resource selection, the transmitting UE randomly selects a resource in the resource selection window to transmit the data packet. For periodic traffic, the UE may reserve the corresponding resource in the next period for the transmission of the next TB. As there are no channel sensing operations, the random resource selection can significantly reduce the power consumption. On the other hand, the benefit of power saving is achieved at the cost of potential increased collisions with other UEs (including sensing UEs and other random selection UEs).

The random resource selection can be used under the following scenarios:

- C1: Some UEs, e.g. P-UEs, may have no or limited SL receiving capability or have no sufficient battery capacity to support SL receiving. As agreed in previous RAN1 meetings, SL UEs that are not capable of performing reception of any SL signals and channels are called reception type-A UEs, and UEs which can receive only PSFCH and S-SSB are called SL reception type-B UEs. In some scenarios, these UEs need to transmit sidelink packets to nearby UEs (e.g. to vehicle UEs for traffic safety purpose). In this case, they have to use random resource selection.

- C2: For UEs that have sensing capability (such as reception type-D UE) but still intend to keep power consumption as low as possible, they may have to use random resource selection at least for the initial transmission if the latency requirement is stringent.

- C3: The UEs may use random resource selection under the condition that both random selection and (partial) sensing based RA are configured for the resource pool and at the same time, when the measured CBR of the pool is sufficiently low.

As mentioned above, the cost of random resource selection is the potential collisions with other UEs e.g. sensing UEs or other random resource selection UEs. This issue will further deteriorate if persistent collisions occur. So, enhancement to the random resource selection is desired to alleviate the potential collisions.

The resource reservation by a UE with random resource selection can be pre-empted by a UE with sensing. But this UE can’t identify the situation leading to collision. We may consider increasing the priority at PHY for a UE with random selection or adding SCI bits to indicate sensing capability (resource selection scheme).

The above issue also exists in the case that the shared resource pool is also used by Rel-16 sensing UEs. Adding SCI bits to indicate sensing capability doesn’t work since Rel-16 UEs can’t identify newly added SCI bits. On the contrary, increasing PHY priority for a UE with random selection is a feasible choice for backward compatibility with Rel-16 sensing UEs. We may use the increased priority value in the priority field in the 1st-stage SCI. This can increase the capability of a Rel-17 UE with random resource selection to protect itself from being pre-empted by Rel-16 UEs. Accordingly, to retain proper operations between Rel-17 UEs with different resource selection schemes, an extra priority field containing the original priority value associated with QoS requirements can be added to the SCI.

For backward compatibility, the collision issues with Rel-16 UEs may be also addressed by using, for example, conditions to control the UEs using random resource selection. The conditions may consider, for example, whether the UEs may use random resource selection per a TB or more than one consecutive TBs, CBR conditions, uses of SL HARQ Option-2 so that the UEs using random resource selection may reselect resources based on HARQ NACKs when persistent collisions happen.

Observation 1: Random resource selection could be used under the conditions e.g. with no sensing capability, no sensing results available for urgent use or when the CBR of the pool is sufficiently low.

Observation 2: In case the shared resource pool is also used by Rel-16 sensing UEs, backward compatibility needs to be considered.

Although we are supportive to Option 2 for backward compatibility, as a compromise, we can support original Option 12.

Proposal 1: For random resource selection, we support original Option 12: No special consideration.

Proposal 2: For backward compatibility, consider Option 2: Increase the priority for the transmission based on random selection and indicate the new priority value in the priority field in the 1st-stage SCI.

There is an FFS item on “whether/how re-evaluation and pre-emption can be supported by UEs performing random resource selection that do perform sensing” left from RAN1#104-e meeting. The question is whether a type-D UE (w/ sensing capability) is allowed to perform re-evaluation/pre-emption while random selection is used for resource allocation. While the UE can select random selection, it can choose to use sensing as well. There is no clear advantage for a type-D UE because the UE has its sensing capability anyway.

Proposal 3: For the FFS item on “whether/how re-evaluation and pre-emption can be supported by UEs performing random resource selection that do perform sensing”, we support closing this FFS issue without agreement/conclusion.

## Partial sensing-based RA

### Re-evaluation and pre-emption checking for aperiodic transmission

In RAN1 #107b-e meeting, the procedure for the re-evaluation and pre-emption checking for aperiodic transmission is agreed, following similar approach for periodic transmission. The agreement is:

Agreement When UE is triggered to perform re-evaluation and pre-emption checking for aperiodic transmission (Prsvp_TX=0) in slot n, - The candidate resource set (SA) is initialized to the remaining Y’ candidate slots that starts from slot tyiSL and ends at the last slot of the Y’ candidate slots. - tyiSL is the first candidate slot after slot n+T3. - UE may perform PBPS for periodic sensing occasions after the resource (re)selection when sl-MultiReserveResource is enabled for the mode 2 Tx resource pool - It is up to UE implementation - UE performs CPS starting from at least M consecutive logical slots earlier than tyiSL to Tproc,0SL+Tproc,1SL slots earlier than tyiSL. - FFS: When the minimum M slots for CPS cannot be guaranteed, - All available sensing results not earlier than n–T0 for the resource pool indicated by higher layer are applied for re-evaluation and pre-emption checking procedures

One issue remains open when the contiguous partial sensing (CPS) does not have minimum M slots. Because the contiguous partial sensing window after slot n, there might be not enough slots between the trigger slot and tyiSL-(Tproc,0SL+Tproc,1SL).

This scenario can happen for resource (re)selection with CPS. RAN1 has agreed that UE may perform two options, depending on UE’s implementation:

- Option A: the UE ensures the Y’min criterion is fulfilled

- Option B: UE performs random resource selection

However, for resource re-evaluation and pre-emption checking with aperiodic transmission, it is not a good practice to allow random resource selection during resource re-evaluation/pre-emption checking. UE’s behavior shall be clearly defined, different from initial resource selection with continuous partial sensing.

For UE performing re-evaluation and pre-emption checking, the UE may have sensing results from other sources (e.g. from PBPS if it is configured), or the UE just uses its remaining slots for sensing, regardless from the minimum M slots. From this point of view, we would support the proposal to let UE use all available sensing results when the minimum M slots for CPS cannot be guaranteed.

Proposal 4: For resource re-evaluation and pre-emption checking for aperiodic Tx, when there is no sufficient M slots for contiguous partial sensing, UE shall use all available sensing results.

### Working assumption for pre-configured CPS sensing window size

There is a working assumption on the value of pre-configured M, from the RAN1 agreement in the last meeting:

Agreement When UE performs at least contiguous partial sensing in a mode 2 Tx pool for a resource (re)selection procedure and re-evaluation/pre-emption checking triggered by aperiodic transmission (Prsvp_TX=0) in slot n, - For minimum size M of the CPS monitoring window [n+TA, n+TB]: - By default, M is 31 unless (pre-)configured with another value - The range of (pre-)configured M is from 0 (working assumption) to 30

The contesting issue is whether the CPS can be allowed to turned off for aperiodic transmission when the value of M=0.

It is not clear about the use case when the higher layer configures continuous partial sensing for the UE and at the same time, the higher layer configures M=0, which practically turns the CPS sensing off. There is no defined UE behavior when M=0 is configured. It is even not clear whether UE shall perform random selection. If the intention is to trigger random selection, RAN1 shall have an agreement on this. We’d like not to confirm this WA; instead, other values of M (such as M=4 as originally proposed) other than 0 should be used.

Proposal 5: Support to use M=4 as the minimum configurable value for contiguous partial sensing, where M is the minimum size of CPS sensing window.

# DRX impact

## DRX impact on resolution allocation when DRX active timing information is provided

A RAN2 LS on the DRX impact on Rx UE was discussed in RAN1 #106b-e and RAN1 #107-e meetings. A set of working assumptions was reached in RAN1 #106b-e meeting, and one of working assumption is confirmed in RAN1 #107-e. When the higher layer provides information about SL DRX active time of Rx UE, PHY layer shall use this information to select and report candidate resources. The WA confirmation agreement is shown below:

Agreement When SL DRX active time of Rx-UE is provided by the higher layer for candidate resource selection (including resource (re)selection and re-evaluation/pre-emption checking), the following working assumption is confirmed with option 2 as agreement (with modification in RED) Working Assumption (RAN1#106bis-e) When PHY layer is indicated with an active time of RX UE from MAC layer for candidate resource selection, a restriction is applied in PHY layer so that at least a subset of candidate resources reported to MAC layer is located within the indicated active time of the RX UE. The following options will be further discussed in RAN1 to restrict resources for candidate resource selection taking into account the indicated active time from MAC layer: - Option 1: PHY layer selects and reports candidate resources only within the indicated active time of the RX UE - Option 2: PHY layer selects and reports candidate resources in which at least a subset of the candidate resources is within the indicated active time of the RX UE - FFS: Details on when the number of subsets of candidate resource is less than the threshold - FFS: The subset of candidate resource outside of the active time should consider each inactive time period - FFS: UE selection of resource selection window to overlap with indicated RX UE active time - FFS: Whether it is up to UE implementation to report candidate resources only within the indicated active time of the RX UE - Option 3: PHY layer selects and reports an additional candidate resource set of candidate resources within the indicated active time of the RX UE

From the original RAN2 LS (R2-2108997), RAN2 agreements were listed with indications that the L2 Tx UE can maintain a set of timers, each of which can be served as the criterion to determine the allowable transmission time for each RX UE. RAN2 asked whether such information could be applicable in RAN1 to consider certain restrictions for Rx DTX UE. From the RAN2 LS, the DRX timing information is provided with a set of timers:

- For unicast, the TX UE maintains a set of timers per pair of source Layer-2 ID and destination Layer-2 ID corresponding to the SL DRX timers in the RX UE. For groupcast/broadcast, the TX UE maintains a set of timers per destination Layer-2 ID corresponding to the SL DRX timers in the RX UE. The TX UE uses the timers as part of the criterion for determining the allowable transmission time for each RX UE. - For transmissions to RX UE(s) using SL DRX operation, LCP restrictions ensure that a TX UE transmits data in the active time of the RX UE(s).

Following the resource selection procedure (TS 38.214), either full sensing or partial sensing shall report a resource set SA to higher layers, subject to sensing results. With consideration of DRX, if the information about a Rx UE active time is available (from MAC CE, for example), the resource selection procedure may be modified to exclude the relative resources, which are not fit into the Rx UE active time, from the resource set SA. The Tx UE will report the SA resource set to higher layers so that the proper resource selection will fit the Rx UE reception. This could be a possible approach to include the Rx UE active time information in the RAN1 resource selection procedure. This approach directly relates to Option 1, where PHY layer selects and reports candidate resource only within the indicated active time of the Rx UE.

The confirmed WA (Option 2) will allow reporting resource outside of active time of the Rx UE, as long as “at least a subset of the candidate resources in within the indicated active time of the Rx UE”. Then it is up to L2 to determine the actual resource allocation based on the reporting. This agreement is quite loose on the candidate resources within the indicated active time of the Rx UE. UE at L1 can also select to report candidate resources only with the indicated active time of the Rx UE. From this point, Option 1 could be also supported.

Quite of open issues are still remaining as indicated in the agreement. The basic question is to what degree the candidate resource outside of the active time should be considered and reported. Shall all resources outside of active time be considered for all inactive time period? What will happen when the resource selection window overlaps with the indicated Rx UE active time?

During the RAN1 discussion, there were some discussions on the interpretation of “active time” of the Rx UE. If the Tx UE reports the candidate resource based on the current DRX time, provided by the higher layer, the reported resource might be too restrictive if the Rx UE extends its active time, which would provide more possible candidate resources. When the Rx UE’s DTX active time changes, reporting candidate resource outside of active time would be beneficial in this scenario.

Since the usage of a set of timers is the basis to determine the Rx UE DRX timing, it would imply that the DRX timing is usually maintained periodically. Rx UE can extend its active time; but the question is how often such active-time extension happens. Given that DRX is a power saving feature, UE will likely operate at power saving mode to maintain a regular DRX schedule. If this is the case, UE may have some freedom to report candidate resources outside the indicated active time of the Rx UE. UE may have choice to report some candidate resources outside active time or to only report candidate resource inside active time. Therefore, it is up to UE implementation to report candidate resources only within the indicated active time of the Rx UE. We have this proposal:

Proposal 6: Up to UE implementation to report candidate resources outside the indicated active time of the Rx UE, when MAC layer provides information about DRX active time of RX UE.

## SL sensing in SL DRX inactive time

In RAN1 107b-e meeting, an agreement is reached to support SL sensing in SL DRX inactive time with a higher layer (pre-)configuration:

Agreement Whether UE performs SL reception of PSCCH and RSRP measurement for partial sensing on slots in SL DRX inactive time is enabled/disabled by (pre-)configuration per resource pool when partial sensing is configured in the UE by a higher layer. - When it is enabled, - When UE performs periodic-based partial sensing for a given Preserve, UE monitors only the default periodic sensing occasion. - When UE performs contiguous partial sensing, UE monitors a minimum of M slots for CPS. - Note, when it is disabled, the UE is not required to perform SL reception of PSCCH and RSRP measurement in SL DRX inactive time. - Note: no further optimization on the resource (re)selection procedure with regard to SL DRX operation is specified in Rel.17. - FFS the case when full sensing is configured in the UE by a higher layer

Based on the agreement, a higher layer (pre-)configuration per resource pool can enable/disable partial sensing in SL DRX inactive time. The supported partial sensing schemes are periodic-based partial sensing and contiguous partial sensing.  A new RRC parameter partialSensingInactiveTime is used to enable/disable partial sensing in SL DRX inactive time. An open question is that whether this enabling/disabling (pre-)configuration can be extended to support full sensing in SL DRX inactive time.

The SL DRX functionality is a SL power saving scheme. When the UE is (pre-)configured with SL DRX functionality, UE shall operate in power saving mode, under which the partial sensing is configured by higher layer. Therefore, enabling sensing in SL DRX inactive time will improve partial sensing performance at the expense of SL power saving. If the UE is configured for full sensing, this usually indicates that the UE is not operating at SL power saving mode or the UE does not support partial sensing. What is the use case to enable/disable the sensing in SL DRX inactive time? Unless the use case is clarified, there is no clear motivation to extend this sensing enabling/disabling functionality to full sensing.

Proposal 7: There is no need to extend the functionality of enabling/disabling sensing in SL DRX inactive time to full sensing UE.

# Conclusion

In this contribution, we provide our views on enhancement to sidelink resource allocation for power saving, summarized in the following observations and proposals:

On random resource selection:

Observation 1: Random resource selection could be used under the conditions e.g. with no sensing capability, no sensing results available for urgent use or when the CBR of the pool is sufficiently low.

Observation 2: In case the shared resource pool is also used by Rel-16 sensing UEs, backward compatibility needs to be considered.

Proposal 1: For random resource selection, we support original Option 12: No special consideration.

Proposal 2: For backward compatibility, consider Option 2: Increase the priority for the transmission based on random selection and indicate the new priority value in the priority field in the 1st-stage SCI.

Proposal 3: For the FFS item on “whether/how re-evaluation and pre-emption can be supported by UEs performing random resource selection that do perform sensing”, we support closing this FFS issue without agreement/conclusion.

On partial sensing:

Proposal 4: For resource re-evaluation and pre-emption checking for aperiodic Tx, when there is no sufficient M slots for contiguous partial sensing, UE shall use all available sensing results.

Proposal 5: Support to use M=4 as the minimum configurable value for contiguous partial sensing, where M is the minimum size of CPS sensing window.

On DRX impact on resource selection:

Proposal 6: Up to UE implementation to report candidate resources outside the indicated active time of the Rx UE, when MAC layer provides information about DRX active time of RX UE.

Proposal 7: There is no need to extend the functionality of enabling/disabling sensing in SL DRX inactive time to full sensing UE.

# References

- RP-202846 - WID revision: NR sidelink enhancement

- RAN1 Chair report, Meeting #106-e, Aug.16th – 27th, 2021

- RAN1 Chair report, Meeting #106bis-e, Oct.11th – 19th, 2021

- R2-2108997, LS to RAN1 on RAN2 Agreements Related to Resource Selection, RAN2 #115-e, Aug 16-27, 2021

- RAN1 Chair report, Meeting #107-e, Nov.11th – 19th, 2021

- RAN1 Chair report, Meeting #107b-e, Jan. 17 – 25, 2022

# Appendix: Previous agreements on Rel-17 resource allocation for power saving

## RAN1 #103e

Conclusion

- SL reception Type A and Type D should be used as the reference for evaluation and designing of SL power saving features in R17.

  - Type A: UE is not capable of performing reception of any SL signals and channels, FFS with exception of performing PSFCH and S-SSB reception (aim to conclude in RAN1#104-e)

  - Type D: UE is capable of performing reception of all SL signals and channels defined in R16. It does not preclude UE to perform reception of a subset of SL signals/channels

  - If there are evaluations with assumptions other than the above reference, the detailed assumptions need to be reported

  - Note: the types and the associated capability defined here are not intended to be defined as Rel-17 UE features as is.

Agreements:

- Partial sensing based RA is supported as a power saving RA scheme

  - FFS details

- Random resource selection is supported as a power saving RA scheme

  - FFS any changes or enhancement

  - FFS on conditions to apply random resource selection

Agreements:

- In R17, a SL Mode 2 Tx resource pool can be (pre-)configured to enable full sensing only, partial sensing only, random resource selection only, or any combination(s) thereof

  - FFS details, including usage, potential restrictions, whether/how any enhancement or condition is needed for the coexistence of full sensing and power saving RA scheme(s) in a same resource pool, etc.

Agreements:

- Re-evaluation and pre-emption checking are not supported by UEs that do not perform any sensing (i.e. PSCCH reception)

- Re-evaluation and pre-emption checking are supported by UEs that perform sensing

  - FFS details and any conditions(s) in which re-evaluation and pre-emption can be performed

- FFS whether/how re-evaluation and pre-emption can be supported by UEs performing random resource selection that do perform sensing

- Note: details about sensing in this context, including when it is performed, are not decided yet.

Agreements:

- Further study congestion control based on CBR and CR for power saving RA schemes

  - Identify necessary changes from R16 CBR/CR (if any), including transmission resource selection and transmission parameters that can be adjusted and applicable to power savings RA schemes

  - Note: this is not intended to require all UEs to perform sensing for the purpose of CBR measurement

## RAN1 #104e

Agreements:

- Random resource selection is applicable to both periodic and aperiodic transmissions

  - FFS conditions for random resource selection

Conclusion:

- PSFCH reception is not included for Type A UE

- S-SSB reception is not included for Type A UE

- SL reception Type B is additionally added

  - Type B: Same as Type A with an exception of performing PSFCH and S-SSB reception

- Note: the same conditions as in RAN1#103-e regarding the context of the discussion of Type A and Type D still apply (also applicable to type B)

Agreements: In a resource pool (pre-)configured with at least partial sensing, if UE performs periodic-based partial sensing, at least when the reservation for another TB (when carried in SCI) is enabled for the resource pool and resource selection/reselection is triggered at slot n, it is up to UE implementation to determine a set of Y candidate slots within a resource selection window, where

- FFS condition(s) and timing(s) for which periodic-based partial sensing is performed by UE

- The resource selection window is [n+T1, n+T2]

  - As a baseline, T1 and T2 are defined in the same way as in R16 NR-V2X according to step 1 [TS 38.214 Sec. 8.1.4]

  - Further discuss whether or not to introduce a threshold to re-define T1 and T2 such that

    - T1 ≥ 0 (subject to processing time constraint Tproc, 1), and T2 ≤ remaining PDB

    - T2-T1 ≤ (pre-)configured threshold

- A minimum value for Y is (pre-)configured from a range of values, FFS details

- FFS any restriction to determine Y candidate slots (including its relationship with SL-DRX)

- FFS whether the resource selection window [n+T1, n+T2] should be confined within a set of periodic set of resources and its relationship with SL-DRX

- Note: The terminology “periodic-based partial sensing” is based on the “partial sensing” used in LTE-V and it is intended to be used for the design and discussion of partial sensing in Rel-17.

Agreements: In a resource pool (pre-)configured with at least partial sensing, if UE performs periodic-based partial sensing, at least when the reservation for another TB (when carried in SCI) is enabled for the resource pool and resource selection/reselection is triggered at slot n, the UE monitors slots of at least one a set of periodic sensing occasions, where a periodic sensing occasion is a set of slots according to

if tvSL is included in the set of Y candidate slots.

- Preserve is a periodicity value from the configured set of possible resource reservation periods allowed in the resource pool (sl-ResourceReservePeriodList). Down select to one:

  - Option 1:  Preserve corresponds to all values from the configured set sl-ResourceReservePeriodList

  - Option 2:  PreservePreserve Preserve corresponds to a subset of values from the configured set sl-ResourceReservePeriodList

    - FFS how to determine the subset (e.g., by (pre-)configuration, UE determination)

  - Option 3: PreservePreserve Preserve is a common divisor among values in the configured set sl-ResourceReservePeriodList

  - Option 4: FFS others

- k equals tois selected according to (down select to one)

  - Option 1: Only the most recent sensing occasion within sensing window for a given reservation periodicity before the resource (re)selection trigger or the set of Y candidate slots subject to processing time restriction

  - Option 2: The two most recent sensing occasions within sensing window for a given reservation periodicity before the resource (re)selection trigger or the set of Y candidate slots subject to processing time restriction

  - Option 3: All possible sensing occasions after n –T0

  - Option 4: Only one periodic sensing occasion for one reservation period. The k value is up to UE implementation. Max value for k is (pre-)configured.

  - Option 5: k is (pre-)configured, including multiple values

  - Option 6: (pre-)configuration of a bitmap, same as in LTE-V

  - Option 7: FFS others

- FFS relationship between periodic sensing occasions and SL-DRX

- FFS condition(s) and timing(s) for which periodic-based partial sensing is performed by UE

- Note: companies are encouraged to show performance data for the down selections

Agreements:

- In a resource pool (pre-)configured with at least partial sensing, if UE performs contiguous partial sensing and resource (re-)selection is triggered in slot n, support the following option:

  - Option 1: For the purpose of resource (re-)selection, the UE monitors slots between [n+TA, n+TB] and performs identification of candidate resources, in or after slot n+TB, based on all available sensing results, including periodic-based partial sensing results (if applicable).

    - FFS TA, TB (including the possibility of equal to zero, positive or negative) and remaining details (in particular, whether there should be exclusion of slots, changes in TA/TB values for different purposes, etc.)

    - FFS whether n can be replaced by e.g., index of some of Y candidate slots

  - FFS condition(s) in which contiguous partial sensing is performed by UE

  - FFS interaction with SL-DRX, if any

  - FFS interaction with periodic-based partial sensing, if any

  - Other options are not precluded

  - Note: This option is not to replace random resource selection only without sensing or re-evaluation and pre-emption checking

## RAN1 #104b-e

Conclusion:

- In periodic-based partial sensing,

  - It is not necessary to further discuss whether or not to introduce a threshold to re-define T1 and T2.

Agreements:

- In periodic-based partial sensing,

- For the set of Preserve values, down-select to one of the following in RAN1#105-e

    - Alt.1: Preserve corresponds to all values from the configured set sl-ResourceReservePeriodList

    - Alt.2: A set of Preserve values is (pre-)configured and includes up to the full set of values from the configured set sl-ResourceReservePeriodList

      - FFS if support multiple sets of Preserve values based on one or more metrics

      - FFS whether/how to restrict the set of values

- For the k value, down-selection to one of the following in RAN1#105-e (further refinement of each of the alternatives is possible)

        - Alt 1: Option 1 as in RAN1#104-e

        - Alt 2: A modified Option 5 as in RAN1#104-e, where the modification is such that it also includes option 1

          - FFS how to (pre-)configure (e.g. including bitmap), whether a maximum number of k values is needed, and whether it can be up to UE implementation to select a k value based on the (pre-)configuration

        - FFS details, e.g., sensing before the resource (re)selection trigger or the first slot of the set of Y candidate slots subject to processing time restriction, etc.

      - Note: companies are encouraged to provide more evaluations

Agreement:

- When periodic-based partial sensing is potentially performed by UE in a mode 2 Tx resource pool provided by higher layer, at least all of the followings are met:

  - Periodic reservation for another TB (sl-MultiReserveResource) is enabled for the resource pool

  - The resource pool is (pre-)configured to enable partial sensing

  - Partial sensing configured by higher layer in the UE

## RAN1 #105-e

Agreement:

- For the set of Preserve values in periodic-based partial sensing,

  - If no (pre-)configuration (i.e., by default), Preserve corresponds to all values from the (pre-)configured set sl-ResourceReservePeriodList.

  - Otherwise, a single set of Preserve values can be (pre-)configured, where the set of Preserve values are restricted to a subset of the (pre-)configured set sl-ResourceReservePeriodList

    - This is per mode 2 Tx resource pool (pre-)configuration

    - A UE by implementation may also monitor other sl-ResourceReservePeriodList values not part of the restricted subset

      - In particular, the UE may additionally monitor occasions corresponding to P_RSVP_Tx

        - FFS whether the monitoring can be mandatory

Agreement:

·  In periodic-based partial sensing for resource (re)selection, the UE at least monitors in periodic sensing occasion(s) for a given reservation periodicity before the first slot of the selected Y candidate slots subject to processing time restriction for the identification of candidate resources.

o   The processing time restriction includes Tproc,0SL  and Tproc,1SL.

o   Aspects relating to sensing during SL DRX are to be discussed separately

- Relationship to re-evaluation and pre-emption operation for periodic-based partial sensing to be discussed separately

  - FFS details including whether monitoring of periodic sensing occasions between triggering slot n and the first slot of the selected Y candidate slots subject to processing time restriction is performed as part of resource (re)selection or re-evaluation and pre-emption checking

Agreement:

- For the k value in periodic-based partial sensing for resource (re)selection,

  - By default, the UE monitors the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger slot n or the first slot of the set of Y candidate slots subject to processing time restriction.

  - If (pre-)configured, UE additionally monitors periodic sensing occasions that correspond to a set of values which can be (pre-)configured with at least one value

    - (Working assumption) Possible values correspond to the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger slot n or the first slot of the set of Y candidate slots, and the last periodic sensing occasion prior to the most recent one for the given reservation periodicity are included.

    - FFS: whether/which other values and details of the (pre-)configuration (e.g. max number of values or sensing occasions)

    - FFS: whether a value denotes a specific occasion to monitor or the earliest occasion to start the monitoring.

  - FFS relationship between periodic-based partial sensing occasions and SL-DRX

  - Note:

    - This is for the case when the resource (re)selection triggering slot n is expected by UE

Agreement:

- For random resource selection,

  - Reuse the maximum distance separation of 32 logical slots for a HARQ retransmission resource reserved by a prior SCI for the same TB, which was defined in R16 for full sensing operation.

  - SL HARQ feedback enabled transmission is supported (FFS applicable conditions if any)

    - The minimum HARQ feedback time gap (Z) shall be respected between any two selected resources of a TB where a HARQ feedback for the first of these resources is expected.

- FFS the impact of resource collision when random resource selection is performed by a UE which does not perform sensing / re-evaluation and pre-emption checking in a resource pool with mixed RA schemes (e.g. for low priority or any priority transmissions).

  - Including study potential solution(s) if the impact is not negligible (e.g. threshold based, raising priority, minimum time gap, pattern based, a priori SCI reserving initial transmissions, resource pool partitioning, and etc.).

Agreement: In contiguous partial sensing for resource (re)selection, TA and TB values can be zero, positive or negative

- TA and TB values or range depend on different operating scenarios or conditions (e.g., periodic/aperiodic traffic, predictability of triggering slot n, remaining PDB, re-evaluation/pre-emption checking, HARQ feedback, CBR/CR parameter, power saving, etc)

  - FFS details

- FFS: details of how periodic-based partial sensing and contiguous partial sensing are used for re-evaluation and pre-emption checking. Including how to reduce UE’s power consumption (caused by additional sensing operation of re-evaluation/pre-emption) after its resource selection, with the considerations of different operating scenarios or conditions (e.g., pre-emption enabled/disabled, HARQ-ACK enabled/disabled, etc).

## RAN1 #106-e

Agreement

In periodic-based partial sensing, UE monitoring of periodic sensing occasions between triggering slot n and the first slot of the selected Y candidate slots subject to processing time restriction is performed as part of resource (re)selection.

Agreement

Conditions in which contiguous partial sensing is performed by UE, when at least all of the followings are met:

- L1 [is expected to be or] is triggered by higher layer to report resources for resource (re-)selection in a mode 2 Tx pool

  - FFS: When the trigger will be received by L1

- The resource pool is (pre-)configured to enable partial sensing

- Partial sensing is configured by higher layer in the UE

Agreement

For a resource pool (pre-)configured with at least partial sensing and UE is configured by its higher layer for partial sensing,

- Periodic-based partial sensing and contiguous partial sensing schemes are supported for resource re-evaluation and pre-emption checking

  - FFS details of partial sensing for re-evaluation and pre-emption checking, including any restrictions / conditions on performing PBPS and CPS, subset of resources, timing, candidate resource set (SA) and etc

- Same as in Rel-16, the higher layer indicates a set of resources (r0,r1,r2,…) and/or a set of resources (r0',r1',r2',…) for re-evaluation and/or pre-emption checking, respectively

  - Pre-emption checking is enabled according to the Release-16 interpretation of sl-PreemptionEnable.

    - FFS: If additional enhancements are needed for enabling/disabling

- The triggering of re-evaluation and pre-emption checking is as in R16.

Agreement

When UE performs only contiguous partial sensing (CPS) in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) disabled, and a resource (re)selection is triggered in slot n,

- The resource selection window (RSW) is [n+T1, n+T2] where T2 is defined based on step 1) of Rel-16 TS 38.214 Sec. 8.1.4

  - FFS whether the resource selection window [n+T1, n+T2] should be confined within a set of periodic set of resources and its relationship with SL-DRX

- On the sensing window [n+TA, n+TB] for CPS,

  - Details of TA and TB values based on the agreements from previous RAN1 meetings

  - FFS whether and how to define a minimum CPS window size, including (pre-)configurability and the case when TB - TA is smaller than the minimum CPS window size

  - FFS whether and how to define a maximum value / upper bound for TB with respect at least to the minimum RSW size and the remaining PDB, including (pre-)configurability

- FFS how a set of candidate resource (SA) is initialized considering candidate single-slot resources, including

  - Whether and how to define a minimum size for the RSW (e.g., Rel-16 T2min), including (pre-)configurability

  - Whether the set SA is confined within a set of Y candidate slots within the RSW

- UE performs resource exclusion from the set SA based on at least all available sensing results and based on step 6) and 7) of Rel-16 TS 38.214 Sec. 8.1.4

- Note, re-evaluation and pre-emption checking in a resource pool with periodic reservation for another TB (sl-MultiReserveResource) disabled is considered separately.

- FFS: Details on T1

Agreement

For random resource selection in a resource pool (pre-)configured with full/partial sensing and random resource selection, down-select to one of the followings in RAN1#106bis-e

- Option 1: A priority threshold value or a range of priority levels is (pre-)configured for the resource pool, below or within which random resource selection is allowed

  - Note, lower value means higher priority

  - FFS whether resource pool partitioning can be additionally applied

- Option 2: Increase the priority for the transmission based on random selection and indicate the new priority value in the priority field in the 1st-stage SCI

  - FFS: An extra field is added in SCI for indicating the original priority value associated with QoS requirement,

  - FFS: A 1-bit field in the SCI indicates that the UE is performing random resource selection, or

  - FFS: An extra field is added in SCI for indicating the mapping to the original priority value associated with QoS requirement.

- Option 7: Exclude resources reserved by UE performing random selection without re-evaluation / pre-emption checking, regardless of their priorities. E.g. a 1-bit field in the SCI indicates that the UE is performing random resource selection and not performing re-evaluation and pre-emption checking

- Option 12: No special consideration

Agreement

When UE performs periodic-based and contiguous partial sensing schemes in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) enabled,

- For a resource (re)selection procedure triggered by aperiodic transmission (Prsvp_TX=0) in slot n,

  - The resource selection window (RSW) is [n+T1, n+T2], and T1 and T2 are defined in the same way according to step 1) of Rel-16 TS 38.214 Sec. 8.1.4

    - FFS whether UE determines a new set of Y candidate slots within the RSW and monitors corresponding periodic sensing occasions between slot n and the first slot of the new Y candidate slots subject to processing constraints

    - FFS how to initialize a set of candidate resource (SA) for the triggered resource (re)selection procedure and which partial sensing scheme(s) and results can be used for resource exclusion in the resource (re)selection procedure

    - FFS whether the resource selection window [n+T1, n+T2] should be confined within a set of periodic set of resources and its relationship with SL-DRX

- Note, re-evaluation and pre-emption checking based on periodic-based and contiguous partial sensing schemes is considered separately

Agreement

When UE performs periodic-based and contiguous partial sensing schemes in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) enabled,

- For a resource (re)selection procedure triggered by periodic transmission (Prsvp_TX≠0) in slot n

  - A set of candidate resource (SA) is initialized to the set of selected Y candidate slots of PBPS

    - UE performs contiguous partial sensing in [n+TA, n+TB] for resource exclusion from the initialized candidate resource set (SA)

      - FFS details of TA and TB based on the agreement(s) from previous RAN1 meetings

- Note, re-evaluation and pre-emption checking based on periodic-based and contiguous partial sensing schemes is considered separately

FFS: The condition under which UE performs periodic-based and contiguous partial sensing schemes in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) enabled

R1-2108265	FL summary for AI 8.11.1.1 – resource allocation for power saving (EOM)	Moderator (OPPO)

[106-e-NR-R17-Sidelink-02] Reply LS to R1-2106413 (LS on time gap information in SCI, RAN2) by August 20 – Kevin (OPPO)

Agreement

Regarding RAN2’s question, in RAN1’s opinion it is feasible, other than in the following exceptional cases:

- SL transmission dropping due to prioritization or congestion control

- Due to re-evaluation, a re-selected resource is earlier than a reserved resource by UE implementation in Mode 2

- If (pre-)configured with many-to-one mapping between Tx and Rx resource pools in some cases (e.g., when PSFCH is not configured)

The final LS is in R1-2108622.

R1-2108226	Moderator summary for [106-e-NR-R17-Sidelink-02] Reply LS to R1-2106413	Moderator (OPPO)

[106-e-NR-R17-Sidelink-03] Reply LS to R1-2100021 (LS to RAN1 on SL DRX design, RAN2, from RAN1#104-e) by August 25 - Yuzhou (ZTE)

Agreement

A UE can perform SL reception of PSCCH and RSRP measurement for sensing during its SL DRX inactive time.

- FFS: When such reception and measurement is performed, whether it is subject to specification, or is up to UE implementation

- FFS: Other details

Agreement

LS to RAN2 on SL DRX design is endorsed in R1-2108580.

## RAN1 #106b-e

Agreement

In the agreement from RAN1#105-e, the working assumption is confirmed and the FFS bullet (in RED) is closed without any agreement.

Agreement from RAN1#105-e: - For the k value in periodic-based partial sensing for resource (re)selection, - By default, the UE monitors the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger slot n or the first slot of the set of Y candidate slots subject to processing time restriction. - If (pre-)configured, UE additionally monitors periodic sensing occasions that correspond to a set of values which can be (pre-)configured with at least one value - (Working assumption) Possible values correspond to the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger slot n or the first slot of the set of Y candidate slots, and the last periodic sensing occasion prior to the most recent one for the given reservation periodicity are included. - FFS: whether/which other values and details of the (pre-)configuration (e.g. max number of values or sensing occasions) - FFS: whether a value denotes a specific occasion to monitor or the earliest occasion to start the monitoring. - FFS relationship between periodic-based partial sensing occasions and SL-DRX - Note: - This is for the case when the resource (re)selection triggering slot n is expected by UE

Agreement

When UE performs periodic-based and contiguous partial sensing schemes in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) enabled,

- For a resource (re)selection procedure triggered by periodic transmission (Prsvp_TX≠0) in slot n, TA and TB for the CPS monitoring window is defined according to one of the followings:

  - n+TA is M logical slots earlier than slot ty0SL, and n+TB is Tproc,0SL+Tproc,1SL slots earlier than ty0SL, where ty0SL is the first slot of the selected Y candidate slots of PBPS, and Tproc,0SL, Tproc,1SL are in units of physical time/slots.

    - By default, M is 31 unless (pre-)configured with another value.

Agreement

For the periodic sensing occasion(s) (PSO(s)) that a UE needs to additionally monitored in PBPS, it shall be (pre-)configured jointly for all Preserve values.

- The UE is not required to monitor PSOs earlier than n–T0 if the UE is triggered to do resource (re)selection in slot n, where T0 is (pre-)configured

Agreement

When UE performs at least contiguous partial sensing in a mode 2 Tx pool for a resource (re)selection procedure triggered by aperiodic transmission (Prsvp_TX=0) in slot n, TA and TB for CPS monitoring window and a candidate resource set (SA) is initialized according to potentially one of the following approaches (final decision in RAN1#107-e). Other approaches are not precluded and the details in each approach can still be updated.

- Approach 1: (SA is initialized based on at least slots with PBPS and/or CPS results and guarantee a minimum of M slots for CPS)

  - The UE selects a set of Y’ candidate slots with corresponding PBPS and/or CPS results (if available) within the RSW.

    - FFS how to handle the case if the total number of Y’ candidate slots is less than a (pre-)configured threshold Y’min without dropping the aperiodic transmission

    - FFS whether the Y’ candidate slots for aperiodic transmission is the same as the Y candidate slots in PBPS for periodic transmission of another TB(s)

    - FFS whether/how to prioritize/select resources based on partial sensing results.

    - FFS: How to select Y’ in case of CPS only

  - Candidate resource set (SA) is initialized to the set of all single-slot candidate resources in the selected Y’ candidate slots.

  - For the CPS monitoring window [n+TA, n+TB]:

    - TA and TB are both selected such that UE has sensing results for a minimum of M consecutive logical slots before ty0, where ty0 is the first slot of the selected Y’ candidate slots.

      - FFS: By default, M is 31 unless (pre-)configured with another value, or M is (pre-)configured based on transmission priority

      - FFS the range of (pre-)configured M from a TBD lowest value up to 30

      - FFS: how to handle the case when the minimum M slots for CPS cannot be guaranteed

  - FFS: RSW in case of CPS only

- Approach 2: (SA is initialized based on all candidate single-slot resources and guarantee a minimum of M slots for CPS)

  - Candidate resource set (SA) is initialized to the set of all candidate single-slot resources in [n+TB+Tproc,0+Tproc,1, n+T2], where TB is selected by the UE such that length of [n+TB+Tproc,0+Tproc,1, n+T2] ≥ T2min.

    - Tproc,0, Tproc,1 are in units of physical time/slots

    - FFS whether/how to prioritize/select resources based on partial sensing results (if PBPS is performed).

  - For the CPS monitoring window [n+TA, n+TB]:

    - TA = X

      - FFS value X for TA including X=1 and negative value

    - TB is selected such that UE has sensing results for a minimum of M consecutive logical slots before the start of (n+TB+Tproc,0+Tproc,1).

      - FFS: By default, M is 31 unless (pre-)configured with another value, or M is (pre-)configured based on transmission priority

      - FFS the range of (pre-) configured M from a TBD lowest value up to 30

      - FFS: how to handle the case when the minimum M slots for CPS cannot be guaranteed

- Approach 3: (independent approach for different case)

  - When UE additionally performs periodic-based partial sensing in the resource pool, the above Approach 1 applies.

  - When UE does not perform periodic-based partial sensing in a resource pool that does not allow resource reservation for another TB, the above Approach 2 applies.

Working Assumption

In a resource pool (pre-)configured to enable partial sensing, when UE is configured with partial sensing by its higher layer, the resources for which the UE performs re-evaluation and/or pre-emption checking are for the initial transmission and retransmissions of every TB according to Rel-16 specification based on partial sensing results.

- Same as in Rel-16, for periodic transmission, re-evaluation check is not applied to the resources that have been signalled in current period or previous periods, except that it is up to UE implementation whether to apply re-evaluation check to the resources in non-initial reservation period that have been signalled neither in the immediate last nor in the current period.

- The resource in the main bullet is the set of resources (r0,r1,r2,…) and/or the set of resources (r0',r1',r2',…)  for re-evaluation and/or pre-emption checking, respectively, which has been agreed in RAN1 #106-e.

## RAN1 #107-e

Agreement

When UE performs at least contiguous partial sensing in a mode 2 Tx pool for a resource (re)selection procedure triggered by aperiodic transmission (Prsvp_TX=0) in slot n, the general design framework in Approach 1 from RAN1#106bis-e in below is adopted. Note that, the details can still be updated.

- Approach 1: (SA is initialized based on at least slots with PBPS and/or CPS results and guarantee a minimum of M slots for CPS)

  - The UE selects a set of Y’ candidate slots with corresponding PBPS and/or CPS results (if available) within the RSW.

    - FFS how to handle the case if the total number of Y’ candidate slots is less than a (pre-)configured threshold Y’min without dropping the aperiodic transmission

    - FFS whether the Y’ candidate slots for aperiodic transmission is the same as the Y candidate slots in PBPS for periodic transmission of another TB(s)

    - FFS whether/how to prioritize/select resources based on partial sensing results.

    - FFS: How to select Y’ in case of CPS only

  - Candidate resource set (SA) is initialized to the set of all single-slot candidate resources in the selected Y’ candidate slots.

  - For the CPS monitoring window [n+TA, n+TB]:

    - TA and TB are both selected such that UE has sensing results for a minimum of M consecutive logical slots before ty0, where ty0 is the first slot of the selected Y’ candidate slots.

      - FFS: By default, M is 31 unless (pre-)configured with another value, or M is (pre-)configured based on transmission priority

      - FFS the range of (pre-)configured M from a TBD lowest value up to 30

      - FFS: how to handle the case when the minimum M slots for CPS cannot be guaranteed

  - FFS: RSW in case of CPS only

Agreement

When SL DRX active time of Rx-UE is provided by the higher layer for candidate resource selection (including resource (re)selection and re-evaluation/pre-emption checking), the following working assumption is confirmed with option 2 as agreement (with modification in RED)

Working Assumption (RAN1#106bis-e)

When PHY layer is indicated with an active time of RX UE from MAC layer for candidate resource selection, a restriction is applied in PHY layer so that at least a subset of candidate resources reported to MAC layer is located within the indicated active time of the RX UE. The following options will be further discussed in RAN1 to restrict resources for candidate resource selection taking into account the indicated active time from MAC layer:

- Option 1: PHY layer selects and reports candidate resources only within the indicated active time of the RX UE

- Option 2: PHY layer selects and reports candidate resources in which at least a subset of the candidate resources is within the indicated active time of the RX UE

  - FFS: Details on when the number of subsets of candidate resource is less than the threshold

  - FFS: The subset of candidate resource outside of the active time should consider each inactive time period

  - FFS: UE selection of resource selection window to overlap with indicated RX UE active time

  - FFS: Whether it is up to UE implementation to report candidate resources only within the indicated active time of the RX UE

- Option 3: PHY layer selects and reports an additional candidate resource set of candidate resources within the indicated active time of the RX UE

R1-2112526	FL summary for AI 8.11.1.1 – resource allocation for power saving (before 3rd GTW)	Moderator (OPPO

Agreement

When UE performs at least contiguous partial sensing in a mode 2 Tx pool for a resource (re)selection procedure triggered by aperiodic transmission (Prsvp_TX=0) in slot n,

- The UE selects a set of Y’ candidate slots with corresponding PBPS and/or CPS results (if available) within the RSW.

  - If the total number of Y’ candidate slots is less than a (pre-)configured threshold Y’min,

    - How UE includes other candidate slots is up to UE implementation

- Candidate resource set (SA) is initialized to the set of all single-slot candidate resources in the selected Y’ candidate slots.

- For the CPS monitoring window [n+TA, n+TB]:

  - TA and TB are both selected such that UE has sensing results starting at M consecutive logical slots before ty0 and ending at Tproc,0 + Tproc,1 slots earlier than ty0.

    - FFS: By default, M is 31 unless (pre-)configured with another value, or where M is (pre-)configured based on transmission priority

    - FFS: The range of (pre-)configured M from a TBD lowest value up to 30

    - When the minimum M slots for CPS cannot be guaranteed, support both

      - Option A, the UE ensures the Y’min criterion is fulfilled

      - Option B: UE performs random resource selection

      - When the UE performs Option A or Option B is up to UE implementation

Conclusion No additional triggering enhancement on top of existing Rel-16 mechanism in re-evaluation and pre-emption checking for partial sensing UEs in Rel-17, including enabling / disabling re-evaluation by (pre-)configuration.

- This does not restrict the triggering of re-evaluation and pre-emption checking due to inter-UE coordination message in scheme 2 (if agreed).

Agreement

When UE is triggered to perform re-evaluation and pre-emption checking for periodic transmission (Prsvp_TX≠0) in slot n,

- During the qth reservation period (q=0,1,2,…, Cresel-1), candidate resource set (SA) is initialized to the remaining Y candidate slots starts from slot tyiSL and ends at the last slot of the Y candidate slots, where the slot indices of the remaining Y candidate slots are equal to [q x Prsvp_Tx + tySL], where tySL is a slot index of Y candidate slots used in the initial resource (re)selection.

  - tyiSL is the first candidate slot after slot n+T3.

  - FFS whether/how to handle the case when number of the remaining Y candidate slots is less than Ymin.

- Scheme 1:

  - UE performs PBPS for the remaining Y candidate slots according to ty'-k×PreserveSL, where ty'SL is a slot belong to the remaining Y candidate slots, and k and Preserve are the same as resource (re)selection.

  - UE performs CPS starts from M logical slots earlier than tyiSL to Tproc,0SL+Tproc,1SL slots earlier than tyiSL.

    - By default, M is 31 unless (pre-)configured with another value.

Agreement

When UE performs random resource selection, LTE principle is reused:

- The UE is not required to measure CBR.

- When no SL CBR measurement result is available, a (pre-)configured SL CBR value is used.

Working assumption

For UE performs partial sensing or random resource selection, Rel-16 SL CR evaluation is directly reused.

Agreement

For SL CBR measurement in partial sensing, select one option in the following:

- Option 1, 2, 3: SL RSSI is measured for slots in which the UE performs partial sensing and PSCCH/PSSCH reception over a SL CBR measurement window defined in Rel-16. The calculation of SL CBR is limited within the slots for which the SL RSSI is measured.

  - If the number of SL RSSI measurement slots is below a (pre-)configured threshold, FFS the following or other options.

- Option 1: a (pre-)configured SL CBR value is used.

- Option 2: the UE additionally measure a set of slots within the SL CBR measurement window to meet the threshold.

- Option 3: the UE measures an additional set of slots which can be extended outside the SL CBR measurement window to meet the threshold.

- FFS whether the set of slots in option 2/3 are (pre-) configured or selected by UE implementation.

  - Option 4: LTE principle is reused:

- The UE is not required to measure CBR.

- When no SL CBR measurement result is available, a (pre-)configured SL CBR value is used

## RAN1 #107b-e

Agreement

When UE is configured to perform partial sensing by a UE higher layer (including when SL DRX is configured), SL RSSI is measured in slots where the UE performs partial sensing and PSCCH/PSSCH reception over the SL CBR measurement window defined in Rel-16. The calculation of SL CBR is limited within the slots for which the SL RSSI is measured.

- If the number of SL RSSI measurement slots is below a (pre-)configured threshold, a (pre-)configured SL CBR value is used.

R1-2200723	FL summary #4 for AI 8.11.1.1 – resource allocation for power saving	Moderator (OPPO)

Agreement

When UE is triggered to perform re-evaluation and pre-emption checking for aperiodic transmission (Prsvp_TX=0) in slot n,

- The candidate resource set (SA) is initialized to the remaining Y’ candidate slots that starts from slot tyiSL and ends at the last slot of the Y’ candidate slots.

  - tyiSL is the first candidate slot after slot n+T3.

- UE may perform PBPS for periodic sensing occasions after the resource (re)selection when sl-MultiReserveResource is enabled for the mode 2 Tx resource pool

  - It is up to UE implementation

- UE performs CPS starting from at least M consecutive logical slots earlier than tyiSL to Tproc,0SL+Tproc,1SL slots earlier than tyiSL.

  - FFS: When the minimum M slots for CPS cannot be guaranteed,

- All available sensing results not earlier than n–T0 for the resource pool indicated by higher layer are applied for re-evaluation and pre-emption checking procedures

Agreement

When UE performs at least contiguous partial sensing in a mode 2 Tx pool for a resource (re)selection procedure and re-evaluation/pre-emption checking triggered by aperiodic transmission (Prsvp_TX=0) in slot n,

- For minimum size M of the CPS monitoring window [n+TA, n+TB]:

  - By default, M is 31 unless (pre-)configured with another value

  - The range of (pre-)configured M is from 0 (working assumption) to 30

R1-2200786	FL summary #5 for AI 8.11.1.1 – resource allocation for power saving	Moderator (OPPO)

Agreement

When UE performs only contiguous partial sensing (CPS) in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) disabled, and a resource (re)selection is triggered in slot n,

- T1 is defined based on step 1) of Rel-16 TS 38.214 Sec. 8.1.4.

  - No update to specification is necessary due to this agreement

- Note: The selected Y’ slots do not overlap with the sensing window

Agreement

Whether UE performs SL reception of PSCCH and RSRP measurement for partial sensing on slots in SL DRX inactive time is enabled/disabled by (pre-)configuration per resource pool when partial sensing is configured in the UE by a higher layer.

- When it is enabled,

  - When UE performs periodic-based partial sensing for a given Preserve, UE monitors only the default periodic sensing occasion.

  - When UE performs contiguous partial sensing, UE monitors a minimum of M slots for CPS.

- Note, when it is disabled, the UE is not required to perform SL reception of PSCCH and RSRP measurement in SL DRX inactive time.

- Note: no further optimization on the resource (re)selection procedure with regard to SL DRX operation is specified in Rel.17.

- FFS the case when full sensing is configured in the UE by a higher layer