3GPP TSG RAN WG1 Meeting #108-e	R1-2200983

e-Meeting, Feb. 21 – Mar. 3, 2022

Agenda Item:	8.11.1.2

Source:	FUTUREWEI

Title:	Discussion on techniques for inter-UE coordination

Document for:	Discussion and decision

# Introduction

At RAN#86, a WI on sidelink enhancements was agreed for Rel-17 [1] and modified in [2].  The latest update of the WID is in [3]. In this WI, an objective on resource allocation enhancements to enhance reliability is:

- Study the feasibility and benefit of the enhancement(s) in mode 2 for enhanced reliability and reduced latency in consideration of both PRR and PIR defined in TR37.885 (by RAN#91), and specify the identified solution if deemed feasible and beneficial [RAN1, RAN2]

  - Inter-UE coordination with the following.

    - A set of resources is determined at UE-A. This set is sent to UE-B in mode 2, and UE-B takes this into account in the resource selection for its own transmission.

In RAN1#104e, an LS was sent to RAN [4] informing the status the study, with a companion document [5] capturing detailed observations from the evaluation results for inter-UE coordination in Mode 2.

Conclusion: - RAN1 concludes that the inter-UE coordination in Mode 2 is feasible, and is beneficial (e.g., reliability, etc.) compared to Rel-16 Mode 2 RA, and thus recommends specification of the feature. - The detailed observations can be found in the attachment of the LS

It was concluded in RAN#91-e and RAN#92-e that no WID update is necessary. WGs continue specifying inter-UE coordination.

At RAN1#107-e, a set of agreements was reached and is listed in the Appendix.

Since the Rel-17 SL WI normative work was not finished in RAN1#107-e, the scope was discussed in RAN#94-e. It was agreed in RAN#94-e that RAN1 is to complete the remaining normative work for Rel-17 NR sidelink enhancement by Q1 of 2022 and all RAN1 decisions that impact other WGs should be finalized in RAN1#107bis-e.

In the status report of Rel-17 SL WI for RAN#94-e [6], a list of remaining open issues on inter-UE coordination was summarized and copied below.

  - Scheme 1

    - Finalization of contents and containers of UE-A’s inter-UE coordination information and UE-B’s explicit request, including determination of destination UE(s) for UE-A’s inter-UE coordination information and UE-B’s explicit request

    - Finalization of behaviour of UE-B receiving resource set(s) from UE-A(s)

    - Finalization of when and with which information UE-A generates and/or transmits an inter-UE coordination information, including triggering based on condition(s) other than an explicit request

    - Finalization of when UE-B generates and/or transmits an explicit request

    - Finalization of resource selection and/or multiplexing with sidelink transmissions for UE-A’s inter-UE coordination information and UE-B’s explicit request

    - Finalization of prioritization of inter-UE coordination information and explicit request

    - Combination of preferred/non-preferred resources with explicit request/condition triggers

  - Scheme 2

    - Finalization of determination of PSFCH resource/index for conflict indication

    - Finalization of behaviour of UE-B receiving a conflict indication from UE-A

    - Finalization of prioritization of conflict indication

    - Finalization of how to determine UE-B among UEs scheduling conflicting TBs, including whether/how to handle, or differently handle, the case when at least one of UEs scheduling conflicting TBs doesn’t support Scheme 2

Most remaining open issues were discussed in RAN1#107bis-e. A set of agreements was reached and listed in the Appendix. In this contribution, we continue the discussions on the remaining open issues but not yet resolved or not discussed in RAN1#107bis-e.

# Discussions

## Inter-UE Coordination Scheme 1

### Content for coordination information

For the content or the format of the coordination information, it was agreed that the Rel-16 TRIV/FRIV pair is reused to specify the resources in the preferred and non-preferred resource sets. In Rel-16, for the TRIV, the first resource slot is the one where the SCI-1 transmitted. For coordination information, the first resource location has to be indicated separately. In RAN1#107bis-e, a consensus was reached that the first resource location is indicated by a slot offset to a common reference slot. For the slot offset, two alternatives were proposed, one is preconfigured candidate slot offset list (then the index is indicated in the coordination information), and the other one is to indicate the slot offset to the reference slot directly in the coordination information. Eventually, alt 2 was adopted as in the following working assumption agreed in RAN1#107bis-e.

Working Assumption First resource location of each TRIV is a slot offset with respect to a reference slot - The slot offset is the number of logical slots from the reference slot - The value range of slot offsets is from 0 to maximum value that is (pre)configurable up to [256] - FFS: The detailed value range including granularity - Slot offset for each TRIV to indicate the set of resources is separately indicated by inter-UE coordination information - For the reference slot, - The reference slot is the slot indicated by the inter-UE coordination information in a form of combination of DFN index and slot index

The remaining issue to be determined is the upper bound of the preconfigurable maximum value of the slot offsets and granularity of preconfigured maximum value. Currently, the upper bound of the maximum value is 256 as in the bracket.

The initial proposal from the FL on this value was 8000, which is determined based on the maximum reservation periodicity of 1000ms. Since the largest SCS supported by Rel-16 NR V2X is 120kHz, the total number of slots for the periodicity of 1000ms is then 8000. For the upper bound, we prefer 8000 which covers the worst case, although it may cost 12 bits for each indication. Then for SCS of 60kHz, 30kHz, and 15kHz, the corresponding maximum number of slots in the periodicity of 1000ms is then 4000, 2000, and 1000, respectively. It may be natural to include 4000, 2000, 1000 in the preconfigured list. However, since the configured maximum value can be used in any SCS, to maximize the usage of one size of bit-width, it would be better to choose 2k-1 as the maximum values, so that for each configuration with a consecutive lower value, one bit is saved. Therefore, we can then include 4195, 2047, and 1023 in the preconfigured list of the maximum values. Since it may be possible to introduce a higher SCS in the future releases for sidelink in FR2, for forward looking, we then propose to use 8191 (2^13-1) as the upper bound of maximum value instead of 8000. The lowest value can be 31.

Proposal 1: The value range of slot offsets for the first resource location is from 0 to maximum value that is (pre)configurable up to 8195

  - The preconfigured maximum value list is represented by 2k-1, where k=5, 6, …, 13.

### Content of UE-B’s explicit request

Inter-UE coordination can be explicitly triggered by UE-B when UE-B needs some coordination information from UE A. The coordination can be explicitly triggered with a trigger message sent to UE A. Besides coordination triggering, some information for coordination is also needed for UE-A to obtain and send the coordination message to UE B e.g., parameters for UE A sensing procedures and coordination resource pool, etc. Explicit coordination triggering is suitable for both periodic traffic and aperiodic traffic at UE B. The triggering information sent in the request for condition 1-A-1 preferred resource set was agreed, which includes the priority value, number of subchannels, and resource reservation interval. However, no agreement is reached yet for the condition 1-B-1 for the non-preferred resource. The agreement on starting and ending time of the resource selection window is for Scheme 1 in general though.

For Condition 1-B-1, if inter-UE coordination information is triggered by an explicit request, we think three parameters for condition 1-A-1 are also needed to form non-preferred set. Although the number of sub-channels may not be needed, which may result in different form in the container, it is preferable to have a unified design with the preferred resource set.

Proposal 2: The additional parameters for explicit request condition 1-B-1 are the same as those for condition 1-A-1, i.e.,

- Priority value to be used for PSCCH/PSSCH transmission

- Number of sub-channels to be used for PSSCH/PSCCH transmission in a slot

- Resource reservation interval

### Remaining issues on information needed of UE-A generating coordination information

For inter-UE coordination information transmission triggered by a condition other than explicit request, to form preferred resource set with condition 1-A-1, several parameters are needed for UE-A to perform resource selection. In RAN1#107bis-e, we have the following agreement.

Agreement For determining preferred resource set in Scheme 1, when inter-UE coordination information transmission is triggered by a condition other than explicit request reception, - Values of following parameters are (pre)configured for a resource pool. If there is no (pre)configuration, UE-A determines by its implementation the values of the following parameters - prio_TX - L_subCH - P_rsvp_TX - UE-A determines by its implementation values of following parameters - n+T_1, n+T_2 - FFS: Whether/how to support (pre)configuration of n+T_1 and n+T_2 - Note that it is up to RAN2 decision whether/how the values of these parameters are provided by PC5-RRC signaling from UE-B to UE-A and UE-A uses the received information to determine the preferred resource set

One remaining issue is whether/how to support (pre)configuration of n+T_1 and n+T_2. We prefer to support (pre)configuration of a RSW for UE-A to form preferred set. For the (pre)configuration, we propose to preconfigure T1 and T2 or in the form of T1 and RSW size, i.e., T2-T1 with a specified reference slot n.  For the coordination triggered by a condition, two alternatives have been agreed, i.e., alt 1 up to UE-A’s implementation and alt 2 when UE-A has a data to be transmitted together to UE-B. For both alternatives, the reference slot n can be specified as slot when UE-A starts to process the sensing results and perform resource selection for transmitting coordination information to UE-B.

Proposal 3: For determining preferred resource set in Scheme 1, when inter-UE coordination information transmission is triggered by a condition other than explicit request reception,

- Support (pre)configuration of n+T_1 and n+T_2, where T_1 and T_2 are preconfigured values and n is the reference slot when UE-A start to process the sensing results and perform resource selection for transmitting the coordination information to UE-B.

Similarly, for non-preferred resource set as Condition 1-B-1, if inter-UE coordination information is triggered by a condition, to have unified design, the same parameters are needed for generating coordination information. Also, the parameters can be obtained in the same way as that for the preferred resource set.

Proposal 4: For determining non-preferred resource set in Scheme 1, when inter-UE coordination information transmission is triggered by a condition other than explicit request reception,

- Values of following parameters are (pre)configured for a resource pool. If there is no (pre)configuration, UE-A determines by its implementation the values of the following parameters

  - prio_TX

  - L_subCH

  - P_rsvp_TX

- UE-A determines by its implementation values of following parameters if not preconfigured

  - n+T_1, n+T_2

- If preconfigured, T_1 and T_2 are preconfigured values with a reference slot n, where n is the slot when UE-A is triggered to process the sensing results to form coordination information and perform resource selection for transmitting the coordination information to UE-B.

- It is up to RAN2 decision whether/how the values of these parameters are provided by PC5-RRC signaling from UE-B to UE-A and UE-A uses the received information to determine the non-preferred resource set

### Prioritization of inter-UE coordination information

In scheme 1, for the inter-UE coordination information triggered by a condition, we have the following agreement on the priority value of the inter-UE coordination information in RAN1#107bis-e.

For inter-UE coordination information triggered by a condition other than explicit request reception in Scheme 1, the priority value of the inter-UE coordination information is (pre)configured priority value if it is provided by (pre)configuration. - FFS: Otherwise, the priority value is determined by UE-A’s implementation. - For the case when inter-UE coordination information is transmitted together with other data, the priority value of the multiplexed sidelink transmission is determined by the smallest priority value between the inter-UE coordination information and data

One remaining issue is whether to support that the priority value is determined by UE-A’s implementation.

For coordination triggered by a condition, UE-A can trigger the coordination when UE-A detects the potential conflict of the scheduled PSSCH by UE-B as in Scheme 2. In this case, the priority value of UE-B’s data can be detected by UE-A based on the SCI from UE-B prior to the transmission. Although we prefer to have a preconfigured case where the priority value is indicated by UE-B’s prior SCI, it can also be achieved by UE-A’s implementation. Therefore, we propose to support the priority value determined by UE-A’s implementation.

Proposal 5: For inter-UE coordination triggered by a condition in Scheme 1, support the priority value determined by UE-A’s implementation if it is not provided by (pre)configuration.

### Combination of preferred/non-preferred resources with explicit request/condition triggers

Based on RAN1 agreements on inter-UE coordination scheme 1, two types of inter-UE coordination information and two mechanisms to trigger inter-UE coordination information transmission are supported and listed as follows

- Types of inter-UE coordination information signaling

  - Option A: Set of resources preferred for UE-B’s transmission

  - Option B: Set of resources non-preferred for UE-B’s transmission

- Mechanisms to trigger inter-UE coordination information transmission

  - Option 1: Triggered by an explicit request

  - Option 2: Triggered by a condition other than explicit request reception

Both types of coordination information (Options A and B) can be supported in the coordination triggered by an explicit request (option 1), or coordination triggered by a condition (option 2). Therefore, there is no technical issue on any combinations.  On the other hand, in term of usage of coordination information at UE-B, two types of coordination information at UE-B are not mutually exclusive. Hence, we should not preclude the case that UE A sends both types of information in one coordination process, i.e., to assist the resource selection for the same transmission at UE-B. Based on the agreements on the indication of resource type, the two types of coordination information can be sent separately in two transmissions or in two request/transmission pairs. Therefore, we support all four combinations of the resource types and coordination triggering mechanisms.

Proposal 6: For Scheme 1, support following combinations

- Inter-UE coordination information of preferred resource set triggered by an explicit request

- Inter-UE coordination information of non-preferred resource set triggered by an explicit request

- Inter-UE coordination information of preferred resource set triggered by a condition other than the explicit request reception

- Inter-UE coordination information of non-preferred resource set triggered by a condition other than the explicit request reception

### Cast Type

The cast type for coordination transmission as well as the transmission of the explicit request was discussed in RAN1#107bis-e. For most cases, agreements were reached. For inter-UE coordination triggered by a condition, we have following working assumption from RAN1#107bis-e on the transmission of coordination information, which has several FFS’s to be resolved.

Working Assumption For Scheme 1, following cast type(s) are supported for inter-UE coordination information transmission triggered by a condition other than explicit request reception - Groupcast/Broadcast for non-preferred resource set, FFS for preferred resource set - FFS: Under which conditions groupcast/broadcast can be supported - Unicast - FFS: Under which conditions unicast can be supported

For inter-UE coordination information triggered by a condition other than explicit request reception in Scheme 1, UE-A can support multiple UE-Bs, e.g., in the cases of RSU, truck platooning, or public safety (fire scene), the RSU, leading truck, or commander UE, can coordinate multiple UE-B’s. Therefore, UE-A can groupcast the non-preferred resource sets to multiple UE-Bs to avoid resource collision and/or protect their own transmission/reception. However, the situation is different for the preferred resource set. If UE-A groupcasts/broadcasts the preferred resource set to multiple UE-B’s, every UE-B will prioritize the resources in the same preferred set for its resource selection. Therefore, when the size of preferred resource set is small and the number of UE-Bs for broadcast/groupcast is large, the probability of conflict is high. If the size of preferred resource set is large and the number of UE-B’s is small, it is ok to send the preferred resource set to more-than-one UE-B’s. Since it is difficult to control the number of UEs in broadcast, it is not preferred to support broadcast for the preferred resource set. It is fine to support groupcast for preferred resource set when the size of groupcast is small and the size of preferred resource set is large.

Proposal 7: For Scheme 1, following cast type(s) are supported for inter-UE coordination information transmission triggered by a condition other than explicit request reception,

- Do not support broadcast for preferred resource set and support groupcast for preferred resource set.

For coordination triggered by a condition, as aforementioned, two alternative conditions were agreed, i.e., alt 1 up to UE-A’s implementation and alt 2 when UE-A has a data to be transmitted together to UE-B. As in the exemplary cases of RSU, truck platooning, or public safety mentioned above, the RSU, leading truck, or commander UE can be UE-A. Then groupcast/broadcast of non-preferred resource set should be supported when the coordination is triggered up to UE-A’s implementation in alt. 1. For the alt. 2 scenarios, although the cases where UE-A has the data to send to multiple UE-Bs and these UE-B’s have the data to send UE-As are not common but still happen, it is preferred to support groupcast/broadcast of coordination information for this alternative condition. If groupcast for preferred set is supported, similarly, it can be supported for both conditions.

Proposal 8: For Scheme 1, following cast type(s) are supported for inter-UE coordination information transmission triggered by a condition other than explicit request reception,

- Support groupcast/broadcast for non-preferred resource set for both alt 1 and alt 2 conditions for triggering coordination.

- Support groupcast for preferred resource set for both alt 1 and alt 2 conditions for triggering coordination.

Also as mentioned above, UE-A can trigger the coordination when UE-A detects the potential conflict of the scheduled PSSCH by UE-B as in Scheme 2. UE-A can then send either preferred resource set, non-preferred resource set, or both to UE-B to help UE-B on the resource selection particularly when UE-A is the destination of the TB to be transmitted from UE-B. On the other hand, UE-A can also send the non-preferred resource set to UE-B even when UE-A is not the Rx UE of UE-B. Therefore, unicast should be supported for both preferred and non-preferred resource set. In term of conditions, unicast is certainly supported for alt 1 where coordination triggering condition is up-to UE-A’s implementation. For unicast, it is very common that UE-B and UE-A have data to send to each other. Therefore, when UE-A has data to be transmitted to UE-B, mostly UE-B has data to be transmitted to UE-A. Therefore, unicast should be supported for the alt 2 condition.

Proposal 9: For Scheme 1, following cast type(s) are supported for inter-UE coordination information transmission triggered by a condition other than explicit request reception,

- Support unicast for both preferred resource set and nonpreferred resource set.

- Support unicast for both alt 1 and alt 2 conditions for triggering coordination.

## Inter-UE Coordination Scheme 2

### PSFCH for conflict indication and behaviour of UE-B receiving a conflict indication from UE-A

For inter-UE coordination scheme 2, it was agreed in RAN1#106bis-e that PSFCH format 0 is used to convey the presence of expected/potential resource conflict on reserved resource(s) indicated by UE-B’s SCI. It was also agreed in RAN1#107-e, the PSFCH occasion is derived by a slot where UE-B’s SCI is transmitted with reusing PSSCH-to-PSFCH timing as specified in TS 38.213 or by a slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B’s SCI, i.e., the latest PSFCH slot at least T3 slots before the PSSCH resource. In RAN1#107bis-e, the PSFCH location was further discussed on resource index, PSFCH sequence, conflict indication. However, no consensus was reached on the latest proposal from the FL which is copied below.

- For Scheme 2, - m_0 for a resource conflict indication is derived in the same way as specified for HARQ-ACK information in TS 38.213 Section 16.3 - m_CS for a resource conflict indication for UE-B’s current TB transmission is 0 - FFS: Whether/How the conflict in periodic transmission is indicated by UE-A and handled by UE-B - a UE expects that different PRBs are (pre)configured between for conflict indication and HARQ-ACK information

There are two issues that have different views. First on the PSFCH resource, most companies support different PRBs between conflict indication and HARQ-ACK information. However, we support the same PRB set used for conflict indication and HARQ-ACK information to be able to utilize all PSFCH resources. To avoid the potential conflict between the PSFCH for conflict indication and HARQ-ACK, a preconfigured offset on PSFCH index can be applied.

Proposal 10: For scheme 2, support same PSFCH PRB set for the conflict indication.

Second issue is whether and how the conflict in periodic transmission is indicated by UE-A and handled by UE-B. Based on current agreed behavior of UE-B, PHY layer at UE-B reports resources overlapping with the next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission to higher layer. Therefore, once a conflict is detected, UE-B will reselect the resources for all its subsequent periodic transmissions. It is then unnecessary to indicate the conflict in periodic transmission from UE-A. Consequently, we do not need to specify the UE-B’s behavior for the indication of conflict in periodic transmissions.

Proposal 11: For scheme 2, there is no need to indicate the conflict in periodic transmission or UE-B’s behavior on the conflict.

### Determination of UE-B

In RAN1#107-e, during the discussions of additional criteria to determine resources where expected/potential resource conflict occurs for Condition 2-A-1 in Scheme 2, an issue was raised that UE-B should be determined first when applying the criteria. After discussions, the following working assumption was made.

Working assumption (from RAN1#107-e) For Condition 2-A-1 in Scheme 2, when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is enabled or when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is disabled and the destination UE of the conflicting TBs is UE-A, for each pair of UEs scheduling the conflicting TBs, a UE with the higher priority value is UE-B. - FFS whether/how to set additional condition for UE-A to send PSFCH. - Conclude on whether/how to handle, or differently handle, the case when at least one of UEs scheduling conflicting TBs doesn’t support Scheme 2 at the subsequent meetings

Further in RAN1#107bis-e, one bit indication on whether a UE supports of receiving conflict indication in inter-UE coordination Scheme 2 as in the following working assumptions.

Working Assumption (from RAN1#107bis-e) For Scheme 2, (pre)configuration is supported to enable or disable that 1 LSB of reserved bits of a SCI format 1-A is used to indicate of whether UE scheduling a conflict TB can be UE-B or not. - FFS: UE-A's behavior for the case when at least one of UEs scheduling conflicting TBs is not capable of receiving the conflict indication

With the 1-bit indication, UE-A knows whether a UE supports Scheme 2 Rx or not. However, since the 1-bit indication enabled or disabled by pre-configuration, once it is disabled, there is still an unknown state on whether the UE supports Scheme 2 Rx or not. We now further discuss behavior of UE-A on determining UE-B based current agreed working assumptions.

First, there is one case that was not considered in the working assumption from RAN1#107-e, i.e., the two UEs have the same priority value. Based on the working assumption, none of the UEs will be UE-B, which is not desired when the two UEs have the same lowest priority vale. In this case, we can determine UE-B considering the following different scenarios.

- If UE-A is the destination of both UEs, the one with lower RSRP measurement should be UE-B as the one with high RSRP has higher reliability or high data rate (better resource utilization)

- If UE-A is neither the destination of the two UEs, one of the UEs is randomly chosen to be UE-B, which is effectively up-to UE-A’s implementation.

- If UE-A is the destination of one UE but not the other, to protect its own data, UE-A will report the conflict to the other UE. Therefore, the UE with the scheduled TB to the UE other than UE-A is UE-B.

Proposal 12: For Condition 2-A-1 in Scheme 2, when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is enabled or when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is disabled and the destination UE of the conflicting TBs is UE-A, for each pair of UEs scheduling the conflicting TBs, if they have the same priority value,

- if UE-A is the destination of both UEs, the one with lower RSRP is UE-B;

- if UE-A is neither the destination of the two UEs, it is up-to UE-A’s implementation to select one of the UEs to be UE-B;

- if UE-A is the destination of one UE but not the other, the UE with the scheduled TB to the UE other than UE-A is UE-B.

Second, the working assumption from RAN1#107-e only supports the case where both UEs support Scheme 2 since there is a subbullet to conclude whether/how other cases. Based on the working assumption from RAN1#107bis-e, it is possible that a UE indicates that it cannot be UE-B. The UE-A’s behavior or how to determine UE-B has to be specified.

Among the UEs scheduling the same conflict TBs, the UEs who do not support Scheme 2 cannot be UE-B. Then if there is at least one of UEs scheduling conflicting TBs who does not support Scheme 2, all the UEs who support Scheme 2 are UE-Bs and UE-A needs to report the conflict to these UEs.

Proposal 13: For Condition 2-A-1 in Scheme 2, when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is enabled or when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is disabled and the destination UE of the conflicting TBs is UE-A, among the UEs scheduling the same conflict TBs, if at least one of UEs scheduling conflicting TBs does not support Scheme 2

- all the UEs who support Scheme 2 are UE-Bs.

As aforementioned, since the 1-bit indication enabled or disabled by pre-configuration, it is possible that UE-A does not know whether the UE supports Scheme 2 Rx or not.  In this case, we can follow the above procedure and allow the unknown UEs to be UE-Bs if at least one of UEs scheduling conflicting TBs is known to not support Scheme 2 Rx. If all UEs scheduling the same conflict TB are unknown or supporting Scheme 2, the UEs who support Scheme 2 are UE-Bs. Then for the rest of UEs who are unknown to supporting Scheme 2, we can follow the rule in the work assumption from RAN#107-e and the above proposal when the UEs have the same lowest priority value.

Proposal 14: For Condition 2-A-1 in Scheme 2, when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is enabled or when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is disabled and the destination UE of the conflicting TBs is UE-A, among the UEs scheduling the same conflict TB,

- if at least one of UEs scheduling conflicting TBs does not support Scheme 2 Rx, all other UEs with unknown capability or supporting Scheme 2 Rx are UE-B’s;

- if all UEs are unknown or supporting Scheme 2 Rx,

  - all UEs supporting scheme 2 Rx are UE-B’s

  - if more-than one UEs are unknown of supporting Scheme 2 Rx, for each pair of UE,

    - UE with the higher priority value is UE-B

    - if they have the same priority value, the one with lower RSRP measurement is UE-B if UE-A is the destination of both UEs, or up-to UE-A’s implementation to select one to be UE-B if UE-A is the destination of neither UEs, or the UE with the scheduled TB to the UE other than UE-A is UE-B.

### Cast Type

For Scheme 2, if UE-A is a destination UE of TB transmitted by UE-B, UE-A reports the conflict via PSFCH channel. Currently, PSFCH detection is supported in unicast and groupcast. Therefore, at least unicast and groupcast should be supported as UE-B’s transmission.

In the cases of RSU, truck platooning, or public safety (fire scene), the RSU, leading truck, or commander UE can report a conflict even if it is not a receiver or one of the receivers for other UE’s transmission. Again, in terms of cast type for UE-B’s transmission, at least unicast and groupcast should be supported as UE-B can detect PSFCH.

Overall, regardless of UE-A being the destination of a TB of UE-B, at least unicast and groupcast of UE-B’s transmissions should be supported in Scheme 2.

Proposal 15: For inter-UE coordination information in Scheme 2, at least when UE-B’s transmission is unicast or groupcast, UE-A can transmit the inter-UE coordination information to UE-B.

## Combination of Scheme 1 and Scheme 2

Although it may be separately configured, scheme 1 with condition-based feedback and scheme 2 can be enabled and performed in one coordination process, i.e., conditioned on the expected/potential conflict detected by UE-A. Besides reporting the conflict, UE-A can also send the preferred and/or non-preferred resource set to UE-B to avoid further conflict after resource reselection. It can be achieved with coordination scheme 1 triggered by condition as discussed before.

Proposal 16: The combination of Scheme 1 and Scheme 2 is not prohibited.

- If (pre-)configured or up-to UE-A’s implementation, in addition to report the conflict as in Scheme 2, UE-A may also send preferred and/or non-preferred resources, as in Scheme 1, to UE-B for resource reselection at UE-B.

# Conclusion

The remaining open issues for inter-UE coordination were discussed. We propose the following:

Proposal 1: The value range of slot offsets for the first resource location is from 0 to maximum value that is (pre)configurable up to 8195

  - The preconfigured maximum value list is represented by 2k-1, where k=5, 6, …, 13.

Proposal 2: The additional parameters for explicit request condition 1-B-1 are the same as those for condition 1-A-1, i.e.,

- Priority value to be used for PSCCH/PSSCH transmission

- Number of sub-channels to be used for PSSCH/PSCCH transmission in a slot

- Resource reservation interval

Proposal 3: For determining preferred resource set in Scheme 1, when inter-UE coordination information transmission is triggered by a condition other than explicit request reception,

- Support (pre)configuration of n+T_1 and n+T_2, where T_1 and T_2 are preconfigured values and n is the reference slot when UE-A start to process the sensing results and perform resource selection for transmitting the coordination information to UE-B.

Proposal 4: For determining non-preferred resource set in Scheme 1, when inter-UE coordination information transmission is triggered by a condition other than explicit request reception,

- Values of following parameters are (pre)configured for a resource pool. If there is no (pre)configuration, UE-A determines by its implementation the values of the following parameters

  - prio_TX

  - L_subCH

  - P_rsvp_TX

- UE-A determines by its implementation values of following parameters if not preconfigured

  - n+T_1, n+T_2

- If preconfigured, T_1 and T_2 are preconfigured values with a reference slot n, where n is the slot when UE-A is triggered to process the sensing results to form coordination information and perform resource selection for transmitting the coordination information to UE-B.

- It is up to RAN2 decision whether/how the values of these parameters are provided by PC5-RRC signaling from UE-B to UE-A and UE-A uses the received information to determine the non-preferred resource set

Proposal 5: For inter-UE coordination triggered by a condition in Scheme 1, support the priority value determined by UE-A’s implementation if it is not provided by (pre)configuration.

Proposal 6: For Scheme 1, support following combinations

- Inter-UE coordination information of preferred resource set triggered by an explicit request

- Inter-UE coordination information of non-preferred resource set triggered by an explicit request

- Inter-UE coordination information of preferred resource set triggered by a condition other than the explicit request reception

- Inter-UE coordination information of non-preferred resource set triggered by a condition other than the explicit request reception

Proposal 7: For Scheme 1, following cast type(s) are supported for inter-UE coordination information transmission triggered by a condition other than explicit request reception,

- Do not support broadcast for preferred resource set and support groupcast for preferred resource set.

Proposal 8: For Scheme 1, following cast type(s) are supported for inter-UE coordination information transmission triggered by a condition other than explicit request reception,

- Support groupcast/broadcast for non-preferred resource set for both alt 1 and alt 2 conditions for triggering coordination.

- Support groupcast for preferred resource set for both alt 1 and alt 2 conditions for triggering coordination.

Proposal 9: For Scheme 1, following cast type(s) are supported for inter-UE coordination information transmission triggered by a condition other than explicit request reception,

- Support unicast for both preferred resource set and nonpreferred resource set.

- Support unicast for both alt 1 and alt 2 conditions for triggering coordination.

Proposal 10: For scheme 2, support same PSFCH PRB set for the conflict indication.

Proposal 11: For scheme 2, there is no need to indicate the conflict in periodic transmission or UE-B’s behavior on the conflict.

Proposal 12: For Condition 2-A-1 in Scheme 2, when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is enabled or when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is disabled and the destination UE of the conflicting TBs is UE-A, for each pair of UEs scheduling the conflicting TBs, if they have the same priority value,

- if UE-A is the destination of both UEs, the one with lower RSRP is UE-B;

- if UE-A is neither the destination of the two UEs, it is up-to UE-A’s implementation to select one of the UEs to be UE-B;

- if UE-A is the destination of one UE but not the other, the UE with the scheduled TB to the UE other than UE-A is UE-B.

Proposal 13: For Condition 2-A-1 in Scheme 2, when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is enabled or when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is disabled and the destination UE of the conflicting TBs is UE-A, among the UEs scheduling the same conflict TBs, if at least one of UEs scheduling conflicting TBs does not support Scheme 2

- all the UEs who support Scheme 2 are UE-Bs.

Proposal 14: For Condition 2-A-1 in Scheme 2, when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is enabled or when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is disabled and the destination UE of the conflicting TBs is UE-A, among the UEs scheduling the same conflict TB,

- if at least one of UEs scheduling conflicting TBs does not support Scheme 2 Rx, all other UEs with unknown capability or supporting Scheme 2 Rx are UE-B’s;

- if all UEs are unknown or supporting Scheme 2 Rx,

  - all UEs supporting scheme 2 Rx are UE-B’s

  - if more-than one UEs are unknown of supporting Scheme 2 Rx, for each pair of UE,

    - UE with the higher priority value is UE-B

    - if they have the same priority value, the one with lower RSRP measurement is UE-B if UE-A is the destination of both UEs, or up-to UE-A’s implementation to select one to be UE-B if UE-A is the destination of neither UEs, or the UE with the scheduled TB to the UE other than UE-A is UE-B.

Proposal 15: For inter-UE coordination information in Scheme 2, at least when UE-B’s transmission is unicast or groupcast, UE-A can transmit the inter-UE coordination information to UE-B.

Proposal 16: The combination of Scheme 1 and Scheme 2 is not prohibited.

- If (pre-)configured or up-to UE-A’s implementation, in addition to report the conflict as in Scheme 2, UE-A may also send preferred and/or non-preferred resources, as in Scheme 1, to UE-B for resource reselection at UE-B.

# References

RP-193257, “NR sidelink enhancement”, RAN#86-e, Dec 2019.

RP-201385, “WID revision: NR sidelink enhancement,” LG Electronics, RAN#88-e, June 29-July 3, 2020.

RP-202846, “WID revision: NR sidelink enhancement”, RAN#90-e, Dec. 7-11, 2020.

R1-2102168, “LS on Mode 2 enhancements in NR sidelink”, LGE, RAN1#104e, Jan. 25-Feb. 5, 2021

R1-2102166, “Detailed observations from evaluation results”, LGE, RAN1#104e, Jan. 25-Feb. 5, 2021

RP-212880, “Status report of WI: NR Sidelink enhancement”, RAN1, Dec. 6-17, 2021.

# Appendix

The agreements from RAN1#107bis-e are listed below.

Agreement

For Scheme 1, when the inter-UE coordination information transmission is triggered by UE-B’s explicit request,

- Starting/Ending time locations of resource selection window is provided by UE-B’s explicit request

    - Starting/Ending time locations of resource selection window is a form of combination of DFN index and slot index

Agreement

- When PSFCH occasion is derived by a slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B’s SCI, time gap between the PSFCH and SCI(s) scheduling conflicting TBs is larger than or equal to X value

    - X = sl-MinTimeGapPSFCH

- UE does not transmit the conflict indicator or receive the conflict indicator if the timeline is not satisfied

Agreement

For Scheme 1, a resource pool level (pre-)configuration can enable one of the following alternatives:

- (Working Assumption) Alt1: MAC CE and 2nd SCI are used as the container of an explicit request transmission from UE-B to UE-A

    - A single format SCI 2-C is used for inter-UE coordination information and request

      - 1 bit in format 2-C is used to indicate whether the SCI is used for request to coordination information or for conveying coordination information

    - SCI 2-C is UE RX optional

    - It is up to UE implementation to additionally use 2nd SCI (for UE-B).

- Alt2: MAC CE is used as the container of an explicit request transmission from UE-B to UE-A

Conclusion

For Scheme 2, there is no consensus to support the indication of the following

- Condition type of a resource conflict

- Time location of a resource conflict

Agreement

For Scheme 2,

- The PHY layer reports S_A after Step 7) of TS 38.214 Section 8.1.4 to higher layer.

- When UE-B receives a conflict indicator for resource(s) indicated by its SCI,

  - PHY layer at UE-B reports resources overlapping with the next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission to higher layer.

    - If (pre)configured, the PHY layer reports resources in a slot including the next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission to higher layer.

  - Higher layer at UE-B re-selects the resource(s) indicated by the conflict indicator among the S_A excluding the reported resources.

- FFS: Whether/How the conflict in periodic transmission is indicated by UE-A and handled by UE-B

Agreement

For PSFCH TX/RX or TX/TX prioritization in Scheme 2,

- Priority value of PSFCH TX for a resource conflict indication is the smallest priority value of the conflicting TBs

- Priority value of PSFCH RX for a resource conflict indication is priority value indicated by UE-B’s SCI

- For PSFCH TX/RX or TX/TX prioritization between SL HARQ-ACK feedback(s) and resource conflict indication(s), PSFCH TX/RX for SL HARQ-ACK feedback is always prioritized over PSFCH TX/RX for a resource conflict indication

Agreement

For Scheme 1, unicast is supported for an explicit request transmission for inter-UE coordination information

- Unicast is used for the inter-UE coordination information transmission triggered by the explicit request

Working Assumption

For Scheme 1, following cast type(s) are supported for inter-UE coordination information transmission triggered by a condition other than explicit request reception

- Groupcast/Broadcast for non-preferred resource set, FFS for preferred resource set

  - FFS: Under which conditions groupcast/broadcast can be supported

- Unicast

- FFS: Under which conditions unicast can be supported

Agreement

For determining preferred resource set in Scheme 1, the value of Cresel is determined by UE-A according to Rel-16 procedure.

- This information is not conveyed to/from UE-B

- When inter-UE coordination information is triggered by UE-B’s request, P_rsvp_TX used for determining SL_RESOURCE_RESELECTION_COUNTER according to Rel-16 procedure is provided by resource reservation interval indicated by UE-B’s request

Agreement

For the indication of resource set in Scheme 1, the value of Sl-MaxNumPerReserve is fixed to 3.

Agreement

The following working assumption is confirmed with modification in RED.

- MAC CE or 2nd SCI are used as the container of inter-UE coordination information transmission from UE A to UE B.

      - For the indication of resource set, the following is supported:

        - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request.

          - First resource location of each TRIV is separately indicated by the inter-UE coordination information

        - If [N <= 3], MAC CE is used and it is up to UE implementation to additionally use 2nd SCI. When 2nd SCI and MAC CE are both used, the same resource set is indicated in the 2nd SCI and the MAC CE. If [N > 3], only MAC CE is used.

          - FFS: UE capability details

          - 2nd SCI is UE RX optional

          - The field size of the indication of resource set in a SCI format 2-C is determined by [N=3]

Agreement

- For inter-UE coordination information transmission in Scheme 1,

  - Inter-UE coordination information can be multiplexed with other data only if the source/destination ID pair is the same

    - Retransmission of the TB carrying inter-UE coordination information is supported

- For explicit request transmission in Scheme 1,

  - Explicit request can be multiplexed with other data only if the source/destination ID pair is the same

    - Retransmission of the TB carrying request is supported

Agreement

For inter-UE coordination triggered by an explicit request in Scheme 1, whether or not to transmit the inter-UE coordination information upon the request reception is determined by UE-A’s implementation subject to the following procedures.

- Rel-16 procedure of UL/SL prioritization, LTE SL/NR SL prioritization, and congestion control

Agreement

For inter-UE coordination triggered by a condition rather than request reception in Scheme 1,

- A resource pool level (pre-)configuration can enable one of the following alternatives:

  - Alt 1: it is up to UE-A’s implementation whether or not to trigger the inter-UE coordination information generation.

  - Alt 2: the inter-UE coordination information generation can be triggered only when UE-A has data to be transmitted together with the inter-UE coordination information to UE-B

- Note: Rel-16 procedure of UL/SL prioritization, LTE SL/NR SL prioritization, and congestion control is applied to the transmission of the inter-UE coordination information triggered by a condition.

Agreement

For inter-UE coordination triggered by UE-B’s explicit request in Scheme 1,

- A resource pool level (pre-)configuration can enable one of the following alternatives:

  - Alt 1: it is up to UE-B’s implementation whether or not to trigger the request generation

  - Alt 2: the request generation can be triggered only when UE-B has data to be transmitted to UE-A

- Note: Rel-16 procedure of UL/SL prioritization, LTE SL/NR SL prioritization, and congestion control is applied to the transmission of the request transmission.

Agreement

For Scheme 1 with preferred resource set Option A,

  - MAC layer selects resources using S_A and the received preferred resource set

    - MAC layer firstly selects resources for transmissions within the intersection of S_A and the preferred resource set until it becomes impossible to select a resource within the intersection under the constraint defined in Rel-16.

      - It is up to the UE whether to use the preferred resource set from SCI format 2-C and/or MAC CE

    - After this, if the number of selected resources is smaller than the required number of transmissions for a TB, MAC layer selects resources for the remaining transmissions outside the intersection but inside S_A under the constraint defined in Rel-16.

Agreement

For Scheme 1 with preferred resource set Option B,

  - MAC layer selects resources belonging to the received preferred resource set under the constraint defined in Rel-16

    - It is up to the UE whether to use the preferred resource set from SCI format 2-C and/or MAC CE

Agreement

For inter-UE coordination information triggered by an explicit request in Scheme 1, the priority value of the inter-UE coordination information is (pre)configured priority value if it is provided by (pre)configuration. Otherwise, the priority value is the same as indicated by UE-B’s explicit request.

- For the case when inter-UE coordination information is transmitted together with other data, the priority value of the multiplexed sidelink transmission is determined by the smallest priority value between the inter-UE coordination information and data

Agreement

For inter-UE coordination information triggered by an explicit request in Scheme 1, the priority value of explicit request is (pre)configured priority value if it is provided by (pre)configuration. Otherwise, the priority value is the same as that of a TB to be transmitted by UE-B.

- For the case when the explicit request is transmitted together with other data, the priority value of the multiplexed sidelink transmission is determined by the smallest priority value between the explicit request and data

Agreement

For inter-UE coordination information triggered by a condition other than explicit request reception in Scheme 1, the priority value of the inter-UE coordination information is (pre)configured priority value if it is provided by (pre)configuration.

- FFS: Otherwise, the priority value is determined by UE-A’s implementation.

- For the case when inter-UE coordination information is transmitted together with other data, the priority value of the multiplexed sidelink transmission is determined by the smallest priority value between the inter-UE coordination information and data

Agreement

- For sidelink transmission carrying inter-UE coordination information in Scheme 1,

  - UE-A performs its resource (re)selection according to the same procedure in TS 38.214 Section 8.1.4 to transmit the inter-UE coordination information to UE-B.

- For sidelink transmission carrying request in Scheme 1,

  - UE-B performs its resource (re)selection according to the same procedure in TS 38.214 Section 8.1.4 to transmit the request for the inter-UE coordination information to UE-A if UE-B performs sensing/resource exclusion. Otherwise, at least UE-B can perform random selection

- Note: RAN1 does not pursue specific enhancement of Rel-17 resource (re)selection for the transmission of inter-UE coordination information and its request.

Working Assumption

First resource location of each TRIV is a slot offset with respect to a reference slot

- The slot offset is the number of logical slots from the reference slot

  - The value range of slot offsets is from 0 to maximum value that is (pre)configurable up to [256]

    - FFS: The detailed value range including granularity

  - Slot offset for each TRIV to indicate the set of resources is separately indicated by inter-UE coordination information

- For the reference slot,

  - The reference slot is the slot indicated by the inter-UE coordination information in a form of combination of DFN index and slot index

Agreement

For determining preferred resource set in Scheme 1, when inter-UE coordination information transmission is triggered by a condition other than explicit request reception,

- Values of following parameters are (pre)configured for a resource pool. If there is no (pre)configuration, UE-A determines by its implementation the values of the following parameters

  - prio_TX

  - L_subCH

  - P_rsvp_TX

- UE-A determines by its implementation values of following parameters

  - n+T_1, n+T_2

- FFS: Whether/how to support (pre)configuration of n+T_1 and n+T_2

- Note that it is up to RAN2 decision whether/how the values of these parameters are provided by PC5-RRC signaling from UE-B to UE-A and UE-A uses the received information to determine the preferred resource set

Agreement

For inter-UE coordination information is triggered by UE-B’s request,

- A resource pool level (pre-)configuration can enable one of the following alternatives:

  - Alt 1:

    - Resource set type to be provided by inter-UE coordination information transmission is determined by UE-A’s implementation and its information is indicated by UE-A’s inter-UE coordination information

      - UE-A’s inter-UE coordination information indicates either preferred resource set or non-preferred resource set

  - Alt 2:

    - Resource set type to be provided by inter-UE coordination information transmission is indicated by UE-B’s request

      - UE-B’s request indicates either preferred resource set or non-preferred resource set

  - Note that it is up to RAN2 decision whether/how UE-B provides its support of sensing/resource exclusion to UE-A via PC5-RRC signaling and UE-A uses the received information to determine the type of resource set to be transmitted to UE-B

Agreement

For inter-UE coordination information is triggered by a condition other than explicit request reception,

- Resource set type to be provided by inter-UE coordination information transmission is determined by UE-A’s implementation and its information is indicated by UE-A’s inter-UE coordination information

  - UE-A’s inter-UE coordination information indicates either preferred resource set or non-preferred resource set

Working Assumption

For Scheme 2, (pre)configuration is supported to enable or disable that 1 LSB of reserved bits of a SCI format 1-A is used to indicate of whether UE scheduling a conflict TB can be UE-B or not.

- FFS: UE-A's behavior for the case when at least one of UEs scheduling conflicting TBs is not capable of receiving the conflict indication