3GPP TSG RAN WG1 Meeting#108-e	R1-2201008

e-Meeting, February 21st – March 3rd, 2022

Agenda item:		8.12.3

Source:	Nokia, Nokia Shanghai Bell

Title:		Remaining issues on Broadcast / Multicast for RRC_IDLE / RRC_INACTIVE UEs

Document for:		Discussion and Decision

# Introduction

In this contribution, we provide our considerations on remaining issues of maintenance for broadcast/multicast for RRC_IDLE/INACTIVE UEs. In our companion contribution, we also provide the maintenance considerations related to group scheduling mechanism to support broadcast/multicast services for RRC_CONNECTED UEs in [1], and related to improve reliability of broadcast/multicast services for RRC_CONNECTED UEs in [2].

# Discussion

## CFR/BWP considerations for broadcast services of RRC_IDLE/INACTIVE UEs

Based on the outcome from RAN2#116bis-e, the following agreement has been made regarding on support of CFR case E for broadcast services of RRC_IDLE/INACTIVE UEs:

Agreement:

- CFR case E:

- In the email discussion (R2-2201838) during the meeting basically all the companies were fine to support CFR case E and thus we agreed:

  - RAN2 confirms to support CFR Case E.

  - It is supported by configuring a CFR for MBS broadcast, which fully contains the CORESET#0 in the frequency domain and has the same CP&SCS as the initial BWP.

CFR Case E is supported based on RAN2 discussions, and the configured CFR for broadcast reception is considered as a common frequency resource in addition to the CORESET#0 initial BWP for RRC_IDLE/INACTIVE UEs. Moreover, during the email discussion in RAN1#107b-e, it is understood that single CFR in Rel17 MBS for broadcast reception for RRC_IDLE/INACTIVE UEs is preferred by majority companies.

Observation-1: CFR Case E is supported based on RAN2 outcome agreement.

Observation-2: the configured single CFR for broadcast reception is considered as a common frequency resource in addition to the CORESET#0 initial BWP for RRC_IDLE/INACTIVE UEs.

## 38.213 TP

We have the following text proposals based on our understanding from email discussions of previous RAN1 meetings in Rel17 MBS.

- Based on the latest update from the editor according to the endorsed TP from RAN1#107bis-e, the sentence by using “neither … nor” and following by “is NOT provided” created confusion. Our understanding of this case is that, if “pdcch-Config-MCCH” or “both pdcch-Config-MCCH and pdcch-Config-MTCH” is not provided, searchSpaceZero in PDCCH-ConfigCommon will be applied for both MCCH PDCCH and MTCH PDCCH. Thus, we have the following TP based on the endorsed TP from RAN1#107bis-e meeting to avoid confusion:

Table-1

10.1 UE procedure for determining physical downlink control channel assignment A set of PDCCH candidates for a UE to monitor is defined in terms of PDCCH search space sets. A search space set can be a CSS set or a USS set. A UE monitors PDCCH candidates in one or more of the following search spaces sets - a Type0-PDCCH CSS set on the primary cell of the MCG configured by - pdcch-ConfigSIB1 in MIB or by searchSpaceSIB1 in PDCCH-ConfigCommon or by searchSpaceZero in PDCCH-ConfigCommon for a DCI format 1_0 with CRC scrambled by a SI-RNTI, or - searchSpaceZero in PDCCH-ConfigCommon, when pdcch-Config-MCCH or when both pdcch-Config-MCCH and pdcch-Config-MTCH areis not provided, for a DCI format 4_0 with CRC scrambled by a MCCH-RNTI or a G-RNTI - a Type0A-PDCCH CSS set configured by searchSpaceOtherSystemInformation in PDCCH-ConfigCommon for a DCI format 1_0 with CRC scrambled by a SI-RNTI on the primary cell of the MCG ---------------------------- Other parts are omitted. ----------------------------

Proposal-1: The endorsed TP from RAN1#107bis-e may create confusion. Thus, it is proposed the TP in Table-1 to avoid the confusion.

- Latest update is mixed of “searchSpaceBroadcast” and “searchSpace-Broadcast”, and it is better to align all to “searchSpaceBroadcast”, as shown in below Table-2.

Table-2

---------------------------- Other parts are omitted. ---------------------------- A UE does not expect to be configured CSS sets, except for CSS sets provided by searchSpace-Broadcast searchSpaceBroadcast or by searchSpace-Multicast, that result to corresponding total, or per scheduled cell, numbers of monitored PDCCH candidates and non-overlapped CCEs per slot, per group of Xs slots for a corresponding combination Xs,Ys, or per span that exceed the corresponding maximum numbers per slot, or per group of Xs slots for a corresponding combination Xs,Ys, or per span, respectively. For same cell scheduling or for cross-carrier scheduling, a UE does not expect a number of PDCCH candidates, and a number of corresponding non-overlapped CCEs per slot or per span on a secondary cell to be larger than the corresponding numbers that the UE is capable of monitoring on the secondary cell per slot or per span, respectively. If a UE is provided monitoringCapabilityConfig = r16monitoringcapability for the primary cell, except the first span of each slot, the UE does not expect a number of PDCCH candidates and a number of corresponding non-overlapped CCEs per span on the primary cell to be larger than the corresponding numbers that the UE is capable of monitoring on the primary cell per span. For cross-carrier scheduling, the number of PDCCH candidates for monitoring and the number of non-overlapped CCEs per span or per slot are separately counted for each scheduled cell. For all search space sets within a slot n, or within a group of Xs slots for a corresponding combination Xs,Ys, or within a span in slot n, denote by Scss a set of CSS sets, except for CSS sets provided by searchSpace-Broadcast searchSpaceBroadcast or by searchSpace-Multicast, with cardinality of Icss and by Suss a set of USS sets and CSS sets provided by searchSpace-Broadcast or by searchSpace-Multicast with cardinality of Juss. The location of search space sets sj, 0≤j<Juss, in Suss is according to an ascending order of the search space set index. ---------------------------- Other parts are omitted. ----------------------------

Proposal-2: Latest update is mixed of “searchSpaceBroadcast” and “searchSpace-Broadcast”, and it is better to align all to the agreed RRC parameter “searchSpaceBroadcast”.

- The paragraph shown in below Table-3 should be removed.

Table-3

18 Multicast Broadcast Services ---------------------------- Other parts are omitted. ---------------------------- A UE can be configured by cfr-Config-MCCH-MTCH an MBS frequency resource for PDCCH and PDSCH receptions providing MCCH and MTCH [12, TS 38.331]; otherwise, the MBS frequency resource is same as for the CORESET with index 0 that is associated with the Type0-PDCCH CSS set for PDCCH and PDSCH receptions providing MCCH and MTCH. In clauses referring to a higher layer parameter value provided by PDCCH-ConfigCommon or PDSCH-ConfigCommon, when applicable a corresponding higher layer parameter value for MCCH/MTCH PDCCH receptions or PDSCH receptions, respectively, is provided as described in [12, TS 38.331]. A UE can be configured by cfr-Config-Broadcast, an MBS frequency resource within the initial DL BWP for PDCCH and PDSCH receptions [4, TS 38.211]. If cfr-Config- Broadcast does not include locationAndBandwidth-Broadcast, the MBS frequency resource is the initial DL BWP. A UE monitors PDCCH for scheduling PDSCH receptions for MCCH or MTCH as described in clause 10.1. ---------------------------- Other parts are omitted. ----------------------------

Proposal-3: The paragraph shown in below Table-3 should be removed.

# Conclusions

In this contribution, we have discussed the remaining issues on support 5G/NR broadcast for RRC_IDLE/INACTIVE UEs, and we have the following observations and proposals:

Observation-1: CFR Case E is supported based on RAN2 outcome agreement.

Observation-2: the configured single CFR for broadcast reception is considered as a common frequency resource in addition to the CORESET#0 initial BWP for RRC_IDLE/INACTIVE UEs.

Proposal-1: Relate to TS38.213, the endorsed TP from RAN1#107bis-e may create confusion. Thus, it is proposed the TP in Table-1 to avoid the confusion.

Proposal-2: Relate to TS38.213, the latest update is mixed of “searchSpaceBroadcast” and “searchSpace-Broadcast”, and it is better to align all to the agreed RRC parameter “searchSpaceBroadcast”.

Proposal-3: Relate to TS38.213, the paragraph shown in below Table-3 should be removed.

# References

- R1-2201006, “Remaining Issues on Group Scheduling Mechanisms for RRC_CONNECTED UEs supporting MBS”, RAN1#108-e.

- R1-2201007, “Remaining Issues on Reliability Improvements for RRC_CONNECTED UEs supporting MBS”, RAN1#108-e.