3GPP TSG RAN WG1 #108-e                                            R1-2201025

e-Meeting, February 21st – March 3rd, 2022

Source:	New H3C

Title:	UE features for IIoT and URLLC

Agenda Item:	8.16.3

Document for:	Discussion and Decision

- Introduction

In this contribution, we further provide our considerations on UE features for Rel-17 IIoT/URLLC.

- Discussion

In [1], the following UE features on IIoT and URLLC were agreed in RAN1#107b-e meeting.

2.1 For FG 25-9 and FG 25-10

| 25. NR_IIOT_URLLC_enh | 25-9 | Semi-static PUCCH cell switching | Semi-static PUCCH cell switching using configured time-domain domain pattern of applicable PUCCH cell / carrier FFS whether to separate the capability for different numerologies |  | Yes | N/A |  | Per UE | No (TDD only) | No | N/A | Note: this feature applies to cells in the same TAG only | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-10 | PUCCH cell switching based on dynamic indication | PUCCH cell switching based on dynamic indication in the DCI scheduling the PUCCH FFS whether to separate the capability for different numerologies |  | Yes | N/A |  | Per UE | No (TDD only) | No | N/A | Note: this feature applies to cells in the same TAG only | Optional with capability signaling |

| New H3C’s view | Regarding whether to separate the capability for different numerologies for FGs 25-9 and 25-10, we support FGs 25-9 and 25-10 are split into ones for the capability for same numerology and the others for different numerologies between switchable carriers because UE implementation for PUCCH cell switch with different numerology is more complicated compared with same numerology. |
| --- | --- |

2.2 For FG 25-12

| 25. NR_IIOT_URLLC_enh | 25-12 | UE initiating a semi-static channel occupancy with configurations dependent on gNB semi-static channel access configurations | Support initiating a semi-static channel access occupancy by the UE where the corresponding period is the same as, integer multiple of, or inter-factor of the period configured for a semi-static channel occupancy that can be initiated by gNB. | 10-1a | Yes | N/A |  | Per band | N/A | N/A | N/A | The signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| New H3C’s view | We suggest adding 3 components to FG 25-12 as follows in order to guarantee which features are included in FG 25-12: - Component2: Sensing to initiate a semi-static CO or transmit after a gap greater than 16us from any transmission burst within a UE-initiated CO - Component 3: Determination of COT initiator assumption based on rules for configured UL - Component 4: Validating COT initiator assumption indicated in UL scheduling DCI |
| --- | --- |

2.3 For FG 25-18

| 25. NR_IIOT_URLLC_enh | 25-18 | Parallel PUCCH and PUSCH transmission across CCs in inter-band CA | Support simultaneous PUCCH/PUSCH transmissions on different cells [at least] for inter-band CA. |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| New H3C’s view | We suggest removing [at least] because there is already conclusion on no supporting simultaneous PUCCH/PUSCH transmissions on different cells for intra-band CA in Rel-17. |
| --- | --- |

2.4 For FG 25-19

| 25. NR_IIOT_URLLC_enh | 25-19 | RTT-based Propagation delay compensation based on CSI-RS for tracking and SRS | Support RTT-based Propagation delay compensation for time synchronization of the Uu interface based on CSI-RS for tracking and SRS | 2-51, 2-53 | Yes | N/A |  | FS | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-19a | RTT-based Propagation delay compensation based on DL PRS and SRS | Support RTT-based Propagation delay compensation for time synchronization of the Uu interface based on DL PRS and SRS | 25-19, 13-1, 2-53 | Yes | N/A |  | FS | N/A | N/A | N/A | Note: FG 13-1 is now only reported to LMF. If UE reports the support of this FG, it needs to report FG 13-1 to gNB also. | Optional with capability signaling |

| New H3C’s view | We propose the type of FG25-19 and FG25-19a are per UE due to dependency on 2-53. |
| --- | --- |

- Conclusion

In this contribution, we discuss about the UE features list for Rel-17 URLLC/IIoT with the proposals in above table.

- Reference

- R1-2200780, Updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e