3GPP TSG RAN WG1 #108	R1-2201030

e-Meeting, February 21st – March 3rd, 2022

Agenda item:		7.1

Source:	Nokia, Nokia Shanghai Bell

Title:	TPC accumulation, interpretation of KPUSCH(i)

Document for:		Discussion and Decision

# Introduction

In RAN1#107bis discussions for Coverage Enhancements, two interpretations for the KPUSCH(i) for PUSCH slot aggregation were identified [2]. This Rel-15 related ambiguity is causing a blocking situation in Rel-17 Coverage Enhancements WI completion. Thus it is important, not just for Rel-15 clarity, but also for Rel-17 work completion to resolve this question in a timely manner.

# Two interpretations of KPUSCH(i)

The TS38.213 definition for KPUSCH(i) for a PUSCH scheduled with a DCI is as follows [1, Subclause 7.1.1]:

If a UE transmits a PUSCH on active UL BWP of carrier of serving cell using parameter set configuration with index and PUSCH power control adjustment state with index , the UE determines the PUSCH transmission power in PUSCH transmission occasion as [dBm] where, […] - For the PUSCH power control adjustment state for active UL BWP of carrier of serving cell in PUSCH transmission occasion […] - is the PUSCH power control adjustment state for active UL BWP of carrier of serving cell and PUSCH transmission occasion if the UE is not provided tpc-Accumulation, where […] - If a PUSCH transmission is scheduled by a DCI format, KPUSCH(i) is a number of symbols for active UL BWP b of carrier f of serving cell c after a last symbol of a corresponding PDCCH reception and before a first symbol of the PUSCH transmission

The contentious question was how to understand KPUSCH(i) in the last sub-bullet of the quote above in context of PUSCH repetition [2].

Interpretation 1: KPUSCH(i) is defined as the number of OFDM symbols after a last symbol of a corresponding PDCCH reception and before a first symbol of the PUSCH transmission occasion i.

- With this interpretation, value of KPUSCH(i) for a PUSCH transmission occasion can be different for each PUCCH transmission occasion among the transmission occasions that constitute a PUSCH aggregate trasnmitting several repetitions of one TB.

Interpretation 2: KPUSCH(i) is defined as the number of OFDM symbols after a last symbol of a corresponding PDCCH reception and before a first symbol of the first PUSCH transmission repetition for a TB.

- With this interpretation, value of KPUSCH(i) for all PUSCH transmission occasions are the same for the TB.

The main difference between the two interpretations is that the 1st interpretation freezes the power control to the same setting for all repeated instances of one TB (one slot aggregate), while the 2nd interpretation includes subsequent TPC commands during the transmission of the slot aggregate, if received in time to be able to be accounted for. However, when reading the hierarchy, it should be clear that all the steps correspond to the transmission occasion i.

Proposal 1: Adopt interpretation 1: KPUSCH(i) is defined as the number of OFDM symbols after a last symbol of a corresponding PDCCH reception and before a first symbol of the PUSCH transmission occasion i.

Proposal 2: Agree to an accompanying CR from Rel-15 or Rel-16 onwards to the TS 38.213 in [3].

# References

- 3GPP TS 38.213	Physical layer procedures for control, V15.14.0

- R1-2200790	[107bis-e-NR-R17-CovEnh-03] Summary of email discussion on joint channel estimation for PUSCH, moderator (China Telecom)

- R1-2201031 38.213CRdraft (Rel15, F) Correction of K_PUSCH(i) in TPC accumulation, Nokia, Nokia Shanghai Bell