3GPP TSG-RAN WG1 Meeting #108-e		          R1-2201048

E-Meeting, February 21st – March 3rd, 2022

Title:	[Draft] LS reply on PDCCH skipping

Response to:	R1-2200884 (R2-2201960)

Release:	Rel-17

Work Item:	NR_UE_pow_sav_enh-Core

Source:	RAN1[vivo]

To:	RAN2

Contact Person:

Name:	Xiaodong SHEN

E-mail Address:	shenxiaodong@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	No

1. Overall Description:

RAN2 has discussed the impact of PDCCH skipping on SR and RA procedure. Following agreements were made in RAN2#116bis-e:

- UE ignores PDCCH skipping while the SR is pending.

- If PDCCH skipping is applied to RNTI(s) monitored during RAR/MsgB window, the UE ignores PDCCH skipping during the RAR/MsgB window.

- UE ignores PDCCH skipping while contention resolution timer is running.

RAN1 would like to thank RAN2 for sending their agreements and questions in LS R1-2200884 (R2-2201960) on PDCCH skipping.

Please find RAN1 answers on the questions below.

Question 1: RAN2 would like to know whether PDCCH skipping is applied to RNTI(s) monitored during RAR/MsgB window for RAR/MsgB reception.

Answer1: RAN1 has agreed that PDCCH based monitoring adaptation is for PDCCH USS and type-3 CSS. RAN 1 has not agreed to skip PDCCH monitoring for type 0/0A/1/2 CSS so far. According to this, during RAR/MsgB window for RAR/MsgB reception, PDCCH skipping applies to PDCCH monitoring for USS (including C-RNTI, MCS-C-RNTI, SP-CSI-RNTI, CS-RNTI(s)) and type-3 CSS (including INT-RNTI, SFI-RNTI, TPC-PUSCH-RNTI, TPC-PUCCH-RNTI, TPC-SRS-RNTI, CI-RNTI and, only for the primary cell, C-RNTI, MCS-C-RNTI, CS-RNTI(s), [PS-RNTI, G-RNTI, or G-CS-RNTI]), and does not apply to other search space type or RNTI(s)

Question 2: RAN2 would like to know if RAN1 prefers to capture the above agreements in RAN1 specification or prefer these to be captured in MAC specification.

Answer2: RAN1 has not agreed on any indication to higher layer of the PDCCH skipping or SSSG switching state and RAN1 thinks handling it in physical layer is feasible. Hence, capturing the above agreement in RAN1 specification is preferred.

RAN2 also discussed the impact of PDCCH skipping on DCP monitoring. If DCP monitoring occasions overlap with PDCCH skipping duration, UE cannot monitor DCP. To address this, RAN2 has discussed following approaches: Approach 1: If UE can not monitor DCP due to PDCCH skipping, physical layer of UE does not report Wake-up indication bit to higher layer. Higher layer reuses the ps-Wakeup to determine whether to start the drx-onDurationTimer or not. Approach 2: If UE can not monitor DCP due to PDCCH skipping, physical layer of UE reports a value of 1 for Wake-up indication bit to higher layer. Approach 3: PDCCH skipping only applies in Active Time and hence DCP cannot be missed due to PDCCH skipping. Question 3: RAN2 would like to ask RAN1 whether a) Physical layer of UE reports a value of 1 for Wake-up indication bit to higher layer or b) Physical layer of UE does not report Wake-up indication bit to higher layer, in case UE cannot monitor DCP due to PDCCH skipping; or if Approach 3 can be assumed.

Answer 3: Current TS38.213 section 10.3 provides description of handling invalid WUS monitoring as follows,

If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE

-	is not required to monitor PDCCH for detection of DCI format 2_6, as described in Clauses 10, 11.1, 12, and in Clause 5.7 of [11, TS 38.321] for all corresponding PDCCH monitoring occasions outside Active Time prior to a next long DRX cycle, or

-	does not have any PDCCH monitoring occasions for detection of DCI format 2_6 outside Active Time of a next long DRX cycle

the physical layer of the UE reports a value of 1 for the Wake-up indication bit to higher layers for the next long DRX cycle.

PDCCH skipping is described in section 10. So when UE is not required to monitor PDCCH for detection of DCI format 2_6, as described in Clauses 10, 11.1, 12, the current specification defines the UE behavior is Approach 2. No additional specification impact is needed.

RAN2 also discussed potential impact on CSI/SRS transmissions due to PDCCH skipping. Question 4: RAN2 would like to know whether UE should continue transmitting CSI/SRS during the PDCCH skipping duration or not?

Answer 4: RAN1 has not agreed on any UE behavior to skip CSI/SRS transmission during PDCCH skipping. Hence, transmitting CSI/SRS is not impacted by PDCCH skipping.

2. Actions:

To RAN2

ACTION: 	RAN1 respectfully ask RAN2 to take above information into account.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #109-e    16 – 27 May 2022                    E-Meeting

TSG-RAN WG1 Meeting #110      22 – 26 Aug 2022                    Toulouse, FR