3GPP TSG RAN WG1 #108-e		R1-2201058

e-Meeting, February 21st – March 3rd, 2022

Title:		         Draft reply LS on the L1 aspects of small data transmission

Response to:	R2-2201828 (R1-2200881)

Release:	Rel-17

Work Item:	NR_SmallData_INACTIVE-Core, NR_redcap

Source:	vivo [To be RAN1]

To:	RAN2

Cc:

Contact person:	Zhipeng Lin

E-mail Address:	zhipeng.lin@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN1 thanks RAN2 for the reply LS in R2-2201828, where following question is asked:

=> RAN2 confirms that SDT will be configured only on initial BWP and there is no L1 ACK feedback for CG-SDT. For the above agreement, RAN2 would like RAN1 to confirm that the separate BWP in case of REDCAP can still be considered as the initial BWP and SDT resources can hence be configured on this BWP for REDCAP UEs.

RAN1 has discussed this question and confirmed that SDT resources can be configured on the separate initial UL BWP for REDCAP UEs.

Specifically, following aspects are concluded in RAN1:

- To support RA-SDT for RedCap UEs in the separate initial UL BWP, separate PRACH resources for SDT should be configured in the separate initial UL BWP and RedCap UEs shall use only the separate initial UL BWP to perform RA-SDT.

- To support CG-SDT for RedCap UEs in the separate initial UL BWP,

  - CG PUSCH resources are configured in the separate initial UL BWP. The CD-SSBs in initial DL BWP are used for TA validation and for mapping to the CG PUSCH resources.

  - In case a separate initial DL BWP is configured for RedCap UEs, the UE-specific search space sdt-CG-SearchSpace configured in RRC release message for SDT should have associated CORESET being configured in this separate initial DL BWP.

# 2	Actions

To RAN2

ACTION: 	RAN1 respectfully asks RAN2 to take the above information into account for their specification work.

# 3	Dates of next RAN1 meetings

TSG RAN1 Meeting #109-e	16th - 27th May 2022		e-Meeting

TSG RAN1 Meeting #110		22nd - 26th Aug. 2022 		Toulouse, FR