3GPP TSG RAN WG1 #108-e		R1-2201059

e-Meeting, February 21st – March 3rd, 2022

Source:	vivo

Title:	Discussion on RAN4 LS on CORESET#0 impact of CBW narrower than 40MHz of n79

Agenda Item:	5

Document for:	Discussion and Decision

- Introduction

In [1], RAN4 asked RAN1 to solve the non-backward compatibility issue on determination of the parameters for PDCCH monitoring in type-0 CSS caused by narrower CBW introduced for band n79. In this contribution, we provide our views on this issue.

- Discussion

In [1], RAN4 also provide four alternatives to solve this NBC issue, as follows.

RAN4 has come up with four potential solutions as follows: - Alt-1: Add narrower channel bandwidth to n79, and indicate different table to legacy UE and new UE for CORESET#0 configuration. - Alt-1a: Add narrower channel bandwidth to n79, and indicate the same table 13.6 to legacy UE and new UE for CORESET#0 configuration. - Rationale: Legacy UE not supporting channel bandwidth lower than 40MHz will always look at table 13.6, this table shall then be the common one. - Alt-1b: Add narrower channel bandwidth to n79, and indicate: - A UE synchronizing on a GSCN with step 4 shall look at the table 13.6. - Rationale: This enables new and legacy to connect in band n79 using the same CORESET#0 configuration. - A UE synchronizing on a GSCN with step 1 but not step 4 shall look at the table 13.4. - Rationale: Table 3.4 offers more flexibility on CORESET#0 configuration - Alt-2: Add narrower channel bandwidth to new band nX instead of n79.

RAN4 also provided analysis on pros/cons and spec impacts for these four alternatives, as follows.

|  | Pros | Cons | Specification impact |
| --- | --- | --- | --- |
| Alternative 1 | - Limited specification effort (Rel-17) | - Only one CORESET#0 configuration can be used for n79 | - Add clarification on restriction of CORESET#0 configuration in RAN1 |
| Alternative 1a | - Limited specification (Rel-17) - More configuration flexibility comparing to Alt.1 | - Lose some configuration flexibility compared with 10MHz table. | - Add clarification on CORESET #0 configuration table determination in RAN1 |
| Alternative 1b | - Limited specification (Rel-17) - More configuration flexibility comparing to Alt.1a for the new Rel-17 UEs | - UE should check on which GSCN it has synchronized | - Add clarification on CORESET #0 configuration table determination in RAN1 |
| Alternative 2 | - No impact on RAN1 | - New band shall be introduced (Rel-18) - Potential increase in UE complexity to support both n79 and nX | - Previous RAN4 agreements on n79 is reverted - Add new band in RAN4 |

In these four alternatives, Alt-1 would put too much restrictions on NW deployment for CORESET#0 configuration due to only one common entry across table 13-4 and table 13-6 is available. Hence, Alt-1 is not preferred.

Alt-1a can achieve more configuration flexibility compared to Alt-1, although not as flexible as Alt-1b and Alt-2. To support Alt-1a, RAN1 spec needs to capture band n79 as an exceptional case in determination of which table to look up. However, frequency band number is transparent in RAN1 spec, and RAN1 spec never had any UE behavior for a specific numbered frequency band.

Observation 1: To support Alt-1a, RAN1 spec needs to capture band n79 as an exceptional case in determination of which table to look up. However, frequency band number is transparent in RAN1 spec, and RAN1 spec never had any UE behavior for a specific numbered frequency band.

Alt-1b have more RAN1 spec impacts compared to Alt-1a, the table to look up is not only determined based on a specific numbered frequency band, but also need to consider some specific GSCNs. Similarly, GSCN is transparent to RAN1 spec, and RAN1 spec never had any UE behavior for specific GSCNs either. Hence, both Alt-1a and Alt-1b are not preferred from RAN1 perspective.

Observation 2: To support Alt-1b, which table to look up is not only determined based on frequency band number, but also based on GSCN. However, GSCN is also transparent in RAN1 spec, and RAN1 spec never had any GSCN specific UE behavior either.

Alt-2 has no RAN1 spec impacts. Table 13-4 can be used for CORESET#0 in band n79 with narrower CBW, and high NW deployment flexibility can be achieved. Hence, Alt-2 is preferred from RAN1 perspective.

Proposal 1: Alt-2, which has no RAN1 spec impacts and better NW deployment flexibility, is preferred from RAN1 perspective.

Proposal 2: Inform the following conclusions in reply LS to RAN4

- Frequency band number and GSCN are transparent in RAN1 spec, and RAN1 spec never had any UE behavior for a specific numbered frequency band or for specific GSCNs.

- Alt-2, add narrower channel bandwidth to new band nX instead of n79, should be supported.

- Conclusion

In this contribution, we discussed the issue raised by RAN4 in [1], and we have the following observations and proposals.

Observation 1: To support Alt-1a, RAN1 spec needs to capture band n79 as an exceptional case in determination of which table to look up. However, frequency band number is transparent in RAN1 spec, and RAN1 spec never had any UE behavior for a specific numbered frequency band.

Observation 2: To support Alt-1b, which table to look up is not only determined based on frequency band number, but also based on GSCN. However, GSCN is also transparent in RAN1 spec, and RAN1 spec never had any GSCN specific UE behavior either.

Proposal 1: Alt-2, which has no RAN1 spec impacts and better NW deployment flexibility, is preferred from RAN1 perspective.

Proposal 2: Inform the following conclusions in reply LS to RAN4

- Frequency band number and GSCN are transparent in RAN1 spec, and RAN1 spec never had any UE behavior for a specific numbered frequency band or for specific GSCNs.

- Alt-2, add narrower channel bandwidth to new band nX instead of n79, should be supported.

- References

- R1-2200907(R4-2202286) LS on CORESET#0 impact of CBW narrower than 40MHz of n79.