3GPP TSG-RAN WG1 Meeting #108-e	R1-2201073

# e-Meeting, February 21st - March 3rd

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev |  | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on UE procedure for receiving HARQ-ACK on sidelink |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2022-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - In Section 16.3.1 in 38.213, the generation of HARQ-ACK from PHY layer at a TX UE side in the following cases are specified: - Groupcast option1: when a TX UE receives or does not receive a PSFCH on a PSFCH reception occasion - Groupcast option2/unicast: when a TX UE receives a PSFCH on a PSFCH reception occasion (note: how to determine the acknowledgment by MAC for the case where PSFCH is not detected for groupcast option2 or unicast is captured in section 5.22.1.3.2 in 38.321; NACK is delivered based on the following text: 1> if an acknowledgement corresponding to the PSSCH transmission in clause 5.22.1.3.1a is obtained from the physical layer: 2> deliver the acknowledgement to the corresponding Sidelink HARQ entity for the Sidelink process; 1> else: 2> deliver a negative acknowledgement to the corresponding Sidelink HARQ entity for the Sidelink process;’) However, the first sentence ‘if UE receives a PSFCH’ for groupcast option1 in 16.3.1 is contradictory with the sub-bullet ‘if the UE determines absence of PSFCH reception’, thus should be corrected. Agreements in RAN1#102: - Prepare a TP to capture UE behavior that physical layer reports HARQ-ACK information of the received PSFCH to higher layer. Detailed wording will be discussed during the TP phase with the following principle: - Groupcast option 1: UE reports ACK when the UE determines absence of PSFCH reception for the PSFCH reception occasion; otherwise, reports NACK. - Groupcast option 2: UE reports ACK if the UE determines ACK in PSFCH resources corresponding to every identity M_ID of the UEs that the UE expects to receive the PSSCH, as described in Clause 16.3; otherwise reports NACK; - Unicast: UE reports HARQ-ACK information with same value as a value of HARQ-ACK information the UE determines from the PSFCH reception - Discuss during the TP phase whether or not a clarification is necessary to separately handle the potential collision case if any - In section 16.1, an incorrect RRC parameter name TDD-UL-ConfigurationCommon is used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Modify the main bullet ‘if UE receives a PSFCH’ to ‘for a PSFCH reception occasion’ for groupcast option1 to include the case of no detected PSFCH. - Correct the RRC parameter name to tdd-UL-DL-ConfigurationCommon |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect description for HARQ-ACK generation in the case of groupcast option1. Misaligned RRC parameter names between 38.331 and 38.213. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 16.1, 16.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: This CR aligns with RAN1 common understanding. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 16.1	Synchronization procedures

======omitted======

-	a5, a6, a7,a8, a9, a10, a11 are the 7th to 1st LSBs of uslotsSL, respectively

-	for a0=0, uslotsSL=uslots*2μ-μref+usym*2μ-μrefL+I1

-	for a0=1, uslotsSL=uslots,2*2μ-μref+usym,2*2μ-μrefL+I2w*P*2μ+1w+uslots*2μ-μref+usym*2μ-μrefL+I1w

where

-	L is the number of symbols in a slot: L=12 if cyclicPrefix = "ECP"; else, L=14

-	I1 is 1 if usym*2μ-μref mod L≥ L-Y, else I1 is 0

-	I2 is 1 if usym,2*2μ-μref mod L≥L-Y, else I2 is 0

-	Y is the sidelink starting symbol index provided by sl-StartSymbol

-	w is the granularity of slots indication as described in Table 16.1-2

-	μref, uslots, usym, uslots,2, usym,2 are the parameters of tdd-UL-DL-ConfigurationCommon as described in clause 11.1, or the parameters of sl-TDD-Configuration as defined in [9.3, TS 38.331]

-	μ=0, 1, 2, 3 corresponds to SL SCS as defined in [4, TS 38.211]

======omitted======

### 16.3.1	UE procedure for receiving HARQ-ACK on sidelink

A UE that transmitted a PSSCH scheduled by a SCI format 2-A or a SCI format 2-B that indicates HARQ feedback enabled, attempts to receive associated PSFCHs according to PSFCH resources determined as described in clause 16.3. The UE determines an ACK or a NACK value for HARQ-ACK information provided in each PSFCH resource as described in [8-4, TS 38.101-4]. The UE does not determine both an ACK value and a NACK value at a same time for a PSFCH resource.

For each PSFCH reception occasion, from a number of PSFCH reception occasions, the UE generates HARQ-ACK information to report to higher layers. For generating the HARQ-ACK information, the UE can be indicated by a SCI format to perform one of the following

-	if the UE receives a PSFCH associated with a SCI format 2-A with Cast type indicator field value of "10"

-	report to higher layers HARQ-ACK information with same value as a value of HARQ-ACK information that the UE determines from the PSFCH reception

-	if the UE receives a PSFCH associated with a SCI format 2-A with Cast type indicator field value of "01"

-	report an ACK value to higher layers if the UE determines an ACK value from at least one PSFCH reception occasion from the number of PSFCH reception occasions in PSFCH resources corresponding to every identity MID of UEs that the UE expects to receive corresponding PSSCHs as described in clause 16.3; otherwise, report a NACK value to higher layers

-	for a PSFCH reception occasion associated with a SCI format 2-B or a SCI format 2-A with Cast type indicator field value of "11"

-	report to higher layers an ACK value if the UE determines absence of PSFCH reception for the PSFCH reception occasion; otherwise, report a NACK value to higher layers