3GPP TSG RAN WG1 #108-e                                                           R1-2201078

e-Meeting, February 21th – March 3rd, 2022

Source:	vivo

Title:	Maintenance on multi beam enhancement

Agenda Item:	8.1.1

Document for:	Discussion and Decision

Introduction

The 3GPP Rel-17 NR MIMO WID includes enhancement on multi-beam operation. In RAN1 #102-e ~ #107-e meetings, some issues were discussed, including unified TCI framework, L1/L2-centric inter-cell beam management, MPUE and MPE, etc. [1~8] In this contribution, we share our views on the remaining issues for Rel-17 NR FeMIMO on multi-beam operations.

Unified TCI framework

TCI state for non-UE-dedicated channel

Agreement On Rel.17 unified TCI framework, for intra-cell beam indication, the following DL RSs can share the same indicated Rel-17 TCI state as UE-dedicated reception on PDSCH and for UE-dedicated reception on all or subset of CORESETs in a CC: - DMRS(s) associated with non-UE-dedicated reception on CORESET(s) and the associated PDSCH - FFS (to be concluded in RAN1#106bis-e): Non-UE-dedicated PUCCH and non-UE-dedicated PUSCH On Rel.17 beam indication enhancements for inter-cell beam management, the supported Rel-17 MAC-CE-based and/or DCI-based beam indication (at least using DCI formats 1_1/1_2 with and without DL assignment including the associated MAC-CE-based TCI state activation) applies to: - The channels and signals as for intra-cell beam management except for non-UE dedicated channels/signals Agreement On Rel.17 unified TCI framework, for Rel-17 unified TCI, for DL or UL channels/signals that can share the same indicated Rel-17 TCI state as UE-dedicated reception on PDSCH/PDCCH or dynamic-grant/configured-grant based PUSCH, all of dedicated PUCCH resources (via Rel-17 MAC-CE/DCI TCI state update): - For DL: A non-UE dedicated PDCCH/PDSCH associated with the serving cell PCI or AP CSI-RS for BM or CSI (per previous agreements) sharing the same indicated Rel-17 TCI state as UE-dedicated reception on PDSCH/PDCCH (via Rel-17 MAC-CE/DCI TCI state update) is configured via RRC. - For UL: An SRS for BM, for antenna switching, or for codebook/non-codebook based uplink transmission (per previous agreements) sharing the same indicated Rel-17 TCI state as dynamic-grant/configured-grant based PUSCH, all of dedicated PUCCH resources (via Rel-17 MAC-CE/DCI TCI state update) is configured via RRC. Note: The details of this RRC configuration (e.g. whether via a new RRC parameter or other means) is up to RAN2. This does not imply that a new RRC parameter(s) is necessary from RAN1 point of view. FFS: Relevant UE capability to be discussed under UE feature agenda item. Agreement For Rel-17 unified TCI framework, on applying the indicated Rel-17 TCI state to PDCCH reception and the respective PDSCH reception: - For discussion purposes, define as follows: - ‘CORESET A’: A CORESET other than CORESET#0 associated with only UE-dedicated reception on PDCCH in a CC, comprising CORESETs in association with: - [USS and/or CSS Type 3] - ‘CORESET B’: A CORESET other than CORESET#0 associated with only non-UE-dedicated reception on PDCCH in a CC, comprising CORESETs in association with: - [CSS or CSS other than Type 3] - ‘CORESET C’: A CORESET other than CORESET#0 associated with both UE-dedicated and non-UE-dedicated reception on PDCCH in a CC - CORESET#0 - For Rel-17 TCI state indication, support per CORESET determination as follows: - For any PDCCH reception on a ‘CORESET A’ and the respective PDSCH reception, UE always applies the indicated Rel-17 TCI state. - For any PDCCH reception on a ‘CORESET B’ and the respective PDSCH reception, whether or not UE to apply the indicated Rel-17 TCI state associated with the serving cell is determined per CORESET by RRC - FFS: For intra-cell BM, whether CORESET C is supported or not - If CORESET C is supported, the TCI state of CORESET C - FFS: For inter-cell BM, whether CORESET C is supported or not - If CORESET C is supported, the TCI state of CORESET C - FFS: The TCI state of CORESET 0

In RAN1 #107-e meeting, for discussion purposes four types of CORESET are defined as CORESET A, CORESET B, CORESET C and CORESET#0. In our understanding, non-UE-dedicated channels include the PDCCH on the   CORESETs associated with type 0/0A/1/2 CSS set and the scheduled PDSCH by the PDCCH, where the non-UE-dedicated CORESET is mainly used to transmit common information, such as SIB, MIB and paging information. For CSS Type 3, the similar rule of TCI state determination for USS can be reused. The CORESET A refers the CORESET in association with only USS, only CSS Type3, or USS+CSS Type3, the CORESET B refers the CORESET in association with CSS other than Type3.

- The non-UE-dedicated channels include the PDCCH on the CORESETs associated with type 0/0A/1/2 CSS set and the scheduled PDSCH by the PDCCH.

- Support the following definition for CORESET A and CORESET B

  - ‘CORESET A’: A CORESET other than CORESET#0 associated with only UE-dedicated reception on PDCCH in a CC, comprising CORESETs in association with:

    - [USS and/or CSS Type 3]

  - ‘CORESET B’:  A CORESET other than CORESET#0 associated with only non-UE-dedicated reception on PDCCH in a CC, comprising CORESETs in association with:

    - [CSS or CSS other than Type 3]

In RAN1 #106-e meeting, it is already agreed that for intra-cell beam indication the same indicated Rel-17 TCI state as UE-dedicated reception on PDSCH and for UE-dedicated reception on all or subset of CORESETs in a CC can be applied to the DMRS(s) associated with non-UE-dedicated reception on CORESET(s) and the associated PDSCH, where the application of same indicated Rel-17 TCI state is configured via RRC. For beam indication of inter-cell beam management, in the RAN1#106-e meeting it had been agreed that the TCI states associated with a PCI different from the serving cell can only be applied for UE-dedicated channels/RSs, and the QCL assumption of other channels/RSs, e.g. non-UE-dedicated channels/RSs, can be configured and updated by the legacy (Rel-15/16) TCI state update signaling/configuration mechanism.

From the agreements of RAN1 #106-e meeting it can be seen that for TCI state determination for PDCCH reception, there are three cases to be considered, i.e. UE-dedicated reception PDCCH, non-UE-dedicated reception on PDCCH for intra-cell, non-UE-dedicated reception on PDCCH for inter-cell. The first PDCCH above are always received using the indicated Rel-17 TCI state, the second one can be configured by RRC to using the indicated Rel-17 TCI state, but last one associated with serving cell PCI is received using different TCI state from that of UE-dedicated channel.

Based on the analysis above, the application of same indicated Rel-17 TCI state to a CORESET B is configured via RRC only in case of intra-cell beam indication, but for inter-cell beam indication the indicated Rel-17 TCI state associated with a PCI different from the serving cell is never applied for a CORESET B, because the CORESET B is associated with CSS, i.e. a CORESET B is non-UE-dedicated CORESET.

For a CORESET C associated with USS and CSS other than Type3, i.e. with both UE-dedicated and non-UE-dedicated reception on PDCCH, the same solution with CORESET B can be reused. There is no much difference between CORESET B and CORESET C for the CSS reception part. For the USS part, with RRC signaling it provides the most flexibility. For intra-cell beam indication, for any PDCCH reception on a CORESET C and the respective PDSCH reception, whether or not UE to apply the indicated Rel-17 TCI state associated with the serving cell is determined per CORESET by RRC. Due to the CORESET C comprising the non-UE-dedicated reception on a PDCCH, for inter-cell beam indication, the TCI state associated with a PCI different from the serving cell is not applied to CORESET C.

For CORESET#0, it can also be considered that CORESET#0 is similar as CORESET C and can follow the same rule as CORESET B. CORESET#0 and related scheduling and feedback is more sensitive to robustness of the beam, blockage will cause frequent beam pair link interruption. On the other hand, if CORESET#0 uses a wide beam, it can maintain the basic RRC/MAC CE connection and small packet scheduling in case of blockage. Thus, if the indicated Rel-17 TCI state is not applied to CORESET#0, the legacy Rel-15/16 signaling is used for beam indication of CORESET#0 and the respective channels which include the PUSCH/PDSCH scheduled/activated and PUCCH transmission triggered by CORESET#0 to increase the robustness of beam link.

- Rel-17 TCI state indication of CORESET C (if supported) and CORESET#0 follows the same rule as CORESET B, i.e.

- for intra-cell BM, for any PDCCH reception on a CORESET C or CORESET#0 and the respective PDSCH reception, whether or not UE to apply the indicated Rel-17 TCI sate associated with the serving cell is determined per CORESET by RRC.

- for inter-cell BM, for any PDCCH reception on a CORESET C or CORESET#0 and the respective PDSCH reception, the indicated Rel-17 TCI state associated with a PCI different from the serving cell is never applied.

- if the indicated Rel-17 TCI state is not applied to CORESET#0, the legacy Rel-15/16 signaling is reused forCORESET#0.

There is no clear definition of ‘non-UE-dedicated PUCCH’ and ‘non-UE-dedicated PUSCH’ in spec. In general, before RRC connected mode, so-called ‘non-UE-dedicated PUCCH’ is provided by pucch-ResourceCommon for transmission of HARQ-ACK information in an initial UL BWP when a UE does not have dedicated PUCCH resource configuration. For PUSCH transmission scheduled by a RAR UL grant in random access procedure, the UE uses the same RS resource index as for a corresponding PRACH transmission, which is also applied for ‘non-UE-dedicated PUCCH’. So the indicated Rel-17 TCI state does not need to be applied to ‘non-UE-dedicated PUCCH’ and ‘non-UE-dedicated PUSCH’.

In section 5.1.5 of TS 38.214, the Rel-17 TCI state is indicated to provide a reference, if applicable, for determining UL TX spatial filter for dynamic-grant and configured-grant based PUSCH and PUCCH resource in a CC, and SRS. The PUCCH and PUSCH associated with CORESET#0 can be considered as an exception due to the requirement of robustness of transmission and basic connection for CORESET#0 and related feedback information. Similar to the respective PDSCH, we prefer that the UL channels associated with CORESET#0 should also apply the same Rel-17 TCI state with CORESET#0.

- The PUSCH scheduled/activated and PUCCH triggered by CORESET#0 follows the beam indicated/activated for CORESET #0.

Unified TCI configuration

Working Assumption The UE is not expected to be configured with Rel-15/Rel-16 TCI/SpatialRelationInfo if the UE is configured with Rel-17 TCI in any CC in a band - The CC list for Rel-16 multi-CC beam indication should not contain any CC configured with Rel-17 TCI

According to the working assumption of RAN1 #107-e meeting, the Rel-17 unified TCI and Rel-15/16 TCI/SpatialRelationInfo cannot be configured simultaneously. Band level restriction is a good compromise to reduce UE complexity. Although the CC level restriction is very flexible, for example, Rel-17 unified TCI is configured for a CC and multi-TRP enhancement in Rel-17 based on Rel-15/16 TCI/SpatialRelationInfo is configured for another CC in same band, but the configuration overhead and UE implementation for CC level restriction are increased.

- Confirm the following working assumption:

- The UE is not expected to be configured with Rel-15/Rel-16 TCI/SpatialRelationInfo if the UE is configured with Rel-17 TCI in any CC in a band

  - The CC list for Rel-16 multi-CC beam indication should not contain any CC configured with Rel-17 TCI

Unified TCI for RS

Agreement On Rel-17 unified TCI framework, any SRS resource or resource set that is a valid target signal of a Rel-15/16 spatial relation based on the Rel-15/16 spatial relation rules (on source-target relations) can be configured as a target signal of a Rel-17 UL or, if applicable, joint TCI (hence the Rel-17 UL or, if applicable, joint TCI state pool). - This feature does not require a new type of source RS on top of the ones supported for UL TCI and joint DL /UL TCI - Note: This doesn’t preclude the content of pending proposal 1.E (on CSI -RS for CSI as QCL Type-A/D source RS) to be agreed in the future - Note: This does not imply that DL and UL TCI state pools are separate or shared for separate DL /UL TCI (this issue is up to RAN2) - Note: A Rel-17 UE is not required to support both this feature and optional Rel-16 features of SRS spatial relation info within a same band

In RAN1 #107-e meeting, any SRS resource or resource set that is a valid target signal of a Rel-15/16 spatial relation can be configured as a target signal of a Rel-17 unified TCI. There are different optional solutions for UL PC parameters of SRS not sharing the same indicated Rel-17 TCI state. For example, the legacy Rel-15/16 PC parameters configuration configuration/activation signaling for SRS not sharing the same indicated Rel-17 TCI state or for SRS regardless of whether the SRS uses the same indicated Rel-17 TCI state.

For the former, two PC configurations, i.e. Rel-17 PC parameters set associated with TCI state and legacy PC parameters for SRS resource set, need to be configured to increase the UE memory overhead. For the latter, the gNB cannot configure the PC parameters associated with Rel-17 TCI state for SRS, but only reuse legacy signaling to configure and activate the PC parameters for a SRS resource set. Moreover, the benefits of reusing legacy PC configuration/activation signaling also include increase the limitation for Rel-17 TCI state, e.g. the PC parameters for TCI states should be same for all SRS resources in a set, in particular for the UE only supporting beam alignment.

- For any SRS resource or resource set that does not share the same indicated Rel-17 TCI state but can be configured as a target signal of Rel-17 TCI, the Rel-15/16 principle for SRS UL PC parameter setting configuration/activation is used.

The application of same indicated Rel-17 TCI state for aperiodic CSI-RS and aperiodic SRS can be flexibly configured by gNB per resource set. For aperiodic CSI-RS, when the triggering offset is smaller than the threshold beamSwitchTiming, the legacy rule for default QCL assumption of aperiodic CSI-RS can be reused, i.e. the QCL assumption of aperiodic CSI-RS follows the QCL assumption of PDSCH with indicated beam, P-/SP-CSI-RS, aperiodic CSI-RS with indicated beam, or lowest CORESET. If there are no other DL signal and CORESET, the joint TCI state or separate DL TCI state corresponding to the lowest TCI codepoint is used.

In default QCL rule mentioned above, the same indicated Rel-17 TCI state as UE-dedicated reception on PDSCH and for UE-dedicated reception on all or subset of CORESETs in a CC can be applied for aperiodic CSI-RS for BM and for CSI when the triggering offset is smaller than the threshold beamSwitchTiming except for the case that the other DL signal in the same symbols at the aperiodic CSI-RS is CSI-RS or non-UE-dedicated reception on PDCCH and the associated PDSCH which can be configured to not share same TCI state as UE-dedicated reception. The QCL assumption of aperiodic CSI-RS follows the QCL assumption of these other DL signals above like legacy default beam rule, where the QCL assumption may be different from the that of UE-dedicated reception.

- For the following other DL signals in the same symbols at the aperiodic CSI-RS, aperiodic CSI-RS for BM and for CSI does not share the same indicated Rel-17 TCI state as UE-dedicated reception on PDSCH and for UE-dedicated reception on all or subset of CORESETs in a CC, except

  - P-/SP-CSI-RS

  - Non-UE-dedicated reception on PDCCH and the associated PDSCH or aperiodic CSI-RS, which is configured to not share same TCI state as UE-dedicated reception.

In RAN1#106bis-e, it was agreed that an SRS for BM, for antenna switching, or for codebook/non-codebook based uplink transmission sharing the same indicated Rel-17 TCI state as dynamic-grant/configured-grant based PUSCH, all of dedicated PUCCH resources is configured via RRC. If an SRS is configured as a valid target signal of a Rel-17 UL or, if applicable, joint TCI state, Rel-17 mechanism(s) which reuse the Rel-15/16 TCI state update signaling/configuration design(s) are used to update/configure such SRS with Rel-17 TCI state(s), where SRS that does not share the same indicated Rel-17 TCI state as dynamic-grant/configured-grant based PUSCH, all of dedicated PUCCH resources in a CC.

In order to quickly update the beam indication for periodic SRS that is configured as valid target signal of Rel-17 TCI state but not to share the same TCI state as dynamic-grant/configured-grant based PUSCH, all or subset of dedicated PUCCH resources, MAC CE can be used to update QCL configuration for periodic SRS.

- MAC CE can be used to update QCL configuration for periodic SRS that is valid target signal of Rel-17 TCI state but not to share the same Rel-17 TCI state as dynamic-grant/configured-grant based PUSCH, all or subset of dedicated PUCCH resources.

This method can also be extended to periodic CSI-RS.  For example, for periodic CSI-RS that is configured not to share the same indicated Rel-17 TCI state as UE-dedicated reception, MAC CE also can be used to update QCL configuration containing QCL RS source(s) and QCL type(s).

In Rel-15/16, for each aperiodic CSI-RS resource in a CSI-RS resource set associated with each CSI triggering state, the UE is indicated the quasi co-location configuration of quasi co-location RS source(s) and quasi co-location type(s) through higher layer signaling of qcl-info which contains a list of references to TCI-State's for the aperiodic CSI-RS resources associated with the CSI triggering state. In Rel-17 unified TCI, if aperiodic CSI-RS is configured to follow the indicated TCI state as UE reception, UE ignores qcl-info configured for the NZP-CSI-RS. But if aperiodic CSI-RS is configured to not follow the indicated TCI state, to dynamically update TCI state of aperiodic CSI-RS, similar to SP-CSI-RS resource set activation/deactivation, a MAC CE can be used to update qcl-info for NZP-CSI-RS resources in higher layer parameter CSI-AperiodicTriggerState, where the usage of CSI-RS may be for beam management or CSI acquisition.

- For periodic CSI-RS and aperiodic CSI-RS that is valid target signal of Rel-17 TCI state but not to share same TCI state as UE-dedicated reception on PDSCH and for UE-dedicated reception on all or subset of CORESETs in a CC, MAC CE can be used to update QCL assumption for the CSI-RS.

Power control parameters

Agreement On the setting of UL PC parameters except for PL-RS (P0, alpha, closed loop index) for Rel.17 unified TCI framework, the setting of (P0, alpha, closed loop index) for SRS can also be associated with UL or (if applicable) joint TCI state. - If not associated, the setting(s) of (P0, alpha, closed loop index) for SRS per BWP is independent of the UL or (if applicable) joint TCI states - This is only applicable for SRS sets using Rel-17 TCI state to determine their spatial relation. - FFS: Whether more than one parameter sets can be configured, e.g. for different traffics Agreement On Rel.17 unified TCI framework, for the case when the settings of (P0, alpha, closed loop index) for PUSCH, PUCCH, and/or SRS are associated with UL or (if applicable) joint TCI states per BWP, for each of the PUSCH, PUCCH, and/or SRS, one individual setting is optionally associated with each of the UL or (if applicable) joint TCI states in a BWP via RRC - FFS: MAC-CE based update for the closed loop index associated with UL or (if applicable) joint TCI state - Above is applicable for eMBB - FFS: Details on power control setting for URLLC

According to the agreements of RAN1 #106bis-e meeting, for Rel-17 UL PC parameter setting, gNB can configure the association between multiple settings and the UL or (if applicable) joint TCI state by RRC, where one individual setting associated with each of the PUSCH, PUCCH and/or SRS. The settings do not need to be dynamically associated with TCI state. The setting is used when the associated UL or (if applicable) joint TCI state is activated by MAC CE.

It is not necessary to update the closed loop index associated with UL or (if applicable) joint TCI state based on MAC CE. In Rel-15/Rel-16, the spatial relation and PC parameters of PUSCH is determined by SRI without expanding the number of closed loop index and without dynamic update of closed loop index for the SRI-PUSCH-PowerControl that is mapped by a SRI for adequate performance. Even in case of mTRP, when gNB dynamically schedules PUSCH transmission to single TRP or multiple TRPs, for power control of PUSCH two close loop indexes are for two TRPs without any update on the closed loop index. Thus, there is no motivation to introduce MAC CE based update for the close loop index associated with UL or joint TCI state.

- Do not support MAC-CE based update for the closed loop index associated with UL or (if applicable) joint TCI state.

For the setting configuration, different setting lists can be configured for PUCCH, PUSCH and SRS, the TCI state ID is associated with different settings from the lists. When gNB indicates a TCI state ID by DCI, the common beam is applied to PUCCH, PUSCH and SRS, but different PC parameter settings associated with this indicated TCI state ID are used for PUCCH, PUSCH and SRS respectively. If the association is not configured by RRC, a default setting or indicated setting from the setting list can be used.

- If the association is not configured by RRC, a default setting from the setting list can be used.

In general, PC parameters required by eMBB and URLLC are different, which can be achieved by SRI field and open-loop power control parameter set indication field in DCI format. For example, additional P0 list is configured, and UE determines P0 or additional P0 based on the value of SRI field and OLPC field for eMBB and URLLC.

In Rel-17 unified TCI framework, the similar scheme can be reused for PC parameter setting determination, where SRI is replaced with TCI state ID to determine PC parameters. For example, a TCI state ID is associated with two settings from different lists for eMBB and URLLC, where the additional setting just includes P0 and other parameters are only configured in one setting. Based on the value of TCI state ID and the open-loop power control parameter set indication field in beam indication DCI, PC parameters from the settings associated with the TCI state ID can be applied for specific service.

- UL PC parameter settings from different setting lists including additional P0 list are associated with an UL or (if applicable) joint TCI state for different services.

  - TCI state ID and OLPC are used to determine PC parameters.

In Rel-16, PC parameters including P0, alpha, and closedloopindex for PUSCH configured in ConfiguredGrantConfig are utilized for both type 1 and type 2 CG. In current Rel-17 spec, PC parameter setting associated with TCI state is used for both DG and CG, which means that transmission power of CG will be comparable with DG. Considering that CG usually carries URLLC data which requires higher transmission power to acquire better performance, it’s better to reuse PC setting configured in ConfiguredGrantConfig. So, we propose to clarify that UL PC parameter setting included or associated in unified TCI state is not applied for CG.

- Clarify that UL PC parameter setting included or associated in unified TCI state is not applied for a PUSCH (re)transmission configured by ConfiguredGrantConfig.

- Support the following TP.

TS 38.213 7.1 Uplink Power control Uplink power control determines a power for PUSCH, PUCCH, SRS, and PRACH transmissions. A UE does not expect to simultaneously maintain more than four pathloss estimates per serving cell for all PUSCH/PUCCH/SRS transmissions as described in clauses 7.1.1, 7.2.1, and 7.3.1, except for SRS transmissions configured by SRS-PosResourceSet as described in clause 7.3.1. If the UE is provided a number of RS resources for pathloss estimation for PUSCH/PUCCH/SRS transmissions that is larger than 4, the UE maintains for pathloss estimation RS resources corresponding to RS resource indexes qd as described in clauses 7.1.1, 7.2.1, and 7.3.1. If an RS resource updated by MAC CE, as described in clauses 7.1.1, 7.2.1 and 7.3.1, is one from the RS resources the UE maintains for pathloss estimation for PUSCH/PUCCH/SRS transmissions, the UE applies the pathloss estimation based on the RS resources starting from the first slot that is after slot k+3∙Nslotsubframe, μ+2μ∙kmac where k is the slot where the UE would transmit a PUCCH or PUSCH with HARQ-ACK information for the PDSCH providing the MAC CE, μ is the SCS configuration for the PUCCH or PUSCH, respectively, that is determined in the slot when the MAC CE command is applied and kmac is a number of slots for SCS configuration μ=0 provided by K-Mac or kmac=0 if K-Mac is not provided. A PUSCH/PUCCH/SRS/PRACH transmission occasion i is defined by a slot index ns,fμ within a frame with system frame number SFN, a first symbol S within the slot, and a number of consecutive symbols L. For a PUSCH transmission with repetition Type B, a PUSCH transmission occasion is a nominal repetition [6, TS 38.214]. In the remaining of this clause, if a UE is provided TCI-State_r17 and for an indicated TCI-State_r17 as described in [6, TS 38.214] - in clauses 7.1.1, 7.2.1, and 7.3.1, the RS index qd for obtaining the downlink pathloss estimate for PUSCH, PUCCH, and SRS transmission is provided by PL-RS associated with or included in the indicated TCI-StateID_r17 - in clause 7.1.1, if p0-Alpha-CLID-PUSCH-Set is provided, the values of PO_UE_PUSCH,b,f,cj, αb,f,cj, and the PUSCH power control adjustment state l are provided by p0-Alpha-CLID-PUSCH-Set associated with the indicated TCI-StateID_r17 except for a PUSCH (re)transmission configured by ConfiguredGrantConfig - in clause 7.2.1, if p0-Alpha-CLID-PUCCHSet is provided, the values of PO_PUCCH,b,f,cqu and the PUCCH power control adjustment state l are provided by p0-Alpha-CLID-PUCCH-Set associated with the indicated TCI-StateID_r17 - in clause 7.3.1, if p0-Alpha-CLID-SRS-Set is provided, the values of PO_SRS,b,f,cqs, αSRS,b,f,cqs, and SRS power control adjustment state l are provided by p0-Alpha-CLID-SRS-Set associated with the indicated TCI-StateID_r17 In the remaining of this clause, if a PDCCH reception by a UE includes two PDCCH candidates from corresponding search space sets, as described in clause 10.1 - a PDCCH monitoring occasion is the union of the PDCCH monitoring occasions for the two PDCCH candidates - the end of the PDCCH reception is the end of the PDCCH candidate that ends later The PDCCH reception includes the two PDCCH candidates also when the UE is not required to monitor one of the two PDCCH candidates as described in clauses 10, 11.1, and 11.1.1.

Application of unified TCI for FR1

For application of unified TCI for FR1 UL channels, it should be clarified whether joint TCI and separate TCI should both be supported:

- if joint TCI mode is configured, the QCL-TypeD RS does not need to be included in IE TCI-State, but the PLRS should be included in or associated with the joint TCI state. To ensure the flexibility of beam indication and power control, various combination of QCL-Type RS other than QCL-TypeD RS and PLRS should be considered.

- If separate TCI mode is configured, no parameters are required to be configured in the UL TCI state, just PLRS needs to be included in but is associated with the UL TCI state ID. Obviously, the RRC signaling overhead for the number of TCI states can be reduced for FR1 UL channels.

- Another issue that needs to be considered is whether UL TCI can be configured with QCL-TypeD RS as optional.

- Clarify whether two TCI modes (joint TCI, separate TCI) are needed for application of unified TCI for FR1 UL channels.

- If separate TCI mode is also supported in FR1, inform RAN2 that QCL-TypeD RS configuration for UL TCI state should be optional.

For FR1, the QCL-TypeD RS is absent in joint or UL TCI state, the PLRS determination rule should be clarified, i.e. other QCL-type RS other than QCL-TypeD RS is used for pathloss estimation for joint TCI, or PLRS is mandatory to be included in or associated with the joint or UL TCI state.

- For FR1 UL channels, clarify the PLRS determination rule, e.g.

  - for joint TCI, other QCL-type RS other than QCL-TypeD RS is used as PLRS, or PLRS is mandatory to be included in or associated with the joint TCI state,

  - for separate TCI, PLRS is mandatory to be included in or associated with the UL TCI state.

CA case

RAN1#105-e meeting supports that the source RS determined from the indicated common TCI state ID can be one source RS across CCs or CC-specific source RS. For Rel-17 power control, a PLRS is either included in or associated with UL TCI state or (if applicable) joint TCI state, and the setting of (P0, alpha, closed loop index) can be associated with UL or (if applicable) joint TCI state. For CA case, similarly to common TCI state update across CCs, a common PLRS ID and setting ID can be determined by the indicated common TCI state ID to provide power control parameters for a set of configured CCs/BWPs.

From the common PLRS ID one PLRS across CCs or CC-specific PLRS can be configured to maintain pathloss estimation for each CC in a set of configured CCs. In case of beam alignment, the PLRS for each CC is the spatial relation RS determined from the indicated common TCI state ID. But in case of beam mis-alignment, the determination of PLRS for each CC can follow the spatial relation RS, e.g. both PLRS and spatial relation RS are across CCs or CC-specific. For PC setting (P0, alpha, close loop index), it is also necessary to clarify whether the setting for each CC is across CCs or CC-specific.

- Clarify whether the PLRS and power control setting are shared across CCs or CC-specific when the TCI state is shared across the band.

Beam application time

Agreement On Rel-17 DCI-based beam indication, regarding application time of the beam indication, the UE can assume that one beam application time (BAT) for a given SCS is configured for all the CCs configured with the common TCI state ID update, - Note: It was agreed that the BAT associated with the carrier(s) (hence BWP(s)/CC(s)) on which the beam indication applies is determined based on the carrier with the smallest SCS among the carrier(s) (hence BWP(s)/CC(s)) applying the beam indication - TBD (maintenance): whether a second configured BAT is also supported, e.g. for MPUE or inter-cell BM - The detailed signaling of the BAT is up to RAN2 - FFS: For CC(s) not configured with a common TCI state ID update

In Rel-17, one beam application time is enough for DCI-based beam indication. For MPUE, in the enhance beam report, an index of UE capability value set, indicating the maximum supported number of SRS antenna ports, is reported along with the pair of SSBRI/CRI and L1-RSRP. the UE panel corresponding to the index can be considered as active, where the panel activation status is determined by UE. Thus, there is no need to consider additional delay for panel selection. In RAN4 there is no definition of panel switching requirement to support two BATs for MPUE. For inter-cell beam management, the UE can transmit/receive channel to/from TRP with different PCI through beam switching to non-serving cell. The measurement scheme of non-serving cell SSB is discussed and specified in issue2. The additional delay is also not required due to the maintenance of non-serving cell SSB measurement.

- Don’t support a second configured BAT, e.g. for MPUE or inter-cell BM.

In RAN1 #107-e meeting, it was agreed that one BAT for a given SCS is configured for all the CCs configured with the common TCI state ID update. The following was agreed in RAN2.

IT shall be possible to configure the parameter BeamAppTime differnet for different SCS

FFS if parameter BeamAppTime is under the cell group config.

Our understanding is that the values if further agreed to be a cell group level can be used for all CCs in a CC list. For example, the BAT list is configured under the cell group config, where each BAT value corresponds to a given SCS. For the CC(s) included in the set of configured CCs, dependent on which CC/BWP is used to determine the application time of beam indication, the corresponding BAT value is selected, i.e. the BAT corresponding to the target BWP with smallest SCS. The above unified BAT determination rule can be reused for the CC(s) not configured with a common TCI state ID update, the beam application time of the active BWP on this CC is the BAT corresponding to the SCS of this active BWP in the BAT list under the cell group config.

- The BAT list can be configured under the cell group config and applied for each CC in the CG, regardless of whether the CC is configured with a common TCI state ID update.

- For CC(s) not configured with a common TCI state ID update, the BAT is determined by the SCS of the active BWP on the CC(s).

In section 5.1.5 of TS 38.214, BAT of the indicated TCI state is described as follows, i.e. the new indicated TCI state should be applied starting from the first slot that is at lease BeamAppTime-r17 symbols after the last symbol of the PUCCH.

When the UE would transmit the last symbol of a PUCCH with HARQ-ACK information corresponding to the DCI carrying the TCI-State indication and without DL assignment, or corresponding to the PDSCH scheduling by the DCI carrying the TCI -State indication, and if the indicated TCI-State is different from the previously indicated one, the indicated [TCI-State] with [tci-StateId_r17] should be applied starting from the first slot that is at least BeamAppTime_r17 symbols after the last symbol of the PUCCH. The first slot and the BeamAppTime_r17 symbols are both determined on the carrier with the smallest SCS among the carrier(s) applying the beam indication. The UE can assume one indicated [TCI-State] with [tci-StateId_r17] for DL and UL, for DL only, or for UL only at a time.

If the UE would transmit PUCCH/PUSCH/PUCCH over multiple slots, e.g. multi-slot PDSCH, PUCCH repetition, PUSCH repetition, multi-slot PUSCH, multiple PDSCHs/PUSCHs scheduled by DCI and so on, the transmission occasions of these DL/UL channels can span multiple slots. When the first slot of BAT is a slot in the transmission occasions, it is unclear whether the new indicated TCI state needs to be applied for the transmission occasions after the BAT. If the TCI state of the transmission occasions after the BAT also is based on the TCI state in the first occasion, there may be some issues. For example, the BAT is postponed until the last transmission occasion of these DL/UL channels. Or if another channel overlaps with the transmission occasion after the BAT, at this time the UE needs to maintain multiple beams, which is contrary to the purpose of the unified TCI framework and also requires to define new behaviors to clarify the priority of these overlapped channels. As a unified solution to simplify spec, we think the new indicated TCI state is also applied to the transmission occasion after BAT for these channels across multi-slots.

- Clarify that the indicated Rel-17 TCI state should be applied for the transmission occasion after BAT for the DL/UL channel over multiple slots,

  - e.g. multi-slot PDSCH, PUCCH repetition, PUSCH repetition, multi-slot PUSCH, multiple PDSCHs/PUSCHs scheduled by DCI.

In case of CA, for a single common BAT, the first slot and the Y symbols are both determined on the carrier with the smallest SCS among the carrier(s) applying the beam indication. However, if the CC carrying the ACK of the beam indication and target CCs belong to different TAGs, it is necessary to clarify how to determine the beam application time of each CC.

Figure 1. Beam application time considering different TAGs for the CC carrying ACK of beam indication and target CCs

For example, when the CC carrying the ACK of the beam indication belong to TAG1 in FR1, target CC(s) belong to TAG2 in FR2, the starting point of Y symbols on target CC with smallest SCS should be mapped from the last symbol of the ACK of beam indication on the CC carrying the ACK of the beam indication considering the TA value of TAG1 and TAG2. If target CCs applying beam indication belong to different TAG, the CC with smallest SCS in each TAG can be used as the reference to determine the beam application time for all target CCs belonging to this TAG.

- For determination of beam application time, clarify whether TA (time advance) is taken into account when determining the starting time, i.e., the last symbol of the PUCCH carrying the ACK of the beam indication.

- For the case when PUCCH is FR1 and beam application time is in FR2 where two TAGs are applied separately for FR1 and FR2, clarify whether the time of last symbol of PUCCH is determined in FR1 first and then map the time to the applied CCs. Counting of Y symbols are conducted on the CCs where the beam is applied.

BFR for unified TCI framework

Agreement On Rel-17 unified TCI framework, for intra-cell beam management, after X symbols from the UE receives the BFRR from NW, the UE assumes the same QCL parameter as the ones associated with the index q new for all PDSCH /PDCCH receptions in a CC, as well as other signals/channels configured to sharing the same indicated Rel-17 TCI state as PDSCH /PDCCH reception. - The above applies to Rel-15 SpCell BFR, Rel-16 CBRA based SpCell BFR, and Rel-16 SCell BFR - Note: q new is a candidate beam identified by the UE in set q1. q1 is the set of candidate beams Agreement On Rel-17 unified TCI framework, after X symbols from the UE receives the BFRR from NW, the UE uses the same UL spatial filter as the one associated with the index q new or the last PRACH transmission for all PUSCH transmissions and all of PUCCH resources in a CC, as well as other signals/channels configured to sharing the same indicated Rel-17 TCI state as PUSCH and all of PUCCH resources. - The above applies to Rel-15/16 SpCell BFR, Rel-16 CBRA based SpCell BFR, and Rel-16 SCell BFR - Note: q new is a candidate beam identified by the UE in set q1. q1 is the set of candidate beams - FFS: UL PC control including q u, q d, and closed loop index

In order to recovery beam pair link quickly, BFR procedure similar to Rel-15/ 16 is applied to Rel-17 unified TCI framework. In Rel-17 BFR procedure, only beam reset behavior is agreed and other BFR steps remain unchanged like Rel-15/16, e.g. beam failure detection, candidate beam detection, beam failure request transmission and beam failure response reception.  There are some remaining issues in the agreements of RAN1 #107-e meeting. The same values with Rel-15/16 can be used, i.e. X=28 for new beam application, qu=0, qd=qnew, and l=0 for UL PC parameters determination.

- X=28 for new beam application.

- qu=0, qd=qnew, and l=0 for UL PC parameters determination.

For CA case in Rel-17 unified TCI framework, a common TCI state is used for all target CCs in a set of configured CCs, where the common TCI state ID determines one source RS across CCs or CC-specific source RSs with a same QCL-TypeD RS to provide QCL-TypeD indication and to determine UL Tx spatial filter. For a set of configured CCs, it is enough to configure BFR on only one cell, e.g. on PCell or a SCell. When beam failure is declared on the cell, it can be considered that beam failure occurs on all cells in the set of configured CCs. Beam reset is required on all cells in the set of configured CCs by using new beam.

- In Rel-17 unified TCI framework, for CA case, BFR can be configured only on one cell in a set of configured CCs.

- New beam indicated by BFRQ is used for beam reset on all cells in a set of configured CCs.

For SCell BFR in Rel-17 unified TCI framework, the beam reset is only applied for the PUCCH, PUSCH and SRS in FR2. For FR1, the new beam application for these UL channels/RSs should be excluded.

- Support the following TP.

TS 38.213 6 Link recovery procedures < Unchanged parts are omitted > If a UE is provided TCI-State_r17 indicating a unified TCI state, after X symbols from a last symbol of a PDCCH reception with a DCI format scheduling a PUSCH transmission with a same HARQ process number as for the transmission of the first PUSCH and having a toggled NDI field value, the UE - monitors PDCCH in all CORESETs, and receives PDSCH and aperiodic CSI-RS in a resource from a CSI-RS resource set using the same antenna port quasi co-location parameters as the ones associated with the corresponding index qnew, if any - transmits, when applicable, PUCCH, PUSCH and SRS that uses a same spatial domain filter with same indicated TCI state as for the PUCCH and PUSCH, using a same spatial domain filter as the one corresponding to qnew, if any < Unchanged parts are omitted >

Inter-cell mobility

The following agreements was achieved for L1/L2-centric mobility in RAN1 #107-e meeting.

Agreement On Rel-17 enhancements for inter-cell beam management and inter-cell mTRP, Rel-15 L1-RSRP reporting format is reused for all SSBRI-RSRP pairs in one L1-RSRP reporting instance, i.e. for K>1, (K-1) 4-bit differential L1-RSRP(s) calculated relative to the reference (absolute) 7-bit L1-RSRP Agreement On Rel-17 enhancements for inter-cell beam management and inter-cell mTRP, a CSI-SSB-ResourceSet configured for L1-RSRP measurement/reporting includes at least a set of SSB indices where PCI indices are associated with the set of SSB indices, respectively. The PCI indices refer to PCIs within the set of PCIs configured for inter-cell beam management or inter-cell multi-TRP. - The additionalInfo associated with SSB(s) with PCI(s) different from the serving cell agreed in RAN1 Agenda Item 8.1.2.2 is also applicable to inter-cell BM - Detailed signaling design is up to RAN2 - FFS (to be concluded in RAN1#107-e): Whether the above L1-RSRP measurement/reporting also includes group-based beam report for inter-cell mTRP Conclusion On Rel-17 enhancements for inter-cell beam management and inter-cell mTRP, in Rel-17, there is no consensus on supporting event-driven beam reporting Conclusion On Rel-17 enhancements for inter-cell beam management, - If UE is receiving DL data from TRP with different PCI on dedicated channels, the UE is not required to receive short message (e.g. paging) and system information from serving cell TRP at the same time Send an LS to RAN2 to provide the following RAN1 conclusion in response to their question – Xi (Huawei) Conclusion On Rel-17 enhancements for inter-cell beam management and inter-cell mTRP , in Rel-17, there is no consensus that the agreed L1-RSRP measurement/reporting also includes group-based beam report for inter-cell mTRP Conclusion On Rel-17 enhancements for inter-cell beam management and inter-cell mTRP , in Rel-17, on the UE behavior when there is overlap for L1-RSRP measurement for SSB associated with serving cell PCI and PCIs different from the serving cell PCI - Prepare an LS to RAN4 informing and recommending RAN4 to investigate the issue - Note: Discussion in UE feature agenda on this issue is not ruled out LS to RAN4 is endorsed in R1-2112762.

Enhanced beam indication with unified framework

In Rel-15, for PDSCH, if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, and for aperiodic CSI-RS, if the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet is smaller than beamSwitchTiming, their QCL assumption may follow the TCI state of the CORESET associated with a monitored search space with the lowest controlResourceSetId. However, in Rel-17, DL channels/RSs are divided into two categories, non-UE-dedicated and UE-dedicated, where in inter-cell cases, non-UE-dedicated DL channels/RSs are defined as the DL channels/RSs associated with non-UE-dedicated CORESET and follow R15/16 TCI state deactivation/activation and indication mechanism for inter-cell cases, while the QCL assumption of DL UE-dedicated channels/RSs follows the indicated TCI-state-r17. Therefore, the default QCL assumption of DL channels/RSs should also be determined separately. UE-dedicated DL channels/RSs shall follow the previous indicated TCI-state-r17 if the channels/RSs starts earlier than the beam application time, and non-UE-dedicated DL channels/RSs (at least include PDSCH and aperiodic CSI-RS) should be clarified to follow the TCI state of the non-UE-dedicated CORESET associated with a monitored search space with the lowest controlResourceSetId.

For inter-cell cases, default beam mechanism should be determined separately.

  - For non-UE-dedicated DL channels/RSs, follow the TCI state of the non-UE-dedicated CORESET associated with a monitored search space with the lowest controlResourceSetId.;

  - For UE-dedicated DL channels/RSs, follow the previous indicated TCI-state-r17;

Besides the default beam, another remaining issue is beam determination/update mechanism across CCs. if only one TCI pool is configured, for intra-cell beam indication mechanism, other CCs will refer to the reference BWP/CC to obtain their QCL assumption information respectively; while for inter-cell beam indication mechanism, e.g., some TCI states associating with a PCI different from that of the serving cell, the beam determination/update mechanism across CCs especially when the activated/indicated TCI state associating with a PCI different serving cell should be clarified.

For inter-cell case with one TCI pool configured within a set of CCs, when different PCIs are associated with the TCI states in different CCs, it should be allowed that the same TCI state ID can refer to different PCI on different CCs.

Inter-cell beam measurement

It has already been agreed to support inter-cell L1-RSRP measurement inside and outside SMTC window in RAN4#101bis-e meeting. And for the case of measurement outside SMTC window for FR1 and FR2, the timing offset between a serving cell and a cell with different PCI is within one CP and take one FFT as the assumption. As for measurement performed inside SMTC window, the timing offset and measurement behaviour is still in discussion.

In Rel-15/16, beam measurement and report only consider SSB/CSI-RS resources in serving cell. And if the higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to "Configured", the UE shall derive the channel measurements for computing L1-RSRP based on only the most recent occasion of SS/PBCH associated with the CSI resource setting.  However, in Rel-17, beam measurement involves multiple cells and SSBs associated with different PCIs may overlap in time. And for the overlapped SSBs, UE cannot perform one-shot L1-RSRP measurements (i.e. M=1) on the same SSB occasion simultaneously for both serving cell and a cell with a different PCI, since they may have different Rx beam assumption and it is very difficult to adjust AGC so as to ensure performance. Therefore, to avoid the ambiguity, for inter-cell beam measurement, the higher layer parameter timeRestrictionForChannel Measurements should be always set to “notConfigured”.

For inter-cell beam measurement, the higher layer parameter timeRestrictionForChannel Measurements should be always set to “notConfigured”

Resource allocation of DL

For resource allocation of DL channels, PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around non-serving cell SSB with the same PCI, which means per PCI rate match, has been agreed in AI.8.1.2.2.

From UE measurement perspective, when UE is configured to measure on SSBs while still receiving PDSCH on overlapped resources, there would be performance degradations and additional UE complexities to guarantee the corresponding performance. In legacy UE implementation, there is no such simultaneous L1-RSRP measurement and PDSCH reception on the same RE case. Three potential rate match schemes for the serving cell are listed as following:

scheme 1: only SSBs associated with the serving cell PCI are used to determine the rate match pattern for PDCCH and PDSCH;

scheme 2: SSBs associated with the serving cell PCI, and in CSI-SSB-ResourceSet, SSBs associated with PCI different from the serving cell PCI are used to determine the rate match pattern for PDCCH and PDSCH.

scheme 3: both SSBs associated with the serving cell PCI and SSBs associated with PCI different from the serving cell PCI are used to determine the rate match pattern for PDCCH and PDSCH.

Option2 seems a reasonable solution to make UE implementation feasible. If it is not agreed, simultaneous reception of PDCCH/PDSCH and SSBs for L1-RSRP should be optional UE capability.

When UE is configured to measure on SSBs while still receiving PDSCH on overlapped resources Support either one of the following two options:

- Option1: The PDCCH/PDSCH should be rate matched around the SSBs configured for L1-RSRP measurement.

- Option2: Support UE capability for simultaneous reception of PDCCH/PDSCH and SSBs for L1-RSRP measurement on the same REs.

Support the following TP for option1.

TS 38.213 10 UE procedure for receiving control information <unchanged parts are omitted> For monitoring of a PDCCH candidate by a UE, if the UE - has received ssb-PositionsInBurst in SIB1 and has not received ssb-PositionsInBurst in ServingCellConfigCommon for a serving cell, and - does not monitor PDCCH candidates in a Type0-PDCCH CSS set, and - at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in SIB1, or overlaps with at least one RE of a candidate SS/PBCH block corresponding to a SS/PBCH block index provided by CSI-SSB-ResourceSet if AdditionalPCIInfo is configured, the UE is not required to monitor the PDCCH candidate. For monitoring of a PDCCH candidate by a UE, if the UE - has received ssb-PositionsInBurst in ServingCellConfigCommon for a serving cell, and - does not monitor PDCCH candidates in a Type0-PDCCH CSS set, and - at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in ServingCellConfigCommon, or overlaps with at least one RE of a candidate SS/PBCH block corresponding to a SS/PBCH block index provided by CSI-SSB-ResourceSet if AdditionalPCIInfo is configured, the UE is not required to monitor the PDCCH candidate. For monitoring of a PDCCH candidate by a UE, if the UE - has received ssb-PositionsInBurst in AdditionalPCIInfo for a serving cell, and - at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in AdditionalPCIInfo with same physical cell identity as the one associated with a RS having same quasi-collocation properties as a CORESET for the PDCCH candidate, or overlaps with at least one RE of a candidate SS/PBCH block corresponding to a SS/PBCH block index provided by CSI-SSB-ResourceSet, the UE is not required to monitor the PDCCH candidate. <unchanged parts are omitted>

TS 38.214 5.1.4 PDSCH resource mapping <unchanged parts are omitted> When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 0, the UE shall assume that no SS/PBCH block is transmitted in REs used by the UE for a reception of the PDSCH. When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 1, RA-RNTI, MSGB-RNTI, P-RNTI or TC-RNTI, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurs, and if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted. A UE expects a configuration provided by ssb-PositionsInBurst in ServingCellConfigCommon to be same as a configuration provided by ssb-PositionsInBurst in SIB1. When receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or PDSCHs with SPS, the REs corresponding to the configured or dynamically indicated resources in Clauses 5.1.4.1, 5.1.4.2 are not available for PDSCH. Furthermore, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources, the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted. If the PDSCH resource allocation overlaps with PRBs containing a candidate SS/PBCH block corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in AdditionalPCIInfo with same physical cell identity as the one associated with a RS having same quasi-collocation properties as the PDSCH, the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted. The UE shall assume that the PRBs containing SS/PBCH block configured in CSI-SSB-ResourceSet are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted if AdditionalPCIInfo is configured. A UE is not expected to handle the case where PDSCH DM-RS REs are overlapping, even partially, with any RE(s) not available for PDSCH. For operation with shared spectrum channel access, SS/PBCH block transmission according to ssb-PositionsInBurst represents all of the candidate SS/PBCH blocks corresponding to SS/PBCH block indices provided by ssb-PositionsInBurst as described in Clause 4.1 of [6, TS 38.213]. <unchanged parts are omitted>

On PUSCH/PUCCH transmission in serving cell

In Rel-17 coverage enhancement AI, PUSCH repetition type A is introduced where PUSCH repetition is counted based on “available slot”. Available slot is mostly determined based on semi-static configurations of various signals/channels. Slot where SSB is transmitted is not available for PUSCH repetition transmission. It was discussed how/whether to take inter-cell mTRP into account for available slot determination since SSBs from TRP associated with different PCI can be overlapping or non-overlapping in time. In the case of inter-cell mTRP operation and Rel-17 PUSCH repetition type A enabled, there could be 3 options for available slot determination as listed below, where option 1 means only the SSBs with serving cell PCI are considered for available slot determination, option 2 means SSBs with PCI different from the serving cell PCI are also considered for available slot determination on top of considering SSBs with serving cell PCI, option 3 means the SSBs associated with a PCI are considered for available slot determination if the PCI is the associated with the TCI state configured for the PUSCH transmission.

•	Option 1: SSBs with PCI different from the serving cell PCI are only used for actual PUSCH repetition determination after the available slots are determined

•	Option 2: Both SSBs with serving cell PCI and SSBs with PCI different from the serving cell PCI are used for the available slot determination

•	Option 3: SSBs with the same PCI as the SSB used to determine the spatial relation of the PUSCH, are used to determine the available slots. Other set of SSBs are used for actual PUSCH repetition determination after the available slots are determined.

In MIMO, according to the agreements reached so far in inter-cell mTRP and inter-cell BM it is not clear whether PUSCH/PUCCH can be transmitted in the serving cell on the symbols (slot) where SSB from TRP associated with different PCI than serving cell PCI is being transmitted. There could be different options which could have different impact on available slot determination in coverage enhancement AI. For example, whether PUSCH transmission in severing is cancelled due to collision with SSB from TRP associated with different PCI than serving cell PC, or UE doesn’t expect gNB to schedule PUSCH transmission in severing on the symbols where it collides with SSB from TRP associated with different PCI than serving cell PC. Whether PUCCH transmission in serving is postponed when it collides with SSB from TRP associated with different PCI than serving cell PCI.

UE behavior on PUSCH/PUCCH transmission in the serving cell on the symbols where SSB from TRP associated with different PCI than serving cell PCI is being transmitted should be discussed and concluded.

Beam selection for multi-panel UE

Working Assumption Support the UE reporting a list of UE capability value sets - Each UE capability value set comprises the max supported number of SRS ports - For any two different value sets, at least one capability value needs to be different - FFS: If in addition also identical value sets are allowed. - Whether the UE capability value set can be common across all BWPs/CCs in same band or BC can be discussed in UE feature session Agreement On Rel.17 enhancements to facilitate UE -initiated panel activation and selection via UE reporting a list of UE capability value sets, the correspondence between each reported CSI-RS and/or SSB resource index and one of the UE capability value sets in the reported list is determined by the UE (analogous to Rel-15/16) and is informed to NW in a beam reporting instance. - The Rel-15/16 beam reporting framework is used, i.e. the index of corresponding UE capability value set is reported along with the pair of SSBRI/CRI and L1-RSRP/SINR (up to 4 pairs, with 7-bit absolute and 4-bit differential) in the beam reporting UCI and down select (maintenance) between the following two options: - Option 1: UE can report one index for all the reported CRIs/SSBRIs in one beam reporting - Option 2: UE can report one index for each reported CRI/SSBRI in one beam reporting. - FFS: whether/how to take DL-only panel into account in the report - FFS: Time-domain behaviour, e.g. the support periodic, semi-persistent, and aperiodic reporting - FFS: Semi-persistent and/or aperiodic reporting is triggered only when periodic reporting is configured - (Working assumptions): Support acknowledgement mechanism of the reported correspondence from NW to UE, which doesn't preclude reusing/reinterpreting existing signaling/procedure - FFS (maintenance): the application time for the reported correspondence (if any), the exact acknowledgement mechanism and whether spec impact is needed, e.g. based on TCI state update, BFR response like mechanism, including the application time for the reported correspondence, if any - No new DCI format and no new RNTI are introduced for this function. Conclusion On Rel.17 enhancements to facilitate UE -initiated panel activation and selection via UE reporting a list of UE capability value sets, other than the max supported number of SRS ports (note: currently pending endorsement in proposal 4.A), there is no consensus on supporting another UE capability type

In RAN1 #107-e meeting, the Rel-15/16 beam report is enhanced to support panel selection for MPUE. The index of UE capability value set, indicating the maximum supported number of SRS antenna ports, is reported along with the pair of SSBRI/CRI and L1-RSRP/L1-SINR in the beam reporting UCI.

For the beam report UCI format, the option 2, i.e. UE can report one index for each reported SSBR/CRI in one beam reporting, can support the flexibility of panel activation at UE. The two options, representing two different UCI formats, may need to be configured by gNB based on UE panel status reporting, which introduces an additional restriction and unnecessary UE implementation exposure, e.g. only one activated UE panel or multiple activated UE panels. But in fact, the panel activation status is determined by UE and transparent to the gNB, and the panel activation status can be changed at any time by UE. In beam report, the UE panels corresponding to the indexes can be considered as active. Obviously, the option 2 provides a more flexible reporting scheme.

- For beam reporting UCI, support option2, i.e. UE can report one index for each reported SSBRI/CRI in one beam report.

For DL-only panel selection, there is no need to introduce a mixed report format with DL panel selection and UL panel selection. For Rel-17 unified TCI framework, joint TCI and separate TCI are configured by RRC. When joint TCI is configured, the enhanced beam report for MPUE is enough to indicate the optimal TCI state for DL and UL, where the optimal UE panel for DL and for UL are the same and the DL-only panel is not required. When separate TCI is configured, it is also not required to include DL-only panel and UL-only panel in one beam reporting. The different beam report type can be configured by gNB. For example, RRC parameter reportQuantity set to legacy value in Rel-15/16 or new RRC parameter in Rel-17 can be used to indicate DL-only or UL-only panel selection.

- Don’t support a mixed report format for DL-only panel selection and UL-only panel selection.

The correspondence between SSBRI/CRI and UE capability value set is determined by UE and is informed to gNB in a beam reporting instance. When the gNB schedules transmission and reception of different channel/RS based on beam report, the joint/separate TCI state in Rel-17 in beam indication can implicitly be used to indicate which UE panel is for channel/RS transmission and reception, which achieves the purpose of the alignment of panel status between gNB and UE.

For the issue of beam report UCI loss, new acknowledgement mechanism is not required in Rel-17. BWP switch scheme can be used for UL panel selection. For example, two capability value sets are reported by UE and the gNB configures two index values, i.e. index1 and index2, where index1 corresponds to panel1 supporting 4 SRS antenna ports and index2 corresponds to panel2 supporting SRS antenna ports, respectively. Furthermore, 4 SRS antenna ports is configured for BWP1 and 2 SRS antenna ports is configured for BWP2. The most recent old beam report with {(index1, CRI2, L1-RSRP1)，(index2, CRI1, L1-RSRP2)} is correctly received by the gNB, but new beam report with {(index1, CRI1, L1-RSRP3)，(index2, CRI2, L1-RSRP4)} is lost. If the gNB is ready to schedule 4-port UL transmission, BWP1 needs to be switched firstly for panel1 selection when current active BWP is BWP2. Based on the new beam report, the TCI state corresponding to CRI1 should be optimal on UE panel1. But due to the new beam report was lost, the indicated TCI state is based on the old beam report, i.e. the gNB considers CRI2 as the source RS of the optimal TCI state. The subsequent triggered SRS and scheduled PUSCH with 4 ports on UE panel1 are based on CRI2, e.g. not optimal beam on panel1. Similar to legacy gNB implementation, for example, when the gNB detects the deterioration of uplink performance, the gNB can re-trigger the beam report to update the correspondence between index and CRI. In addition, the same situation also exists for periodic SRS or CG PUSCH with 4 ports and non-optimal beam on panel1. At this point, the gNB can update the UL beam for these signals by legacy signaling based on a new triggered beam report.

From the analysis above, the UL panel selection with different supported number of ports can be realized through BWP switching, regardless of whether the beam report including the correspondence is lost. The UL panel supported a specific number of ports is selected via BWP switching signaling from the gNB. If the indicated TCI state is not optimal, the gNB can trigger another beam report based on gNB implementation for the case of misalignment issue.

- BWP switching scheme can be used for UL panel selection.

- It is not necessary to define new acknowledgement mechanism for beam report in Rel-17.

For the time-domain behavior of beam report, there is no need to introduce new triggering scheme for semi-persistent and/or aperiodic reporting. When uplink signal is not triggered or scheduled, the gNB does not need to know the correspondence between the SSBRI/CRI and the index of UE capability value set. If the periodic beam report is not configured, when uplink signal is triggered or scheduled, the gNB can determine whether to trigger aperiodic beam report based on the uplink performance to update the correspondence. It is up to gNB implementation for time-domain behavior configuration.

- Support to reuse legacy time-domain behavior of beam report, e.g. periodic, semi-persistent and aperiodic reporting.

In section 5.2.1.4.3 of TS 38.214, BAT of the indicated TCI state is described as follows, i.e. the enhanced beam report is introduced for L1-RSRP reporting.

When the UE is configured with [capabilitycinIndexReporting] an index of UE capability value set, indicating the maximum supported number of SRS antenna ports, is reported along with the pair of SSBRI/CRI and L1-RSRP.

In the agreement of RAN1 #107-e meeting, the Rel-15/16 beam reporting framework is used, i.e. the index of corresponding UE capability value set is reported along with the pair of SSBRI/CRI and L1-RSRP/SINR. So for L1-SINR reporting, the enhanced beam report also should be added in Rel-17 spec.

- Support the proposed TP for L1-SINR reporting.

TS 38.214 5.2.1.4.4 L1-SINR Reporting For L1-SINR computation, for channel measurement the UE may be configured with NZP CSI-RS resources and/or SS/PBCH Block resources, for interference measurement the UE may be configured with NZP CSI-RS or CSI-IM resources. - for channel measurement, the UE may be configured with CSI-RS resource setting with up to 16 resource sets, with a total of up to 64 CSI-RS resources or up to 64 SS/PBCH Block resources. For L1-SINR reporting, if the higher layer parameter nrofReportedRS in CSI-ReportConfig is configured to be one, the reported L1-SINR value is defined by a 7-bit value in the range [-23, 40] dB with 0.5 dB step size, and if the higher layer parameter nrofReportedRS is configured to be larger than one, or if the higher layer parameter groupBasedBeamReporting is configured as 'enabled', the UE shall use differential L1-SINR based reporting, where the largest measured value of L1-SINR is quantized to a 7-bit value in the range [-23, 40] dB with 0.5 dB step size, and the differential L1-SINR is quantized to a 4-bit value. The differential L1-SINR is computed with 1 dB step size with a reference to the largest measured L1-SINR value which is part of the same L1-SINR reporting instance. When NZP CSI-RS is configured for channel measurement and/or interference measurement, the reported L1-SINR values should not be compensated by the power offset(s) given by higher layer parameter powerControOffsetSS or powerControlOffset. When one or two resource settings are configured for L1-SINR measurement - If the higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to 'notConfigured', the UE shall derive the channel measurements for computing L1-SINR reported in uplink slot n based on only the SSB or NZP CSI-RS, no later than the CSI reference resource, (defined in TS 38.211[4]) associated with the CSI resource setting. - If the higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to 'configured', the UE shall derive the channel measurements for computing L1-SINR reported in uplink slot n based on only the most recent, no later than the CSI reference resource, occasion of SSB or NZP CSI-RS (defined in [4, TS 38.211]) associated with the CSI resource setting. - If the higher layer parameter timeRestrictionForInterferenceMeasurements in CSI-ReportConfig is set to 'notConfigured', the UE shall derive the interference measurements for computing L1-SINR reported in uplink slot n based on only the CSI-IM or NZP CSI-RS for interference measurement (defined in [4, TS 38.211]) or NZP CSI-RS for channel and interference measurement no later than the CSI reference resource associated with the CSI resource setting. - If the higher layer parameter timeRestrictionForInterferenceMeasurements in CSI-ReportConfig is set to 'configured', the UE shall derive the interference measurements for computing the L1-SINR reported in uplink slot n based on the most recent, no later than the CSI reference resource, occasion of CSI-IM or NZP CSI-RS for interference measurement (defined in [4, TS 38.211]) or NZP CSI-RS for channel and interference measurement associated with the CSI resource setting. When the UE is configured with [capabilitycinIndexReporting] an index of UE capability value set, indicating the maximum supported number of SRS antenna ports, is reported along with the pair of SSBRI/CRI and L1-SINR.

MPE issue for multi-panel UE

Conclusion On Rel.17 enhancements to facilitate MPE mitigation, there is no consensus on a specification-based criterion for selecting N from a candidate SSB/CSI-RS resource pool Agreement On Rel.17 enhancements to facilitate MPE mitigation, support N=1, 2, 3, and 4 - N is defined as the number of reported measurements - UE reports supported largest N value as a UE capability Agreement On Rel.17 enhancements to facilitate MPE mitigation, confirm the following working assumption (in the midst of the previous agreement) as an agreement with the following refinement (highlighted in red): On Rel.17 enhancements to facilitate MPE mitigation, support the following enhancement on the Rel-16 event-triggered P-MPR-based reporting (included in the PHR report when a threshold is reached, reported via MAC-CE): - In addition to the existing field in the PHR MAC-CE, N≥1 P-MPR values can be reported - The N P-MPR values are reported together with the following: - (Working Assumption) For each P-MPR value, up to M SSBRI(s)/CRI(s), where the SSBRI(s)/CRI(s) is selected by the UE from a candidate SSB/CSI-RS resource pool (FFS: how to perform the selection) - Support M=1 - FFS: The supported value(s) of M - FFS: Additional reporting quantities, e.g. SSBRI/CRI, MPR+DL RSRP, or modified virtual PHR - FFS: additional signaling (e.g. CSI triggering) from the NW. Agreement On Rel.17 enhancements to facilitate MPE mitigation, the candidate resource pool corresponds to a CSI-RS/SSB resource set configured via RRC (details up to RAN2)

The UE can apply P-MPR to ensure compliance with applicable electromagnetic power density exposure requirements and addressing unwanted emissions/self-defense requirements. Based on P-MPR report the gNB can obtain the real-time status of the UE accurately, and adjust the scheduling scheme and panel/beam selection accordingly.

In RAN1 #106bis-e meeting, it is agreed that enhanced Rel-16 event-triggered P-MPR-based reporting is supported, where N pairs of P-MPR and SSBRI/CRI are reported. If there is no SSBRI/CRI with a better measurement result at the UE, only P-MPR without associated SSBRI/CRI in MAC CE does not facilitate beam indication by gNB. We prefer that N is defined as the maximum number of reported measurements and the UE selects and reports up to N pairs of P-MPR(s) and SSBRI(s)/CRI(s) from a candidate SSB/CSI-RS resource pool.

- The value of N is configured by gNB and is the maximum number of reported measurements in one P-MPR report in addition to legacy P-MPR.

- In a MPE report, in addition to legacy P-MPR, the UE selects and reports up to N pairs of P-MPR(s) and SSBRI(s)/CRI(s) from a candidate SSB/CSI-RS resource pool.

In current RAN2 spec TS38.321, if mpe-Reporting-FR2 is configured, the MAC CE entity shall trigger MPE reporting when the triggering condition is met. In single entry PHR MAC CE or multiple entry PHR MAC CE, the applied power backoff to meet MPE requirement is indicated, where the measured values of P-MPR levels in dB are included.

When SSBRI(s)/CRI(s) in beam report is not included in latest P-MPR report, UE should report P-MPR MAC CE to gNB. For example, {P-MPR1, CRI1} is included in latest P-MPR report, but based on subsequent beam measurement, a new beam report is transmitted to gNB which indicate that the beam pair link according to CRI1 quality is worse than CRI2. At this time the UE needs to report new pair of P-MPR and CRI, so that the gNB can accurately select beam with better quality. Thus, it is necessary to introduce new triggering conditions to ensure that gNB can obtain the pair of P-MPR and SSBRI/CRI in time.

The legacy triggering condition in TS38.321 needs to be modified to accommodate P-MPR report per measurement by replacing measured P-MPR with at least one measured P-MPR. MAC CE carrying P-MPR report different from legacy PHR report is transmitted based on the configuration and the triggering condition for MPE.

- if mpe-Reporting-FR2 is configured, and mpe-ProhibitTimer is not running: -the measured P-MPR applied to meet FR2 MPE requirements as specified in TS 38.101-2 [15] is equal to or larger than mpe-Threshold for at least one activated FR2 Serving Cell since the last transmission of a PHR in this MAC entity; or -the measured P-MPR applied to meet FR2 MPE requirements as specified in TS 38.101-2 [15] has changed more than phr-Tx-PowerFactorChange dB for at least one activated FR2 Serving Cell since the last transmission of a PHR due to the measured P-MPR applied to meet MPE requirements being equal to or larger than mpe-Threshold in this MAC entity.

- Introduce new triggering condition of P-MPR report.

  - SSBRI(s)/CRI(s) in beam report is not included in latest P-MPR report.

- Modify legacy triggering condition of P-MPR report by replacing measured P-MPR with at least one measured P-MPR.

Conclusions

- In this contribution, we analyze potential enhancements of multi-beam operation in Rel-17. To summarize, we have following proposals and TPs.

- The non-UE-dedicated channels include the PDCCH on the CORESETs associated with type 0/0A/1/2 CSS set and the scheduled PDSCH by the PDCCH.

- Support the following definition for CORESET A and CORESET B

  - ‘CORESET A’: A CORESET other than CORESET#0 associated with only UE-dedicated reception on PDCCH in a CC, comprising CORESETs in association with:

    - [USS and/or CSS Type 3]

  - ‘CORESET B’:  A CORESET other than CORESET#0 associated with only non-UE-dedicated reception on PDCCH in a CC, comprising CORESETs in association with:

    - [CSS or CSS other than Type 3]

- Rel-17 TCI state indication of CORESET C (if supported) and CORESET#0 follows the same rule as CORESET B, i.e.

- for intra-cell BM, for any PDCCH reception on a CORESET C or CORESET#0 and the respective PDSCH reception, whether or not UE to apply the indicated Rel-17 TCI sate associated with the serving cell is determined per CORESET by RRC.

- for inter-cell BM, for any PDCCH reception on a CORESET C or CORESET#0 and the respective PDSCH reception, the indicated Rel-17 TCI state associated with a PCI different from the serving cell is never applied.

- if the indicated Rel-17 TCI state is not applied to CORESET#0, the legacy Rel-15/16 signaling is reused forCORESET#0.

- The PUSCH scheduled/activated and PUCCH triggered by CORESET#0 follows the beam indicated/activated for CORESET #0.

- Confirm the following working assumption:

- The UE is not expected to be configured with Rel-15/Rel-16 TCI/SpatialRelationInfo if the UE is configured with Rel-17 TCI in any CC in a band

  - The CC list for Rel-16 multi-CC beam indication should not contain any CC configured with Rel-17 TCI

- For any SRS resource or resource set that does not share the same indicated Rel-17 TCI state but can be configured as a target signal of Rel-17 TCI, the Rel-15/16 principle for SRS UL PC parameter setting configuration/activation is used.

- For the following other DL signals in the same symbols at the aperiodic CSI-RS, aperiodic CSI-RS for BM and for CSI does not share the same indicated Rel-17 TCI state as UE-dedicated reception on PDSCH and for UE-dedicated reception on all or subset of CORESETs in a CC, except

  - P-/SP-CSI-RS

  - Non-UE-dedicated reception on PDCCH and the associated PDSCH or aperiodic CSI-RS, which is configured to not share same TCI state as UE-dedicated reception.

- MAC CE can be used to update QCL configuration for periodic SRS that is valid target signal of Rel-17 TCI state but not to share the same Rel-17 TCI state as dynamic-grant/configured-grant based PUSCH, all or subset of dedicated PUCCH resources.

- For periodic CSI-RS and aperiodic CSI-RS that is valid target signal of Rel-17 TCI state but not to share same TCI state as UE-dedicated reception on PDSCH and for UE-dedicated reception on all or subset of CORESETs in a CC, MAC CE can be used to update QCL assumption for the CSI-RS.

- Do not support MAC-CE based update for the closed loop index associated with UL or (if applicable) joint TCI state.

- If the association is not configured by RRC, a default setting from the setting list can be used.

- UL PC parameter settings from different setting lists including additional P0 list are associated with an UL or (if applicable) joint TCI state for different services.

  - TCI state ID and OLPC are used to determine PC parameters.

- Clarify that UL PC parameter setting included or associated in unified TCI state is not applied for a PUSCH (re)transmission configured by ConfiguredGrantConfig.

- Support the following TP.

TS 38.213 7.1 Uplink Power control Uplink power control determines a power for PUSCH, PUCCH, SRS, and PRACH transmissions. A UE does not expect to simultaneously maintain more than four pathloss estimates per serving cell for all PUSCH/PUCCH/SRS transmissions as described in clauses 7.1.1, 7.2.1, and 7.3.1, except for SRS transmissions configured by SRS-PosResourceSet as described in clause 7.3.1. If the UE is provided a number of RS resources for pathloss estimation for PUSCH/PUCCH/SRS transmissions that is larger than 4, the UE maintains for pathloss estimation RS resources corresponding to RS resource indexes qd as described in clauses 7.1.1, 7.2.1, and 7.3.1. If an RS resource updated by MAC CE, as described in clauses 7.1.1, 7.2.1 and 7.3.1, is one from the RS resources the UE maintains for pathloss estimation for PUSCH/PUCCH/SRS transmissions, the UE applies the pathloss estimation based on the RS resources starting from the first slot that is after slot k+3∙Nslotsubframe, μ+2μ∙kmac where k is the slot where the UE would transmit a PUCCH or PUSCH with HARQ-ACK information for the PDSCH providing the MAC CE, μ is the SCS configuration for the PUCCH or PUSCH, respectively, that is determined in the slot when the MAC CE command is applied and kmac is a number of slots for SCS configuration μ=0 provided by K-Mac or kmac=0 if K-Mac is not provided. A PUSCH/PUCCH/SRS/PRACH transmission occasion i is defined by a slot index ns,fμ within a frame with system frame number SFN, a first symbol S within the slot, and a number of consecutive symbols L. For a PUSCH transmission with repetition Type B, a PUSCH transmission occasion is a nominal repetition [6, TS 38.214]. In the remaining of this clause, if a UE is provided TCI-State_r17 and for an indicated TCI-State_r17 as described in [6, TS 38.214] - in clauses 7.1.1, 7.2.1, and 7.3.1, the RS index qd for obtaining the downlink pathloss estimate for PUSCH, PUCCH, and SRS transmission is provided by PL-RS associated with or included in the indicated TCI-StateID_r17 - in clause 7.1.1, if p0-Alpha-CLID-PUSCH-Set is provided, the values of PO_UE_PUSCH,b,f,cj, αb,f,cj, and the PUSCH power control adjustment state l are provided by p0-Alpha-CLID-PUSCH-Set associated with the indicated TCI-StateID_r17 except for a PUSCH (re)transmission configured by ConfiguredGrantConfig - in clause 7.2.1, if p0-Alpha-CLID-PUCCHSet is provided, the values of PO_PUCCH,b,f,cqu and the PUCCH power control adjustment state l are provided by p0-Alpha-CLID-PUCCH-Set associated with the indicated TCI-StateID_r17 - in clause 7.3.1, if p0-Alpha-CLID-SRS-Set is provided, the values of PO_SRS,b,f,cqs, αSRS,b,f,cqs, and SRS power control adjustment state l are provided by p0-Alpha-CLID-SRS-Set associated with the indicated TCI-StateID_r17 In the remaining of this clause, if a PDCCH reception by a UE includes two PDCCH candidates from corresponding search space sets, as described in clause 10.1 - a PDCCH monitoring occasion is the union of the PDCCH monitoring occasions for the two PDCCH candidates - the end of the PDCCH reception is the end of the PDCCH candidate that ends later The PDCCH reception includes the two PDCCH candidates also when the UE is not required to monitor one of the two PDCCH candidates as described in clauses 10, 11.1, and 11.1.1.

- Clarify whether two TCI modes (joint TCI, separate TCI) are needed for application of unified TCI for FR1 UL channels.

- If separate TCI mode is also supported in FR1, inform RAN2 that QCL-TypeD RS configuration for UL TCI state should be optional.

- For FR1 UL channels, clarify the PLRS determination rule, e.g.

  - for joint TCI, other QCL-type RS other than QCL-TypeD RS is used as PLRS, or PLRS is mandatory to be included in or associated with the joint TCI state,

  - for separate TCI, PLRS is mandatory to be included in or associated with the UL TCI state.

- Clarify whether the PLRS and power control setting are shared across CCs or CC-specific when the TCI state is shared across the band.

- Don’t support a second configured BAT, e.g. for MPUE or inter-cell BM.

- The BAT list can be configured under the cell group config and applied for each CC in the CG, regardless of whether the CC is configured with a common TCI state ID update.

- For CC(s) not configured with a common TCI state ID update, the BAT is determined by the SCS of the active BWP on the CC(s).

- Clarify that the indicated Rel-17 TCI state should be applied for the transmission occasion after BAT for the DL/UL channel over multiple slots,

  - e.g. multi-slot PDSCH, PUCCH repetition, PUSCH repetition, multi-slot PUSCH, multiple PDSCHs/PUSCHs scheduled by DCI.

- For determination of beam application time, clarify whether TA (time advance) is taken into account when determining the starting time, i.e., the last symbol of the PUCCH carrying the ACK of the beam indication.

- For the case when PUCCH is FR1 and beam application time is in FR2 where two TAGs are applied separately for FR1 and FR2, clarify whether the time of last symbol of PUCCH is determined in FR1 first and then map the time to the applied CCs. Counting of Y symbols are conducted on the CCs where the beam is applied.

- X=28 for new beam application.

- qu=0, qd=qnew, and l=0 for UL PC parameters determination.

- In Rel-17 unified TCI framework, for CA case, BFR can be configured only on one cell in a set of configured CCs.

- New beam indicated by BFRQ is used for beam reset on all cells in a set of configured CCs.

- Support the following TP.

TS 38.213 6 Link recovery procedures < Unchanged parts are omitted > If a UE is provided TCI-State_r17 indicating a unified TCI state, after X symbols from a last symbol of a PDCCH reception with a DCI format scheduling a PUSCH transmission with a same HARQ process number as for the transmission of the first PUSCH and having a toggled NDI field value, the UE - monitors PDCCH in all CORESETs, and receives PDSCH and aperiodic CSI-RS in a resource from a CSI-RS resource set using the same antenna port quasi co-location parameters as the ones associated with the corresponding index qnew, if any - transmits, when applicable, PUCCH, PUSCH and SRS that uses a same spatial domain filter with same indicated TCI state as for the PUCCH and PUSCH, using a same spatial domain filter as the one corresponding to qnew, if any < Unchanged parts are omitted >

For inter-cell cases, default beam mechanism should be determined separately.

  - For non-UE-dedicated DL channels/RSs, follow the TCI state of the non-UE-dedicated CORESET associated with a monitored search space with the lowest controlResourceSetId.;

  - For UE-dedicated DL channels/RSs, follow the previous indicated TCI-state-r17;

For inter-cell case with one TCI pool configured within a set of CCs, when different PCIs are associated with the TCI states in different CCs, it should be allowed that the same TCI state ID can refer to different PCI on different CCs.

For inter-cell beam measurement, the higher layer parameter timeRestrictionForChannel Measurements should be always set to “notConfigured”

Support either one of the following two options:

- Option1: The PDCCH/PDSCH should be rate matched around the SSBs configured for L1-RSRP measurement.

- Option2: Support UE capability for simultaneous reception of PDCCH/PDSCH and SSBs for L1-RSRP measurement on the same REs.

Support the following TP for option1.

TS 38.213 10 UE procedure for receiving control information <unchanged parts are omitted> For monitoring of a PDCCH candidate by a UE, if the UE - has received ssb-PositionsInBurst in SIB1 and has not received ssb-PositionsInBurst in ServingCellConfigCommon for a serving cell, and - does not monitor PDCCH candidates in a Type0-PDCCH CSS set, and - at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in SIB1, or overlaps with at least one RE of a candidate SS/PBCH block corresponding to a SS/PBCH block index provided by CSI-SSB-ResourceSet if AdditionalPCIInfo is configured, the UE is not required to monitor the PDCCH candidate. For monitoring of a PDCCH candidate by a UE, if the UE - has received ssb-PositionsInBurst in ServingCellConfigCommon for a serving cell, and - does not monitor PDCCH candidates in a Type0-PDCCH CSS set, and - at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in ServingCellConfigCommon, or overlaps with at least one RE of a candidate SS/PBCH block corresponding to a SS/PBCH block index provided by CSI-SSB-ResourceSet if AdditionalPCIInfo is configured, the UE is not required to monitor the PDCCH candidate. For monitoring of a PDCCH candidate by a UE, if the UE - has received ssb-PositionsInBurst in AdditionalPCIInfo for a serving cell, and - at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in AdditionalPCIInfo with same physical cell identity as the one associated with a RS having same quasi-collocation properties as a CORESET for the PDCCH candidate, or overlaps with at least one RE of a candidate SS/PBCH block corresponding to a SS/PBCH block index provided by CSI-SSB-ResourceSet, the UE is not required to monitor the PDCCH candidate. <unchanged parts are omitted>

TS 38.214 5.1.4 PDSCH resource mapping <unchanged parts are omitted> When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 0, the UE shall assume that no SS/PBCH block is transmitted in REs used by the UE for a reception of the PDSCH. When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 1, RA-RNTI, MSGB-RNTI, P-RNTI or TC-RNTI, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurs, and if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted. A UE expects a configuration provided by ssb-PositionsInBurst in ServingCellConfigCommon to be same as a configuration provided by ssb-PositionsInBurst in SIB1. When receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or PDSCHs with SPS, the REs corresponding to the configured or dynamically indicated resources in Clauses 5.1.4.1, 5.1.4.2 are not available for PDSCH. Furthermore, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources, the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted. If the PDSCH resource allocation overlaps with PRBs containing a candidate SS/PBCH block corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in AdditionalPCIInfo with same physical cell identity as the one associated with a RS having same quasi-collocation properties as the PDSCH, the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted. The UE shall assume that the PRBs containing SS/PBCH block configured in CSI-SSB-ResourceSet are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted if AdditionalPCIInfo is configured. A UE is not expected to handle the case where PDSCH DM-RS REs are overlapping, even partially, with any RE(s) not available for PDSCH. For operation with shared spectrum channel access, SS/PBCH block transmission according to ssb-PositionsInBurst represents all of the candidate SS/PBCH blocks corresponding to SS/PBCH block indices provided by ssb-PositionsInBurst as described in Clause 4.1 of [6, TS 38.213]. <unchanged parts are omitted>

UE behavior on PUSCH/PUCCH transmission in the serving cell on the symbols where SSB from TRP associated with different PCI than serving cell PCI is being transmitted should be discussed and concluded.

- For beam reporting UCI, support option2, i.e. UE can report one index for each reported SSBRI/CRI in one beam report.

- Don’t support a mixed report format for DL-only panel selection and UL-only panel selection.

- BWP switching scheme can be used for UL panel selection.

- It is not necessary to define new acknowledgement mechanism for beam report in Rel-17.

- Support to reuse legacy time-domain behavior of beam report, e.g. periodic, semi-persistent and aperiodic reporting.

- Support the proposed TP for L1-SINR reporting.

TS 38.214 5.2.1.4.4 L1-SINR Reporting For L1-SINR computation, for channel measurement the UE may be configured with NZP CSI-RS resources and/or SS/PBCH Block resources, for interference measurement the UE may be configured with NZP CSI-RS or CSI-IM resources. - for channel measurement, the UE may be configured with CSI-RS resource setting with up to 16 resource sets, with a total of up to 64 CSI-RS resources or up to 64 SS/PBCH Block resources. For L1-SINR reporting, if the higher layer parameter nrofReportedRS in CSI-ReportConfig is configured to be one, the reported L1-SINR value is defined by a 7-bit value in the range [-23, 40] dB with 0.5 dB step size, and if the higher layer parameter nrofReportedRS is configured to be larger than one, or if the higher layer parameter groupBasedBeamReporting is configured as 'enabled', the UE shall use differential L1-SINR based reporting, where the largest measured value of L1-SINR is quantized to a 7-bit value in the range [-23, 40] dB with 0.5 dB step size, and the differential L1-SINR is quantized to a 4-bit value. The differential L1-SINR is computed with 1 dB step size with a reference to the largest measured L1-SINR value which is part of the same L1-SINR reporting instance. When NZP CSI-RS is configured for channel measurement and/or interference measurement, the reported L1-SINR values should not be compensated by the power offset(s) given by higher layer parameter powerControOffsetSS or powerControlOffset. When one or two resource settings are configured for L1-SINR measurement - If the higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to 'notConfigured', the UE shall derive the channel measurements for computing L1-SINR reported in uplink slot n based on only the SSB or NZP CSI-RS, no later than the CSI reference resource, (defined in TS 38.211[4]) associated with the CSI resource setting. - If the higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to 'configured', the UE shall derive the channel measurements for computing L1-SINR reported in uplink slot n based on only the most recent, no later than the CSI reference resource, occasion of SSB or NZP CSI-RS (defined in [4, TS 38.211]) associated with the CSI resource setting. - If the higher layer parameter timeRestrictionForInterferenceMeasurements in CSI-ReportConfig is set to 'notConfigured', the UE shall derive the interference measurements for computing L1-SINR reported in uplink slot n based on only the CSI-IM or NZP CSI-RS for interference measurement (defined in [4, TS 38.211]) or NZP CSI-RS for channel and interference measurement no later than the CSI reference resource associated with the CSI resource setting. - If the higher layer parameter timeRestrictionForInterferenceMeasurements in CSI-ReportConfig is set to 'configured', the UE shall derive the interference measurements for computing the L1-SINR reported in uplink slot n based on the most recent, no later than the CSI reference resource, occasion of CSI-IM or NZP CSI-RS for interference measurement (defined in [4, TS 38.211]) or NZP CSI-RS for channel and interference measurement associated with the CSI resource setting. When the UE is configured with [capabilitycinIndexReporting] an index of UE capability value set, indicating the maximum supported number of SRS antenna ports, is reported along with the pair of SSBRI/CRI and L1-SINR.

- The value of N is configured by gNB and is the maximum number of reported measurements in one P-MPR report in addition to legacy P-MPR.

- In a MPE report, in addition to legacy P-MPR, the UE selects and reports up to N pairs of P-MPR(s) and SSBRI(s)/CRI(s) from a candidate SSB/CSI-RS resource pool.

- Introduce new triggering condition of P-MPR report.

  - SSBRI(s)/CRI(s) in beam report is not included in latest P-MPR report.

- Modify legacy triggering condition of P-MPR report by replacing measured P-MPR with at least one measured P-MPR.

References

Chairman’s notes for 3GPP TSG RAN WG1 #102-e Meeting.

Chairman’s notes for 3GPP TSG RAN WG1 #103-e Meeting.

Chairman’s notes for 3GPP TSG RAN WG1 #104-e Meeting.

Chairman’s notes for 3GPP TSG RAN WG1 #104b-e Meeting.

Chairman’s notes for 3GPP TSG RAN WG1 #105-e Meeting.

Chairman’s notes for 3GPP TSG RAN WG1 #106-e Meeting.

Chairman’s notes for 3GPP TSG RAN WG1 #106b-e Meeting.

Chairman’s notes for 3GPP TSG RAN WG1 #107-e Meeting.

3GPP TS 38.213 V17.0.0.

3GPP TS 38.214 V17.0.0.