3GPP TSG RAN WG1#108-e		R1-2201090

e-Meeting, February 21st – March 3rd, 2022

Source:	vivo

Title:	Remaining issues on HARQ-ACK enhancements for Rel-17 URLLC

Agenda Item:	8.3.1

Document for:	Discussion and Decision

- Introduction

During RAN1#107b-e meeting, a lot of agreements have been achieved for HARQ-ACK enhancements for Rel-17 URLLC, which are listed in Annex for reference.

In this contribution, we will discuss some remaining issues on HARQ-ACK enhancements for Rel-17 URLLC.

- Discussion

During several latest RAN1 meetings, interaction between HARQ-ACK enhancement features, including SPS HARQ-ACK deferral, enhanced Type-3 codebook, one-shot triggering of HARQ-ACK re-transmission, dynamic PUCCH cell switching, semi-static PUCCH cell switching and PUCCH repetition, has been discussed extensively. Table 1 summarizes the progress achieved so far as below.

Table 1 – Support of the joint operations of Rel-17 URLLC features

|  | SPS HARQ-ACK Deferral | Enh. Type-3 Codebook | One-shot triggering | Dynamic PUCCH Cell Switching | Semi-static PUCCH Cell Switching | PUCCH Repetition |
| --- | --- | --- | --- | --- | --- | --- |
| SPS HARQ-ACK Deferral | - | Y | Y | N | Y | N |
| Enh. Type-3 Codebook | - | - | N | Y | Y | FFS |
| One-shot triggering | - | - | - | N | Y | Y |
| Dynamic PUCCH Cell Switching | - | - | - | - | N | Y |
| Semi-static PUCCH Cell Switching | - | - | - | - |  | FFS |
| PUCCH Repetition | - | - | - | - | - | - |

Note:

- Y: means interaction between the two features is supported.

- N: means interaction between the two features is not supported.

- FFS: means no conclusion/agreement regarding interaction between the two features has been achieved, and some (further) clarifications or discussions are required to determine if interaction between the two features is supported or not.

One FFS is about interaction between enhanced Type-3 codebook and PUCCH repetition from the above table. A PUCCH resource designated for conveying an enhanced Type-3 codebook may have a repetition factor larger than 1, and no special handling is required, therefore interaction between these two features can be supported naturally without any clarification.

Observation 1: Interaction between enhanced Type-3 codebook and PUCCH repetition can be supported naturally without any clarification.

Regarding interaction between semi-static PUCCH cell switching and PUCCH repetition, it will be discussed in sub-section 2.4.1.

In the following sub-sections, remaining issues for HARQ-ACK enhancements will be discussed at first, then followed by the discussions on interaction of HARQ-ACK enhancements with Rel-17 intra-UE multiplexing, as well as with Rel-17 simultaneous PUCCH/PUSCH transmission.

  - Remaining issues for enhanced Type-3 codebook

- 2.1.1. Codebook construction

In [2], codebook determination for enhanced Type-3 codebook mainly reuses that for Rel-16 Type-3 codebook, except for some special handling, which is referenced as below for convenience.

-------------------------------------------------------Start of text in [2]-----------------------------------------------------------

9.1.4	Type-3 HARQ-ACK codebook determination

If a UE is provided pdsch-HARQ-ACK-OneShotFeedback, the UE determines o0ACK,o1ACK,…,oOACK-1ACK HARQ-ACK information bits, for a total number of OACK HARQ-ACK information bits, of a Type-3 HARQ-ACK codebook according to the following procedure. If the UE is provided pdsch-HARQ-ACK-enhType3List and a DCI format scheduling PDSCH reception and triggering the Type-3 HARQ-ACK codebook includes a Type3-subcodebook-index field that provides a value for pdsch-HARQ-ACK-enhType3Index, the UE determines a number of indicated serving cells NcellsDL,ind and a number of indicated HARQ processes NHARQ,cDL,ind for each indicated serving cell c from the entry in pdsch-HARQ-ACK-enhType3List corresponding to the pdsch-HARQ-ACK-enhType3Index value. If the DCI format does not include the Type3-subcodebook-index field, the pdsch-HARQ-ACK-enhType3Index value is zero.

Set NcellsDL to the number of configured serving cells or, when applicable, to NcellsDL,ind

Set NHARQ,cDL to the value of nrofHARQ-ProcessesForPDSCH for serving cell c, if provided; else, set NHARQ,cDL=8. When applicable, set NHARQ,cDL to NHARQ,cDL,ind

Set NTB,cDL to the value of maxNrofCodeWordsScheduledByDCI for serving cell c if harq-ACK-SpatialBundlingPUCCH is provided and NDIHARQ=0, or if harq-ACK-SpatialBundlingPUCCH is not provided, or if maxCodeBlockGroupsPerTransportBlock is provided for serving cell c; else, set NTB,cDL=1

Set NHARQ-ACK,cCBG/TB,max to the number of HARQ-ACK information bits per TB for PDSCH receptions on serving cell c as described in clause 9.1.1 if maxCodeBlockGroupsPerTransportBlock is provided for serving cell c and pdsch-HARQ-ACK-OneShotFeedbackCBG or pdsch-HARQ-ACK-enhType3CBG is provided; else, set NHARQ-ACK,cCBG/TB,max=0

Set NDIHARQ=0 if pdsch-HARQ-ACK-OneShotFeedbackNDI or pdsch-HARQ-ACK-enhType3NDI is provided; else set NDIHARQ=1

Set c=0 – serving cell index

Set h=0 – HARQ process number

Set t=0 – TB index

Set g=0 – CBG index

Set j=0

while c<NcellsDL

while h<NHARQ,cDL

……

h=h+1

end while

h=0

c=c+1

end while

……

If

-	a UE detects a DCI format that includes a One-shot HARQ-ACK request field with value 1, and

-	the CRC of the DCI is scrambled by a C-RNTI or an MCS-C-RNTI, and

-	resourceAllocation = resourceAllocationType0 and all bits of the frequency domain resource assignment field in the DCI format are equal to 0, or

-	resourceAllocation = resourceAllocationType1 and all bits of the frequency domain resource assignment field in the DCI format are equal to 1, or

-	resourceAllocation = dynamicSwitch and all bits of the frequency domain resource assignment field in the DCI format are equal to 0 or 1

the DCI format provides a request for a Type-3 HARQ-ACK codebook report and does not schedule a PDSCH reception. If the UE is provided pdsch-HARQ-ACK-enhType3List and the DCI format includes a Type3-subcodebook-index field that provides a value for pdsch-HARQ-ACK-enhType3Index, the UE determines a number of indicated serving cells NcellsDL,ind and a number of indicated HARQ processes NHARQ,cDL,ind for each indicated serving cell c from the entry in pdsch-HARQ-ACK-enhType3List corresponding to the pdsch-HARQ-ACK-enhType3Index value. If the DCI format does not include the Type3-subcodebook-index field, the pdsch-HARQ-ACK-enhType3Index value is provided by the value of MCS field in the DCI format. The UE is expected to provide HARQ-ACK information in response to the request for the Type-3 HARQ-ACK codebook after N symbols from the last symbol of a PDCCH providing the DCI format, where the value of N for μ=0,1,2 is provided in clause 10.2 by replacing "SPS PDSCH release" with "DCI format".

……

-------------------------------------------------------End of text in [2]-----------------------------------------------------------

Based on the following agreement achieved during RAN1#107-e meeting, it can be understood that serving cell indexes for the subset of serving cells, as well as HARQ process numbers for the subset of configured HARQ processes for a serving cell, involved in an enhanced Type-3 codebook, may not be consecutive. But according to the corresponding text in [2], which is listed above for convenience, the configuration of an enhanced Type-3 codebook only provides a number of indicated serving cells NcellsDL,ind and a number of indicated HARQ processes NHARQ,cDL,ind for each indicated serving cell c, and the indexes of the indicated serving cells, as well as the numbers of the indicated HARQ processes for each indicated serving cell, are regarded as consecutive, based on the text highlighted in yellow background. In other words, there is a mismatch between the agreement(s) and corresponding description in the CR.

Agreement

One enhanced Type 3 HARQ-ACK codebook is RRC configured either as:

        - a subset of CC, i.e., all HARQ processes of the subset of CCs are part of the codebook, OR

| pdsch-HARQ-ACK-enhType3perCC | Configure the one enhanced Type 3 HARQ-ACK codebook using per CC configuration | (1..maxNrofServingCells) of Integer (0,1) |
| --- | --- | --- |

        - a subset of configured HARQ processes per CC, i.e., different subsets of HARQ processes can be configured for each CC.

| pdsch-HARQ-ACK-enhType3perHARQ | Configure the one enhanced Type 3 HARQ-ACK codebook using a per HARQ process and CC configuration | (1..maxNrofServingCells) of Bit String (Size (16)) |
| --- | --- | --- |

To resolve the above issue, the following TP can be considered, with the modified text in red.

-------------------------------------------------------Start text proposal-----------------------------------------------------------

If a UE is provided pdsch-HARQ-ACK-OneShotFeedback, the UE determines o0ACK,o1ACK,…,oOACK-1ACK HARQ-ACK information bits, for a total number of OACK HARQ-ACK information bits, of a Type-3 HARQ-ACK codebook according to the following procedure. If the UE is provided pdsch-HARQ-ACK-enhType3List and a DCI format scheduling PDSCH reception and triggering the Type-3 HARQ-ACK codebook includes a Type3-subcodebook-index field that provides a value for pdsch-HARQ-ACK-enhType3Index, the UE determines for each configured serving cell c if at least one configured HARQ process for it is involved in the Type-3 HARQ-ACK codebook determination, and when applicable, determines if HARQ process number h for serving cell c is involved in the Type-3 HARQ-ACK codebook determination,  a number of indicated serving cells NcellsDL,ind and a number of indicated HARQ processes NHARQ,cDL,ind for each indicated serving cell c from the entry in pdsch-HARQ-ACK-enhType3List corresponding to the pdsch-HARQ-ACK-enhType3Index value. If the DCI format does not include the Type3-subcodebook-index field, the pdsch-HARQ-ACK-enhType3Index value is zero.

Set NcellsDL to the number of configured serving cells or, when applicable, to NcellsDL,ind

Set NHARQ,cDL to the value of nrofHARQ-ProcessesForPDSCH for serving cell c, if provided; else, set NHARQ,cDL=8. When applicable, set NHARQ,cDL to NHARQ,cDL,ind

Set NTB,cDL to the value of maxNrofCodeWordsScheduledByDCI for serving cell c if harq-ACK-SpatialBundlingPUCCH is provided and NDIHARQ=0, or if harq-ACK-SpatialBundlingPUCCH is not provided, or if maxCodeBlockGroupsPerTransportBlock is provided for serving cell c; else, set NTB,cDL=1

Set NHARQ-ACK,cCBG/TB,max to the number of HARQ-ACK information bits per TB for PDSCH receptions on serving cell c as described in clause 9.1.1 if maxCodeBlockGroupsPerTransportBlock is provided for serving cell c and pdsch-HARQ-ACK-OneShotFeedbackCBG or pdsch-HARQ-ACK-enhType3CBG is provided; else, set NHARQ-ACK,cCBG/TB,max=0

Set NDIHARQ=0 if pdsch-HARQ-ACK-OneShotFeedbackNDI or pdsch-HARQ-ACK-enhType3NDI is provided; else set NDIHARQ=1

Set c=0 – serving cell index

Set h=0 – HARQ process number

Set t=0 – TB index

Set g=0 – CBG index

Set j=0

while c<NcellsDL

if the UE is not provided pdsch-HARQ-ACK-enhType3List, or, if the UE is provided pdsch-HARQ-ACK-enhType3List and the UE determines that for serving cell c at least one configured HARQ process is involved in the Type-3 HARQ-ACK codebook determination

while h<NHARQ,cDL

if the UE is not provided pdsch-HARQ-ACK-enhType3List, or, if the UE is provided pdsch-HARQ-ACK-enhType3List and the UE determines that HARQ process number h for serving cell c is involved in the Type-3 HARQ-ACK codebook determination

……

end if

h=h+1

end while

end if

h=0

c=c+1

end while

……

If

-	a UE detects a DCI format that includes a One-shot HARQ-ACK request field with value 1, and

-	the CRC of the DCI is scrambled by a C-RNTI or an MCS-C-RNTI, and

-	resourceAllocation = resourceAllocationType0 and all bits of the frequency domain resource assignment field in the DCI format are equal to 0, or

-	resourceAllocation = resourceAllocationType1 and all bits of the frequency domain resource assignment field in the DCI format are equal to 1, or

-	resourceAllocation = dynamicSwitch and all bits of the frequency domain resource assignment field in the DCI format are equal to 0 or 1

the DCI format provides a request for a Type-3 HARQ-ACK codebook report and does not schedule a PDSCH reception. If the UE is provided pdsch-HARQ-ACK-enhType3List and the DCI format includes a Type3-subcodebook-index field that provides a value for pdsch-HARQ-ACK-enhType3Index, the UE determines for each configured serving cell c if at least one configured HARQ process for it is involved in the Type-3 HARQ-ACK codebook determination, and when applicable, determines if HARQ process number h for serving cell c is involved in the Type-3 HARQ-ACK codebook determination,a number of indicated serving cells NcellsDL,ind and a number of indicated HARQ processes NHARQ,cDL,ind for each indicated serving cell c from the entry in pdsch-HARQ-ACK-enhType3List corresponding to the pdsch-HARQ-ACK-enhType3Index value. If the DCI format does not include the Type3-subcodebook-index field, the pdsch-HARQ-ACK-enhType3Index value is provided by the value of MCS field in the DCI format.  The UE is expected to provide HARQ-ACK information in response to the request for the Type-3 HARQ-ACK codebook after N symbols from the last symbol of a PDCCH providing the DCI format, where the value of N for μ=0,1,2 is provided in clause 10.2 by replacing "SPS PDSCH release" with "DCI format".

……

--------------------------------------------------------End text proposal----------------------------------------------------------

Proposal 1: Consider the text proposal for enhanced Type-3 codebook determination in TS38.213.

  - Remaining issues for one-shot triggering of HARQ-ACK re-transmission

- 2.2.1. UE behavior when no original codebook is generated

During RAN1#107b-e meeting, the following conclusion was achieved for HARQ-ACK codebook size indication in a triggering DCI.

Conclusion

There is no consensus on the support of HARQ-ACK CB size indication in the triggering DCI for HARQ-ACK re-transmission

Since no HARQ-ACK codebook size indication will be included in a triggering DCI, it is better to clarify UE behavior when the UE did not generate a HARQ-ACK codebook with the indicated PHY priority for corresponding PUCCH transmission in the original PUCCH slot.

During RAN1#107b-e meeting the corresponding UE behavior has been discussed without an explicit conclusion, and the proposed conclusion during the discussion is listed as below.

Proposal 3.9.1: If HARQ-ACK CB size indication in the triggering DCI for HARQ-ACK re-transmission is NOT supported, take the following conclusion:

- Proposed Conclusion: The UE ignores the one-shot triggering DCI for new PUCCH determination if it schedules a re-transmission of HARQ-ACK information from a PUCCH slot/sub-slot on which the UE did not generate a valid HARQ-ACK CB

In our opinion, it is natural for the UE to ignore the triggering DCI, since no original HARQ-ACK codebook is available for re-transmission, and the UE cannot generate required NACK sequence due to no explicit HARQ-ACK codebook size indication as well.

Specifically, when the UE receives a triggering DCI, and did not generate an HARQ-ACK codebook with the indicated PHY priority for corresponding PUCCH transmission in the original PUCCH slot, the triggering DCI will be discarded as if the UE did not receive the triggering DCI. As a result, no corresponding PUCCH transmission will be determined in the PUCCH slot designated for HARQ-ACK re-transmission.

We are fine to clarify the above UE behavior as a conclusion.

Proposal 2: Clarify the following as a conclusion:

- When a UE receives a one-shot triggering DCI for HARQ-ACK re-transmission, and did not generate an HARQ-ACK codebook with the indicated PHY priority for corresponding PUCCH transmission in the original PUCCH slot, the UE ignores the triggering DCI, without determining corresponding PUCCH transmission in the PUCCH slot designated for HARQ-ACK re-transmission.

  - Remaining issues for dynamic PUCCH cell switching

- 2.3.1. Handling of SPS HARQ-ACK

When dynamic PUCCH cell switching is enabled, how to report HARQ-ACK corresponding to an SPS activation DCI, as well as SPS HARQ-ACK (i.e. HARQ-ACK of SPS PDSCH without associated DCI) corresponding to the SPS activation DCI, was discussed extensively during RAN1#107b-e meeting without any conclusion/decision yet. The latest proposal for this issue in the end of RAN1#107b-e meeting is listed as below for convenience.

Alternative Proposal 5 (from HW): For dynamic PUCCH cell switching, the HARQ-ACK of SPS PDSCH without associated DCI is to be transmitted on PCell / SPCell / PUCCH-SCell independently of the dynamically indicated PUCCH cell in the SPS activation DCI. Only the HARQ-ACK corresponding to the first SPS PDSCH activated by Activation DCI uses the PUCCH cell based on the indication in the activation DCI (based on the earlier agreement).

- The UE does not expected to be dynamically indicated for PUCCH transmission on the PUCCH sSCell in the SPS activation DCI in a slot overlapping with a PCell / SPCell / PUCCH-SCell slot where SPS HARQ-ACK subject to another SPS config would be transmitted.

It is common understanding that for dynamic PUCCH cell switching, SPS HARQ-ACK is always to be transmitted on the PCell/PSCell/PUCCH-SCell.

In addition, it was agreed during RAN1#106-e meeting that HARQ-ACK corresponding to the PDSCH scheduled by an SPS activation DCI is reported on the PUCCH cell indicated by the SPS activation DCI.

Agreement

In addition to HARQ-Ack of PDSCH dynamically scheduled by a DCI indicating a PUCCH carrier, the dynamic target carrier indication also applies to:

- HARQ-ACK corresponding to the first SPS PDSCH activated by Activation DCI based on the indication in the activation DCI

- HARQ-ACK corresponding to the SPS Release DCI based on the indication in the release DCI

- triggered PUCCH for Rel-16 Type 3 CB, Rel-17 enh. Type 3 CB of smaller size and Rel-17 one-shot triggering for HARQ-Ack retransmission based on the indication in the triggering DCI

- FFS: Additional cases

The controversial points for now are whether an SPS activation DCI (when it contains a field for indicating the target PUCCH cell) should always indicate the PCell/PSCell/PUCCH-SCell as the target PUCCH cell , and whether it is necessary to use the same PUCCH cell to transmit the HARQ-ACK corresponding to the PDSCH scheduled by an SPS activation DCI and  SPS HARQ-ACK corresponding to the SPS activation DCI.

In our opinion, in Rel-15/16, the first PDSCH or PUSCH corresponding to an SPS activation DCI or Type 2 CG activation DCI is always regarded as dynamically scheduled PDSCH/PUSCH by the activation DCI. Therefore, it is also natural that HARQ-ACK corresponding to the PDSCH scheduled by an SPS activation DCI is regarded as dynamically scheduled HARQ-ACK, and should benefit from dynamic PUCCH cell switching. In other words, an SPS activation DCI, when applicable, can indicate the PUCCH sSCell as the target PUCCH cell as required, for flexibility or shorter latency.

Nevertheless, based on the following conclusion achieved during RAN1#107-e meeting, it is reasonable to avoid HARQ-ACK corresponding the PDSCH scheduled by an SPS activation DCI to be reported on the PUCCH sSCell and in a PUCCH slot overlapping with a PUCCH slot on the PCell/PSCell/PUCCH-SCell where SPS HARQ-ACK corresponding to another SPS Config, as well as other UCI, if any, would be transmitted.

Conclusion

For dynamic PUCCH cell switching, the UE does not expect a PUCCH slot with UCI on PCell /SPCell / PUCCH SCell to overlap with a PUCCH slot with HARQ-ACK on the dynamically indicated alternative PUCCH PUCCH cell.

- The UCI on PCell /SPCell / PUCCH SCell dropped due to collision with semi-static DL symbols, SSB, and symbols indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set is exempted and is not multiplexed on the PUCCH on the alternative PUCCH cell.

Therefore, we support the latest proposal made in the end of RAN1#107b-e meeting.

Proposal 3: For dynamic PUCCH cell switching, SPS HARQ-ACK is always transmitted on the PCell/PSCell/PUCCH-SCell, and HARQ-ACK corresponding to the PDSCH scheduled by an SPS activation DCI is reported on the target PUCCH cell indicated by the SPS activation DCI.

- The UE does not expect to be dynamically indicated for PUCCH transmission on the PUCCH sSCell in the SPS activation DCI in a PUCCH slot overlapping with a PUCCH slot on the PCell/PSCell/PUCCH-SCell where SPS HARQ-ACK corresponding to another SPS Config, as well as other UCI, if any, would be transmitted.

When HARQ-ACK corresponding to the PDSCH scheduled by an SPS activation DCI is reported on the PUCCH sSCell, how to determine the applicable K1 value for the PDSCH, as well as other SPS PDSCH(s) corresponding to the SPS activation DCI, was also discussed during RAN1#107b-e meeting, with the corresponding proposal listed as below.

Proposal 6.3.1: For dynamic PUCCH cell switching, if the HARQ-ACK for the first SPS PDSCH is indicated on the PUCCH sSCell based on the activation DCI,

- the UE determines for the first SPS PDSCH a k1 value from the PUCCH sSCell’s K1 set according to the K1 indicator field in the activation DCI

- the UE determines for the other SPS PDSCHs without associated DCI a k1 value from PCell’s K1 set according to the K1 indicator field in the activation DCI

The above proposal is straightforward and reasonable from our perspective. Since all applicable K1 values are based on the K1 indicator field in the SPS activation DCI, it is still under the gNB’s control with sufficient flexibility. In addition, the applicable K1 value on a PUCCH cell is always contained in the K1 set for the PUCCH cell, thus legacy Type-1 codebook construction can be reused without special handling.

Proposal 4: For dynamic PUCCH cell switching, when HARQ-ACK corresponding to the PDSCH scheduled by an SPS activation DCI is indicated to be reported on the PUCCH sSCell, the K1 value for SPS PDSCH(s) corresponding to the SPS activation DCI is determined based on the K1 indicator field in the SPS activation DCI, as well as the K1 set for the PCell/PSCell/PUCCH-SCell.

- 2.3.2. Handling of SR/CSI transmission

For SR, an SR configuration may be associated to zero, one or more SR resource configurations, each of which is configured on a UL BWP of a respective PUCCH cell. When this SR configuration is triggered, PUCCH resource(s) of the associated SR resource configuration(s) on corresponding PUCCH cell(s) will be used.

For CSI, a CSI report configuration for periodic or semi-persistent CSI reporting on PUCCH may be configured on a PUCCH cell, and CSI reporting for the CSI report configuration will be performed on the PUCCH cell.

When dynamic PUCCH cell switching is enabled, PUCCH may be transmitted on the PUCCH sSCell, in addition to the PCell/PSCell/PUCCH-SCell. Based on agreements and conclusions achieved so far, only dynamically scheduled HARQ-ACK (i.e. HARQ-ACK in response to a DCI) can be transmitted on the PUCCH sSCell. It should be clarified further whether SR resource configuration(s) and/or CSI report configuration(s) can be configured for the PUCCH sSCell.

In Rel-17 maintenance phase, SR resource configuration(s) and/or CSI report configuration(s) should not be configured, i.e. in the PUCCH Config(s), for the PUCCH sSCell for simplicity from our perspective. Otherwise, to avoid simultaneous PUCCH transmissions on different PUCCH cells, as well as UCI multiplexing across different PUCCH cells, many efforts may be expected with marginal benefit.

Proposal 5: For dynamic PUCCH cell switching, further clarify that SR resource configuration(s) and/or CSI report configuration(s) cannot be configured in the PUCCH Config(s) for the PUCCH sSCell.

  - Remaining issues for semi-static PUCCH cell switching

- 2.4.1. Interaction with PUCCH repetition

Since RAN1#106b-e meeting, joint operation between semi-static PUCCH cell switching and PUCCH repetition has been discussed for several times, with no agreement achieved so far. The latest version of corresponding proposal formulated by the moderator during RAN1#107b-e meeting is listed as below for convenience.

Mod Proposal 11: For semi-static PUCCH cell switching, a PUCCH repetition transmission on a different target PUCCH cell from the PUCCH cell of the first PUCCH repetition is not supported

- A PUCCH slot mapped to different PUCCH cell is considered as invalid for PUCCH repetition and is not counted towards the total number of PUCCH repetitions / the repetition is post-poned as in Rel-16. Example figure for 4 repetitions:

For a single PUCCH resource, if at least one repetition is transmitted on a different PUCCH cell with potential different PUCCH configurations, it may be hard to combine corresponding repetitions for PUCCH detection. To avoid this undesirable case and corresponding complexity, a PUCCH repetition transmission on a different target PUCCH cell from the PUCCH cell of the first PUCCH repetition should not be supported. As a result, a PUCCH repetition mapped to a different target PUCCH cell from the PUCCH cell of the first PUCCH repetition can be dropped, and counted towards the total number of PUCCH repetitions, for simplicity and to keep the latency independent of the time domain pattern. Alternatively, a PUCCH slot mapped to a different target PUCCH cell can be considered as invalid for PUCCH repetition, and not counted towards the total number of PUCCH repetitions, which is exactly the intention of the above proposal, where the reliability can be guaranteed, but the latency may be extended depending on the time domain pattern.

Proposal 6: For semi-static PUCCH cell switching in conjunction with PUCCH repetition, a PUCCH repetition mapping to a different target PUCCH cell from the target PUCCH cell of the first PUCCH repetition is not supported.

- 2.4.2. Handling of SPS HARQ-ACK

For semi-static PUCCH cell switching, SPS HARQ-ACK is expected to be reported on a PUCCH cell determined based on the time domain pattern configured for a PUCCH cell group.

For each SPS Config, SPS HARQ-ACK PUCCH resource can be configured by SPS-Config->n1PUCCH-AN. Alternatively, a list of SPS HARQ-ACK PUCCH resources can be configured for all SPS HARQ-ACK transmitted on a PUCCH cell by PUCCH-Config->sps-PUCCH-AN-List-r16.

When SPS-Config->n1PUCCH-AN is configured for an SPS Config, in Rel-15/16 it can identify a unique PUCCCH resource on the unique PUCCH cell in the PUCCH cell group to which the serving cell of the SPS Config belongs. When SPS HARQ-ACK can be transmitted on the PCell/PSCell/PUCCH-SCell or the alternative PUCCH sSCell based on the time domain pattern, there are potentially two PUCCH cells providing SPS HARQ-ACK PUCCH resources for an SPS Config, how to understand or extend SPS-Config->n1PUCCH-AN should be determined. Two alternatives can be identified, described as below.

- Alt. 1: SPS-Config->n1PUCCH-AN corresponds to a configured PUCCH resource on the PCell/PSCell/PUCCH-SCell, as well as a configured PUCCH resource on the PUCCH sSCell. The two configured PUCCH resources has the same resource ID but in different PUCCH cells. It is up to gNB implementation to guarantee that the configured PUCCH resource(s), when to be used, is valid.

- Alt. 2: SPS-Config->n1PUCCH-AN can be extended so that two resource IDs can be configured independently for the PCell/PSCell/PUCCH-SCell and the PUCCH sSCell, respectively. For example, legacy parameter n1PUCCH-AN can be used to indicate the ID of a configured PUCCH resource on the PCell/PSCell/PUCCH-SCell, and a new parameter can be introduced to indicate the ID of a configured PUCCH resource on the PUCCH sSCell.

Alt. 1 can keep legacy parameter list unchanged, but the understanding or description for specific parameter(s) should be updated. In contrast, Alt. 2 will extend the parameter list, but has more flexibility.

Proposal 7: For semi-static PUCCH cell switching, where there are potentially two PUCCH cells providing SPS HARQ-ACK PUCCH resources for an SPS Config, how to understand or extend SPS-Config->n1PUCCH-AN should be determined, and the following two alternatives can be considered:

- Alt. 1: SPS-Config->n1PUCCH-AN corresponds to a configured PUCCH resource on the PCell/PSCell/PUCCH-SCell, as well as a configured PUCCH resource on the PUCCH sSCell, where the two configured PUCCH resources have the same resource ID.

- Alt. 2: SPS-Config->n1PUCCH-AN can be extended so that two resource IDs can be configured independently for the PCell/PSCell/PUCCH-SCell and the PUCCH sSCell, respectively.

- 2.4.3. Handling of SR/CSI transmission

During RAN1#107b-e meeting, the following proposal was discussed briefly without any decision yet.

Proposal 6.2.4: When CSI reporting on PUCCH is configured on both PCell/PScell/PUCCH-Scell and alternative PUCCH sScell, the PUCCH cell pattern is applied to determine whether CSI PUCCH will be transmitted or not.

In our opinion, according to the following agreement achieved during RAN1#106-e meeting, how to report CSI, as well as SR, should be determined when semi-static PUCCH cell switching is enabled.

Agreement

Semi-static PUCCH carrier switching is applicable to all UCI types incl. HARQ-ACK, SR and CSI.

As described in sub-section 2.3.2, SR resource configuration(s), as well as CSI report configuration(s), can be configured for any PUCCH cell, i.e. the PCell/PSCell/PUCCH-SCell and/or the PUCCH sSCell.

Regarding SR transmission, when an SR configuration is triggered, PUCCH resource(s) of the associated SR resource configuration(s) on corresponding PUCCH cell(s) will be validated based on the time domain pattern. In other words, for a PUCCH slot of a PUCCH cell where one or more SR PUCCH resources of the PUCCH cell corresponding to the triggered SR configuration locate, if the PUCCH cell is not the applicable target PUCCH cell in the PUCCH slot based on the time domain pattern, the one or more SR PUCCH resources are invalid, or corresponding SR PUCCH transmission(s) will be skipped.

Regarding CSI transmission, similarly, for a PUCCH slot of a PUCCH cell where a CSI PUCCH resource of the PUCCH cell corresponding to a CSI report configuration of periodic CSI reporting or semi-persistent CSI reporting on PUCCH locates, if the PUCCH cell is not the applicable target PUCCH cell in the PUCCH slot based on the time domain pattern, the CSI PUCCH resource is invalid, or corresponding SR PUCCH transmission will be skipped.

Proposal 8: For semi-static PUCCH cell switching, when an SR configuration is triggered, PUCCH resource(s) of the associated SR resource configuration(s) on corresponding PUCCH cell(s) will be validated based on the time domain pattern.

Proposal 9: For semi-static PUCCH cell switching, when CSI reporting on PUCCH is configured on both the PCell/PSCell/PUCCH-SCell and the PUCCH sSCell, corresponding CSI PUCCH resource(s) on a PUCCH cell will be validated based on the time domain pattern.

If companies think the related behaviors in the above two proposals has been agreed or are clear enough based on the agreement cited above, we are also fine to clarify these behaviors in corresponding conclusions.

  - Interaction with Rel-17 intra-UE multiplexing

Regarding interaction of HARQ-ACK enhancements with Rel-17 intra-UE multiplexing, each feature in conjunction with Rel-17 intra-UE multiplexing will be analysed in the following.

- Interaction of SPS HARQ-ACK deferral with Rel-17 intra-UE multiplexing

Interaction of SPS HARQ-ACK deferral with Rel-17 intra-UE multiplexing was discussed during RAN1#107-e meeting without conclusion/decision yet, and the latest proposal is referenced as below for convenience. It was deprioritized during RAN1#107b-e meeting as well, to wait for more progress in AI 8.3.3.

Proposal 2.3.2: Support simultaneous configuration of Rel-17 intra-UE multiplexing of different priorities and SPS HARQ-ACK deferral

- If after the Rel-17 multiplexing operation into a PUCCH or PUSCH if any, and if the UE would be transmitting SPS HARQ-ACK using the PUCCH SPS-PUCCH-AN-List-r16 or n1PUCCH-AN,

  - which is not valid in the initial slot, the SPS HARQ-ACK configured for deferral is deferred; and

  - which is valid in a next PUCCH slot, the next PUCCH slot is determined as target PUCCH slot.

- For the target slot determination, LP SPS HARQ-ACK is considered as LP HARQ-ACK payload and HP SPS HARQ-ACK for deferral is treated as HP HARQ-ACK playload for the target slot determination using the Rel-17 intra-UE multiplexing framework.

Besides, a conclusion regarding PHY priorities was achieved during RAN1#106b-e meeting, listed as below.

Conclusion

If the UE is not configured with Rel-17 Intra-UE multiplexing, SPS HARQ for deferral of different PHY priorities can be separately deferred with the target PUCCHs separately determinated according to their respective PHY priorities.

- FFS on the PHY priority handling for SPS HARQ deferral if the UE configured with Rel-17 Intra-UE multiplexing

In our opinion, simultaneous configuration of Rel-17 intra-UE multiplexing of different PHY priorities and SPS HARQ-ACK deferral can be supported with minor clarification.

If after performing the Rel-17 intra-UE multiplexing operation into a PUCCH or PUSCH if any, and if the UE would be transmitting SPS HARQ-ACK using the HARQ-ACK PUCCH resource configured by sps-PUCCH-AN-List-r16 or n1PUCCH-AN which is not valid in the initial PUCCH slot, the SPS HARQ-ACK configured for deferral is deferred.

When a deferral is triggered, SPS HARQ-ACK configured for deferral and with different PHY priorities can be separately deferred to determine a corresponding target PUCCH slot.

In a candidate target PUCCH slot for deferred SPS HARQ-ACK of a PHY priority, if after performing the Rel-17 intra-UE multiplexing operation into a PUCCH or PUSCH if any, the UE would be either (i) transmitting HARQ-ACK including the deferred SPS HARQ-ACK using a PUCCH/PUSCH other than an HARQ-ACK PUCCH resource configured by sps-PUCCH-AN-List-r16 or n1PUCCH-AN or (ii) transmitting HARQ-ACK including the deferred SPS HARQ-ACK using a PUCCH resource configured by sps-PUCCH-AN-List-r16 or n1PUCCH-AN being regarded as valid, the candidate target PUCCH slot is regarded as the target PUCCH slot determined for the deferred SPS HARQ-ACK of the PHY priority, where the deferred SPS HARQ-ACK will not be deferred further. Note when deferred SPS HARQ-ACK of two PHY priorities are involved in the above case (i) or case (ii), the target PUCCH slot for deferred SPS HARQ-ACK of each PHY priority is determined respectively at the same time. It can also be understood that PUCCH slots for different PHY priorities may has different length.

Proposal 10: Support simultaneous configuration of Rel-17 intra-UE multiplexing of different PHY priorities and SPS HARQ-ACK deferral.

- Interaction of enhanced Type-3 codebook with Rel-17 intra-UE multiplexing

When an enhanced Type-3 codebook is triggered by a DCI format, the DCI format will indicate the PHY priority for the enhanced Type-3 codebook, which is also used to determine the PHY priority of the PUCCH transmission conveying the enhanced Type-3 codebook. In this regard, configuration of enhanced Type-3 codebook does not has any impact on Rel-17 intra-UE multiplexing. Therefore, simultaneous configuration of these two features can be supported naturally without any clarification.

Proposal 11: Support simultaneous configuration of Rel-17 intra-UE multiplexing of different PHY priorities and enhanced Type-3 codebook.

- Interaction of one-shot triggering with Rel-17 intra-UE multiplexing

Regarding one-shot triggering, the following agreements have been achieved.

Agreement

A single DCI triggering the Rel-17 one-shot triggering (by a DL assignment) of HARQ-ACK re-transmission on a PUCCH resource other than enhanced Type 2 or (enhanced) Type 3 HARQ-ACK CB can trigger the re-transmission of HARQ-ACK information of only a single HARQ-ACK CB.

Agreement

For Rel-17 one-shot triggering for HARQ-ACK re-transmission, the UE does not expect more than one triggering DCI for Rel-17 one-shot feedback indicating the same PUCCH slot for the re-transmission of HARQ-ACK CBs of different PUCCH slots to be re-transmitted

- Note: i.e. only a single HARQ-ACK codebook / PUCCH occasion can be re-transmitted in a PUCCH slot

Agreement

Support PHY priority handling for the Rel-17 one-shot triggering (by a DL assignment) of HARQ-ACK re-transmission on a PUCCH resource other than enhanced Type 2 or (enhanced) Type 3 HARQ-ACK CB.

- The indicated PHY priority in the triggering DCI defines the PHY priority of the PUCCH carrying the re-transmitted HARQ-ACK information.

- The indicated PHY priority in the triggering DCI is used to determine the HARQ-ACK information to be re-transmitted corresponding to the indicated PHY priority.

Based on the above agreements, a triggering DCI can only trigger a single HARQ-ACK codebook of a PHY priority, and one-shot triggering cannot change the PHY priority of an HARQ-ACK codebook. Furthermore, in a PUCCH slot of a PHY priority, at most one re-transmitted HARQ-ACK codebook of the PHY priority can be triggered to be reported in the PUCCH slot.

In our opinion, even if a HP HARQ-ACK codebook and a LP HARQ-ACK codebook are multiplexed into a PUCCH or PUSCH based on the rules of Rel-17 intra-UE multiplexing, when a re-transmission of either of the two HARQ-ACK codebooks is required, gNB can simply issue a triggering DCI indicating the PHY priority of the HARQ-ACK codebook required to be re-transmitted, and indicating the original PUCCH conveying the HARQ-ACK codebook by HARQ_retx_offset. If both of the two HARQ-ACK codebooks are required to be re-transmitted, gNB can simply issue two separate triggering DCIs, from which one is used for each HARQ-ACK codebook, respectively.

With the above understanding, simultaneous configuration of Rel-17 intra-UE multiplexing of different PHY priorities and one-shot triggering can be supported naturally.

Proposal 12: Support simultaneous configuration of Rel-17 intra-UE multiplexing of different PHY priorities and one-shot triggering.

- Interaction of dynamic PUCCH cell switching with Rel-17 intra-UE multiplexing

With the following conclusion achieved during RAN1#107-e meeting, UCI multiplexing across different PUCCH cells is avoided. Therefore, simultaneous configuration of Rel-17 intra-UE multiplexing of different PHY priorities and dynamic PUCCH cell switching can be supported without any additional complexity.

Conclusion

For dynamic PUCCH cell switching, the UE does not expect a PUCCH slot with UCI on PCell /SPCell / PUCCH SCell to overlap with a PUCCH slot with HARQ-ACK on the dynamically indicated alternative PUCCH PUCCH cell.

- The UCI on PCell /SPCell / PUCCH SCell dropped due to collision with semi-static DL symbols, SSB, and symbols indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set is exempted and is not multiplexed on the PUCCH on the alternative PUCCH cell.

Proposal 13: Support simultaneous configuration of Rel-17 intra-UE multiplexing of different PHY priorities and dynamic PUCCH cell switching.

- Interaction of semi-static PUCCH cell switching with Rel-17 intra-UE multiplexing

For semi-static PUCCH cell switching, PUCCH transmission(s) is(are) performed on at most one PUCCH cell at any time based on the time domain pattern. Therefore, Rel-17 intra-UE multiplexing of different PHY priorities can be performed on the single PUCCH cell determined by the time domain pattern without any additional complexity.

Proposal 14: Support simultaneous configuration of Rel-17 intra-UE multiplexing of different PHY priorities and semi-static PUCCH cell switching.

  - Interaction with Rel-17 simultaneous PUCCH/PUSCH transmission

During RAN1#107b-e meeting, the following agreement was achieved for simultaneous PUCCH/PUSCH transmission. With respect to SPS HARQ-ACK deferral, a PUSCH that can be simultaneously transmitted with a PUCCH conveying SPS HARQ-ACK in initial/intermediate/target PUCCH slot is excluded from overlapping channels for multiplexing the UCI of the PUCCH and for intra-UE prioritization with the PUCCH. With this understanding, simultaneous configuration of Rel-17 simultaneous PUCCH/PUSCH transmission and SPS HARQ-ACK deferral can be supported naturally.

Agreement

If the simultaneous PUCCH/PUSCH transmission is enabled, a PUSCH that can be simultaneously transmitted with a PUCCH is excluded from overlapping channels for multiplexing the UCI of the PUCCH and for intra-UE prioritization with the PUCCH.

- Note: For intra-UE multiplexing, above is for step 2-2. For intra-UE prioritization, above is applied after step 1.

- FFS: How to capture this in the specifications

Regarding re-transmission of cancelled HARQ-ACK, including enhanced Type-3 codebook triggering and one-shot triggering, it mainly deals with PUCCH(s) conveying HARQ-ACK, and no additional specification effort can be seen when configured jointly with Rel-17 simultaneous PUCCH/PUSCH transmission.

Regarding PUCCH cell switching, including dynamic PUCCH cell switching and semi-static PUCCH cell switching, based on agreements/conclusions achieved so far, it can also be configured jointly with Rel-17 simultaneous PUCCH/PUSCH transmission without additional complexity.

In summary, each feature of HARQ-ACK enhancements can be configured jointly with Rel-17 simultaneous PUCCH/PUSCH transmission without additional complexity.

Observation 2: Each feature of HARQ-ACK enhancements can be configured jointly with Rel-17 simultaneous PUCCH/PUSCH transmission without additional complexity.

- Conclusion

In this contribution, some remaining issues on HARQ-ACK enhancements for URLLC Rel-17 have been discussed, and the proposals and observations made are summarized as below:

Observation 1: Interaction between enhanced Type-3 codebook and PUCCH repetition can be supported naturally without any clarification.

Proposal 1: Consider the text proposal for enhanced Type-3 codebook determination in TS38.213.

Proposal 2: Clarify the following as a conclusion:

- When a UE receives a one-shot triggering DCI for HARQ-ACK re-transmission, and did not generate an HARQ-ACK codebook with the indicated PHY priority for corresponding PUCCH transmission in the original PUCCH slot, the UE ignores the triggering DCI, without determining corresponding PUCCH transmission in the PUCCH slot designated for HARQ-ACK re-transmission.

Proposal 3: For dynamic PUCCH cell switching, SPS HARQ-ACK is always transmitted on the PCell/PSCell/PUCCH-SCell, and HARQ-ACK corresponding to the PDSCH scheduled by an SPS activation DCI is reported on the target PUCCH cell indicated by the SPS activation DCI.

- The UE does not expect to be dynamically indicated for PUCCH transmission on the PUCCH sSCell in the SPS activation DCI in a PUCCH slot overlapping with a PUCCH slot on the PCell/PSCell/PUCCH-SCell where SPS HARQ-ACK corresponding to another SPS Config, as well as other UCI, if any, would be transmitted.

Proposal 4: For dynamic PUCCH cell switching, when HARQ-ACK corresponding to the PDSCH scheduled by an SPS activation DCI is indicated to be reported on the PUCCH sSCell, the K1 value for SPS PDSCH(s) corresponding to the SPS activation DCI is determined based on the K1 indicator field in the SPS activation DCI, as well as the K1 set for the PCell/PSCell/PUCCH-SCell.

Proposal 5: For dynamic PUCCH cell switching, further clarify that SR resource configuration(s) and/or CSI report configuration(s) cannot be configured in the PUCCH Config(s) for the PUCCH sSCell.

Proposal 6: For semi-static PUCCH cell switching in conjunction with PUCCH repetition, a PUCCH repetition mapping to a different target PUCCH cell from the target PUCCH cell of the first PUCCH repetition is not supported.

Proposal 7: For semi-static PUCCH cell switching, where there are potentially two PUCCH cells providing SPS HARQ-ACK PUCCH resources for an SPS Config, how to understand or extend SPS-Config->n1PUCCH-AN should be determined, and the following two alternatives can be considered:

- Alt. 1: SPS-Config->n1PUCCH-AN corresponds to a configured PUCCH resource on the PCell/PSCell/PUCCH-SCell, as well as a configured PUCCH resource on the PUCCH sSCell, where the two configured PUCCH resources have the same resource ID.

- Alt. 2: SPS-Config->n1PUCCH-AN can be extended so that two resource IDs can be configured independently for the PCell/PSCell/PUCCH-SCell and the PUCCH sSCell, respectively.

Proposal 8: For semi-static PUCCH cell switching, when an SR configuration is triggered, PUCCH resource(s) of the associated SR resource configuration(s) on corresponding PUCCH cell(s) will be validated based on the time domain pattern.

Proposal 9: For semi-static PUCCH cell switching, when CSI reporting on PUCCH is configured on both the PCell/PSCell/PUCCH-SCell and the PUCCH sSCell, corresponding CSI PUCCH resource(s) on a PUCCH cell will be validated based on the time domain pattern.

Proposal 10: Support simultaneous configuration of Rel-17 intra-UE multiplexing of different PHY priorities and SPS HARQ-ACK deferral.

Proposal 11: Support simultaneous configuration of Rel-17 intra-UE multiplexing of different PHY priorities and enhanced Type-3 codebook.

Proposal 12: Support simultaneous configuration of Rel-17 intra-UE multiplexing of different PHY priorities and one-shot triggering.

Proposal 13: Support simultaneous configuration of Rel-17 intra-UE multiplexing of different PHY priorities and dynamic PUCCH cell switching.

Proposal 14: Support simultaneous configuration of Rel-17 intra-UE multiplexing of different PHY priorities and semi-static PUCCH cell switching.

Observation 2: Each feature of HARQ-ACK enhancements can be configured jointly with Rel-17 simultaneous PUCCH/PUSCH transmission without additional complexity.

- References

- RP-201310, Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR, July, 2020.

- TS 38.213, Physical layer procedures for control, v17.0.0, Dec. 2021.

- Annex

Agreements achieved during RAN1#107b-e meeting are listed as follows.

SPS HARQ-ACK deferral:

Conclusion

There is no consensus to support SPS HARQ-ACK deferral for half-duplex CA UEs in Rel-17.

Conclusion

There is no consensus to support joint operation of SPS HARQ-ACK deferral and PUCCH repetition in Rel-17.

Conclusion

There is no consensus to support joint configuration of PUCCH cell switching based on dynamic indication and SPS HARQ-ACK deferral in Rel-17.

Conclusion

The operation of simultaneous configuration of Rel-16 Type 3 HARQ-ACK codebook or enhanced Type 3 HARQ-ACK codebook triggering and SPS HARQ-ACK deferral is further clarified as:

- If the UE detects a DCI format in a PDCCH reception that triggers a PUCCH transmission with a Type-3 or enhanced Type-3 HARQ-ACK codebook in a slot, the UE stops the procedure to determine the earliest second slot in that slot.

- The pending SPS HARQ information for deferral is not appended to the Type-3 or enhanced Type 3 CB in that slot.

Agreement

The following TP to 38.213 is endorsed for the editor’s CR.

9.2.5.4 UE procedure for deferring HARQ-ACK for SPS PDSCH If a UE is provided spsHARQdeferral and, after performing the procedures in clauses 9 and 9.2.5 to resolve overlapping among PUCCHs and PUSCHs in a first slot, the UE determines a PUCCH resource for a PUCCH transmission with first HARQ-ACK information bits for SPS PDSCH receptions that the UE would report for a first time, and the PUCCH resource - is provided by SPS-PUCCH-AN-List as described in clause 9.2.1, or by n1PUCCH-AN if SPS-PUCCH-AN-List is not provided - overlaps with a symbol indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigDedicated, or indicated for a SS/PBCH block by ssb-PositionsInBurst, or belonging to a CORESET associated with a Type0-PDCCH CSS set the UE - determines an earliest second slot and, after performing the procedures in clauses 9 and 9.2.5 to resolve overlapping among PUCCHs and PUSCHs, a PUSCH or a PUCCH in the earliest second slot to multiplex HARQ-ACK information bits that include second HARQ-ACK information bits from the first HARQ-ACK information bits - if the UE detects a DCI format in a PDCCH reception that triggers a PUCCH transmission with a Type-3 HARQ-ACK codebook in a slot as described in clause 9.1.4, the UE stops the procedure to determine the earliest second slot in the slot - if the UE is provided a periodic cell switching pattern for PUCCH transmissions by pucch-sSCellPattern, the UE determines the earliest second slot and a corresponding cell based on the periodic cell switching pattern as described in clause 9.A

Agreement

Support the simultaneous configuration of one-shot triggering of HARQ re-transmission and SPS deferral

- One-shot HARQ-ACK re-transmission can trigger re-transmission SPS HARQ-ACK enabled with deferring from the initial SPS HARQ deferral slot.

- If the PUCCH slot indicated by the HARQ_retx_offset is the ‘target’ or earliest ‘second’ slot for SPS HARQ-ACK deferral, the HARQ-ACK CB including the deferred SPS HARQ-ACK bits will be retransmitted in the new retransmission PUCCH triggered by one-shot triggering DCI.

- For the SPS HARQ-ACK deferral procedure, the PUCCH slot with a one-shot triggered HARQ-ACK CB is regarded as a valid potential target PUCCH slot for SPS HARQ-ACK deferral with same PHY priority (at least for operation with Rel-16 PHY prioritization) as the PHY priority of the triggered one-shot HARQ-ACK re-transmission.

    - If the PUCCH slot with a one-shot triggered HARQ-ACK CB is determined by the UE as target or earliest second PUCCH slot for SPS HARQ-ACK deferral, the deferred SPS HARQ-ACK in a target slot is appended to the re-transmitted HARQ-ACK CB and initial, new HARQ-ACK (if any) following the operation of SPS HARQ-ACK deferral procedure.

Re-transmission of cancelled HARQ-ACK:

Agreement

Re-add the RRC parameter for the DCI field configuration in row 17 of the Enh. Type-3 HARQ-ACK codebook for the primary PUCCH cell group (that was lost when moving from v006 to v007 in the final RRC parameter discussions in RAN1#107-e, currently we only have the configuration for the secondary PUCCH cell group) i.e.,

| pdsch-HARQ-ACK-enhType3DCIfield | Enables the enhanced Type 3 CB through a new DCI field to indicate the enhanced Type 3 HARQ-ACK codebook in the primary cell group if the more than one enhanced Type 3 HARQ-ACK codebook is configured for the primary PUCCH cell group. | Enabled |
| --- | --- | --- |

Agreement

Support separate configuration of the DCI field presence for enh. Type 3 HARQ-ACK CB for DCI format 1_2 (i.e. pdsch-HARQ-ACK-enhType3DCIfieldDCI-1-2 as discussed in RAN1#107-e)

Agreement

RAN1 confirms the following RAN1#107-e working assumption:

Working Assumption For one-shot triggering of HARQ re-transmission, in addition to one-shot triggering of HARQ re-transmission after the initial PUCCH transmission slot, the triggering is supported before the initial PUCCH transmission slot - Re-transmission triggering does not change processing for the initial PUCCH transmission (i.e., HARQ multiplexing / dropping / transmission) - The UE expects the PUCCH carrying the HARQ-ACK re-transmission to be scheduled in a slot/sub-slot after the initial PUCCH transmission slot/sub-slot. - The support for the triggering before the initial PUCCH transmission slot is subject to separate UE capability indication

Conclusion

For one-shot HARQ re-transmission on PUCCH, the UE determines no PDSCH is scheduled when the triggering bit is set to ‘1’ (i.e. the UE does not need to in addition check any specific resource allocation setting).

Conclusion

There is no consensus on the support of HARQ-ACK CB size indication in the triggering DCI for HARQ-ACK re-transmission

Conclusion

There is no consensus to support the following in Rel-17:

- For one-shot HARQ re-transmission on PUCCH, if certain HARQ process IDs of the requested HARQ CB to be retransmitted is replaced by new HARQ bits, the UE transmits the new content of HARQ process(es) being updated.

Agreement

For one-shot HARQ-ACK re-transmission, the value range for HARQ re-tx offset is given by [min_HARQ_retx_offset_value, max_HARQ_retx_offset_value] with an indication of 1 slot / sub-slot within that range.

- FFS the fixed value of min_HARQ_retx_offset_value

- FFS the fixed value of max_HARQ_retx_offset_value

Agreement

For one-shot HARQ-ACK re-transmission,

- the minimum value for the HARQ re-tx offset min_HARQ_retx_offset_value is -7.

- the maximum value for the HARQ re-tx offset max_HARQ_retx_offset_value is 24.

- Note: UE capability reporting on the UE supported value range for HARQ_retx_offset in the scope of [min_HARQ_retx_offset_value, max_HARQ_retx_offset_value ] that can be indicated by the gNB for the UE can be further discussed in UE capabilities

Agreement

For one-shot triggering of HARQ-ACK re-transmission, the HARQ_retx_offset is indicated by the bits of the MCS field for transport block 1.

Agreement

For PUCCH repetition and one-shot HARQ-ACK re-transmission, if the gNB triggers the HARQ-ACK CB re-transmission from a PUCCH slot indicated by HARQ_retx_offset where a HARQ-ACK in a first PUCCH is dropped due to overlapping with another, second PUCCH, where the first PUCCH and second PUCCH have the same L1 priority, and at least one of the first PUCCH and the second PUCCH is subject to a repetition, the UE re-transmits the HARQ-ACK CB of the second PUCCH from the slot.

PUCCH repetition enhancements:

None.

Sub-slot based Type-1 codebook:

None.

PUCCH cell switching for HARQ-ACK:

Conclusion

There is no consensus for introducing further specification support for the following

- PUCCH cell switching between cells with shared spectrum channel access (in any mode)

- PUCCH cell switching between a cell with licensed spectrum and a cell with shared spectrum channel access (in any mode)

Conclusion

There is no consensus to support MAC CE activation indicating a set of values of pucch-SpatialRelationInfoId applicable to the alternative PUCCH sSCell for PUCCH cell switching in Rel-17.

Agreement

For dynamic PUCCH cell switching, the Type 1 HARQ-ACK codebook construction is based on the k1 set(s) of the dynamically indicated PUCCH cell.

Agreement

For PUCCH cell switching based on semi-static time domain pattern, the Type 1 HARQ-ACK codebook construction is based on the k1 set(s) of the PCell / SPCell / PUCCH SCell.