3GPP TSG RAN WG1 #108-e		R1-2201100

e-Meeting, February 21st – March 3rd, 2022

Source:	vivo, Guangdong Genius

Title:                     Remaining issues on RedCap half-duplex operation

Agenda Item:	8.6.1.2

Document for:	Discussion and Decision

- Introduction

In the RAN1#107-e meeting, following agreements were made for FDD half-duplex RedCap UEs [1]:

Agreement: - For Case 5 of dynamically scheduled UL transmission vs. SSB, support Option 2 at least for dynamically scheduled UL transmission other than Msg3 (re)transmission and PUCCH for Msg4 - Option 2: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over dynamically scheduled UL transmission Agreement: - For MsgA PUSCH occasion overlapping with dynamic or semi-static DL reception, leave it to UE implementation to prioritize the DL reception or MsgA PUSCH transmission Agreement: - For the case of the “back-to-back” non-overlapping UL/DL without sufficient gap between cell specific configured DL and cell-specific configured UL, e.g., SSB or PDCCH in CSS vs. valid RO, it is up to UE implementation to ensure that the switching time is satisfied Agreement:  The “back-to-back” non-overlapping UL/DL without sufficient gap between cell-specific configured DL and dedicated configured UL may happen, i.e., allowed for HD-FDD UEs - E.g., SSB vs. CG PUSCH, PUCCH or SRS - Configured UL transmission is cancelled (as in the overlapping case) - The “back-to-back” non-overlapping UL/DL without sufficient gap between dedicated configured DL and cell-specific configured UL may happen, i.e., allowed for HD-FDD UEs - E.g., PDCCH in USS, SPS PDSCH, CSI-RS or DL PRS vs. valid RO - Leave it to UE implementation to cancel either DL reception or UL transmission to ensure sufficient switching time Agreement:  No additional UE behavior for DL/UL collision handling is specified in Rel-17 if SFI monitoring is supported for HD-FDD RedCap UEs.

In this contribution, we provide our views on the remaining issues on half duplex FDD operation for RedCap.

- Collison handling for Case 5 of Msg3 (re)transmission and PUCCH for Msg4 vs. SSB

For RedCap HD-FDD operation, one remaining issue is about DL/UL collision handling for Case 5 SSB vs. dynamic scheduled Msg.3 (re)transmission and PUCCH for Msg4. For other dynamic scheduled UL transmissions vs. SSB, it was agreed that SSB is prioritized over dynamically scheduled UL transmission which follows the existing collision handling principles of Rel-15/16 for NR TDD. We failed to find strong motivation/benefit to have a special or different handling for Msg.3 (re)transmission and PUCCH for Msg4 vs. SSB. Therefore, we propose to have a unified solution for all dynamic scheduled UL transmissions vs. SSB.

Proposal: For Case 5 of SSB overlapping with Msg3 (re)transmission and PUCCH for Msg4, reuse the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over Msg3 and PUCCH for Msg4.

- Conclusion

This contribution discusses one remaining issue for HD-FDD operation for RedCap. The proposal is following:

Proposal: For Case 5 of SSB overlapping with Msg3 (re)transmission and PUCCH for Msg4, reuse the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over Msg3 and PUCCH for Msg4.

6 	Reference

- 3GPP RAN1#107-e meeting, Chairman’s notes.