3GPP TSG RAN WG1#108-e		R1-2201103

e-Meeting, February 21st – March 3rd, 2022

Source:	vivo

Title:	Remaining issues on DCI-based power saving adaptation in connected mode

Agenda Item:	8.7.2

Document for:	Discussion and Decision

- Introduction

In this contribution, we further discuss the remaining issues for completion of the PDCCH monitoring adaptation.

- Remaining issues for PDCCH monitoring adaptation

  - Monitoring Type 0/1/1A/2 CSS

The main use case as argued in the discussion from previous meetings is that when the UE has not been provided a Type3-PDCCH CSS set or a USS set and the UE has received a C-RNTI and has been provided a, e.g., Type1-PDCCH CSS set, the UE monitors PDCCH candidates for DCI format 0_0 and DCI format 1_0 with CRC scrambled by the C-RNTI in the Type1-PDCCH CSS set.

If we do not allow skipping Type1-PDCCH CSS set, then in such case, the power saving benefit cannot been provided.

However, it seems further clarification is needed on the use case why the network does not provide a Type3-PDCCH CSS set or a USS set. If it is provided, then UE does not need to monitoring the Type 0/1/1A/2 CSS so frequently.

Proposal 1: Type0/0A/1/2-PDCCH CSS monitoring is not impacted by Rel-17 PDCCH monitoring adaptation

  - Timer triggered for SSSGs switching

Timer behaviors

In the last meeting, it is finally agreed as follows, need to down-select in this meeting.

Agreement If a UE is provided with a timer value by searchSpaceSwitchTimer-r17 for PDCCH monitoring on a serving cell and the timer is running, the UE - resets the timer after a slot of the active DL BWP of the serving cell when the UE detects a DCI format in a PDCCH reception in the slot o Alt 2a: for the Type3-PDCCH CSS set or the USS set with group index of either 1 or 2 o Alt 2b: for the Type3-PDCCH CSS set or the USS set o Alt 2c: with CRC scrambled by C-RNTI/CS-RNTI/MCS-C-RNTI - otherwise, decrease the timer value by one after each slot. - FFS: When the timer expires in a slot

Firstly, for clarification, Alt 2c is also restricted in the Type3-PDCCH CSS set and USS set considering the current agreement says the PDCCH monitoring applies to Type3-PDCCH CSS set and USS set. Whether others can be adapted need more discussion.

And for Alt 2c, the PDCCH monitoring is adapted according to the ‘traffic’ (which says the CRC scrambled by  C-RNTI/CS-RNTI/MCS-C-RNTI). We think this seems to be a reasonable to reset timer. Different to Rel-16 timer, which is used to align with the gNB and UE behaviour, In Rel-17, the SSSG switching is based on scheduling DCI which relies on HARQ-ACK. It is main used for counting how much time the UE is free of ‘traffic’ arrival.

Proposal 2:

- If a UE is provided with a timer value by searchSpaceSwitchTimer-r17 for PDCCH monitoring on a serving cell and the timer is running, the UE resets the timer after a slot of the active DL BWP of the serving cell when the UE detects a DCI format in a PDCCH reception in the slot for the Type3-PDCCH CSS set or the USS set with CRC scrambled by C-RNTI/CS-RNTI/MCS-C-RNTI.

- Otherwise, decrease the timer value by one after each slot.

Secondly, for timer expiry, one of the discussing point is as follows,

When the timer expires in a slot:

- Alt 1: UE switches to the default SSSG (SSSG 0) when it receives the indication of PDCCH skipping

- Alt 2:

  - If the UE has not been indicated skipping PDCCH monitoring for a duration overlapping in time with the slot, the UE monitors PDCCH on the serving cell according to search space sets with group index 0;

  - If the UE has been indicated skipping PDCCH monitoring for a duration overlapping in time with the slot, the UE starts to monitor PDCCH until the completion of the PDCCH skipping for the duration on the serving cell according to search space sets with group index 0.

Alt 2 allows the network can indicate a PDCCK skipping duration longer than the timer duration. However, the case when UE misses skipping DCI need to be considered. In such case, the UE behaviour may be ambiguous. And the benefits of such design may be not clear.

Proposal 3: When the timer expires in a slot, UE switches to the default SSSG (SSSG 0) when it receives the indication of PDCCH skipping

  - Exception cases for PDCCH skipping

- SR and PDCCH skipping

RAN 2 LS [R1-2200884] states RAN2 discussed and propose that UE ignores PDCCH skipping while the SR is pending.

According to the RAN2 proposed ‘UE ignores PDCCH skipping while the SR is pending’, it is still unclear the following two cases,

- Case 1: Whether the PDCCH skipping indication is applied before the SR is pending, or

- Case 2: Whether the PDCCH skipping indication is applied after the SR pending occurs

For case 1, it is natural that the prior PDCCH skipping indication should be discard since the SR transmission (from RAN1 description perspective) occurs. Hence a simple description as follows would be proper.

If PDCCH skipping is applied/activated, UE starts to performs Beh 1 at the first slot after the last OFDM symbol of the SR transmission.

For case 2, the PDCCH indication is after the SR transmission, it seems that it should depend on gNB implementation if gNB wants UE to skip the PDCCH monitoring after a SR transmission. It is also not clear whether L1 should apply PDCCH skipping or not since L1 may not have the knowledge whether SR pending is terminated or not.

Hence, follow the principle ‘If PDCCH skipping is applied/activated, UE starts to performs Beh 1 at the first slot after the last OFDM symbol of the SR transmission’ can leave the gNB/UE flexibility that after SR transmission, if UE received a Grant (Figure 1) with skipping indication, the UE can still skip PDCCH monitoring to save power.

Figure 1. gNB scheduled PDCCH skipping during SR transmission (case 2)

Secondly, RAN1 has not agreed on any indication to higher layer of the PDCCH skipping or SSSG switching state. Hence, it is transparent to MAC layer. Capturing the above agreement in RAN1 specification is preferred.

Proposal 4: Agree the following conclusions and capture it in the TS38.213.

-  If PDCCH skipping is activated, PDCCH skipping is not activated (i.e., Beh 1) at the first slot after the last OFDM symbol of the SR transmission.

- RACH and PDCCH skipping

RAN 2 agrees the followings related to RACH procedure and ask whether RAN1 prefers to capture the above agreements in RAN1 specification or prefer these to be captured in MAC specification.

-  If PDCCH skipping is applied to RNTI(s) monitored during RAR/MsgB window, the UE ignores PDCCH skipping during the RAR/MsgB window.

-  UE ignores PDCCH skipping while contention resolution timer is running.

RAN1 has not agreed on any indication to higher layer of the PDCCH skipping or SSSG switching state and it can be properly handled in physical layer is feasible. Hence, capturing the above agreement in RAN1 specification is preferred.

From RAN1 perspective, during RAR/MsgB window or contention resolution timer, UE only ignores the PDCCH skipping for RACH procedure related PDCCH monitoring, other PDCCH monitoring behaviour not related to this RACH procedure should keep unchanged.

Hence, the following PDCCH monitoring is impacted during RAR/MsgB window or contention resolution timer, i.e., PDCCH skipping is ignored.

-  a DCI format in response to a PRACH transmission (i.e., RAR), or

-  a DCI format in response to a transmission of a PRACH and a PUSCH, or to a transmission of only a PRACH if the PRACH preamble is mapped to a valid PUSCH occasion (i.e., MsgB), or

-  a DCI format in response to a PUSCH transmission scheduled by a RAR UL grant (i.e. Msg4 when contention resolution timer is running)

Proposal 5: Agree the following conclusions and capture it in the TS38.213.

If a UE received a DCI format indicating skipping PDCCH monitoring for a duration that overlaps with at least one symbol of a window provided by ra-ResponseWindow or msgB-ResponseWindow or the duration where ra-ContentionResolutionTimer is running which is controlled by higher layers [11, TS 38.321], the UE skips monitoring PDCCH according to Type3-PDCCH CSS sets or USS sets on the serving cell, except for monitoring a DCI format in response to

- a PRACH transmission, or

- a transmission of a PRACH and a PUSCH, or to a transmission of only a PRACH if the PRACH preamble is mapped to a valid PUSCH occasion, or

- a PUSCH transmission scheduled by a RAR UL grant

- Rel-16 DCP and PDCCH skipping

RAN2 also discussed the impact of PDCCH skipping on DCP monitoring. If DCP monitoring occasions overlap with PDCCH skipping duration, UE cannot monitor DCP. To address this, RAN2 has discussed following approaches in RAN2 LS to RAN 1 [R1-2200884]:

Approach 1: If UE can not monitor DCP due to PDCCH skipping, physical layer of UE does not report Wake-up indication bit to higher layer. Higher layer reuses the ps-Wakeup to determine whether to start the drx-onDurationTimer or not.

Approach 2: If UE can not monitor DCP due to PDCCH skipping, physical layer of UE reports a value of 1 for Wake-up indication bit to higher layer.

Approach 3: PDCCH skipping only applies in Active Time and hence DCP cannot be missed due to PDCCH skipping.

Question 3: RAN2 would like to ask RAN1 whether a) Physical layer of UE reports a value of 1 for Wake-up indication bit to higher layer or b) Physical layer of UE does not report Wake-up indication bit to higher layer, in case UE cannot monitor DCP due to PDCCH skipping; or if Approach 3 can be assumed.

Current TS38.213 section 10.3 provides description of invalid WUS monitoring as follows,

TS38.213 section 10.3 If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE - is not required to monitor PDCCH for detection of DCI format 2_6, as described in Clauses 10, 11.1, 12, and in Clause 5.7 of [11, TS 38.321] for all corresponding PDCCH monitoring occasions outside Active Time prior to a next long DRX cycle, or - does not have any PDCCH monitoring occasions for detection of DCI format 2_6 outside Active Time of a next long DRX cycle the physical layer of the UE reports a value of 1 for the Wake-up indication bit to higher layers for the next long DRX cycle.

PDCCH skipping is described in section 10. So when UE is not required to monitor PDCCH for detection of DCI format 2_6, as described in Clauses 10, 11.1, 12, the current specification defines the UE behavior is Approach 2. No additional specification impact is needed.

Proposal 6: the current specification defines the UE behavior for PDCCH skipping and DCP overlap is Approach 2. No additional specification impact is needed.

- MBS and PDCCH skipping

PDCCH skipping can be applied to Type 3 CSS as follows,

-	a Type3-PDCCH CSS set configured by

-	SearchSpace in PDCCH-Config with searchSpaceType = common for DCI formats with CRC scrambled by INT-RNTI, SFI-RNTI, TPC-PUSCH-RNTI, TPC-PUCCH-RNTI, TPC-SRS-RNTI, or CI-RNTI and, only for the primary cell, C-RNTI, MCS-C-RNTI, CS-RNTI(s), or PS-RNTI, or

-	SearchSpace-Multicast in PDCCH-Config-Multicast for DCI formats with CRC scrambled by G-RNTI, or G-CS-RNTI, or

-	searchSpaceBroadcast in pdcch-Config-MCCH and pdcch-Config-MTCH on a secondary cell for a DCI format 4_0 with CRC scrambled by a MCCH-RNTI or a G-RNTI, and

Multicast and broadcast are specified in Rel-17. It is agreed in RAN2 that separate DRX configuration is for multicast and broadcast. Hence, independent control of the PDCCH for unicast and multicast/broadcast is pursued. The PDCCH skipping should not applied to PDCHC for multicast and broadcast, i.e., PDCCH skipping is not applied to DCI format with CRC scrambled by G-RNTI, or G-CS-RNTI, or MCCH-RNTI

Proposal 7: Agree the following conclusions and capture it in the TS38.213.

If a UE is indicated to skip PDCH for a duration, the UE skips monitoring PDCCH according to Type3-PDCCH CSS sets or USS sets on the serving cell, except for monitoring a DCI format with CRC scrambled by G-RNTI, or G-CS-RNTI, or MCCH-RNTI.

  - Application delay and interaction with other procedure

For SSSG switching, it can be seen that UE is still possible to monitor PDCCH sparsely during the retransmission period (either by configuration or switching to a more sparse SSSG for the last packet scheduling), therefore, it is suggested that SSSG switching interacted with HARQ-ACK retransmission is not considered in Rel-17.

For PDCCH skipping, in order to address the retransmission latency issues, PDCCH monitoring adaptation with HARQ interaction need to be considered. So far, RAN1 does not have any L1 indication to higher layer of the PDCCH skipping or SSSG switching states. To simplify the design and avoid cross-layer interaction, e.g., MAC and PHY, it is suggested the solution is transparent to MAC layer. For Alt 2 (skipping indication applied after HARQ-ACK transmission) and Alt 3 (skipping indication applied before HARQ-ACK transmission), the main differences seems to be power saving gain but considers Alt 3 has more complicated specification impact w.r.t. UE behaviour and state machine control , we expect Alt 2 to be good compromise to move forward.

Proposal 8:

For PDCCH monitoring adaptation for SSSG switching,

- Interaction with retransmission by PDCCH monitoring adaptation for SSSG switching is NOT supported.

- down-select one of the following options:

- Alt 1a: the UE applies Beh 1A on the serving cell at the first slot after the last OFDM symbol of the PDCCH transmission.

- Alt 1b: the UE applies Beh 1A on the serving cell is applied in the next Zµ slot, where Definition of Zµ is described in Table 5.3.1-1 in TS38.214

For PDCCH monitoring adaptation for PDCCH skipping,

- the UE applies Beh 1A next slot after the last OFDM symbol of ACK transmission/PUSCH transmission, otherwise the indication is not applied.

  - DCI field

How many bits for PDCCH monitoring adaptation if PDCCHSkippingDurationList is not configured and if searchSpaceGroupIdList-r17 is configured is still remains open.

Three options are considered during the discussion as follows,

For DCI format 0_1, DCI format 0_2, DCI format 1_1 and DCI format 1_2,

- 1 or 2 bits, if PDCCHSkippingDurationList is not configured and if searchSpaceGroupIdList-r17 is configured

Option 1:

-     1 bit if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0 and search space set(s) with group index 1, and if the UE is not configured by searchSpaceGroupIdList-r17 with any search space set with group index 2;

-     2 bits if the UE is configured by searchSpaceGroupList-r17 with search space set(s) with group index 2.

Option 2:

-     1 bit if the UE is not configured by searchSpaceGroupIdList-r17 with any search space set with group index 2;

-     2 bits if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 2;

Option 3:

-     1 bit if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0 and search space set(s) with group index 1, and if the UE is not configured by searchSpaceGroupIdList-r17 with any search space set with group index 2;

-     2 bits if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0, search space set(s) with group index 1 and search space set(s) with group index 2;

For searchSpaceGroupIdList-r17, it is agreed that it is any combination of {0,…,maxSearchSpaceGroups-r17-1}, hence it is not precluded that the UE may not be configured with any SSSG#0,1 but has SSSG#2.

Option 2 provides maximum flexibility of configuration and better usage.

However, during the last meeting discussion, it is also proposed by other companies that an explicit indication of the number of SSSG. The main reason can be to avoid an ambiguity in deciding the number of configured SSSG when an empty SSSG is allowed [R1-2100608]. It is also feasibility and a clear solution.

Proposal 9: adopt either one of the following text proposals in TS38.212

For DCI format 0_1, DCI format 0_2, DCI format 1_1 and DCI format 1_2,

- 1 or 2 bits, if PDCCHSkippingDurationList is not configured and if searchSpaceGroupIdList-r17 is configured

  - Option A:

-     1 bit if the UE is not configured by searchSpaceGroupIdList-r17 with any search space set with group index 2;

-     2 bits if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 2;

  - Option B:

-	1 bit if the UE is configured numOfSSSG = 2

-	2 bits if the UE is configured numOfSSSG = 3

-  Note: numOfSSSG is per BWP configured

  - Handling of the indication change

Applying another PDCCH monitoring adaptation before an old PDCCH monitoring adaptation indication being applied should be excluded in order to avoid chain reaction. From specification perspective, it is to say that when the UE is scheduled with DCI format 0_1 /1_1/0_2/1_2 with a 'PDCCH monitoring adaptation indication’ field in slot n, the UE does not expect to be scheduled 0_1 /1_1/0_2/1_2 with a 'PDCCH monitoring adaptation indication’ field indicating change to another SSSG or skipping for the same active BWP of the scheduling cell before slot n+X of the scheduling cell, where X is the value of the application delay.

Proposal 10: adopt the following text proposals in TS38.213

When the UE is scheduled with DCI format 0_1 /1_1/0_2/1_2 with a 'PDCCH monitoring adaptation indication’ field in slot n, the UE does not expect to be scheduled 0_1 /1_1/0_2/1_2 with a 'PDCCH monitoring adaptation indication’ field indicating change to another SSSG or skipping for the same active BWP of the scheduling cell before slot n+X of the scheduling cell, where X is the value of the application delay.

- Conclusion

In this contribution, the following are proposed,

Proposal 1: Type0/0A/1/2-PDCCH CSS monitoring is not impacted by Rel-17 PDCCH monitoring adaptation

Proposal 2:

- If a UE is provided with a timer value by searchSpaceSwitchTimer-r17 for PDCCH monitoring on a serving cell and the timer is running, the UE resets the timer after a slot of the active DL BWP of the serving cell when the UE detects a DCI format in a PDCCH reception in the slot for the Type3-PDCCH CSS set or the USS set with CRC scrambled by C-RNTI/CS-RNTI/MCS-C-RNTI.

- Otherwise, decrease the timer value by one after each slot.

Proposal 3: When the timer expires in a slot, UE switches to the default SSSG (SSSG 0) when it receives the indication of PDCCH skipping

Proposal 4: Agree the following conclusions and capture it in the TS38.213.

-  If PDCCH skipping is activated, PDCCH skipping is not activated (i.e., Beh 1) at the first slot after the last OFDM symbol of the SR transmission.

Proposal 5: Agree the following conclusions and capture it in the TS38.213.

If a UE received a DCI format indicating skipping PDCCH monitoring for a duration that overlaps with at least one symbol of a window provided by ra-ResponseWindow or msgB-ResponseWindow or the duration where ra-ContentionResolutionTimer is running which is controlled by higher layers [11, TS 38.321], the UE skips monitoring PDCCH according to Type3-PDCCH CSS sets or USS sets on the serving cell, except for monitoring a DCI format in response to

- a PRACH transmission, or

- a transmission of a PRACH and a PUSCH, or to a transmission of only a PRACH if the PRACH preamble is mapped to a valid PUSCH occasion, or

- a PUSCH transmission scheduled by a RAR UL grant

Proposal 6: the current specification defines the UE behavior for PDCCH skipping and DCP overlap is Approach 2. No additional specification impact is needed.

Proposal 7: Agree the following conclusions and capture it in the TS38.213.

If a UE is indicated to skip PDCH for a duration, the UE skips monitoring PDCCH according to Type3-PDCCH CSS sets or USS sets on the serving cell, except for monitoring a DCI format with CRC scrambled by G-RNTI, or G-CS-RNTI, or MCCH-RNTI.

Proposal 8:

For PDCCH monitoring adaptation for SSSG switching,

- Interaction with retransmission by PDCCH monitoring adaptation for SSSG switching is NOT supported.

- down-select one of the following options:

- Alt 1a: the UE applies Beh 1A on the serving cell at the first slot after the last OFDM symbol of the PDCCH transmission.

- Alt 1b: the UE applies Beh 1A on the serving cell is applied in the next Zµ slot, where Definition of Zµ is described in Table 5.3.1-1 in TS38.214

For PDCCH monitoring adaptation for PDCCH skipping,

- the UE applies Beh 1A next slot after the last OFDM symbol of ACK transmission/PUSCH transmission, otherwise the indication is not applied.

Proposal 9: adopt either one of the following text proposals in TS38.212

For DCI format 0_1, DCI format 0_2, DCI format 1_1 and DCI format 1_2,

- 1 or 2 bits, if PDCCHSkippingDurationList is not configured and if searchSpaceGroupIdList-r17 is configured

  - Option A:

-     1 bit if the UE is not configured by searchSpaceGroupIdList-r17 with any search space set with group index 2;

-     2 bits if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 2;

  - Option B:

-	1 bit if the UE is configured numOfSSSG = 2

-	2 bits if the UE is configured numOfSSSG = 3

-  Note: numOfSSSG is per BWP configured

Proposal 10: adopt the following text proposals in TS38.213

When the UE is scheduled with DCI format 0_1 /1_1/0_2/1_2 with a 'PDCCH monitoring adaptation indication’ field in slot n, the UE does not expect to be scheduled 0_1 /1_1/0_2/1_2 with a 'PDCCH monitoring adaptation indication’ field indicating change to another SSSG or skipping for the same active BWP of the scheduling cell before slot n+X of the scheduling cell, where X is the value of the application delay.

- References

- RP-200938, Revised WID UE Power Saving Enhancements for NR, MediaTek

- 3GPP TS 38.213 v16.1.0.

- 3GPP TR 38.840 Study on UE power saving

- R1-2112878, 	Final FL summary of DCI-based power saving adaptation, Moderator (vivo), RAN1#107-E

- R1-2108988, Discussion on DCI-based power saving adaptation in connected mode, vivo, RAN1#106bis-E

- R1-2201047	Discussion on PDCCH skipping, vivo, RAN1#108-E

- R1-2201048	Draft LS reply on PDCCH skipping, vivo, RAN1#108-E