3GPP TSG RAN WG1 #108-e	                                                                                   R1-2201104

e-Meeting, February 21st – March 3rd, 2022

Source:	vivo

Title:	Remaining issues on enhancement for PUSCH repetition type A

Agenda Item:	8.8.1.1

Document for:	Discussion and Decision

# Introduction

In RAN1#107bis-e meeting, some remaining issues are further discussed in this contribution for enhancements of Type A PUSCH repetition, including

- available slot determination for inter-cell mTRP or CA configuration,

- text proposal of 38.214 on capturing inter-slot FH of Msg3 repetitions.

# Available slot determination in case of inter-cell mTRP or CA configuration

In RAN1#106bis-e meeting, it has been agreed that only tdd-UL-DL-ConfigurationCommon, tdd-UL-DL-ConfigurationDedicated and ssb-PositionsInBurst are considered for the determination of available slots.

Agreement (RAN1#106bis-AI 8.8.1.1) - Only tdd-UL-DL-ConfigurationCommon, tdd-UL-DL-ConfigurationDedicated and ssb-PositionsInBurst are considered for the determination of available slots. - Any other RRC configuration is not considered for the determination of available slots.

For inter-cell mTRP case or CA case, ssb-PositionsInBurst may be configured for different mTRPs and different serving cells respectively. Whether all of these SSBs should be considered for available slot determination for Type A PUSCH repetition transmissions in one serving cell should be discussed and concluded in RAN1. These 2 cases will be discussed in the following 2 sections.

## 2.1 SSB for available slot determination in multiple TRPs

In NR Rel-17 feMIMO work item, following agreements have been made with respect to different SSB sets with different PCIs transmitted from mTRPs in a same serving cell.

Agreement (RAN1#106bis-AI 8.1.2.2) - Center frequency, SCS, SFN offset are assumed to be the same for SSBs from the serving cell and the configured SSBs with PCI different from the serving cell for inter-cell multi TRP operation. - The information related to “SSB time domain position” for SSB with PCI different from the serving cell consists of [halfFrameIndex and] ssb-PositionsInBurst Agreement (RAN1#106bis-AI 8.1.2.2) Support two independent X values (X1, X2) are reported as a UE capability for two different assumptions on additional SSB time domain position and periodicity with respect to serving cell SSB. - X1 (Case 1)= The maximum number of configured additional PCIs when each configuration of SSB time domain positions and periodicity of the additional PCIs is the same as SSB time domain positions and periodicity of the serving cell PCI - X2 (Case 2)= The maximum number of configured additional PCIs when the configurations of SSB time domain positions and periodicity of the additional PCIs is not according to Case 1 - Note: By definition, Case 1 and Case 2 cannot be enabled simultaneously - Supported values for X1 and X2 includes at least 0,1,2,3 and 7. FFS on other values - This UE capability has FR1 and FR2 differentiation (FFS : Whether this UE capability is per UE or per band)

According to above agreements, for multi-TRP inter-cell operation, UE may be configured with SSBs on different time domain positions for different TRPs. One of the motivations for multi-TRP inter-cell operation is also coverage enhancement, in which case other coverage enhancement features, e.g. counting PUSCH repetition based on available slots, may also be enabled at same time. Hence, it should be discussed in RAN1 on whether all the multiple sets of SSBs  transmitted from multiple TRPs in the same serving cell should be considered with respect to available slot determination for Type A PUSCH repetition.

In RAN1 #107bis-e meetings, companies have different understandings on this issue (issue 2-2 in [1]). In our view, there could be 3 options listed below, where option 1 means only the SSBs with serving cell PCI are considered for available slot determination, option 2 means SSBs with PCI different from the serving cell PCI are also considered for available slot determination on top of considering SSBs with serving cell PCI, option 3 means the SSBs with a PCI are considered for available slot determination if the PCI is the associated with the TCI state configured for the PUSCH transmission.

- Option 1: SSBs with PCI different from the serving cell PCI are only used for actual PUSCH repetition determination after the available slots are determined

- Option 2: Both SSBs with serving cell PCI and SSBs with PCI different from the serving cell PCI are used for the available slot determination

- Option 3: SSBs with the same PCI as the SSB used to determine the spatial relation of the PUSCH, are used to determine the available slots. Other set of SSBs are used for actual PUSCH repetition determination after the available slots are determined.

According to following excerpt from TS38.214 v17.0.0, PUSCH spatial Tx filter for PUSCH transmission can be determined according to SSBs associated with  different PCI from the PCI of the serving cell, i.e., SSBs from a second TRP.

Section 6.1 of TS 38.214 v17.0.0 ...... When the UE is configured [TCI-State](s) with [tci-StateId_r17] for UL, the UE shall perform PUSCH transmission corresponding to a Type 1 configured grant or a Type 2 configured grant or a dynamic grant according to the spatial relation, if applicable, with a reference to the RS for determining UL Tx spatial filter or the RS configured with qcl-Type set to 'typeD' of the indicated [TCI-State] with [tci-StateId_r17]. The reference RS can be a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info or, in case [TCI-State] with [tci-StateId_r17] is for UL only, an SRS resource with the higher layer parameter usage set to 'beamManagement', or SS/PBCH block associated with the same or different PCI from the PCI of the serving cell.

When UL beam for PUSCH transmission is determined based on SSBs from a second TRP, the PUSCH transmission is targeted for the second TRP. In such case, it does not seem to make sense to only determine the available slots for this PUSCH transmission based on the SSBs from the serving cell TRP, i.e., SSBs with the serving cell PCI.

Observation 1:

- In current specification, UL beam for PUSCH transmission can be determined based on the SSBs from a second TRP with different PCI. In such case, it is more reasonable to consider at least such SSBs for available slots determination.

Hence, we prefer either option 2 or option 3, and propose RAN1 to agree on one of the options to apply for the determination of enhanced Type A PUSCH repetition transmission.

Proposal 1:

- RAN1 should further discuss and down-select from the following 3 options for Type A PUSCH repetition available slot determination with respect to the multiple SSBs with different PCIs from mTRPs:

  - Option 1: SSBs with PCI different from the serving cell PCI are only used for actual PUSCH repetition determination after the available slots are determined

  - Option 2: Both SSBs with serving cell PCI and SSBs with PCI different from the serving cell PCI are used for the available slot determination

  - Option 3: SSBs with the same PCI as the SSB used to determine the spatial relation of the PUSCH, are used to determine the available slots. Other set of SSBs are used for actual PUSCH repetition determination after the available slots are determined.

## 2.2 SSB for available slot determination in CA

Another remaining issue is the available slot determination for Type A PUSCH with respect to multiple SSBs transmitted from different CCs in carrier aggregation case.

As is known for single carrier TDD case, according to following text copied from section 11.1 of 38.213 v17.0.0, the PUSCH transmission will be dropped when colliding with SSB transmissions configured for the serving cell.

For operation on a single carrier in unpaired spectrum, for a set of symbols of a slot indicated to a UE by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon, for reception of SS/PBCH blocks, the UE does not transmit PUSCH, PUCCH, PRACH in the slot if a transmission would overlap with any symbol from the set of symbols and the UE does not transmit SRS in the set of symbols of the slot. The UE does not expect the set of symbols of the slot to be indicated as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, when provided to the UE.

For CA case (inter-band or intra-band), the collision handling is performed according to following text in the same section of 38.213 v17.0.0.

If a UE - is configured with multiple serving cells and is provided with directionalCollisionHandling-r16 = 'enabled' for a set of serving cell(s) among the multiple serving cells, and - indicates support of half-DuplexTDD-CA-SameSCS-r16 capability, and - is not configured to monitor PDCCH for detection of DCI format 2_0 on any of the multiple serving cells, for a set of symbols of a slot that are indicated to the UE for reception of SS/PBCH blocks in a first cell of the multiple serving cells by ssb-PositionsInBurst in SystemInformationBlockType1 or by ssb-PositionsInBurst in ServingCellConfigCommon, when provided to the UE, the UE does not transmit PUSCH, PUCCH, or PRACH in the slot if a transmission would overlap with any symbol from the set of symbols, and the UE does not transmit SRS in the set of symbols of the slot in - any of the multiple serving cells if the UE is not capable of simultaneous transmission and reception as indicated by simultaneousRxTxInterBandCA among the multiple serving cells, and - any one of the cells corresponding to the same band as the first cell, irrespective of any capability indicated by simultaneousRxTxInterBandCA.

In RAN1 #107bis-e, in the discussions on this issue (issue 2-3 in [1]), there’s no conclusion made. It is still necessary to make conclusion that legacy rules mentioned above are reused after available slot is determined for Type A PUSCH repetition transmissions, i.e. only the SSBs transmitted in the serving cell are considered for available slot determination for the Type A PUSCH repetition transmissions.

Proposal 2:

- RAN1 to conclude that only the SSBs transmitted within the serving cell are considered for available slot determination for the Type A PUSCH transmissions if CA is configured.

# Text proposal for FH of Msg3 repetition

In RAN1 #107bis-e meeting, it has been identified that something is wrong when both Msg3 repetition and TboMS were captured in this section 6.3.1 of 38.214 V17.0.0 for Rel-17. Some TPs were discussed without any conclusion in the end.

In our view, the original intention of the TP is to preclude Msg3 repetition (treated as a kind of PUSCH repetition Type A) when describing the usage of frequencyHopping and frequencyHoppingDCI-0-2 provided in RRC configurations which are not intended for Msg3 repetition. It would be natural to simply preclude Msg3 directly as is shown in TP#1.

------------------------------------ start of TP1 to section 6.3.1 of 38.214 V17.0.0 ---------------------------------------------------

### 6.3.1	Frequency hopping for PUSCH repetition Type A and for TB processing over multiple slots

For PUSCH repetition Type A other than the PUSCH scheduled by  0_1 or 0_2 RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI and for TB processing over multiple slots (as determined according to procedures defined in Clause 6.1.2.1 for scheduled PUSCH, or Clause 6.1.2.3 for configured PUSCH), a UE is configured for frequency hopping by the higher layer parameter frequencyHoppingDCI-0-2 in pusch-Config for PUSCH transmission scheduled by DCI format 0_2, and by frequencyHopping provided in pusch-Config for PUSCH transmission scheduled by a DCI format other than 0_2, and by frequencyHopping provided in configuredGrantConfig for configured PUSCH transmission. For PUSCH repetition Type A scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, a UE is configured for frequency hopping by the frequency hopping flag information field of the RAR UL grant, and by the frequency hopping flag information field of DCI format 0_0 with CRC scrambled by TC-RNTI, respectively. One of two frequency hopping modes can be configured:

-	Intra-slot frequency hopping, applicable to single slot and multi-slot configured PUSCH transmission and multi-slot PUSCH transmission scheduled by DCI format 0_1 or 0_2 and each of multiple PUSCH transmissions scheduled by a DCI if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is configured.

-	Inter-slot frequency hopping, applicable to multi-slot PUSCH transmission.

------------------------------------ end of TP1 to section 6.3.1 of 38.214 V17.0.0 ---------------------------------------------------

Another TP proposed in the end of the meeting is to include some text to support Type A PUSCH repetition scheduled by DCI 0_0, which tries to capture all DCI formats except Msg3. However, repetition is not supported for PUSCH scheduled by DCI format 0_0 with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI since the pusch-AggregationFactor is only applicable to PUSCH scheduled by non-fallback DCI. This TP is not correct in our understanding.

For PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2 and for TB processing over multiple slots (as determined according to procedures defined in Clause 6.1.2.1 for scheduled PUSCH, or Clause 6.1.2.3 for configured PUSCH) and for PUSCH repetition Type A scheduled by DCI format 0_0 with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI

According to above and considering intra-slot FH configuration should be captured somehow in the spec. for a normal PUSCH (not Msg3) scheduled by fallback DCI though such PUSCH is not repeated, we have following proposal.

Proposal 3:

- RAN1 to adopt TP1 to section 6.3.1 of 38.213 v17.0.0 for correction of FH of Type A PUSCH repetition.

# Conclusion

In this contribution, we discuss the remaining issues of enhancement on PUSCH repetition type A, and have following  observations and proposals:

Observation 1:

- In current specification, UL beam for PUSCH transmission can be determined based on the SSBs from a second TRP with different PCI. In such case, it is more reasonable to consider at least such SSBs for available slots determination.

Proposal 1:

- RAN1 should further discuss and down-select from the following 3 options for Type A PUSCH repetition available slot determination with respect to the multiple SSBs with different PCIs from mTRPs:

  - Option 1: SSBs with PCI different from the serving cell PCI are only used for actual PUSCH repetition determination after the available slots are determined

  - Option 2: Both SSBs with serving cell PCI and SSBs with PCI different from the serving cell PCI are used for the available slot determination

  - Option 3: SSBs with the same PCI as the SSB used to determine the spatial relation of the PUSCH, are used to determine the available slots. Other set of SSBs are used for actual PUSCH repetition determination after the available slots are determined.

Proposal 2:

- RAN1 to conclude that only the SSBs transmitted within the serving cell are considered for available slot determination for the Type A PUSCH transmissions if CA is configured.

Proposal 3:

- RAN1 to adopt TP1 to section 6.3.1 of 38.213 v17.0.0 for correction of FH of Type A PUSCH repetition.

# References

- R1-2200789, FL Summary #4 on Enhancements on PUSCH repetition type A, 3GPP TSG RAN WG1 #107bis-e, January, 2022.